3GPP TSG RAN WG1 #115		R1-2311849

Chicago, USA, November 13th – November 17th, 2023

Agenda item:	8.5.1

Source:	Samsung

Title:	Remaining issues on NES techniques in spatial and power domains

Document for:	Discussion and decision

# Introduction

In this contribution, we discuss remaining issues on NES techniques in spatial domain (SD) and power domain (PD) for the maintenance of the Rel-18 NES WI.

# Remaining issues on CSI resource and report configurations

## Non-PMI Port Indication

In RAN1#114-bis [1], the following agreement has been made regarding reportQuantity for Rel-18 NES.

Agreement Support gNB can configure report quantities of 'cri-RI-i1-CQI', 'cri-RI-CQI', or 'cri-RI-i1'.

Accordingly, for NES CSI report, non-PMI based CSI feedback can be performed when the higher layer parameter reportQuantity set to ‘cri-RI-CQI’. For non-PMI based CSI feedback, gNB can configure the higher layer parameter non-PMI-PortIndication, which contains a sequence indicating CSI-RS ports for each layer for RI/CQI calculation. In case of type 1 SD adaptation, gNB can provide multiple sub-configurations to a UE, which indicates antenna port subset using the higher layer bitmap parameter [port-subsetIndicator]. Then, there is a remaining question on how to apply the non-PMI-PortIndication to multiple sub-configurations. One simple approach is to separately provide non-PMI-PortIndication for each sub-configuration for type 1 SD adaptation. When a UE is not configured with non-PMI-PortIndication, non-PMI based CSI feedback also can be supported with following TP.

Proposal 1: Adopt the following TP for TS 38.214 Clause 5.2.1.4.2 Report Quantity Configurations for providing separate non-PMI-PortIndication configurations for each CSI report sub-configuration.

TP for TS 38.214 Clause 5.2.1.4.2 Report Quantity Configurations <omitted texts> If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-CQI', - if the UE is configured with higher layer parameter non-PMI-PortIndication contained in a CSI-ReportConfig, r ports are indicated in the order of layer ordering for rank r and each CSI-RS resource in the CSI resource setting is linked to the CSI-ReportConfig based on the order of the associated NZP-CSI-RS-ResourceId in the linked CSI resource setting for channel measurement given by higher layer parameter resourcesForChannelMeasurement. The configured higher layer parameter non-PMI-PortIndication contains a sequence of port indices, where are the CSI-RS port indices associated with rank ν and where is the number of ports in the CSI-RS resource. The UE shall only report RI corresponding to the configured fields of PortIndexFor8Ranks. If the UE is configured with a CSI-ReportConfig that contains a list of sub-configurations, the higher layer parameter non-PMI-PortIndication is separately provided for each sub-configuration and P corresponds to the number of bits with value 1 in the bitmap [port-subsetIndicator] for the corresponding sub-configuration. - if the UE is not configured with higher layer parameter non-PMI-PortIndication, the UE assumes, for each CSI-RS resource in the CSI resource setting linked to the CSI-ReportConfig, that the CSI-RS port indices are associated with ranks where is the number of ports in the CSI-RS resource. If the UE is configured with a CSI-ReportConfig that contains a list of sub-configurations, P corresponds to the number of bits with value 1 in the bitmap [port-subsetIndicator] for the corresponding sub-configuration. - When calculating the CQI for a rank, the UE shall use the ports indicated for that rank for the selected CSI-RS resource. The precoder for the indicated ports shall be assumed to be the identity matrix scaled by . <omitted texts>

## Time domain restrictions on CSI-RS resource measurement

In TS 38.214 [2], the following time domain restriction for CSI-RS resources are specified for mTRP CSI. The intention of the following restriction is to ensure that the reported CSIs associated with different hypothesis and different CSI-RS resources are based on the CSI-RS measurements, which are close enough in time such that the correlation between the reported CSIs remains.

TS 38.214 Clause 5.2.2.3.1 NZP CSI-RS <omitted texts> For a CSI-RS Resource Set for channel measurement configured with two Resource Groups and N Resource Pairs, the slot offsets of the two resources in a Resource Pair are configured within X∈{1,2} slots, without DL/UL switching in between the two resources, where X=1 implies that the two resources are configured in the same slot, and X=2 implies that the two resources are configured within two adjacent slots. For a NZP-CSI-RS-ResourceSet for channel measurement with 1<K≤4 resources and linked to a CSI-ReportConfig configured with codebookType set to 'typeII-CJT-r18' or 'typeII-CJT-PortSelection-r18', the slot offsets of the K CSI-RS resources are configured within X∈{1,2} slots, without DL/UL switching in between the two resources, where X=1 implies that the K resources are configured in the same slot, and X=2 implies that the K resources are configured within two adjacent slots. <omitted texts>

In our view, similar restriction should be imposed to the CSI-RS resources for the CSI-ReportConfig containing sub-configuration(s) since each sub-configuration may correspond to different CSI-RS resources if each of the sub-configuration is provided with a list of CSI-RS resources (i.e., Type 2 SD adaptation). If there is no such time domain restriction, the CSIs corresponding to different sub-configurations may be based on the measurement with long time gap, which makes the comparison between those CSIs (for NES adaptation pattern selection) less meaningful.

Proposal 2: Adopt the following TP for TS 38.214 Clause 5.2.2.3.1 Resource Setting configuration providing a description on the time domain restriction of CSI-RS resource configuration for NES.

TS 38.214 Clause 5.2.2.3.1 NZP CSI-RS <omitted texts> For a CSI-RS Resource Set for channel measurement configured with two Resource Groups and N Resource Pairs, the slot offsets of the two resources in a Resource Pair are configured within X∈{1,2} slots, without DL/UL switching in between the two resources, where X=1 implies that the two resources are configured in the same slot, and X=2 implies that the two resources are configured within two adjacent slots. For a NZP-CSI-RS-ResourceSet for channel measurement with 1<K≤4 resources and linked to a CSI-ReportConfig configured with codebookType set to 'typeII-CJT-r18' or 'typeII-CJT-PortSelection-r18', the slot offsets of the K CSI-RS resources are configured within X∈{1,2} slots, without DL/UL switching in between the two resources, where X=1 implies that the K resources are configured in the same slot, and X=2 implies that the K resources are configured within two adjacent slots. For a NZP-CSI-RS-ResourceSet for channel measurement with K resources and linked to a CSI-ReportConfig that contains a list of sub-configurations and each of the sub-configuration is provided with nzp-CSI-RS-resourceList, the slot offsets of the K CSI-RS resources are configured within X∈{1,2} slots, without DL/UL switching in between the two resources, where X=1 implies that the K resources are configured in the same slot, and X=2 implies that the K resources are configured within two adjacent slots. The bandwidth and initial common resource block (CRB) index of a CSI-RS resource within a BWP …. <omitted texts>

## Parameter configuration/determination for sub-configuration

In RAN1 #114-bis [1], the following notes were captured to facilitate the discussion in RAN1#115.

For RAN1#115: - For Type 2 SD adaptation or joint operation of Type 2 SD and PD adaptation, whether the list of NZP CSI-RS resources configured for a sub-configuration is identical to or has no intersection with the list of NZP CSI-RS resources configured for the other sub-configuration(s) within the same CSI-ReportConfig. - Companies are encouraged to check the example as provided in section 2.5 in R1- 2309647 - For Type 1 SD adaptation, or joint operation of Type 1 SD and PD adaptation, - ‘typeI-SinglePanel-codebookSubsetRestriction-i2’ is configured for each sub-configuration - if a UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-CQI', the UE expects to be configured with higher layer parameter non-PMI-PortIndication in each sub-configuration - if all the sub-configurations are configured with port antenna subset indication, the codebook subset restriction, ri restriction, N1 and N2 (and Ng when applicable) should be configured separately in each sub-configuration, instead of being configured in CodebookConfig in the CSI report configuration; otherwise, the CodebookConfig should be configured as legacy in the CSI report configuration.

The first highlighted aspect is about whether to impose additional restriction/description for the case of Type 2 SD adaptation and/or joint SD/PD adaptation. In our view, the current specification already allows the lists of CSI-RS resources to be different or the same across different sub-configurations. Also, for each sub-configuration, a power offset parameter can be optionally added to a sub-configuration. The power offset parameter is applicable to the list of CSI-RS resources corresponding to the sub-configuration. Since the current mechanism can provide sufficient flexibility, additional specification description on this aspect is not necessary.

The second highlighted aspect is about whether the high layer parameter typeI-SinglePanel-codebookSubsetRestriction-i2 should be configured per sub-configuration for Type 1 SD adaptation. This parameter is for the Type-I wideband single panel codebook with report quantity set to 'cri-RI-i1-CQI'. Since each sub-configuration may correspond to different port subsets in Type 1 SD adaptation, typeI-SinglePanel-codebookSubsetRestriction-i2 should be configured per sub-configuration to accommodate the corresponding port subset. Hence, if the CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RI-i1-CQI' and if the higher layer parameter codebookType corresponding to the sub-configuration is set to 'typeI-SinglePanel' and if high layer parameter pmi-FormatIndicator is set to 'widebandPMI', typeI-SinglePanel-codebookSubsetRestriction-i2 should be configured per sub-configuration.

Similar to typeI-SinglePanel-codebookSubsetRestriction-i2, it should be further discussed that whether the high layer parameter codebookMode shall be configured per sub-configuration for Type 1 SD adaptation. In the case when all the sub-configurations correspond to the 'typeI-SinglePanel' or in the case when all the sub-configurations correspond to the 'typeI-MultiPanel', there is no problem with a common codebookMode configuration in CodebookConfig which applies to all the sub-configurations. However, in the case when there is a mixture of sub-configuration(s) with 'typeI-SinglePanel' and sub-configuration(s) with 'typeI-MultiPanel', the codebookMode for single panel codebook and the codebookMode for multi-panel codebook have totally different meaning and interpretation. For example, if mode 2 is intended for multiple-panel codebook, the corresponding single-panel codebook is also mandated to use mode 2. However, for single-panel codebook that results from the turn-off of multiple panels, mode 1 is more typical since it can use the same 1-stage precoder and the same spatial oversampling granularity (similar to the fallback of multi-panel codebook) as for the corresponding multiple panel codebook. Therefore, if a CSI-ReportConfig that contains a mix of sub-configuration(s), wherein some sub-configuration(s) correspond to 'typeI-SinglePanel' and some other sub-configuration(s) correspond to 'typeI-MultiPanel', each sub-configuration can be configured with the higher layer parameter codebookMode.

Proposal 3: Adopt the following TP for TS 38.214 Clause 5.2.1.4.2 Report Quantity Configurations providing a description on the configuration of typeI-SinglePanel-codebookSubsetRestriction-i2 and codebookMode.

TP for TS 38.214 Clause 5.2.1.4.2 Report Quantity Configurations <omitted texts> If the UE is configured with a CSI-ReportConfig that contains a list of sub-configurations, provided by the higher layer parameter [csi-ReportSubConfigList]: - the UE expects to be configured with the higher layer parameter codebookType set to 'typeI-SinglePanel' or 'typeI-MultiPanel'. If the UE indicates a capability for supporting mixed codebook combination in a slot with [ABC], each sub-configuration can be configured with the higher layer parameter codebookType set to 'typeI-SinglePanel' or 'typeI-MultiPanel'. - Each sub-configuration can be configured with an antenna port subset using the higher layer bitmap parameter [port-subsetIndicator] which contains the bit sequence p0,p1,...,pPm-1, where p0 is the MSB and pPm-1 is the LSB, bit pi corresponds to antenna port 3000+i, and Pm is the number of ports nrofPorts configured for the CSI-RS resources(s) within the NZP-CSI-RS-ResourceSet contained in the CSI-ResourceConfig for channel measurement that corresponds to the CSI-ReportConfig. A bit value 0 in [port-subsetIndicator] indicates that the corresponding antenna port is disabled for the sub-configuration, whereas bit value 1 indicates that the antenna port is enabled and belongs to the antenna port subset for the sub-configuration. For the derivation of PMI, antenna ports corresponding to all bits with value of 1 in [port-subsetIndicator] are mapped to consecutive antenna ports starting at CSI-RS antenna port 3000 in increasing order of the bit position in [port-subsetIndicator]. - If a sub-configuration is configured with an antenna port subset, then the sub-configuration can be configured with a [RI restriction parameter] and, if the number of antenna ports of the subset greater than 2, with [n1-n2 parameter] if the higher layer parameter codebookType is set to 'typeI-SinglePanel' or with [ng-n1-n2 parameter] if the higher layer parameter codebookType is set to 'typeI-MultiPanel', and, if the corresponding number of antenna ports of the subset is 2, with twoTX-CodebookSubsetRestriction, where the parameters [RI restriction], [n1-n2], [ng-n1-n2], twoTX-CodebookSubsetRestriction are as described in Clauses 5.2.2.2.1 and 5.2.2.2.2. - If a sub-configuration is configured with an antenna port subset, and if higher layer parameter reportQuantity is set to 'cri-RI-i1-CQI', and if the higher layer parameter codebookType is set to 'typeI-SinglePanel', and if higher layer parameter pmi-FormatIndicator is set to 'widebandPMI', then the sub-configuration can be configured with higher layer parameter typeI-SinglePanel-codebookSubsetRestriction-i2, where twoTX-CodebookSubsetRestriction are as described in Clause 5.2.2.2.2. - If a sub-configuration is configured with an antenna port subset, and if the CSI-ReportConfig that contains a mix of sub-configuration(s) corresponding to 'typeI-SinglePanel' and sub-configuration(s) corresponding to 'typeI-MultiPanel', then the sub-configuration can be configured with the higher layer parameter codebookMode. - A sub-configuration can be configured with a list of NZP CSI-RS resources, provided by [nzp-CSI-RS-resourceList], which indicates one or more NZP CSI-RS resources, within the NZP-CSI-RS-ResourceSet contained in the CSI-ResourceConfig for channel measurement which corresponds to the CSI-ReportConfig.[The list of NZP CSI-RS resources is identical to or has no intersection with a list of NZP CSI-RS resources configured for any other sub-configuration(s) within the CSI-ReportConfig.] - A sub-configuration can be configured with a power offset provided by [powerOffset]. - If a sub-configurations is not configured with [nzp-CSI-RS-resourceList] then the sub-configuration shall be associated with all the NZP CSI-RS resources within the NZP-CSI-RS-ResourceSet contained in the CSI-ResourceConfig for channel measurement which corresponds to the CSI-ReportConfig. - the UE reports CSI(s) for one or more sub-configurations according to Clauses 5.2.1.5.1, 5.2.1.5.2, 5.2.3 and 5.2.4, and according to the higher layer parameter reportQuantity configured for that CSI-ReportConfig. - The UE does not expect the higher layer parameter reportQuantity to be set to ‘cri-RSRP’, ‘cri-SINR’, or ‘cri-SINR- Index’. <omitted texts>

The third highlighted aspect is about the relation between the codebook related parameters in sub-configurations and the CodebookConfig in CSI-ReportConfig. In our view, the CodebookConfig should be anyway configured in CSI-ReportConfig and the codebook related parameters in sub-configurations should override the parameters in the CodebookConfig in its entirety or subset of parameters when applicable. In this way, parameters configured in CodebookConfig and parameters configured in sub-configurations can be used without contradiction.

Proposal 4: Adopt the following TP for TS 38.214 Clause 5.2.1.4.2 Report Quantity Configurations providing a description on applicable parameters for the determination of CSI for a sub-configuration.

TP for TS 38.214 Clause 5.2.1.4.2 Report Quantity Configurations <omitted texts> If the UE is configured with a CSI-ReportConfig that contains a list of sub-configurations, provided by the higher layer parameter [csi-ReportSubConfigList]: - the UE expects to be configured with the higher layer parameter codebookType set to 'typeI-SinglePanel' or 'typeI-MultiPanel'. If the UE indicates a capability for supporting mixed codebook combination in a slot with [ABC], each sub-configuration can be configured with the higher layer parameter codebookType set to 'typeI-SinglePanel' or 'typeI-MultiPanel'. - Each sub-configuration can be configured with an antenna port subset using the higher layer bitmap parameter [port-subsetIndicator] which contains the bit sequence p0,p1,...,pPm-1, where p0 is the MSB and pPm-1 is the LSB, bit pi corresponds to antenna port 3000+i, and Pm is the number of ports nrofPorts configured for the CSI-RS resources(s) within the NZP-CSI-RS-ResourceSet contained in the CSI-ResourceConfig for channel measurement that corresponds to the CSI-ReportConfig. A bit value 0 in [port-subsetIndicator] indicates that the corresponding antenna port is disabled for the sub-configuration, whereas bit value 1 indicates that the antenna port is enabled and belongs to the antenna port subset for the sub-configuration. For the derivation of PMI, antenna ports corresponding to all bits with value of 1 in [port-subsetIndicator] are mapped to consecutive antenna ports starting at CSI-RS antenna port 3000 in increasing order of the bit position in [port-subsetIndicator]. - If a sub-configuration is configured with an antenna port subset, then the sub-configuration can be configured with a [RI restriction parameter] and, if the number of antenna ports of the subset greater than 2, with [n1-n2 parameter] if the higher layer parameter codebookType is set to 'typeI-SinglePanel' or with [ng-n1-n2 parameter] if the higher layer parameter codebookType is set to 'typeI-MultiPanel', and, if the corresponding number of antenna ports of the subset is 2, with twoTX-CodebookSubsetRestriction, where the parameters [RI restriction], [n1-n2], [ng-n1-n2], twoTX-CodebookSubsetRestriction are as described in Clauses 5.2.2.2.1 and 5.2.2.2.2. - A sub-configuration can be configured with a list of NZP CSI-RS resources, provided by [nzp-CSI-RS-resourceList], which indicates one or more NZP CSI-RS resources, within the NZP-CSI-RS-ResourceSet contained in the CSI-ResourceConfig for channel measurement which corresponds to the CSI-ReportConfig.[The list of NZP CSI-RS resources is identical to or has no intersection with a list of NZP CSI-RS resources configured for any other sub-configuration(s) within the CSI-ReportConfig.] - A sub-configuration can be configured with a power offset provided by [powerOffset]. - If a sub-configurations is not configured with [nzp-CSI-RS-resourceList] then the sub-configuration shall be associated with all the NZP CSI-RS resources within the NZP-CSI-RS-ResourceSet contained in the CSI-ResourceConfig for channel measurement which corresponds to the CSI-ReportConfig. - the UE reports CSI(s) for one or more sub-configurations according to Clauses 5.2.1.5.1, 5.2.1.5.2, 5.2.3 and 5.2.4, and according to the higher layer parameter reportQuantity configured for that CSI-ReportConfig. - the UE reports CSI for a sub-configuration according to the parameters configured in the CodebookConfig provided in the sub-configuration when present; otherwise, if a parameter is present in the CodebookConfig provided in the CSI-ReportConfig but not in the CodebookConfig provided in the sub-configuration, the parameter configured in the CodebookConfig provided in the CSI-ReportConfig is used. - The UE does not expect the higher layer parameter reportQuantity to be set to ‘cri-RSRP’, ‘cri-SINR’, or ‘cri-SINR- Index’. <omitted texts>

# Remaining Issues on Multi-CSI Reporting

## Mapping between CRI and resources for a sub-configuration

In RAN1 #113 [3], it was agreed that a list of CSI-RS resource ID will be provided per sub-configuration in a CSI report configuration for Type-2 SD adaptation.

Agreement For the sub-configuration(s) in a CSI report configuration with L>1, - for Type 1 SD with A1-2-revised, the following is configured in each sub-configuration - codebook subset restriction, - rank restriction - N1, N2 and Ng - FFS: the case when the number of ports is less than 4 - for Type 2 SD adaptation with A1-1-revised, for each sub-configuration - a list of CSI-RS resource ID - FFS: codebookConfig (including codebookSubsetRestriction/ ri-Restriction) - FFS: CQI table indication - FFS: reportFreqConfiguration - FFS: report quantity Above is agreed in addition to what was agreed in previous RAN1 agreements

Accordingly, for type 2 SD adaptation, a list of CSI-RS resources will be configured per sub-configuration, and the CRI for the sub-configuration shall be chosen from the provided list of CSI-RS resources. For clarification on the UE behaviour, the following TP is made.

Proposal 5: Adopt the following TP for TS 38.214 Clause 5.2.1.4.2 Report Quantity Configurations for the clarification on the selection of CRI when a list of CSI-RS resources is provided in csi-ReportSubConfig.

TP for TS 38.214 Clause 5.2.1.4.2 Report Quantity Configurations <omitted texts> If the UE is configured with a CSI-ReportConfig that contains a list of sub-configurations, provided by the higher layer parameter [csi-ReportSubConfigList]: - the UE expects to be configured with the higher layer parameter codebookType set to 'typeI-SinglePanel' or 'typeI-MultiPanel'. If the UE indicates a capability for supporting mixed codebook combination in a slot with [ABC], each sub-configuration can be configured with the higher layer parameter codebookType set to 'typeI-SinglePanel' or 'typeI-MultiPanel'. - Each sub-configuration can be configured with an antenna port subset using the higher layer bitmap parameter [port-subsetIndicator] which contains the bit sequence p0,p1,...,pPm-1, where p0 is the MSB and pPm-1 is the LSB, bit pi corresponds to antenna port 3000+i, and Pm is the number of ports nrofPorts configured for the CSI-RS resources(s) within the NZP-CSI-RS-ResourceSet contained in the CSI-ResourceConfig for channel measurement that corresponds to the CSI-ReportConfig. A bit value 0 in [port-subsetIndicator] indicates that the corresponding antenna port is disabled for the sub-configuration, whereas bit value 1 indicates that the antenna port is enabled and belongs to the antenna port subset for the sub-configuration. - If a sub-configuration is configured with an antenna port subset, then the sub-configuration can be configured with a [RI restriction parameter] and, if the number of antenna ports of the subset greater than 2, with [n1-n2 parameter] if the higher layer parameter codebookType is set to 'typeI-SinglePanel' or with [ng-n1-n2 parameter] if the higher layer parameter codebookType is set to 'typeI-MultiPanel', and, if the corresponding number of antenna ports of the subset is 2, with twoTX-CodebookSubsetRestriction, where the parameters [RI restriction], [n1-n2], [ng-n1-n2], twoTX-CodebookSubsetRestriction are as described in Clauses 5.2.2.2.1 and 5.2.2.2.2. - A sub-configuration can be configured with a list of NZP CSI-RS resources, provided by [nzp-CSI-RS-resourceList], which indicates one or more NZP CSI-RS resources, within the NZP-CSI-RS-ResourceSet contained in the CSI-ResourceConfig for channel measurement which corresponds to the CSI-ReportConfig. [The list of NZP CSI-RS resources is identical to or has no intersection with a list of NZP CSI-RS resources configured for any other sub-configuration(s) within the CSI-ReportConfig.] - If a sub-configuration is configured with a list of NZP CSI-RS resources with more than one resources, the UE shall derive the CSI parameters other than CRI conditioned on the reported CRI, where the CRI k (k ≥ 0) for the sub-configuration corresponds to the configured (k+1)-th entry of associated nzp-CSI-RS-Resources indicated by the list in the corresponding NZP-CSI-RS-ResourceSet for channel measurement. - A sub-configuration can be configured with a power offset provided by [powerOffset]. - If a sub-configurations is not configured with [nzp-CSI-RS-resourceList] then the sub-configuration shall be associated with all the NZP CSI-RS resources within the NZP-CSI-RS-ResourceSet contained in the CSI-ResourceConfig for channel measurement which corresponds to the CSI-ReportConfig. - the UE reports CSI(s) for one or more sub-configurations according to Clauses 5.2.1.5.1, 5.2.1.5.2, 5.2.3 and 5.2.4, and according to the higher layer parameter reportQuantity configured for that CSI-ReportConfig. <omitted texts>

## CSI dropping when no valid CSI reference resource

In RAN1#114bis, one of the remaining proposals from chair’s note includes [1]:

For RAN1#115: Further discuss in the next meeting for the following case: A UE configured with CSI report configuration with e.g. two sub-configurations (sub-config#1 and sub-config#2), each with e.g. two CSI-RS resources, none of the CSI-RS Tx occasions of sub-config#1 meet the CSI reference resource, i.e. they are later than CSI ref. resource, the UE shall - Alt 1: report the CSI report, according to current spec - Alt 2: report the CSI sub-report#2 only and drop the CSI sub-report#1 only - Alt 3: drop the entire CSI report

For type 2 SD adaptation, if UE is configured with a CSI-ReportConfig that contains multiple sub-configurations associated with different CSI-RS resources and one of the sub-configurations doesn’t have a valid downlink slot for the CSI reference resource, it is necessary to clarify the UE behavior for CSI omission. According to current specification, if there is no valid downlink slot for the CSI reference resource or there is no qualifying CSI-RS transmission corresponding to a CSI Report Setting, a CSI reporting is omitted or dropped, and it was concluded in Rel-18 NES that the CSI reference resource is defined in CSI reporting level. Furthermore, the described situation in the above seems a corner case and not so critical as such situation can be avoided by gNB scheduling. Therefore, we suggest Alt 3 without spec change to inherit current implementation and minimize spec change.

Proposal 6: Support Alt 3 for CSI omission per sub-configuration when there is no valid downlink slot for the CSI reference resource.

## CSI omission

In RAN1 #114b, the following note was captured in the Chairman’s note for further discussion in RAN1 #115 [1].

For RAN1#115 - Further check whether there is any issue according to the current specification that, for the CSI mapping of a CSI report configuration having L sub-configurations, Part 2 wideband CSIs have the same priority and are dropped per sub-configuration level in the ascending order of sub-configuration index. - Further check whether part 1 CSI can already be dropped in legacy for a CSI report configuration, and if so, then Part 1 CSIs are dropped per sub-configuration level, in the ascending order of sub-configuration index. - Companies are encouraged to provide TPs for necessary changes

Part 1 CSI

For CSI reporting on PUCCH (when multiplexed with HARQ-ACK, SR) and for single part CSI (P-CSI on PUCCH format 2,3,4 or SP-CSI on PUCCH format 2,3,4 with Type-I wideband), in the legacy behavior as described in TS 38.213 Clause 9.2.5.2 [4], a UE can drop CSI Part 1, i.e., a single part CSI, at report level according to priority values in TS 38.214 [2]. Analogous to the legacy behavior, dropping of CSI Part 1 for a CSI report with multiple sub-reports can be performed at a sub-report level from higher sub-report index to lower sub-report index.

For CSI reporting on PUCCH (when multiplexed with HARQ-ACK, SR) and for two-part CSI (SP-CSI on PUCCH format 3,4 with Type-1 sub-band), in the legacy behavior as described in TS 38.213 9.2.5.2 [4], UE drops CSI Part 2 first according to priority values in TS 38.214 [2], and after dropping all the CSI Part 2, the UE can drop CSI Part 1 at report level according to priority values in TS 38.214 [2]. Assuming that all the CSI Part 2 is dropped, as in the legacy UE behavior, dropping of CSI Part 1 for a CSI report with multiple sub-reports can be performed at a sub-report level from higher sub-report index to lower sub-report index.

Proposal 7: For CSI reporting on PUCCH, CSI Part 1 for a CSI report with sub-reports can be dropped at a sub-report level from a higher sub-report index to a lower sub-report index.

For CSI reporting on PUSCH, it’s always two-part CSI when CSI report quantities include RI. Current TS 38.214 [2] only describes Part 2 CSI dropping. Part 1 CSI is expected to be transmitted in its entirety, i.e., no Part 1 CSI dropping. Following the legacy behavior, we may not specify anything regarding Part 1 CSI dropping regardless of whether a CSI report includes multiple sub-reports or not. In case if it has to be specified, e.g., the payload size of Part 1 CSI itself can be large due to multiple sub-reports, it can be dropped entirely, i.e., either entirely transmitted or entirely dropped.

Proposal 8: For CSI reporting on PUSCH, no specification is needed regarding Part 1 CSI dropping as in the legacy, i.e., Part 1 CSI is transmitted in its entirety.

Part 2 wideband CSI

Regarding “Further check whether there is any issue according to the current specification that, for the CSI mapping of a CSI report configuration having L sub-configurations, Part 2 wideband CSIs have the same priority and are dropped per sub-configuration level in the ascending order of sub-configuration index.”, we think there is an issue with sub-report level dropping of Part 2 wideband CSI. Currently all the Part 2 wideband CSIs from Nrep CSI reports are mapped to the same priority level 0 and the same priority level information is dropped entirely in the current specification [2]. If we allow sub-report level dropping of Part 2 wideband CSI, when there are multiplexing of a legacy CSI without sub-reports and Rel-18 NES-CSI with sub-reports, there will be a conflict in the UE behavior, i.e., how to drop at sub-report level for Rel-18 NES-CSI with sub-reports, while all other legacy CSIs are supposedly dropped at once?

Proposal 9: Part 2 wideband CSIs mapped to priority level 0 is dropped at its entirety as in the current specification regardless of whether a CSI report consists of sub-reports or not.

# Remaining issues related to CSI resources

## CSI-RS resources for interference measurement (IM)

As captured in the FL summary [5], in RAN1 #114b, there was a discussion on the impact on the CSI-RS resources for IM and it is largely understood that no spec impact is needed regarding CSI-IM. On the other hand, there was a controversy on whether NZP CSI-RS resources for IM is supported for NES and if there is any spec impact, if supported. It is noted that optimizing MU-MIMO performance is not the objective of NES WI, while not making the existing feature obsolete either. Unless there is an identification that the existing machinery is broken, RAN1 should avoid any optimization or enhancement on non-NES features based on a preference during the Rel-18 NES maintenance phase.

Proposal 10: For NES, CSI-IM and NZP CSI-RS for IM are supported without further specification impact.

## Active CSI-RS resource and active CSI-RS port counting

In RAN1#114 [6], the following agreements were made regarding CSI-RS port counting:

Agreement For a CSI report configuration containing sub-configuration(s), if a CSI-RS resource is referred by M sub-configurations among X sub-configurations, the CSI-RS resource is counted M times and CSI-RS ports within the CSI-RS resource are counted by - Option 2A: maxs=1MPs, P for Type 1 SD adaptation, and M×P for Type 2 SD or PD adaptation. - P is nrofPorts configured in NZP-CSI-RS-Resource and Ps is the number of CSI-RS ports in sub-configuration s derived from port subset indication. - It is understood that further discussions are necessary

As it is noted in the agreement, further discussions are necessary regarding the value of X. One approach that received the most support in the previous meeting was such that:

- For aperiodic CSI-RS resources, X=N (the number of sub-configurations triggered for CSI reporting)

- For semi-persistent or periodic CSI-RS resource, X=L (the number of sub-configurations in the CSI report configuration).

As described in the current specification as captured below, the active time period for AP CSI-RS resource for port counting is based on the time period between AP CSI triggering and the end of the corresponding CSI report. In this case, X = N is a natural choice. Meanwhile, the active time period for SP/P CSI-RS resource for port counting is based on the configuration/activation of resources rather than the triggering of the corresponding CSI report. Since the current definition of active time period of SP/P CSI-RS resource is irrelevant to CSI triggering of the CSI report, X = L is a safer choice in this case.

Rel-17 TS 38.214 Clause 5.2.1.6 Error! Reference source not found. In any slot, the UE is not expected to have more active CSI-RS ports or active CSI-RS resources in active BWPs than reported as capability. NZP CSI-RS resource is active in a duration of time defined as follows. For aperiodic CSI-RS, starting from the end of the PDCCH containing the request and ending at the end of the scheduled PUSCH containing the report associated with this aperiodic CSI-RS. When the PDCCH candidates are associated with a search space set configured with searchSpaceLinking, for the purpose of determining the NZP CSI-RS resource active duration, the PDCCH candidate that ends later in time among the two linked PDCCH candidates is used. For semi-persistent CSI-RS, starting from the end of when the activation command is applied, and ending at the end of when the deactivation command is applied. For periodic CSI-RS, starting when the periodic CSI-RS is configured by higher layer signalling, and ending when the periodic CSI-RS configuration is released. If a CSI-RS resource is referred N times by one or more CSI Reporting Settings, the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted N times. For a CSI-RS Resource Set for channel measurement configured with two Resource Groups and 𝑁 Resource Pairs, if a CSI-RS resource is referred 𝑋 times by one of the 𝑀 CSI-RS resources, where 𝑀 is defined in clause 5.2.1.4.2, and/or one or two Resource Pairs, the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted 𝑋 times.

Proposal 11: For the agreement made in RAN1 #114 regarding active CSI-RS port counting,

- X=N (the number of sub-configurations triggered for CSI reporting) for aperiodic CSI-RS resource.

- X=L (the number of sub-configurations in the CSI report configuration) for semi-persistent CSI-RS resource or periodic CSI-RS resource.

Despite the majority support, the main concern on the above proposal was such that the the maximum number of simultaneous CSI-RS resources and CSI-RS ports across simultaneous CSI-RS resources, i.e., totalNumberPortsSimultaneousNZP-CSI-RS-PerCC can limit the utilization of the NES feature. Based on TS 38.306 [7], the current value range of totalNumberPortsSimultaneousNZP-CSI-RS-PerCC is from 2 to 256. If the value totalNumberPortsSimultaneousNZP-CSI-RS-PerCC reported by an NES UE is large enough (e.g., 256), gNB can still configure or trigger a sufficient number of sub-configurations. However, if an NES UE reports a rather small value of totalNumberPortsSimultaneousNZP-CSI-RS-PerCC (e.g., 32), it can limit the number of sub-configurations that can be configured or triggered by the gNB.

To solve this problem, we can take the UE feature (i.e., FG 23-7-1 as in Appendix) specified for MIMO mTRP as a precedent, where a new capability is introduced to indicate the maximum total number of ports can be referred by all measurement hypotheses (i.e., maxTotalNumTx-PortsNZP-CSI-RS). For NES UE feature, similar UE capability can be introduced. The upper bound of this parameter can be 256 (as legacy). Meanwhile, the lower bound of this parameter can be reasonably increased to keep the NES feature useful.

## Restriction on total number of CSI-RS resources for channel measurement

In RAN1#113 [3], the following agreements were made regarding different types of CSI-RS resource associations with different types of SD adaptations:

Agreement - For A1-1-revised for Type 2, one or more CSI-RS resources from a CSI-RS resource set for channel measurement can be associated with the same sub-configuration provided in a CSI report configuration - Resources in the resource set for channel measurement have the same number of antenna ports - For A1-2-revised for Type 1, all CSI-RS resource(s) (which can be one or more) in the CSI-RS resource set for channel measurement are associated with each sub-configuration provided in a CSI report configuration - i.e. each CSI-RS resource is associated with all the sub-configurations - Resources in the resource set for channel measurement have the same number of antenna ports - FFS: restriction on total number of CSI-RS resources for channel measurement in a CSI-ReportConfig and/or sub-configuration.

In the current specification, if the number of CSI-RS resource configured within the resources set (e.g., Ks) is 1, the resource in the set shall contain at most 32 CSI-RS ports. If Ks is 2, the resource in the set shall contain at most 16 CSI-RS ports. If 2 < Ks ≤ 8, the resource in the set shall contain at most 8 CSI-RS ports.

In the case of the CSI-ReportConfig with sub-configurations and for Type 1 SD adaptation, a restriction should be considered based on the number of ports referred by each of the multiple sub-configurations. In particular, the above restriction should be based on the maximum number of port subset indicated by each of the sub-configuration. For example, a CSI-ReportConfig is with sub-configuration#1 and sub-configuration#2 and is configured with resource set for channel measurement with 4 resources. Since 2 < 4 ≤ 8, the maximum number of (or any of) port subset indicated by sub-configuration#1 and sub-configuration#2 shall be less than 8.

In the case of the CSI-ReportConfig with sub-configurations and for Type 2 SD adaptation, a restriction should be considered based on the number of resources (e.g., K) referred by each of the multiple sub-configurations. In particular, the above restriction should be based on the maximum number of resources referred by each of the multiple sub-configurations (e.g., resources indicated by the list of CSI-RS resource ID per sub-configuration). For example, a CSI-ReportConfig is with sub-configuration#1 and sub-configuration#2, where the list corresponding to sub-configuration#1 indicates {CSI-RS#1, CSI-RS#2, CSI-RS#3} and the list corresponding to sub-configuration#2 indicates {CSI-RS#4, CSI-RS#5}. Since the maximum number of resources referred by sub-configuration#1 and sub-configuration#2 is 3, the resource in the set shall contain at most 8 CSI-RS ports.

Proposal 12: For a CSI-ReportConfig with sub-configurations, legacy rule for resource restriction is reused as a baseline with the following modification:

- The maximum number of ports corresponding to each of the L sub-configurations is used for the determination of the number of antenna ports.

- The maximum number of resources corresponding to each of the L sub-configurations is used for the determination of the number of CSI-RS resources.

# Other remaining issues

## CSI computation time

In RAN1#114bis, it was agreed that the CSI computation time for a CSI-configuration containing a list of sub-configurations, only (Z2, Z’2) of Table 5.4-2 is used. Another remaining issue related to CSI computation time is its definition. Based on the description in TS 38.214 [8], the definition of CSI computation time (Z’) is associated with the latest symbol of CSI-RS for channel measurement, CSI-RS/CSI-IM for interference measurement. In the case of a CSI report with sub-configurations, those CSI-RS or CSI-IM should correspond to N triggered sub-configurations (i.e., the latest of CSI-RS for channel measurement, CSI-RS for interference measurement and CSI-IM for all N sub-configurations). Moreover, in the case of Type 2 SD adaptation, those CSI-RS and CSI-IM are the resources referred by the corresponding sub-configurations (i.e., the resources referred based on a list of CSI-RS IDs provided for the sub-configurations).

Proposal 13: For a CSI-ReportConfig with sub-configurations, the definition of the corresponding CSI computation time should be based on the CSI-RS resources for channel measurement, the CSI-RS resources for interference measurement and the CSI-IM resources for all N sub-configurations.

- If a sub-configuration is provided with a list of CSI-RS IDs (i.e., Type 2 SD adaptation), the CSI-RS and CSI-IM for the sub-configurations are the resources determined based on the provided list of CSI-RS IDs.

Adopt the following TP for TS 38.214 Clause 5.4 UE CSI computation time.

TP for TS 38.214 Clause 5.4 UE CSI computation time <omitted texts> When the CSI request field on a DCI triggers a CSI report(s) on PUSCH, the UE shall provide a valid CSI report for the n-th triggered report, - if the first uplink symbol to carry the corresponding CSI report(s) including the effect of the timing advance, starts no earlier than at symbol Zref, and - if the first uplink symbol to carry the n-th CSI report including the effect of the timing advance, starts no earlier than at symbol Z'ref(n), where Zref is defined as the next uplink symbol with its CP starting after the end of the last symbol of the PDCCH triggering the CSI report(s), and where Z'ref(n), is defined as the next uplink symbol with its CP starting after the end of the last symbol in time of the latest of: aperiodic CSI-RS resource for channel measurements, aperiodic CSI-IM used for interference measurements, and aperiodic NZP CSI-RS for interference measurement, when aperiodic CSI-RS is used for channel measurement for the n-th triggered CSI report, and where Tswitch is defined in clause 6.4 and is applied only if Z1 of table 5.4-1 is applied. If the n-th triggered report for which the CSI-ReportConfig containing a list of L sub-configurations, the aperiodic CSI-RS resource for channel measurements, aperiodic CSI-IM used for interference measurements, and aperiodic NZP CSI-RS for interference measurement corresponds to the N indicated sub-configurations out of L sub-configurations contained in the CSI-ReportConfig. <omitted texts>

## CPU occupation time

Regarding CPU occupation time, the following proposal was made in RAN1 #114 FL summary [9].

#### FL4 P8-5 For CPU occupation time, discuss the following determination - A periodic CSI report with L sub-configs, - occupies CPU(s) from the first symbol the earliest RS resource for channel or interference measurement within L configured sub-configs, until the last symbol of the configured PUSCH/PUCCH carrying the report. - An aperiodic CSI report with sub-config(s), - occupies CPU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report. - An initial semi-persistent CSI report on PUSCH , - occupies CPU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report. - A semi-persistent CSI report on PUSCH (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report) - occupies CPU(s) from the first symbol of the earliest one of each CSI-RS/CSI-IM/SSB resource for channel or interference measurement within N activated sub-configs, until the last symbol of the configured PUSCH/PUCCH carrying the report. - A semi-persistent CSI report on PUCCH, - occupies CPU(s) from the first symbol of the earliest one of each CSI-RS/CSI-IM/SSB resource for channel or interference measurement within L configured sub-configs, until the last symbol of the configured PUSCH/PUCCH carrying the report.

In the case of a CSI report with sub-configurations, the corresponding CPU occupation time needs to be clarified. Based on the description in Rel-17 TS 38.214 [8], the CPU occupation time is associated with the earliest measurement occasion among the CSI-RS/CSI-IM resource for channel or interference measurement. In the case of CSI report with sub-configurations, those CSI-RS or CSI-IM shall correspond to the N triggered sub-configurations (i.e., the earliest of CSI-RS/CSI-IM resource for channel or interference measurement for all N sub-configurations, and N = L in the case of periodic CSI report). Moreover, in the case of Type 2 SD adaptation, those CSI-RS and CSI-IM are the resources referred by the corresponding sub-configurations (i.e., the resources referred based on lists of CSI-RS IDs provided by the sub-configurations).

Proposal 14: For a CSI-ReportConfig with sub-configurations, the determination of CPU occupation time is based on CSI-RS/CSI-IM for channel or interference measurement referred by all the N triggered sub-configurations, where N = L for periodic CSI reporting.

Adopt the following TP for TS 38.214 Clause 5.2.1.6 CSI processing criteria.

TP for TS 38.214 Clause 5.2.1.6 CSI processing criteria <omitted texts> For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity not set to 'none', the CPU(s) are occupied for a number of OFDM symbols as follows: - A periodic or semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report) occupies CPU(s) from the first symbol of the earliest one of each CSI-RS/CSI-IM/SSB resource for channel or interference measurement, respective latest CSI-RS/CSI-IM/SSB occasion no later than the corresponding CSI reference resource, until the last symbol of the configured PUSCH/PUCCH carrying the report. For a periodic CSI report with CSI-ReportConfig providing a list of L sub-configurations, the CSI-RS/CSI-IM resource corresponds to the L sub-configurations provided in the CSI-ReportConfig. For a semi-persistent CSI report with CSI-ReportConfig providing a list of L sub-configurations, the CSI-RS/CSI-IM resource corresponds to the N indicated sub-configurations from the L sub-configurations provided in the CSI-ReportConfig. <omitted texts>

# Conclusion

In this contribution, we discussed impacts of spatial and power domain adaptations and, then, discussed necessary enhancements to support the adaptations. The following observations and proposals were made.

Proposal 1: Adopt the following TP for TS 38.214 Clause 5.2.1.4.2 Report Quantity Configurations for providing separate non-PMI-PortIndication configurations for each CSI report sub-configuration.

TP for TS 38.214 Clause 5.2.1.4.2 Report Quantity Configurations <omitted texts> If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-CQI', - if the UE is configured with higher layer parameter non-PMI-PortIndication contained in a CSI-ReportConfig, r ports are indicated in the order of layer ordering for rank r and each CSI-RS resource in the CSI resource setting is linked to the CSI-ReportConfig based on the order of the associated NZP-CSI-RS-ResourceId in the linked CSI resource setting for channel measurement given by higher layer parameter resourcesForChannelMeasurement. The configured higher layer parameter non-PMI-PortIndication contains a sequence of port indices, where are the CSI-RS port indices associated with rank ν and where is the number of ports in the CSI-RS resource. The UE shall only report RI corresponding to the configured fields of PortIndexFor8Ranks. If the UE is configured with a CSI-ReportConfig that contains a list of sub-configurations, the higher layer parameter non-PMI-PortIndication is separately provided for each sub-configuration and P corresponds to the number of bits with value 1 in the bitmap [port-subsetIndicator] for the corresponding sub-configuration. - if the UE is not configured with higher layer parameter non-PMI-PortIndication, the UE assumes, for each CSI-RS resource in the CSI resource setting linked to the CSI-ReportConfig, that the CSI-RS port indices are associated with ranks where is the number of ports in the CSI-RS resource. If the UE is configured with a CSI-ReportConfig that contains a list of sub-configurations, P corresponds to the number of bits with value 1 in the bitmap [port-subsetIndicator] for the corresponding sub-configuration. - When calculating the CQI for a rank, the UE shall use the ports indicated for that rank for the selected CSI-RS resource. The precoder for the indicated ports shall be assumed to be the identity matrix scaled by . <omitted texts>

Proposal 2: Adopt the following TP for TS 38.214 Clause 5.2.2.3.1 Resource Setting configuration providing a description on the time domain restriction of CSI-RS resource configuration for NES.

TS 38.214 Clause 5.2.2.3.1 NZP CSI-RS <omitted texts> For a CSI-RS Resource Set for channel measurement configured with two Resource Groups and N Resource Pairs, the slot offsets of the two resources in a Resource Pair are configured within X∈{1,2} slots, without DL/UL switching in between the two resources, where X=1 implies that the two resources are configured in the same slot, and X=2 implies that the two resources are configured within two adjacent slots. For a NZP-CSI-RS-ResourceSet for channel measurement with 1<K≤4 resources and linked to a CSI-ReportConfig configured with codebookType set to 'typeII-CJT-r18' or 'typeII-CJT-PortSelection-r18', the slot offsets of the K CSI-RS resources are configured within X∈{1,2} slots, without DL/UL switching in between the two resources, where X=1 implies that the K resources are configured in the same slot, and X=2 implies that the K resources are configured within two adjacent slots. For a NZP-CSI-RS-ResourceSet for channel measurement with K resources and linked to a CSI-ReportConfig that contains a list of sub-configurations and each of the sub-configuration is provided with nzp-CSI-RS-resourceList, the slot offsets of the K CSI-RS resources are configured within X∈{1,2} slots, without DL/UL switching in between the two resources, where X=1 implies that the K resources are configured in the same slot, and X=2 implies that the K resources are configured within two adjacent slots. The bandwidth and initial common resource block (CRB) index of a CSI-RS resource within a BWP …. <omitted texts>

Proposal 3: Adopt the following TP for TS 38.214 Clause 5.2.1.4.2 Report Quantity Configurations providing a description on the configuration of typeI-SinglePanel-codebookSubsetRestriction-i2 and codebookMode.

TP for TS 38.214 Clause 5.2.1.4.2 Report Quantity Configurations <omitted texts> If the UE is configured with a CSI-ReportConfig that contains a list of sub-configurations, provided by the higher layer parameter [csi-ReportSubConfigList]: - the UE expects to be configured with the higher layer parameter codebookType set to 'typeI-SinglePanel' or 'typeI-MultiPanel'. If the UE indicates a capability for supporting mixed codebook combination in a slot with [ABC], each sub-configuration can be configured with the higher layer parameter codebookType set to 'typeI-SinglePanel' or 'typeI-MultiPanel'. - Each sub-configuration can be configured with an antenna port subset using the higher layer bitmap parameter [port-subsetIndicator] which contains the bit sequence p0,p1,...,pPm-1, where p0 is the MSB and pPm-1 is the LSB, bit pi corresponds to antenna port 3000+i, and Pm is the number of ports nrofPorts configured for the CSI-RS resources(s) within the NZP-CSI-RS-ResourceSet contained in the CSI-ResourceConfig for channel measurement that corresponds to the CSI-ReportConfig. A bit value 0 in [port-subsetIndicator] indicates that the corresponding antenna port is disabled for the sub-configuration, whereas bit value 1 indicates that the antenna port is enabled and belongs to the antenna port subset for the sub-configuration. For the derivation of PMI, antenna ports corresponding to all bits with value of 1 in [port-subsetIndicator] are mapped to consecutive antenna ports starting at CSI-RS antenna port 3000 in increasing order of the bit position in [port-subsetIndicator]. - If a sub-configuration is configured with an antenna port subset, then the sub-configuration can be configured with a [RI restriction parameter] and, if the number of antenna ports of the subset greater than 2, with [n1-n2 parameter] if the higher layer parameter codebookType is set to 'typeI-SinglePanel' or with [ng-n1-n2 parameter] if the higher layer parameter codebookType is set to 'typeI-MultiPanel', and, if the corresponding number of antenna ports of the subset is 2, with twoTX-CodebookSubsetRestriction, where the parameters [RI restriction], [n1-n2], [ng-n1-n2], twoTX-CodebookSubsetRestriction are as described in Clauses 5.2.2.2.1 and 5.2.2.2.2. - If a sub-configuration is configured with an antenna port subset, and if higher layer parameter reportQuantity set to 'cri-RI-i1-CQI', and if the higher layer parameter codebookType set to 'typeI-SinglePanel', and if higher layer parameter pmi-FormatIndicator set to 'widebandPMI', then the sub-configuration can be configured with higher layer parameter typeI-SinglePanel-codebookSubsetRestriction-i2, where twoTX-CodebookSubsetRestriction are as described in Clause 5.2.2.2.2. - If a sub-configuration is configured with an antenna port subset, and if the CSI-ReportConfig that contains a mix of sub-configuration(s) each corresponding to 'typeI-SinglePanel' and some other sub-configuration(s) each corresponding to 'typeI-MultiPanel', then the sub-configuration can be configured with the higher layer parameter codebookMode. - A sub-configuration can be configured with a list of NZP CSI-RS resources, provided by [nzp-CSI-RS-resourceList], which indicates one or more NZP CSI-RS resources, within the NZP-CSI-RS-ResourceSet contained in the CSI-ResourceConfig for channel measurement which corresponds to the CSI-ReportConfig.[The list of NZP CSI-RS resources is identical to or has no intersection with a list of NZP CSI-RS resources configured for any other sub-configuration(s) within the CSI-ReportConfig.] - A sub-configuration can be configured with a power offset provided by [powerOffset]. - If a sub-configurations is not configured with [nzp-CSI-RS-resourceList] then the sub-configuration shall be associated with all the NZP CSI-RS resources within the NZP-CSI-RS-ResourceSet contained in the CSI-ResourceConfig for channel measurement which corresponds to the CSI-ReportConfig. - the UE reports CSI(s) for one or more sub-configurations according to Clauses 5.2.1.5.1, 5.2.1.5.2, 5.2.3 and 5.2.4, and according to the higher layer parameter reportQuantity configured for that CSI-ReportConfig. - The UE does not expect the higher layer parameter reportQuantity to be set to ‘cri-RSRP’, ‘cri-SINR’, or ‘cri-SINR- Index’. <omitted texts>

Proposal 4: Adopt the following TP for TS 38.214 Clause 5.2.1.4.2 Report Quantity Configurations providing a description on applicable parameters for the determination of CSI for a sub-configuration.

TP for TS 38.214 Clause 5.2.1.4.2 Report Quantity Configurations <omitted texts> If the UE is configured with a CSI-ReportConfig that contains a list of sub-configurations, provided by the higher layer parameter [csi-ReportSubConfigList]: - the UE expects to be configured with the higher layer parameter codebookType set to 'typeI-SinglePanel' or 'typeI-MultiPanel'. If the UE indicates a capability for supporting mixed codebook combination in a slot with [ABC], each sub-configuration can be configured with the higher layer parameter codebookType set to 'typeI-SinglePanel' or 'typeI-MultiPanel'. - Each sub-configuration can be configured with an antenna port subset using the higher layer bitmap parameter [port-subsetIndicator] which contains the bit sequence p0,p1,...,pPm-1, where p0 is the MSB and pPm-1 is the LSB, bit pi corresponds to antenna port 3000+i, and Pm is the number of ports nrofPorts configured for the CSI-RS resources(s) within the NZP-CSI-RS-ResourceSet contained in the CSI-ResourceConfig for channel measurement that corresponds to the CSI-ReportConfig. A bit value 0 in [port-subsetIndicator] indicates that the corresponding antenna port is disabled for the sub-configuration, whereas bit value 1 indicates that the antenna port is enabled and belongs to the antenna port subset for the sub-configuration. For the derivation of PMI, antenna ports corresponding to all bits with value of 1 in [port-subsetIndicator] are mapped to consecutive antenna ports starting at CSI-RS antenna port 3000 in increasing order of the bit position in [port-subsetIndicator]. - If a sub-configuration is configured with an antenna port subset, then the sub-configuration can be configured with a [RI restriction parameter] and, if the number of antenna ports of the subset greater than 2, with [n1-n2 parameter] if the higher layer parameter codebookType is set to 'typeI-SinglePanel' or with [ng-n1-n2 parameter] if the higher layer parameter codebookType is set to 'typeI-MultiPanel', and, if the corresponding number of antenna ports of the subset is 2, with twoTX-CodebookSubsetRestriction, where the parameters [RI restriction], [n1-n2], [ng-n1-n2], twoTX-CodebookSubsetRestriction are as described in Clauses 5.2.2.2.1 and 5.2.2.2.2. - A sub-configuration can be configured with a list of NZP CSI-RS resources, provided by [nzp-CSI-RS-resourceList], which indicates one or more NZP CSI-RS resources, within the NZP-CSI-RS-ResourceSet contained in the CSI-ResourceConfig for channel measurement which corresponds to the CSI-ReportConfig.[The list of NZP CSI-RS resources is identical to or has no intersection with a list of NZP CSI-RS resources configured for any other sub-configuration(s) within the CSI-ReportConfig.] - A sub-configuration can be configured with a power offset provided by [powerOffset]. - If a sub-configurations is not configured with [nzp-CSI-RS-resourceList] then the sub-configuration shall be associated with all the NZP CSI-RS resources within the NZP-CSI-RS-ResourceSet contained in the CSI-ResourceConfig for channel measurement which corresponds to the CSI-ReportConfig. - the UE reports CSI(s) for one or more sub-configurations according to Clauses 5.2.1.5.1, 5.2.1.5.2, 5.2.3 and 5.2.4, and according to the higher layer parameter reportQuantity configured for that CSI-ReportConfig. - the UE reports CSI for a sub-configuration according to the parameters configured in the CodebookConfig provided in the sub-configuration when present; otherwise, if a parameter is present in the CodebookConfig provided in the CSI-ReportConfig but not in the CodebookConfig provided in the sub-configuration, the parameter configured in the CodebookConfig provided in the CSI-ReportConfig is used. - The UE does not expect the higher layer parameter reportQuantity to be set to ‘cri-RSRP’, ‘cri-SINR’, or ‘cri-SINR- Index’. <omitted texts>

Proposal 5: Adopt the following TP for TS 38.214 Clause 5.2.1.4.2 Report Quantity Configurations for the clarification on the selection of CRI when a list of CSI-RS resources is provided in csi-ReportSubConfig.

TP for TS 38.214 Clause 5.2.1.4.2 Report Quantity Configurations <omitted texts> If the UE is configured with a CSI-ReportConfig that contains a list of sub-configurations, provided by the higher layer parameter [csi-ReportSubConfigList]: - the UE expects to be configured with the higher layer parameter codebookType set to 'typeI-SinglePanel' or 'typeI-MultiPanel'. If the UE indicates a capability for supporting mixed codebook combination in a slot with [ABC], each sub-configuration can be configured with the higher layer parameter codebookType set to 'typeI-SinglePanel' or 'typeI-MultiPanel'. - Each sub-configuration can be configured with an antenna port subset using the higher layer bitmap parameter [port-subsetIndicator] which contains the bit sequence p0,p1,...,pPm-1, where p0 is the MSB and pPm-1 is the LSB, bit pi corresponds to antenna port 3000+i, and Pm is the number of ports nrofPorts configured for the CSI-RS resources(s) within the NZP-CSI-RS-ResourceSet contained in the CSI-ResourceConfig for channel measurement that corresponds to the CSI-ReportConfig. A bit value 0 in [port-subsetIndicator] indicates that the corresponding antenna port is disabled for the sub-configuration, whereas bit value 1 indicates that the antenna port is enabled and belongs to the antenna port subset for the sub-configuration. - If a sub-configuration is configured with an antenna port subset, then the sub-configuration can be configured with a [RI restriction parameter] and, if the number of antenna ports of the subset greater than 2, with [n1-n2 parameter] if the higher layer parameter codebookType is set to 'typeI-SinglePanel' or with [ng-n1-n2 parameter] if the higher layer parameter codebookType is set to 'typeI-MultiPanel', and, if the corresponding number of antenna ports of the subset is 2, with twoTX-CodebookSubsetRestriction, where the parameters [RI restriction], [n1-n2], [ng-n1-n2], twoTX-CodebookSubsetRestriction are as described in Clauses 5.2.2.2.1 and 5.2.2.2.2. - A sub-configuration can be configured with a list of NZP CSI-RS resources, provided by [nzp-CSI-RS-resourceList], which indicates one or more NZP CSI-RS resources, within the NZP-CSI-RS-ResourceSet contained in the CSI-ResourceConfig for channel measurement which corresponds to the CSI-ReportConfig. [The list of NZP CSI-RS resources is identical to or has no intersection with a list of NZP CSI-RS resources configured for any other sub-configuration(s) within the CSI-ReportConfig.] - If a sub-configuration is configured with a list of NZP CSI-RS resources with more than one resources, the UE shall derive the CSI parameters other than CRI conditioned on the reported CRI, where the CRI k (k ≥ 0) for the sub-configuration corresponds to the configured (k+1)-th entry of associated nzp-CSI-RS-Resources indicated by the list in the corresponding NZP-CSI-RS-ResourceSet for channel measurement. - A sub-configuration can be configured with a power offset provided by [powerOffset]. - If a sub-configurations is not configured with [nzp-CSI-RS-resourceList] then the sub-configuration shall be associated with all the NZP CSI-RS resources within the NZP-CSI-RS-ResourceSet contained in the CSI-ResourceConfig for channel measurement which corresponds to the CSI-ReportConfig. - the UE reports CSI(s) for one or more sub-configurations according to Clauses 5.2.1.5.1, 5.2.1.5.2, 5.2.3 and 5.2.4, and according to the higher layer parameter reportQuantity configured for that CSI-ReportConfig. <omitted texts>

Proposal 6: Support Alt 3 for CSI omission per sub-configuration when there is no valid downlink slot for the CSI reference resource.

Proposal 7: For CSI reporting on PUCCH, CSI Part 1 for a CSI report with sub-reports can be dropped at a sub-report level from a higher sub-report index to a lower sub-report index.

Proposal 8: For CSI reporting on PUSCH, no specification is needed regarding Part 1 CSI dropping as in the legacy, i.e., Part 1 CSI is transmitted in its entirety.

Proposal 9: Part 2 wideband CSIs mapped to priority level 0 is dropped at its entirety as in the current specification regardless of whether a CSI report consists of sub-reports or not.

Proposal 10: For NES, CSI-IM and NZP CSI-RS for IM are supported without further specification impact.

Proposal 11: For the agreement made in RAN1 #114 regarding active CSI-RS port counting,

- X=N (the number of sub-configurations triggered for CSI reporting) for aperiodic CSI-RS resource.

- X=L (the number of sub-configurations in the CSI report configuration) for semi-persistent CSI-RS resource or periodic CSI-RS resource.

Proposal 12: For a CSI-ReportConfig with sub-configurations, legacy rule for resource restriction is reused as a baseline with the following modification:

- The maximum number of ports corresponding to each of the L sub-configurations is used for the determination of the number of antenna ports.

- The maximum number of resources corresponding to each of the L sub-configurations is used for the determination of the number of CSI-RS resources.

Proposal 13: For a CSI-ReportConfig with sub-configurations, the definition of the corresponding CSI computation time should be based on the CSI-RS resources for channel measurement, the CSI-RS resources for interference measurement and the CSI-IM resources for all N sub-configurations.

- If a sub-configuration is provided with a list of CSI-RS IDs (i.e., Type 2 SD adaptation), the CSI-RS and CSI-IM for the sub-configurations are the resources determined based on the provided list of CSI-RS IDs.

Adopt the following TP for TS 38.214 Clause 5.4 UE CSI computation time.

TP for TS 38.214 Clause 5.4 UE CSI computation time <omitted texts> When the CSI request field on a DCI triggers a CSI report(s) on PUSCH, the UE shall provide a valid CSI report for the n-th triggered report, - if the first uplink symbol to carry the corresponding CSI report(s) including the effect of the timing advance, starts no earlier than at symbol Zref, and - if the first uplink symbol to carry the n-th CSI report including the effect of the timing advance, starts no earlier than at symbol Z'ref(n), where Zref is defined as the next uplink symbol with its CP starting after the end of the last symbol of the PDCCH triggering the CSI report(s), and where Z'ref(n), is defined as the next uplink symbol with its CP starting after the end of the last symbol in time of the latest of: aperiodic CSI-RS resource for channel measurements, aperiodic CSI-IM used for interference measurements, and aperiodic NZP CSI-RS for interference measurement, when aperiodic CSI-RS is used for channel measurement for the n-th triggered CSI report, and where Tswitch is defined in clause 6.4 and is applied only if Z1 of table 5.4-1 is applied. If the n-th triggered report for which the CSI-ReportConfig containing a list of L sub-configurations, the aperiodic CSI-RS resource for channel measurements, aperiodic CSI-IM used for interference measurements, and aperiodic NZP CSI-RS for interference measurement corresponds to the N indicated sub-configurations out of L sub-configurations contained in the CSI-ReportConfig. <omitted texts>

Proposal 14: For a CSI-ReportConfig with sub-configurations, the determination of CPU occupation time is based on CSI-RS/CSI-IM for channel or interference measurement referred by all the N triggered sub-configurations, where N = L for periodic CSI reporting.

Adopt the following TP for TS 38.214 Clause 5.2.1.6 CSI processing criteria.

TP for TS 38.214 Clause 5.2.1.6 CSI processing criteria <omitted texts> For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity not set to 'none', the CPU(s) are occupied for a number of OFDM symbols as follows: - A periodic or semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report) occupies CPU(s) from the first symbol of the earliest one of each CSI-RS/CSI-IM/SSB resource for channel or interference measurement, respective latest CSI-RS/CSI-IM/SSB occasion no later than the corresponding CSI reference resource, until the last symbol of the configured PUSCH/PUCCH carrying the report. For a periodic CSI report with CSI-ReportConfig providing a list of L sub-configurations, the CSI-RS/CSI-IM resource corresponds to the L sub-configurations provided in the CSI-ReportConfig. For a semi-persistent CSI report with CSI-ReportConfig providing a list of L sub-configurations, the CSI-RS/CSI-IM resource corresponds to the N indicated sub-configurations from the L sub-configurations provided in the CSI-ReportConfig. <omitted texts>

# Appendix:

Table 1 The description of FG23-7-1

| 23-7-1 | Basic Features of CSI Enhancement for Multi-TRP | 1. Support of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Support of N=1 2, Maximum number of NZP CSI-RS resources in one CSI-RS resource set: Ks,max 3. CSI report mode selection of mode 1 with X=0 and/or mode 2 4. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is b) Maximum number of Tx ports in one NZP CSI-RS resource associated with an NCJT measurement hypothesis c) Maximum total number of CMRs for NCJT measurement d) Maximum total number of Tx ports of NZP CSI-RS resources associated with NCJT measurement hypotheses 5. Supported codebook modes for NCJT CSI | mTRP-CSI-EnhancementPerBC-r17 { maxNumNZP-CSI-RS-r17, cSI-Report-mode-r17, supportedComboAcrossCCs-r17, codebookMode-NCJT-r17 } | Component 2 candidate value set: {2, 3, 4, 5, 6, 7, 8} Component 3 candidate value set: {mode 1 with X=0, mode 2, both} Component 4 candidate values: a) {2, 4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {2,3,4, …, 256} Component 5 candidate values: {mode 1, both mode 1 and mode 2} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- |

# References:

- “RAN1 Chair’s Notes”, 3GPP TSG RAN WG1 #114b, Xiamen, China.

- 3GPP TS 38.214, v18.0.0 (2023-09), “NR; Physical layer procedures for data (Release 18)”.

- “RAN1 Chair’s Notes”, 3GPP TSG RAN WG1 #113, Incheon, Korea.

- 3GPP TS 38.213, v17.7.0 (2023-09), “NR; Physical layer procedures for control (Release 17)”.

- R1-2310574, FL summary#5 for SD and PD adaptation for R18 NES, 3GPP TSG RAN WG1 #114b, Xiamen, China.

- “RAN1 Chair’s Notes”, 3GPP TSG RAN WG1 #114, Toulouse, France.

- 3GPP TS 38.306, v17.5.0, “NR; User Equipment (UE) radio access capabilities”.

- 3GPP TS 38.214, v17.6.0, “NR; Physical layer procedures for data”.

- R1-2308600, “Final summary for SD and PD adaptation for R18 NES”, Moderator (Huawei), 3GPP TSG RAN WG1 #114, Toulouse, France.