3GPP TSG RAN WG1 #115		R1-2311875

Chicago, USA, November 13th – 17th, 2023

Agenda item:	8.16.9

Source:	Samsung

Title:	UE features for dedicated spectrum less than 5MHz

Document for:	Discussion and Decision

# Introduction

In RAN1#114b [1], the following agreements were made, regarding the UE features for the dedicated spectrum with less than 5MHz bandwidth.

Agreement

- FG 51-x is revised as follows

| 51. NR_FR1_lessthan_5MHz_BW | 51-x1 | Support for 3 MHz channel bandwidth | 1) Reception of 12 PRB PBCH based on RB-level puncturing 2) Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS 3) Reception of 15 PRB and CORESET0 |  | Yes | N/A | UE is not able to support 3 MHz channel bandwidth with 15 PRB CORESET0 | Per Band | FDD only | FR1 only | N/A | This FG is supported for 15 kHz SCS only This FG is applicable only when an associated SS/PBCH block is located according to Table 5.4.3.3-2 in TS 38.101-1 in Rel-18 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- Introduce following FG

| 51. NR_FR1_lessthan_5MHz_BW | 51-3 | Support 5 MHz channel bandwidth with 20 PRB CORESET0 | 1) Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS 2) Reception of 20 PRB CORESET0 |  | Yes | N/A | UE is not able to support 5 MHz channel bandwidth with 20 PRB CORESET0 | Per UE | FDD only | FR1 only | N/A | This FG is supported for 15 kHz SCS only This FG is only applicable when an associated SS/PBCH block is located in band n100 at GSCN 41638 of Table 5.4.3.1-3 in TS 38.101-1 in Rel-18. [Note: The UE supporting this FG supports configuration of 20 PRB BWP operation] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

In this contribution, we discuss the remaining details of UE features for dedicated spectrum less than 5MHz.

# Discussion

12 PRB CORESET#0 may not be only applicable for transmission bandwidth with 12 RBs, and such restriction can be fully up to gNB’s implementation. In this sense, the highlighted part in FG 51-2 shall be removed.

We also addressed some typos in current text.

Proposal: Update the FG 51-2 for supporting smaller than 5 MHz channel bandwidth (changes in red):

| 51. NR_FR1_lessthan_5MHz_BW | 51-2 | Support 12 PRB CORESET0 | 1) Reception of 12 PRB CORESET0 | 51-1 | Yes | N/A | UE is not able to support 3 MHz channel bandwidth with 12 PRB CORESET0 | Per Band | FDD only | FR1 only | N/A | This FG is supported for 15 kHz SCS only [This FG is applicable to the case when transmission BW is limited within 12 PRB] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# Conclusion

Proposal from this contribution is summarized as follow:

Proposal: Update the FG 51-2 for supporting smaller than 5 MHz channel bandwidth (changes in red):

| 51. NR_FR1_lessthan_5MHz_BW | 51-2 | Support 12 PRB CORESET0 | 1) Reception of 12 PRB CORESET0 | 51-1 | Yes | N/A | UE is not able to support 3 MHz channel bandwidth with 12 PRB CORESET0 | Per Band | FDD only | FR1 only | N/A | This FG is supported for 15 kHz SCS only [This FG is applicable to the case when transmission BW is limited within 12 PRB] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# Reference

[1] Chairman notes, 3GPP RAN1#114b, Xiamen, China, October, 2023.