# 3GPP TSG RAN WG1 #115							R1-2312059

# Chicago, USA, November 13th – November 17th, 2023

Agenda item:	8.15.2

Source: 	Qualcomm Incorporated

Title: 	Self-evaluation results for NR NTN

Document for:	Discussion and Decision

# Background

Agreement

For DL and UL overhead:

- DL overhead is assumed to be 0.14

- UL overhead is assumed to be 0.08

Agreement

For the evaluation metrics that require usage of beam area, a beam area of 1415 km2 is assumed.

Agreement

The modeling of delay / RTT for SLS is up to companies to report.

Agreement

All the beams are considered for computing the area of connection density.

Agreement

For SLS to LLS metric of connection density, “pre-processing SINR” should be used instead of “pre-processing SNR”.

Agreement

Elevation angle of 90° is used for determining mean and standard deviation values of K-factor and delay spread for NTN TDL-C Rural channel model.

Agreement

For mMTC LLS (NR and eMTC), 16-QAM can be used in addition to QPSK to derive the SNR to SE mapping.

Agreement

For connection density evaluation with full buffer system-level simulation followed by link-level simulation, for SINR CDF distribution derivation, all the beams are assumed to be fully loaded.

Agreement

3GPP will report peak data rate and spectral efficiency based on

- Rmax=822/1024 and Qm=6 for DL

- Rmax=553/1024 for and Qm=4 for UL

Agreement

The results of CL, Geometry SIR and Geometry SINR simulated on DL are to be reported within the attached template.

In RAN1#114, the following was agreed.

In RAN1#114b, the following was agreed

Agreement

The attached template is used to collect evaluation results for eMBBs 5th percentile user spectral efficiency, average spectral efficiency, user experienced data rate and area traffic capacity.

Note: OH as agreed in 9.14.1 will be removed from the template.

Agreement

The attached template is used to collect evaluation results for the TPR on reliability.

Agreement

The attached template is used to collect evaluation results for the TPR on connection density.

Agreement

RAN1 to provide at RAN1#114-bis evaluation results on Energy efficiency, including both network and device.

Agreement

For NR NTN Connection Density LLS:

- Agree 10% BLER as target for throughput for Full Buffer

- SINR - user spectral efficiency (SE) mapping graph for Full Buffer, where:

  - SE = nominal SE × (1-BLER)

(several agreements omitted for brevity):

Agreement

The same energy efficiency aspects from the terrestrial self-evaluation in the report TR 37.910 apply for NTN as well on network side and device side. Thereby the text on the energy efficiency requirement from TR 37.910 can be reused for TR 37.911 with changes as necessary, considering only 15 kHz subcarrier spacing.

This document is a minor revision of R1-2310391. No new additional evaluation results have been added.

# Calibration curves

The following agreement was reached in RAN1#114:

The results of CL, Geometry SIR and Geometry SINR simulated on DL are to be reported within the attached template.

In line with this agreement, we present the CDFs of coupling loss, geometry, and SINR for case 9 and 10 in attached spreadsheet.

The simulation was performed according to Section 8.1.1 [1] with Set-1 satellite parameters and handheld UEs. Note that two additional tiers of beams for FRF 1 and three additional tiers of beams for FRF 3 were simulated for intra-satellite interference modelling. However, only the statistics of the UEs connecting to the inner 19 beams were collected (see Fig. 6.1.1.1-1 [1] for the beam layout based on FRF configurations). The CDF of coupling loss, geometry, and SINR for case 9 (FRF 1) and case 10 (FRF 3) observed for DL transmissions are presented in Fig. 2.1. These CDFs are in agreement with the average performance reported by different companies [2].  Note that scintillation and atmospheric losses were set to 0, as per agreed guidelines. As we observe in Fig 2, coupling loss for FRF 1 and 3 are the same. However, the interference for FRF 3 is less than FRF 1, which leads to better geometry SIR and SINR.

| Figure 2.1: Coupling loss for Case 9 and 10 DL transmissions | Figure 2.2: Geometry for Case 9 and 10 DL transmissions |
| --- | --- |
| Figure 2.3: SINR for Case 9 and 10 DL transmissions |  |

# eMBB-s: Spectral efficiency, area traffic capacity, user experienced data rate

## Evaluation assumptions

For the evaluation of 5th percentile user spectral efficiency, user experienced data rate, average spectral efficiency, and area traffic capacity, we followed the steps outlined in [3]. We only considered full-buffer traffic for the system-level simulation (SLS). All simulation assumptions for these metrics are in agreement with [4] and listed in Table A.1 in the appendix. To put the simulation results into perspective, we clarify few simulation assumptions in this section.

- For DL, we assumed 1Tx, 2 Rx and 1Tx, 4 Rx transmission with cross polarized UE antennas. For UL, we assumed 1 Tx, 1 Rx transmission.

- For UL, the scheduling scheduling granularity was set to 10 RBs.

- For calculation of spectral efficiency, the channel bandwidth was set to 30 MHz for FRF 1 and FRF 3.

## Evaluation results for spectral efficiency and user experienced data rate

The results for DL and UL are presented in Table 1. For FRF1, no metric meets the requirement. For FRF 3 and 2 UE antennas, all metrics meet the requirements except DL average spectral efficiency and user experienced data rate. We further considered UEs with 4 Rx antennas, which are able to meet the requirement for average spectral efficiency and user experienced data rate.

Table 1 Evaluation results for eMBB

| SE | Number of UE antennas |  |  | Frequency reuse factor |  | Requirement [5] |  |  | Reported Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL Spectral efficiency | 2 |  |  | FRF = 1 |  | Average [bit/s/Hz/TRxP] | 0.5 |  | 0.382 |  |
|  |  |  |  |  |  | 5th percentile [bit/s/Hz] | 0.03 |  | 0.0159 |  |
|  |  |  |  | FRF = 3 |  | Average [bit/s/Hz/TRxP] | 0.5 |  | 0.4914 |  |
|  |  |  |  |  |  | 5th percentile [bit/s/Hz] | 0.03 |  | 0.0307 |  |
|  | 4 |  |  | FRF = 3 |  | Average [bit/s/Hz/TRxP] | 0.5 |  | 0.5054 |  |
|  |  |  |  |  |  | 5th percentile [bit/s/Hz] | 0.03 |  | 0.0337 |  |  |  |
| SE | Number of UE antennas |  |  | Frequency reuse factor |  | Requirement [5] |  |  | Reported Value |  |  |  |
| UL spectral efficiency | 1 |  |  | FRF = 1 |  | Average [bit/s/Hz/TRxP] | 0.1 |  | 0.0779 |  |  |  |
|  |  |  |  |  |  | 5th percentile [bit/s/Hz] | 0.003 |  | 0.0022 |  |  |  |
|  |  |  |  | FRF = 3 |  | Average [bit/s/Hz/TRxP] | 0.1 |  | 0.1551 |  |  |  |
|  |  |  |  |  |  | 5th percentile [bit/s/Hz] | 0.003 |  | 0.0043 |  |  |  |
| User experienced data rate |  | Number of UE antennas | Frequency reuse factor |  | Requirement [5] |  |  |  |  | Reported Value |  |
| DL |  | 2 | FRF = 1 |  | User experienced data rate (Mbit/s) |  |  | 1 |  | 0.476 |  |
|  |  |  | FRF = 3 |  | User experienced data rate (Mbit/s) |  |  | 1 |  | 0.9205 |  |
|  |  | 4 | FRF = 3 |  | User experienced data rate (Mbit/s) |  |  | 1 |  | 1.011 |  |
| User experienced data rate |  | Number of UE antennas | Frequency reuse factor |  | Req. |  |  |  |  | Reported Value |  |
| UL |  |  | FRF = 1 |  | User experienced data rate (Mbit/s) |  |  | 0.1 |  | 0.0665 |  |
|  |  |  | FRF = 3 |  | User experienced data rate (Mbit/s) |  |  | 0.1 |  | 0.133 |  |

Proposal 1: Capture the evaluation results in Table 1 in TR 37.911

## Evaluation results for area traffic capacity

For area traffic capacity evaluation, we first need to compute the geographical area covered a beam. In RAN1#114, the following was agreed.

For the evaluation metrics that require usage of beam area, a beam area of 1415 km2 is assumed.

Based on this agreement and the evaluation results reported in Table 1, values of area traffic capacity are reported in Table 3. We observe that FRF 1 and 3 meet the requirements for DL and UL transmissions.

Table 2: Area Traffic Capacity

| Area traffic Capacity | Number of UE antennas | Frequency reuse factor | Requirement [5] |  | Reported Value |
| --- | --- | --- | --- | --- | --- |
| DL | 2 | FRF = 1 | Area traffic capacity (kbit/s/km2) | 8 | 8.0989 |
|  |  | FRF = 3 | Area traffic capacity (bit/s/km2) | 8 | 10.419 |
|  | 4 | FRF = 3 | Area traffic capacity (bit/s/km2) | 8 | 10.707 |
| Area traffic Capacity |  | Frequency reuse factor | Requirement [5] |  | Reported Value |
| UL |  | FRF = 1 | Area traffic capacity (bit/s/km2) | 1.5 | 1.6537 |
|  | 1 | FRF = 3 | Area traffic capacity (bit/s/km2) | 1.5 | 3.2862 |

Proposal 3: Capture the UL evaluation results in Table 2 in TR 37.911

# eMBB-s: Mobility

## Evaluation assumptions

For mobility evaluations, we follow the procedure mentioned in [3]. We performed simulations for FRF 3 only. First, we obtained the CDF of pre-processing SINR for UL transmissions from the SLS. The parameters that are used for LLS are presented in the Table 4 below.

Table 3: LLS parameters for eMBB-s Mobility

| Physical channel | PUSCH |
| --- | --- |
| Simulation bandwidth (PRB) | 5 RBs (Consistent with SLS BW) |
| Number of users in simulation | 1 |
| Link-level Channel model | NTN TDL-C Rural |
| DMRS config | 2 symbol DMRS |
| Antenna configuration at Satellite | 1 Rx |
| Antenna configuration at UE | 1 Tx |
| Transmission mode | SISO |
| Transmission rank | 1 |
| TBS | Based on MCS Sweep |
| Modulation order | Based on MCS Sweep |
| Number of repetitions | [1,2,4,8] |
| Channel estimation | LMMSE |
| Channel coding scheme | LDPC |
| Doppler spread | 463 Hz |

## Evaluation results

Figure 4.2.1: CDF of UL pre-processing SINR from SLS for FRF 3 with RBG size set to 5 PRBs

The CDF of uplink pre-processing SINR obtained from SLS for FRF 3 is shown in Fig 4.2.1. We obtain the 50th percentile SINR from the CDF above and perform mobility analysis of a UE moving at 250 km/hr. We aim to find the maximum spectral efficiency that can be achieved under the reliability constraints provided in [5] (packet error ratio < 1%). At the 50th percentile SINR, we run a sweep over all the MCS’es (Table 6.1.4.1-1 [6]) with up to 8 repetitions. The spectral efficiency ρ at repetition number rep with MCS m and throughput of Tputm, bandwidth of W and BLER ηrep,m can be written as:

ρm, rep=1-ηrep,m×Tputmrep×W,

where Tputm is the throughput for MCS m and can be obtained by dividing TBSize for MCS m by 1 ms (slot duration). Note that the BLER, ηrep,m, is a function of repetition number and MCS index as well. The maximum supported spectral efficiency ρmaxmobility under the given reliability constraints can be obtained by solving the following:

ρmaxmobility=maxm,repρ(m,rep) s.t.ηrep,m<1%

The maximum achievable spectral efficiency with its reliability has been reported in Table 5. Note that the spectral efficiency is the same as normalized traffic channel link data rate. The number of repetitions needed to achieve maximum spectral efficiency is 4 with MCS 7 (TBSize = 736 bits).

Table 4: Performance evaluation of mobility

| Parameter | Value |
| --- | --- |
| FRF | 3 |
| 50 percentile SINR | 0.95 dB |
| System Bandwidth (W) | 0.9 MHz (5 PRBs) |
| Packet Error Ratio | 0.62% (requirement: <1%) |
| Normalized traffic channel link data rate | 0.203 bps/Hz (requirement: 0.005 bps/Hz) |

From Table 4, we observe that NR-NTN meets the packet error ratio requirement of (< 1% [1]), while delivering normalized traffic channel link data rate of 0.203 bps/Hz (requirement is 0.005 bps/Hz [5]).

Proposal 4: Capture the evaluation assumptions in Table 3 and evaluation results in Table 4 in TR 37.911.

# HRC-s: Reliability

## Evaluation assumptions

For reliability simulations, we again consider FRF 3 only for NR-NTN UL and DL. We follow the procedure from [3] to obtain the CDF of SINR from SLS. The parameters for LLS are reported in Table 5. To meet the target requirement for reliability, we consider 16 repetitions for UL. These repetitions span a total duration of 36 ms. For the first 8 ms, PUSCH with RV 0 is transmitted over 8 slots of TBoMS. Then, there is a gap of 20ms to leverage time diversity. After that, PUSCH with RV 2 is transmitted with another 8 slots of TBoMS.

Table 5: Parameters for LLS Reliability for NR UL and DL

|  | NR Uplink | NR Downlink |
| --- | --- | --- |
| Physical channel | PUSCH | PDSCH |
| SCS | 15 kHz | 15 kHz |
| Simulation bandwidth (PRBs) | 5 RBs | 8 PRBs for 64 QAM MCS Table, 30 RBs for low SE MCS Table from [3] |
| Number of users in simulation | 1 | 1 |
| Link-level Channel model | NTN TDL-C Rural | NTN TDL-C Rural |
| DMRS config | 2 DMRS per slot | 2 symbol DMRS |
| Antenna configuration at Satellite | 1Rx | 1Tx |
| Antenna configuration at UE | 1Tx | 2 Rx |
| Transmission mode | SISO | SISO |
| Transmission rank | 1 | 1 |
| TBS | 256 | 256 |
| Modulation order | QPSK | QPSK |
| Number of repetitions | 16 (8 TBoMS + 20ms + 8 TBoMS) | 1 |
| Channel estimation | LMMSE | LMMSE |
| Channel coding scheme | LDPC | LDPC |
| Doppler spread | 5 Hz | 5 Hz |
| MCS | MCS 2 from Table 6.1.4.1-2 of [3] | MCS 0 |

## Evaluation results

The CDF for UL SINR from SLS for FRF 3 has been shown in Fig. 4.2.1. The CDF for DL SINR from SLS for FRF 3 has been shown in Fig. 5.2.1. We pick the 5 percentile pre-processing SINR point and proceed with the reliability analysis under the guidance from [3].

Figure 5.2.1: CDF of DL pre-processing SINR from SLS for FRF 3

The results for UL and DL reliability after LLS have been reported in Table 6. According to [5], the reliability requirement is 1-10-3 or 99.9%. We observe from Table 6 that we can meet the reliability requirements for both UL and DL NR-NTN.

Table 6: Performance evaluation of reliability

| Parameter | Mode |  |
| --- | --- | --- |
|  | Uplink | Downlink |
| FRF | 3 | 3 |
| 5 percentile SINR | -0.63 dB | 6.79 dB |
| Scheduled Bandwidth | 0.9 MHz (5 PRBs) | 8 PRBs, 30 PRBs |
| Reliability | 99.96 % | 99.97 %, 99.99% |
| Connection Time Duration | 36 ms | 1 ms |

Proposal 5: Capture the evaluation assumptions in Table 5 and evaluation results in Table 6 in TR 37.911.

# mMTC-s: Connection density

## Evaluation assumptions

For connection density evaluations with NR-NTN, we follow the procedure mentioned in [3] for full buffer scenario. We obtain the CDF of pre-processing SINR for UL transmissions from SLS for both FRF=1 and 3. We assume that each device is scheduled over 1 PRB which maximizes the number of supported devices per cell.

The LLS parameters have been provided in the Table 7 below.

Table 7: LLS parameters for NR-NTN mMTC evaluations

| mMTC NR |  |
| --- | --- |
| Physical channel | PUSCH |
| Simulation bandwidth (=bandwidth allocated per UE) | 1 PRB |
| SCS | 15kHz |
| Link-level Channel model | NTN TDL-C Rural |
| Antenna configuration at Satellite | 1Rx |
| Antenna configuration at UE | 1Tx |
| Transmission mode | SISO |
| Transmission rank | 1 |
| TBS | Based on MCS Sweep |
| Modulation order | Based on MCS Sweep |
| Number of repetitions | 1,2,4,8 (TBoMS) |
| Channel estimation | LMMSE |
| Channel coding scheme | LDPC |
| Doppler spread | 5 Hz |
| UL DMRS config | - DMRS per slot |

## Evaluation results

The CDFs of uplink SINR with full buffer assumption for mMTC and FRF = 1,3 are shown in Fig. 6.2.1.

Figure 6.2.1. CDF of UL pre-processing SINR

Based on the CDF reported above, we calculate the maximum spectral efficiency that can be supported at a particular SINR. This is done by doing a sweep over different MCS’es (Table 6.1.4.1-1 of [6]). The spectral efficiency ρ at SINR γ at MCS m with throughput of Tputm for repetition rep, bandwidth per user Wuser and BLER ηm can be written as:

ργ, m,rep=1-ηmγ,rep×Tputm,repWuser

Tputm,rep is the throughput for MCS m at repetition rep and can be obtained by dividing TBSize for MCS m by slot duration for TBoMS (1 rep = 1 ms). Wuser is 180 kHz. Note that BLER, ηm, in the equation above is also a function of SINR, repetition number and MCS index. BLER is output of LLS. The maximum supported spectral efficiency at a given SINR with a 10% BLER constraint can be calculated as:

ρmaxγ=maxm,repργ, m,rep s.t ηmγ,rep<10%

ρmax has been plotted in Fig. 6.2.2 as SINR as varied for FRF =1 and 3. Note that each SINR corresponds to a percentile value from the CDF reported in Fig. 6.2.1.  The maximum supported rate is  Rmaxγ=ρmaxγWuser.

Figure 6.2.2: Maximum supported spectral efficiency for NR-NTN mMTC

Based on the above rate at 1 percentile of SINR (99th percentile of delay), we calculate the delay per user as per the procedure in [3]. We report the delay in for different system bandwidth and different FRF in Table 8.

| Parameter | Values |  |  |  |
| --- | --- | --- | --- | --- |
|  | For FRF = 1 |  | For FRF = 3 |  |
| Service Profile | Full buffer |  | Full buffer |  |
| 1 percentile pre-processing SINR | -5.08 dB |  | 3.27 dB |  |
| System Bandwidth (W) | 10 MHz, 180 KHz |  | 10 MHz, 180 KHz |  |
| 99th Percentile Delay | 0.0088 s |  | 0.0044 s |  |
| Inter-packet arrival time | 1 message/2 hours/device | 1 message/day/device | 1 message/2 hours/device | 1 message/day/device |
| Connection Density | 41,513 Devices/ km2 , 747 Devices/ km2 | 498,158 Devices/ km2 , 8966 Devices/km2 | 129,041 Devices/ km2 , 2322 Devices/ km2 | 1,548,494 Devices/km2, 27,972 Devices/km2 |

Additionally, based on different inter-packet arrival times and system bandwidths, we report the connection density in Table 8. Area of 1415 km2 is considered for connection density calculation, as per the agreement. The connection density requirement according to [5] is 500 Devices/km2 and 99th percentile delay requirement is less than 10s. We see that both the requirements are met for the case of NR-NTN mMTC for all FRF, system bandwidths and inter-packet arrival times.

Table 8: Performance metrics for NR-NTN mMTC

Proposal 6: Capture the evaluation assumptions in Table 7 and evaluation results in Table 8 in TR 37.911.

# Energy Efficiency

In this section we propose a text proposal for energy efficiency of NR NTN. The evaluation methodology is referring to Sect. 5.8 of TR 37.910 with minor changes to adapt to the NTN specifics. As agreed in RAN1#114b, only 15kHz SCS is included in the evaluations.

<Begin text proposal>

As defined in Report ITU-R M.2410 [4], network energy efficiency is the capability of a RIT/SRIT to minimize the radio access network energy consumption in relation to the traffic capacity provided. Device energy efficiency is the capability of the RIT/SRIT to minimize the power consumed by the device modem in relation to the traffic characteristics.

The RIT/SRIT shall have the capability to support a high sleep ratio and long sleep duration.

The sleep ratio is the fraction of unoccupied time resources (for the network) or sleeping time (for the device) in a period of time corresponding to the cycle of the control signaling (for the network) or the cycle of discontinuous reception (for the device) when no user data transfer takes place. The sleep duration is the continuous period of time with no transmission (for network and device) and reception (for the device).

## Network side

#### 7.1.1	NR

The sleep ratio and sleep duration for NR network under unloaded case are evaluated.

When no data transfer takes place, NR network will keep periodical transmission of SS/PBCH blocks and RMSI (remaining minimum system information), as well as paging signal in order for UEs to detect and access the radio network. The following mechanisms for SS/PBCH block, RMSI and paging are assumed for the evaluation.

For SS/PBCH block transmission, the following configurations are considered in evaluation.

-	One SS/PBCH block occupies 4 OFDM symbols with 20 RBs in one slot.

-	One or multiple SS/PBCH block(s) compose an "SS burst set" (SSB set).

-	Denote L as the number of SS/PBCH blocks in an SSB set, where L can be 1~ 64. For below 3 GHz, the maximum value of L is 4; for below 7.125 GHz, the maximum value of L is 8.

-	One SSB set transmission is confined to a half radio frame (5 ms) window

-	The SSB set periodicity (PSSB) can be configured to be {5, 10, 20, 40, 80, 160} ms

-	The following mapping is used in a half radio frame for 15kHz SCS

-	2 SS/PBCH blocks is transmitted in one slot. And the L SS/PBCH blocks in an SSB set is transmitted in successive slots from the first slot in one SSB set period.

For RMSI transmission, the following configurations are considered in evaluation.

-	One RMSI transmission occupies 2 OFDM symbols in one slot.

-	RMSI is multiplexed with SS/PBCH block using the following ways:

-	For FR1, RMSI is time division multiplexed (TDMed) with SS/PBCH block.

-	SS/PBCH block and RMSI could be transmitted in the same slot for both TDM and FDM.

-	RMSI periodicity (PRMSI) is assumed as follows:

-	20ms for SSB set periodicity less than or equal to 20ms;

-	Otherwise RMSI periodicity equals to SSB set periodicity.

-	The following mapping is used

-	One RMSI transmission corresponds to one SS/PBCH block

-	If L SS/PBCH block is transmitted, then L RMSI transmissions are required.

-	One slot accommodates 2 RMSI transmissions.

-	The offset of RMSI transmission can be set as {0, 2, 5, 7}ms with respect to every 20ms time point. In the evaluation, the offset value that allows the closest RMSI transmission to SS/PBCH block transmission is selected.

For paging occasion,

-	The periodicity of paging occasion is the same as that of SSB set, and it is FDMed with an SS block.

Figure 7-1 illustrates NR SS/PBCH block and RMSI transmission which employs the above mentioned mechanism.

Half frame

(

5

ms

)

SS block

SS block

SS block

SS block

“SSB set”

“SSB set”

periodicity

Half frame

(

5

ms

)

RMSI

RMSI

RMSI

RMSI

Figure 7-1 Illustration of NR SS/PBCH block and RMSI transmission

##### 7.1.1	Evaluation of sleep ratio

Based on the above mechanisms, the sleep ratio per slot basis and per symbol basis are given as follows,

where  indicates the ceiling of x,  is the numerology (as defined in TS 38.211 [7], e.g., =0 for 15 kHz SCS, L is the number of SS/PBCH blocks in one SSB set, PSSB is the SSB set periodicity, PRMSI is the RSMI periodicity, and =1 for FR1.

Evaluation results are shown in Table 7.1-1 and Table 7.1-2, respectively, for slot level and symbol level sleep ratio. It is observed that with SSB set period of 5ms, more than 80% of sleep ratio can be obtained by NR network; with SSB set period of larger than 10ms, more than 90% of sleep ratio can be obtained by NR network. Higher sleep ratio is expected with finer sleep granularity, e.g., in symbol level. Note that a subset of configurations in terms of number of SSB per set is used to derive the results.

Therefore NR network can achieve high sleep ratio in unloaded case.

Table 7.1-1 NR network sleep ratio in slot level

| SSB configuration |  | SSB set periodicity PSSB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SCS [kHz] | Number of SS/PBCH block per SSB set, L | 5ms | 10ms | 20ms | 40ms | 80ms | 160ms |
| 15kHz | 1 | 80.00% | 90.00% | 95.00% | 97.50% | 98.75% | 99.38% |
|  | 2 | 80.00% | 90.00% | 95.00% | 97.50% | 98.75% | 99.38% |

Table 7.1-2 NR network sleep ratio in symbol level

| SSB configuration |  | SSB set periodicity PSSB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SCS [kHz] | Number of SS/PBCH block per SSB set, L | 5ms | 10ms | 20ms | 40ms | 80ms | 160ms |
| 15kHz | 1 | 93.57% | 96.43% | 97.86% | 98.93% | 99.46% | 99.73% |
|  | 2 | 87.14% | 92.86% | 95.71% | 97.86% | 98.93% | 99.46% |

##### 7.1.1.2   Evaluation of sleep duration

Based on the above mechanisms, evaluation results of sleep duration are provided in Table 7.1-3. It is observed that with SSB set period of 160 ms, more than 150ms sleep duration can be obtained by NR network. Therefore NR network can achieve long sleep duration in unloaded case.

Therefore, NR meets network side energy efficiency requirement.

Table 7.1-3 NR network sleep duration (ms) in slot level

| SSB configuration |  | SSB set periodicity PSSB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SCS [kHz] | Number of SS/PBCH block per SSB set, L | 5ms | 10ms | 20ms | 40ms | 80ms | 160ms |
| 15kHz | 1 | 4.00 | 9.00 | 19.00 | 39.00 | 79.00 | 159.00 |
|  | 2 | 4.00 | 9.00 | 19.00 | 39.00 | 79.00 | 159.00 |

## Device side

#### 7.2.1	NR

The sleep ratio and sleep duration for NR UEs under unloaded case are evaluated.

For NR, DRX is supported for UEs in idle, inactive and connected states.

##### 7.2.1.1	Evaluation of sleep ratio

For idle state and inactive state, the UE should monitor one paging occasion per discontinuous reception (DRX) cycle (which equals to the paging cycle), and the UE can use DRX to reduce power consumption. Before paging receiving, the SSB monitoring is needed. Also RRM measurement(s), including intra- and inter-cell shall be performed.

The DRX cycle for idle state / inactive state UE consists of an "On Duration" during which the UE should perform SSB monitoring, paging monitoring and RRM measurement, and an “Off Duration” during which the UE can skip reception of downlink channels to save energy. It is illustrated in Figure 7-2.

Therefore the sleep ratio is determined by the length of "On Duration" and the length of one paging cycle.

Figure 7-2 Illustration of DRX cycle in connected state

When DRX is used, the UE wakes up and receives SSB for synchronization, listens to PDCCH only on specific paging occasion defined in-terms of paging frame and subframe within period of NPC_RF radio frames defined by the DRX cycle (paging cycle) of the cell and performs RRM measurement. The UE can remain in sleep mode for remaining duration within DRX cycle.

For synchronization, one SSB-burst set is assumed for short paging cycle (e.g., 320ms). Further, it is assumed that synchronization signal can be located in the same slot as paging-on slot and UE can finish network synchronization before paging monitoring. For longer paging cycle, one SSB-burst can still be assumed. In addition, to improve synchronization accuracy, the case of two SSB-burst sets is also evaluated. In this case, UE needs additional time up to one SSB cycle for SSB reception.

For paging monitoring, a paging occasion can consist of multiple time slots (e.g. slot or OFDM symbol) where paging DCI can be sent. In the evaluation, it is assumed that one paging occasion consists of one slot. On the other hand, one paging cycle consists of one or multiple Paging Frames. One Paging Frame may contain one or multiple paging occasion(s) or starting point of a PO. In the evaluation, it is assumed that one Paging Frame contains one paging occasion and time for paging monitoring is not longer than that of one SSB burst.

RRM measurement is based on SS/PBCH. In the evaluation, it is assumed that RRM measurement takes place in “On Duration” time, and the RRM measurement time is assumed to be 3ms for FR1 (see TS 38.133 [12]).

In addition to the above procedure, transition time is needed for UE to switch on / off its components. 10ms transition time is assumed for evaluation; but further reduced value is possible.

Based on the above analysis, the idle mode sleep ratio is evaluated with the configurations shown in Table 7.2-1. It is observed that more than 90% sleep ratio is achieved in idle mode by NR device.

Table 7.2-1 NR device sleep ratio in slot level (for idle / inactive mode)

|  | Paging cycle NPC_RF *10 (ms) | SCS(kHz) | SSB L | SSB reception time(ms) | SSB cycle (ms) | Number of SSB burst set | RRM measurement time per DRX (ms) | Transition time(ms) | Sleep ratio |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RRC-Idle/Inactive | 2560 | 15 | 2 | 1 | -- | 1 | 3 | 10 | 99.5% |
|  | 2560 | 15 | 2 | 1 | 160 | 2 | 3 | 10 | 93.2% |

NOTE:	For SSB period, "--" is assumed that SSB reception is during DRX-On time.

For connected state, if there is no data transmission in either downlink or uplink direction, the DRX mode is switched on.

The DRX cycle for connected state UE consists of an “On Duration” during which the UE should perform SSB monitoring, PDCCH monitoring (reflected as DRX-onDurationTimer), and RRM measurement, and an “Off Duration” during which the UE can skip reception of downlink channels to save energy. Also, transition time is assumed in “On Duration”.  The connected mode sleep ratio for different DRX cycles is shown in Table 7.2-2. Therefore NR device can achieve high sleep ratio for both idle/inactive state and connected state in unloaded case.

Table 7.2-2 NR device sleep ratio in slot level (for connected mode)

|  | DRX cycle TSC_ms * MSC (ms) | Number of SSB burst set | DRX-onDurationTimer(ms) | RRM measurement time per DRX (ms) | Transition time(ms) | Sleep ratio |
| --- | --- | --- | --- | --- | --- | --- |
| RRC-Connected | 320 | 1 | 10 | 3 | 10 | 92.8% |
|  | 2560 | 1 | 100 | 3 | 10 | 95.6% |
|  | 10240 | 1 | 1600 | 3 | 10 | 84.2% |

NOTE:	For SSB period, "--" is assumed that SSB reception is during DRX-On time.

##### 7.2.1.2	Evaluation of sleep duration

The sleep duration for NR UE in idle mode is 2546ms for paging cycle of 2560ms with the assumed parameters.

The sleep duration of NR UE in connected state is 8627ms for paging cycle of 10240ms with the assumed parameters.

Consequently NR device can achieve very long sleep duration in both idle mode and connected mode.

It is therefore concluded that NR meets device side energy efficiency requirement.

<End text proposal>

Proposal 6: Endorse the text proposal on energy efficiency in Section 7.

# 8	Conclusions

Proposal 1 (eMBB-s): Capture the evaluation results in Table 1 in TR 37.911

Proposal 2 (eMBB-s): Capture the UL evaluation results in Table 2 in TR 37.911

Proposal 3 (eMBB-s): Capture the evaluation assumptions in Table 3 and evaluation results in Table 4 in TR 37.911.

Proposal 4 (HRC-s): Capture the evaluation assumptions in Table 5 and evaluation results in Table 6 in TR 37.911.

Proposal 5 (mMTC-s): Capture the evaluation assumptions in Table 7 and evaluation results in Table 8 in TR 37.911.

Proposal 6 (Energy Efficiency): Endorse the text proposal on energy efficiency in Section 7.

# References

[1] TR38.821, “Solutions for NR to support non-terrestrial networks (NTN)”, v16.1.0.

[2] R1-1913404 "System Level Calibration Results for NTN on DL transmissions", Thales, submitted to RAN1#99

[3] ITU-R, “Report  ITU-R  M.2412-0”.

[4] R1-2304123, “Feature lead summary #4 on evaluation methodology for IMT-2020 Satellite”.

[5] ITU-R, “Report  ITU-R  M.2514-0”.

[6] TS 38.214 “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Physical layer procedures for data”, v17.0.0

# Annex

Table A.1 : Parameters for system-level simulation

| Satellite orbit | LEO-600 |
| --- | --- |
| Satellite altitude | 600 km |
| Satellite antenna pattern | Section 6.4.1 in [1] |
| Satellite antenna polarization | Circular |
| Satellite antenna number | 1 Tx / 1 Rx per beam |
| 3 dB beam width | 4.41 degrees |
| Satellite EIRP density | 34 dBW/MHz |
| Satellite antenna gain | 30 dBi |
| Satellite G/T | 1.1 dB/K |
| Central beam center elevation | 90 deg |
| UE anntenna type | Handheld, (1,1,2) with omni-directional antenna element |
| UE antenna polarization | Linear: +/- 45deg X-pol |
| UE Rx Antenna gain | 0 dBi |
| UE antenna temperature | 290 K |
| UE noise figure | 7 dB |
| FRF | 1 or 3 |
| Carrier frequency | 2 GHz |
| SCS | 15 kHz |
| Channel bandwidth | 30 MHz for FRF1, 10MHz for FRF3 |
| Scenario | Rural-eMBB-s |
| UE deployment | 100% outdoor and uniformly distributed over the area |
| LOS condition | 100% LOS |
| Spot beam pattern and frequency reuse factor | Hexagonal pattern, 19 inner beams, Total beams: 61 beams for FRF=1, 127 beams for FRF=3. |
| UE density | 10 UEs per beam |
| UE mobility | 0 (Stationary) |
| Satellite mobility | 0 (Doppler spread is assumed to be compensated) |
| Large scale channel model | large scale model of Section 6.6 in 38.811 |
| Small scale channel model | Frequency selective fading model of Section 6.7.2 in 38.811 |
| Handover margin | 0dB (handover was not simulated) |
| UE attachment | RSRP |
| Traffic model | Full-buffer |
| Scheduling scheme | PF and SU-MIMO |
| Receiver type | MMSE-IRC |
| Channel estimation | Realistic |
| Frequency offset | 0ppm |
| Frequency drift | 0ppm |
| DL CSI measurement | CQI only (1 layer / 1-port CSI-RS) |
| PRB bundling | wideband |
| Codeword (CW) | SCW |
| Transmission scheme | One layer |
| Frame structure | FDD |
| Overhead | 0.14 (same as for peak data rate calculation) |