3GPP TSG RAN WG1 #115	                                   					       												                           R1-2312079

Chicago, USA, November 13th – November 17th, 2023

Agenda Item:	8.16.1

Source:	Moderator (AT&T)

Title:	Summary of UE features for NR MIMO evolution

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email discussion [115-R18-UE_features-02] during RAN1 #115. According to the Chair’s Notes:

[115-R18-UE_features-02] Email discussion on UE features for MIMO, positioning, NCR, NR-NTN, IoT-NTN, BWP without restriction, NW energy saving, mobility enhancement – Ralf (AT&T) - To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

The following was discussed and/or agreed during RAN1 #115 within the scope of [115-R18-UE_features-02]. All proposals are based on the latest RAN1 UE features list for Rel-18 in [1].

# Summary of Contributions Submitted to RAN1 #115

The following is the moderator’s summary of contributions submitted to RAN1 #115 in this agenda item.

| 40. NR_MIMO_evo_DL_UL | 40-1-1 | Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC | FFS: maximum number of configured joint TCI states per CC per BWP FFS: maximum number of activated joint TCI states across all CCs FFS: what to do/signal about additional support of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH | FFS | yes | n/a | Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC is not supported | Per band | n/a | n/a | n/a | Note: FG 16-2b-0 can be used to indicate support of two default beams | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-1-1 - FFS1: Candidate values for the maximum number of configured joint TCI states per CC per BWP {8, 12, 16, 24, 32, 48, 64, 128} is added in the notes - FFS2: Candidate values for the maximum number of activated joint TCI states across all CCs {2, 4, 8, 16} is added in the notes - FFS3: What to do/signal about additional support of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH – It is unclear and this FFS may be removed. 40. NR_MIMO_evo_DL_UL; 40-1-1; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC; FFS: maximum number of configured joint TCI states per CC per BWP FFS: maximum number of activated joint TCI states across all CCs FFS: what to do/signal about additional support of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH; FFS; yes; n/a; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC is not supported; Per band; n/a; n/a; n/a; Note: FG 16-2b-0 can be used to indicate support of two default beams Candidate values for the maximum number of configured joint TCI states per CC per BWP {8, 12, 16, 24, 32, 48, 64, 128} Candidate values for the maximum number of activated joint TCI states across all CCs {1, 2, 4, 8, 16}; Optional with capability signalling. |
| Ericsson [3] | The features related to TCI states are typically provided per band, or per UE. We propose to reuse this for the features related to the multi-TRP extension of the unified TCI framework: As baseline, the UE features related to the multi-TRP extension of the unified TCI framework are provided per band. When designing the features for unified TCI framework extension for multi-TRP, it is useful to revisit how the UE features for the unified TCI framework in Rel-17 was designed. Many of the choices made in Rel-17 are relevant also when designing the UE features for the Rel-18 framework. It is useful to revisit the main structure of the Rel-17 UE features for the unified TCI framework (FG 23-1-1x and FG 23-1-10x), in particular for the basic functionality: FG 23-1-1 included intra-cell operation, one activated TCI state, and only MAC CE-based beam indication. FG 23-1-1a included intra- and inter-cell operation, multiple activated TCI states, and MAC CE-based indication. FG 23-1-1b included intra- and inter-cell operation, multiple activated TCI states, and DCI-based indication. Currently, this structure is not used. “FFS: maximum number of configured joint TCI states per CC per BWP”. Following the Rel-17 solution, this would be a component in FG 40-1-1. “FFS: maximum number of activated joint TCI states across all CCs”. Following the Rel-17 solution, this would be a component in FG 40-1-1. “FFS: what to do/signal about additional support of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH.” It is unclear what this means. The FFS can be removed. There is no need for any pre-requisite for this feature group. For example, a UE that advertises support for FG 40-1-1 would obviously support single-DCI multi-TRP. 40-1-1; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC; maximum number of configured joint TCI states per CC per BWP maximum number of activated joint TCI states across all CCs; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC is not supported; Per band; Note: FG 16-2b-0 can be used to indicate support of two default beams. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-1-1; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC; FFS: 1. Maximum number of configured joint TCI states per CC per BWP FFS: 2. Maximum number of activated joint TCI states across all CCs FFS: what to do/signal about additional support of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH 3. Support of a single default beam for PDSCH with scheduling offset less than a threshold; FFS; yes; n/a; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC is not supported; Per band; n/a; n/a; n/a; Note: FG 16-2b-0 can be used to indicate support of two default beams; Optional with capability signalling. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-1-1; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC; FFS: maximum number of configured joint TCI states per CC per BWP FFS: maximum number of activated joint TCI states across all CCs Comment: Maximum number in Rel-16/17 can be reused. FFS: what to do/signal about additional support of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH; FFS 23-10-1, at least one of {16-2b-1/2/3/4/5, 23-2-1, 23-3-1, 23-3-1-1/3, 23-3-2/3, 23-6-1/1b, 23-6-1-1, 23-6-2/2b}; yes; n/a; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC is not supported; Per band; n/a; n/a; n/a; Note: FG 16-2b-0 can be used to indicate support of two default beams; Optional with capability signalling. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-1-1; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC; FFS: maximum number of configured joint TCI states per CC per BWP FFS: maximum number of activated joint TCI states across all CCs FFS: what to do/signal about additional support of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH; FFS; yes; n/a; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC is not supported; Per band; n/a; n/a; n/a; Note: FG 16-2b-0 can be used to indicate support of two default beams; Optional with capability signalling. |
| OPPO [8] | 40. NR_MIMO_evo_DL_UL; 40-1-1; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC; FFS: separate rows intra-cell and inter-cell multi-TRP FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] FFS: maximum number of configured joint TCI states per CC per BWP FFS: maximum number of activated joint TCI states across all CCs FFS: what to do/signal about additional support of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH; FFS; yes; n/a; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC is not supported; Per band; n/a; n/a; n/a; Note: FG 16-2b-0 can be used to indicate support of two default beams; Optional with capability signalling. |
| CATT [9] | 40-1-1: - The maximum number of configured joint TCI states per CC per BWP has been specified in Rel-17 with candidate value {8, 12, 16, 24, 32, 48, 64, 128}. These values could be reused for Rel-18. - The maximum number of activated joint TCI states across all CCs has been specified in Rel-17 with candidate value {1, 2, 4, 8, 16}. These candidate values are specified for sTRP. Considering Rel-18 mTRP, more TCI states are needed. The recommended candidate values are {2, 4, 8, 16, 32}. - In MIMO session, it has been discussed that whether specification should restrict that two indicated joint/DL TCI states must be associated with different TRPs. In our opinion, gNB is responsible to indicate appropriate beam(s) for PDCCH and PDSCH/PUCCH/PUSCH. It is not necessary to introduce such restriction. Therefore, we don’t see the need of signaling about additional supportive of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH. The support of different TCI state for different channels should be a basic feature of Rel-18 UE. - The prerequisite of 40-1-1 should be 23-1-1 (unified TCI). |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-1-1; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC; FFS: maximum number of configured joint TCI states per CC per BWP FFS: maximum number of activated joint TCI states across all CCs FFS: what to do/signal about additional support of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH; FFS 23-1-1, 16-2b-1 or 16-2b-2 or 16-2b-3 or 16-2b-4 or 16-2b-5 or 23-2-1 or 23-3-1 or 23-3-2 or 23-3-3 or 23-6-1 or 23-6-2 or 23-6-1-1 or 23-6-1b or 23-6-2b; yes; n/a; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC is not supported; Per band; n/a; n/a; n/a; Note: FG 16-2b-0 can be used to indicate support of two default beams; Optional with capability signalling. |
| LG Electronics [11] | 40. NR_MIMO_evo_DL_UL; 40-1-1; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC; FFS: maximum number of configured joint TCI states per CC per BWP FFS: maximum number of activated joint TCI states across all CCs; FFS; yes; n/a; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC is not supported; Per band; n/a; n/a; n/a; Note: FG 16-2b-0 can be used to indicate support of two default beams; Optional with capability signalling. |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-1-1 - Components: - We think the first two FFS can be supported. We think “per CC per BWP” should be “per BWP per CC”. - Regarding to the last FFS, we are not ok with the current text. If the last FFS part is supported, when UE is indicated with two TCI states (TCI#1, TCI#2), gNB must use the same TCI between the scheduling DCI and the scheduled PDSCH. It looks M-TRP PDSCH can be only scheduled by SFN-PDCCH. Even for S-TRP PDSCH, we think DCI with TCI#1 schedules PDSCH with TCI#2 should be automatically supported if UE supports TCI state selection field. We think the motivation of the FFS is to ensure that one UE can assume that TCI state of the scheduling DCI can be the same as one of the TCI states of the scheduled PDSCH. We suggest updating the FFS to have the following separate two FGs. Or, another possible option is just remove the last FFS. - FG1 (optional): Support TCI state selection field. UE assumes TCI state of the scheduling PDCCH is the same as one of the TCI states of the scheduled PDSCH. - FG2 (optional): Support TCI state selection field. UE assumes TCI state of the scheduling PDCCH can be the same or different as one of the TCI states of the scheduled PDSCH. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-1-1; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC; - The maximum number of configured joint TCI state per BWP per CC in a band. - Two MAC-CE activated joint TCI-states per CC in a band. - MAC-CE based TCI-state indication for two active TCI-states. - The maximum number of MAC-CE activated joint TCI-states across all CC(s) in a band.; FFS; yes; n/a; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC is not supported; Per band; n/a; n/a; n/a; Component 1 candidate values {8, 12, 16, 24, 32, 48, 64, 128} Component 4 candidate value {2, 4, 8, 16, 32} Note: FG 16-2b-0 can be used to indicate support of two default beams; Optional with capability signalling. |
| Samsung [16] | 40. NR_MIMO_evo_DL_UL; 40-1-1; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC; FFS: maximum number of configured joint TCI states per CC per BWP FFS: maximum number of activated joint TCI states across all CCs FFS: what to do/signal about additional support of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH; Note: FG 16-2b-0 can be used to indicate support of two default beams; Optional with capability signalling. |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | Proposal 1-1: For FG 40-1-1, support the following changes on the FFS - Add the following items in the prerequisite field - 23-1-1, and at least one of 16-2b-1, 16-2b-2, 16-2b-3, 16-2b-4, 16-2b-5, 23-2-1, 23-3-1, 23-3-2, 23-3-3, 23-6-1, 23-6-2, 40-6-1, 40-6-1a, 40-6-2, 40-6-2a, 40-6-3a, 40-6-3b, 40-6-4 - Support the following FFS, with candidate value {8, 12, 16, 24, 32, 48, 64, 128} - FFS: maximum number of configured joint TCI states per CC per BWP - Support the following FFS, with candidate value {2, 4, 6, 8, 16} - FFS: maximum number of activated joint TCI states across all CCs 40. NR_MIMO_evo_DL_UL; 40-1-1; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC; FFS: maximum number of configured joint TCI states per CC per BWP FFS: maximum number of activated joint TCI states across all CCs FFS: what to do/signal about additional support of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH; FFS; yes; n/a; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC is not supported; Per band; n/a; n/a; n/a; Note: FG 16-2b-0 can be used to indicate support of two default beams; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-1-1c; DCI format 1_1 and if supported 1_2 configured with TCI selection field; Support of DCI format 1_1 and if supported 1_2 configured with TCI selection field; At least one of {40-1-1/1a/2/2a}; yes; n/a; DCI format 1_1 and if supported 1_2 configured with TCI selection field are not supported; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| Huawei/HiSilicon [19] | - For FG 40-1-1, support following components: - The maximum number of configured joint TCI states per BWP per CC - The maximum number of activated joint TCI states across all CC For FG 40-1-1, there is an FFS: what to do/signal about additional support of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH. In our view, allowing PDCCH from one TRP to schedule PDSCH/PUCCH/PUSCH from the other TRP is a basic feature in sDCI-based transmission schemes and should be supported along with FG 40-1-1. Therefore, we propose to include it as a component of FG 40-1-1. Proposal 2-2: Support the following component for FG 40-1-1: - Support of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH. 40. NR_MIMO_evo_DL_UL; 40-1-1; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC; FFS: maximum number of configured joint TCI states per CC per BWP FFS: maximum number of activated joint TCI states across all CCs FFS: what to do/signal about additional support of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH; FFS 23-1-1; yes; n/a; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC is not supported; Per band; n/a; n/a; n/a; Note: FG 16-2b-0 can be used to indicate support of two default beams; Optional with capability signalling. |

| 40. NR_MIMO_evo_DL_UL | 40-1-1a | Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC | FFS: maximum number of activated joint TCI states per CC | FFS | yes | n/a | Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC is not supported | Per band | n/a | n/a | n/a | Note: FG 16-2b-0 can be used to indicate support of two default beams | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-1-1a - FFS1: Candidate values for the maximum number of activated joint TCI states per CC { 2,3, 4,5,6,7, 8}. It is added in the notes 40. NR_MIMO_evo_DL_UL; 40-1-1a; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC; FFS: maximum number of activated joint TCI states per CC; FFS 40-1-1; yes; n/a; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC is not supported; Per band; n/a; n/a; n/a; Note: FG 16-2b-0 can be used to indicate support of two default beams Candidate values for the maximum number of activated joint TCI states per CC { 2,3, 4,5,6,7, 8}; Optional with capability signalling. |
| Ericsson [3] | Assuming that we follow the structure from Rel-17, this FG does should also apply to inter-cell operation, as this is an integral part of the unified TCI framework. “FFS: maximum number of activated joint TCI states per CC.” Following the Rel-17 solution, this would be a component in FG 40-1-1a. Following the Rel-17 solution, one additional component is needed in FG 40-1-1a: maximum number of activated joint TCI states across all CCs. FG 40-1-1 is a pre-requisite FG. Following Rel-17, and also FG 40-1-7a, description of the indication modes should be clarified. Also, there is a need to add the beam application time. 40-1-1a; Unified TCI with joint DL/UL TCI update for single-DCI based intra- and inter-cell multi-TRP with multiple activated TCI codepoints per CC; - TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2 without DL assignment) - The minimum beam application time in symbols maximum number of activated joint TCI states per CC maximum number of activated joint TCI states across all CCs; FG 40-1-1; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell and intra-cell multi-TRP with multiple activated TCI codepoints per CC is not supported; Per band; Note: FG 16-2b-0 can be used to indicate support of two default beams. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-1-1a; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC; FFS: Maximum number of activated joint TCI states per CC; FFS 40-1-1; yes; n/a; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC is not supported; Per band; n/a; n/a; n/a; Note: FG 16-2b-0 can be used to indicate support of two default beams; Optional with capability signalling. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-1-1a; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC; FFS: maximum number of activated joint TCI states per CC Comment: Maximum number in Rel-16/17 can be reused.; FFS 23-10-1b, 40-1-1; yes; n/a; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC is not supported; Per band; n/a; n/a; n/a; Note: FG 16-2b-0 can be used to indicate support of two default beams; Optional with capability signalling. |
| Intel Corporation [7] |  |
| OPPO [8] | 40. NR_MIMO_evo_DL_UL; 40-1-1a; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC; FFS: separate rows intra-cell and inter-cell multi-TRP FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] FFS: maximum number of activated joint TCI states across all CCs Maximum number of configured joint TCI states per BWP per CC in a band; FFS; yes; n/a; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC is not supported; Per band; n/a; n/a; n/a; Note: FG 16-2b-0 can be used to indicate support of two default beams; Optional with capability signalling. |
| CATT [9] | 40-1-1a: - The prerequisite of 40-1-1a should be 40-1-1. |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-1-1a; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC; FFS: maximum number of activated joint TCI states per CC; FFS 23-1-1b, 40-1-1; yes; n/a; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC is not supported; Per band; n/a; n/a; n/a; Note: FG 16-2b-0 can be used to indicate support of two default beams; Optional with capability signalling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-1-1a/40-1-2/40-1-2a - Components: - We think all the FFS can be supported. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-1-1a; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC; - TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) - The maximum number of MAC-CE activated joint TCI states per CC in a band. - The minimum beam application time in Y symbols per SCS; FFS; yes; n/a; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC is not supported; Per band; n/a; n/a; n/a; Component 2 candidate value {2, 4, 8, 10, 12, 14, 16} Component 3 candidate value {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336}, where {84, 98, 112, 224, 336} only can be indicated in FR2 Note: FG 16-2b-0 can be used to indicate support of two default beams; Optional with capability signalling. |
| Samsung [16] | 40. NR_MIMO_evo_DL_UL; 40-1-1a; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC; FFS: maximum number of activated joint TCI states per CC; Note: FG 16-2b-0 can be used to indicate support of two default beams; Optional with capability signalling. |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | Proposal 1-2: For FG 40-1-1a, support the following changes on the FFS - Add the following items in the prerequisite field - 40-1-1 - Support the following FFS, with candidate value {3, 4, 5, 6, 7, 8} - FFS: maximum number of activated joint TCI states per CC 40. NR_MIMO_evo_DL_UL; 40-1-1a; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC; FFS: maximum number of activated joint TCI states per CC; FFS; yes; n/a; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC is not supported; Per band; n/a; n/a; n/a; Note: FG 16-2b-0 can be used to indicate support of two default beams; Optional with capability signalling. |
| Huawei/HiSilicon [19] | - For FG 40-1-1a, support following components: - The maximum number of activated TCI state per BWP per CC - Note: The maximum number of configured joint TCI states per BWP per CC is not needed for this FG, as its prerequisite FG is FG 40-1-1. 40. NR_MIMO_evo_DL_UL; 40-1-1a; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC; FFS: maximum number of activated joint TCI states per CC; FFS 40-1-1; yes; n/a; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC is not supported; Per band; n/a; n/a; n/a; Note: FG 16-2b-0 can be used to indicate support of two default beams; Optional with capability signalling. |

| 40. NR_MIMO_evo_DL_UL | 40-1-1b | TRP-specific BFR with unified TCI framework with Unified TCI | Support for TRP-specific BFR with unified TCI framework with Unified TCI | FFS | yes | n/a |  | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-1-1b - The FG “TRP-specific BFR with unified TCI framework with Unified TCI” is a little confusing and should be modified to “TRP specific BFR with unified TCI framework for single-DCI based multi-TRP” 40. NR_MIMO_evo_DL_UL; 40-1-1b; TRP-specific BFR with unified TCI framework with Unified TCI TRP specific BFR with unified TCI framework for single-DCI based multi-TRP; Support for TRP-specific BFR with unified TCI framework with Unified TCI; FFS; yes; n/a; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| Ericsson [3] | There is no need for any pre-requisite FG. If the UE reports support for FG 40-1-1c, the UE would have to support mTRP BFR. 40-1-1b; TRP-specific BFR with unified TCI framework with Unified TCI; Support for TRP-specific BFR with unified TCI framework with Unified TCI; Per band. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-1-1b; TRP-specific BFR with unified TCI framework with Unified TCI for single-DCI based multi-TRP; Support for TRP-specific BFR with unified TCI framework with Unified TCI for single-DCI based multi-TRP; FFS 40-1-1; yes; n/a; TRP-specific BFR with unified TCI framework with Unified TCI for single-DCI based multi-TRP is not supported; Per band; n/a; n/a; n/a; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-1-7b; TRP-specific BFR with unified TCI framework with Unified TCI for multi-DCI based multi-TRP; Support for TRP-specific BFR with unified TCI framework with Unified TCI for multi-DCI based multi-TRP; 40-1-7; yes; n/a; TRP-specific BFR with unified TCI framework with Unified TCI for multi-DCI based multi-TRP is not supported; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-1-1bx; TRP-specific BFR with unified TCI framework with Unified TCI Comment: TRP-specific BFR can be applied to both single-DCI based multi-TRP and multi-DCI based multi-TRP. It is better to assign a FG number that is not under 40-1-1.; Support for TRP-specific BFR with unified TCI framework with Unified TCI; FFS 23-5-2, one of {40-1-1/1a, 40-1-2/2a, 40-1-7/7a, 40-1-9/9a}; yes; n/a; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-1-1b; TRP-specific BFR with unified TCI framework with Unified TCI; Support for TRP-specific BFR with unified TCI framework with Unified TCI; FFS 23-5-2, At least one of {40-1-1/1a/2/2a/7/7a/9/9a}; yes; n/a; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-1-1b - Prerequisite FG: - It should be FG23-5-2 (MTRP BFR based on two BFD-RS sets) and at least of one of FG40-1-1, FG40-1-2, FG40-1-7, or FG40-1-9. |
| Apple [15] |  |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | Proposal 1-3: For FG 40-1-1b, support the following changes on the FFS - Add the following items in the prerequisite field - Atl least one of 40-1-1, 40-1-7 |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-1-1b; TRP-specific BFR with unified TCI framework with Unified TCI; Support for TRP-specific BFR with unified TCI framework with Unified TCI; FFS 23-5-2, 40-1-1 or 40-1-7; yes; n/a; Per band; n/a; n/a; n/a; Optional with capability signalling. |

| 40. NR_MIMO_evo_DL_UL | 40-1-1c | DCI format 1_1 and if supported 1_2 configured with TCI selection field | Support of DCI format 1_1 and if supported 1_2 configured with TCI selection field | At least one of {40-1-1/1a/2/2a} | yes | n/a | DCI format 1_1 and if supported 1_2 configured with TCI selection field are not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] | Some edits to improve clarity. Only FG 40-1-1 is needed as pre-requisite. 40-1-1c; TCI selection field in DCI format 1_1 and if supported 1_2 configured; Support TCI selection field in DCI format 1_1 and if supported 1_2 configured; 40-1-1; TCI selection field in DCI format 1_1 and if supported 1_2 configured is not supported; Per band. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-1-1c; DCI format 1_1 and if supported 1_2 configured with TCI selection field; Support of DCI format 1_1 and if supported 1_2 configured with TCI selection field; At least one of {40-1-1/1a/2/2a}; yes; n/a; DCI format 1_1 and if supported 1_2 configured with TCI selection field are not supported; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-1-1c; DCI format 1_1 and if supported 1_2 configured with TCI selection field; Support of DCI format 1_1 and if supported 1_2 configured with TCI selection field; At least one of {40-1-1/1a/2/2a}; yes; n/a; DCI format 1_1 and if supported 1_2 configured with TCI selection field are not supported; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] |  |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] |  |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-1-2 | Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC | FFS: maximum number of configured DL TCI states per CC per BWP FFS: maximum number of configured UL TCI states per CC per BWP FFS: maximum number of activated DL TCI states across all CCs FFS: maximum number of activated UL TCI states across all CCs | FFS | yes | n/a | Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC is not supported | Per band | n/a | n/a | n/a | Note: FG 16-2b-0 can be used to indicate support of two default beams | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-1-2 - FFS1: maximum number of configured DL TCI states per CC per BWP. The candidate values of {4,8,12,16,24,32,48,64,128} are added in the notes - FFS2: maximum number of configured UL TCI states per CC per BWP. The candidate values of {4,8,12,16,24,32,48,64,128} are added in the notes - FFS3: maximum number of activated DL TCI states across all CCs. The candidate values of {1,2,4,8,16 } are added in the notes - FFS4: maximum number of activated UL TCI states across all CCs. The candidate values of {1,2,4,8,16 } are added in the notes 40. NR_MIMO_evo_DL_UL; 40-1-2; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC; FFS: maximum number of configured DL TCI states per CC per BWP FFS: maximum number of configured UL TCI states per CC per BWP FFS: maximum number of activated DL TCI states across all CCs FFS: maximum number of activated UL TCI states across all CCs; FFS 40-1-1; yes; n/a; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC is not supported; Per band; n/a; n/a; n/a; Note: FG 16-2b-0 can be used to indicate support of two default beams Candidate values for the maximum number of activated DL TCI states per CC per BWP {4,8,12,16,24,32,48,64,128} Candidate values for the maximum number of activated UL TCI states per CC per BWP {4,8,12,16,24,32,48,64,128} Candidate values for the maximum number of activated DL TCI states across all CC {1,2,4,8,16 } Candidate values for the maximum number of activated UL TCI states across all CC{1,2,4,8,16}; Optional with capability signalling. |
| Ericsson [3] | The input on FG 40-1-2 essentially mirrors the input on FG 40-1-1. Note that FG 40-1-1 will be a pre-requisite for FG 40-1-2. “FFS: maximum number of configured DL TCI states per CC per BWP”. Following the Rel-17 solution, this should be a component in FG 40-1-2. “FFS: maximum number of configured UL TCI states per CC per BWP”. Following the Rel-17 solution, this should be a component in FG 40-1-2. “FFS: maximum number of activated DL TCI states across all CCs”. Following the Rel-17 solution, this should be a component in FG 40-1-2. “FFS: maximum number of activated UL TCI states across all CCs”. Following the Rel-17 solution, this should be a component in FG 40-1-2. 40-1-2; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC; maximum number of configured DL TCI states per CC per BWP maximum number of configured UL TCI states per CC per BWP maximum number of activated DL TCI states across all CCs maximum number of activated UL TCI states across all CCs; 40-1-1; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC is not supported; Per band; Note: FG 16-2b-0 can be used to indicate support of two default beams. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-1-2; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC; FFS: 1. Maximum number of configured DL TCI states per CC per BWP FFS: 2. Maximum number of configured UL TCI states per CC per BWP FFS: 3. Maximum number of activated DL TCI states across all CCs FFS: 4. Maximum number of activated UL TCI states across all CCs; FFS 40-1-1; yes; n/a; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC is not supported; Per band; n/a; n/a; n/a; Note: FG 16-2b-0 can be used to indicate support of two default beams; Optional with capability signalling. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-1-2; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC; FFS: maximum number of configured DL TCI states per CC per BWP FFS: maximum number of configured UL TCI states per CC per BWP FFS: maximum number of activated DL TCI states across all CCs FFS: maximum number of activated UL TCI states across all CCs Comment: Maximum number in Rel-16/17 can be reused.; FFS 23-10-1, at least one of {16-2b-1/2/3/4/5, 23-2-1, 23-3-1, 23-3-1-1/3, 23-3-2/3, 23-6-1/1b, 23-6-1-1, 23-6-2/2b}; yes; n/a; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC is not supported; Per band; n/a; n/a; n/a; Note: FG 16-2b-0 can be used to indicate support of two default beams; Optional with capability signalling. |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-1-2: - With single activated TCI codepoint, for Rel-18 mTRP, the candidate values for the maximum number of configured DL or UL TCI states per CC per BWP could be the same as Rel-17, i.e. 128 and 64 respectively. - With single activated TCI codepoint, for Rel-18 mTRP, the candidate values for the maximum number of activated DL or UL TCI states across all CCs could be {2,4,8,16,32}. - The prerequisite of 40-1-2 should be 40-1-1. |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-1-2; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC; FFS: maximum number of configured DL TCI states per CC per BWP FFS: maximum number of configured UL TCI states per CC per BWP FFS: maximum number of activated DL TCI states across all CCs FFS: maximum number of activated UL TCI states across all CCs; FFS 23-10-1, 16-2b-1 or 16-2b-2 or 16-2b-3 or 16-2b-4 or 16-2b-5 or 23-2-1 or 23-3-1 or 23-3-2 or 23-3-3 or 23-6-1 or 23-6-2 or 23-6-1-1 or 23-6-1b or 23-6-2b; yes; n/a; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC is not supported; Per band; n/a; n/a; n/a; Note: FG 16-2b-0 can be used to indicate support of two default beams; Optional with capability signalling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-1-1a/40-1-2/40-1-2a - Components: - We think all the FFS can be supported. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-1-2; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC; - The maximum number of configured separate DL TCI state per BWP per CC in a band. - The maximum number of configured separate UL TCI state per BWP per CC in a band. - Two MAC-CE activated DL TCI-states per CC in a band. - Two MAC-CE activated UL TCI-states per CC in a band. - MAC-CE based TCI-state indication. - The maximum number of MAC-CE activated DL TCI states across all CC(s) in a band. - The maximum number of MAC-CE activated UL TCI states across all CC(s) in a band.; FFS; yes; n/a; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC is not supported; Per band; n/a; n/a; n/a; Component 1 candidate values {8, 12, 16, 24, 32, 48, 64, 128} Component 2 candidate values {8, 12, 16, 24, 32, 48, 64, 128} Component 6 candidate value {2, 4, 8, 16, 32} Component 7 candidate value {2, 4, 8, 16, 32} Note: FG 16-2b-0 can be used to indicate support of two default beams; Optional with capability signalling. |
| Samsung [16] | 40. NR_MIMO_evo_DL_UL; 40-1-2; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC; FFS: maximum number of configured DL TCI states per CC per BWP FFS: maximum number of configured UL TCI states per CC per BWP FFS: maximum number of activated DL TCI states across all CCs FFS: maximum number of activated UL TCI states across all CCs; Note: FG 16-2b-0 can be used to indicate support of two default beams; Optional with capability signalling. |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | Proposal 1-4: For FG 40-1-2, support the following changes on the FFS - Add the following items in the prerequisite field - 23-10-1 - Support the following FFS, with candidate value {4, 8, 12, 16, 24, 32, 48, 64, 128} - FFS: maximum number of activated joint TCI states per CC - Support the following FFS, with candidate value {4, 8, 12, 16, 24, 32, 48, 64} - FFS: maximum number of configured UL TCI states per CC per BWP - Support the following FFS, with candidate value {1, 2, 4, 8, 16} - FFS: maximum number of activated DL TCI states across all CCs - Support the following FFS, with candidate value {1, 2, 4, 8, 16} - FFS: maximum number of activated UL TCI states across all CCs 40. NR_MIMO_evo_DL_UL; 40-1-2; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC; FFS: maximum number of configured DL TCI states per CC per BWP FFS: maximum number of configured UL TCI states per CC per BWP FFS: maximum number of activated DL TCI states across all CCs FFS: maximum number of activated UL TCI states across all CCs; FFS; yes; n/a; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC is not supported; Per band; n/a; n/a; n/a; Note: FG 16-2b-0 can be used to indicate support of two default beams; Optional with capability signalling. |
| Huawei/HiSilicon [19] | - For FG 40-1-2, support following components: - The maximum number of configured DL TCI states per BWP per CC - The maximum number of configured UL TCI states per BWP per CC - The maximum number of activated DL TCI states cross all CC - The maximum number of activated UL TCI states cross all CC 40. NR_MIMO_evo_DL_UL; 40-1-2; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC; FFS: maximum number of configured DL TCI states per CC per BWP FFS: maximum number of configured UL TCI states per CC per BWP FFS: maximum number of activated DL TCI states across all CCs FFS: maximum number of activated UL TCI states across all CCs; FFS 40-1-1 23-10-1; yes; n/a; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC is not supported; Per band; n/a; n/a; n/a; Note: FG 16-2b-0 can be used to indicate support of two default beams; Optional with capability signalling. |

| 40. NR_MIMO_evo_DL_UL | 40-1-2a | Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC | FFS: maximum number of activated DL TCI states across all CCs FFS: maximum number of activated UL TCI states across all CCs | FFS | yes | n/a | Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC is not supported | Per band | n/a | n/a | n/a | Note: FG 16-2b-0 can be used to indicate support of two default beams | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-1-2a - FFS1: maximum number of activated DL TCI states across all CCs. The candidate values of {1,2,4,8,16 } are added in the notes - FFS2: maximum number of activated UL TCI states across all CCs. The candidate values of {1,2,4,8,16 } are added in the notes 40. NR_MIMO_evo_DL_UL; 40-1-2a; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC; FFS: maximum number of activated DL TCI states across all CCs FFS: maximum number of activated UL TCI states across all CCs; FFS 40-1-2; yes; n/a; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC is not supported; Per band; n/a; n/a; n/a; Note: FG 16-2b-0 can be used to indicate support of two default beams Candidate values for maximum number of activated DL TCI states across all CCs {1,2,4,8,16 } Candidate values for maximum number of activated UL TCI states across all CCs { 1,2,4,8,16}; Optional with capability signalling. |
| Ericsson [3] | Assuming that we follow the structure from Rel-17, this FG does should also apply to inter-cell operation, as this is an integral part of the unified TCI framework. “FFS: maximum number of activated DL TCI states across all CCs”: Following the Rel-17 solution, this should be a component in FG 40-1-2a “FFS: maximum number of activated UL TCI states across all CCs”: Following the Rel-17 solution, this should be a component in FG 40-1-2a Following the Rel-17 solution, one additional component is needed in FG 40-1-2a: maximum number of activated DL TCI states per CC Following the Rel-17 solution, one additional component is needed in FG 40-1-2a: maximum number of activated UL TCI states per CC Following Rel-17, and also FG 40-1-7a, description of the indication modes should be clarified. Also, there is a need to add the beam application time. 40-1-2a; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell and inter-cell multi-TRP with multiple activated TCI codepoints per CC; - TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2 without DL assignment) - The minimum beam application time in symbols maximum number of activated DL TCI states across all CCs maximum number of activated DL TCI states per CC maximum number of activated UL TCI states across all CCs maximum number of activated UL TCI states per CC; 40-1-2; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell and inter-cell multi-TRP with multiple activated TCI codepoints per CC is not supported; Per band; Note: FG 16-2b-0 can be used to indicate support of two default beams. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-1-2a; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC; FFS: 1. Maximum number of activated DL TCI states per CC across all CCs FFS: 2. Maximum number of activated UL TCI states per CC across all CCs; FFS 40-2-1; yes; n/a; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC is not supported; Per band; n/a; n/a; n/a; Note: FG 16-2b-0 can be used to indicate support of two default beams; Optional with capability signalling. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-1-2a; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC; FFS: maximum number of activated DL TCI states across all CCs FFS: maximum number of activated UL TCI states across all CCs Comment: Maximum number in Rel-16/17 can be reused.; FFS 23-10-1b, 40-1-2; yes; n/a; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC is not supported; Per band; n/a; n/a; n/a; Note: FG 16-2b-0 can be used to indicate support of two default beams; Optional with capability signalling. |
| Intel Corporation [7] |  |
| OPPO [8] | 40. NR_MIMO_evo_DL_UL; 40-1-2a; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC; FFS: separate rows intra-cell and inter-cell multi-TRP FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] FFS: maximum number of activated DL TCI states per CCacross all CCs FFS: maximum number of activated UL TCI states per CCacross all CCs Maximum number of configured DL TCI states per BWP per CC in a band Maximum number of configured UL TCI states per BWP per CC in a band; FFS; yes; n/a; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC is not supported; Per band; n/a; n/a; n/a; Note: FG 16-2b-0 can be used to indicate support of two default beams; Optional with capability signalling. |
| CATT [9] | 40-1-2a: - The prerequisite of 40-1-2a should be 40-1-2. |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-1-2a; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC; FFS: maximum number of activated DL TCI states across all CCs FFS: maximum number of activated UL TCI states across all CCs; FFS 23-10-1b, 40-1-2; yes; n/a; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC is not supported; Per band; n/a; n/a; n/a; Note: FG 16-2b-0 can be used to indicate support of two default beams; Optional with capability signalling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-1-1a/40-1-2/40-1-2a - Components: - We think all the FFS can be supported. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-1-2a; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC; - TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) - The maximum number of MAC-CE activated DL TCI states per CC in a band. - The maximum number of MAC-CE activated UL TCI states per CC in a band. - The minimum beam application time in Y symbols per SCS; FFS; yes; n/a; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC is not supported; Per band; n/a; n/a; n/a; Component 2 candidate value {2, 4, 8, 10, 12, 14, 16} Component 3 candidate value {2, 4, 8, 10, 12, 14, 16} Component 4 candidate value {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336}, where {84, 98, 112, 224, 336} only can be indicated in FR2. Note: FG 16-2b-0 can be used to indicate support of two default beams; Optional with capability signalling. |
| Samsung [16] | 40. NR_MIMO_evo_DL_UL; 40-1-2a; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC; FFS: maximum number of activated DL TCI states across all CCs FFS: maximum number of activated UL TCI states across all CCs; Note: FG 16-2b-0 can be used to indicate support of two default beams; Optional with capability signalling. |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | Proposal 1-5: For FG 40-1-2a, support the following changes on the FFS - Add the following items in the prerequisite field - 40-1-2 - Support the following FFS with this change, with candidate value {1, 2, 3, 4, 5, 6, 7, 8} - FFS: maximum number of activated DL TCI states per CC - Support the following FFS with this change, with candidate value {1, 2, 3, 4, 5, 6, 7, 8} - FFS: maximum number of activated UL TCI states per CC 40. NR_MIMO_evo_DL_UL; 40-1-2a; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC; FFS: maximum number of activated DL TCI states across all CCs FFS: maximum number of activated UL TCI states across all CCs; FFS; yes; n/a; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC is not supported; Per band; n/a; n/a; n/a; Note: FG 16-2b-0 can be used to indicate support of two default beams; Optional with capability signalling. |
| Huawei/HiSilicon [19] | - For FG 40-1-2a, support following components: - The maximum number of activated DL TCI state per BWP per CC - The maximum number of activated UL TCI state per BWP per CC - Note: The maximum number of configured DL/UL TCI states per BWP per CC is not needed for this FG, as its prerequisite FG is FG 40-1-2. 40. NR_MIMO_evo_DL_UL; 40-1-2a; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC; FFS: maximum number of activated DL TCI states across all CCs FFS: maximum number of activated UL TCI states across all CCs; FFS 40-1-2; yes; n/a; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC is not supported; Per band; n/a; n/a; n/a; Note: FG 16-2b-0 can be used to indicate support of two default beams; Optional with capability signalling. |

| 40. NR_MIMO_evo_DL_UL | 40-1-3 | Per aperiodic CSI-RS resource/resource set configuration for TCI selection in S-DCI based MTRP | Support of per aperiodic CSI-RS resource/resource set configuration for TCI selection in S-DCI based MTRP |  | yes | n/a |  | Per band | n/a | n/a | n/a | Component candidate values: {per resource, per resource set, both} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] |  |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-1-3; Per aperiodic CSI-RS resource/resource set configuration for TCI selection in S-DCI based MTRP; Support of per aperiodic CSI-RS resource/resource set configuration for TCI selection in S-DCI based MTRP; 40-1-1; yes; n/a; Per aperiodic CSI-RS resource/resource set configuration for TCI selection in S-DCI based MTRP is not supported; Per band; n/a; n/a; n/a; Component candidate values: {per resource, per resource set, both}; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-1-3a; Per aperiodic CSI-RS resource/resource set configuration for TCI selection in M-DCI based MTRP; Support of per aperiodic CSI-RS resource/resource set configuration for TCI selection in M-DCI based MTRP; 40-1-7; yes; n/a; Per aperiodic CSI-RS resource/resource set configuration for TCI selection in M-DCI based MTRP is not supported; Per band; n/a; n/a; n/a; Component candidate values: {per resource, per resource set, both}; Optional with capability signalling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-1-3; Per aperiodic CSI-RS resource/resource set configuration for TCI selection in S-DCI based MTRP; Support of per aperiodic CSI-RS resource/resource set configuration for TCI selection in S-DCI based MTRP; At least one of {40-1-1/1a/2/2a/7/7a/9/9a}; yes; n/a; Per band; n/a; n/a; n/a; Component candidate values: {per resource, per resource set, both}; Optional with capability signalling. |
| LG Electronics [11] | 40-1-3 - In addition to the case for S-DCI based MTRP, separate FG for M-DCI based MTRP is needed according to the following agreement in RAN1#113. Agreement On unified TCI framework extension for M-DCI based MTRP, An RRC configuration can be provided to an aperiodic CSI-RS resource set or a CSI-RS resource in an aperiodic CSI-RS resource set to inform that the UE shall apply the first or the second indicated joint/DL TCI state to the aperiodic CSI-RS resource set or to the CSI-RS resource in the aperiodic CSI-RS resource set, if the aperiodic CSI-RS resource set for CSI/BM is configured to follow unified TCI state - The first and the second indicated joint/DL TCI states correspond to the indicated joint/UDL TCI states specific to coresetPoolIndex value 0 and value 1, respectively. - Above applies at least if the offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in the aperiodic CSI-RS resource set is equal to or larger than a threshold (if the threshold is needed) - Support of ‘per CSI-RS resource set’ or ‘per CSI-RS resource’ RRC configuration is up to UE capability. Proposal 2: Add an FG (i.e. 40-1-3a) on per AP CSI-RS resource/resource set configuration for TCI selection in M-DCI based MTRP 40. NR_MIMO_evo_DL_UL; 40-1-3; Per aperiodic CSI-RS resource/resource set configuration for TCI selection in S-DCI based MTRP; Per aperiodic CSI-RS resource/resource set configuration for TCI selection in S-DCI based MTRP; yes; n/a; Per band; n/a; n/a; n/a; Component candidate values: {per resource, per resource set, both}; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-1-3a; Per aperiodic CSI-RS resource/resource set configuration for TCI selection in M-DCI based MTRP; Per aperiodic CSI-RS resource/resource set configuration for TCI selection in M-DCI based MTRP; yes; n/a; Per band; n/a; n/a; n/a; Component candidate values: {per resource, per resource set, both}; Optional with capability signalling. |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] |  |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] |  |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-1-4 | Two TCI states for CJT Tx scheme for PDSCH | Support of two TCI states for CJT Tx scheme for PDSCH [FFS: for CSI-RS] |  | yes | n/a | Two TCI states for CJT Tx scheme for PDSCH are not supported | Per band | n/a | n/a | n/a | Component candidate values: {CJT Scheme-A, CJT scheme-B, both} CJT Scheme-A: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint/DL TCI states with respect to QCL-TypeA CJT Scheme-B: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint/DL TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint/DL TCI state | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-1-4 - Support of two TCI states for CJT Tx scheme for PDSCH [FFS: for CSI-RS]: we recommend removing “for CSI RS” 40. NR_MIMO_evo_DL_UL; 40-1-4; Two TCI states for CJT Tx scheme for PDSCH; Support of two TCI states for CJT Tx scheme for PDSCH [FFS: for CSI-RS]; yes; n/a; Two TCI states for CJT Tx scheme for PDSCH are not supported; Per band; n/a; n/a; n/a; Component candidate values: {CJT Scheme-A, CJT scheme-B, both} CJT Scheme-A: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint/DL TCI states with respect to QCL-TypeA CJT Scheme-B: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint/DL TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint/DL TCI state; Optional with capability signaling. |
| Ericsson [3] | The FFS can be removed: the FG is only related to PDSCH, and it is unclear what “FFS: for CSI-RS” means. CSI-RS cannot be configured with two TCI states. 40-1-4; Two TCI states for CJT Tx scheme for PDSCH; Support of two TCI states for CJT Tx scheme for PDSCH [; Two TCI states for CJT Tx scheme for PDSCH are not supported; Per band; Component candidate values: {CJT Scheme-A, CJT scheme-B, both}. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-1-4; Two TCI states for CJT Tx scheme for PDSCH; Support of two TCI states for CJT Tx scheme for PDSCH [FFS: for CSI-RS]; yes; n/a; Two TCI states for CJT Tx scheme for PDSCH are not supported; Per band; n/a; n/a; n/a; Component candidate values: {CJT Scheme-A, CJT scheme-B, both} CJT Scheme-A: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint/DL TCI states with respect to QCL-TypeA CJT Scheme-B: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint/DL TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint/DL TCI state; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-1-4; Two TCI states for CJT Tx scheme for PDSCH; Support of two TCI states for CJT Tx scheme for PDSCH [FFS: for CSI-RS] Comment: There is no restriction for QCL configuration of CSI-RS.; 40-1-1, 40-1-1a; yes; n/a; Two TCI states for CJT Tx scheme for PDSCH are not supported; Per band; n/a; n/a; n/a; Component candidate values: {CJT Scheme-A, CJT scheme-B, both} CJT Scheme-A: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint/DL TCI states with respect to QCL-TypeA CJT Scheme-B: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint/DL TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint/DL TCI state; Optional with capability signaling. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-1-4; Two TCI states for CJT Tx scheme for PDSCH; Support of two TCI states for CJT Tx scheme for PDSCH [FFS: for CSI-RS]; yes; n/a; Two TCI states for CJT Tx scheme for PDSCH are not supported; Per band; n/a; n/a; n/a; Component candidate values: {CJT Scheme-A, CJT scheme-B, both} CJT Scheme-A: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint/DL TCI states with respect to QCL-TypeA CJT Scheme-B: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint/DL TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint/DL TCI state; Optional with capability signaling. |
| OPPO [8] | 40. NR_MIMO_evo_DL_UL; 40-1-4; Support of Two TCI states for CJT Tx scheme for PDSCH; FFS: how to signal QCL subset Support of two TCI states for CJT Tx scheme for PDSCH [FFS: for CSI-RS]; yes; n/a; Two TCI states for CJT Tx scheme for PDSCH are not supported; Per band; n/a; n/a; n/a; Component candidate values: {CJT Scheme-A, CJT scheme-B, both} CJT Scheme-A: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint/DL TCI states with respect to QCL-TypeA CJT Scheme-B: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint/DL TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint/DL TCI state; Optional with capability signaling. |
| CATT [9] | 40-1-4: - 40-1-4 is to support two TCI states for CJT Tx scheme for PDSCH. With FG 40-1-4, NW would correspondingly configure more than one TCI states for the CSI-RS as CMR in a CSI-ReportConfig. Therefore, it is not necessary to introduce additional FG for CSI-RS for CJT. |
| Xiaomi [10] | As for the FG 40-1-4, only two TCI states for CJT Tx scheme for PDSCH is supported, two TCI states for CJT Tx scheme for CSI-RS is not supported. Proposal 2-1: For the FG 40-1-4, remove ‘[FFS: for CSI-RS]’ in the components. 40. NR_MIMO_evo_DL_UL; 40-1-4; Two TCI states for CJT Tx scheme for PDSCH; Support of two TCI states for CJT Tx scheme for PDSCH [FFS: for CSI-RS]; 40-3-1-1; yes; n/a; Two TCI states for CJT Tx scheme for PDSCH are not supported; Per band; n/a; n/a; n/a; Component candidate values: {CJT Scheme-A, CJT scheme-B, both} CJT Scheme-A: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint/DL TCI states with respect to QCL-TypeA CJT Scheme-B: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint/DL TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint/DL TCI state; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] | For FG 40-1-4, there is one open issue on whether to include the CSI-RS for the feature of supporting two TCI states for CJT scheme. Since the UE behavior to receive CSI-RS and PDSCH should be the same, it is necessary to include the CSI-RS to reduce the UE complexity. Proposal 2: Support to include CSI-RS for FG 40-1-4 as a separate feature group 40-1-4a, similar to PDSCH for CJT. 40. NR_MIMO_evo_DL_UL; 40-1-4; Support of Two TCI states for CJT Tx scheme for PDSCH; FFS: how to signal QCL subset Support of two TCI states for CJT Tx scheme for PDSCH; yes; n/a; Two TCI states for CJT Tx scheme for PDSCH are not supported; Per band; n/a; n/a; n/a; Component candidate values: {CJT Scheme-A, CJT scheme-B, both} CJT Scheme-A: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint/DL TCI states with respect to QCL-TypeA CJT Scheme-B: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint/DL TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint/DL TCI state; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-1-4-1; Support of more than one TCI states across CSI-RS resources configured as CMR in a CSI-ReportConfig for CJT CSI report; Support of more than one TCI states across CSI-RS resources configured as CMR in a CSI-ReportConfig for CJT CSI report; yes; n/a; More than one TCI states for CSI-RS configured as CMR in a CSI-ReportConfig for CJT CSI report are not supported; Per band; n/a; n/a; Optional with capability signaling. |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] |  |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | Proposal 1-6: For FG 40-1-4, support the following changes on the FFS - Do not support the following FFS, which is not discussed in 8.1.1.1 - FFS: for CSI-RS 40. NR_MIMO_evo_DL_UL; 40-1-4; Two TCI states for CJT Tx scheme for PDSCH; Support of two TCI states for CJT Tx scheme for PDSCH [FFS: for CSI-RS]; yes; n/a; Two TCI states for CJT Tx scheme for PDSCH are not supported; Per band; n/a; n/a; n/a; Component candidate values: {CJT Scheme-A, CJT scheme-B, both} CJT Scheme-A: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint/DL TCI states with respect to QCL-TypeA CJT Scheme-B: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint/DL TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint/DL TCI state; Optional with capability signaling. |
| Huawei/HiSilicon [19] | For FG 40-1-4, there is an FFS: for CSI-RS. This FFS is not clear enough and we failed to see any necessity to discuss supporting two TCI-state for CSI-RS as each CSI-RS is transmitted with one TCI-state. So, we propose to remove this FFS. Proposal 2-3: For FG 40-4-1, remove the “FFS: for CSI-RS”. 40. NR_MIMO_evo_DL_UL; 40-1-4; Two TCI states for CJT Tx scheme for PDSCH; Support of two TCI states for CJT Tx scheme for PDSCH [FFS: for CSI-RS]; yes; n/a; Two TCI states for CJT Tx scheme for PDSCH are not supported; Per band; n/a; n/a; n/a; Component candidate values: {CJT Scheme-A, CJT scheme-B, both} CJT Scheme-A: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint/DL TCI states with respect to QCL-TypeA CJT Scheme-B: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint/DL TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint/DL TCI state; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-1-7 | Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC | 1. Support of [intra-cell] mTRP operation for M-DCI [2. Support of a single default beam for PDSCH with scheduling offset less than a threshold unless UE indicates the support of two default beams] 3. Maximum number of configured joint TCI states per BWP per CC 4. Maximum number of activated joint TCI states across all CCs per ‘coresetPoolIndex’ value 5. One MAC-CE activates one joint TCI-states per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value | [16-2a, 23-4, 40-6-1, 40-6-2, 40-6-3a, 40-6-3b, 40-6-4] | yes | n/a | Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC is not supported | [Per band] | n/a | n/a | n/a | Component 3 candidate values: {8, 12, 16, 24, 32, 48, 64, 128} Component 4 candidate values: {2, 4, 8, 16} Note: activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH transmissions | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-1-7; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC; 1. Support of [intra-cell] mTRP operation for M-DCI [2. Support of a single default beam for PDSCH with scheduling offset less than a threshold unless UE indicates the support of two default beams] 3. Maximum number of configured joint TCI states per BWP per CC 4. Maximum number of activated joint TCI states across all CCs per ‘coresetPoolIndex’ value 5. One MAC-CE activates one joint TCI-states per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value; [16-2a, 23-4, 40-6-1, 40-6-2, 40-6-3a, 40-6-3b, 40-6-4]; yes; n/a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC is not supported; [Per band]; n/a; n/a; n/a; Component 3 candidate values: {8, 12, 16, 24, 32, 48, 64, 128} Component 4 candidate values: {2, 4, 8, 16} Note: activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH transmissions; Optional with capability signaling. |
| Ericsson [3] | This feature group should only describe multi-DCI based intra-cell mTRP operation. Inter-cell operation should be a separate FG, added as FG 40-1-7d. “[2. Support of a single default beam for PDSCH with scheduling offset less than a threshold unless UE indicates the support of two default beams]”. This is not needed. The FG should be signalled per band, in line with RAN2 recommendations. 40-1-7; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC; 1. Support of intra-cell mTRP operation for M-DCI 3. Maximum number of configured joint TCI states per BWP per CC 4. Maximum number of activated joint TCI states across all CCs per ‘coresetPoolIndex’ value 5. One MAC-CE activates one joint TCI-states per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC is not supported; Per band; Component 3 candidate values: {8, 12, 16, 24, 32, 48, 64, 128} Component 4 candidate values: {2, 4, 8, 16} Note: activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH transmissions. |
| ZTE [4] | - FG 40-1-7 should be assumed as a basic FG for multi-DCI based multi-TRP operation in terms of unified TCI extension. Then, regarding support of a single default beam for PDSCH with scheduling offset less than a threshold, and then we are okay not to mention that as in the component. However, as what we did for 40-1-1, we need to add one more note for clarifying that ‘Note: FG 16-2a-6 can be used to indicate support of two default beams’. 40. NR_MIMO_evo_DL_UL; 40-1-7; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC; 1. Support of [intra-cell] mTRP operation for M-DCI [2. Support of a single default beam for PDSCH with scheduling offset less than a threshold unless UE indicates the support of two default beams] 3. Maximum number of configured joint TCI states per BWP per CC 4. Maximum number of activated joint TCI states across all CCs per ‘coresetPoolIndex’ value 5. One MAC-CE activates one joint TCI-states per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value; [16-2a, 23-4, 40-6-1, 40-6-2, 40-6-3a, 40-6-3b, 40-6-4]; yes; n/a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC is not supported; [Per band]; n/a; n/a; n/a; Component 3 candidate values: {8, 12, 16, 24, 32, 48, 64, 128} Component 4 candidate values: {2, 4, 8, 16} Note: activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH transmissions Note: FG 16-2a-6 can be used to indicate support of two default beams; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-1-7; Unified TCI with joint DL/UL TCI update for multi-DCI based intra-cell multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC; 1. Support of [intra-cell] mTRP operation for M-DCI [2. Support of a single default beam for PDSCH with scheduling offset less than a threshold unless UE indicates the support of two default beams] 3. Maximum number of configured joint TCI states per BWP per CC 4. Maximum number of activated joint TCI states across all CCs per ‘coresetPoolIndex’ value 5. One MAC-CE activates one joint TCI-states per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value; 23-10-1, [16-2a, 23-4, 40-6-1, 40-6-2, 40-6-3a, 40-6-3b, 40-6-4]; yes; n/a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC is not supported; [Per band]; n/a; n/a; n/a; Component 3 candidate values: {8, 12, 16, 24, 32, 48, 64, 128} Component 4 candidate values: {2, 4, 8, 16} Note: activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH transmissions; Optional with capability signaling. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-1-7; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC; 1. Support of [intra-cell and inter-cell] mTRP operation for M-DCI [2. Support of a single default beam for PDSCH with scheduling offset less than a threshold unless UE indicates the support of two default beams] 3. Maximum number of configured joint TCI states per BWP per CC 4. Maximum number of activated joint TCI states across all CCs per ‘coresetPoolIndex’ value 5. One MAC-CE activates one joint TCI-states per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value; [16-2a, 23-4, 40-6-1, 40-6-2, 40-6-3a, 40-6-3b, 40-6-4]; yes; n/a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC is not supported; [Per band]; n/a; n/a; n/a; Component 3 candidate values: {8, 12, 16, 24, 32, 48, 64, 128} Component 4 candidate values: {2, 4, 8, 16} Note: activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH transmissions; Optional with capability signaling. |
| OPPO [8] | 40. NR_MIMO_evo_DL_UL; 40-1-7; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per BWP per CC; 1. Support of [intra-cell] mTRP operation for M-DCI with joint TCI state [2. Support of a single default beam for PDSCH with scheduling offset less than a threshold unless UE indicates the support of two default beams] 3. Maximum number of configured joint TCI states per CORESETPoolIndex per BWP per CC 4. Maximum number of activated joint TCI states across all CCs per ‘coresetPoolIndex’ value 5. One MAC-CE activates one joint TCI-states per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value; [16-2a, 23-4, 40-6-1, 40-6-2, 40-6-3a, 40-6-3b, 40-6-4]; yes; n/a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC is not supported; [Per band]; n/a; n/a; n/a; Component 3 candidate values: {8, 12, 16, 24, 32, 48, 64, 128} Component 4 candidate values: {2, 4, 8, 16} Note: activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH transmissions; Optional with capability signaling. |
| CATT [9] | 40-1-7: - According to the agreement of RAN1#114bis, if the UE doesn’t support the capability of default beam per coresetPoolIndex for M-DCI based MTRP in FR2, the UE shall apply the indicated joint/DL TCI state specific to coresetPoolIndex value 0 to the scheduled PDSCH reception. In our opinion, the supportive of single default beam is UE behavior instead of UE capability, which should not be defined as a component of FG 40-1-7. - The prerequisite of 40-1-7 should be 23-1-1 (unified TCI). |
| Xiaomi [10] | As for FG 40-1-7, M-DCI based MTRP can be used for both intra-cell and inter-cell, thus we propose to remove ‘intra-cell’ in the components. In addition, the FG for S-DCI based MTRP should be removed from the Prerequisite feature groups. Proposal 2-2: For the FG 40-1-7, remove ‘[intra-cell]’ in the components. 40. NR_MIMO_evo_DL_UL; 40-1-7; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC; 1. Support of [intra-cell] mTRP operation for M-DCI [2. Support of a single default beam for PDSCH with scheduling offset less than a threshold unless UE indicates the support of two default beams] 3. Maximum number of configured joint TCI states per BWP per CC 4. Maximum number of activated joint TCI states across all CCs per ‘coresetPoolIndex’ value 5. One MAC-CE activates one joint TCI-states per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value; 23-1-1, [16-2a, 23-4, 40-6-1, 40-6-2, 40-6-3a, 40-6-3b, 40-6-4]; yes; n/a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC is not supported; [Per band]; n/a; n/a; n/a; Component 3 candidate values: {8, 12, 16, 24, 32, 48, 64, 128} Component 4 candidate values: {2, 4, 8, 16} Note: activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH transmissions; Optional with capability signaling. |
| LG Electronics [11] | 40-1-7 - According to the following agreement in RAN1#114bis, it is prerequisite to support the capability of default beam per coresetPoolIndex for two default beams. Agreement On unified TCI framework extension for M-DCI based MTRP, if the scheduling offset between the last symbol of the PDCCH carrying a scheduling DCI and the first symbol of the scheduled PDSCH is smaller than a threshold: - If the UE doesn’t support the capability of default beam per coresetPoolIndex for M-DCI based MTRP in FR2: - The UE shall apply the indicated joint/DL TCI state specific to coresetPoolIndex value 0 to the scheduled PDSCH reception - The UE doesn’t expect to be scheduled with PDSCH with scheduling offset less than a threshold of the PDSCH if scheduled by a CORESET associated with coresetPoolIndex value 1 - Note: If the UE supports the capability of default beam per coresetPoolIndex for M-DCI based MTRP in FR2, UE can use both indicated joint/DL TCI states to buffer the received signal before a threshold.. Proposal 3: Modify the second component as: Support of default beam per coresetPoolIndex. 40. NR_MIMO_evo_DL_UL; 40-1-7; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC; 1. Support of [intra-cell] mTRP operation for M-DCI 2. Support of default beam per coresetPoolIndex 3. Maximum number of configured joint TCI states per BWP per CC 4. Maximum number of activated joint TCI states across all CCs per ‘coresetPoolIndex’ value 5. One MAC-CE activates one joint TCI-states per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value; [16-2a, 23-4, 40-6-1, 40-6-2, 40-6-3a, 40-6-3b, 40-6-4]; yes; n/a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC is not supported; [Per band]; n/a; n/a; n/a; Component 3 candidate values: {8, 12, 16, 24, 32, 48, 64, 128} Component 4 candidate values: {2, 4, 8, 16} Note: activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH transmissions; Optional with capability signaling. |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-1-7 - Component 1: - It is fine to agree with “intra-cell” if there is another row for “inter-cell”. - Component 2: - It should be removed to be consistent with single DCI case (e.g. FG40-1-1). Then, we can add Note: FG 16-2a-6 can be used to indicate support of two default beams. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-1-7; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC; 1. Support of mTRP operation for M-DCI 2. Support of a single default beam for PDSCH with scheduling offset less than a threshold unless UE indicates the support of two default beams 3. Maximum number of configured joint TCI states per BWP per CC 4. Maximum number of activated joint TCI states across all CCs per ‘coresetPoolIndex’ value 5. One MAC-CE activates one joint TCI-states per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value; [16-2a, 23-4, 40-6-1, 40-6-2, 40-6-3a, 40-6-3b, 40-6-4]; yes; n/a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC is not supported; Per ban]; n/a; n/a; n/a; Component 1 candidate values {Intra-cell, intra-cell and inter-cell} Component 3 candidate values: {8, 12, 16, 24, 32, 48, 64, 128} Component 4 candidate values: {2, 4, 8, 16} Note: activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH transmissions; Optional with capability signaling. |
| Samsung [16] | For the FGs related to multi-DCI based multi-TRP operation including FGs 40-1-7/40-1-7a/40-1-9/40-1-9a, both intra-cell and inter-cell operations should be supported. It is worth noting here that RAN1 has agreed that multi-DCI based multi-TRP can be applied/extended to inter-cell operation under the Rel-18 unified TCI framework (eUTCI) by associating activated TCI state(s) specific to a coresetPoolIndex value to a PCI value. Hence, we propose the following: Proposal 2: For FGs 40-1-7/40-1-7a/40-1-9/40-1-9a, support both intra-cell and inter-cell mTRP operations for M-DCI. |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | Proposal 1-7: For FG 40-1-7, support the following changes on the FFS - Add the following items in the prerequisite field, and remove all other FGs - 23-1-1, and at least one of 16-2a, 40-6-3a, 40-6-3b - Support keeping “intra-cell” in the following component - Support of [intra-cell] mTRP operation for M-DCI - Support adding a separate new FG row for “inter-cell” - Support to keep the following component - [2. Support of a single default beam for PDSCH with scheduling offset less than a threshold unless UE indicates the support of two default beams] - Fine for “per band” 40. NR_MIMO_evo_DL_UL; 40-1-7; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC; 1. Support of [intra-cell] mTRP operation for M-DCI [2. Support of a single default beam for PDSCH with scheduling offset less than a threshold unless UE indicates the support of two default beams] 3. Maximum number of configured joint TCI states per BWP per CC 4. Maximum number of activated joint TCI states across all CCs per ‘coresetPoolIndex’ value 5. One MAC-CE activates one joint TCI-states per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value; [16-2a, 23-4, 40-6-1, 40-6-2, 40-6-3a, 40-6-3b, 40-6-4]; yes; n/a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC is not supported; [Per band]; n/a; n/a; n/a; Component 3 candidate values: {8, 12, 16, 24, 32, 48, 64, 128} Component 4 candidate values: {2, 4, 8, 16} Note: activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH transmissions; Optional with capability signaling. |
| Huawei/HiSilicon [19] | For FG 40-1-7, one remaining issue is whether restrict it to intra-cell case. Since multi-DCI based multi-TRP include both intra-cell and inter-cell cases, restricting FG 40-1-7 to intra-cell case will lead to the need of introducing another FG for the inter-cell case. For simplicity, we prefer to avoid such restriction. Then, FG 40-1-7 is only used to report that the UE supports unified TCI for multi-DCI based multi-TRP. As for whether it supports unified TCI for intra-cell or inter-cell multi-DCI based multi-TRP, it depends on whether the UE support intra-cell or inter-cell multi-DCI based multi-TRP. For example, if UE support FG 40-1-7 and intra-cell multi-DCI based multi-TRP FG 16-2a, then it supports unified TCI for intra-cell multi-DCI based multi-TRP. Similarly, if UE support FG 40-1-7 and inter-cell multi-DCI based multi-TRP FG 23-4, then it supports unified TCI for inter-cell multi-DCI based multi-TRP. Proposal 2-4: For FG 40-1-7, do not restrict it to the intra-cell case. For FG 40-1-7, there is an FFS “Support of a single default beam for PDSCH with scheduling offset less than a threshold unless UE indicates the support of two default beams”. In the legacy mDCI based mTRP FG 16-2a, we don’t have such a component. To our understanding, supporting a single default beam is a basic feature by default. There is no need to have a component for it. Proposal 2-5: For FG 40-1-7, remove the “Support of a single default beam for PDSCH with scheduling offset less than a threshold unless UE indicates the support of two default beams”. For FG 40-1-7a, one remaining issue is whether to have a component on the minimum beam application time in symbols. We don't think this is needed as FG 23-1-1b is the prerequisite of FG 40-1-7a and the minimum beam application time in symbols is already included in 23-1-1b. Proposal 2-6: For 40-1-7, remove the component: the minimum beam application time in symbols. 40. NR_MIMO_evo_DL_UL; 40-1-7; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC; 1. Support of [intra-cell] mTRP operation for M-DCI [2. Support of a single default beam for PDSCH with scheduling offset less than a threshold unless UE indicates the support of two default beams] 3. Maximum number of configured joint TCI states per BWP per CC 4. Maximum number of activated joint TCI states across all CCs per ‘coresetPoolIndex’ value 5. One MAC-CE activates one joint TCI-states per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value; [16-2a, 23-4, 40-6-1, 40-6-2, 40-6-3a, 40-6-3b, 40-6-4] 23-1-1; yes; n/a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC is not supported; [Per band]; n/a; n/a; n/a; Component 3 candidate values: {8, 12, 16, 24, 32, 48, 64, 128} Component 4 candidate values: {2, 4, 8, 16} Note: activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH transmissions; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-1-7a | Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC | 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2 without DL assignment) 2. Maximum number of MAC-CE activated joint TCI states per BWP per CC per coresetpoolindex [3. The minimum beam application time in symbols] | [FFS] | yes | n/a | Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC is not supported | [Per band] | n/a | n/a | n/a | Component 2 candidate values: {2,3,4,5,6,7,8) Component 3 candidate values: {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336}, where {84, 98, 112, 224, 336 } only can be applicable in FR2 | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-1-7a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC; 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2 without DL assignment) 2. Maximum number of MAC-CE activated joint TCI states per BWP per CC per coresetpoolindex [3. The minimum beam application time in symbols]; [FFS] 40-1-7; yes; n/a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {2,3,4,5,6,7,8) Component 3 candidate values: {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336}, where {84, 98, 112, 224, 336 } only can be applicable in FR2; Optional with capability signaling. |
| Ericsson [3] | This is the first time the DCI-based TCI state indication occurs. “[3. The minimum beam application time in symbols].”: this should be a component in FG 40-1-7a. It seems limiting to use the same value as in FG 23-1-1b or FG 23-10-1b. As a pre-requisite, it is sufficient to list FG 40-1-7. The FG should be signalled per band, in line with RAN2 recommendations. 40-1-7a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC; 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2 without DL assignment) 2. Maximum number of MAC-CE activated joint TCI states per BWP per CC per coresetpoolindex 3. The minimum beam application time in symbols; 40-1-7; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC is not supported; Per band; Component 2 candidate values: {2,3,4,5,6,7,8) Component 3 candidate values: {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336}, where {84, 98, 112, 224, 336 } only can be applicable in FR2. |
| ZTE [4] | - FG 40-1-7a, we do NOT identify the necessity of adding additional component of ‘the minimum beam application time in symbols’ which has been considered as in/aligned with Rel-17 single TCI framework. 40. NR_MIMO_evo_DL_UL; 40-1-7a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC; 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2 without DL assignment) 2. Maximum number of MAC-CE activated joint TCI states per BWP per CC per coresetpoolindex [3. The minimum beam application time in symbols]; [FFS] 40-1-7; yes; n/a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {2,3,4,5,6,7,8) Component 3 candidate values: {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336}, where {84, 98, 112, 224, 336 } only can be applicable in FR2; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-1-7b; TRP-specific BFR with unified TCI framework with Unified TCI for multi-DCI based multi-TRP; Support for TRP-specific BFR with unified TCI framework with Unified TCI for multi-DCI based multi-TRP; 40-1-7; yes; n/a; TRP-specific BFR with unified TCI framework with Unified TCI for multi-DCI based multi-TRP is not supported; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-1-7a; Unified TCI with joint DL/UL TCI update for multi-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC; 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2 without DL assignment) 2. Maximum number of MAC-CE activated joint TCI states per BWP per CC per coresetpoolindex [3. The minimum beam application time in symbols]; [FFS] 23-10-1b, 40-1-7; yes; n/a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {2,3,4,5,6,7,8) Component 3 candidate values: {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336}, where {84, 98, 112, 224, 336 } only can be applicable in FR2; Optional with capability signaling. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-1-7a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC; 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2 without DL assignment) 2. Maximum number of MAC-CE activated joint TCI states per BWP per CC per coresetpoolindex [3. The minimum beam application time in symbols]; [FFS]; yes; n/a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {2,3,4,5,6,7,8) Component 3 candidate values: {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336}, where {84, 98, 112, 224, 336 } only can be applicable in FR2; Optional with capability signaling. |
| OPPO [8] | 40. NR_MIMO_evo_DL_UL; 40-1-7a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC; 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2 without DL assignment) 2. Maximum number of MAC-CE activated joint TCI states per BWP per CC across all CCs per coresetpoolindex [3. The minimum beam application time in symbols]; [FFS]; yes; n/a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {2,3,4,5,6,7,8) Component 3 candidate values: {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336}, where {84, 98, 112, 224, 336 } only can be applicable in FR2; Optional with capability signaling. |
| CATT [9] | 40-1-7a: - The prerequisite of 40-1-7a should be 40-1-7. |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-1-7a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC; 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2 without DL assignment) 2. Maximum number of MAC-CE activated joint TCI states per BWP per CC per coresetpoolindex [3. The minimum beam application time in symbols]; 40-1-7 23-1-1b; yes; n/a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {2,3,4,5,6,7,8) Component 3 candidate values: {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336}, where {84, 98, 112, 224, 336 } only can be applicable in FR2; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-1-7a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC; 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2 without DL assignment) 2. Maximum number of MAC-CE activated joint TCI states per BWP per CC per coresetpoolindex 3. The minimum beam application time in symbols; [FFS]; yes; n/a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC is not supported; Per band; n/a; n/a; n/a; Component 2 candidate values: {2,3,4,5,6,7,8) Component 3 candidate values: {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336}, where {84, 98, 112, 224, 336 } only can be applicable in FR2; Optional with capability signaling. |
| Samsung [16] | For the FGs related to multi-DCI based multi-TRP operation including FGs 40-1-7/40-1-7a/40-1-9/40-1-9a, both intra-cell and inter-cell operations should be supported. It is worth noting here that RAN1 has agreed that multi-DCI based multi-TRP can be applied/extended to inter-cell operation under the Rel-18 unified TCI framework (eUTCI) by associating activated TCI state(s) specific to a coresetPoolIndex value to a PCI value. Hence, we propose the following: Proposal 2: For FGs 40-1-7/40-1-7a/40-1-9/40-1-9a, support both intra-cell and inter-cell mTRP operations for M-DCI. |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | Proposal 1-8: For FG 40-1-7a, support the following changes on the FFS - Add the following items in the prerequisite field - 40-1-7 - Fine for “per band” 40. NR_MIMO_evo_DL_UL; 40-1-7a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC; 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2 without DL assignment) 2. Maximum number of MAC-CE activated joint TCI states per BWP per CC per coresetpoolindex [3. The minimum beam application time in symbols]; [FFS]; yes; n/a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {2,3,4,5,6,7,8) Component 3 candidate values: {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336}, where {84, 98, 112, 224, 336 } only can be applicable in FR2; Optional with capability signaling. |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-1-7a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC; 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2 without DL assignment) 2. Maximum number of MAC-CE activated joint TCI states per BWP per CC per coresetpoolindex [3. The minimum beam application time in symbols]; [FFS] 40-1-7, 23-1-1b; yes; n/a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {2,3,4,5,6,7,8) Component 3 candidate values: {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336}, where {84, 98, 112, 224, 336 } only can be applicable in FR2; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-1-9 | Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC | 1. Maximum number of configured DL TCI states per BWP per CC 2. Maximum number of configured UL TCI states per BWP per CC 3. Maximum number of activated DL TCI states across all CC 4. Maximum number of activated UL TCI states across all CC 5. One MAC-CE activated DL TCI-state per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value. 6. One MAC-CE activated UL TCI-state per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value. | [23-10-1] | yes | n/a | Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC is not supported | [Per band] | n/a | n/a | n/a | Component 1 candidate value {8, 12, 16, 24, 32, 48, 64, 128} Component 2 candidate value {8, 12, 16, 24, 32, 48, 64} Component 3 candidate values: {1, 2, 4, 8, 16} Component 4 candidate values: {1, 2, 4, 8, 16} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-1-9; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC; 1. Maximum number of configured DL TCI states per BWP per CC 2. Maximum number of configured UL TCI states per BWP per CC 3. Maximum number of activated DL TCI states across all CC 4. Maximum number of activated UL TCI states across all CC 5. One MAC-CE activated DL TCI-state per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value. 6. One MAC-CE activated UL TCI-state per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value.; [23-10-1]; yes; n/a; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC is not supported; [Per band]; n/a; n/a; n/a; Component 1 candidate value {8, 12, 16, 24, 32, 48, 64, 128} Component 2 candidate value {8, 12, 16, 24, 32, 48, 64} Component 3 candidate values: {1, 2, 4, 8, 16} Component 4 candidate values: {1, 2, 4, 8, 16}; Optional with capability signaling. |
| Ericsson [3] | As a pre-requisite, it is sufficient to list FG 40-1-7. The FG should be signalled per band, in line with RAN2 recommendations. 40-1-9; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC; 1. Maximum number of configured DL TCI states per BWP per CC 2. Maximum number of configured UL TCI states per BWP per CC 3. Maximum number of activated DL TCI states across all CC 4. Maximum number of activated UL TCI states across all CC 5. One MAC-CE activated DL TCI-state per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value. 6. One MAC-CE activated UL TCI-state per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value.; 40-1-7; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC is not supported; Per band; Component 1 candidate value {8, 12, 16, 24, 32, 48, 64, 128} Component 2 candidate value {8, 12, 16, 24, 32, 48, 64} Component 3 candidate values: {1, 2, 4, 8, 16} Component 4 candidate values: {1, 2, 4, 8, 16}. |
| ZTE [4] | - After that, for FG 40-1-9/9a, the UE can separately indicate the corresponding configured/activated DL/UL TCI state. 40. NR_MIMO_evo_DL_UL; 40-1-9; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC; 1. Maximum number of configured DL TCI states per BWP per CC 2. Maximum number of configured UL TCI states per BWP per CC 3. Maximum number of activated DL TCI states across all CC 4. Maximum number of activated UL TCI states across all CC 5. One MAC-CE activated DL TCI-state per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value. 6. One MAC-CE activated UL TCI-state per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value.; [23-10-1] 40-1-7; yes; n/a; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC is not supported; [Per band]; n/a; n/a; n/a; Component 1 candidate value {8, 12, 16, 24, 32, 48, 64, 128} Component 2 candidate value {8, 12, 16, 24, 32, 48, 64} Component 3 candidate values: {1, 2, 4, 8, 16} Component 4 candidate values: {1, 2, 4, 8, 16} Note: FG 16-2a-6 can be used to indicate support of two default beams; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-1-9; Unified TCI with separate DL/UL TCI update for multi-DCI based intra-cell multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC; 1. Maximum number of configured DL TCI states per BWP per CC 2. Maximum number of configured UL TCI states per BWP per CC 3. Maximum number of activated DL TCI states across all CC 4. Maximum number of activated UL TCI states across all CC 5. One MAC-CE activated DL TCI-state per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value. 6. One MAC-CE activated UL TCI-state per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value.; [23-10-1], 16-2a; yes; n/a; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC is not supported; [Per band]; n/a; n/a; n/a; Component 1 candidate value {8, 12, 16, 24, 32, 48, 64, 128} Component 2 candidate value {8, 12, 16, 24, 32, 48, 64} Component 3 candidate values: {1, 2, 4, 8, 16} Component 4 candidate values: {1, 2, 4, 8, 16}; Optional with capability signaling. |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-1-9; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC; 1. Maximum number of configured DL TCI states per BWP per CC 2. Maximum number of configured UL TCI states per BWP per CC 3. Maximum number of activated DL TCI states across all CC 4. Maximum number of activated UL TCI states across all CC 5. One MAC-CE activated DL TCI-state per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value. 6. One MAC-CE activated UL TCI-state per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value.; 23-10-1, 16-2a; yes; n/a; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC is not supported; [Per band]; n/a; n/a; n/a; Component 1 candidate value {8, 12, 16, 24, 32, 48, 64, 128} Component 2 candidate value {8, 12, 16, 24, 32, 48, 64} Component 3 candidate values: {1, 2, 4, 8, 16} Component 4 candidate values: {1, 2, 4, 8, 16}; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-1-9; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC; 1. Maximum number of configured DL TCI states per BWP per CC 2. Maximum number of configured UL TCI states per BWP per CC 3. Maximum number of activated DL TCI states across all CC 4. Maximum number of activated UL TCI states across all CC 5. One MAC-CE activated DL TCI-state per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value. 6. One MAC-CE activated UL TCI-state per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value. 7. Support unified TCI with separate DL/UL TCI update for multi-DCI intra-cell and inter-cell multi-TRP operation; [23-10-1]; yes; n/a; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC is not supported; Per band; n/a; n/a; n/a; Component 1 candidate value {8, 12, 16, 24, 32, 48, 64, 128} Component 2 candidate value {8, 12, 16, 24, 32, 48, 64} Component 3 candidate values: {1, 2, 4, 8, 16} Component 4 candidate values: {1, 2, 4, 8, 16} Component 7 candidate values {Intra-cell, intra-cell and inter-cell}; Optional with capability signaling. |
| Samsung [16] | For the FGs related to multi-DCI based multi-TRP operation including FGs 40-1-7/40-1-7a/40-1-9/40-1-9a, both intra-cell and inter-cell operations should be supported. It is worth noting here that RAN1 has agreed that multi-DCI based multi-TRP can be applied/extended to inter-cell operation under the Rel-18 unified TCI framework (eUTCI) by associating activated TCI state(s) specific to a coresetPoolIndex value to a PCI value. Hence, we propose the following: Proposal 2: For FGs 40-1-7/40-1-7a/40-1-9/40-1-9a, support both intra-cell and inter-cell mTRP operations for M-DCI. |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | Proposal 1-9: For FG 40-1-9, support the following changes on the FFS - Add the following items in the prerequisite field, and remove all other FGs - 23-10-1, and at least one of 16-2a, 40-6-3a, 40-6-3b - Fine for “per band” 40. NR_MIMO_evo_DL_UL; 40-1-9; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC; 1. Maximum number of configured DL TCI states per BWP per CC 2. Maximum number of configured UL TCI states per BWP per CC 3. Maximum number of activated DL TCI states across all CC 4. Maximum number of activated UL TCI states across all CC 5. One MAC-CE activated DL TCI-state per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value. 6. One MAC-CE activated UL TCI-state per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value.; [23-10-1]; yes; n/a; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC is not supported; [Per band]; n/a; n/a; n/a; Component 1 candidate value {8, 12, 16, 24, 32, 48, 64, 128} Component 2 candidate value {8, 12, 16, 24, 32, 48, 64} Component 3 candidate values: {1, 2, 4, 8, 16} Component 4 candidate values: {1, 2, 4, 8, 16}; Optional with capability signaling. |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-1-9; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC; 1. Maximum number of configured DL TCI states per BWP per CC 2. Maximum number of configured UL TCI states per BWP per CC 3. Maximum number of activated DL TCI states across all CC 4. Maximum number of activated UL TCI states across all CC 5. One MAC-CE activated DL TCI-state per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value. 6. One MAC-CE activated UL TCI-state per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value.; [23-10-1] 40-1-7,; yes; n/a; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC is not supported; [Per band]; n/a; n/a; n/a; Component 1 candidate value {8, 12, 16, 24, 32, 48, 64, 128} Component 2 candidate value {8, 12, 16, 24, 32, 48, 64} Component 3 candidate values: {1, 2, 4, 8, 16} Component 4 candidate values: {1, 2, 4, 8, 16}; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-1-9a | Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC | 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1 and if supported 1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1 and if supported 1_2 without DL assignment) 2. maximum number of activated DL TCI states per CORESETPoolIndex per BWP per CC 3. maximum number of activated UL TCI states per CORESETPoolIndex per BWP per CC | [23-10-1b, 40-1-9] | yes | n/a | Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC is not supported | [Per band] | n/a | n/a | n/a | Component 2 candidate values: {1, 2, 3, 4, 5, 6, 7, 8} Component 3 candidate values: {1, 2, 3, 4, 5, 6, 7, 8,} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-1-9a; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC; 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1 and if supported 1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1 and if supported 1_2 without DL assignment) 2. maximum number of activated DL TCI states per CORESETPoolIndex per BWP per CC 3. maximum number of activated UL TCI states per CORESETPoolIndex per BWP per CC; [23-10-1b, 40-1-9]; yes; n/a; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {1, 2, 3, 4, 5, 6, 7, 8} Component 3 candidate values: {1, 2, 3, 4, 5, 6, 7, 8,}; Optional with capability signalling. |
| Ericsson [3] | As a pre-requisite, it is sufficient to list FG 40-1-7a. The FG should be signalled per band, in line with RAN2 recommendations. We prefer to add a separate component for beam application time, just as for Rel-17. 40-1-9a; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC; 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1 and if supported 1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1 and if supported 1_2 without DL assignment) 2. maximum number of activated DL TCI states per CORESETPoolIndex per BWP per CC 3. maximum number of activated UL TCI states per CORESETPoolIndex per BWP per CC 4. The minimum beam application time in symbols; 40-1-9 40-1-7a; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC is not supported; Per band; Component 2 candidate values: {1, 2, 3, 4, 5, 6, 7, 8} Component 3 candidate values: {1, 2, 3, 4, 5, 6, 7, 8,}. |
| ZTE [4] | - After that, for FG 40-1-9/9a, the UE can separately indicate the corresponding configured/activated DL/UL TCI state. 40. NR_MIMO_evo_DL_UL; 40-1-9a; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC; 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1 and if supported 1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1 and if supported 1_2 without DL assignment) 2. maximum number of activated DL TCI states per CORESETPoolIndex per BWP per CC 3. maximum number of activated UL TCI states per CORESETPoolIndex per BWP per CC; [23-10-1b, 40-1-9] 40-1-9; yes; n/a; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {1, 2, 3, 4, 5, 6, 7, 8} Component 3 candidate values: {1, 2, 3, 4, 5, 6, 7, 8,}; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-1-9b; Unified TCI indication for M-DCI based inter-cell mTRP; Support of unified TCI indication for inter-cell mTRP in M-DCI; 40-1-7; yes; n/a; Unified TCI indication for M-DCI based inter-cell mTRP is not supported; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-1-9a; Unified TCI with separate DL/UL TCI update for multi-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC; 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1 and if supported 1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1 and if supported 1_2 without DL assignment) 2. maximum number of activated DL TCI states per CORESETPoolIndex per BWP per CC 3. maximum number of activated UL TCI states per CORESETPoolIndex per BWP per CC; [23-10-1b, 40-1-9]; yes; n/a; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {1, 2, 3, 4, 5, 6, 7, 8} Component 3 candidate values: {1, 2, 3, 4, 5, 6, 7, 8,}; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-1-y; Unified TCI with separate DL/UL TCI update for multi-DCI based inter-cell multi-TRP; 1. Support of inter-cell mTRP operation for M-DCI; 23-4, at least one of {40-1-7/7a, 40-1-9/9a}. |
| Intel Corporation [7] |  |
| OPPO [8] | 40. NR_MIMO_evo_DL_UL; 40-1-9a; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC; 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1 and if supported 1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1 and if supported 1_2 without DL assignment) 2. maximum number of activated DL TCI states per CORESETPoolIndex per BWP per CC across all CCs 3. maximum number of activated UL TCI states per CORESETPoolIndex per BWP per CC across all CCs; [23-10-1b, 40-1-9]; yes; n/a; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {1, 2, 3, 4, 5, 6, 7, 8} Component 3 candidate values: {1, 2, 3, 4, 5, 6, 7, 8,}; Optional with capability signalling. |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-1-9a; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC; 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1 and if supported 1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1 and if supported 1_2 without DL assignment) 2. maximum number of activated DL TCI states per CORESETPoolIndex per BWP per CC 3. maximum number of activated UL TCI states per CORESETPoolIndex per BWP per CC; 23-10-1b, 16-2a, 40-1-9; yes; n/a; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {1, 2, 3, 4, 5, 6, 7, 8} Component 3 candidate values: {1, 2, 3, 4, 5, 6, 7, 8,}; Optional with capability signalling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-1-9a; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC; 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1 and if supported 1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1 and if supported 1_2 without DL assignment) 2. maximum number of activated DL TCI states per CORESETPoolIndex per BWP per CC 3. maximum number of activated UL TCI states per CORESETPoolIndex per BWP per CC 4. The minimum beam application time in Y symbols per SCS; [23-10-1b, 40-1-9]; yes; n/a; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC is not supported; Per band; n/a; n/a; n/a; Component 2 candidate values: {1, 2, 3, 4, 5, 6, 7, 8} Component 3 candidate values: {1, 2, 3, 4, 5, 6, 7, 8,} Component 4 candidate value {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336}, where {84, 98, 112, 224, 336} only can be indicated in FR2.; Optional with capability signalling. |
| Samsung [16] | For the FGs related to multi-DCI based multi-TRP operation including FGs 40-1-7/40-1-7a/40-1-9/40-1-9a, both intra-cell and inter-cell operations should be supported. It is worth noting here that RAN1 has agreed that multi-DCI based multi-TRP can be applied/extended to inter-cell operation under the Rel-18 unified TCI framework (eUTCI) by associating activated TCI state(s) specific to a coresetPoolIndex value to a PCI value. Hence, we propose the following: Proposal 2: For FGs 40-1-7/40-1-7a/40-1-9/40-1-9a, support both intra-cell and inter-cell mTRP operations for M-DCI. |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | Proposal 1-10: For FG 40-1-9a, support the following changes on the FFS - Add the following items in the prerequisite field, and remove all other FGs - 40-1-9 - Fine for “per band” 40. NR_MIMO_evo_DL_UL; 40-1-9a; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC; 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1 and if supported 1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1 and if supported 1_2 without DL assignment) 2. maximum number of activated DL TCI states per CORESETPoolIndex per BWP per CC 3. maximum number of activated UL TCI states per CORESETPoolIndex per BWP per CC; [23-10-1b, 40-1-9]; yes; n/a; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {1, 2, 3, 4, 5, 6, 7, 8} Component 3 candidate values: {1, 2, 3, 4, 5, 6, 7, 8,}; Optional with capability signalling. |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-1-9a; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC; 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1 and if supported 1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1 and if supported 1_2 without DL assignment) 2. maximum number of activated DL TCI states per CORESETPoolIndex per BWP per CC 3. maximum number of activated UL TCI states per CORESETPoolIndex per BWP per CC; [23-10-1b, 40-1-9]; yes; n/a; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {1, 2, 3, 4, 5, 6, 7, 8} Component 3 candidate values: {1, 2, 3, 4, 5, 6, 7, 8,}; Optional with capability signalling. |

| 40. NR_MIMO_evo_DL_UL | 40-1-12 | Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP | 1. Support of common multi-CC TCI state ID update and activation for single-DCI based multi-TRP 2. Maximum number of CC list(s) | [40-1-1, 40-1-1, 40-1-2] | yes | n/a | Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP is not supported | [Per band] | n/a | n/a | n/a | Component 2 candidate values: {1,2,3,4} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-1-12; Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP; 1. Support of common multi-CC TCI state ID update and activation for single-DCI based multi-TRP 2. Maximum number of CC list(s); [40-1-1, 40-1-1, 40-1-2]; yes; n/a; Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {1,2,3,4}; Optional with capability signaling. |
| Ericsson [3] | As a pre-requisite, it is sufficient to list FG 40-1-1. The FG should be signalled per band, in line with RAN2 recommendations. Also, the corresponding Rel-17 feature was specified per band. 40-1-12; Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP; 1. Support of common multi-CC TCI state ID update and activation for single-DCI based multi-TRP 2. Maximum number of CC list(s); 40-1-1; Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP is not supported; Per band; Component 2 candidate values: {1,2,3,4}. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-1-12; Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP; 1. Support of common multi-CC TCI state ID update and activation for single-DCI based multi-TRP 2. Maximum number of CC list(s); [40-1-1, 40-1-1, 40-1-2]; yes; n/a; Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {1,2,3,4}; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-1-12; Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP; 1. Support of common multi-CC TCI state ID update and activation for single-DCI based multi-TRP 2. Maximum number of CC list(s); 23-10-1e, 23-1-1f, [at least one of {40-1-1/1a, 40-1-1, 40-1-2/2a}]; yes; n/a; Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {1,2,3,4}; Optional with capability signaling. |
| Intel Corporation [7] |  |
| OPPO [8] | 40. NR_MIMO_evo_DL_UL; 40-1-12; Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP; - Support of common multi-CC TCI state ID update and activation for single-DCI based multi-TRP - Maximum number of CC list(s) per cell group - Support of reference BWP/CC configured with reference TCI state pool shared by a set of BWP/CC - The maximum number of configured TCI state pools across all BWPs and all CCs in a band; [40-1-1, 40-1-1, 40-1-2]; yes; n/a; Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {1,2,3,4}; Optional with capability signaling. |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-1-12; Common multi-CC TCI state ID update and activation for single-DCI based intra-cell multi-TRP; 1. Support of common multi-CC TCI state ID update and activation for single-DCI based multi-TRP 2. Maximum number of CC list(s); [40-1-1, 40-1-1a, 40-1-2, 40-1-2a]; yes; n/a; Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {1,2,3,4}; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-1-12; Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP; 1. Support of common multi-CC TCI state ID update and activation for single-DCI based multi-TRP 2. Maximum number of CC list(s); [40-1-1, 40-1-1, 40-1-2]; yes; n/a; Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP is not supported; Per band; n/a; n/a; n/a; Component 2 candidate values: {1,2,3,4}; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | Proposal 1-11: For FG 40-1-12, support the following changes on the FFS - Add the following items in the prerequisite field, and remove all other FGs - 40-1-1 - Fine for “per band” 40. NR_MIMO_evo_DL_UL; 40-1-12; Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP; 1. Support of common multi-CC TCI state ID update and activation for single-DCI based multi-TRP 2. Maximum number of CC list(s); [40-1-1, 40-1-1, 40-1-2]; yes; n/a; Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {1,2,3,4}; Optional with capability signaling. |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-1-12; Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP; 1. Support of common multi-CC TCI state ID update and activation for single-DCI based multi-TRP 2. Maximum number of CC list(s); [40-1-1, 40-1-1, 40-1-2]; yes; n/a; Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {1,2,3,4}; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-1-13 | Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP | 1. Support of common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP 2. Maximum number of CC list(s) | [40-1-7] | yes | n/a | Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP is not supported | [Per band] | n/a | n/a | n/a | Component 2 candidate values: {1,2,3,4} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-1-13; Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP; 1. Support of common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP 2. Maximum number of CC list(s); [40-1-7]; yes; n/a; Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {1,2,3,4}; Optional with capability signaling. |
| Ericsson [3] | As a pre-requisite, it is sufficient to list FG 40-1-7. The FG should be signalled per band, in line with RAN2 recommendations. Also, the corresponding Rel-17 feature was specified per band. 40-1-13; Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP; 1. Support of common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP 2. Maximum number of CC list(s); 40-1-7; Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP is not supported; Per band; Component 2 candidate values: {1,2,3,4}. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-1-13; Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP; 1. Support of common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP 2. Maximum number of CC list(s); [40-1-7]; yes; n/a; Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {1,2,3,4}; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-1-13; Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP; 1. Support of common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP 2. Maximum number of CC list(s); 23-10-1e, 23-1-1f, [at least one of {40-1-7/7a, 40-1-9/9a}]; yes; n/a; Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {1,2,3,4}; Optional with capability signaling. |
| Intel Corporation [7] |  |
| OPPO [8] | 40. NR_MIMO_evo_DL_UL; 40-1-13; Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP; - Support of common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP - Maximum number of CC list(s) per cell group - Support of reference BWP/CC configured with reference TCI state pool shared by a set of BWP/CC - The maximum number of configured TCI state pools across all BWPs and all CCs in a band; [40-1-7]; yes; n/a; Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {1,2,3,4}; Optional with capability signaling. |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-1-13; Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP; 1. Support of common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP 2. Maximum number of CC list(s); [40-1-7, 40-1-7a, 40-1-9, 40-1-9a]; yes; n/a; Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {1,2,3,4}; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-1-13; Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP; 1. Support of common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP 2. Maximum number of CC list(s); [40-1-7]; yes; n/a; Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP is not supported; Per band; n/a; n/a; n/a; Component 2 candidate values: {1,2,3,4}; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | Proposal 1-12: For FG 40-1-13, support the following changes on the FFS - Add the following items in the prerequisite field, and remove all other FGs - 40-1-7 - Fine for “per band” 40. NR_MIMO_evo_DL_UL; 40-1-13; Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP; 1. Support of common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP 2. Maximum number of CC list(s); [40-1-7]; yes; n/a; Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {1,2,3,4}; Optional with capability signaling. |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-1-13; Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP; 1. Support of common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP 2. Maximum number of CC list(s); [40-1-7]; yes; n/a; Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {1,2,3,4}; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-2-1 | Basic feature for multi-DCI based intra-cell Multi-TRP operation with two TA enhancement | FFS: Maximum number of n-TimingAdvanceOffset value per serving cell | 16-2a | yes | n/a |  | [Per band] | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-2-1 - FFS: Maximum number of n-TimingAdvanceOffset values per serving cell. The candidate values are added in notes column based on the agreement below. 40. NR_MIMO_evo_DL_UL; 40-2-1; Basic feature for multi-DCI based intra-cell Multi-TRP operation with two TA enhancement; FFS: Maximum number of n-TimingAdvanceOffset value per serving cell; 16-2a; yes; n/a; [Per band]; n/a; n/a; n/a; There is one n-TimingAdvanceOffset values and the candidate values are in Tc units {0, 25600, 39936}; Optional with capability signalling. |
| Ericsson [3] | “FFS: Maximum number of n-TimingAdvanceOffset value per serving cell”. - In RAN1#114, RAN1 made the following agreement Agreement For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement and PDCCH order CFRA, indicate a representation of the TAG ID with 1 bit (either the first TAG ID or the second TAG ID for the serving cell) as part of TA command in RAR Note: For intra-cell multi-DCI based Multi-TRP operation, only a single NTA,offset is configured. - So, for intra-cell, RAN1 has excluded the possibility to configure more than one n-TimingAdvanceOffset values per serving cell. Hence, the FFS can be deleted. - In this FG, the type is in brackets. We propose to follow the RAN2 guidance in [1], and specify this per band. - We propose to add “The maximum number of TAGs all CCs” in FG 40-2-8 as a component of 40-2-1. 40-2-1; Basic feature for multi-DCI based intra-cell Multi-TRP operation with two TA enhancement; The maximum number of TAGs all CCs; 16-2a; Two TA enhancement is not supported for intra-cell operation; Per band. |
| ZTE [4] | - For FG 40-2-1, - As per the following agreement endorsed in RAN1#114 meeting, the FFS part on maximum number of n-TimingAdvanceOffset value per serving cell should be removed. Agreement For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement and PDCCH order CFRA, indicate a representation of the TAG ID with 1 bit (either the first TAG ID or the second TAG ID for the serving cell) as part of TA command in RAR Note: For intra-cell multi-DCI based Multi-TRP operation, only a single NTA,offset is configured.. - Component-1 as approved in R1-2308574 should be captured. - The following components should be added to capture the agreements endorsed for basic features of MDCI MTRP in intra-cell operation with two TAs: - Support of joint/separate DL/UL TCI states of UL signals/channels associated to one CORESET Pool Index correspond to one TAG - Support of maximum receiving timing difference between the DL reference timings from two TRPs is within a CP - Support of RAR-based solution where RAR is only received from the TRP that is associated with Type 1 CSS - The prerequisite FGs should additionally include FG 40-1-7, FG 40-1-7a, FG 40-1-9, FG 40-1-9a, because of the new feature that MDCI MTRP enabled with unified TCI state in Rel-18. - Granularity can be “Per FSPC” to keep alignment with that of its prerequisite FG 16-2a. 40. NR_MIMO_evo_DL_UL; 40-2-1; Basic feature for multi-DCI based intra-cell Multi-TRP operation with two TA enhancement; FFS: Maximum number of n-TimingAdvanceOffset value per serving cell - Support of multi-DCI based intra-cell Multi-TRP operation with two TA enhancement - Support of joint/separate DL/UL TCI states of UL signals/channels associated to one CORESET Pool Index correspond to one TAG - Support of maximum receiving timing difference between the DL reference timings from two TRPs is within a CP - Support of RAR-based solution where RAR is only received from the TRP that is associated with Type 1 CSS; 16-2a, 40-1-7, 40-1-7a, 40-1-9, 40-1-9a; yes; n/a; [Per band] Per FSPC; n/a; n/a; n/a; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-2-9; Two n-TimingAdvanceOffset value in serving cell; Support of two configured n-TimingAdvanceOffset values in serving cell; 40-2-2; yes; n/a; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-2-1; Basic feature for multi-DCI based intra-cell Multi-TRP operation with two TA enhancement; Maximum number of n-TimingAdvanceOffset value per serving cell; 16-2a; yes; n/a; [Per band]; n/a; n/a; n/a; candidate values: {1,2}; Optional with capability signalling. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-2-1; Basic feature for multi-DCI based intra-cell Multi-TRP operation with two TA enhancement; FFS: Maximum number of n-TimingAdvanceOffset value per serving cell; 16-2a; yes; n/a; [Per band]; n/a; n/a; n/a; Optional with capability signalling. |
| OPPO [8] | In previous agreement, the following UE capabilities were explicitly agreed. - For the case of two DL reference timing, support of Rx timing difference between two DL reference timings can be assumed as larger than a CP length is up to UE capability For intra-cell MTRP and inter-cell MTRP, the following UE capabilities should be discussed for two TA enhancement. First, similar to the UE feature of UTCI, we also should split the joint TCI states and separate DL/UL TCI state, just in case that UE only support one type of uTCI state, either joint or separate. So, we suggest to add UE feature 40-2-1a and 40-2-2a as below. Secondly, in our understanding, each TAG has its own n-TimingAdvanceOffset, but some UE may only support one single TA offset value per serving cell. Hence, we suggest to make a component, associated with the basic feature. 40. NR_MIMO_evo_DL_UL; 40-2-1; Basic feature for multi-DCI based intra-cell Multi-TRP operation with two TA enhancement using joint TCI state; FFS: split joint TCI states and separate DL/UL TCI states into two rows/FGs FFS: what to do/signal when UE also supports multi-DCI based STxMP PUSCH+PUSCH FFS: what to do/signal about absolute TA command MAC CE FFS: Maximum number of n-TimingAdvanceOffset value per serving cell. 40. NR_MIMO_evo_DL_UL; 40-2-1a; Basic feature for multi-DCI based intra-cell Multi-TRP operation with two TA enhancement using separate DL/UL TCI state; Maximum number of n-TimingAdvanceOffset value per serving cell. |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-2-1; Basic feature for multi-DCI based intra-cell Multi-TRP operation with two TA enhancement; FFS: Maximum number of n-TimingAdvanceOffset value per serving cell 1. Supported maximum number of n-TimingAdvanceOffset value per serving cell.; 16-2a; yes; n/a; [Per band]; n/a; n/a; n/a; Component 1 candidate values: {1,2}; Optional with capability signalling. |
| LG Electronics [11] | 40-2-1 - Add component as “Support of multi-DCI based intra-cell Multi-TRP operation with two TA enhancement”. - Regarding FFS for maximum number of n-TimingAdvanceOffset value per serving cell, it is not needed for 40-2-1 according to the agreement made in RAN1 #114 (captured below). Agreement For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement and PDCCH order CFRA, indicate a representation of the TAG ID with 1 bit (either the first TAG ID or the second TAG ID for the serving cell) as part of TA command in RAR Note: For intra-cell multi-DCI based Multi-TRP operation, only a single NTA,offset is configured.. Proposal 4: Add component and remove the FFS in 40-2-1. 40. NR_MIMO_evo_DL_UL; 40-2-1; Basic feature for multi-DCI based intra-cell Multi-TRP operation with two TA enhancement; Support of multi-DCI based intra-cell Multi-TRP operation with two TA enhancement; 16-2a; yes; n/a; [Per band]; n/a; n/a; n/a; Optional with capability signalling. |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-2-1 - Components: Regarding the FFS, it was agreed in last meeting that for intra-cell multi-DCI based Multi-TRP operation, only a single NTA,offset is configured. No component or separate row is needed. - Prerequisite FG: In RAN1#113, it was concluded that there is no consensus on support two TA for Rel-15/16 spatial relation framework. Two TA is applied only when unified TCI framework is applied, thus, FG40-1-7 or 40-1-9 (Rel-18 unified TCI for M-DCI M-TRP) should be its prerequisites. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-2-1; Basic feature for multi-DCI based intra-cell Multi-TRP operation with two TA enhancement; - Support of 2 TA/TAG within a serving cell for intra-cell multi-DCI multi-TRP - Rx timing difference between different TRP is no larger than CP length. - The UL/joint TCI states of UL signals/channels associated to one coresetPoolIndex correspond to one TAG; 16-2a; yes; n/a; Per band; n/a; n/a; n/a; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-2-10; Maximum number of n-TimingAdvanceOffset value per serving cell; - Maximum number of n-TimingAdvanceOffset value supported per serving cell; Per band; Component 1 candidate value: {1, 2}; Optional with capability signalling. |
| Samsung [16] | Support of FG 40-2-1 and/or FG 40-2-2 would require a significant UE complexity increase in terms of control logics and baseband processing. To facilitate alternative less complex UE implementations for support of this basic feature, the signalling granularity of FG 40-2-1 and FG 40-2-2 should be per feature set (i.e. ‘per FS’). Proposal 5: Signalling granularity of FG 40-2-1 and FG 40-2-2 should be ‘per FS’. Regarding the maximum number of n-TimingAdvanceOffset, the following agreement has been made in RAN1#114. Agreement For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement and PDCCH order CFRA, indicate a representation of the TAG ID with 1 bit (either the first TAG ID or the second TAG ID for the serving cell) as part of TA command in RAR Note: For intra-cell multi-DCI based Multi-TRP operation, only a single NTA,offset is configured.. Therefore, we suggest for intra-cell operation, the maximum number of n-TimingAdvanceOffset value per serving cell is one. For inter-cell operation, the maximum number of n-TimingAdvanceOffset value per serving cell is two. Proposal 6: For FG 40-2-1 and FG 40-2-2, the maximum n-TimingAdvanceOffset value per serving cell is one for intra-cell operation, otherwise, n-TimingAdvanceOffset value per serving cell is two for inter-cell operation. |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | - Regarding “FFS: Maximum number of n-TimingAdvanceOffset value per serving cell” in FGs 40-2-1/2, we this it is needed as a component (or as a separate FG) for 40-2-2, as value of two should not be mandatory for the basic feature. For FG 40-2-1, it is not needed as it is already agreed that only one n-TimingAdvanceOffset can be configured for the case of intra-cell. - Furthermore, it is better to add the following note to FGs 40-2-1, 40-2-2, and 40-2-7: “Note: Rx timing difference between the two DL reference timings associated with the two TAGs is within a CP”. This makes it more clear / explicit that Rx timing difference exceeding the CP can be assumed only if FG 40-2-6 is supported. 40. NR_MIMO_evo_DL_UL; 40-2-1; Basic feature for multi-DCI based intra-cell Multi-TRP operation with two TA enhancement; Support of two TAGs in a same CC for intra-cel multi-DCI based multi-TRP; 16-2a; yes; n/a; FSPC; n/a; n/a; n/a; Note: Rx timing difference between the two DL reference timings associated with the two TAGs is within a CP; Optional with capability signalling. |
| Huawei/HiSilicon [19] | For FG 40-2-1 and FG 40-2-2, there is an FFS on maximum number of n-TimingAdvanceOffset value per serving cell. It was agreed that the maximum number of TA offset for intra-cell case is one. So, the FFS in 40-2-1 can be directly removed. For FG 40-2-2, it should have a component on the maximum number of n-TimingAdvanceOffset. Proposal 2-7: For 40-2-1, remove the “FFS: Maximum number of n-TimingAdvanceOffset value per serving cell”. 40. NR_MIMO_evo_DL_UL; 40-2-1; Basic feature for multi-DCI based intra-cell Multi-TRP operation with two TA enhancement; FFS: Maximum number of n-TimingAdvanceOffset value per serving cell; 16-2a; yes; n/a; [Per band]; n/a; n/a; n/a; Optional with capability signalling. |

| 40. NR_MIMO_evo_DL_UL | 40-2-2 | Basic feature for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement | FFS: Maximum number of n-TimingAdvanceOffset value per serving cell | 16-2a | yes | n/a |  | [Per band] | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-2-2 - FFS: Maximum number of n-TimingAdvanceOffset values per serving cell. The candidate values are added in notes column based on the agreement below. Agreement (RAN1#110 Toulouse) For multi-DCI multi-TRP operation with two TAs, up to two n-TimingAdvanceOffset value per serving cell is supported Agreement(RAN1#114 Toulouse) For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement and PDCCH order CFRA, indicate a representation of the TAG ID with 1 bit (either the first TAG ID or the second TAG ID for the serving cell) as part of TA command in RAR Note: For intra-cell multi-DCI based Multi-TRP operation, only a single NTA,offset is configured.. 40. NR_MIMO_evo_DL_UL; 40-2-2; Basic feature for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement; FFS: Maximum number of n-TimingAdvanceOffset value per serving cell; 16-2a; yes; n/a; [Per band]; n/a; n/a; n/a; There are up to two n-TimingAdvanceOffset values for serving cell, and the candidate values are in Tc units {0, 25600, 39936}; Optional with capability signalling. |
| Ericsson [3] | “FFS: Maximum number of n-TimingAdvanceOffset value per serving cell” We do not see that it is possible to define more than one n-TimingAdvanceOffset value per serving cell, since this is a cell-specific quantity: it determines the offset between UL and DL and is the same for all UEs in a cell. For now, this FFS can remain. We propose to add FG 40-2-8 as a component of 40-2-2. In this FG, the type is in brackets. We propose to follow the RAN2 guidance in [1], and specify this per band. 40-2-2; Basic feature for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement; FFS: Maximum number of n-TimingAdvanceOffset value per serving cell The maximum number of TAGs all CCs; 16-2a; Two TA enhancement is not supported for inter-cell operation; Per band. |
| ZTE [4] | - For FG 40-2-2, - Component-1 as approved in R1-2308574 should be captured. - The following components should be added to capture the agreements endorsed for basic features of MDCI MTRP in intra-cell operation with two TAs: - Support of joint/separate DL/UL TCI states of UL signals/channels associated to one CORESET Pool Index correspond to one TAG - Support of maximum receiving timing difference between the DL reference timings from two TRPs is within a CP - Support of RAR-based solution where RAR is only received from the TRP that is associated with Type 1 CSS - Regarding the FFS, it is proper to specify the feature that “two n-TimingAdvanceOffset value per serving cell” as an independent FG, which is added as below, i.e., FG 40-2-9. - The prerequisite FG 16-2a should be replaced to FG 23-4, and additionally including FG 40-1-7, FG 40-1-7a, FG 40-1-9, FG 40-1-9a, because of the new feature that MDCI MTRP enabled with unified TCI state in Rel-18. - Granularity should be “Per band” to keep alignment with that of its prerequisite FG 23-4. 40. NR_MIMO_evo_DL_UL; 40-2-2; Basic feature for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement; - Support of multi-DCI based inter-cell Multi-TRP operation with two TA enhancement - Support of joint/separate DL/UL TCI states of UL signals/channels associated to one CORESET Pool Index correspond to one TAG - Support of maximum receiving timing difference between the DL reference timings from two TRPs is within a CP - Support of RAR-based solution where RAR is only received from the TRP that is associated with Type 1 CSS - Support of one configured n-TimingAdvanceOffset value in serving cell FFS: Maximum number of n-TimingAdvanceOffset value per serving cell; 16-2a; yes; n/a; [Per band] Per band; n/a; n/a; n/a; Optional with capability signalling. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-2-2; Basic feature for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement; Maximum number of n-TimingAdvanceOffset value per serving cell; 16-2a; yes; n/a; [Per band]; n/a; n/a; n/a; candidate values: {1,2}; Optional with capability signalling. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-2-2; Basic feature for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement; FFS: Maximum number of n-TimingAdvanceOffset value per serving cell; 16-2a; yes; n/a; [Per band]; n/a; n/a; n/a; Optional with capability signalling. |
| OPPO [8] | In previous agreement, the following UE capabilities were explicitly agreed. - For the case of two DL reference timing, support of Rx timing difference between two DL reference timings can be assumed as larger than a CP length is up to UE capability For intra-cell MTRP and inter-cell MTRP, the following UE capabilities should be discussed for two TA enhancement. First, similar to the UE feature of UTCI, we also should split the joint TCI states and separate DL/UL TCI state, just in case that UE only support one type of uTCI state, either joint or separate. So, we suggest to add UE feature 40-2-1a and 40-2-2a as below. Secondly, in our understanding, each TAG has its own n-TimingAdvanceOffset, but some UE may only support one single TA offset value per serving cell. Hence, we suggest to make a component, associated with the basic feature. 40. NR_MIMO_evo_DL_UL; 40-2-2; Basic feature for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement using joint TCI state; FFS: split joint TCI states and separate DL/UL TCI states into two rows/FGs FFS: what to do/signal when UE also supports multi-DCI based STxMP PUSCH+PUSCH FFS: what to do/signal about absolute TA command MAC CE FFS: Maximum number of n-TimingAdvanceOffset value per serving cell. 40. NR_MIMO_evo_DL_UL; 40-2-2a; Basic feature for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement using separate DL/UL TCI state; Maximum number of n-TimingAdvanceOffset value per serving cell. |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-2-2; Basic feature for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement; FFS: Maximum number of n-TimingAdvanceOffset value per serving cell 1. Supported maximum number of n-TimingAdvanceOffset value per serving cell.; 16-2a; yes; n/a; [Per band]; n/a; n/a; n/a; Component 1 candidate values: {1,2}; Optional with capability signalling. |
| LG Electronics [11] | 40-2-2 - Add component as “Support of multi-DCI based inter-cell Multi-TRP operation with two TA enhancement”. Proposal 5: Add component in 40-2-2. 40. NR_MIMO_evo_DL_UL; 40-2-2; Basic feature for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement; Support of multi-DCI based inter-cell Multi-TRP operation with two TA enhancement FFS: Maximum number of n-TimingAdvanceOffset value per serving cell; 16-2a; yes; n/a; [Per band]; n/a; n/a; n/a; Optional with capability signalling. |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-2-2 - Components: Regarding the FFS, support up to two n-TimingAdvanceOffset value per serving cell can be basic feature. No separate row is needed. - Prerequisite FG: Similar as 40-2-1, FG40-1-7 or 40-1-9 (Rel-18 unified TCI for M-DCI M-TRP) should be its prerequisite. Furthermore, 23-4 (inter-cell mTRP) should be its prerequisite. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-2-2; Basic feature for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement; - serving cell for inter-cell multi-DCI multi-TRP - Rx timing difference between different TRP is no larger than CP length. - The UL/joint TCI states of UL signals/channels associated to one coresetPoolIndex correspond to one TAG.; 16-2a; yes; n/a; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| Samsung [16] | Support of FG 40-2-1 and/or FG 40-2-2 would require a significant UE complexity increase in terms of control logics and baseband processing. To facilitate alternative less complex UE implementations for support of this basic feature, the signalling granularity of FG 40-2-1 and FG 40-2-2 should be per feature set (i.e. ‘per FS’). Proposal 5: Signalling granularity of FG 40-2-1 and FG 40-2-2 should be ‘per FS’. Regarding the maximum number of n-TimingAdvanceOffset, the following agreement has been made in RAN1#114. Agreement For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement and PDCCH order CFRA, indicate a representation of the TAG ID with 1 bit (either the first TAG ID or the second TAG ID for the serving cell) as part of TA command in RAR Note: For intra-cell multi-DCI based Multi-TRP operation, only a single NTA,offset is configured.. Therefore, we suggest for intra-cell operation, the maximum number of n-TimingAdvanceOffset value per serving cell is one. For inter-cell operation, the maximum number of n-TimingAdvanceOffset value per serving cell is two. Proposal 6: For FG 40-2-1 and FG 40-2-2, the maximum n-TimingAdvanceOffset value per serving cell is one for intra-cell operation, otherwise, n-TimingAdvanceOffset value per serving cell is two for inter-cell operation. |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-2-2; Basic feature for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement; 1. Support of two TAGs in a same CC inter-cel multi-DCI based multi-TRP 2. Maximum number of n-TimingAdvanceOffset value per serving cell; 16-2a; yes; n/a; FSPC; n/a; n/a; n/a; Note: Rx timing difference between the two DL reference timings associated with the two TAGs is within a CP Component 2 candidate values: {1,2}; Optional with capability signalling. |
| Huawei/HiSilicon [19] | For FG 40-2-1 and FG 40-2-2, there is an FFS on maximum number of n-TimingAdvanceOffset value per serving cell. It was agreed that the maximum number of TA offset for intra-cell case is one. So, the FFS in 40-2-1 can be directly removed. For FG 40-2-2, it should have a component on the maximum number of n-TimingAdvanceOffset. Proposal 2-8: For 40-2-2, introduce a component on the maximum number of n-TimingAdvanceOffset value per serving cell. 40. NR_MIMO_evo_DL_UL; 40-2-2; Basic feature for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement; FFS: Maximum number of n-TimingAdvanceOffset value per serving cell; 16-2a; yes; n/a; [Per band]; n/a; n/a; n/a; Optional with capability signalling. |

| 40. NR_MIMO_evo_DL_UL | 40-2-3 | TAG ID indication via absolute TA command MAC CE | Support indicating one of two TAG IDs configured in the SpCell via absolute TA command MAC CE. |  | yes | n/a |  | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] | 40-2-3; TAG ID indication via absolute TA command MAC CE; Support indicating one of two TAG IDs configured in the SpCell via absolute TA command MAC CE.; TAG indication in absolute TA command MAC CE is not supported; Per band. |
| ZTE [4] |  |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-2-3; TAG ID indication via absolute TA command MAC CE; Support indicating one of two TAG IDs configured in the SpCell via absolute TA command MAC CE.; yes; n/a; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | FG 40-2-3 - Prerequisite FG: FG 40-2-1 or 40-2-2 should be its prerequisite. In addition, it is used only for 2-step RACH, thus FG 9-1 (2-step RACH) should be its prerequisite. |
| Apple [15] |  |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] |  |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-2-4 | Support of a PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA | Support of cross-TRP PDCCH order based on CFRA for inter-cell [WA: and intra-cell] multi-DCI based mTRP |  | yes | N/A |  | [Per band] | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-2-4 - The Cross TRP PDCCH order based on CFRA for intra-cell is agreed in the Ran1 114bis. We recommend removing the text ”WA” Agreement(RAN1#114bis Xiamen) The following working assumption is confirmed - “For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement, support the case where a PDCCH order sent by TRPX triggers RACH procedure towards either TRPX or TRPY.” Above confirmation does not change power control for the same TRP PDCCH order.. 40. NR_MIMO_evo_DL_UL; 40-2-4; Support of a PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA; Support of cross-TRP PDCCH order based on CFRA for inter-cell [WA: and intra-cell] multi-DCI based mTRP; yes; N/A; [Per band]; No; No; N/A; Optional with capability signaling. |
| Ericsson [3] | - In RAN#114bis, RAN1 made the following agreement Agreement The following working assumption is confirmed - “For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement, support the case where a PDCCH order sent by TRPX triggers RACH procedure towards either TRPX or TRPY.” Above confirmation does not change power control for the same TRP PDCCH order. - Hence, cross-TRP PRACH triggering is supported both for intra-cell and inter-cell. In our view, different FGs are needed for the two cases. We propose to described the intra-cell case in FG 40-2-4 and the inter-cell case in FG 40-2-4a. - In this FG, the type is in brackets. We propose to follow the RAN2 guidance in [1], and specify this per band. 40-2-4; Support of a PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different intra-cell TRP based on CFRA; Support of cross-TRP PDCCH order based on CFRA for intra-cell multi-DCI based mTRP; Intra-cell cross-TRP PDCCH ordered PRACH transmission is not supported; Per band. |
| ZTE [4] | - For FG 40-2-4, - Its prerequisite should at least include FG 40-2-2, and FG40-2-1 can be pending once the WA for the case of intra-cell operation is confirmed. - The following components should be added to capture the agreements endorsed for basic features of cross-TRP PDCCH order based CFRA: - PDCCH scheduling RAR is always received from serving cell - Support of one additional PRACH configuration is supported for each configured additional PCI different from the PCI of the serving cell when multi-DCI based intra-cell Multi-TRP operation - Granularity should be “Per band” to keep alignment with that of its prerequisite FG 40-2-2. 40. NR_MIMO_evo_DL_UL; 40-2-4; Support of a PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA; - Support of cross-TRP PDCCH order based on CFRA for inter-cell [WA: and intra-cell] multi-DCI based mTRP - PDCCH scheduling RAR is always received from serving cell - Support of one additional PRACH configuration is supported for each configured additional PCI different from the PCI of the serving cell when multi-DCI based intra-cell Multi-TRP operation; [40-2-1], 40-2-2; yes; N/A; [Per band] Per band; No; No; N/A; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-2-4; Support of a PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA; Support of cross-TRP PDCCH order based on CFRA for inter-cell and intra-cell multi-DCI based mTRP; yes; N/A; [Per band]; No; No; N/A; Optional with capability signaling. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-2-4; Support of a PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA; Support of cross-TRP PDCCH order based on CFRA for inter-cell [WA: and intra-cell ] multi-DCI based mTRP; yes; N/A; [Per band]; No; No; N/A; Optional with capability signaling. |
| OPPO [8] | 40. NR_MIMO_evo_DL_UL; 40-2-4; Support of a PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA; Support of cross-TRP PDCCH order based on CFRA for inter-cell [WA: and intra-cell] multi-DCI based mTRP FFS: merge with 40-2-2 FFS: inter-/intra-cell; yes; N/A; [Per band]; No; No; N/A; Optional with capability signaling. |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-2-4; Support of a PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA; Support of cross-TRP PDCCH order based on CFRA for inter-cell [WA: and intra-cell] multi-DCI based mTRP; 40-2-1, 40-2-2; yes; N/A; [Per band]; No; No; N/A; Optional with capability signaling. |
| LG Electronics [11] | 40-2-4 - Remove square bracket for intra-cell multi-DCI based mTRP, since the following WA had been confirmed in RAN1 #114bis as below. Agreement The following working assumption is confirmed - “For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement, support the case where a PDCCH order sent by TRPX triggers RACH procedure towards either TRPX or TRPY.” Above confirmation does not change power control for the same TRP PDCCH order.. Proposal 6: Remove square bracket for intra-cell multi-DCI based mTRP in 40-2-4. 40. NR_MIMO_evo_DL_UL; 40-2-4; Support of a PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA; Support of cross-TRP PDCCH order based on CFRA for inter-cell and intra-cell multi-DCI based mTRP; yes; N/A; [Per band]; No; No; N/A; Optional with capability signaling. |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | FG 40-2-4 - Component: WA of cross TRP PDCCH order RACH for intra-cell M-TRP was confirmed. “[WA: and intra-cell]” should be replaced by “and intra-cell”. - Prerequisite FG: FG 40-2-1 or 40-2-2 should be added to prerequisite FG. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-2-4; Support of a PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA; Support of cross-TRP PDCCH order based on CFRA for inter-cell [WA: and intra-cell] multi-DCI based mTRP; yes; N/A; Per band; No; No; N/A; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | Agreement The following working assumption is confirmed - “For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement, support the case where a PDCCH order sent by TRPX triggers RACH procedure towards either TRPX or TRPY.” Above confirmation does not change power control for the same TRP PDCCH order. Lastly, for FG 40-2-4, given the following agreement in RAN1 #114bis, which confirms the WA for intra-cell, it is better to split it to 40-2-4a and 40-2-4b for intra-cell and inter-cell separately: 40. NR_MIMO_evo_DL_UL; 40-2-4b; Support of a PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA for inter-cell; Support of cross-TRP PDCCH order based on CFRA for inter-cell multi-DCI based mTRP; yes; N/A; Per band; No; No; N/A; Optional with capability signaling. |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-2-4; Support of a PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA; Support of cross-TRP PDCCH order based on CFRA for inter-cell [WA: and intra-cell] multi-DCI based mTRP; yes; N/A; [Per band]; No; No; N/A; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-2-5 | TCI states of one coresetPoolIndex associated to both TAGs | Support of UL/joint TCI states of UL signals/channels associated to one coresetPoolIndex correspond to both TAGs This is a WA | FG 40-2-1 or 40-2-2 | yes | n/a |  | [Per band] | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-2-5 - Based on the agreements below the” working assumption” is not confirmed. Thus, This FG needs to be removed. Agreement (RAN1#112 Athens) For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, support the following: Associate TAG to TCI-state - Associate TAG ID with UL/joint TCI state - For UL transmission, the TAG ID associated with the UL/joint TCI state is utilized - A baseline is UE expects that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESET Pool Index correspond to one TAG - Working Assumption: A UE may report that it supports that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESETPoolIndex correspond to both TAGs FFS: on how to handle association when Rel-15/16 spatial relation framework is used for - PUCCH - DG/CG Type 1/Type 2 PUSCH - AP/SP/P SRS Agreement (RAN1#113 Incheon) For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, the baseline feature is revised as follows: - UE expects that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESET Pool Index correspond to one TAG - Association of TAG ID with UL/joint TCI state is via RRC configuration - Above does not impact the association of the indicated TCI states and coresetPoolIndex values as agreed in previous meetings in 9.1.1.1.. 40. NR_MIMO_evo_DL_UL; 40-2-5; TCI states of one coresetPoolIndex associated to both TAGs; Support of UL/joint TCI states of UL signals/channels associated to one coresetPoolIndex correspond to both TAGs This is a WA; FG 40-2-1 or 40-2-2; yes; n/a; [Per band]; n/a; n/a; n/a; Optional with capability signalling. |
| Ericsson [3] | - In this FG, the type is in brackets. We propose to follow the RAN2 guidance in [1], and specify this per band. 40-2-5; TCI states of one coresetPoolIndex associated to both TAGs; Support of UL/joint TCI states of UL signals/channels associated to one coresetPoolIndex correspond to both TAGs This is a WA; FG 40-2-1 or 40-2-2; All UL channels associated to one coresetPoolIndex must correspond to one TAG; Per band. |
| ZTE [4] |  |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-2-5; TCI states of one coresetPoolIndex associated to both TAGs; Support of UL/joint TCI states of UL signals/channels associated to one coresetPoolIndex correspond to both TAGs This is a WA; FG 40-2-1 or 40-2-2; yes; n/a; [Per band]; n/a; n/a; n/a; Optional with capability signalling. |
| OPPO [8] | For FG 40-2-5, since the WA on associating one CORESETPoolIndex to 2 TAGs is still pending, we suggest to hold the discussion on it until RAN1 makes final decision. |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-2-5; TCI states of one coresetPoolIndex associated to both TAGs; Support of UL/joint TCI states of UL signals/channels associated to one coresetPoolIndex correspond to both TAGs This is a WA; FG 40-2-1 or 40-2-2; yes; n/a; [Per band]; n/a; n/a; n/a; Optional with capability signalling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | FG 40-2-5 - Component: This WA has not been confirmed in RAN1. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-2-5; TCI states of one coresetPoolIndex associated to both TAGs; Support of UL/joint TCI states of UL signals/channels associated to one coresetPoolIndex correspond to both TAGs This is a WA; FG 40-2-1 or 40-2-2; yes; n/a; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] |  |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-2-5; TCI states of one coresetPoolIndex associated to both TAGs; Support of UL/joint TCI states of UL signals/channels associated to one coresetPoolIndex correspond to both TAGs This is a WA; FG 40-2-1 or 40-2-2; yes; n/a; [Per band]; n/a; n/a; n/a; Optional with capability signalling. |

| 40. NR_MIMO_evo_DL_UL | 40-2-6 | Rx timing difference larger than CP length |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-2-6; Rx timing difference larger than CP length; yes; N/A; [Per band]; n/a; n/a; N/A; Optional with capability signaling. |
| Ericsson [3] | - A description is missing. - Regarding the type, we propose to follow the RAN2 guidance in [1], and specify this per band. 40-2-6; Rx timing difference larger than CP length; Support reception with Rx timing difference larger than the CP; Rx timing difference larger than CP is not supported; Per band. |
| ZTE [4] | - For FG 40-2-6, - The following components should be added: - Maximum value of Rx timing difference - Its prerequisite should include FG 40-2-1, FG 40-2-2. 40. NR_MIMO_evo_DL_UL; 40-2-6; Rx timing difference larger than CP length; Maximum value of Rx timing difference; 40-2-1, 40-2-2. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] | For FG 40-2-6, i.e. Two TAs for multi-DCI STxMP PUSCH+PUSCH, we think it should be kept as a separate row. In UE feature agenda of STxMP, UE reports its support of fully overlapped, partially overlapped or non-overlapped M-DCI based PUSCH + PUSCH. Once a UE reports fully overlapped PUSCH + PUSCH, then this UE may be able to only apply a single TA for both PUSCH. From this sense, FG 40-2-6 should be kept as a separate feature. |
| CATT [9] | 40-2-6: - According to the following RAN1 agreement, as an optional UE capability, Rx timing difference between the two DL reference timings can be assumed to be larger than CP length. Agreement For multi-DCI multi-TRP operation with two TAs in a CC, two DL reference timings are supported where each DL reference timing is associated with one TAG - baseline assumption is that the Rx timing difference between the two DL reference timings is no larger than CP length - as an optional UE capability, Rx timing difference between the two DL reference timings can be assumed to be larger than CP length. Proposal 5: Revise the feature 40-2-6 as follows: 40. NR_MIMO_evo_DL_UL; 40-2-6; Rx timing difference larger than CP length; Support the Rx timing difference between the two DL reference timings to be larger than CP length; 40-2-1, 40-2-2; n/a; n/a; n/a; n/a; Optional with capability signalling. |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-2-6; Rx timing difference larger than CP length; 1. Support of the Rx timing difference between the two DL reference timings is larger than CP length.; 40-2-1, 40-2-2; yes; n/a; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | FG 40-2-6 - Prerequisite FG: FG 40-2-1 or 40-2-2 should be added to prerequisite FG. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-2-6; Rx timing difference larger than CP length; - Support of maximum Rx timing difference between the two DL reference timings larger than CP; Per FS; Optional with capability signalling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-2-6; Rx timing difference larger than CP length; Maximum receive timing difference between the DL transmissions from two TRPs can exceed a CP for multi-DCI based mTRP operation; 16-2a; Yes; N/A; FSPC; No; No; N/A; Optional with capability signaling. |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-2-6; Rx timing difference larger than CP length. |

| 40. NR_MIMO_evo_DL_UL | 40-2-7 | Two TAs for multi-DCI STxMP PUSCH+PUSCH | Support of two TAs for multi-DCI STxMP PUSCH+PUSCH | [40-2-1 or 40-2-2, FFS more] | yes | n/a |  | [Per band] | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-2-7; Two TAs for multi-DCI STxMP PUSCH+PUSCH; Support of two TAs for multi-DCI STxMP PUSCH+PUSCH; [40-2-1 or 40-2-2, FFS more]; yes; n/a; [Per band]; n/a; n/a; n/a; Optional with capability signaling. |
| Ericsson [3] | - In this FG, the type is in brackets. We propose to follow the RAN2 guidance in [1], and specify this per band. - The pre-requisites are in brackets. We propose to remove the brackets around “FG 40-2-1 or 40-2-2”, while leaving the FFS, until the STxMP FGs stabilize. 40-2-7; Two TAs for multi-DCI STxMP PUSCH+PUSCH; Support of two TAs for multi-DCI STxMP PUSCH+PUSCH; [40-2-1 or 40-2-2, FFS more]; Two TA enhancement is not supported for STxMP; Per band. |
| ZTE [4] |  |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-2-7; Two TAs for multi-DCI STxMP PUSCH+PUSCH; Support of two TAs for multi-DCI STxMP PUSCH+PUSCH; [40-2-1 or 40-2-2, FFS more]; yes; n/a; [Per band]; n/a; n/a; n/a; Optional with capability signalling. |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-2-7; Two TAs for multi-DCI STxMP PUSCH+PUSCH; Support of two TAs for multi-DCI STxMP PUSCH+PUSCH; [40-2-1 or 40-2-2, FFS more]; yes; n/a; [Per band]; n/a; n/a; n/a; Optional with capability signalling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | FG 40-2-7 - Prerequisite FG: FG 40-6-3a or 40-6-3b (CB or NCB M-DCI STxMP) should be added to prerequisite FG. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-2-7; Two TAs for multi-DCI STxMP PUSCH+PUSCH; Support of two TAs for multi-DCI STxMP PUSCH+PUSCH; [40-2-1 or 40-2-2, FFS more]; yes; n/a; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| Samsung [16] | The overlap between UL transmissions with different TAs can have any duration. Therefore, we suggest to add a note for FG 40-2-7: “support of any overlap duration with no reduction of any of UL transmissions for two UL transmissions in multi-DCI based Multi-TRP with 2 TA” Proposal 7: Add the following note to FG 40-2-7: “support of any overlap duration with no reduction of any of UL transmissions for two UL transmissions in multi-DCI based Multi-TRP with 2 TA” |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | - Regarding pre-requisite for FG 40-2-7, it should be “40-6-3a or 40-6-3b” (support of multi-DCI STxMP feature itself). - Furthermore, it is better to add the following note to FGs 40-2-1, 40-2-2, and 40-2-7: “Note: Rx timing difference between the two DL reference timings associated with the two TAGs is within a CP”. This makes it more clear / explicit that Rx timing difference exceeding the CP can be assumed only if FG 40-2-6 is supported. 40. NR_MIMO_evo_DL_UL; 40-2-7; Two TAs for multi-DCI STxMP PUSCH+PUSCH; Support of two TAs for multi-DCI STxMP PUSCH+PUSCH; 40-6-3a or 40-6-3b; yes; n/a; FSPC; n/a; n/a; n/a; Note: Rx timing difference between the two DL reference timings associated with the two TAGs is within a CP; Optional with capability signalling. |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-2-7; Two TAs for multi-DCI STxMP PUSCH+PUSCH; Support of two TAs for multi-DCI STxMP PUSCH+PUSCH; [40-2-1 or 40-2-2, FFS more] 40-6-3a; yes; n/a; [Per band]; n/a; n/a; n/a; Optional with capability signalling. |

| 40. NR_MIMO_evo_DL_UL | 40-2-8 | The maximum number of TAGs all CCs |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-2-8 - The maximum number of TAGs all CCs. The candidate values are added in notes column based on legacy values from the specification. In addition, RAN2 also assumes the current 4 TAGs per cell group is sufficient to support Rel-18 mDCI mTRP with 2 TAs 40. NR_MIMO_evo_DL_UL; 40-2-8; The maximum number of TAGs all CCs; 40-2-1; yes; n/a; Per band; n/a; n/a; n/a; The candidate values are {2,3,4}; Optional with capability signaling. |
| Ericsson [3] | - We propose that this is captured as a component in FG 40-2-1 and FG 40-2-2. Then FG 40-2-8 can be removed. |
| ZTE [4] | - For FG 40-2-8, - Its prerequisite should include FG 40-2-1, FG 40-2-2. 40. NR_MIMO_evo_DL_UL; 40-2-8; The maximum number of TAGs all CCs; 40-2-1, 40-2-2. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-2-8; The maximum number of TAGs all CCs; 1. Supported maximum number of TAGs per cell group.; 40-2-1, 40-2-2; yes; n/a; Per band; n/a; n/a; n/a; Component 1 candidate values: {2,3,4}; Optional with capability signalling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | FG 40-2-8 - Component: maximum number of TAGs across all CCs. - Candidate value: {2,3,4} - Prerequisite: 40-2-1 or 40-2-2 |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-2-8; The maximum number of TAGs all CCs; - Maximum number of TAGs that can be supported across all intra-frequency and inter-frequency CCs.; Per BC; Component 1 candidate value: {2, 3, 4[, 5, 6, 7, 8]} Note: the UE only supports the configuration where all UL CCs of the same frequency band are configured with up to 2 Timing Advance Group ID.; Optional with capability signalling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | - For FG 40-2-8, the same description of legacy FG “supportedNumberTAG” in 38.306 applies to this FG as well, with the difference that FG 40-2-8 is applicable when at least one CC is configured with 2 TAGs. 40. NR_MIMO_evo_DL_UL; 40-2-8; The maximum number of TAGs across all CCs; Supported maximum number of TAGs across all CCs when at least one CC is configured with 2 TAGs.; Yes; N/A; Band combination; No; No; N/A; {2,3,4} Note: The same description of “supportedNumberTAG” in 38.306 applies to this FG as well.; Optional with capability signaling. |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-2-8; The maximum number of TAGs all CCs; - The maximum number of TAGs all CCs; 40-2-1 or 40-2-2; yes; n/a; Per band; n/a; n/a; n/a; Component 1 candidate values: {1,2,3,4}; Optional with capability signalling. |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-1 | Support of mode 2 for Rel-16-based CJT type-II codebook | Support of N=N_TRP only Support of N_L=1 only 1. Support of mode 2 for Rel-16 eType-II codebook refinement for multi-TRP CJT 2. Support for PMI subband R=1. 3. Support of parameter combinations with L=2,4 4. Support of rank 1,2 5. A list of supported combinations, up to 16, [across all CCs] simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT] 6. Supported frequency basis selection mode 2, i.e., common frequency basis selection among different TRPs | [2-35] | Yes | N/A |  | Per band and Per BC | No | N/A | N/A | FFS: CPU occupation Component 5 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} Note: A-CSI is supported, and whether UE supports SP-CSI on PUSCH is dependent on FG2-32b Note: A UE that supports CSI enhancement for Rel. 16 based type-II CJT must support this FG | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-3-1-1; Support of mode 2 for Rel-16-based CJT type-II codebook; Support of N=N_TRP only Support of N_L=1 only 1. Support of mode 2 for Rel-16 eType-II codebook refinement for multi-TRP CJT 2. Support for PMI subband R=1. 3. Support of parameter combinations with L=2,4 4. Support of rank 1,2 5. A list of supported combinations, up to 16, [across all CCs] simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT] 6. Supported frequency basis selection mode 2, i.e., common frequency basis selection among different TRPs; [2-35]; Yes; N/A; Per band and Per BC; No; N/A; N/A; FFS: CPU occupation Component 5 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} Note: A-CSI is supported, and whether UE supports SP-CSI on PUSCH is dependent on FG2-32b Note: A UE that supports CSI enhancement for Rel. 16 based type-II CJT must support this FG; Optional with capability signaling. |
| Ericsson [3] |  |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-3-1-1; Support of mode 2 for Rel-16-based CJT type-II codebook; Support of N=N_TRP only Support of N_L=1 only 1. Support of mode 2 for Rel-16 eType-II codebook refinement for multi-TRP CJT 2. Support for PMI subband R=1. 3. Support of parameter combinations with L=2,4 4. Support of rank 1,2 5. A list of supported combinations, up to 16, [across all CCs] simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT] 6. Supported frequency basis selection mode 2, i.e., common frequency basis selection among different TRPs 7. Value of X for CPU occupation (OCPU = X.NTRP); [2-35]; Yes; N/A; Mode 2 for Rel-16-based CJT type-II codebook is not supported; Per band and Per BC; No; N/A; N/A; FFS: CPU occupation Component 5 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} Component 7 candidate values: {1, 1.5, 2} Note: A-CSI is supported, and whether UE supports SP-CSI on PUSCH is dependent on FG2-32b Note: A UE that supports CSI enhancement for Rel. 16 based type-II CJT must support this FG Note: When NTRP=1 is configured, legacy OCPU applies, i.e. OCPU =1; Optional with capability signaling. |
| Fujitsu [5] | 40-3-1-1; Support of mode 2 for Rel-16-based CJT type-II codebook; Support of N=N_TRP only Support of N_L=1 only 1. Support of mode 2 for Rel-16 eType-II codebook refinement for multi-TRP CJT 2. Support for PMI subband R=1. 3. Support of parameter combinations with L=2,4 4. Support of rank 1,2 5. A list of supported combinations, up to 16, [across all CCs] simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT] 6. Supported frequency basis selection mode 2, i.e., common frequency basis selection among different TRPs; [2-35]; FFS: CPU occupation Component 5 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} Note: A-CSI is supported, and whether UE supports SP-CSI on PUSCH is dependent on FG2-32b Note: A UE that supports CSI enhancement for Rel. 16 based type-II CJT must support this FG. |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-3-1-1; Support of mode 2 for Rel-16-based CJT type-II codebook; Support of N=N_TRP only Support of N_L=1 only 1. Support of mode 2 for Rel-16 eType-II codebook refinement for multi-TRP CJT 2. Support for PMI subband R=1. 3. Support of parameter combinations with L=2,4 4. Support of rank 1,2 5. A list of supported combinations, up to 16, [across all CCs] simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT] 6. Supported frequency basis selection mode 2, i.e., common frequency basis selection among different TRPs; [2-35]; Yes; N/A; Per band and Per BC; No; N/A; N/A; FFS: CPU occupation Component 5 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} Note: A-CSI is supported, and whether UE supports SP-CSI on PUSCH is dependent on FG2-32b Note: A UE that supports CSI enhancement for Rel. 16 based type-II CJT must support this FG; Optional with capability signaling. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-3-1-1; Support of mode 2 for Rel-16-based CJT type-II codebook; Support of N=N_TRP only Support of N_L=1 only 1. Support of mode 2 for Rel-16 eType-II codebook refinement for multi-TRP CJT 2. Support for PMI subband R=1. 3. Support of parameter combinations with L=2,4 4. Support of rank 1,2 5. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT 6. Supported frequency basis selection mode 2, i.e., common frequency basis selection among different TRPs 7. Value of X for CPU occupation (OCPU = X.N_TRP),; 2-35; Yes; N/A; Per band and Per BC; No; N/A; N/A; Component 5 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} Component 6 candidate values: {1, 1.5, 2} Note: A-CSI is supported, and whether UE supports SP-CSI on PUSCH is dependent on FG2-32b Note: A UE that supports CSI enhancement for Rel. 16 based type-II CJT must support this FG; Optional with capability signaling. |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-3-1-1; Support of mode 2 for Rel-16-based CJT type-II codebook; Support of N=N_TRP only Support of N_L=1 only 1. Support of mode 2 for Rel-16 eType-II codebook refinement for multi-TRP CJT 2. Support for PMI subband R=1. 3. Support of parameter combinations with L=2,4 4. Support of rank 1,2 5. A list of supported combinations, up to 16, [across all CCs] simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT] 6. Supported frequency basis selection mode 2, i.e., common frequency basis selection among different TRPs; [2-35], 16-3a; Yes; N/A; Per band and Per BC; No; N/A; N/A; FFS: CPU occupation Component 5 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} Note: A-CSI is supported, and whether UE supports SP-CSI on PUSCH is dependent on FG2-32b Note: A UE that supports CSI enhancement for Rel. 16 based type-II CJT must support this FG; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] | In RAN1#114 meeting [1], we have the following agreement on the CPU occupation for Rel-18 CJT codebook: Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding the CPU occupation, - X is a “common value” and not dependent on NTRP or any other parameter value, reported by the UE from a set of candidate values {1, 1.5, 2} according to UE capability. So, we suggest that the component of value of X for CPU occupation should be introduced for FG 40-3-1-1/40-3-1-5 and the candidate value could be {1, 1.5, 2}: Proposal 3: For FG 40-3-1-1, support to introduce the component of value of X for CPU occupation: 40. NR_MIMO_evo_DL_UL; 40-3-1-1; Support of mode 2 for Rel-16-based CJT type-II codebook; Support of N=N_TRP only Support of N_L=1 only 1. Support of mode 2 for Rel-16 eType-II codebook refinement for multi-TRP CJT 2. Support for PMI subband R=1. 3. Support of parameter combinations with L=2,4 4. Support of rank 1,2 5. A list of supported combinations, up to 16, [across all CCs] simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT] 6. Supported frequency basis selection mode 2, i.e., common frequency basis selection among different TRPs The value of X for CPU occupation; [2-35]; Candidate value X for CPU occupation {1, 1.5, 2} Component 5 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} Note: A-CSI is supported, and whether UE supports SP-CSI on PUSCH is dependent on FG2-32b Note: A UE that supports CSI enhancement for Rel. 16 based type-II CJT must support this FG. |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-3-1-1 - Prerequisite FG: OK with 2-35. - Component: - OK with ‘across all CCs’. - Support to add CPU occupation with the max value as a component of the basic feature. And a note should be added to clarify that when NTRP=1 is configured, legacy OCPU applies, i.e. OCPU =1. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-3-1-1; Support of mode 2 for Rel-16-based CJT type-II codebook; Support of N=N_TRP only Support of N_L=1 only 1. Support of mode 2 for Rel-16 eType-II codebook refinement for multi-TRP CJT 2. Support for PMI subband R=1. 3. Support of parameter combinations with L=2,4 4. Support of rank 1,2 5. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT] 6. Supported frequency basis selection mode 2, i.e., common frequency basis selection among different TRPs; [2-35]; Yes; N/A; Per band and Per BC; No; N/A; N/A; FFS: CPU occupation Component 5 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} Note: A-CSI is supported, and whether UE supports SP-CSI on PUSCH is dependent on FG2-32b Note: A UE that supports CSI enhancement for Rel. 16 based type-II CJT must support this FG; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] | 40. NR_MIMO_evo_DL_UL; 40-3-1-1; Support of mode 2 for Rel-16-based CJT type-II codebook; - Value of X for CPU occupation (OCPU = X.NTRP) when NTRP > 1 - Capability index on CSI computation time; Component 1 candidate values {1,1.5,2} Component 2 candidate values {1,2}; Optional with capability signalling. |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-3-1-1; Support of mode 2 for Rel-16-based CJT type-II codebook; Support of N=N_TRP only Support of N_L=1 only 1. Support of mode 2 for Rel-16 eType-II based codebook refinement for multi-TRP CJT i.e., common frequency basis selection among different TRPs 2. Support for PMI subband R=1. 3. Support of parameter combinations with L=2,4 4. Support of rank 1,2 5. A list of supported parameter combinations: {1,2} with NTRP=1, and {1,2,3,4} with NTRP=2, up to 16, [across all CCs] simultaneously, where each combination is 6. Triplet {a,b,c}: a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT]\ 6. Supported frequency basis selection mode 2, i.e., common frequency basis selection among different TRPs 7, Value of X for CPU occupation (OCPU = X.NTRP) Note: A UE that supports CSI enhancement for Rel. 16 based type-II CJT must support this FG; [2-35]; Yes; N/A; Per band and Per BC; No; N/A; N/A; FFS: CPU occupation Component 7 candidate values:X={1, 1.5, 2} Component 5 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} Note: A-CSI is supported, and whether UE supports SP-CSI on PUSCH is dependent on FG2-32b; Optional with capability signaling. |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-1a | Support of mode 1 for Rel-16-based CJT type-II codebook with FD basis selection integer frequency offset | [1. Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with PMI subband R=1. 2. Support of parameter combinations with L=2,4 3. Support of rank 1,2] 4. A list of supported combinations, up to 16, [across all CCs] simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT] 5. Supported frequency basis selection mode 1, i.e., common frequency basis selection among different TRPs with FD basis selection integer frequency offset | 40-3-1-1 | Yes | N/A | Mode 1 for Rel-16-based CJT type-II codebook with FD basis selection integer frequency offset is not supported | Per band and Per BC | N/A | N/A | N/A | Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-3-1-1a - We recommend confirming the following “ - 1. Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with PMI subband R=1. - 2. Support of parameter combinations with L=2,4 - 3. Support of rank 1,2 40. NR_MIMO_evo_DL_UL; 40-3-1-1a; Support of mode 1 for Rel-16-based CJT type-II codebook with FD basis selection integer frequency offset; [1. Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with PMI subband R=1. 2. Support of parameter combinations with L=2,4 3. Support of rank 1,2] 4. A list of supported combinations, up to 16, [across all CCs] simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT] 5. Supported frequency basis selection mode 1, i.e., common frequency basis selection among different TRPs with FD basis selection integer frequency offset; 40-3-1-1; Yes; N/A; Mode 1 for Rel-16-based CJT type-II codebook with FD basis selection integer frequency offset is not supported; Per band and Per BC; N/A; N/A; N/A; Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256}; Optional with capability signaling. |
| Ericsson [3] |  |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-3-1-1a; Support of mode 1 for Rel-16-based CJT type-II codebook with FD basis selection integer frequency offset; [1. Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with PMI subband R=1. 2. Support of parameter combinations with L=2,4 3. Support of rank 1,2] 4. A list of supported combinations, up to 16, [across all CCs] simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT] 5. Supported frequency basis selection mode 1, i.e., common frequency basis selection among different TRPs with FD basis selection integer frequency offset; 40-3-1-1; Yes; N/A; Mode 1 for Rel-16-based CJT type-II codebook with FD basis selection integer frequency offset is not supported; Per band and Per BC; N/A; N/A; N/A; Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256}; Optional with capability signaling. |
| Fujitsu [5] | 40-3-1-1a; Support of mode 1 for Rel-16-based CJT type-II codebook with FD basis selection integer frequency offset; [1. Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with PMI subband R=1. 2. Support of parameter combinations with L=2,4 3. Support of rank 1,2] 4. A list of supported combinations, up to 16, [across all CCs] simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT] 5. Supported frequency basis selection mode 1, i.e., common frequency basis selection among different TRPs with FD basis selection integer frequency offset; 40-3-1-1; Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256}. |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-3-1-1a; Support of mode 1 for Rel-16-based CJT type-II codebook with FD basis selection integer frequency offset; [1. Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with PMI subband R=1. 2. Support of parameter combinations with L=2,4 3. Support of rank 1,2] 4. A list of supported combinations, up to 16, [across all CCs] simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT] 5. Supported frequency basis selection mode 1, i.e., common frequency basis selection among different TRPs with FD basis selection integer frequency offset; 40-3-1-1; Yes; N/A; Mode 1 for Rel-16-based CJT type-II codebook with FD basis selection integer frequency offset is not supported; Per band and Per BC; N/A; N/A; N/A; Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256}; Optional with capability signaling. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-3-1-1a; Support of mode 1 for Rel-16-based CJT type-II codebook with FD basis selection integer frequency offset; 4. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT] 5. Supported frequency basis selection mode 1, i.e., common frequency basis selection among different TRPs with FD basis selection integer frequency offset; 40-3-1-1; Yes; N/A; Mode 1 for Rel-16-based CJT type-II codebook with FD basis selection integer frequency offset is not supported; Per band and Per BC; N/A; N/A; N/A; Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256}; Optional with capability signaling. |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-3-1-1a - Component: support the yellow text, to align with 40-3-1-1. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-3-1-1a; Support of mode 1 for Rel-16-based CJT type-II codebook with FD basis selection integer frequency offset; 1. Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with PMI subband R=1. 2. Support of parameter combinations with L=2,4 3. Support of rank 1,2 4. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT] 5. Supported frequency basis selection mode 1, i.e., common frequency basis selection among different TRPs with FD basis selection integer frequency offset; 40-3-1-1; Yes; N/A; Mode 1 for Rel-16-based CJT type-II codebook with FD basis selection integer frequency offset is not supported; Per band and Per BC; N/A; N/A; N/A; Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256}; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-1-1b; Maximum number of TRPs supported for Rel-16-based CJT Type-II codebook; 1, Maximum number of TRPs supported for Rel-16-based CJT Type-II codebook; 40-3-1-1; Yes; N/A; Rel-16-based CJT type-II codebook with FD basis selection integer frequency offset is not supported; Per band and Per BC; N/A; N/A; N/A; Component 1 candidate values: {2,3,4}; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] | 40. NR_MIMO_evo_DL_UL; 40-3-1-1a; Support of mode 1 for Rel-16-based CJT type-II codebook with FD basis selection integer frequency offset; - Value of X for CPU occupation (OCPU = X.NTRP) when NTRP > 1 - Capability index on CSI computation time; Component 1 candidate values {1,1.5,2} Component 2 candidate values {1,2}; Optional with capability signalling. |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-3-1-1a; Support of mode 1 for Rel-16-based CJT type-II codebook with FD basis selection integer frequency offset; [1. Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with PMI subband R=1. 2. Support of parameter combinations with L=2,4 3. Support of rank 1,2] 4. A list of supported parameter combinations, {1,2} with NTRP=1, and {1,2,3,4} with NTRP=2,up to 16, [across all CCs] simultaneously, where each combination is 6. Triplet {a,b,c}: a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT]\ 5. Supported frequency basis selection mode 1, i.e., common frequency basis selection among different TRPs with FD basis selection integer frequency offset; 40-3-1-1; Yes; N/A; Mode 1 for Rel-16-based CJT type-II codebook with FD basis selection integer frequency offset is not supported; Per band and Per BC; N/A; N/A; N/A; Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256}; Optional with capability signaling. |
| Huawei/HiSilicon [19] | For CJT CSI codebook, similar to legacy Type I single panel codebook in FG 2-36, Type I multi panel codebook in FG 2-40, Type II codebook in FG 2-41 and etc., a list of supported capability combinations where each combination contains {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CC should be introduced. The gNB can configure multiple CSI measurement according to the above listed combinations. For example, if UE reports {16, 8, 128}, then gNB can configure CSI report #1 and CSI report #2 based on {16, 4, 64} and {16, 4, 64}, respectively; or the gNB can configure CSI report #1, CSI report #2, CSI report #3, and CSI report #4 based on {16, 2, 64}, {16, 2, 64}, {16, 2, 64} and {16, 2, 64}, respectively; or the gNB can configure CSI report #1, CSI report #2, and CSI report #3 based on {16, 2, 64}, {16, 2, 64}, {16, 4, 64}, respectively. However, it is quite complicated for UE preforming CJT CSI measurement on 4 CSI-RS resources than 2 CSI-RS resources. Therefore, to avoid the configuration of the base station exceeding UE capability, a list of supported combinations where each combination contains {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement should be introduced. In RAN1#114b, FGs for CJT CSI enhancement in [1] were agreed to be adopted. FG 40-3-1-1 and FG 4-3-1-1a for Rel-16-based CJT, and FG 40-3-1-5, FG 40-3-1-7, FG 40-3-1-8, and FG 40-3-1-5a for Rel-17-based CJT are endorsed However, whether each combination is across all CCs is FFS because it is quite complicated for UE performing one CJT CSI measurement. To avoid the configuration of the base station exceeding UE capability, we have the following two alternatives. - Alt 1: Introduce a list of combination for one CJT CSI measurement in addition to a list of combinations across all CC. - Alt 2: Similar to FG 2-36/FG 2-40, introduce maximum number of CSI-RS resource in a resource set Thus, we have the following Proposal: Proposal 3-1: adopt the either Alt1 or Alt 2 to avoid the configuration of one CJT CSI measurement exceeding UE capability. - Alt 1: Introduce a list of combination for one CJT CSI measurement in addition to a list of combinations across all CC, where each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement. - Alt 2: Similar to FG 2-36/FG 2-40, introduce maximum number of CSI-RS resource in a resource set 40. NR_MIMO_evo_DL_UL; 40-3-1-1a; Support of mode 1 for Rel-16-based CJT type-II codebook with FD basis selection integer frequency offset; [1. Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with PMI subband R=1. 2. Support of parameter combinations with L=2,4 3. Support of rank 1,2] 4. A list of supported combinations, up to 16, [across all CCs] simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT] 5. Supported frequency basis selection mode 1, i.e., common frequency basis selection among different TRPs with FD basis selection integer frequency offset 6. A list of supported combinations, up to 16, per measurement, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT; 40-3-1-1; Yes; N/A; Mode 1 for Rel-16-based CJT type-II codebook with FD basis selection integer frequency offset is not supported; Per band and Per BC; N/A; N/A; N/A; Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256}; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-2 | Support for FD basis selection fractional offset mode for Rel-16-based CJT codebook with mode1 | Supported frequency basis selection mode 1 with FD basis selection fractional frequency offset for Rel-16 eType-II based CJT codebook | 40-3-1-1a | Yes | N/A | FD basis selection fractional offset mode for Rel-16-based CJT codebook with mode 1 is not supported | Per band and Per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] |  |
| ZTE [4] |  |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-3-1-2; Support for FD basis selection fractional offset mode for Rel-16-based CJT codebook with mode1; Supported frequency basis selection mode 1 with FD basis selection fractional frequency offset for Rel-16 eType-II based CJT codebook; 40-3-1-1a; Yes; N/A; FD basis selection fractional offset mode for Rel-16-based CJT codebook with mode 1 is not supported; Per band and Per BC; N/A; N/A; N/A; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] | 40. NR_MIMO_evo_DL_UL; 40-3-1-2; Support for FD basis selection fractional offset mode for Rel-16-based CJT codebook with mode 1; - Value of X for CPU occupation (OCPU = X.NTRP) when NTRP > 1 - Capability index on CSI computation time; Component 1 candidate values {1,1.5,2} Component 2 candidate values {1,2}; Optional with capability signalling. |
| Qualcomm Incorporated [18] |  |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-3 | Support R=2 for Rel-16-based CJT codebook | 1. Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with PMI subbands R=2 2. {Max # of Tx ports in one resource set, Max # of resource sets, total # of Tx ports}, [across all CCs] simultaneously, with R=2 | 40-3-1-1 | Yes | N/A | R=2 for Rel-16-based CJT codebook is not supported | Per-band Per-BC | N/A | N/A | N/A | Component 2 candidate values: a) {4,8,12,16,24,32} b) {2 to 64} c) {4 to 256} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-3-1-3; Support R=2 for Rel-16-based CJT codebook; 1. Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with PMI subbands R=2 2. {Max # of Tx ports in one resource set, Max # of resource sets, total # of Tx ports}, [across all CCs] simultaneously, with R=2; 40-3-1-1; Yes; N/A; R=2 for Rel-16-based CJT codebook is not supported; Per-band Per-BC; N/A; N/A; N/A; Component 2 candidate values: a) {4,8,12,16,24,32} b) {2 to 64} c) {4 to 256}; Optional with capability signaling. |
| Ericsson [3] |  |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-3-1-3; Support R=2 for Rel-16-based CJT codebook; 1. Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with PMI subbands R=2 2. {Max # of Tx ports in one resource set, Max # of resource sets, total # of Tx ports}, [across all CCs] simultaneously, with R=2; 40-3-1-1; Yes; N/A; R=2 for Rel-16-based CJT codebook is not supported; Per-band Per-BC; N/A; N/A; N/A; Component 2 candidate values: a) {4,8,12,16,24,32} b) {2 to 64} c) {4 to 256}; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-3-1-3; Support R=2 for Rel-16-based CJT codebook; 1. Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with PMI subbands R=2 2. {Max # of Tx ports in one resource set, Max # of resource sets, total # of Tx ports}, [across all CCs] simultaneously, with R=2; 40-3-1-1; Yes; N/A; R=2 for Rel-16-based CJT codebook is not supported; Per-band Per-BC; N/A; N/A; N/A; Component 2 candidate values: a) {4,8,12,16,24,32} b) {2 to 64} c) {4 to 256}; Optional with capability signaling. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-3-1-3; Support R=2 for Rel-16-based CJT codebook; 1. Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with PMI subbands R=2 2. {Max # of Tx ports in one resource set, Max # of resource sets, total # of Tx ports}, across all CCssimultaneously, with R=2; 40-3-1-1; Yes; N/A; R=2 for Rel-16-based CJT codebook is not supported; Per-band Per-BC; N/A; N/A; N/A; Component 2 candidate values: a) {4,8,12,16,24,32} b) {2 to 64} c) {4 to 256}; Optional with capability signaling. |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-3-1-3/40-3-1-7/40-3-1-8 - Component: - OK with ‘across all CCs’. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-3-1-3; Support R=2 for Rel-16-based CJT codebook; 1. Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with PMI subbands R=2 2. {Max # of Tx ports in one resource set, Max # of resource sets, total # of Tx ports}, across all CCs simultaneously, with R=2; 40-3-1-1; Yes; N/A; R=2 for Rel-16-based CJT codebook is not supported; Per-band Per-BC; N/A; N/A; N/A; Component 2 candidate values: a) {4,8,12,16,24,32} b) {2 to 64} c) {4 to 256}; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-3-1-3; Support R=2 for Rel-16-based CJT codebook; 1. Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with PMI subbands R=2 2. {Max # of Tx ports in one resource set, Max # of resource sets, total # of Tx ports}, [across all CCs] simultaneously, with R=2; 40-3-1-1, or additionally, 40-3-1-1a; Yes; N/A; R=2 for Rel-16-based CJT codebook is not supported; Per-band Per-BC; N/A; N/A; N/A; Component 2 candidate values: a) {4,8,12,16,24,32} b) {2 to 64} c) {4 to 256}; Optional with capability signaling. |
| Huawei/HiSilicon [19] | For CJT CSI codebook, similar to legacy Type I single panel codebook in FG 2-36, Type I multi panel codebook in FG 2-40, Type II codebook in FG 2-41 and etc., a list of supported capability combinations where each combination contains {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CC should be introduced. The gNB can configure multiple CSI measurement according to the above listed combinations. For example, if UE reports {16, 8, 128}, then gNB can configure CSI report #1 and CSI report #2 based on {16, 4, 64} and {16, 4, 64}, respectively; or the gNB can configure CSI report #1, CSI report #2, CSI report #3, and CSI report #4 based on {16, 2, 64}, {16, 2, 64}, {16, 2, 64} and {16, 2, 64}, respectively; or the gNB can configure CSI report #1, CSI report #2, and CSI report #3 based on {16, 2, 64}, {16, 2, 64}, {16, 4, 64}, respectively. However, it is quite complicated for UE preforming CJT CSI measurement on 4 CSI-RS resources than 2 CSI-RS resources. Therefore, to avoid the configuration of the base station exceeding UE capability, a list of supported combinations where each combination contains {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement should be introduced. In RAN1#114b, FGs for CJT CSI enhancement in [1] were agreed to be adopted. FG 40-3-1-1 and FG 4-3-1-1a for Rel-16-based CJT, and FG 40-3-1-5, FG 40-3-1-7, FG 40-3-1-8, and FG 40-3-1-5a for Rel-17-based CJT are endorsed However, whether each combination is across all CCs is FFS because it is quite complicated for UE performing one CJT CSI measurement. To avoid the configuration of the base station exceeding UE capability, we have the following two alternatives. - Alt 1: Introduce a list of combination for one CJT CSI measurement in addition to a list of combinations across all CC. - Alt 2: Similar to FG 2-36/FG 2-40, introduce maximum number of CSI-RS resource in a resource set Thus, we have the following Proposal: Proposal 3-1: adopt the either Alt1 or Alt 2 to avoid the configuration of one CJT CSI measurement exceeding UE capability. - Alt 1: Introduce a list of combination for one CJT CSI measurement in addition to a list of combinations across all CC, where each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement. - Alt 2: Similar to FG 2-36/FG 2-40, introduce maximum number of CSI-RS resource in a resource set 40. NR_MIMO_evo_DL_UL; 40-3-1-3; Support R=2 for Rel-16-based CJT codebook; 1. Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with PMI subbands R=2 2. {Max # of Tx ports in one resource set, Max # of resource sets, total # of Tx ports}, [across all CCs] simultaneously, with R=2 3. {Max # of Tx ports in one resource set, Max # of resource sets, total # of Tx ports}, per measurement, with R=2; 40-3-1-1; Yes; N/A; R=2 for Rel-16-based CJT codebook is not supported; Per-band Per-BC; N/A; N/A; N/A; Component 2 candidate values: a) {4,8,12,16,24,32} b) {2 to 64} c) {4 to 256}; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-4 | Support pv={1/2,1/2,1/2,1/2} and beta=1/2 for Rel-16-based CJT codebook | 1. Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with parameter combination pv={1/2,1/2,1/2,1/2} and beta=1/2 | 40-3-1-1 | Yes | N/A | pv={1/2,1/2,1/2,1/2} and beta=1/2 for Rel-16-based CJT codebook are not supported | Per band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] |  |
| ZTE [4] |  |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-3-1-4; Support pv={1/2,1/2,1/2,1/2} and beta=1/2 for Rel-16-based CJT codebook; 1. Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with parameter combination pv={1/2,1/2,1/2,1/2} and beta=1/2; 40-3-1-1; Yes; N/A; pv={1/2,1/2,1/2,1/2} and beta=1/2 for Rel-16-based CJT codebook are not supported; Per band and per BC; N/A; N/A; N/A; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] |  |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-5 | Support of mode 2 for Rel-17-based CJT type-II codebook | Support of N=N_TRP only Support of N_L=1 only 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT 2. Support of PMI subband R=1. 3. Support of parameter combinations with M=1 4. Support of rank 1,2 5. A list of supported combinations, up to 16, [across all CCs] simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT 6. Supported frequency basis selection mode 2, i.e., common frequency basis selection among different TRPs | [2-35] | Yes | N/A |  | Per-band and Per-BC | No | N/A | N/A | FFS: CPU occupation Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} Note: A-CSI is supported, and whether UE supports SP-CSI on PUSCH is dependent on FG2-32b Note: A UE that supports CSI enhancement for Rel 17 based type-II CJT must support this FG | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] |  |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-3-1-5; Support of mode 2 for Rel-17-based CJT type-II codebook; Support of N=N_TRP only Support of N_L=1 only 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT 2. Support of PMI subband R=1. 3. Support of parameter combinations with M=1 4. Support of rank 1,2 5. A list of supported combinations, up to 16, [across all CCs] simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT 6. Supported frequency basis selection mode 2, i.e., common frequency basis selection among different TRPs 7. Value of X for CPU occupation (OCPU = X.NTRP); [2-35]; Yes; N/A; Per-band and Per-BC; No; N/A; N/A; FFS: CPU occupation Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} Component 7 candidate values: {1, 1.5, 2} Note: A-CSI is supported, and whether UE supports SP-CSI on PUSCH is dependent on FG2-32b Note: A UE that supports CSI enhancement for Rel 17 based type-II CJT must support this FG Note: When NTRP=1 is configured, legacy OCPU applies, i.e. OCPU =1; Optional with capability signaling. |
| Fujitsu [5] | 40-3-1-5; Support of mode 2 for Rel-17-based CJT type-II codebook; Support of N=N_TRP only Support of N_L=1 only 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT 2. Support of PMI subband R=1. 3. Support of parameter combinations with M=1 4. Support of rank 1,2 5. A list of supported combinations, up to 16, [across all CCs] simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT 6. Supported frequency basis selection mode 2, i.e., common frequency basis selection among different TRPs; [2-35]; FFS: CPU occupation Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} Note: A-CSI is supported, and whether UE supports SP-CSI on PUSCH is dependent on FG2-32b Note: A UE that supports CSI enhancement for Rel 17 based type-II CJT must support this FG. |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-3-1-5; Support of mode 2 for Rel-17-based CJT type-II codebook; Support of N=N_TRP only Support of N_L=1 only 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT 2. Support of PMI subband R=1. 3. Support of parameter combinations with M=1 4. Support of rank 1,2 5. A list of supported combinations, up to 16, [across all CCs] simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT 6. Supported frequency basis selection mode 2, i.e., common frequency basis selection among different TRPs; [2-35]; Yes; N/A; Per-band and Per-BC; No; N/A; N/A; FFS: CPU occupation Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} Note: A-CSI is supported, and whether UE supports SP-CSI on PUSCH is dependent on FG2-32b Note: A UE that supports CSI enhancement for Rel 17 based type-II CJT must support this FG; Optional with capability signaling. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-3-1-5; Support of mode 2 for Rel-17-based CJT type-II codebook; Support of N=N_TRP only Support of N_L=1 only 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT 2. Support of PMI subband R=1. 3. Support of parameter combinations with M=1 4. Support of rank 1,2 5. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT 6. Supported frequency basis selection mode 2, i.e., common frequency basis selection among different TRPs 7. Value of X for CPU occupation (OCPU = X.N_TRP),; 2-35; Yes; N/A; Per-band and Per-BC; No; N/A; N/A; Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} Component 7 candidate values: {1, 1.5, 2} Note: A-CSI is supported, and whether UE supports SP-CSI on PUSCH is dependent on FG2-32b Note: A UE that supports CSI enhancement for Rel 17 based type-II CJT must support this FG; Optional with capability signaling. |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-3-1-5; Support of mode 2 for Rel-17-based CJT type-II codebook; Support of N=N_TRP only Support of N_L=1 only 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT 2. Support of PMI subband R=1. 3. Support of parameter combinations with M=1 4. Support of rank 1,2 5. A list of supported combinations, up to 16, [across all CCs] simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT 6. Supported frequency basis selection mode 2, i.e., common frequency basis selection among different TRPs; [2-35], 23-9-1; Yes; N/A; Per-band and Per-BC; No; N/A; N/A; FFS: CPU occupation Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} Note: A-CSI is supported, and whether UE supports SP-CSI on PUSCH is dependent on FG2-32b Note: A UE that supports CSI enhancement for Rel 17 based type-II CJT must support this FG; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] | In RAN1#114 meeting [1], we have the following agreement on the CPU occupation for Rel-18 CJT codebook: Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding the CPU occupation, - X is a “common value” and not dependent on NTRP or any other parameter value, reported by the UE from a set of candidate values {1, 1.5, 2} according to UE capability. So, we suggest that the component of value of X for CPU occupation should be introduced for FG 40-3-1-1/40-3-1-5 and the candidate value could be {1, 1.5, 2}: Proposal 3: For FG 40-3-1-5, support to introduce the component of value of X for CPU occupation: 40. NR_MIMO_evo_DL_UL; 40-3-1-5; Support of mode 2 for Rel-17-based CJT type-II codebook; Support of N=N_TRP only Support of N_L=1 only 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT 2. Support of PMI subband R=1. 3. Support of parameter combinations with M=1 4. Support of rank 1,2 5. A list of supported combinations, up to 16, [across all CCs] simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT 6. Supported frequency basis selection mode 2, i.e., common frequency basis selection among different TRPs The value of X for CPU occupation; [2-35]; Candidate value X for CPU occupation {1, 1.5, 2} Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} Note: A-CSI is supported, and whether UE supports SP-CSI on PUSCH is dependent on FG2-32b Note: A UE that supports CSI enhancement for Rel 17 based type-II CJT must support this FG. |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-3-1-5 - Prerequisite FG: OK with 2-35. - Component: - Support to add CPU occupation with the max value as a component of the basic feature. |
| Apple [15] |  |
| Samsung [16] |  |
| MediaTek Inc. [17] | 40. NR_MIMO_evo_DL_UL; 40-3-1-5; Support of mode 2 for Rel-17-based CJT type-II codebook; - Value of X for CPU occupation (OCPU = X.NTRP) when NTRP > 1 - Capability index on CSI computation time; Component 1 candidate values {1,1.5,2} Component 2 candidate values {1,2}; Optional with capability signalling. |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-3-1-5; Support of mode 2 for Rel-17-based CJT type-II codebook; Support of N=N_TRP only Support of N_L=1 only 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT 2. Support of PMI subband R=1. 3. Support of parameter combinations with M=1 4. Support of rank 1,2 5. A list of supported parameter combinations, up to 16, [across all CCs] simultaneously, where each combination is 6. Triplet {a,b,c}: a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT 6. Supported frequency basis selection mode 2, i.e., common frequency basis selection among different TRPs 7, Value of X for CPU occupation (OCPU = X.NTRP) Note: A UE that supports CSI enhancement for Rel 17 based type-II CJT must support this FG; [2-35]; Yes; N/A; Per band and Per BC; No; N/A; N/A; FFS: CPU occupation Component 7 candidate values:X={1, 1.5, 2} Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} Note: A-CSI is supported, and whether UE supports SP-CSI on PUSCH is dependent on FG2-32b; Optional with capability signaling. |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-5a | Support of mode 1 for Rel-17-based CJT type-II codebook with FD basis selection integer frequency offset | 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with PMI subband R=1. 2. Support of parameter combinations with M=1 3. Support of rank 1,2 4. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT 5. Supported frequency basis selection mode 1, i.e., common frequency basis selection among different TRPs with FD basis selection integer frequency offset | 40-3-1-5 | Yes | N/A | Mode 1 for Rel-17-based CJT type-II codebook with FD basis selection integer frequency offset is not supported | Per band and per BC | N/A | N/A | N/A | Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] |  |
| ZTE [4] |  |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-3-1-5a; Support of mode 1 for Rel-17-based CJT type-II codebook with FD basis selection integer frequency offset; 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with PMI subband R=1. 2. Support of parameter combinations with M=1 3. Support of rank 1,2 4. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT 5. Supported frequency basis selection mode 1, i.e., common frequency basis selection among different TRPs with FD basis selection integer frequency offset; 40-3-1-5; Yes; N/A; Mode 1 for Rel-17-based CJT type-II codebook with FD basis selection integer frequency offset is not supported; Per band and per BC; N/A; N/A; N/A; Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256}; Optional with capability signaling. |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] |  |
| Samsung [16] |  |
| MediaTek Inc. [17] | 40. NR_MIMO_evo_DL_UL; 40-3-1-5a; Support of mode 1 for Rel-17-based CJT type-II codebook with FD basis selection integer frequency offset; - Value of X for CPU occupation (OCPU = X.NTRP) when NTRP > 1 - Capability index on CSI computation time; Component 1 candidate values {1,1.5,2} Component 2 candidate values {1,2}; Optional with capability signalling. |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-3-1-5a; Support of mode 1 for Rel-17-based CJT type-II codebook with FD basis selection integer frequency offset; 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with PMI subband R=1. 2. Support of parameter combinations with M=1 3. Support of rank 1,2 4. A list of supported parameter combinations, up to 16, across all CCs simultaneously, where each combination is 6. Triplet {a,b,c}: a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT 5. Supported frequency basis selection mode 1, i.e., common frequency basis selection among different TRPs with FD basis selection integer frequency offset; 40-3-1-5; Yes; N/A; Mode 1 for Rel-17-based CJT type-II codebook with FD basis selection integer frequency offset is not supported; Per band and per BC; N/A; N/A; N/A; Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256}; Optional with capability signaling. |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-6 | Support for FD basis selection fractional offset mode for Rel-17-based CJT codebook with mode1 | 1. Supported frequency basis selection mode 1 with FD basis selection fractional frequency offset for Rel-17 FeType-II port selection based CJT codebook | 40-3-1-5a | Yes | N/A | FD basis selection fractional offset mode for Rel-17-based CJT codebook with mode1 is not supported | Per band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] |  |
| ZTE [4] |  |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] |  |
| Samsung [16] |  |
| MediaTek Inc. [17] | 40. NR_MIMO_evo_DL_UL; 40-3-1-6; Support for FD basis selection fractional offset mode for Rel-17-based CJT codebook with mode1; - Value of X for CPU occupation (OCPU = X.NTRP) when NTRP > 1 - Capability index on CSI computation time; Component 1 candidate values {1,1.5,2} Component 2 candidate values {1,2}; Optional with capability signalling. |
| Qualcomm Incorporated [18] |  |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-7 | Support of M=2 and R=1 for Rel-17-based CJT codebook | 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with M=2 and PMI subband R=1 2. {Max # of Tx ports in one resource set, Max # of resources and total # of Tx ports}, [across all CCs] simultaneously, with M=2 and R=1 | 40-3-1-5 or 40-3-1-5a | Yes | N/A | M=2 and R=1 for Rel-17-based CJT codebook are not supported | Per band and per BC | N/A | N/A | N/A | Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-3-1-7; Support of M=2 and R=1 for Rel-17-based CJT codebook; 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with M=2 and PMI subband R=1 2. {Max # of Tx ports in one resource set, Max # of resources and total # of Tx ports}, [across all CCs] simultaneously, with M=2 and R=1; 40-3-1-5 or 40-3-1-5a; Yes; N/A; M=2 and R=1 for Rel-17-based CJT codebook are not supported; Per band and per BC; N/A; N/A; N/A; Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256}; Optional with capability signaling. |
| Ericsson [3] |  |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-3-1-7; Support of M=2 and R=1 for Rel-17-based CJT codebook; 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with M=2 and PMI subband R=1 2. {Max # of Tx ports in one resource set, Max # of resources and total # of Tx ports}, [across all CCs] simultaneously, with M=2 and R=1; 40-3-1-5 or 40-3-1-5a; Yes; N/A; M=2 and R=1 for Rel-17-based CJT codebook are not supported; Per band and per BC; N/A; N/A; N/A; Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256}; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-3-1-7; Support of M=2 and R=1 for Rel-17-based CJT codebook; 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with M=2 and PMI subband R=1 2. {Max # of Tx ports in one resource set, Max # of resources and total # of Tx ports}, [across all CCs] simultaneously, with M=2 and R=1; 40-3-1-5 or 40-3-1-5a; Yes; N/A; M=2 and R=1 for Rel-17-based CJT codebook are not supported; Per band and per BC; N/A; N/A; N/A; Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256}; Optional with capability signaling. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-3-1-7; Support of M=2 and R=1 for Rel-17-based CJT codebook; 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with M=2 and PMI subband R=1 2. {Max # of Tx ports in one resource set, Max # of resources and total # of Tx ports}, across all CCssimultaneously, with M=2 and R=1; 40-3-1-5 or 40-3-1-5a; Yes; N/A; M=2 and R=1 for Rel-17-based CJT codebook are not supported; Per band and per BC; N/A; N/A; N/A; Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256}; Optional with capability signaling. |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-3-1-3/40-3-1-7/40-3-1-8 - Component: - OK with ‘across all CCs’. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-3-1-7; Support of M=2 and R=1 for Rel-17-based CJT codebook; 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with M=2 and PMI subband R=1 2. {Max # of Tx ports in one resource set, Max # of resources and total # of Tx ports}, across all CCs simultaneously, with M=2 and R=1; 40-3-1-5 or 40-3-1-5a; Yes; N/A; M=2 and R=1 for Rel-17-based CJT codebook are not supported; Per band and per BC; N/A; N/A; N/A; Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256}; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-3-1-7; Support of M=2 and R=1 for Rel-17-based CJT codebook; 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with M=2 and PMI subband R=1 2. {Max # of Tx ports in one resource set, Max # of resources and total # of Tx ports}, [across all CCs] simultaneously, with M=2 and R=1; 40-3-1-5 or 40-3-1-5a; Yes; N/A; M=2 and R=1 for Rel-17-based CJT codebook are not supported; Per band and per BC; N/A; N/A; N/A; Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256}; Optional with capability signaling. |
| Huawei/HiSilicon [19] | For CJT CSI codebook, similar to legacy Type I single panel codebook in FG 2-36, Type I multi panel codebook in FG 2-40, Type II codebook in FG 2-41 and etc., a list of supported capability combinations where each combination contains {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CC should be introduced. The gNB can configure multiple CSI measurement according to the above listed combinations. For example, if UE reports {16, 8, 128}, then gNB can configure CSI report #1 and CSI report #2 based on {16, 4, 64} and {16, 4, 64}, respectively; or the gNB can configure CSI report #1, CSI report #2, CSI report #3, and CSI report #4 based on {16, 2, 64}, {16, 2, 64}, {16, 2, 64} and {16, 2, 64}, respectively; or the gNB can configure CSI report #1, CSI report #2, and CSI report #3 based on {16, 2, 64}, {16, 2, 64}, {16, 4, 64}, respectively. However, it is quite complicated for UE preforming CJT CSI measurement on 4 CSI-RS resources than 2 CSI-RS resources. Therefore, to avoid the configuration of the base station exceeding UE capability, a list of supported combinations where each combination contains {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement should be introduced. In RAN1#114b, FGs for CJT CSI enhancement in [1] were agreed to be adopted. FG 40-3-1-1 and FG 4-3-1-1a for Rel-16-based CJT, and FG 40-3-1-5, FG 40-3-1-7, FG 40-3-1-8, and FG 40-3-1-5a for Rel-17-based CJT are endorsed However, whether each combination is across all CCs is FFS because it is quite complicated for UE performing one CJT CSI measurement. To avoid the configuration of the base station exceeding UE capability, we have the following two alternatives. - Alt 1: Introduce a list of combination for one CJT CSI measurement in addition to a list of combinations across all CC. - Alt 2: Similar to FG 2-36/FG 2-40, introduce maximum number of CSI-RS resource in a resource set Thus, we have the following Proposal: Proposal 3-1: adopt the either Alt1 or Alt 2 to avoid the configuration of one CJT CSI measurement exceeding UE capability. - Alt 1: Introduce a list of combination for one CJT CSI measurement in addition to a list of combinations across all CC, where each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement. - Alt 2: Similar to FG 2-36/FG 2-40, introduce maximum number of CSI-RS resource in a resource set 40. NR_MIMO_evo_DL_UL; 40-3-1-7; Support of M=2 and R=1 for Rel-17-based CJT codebook; 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with M=2 and PMI subband R=1 2. {Max # of Tx ports in one resource set, Max # of resources and total # of Tx ports}, [across all CCs] simultaneously, with M=2 and R=1 3. {Max # of Tx ports in one resource set, Max # of resources and total # of Tx ports}, per measurement, with M=2 and R=1; 40-3-1-5 or 40-3-1-5a; Yes; N/A; M=2 and R=1 for Rel-17-based CJT codebook are not supported; Per band and per BC; N/A; N/A; N/A; Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256}; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-8 | Support R=2 for Rel-17-based CJT codebook | 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with PMI subband R=2. 2. {Max # of Tx ports in one resource set, Max # of resources and total # of Tx ports}, [across all CCs] simultaneously, with R=2 | 40-3-1-5 or 40-3-1-5a | Yes | N/A | R=2 for Rel-17-based CJT codebook is not supported | Per band and Per BC | N/A | N/A | N/A | Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-3-1-8; Support R=2 for Rel-17-based CJT codebook; 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with PMI subband R=2. 2. {Max # of Tx ports in one resource set, Max # of resources and total # of Tx ports}, [across all CCs] simultaneously, with R=2; 40-3-1-5 or 40-3-1-5a; Yes; N/A; R=2 for Rel-17-based CJT codebook is not supported; Per band and Per BC; N/A; N/A; N/A; Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256}; Optional with capability signaling. |
| Ericsson [3] |  |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-3-1-8; Support R=2 for Rel-17-based CJT codebook; 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with PMI subband R=2. 2. {Max # of Tx ports in one resource set, Max # of resources and total # of Tx ports}, [across all CCs] simultaneously, with R=2; 40-3-1-5 or 40-3-1-5a; Yes; N/A; R=2 for Rel-17-based CJT codebook is not supported; Per band and Per BC; N/A; N/A; N/A; Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256}; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-3-1-8; Support R=2 for Rel-17-based CJT codebook; 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with PMI subband R=2. 2. {Max # of Tx ports in one resource set, Max # of resources and total # of Tx ports}, [across all CCs] simultaneously, with R=2; 40-3-1-5 or 40-3-1-5a; Yes; N/A; R=2 for Rel-17-based CJT codebook is not supported; Per band and Per BC; N/A; N/A; N/A; Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256}; Optional with capability signaling. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-3-1-8; Support R=2 for Rel-17-based CJT codebook; 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with PMI subband R=2. 2. {Max # of Tx ports in one resource set, Max # of resources and total # of Tx ports}, across all CCssimultaneously, with R=2; 40-3-1-5 or 40-3-1-5a; Yes; N/A; R=2 for Rel-17-based CJT codebook is not supported; Per band and Per BC; N/A; N/A; N/A; Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256}; Optional with capability signaling. |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-3-1-3/40-3-1-7/40-3-1-8 - Component: - OK with ‘across all CCs’. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-3-1-8; Support R=2 for Rel-17-based CJT codebook; 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with PMI subband R=2. 2. {Max # of Tx ports in one resource set, Max # of resources and total # of Tx ports}, [across all CCs simultaneously, with R=2; 40-3-1-5 or 40-3-1-5a; Yes; N/A; R=2 for Rel-17-based CJT codebook is not supported; Per band and Per BC; N/A; N/A; N/A; Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256}; Optional with capability signaling. |
| Samsung [16] | 40. NR_MIMO_evo_DL_UL; 40-3-1-8; Support R=2 for Rel-17-based CJT codebook; 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with PMI subband R=2. 2. {Max # of Tx ports in one resource set, Max # of resources and total # of Tx ports}, [across all CCs] simultaneously, with R=2; 40-3-1-5 or 40-3-1-5a; Yes; N/A; R=2 for Rel-17-based CJT codebook is not supported; Per band and Per BC; N/A; N/A; N/A; Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256}; Optional with capability signaling. |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] |  |
| Huawei/HiSilicon [19] | For CJT CSI codebook, similar to legacy Type I single panel codebook in FG 2-36, Type I multi panel codebook in FG 2-40, Type II codebook in FG 2-41 and etc., a list of supported capability combinations where each combination contains {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CC should be introduced. The gNB can configure multiple CSI measurement according to the above listed combinations. For example, if UE reports {16, 8, 128}, then gNB can configure CSI report #1 and CSI report #2 based on {16, 4, 64} and {16, 4, 64}, respectively; or the gNB can configure CSI report #1, CSI report #2, CSI report #3, and CSI report #4 based on {16, 2, 64}, {16, 2, 64}, {16, 2, 64} and {16, 2, 64}, respectively; or the gNB can configure CSI report #1, CSI report #2, and CSI report #3 based on {16, 2, 64}, {16, 2, 64}, {16, 4, 64}, respectively. However, it is quite complicated for UE preforming CJT CSI measurement on 4 CSI-RS resources than 2 CSI-RS resources. Therefore, to avoid the configuration of the base station exceeding UE capability, a list of supported combinations where each combination contains {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement should be introduced. In RAN1#114b, FGs for CJT CSI enhancement in [1] were agreed to be adopted. FG 40-3-1-1 and FG 4-3-1-1a for Rel-16-based CJT, and FG 40-3-1-5, FG 40-3-1-7, FG 40-3-1-8, and FG 40-3-1-5a for Rel-17-based CJT are endorsed However, whether each combination is across all CCs is FFS because it is quite complicated for UE performing one CJT CSI measurement. To avoid the configuration of the base station exceeding UE capability, we have the following two alternatives. - Alt 1: Introduce a list of combination for one CJT CSI measurement in addition to a list of combinations across all CC. - Alt 2: Similar to FG 2-36/FG 2-40, introduce maximum number of CSI-RS resource in a resource set Thus, we have the following Proposal: Proposal 3-1: adopt the either Alt1 or Alt 2 to avoid the configuration of one CJT CSI measurement exceeding UE capability. - Alt 1: Introduce a list of combination for one CJT CSI measurement in addition to a list of combinations across all CC, where each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement. - Alt 2: Similar to FG 2-36/FG 2-40, introduce maximum number of CSI-RS resource in a resource set 40. NR_MIMO_evo_DL_UL; 40-3-1-8; Support R=2 for Rel-17-based CJT codebook; 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with PMI subband R=2. 2. {Max # of Tx ports in one resource set, Max # of resources and total # of Tx ports}, [across all CCs] simultaneously, with R=2 3. {Max # of Tx ports in one resource set, Max # of resources and total # of Tx ports}, per measurement simultaneously, with R=2; 40-3-1-5 or 40-3-1-5a; Yes; N/A; R=2 for Rel-17-based CJT codebook is not supported; Per band and Per BC; N/A; N/A; N/A; Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256}; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-9 | Support for 2NN1N2 >32 for Rel-16 based CJT codebook | Maximum number of ports across all TRPs for one CJT CSI measurement | 40-3-1-1 | Yes | N/A | 2NN1N2 >32 is not supported for Rel-16 based CJT codebook | Per band and Per BC | N/A | N/A | N/A | Component 1 candidate values: {64, 96, 128} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] |  |
| ZTE [4] |  |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] |  |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] |  |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-9a | Support for 2NN1N2 >32 for Rel-17 based CJT codebook | Maximum number of ports across all TRPs for one CJT CSI measurement | 40-3-1-5 | Yes | N/A | 2NN1N2 >32 is not supported for Rel-17 based CJT codebook | Per band and Per BC | N/A | N/A | N/A | Component 1 candidate values: {64, 96, 128} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] |  |
| ZTE [4] |  |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] |  |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] |  |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-11 | Active CSI-RS resources and ports for mixed codebook types including Type-II-CJT in any slot | 1. List of codebook combinations 2. List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination | 40-3-1-1,40-3-1-5,2-36 | Yes | N/A | Active CSI-RS resources and ports for mixed Type-II-CJT codebook types in any slot is not supported | [Per-band Per-BC/Per FS] | N/A | N/A | N/A | [Codebook 1 = {Type I SP, Type I MP} (Codebook 2, Codebook 3) = {(eType-II-CJT R=1, NULL), (eType-II-CJT R=2, NULL), (FeType-II-CJT PS R=1 M=1, NULL), (FeType-II-CJT PS R=1 M=2, NULL), (FeType-II-CJT PS R=2 M=2, NULL), (FFS CB combos…)}] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-3-1-11 - Candidate values for List of codebook combinations [Codebook 1 = {Type I SP, Type I MP} (Codebook 2, Codebook 3) = {(eType-II-CJT R=1, NULL), (eType-II-CJT R=2, NULL), (FeType-II-CJT PS R=1 M=1, NULL), (FeType-II-CJT PS R=1 M=2, NULL), (FeType-II-CJT PS R=2 M=2, NULL), (FFS CB combos…)}] - Confirm the legacy CBs and keep FFS for CB combos - Candidate values for max number of ports per resource, max number of resources, max number of total ports for each codebook combination - Maximum 16 triplets for each codebook combination - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} 40. NR_MIMO_evo_DL_UL; 40-3-1-11; Active CSI-RS resources and ports for mixed codebook types including Type-II-CJT in any slot; 1. List of codebook combinations 2. List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination; 40-3-1-1,40-3-1-5,2-36; Yes; N/A; Active CSI-RS resources and ports for mixed Type-II-CJT codebook types in any slot is not supported; [Per-band Per-BC/Per FS]; N/A; N/A; N/A; [Codebook 1 = {Type I SP, Type I MP} (Codebook 2, Codebook 3) = {(eType-II-CJT R=1, NULL), (eType-II-CJT R=2, NULL), (FeType-II-CJT PS R=1 M=1, NULL), (FeType-II-CJT PS R=1 M=2, NULL), (FeType-II-CJT PS R=2 M=2, NULL), (FFS CB combos…)}] Component 2 candidate values: - Maximum 16 triplets for each codebook combination - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}; Optional with capability signaling. |
| Ericsson [3] |  |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-3-1-11; Active CSI-RS resources and ports for mixed codebook types including Type-II-CJT in any slot; 1. List of codebook combinations 2. List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination; 40-3-1-1,40-3-1-5,2-36; Yes; N/A; Active CSI-RS resources and ports for mixed Type-II-CJT codebook types in any slot is not supported; [Per-band Per-BC/Per FS] Per band and Per BC; N/A; N/A; N/A; [Codebook 1 = {Type I SP, Type I MP} (Codebook 2, Codebook 3) = {(eType-II-CJT R=1, NULL), (eType-II-CJT R=2, NULL), (FeType-II-CJT PS R=1 M=1, NULL), (FeType-II-CJT PS R=1 M=2, NULL), (FeType-II-CJT PS R=2 M=2, NULL), (FFS CB combos…)}]; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-3-1-11; Active CSI-RS resources and ports for mixed codebook types including Type-II-CJT in any slot; 1. List of codebook combinations 2. List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination; 40-3-1-1,40-3-1-5,2-36; Yes; N/A; Active CSI-RS resources and ports for mixed Type-II-CJT codebook types in any slot is not supported; Per-band and Per BC; N/A; N/A; N/A; Codebook 1 = {Type I SP, Type I MP} (Codebook 2, Codebook 3) = {(eType-II-CJT R=1, NULL), (eType-II-CJT R=2, NULL), (FeType-II-CJT PS R=1 M=1, NULL), (FeType-II-CJT PS R=1 M=2, NULL), (FeType-II-CJT PS R=2 M=2, NULL) }; Optional with capability signaling. |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-3-1-11 - Type: OK with Per-band and per BC. In addition, We also support to add new FGs for following. And each of following can have separate rows for Rel-16 based enhancement and Rel-17 based enhancement respectively. - CPU occupation for N_TRP>1 CMRs for CJT CSI. - UE CSI processing time for N_TRP>1 CMRs for CJT CSI. 40. NR_MIMO_evo_DL_UL; 40-3-1-19; CPU counting for NTRP>1 for Rel-16-based CJT type-II codebook; - Value of X for CPU occupation (OCPU = X.NTRP); Yes; N/A; Per-band and per BC; No; N/A; N/A; Candidate values: {1,1.5,2}; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-1-20; CPU counting for NTRP>1 for Rel-17-based CJT type-II codebook; - Value of X for CPU occupation (OCPU = X.NTRP); Yes; N/A; Per-band and per BC; No; N/A; N/A; Candidate values: {1,1.5,2}; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-1-21; Minimum CSI calculation delay requirements for NTRP>1 for AP-CSI reporting for Rel-16-based CJT type-II codebook; - Supported minimum delay requirements; Yes; N/A; Per-band and per BC; No; N/A; N/A; Candidate values: {(Z2, Z'2), Z2+Z'2, 2Z'2)}; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-1-22; Minimum CSI calculation delay requirements for NTRP>1 for AP-CSI reporting for Rel-16-based CJT type-II codebook; - Supported minimum delay requirements; Yes; N/A; Per-band and per BC; No; N/A; N/A; Candidate values: {(Z2, Z'2), Z2+Z'2, 2Z'2)}; Optional with capability signaling. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-3-1-11; Active CSI-RS resources and ports for mixed codebook types including Type-II-CJT in any slot; 1. List of codebook combinations 2. List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination; 40-3-1-1,40-3-1-5,2-36; Yes; N/A; Active CSI-RS resources and ports for mixed Type-II-CJT codebook types in any slot is not supported; Per-band and Per-BC; N/A; N/A; N/A; [Codebook 1 = {Type I SP, Type I MP} (Codebook 2, Codebook 3) = {(eType-II-CJT R=1, NULL), (eType-II-CJT R=2, NULL), (FeType-II-CJT PS R=1 M=1, NULL), (FeType-II-CJT PS R=1 M=2, NULL), (FeType-II-CJT PS R=2 M=2, NULL), (FFS CB combos…)}]; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-3-1-11; Active CSI-RS resources and ports for mixed codebook types including Type-II-CJT in any slot; 1. List of codebook combinations 2. List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination; 40-3-1-1,40-3-1-5,2-36, 2-40; Yes; N/A; Active CSI-RS resources and ports for mixed Type-II-CJT codebook types in any slot is not supported; [Per-band Per-BC/Per FS]; N/A; N/A; N/A; [Codebook 1 = {Type I SP, Type I MP} (Codebook 2, Codebook 3) = {(eType-II-CJT R=1, NULL), (eType-II-CJT R=2, NULL), (FeType-II-CJT PS R=1 M=1, NULL), (FeType-II-CJT PS R=1 M=2, NULL), (FeType-II-CJT PS R=2 M=2, NULL), (FFS CB combos…)}]; Optional with capability signaling. |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-12 | Support of Rank 3 and 4 for Rel-16-based CJT type-II codebook | 1. Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with rank 3,4 | 40-3-1-1 | Yes | N/A | Rank 3 and 4 for Rel-16-based CJT type-II codebook is not supported | Per-band and Per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] |  |
| ZTE [4] |  |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] |  |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] |  |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-13 | Support of Rank 3 and 4 for Rel-17-based CJT type-II codebook | 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with rank 3,4 | 40-3-1-5 | Yes | N/A | Rank 3 and 4 for Rel-17-based CJT type-II codebook is not supported | Per-band and Per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] |  |
| ZTE [4] |  |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] |  |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] |  |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-14 | Support of Support of L=6 for Rel-16-based CJT type-II codebook | 1. Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with parameter combination with L=6 | 40-3-1-1 | Yes | N/A | Support of L=6 for Rel-16-based CJT type-II codebook is not supported | Per-band and Per BC | N/A | N/A | N/A | Note: Only for N_TRP=1 | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] |  |
| ZTE [4] |  |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] |  |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] |  |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-15 | dynamic selection of N<=N_TRP for Rel-16-based CJT type-II codebook | 1. Support of selection of N <= N_TRP CSI-RS resource by UE for multi-TRP CJT based on Rel-16 eType-II codebook | 40-3-1-1 | Yes | N/A | Dynamic selection of N<N_TRP for Rel-16-based CJT type-II codebook is not supported | Per-band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] |  |
| ZTE [4] |  |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] |  |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] |  |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-16 | dynamic selection of N<=N_TRP for Rel-17-based CJT type-II codebook | 1. Support of selection of N <= N_TRP CSI-RS resource by UE for multi-TRP CJT based on Rel-17 FeType-II port selection codebook | 40-3-1-5 | Yes | N/A | Dynamic selection of N<N_TRP for Rel-17-based CJT type-II codebook is not supported | Per-band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] |  |
| ZTE [4] |  |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] |  |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] |  |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-17 | Support for N_L>1 combinations of number of SD basis across CSI-RS resources for Rel-16-based CJT type-II codebook | 1. Maximum number of lists for spatial basis selection, i.e., N_L, for multi-TRP CJT based on Rel-16 eType-II codebook | 40-3-1-1 | Yes | N/A | N_L>1 combinations across for Rel-16-based CJT type-II codebook is not supported | Per band and Per BC | N/A | N/A | N/A | Candidate values for component 1: N_L = {2,4} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] |  |
| ZTE [4] |  |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] |  |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] |  |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-18 | Support for N_L>1 combinations of number of ports across CSI-RS resources for Rel-17-based CJT type-II codebook | 1. Maximum number of lists for ports selection, i.e., NL, for multi-TRP CJT based on Rel-17 FeType-II port selection codebook | 40-3-1-5 | Yes | N/A | N_L>1 combinations across for Rel-17-based CJT type-II codebook is not supported | Per band and Per BC | N/A | N/A | N/A | Candidate values for component 1: N_L = {2,4} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] |  |
| ZTE [4] |  |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] |  |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] |  |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-23 | Unequal number of spatial basis selection configuration for multi-TRP CJT | 1. Support of unequal number of spatial basis selection configuration across CSI-RS resources for multi-TRP CJT including Rel-16 eType-II codebook refinement | 40-3-1-1 | Yes | N/A | Unequal number of spatial basis selection configuration for multi-TRP CJT is not supported | Per-band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] |  |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-3-1-23; Unequal number of spatial basis selection configuration for multi-TRP CJT for Rel-16-based CJT codebook; 1. Support of unequal number of spatial basis selection configuration across CSI-RS resources for multi-TRP CJT including Rel-16 eType-II codebook refinement; 40-3-1-1; Yes; N/A; Unequal number of spatial basis selection configuration for multi-TRP CJT for Rel-16-based CJT codebook is not supported; Per-band and per BC; N/A; N/A; N/A; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] |  |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] |  |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-23a | Unequal number of port selection configuration for multi-TRP CJT | 1. Support of unequal number of port selection configuration across CSI-RS resources for multi-TRP CJT including Rel-17 FeType-II port selection codebook refinement | 40-3-1-5 | Yes | N/A | Unequal number of port selection configuration for multi-TRP CJT is not supported | Per-band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] |  |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-3-1-23a; Unequal number of port selection configuration for multi-TRP CJT for Rel-17-based CJT codebook; 1. Support of unequal number of port selection configuration across CSI-RS resources for multi-TRP CJT including Rel-17 FeType-II port selection codebook refinement; 40-3-1-5; Yes; N/A; Unequal number of port selection configuration for multi-TRP CJT for Rel-17-based CJT codebook is not supported; Per-band and per BC; N/A; N/A; N/A; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] |  |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] |  |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-2-1 | Support of Rel-16-based doppler CSI | 1. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) 2. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 3. Support parameter combinations with L=2,4 4. Support for rank = 1,2 5. A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously [6. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting] 7. Value of Y for CPU occupation (OCPU = Y.N4), when P/SP-CSI-RS is configured for CMR 8. Value of Y for CPU occupation (OCPU = Y.K), when A-CSI-RS is configured for CMR 9. Support for the size of DD-basis, N4=1 10. Scaling factor for active resource counting Kp | 2-35 | Yes | N/A | Rel-16 based Type II doppler codebook is not supported | Per-band and Per-BC | N/A | N/A | N/A | Component 7 candidate values: {1, 2, 3} Component 8 candidate values: {1, 2, 3} Component 10 candidate values: {1, 2, 4} Note: When N4=1, OCPU =4 Note: OCPU ≥ 4 when P/SP-CSI-RS is configured for CMR Note: when K=12, OCPU =8 Note: A UE that supports CSI enhancement for Rel. 16 based type-II doppler must support this FG | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-3-2-1: Support of Rel-16-based doppler CSI - A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously - a. {1,2,4,8} - b. {4, 8, 12, 16, 24, 32} - c.{2,3,4 … 64} - d. {4, …, 256} - We recomend confirming component 6 ” A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting. The candidate vaules are - a. {1,2,4,8} - b. {4, 8, 12, 16, 24, 32} - c. {4, 8, 12} - d. .{4, .., 256} 40. NR_MIMO_evo_DL_UL; 40-3-2-1; Support of Rel-16-based doppler CSI; 1. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) 2. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 3. Support parameter combinations with L=2,4 4. Support for rank = 1,2 5. A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously [6. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting] 7. Value of Y for CPU occupation (OCPU = Y.N4), when P/SP-CSI-RS is configured for CMR 8. Value of Y for CPU occupation (OCPU = Y.K), when A-CSI-RS is configured for CMR 9. Support for the size of DD-basis, N4=1 10. Scaling factor for active resource counting Kp; 2-35; Yes; N/A; Rel-16 based Type II doppler codebook is not supported; Per-band and Per-BC; N/A; N/A; N/A; Component 5 candidate values a. {1,2,4,8} b. {4, 8, 12, 16, 24, 32} c. {2,3,4 … 64} d. {4, …, 256} Component 6 candidate values a. {4, 8, 12, 16, 24, 32} b. {4,8,12} c. {4, …, 256} Component 7 candidate values: {1, 2, 3} Component 8 candidate values: {1, 2, 3} Component 10 candidate values: {1, 2, 4} Note: When N4=1, OCPU =4 Note: OCPU ≥ 4 when P/SP-CSI-RS is configured for CMR Note: when K=12, OCPU =8 Note: A UE that supports CSI enhancement for Rel. 16 based type-II doppler must support this FG; Optional with capability signaling. |
| Ericsson [3] |  |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-3-2-1; Support of Rel-16-based doppler CSI; 1. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) 2. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 3. Support parameter combinations with L=2,4 4. Support for rank = 1,2 5. A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports of resources} across all CCs simultaneously [6. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting] 7. Value of Y for CPU occupation (OCPU = Y.N4), when P/SP-CSI-RS is configured for CMR 8. Value of Y for CPU occupation (OCPU = Y.K), when A-CSI-RS is configured for CMR 9. Support for the size of DD-basis, N4=1 10. Scaling factor for active resource counting Kp; 2-35; Yes; N/A; Rel-16 based Type II doppler codebook is not supported; Per-band and Per-BC; N/A; N/A; N/A; Component 5 candidate values: a) Max # of Tx ports in one resource : {4, 8, 12, 16, 24, 32} b) Max # of resources: {2,3,4 … 64} c) Total # of Tx ports: {4, …, 256} Component 7 candidate values: {1, 2, 3} Component 8 candidate values: {1, 2, 3} Component 10 candidate values: {1, 2, 4} Note: When N4=1, OCPU =4 Note: OCPU ≥ 4 when P/SP-CSI-RS is configured for CMR Note: when K=12, OCPU =8 Note: A UE that supports CSI enhancement for Rel. 16 based type-II doppler must support this FG; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-3-2-1; Support of Rel-16-based doppler CSI; 1. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) 2. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 3. Support parameter combinations with L=2,4 4. Support for rank = 1,2 5. A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously [6. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting] 7. Value of Y for CPU occupation (OCPU = Y.N4), when P/SP-CSI-RS is configured for CMR 8. Value of Y for CPU occupation (OCPU = Y.K), when A-CSI-RS is configured for CMR 9. Support for the size of DD-basis, N4=1 10. Scaling factor for active resource counting Kp 11. W additional symbols of Z for P/SP CSI-RS based Type II Doppler CSI compared with legacy Z; 2-35; Yes; N/A; Rel-16 based Type II doppler codebook is not supported; Per-band and Per-BC; N/A; N/A; N/A; Component 7 candidate values: {1, 2, 3} Component 8 candidate values: {1, 2, 3} Component 10 candidate values: {1, 2, 4} Component 11 candidate values: {0, 14, 28, 56} Note: When N4=1, OCPU =4 Note: OCPU ≥ 4 when P/SP-CSI-RS is configured for CMR Note: when K=12, OCPU =8 Note: A UE that supports CSI enhancement for Rel. 16 based type-II doppler must support this FG; Optional with capability signaling. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-3-2-1; Support of Rel-16-based doppler CSI; 1. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) 2. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 3. Support parameter combinations with L=2,4 4. Support for rank = 1,2 5. A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously 7. Value of Y for CPU occupation (OCPU = Y.N4), when P/SP-CSI-RS is configured for CMR 8. Value of Y for CPU occupation (OCPU = Y.K), when A-CSI-RS is configured for CMR 9. Support for the size of DD-basis, N4=1 10. Scaling factor for active resource counting Kp; 2-35; Yes; N/A; Rel-16 based Type II doppler codebook is not supported; Per-band and Per-BC; N/A; N/A; N/A; Component 7 candidate values: {1, 2, 3} Component 8 candidate values: {1, 2, 3} Component 10 candidate values: {1, 2, 4} Note: When N4=1, OCPU =4 Note: OCPU ≥ 4 when P/SP-CSI-RS is configured for CMR Note: when K=12, OCPU =8 Note: A UE that supports CSI enhancement for Rel. 16 based type-II doppler must support this FG; Optional with capability signaling. |
| OPPO [8] |  |
| CATT [9] | 40-3-2-1: - According to the following agreement, one separate FG corresponding to parameter combinations with L=6 should be introduced for Rel-16-based Doppler codebook. Agreement For the Type-II codebook refinement for high/medium velocities based on Rel-16 eType-II regular codebook, at least the following Parameter Combinations are supported 4; 1/4; 1/4; 1/4. 4; 1/4; 1/4; 1/2. 4 (*); 1/2; 1/4; 1/2. 4 (*); 1/4; 1/4; 3/4. 6 (*); 1/4; --; 1/2. 6 (*); 1/4; --; 3/4. (*) Note: From legacy. For L=6, the same restriction and UE optionality as legacy apply. - According to the following agreement, rank=1,2,3,4 is supported. Same to Rel-16 and Rel-17 Type II codebook, rank=1,2 is basic feature and rank=3,4 is optional subject to UE capability. Therefore, one separate FG corresponding to rank=3,4 should be introduced for Rel-16-based Doppler codebook. Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, support RI={1,2,3,4}.. - According to the following agreement, one separate FG corresponding to Capability 2 for Z/Z’ should be introduced for Rel-16-based Doppler codebook. Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z/Z’ - For N4=1: reuse legacy Z’ values - For N4>1, introduce two UE capabilities: - Capability 1: Reuse legacy Z’ values - Capability 2: Legacy Z’ values + r. Proposal 6: For Rel-16-based Doppler codebook, three additional FG corresponding to the following UE feature should be added. - Support parameter combinations with L=6 - Support rank=3,4 - Support Capability 2 for Z/Z’ |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-3-2-1; Support of Rel-16-based doppler CSI; 1. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) 2. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 3. Support parameter combinations with L=2,4 4. Support for rank = 1,2 5. A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously [6. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting] 7. Value of Y for CPU occupation (OCPU = Y.N4), when P/SP-CSI-RS is configured for CMR 8. Value of Y for CPU occupation (OCPU = Y.K), when A-CSI-RS is configured for CMR 9. Support for the size of DD-basis, N4=1 10. Scaling factor for active resource counting Kp; 2-35; Yes; N/A; Rel-16 based Type II doppler codebook is not supported; Per-band and Per-BC; N/A; N/A; N/A; Candidate values for component 5: - Max# of time unit: - {d∙K for aperoidc CSI-RS - {1T, 2T, 4T} for periodic or semi-persisten CSI-RS, where T is period of CSI-RS. - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} Component 7 candidate values: {1, 2, 3} Component 8 candidate values: {1, 2, 3} Component 10 candidate values: {1, 2, 4} Note: When N4=1, OCPU =4 Note: OCPU ≥ 4 when P/SP-CSI-RS is configured for CMR Note: when K=12, OCPU =8 Note: A UE that supports CSI enhancement for Rel. 16 based type-II doppler must support this FG; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-3-2-1: - Components: - For Component 6, it was argued that this is for UE’s memory limitation, while component 5 is for UE’s per-slot limitation. We are ok to remove the bracket (i.e., confirm to include this component). - Capture default Z’=2*Z2’ (i.e., r=Z2’) Agreement (RAN1#113) For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z/Z’ - For N4=1: reuse legacy Z’ values - For N4>1, introduce two UE capabilities: - Capability 1: Reuse legacy Z’ values - Capability 2: Legacy Z’ values + r - The value(s) of r>0 can depend on the configured N4 value - FFS: exact value(s) of r Note: Since this pertains Type-II, the relevant values are Z2/Z2’ Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z/Z’ for Capability 2, r=legacy Z2’. - Capture default Z: - Z=Z2+w+r for P/SP CSI-RS, where w values need to be discussed further in UE feature session and r value is equal to legacy Z2’ - Z=Z2+14.(K–1).m + r Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z - Based on the two UE capabilities agreed for Z’: - Capability 1: - For AP CSI-RS: Z=legacy Z+14.(K–1).m - For P/SP CSI-RS: Z= legacy Z+w where w>0 - TBD: Value of w - Capability 2: - For AP CSI-RS: Z= legacy Z+14.(K–1).m + r - For P/SP CSI-RS: Z= legacy Z+w+r - Note: r corresponds to the agreed value for Z’ relaxation in previous agreement. Note: Since this pertains Type-II, the relevant values are Z2/Z2’. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-3-2-1; Support of Rel-16-based doppler CSI; 1. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) 2. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 3. Support parameter combinations with L=2,4 4. Support for rank = 1,2 5. A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously [6. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting] 7. Value of Y for CPU occupation (OCPU = Y.N4), when P/SP-CSI-RS is configured for CMR 8. Value of Y for CPU occupation (OCPU = Y.K), when A-CSI-RS is configured for CMR 9. Support for the size of DD-basis, N4=1 10. Scaling factor for active resource counting Kp; 2-35; Yes; N/A; Rel-16 based Type II doppler codebook is not supported; Per-band and Per-BC; N/A; N/A; N/A; Component 5 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} Component 7 candidate values: {1, 2, 3} Component 8 candidate values: {1, 2, 3} Component 10 candidate values: {1, 2, 4} Note: When N4=1, OCPU =4 Note: OCPU ≥ 4 when P/SP-CSI-RS is configured for CMR Note: when K=12, OCPU =8 Note: A UE that supports CSI enhancement for Rel. 16 based type-II doppler must support this FG; Optional with capability signaling. |
| Samsung [16] | FG 40-3-2-1 and FG 40-3-2-4: for consistency across different FGs, it is preferred to use the one wording ‘Doppler measurement’ or ‘Doppler CSI. Our preference is to use the latter since this enhancement is mainly about Doppler codebook. - Replace ‘Doppler measurement’ with ‘Doppler CSI’ |
| MediaTek Inc. [17] | 40. NR_MIMO_evo_DL_UL; 40-3-2-1; Support of Rel-16-based doppler CSI; - Support of DD unit size d=m when A-CSI-RS is configured for CMR - Support of DD unit size d= CSI-RS periodicity when P/SP-CSI-RS is configured for CMR - Capability index on CSI computation time; Component 3 candidate values {1,2}; Optional with capability signalling. |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-3-2-1; Support of Rel-16-based doppler CSI; 1. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) 2. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 3. Support parameter combinations with L=2,4 4. Support for rank = 1,2 5. A list of supported combinations, each combination is 6. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously [6. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting] 7. Value of Y for CPU occupation (OCPU = Y.N4), when P/SP-CSI-RS is configured for CMR 8. Value of Y for CPU occupation (OCPU = Y.K), when A-CSI-RS is configured for CMR 9. Support for the size of DD-basis, N4=1 10. Scaling factor for active resource counting Kp Note: A UE that supports CSI enhancement for Rel. 16 based type-II doppler must support this FG; 2-35; Yes; N/A; Rel-16 based Type II doppler codebook is not supported; Per-band and Per-BC; N/A; N/A; N/A; Component 7 candidate values: {1, 2, 3} Component 8 candidate values: {1, 2, 3} Component 10 candidate values: {1, 2, 4} Note: When N4=1, OCPU =4 Note: OCPU ≥ 4 when P/SP-CSI-RS is configured for CMR Note: when K=12, OCPU =8; Optional with capability signaling. |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-2-1a | Support of Rel-16-based doppler measurement with N4>1 | 1. Support for the size of DD-basis, N4>1 [2. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously] [3. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting] | 40-3-2-1 | Yes | N/A | Rel-16 based Type II doppler codebook with N4>1 is not supported | Per-band and Per-BC | N/A | N/A | N/A |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-3-2-1a: Support of Rel-16-based doppler measurement with N4>1 - We recommend confirming component 2 “ A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously”. Recommended candidate values are - a. {1,2,4,8} - b. {4, 8, 12, 16, 24, 32} - c. {2,3,4 … 64} - d. {4, …, 256} - We recommend confirming component 3 “A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting”. Recommended candidate values are - a. {1,2,4,8} - b. {4, 8, 12, 16, 24, 32} - c. {4, 8, 12} - d. {4, .., 256} 40. NR_MIMO_evo_DL_UL; 40-3-2-1a; Support of Rel-16-based doppler measurement with N4>1; 1. Support for the size of DD-basis, N4>1 [2. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously] [3. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting]; 40-3-2-1; Yes; N/A; Rel-16 based Type II doppler codebook with N4>1 is not supported; Per-band and Per-BC; N/A; N/A; N/A; Component 2 candidate values a. {1,2,4,8} b. {4, 8, 12, 16, 24, 32} c. {2,3,4 … 64} d. {4, …, 256} Component 3 Candidate values a. {1,2,4,8} b. {4, 8, 12, 16, 24, 32} c. {4, 8, 12} d.{4, .., 256}. |
| Ericsson [3] |  |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-3-2-1a; Support of Rel-16-based doppler measurement with N4>1; 1. Support for the size of DD-basis, N4>1 [2. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously] [3. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting]; 40-3-2-1; Yes; N/A; Rel-16 based Type II doppler codebook with N4>1 is not supported; Per-band and Per-BC; N/A; N/A; N/A; Component 1 candidate values: {2, 4, 8}. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-3-2-1a; Support of Rel-16-based doppler measurement with N4>1; 1. Support for the size of DD-basis, N4>1 [2. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously] [3. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting]; 40-3-2-1; Yes; N/A; Rel-16 based Type II doppler codebook with N4>1 is not supported; Per-band and Per-BC; N/A; N/A; N/A. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-3-2-1a; Support of Rel-16-based doppler measurement with N4>1; 1. Support for the size of DD-basis, N4>1 2. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously; 40-3-2-1; Yes; N/A; Rel-16 based Type II doppler codebook with N4>1 is not supported; Per-band and Per-BC; N/A; N/A; N/A. |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-3-2-1a; Support of Rel-16-based doppler measurement with N4>1; 1. Support for the size of DD-basis, N4>1 [2. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously] [3. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting]; 40-3-2-1; Yes; N/A; Rel-16 based Type II doppler codebook with N4>1 is not supported; Per-band and Per-BC; N/A; N/A; N/A; Candidate values for component 2: - Max N4 {2,4,8} - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-3-2-1a: - Component 2: Ok to confirm - Component 3: Ok to confirm |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-3-2-1a; Support of Rel-16-based doppler measurement with N4>1; 1. Support for the size of DD-basis, N4>1 2. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource - Maximum total number of NZP CSI-RS resource - Maximum total number of Tx ports of NZP CSI-RS resources - Number of predicted CSI (N4); 40-3-2-1; Yes; N/A; Rel-16 based Type II doppler codebook with N4>1 is not supported; Per-band and Per-BC; N/A; N/A; N/A; Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} d) {1, 2, 4, 8}; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-3-2-1a; Support of Rel-16-based doppler measurement with N4>1={2, 4, 8}; 1. Support for the size of DD-basis, N4>1 [2. A list of supported combinations, each combination is 3. {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously] [3. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting]; 40-3-2-1; Yes; N/A; Rel-16 based Type II doppler codebook with N4>1 is not supported; Per-band and Per-BC; N/A; N/A; N/A; Optional with capability signaling. |
| Huawei/HiSilicon [19] | For CSI enhancement for mobility, whether to support a list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one doppler CSI measurement is FFS. In our view, for doppler CSI codebook, multiple CSI-RS occasions measurement, UE-side prediction and PMI calculation in three dimensions would be performed, which increase calculation complexity and the requirement of UE memory. Obviously, the value of N4 affects the calculation complexity of the UE. Therefore, the configuration of supported combinations should be considered together with the value of N4. Therefore, we propose: Proposal 3-2: Support the following components into FG 40-3-2-1a: - {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CC. 40. NR_MIMO_evo_DL_UL; 40-3-2-1a; Support of Rel-16-based doppler measurement with N4>1; 1. Support for the size of DD-basis, N4>1 [2. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously] [3. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting]; 40-3-2-1; Yes; N/A; Rel-16 based Type II doppler codebook with N4>1 is not supported; Per-band and Per-BC; N/A; N/A; N/A. |

| 40. NR_MIMO_evo_DL_UL | 40-3-2-2 | Support R=2 for Rel-16-based doppler codebook | A list of supported combinations {[Max N4,] Max # of Tx ports in one resource, Max # of resources and total # of Tx ports}, across all CCs simultaneously, with R=2 | 40-3-2-1 | Yes | N/A | R=2 for Rel-16-based doppler codebook is not supported | Per band and per BC | N/A | N/A | N/A | Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] |  |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-3-2-2; Support R=2 for Rel-16-based doppler codebook; A list of supported combinations {[Max N4,] Max # of Tx ports in one resource, Max # of resources and total # of Tx ports}, across all CCs simultaneously, with R=2; 40-3-2-1; Yes; N/A; R=2 for Rel-16-based doppler codebook is not supported; Per band and per BC; N/A; N/A; N/A; Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-3-2-2; Support R=2 for Rel-16-based doppler codebook; A list of supported combinations {[Max N4,] Max # of Tx ports in one resource, Max # of resources and total # of Tx ports}, across all CCs simultaneously, with R=2; 40-3-2-1; Yes; N/A; R=2 for Rel-16-based doppler codebook is not supported; Per band and per BC; N/A; N/A; N/A; Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}; Optional with capability signaling. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-3-2-2; Support R=2 for Rel-16-based doppler codebook; A list of supported combinations { Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports}, across all CCs simultaneously, with R=2; 40-3-2-1; Yes; N/A; R=2 for Rel-16-based doppler codebook is not supported; Per band and per BC; N/A; N/A; N/A; Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}; Optional with capability signaling. |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-3-2-2; Support R=2 for Rel-16-based doppler codebook; A list of supported combinations {[Max N4,] Max # of Tx ports in one resource, Max # of resources and total # of Tx ports}, across all CCs simultaneously, with R=2; 40-3-2-1; Yes; N/A; R=2 for Rel-16-based doppler codebook is not supported; Per band and per BC; N/A; N/A; N/A; Candidate values for component 1: - Maximum 16 triplets - Max N4:{2,4,8} - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-3-2-2: - Component: Ok to confirm the yellow part |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-3-2-2; Support R=2 for Rel-16-based doppler codebook; A list of supported combinations {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports}, across all CCs simultaneously, with R=2; 40-3-2-1; Yes; N/A; R=2 for Rel-16-based doppler codebook is not supported; Per band and per BC; N/A; N/A; N/A; Candidate values for component 1: - Maximum 16 triplets - Max N4 {1,2,4,8} - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-3-2-2; Support R=2 for Rel-16-based doppler codebook; A list of supported combinations {[Max N4,] Max # of Tx ports in one resource, Max # of resources and total # of Tx ports}, across all CCs simultaneously, with R=2; 40-3-2-1; Yes; N/A; R=2 for Rel-16-based doppler codebook is not supported; Per band and per BC; N/A; N/A; N/A; Candidate values for component 1: - Maximum 9 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}; Optional with capability signaling. |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-2-3 | Support X=1 based on first and last slot of WCSI, for Rel-16-based doppler codebook | Support of TDCQI = ‘1-2’ | 40-3-2-1 | Yes | N/A | X=1 based on first and last slot of WCSI, for Rel-16-based doppler codebook is not supported | Per band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] |  |
| ZTE [4] |  |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] |  |
| Samsung [16] | FG 40-3-2-3/3a: it needs to be clarified that X is the number of time-domain CQIs - Support X=1 time-domain CQI based on averaging first and last slot of WCSI, for Rel-16-based doppler codebook - Support X=2 time-domain CQIs based on 2 slots for Rel-16-based doppler codebook |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] |  |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-2-3a | Support X=2 CQI based on 2 slots for Rel-16-based doppler codebook | Support of TDCQI = ‘2’ | 40-3-2-1 | Yes | N/A | X=2 CQI based on 2 slots for Rel-16-based doppler codebook is not supported | Per band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] |  |
| ZTE [4] |  |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] |  |
| Samsung [16] | FG 40-3-2-3/3a: it needs to be clarified that X is the number of time-domain CQIs - Support X=1 time-domain CQI based on averaging first and last slot of WCSI, for Rel-16-based doppler codebook - Support X=2 time-domain CQIs based on 2 slots for Rel-16-based doppler codebook |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] |  |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-2-4 | Support of Rel-17-based doppler measurement | 1. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI 2. Support of Rel-17 FeType-II port selection codebook refinement for predicted PMI with PMI subband R=1 3. Support of parameter combinations with M = 1 4. Support for rank = 1,2 5: A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one doppler CSI measurement [6. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting] 7. Value of Y for CPU occupation (OCPU = Y.K), when A-CSI-RS is configured for CMR 8. Support for N4=1 9. Scaling factor for active resource counting Kp | 40-3-2-1, 2-35 | Yes | N/A | Rel-17 based Type II doppler codebook is not supported | Per-band and Per-BC | N/A | N/A | N/A | Component 7 candidate values: {1, 2, 3} Component 9 candidate values: {1, 2, 4} Note: OCPU = 4 when P/SP-CSI-RS is configured for CMR Note: when K=12, OCPU =8 Note: A UE that supports CSI enhancement for Rel. 17-based type-2 doppler must support this FG | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-3-2-4: Support of Rel-17-based doppler measurement - Component 5, A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one doppler CSI measurement. The candidate values are - a. {4, 8, 12, 16, 24, 32} - b. {1, 2,3,4 … 64} - c. {4, …, 256} - We recommend removing component 6, 40. NR_MIMO_evo_DL_UL; 40-3-2-4; Support of Rel-17-based doppler measurement; 1. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI 2. Support of Rel-17 FeType-II port selection codebook refinement for predicted PMI with PMI subband R=1 3. Support of parameter combinations with M = 1 4. Support for rank = 1,2 5: A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one doppler CSI measurement [6. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting] 7. Value of Y for CPU occupation (OCPU = Y.K), when A-CSI-RS is configured for CMR 8. Support for N4=1 9. Scaling factor for active resource counting Kp; 40-3-2-1, 2-35; Yes; N/A; Rel-17 based Type II doppler codebook is not supported; Per-band and Per-BC; N/A; N/A; N/A; Component 5 candidate values a. {4, 8, 12, 16, 24, 32} b. {1, 2,3,4 … 64} c. {4, …, 256} Component 7 candidate values: {1, 2, 3} Component 9 candidate values: {1, 2, 4} Note: OCPU = 4 when P/SP-CSI-RS is configured for CMR Note: when K=12, OCPU =8 Note: A UE that supports CSI enhancement for Rel. 17-based type-2 doppler must support this FG; Optional with capability signaling. |
| Ericsson [3] |  |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-3-2-4; Support of Rel-17-based doppler measurement; 1. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI 2. Support of Rel-17 FeType-II port selection codebook refinement for predicted PMI with PMI subband R=1 3. Support of parameter combinations with M = 1 4. Support for rank = 1,2 5: A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports of resources} across all CCs simultaneously for one doppler CSI measurement [6. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting] 7. Value of Y for CPU occupation (OCPU = Y.K), when A-CSI-RS is configured for CMR 8. Support for N4=1 9. Scaling factor for active resource counting Kp; 40-3-2-1, 2-35; Yes; N/A; Rel-17 based Type II doppler codebook is not supported; Per-band and Per-BC; N/A; N/A; N/A; Component 5 candidate values: a) Max # of Tx ports in one resource : {4, 8, 12, 16, 24, 32} b) Max # of resources: {2,3,4 … 64} c) Total # of Tx ports: {4, …, 256} Component 7 candidate values: {1, 2, 3} Component 9 candidate values: {1, 2, 4} Note: OCPU = 4 when P/SP-CSI-RS is configured for CMR Note: when K=12, OCPU =8 Note: A UE that supports CSI enhancement for Rel. 17-based type-2 doppler must support this FG; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-3-2-4; Support of Rel-17-based doppler measurement; 1. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI 2. Support of Rel-17 FeType-II port selection codebook refinement for predicted PMI with PMI subband R=1 3. Support of parameter combinations with M = 1 4. Support for rank = 1,2 5: A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one doppler CSI measurement [6. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting] 7. Value of Y for CPU occupation (OCPU = Y.K), when A-CSI-RS is configured for CMR 8. Support for N4=1 9. Scaling factor for active resource counting Kp 10. W additional symbols of Z for P/SP CSI-RS based Type II Doppler CSI compared with legacy Z; 40-3-2-1, 2-35; Yes; N/A; Rel-17 based Type II doppler codebook is not supported; Per-band and Per-BC; N/A; N/A; N/A; Component 7 candidate values: {1, 2, 3} Component 9 candidate values: {1, 2, 4} Component 10 candidate values: {0, 14, 28, 56} Note: OCPU = 4 when P/SP-CSI-RS is configured for CMR Note: when K=12, OCPU =8 Note: A UE that supports CSI enhancement for Rel. 17-based type-2 doppler must support this FG; Optional with capability signaling. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-3-2-4; Support of Rel-17-based doppler measurement; 1. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI 2. Support of Rel-17 FeType-II port selection codebook refinement for predicted PMI with PMI subband R=1 3. Support of parameter combinations with M = 1 4. Support for rank = 1,2 5: A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one doppler CSI measurement 7. Value of Y for CPU occupation (OCPU = Y.K), when A-CSI-RS is configured for CMR 8. Support for N4=1 9. Scaling factor for active resource counting Kp; 40-3-2-1, 2-35; Yes; N/A; Rel-17 based Type II doppler codebook is not supported; Per-band and Per-BC; N/A; N/A; N/A; Component 7 candidate values: {1, 2, 3} Component 9 candidate values: {1, 2, 4} Note: OCPU = 4 when P/SP-CSI-RS is configured for CMR Note: when K=12, OCPU =8 Note: A UE that supports CSI enhancement for Rel. 17-based type-2 doppler must support this FG; Optional with capability signaling. |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-3-2-4; Support of Rel-17-based doppler measurement; 1. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI 2. Support of Rel-17 FeType-II port selection codebook refinement for predicted PMI with PMI subband R=1 3. Support of parameter combinations with M = 1 4. Support for rank = 1,2 5: A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one doppler CSI measurement [6. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting] 7. Value of Y for CPU occupation (OCPU = Y.K), when A-CSI-RS is configured for CMR 8. Support for N4=1 9. Scaling factor for active resource counting Kp; 40-3-2-1, 2-35; Yes; N/A; Rel-17 based Type II doppler codebook is not supported; Per-band and Per-BC; N/A; N/A; N/A; Component 7 candidate values: {1, 2, 3} Component 9 candidate values: {1, 2, 4} Note: OCPU = 4 when P/SP-CSI-RS is configured for CMR Note: when K=12, OCPU =8 Note: A UE that supports CSI enhancement for Rel. 17-based type-2 doppler must support this FG; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-3-2-4: - Components: Ok to confirm the yellow part |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-3-2-4; Support of Rel-17-based doppler measurement; 1. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI 2. Support of Rel-17 FeType-II port selection codebook refinement for predicted PMI with PMI subband R=1 3. Support of parameter combinations with M = 1 4. Support for rank = 1,2 5: A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one doppler CSI measurement [6. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting] 7. Value of Y for CPU occupation (OCPU = Y.K), when A-CSI-RS is configured for CMR 8. Support for N4=1 9. Scaling factor for active resource counting Kp; 40-3-2-1, 2-35; Yes; N/A; Rel-17 based Type II doppler codebook is not supported; Per-band and Per-BC; N/A; N/A; N/A; Component 5 candidate values: a) {4, 8, 12, 16, 24, 32} b) {1, 2,3,4 … 64} c) {4, …, 256} Component 7 candidate values: {1, 2, 3} Component 9 candidate values: {1, 2, 4} Note: OCPU = 4 when P/SP-CSI-RS is configured for CMR Note: when K=12, OCPU =8 Note: A UE that supports CSI enhancement for Rel. 17-based type-2 doppler must support this FG; Optional with capability signaling. |
| Samsung [16] | FG 40-3-2-1 and FG 40-3-2-4: for consistency across different FGs, it is preferred to use the one wording ‘Doppler measurement’ or ‘Doppler CSI. Our preference is to use the latter since this enhancement is mainly about Doppler codebook. - Replace ‘Doppler measurement’ with ‘Doppler CSI’ |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-3-2-4; Support of Rel-17-based doppler CSI measurement; 1. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI 2. Support of Rel-17 FeType-II port selection codebook refinement for predicted PMI with PMI subband R=1 3. Support of parameter combinations with M = 1 4. Support for rank = 1,2 5. A list of supported combinations, each combination is 6. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously [6. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting] 7. Value of Y for CPU occupation (OCPU = Y.K), when A-CSI-RS is configured for CMR 8. Support for N4=1 9. Scaling factor for active resource counting Kp Note: A UE that supports CSI enhancement for Rel. 17-based type-2 doppler must support this FG; 40-3-2-1, 2-35; Yes; N/A; Rel-17 based Type II doppler codebook is not supported; Per-band and Per-BC; N/A; N/A; N/A; Component 7 candidate values: {1, 2, 3} Component 9 candidate values: {1, 2, 4} Note: OCPU = 4 when P/SP-CSI-RS is configured for CMR Note: when K=12, OCPU =8; Optional with capability signaling. |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-2-5 | Support of M=2 and R=1 for Rel-17-based doppler codebook | 1. Support of Rel-17 FeType-II port selection codebook refinement for predicted PMI with M=2 and PMI subband R=1 2. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource b) Maximum total number of NZP CSI-RS resource c) Maximum total number of Tx ports of NZP CSI-RS resources | 40-3-2-4 | Yes | N/A | M=2 and R=1 for Rel-17-based doppler codebook is not supported | Per-band and Per-BC | N/A | N/A | N/A | Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] |  |
| ZTE [4] |  |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] |  |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] |  |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-2-6 | Support R=2 for Rel-17-based doppler codebook | 1. Support of Rel-17 FeType-II port selection codebook refinement for predicted PMI with PMI subbands R=2 2. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource b) Maximum total number of NZP CSI-RS resource c) Maximum total number of Tx ports of NZP CSI-RS resources | 40-3-2-4 | Yes | N/A | R=2 for Rel-17-based doppler codebook is not supported | Per band and per BC | N/A | N/A | N/A | Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} d) {2,3,4} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-3-2-6; Support R=2 for Rel-17-based doppler codebook; 1. Support of Rel-17 FeType-II port selection codebook refinement for predicted PMI with PMI subbands R=2 2. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource b) Maximum total number of NZP CSI-RS resource c) Maximum total number of Tx ports of NZP CSI-RS resources; 40-3-2-4; Yes; N/A; R=2 for Rel-17-based doppler codebook is not supported; Per band and per BC; N/A; N/A; N/A; Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} d) {2,3,4}; Optional with capability signaling. |
| Ericsson [3] |  |
| ZTE [4] |  |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-3-2-6; Support R=2 for Rel-17-based doppler codebook; 1. Support of Rel-17 FeType-II port selection codebook refinement for predicted PMI with PMI subbands R=2 2. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource b) Maximum total number of NZP CSI-RS resource c) Maximum total number of Tx ports of NZP CSI-RS resources; 40-3-2-4; Yes; N/A; R=2 for Rel-17-based doppler codebook is not supported; Per band and per BC; N/A; N/A; N/A; Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} d) {2,3,4}; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] |  |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] |  |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-2-7 | support of l = (n – nCSI,ref ) for CSI reference slot for Rel-16 based doppler codebook | 1. Support of l = (n – nCSI,ref ) for CSI reference slot when N4=1 and d>1 2. Support of l = (n – nCSI,ref ) for CSI reference slot when N4>1 | 40-3-2-1 | Yes | N/A | l = (n – nCSI,ref ) for CSI reference slot for Rel-16 based doppler codebook is not supported | Per band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] |  |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-3-2-7; support of l=(n – nCSI,ref ) n+ for CSI reference slot for Rel-16 based doppler codebook; 1. Support of l=(n – nCSI,ref ) n+ for CSI reference slot when N4=1 and d>1 2. Support of l=(n – nCSI,ref ) n+ for CSI reference slot when N4>1 3. Regarding the start of CSI window, the maximum number of  is supported.; 40-3-2-1; Yes; N/A; l=(n – nCSI,ref ) n+ for CSI reference slot for Rel-16 based doppler codebook is not supported; Per band and per BC; N/A; N/A; N/A; Component 3 candidate values: {0, 1, 2}; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] |  |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] |  |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-2-7a | Support of l = (n – nCSI,ref ) for CSI reference slot for Rel-17 based doppler codebook | 1. Support of l = (n – nCSI,ref ) for CSI reference slot when N4=1 and d>1 | 40-3-2-4 | Yes | N/A | l = (n – nCSI,ref ) for CSI reference slot for Rel-17 based doppler codebook is not supported | Per band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] |  |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-3-2-7a; Support of l=(n – nCSI,ref ) n+ for CSI reference slot for Rel-17 based doppler codebook; 1. Support of l=(n – nCSI,ref ) n+ for CSI reference slot when N4=1 and d>1 2. Regarding the start of CSI window, the maximum number of  is supported.; 40-3-2-4; Yes; N/A; l=(n – nCSI,ref ) n+ for CSI reference slot for Rel-17 based doppler codebook is not supported; Per band and per BC; N/A; N/A; N/A; Component 2 candidate values: {0, 1, 2}; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] |  |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] |  |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-3-1 | TDCP (Time Domain Channel Properties) report | 1. Support of Y=1 delay value for TDCP report 2. Basic delay value, component candidate value <= D_basic = 1 slot 3. Support of amplitude report 4. Value of X for CPU occupation (OCPU=(Y+1).X) 5. Support to configure KTRS = 1 TRS resource set 6. Maximum number of simultaneously active CSI-RS resources for TDCP across all CCs Note: counting of simultaneously active CSI-RS resources follows existing specification TS38.214 | [2-35] | Yes | N/A | Time Domain Channel Properties report is not supported | [Per band] | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-3-3-1: TDCP (Time Domain Channel Properties) report - Value of X for CPU occupation (OCPU=(Y+1).X) - Candidate values for X: {1,2} - Maximum number of simultaneously active CSI-RS resources for TDCP across all CCs - Candidate values are {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} (includes all even numbers between 16 and 64) 40. NR_MIMO_evo_DL_UL; 40-3-3-1; TDCP (Time Domain Channel Properties) report; 1. Support of Y=1 delay value for TDCP report 2. Basic delay value, component candidate value <= D_basic = 1 slot 3. Support of amplitude report 4. Value of X for CPU occupation (OCPU=(Y+1).X) 5. Support to configure KTRS = 1 TRS resource set 6. Maximum number of simultaneously active CSI-RS resources for TDCP across all CCs Note: counting of simultaneously active CSI-RS resources follows existing specification TS38.214; [2-35]; Yes; N/A; Time Domain Channel Properties report is not supported; [Per band]; N/A; N/A; N/A; Component 4 Candidate values for X: {1,2} Component 6, candidate values {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} (includes all even numbers between 16 and 64); Optional with capability signaling. |
| Ericsson [3] | For this FG, confirm 2-35 as the pre-requisite. Furthermore, the reporting granularity should be “Per band”. 40-3-3-1; TDCP (Time Domain Channel Properties) report; 1. Support of Y=1 delay value for TDCP report 2. Basic delay value, component candidate value <= D_basic = 1 slot 3. Support of amplitude report 4. Value of X for CPU occupation (OCPU=(Y+1).X) 5. Support to configure KTRS = 1 TRS resource set 6. Maximum number of simultaneously active CSI-RS resources for TDCP across all CCs Note: counting of simultaneously active CSI-RS resources follows existing specification TS38.214; [2-35] [2-35]; Yes; N/A; Time Domain Channel Properties report is not supported; [Per band] Per band; N/A; N/A; N/A. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-3-3-1; TDCP (Time Domain Channel Properties) report; 1. Support of Y=1 delay value for TDCP report 2. Basic delay value, component candidate value <= D_basic = 1 slot 3. Support of amplitude report 4. Value of X for CPU occupation (OCPU=(Y+1).X) 5. Support to configure KTRS = 1 TRS resource set 6. Maximum number of simultaneously active CSI-RS resources for TDCP across all CCs Note: counting of simultaneously active CSI-RS resources follows existing specification TS38.214; [2-35]; Yes; N/A; Time Domain Channel Properties report is not supported; [Per band]; N/A; N/A; N/A; 4. Component candidate values: {1,2} 6. Component candidate values: {2,4,6,8, … 64}; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-3-3-1; TDCP (Time Domain Channel Properties) report; 1. Support of Y=1 delay value for TDCP report 2. Basic delay value, component candidate value <= D_basic = 1 slot 3. Support of amplitude report 4. Value of X for CPU occupation (OCPU=(Y+1).X) 5. Support to configure KTRS = 1 TRS resource set 6. Maximum number of simultaneously active CSI-RS resources for TDCP across all CCs Note: counting of simultaneously active CSI-RS resources follows existing specification TS38.214; [2-35]; Yes; N/A; Time Domain Channel Properties report is not supported; [Per band]; N/A; N/A; N/A; Optional with capability signaling. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-3-3-1; TDCP (Time Domain Channel Properties) report; 1. Support of Y=1 delay value for TDCP report 2. Basic delay value, component candidate value <= D_basic = 1 slot 3. Support of amplitude report 4. Value of X for CPU occupation (OCPU=(Y+1).X) 5. Support to configure KTRS = 1 TRS resource set 6. Maximum number of simultaneously active CSI-RS resources for TDCP across all CCs Note: counting of simultaneously active CSI-RS resources follows existing specification TS38.214; 2-35; Yes; N/A; Time Domain Channel Properties report is not supported; Per band; N/A; N/A; N/A; Optional with capability signaling. |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-3-3-1; TDCP (Time Domain Channel Properties) report; 1. Support of Y=1 delay value for TDCP report 2. Basic delay value, component candidate value <= D_basic = 1 slot 3. Support of amplitude report 4. Value of X for CPU occupation (OCPU=(Y+1).X) 5. Support to configure KTRS = 1 TRS resource set 6. Maximum number of simultaneously active CSI-RS resources for TDCP across all CCs Note: counting of simultaneously active CSI-RS resources follows existing specification TS38.214; [2-35]; Yes; N/A; Time Domain Channel Properties report is not supported; [Per band]; N/A; N/A; N/A; Candidate values of compentent 4, X={1,2}; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-3-3-1: - Prerequisite FG: Ok to confirm 2-35 - Type: per-band. In our view, this feature does not require “per-BC” granularity |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-3-3-1; TDCP (Time Domain Channel Properties) report; 1. Support of Y=1 delay value for TDCP report 2. Basic delay value, component candidate value <= D_basic = 1 slot 3. Support of amplitude report 4. Value of X for CPU occupation (OCPU=(Y+1).X) 5. Support to configure KTRS = 1 TRS resource set 6. Maximum number of simultaneously active CSI-RS resources for TDCP across all CCs Note: counting of simultaneously active CSI-RS resources follows existing specification TS38.214; [2-35]; Yes; N/A; Time Domain Channel Properties report is not supported; per band and per BC; N/A; N/A; N/A; Component 4 candidate values: {1,2} Component 6 candidate values: {2,4,6,8,12,16,20, 24,28,32,40,48,56,64}; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-3-3-1; TDCP (Time Domain Channel Properties) report; 1. Support of Y=1 delay value for TDCP report 2. Basic delay value, component candidate value <= D_basic = 1 slot 3. Support of amplitude report 4. Value of X for CPU occupation (OCPU=(Y+1).X) 5. Support to configure KTRS = 1 TRS resource set 6. Maximum number of simultaneously active CSI-RS resources for TDCP across all CCs Note: counting of simultaneously active CSI-RS resources follows existing specification TS38.214; [2-35]; Yes; N/A; Time Domain Channel Properties report is not supported; [Per band]; N/A; N/A; N/A; Optional with capability signaling. |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-3-2 | Number of delay values | Number Y>1 of delay values for which TDCP is reported | 40-3-3-1 | Yes | N/A | TDCP is not reported for more than 1 delay value | [Per-band Per-BC] | N/A | N/A | N/A | Candidate values: {2,3,4} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] | 40-3-3-2; Number of delay values; Number Y>1 of delay values for which TDCP is reported; 40-3-3-1; Yes; N/A; TDCP is not reported for more than 1 delay value; [Per-band Per-BC] Per Band; N/A; N/A; N/A; Candidate values: {2,3,4}. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-3-3-2; Number of delay values; Number Y>1 of delay values for which TDCP is reported; 40-3-3-1; Yes; N/A; TDCP is not reported for more than 1 delay value; [Per-band Per-BC] Per band; N/A; N/A; N/A; Candidate values: {2,3,4}; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-3-3-2; Number of delay values; Number Y>1 of delay values for which TDCP is reported; 40-3-3-1; Yes; N/A; TDCP is not reported for more than 1 delay value; Per- band; N/A; N/A; N/A; Candidate values: {2,3,4}; Optional with capability signaling. |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-3-3-2; Number of delay values; Number Y>1 of delay values for which TDCP is reported; 40-3-3-1; Yes; N/A; TDCP is not reported for more than 1 delay value; [Per-band Per-BC]; N/A; N/A; N/A; Candidate values: {2,3,4}; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-3-3-2; Number of delay values; Number Y>1 of delay values for which TDCP is reported; 40-3-3-1; Yes; N/A; TDCP is not reported for more than 1 delay value; per band and per BC; N/A; N/A; N/A; Candidate values: {2,3,4}; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] | The Number of delay values with phase reporting: The “Number of delay values” and “Phase report” are two other FGs agreed for TDCP. The current agreement allows the UE to signal capability for phase reporting for all TDCP coefficients, or for none of them. We propose allowing more flexibility in the capability report, where UEs can signal the number of delay values with phase reporting switched on in a separate FG. Proposal 11: Add a new component for the FG “Number of delay values” (FGI 40-3-3-2) to indicate the number of supported delay values with phase reporting switched ON: 40. NR_MIMO_evo_DL_UL; 40-3-3-2; Number of delay values; - Number Y>1 of delay values - Number Y≥1 of delay values with phase reporting switched on; Optional with UE capability. |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-3-3-2; Number of delay values; Number Y>1 of delay values for which TDCP is reported; 40-3-3-1; Yes; N/A; TDCP is not reported for more than 1 delay value; [Per-band Per-BC]; N/A; N/A; N/A; Candidate values: {2,3,4}; Optional with capability signaling. |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-3-1a | Supported maximum delay value larger than D_basic | Support of maximum delay value larger than D_basic =1 slot | 40-3-3-1 | Yes | N/A | delay value(s) larger than D_basic are not supported | [Per-band Per-BC] | N/A | N/A | N/A | Candidate values: {2 slots, 3 slots, 4 slots, 5 slots, 6 slots, 10 slots} [Note: 10 slots is only applicable for SCS >= 30 kHz, and 6 slots is maximum for SCS = 15 kHz] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] | 40-3-3-1a; Supported maximum delay value larger than D_basic; Support of maximum delay value larger than D_basic =1 slot; 40-3-3-1; Yes; N/A; delay value(s) larger than D_basic are not supported; [Per-band Per-BC] Per Band; N/A; N/A; N/A; Candidate values: {2 slots, 3 slots, 4 slots, 5 slots, 6 slots, 10 slots} [Note: 10 slots is only applicable for SCS >= 30 kHz, and 6 slots is maximum for SCS = 15 kHz]. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-3-3-1a; Supported maximum delay value larger than D_basic; Support of maximum delay value larger than D_basic =1 slot; 40-3-3-1; Yes; N/A; delay value(s) larger than D_basic are not supported; [Per-band Per-BC] Per band; N/A; N/A; N/A; Candidate values: {2 slots, 3 slots, 4 slots, 5 slots, 6 slots, 10 slots} [Note: 10 slots is only applicable for SCS >= 30 kHz, and, if 10 slots is reported, 6 slots is maximum for SCS = 15 kHz]; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-3-3-1a; Supported maximum delay value larger than D_basic; Support of maximum delay value larger than D_basic =1 slot; 40-3-3-1; Yes; N/A; delay value(s) larger than D_basic are not supported; Per-band; N/A; N/A; N/A; Candidate values: {2 slots, 3 slots, 4 slots, 5 slots, 6 slots, 10 slots} Note: 10 slots is only applicable for SCS >= 30 kHz, and 6 slots is maximum for SCS = 15 kHz; Optional with capability signaling. |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] | Accordinng to the following agreement, only when D=10, it requires that the SCS should be larger than 30 kHz. For D=6, there is no any additional resturiction. Agreement For the Rel-18 TRS-based TDCP reporting, regarding the value of parameter D, - Dbasic = 1 slot - Confirm the working assumption on the support for D=6 - Confirm the working assumption on the support for D=10 (only for >=30kHz SCS) 40. NR_MIMO_evo_DL_UL; 40-3-3-1a; Supported maximum delay value larger than D_basic; Support of maximum delay value larger than D_basic =1 slot; 40-3-3-1; Yes; N/A; delay value(s) larger than D_basic are not supported; [Per-band Per-BC]; N/A; N/A; N/A; Candidate values: {2 slots, 3 slots, 4 slots, 5 slots, 6 slots, 10 slots} [Note: 10 slots is only applicable for SCS >= 30 kHz, and 6 slots is maximum for SCS = 15 kHz]; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-3-3-1a: - Note: We believe the Note should be confirmed as it is the intention of the agreement - Type: per-band, following 40-3-3-1 |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-3-3-1a; Supported maximum delay value larger than D_basic; Support of maximum delay value larger than D_basic =1 slot; 40-3-3-1; Yes; N/A; delay value(s) larger than D_basic are not supported; per band and per BC; N/A; N/A; N/A; Candidate values: {2 slots, 3 slots, 4 slots, 5 slots, 6 slots, 10 slots} Note: 10 slots is only applicable for SCS >= 30 kHz, and 6 slots is maximum for SCS = 15 kHz; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-3-3-1a; Supported maximum delay values larger than D_basic; Support of maximum delay value larger than D_basic =1 slot; 40-3-3-1; Yes; N/A; delay value(s) larger than D_basic are not supported; [Per-band Per-BC]; N/A; N/A; N/A; Candidate values: {2 slots, 3 slots, 4 slots, 5 slots, 6 slots, 10 slots} [Note: 10 slots is only applicable for SCS >= 30 kHz, and 6 slots is maximum for SCS = 15 kHz]; Optional with capability signaling. |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-3-4 | Phase report | Support of phase report for Y>=1 | 40-3-3-1 | Yes | N/A | Phase report for Y>=1 is not supported | [Per-band Per-BC] | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] | 40-3-3-4; Phase report; Support of phase report for Y>=1; 40-3-3-1; Yes; N/A; Phase report for Y>=1 is not supported; [Per-band Per-BC] Per Band; N/A; N/A; N/A. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-3-3-4; Phase report; Support of phase report for Y>=1; 40-3-3-1; Yes; N/A; Phase report for Y>=1 is not supported; [Per-band Per-BC] Per band; N/A; N/A; N/A; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-3-3-4; Phase report; Support of phase report for Y>=1; 40-3-3-1; Yes; N/A; Phase report for Y>=1 is not supported; Per-band; N/A; N/A; N/A; Optional with capability signaling. |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-3-3-4; Phase report; Support of phase report for Y>=1; 40-3-3-1; Yes; N/A; Phase report for Y>=1 is not supported; [Per-band Per-BC]; N/A; N/A; N/A; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-3-3-4; Phase report; Support of phase report for Y>=1; 40-3-3-1; Yes; N/A; Phase report for Y>=1 is not supported; per band and per BC; N/A; N/A; N/A; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-3-3-4; Phase report; Support of phase report for Y>=1; 40-3-3-1; Yes; N/A; Phase report for Y>=1 is not supported; [Per-band Per-BC]; N/A; N/A; N/A; Optional with capability signaling. |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-3-5 | Number of CSI-RS resources for TDCP | 1. Maximum number of configured CSI-RS resources for TDCP per CC 2. Maximum number of configured CSI-RS resources for TDCP across all CCs 3. Maximum number of simultaneously active CSI-RS resources for TDCP per CC Note: counting of simultaneously active CSI-RS resources follows existing specification TS38.214 | 40-3-3-1 | Yes | N/A | Number of CSI-RS resources for TDCP is not reported | [Per-band Per-BC] | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-3-3-5: Number of CSI-RS resources for TDCP - Maximum number of configured CSI-RS resources for TDCP per CC - Candidates values are {1,2,..,32} - Maximum number of configured CSI-RS resources for TDCP across all CCs - Candidate vales are {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} (includes all even numbers between 16 and 64) - Maximum number of simultaneously active CSI-RS resources for TDCP per CC - Candidate vales are {1, 2, 3 …32} 40. NR_MIMO_evo_DL_UL; 40-3-3-5; Number of CSI-RS resources for TDCP; 1. Maximum number of configured CSI-RS resources for TDCP per CC 2. Maximum number of configured CSI-RS resources for TDCP across all CCs 3. Maximum number of simultaneously active CSI-RS resources for TDCP per CC Note: counting of simultaneously active CSI-RS resources follows existing specification TS38.214; 40-3-3-1; Yes; N/A; Number of CSI-RS resources for TDCP is not reported; [Per-band Per-BC]; N/A; N/A; N/A; Component 1, candidate values {1,2,..,32} Component 2, candidate values {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} (includes all even numbers between 16 and 64) Component 3, candidate values {1, 2, 3 …32}; Optional with capability signaling. |
| Ericsson [3] | 40-3-3-5; Number of CSI-RS resources for TDCP; 1. Maximum number of configured CSI-RS resources for TDCP per CC 2. Maximum number of configured CSI-RS resources for TDCP across all CCs 3. Maximum number of simultaneously active CSI-RS resources for TDCP per CC Note: counting of simultaneously active CSI-RS resources follows existing specification TS38.214; 40-3-3-1; Yes; N/A; Number of CSI-RS resources for TDCP is not reported; [Per-band Per-BC] Per Band; N/A; N/A; N/A. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-3-3-5; Number of CSI-RS resources for TDCP; 1. Maximum number of configured CSI-RS resources for TDCP per CC 2. Maximum number of configured CSI-RS resources for TDCP across all CCs 3. Maximum number of simultaneously active CSI-RS resources for TDCP per CC Note: counting of simultaneously active CSI-RS resources follows existing specification TS38.214; 40-3-3-1; Yes; N/A; Number of CSI-RS resources for TDCP is not reported; [Per-band Per-BC] Per band; N/A; N/A; N/A; 1. Component candidate values: {2, 4, 8, 10, 12, 16, 32, 64} 2. Component candidate values: {2, 4, 8, 10, 12, 16, 32, 64, 128} 3. Component candidate values: {2,4,6,8,10,12}; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-3-3-5; Number of CSI-RS resources for TDCP; 1. Maximum number of configured CSI-RS resources for TDCP per CC 2. Maximum number of configured CSI-RS resources for TDCP across all CCs 3. Maximum number of simultaneously active CSI-RS resources for TDCP per CC Note: counting of simultaneously active CSI-RS resources follows existing specification TS38.214; 40-3-3-1; Yes; N/A; Number of CSI-RS resources for TDCP is not reported; Per-band and Per BC; N/A; N/A; N/A; Optional with capability signaling. |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-3-3-5; Number of CSI-RS resources for TDCP; 1. Maximum number of configured CSI-RS resources for TDCP per CC 2. Maximum number of configured CSI-RS resources for TDCP across all CCs 3. Maximum number of simultaneously active CSI-RS resources for TDCP per CC Note: counting of simultaneously active CSI-RS resources follows existing specification TS38.214; 40-3-3-1; Yes; N/A; Number of CSI-RS resources for TDCP is not reported; [Per-band Per-BC]; N/A; N/A; N/A; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-3-3-5; Number of CSI-RS resources for TDCP; 1. Maximum number of configured CSI-RS resources for TDCP per CC 2. Maximum number of configured CSI-RS resources for TDCP across all CCs 3. Maximum number of simultaneously active CSI-RS resources for TDCP per CC Note: counting of simultaneously active CSI-RS resources follows existing specification TS38.214; 40-3-3-1; Yes; N/A; Number of CSI-RS resources for TDCP is not reported; per band and per BC; N/A; N/A; N/A; Component 1 candidate values: {2,4,6,8} Component 2 candidate values: {2,4,6,8,12,16} Component 3 candidate values: {2,4,6,8,12,16,20,24,28,32}; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-3-3-5; Number of CSI-RS resources for TDCP; 1. Maximum number of configured CSI-RS resources for TDCP per CC 2. Maximum number of configured CSI-RS resources for TDCP across all CCs 3. Maximum number of simultaneously active CSI-RS resources for TDCP per CC Note: counting of simultaneously active CSI-RS resources follows existing specification TS38.214; 40-3-3-1; Yes; N/A; Number of CSI-RS resources for TDCP is not reported; [Per-band Per-BC]; N/A; N/A; N/A; Optional with capability signaling. |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-3-6 | Maximum number of TRS resource sets in a report configuration | Max number of TRS resource sets in a single CSI-RS resource setting | 40-3-3-1 | Yes | N/A | More than 1 TRS resource set in a report configuration is not supported | [Per-band Per-BC] | N/A | N/A | N/A | Candidate values: {2,3} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-3-3-6; Maximum number of TRS resource sets in a report configuration; Max number of TRS resource sets in a single CSI-RS resource setting; 40-3-3-1; Yes; N/A; More than 1 TRS resource set in a report configuration is not supported; [Per-band Per-BC]; N/A; N/A; N/A; Candidate values: {1, 2,3}; Optional with capability signaling. |
| Ericsson [3] | 40-3-3-6; Maximum number of TRS resource sets in a report configuration; Max number of TRS resource sets in a single CSI-RS resource setting; 40-3-3-1; Yes; N/A; More than 1 TRS resource set in a report configuration is not supported; [Per-band Per-BC] Per Band; N/A; N/A; N/A; Candidate values: {2,3}. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-3-3-6; Maximum number of TRS resource sets in a report configuration; Max number of TRS resource sets in a single CSI-RS resource setting; 40-3-3-1; Yes; N/A; More than 1 TRS resource set in a report configuration is not supported; [Per-band Per-BC] Per band; N/A; N/A; N/A; Candidate values: {2,3}; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-3-3-6; Maximum number of TRS resource sets in a report configuration; Max number of TRS resource sets in a single CSI-RS resource setting; 40-3-3-1; Yes; N/A; More than 1 TRS resource set in a report configuration is not supported; Per-band; N/A; N/A; N/A; Candidate values: {2,3}; Optional with capability signaling. |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-3-3-6; Maximum number of TRS resource sets in a report configuration; Max number of TRS resource sets in a single CSI-RS resource setting; 40-3-3-1; Yes; N/A; More than 1 TRS resource set in a report configuration is not supported; [Per-band Per-BC]; N/A; N/A; N/A; Candidate values: {2,3}; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-3-3-6; Maximum number of TRS resource sets in a report configuration; Max number of TRS resource sets in a single CSI-RS resource setting; 40-3-3-1; Yes; N/A; More than 1 TRS resource set in a report configuration is not supported; per band and per BC; N/A; N/A; N/A; Candidate values: {2,3}; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-3-3-6; Maximum number of TRS resource sets in a report configuration; Max number of TRS resource sets in a single CSI-RS resource setting; 40-3-3-1; Yes; N/A; More than 1 TRS resource set in a report configuration is not supported; [Per-band Per-BC]; N/A; N/A; N/A; Candidate values: {2,3}; Optional with capability signaling. |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-4-1 | Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type A | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol | [2-5] | Yes | n/a |  | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-4-1; Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type A; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol; [2-5]; Yes; n/a; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Ericsson [3] | 40-4-1; Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type A; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol; [2-5]; [Per FS]. |
| ZTE [4] | - For both FG 40-4-1 and FG 40-4-2, - The prerequisite FG can be FG 2-5. - Granularity should be “Per FS” to keep alignment with that of its prerequisite FG 2-5. - For both FG 40-4-1 and FG 40-4-1a, - The prerequisite FG can be FG 2-5. - Granularity should be “Per band” to keep alignment with that of its prerequisite FG 2-5. 40. NR_MIMO_evo_DL_UL; 40-4-1; Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type A; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol; [2-5] 2-5; Yes; n/a; [Per FS] Per FS; No; No; n/a; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-4-1: Rel-18 DMRS is enhanced based on Rel-15 DMRS (corresponding to FG 2-5 for PDSCH for mapping type A) and FG 2-5 is mandatory without capability signaling. Therefore, we propose that FG 2-5 is the prerequisite feature group of 40-4-1. |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-4-1; Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type A; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol; [2-5]; Yes; n/a; Per FS; No; No; n/a; Optional with capability signaling. |
| Samsung [16] | - For FG 40-4-1 (Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type A), we are basically fine with having FG 2-5 (Basic downlink DMRS for scheduling type A) only as a pre-requisite, but we are open to discuss whether to need more pre-requisites. Per FS |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-4-1; Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type A; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol; [2-5]; Yes; n/a; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Huawei/HiSilicon [19] | Similar to Rel-15 DMRS FGs, in our view, at least FG 40-4-1, FG 40-4-1j, FG 40-4-1a, FG 40-4-1b, FG 40-4-1c, FG 40-4-1e, FG 40-4-1f, FG 40-4-5, FG 40-4-7 should be Per FS. Proposal 4-4: The reporting granularity of FG 40-4-1, FG 40-4-1j, FG 40-4-1a, FG 40-4-1b, FG 40-4-1c, FG 40-4-1e, FG 40-4-1f, FG 40-4-5, FG 40-4-7 should be Per FS. 40. NR_MIMO_evo_DL_UL; 40-4-1; Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type A; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol; [2-5]; Yes; n/a; [Per FS]; No; No; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-4-1a | Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type B | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol | [2-5] | Yes | n/a | UE does not support basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type B | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-4-1a; Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type B; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol; [2-56]; Yes; n/a; UE does not support basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type B; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Ericsson [3] | 40-4-1a; Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type B; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol; [2-5]; [Per FS]. |
| ZTE [4] | - For both FG 40-4-1 and FG 40-4-1a, - The prerequisite FG can be FG 2-5. - Granularity should be “Per band” to keep alignment with that of its prerequisite FG 2-5. 40. NR_MIMO_evo_DL_UL; 40-4-1a; Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type B; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol; [2-5] 2-5; Yes; n/a; UE does not support basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type B; [Per FS] Per FS; No; No; n/a; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-4-1a: Similar to FG 40-4-1, FG 2-6 is the prerequisite feature group of 40-4-1 for PDSCH for mapping type B. |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-4-1a - Pre-requisite FG: It should be FG2-6 (TypeB), not FG2-5 (TypeA). |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-4-1a; Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type B; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol; [2-5]; Yes; n/a; UE does not support basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type B; Per FS; No; No; n/a; Optional with capability signaling. |
| Samsung [16] | - For FG 40-4-1a (Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type B), we are basically fine with having FG 2-6 (Basic downlink DMRS for scheduling type B) only as a pre-requisite, but we are open to discuss whether to need more pre-requisites. Per FS |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-4-1a; Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type B; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol; [2-5]; Yes; n/a; UE does not support basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type B; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Huawei/HiSilicon [19] | Similar to Rel-15 DMRS FGs, in our view, at least FG 40-4-1, FG 40-4-1j, FG 40-4-1a, FG 40-4-1b, FG 40-4-1c, FG 40-4-1e, FG 40-4-1f, FG 40-4-5, FG 40-4-7 should be Per FS. Proposal 4-4: The reporting granularity of FG 40-4-1, FG 40-4-1j, FG 40-4-1a, FG 40-4-1b, FG 40-4-1c, FG 40-4-1e, FG 40-4-1f, FG 40-4-5, FG 40-4-7 should be Per FS. 40. NR_MIMO_evo_DL_UL; 40-4-1a; Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type B; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol; [2-5]; Yes; n/a; UE does not support basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type B; [Per FS]; No; No; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-4-1b | 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH | Support of 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH | 40-4-1 | Yes | n/a | UE does not support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-4-1b; 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH; Support of 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1; Yes; n/a; UE does not support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Ericsson [3] | 40-4-1b; 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH; Support of 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1; [Per FS]. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-4-1b; 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH; Support of 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1; Yes; n/a; UE does not support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH; [Per FS] Per FS; No; No; n/a; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-4-1b: Per FS. |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-4-1b; 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH; Support of 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1; Yes; n/a; UE does not support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH; Per FS; No; No; n/a; Optional with capability signaling. |
| Samsung [16] | Per FS |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-4-1b; 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH; Support of 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1; Yes; n/a; UE does not support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Huawei/HiSilicon [19] | Similar to Rel-15 DMRS FGs, in our view, at least FG 40-4-1, FG 40-4-1j, FG 40-4-1a, FG 40-4-1b, FG 40-4-1c, FG 40-4-1e, FG 40-4-1f, FG 40-4-5, FG 40-4-7 should be Per FS. Proposal 4-4: The reporting granularity of FG 40-4-1, FG 40-4-1j, FG 40-4-1a, FG 40-4-1b, FG 40-4-1c, FG 40-4-1e, FG 40-4-1f, FG 40-4-5, FG 40-4-7 should be Per FS. 40. NR_MIMO_evo_DL_UL; 40-4-1b; 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH; Support of 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1; Yes; n/a; UE does not support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH; [Per FS]; No; No; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-4-1c | Alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH | Support of alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH | 40-4-1, 5-28 | Yes | n/a | UE does not support alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-4-1c; Alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH; Support of alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1, 5-28; Yes; n/a; UE does not support alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Ericsson [3] | 40-4-1c; Alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH; Support of alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1, 5-28; [Per FS]. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-4-1c; Alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH; Support of alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1, 5-28; Yes; n/a; UE does not support alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH; [Per FS] Per FS; No; No; n/a; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-4-1c: Per FS. |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-4-1c; Alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH; Support of alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1, 5-28; Yes; n/a; UE does not support alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH; Per FS; No; No; n/a; Optional with capability signaling. |
| Samsung [16] | Per FS |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-4-1c; Alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH; Support of alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1, 5-28; Yes; n/a; UE does not support alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Huawei/HiSilicon [19] | Similar to Rel-15 DMRS FGs, in our view, at least FG 40-4-1, FG 40-4-1j, FG 40-4-1a, FG 40-4-1b, FG 40-4-1c, FG 40-4-1e, FG 40-4-1f, FG 40-4-5, FG 40-4-7 should be Per FS. Proposal 4-4: The reporting granularity of FG 40-4-1, FG 40-4-1j, FG 40-4-1a, FG 40-4-1b, FG 40-4-1c, FG 40-4-1e, FG 40-4-1f, FG 40-4-5, FG 40-4-7 should be Per FS. 40. NR_MIMO_evo_DL_UL; 40-4-1c; Alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH; Support of alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1, 5-28; Yes; n/a; UE does not support alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH; [Per FS]; No; No; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-4-1d | 2 symbols FL-DMRS for Rel.18 enhanced DMRS ports for PDSCH | Support of 2 symbols FL-DMRS for Rel.18 enhanced DMRS ports for PDSCH | 40-4-1 | Yes | n/a | UE does not support 2 symbols FL-DMRS for Rel.18 enhanced DMRS ports for PDSCH | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-4-1d; 2 symbols FL-DMRS for Rel.18 enhanced DMRS ports for PDSCH; Support of 2 symbols FL-DMRS for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1; Yes; n/a; UE does not support 2 symbols FL-DMRS for Rel.18 enhanced DMRS ports for PDSCH; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Ericsson [3] | 40-4-1d; 2 symbols FL-DMRS for Rel.18 enhanced DMRS ports for PDSCH; Support of 2 symbols FL-DMRS for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1; [Per FS] Per UE. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-4-1d; 2 symbols FL-DMRS for Rel.18 enhanced DMRS ports for PDSCH; Support of 2 symbols FL-DMRS for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1; Yes; n/a; UE does not support 2 symbols FL-DMRS for Rel.18 enhanced DMRS ports for PDSCH; [Per FS] Per FS; No; No; n/a; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-4-1d: Per FS. |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-4-1d; 2 symbols FL-DMRS for Rel.18 enhanced DMRS ports for PDSCH; Support of 2 symbols FL-DMRS for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1; Yes; n/a; UE does not support 2 symbols FL-DMRS for Rel.18 enhanced DMRS ports for PDSCH; Per FS; No; No; n/a; Optional with capability signaling. |
| Samsung [16] | Per FS |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-4-1d; 2 symbols FL-DMRS for Rel.18 enhanced DMRS ports for PDSCH; Support of 2 symbols FL-DMRS for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1; Yes; n/a; UE does not support 2 symbols FL-DMRS for Rel.18 enhanced DMRS ports for PDSCH; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-4-1d; 2 symbols FL-DMRS for Rel.18 enhanced DMRS ports for PDSCH; Support of 2 symbols FL-DMRS for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1; Yes; n/a; UE does not support 2 symbols FL-DMRS for Rel.18 enhanced DMRS ports for PDSCH; [Per FS]; No; No; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-4-1e | 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH | Support of 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH | 40-4-1 | Yes | n/a | UE does not support 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-4-1e; 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH; Support of 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1; Yes; n/a; UE does not support 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Ericsson [3] | 40-4-1e; 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH; Support of 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1; [Per FS]. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-4-1e; 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH; Support of 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1; Yes; n/a; UE does not support 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH; [Per FS] Per FS; No; No; n/a; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-4-1e: Per FS. 40. NR_MIMO_evo_DL_UL; 40-4-1e; 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH; Support of 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1; Yes; n/a; UE does not support 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-4-1e; 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH; Support of 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1; Yes; n/a; UE does not support 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH; Per FS; No; No; n/a; Optional with capability signaling. |
| Samsung [16] | Per FS |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] |  |
| Huawei/HiSilicon [19] | Similar to Rel-15 DMRS FGs, in our view, at least FG 40-4-1, FG 40-4-1j, FG 40-4-1a, FG 40-4-1b, FG 40-4-1c, FG 40-4-1e, FG 40-4-1f, FG 40-4-5, FG 40-4-7 should be Per FS. Proposal 4-4: The reporting granularity of FG 40-4-1, FG 40-4-1j, FG 40-4-1a, FG 40-4-1b, FG 40-4-1c, FG 40-4-1e, FG 40-4-1f, FG 40-4-5, FG 40-4-7 should be Per FS. 40. NR_MIMO_evo_DL_UL; 40-4-1e; 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH; Support of 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1; Yes; n/a; UE does not support 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH; [Per FS]; No; No; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-4-1f | 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH | Support of 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH | 40-4-1 | Yes | n/a | UE does not support 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-4-1f; 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH; Support of 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1; Yes; n/a; UE does not support 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Ericsson [3] | 40-4-1f; 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH; Support of 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1; [Per FS]. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-4-1f; 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH; Support of 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1; Yes; n/a; UE does not support 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH; [Per FS] Per FS; No; No; n/a; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-4-1f: Per FS. |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-4-1f; 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH; Support of 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1; Yes; n/a; UE does not support 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH; Per FS; No; No; n/a; Optional with capability signaling. |
| Samsung [16] | Per FS |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-4-1f; 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH; Support of 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1; Yes; n/a; UE does not support 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Huawei/HiSilicon [19] | Similar to Rel-15 DMRS FGs, in our view, at least FG 40-4-1, FG 40-4-1j, FG 40-4-1a, FG 40-4-1b, FG 40-4-1c, FG 40-4-1e, FG 40-4-1f, FG 40-4-5, FG 40-4-7 should be Per FS. Proposal 4-4: The reporting granularity of FG 40-4-1, FG 40-4-1j, FG 40-4-1a, FG 40-4-1b, FG 40-4-1c, FG 40-4-1e, FG 40-4-1f, FG 40-4-5, FG 40-4-7 should be Per FS. 40. NR_MIMO_evo_DL_UL; 40-4-1f; 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH; Support of 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1; Yes; n/a; UE does not support 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH; [Per FS]; No; No; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-4-1g | DMRS type for Rel.18 enhanced DMRS ports for PUSCH | Support of DMRS type for Rel.18 enhanced DMRS ports for PUSCH | 40-4-1 | Yes | n/a | UE does not support DMRS type for Rel.18 enhanced DMRS ports for PUSCH | [Per FS] | No | No | n/a | Component 1 candidate values: {etype 1, both etype 1 and etype 2} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-4-1g; DMRS type for Rel.18 enhanced DMRS ports for PUSCH; Support of DMRS type for Rel.18 enhanced DMRS ports for PUSCH; 40-4-1; Yes; n/a; UE does not support DMRS type for Rel.18 enhanced DMRS ports for PUSCH; [Per FS]; No; No; n/a; Component 1 candidate values: {etype 1, both etype 1 and etype 2}; Optional with capability signaling. |
| Ericsson [3] | 40-4-1g; DMRS type for Rel.18 enhanced DMRS ports for PUSCH PDSCH; Support of DMRS type for Rel.18 enhanced DMRS ports for PUSCH PDSCH; 40-4-1; [Per FS] Per UE. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-4-1g; DMRS type for Rel.18 enhanced DMRS ports for PUSCH; Support of DMRS type for Rel.18 enhanced DMRS ports for PUSCH; 40-4-1; Yes; n/a; UE does not support DMRS type for Rel.18 enhanced DMRS ports for PUSCH; [Per FS] Per FS; No; No; n/a; Component 1 candidate values: {etype 1, both etype 1 and etype 2}; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-4-1g; DMRS type for Rel.18 enhanced DMRS ports for PUSCH PDSCH; Support of DMRS type for Rel.18 enhanced DMRS ports for PUSCH PDSCH; 40-4-1; Yes; n/a; UE does not support DMRS type for Rel.18 enhanced DMRS ports for PUSCH PDSCH; [Per FS]; No; No; n/a; Component 1 candidate values: {etype 1, both etype 1 and etype 2} Note: A UE supporting one of FG 40-4-6 or FG 40-4-6a must support one of candidate values of this FG Comment: Add the note like 40-4-6c.; Optional with capability signaling. |
| Intel Corporation [7] |  |
| OPPO [8] | 40. NR_MIMO_evo_DL_UL; 40-4-1g; DMRS type for Rel.18 enhanced DMRS ports for PDSCH; Support of DMRS type for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1; Yes; n/a; UE does not support DMRS type for Rel.18 enhanced DMRS ports for PDSCH; [Per FS]; No; No; n/a; Component 1 candidate values: {etype 1, both etype 1 and etype 2}; Optional with capability signaling. |
| CATT [9] | 40-4-1g: Per UE. 40-1-g: - According to the context, FG 40-1-g should be DMRS type for PDSCH. Therefore, the following modification is proposed. Proposal 9: For FG 40-1-g, change PUSCH to PDSCH. |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-4-1g - Note: If UE supports Rel.18 DMRS ports but not report this FG, it is not clear which DMRS type UE supports. Hence, we suggest to add the following note, to avoid the case UE does not report this FG: - Note: A UE supporting at least one of FG 40-4-1 or FG 40-4-1a must support this FG. - Pre-requisite FG: FG2-5 is fine. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-4-1g; DMRS type for Rel.18 enhanced DMRS ports for PUSCH; Support of DMRS type for Rel.18 enhanced DMRS ports for PUSCH; 40-4-1; Yes; n/a; UE does not support DMRS type for Rel.18 enhanced DMRS ports for PUSCH; Per FS; No; No; n/a; Component 1 candidate values: {etype 1, both etype 1 and etype 2}; Optional with capability signaling. |
| Samsung [16] | Per FS |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-4-1g; DMRS type for Rel.18 enhanced DMRS ports for PUSCH PDSCH; Support of DMRS type for Rel.18 enhanced DMRS ports for PUSCH PDSCH; 40-4-1; Yes; n/a; UE does not support DMRS type for Rel.18 enhanced DMRS ports for PUSCH PDSCH; [Per FS]; No; No; n/a; Component 1 candidate values: {etype 1, both etype 1 and etype 2}; Optional with capability signaling. |
| Huawei/HiSilicon [19] | Besides, in FG 40-4-1g, there is a typo, “PUSCH” should be “PDSCH”. Proposal 4-5: “PUSCH” in FG 40-401g should be changed to “PDSCH”. 40. NR_MIMO_evo_DL_UL; 40-4-1g; DMRS type for Rel.18 enhanced DMRS ports for PDSCH PUSCH; Support of DMRS type for Rel.18 enhanced DMRS ports for PDSCH PUSCH; 40-4-1; Yes; n/a; UE does not support DMRS type for Rel.18 enhanced DMRS ports for PUSCH; [Per FS]; No; No; n/a; Component 1 candidate values: {etype 1, both etype 1 and etype 2}; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-4-1h | 1 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8 | Support of 1 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8 | 40-4-1 | Yes | n/a | 1 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8 is not supported | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-4-1h; 1 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8; Support of 1 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8; 40-4-1; Yes; n/a; 1 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8 is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Ericsson [3] | 40-4-1h; 1 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8; Support of 1 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8; 40-4-1; [Per FS] Per UE. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-4-1h; 1 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8; Support of 1 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8; 40-4-1; Yes; n/a; 1 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8 is not supported; [Per FS] Per FS; No; No; n/a; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-4-1h: Per band. |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-4-1h - Note: Since 1-port DL PTRS (FG2-46) is mandatory in FR2 in Rel.15, we suggest to add the following note, to avoid the situation UE does not support this FG in FR2: - Note: A UE supporting FG2-46 and at least one of FG 40-4-1 or FG 40-4-1a must support this FG. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-4-1h; 1 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8; Support of 1 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8; 40-4-1; Yes; n/a; 1 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8 is not supported; Per FS; No; No; n/a; Optional with capability signaling. |
| Samsung [16] | Per FS |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-4-1h; 1 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8; Support of 1 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8; 40-4-1; Yes; n/a; 1 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8 is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-4-1h; 1 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8; Support of 1 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8; 40-4-1; Yes; n/a; 1 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8 is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-4-1i | 2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8 | Support of 2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8 | 40-4-1, FFS more | Yes | n/a | 2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8 is not supported | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-4-1i : 2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8 - This is for S-DCI mTRP. mTRP supports upto 4 layers only . Thus, “with rank 1-8” can be removed. We recommend adding a clarification of “for S-DCI based SDM scheme” at the end of the feature description 40. NR_MIMO_evo_DL_UL; 40-4-1i; 2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8; Support of 2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8for S-DCI based SDM scheme; 40-4-1, FFS more 40-4-5, 16-2b-1a; Yes; n/a; 2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8 is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Ericsson [3] | 40-4-1i; 2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8; Support of 2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8; 40-4-1, FFS more40-4-1h; [Per FS] Per Band. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-4-1i; 2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8; Support of 2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8; 40-4-1, FFS more; Yes; n/a; 2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8 is not supported; [Per FS] Per FS; No; No; n/a; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-4-1i: In Rel-16, UE supports 2-port DL PTRS when the UE performs single DCI based M-TRP. In Rel-18, support of 2-port DL PTRS for Rel-18 enhanced DMRS ports for PDSCH with rank 1-8 also requires that the UE support 2-port DL PTRS and supports single DCI based M-TRP. Therefore, the prerequisite feature group includes FG 16-2b-1a. 40-4-1i: Per band. |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-4-1i; 2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8; Support of 2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8; 40-4-1, FFS more; Yes; n/a; 2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8 is not supported; Per FS; No; No; n/a; Optional with capability signaling. |
| Samsung [16] | - For FG 40-4-1i (2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8), in addition to FG 40-4-1, having FG 16-2b-1a as one of pre-requisites is needed since the FG 16-2b-1a is the only FG for supporting 2-port DL PTRS. - Per FS |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-4-1i; 2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8; Support of 2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8; 40-4-1, FFS more; Yes; n/a; 2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8 is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-4-1i; 2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8; Support of 2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8; 40-4-1, FFS more; Yes; n/a; 2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8 is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-4-1j | Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port for mapping type A | Support of Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port for mapping type A | 40-4-1 | Yes | n/a | Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port is not supported for mapping type A | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-4-1j; Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port for mapping type A; Support of Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port for mapping type A; 40-4-1; Yes; n/a; Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port is not supported for mapping type A; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Ericsson [3] | 40-4-1j; Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port for mapping type A (downlink); Support of Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port for mapping type A; 40-4-1; [Per FS]. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-4-1j; Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port for mapping type A; Support of Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port for mapping type A; 40-4-1; Yes; n/a; Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port is not supported for mapping type A; [Per FS] Per FS; No; No; n/a; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-4-1j: Per band. |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-4-1j; Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port for mapping type A; Support of Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port for mapping type A; 40-4-1; Yes; n/a; Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port is not supported for mapping type A; Per FS; No; No; n/a; Optional with capability signaling. |
| Samsung [16] | Per FS |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-4-1j; Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port for mapping type A; Support of Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port for mapping type A; 40-4-1; Yes; n/a; Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port is not supported for mapping type A; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Huawei/HiSilicon [19] | Similar to Rel-15 DMRS FGs, in our view, at least FG 40-4-1, FG 40-4-1j, FG 40-4-1a, FG 40-4-1b, FG 40-4-1c, FG 40-4-1e, FG 40-4-1f, FG 40-4-5, FG 40-4-7 should be Per FS. Proposal 4-4: The reporting granularity of FG 40-4-1, FG 40-4-1j, FG 40-4-1a, FG 40-4-1b, FG 40-4-1c, FG 40-4-1e, FG 40-4-1f, FG 40-4-5, FG 40-4-7 should be Per FS. 40. NR_MIMO_evo_DL_UL; 40-4-1j; Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port for mapping type A; Support of Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port for mapping type A; 40-4-1; Yes; n/a; Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port is not supported for mapping type A; [Per FS]; No; No; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-4-4 | Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports | Support reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports | 40-4-1 | Yes | n/a | Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports is not supported | [Per FS] | No | No | n/a | Note: If this feature is not supported, UE expects that gNB shall apply at least the following scheduling restriction for PDSCH for FD-OCC 4 in Rel.18 eType 1 DMRS 1) The number of consecutively scheduled PRBs for PDSCH is even 2) The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) is even | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-4-4 - Updated the notes 2 as ” The number of PRBs offset of each set of consecutively scheduled PDSCH from point A (common resource block 0) is even” based on the agreement and text proposal below Agreement(RAN1#114bis Xiamen) Adopt the following text proposal in TS38.214 v18.0.0. - Reason for change: The scheduling restriction of orphan RE or eType1 is specified in current TS 38.214 v18.0.0 clause 5.1.6.2. However, the current scheduling restriction cannot ensure orthogonality because the PRBs not available for PDSCH are variable (e.g. PRBs not available for PDSCH declared by RateMatchPattern are configured with 1RB granularity and a symbol level bitmap, leading to the second restriction can’t avoid some orphan RE cases caused by PDSCH rate matching). - Summary of change: Clarify the offset of the scheduling restriction is the offset of each set of consecutively scheduled PRBs. - Consequences if not approved: Rel-18 eType 1 DMRS ports cannot be orthogonal for scheduled in MU-MIMO scenario for some cases. #### 5.1.6.2 DM-RS reception procedure < Unchanged parts are omitted > For DM-RS configuration enhanced type 1, when UE is not indicating UE capability of [noSchedulingRestriction-r18], the UE shall assume the number of consecutively scheduled PRBs are even, and the offset of the each set of consecutively scheduled PRBs from common resource block 0 is even number. < Unchanged parts are omitted >.. 40. NR_MIMO_evo_DL_UL; 40-4-4; Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports; Support reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports; 40-4-1; Yes; n/a; Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports is not supported; [Per FS]; No; No; n/a; Note: If this feature is not supported, UE expects that gNB shall apply at least the following scheduling restriction for PDSCH for FD-OCC 4 in Rel.18 eType 1 DMRS 1) The number of consecutively scheduled PRBs for PDSCH is even 2) The number of PRBs offset of each set of consecutively scheduled PDSCH from point A (common resource block 0) is even; Optional with capability signaling. |
| Ericsson [3] | 40-4-4; Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports; Support reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports; 40-4-1; [Per FS]. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-4-4; Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports; Support reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports; 40-4-1; Yes; n/a; Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports is not supported; [Per FS] Per FS; No; No; n/a; Note: If this feature is not supported, UE expects that gNB shall apply at least the following scheduling restriction for PDSCH for FD-OCC 4 in Rel.18 eType 1 DMRS 1) The number of consecutively scheduled PRBs for PDSCH is even 2) The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) is even; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-4-4; Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports; Support reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports; 40-4-1; Yes; n/a; Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports is not supported; [Per FS]; No; No; n/a; Note: If this feature is not supported, UE expects that gNB shall apply at least both the following scheduling restriction for PDSCH for FD-OCC 4 in Rel.18 eType 1 DMRS 1) The number of consecutively scheduled PRBs for PDSCH is even 2) The number of PRBs offset of each set of consecutively scheduled PDSCH from point A (common resource block 0) is even Comment: - “at least” should be modified as ‘both” as agreed in RAN1 - “each set of consecutively” should be added as agreed in RAN1; Optional with capability signaling. |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-4-4: Per UE. |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-4-4; Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports; Support reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports; 40-4-1; Yes; n/a; Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports is not supported; Per FS; No; No; n/a; Note: If this feature is not supported, UE expects that gNB shall apply at least the following scheduling restriction for PDSCH for FD-OCC 4 in Rel.18 eType 1 DMRS 1) The number of consecutively scheduled PRBs for PDSCH is even 2) The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) is even; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-4-4a; Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports for PDSCH with fdmSchemeA; Support reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports for PDSCH with fdmSchemeA; 40-4-1, 23-6-1/1b; Yes; n/a; Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports for PDSCH with fdmSchemeA is not supported; Per FS; No; No; n/a; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-4-4b; Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports for PDSCH with fdmSchemeB; Support reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports for PDSCH with fdmSchemeB; 40-4-1, 23-6-2/2b; Yes; n/a; Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports for PDSCH with fdmSchemeB is not supported; Per FS; No; No; n/a; Optional with capability signaling. |
| Samsung [16] | In RAN1#114 [4], a scheduling restriction handling orphan RE problem was extended for PDSCH transmission with fdmSchemeA or fdmSchemeB by extending such original scheduling restrictions as per each of indicated TCI states as follows: Agreement in RAN1#114 If UE does not support the orphan RE capability (i.e. UE can receive PDSCH without the scheduling restriction for FD-OCC length 4 in Rel.18 eType 1 DMRS), all the following scheduling restriction is applied for PDSCH transmission with fdmSchemeA or fdmSchemeB: - 1) The number of consecutively scheduled PRBs for PDSCH for each TCI-state is even. - If the precoding granularity is set to ‘wideband’, the total number of PRBs allocated to UE should be multiple of 4 to ensure the number of consecutively scheduled PRBs for PDSCH for each TCI-state is even. - 2) The number of PRBs offset of scheduled PDSCH for each TCI-state from point A (common resource block 0) is even.. Since pre-requisite FG would be different, if this extension on single-DCI based multi-TRP with FDM scheme A or B is defined, then separate FG can be defined as follows. For type, to avoid defining capabilities with re-requisite on a finer granularity [3] based on RAN2’s guidance, we are fine with per FSPC since FG 16-2b-2 (single-DCI based fdmSchemeA) is per band and FG 16-2b-3 (single-DCI based fdmSchemeB) is per FSPC. Proposal 12: Support the FGs considering single-DCI based PDSCH transmission with fdmSchemeA or fdmSchemeB whether a UE can handle orphan RE problem or not. 40-4-4; Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports; Support reception of PDSCH without the scheduling restriction for Rel.18 enhanced DMRS type 1; 40-4-1; Per FS; Note: If this FG is not reported, UE expects that gNB shall apply the following frequency domain scheduling restrictions for PDSCH for Rel-18 DMRS eType-I. - The number of consecutively scheduled PRBs for PDSCH is even. - The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) is even.. 40-4-4a; Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports with single-DCI based fdmSchemeA or fdmSchemeB; Support reception of PDSCH without the scheduling restriction for Rel.18 enhanced DMRS type 1 with single-DCI based fdmSchemeA or fdmSchemeB; 16-2b-2, and/or 16-2b-3, 40-4-5; Per FSPC; Note: If this FG is not reported, UE expects that gNB shall apply the following frequency domain scheduling restrictions for PDSCH for Rel-18 DMRS eType-I. - The number of consecutively scheduled PRBs per TCI state for PDSCH is even. - If the precoding granularity is set to ‘wideband’, the total number of PRBs allocated to UE should be multiple of 4 to ensure the number of consecutively scheduled PRBs for PDSCH for each TCI-state is even. - The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) per TCI state is even.. |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-4-4; Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports; Support reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports; 40-4-1; Yes; n/a; Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports is not supported; [Per FS]; No; No; n/a; Note: If this feature is not supported, UE expects that gNB shall apply at least the following scheduling restriction for PDSCH for FD-OCC 4 in Rel.18 eType 1 DMRS 1) The number of consecutively scheduled PRBs for PDSCH is even 2) The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) is even; Optional with capability signaling. |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-4-4; Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports; Support reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports; 40-4-1; Yes; n/a; Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports is not supported; [Per FS]; No; No; n/a; Note: If this feature is not supported, UE expects that gNB shall apply at least the following scheduling restriction for PDSCH for FD-OCC 4 in Rel.18 eType 1 DMRS 1) The number of consecutively scheduled PRBs for PDSCH is even 2) The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) is even; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-4-5 | Rel-18 DL DMRS with single DCI based M-TRP | Support of Rel-18 DL DMRS with single DCI based M-TRP | 40-4-1, FFS more | Yes | n/a | Rel-18 DL DMRS with single DCI based M-TRP is not supported | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-4-5; Rel-18 DL DMRS with single DCI based M-TRP; Support of Rel-18 DL DMRS with single DCI based M-TRP; 40-4-1, FFS more16-2b-1; Yes; n/a; Rel-18 DL DMRS with single DCI based M-TRP is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Ericsson [3] | 40-4-5; Rel-18 DL DMRS with single DCI based M-TRP; Support of Rel-18 DL DMRS with single DCI based M-TRP; 40-4-1, FFS more; [Per FS]. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-4-5; Rel-18 DL DMRS with single DCI based M-TRP; Support of Rel-18 DL DMRS with single DCI based M-TRP; 40-4-1, FFS more; Yes; n/a; Rel-18 DL DMRS with single DCI based M-TRP is not supported; [Per FS] Per FS; No; No; n/a; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-4-5: UE supports Rel-18 DL DMRS with single DCI based M-TRP requires that the UE supports Rel-18 DL DMRS and supports single DCI based M-TRP. Therefore, the prerequisite feature group includes FG 40-4-1 and FG 16-2b-1. 40-4-5: Per FSPC. |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-4-5 - Pre-requisite FG: We suggest the following update to add FGs for single DCI multi-TRP (16-2b-1/16-2b-2/16-2b-3/16-2b-4/16-2b-5): - At least one of 40-4-1/40-4-1a, and at least one of 16-2b-1/16-2b-2/16-2b-3/16-2b-4/16-2b-5. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-4-5; Rel-18 DL DMRS with single DCI based M-TRP; Support of Rel-18 DL DMRS with single DCI based M-TRP; 40-4-1, FFS more; Yes; n/a; Rel-18 DL DMRS with single DCI based M-TRP is not supported; Per FS; No; No; n/a; Optional with capability signaling. |
| Samsung [16] | - For FG 40-4-5 (Rel-18 DL DMRS with single DCI based M-TRP), in addition to FG 40-4-1, having at least one of FG 16-2b-1 (Single-DCI based SDM scheme), FG 16-2b-2 (Single-DCI based FDMSchemeA), FG 16-2b-3 (Single-DCI based FDMSchemeB), FG 16-2b-4 (Single-DCI based TDMSchemeA), or FG 16-2b-5 (Single-DCI based inter-slot TDM) as pre-requisites is needed. Per FS |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-4-5; Rel-18 DL DMRS with single DCI based M-TRP; Support of Rel-18 DL DMRS with single DCI based M-TRP; 40-4-1, FFS more; Yes; n/a; Rel-18 DL DMRS with single DCI based M-TRP is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Huawei/HiSilicon [19] | Similar to Rel-15 DMRS FGs, in our view, at least FG 40-4-1, FG 40-4-1j, FG 40-4-1a, FG 40-4-1b, FG 40-4-1c, FG 40-4-1e, FG 40-4-1f, FG 40-4-5, FG 40-4-7 should be Per FS. Proposal 4-4: The reporting granularity of FG 40-4-1, FG 40-4-1j, FG 40-4-1a, FG 40-4-1b, FG 40-4-1c, FG 40-4-1e, FG 40-4-1f, FG 40-4-5, FG 40-4-7 should be Per FS. 40. NR_MIMO_evo_DL_UL; 40-4-5; Rel-18 DL DMRS with single DCI based M-TRP; Support of Rel-18 DL DMRS with single DCI based M-TRP; 40-4-1, FFS more; Yes; n/a; Rel-18 DL DMRS with single DCI based M-TRP is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-4-5a | Additional row(s) for antenna ports (0,2,3) for Rel.18 DMRS ports for single-DCI based M-TRP | Support of additional row(s) for antenna ports (0,2,3) for Rel.18 DMRS ports for single-DCI based M-TRP | 40-4-5 | Yes | n/a | Additional row(s) for antenna ports (0,2,3) for Rel.18 DMRS ports for single-DCI based M-TRP are not supported | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-4-5a; Additional row(s) for antenna ports (0,2,3) for Rel.18 DMRS ports for single-DCI based M-TRP; Support of additional row(s) for antenna ports (0,2,3) for Rel.18 DMRS ports for single-DCI based M-TRP; 40-4-5; Yes; n/a; Additional row(s) for antenna ports (0,2,3) for Rel.18 DMRS ports for single-DCI based M-TRP are not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Ericsson [3] | 40-4-5a; Additional row(s) for antenna ports (0,2,3) for Rel.18 DMRS ports for single-DCI based M-TRP; Support of additional row(s) for antenna ports (0,2,3) for Rel.18 DMRS ports for single-DCI based M-TRP; 40-4-5; [Per FS]. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-4-5a; Additional row(s) for antenna ports (0,2,3) for Rel.18 DMRS ports for single-DCI based M-TRP; Support of additional row(s) for antenna ports (0,2,3) for Rel.18 DMRS ports for single-DCI based M-TRP; 40-4-5; Yes; n/a; Additional row(s) for antenna ports (0,2,3) for Rel.18 DMRS ports for single-DCI based M-TRP are not supported; [Per FS] Per FS; No; No; n/a; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-4-5a: Per FSPC. |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-4-5a; Additional row(s) for antenna ports (0,2,3) for Rel.18 DMRS ports for single-DCI based M-TRP; Support of additional row(s) for antenna ports (0,2,3) for Rel.18 DMRS ports for single-DCI based M-TRP; 40-4-5; Yes; n/a; Additional row(s) for antenna ports (0,2,3) for Rel.18 DMRS ports for single-DCI based M-TRP are not supported; [Per FS; No; No; n/a; Optional with capability signaling. |
| Samsung [16] | Per FS |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-4-5a; Additional row(s) for antenna ports (0,2,3) for Rel.18 DMRS ports for single-DCI based M-TRP for PDSCH; Support of additional row(s) for antenna ports (0,2,3) for Rel.18 DMRS ports for single-DCI based M-TRP; 40-4-5; Yes; n/a; Additional row(s) for antenna ports (0,2,3) for Rel.18 DMRS ports for single-DCI based M-TRP are not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-4-5a; Additional row(s) for antenna ports (0,2,3) for Rel.18 DMRS ports for single-DCI based M-TRP; Support of additional row(s) for antenna ports (0,2,3) for Rel.18 DMRS ports for single-DCI based M-TRP; 40-4-5; Yes; n/a; Additional row(s) for antenna ports (0,2,3) for Rel.18 DMRS ports for single-DCI based M-TRP are not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-4-7 | Rel-18 DL DMRS with M-DCI based M-TRP | Support of Rel-18 DL DMRS with multi- DCI based M-TRP PDSCH operation | FFS | Yes | n/a | Rel-18 DL DMRS with M-DCI based M-TRP is not supported | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-4-7; Rel-18 DL DMRS with M-DCI based M-TRP; Support of Rel-18 DL DMRS with multi- DCI based M-TRP PDSCH operation; FFS 40-4-1; Yes; n/a; Rel-18 DL DMRS with M-DCI based M-TRP is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Ericsson [3] | 40-4-7; Rel-18 DL DMRS with M-DCI based M-TRP; Support of Rel-18 DL DMRS with multi- DCI based M-TRP PDSCH operation; FFS40-4-1; [Per FS]. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-4-7; Rel-18 DL DMRS with M-DCI based M-TRP; Support of Rel-18 DL DMRS with multi- DCI based M-TRP PDSCH operation; FFS; Yes; n/a; Rel-18 DL DMRS with M-DCI based M-TRP is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-4-7: Similar to single DCI based M-TRP, the prerequisite feature group includes FG 40-4-1 and FG 16-2a for the feature of Rel-18 DL DMRS with M-DCI based M-TRP. 40-4-7: Per FSPC. |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-4-7 - Pre-requisite FG: We suggest the following update to add FGs for multi DCI multi-TRP (16-2a): - At least one of 40-4-1/40-4-1a, and 16-2a. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-4-7; Rel-18 DL DMRS with M-DCI based M-TRP; Support of Rel-18 DL DMRS with multi- DCI based M-TRP PDSCH operation; FFS; Yes; n/a; Rel-18 DL DMRS with M-DCI based M-TRP is not supported; Per FS; No; No; n/a; Optional with capability signaling. |
| Samsung [16] | - For FG 40-4-7 (Rel-18 DL DMRS with M-DCI based M-TRP), having FG 40-4-1 and FG 16-2a (Multi-DCI based multi-TRP) as one of pre-requisite is needed. Support per FSPC for FG 40-4-7. |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-4-7 40-4-5b; Rel-18 DL DMRS with M-DCI based M-TRP; Support of Rel-18 DL DMRS with multi- DCI based M-TRP PDSCH operation; FFS; Yes; n/a; Rel-18 DL DMRS with M-DCI based M-TRP is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Huawei/HiSilicon [19] | Similar to Rel-15 DMRS FGs, in our view, at least FG 40-4-1, FG 40-4-1j, FG 40-4-1a, FG 40-4-1b, FG 40-4-1c, FG 40-4-1e, FG 40-4-1f, FG 40-4-5, FG 40-4-7 should be Per FS. Proposal 4-4: The reporting granularity of FG 40-4-1, FG 40-4-1j, FG 40-4-1a, FG 40-4-1b, FG 40-4-1c, FG 40-4-1e, FG 40-4-1f, FG 40-4-5, FG 40-4-7 should be Per FS. 40. NR_MIMO_evo_DL_UL; 40-4-7; Rel-18 DL DMRS with M-DCI based M-TRP; Support of Rel-18 DL DMRS with multi- DCI based M-TRP PDSCH operation; FFS; Yes; n/a; Rel-18 DL DMRS with M-DCI based M-TRP is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-4-6 | Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type A for Rel.18 enhanced DMRS ports | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbols 3) Support 1 symbol FL DMRS and 2 additional DMRS symbols | [2-16] | Yes | n/a | Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type A for Rel.18 enhanced DMRS ports is not supported | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-4-6; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type A for Rel.18 enhanced DMRS ports; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbols 3) Support 1 symbol FL DMRS and 2 additional DMRS symbols; [2-16]; Yes; n/a; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type A for Rel.18 enhanced DMRS ports is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Ericsson [3] | 40-4-6; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type A for Rel.18 enhanced DMRS ports; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbols 3) Support 1 symbol FL DMRS and 2 additional DMRS symbols; [2-16]; [Per FS] Per UE. |
| ZTE [4] | For FG 40-4-6, - The prerequisite FG can be FG 2-16. 40. NR_MIMO_evo_DL_UL; 40-4-6; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type A for Rel.18 enhanced DMRS ports; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbols 3) Support 1 symbol FL DMRS and 2 additional DMRS symbols; [2-16] 2-16; Yes; n/a; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type A for Rel.18 enhanced DMRS ports is not supported; [Per FS] Per FS; No; No; n/a; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-4-6; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type A for Rel.18 enhanced DMRS ports Comment: It is redundant; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbols 3) Support 1 symbol FL DMRS and 2 additional DMRS symbols; [2-16]; Yes; n/a; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type A for Rel.18 enhanced DMRS ports is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Intel Corporation [7] |  |
| OPPO [8] | 40. NR_MIMO_evo_DL_UL; 40-4-6; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type A for Rel.18 enhanced DMRS ports; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbols 3) Support 1 symbol FL DMRS and 2 additional DMRS symbols 4) Support Rank 1-4; [2-16]; Yes; n/a; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type A for Rel.18 enhanced DMRS ports is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |
| CATT [9] | 40-4-6: The prerequisite feature group includes FG 2-16. |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-4-6; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type A for Rel.18 enhanced DMRS ports; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbols 3) Support 1 symbol FL DMRS and 2 additional DMRS symbols; [2-16]; Yes; n/a; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type A for Rel.18 enhanced DMRS ports is not supported; Per FS; No; No; n/a; Optional with capability signaling. |
| Samsung [16] | - For FG 40-4-6 (Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type A for Rel.18 enhanced DMRS ports), we are basically fine with having FG 2-16 (Basic uplink DMRS for scheduling type A) only as a pre-requisite, but we are open to discuss whether to need more pre-requisites. Per FS |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-4-6; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type A for Rel.18 enhanced DMRS ports; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbols 3) Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port; [2-16]; Yes; n/a; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type A for Rel.18 enhanced DMRS ports is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-4-6; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type A for Rel.18 enhanced DMRS ports; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbols 3) Support 1 symbol FL DMRS and 2 additional DMRS symbols; [2-16]; Yes; n/a; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type A for Rel.18 enhanced DMRS ports is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-4-6a | Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type B for Rel.18 enhanced DMRS ports | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol | [2-16a] | Yes | n/a | Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type B for Rel.18 enhanced DMRS ports is not supported | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-4-6a; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type B for Rel.18 enhanced DMRS ports; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol; [2-16a]; Yes; n/a; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type B for Rel.18 enhanced DMRS ports is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Ericsson [3] | 40-4-6a; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type B for Rel.18 enhanced DMRS ports; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol; [2-16a]; [Per FS] Per UE. |
| ZTE [4] | - For FG 40-4-6a, - The prerequisite FG can be FG 2-16a. 40. NR_MIMO_evo_DL_UL; 40-4-6a; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type B for Rel.18 enhanced DMRS ports; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol; [2-16a] 2-16a; Yes; n/a; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type B for Rel.18 enhanced DMRS ports is not supported; [Per FS] Per FS; No; No; n/a; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-4-6a; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type B for Rel.18 enhanced DMRS ports; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol; [2-16a]; Yes; n/a; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type B for Rel.18 enhanced DMRS ports is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Intel Corporation [7] |  |
| OPPO [8] | 40. NR_MIMO_evo_DL_UL; 40-4-6a; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type B for Rel.18 enhanced DMRS ports; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol 3) Support Rank 1-4; [2-16a]; Yes; n/a; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type B for Rel.18 enhanced DMRS ports is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |
| CATT [9] | 40-4-6a: The prerequisite feature group includes FG 2-16a. |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-4-6a; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type B for Rel.18 enhanced DMRS ports; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol; [2-16a]; Yes; n/a; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type B for Rel.18 enhanced DMRS ports is not supported; Per FS; No; No; n/a; Optional with capability signaling. |
| Samsung [16] | - For FG 40-4-6a (Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type B for Rel.18 enhanced DMRS ports), we are basically fine with having FG 2-16a (Basic uplink DMRS for scheduling type B) only as a pre-requisite, but we are open to discuss whether to need more pre-requisites. Per FS |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] |  |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-4-6a; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type B for Rel.18 enhanced DMRS ports; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol; [2-16a]; Yes; n/a; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type B for Rel.18 enhanced DMRS ports is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-4-6c | DMRS type for Rel.18 enhanced DMRS ports for PUSCH | Support of DMRS type for Rel.18 enhanced DMRS ports for PUSCH | 40-4-6 | Yes | n/a | DMRS type for Rel.18 enhanced DMRS ports for PUSCH is not supported | [Per FS] | No | No | n/a | Component candidate values: {eType 1, both eType 1 and eType 2} [Note: A UE supporting one of FG 40-4-6 or FG 40-4-6a must support this FG] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-4-6c; DMRS type for Rel.18 enhanced DMRS ports for PUSCH; Support of DMRS type for Rel.18 enhanced DMRS ports for PUSCH; 40-4-6; Yes; n/a; DMRS type for Rel.18 enhanced DMRS ports for PUSCH is not supported; [Per FS]; No; No; n/a; Component candidate values: {eType 1, both eType 1 and eType 2} [Note: A UE supporting one of FG 40-4-6 or FG 40-4-6a must support this FG]; Optional with capability signaling. |
| Ericsson [3] | 40-4-6c; DMRS type for Rel.18 enhanced DMRS ports for PUSCH; Support of DMRS type for Rel.18 enhanced DMRS ports for PUSCH; 40-4-6; [Per FS] Per UE. |
| ZTE [4] | - For FG 40-4-6c, - The prerequisite FG should include FG 2-16a in additional. - The note “A UE supporting one of FG 40-4-6 or FG 40-4-6a must support this FG” is needed. 40. NR_MIMO_evo_DL_UL; 40-4-6c; DMRS type for Rel.18 enhanced DMRS ports for PUSCH; Support of DMRS type for Rel.18 enhanced DMRS ports for PUSCH; 40-4-6, 40-4-6a; Yes; n/a; DMRS type for Rel.18 enhanced DMRS ports for PUSCH is not supported; [Per FS] Per FS; No; No; n/a; Component candidate values: {eType 1, both eType 1 and eType 2} [Note: A UE supporting one of FG 40-4-6 or FG 40-4-6a must support this FG]; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-4-6c; DMRS type for Rel.18 enhanced DMRS ports for PUSCH; Support of DMRS type for Rel.18 enhanced DMRS ports for PUSCH; 40-4-6; Yes; n/a; DMRS type for Rel.18 enhanced DMRS ports for PUSCH is not supported; [Per FS]; No; No; n/a; Component candidate values: {eType 1, both eType 1 and eType 2} [Note: A UE supporting one of FG 40-4-6 or FG 40-4-6a must support this FG] Comment: Support the note.; Optional with capability signaling. |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-4-6c: Per UE. |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-4-6c - Note: If UE supports Rel.18 DMRS ports but not report this FG, it is not clear which DMRS type UE supports. Hence, we suggest to support the following note: - [Note: A UE supporting one of FG 40-4-6 or FG 40-4-6a must support this FG] |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-4-6c; DMRS type for Rel.18 enhanced DMRS ports for PUSCH; Support of DMRS type for Rel.18 enhanced DMRS ports for PUSCH; 40-4-6; Yes; n/a; DMRS type for Rel.18 enhanced DMRS ports for PUSCH is not supported; Per FS; No; No; n/a; Component candidate values: {eType 1, both eType 1 and eType 2} [Note: A UE supporting one of FG 40-4-6 or FG 40-4-6a must support this FG]; Optional with capability signaling. |
| Samsung [16] | Per FS |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-4-6c; DMRS type for Rel.18 enhanced DMRS ports for PUSCH; Support of DMRS type for Rel.18 enhanced DMRS ports for PUSCH; 40-4-6; Yes; n/a; DMRS type for Rel.18 enhanced DMRS ports for PUSCH is not supported; [Per FS]; No; No; n/a; Component candidate values: {eType 1, both eType 1 and eType 2} [Note: A UE supporting one of FG 40-4-6 or FG 40-4-6a must support this FG]; Optional with capability signaling. |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-4-6c; DMRS type for Rel.18 enhanced DMRS ports for PUSCH; Support of DMRS type for Rel.18 enhanced DMRS ports for PUSCH; 40-4-6; Yes; n/a; DMRS type for Rel.18 enhanced DMRS ports for PUSCH is not supported; [Per FS]; No; No; n/a; Component candidate values: {eType 1, both eType 1 and eType 2} [Note: A UE supporting one of FG 40-4-6 or FG 40-4-6a must support this FG]; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-4-6d | 2 symbols front-loaded DMRS (uplink) for Rel.18 enhanced DMRS ports for PUSCH | Support of 2 symbols front-loaded DMRS (uplink) for Rel.18 enhanced DMRS ports for PUSCH | 40-4-6 | Yes | n/a | 2 symbols front-loaded DMRS (uplink) for Rel.18 enhanced DMRS ports for PUSCH is not supported | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-4-6d; 2 symbols front-loaded DMRS (uplink) for Rel.18 enhanced DMRS ports for PUSCH; Support of 2 symbols front-loaded DMRS (uplink) for Rel.18 enhanced DMRS ports for PUSCH; 40-4-6; Yes; n/a; 2 symbols front-loaded DMRS (uplink) for Rel.18 enhanced DMRS ports for PUSCH is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Ericsson [3] | 40-4-6d; 2 symbols front-loaded DMRS (uplink) for Rel.18 enhanced DMRS ports for PUSCH; Support of 2 symbols front-loaded DMRS (uplink) for Rel.18 enhanced DMRS ports for PUSCH; 40-4-6; [Per FS] Per UE. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-4-6d; 2 symbols front-loaded DMRS (uplink) for Rel.18 enhanced DMRS ports for PUSCH; Support of 2 symbols front-loaded DMRS (uplink) for Rel.18 enhanced DMRS ports for PUSCH; 40-4-6; Yes; n/a; 2 symbols front-loaded DMRS (uplink) for Rel.18 enhanced DMRS ports for PUSCH is not supported; [Per FS] Per FS; No; No; n/a; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-4-6d: Per FS. |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-4-6d; 2 symbols front-loaded DMRS (uplink) for Rel.18 enhanced DMRS ports for PUSCH; Support of 2 symbols front-loaded DMRS (uplink) for Rel.18 enhanced DMRS ports for PUSCH; 40-4-6; Yes; n/a; 2 symbols front-loaded DMRS (uplink) for Rel.18 enhanced DMRS ports for PUSCH is not supported; Per FS; No; No; n/a; Optional with capability signaling. |
| Samsung [16] | Per FS |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-4-6d; 2 symbols front-loaded DMRS (uplink) for Rel.18 enhanced DMRS ports for PUSCH; Support of 2 symbols front-loaded DMRS (uplink) for Rel.18 enhanced DMRS ports for PUSCH; 40-4-6; Yes; n/a; 2 symbols front-loaded DMRS (uplink) for Rel.18 enhanced DMRS ports for PUSCH is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-4-6d; 2 symbols front-loaded DMRS (uplink) for Rel.18 enhanced DMRS ports for PUSCH; Support of 2 symbols front-loaded DMRS (uplink) for Rel.18 enhanced DMRS ports for PUSCH; 40-4-6; Yes; n/a; 2 symbols front-loaded DMRS (uplink) for Rel.18 enhanced DMRS ports for PUSCH is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-4-6e | 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH | Support of 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH | 40-4-6 | Yes | n/a | 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH is not supported | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-4-6e; 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH; Support of 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH; 40-4-6; Yes; n/a; 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Ericsson [3] | 40-4-6e; 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH; Support of 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH; 40-4-6; [Per FS] Per UE. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-4-6e; 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH; Support of 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH; 40-4-6; Yes; n/a; 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH is not supported; [Per FS] Per FS; No; No; n/a; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-4-6e: Per FS. |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-4-6e; 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH; Support of 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH; 40-4-6; Yes; n/a; 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH is not supported; Per FS; No; No; n/a; Optional with capability signaling. |
| Samsung [16] | Per FS |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-4-6e; 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH; Support of 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH; 40-4-6; Yes; n/a; 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-4-6e; 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH; Support of 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH; 40-4-6; Yes; n/a; 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-4-6f | 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH | Support of 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH | 40-4-6 | Yes | n/a | 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH is not supported | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-4-6f; 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH; Support of 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH; 40-4-6; Yes; n/a; 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Ericsson [3] | 40-4-6f; 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH; Support of 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH; 40-4-6; [Per FS] Per UE. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-4-6f; 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH; Support of 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH; 40-4-6; Yes; n/a; 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH is not supported; [Per FS] Per FS; No; No; n/a; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-4-6f: Per FS. |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-4-6f; 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH; Support of 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH; 40-4-6; Yes; n/a; 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH is not supported; Per FS; No; No; n/a; Optional with capability signaling. |
| Samsung [16] | Per FS |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-4-6f; 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH; Support of 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH; 40-4-6; Yes; n/a; 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-4-6f; 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH; Support of 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH; 40-4-6; Yes; n/a; 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-4-6g | 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 | Support of 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 | 40-4-6, FFS more | Yes | n/a | 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 is not supported | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-4-6g; 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4; Support of 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4; 40-4-6, FFS more; Yes; n/a; 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Ericsson [3] | 40-4-6g; 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4; Support of 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4; 40-4-6, FFS more; [Per FS] Per UE. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-4-6g; 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4; Support of 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4; 40-4-6, FFS more; Yes; n/a; 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 is not supported; [Per FS] Per FS; No; No; n/a; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-4-6g: Besides FG 40-4-6, the prerequisite feature group of 40-6-6g shall include the basic UL-PTRS feature, i.e., FG 2-47. 40-4-6g: Per band. |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-4-6g - Note: Since 1-port UL PTRS (FG2-47) is mandatory in FR2 in Rel.15, we suggest to add the following note, to avoid the situation UE does not support this FG in FR2: - Note: A UE supporting FG2-47 and at least one of FG 40-4-6 or FG 40-4-6a must support this FG. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-4-6g; 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4; Support of 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4; 40-4-6, FFS more; Yes; n/a; 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 is not supported; Per FS; No; No; n/a; Optional with capability signaling. |
| Samsung [16] | - For FG 40-4-6g (1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4), in addition to FG 40-4-6, having FG 2-47 (Basic UL PTRS) as one of pre-requisites is needed. Per FS |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-4-6k; 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4; Support of 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4; 40-4-6, FFS more; Yes; n/a; 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-4-6g; 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4; Support of 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4; 40-4-6, FFS more; Yes; n/a; 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-4-6h | 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 | Support of 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 | 40-4-6, FFS more | Yes | n/a | 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 is not supported | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-4-6h; 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8; Support of 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8; 40-4-6, FFS more; Yes; n/a; 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Ericsson [3] | 40-4-6h; 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8; Support of 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8; 40-4-6, FFS more, 40-4-6g; [Per FS] Per UE. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-4-6h; 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8; Support of 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8; 40-4-6, FFS more; Yes; n/a; 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 is not supported; [Per FS] Per FS; No; No; n/a; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-4-6h: A UE supports 1-port UL PTRS for Rel-18 enhanced DMRS ports for PUSCH with rank 5-8 is expected to support 1-port UL PTRS for Rel-18 enhanced DMRS ports for PUSCH with rank 1-4, therefore the prerequisite feature group of 40-4-6h shall be FG 40-4-6g. 40-4-6h: Per band. |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-4-6h; 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8; Support of 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8; 40-4-6, 2-47; Yes; n/a; 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 is not supported; Per FS; No; No; n/a; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-4-6h; 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8; Support of 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8; 40-4-6, FFS more; Yes; n/a; 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 is not supported; Per FS; No; No; n/a; Optional with capability signaling. |
| Samsung [16] | - For FG 40-4-6h (1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8), in addition to FG 40-4-6, having FG 2-47 (Basic UL PTRS) and FG 40-4-10 as pre-requisites are needed. Per FS |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-4-6m; 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8; Support of 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8; 40-4-6, FFS more; Yes; n/a; 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-4-6h; 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8; Support of 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8; 40-4-6, FFS more; Yes; n/a; 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-4-6i | 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 | Support of 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 | 40-4-6, FFS more | Yes | n/a | 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 is not supported | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-4-6i; 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4; Support of 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4; 40-4-6, FFS more; Yes; n/a; 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Ericsson [3] | 40-4-6i; 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4; Support of 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4; 40-4-6, FFS more40-4-6g; [Per FS] Per Band. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-4-6i; 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4; Support of 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4; 40-4-6, FFS more; Yes; n/a; 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 is not supported; [Per FS] Per FS; No; No; n/a; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-4-6i: Besides FG 40-4-6, the prerequisite feature group of 40-4-6i shall include the UL-PTRS feature for 2 ports PTRS, i.e., FG 2-48. 40-4-6i: Per band. |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-4-6i; 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4; Support of 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4; 40-4-6, 2-48; Yes; n/a; 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 is not supported; Per FS; No; No; n/a; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-4-6i; 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4; Support of 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4; 40-4-6, FFS more; Yes; n/a; 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 is not supported; Per FS; No; No; n/a; Optional with capability signaling. |
| Samsung [16] | - For FG 40-4-6i (2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4), in addition to FG 40-4-6, having FG 2-48 (Basic UL PTRS) as one of pre-requisites is needed. Per FS |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-4-6n; 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4; Support of 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4; 40-4-6, FFS more; Yes; n/a; 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-4-6i; 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4; Support of 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4; 40-4-6, FFS more; Yes; n/a; 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-4-6j | 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 | Support of 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 | 40-4-6, FFS more | Yes | n/a | 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 is not supported | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-4-6j; 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8; Support of 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8; 40-4-6, FFS more; Yes; n/a; 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Ericsson [3] | 40-4-6j; 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8; Support of 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8; 40-4-6, FFS more40-4-6h, 40-4-6g; [Per FS] Per Band. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-4-6j; 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8; Support of 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8; 40-4-6, FFS more; Yes; n/a; 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 is not supported; [Per FS] Per FS; No; No; n/a; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-4-6j: A UE supports 2 ports UL PTRS for Rel-18 enhanced DMRS ports for PUSCH with rank 5-8 is expected to support 2 ports UL PTRS for Rel-18 enhanced DMRS ports for PUSCH with rank 1-4, therefore the prerequisite feature group of 40-4-6j shall be FG 40-4-6i. 40-4-6j: Per band. |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-4-6j; 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8; Support of 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8; 40-4-6, 2-48; Yes; n/a; 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 is not supported; Per FS; No; No; n/a; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-4-6j; 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8; Support of 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8; 40-4-6, FFS more; Yes; n/a; 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 is not supported; Per FS; No; No; n/a; Optional with capability signaling. |
| Samsung [16] | - For FG 40-4-6j (2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8), in addition to FG 40-4-6, having FG 2-48 (Basic UL PTRS) and FG 40-4-10 as pre-requisites are needed. Per FS |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-4-6p; 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8; Support of 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8; 40-4-6, FFS more; Yes; n/a; 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-4-6j; 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8; Support of 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8; 40-4-6, FFS more; Yes; n/a; 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-4-10 | DMRS port configuration for PUSCH [with rank 5-8/ with 8Tx] | DMRS port configuration for PUSCH [with rank 5-8 / with 8Tx] for Rel 15 and Rel. 18 | [40-4-6] | Yes | n/a | Port configuration for PUSCH [with rank 5-8 / with 8Tx] is not supported | [Per FS] | No | No | n/a | Candidate values: {Rel. 15 DMRS, Rel. 15 DMRS and Rel. 18 DMRS} [Note: A UE supporting 8 Tx must support this FG] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-4-10; DMRS port configuration for PUSCH [with rank 5-8/ with 8Tx]; DMRS port configuration for PUSCH [with rank 5-8 / with 8Tx] for Rel 15 and Rel. 18; [40-4-6]; Yes; n/a; Port configuration for PUSCH [with rank 5-8 / with 8Tx] is not supported; [Per FS]; No; No; n/a; Candidate values: {Rel. 15 DMRS, Rel. 15 DMRS and Rel. 18 DMRS} [Note: A UE supporting 8 Tx must support this FG]; Optional with capability signaling. |
| Ericsson [3] | 40-4-10; DMRS port configuration for PUSCH [with rank 5-8/ with 8Tx]; DMRS port configuration for PUSCH [with rank 5-8 / with 8Tx] for Rel 15 and Rel. 18; [40-4-6]; [Per FS] Per UE. |
| ZTE [4] | - For FG 40-4-10, - The prerequisite FG 40-4-6 is needed. - The wording “with 8Tx” is redundant due to “PUSCH with rank 5-8” is clear enough to capture this point. - Granularity can be “Per FS” to keep alignment with that of its prerequisite FG. - The note “A UE supporting 8 Tx must support this FG” is can be kept. 40. NR_MIMO_evo_DL_UL; 40-4-10; DMRS port configuration for PUSCH [with rank 5-8/ with 8Tx]; DMRS port configuration for PUSCH [with rank 5-8 / with 8Tx] for Rel 15 and Rel. 18; [40-4-6]; Yes; n/a; Port configuration for PUSCH [with rank 5-8 / with 8Tx] is not supported; [Per FS] Per FS; No; No; n/a; Candidate values: {Rel. 15 DMRS, Rel. 15 DMRS and Rel. 18 DMRS} [Note: A UE supporting 8 Tx must support this FG]; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-4-10; DMRS port configuration for PUSCH [with rank 5-8 / with 8Tx] Comment: This FG is not needed here. We already have the FGs for whether UE supports PUSCH with rank 5-8 and whether UE supports R15 DMRS and/or R18 DMRS, the combination can achieve the same target as this FG; Candidate values: {Rel. 15 DMRS, Rel. 18 DMRs, borh} DMRS port configuration for PUSCH [with rank 5-8 / with 8Tx] for Rel 15 and Rel. 18; [40-4-6]; Yes; n/a; Port configuration for PUSCH [with rank 5-8 / with 8Tx] is not supported; [Per FS]; No; No; n/a; Candidate values: {Rel. 15 DMRS, Rel. 15 DMRS and Rel. 18 DMRS} [Note: A UE supporting 8 Tx must support this FG]; Optional with capability signaling. |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | In TS38.822, UE feature groups on basic UL DMRS for scheduling type A (i.e., FG 2-16) and UL DMRS for scheduling type B (i.e., FG 2-16a) are mandatory without capability signaling. In RAN1 #110bis meeting, the following agreement was achieved: Agreement For more than 4 layers SU-MIMO PUSCH, support - Both Rel.15 Type 1/Type 2 DMRS ports and Rel.18 eType 1/eType 2 DMRS ports. - For UE supporting Rel.18 eType 1/eType 2 DMRS ports, UE can be indicated with either of Rel.15 Type 1/Type 2 DMRS ports or Rel.18 eType 1/eType 2 DMRS ports. - RRC based indication is supported as the baseline. FFS whether DCI based indication is further needed. - For UE not supporting Rel.18 eType 1/eType 2 DMRS ports, UE can be indicated with Rel.15 Type 1/Type 2 DMRS ports only.. It can be seen from the agreement that for PUSCH transmission with more than 4 layers, supporting DMRS port configuration for PUSCH on whether Rel-15 DMRS ports are used or Rel-18 DMRS ports are used is mandatory, no UE feature group is needed. Therefore, we propose to remove FG 40-4-10. Proposal 10: FG 40-4-10 is removed from UE feature list. |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-4-10; DMRS port configuration for PUSCH with rank 5-8/ with 8Tx; DMRS port configuration for PUSCH with rank 5-8 / with 8Txfor Rel 15 and/or Rel. 18; 40-4-6; Yes; n/a; Port configuration for PUSCH with rank 5-8 / with 8Txis not supported; Per FS; No; No; n/a; Candidate values: {Rel. 15 DMRS, Rel. 15 DMRS and Rel. 18 DMRS} [Note: A UE supporting 8 Tx must support this FG]; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-4-10 - Component: For the yellow highlighted part, we think it should be “with 8Tx”. Even if 8Tx is configured, UE can be dynamically indicated with rank 1-4. In this case, it is not clear which FG is used if it is noted as “with rank 5-8”. For clarification, we suggest to updated as: - DMRS port configuration for PUSCH [with rank 5-8/ configured with 8Tx]. - Pre-requisite FG: FG40-4-6 should be removed. It is possible that UE only supports Rel.15 DMRS ports but UE supports 8Tx. Instead, FG40-7-1 (Basic FG for 8Tx) can be pre-requisite FG. - Note: We support to have the note. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-4-10; DMRS port configuration for PUSCH [with rank 5-8/ with 8Tx]; DMRS port configuration for PUSCH [with rank 5-8 / with 8Tx] for Rel 15 and Rel. 18; [40-4-6]; Yes; n/a; Port configuration for PUSCH [with rank 5-8 / with 8Tx] is not supported; Per FS; No; No; n/a; Candidate values: {Rel. 15 DMRS, Rel. 15 DMRS and Rel. 18 DMRS} [Note: A UE supporting 8 Tx must support this FG]; Optional with capability signaling. |
| Samsung [16] | - For FG 40-4-10 (DMRS port configuration for PUSCH with rank 5-8 with 8TX), having FG 2-16 and FG 40-4-6 as pre-requisites are needed. Per FS |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-4-10; DMRS port configuration for PUSCH [with rank 5-8 / with 8Tx]; DMRS port configuration for PUSCH [with rank 5-8 / with 8Tx] for Rel 15 and Rel. 18; [40-4-6]; Yes; n/a; Port configuration for PUSCH [with rank 5-8 / with 8Tx] is not supported; [Per FS]; No; No; n/a; Candidate values: {Rel. 15 DMRS, Rel. 15 DMRS and Rel. 18 DMRS} [Note: A UE supporting 8 Tx must support this FG]; Optional with capability signaling. |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-4-10; DMRS port configuration for PUSCH [with rank 5-8/ with 8Tx]; DMRS port configuration for PUSCH [with rank 5-8 / with 8Tx] for Rel 15 and Rel. 18; [40-4-6]; Yes; n/a; Port configuration for PUSCH [with rank 5-8 / with 8Tx] is not supported; [Per FS]; No; No; n/a; Candidate values: {Rel. 15 DMRS, Rel. 15 DMRS and Rel. 18 DMRS} [Note: A UE supporting 8 Tx must support this FG]; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-5-1 | SRS comb offset hopping | Support of SRS comb offset hopping | FFS | Yes | n/a | SRS comb offset hopping is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-5-1; SRS comb offset hopping; Support of SRS comb offset hopping; FFS 2-52; Yes; n/a; SRS comb offset hopping is not supported; Per band; n/a; n/a; n/a; Optional with capability signaling. |
| Ericsson [3] | The prerequisite should be 2-52 (Basic SRS). 40. NR_MIMO_evo_DL_UL; 40-5-1; SRS comb offset hopping; Support of SRS comb offset hopping; 2-52; Yes; n/a; SRS comb offset hopping is not supported; Per band; n/a; n/a; n/a; Optional with capability signaling. |
| ZTE [4] | - For FG 40-5-1 and FG 40-5-2, both the prerequisites should be [2-53] ‘basic SRS’; 40. NR_MIMO_evo_DL_UL; 40-5-1; SRS comb offset hopping; Support of SRS comb offset hopping; 2-53 FFS; Yes; n/a; SRS comb offset hopping is not supported; Per band; n/a; n/a; n/a; Optional with capability signaling. |
| Fujitsu [5] | 40-5-1; SRS comb offset hopping; Support of SRS comb offset hopping; FFS; Note: UE supporting FG 40-5-1 doesn’t support FG 40-5-6 and FG 40-5-7. |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-5-1; SRS comb offset hopping; Support of SRS comb offset hopping; FFS 2-53; Yes; n/a; SRS comb offset hopping is not supported; Per band; n/a; n/a; n/a; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-5-1: - Prerequisite FG: Not essential, but if it is needed, we think it should be 2-53 |
| Apple [15] |  |
| Samsung [16] | 40-5-1; SRS comb offset hopping; Support of SRS cyclic shift hopping; 2-52 or 2-53; Per band; Applicable for periodic, semi-persistent, and aperiodic SRS with usage of antenna switching. |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-5-1; SRS comb offset hopping; Support of SRS comb offset hopping; Yes; n/a; SRS comb offset hopping is not supported; Per band; n/a; n/a; n/a; Optional with capability signaling. |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-5-2 | SRS cyclic shift hopping | Support of SRS cyclic shift hopping | [2-53] | Yes | n/a | SRS cyclic shift hopping is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-5-2; SRS cyclic shift hopping; Support of SRS cyclic shift hopping; [2-53]; Yes; n/a; SRS cyclic shift hopping is not supported; Per band; n/a; n/a; n/a; Optional with capability signaling. |
| Ericsson [3] | The prerequisite should be 2-52 (Basic SRS). 40. NR_MIMO_evo_DL_UL; 40-5-2; SRS cyclic shift hopping; Support of SRS cyclic shift hopping; 2-52; Yes; n/a; SRS cyclic shift hopping is not supported; Per band; n/a; n/a; n/a; Optional with capability signaling. |
| ZTE [4] | - For FG 40-5-1 and FG 40-5-2, both the prerequisites should be [2-53] ‘basic SRS’; 40. NR_MIMO_evo_DL_UL; 40-5-2; SRS cyclic shift hopping; Support of SRS cyclic shift hopping; [2-53]; Yes; n/a; SRS cyclic shift hopping is not supported; Per band; n/a; n/a; n/a; Optional with capability signaling. |
| Fujitsu [5] | 40-5-2; SRS cyclic shift hopping; Support of SRS cyclic shift hopping; [2-53]; Note: UE supporting FG 40-5-2 doesn’t support FG 40-5-6 and FG 40-5-7. |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-5-2; SRS cyclic shift hopping; Support of SRS cyclic shift hopping; [2-53]; Yes; n/a; SRS cyclic shift hopping is not supported; Per band; n/a; n/a; n/a; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-5-2: - Prerequisite FG: Ok to confirm 2-53 |
| Apple [15] |  |
| Samsung [16] | 40-5-2; SRS cyclic shift hopping; Support of SRS comb offset hopping combined with legacy group/sequence hopping; 2-52 or 2-53; Per band; Applicable for periodic, semi-persistent, and aperiodic SRS with usage of antenna switching. |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-5-2; SRS cyclic shift hopping; Support of SRS cyclic shift hopping; Yes; n/a; SRS cyclic shift hopping is not supported; Per band; n/a; n/a; n/a; Optional with capability signaling. |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-5-1b | SRS comb offset hopping combined with legacy group/sequence hopping | Support of SRS comb offset hopping combined with legacy group/sequence hopping | 40-5-1 | Yes | n/a | SRS comb offset hopping combined with legacy group/sequence hopping is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] |  |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-5-1b; SRS comb offset hopping combined with legacy group/sequence hopping; Support of SRS comb offset hopping combined with legacy group/sequence hopping; 40-5-1; Yes; n/a; SRS comb offset hopping combined with legacy group/sequence hopping is not supported; Per band; n/a; n/a; n/a; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] |  |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] |  |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-5-2b | SRS cyclic shift hopping combined with legacy group/sequence hopping | Support of SRS cyclic shift hopping combined with legacy group/sequence hopping | 40-5-2 | Yes | n/a | SRS cyclic shift hopping combined with legacy group/sequence hopping is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] |  |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-5-2b; SRS cyclic shift hopping combined with legacy group/sequence hopping; Support of SRS cyclic shift hopping combined with legacy group/sequence hopping; 40-5-2; Yes; n/a; SRS cyclic shift hopping combined with legacy group/sequence hopping is not supported; Per band; n/a; n/a; n/a; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] |  |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] |  |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-5-3 | SRS cyclic shift hopping combined with SRS comb offset hopping | Support of SRS cyclic shift hopping combined SRS comb offset hopping | 40-5-1, 40-5-2 | Yes | n/a | SRS cyclic shift hopping combined with SRS comb offset hopping is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] |  |
| ZTE [4] |  |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] |  |
| Samsung [16] | 40-5-3; SRS cyclic shift hopping combined with legacy group/sequence hopping; Support of SRS cyclic shift hopping combined SRS comb offset hopping; 2-52 or 2-53 40-5-1, 40-5-2; Per band; No joint scheme between SRS comb-offset and SRS cyclic shift hopping scheme is supported.. |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] |  |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-5-1a | Comb offset hopping time-domain behavior when repetition factor R>1 | Supported comb offset hopping granularity in time when repetition factor R>1 is configured | 40-5-1 | Yes | n/a |  | Per band | n/a | n/a | n/a | Component candidate values: {‘per SRS symbol’,’per R SRS symbols’, ‘both’} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] | 40. NR_MIMO_evo_DL_UL; 40-5-1a; Comb offset hopping time-domain behavior when repetition factor R>1; Supported comb offset hopping granularity in time when repetition factor R>1 is configured; 40-5-1; Yes; n/a; Per band; n/a; n/a; n/a; Component candidate values: {‘per SRS symbol’, ’per R SRS symbols’, ‘both’}; Optional with capability signaling. |
| ZTE [4] | - For FG 40-5-1a, we prefer to add a note to clarify the component candidate values ‘per SRS symbol’ and ‘per R SRS symbols’ ; 40. NR_MIMO_evo_DL_UL; 40-5-1a; Comb offset hopping time-domain behavior when repetition factor R>1; Supported comb offset hopping granularity in time when repetition factor R>1 is configured; 40-5-1; Yes; n/a; Per band; n/a; n/a; n/a; Component candidate values: {‘per SRS symbol’,’per R SRS symbols’, ‘both’} Note: - ‘per SRS symbol’ indicates the comb offset hopping time-domain behavior depends on the symbol index l’ of each symbol - ‘per R SRS symbol’ indicates the comb offset hopping time-domain behavior depends on the symbol index l’ of the first symbol across R repetition symbols, where R is repetition factor; Optional with capability signaling. |
| Fujitsu [5] | Regarding FG 40-5-1a, the UE can report supporting both options. But these two options can’t be configured at the same time. So, some note text can be added for clarification. 40-5-1a; Comb offset hopping time-domain behavior when repetition factor R>1; Supported comb offset hopping granularity in time when repetition factor R>1 is configured; 40-5-1; Component candidate values: {‘per SRS symbol’,’per R SRS symbols’, ‘both’} Note: when UE indicates supporting ‘both’, only one option can be configured by gNB.. |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] |  |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] |  |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-5-4 | SRS without TDMed 8 Tx ports—antenna switching | Support of SRS with 8 Tx ports—antenna switching FFS: detailed components FFS: separate row for comb8 |  | Yes | n/a | SRS with 8 Tx ports—antenna switching is not supported | [Per band] | n/a | n/a | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-5-4 - FFS1: we recommend removing the this FFS - FFS2: For SRS with 8 Tx ports—antenna switching, we propose having a separate row for comb8 mapping 40. NR_MIMO_evo_DL_UL; 40-5-4; SRS without TDMed 8 Tx ports—antenna switching; Support of SRS with 8 Tx ports—antenna switching FFS: detailed components FFS: separate row for comb8; Yes; n/a; SRS with 8 Tx ports—antenna switching is not supported; [Per band]; n/a; n/a; n/a; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-5-4b; SRS with 8 Tx ports and Comb8 mapping —antenna switching; Yes; n/a; Per band; n/a; n/a; n/a; Optional with capability signaling. |
| Ericsson [3] | This is the FG for supporting an 8-port SRS resource in an SRS resource set with usage ‘antennaSwitching’. In our view, there is no need for a separate FG for comb 8. 40. NR_MIMO_evo_DL_UL; 40-5-4; 8 Tx SRS—antenna switching; Support of SRS with 8 Tx ports—antenna switching; Yes; n/a; SRS with 8 Tx ports—antenna switching is not supported; Per band; n/a; n/a; n/a; Optional with capability signaling. |
| ZTE [4] | - For FG 40-5-4, FG 40-5-4a, FG 40-5-5, and FG 40-5-6, the brackets of ‘Per band’ should be removed; - For FG 40-5-4, FG 40-5-4a, FG 40-5-5, and FG 40-5-6, we prefer to add details of the components including the value of the number of SRS symbols and the value of TDM factor, and the ‘FFS: detailed components’ should be removed; - For FG 40-5-4, FG 40-5-4a, FG 40-5-5, and FG 40-5-6, there is no justified reason to add a separate row for comb 8, and the ‘FFS: separate row for comb 8’ should be removed; 40. NR_MIMO_evo_DL_UL; 40-5-4; SRS without TDMed 8 Tx ports—antenna switching; Support of SRS with 8 Tx ports—antenna switching, the number of SRS symbols Ns = {1, 2, 4, 8, 10, 12, 14} FFS: detailed components FFS: separate row for comb8; Yes; n/a; SRS with 8 Tx ports—antenna switching is not supported; [Per band]; n/a; n/a; n/a; Optional with capability signaling. |
| Fujitsu [5] | In addition, for FG 40-5-4/40-5-4a/40-5-6, currently there is FFS on whether to introduce separate row for Comb-8. In our view, we don’t see the necessity to introduce separate rows for Comb-8. So, the FFS part can be removed. 40-5-4; SRS without TDMed 8 Tx ports—antenna switching; Support of SRS with 8 Tx ports—antenna switching FFS: detailed components FFS: separate row for comb8. |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-5-4; SRS without TDMed 8 Tx ports—antenna switching; Support of SRS with 8 Tx ports—antenna switching FFS: detailed components FFS: separate row for comb8 Comments: no need for separate row for comb8; Yes; n/a; SRS without 8 Tx ports—antenna switching is not supported; [Per band]; n/a; n/a; n/a; Optional with capability signaling. |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-5-4/40-5-6: - Only 8T8R is supported for antenna switching with 8Tx, therefore similar as that for FG 23-8-3(SRS Antenna switching for >4Rx), 40-5-4 and 40-5-6 are per band features, and the components of 40-5-4 and 40-5-6 can be as follows: 1. Support of 8T8R SRS antenna switching; 2. Report the entry number of the first-listed band with UL in the band combination that affects this DL; 3. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL. For component-2 and component-3, the candidate values are {1 to 32} and {1 to 32}, respectively. 40-5-4/4a/6/7: - A UE supports 8Tx SRS with comb-2 and comb-4 may not support 8Tx SRS with comb-8. Therefore separate row of comb-8 is needed for 40-5-4/4a/6/7. - 40-5-4/4a/6/7 can be FGs per band. |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-5-4; SRS without TDMed 8 Tx ports—antenna switching; - Support of SRS with 8 Tx ports—antenna switching - Support of repetition factor R; 2-53，2-55，23-8-5，23-8-8; Yes; n/a; SRS with 8 Tx ports—antenna switching is not supported; Per FS; n/a; n/a; n/a; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-5-4/6: - Components: - The following components should be captured: - 1) Support SRS Tx port switch, - 2) Report whether the uplink TX switching impact to downlink receiving in a band, - 3) Report whether the UL Tx is switched together with UL Tx in another band - Separate row for comb8 isn’t necessary. Comb8 is per-band capability, thus, once this FG is supported for a band combination in which all the corresponding bands support comb8 (FG 23-8-8), the support of FG40-5-4/-5 for comb8 is implied. - Type: Per-BC |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-5-4; SRS without TDMed 8 Tx ports—antenna switching; - Support of 8T8R SRS antenna switching. - Report the entry number of the first-listed band with UL in the band combination that affects this DL. - Report the entry number of the first-listed band with UL in the band combination that switches together with this UL. FFS: detailed components FFS: separate row for comb8; Yes; n/a; SRS with 8 Tx ports—antenna switching is not supported; Per FS; n/a; n/a; n/a; Component 1 candidate values: a combination from the set {t1r1, t2r2, t1r2, t4r4, t2r4, t1r4, t2r6, t1r6, t4r8, t2r8, t1r8} Note: Component 1 indicates the supported xTyR in addition to 8T8R. Component 2 candidate values: {1 to 32} Component 3 candidate values: {1 to 32}; Optional with capability signaling. |
| Samsung [16] | Regarding 8-port SRS, non-TDMed port mapping and TDMed port mapping related FGs have been adopted as in Table. 3. For both non-TDMed and TDMed port mapping, our view is that separate rows for Comb-8 is not needed and it could be as a component of FG 40-5-4 and FG 40-5-4a. For TDMed port mapping, our view is that merging FG 40-5-6 and FG 40-5-7 is not needed and we can keep separate rows for antenna switching and codebook. Regarding FG 40-5-4 and FG 40-5-6 which are for antenna switching, the following aspects are further considered. - FG 23-8-3 (SRS Antenna switching for >4Rx) as one of pre-requisites is needed as this is for 8T8R, hence supporting 8R is necessary. - For indicating whether to support comb-8, FG 23-8-8 (Comb-8 SRS) as one of pre-requisites is needed. - Indicating whether to support 8T8R is needed and enough, since a UE can report by FG 23-8-3 for possible combination of antenna switching configurations considering downgrading as well. - Similar with FG 2-55 and FG 23-8-3, reporting the band indexes that affects this DL and switches together with this UL are needed for these new FGs as well, and this applies only for 8T8R. 40-5-4; SRS without TDMed 8 Tx ports—antenna switching; 1. Support 8T8R 2. Report the entry number of the first-listed band with UL in the band combination that affects this DL 3. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL 4. Whether to support comb-8; 2-55 23-8-3 23-8-8; Per band; Component 1 candidate values: {support, not support} Component 2 candidate values: {1 to 32} Component 3 candidate values: {1 to 32} Component 4 candidate values: {support, not support}. |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-5-4; SRS without TDMed 8 Tx ports—antenna switching; 1. Support 8-ports SRS without TDM for antenna switch, without SRS repetition and without SRS frequency hopping 2. Support 8-ports SRS without TDM for antenna switch, with SRS repetition and without SRS frequency hopping 3. Support 8-ports SRS without TDM for antenna switch, without SRS repetition and with SRS frequency hopping 4. Support 8-ports SRS without TDM for antenna switch, with SRS repetition and with SRS frequency hopping 5. Supported repetition factor R 6. Supported number of SRS symbols m in one slot; Yes; N/A; Per FSPC; No; No; N/A; Candidate value for component 5 = a subset of {1, 2, 4, 5, 6, 7, 8, 10, 12, 14} Candidate value for component 6 = a subset {1, 2, 4, 8, 10, 12, 14}; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-5-4b; 8-port nonTDMed SRS with srs-AntennaSwitching2SP-1Periodic; 1.Support of maximum 2 SP 8-port SRS resource sets and maximum 1 periodic 8-port SRS resource set for SRS antenna switching without TDM; Yes; N/A; Per FSPC; No; No; N/A; Note1: - Applies for all supported xTyR where y<=8. - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS. - For xTyR where y<=4, if UE does not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS. - The two SP-SRS resource sets are not activated at the same time.; Optional with capability signaling. |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-5-4a | SRS without TDMed 8 Tx ports—codebook | Support of SRS with 8 Tx ports—codebook FFS: detailed components FFS: separate row for comb8 |  | Yes | n/a | SRS with 8 Tx ports—codebook is not supported | [Per band] | n/a | n/a | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-5-4a - FFS1: we recommend removing the this FFS - FFS2: For SRS with 8 Tx ports—codebook, we propose having a separate row for comb8 mapping 40. NR_MIMO_evo_DL_UL; 40-5-4a; SRS without TDMed 8 Tx ports—codebook; Support of SRS with 8 Tx ports—codebook FFS: detailed components FFS: separate row for comb8; Yes; n/a; SRS with 8 Tx ports—codebook is not supported; [Per band]; n/a; n/a; n/a; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-5-4c; SRS with 8 Tx ports and Comb8 mapping —codebook; Yes; n/a; Per band; n/a; n/a; n/a; Optional with capability signaling. |
| Ericsson [3] | This is the FG for supporting an 8-port SRS resource in an SRS resource set with usage ‘codebook’. In our view, there is no need for a separate FG for comb 8. 40. NR_MIMO_evo_DL_UL; 40-5-4a; 8 Tx SRS—codebook; Support of SRS with 8 Tx ports—codebook; Yes; n/a; SRS with 8 Tx ports—codebook is not supported; Per band; n/a; n/a; n/a; Optional with capability signaling. |
| ZTE [4] | - For FG 40-5-4, FG 40-5-4a, FG 40-5-5, and FG 40-5-6, the brackets of ‘Per band’ should be removed; - For FG 40-5-4, FG 40-5-4a, FG 40-5-5, and FG 40-5-6, we prefer to add details of the components including the value of the number of SRS symbols and the value of TDM factor, and the ‘FFS: detailed components’ should be removed; - For FG 40-5-4, FG 40-5-4a, FG 40-5-5, and FG 40-5-6, there is no justified reason to add a separate row for comb 8, and the ‘FFS: separate row for comb 8’ should be removed; 40. NR_MIMO_evo_DL_UL; 40-5-4a; SRS without TDMed 8 Tx ports—codebook; Support of SRS with 8 Tx ports—codebook FFS: detailed components FFS: separate row for comb8; Yes; n/a; SRS with 8 Tx ports—codebook is not supported; [Per band]; n/a; n/a; n/a; Optional with capability signaling. |
| Fujitsu [5] | In addition, for FG 40-5-4/40-5-4a/40-5-6, currently there is FFS on whether to introduce separate row for Comb-8. In our view, we don’t see the necessity to introduce separate rows for Comb-8. So, the FFS part can be removed. 40-5-4a; SRS without TDMed 8 Tx ports—codebook; Support of SRS with 8 Tx ports—codebook FFS: detailed components FFS: separate row for comb8. |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-5-4a; SRS without TDMed 8 Tx ports—codebook; Support of SRS with 8 Tx ports—codebook FFS: detailed components Comments: no need for separate row for comb8; Yes; n/a; SRS without 8 Tx ports—codebook is not supported; [Per band]; n/a; n/a; n/a; Optional with capability signaling. |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-5-4/4a/6/7: - A UE supports 8Tx SRS with comb-2 and comb-4 may not support 8Tx SRS with comb-8. Therefore separate row of comb-8 is needed for 40-5-4/4a/6/7. - 40-5-4/4a/6/7 can be FGs per band. |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-5-4a; SRS without TDMed 8 Tx ports—codebook; - Support of SRS with 8 Tx ports—codebook - Support of repetition factor R; ,2-52,23-8-5,23-8-8; Yes; n/a; SRS with 8 Tx ports—codebook is not supported; Per FS; n/a; n/a; n/a; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-5-4a/7: - Components: Other than “Support of SRS with TDMed 8 Tx ports—codebook”, no component is needed - Type: Per-band |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-5-4a; SRS without TDMed 8 Tx ports—codebook; Support of SRS with 8 Tx ports—codebook FFS: detailed components FFS: separate row for comb8; Yes; n/a; SRS with 8 Tx ports—codebook is not supported; Per band; n/a; n/a; n/a; Optional with capability signaling. |
| Samsung [16] | Regarding 8-port SRS, non-TDMed port mapping and TDMed port mapping related FGs have been adopted as in Table. 3. For both non-TDMed and TDMed port mapping, our view is that separate rows for Comb-8 is not needed and it could be as a component of FG 40-5-4 and FG 40-5-4a. 40-5-4a; SRS without TDMed 8 Tx ports—codebook; 1. Support of SRS without TDMed 8 Tx ports—codebook 2. Whether to support comb-8; 2-52 or 2-53 23-8-8; Per band; Component 2 candidate values: {support, not support}. |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-5-4a; SRS without TDMed 8 Tx ports—codebook; 1. Support 8-ports SRS without TDM for codebook PUSCH, without SRS repetition and without SRS frequency hopping 2. Support 8-ports SRS without TDM for codebook PUSCH, with SRS repetition and without SRS frequency hopping 3. Support 8-ports SRS without TDM for codebook PUSCH, without SRS repetition and with SRS frequency hopping 4. Support 8-ports SRS without TDM for codebook PUSCH, with SRS repetition and with SRS frequency hopping 5. Supported repetition factor R 6. Supported number of SRS symbols m in one slot; N/A; Per FSPC; Candidate value for component 5 = a subset of {1, 2, 4, 5, 6, 7, 8, 10, 12, 14} Candidate value for component 6 = a subset {1, 2, 4, 8, 10, 12, 14}; Optional with capability signaling. |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-5-6 | SRS with TDMed 8 Tx ports—antenna switching | Support of SRS with TDMed 8 Tx ports—antenna switching FFS: detailed components FFS: separate row for comb8 |  | Yes | n/a | SRS with TDMed 8 Tx ports—antenna switching is not supported | [Per band] | n/a | n/a | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-5-6 - FFS1: we recommend removing the this FFS - FFS2: For SRS with TDMed 8 Tx ports—antenna switching, we propose having a separate row for comb8 mapping as well 40. NR_MIMO_evo_DL_UL; 40-5-6; SRS with TDMed 8 Tx ports—antenna switching; Support of SRS with TDMed 8 Tx ports—antenna switching FFS: detailed components FFS: separate row for comb8; Yes; n/a; SRS with TDMed 8 Tx ports—antenna switching is not supported; [Per band]; n/a; n/a; n/a; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-5-6a; SRS with TDMed 8 Tx ports with Comb8 mapping—antenna switching and codebook; Yes; n/a; Per band; n/a; n/a; n/a; Optional with capability signaling. |
| Ericsson [3] | This FG is for additionally supporting TDM for an 8-port SRS resource in an SRS resource set with usage ‘antennaSwitching’. In our view, this can be merged with FG 40-5-7 as a UE capability for support TDM. 40. NR_MIMO_evo_DL_UL; 40-5-6; TDM for 8 Tx SRS; Support of 8 Tx SRS with TDM; Yes; n/a; 8 Tx SRS with TDM is not supported; Per band; n/a; n/a; n/a; Optional with capability signaling. |
| ZTE [4] | - For FG 40-5-4, FG 40-5-4a, FG 40-5-5, and FG 40-5-6, the brackets of ‘Per band’ should be removed; - For FG 40-5-4, FG 40-5-4a, FG 40-5-5, and FG 40-5-6, we prefer to add details of the components including the value of the number of SRS symbols and the value of TDM factor, and the ‘FFS: detailed components’ should be removed; - For FG 40-5-4, FG 40-5-4a, FG 40-5-5, and FG 40-5-6, there is no justified reason to add a separate row for comb 8, and the ‘FFS: separate row for comb 8’ should be removed; - FG 40-5-6, it is fine to keep two separate rows for ‘codebook’ and ‘antenna switching’, the ‘FFS: merge with antenna switching’ should be removed. 40. NR_MIMO_evo_DL_UL; 40-5-6; SRS with TDMed 8 Tx ports—antenna switching; Support of SRS with TDMed 8 Tx ports—antenna switching, the TDM factor s=2, the number of SRS symbols Ns = {2, 4, 6, 8, 10, 12, 14} FFS: detailed components FFS: separate row for comb8; Yes; n/a; SRS with TDMed 8 Tx ports—antenna switching is not supported; [Per band]; n/a; n/a; n/a; Optional with capability signaling. |
| Fujitsu [5] | In addition, for FG 40-5-4/40-5-4a/40-5-6, currently there is FFS on whether to introduce separate row for Comb-8. In our view, we don’t see the necessity to introduce separate rows for Comb-8. So, the FFS part can be removed. 40-5-6; SRS with TDMed 8 Tx ports—antenna switching; Support of SRS with TDMed 8 Tx ports—antenna switching FFS: detailed components FFS: separate row for comb8. |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-5-6; SRS with TDMed 8 Tx ports—antenna switching; Support of SRS with TDMed 8 Tx ports—antenna switching FFS: detailed components FFS: separate row for comb8 Comments: no need for separate row for comb8; Yes; n/a; SRS with TDMed 8 Tx ports—antenna switching is not supported; [Per band]; n/a; n/a; n/a; Optional with capability signaling. |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-5-4/40-5-6: - Only 8T8R is supported for antenna switching with 8Tx, therefore similar as that for FG 23-8-3(SRS Antenna switching for >4Rx), 40-5-4 and 40-5-6 are per band features, and the components of 40-5-4 and 40-5-6 can be as follows: 1. Support of 8T8R SRS antenna switching; 2. Report the entry number of the first-listed band with UL in the band combination that affects this DL; 3. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL. For component-2 and component-3, the candidate values are {1 to 32} and {1 to 32}, respectively. 40-5-4/4a/6/7: - A UE supports 8Tx SRS with comb-2 and comb-4 may not support 8Tx SRS with comb-8. Therefore separate row of comb-8 is needed for 40-5-4/4a/6/7. - 40-5-4/4a/6/7 can be FGs per band. |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-5-6; SRS with TDMed 8 Tx ports—antenna switching; - Support of SRS with TDMed 8 Tx ports—antenna switching - Support of repetition factor R - Support number of SRS symbols m; 2-53，2-55，23-8-5，23-8-8; Yes; n/a; SRS with TDMed 8 Tx ports—antenna switching is not supported; Per FS; n/a; n/a; Component candidate values: a combination from the set {t1r1, t2r2, t1r2, t4r4, t2r4, t1r4, t2r6, t1r6, t4r8, t2r8, t1r8,t2r8,t4r8,t8r8} Candidate value for component 3 = a subset {1, 2, 4, 8, 10, 12, 14}; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-5-4/6: - Components: - The following components should be captured: - 1) Support SRS Tx port switch, - 2) Report whether the uplink TX switching impact to downlink receiving in a band, - 3) Report whether the UL Tx is switched together with UL Tx in another band - Separate row for comb8 isn’t necessary. Comb8 is per-band capability, thus, once this FG is supported for a band combination in which all the corresponding bands support comb8 (FG 23-8-8), the support of FG40-5-4/-5 for comb8 is implied. - Type: Per-BC |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-5-6; SRS with TDMed 8 Tx ports—antenna switching; Support of SRS with TDMed 8 Tx ports—antenna switching FFS: detailed components FFS: separate row for comb8; 40-5-4; Yes; n/a; SRS with TDMed 8 Tx ports—antenna switching is not supported; Per FS; n/a; n/a; n/a; Optional with capability signaling. |
| Samsung [16] | For TDMed port mapping, our view is that merging FG 40-5-6 and FG 40-5-7 is not needed and we can keep separate rows for antenna switching and codebook. Regarding FG 40-5-4 and FG 40-5-6 which are for antenna switching, the following aspects are further considered. - FG 23-8-3 (SRS Antenna switching for >4Rx) as one of pre-requisites is needed as this is for 8T8R, hence supporting 8R is necessary. - For indicating whether to support comb-8, FG 23-8-8 (Comb-8 SRS) as one of pre-requisites is needed. - Indicating whether to support 8T8R is needed and enough, since a UE can report by FG 23-8-3 for possible combination of antenna switching configurations considering downgrading as well. - Similar with FG 2-55 and FG 23-8-3, reporting the band indexes that affects this DL and switches together with this UL are needed for these new FGs as well, and this applies only for 8T8R. |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-5-6; SRS with TDMed 8 Tx ports—antenna switching; 1. Support 8-ports SRS with TDM for antenna switch, without SRS repetition and without SRS frequency hopping 2. Support 8-ports SRS with TDM for antenna switch, with SRS repetition and without SRS frequency hopping 3. Support 8-ports SRS with TDM for antenna switch, without SRS repetition and with SRS frequency hopping 4. Support 8-ports SRS with TDM for antenna switch, with SRS repetition and with SRS frequency hopping 5. Supported repetition factor R 6. Support number of SRS symbols m; Yes; N/A; Per FSPC; No; No; N/A; Candidate value for component 5 = a subset of {1, 2, 4, 5, 6, 7, 8, 10, 12, 14} Candidate value for component 6 = a subset {1, 2, 4, 8, 10, 12, 14}; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-5-6a; 8-port TDMed SRS with srs-AntennaSwitching2SP-1Periodic; 1.Support of maximum 2 SP 8-port SRS resource sets and maximum 1 periodic 8-port SRS resource set for SRS antenna switching with TDM; Yes; N/A; Per FSPC; No; No; N/A; Note1: - Applies for all supported xTyR where y<=8. - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS. - For xTyR where y<=4, if UE does not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS. - The two SP-SRS resource sets are not activated at the same time.; Optional with capability signaling. |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-5-7 | SRS with TDMed 8 Tx ports—codebook | Support of SRS with TDMed 8 Tx ports—codebook FFS: detailed components FFS: merge with antenna switching |  | Yes | n/a | SRS with TDMed 8 Tx ports—codebook is not supported | [Per band] | n/a | n/a | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-5-7 - FFS2: For SRS with TDMed 8 Tx ports—codebook, we propose having a common row for antenna switching and codebook 40. NR_MIMO_evo_DL_UL; 40-5-7; SRS with TDMed 8 Tx ports—codebook; Support of SRS with TDMed 8 Tx ports—codebook FFS: detailed components FFS: merge with antenna switching; Yes; n/a; SRS with TDMed 8 Tx ports—codebook is not supported; [Per band]; n/a; n/a; n/a; Optional with capability signaling. |
| Ericsson [3] | This FG is for additionally supporting TDM for an 8-port SRS resource in an SRS resource set with usage ‘codebook. In our view, this can be merged with FG 40-5-6 as a UE capability for support TDM. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-5-7; SRS with TDMed 8 Tx ports—codebook; Support of SRS with TDMed 8 Tx ports—codebook, the TDM factor s=2, the number of SRS symbols Ns = {2, 4, 6, 8, 10, 12, 14} FFS: detailed components FFS: merge with antenna switching; Yes; n/a; SRS with TDMed 8 Tx ports—codebook is not supported; [Per band]; n/a; n/a; n/a; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-5-7; SRS with TDMed 8 Tx ports—codebook; Support of SRS with TDMed 8 Tx ports—codebook FFS: detailed components FFS: merge with antenna switching Comments: it’s better to keep the separate row since they are different usages for PUSCH and PDSCH; Yes; n/a; SRS with TDMed 8 Tx ports—codebook is not supported; [Per band]; n/a; n/a; n/a; Optional with capability signaling. |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 0-5-4/4a/6/7: - A UE supports 8Tx SRS with comb-2 and comb-4 may not support 8Tx SRS with comb-8. Therefore separate row of comb-8 is needed for 40-5-4/4a/6/7. - 40-5-4/4a/6/7 can be FGs per band. |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-5-7; SRS with TDMed 8 Tx ports—codebook; - Support of SRS with TDMed 8 Tx ports—codebook - Support of repetition factor R - Support number of SRS symbols m FFS: detailed components FFS: merge with antenna switching; 2-52，2-55，23-8-5，23-8-8; Yes; n/a; SRS with TDMed 8 Tx ports—codebook is not supported; Per FS; n/a; n/a; Candidate value for component 3 = a subset {1, 2, 4, 8, 10, 12, 14}; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-5-4a/7: - Components: Other than “Support of SRS with TDMed 8 Tx ports—codebook”, no component is needed - Type: Per-band |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-5-7; SRS with TDMed 8 Tx ports—codebook; Support of SRS with TDMed 8 Tx ports—codebook FFS: detailed components FFS: merge with antenna switching; Yes; n/a; SRS with TDMed 8 Tx ports—codebook is not supported; Per band; n/a; n/a; n/a; Optional with capability signaling. |
| Samsung [16] | For TDMed port mapping, our view is that merging FG 40-5-6 and FG 40-5-7 is not needed and we can keep separate rows for antenna switching and codebook. 40-5-7; SRS with TDMed 8 Tx ports—codebook; 1. Support of SRS with TDMed 8 Tx ports—codebook 2. Whether to support comb-8; 2-52 or 2-53 23-8-8; Per band; Note: Only 2 TDMed subsets of ports are supported. Component 2 candidate values: {support, not support}. |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-5-7; SRS with TDMed 8 Tx ports—codebook; 1. Support 8-ports SRS with TDM for codebook PUSCH, without SRS repetition and without SRS frequency hopping 2. Support 8-ports SRS with TDM for codebook PUSCH, with SRS repetition and without SRS frequency hopping 3. Support 8-ports SRS with TDM for codebook PUSCH, without SRS repetition and with SRS frequency hopping 4. Support 8-ports SRS with TDM for codebook PUSCH, with SRS repetition and with SRS frequency hopping 5. Supported repetition factor R 6. Support number of SRS symbols m; Yes; N/A; Per FSPC; No; No; N/A; Candidate value for component 5 = a subset of {1, 2, 4, 5, 6, 7, 8, 10, 12, 14} Candidate value for component 6 = a subset {1, 2, 4, 8, 10, 12, 14}; Optional with capability signaling. |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-6-1 | Single-DCI based STx2P SDM scheme for PUSCH—codebook | 1. Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP for PUSCH—codebook 2. 1 PTRS port for single-DCI based STx2P SDM scheme for PUSCH—codebook 3. Support of two SRS resource sets with usage set to 'codebook' 4. Maximum number of SRS resources in one SRS resource set 5. Maximum number of layers of each panel for Single-DCI STx2P with SDM [6. Maximum total number of layers across both panels for Single-DCI STx2P with SDM] [7. Max number of PUSCH ports associated with on SRS resource set] | 2-14 | Yes | N/A | Single-DCI based STx2P SDM scheme for PUSCH—codebook is not supported | [Per FSPC] | No | FR2 only | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-6-1; Single-DCI based STx2P SDM scheme for PUSCH—codebook; 1. Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP for PUSCH—codebook 2. 1 PTRS port for single-DCI based STx2P SDM scheme for PUSCH—codebook 3. Support of two SRS resource sets with usage set to 'codebook' 4. Maximum number of SRS resources in one SRS resource set 5. Maximum number of layers of each panel for Single-DCI STx2P with SDM [6. Maximum total number of layers across both panels for Single-DCI STx2P with SDM] [7. Max number of PUSCH ports associated with on SRS resource set]; 2-14; Yes; N/A; Single-DCI based STx2P SDM scheme for PUSCH—codebook is not supported; [Per FSPC]; No; FR2 only; n/a; Candidate value for component 4: {1,2,4} Candidate value for component 5:{1,2} Candidate value for component 6:{2,3,4} Note: 1+3 and 3+1 are not supported Candidate value for component 7:{1,2,4}; Optional with capability signaling. |
| Ericsson [3] | This is the FG for supporting codebook-based STxMP SDM scheme. There are two highlighted components for this FG: Maximum total number of layers across both panels for Single-DCI STx2P with SDM. We do not see that the number of layers across both panels need to be reported separately: the number of layers per SRS resource set in component 5 is sufficient. Max number of PUSCH ports associated with on SRS resource set. There are no RAN1 agreements on supporting a max number of PUSCH ports per SRS resource set. Hence, since this is a new feature, which should not be introduced in UE-capability discussions, this component is not needed. - Regarding the type, we propose to follow RAN2 guidance and define FG 40-6-1 per band. 40-6-1; Single-DCI based STx2P SDM scheme for PUSCH—codebook; 1. Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP for PUSCH—codebook 2. 1 PTRS port for single-DCI based STx2P SDM scheme for PUSCH—codebook 3. Support of two SRS resource sets with usage set to 'codebook' 4. Maximum number of SRS resources in one SRS resource set 5. Maximum number of layers of each panel for Single-DCI STx2P with SDM; 2-14; Single-DCI based STx2P SDM scheme for PUSCH—codebook is not supported; Per Band. |
| ZTE [4] | - For FG 40-6-1, - Component-6 is not needed and should be removed, because it can be derived from component-5 that same maximum layers per panel as agreed in RAN1. - Component-7 should be revised as “Maximum number of SRS antenna ports for each SRS resource in each SRS resource set”. 40. NR_MIMO_evo_DL_UL; 40-6-1; Single-DCI based STx2P SDM scheme for PUSCH—codebook; 1. Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP for PUSCH—codebook 2. 1 PTRS port for single-DCI based STx2P SDM scheme for PUSCH—codebook 3. Support of two SRS resource sets with usage set to 'codebook' 4. Maximum number of SRS resources in one SRS resource set 5. Maximum number of layers of each panel for Single-DCI STx2P with SDM [6. Maximum total number of layers across both panels for Single-DCI STx2P with SDM] [7. Max number of PUSCH ports associated with on SRS resource set] 6. Maximum number of SRS antenna ports for each SRS resource in each SRS resource set for Single-DCI STx2P with SDM; 2-14; Yes; N/A; Single-DCI based STx2P SDM scheme for PUSCH—codebook is not supported; [Per FSPC] Per FSPC; No; FR2 only; n/a; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-6-1; Single-DCI based STx2P SDM scheme for PUSCH—codebook; 1. Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP for PUSCH—codebook 2. 1 PTRS port for single-DCI based STx2P SDM scheme for PUSCH—codebook 3. Support of two SRS resource sets with usage set to 'codebook' 4. Maximum number of SRS resources in one SRS resource set 5. Maximum number of layers of each panel for Single-DCI STx2P with SDM 6. Maximum total number of layers across both panels for Single-DCI STx2P with SDM 7. Max number of PUSCH ports associated with one SRS resource set 8. Maximum number of SRS antenna ports for each SRS resource in each SRS resource set; 2-14, one of {40-1-1/1a, 40-1-2/2a}; Yes; N/A; Single-DCI based STx2P SDM scheme for PUSCH—codebook is not supported; [Per FSPC]; No; FR2 only; n/a; Optional with capability signaling. |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-6-1: - It was agreed to indicate the maximal number of layers for single-DCI based STx2P SDM scheme in each panel as those in component 5, it is also necessary to introduce the maximal number of layers for both panels in FG 40-6-1 for flexibility in case the UE capability of the total maximal number for both panels is the same as that in each panel. Besides, it is acceptable for component 7. Proposal 12: Keep component 6 and component 7 for FG 40-6-1. |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-6-1; Single-DCI based STx2P SDM scheme for PUSCH—codebook; 1. Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP for PUSCH—codebook 2. 1 PTRS port for single-DCI based STx2P SDM scheme for PUSCH—codebook 3. Support of two SRS resource sets with usage set to 'codebook' 4. Maximum number of SRS resources in one SRS resource set 5. Maximum number of layers of each panel for Single-DCI STx2P with SDM; 2-14; Yes; N/A; Single-DCI based STx2P SDM scheme for PUSCH—codebook is not supported; Per FSPC; No; FR2 only; Candidate value for component 4: {1,2,4} Candidate value for component 5: {1,2}; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-6-1-1; 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—codebook; Support of 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—codebook; 40-6-1; Yes; N/A; 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—codebook is not supported; Per FSPC; No; FR2 only; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-6-1 - Component - Candidate value of component 4: {1,2,4} - Candidate value of component 5: {1,2} - Component 6 seems duplicated with component 5 and should be removed. - Support component 7 with candidate value as {1,2,4} - In addition, another component max number of SRS ports of each SRS resource should be included with candidate value as {1,2,4} |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-6-1; Single-DCI based STx2P SDM scheme for PUSCH—codebook; 1. Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP for PUSCH—codebook 2. 1 PTRS port for single-DCI based STx2P SDM scheme for PUSCH—codebook 3. Support of two SRS resource sets with usage set to 'codebook' 4. Maximum number of SRS resources in one SRS resource set 5. Maximum number of layers of each panel for Single-DCI STx2P with SDM 6. Maximum total number of layers across both panels for Single-DCI STx2P with SDM 7. Maximum number of SRS antenna ports for each SRS resource in each SRS resource set [7. Max number of PUSCH ports associated with on SRS resource set]; 2-14; Yes; N/A; Single-DCI based STx2P SDM scheme for PUSCH—codebook is not supported; Per FSPC; No; FR2 only; n/a; Component 4 candidate values: {1, 2 ,4} Component 5 candidate values: {1, 2} Component 6 candidate values: {2 ,3, 4} Component 7 candidate values: {1, 2 [,4]} Note: If value 4 is reported for component 4, UE also reports value 4 in FG 16-5c.; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | - For 40-6-1 (SDM, codebook): - A first component is added to make the description aligned with the first component of 40-6-2 (SFN). This is common understanding in RAN1 anyway but should be spelled out for RAN2 to capture it accurately. - The component “Maximum total number of layers across both panels for Single-DCI STx2P with SDM” is unclear as it should be always twice of the already agreed component “Maximum number of layers of each panel for Single-DCI STx2P with SDM”. Hence, the need for signaling this additional number is unclear to us. - The component “Max number of PUSCH ports associated with one SRS resource set” is needed because of simultaneous transmission and given that the number of PUSCH ports may not be the same as the number of layers in codebook-based PUSCH. - Reporting type should be “per FSPC” given that number of layers is indicated (which is always reported per FSPC given the RF / baseband complexity associated with it) - Details of candidate values for various components are also added. 40. NR_MIMO_evo_DL_UL; 40-6-1; Single-DCI based STx2P SDM scheme for PUSCH—codebook; 1. Support of single-DCI based STx2P SDM scheme for codebook based PUSCH 2. Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP for PUSCH—codebook 3. 1 PTRS port for single-DCI based STx2P SDM scheme for PUSCH—codebook 4. Support of two SRS resource sets with usage set to 'codebook' 5. Maximum number of SRS resources in one SRS resource set 6. Maximum number of layers of each panel for Single-DCI STx2P with SDM 7. Max number of PUSCH ports associated with one SRS resource set; 2-14; Yes; N/A; Single-DCI based STx2P SDM scheme for PUSCH—codebook is not supported; Per FSPC; No; FR2 only; n/a; Component 5 candidate values: {1,2,4} Note: If value 4 is reported for component 5, UE also reports value 4 in FG 16-5c. Component 6 candidate values: {1,2} Component 7 candidate values: {1,2,4} Note: If a row of the TPMI consists of all 0’s, the corresponding PUSCH port is not counted.; Optional with capability signaling. |
| Huawei/HiSilicon [19] | For FG 40-6-1 and 40-6-1-1, we have the following proposal: Proposal 5-1: - Regarding FG 40-6-1, support component 6 and 7. - Regarding FG 40-6-1 and FG 40-6-1-1, support Per FS reporting granularity. 40. NR_MIMO_evo_DL_UL; 40-6-1; Single-DCI based STx2P SDM scheme for PUSCH—codebook; 1. Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP for PUSCH—codebook 2. 1 PTRS port for single-DCI based STx2P SDM scheme for PUSCH—codebook 3. Support of two SRS resource sets with usage set to 'codebook' 4. Maximum number of SRS resources in one SRS resource set 5. Maximum number of layers of each panel for Single-DCI STx2P with SDM [6. Maximum total number of layers across both panels for Single-DCI STx2P with SDM] [7. Max number of PUSCH ports associated with on SRS resource set]; 2-14; Yes; N/A; Single-DCI based STx2P SDM scheme for PUSCH—codebook is not supported; [Per FSPC]; No; FR2 only; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-6-1-1 | 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—codebook | Support of 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—codebook | 40-6-1 | Yes | N/A | 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—codebook is not supported | [Per FSPC] | No | FR2 only | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] | - Regarding the type, we propose to follow RAN2 guidance and define FG 40-6-1-1 per band. 40-6-1-1; 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—codebook; Support of 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—codebook; 40-6-1; 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—codebook is not supported; Per Band. |
| ZTE [4] |  |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-6-1-1; 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—codebook; Support of 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—codebook; 40-6-1; Yes; N/A; 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—codebook is not supported; Per FS; No; FR2 only; n/a; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-6-1-1; 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—codebook; Support of 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—codebook; 40-6-1; Yes; N/A; 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—codebook is not supported; Per band; No; FR2 only; n/a; Optional with capability signaling. |
| Huawei/HiSilicon [19] | For FG 40-6-1 and 40-6-1-1, we have the following proposal: Proposal 5-1: - Regarding FG 40-6-1, support component 6 and 7. - Regarding FG 40-6-1 and FG 40-6-1-1, support Per FS reporting granularity. 40. NR_MIMO_evo_DL_UL; 40-6-1-1; 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—codebook; Support of 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—codebook; 40-6-1; Yes; N/A; 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—codebook is not supported; Per FSPC; No; FR2 only; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-6-1a | Single-DCI based STx2P SDM scheme for PUSCH—noncodebook | 1. Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP for PUSCH—noncodebook 2. 1 PTRS port for single-DCI based STx2P SDM scheme for PUSCH—noncodebook 3. Support of two SRS resource sets with usage set to 'noncodebook' 4. Maximum number of SRS resources in one SRS resource set 5. Maximum number of layers of each panel for Single-DCI STx2P with SDM [6. Maximum total number of layers across both panels for Single-DCI STx2P with SDM] [7. Maximum number of simultaneous transmitted SRS resources at one symbol] [8. Maximum number of simultaneous transmitted SRS resources from one SRS resource set at one symbol] | FFS | Yes | N/A | Single-DCI based STx2P SDM scheme for PUSCH—noncodebook is not supported | [Per FSPC] | No | FR2 only | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-6-1a and 40-6-2a - There are no agreements regarding “Maximum number of simultaneous transmitted SRS resources at one symbol” and “Maximum number of simultaneous transmitted SRS resources from one SRS resource set at one symbol] in rel. 18. Thus, they should be deleted. 40. NR_MIMO_evo_DL_UL; 40-6-1a; Single-DCI based STx2P SDM scheme for PUSCH—noncodebook; 1. Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP for PUSCH—noncodebook 2. 1 PTRS port for single-DCI based STx2P SDM scheme for PUSCH—noncodebook 3. Support of two SRS resource sets with usage set to 'noncodebook' 4. Maximum number of SRS resources in one SRS resource set 5. Maximum number of layers of each panel for Single-DCI STx2P with SDM [6. Maximum total number of layers across both panels for Single-DCI STx2P with SDM] [7. Maximum number of simultaneous transmitted SRS resources at one symbol] [8. Maximum number of simultaneous transmitted SRS resources from one SRS resource set at one symbol]; FFS; Yes; N/A; Single-DCI based STx2P SDM scheme for PUSCH—noncodebook is not supported; [Per FSPC]; No; FR2 only; n/a; Candidate value for component 4:{1,2,4} Candidate value for component 5: {1,2} Candidate value for component 6: {2,3,4} Note: 1+3 and 3+1 are not supported; Optional with capability signaling. |
| Ericsson [3] | This is the FG for supporting non-codebook-based STxMP SDM scheme. There are three highlighted components for this FG: Maximum total number of layers across both panels for Single-DCI STx2P with SDM. We do not see that the number of layers across both panels need to be reported separately: the number of layers per SRS resource set in component 5 is sufficient. Maximum number of simultaneous transmitted SRS resources at one symbol. One of component 7 and 8 is needed, which one may depend on whether simultaneous transmission of two SRS resource sets is supported or not, which has not yet been decided in RAN1. Since component 8 works irrespectively of whether simultaneous transmission of two SRS resource sets is supported it can be kept. Maximum number of simultaneous transmitted SRS resources from one SRS resource set at one symbol. One of component 7 and 8 is needed, which one may depend on whether simultaneous transmission of two SRS resource sets is supported or not, which has not yet been decided in RAN1. Since component 8 works irrespectively of whether simultaneous transmission of two SRS resource sets is supported it can be kept.RAN1. Regarding the type, we propose to follow RAN2 guidance and define FG 40-6-1a per band. The pre-requisite FG is also highlighted. In our understanding, FG 2-15 (non-codebook based PUSCH transmission) should be a pre-requisite. 40-6-1a; Single-DCI based STx2P SDM scheme for PUSCH—noncodebook; 1. Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP for PUSCH—noncodebook 2. 1 PTRS port for single-DCI based STx2P SDM scheme for PUSCH—noncodebook 3. Support of two SRS resource sets with usage set to 'noncodebook' 4. Maximum number of SRS resources in one SRS resource set 5. Maximum number of layers of each panel for Single-DCI STx2P with SDM 6. Maximum number of simultaneous transmitted SRS resources from one SRS resource set at one symbol; 2-15; Single-DCI based STx2P SDM scheme for PUSCH—noncodebook is not supported; Per Band. |
| ZTE [4] | - For FG 40-6-1a, - Component-6 is redundant and should be removed, because it can be derived from component-5 that same maximum layers per panel as agreed in RAN1. - Component-7 is needed according to our elaboration in R1-2309018, where the candidate values can include {1, 2, 3, 4}. - Component-8 is needed according to our elaboration in R1-2309018, where the candidate values can include {1, 2}. 40. NR_MIMO_evo_DL_UL; 40-6-1a; Single-DCI based STx2P SDM scheme for PUSCH—noncodebook; 1. Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP for PUSCH—noncodebook 2. 1 PTRS port for single-DCI based STx2P SDM scheme for PUSCH—noncodebook 3. Support of two SRS resource sets with usage set to 'noncodebook' 4. Maximum number of SRS resources in one SRS resource set 5. Maximum number of layers of each panel for Single-DCI STx2P with SDM [6. Maximum total number of layers across both panels for Single-DCI STx2P with SDM] [7. Maximum number of simultaneous transmitted SRS resources at one symbol] [8. Maximum number of simultaneous transmitted SRS resources from one SRS resource set at one symbol]; FFS; Yes; N/A; Single-DCI based STx2P SDM scheme for PUSCH—noncodebook is not supported; [Per FSPC]; No; FR2 only; n/a; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-6-1a; Single-DCI based STx2P SDM scheme for PUSCH—noncodebook; 1. Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP for PUSCH—noncodebook 2. 1 PTRS port for single-DCI based STx2P SDM scheme for PUSCH—noncodebook 3. Support of two SRS resource sets with usage set to 'noncodebook' 4. Maximum number of SRS resources in one SRS resource set 5. Maximum number of layers of each panel for Single-DCI STx2P with SDM 6. Maximum total number of layers across both panels for Single-DCI STx2P with SDM [7. Maximum number of simultaneous transmitted SRS resources at one symbol] 8. Maximum number of simultaneous transmitted SRS resources from one SRS resource set at one symbol; 2-15, one of {40-1-1/1a, 40-1-2/2a}; Yes; N/A; Single-DCI based STx2P SDM scheme for PUSCH—noncodebook is not supported; [Per FSPC]; No; FR2 only; n/a; Optional with capability signaling. |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-6-1a: - As it is agreed to implement single-DCI based STx2P SDM scheme for non-codebook based PUSCH transmission, FG 2-15 (basic UE features for non-codebook based PUSCH transmission in Rel-15) should be applied as the prerequisite FG. - Similar to 40-6-1, it is not necessary to keep component 6 and 7 while keeping component 8 in FG 40-6-1a. Proposal 13: Adopt the following changes for FG 40-6-1a: - The prerequisite of FG 40-6-1a is FG 2-15; - Remove component 6, 7 and keep component 8 for FG 40-6-1a. |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-6-1a; Single-DCI based STx2P SDM scheme for PUSCH—noncodebook; 1. Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP for PUSCH—noncodebook 2. 1 PTRS port for single-DCI based STx2P SDM scheme for PUSCH—noncodebook 3. Support of two SRS resource sets with usage set to 'noncodebook' 4. Maximum number of SRS resources in one SRS resource set 5. Maximum number of layers of each panel for Single-DCI STx2P with SDM; 2-15; Yes; N/A; Single-DCI based STx2P SDM scheme for PUSCH—noncodebook is not supported; Per FSPC; No; FR2 only; Candidate value for component 4: {1,2,4} Candidate value for component 5: {1,2}; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-6-1a - Component: - Candidate value of component 4: {1,2,3,4} - Candidate value of component 5: {1,2} - Component 6 seems duplicated with component 5 and should be removed. - Support component 8 with candidate value as {1,2,3,4} - For component 7, we have not agreed on simultaneous transmission of two SRS resource sets. It can be discussed later. - Prerequisite FG: 2-15 (NCB based PUSCH). |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-6-1a; Single-DCI based STx2P SDM scheme for PUSCH—noncodebook; 1. Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP for PUSCH—noncodebook 2. 1 PTRS port for single-DCI based STx2P SDM scheme for PUSCH—noncodebook 3. Support of two SRS resource sets with usage set to 'noncodebook' 4. Maximum number of SRS resources in one SRS resource set 5. Maximum number of layers of each panel for Single-DCI STx2P with SDM 6. Maximum total number of layers across both panels for Single-DCI STx2P with SDM 7. Maximum number of simultaneous transmitted SRS resources at one symbol 8. Maximum number of simultaneous transmitted SRS resources from one SRS resource set at one symbol; FFS; Yes; N/A; Single-DCI based STx2P SDM scheme for PUSCH—noncodebook is not supported; Per FSPC; No; FR2 only; n/a; Component 4 candidate values: {1, 2 ,3, 4} Component 5 candidate values: {1, 2} Component 6 candidate values: {2 ,3, 4} Component 7 candidate values: {1, 2, 3, 4, 5, ,6, 7, 8} Component 8 candidate values: {1, 2, 3, 4}; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | - For 40-6-1a (SDM, non-codebook): - Similar to above, a first component is added to make the description aligned with the first component of 40-6-2a (SFN). - Similar to above, the component on “Maximum total number of layers across both panels for Single-DCI STx2P with SDM” seems not needed. - The component “Maximum number of simultaneous transmitted SRS resources at one symbol” should be removed because it is unclear if these resources belong to the same or different SRS resource sets. For different SRS resource sets, this means simultaneous SRS transmission with different beams, which is a new feature itself and not agreed. - The last component “Maximum number of simultaneous transmitted SRS resources from one SRS resource set at one symbol” can be kept since it is similar to Rel-15 FG 2-15 (component 3). - Reporting type should be “per FSPC” given that number of layers is indicated (which is always reported per FSPC given the RF / baseband complexity associated with it) - Details of candidate values for various components are also added. 40. NR_MIMO_evo_DL_UL; 40-6-1a; Single-DCI based STx2P SDM scheme for PUSCH—noncodebook; 1. Support of single-DCI based STx2P SDM scheme for non-codebook based PUSCH 2. Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP for PUSCH—noncodebook 3. 1 PTRS port for single-DCI based STx2P SDM scheme for PUSCH—noncodebook 4. Support of two SRS resource sets with usage set to 'noncodebook' 5. Maximum number of SRS resources in one SRS resource set 6. Maximum number of layers of each panel for Single-DCI STx2P with SDM 7. Maximum number of simultaneous transmitted SRS resources from one SRS resource set at one symbol; 2-15; Yes; N/A; Single-DCI based STx2P SDM scheme for PUSCH—noncodebook is not supported; Per FSPC; No; FR2 only; n/a; Component 5 candidate values: {1,2,3,4} Component 6 candidate values: {1,2} Component 7 candidate values: {1,2,3,4}; Optional with capability signaling. |
| Huawei/HiSilicon [19] | For FG 40-6-1a and 40-6-1a-1, we have the following proposal: Proposal 5-2: - Regarding FG 40-6-1a, support component 6, 7 and 8. Prerequisite of FG 40-6-1a is FG 2-15. - Regarding FG 40-6-1a and FG 40-6-1a-1, support Per FS reporting granularity. 40. NR_MIMO_evo_DL_UL; 40-6-1a; Single-DCI based STx2P SDM scheme for PUSCH—noncodebook; 1. Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP for PUSCH—noncodebook 2. 1 PTRS port for single-DCI based STx2P SDM scheme for PUSCH—noncodebook 3. Support of two SRS resource sets with usage set to 'noncodebook' 4. Maximum number of SRS resources in one SRS resource set 5. Maximum number of layers of each panel for Single-DCI STx2P with SDM [6. Maximum total number of layers across both panels for Single-DCI STx2P with SDM] [7. Maximum number of simultaneous transmitted SRS resources at one symbol] [8. Maximum number of simultaneous transmitted SRS resources from one SRS resource set at one symbol]; FFS 2-15; Yes; N/A; Single-DCI based STx2P SDM scheme for PUSCH—noncodebook is not supported; Per FSPC; No; FR2 only; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-6-1a-1 | 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—noncodebook | Support of 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—noncodebook | 40-6-1a | Yes | N/A | 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—noncodebook is not supported | [Per FSPC] | No | FR2 only | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] | - Regarding the type, we propose to follow RAN2 guidance and define FG 40-6-1a-1 per band. 40-6-1a-1; 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—noncodebook; Support of 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—noncodebook; 40-6-1a; 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—noncodebook is not supported; Per Band. |
| ZTE [4] |  |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-6-1a-1; 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—noncodebook; Support of 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—noncodebook; 40-6-1a; Yes; N/A; 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—noncodebook is not supported; Per FSPC; No; FR2 only; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-6-1a-1; 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—noncodebook; Support of 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—noncodebook; 40-6-1a; Yes; N/A; 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—noncodebook is not supported; Per FS; No; FR2 only; n/a; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-6-1a-1; 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—noncodebook; Support of 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—noncodebook; 40-6-1a; Yes; N/A; 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—noncodebook is not supported; Per band; No; FR2 only; n/a; Optional with capability signaling. |
| Huawei/HiSilicon [19] | For FG 40-6-1a and 40-6-1a-1, we have the following proposal: Proposal 5-2: - Regarding FG 40-6-1a, support component 6, 7 and 8. Prerequisite of FG 40-6-1a is FG 2-15. - Regarding FG 40-6-1a and FG 40-6-1a-1, support Per FS reporting granularity. 40. NR_MIMO_evo_DL_UL; 40-6-1a-1; 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—noncodebook; Support of 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—noncodebook; 40-6-1a; Yes; N/A; 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—noncodebook is not supported; Per FSPC; No; FR2 only; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-6-1b | Association between CSI-RS and SRS for noncodebook single-DCI based STx2P SDM scheme for PUSCH | 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook based STxMP SDM scheme for PUSCH 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously | [2-15a, 40-6-1a | Yes | N/A | Associated CSI-RS resources for non-codebook single-DCI based STxMP SDM scheme for PUSCH is not supported | [Per Band] | n/a | FR2 only | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-6-1b; Association between CSI-RS and SRS for noncodebook single-DCI based STx2P SDM scheme for PUSCH; 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook based STxMP SDM scheme for PUSCH 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously; [2-15a, 40-6-1a; Yes; N/A; Associated CSI-RS resources for non-codebook single-DCI based STxMP SDM scheme for PUSCH is not supported; [Per Band]; n/a; FR2 only; n/a; Candidate value for component 2:{1 to 8} Candidate value for component 3:{1 to 8} Candidate value for component 4:{0 to 8} Candidate value for component 5:{ 1 to 16} Candidate value for component 6: {1,2}; Optional with capability signalling. |
| Ericsson [3] | The pre-requisite for this FG should be FG 2-15a. The type should be per band. 40-6-1b; Association between CSI-RS and SRS for noncodebook single-DCI based STx2P SDM scheme for PUSCH; 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook based STxMP SDM scheme for PUSCH 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously; 2-15a, 40-6-1a; Associated CSI-RS resources for non-codebook single-DCI based STxMP SDM scheme for PUSCH is not supported; Per Band. |
| ZTE [4] |  |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-6-1b; Association between CSI-RS and SRS for noncodebook single-DCI based STx2P SDM scheme for PUSCH; 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook based STxMP SDM scheme for PUSCH 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously; 2-15a, 40-6-1a; Yes; N/A; Associated CSI-RS resources for non-codebook single-DCI based STxMP SDM scheme for PUSCH is not supported; Per Band; No; FR2 only; n/a; Optional with capability signalling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-6-1b: - Component: - Candidate value of component 2 can be {1 to 8}. - Candidate value of component 3 can be {1 to 8}. - Candidate value of component 4 can be {0 to 8}. - Candidate value of component 5 can be {1 to 16}. - Candidate value of component 6 can be {1, 2}. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-6-1b; Association between CSI-RS and SRS for noncodebook single-DCI based STx2P SDM scheme for PUSCH; 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook based STxMP SDM scheme for PUSCH 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously; [2-15a, 40-6-1a; Yes; N/A; Associated CSI-RS resources for non-codebook single-DCI based STxMP SDM scheme for PUSCH is not supported; Per band; n/a; FR2 only; n/a; Component 2 candidate values: {1 to 8} Component 3 candidate values: {1 to 8} Component 4 candidate values: {0 to 8} Component 5 candidate values: {1 to 16} Component 6 candidate values: {1 to 2}; Optional with capability signalling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-6-1b; Association between CSI-RS and SRS for noncodebook single-DCI based STx2P SDM scheme for PUSCH; 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook based STxMP SDM scheme for PUSCH 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously; 2-15a, 40-6-1a; Yes; N/A; Associated CSI-RS resources for non-codebook single-DCI based STxMP SDM scheme for PUSCH is not supported; Per Band; n/a; FR2 only; n/a; Component 2: {1 to 8} Component 3: {1 to 8} Component 4: {0 to 8} Component 5: {1 to 16} Component 6: {1,2}; Optional with capability signalling. |
| Huawei/HiSilicon [19] | For FG 40-6-1b, we think, similar to the prerequisite FG 2-15a, the reporting granularity should be Per band. Regarding the reporting granularity of FG 40-6-1-2, it can also follow the reporting granularity of FG 16-2b-1b (new DMRS port entry for DL SDM) which is Per band. Proposal 5-3: For FG 40-6-1b and 40-6-1-2, support Per band reporting granularity. 40. NR_MIMO_evo_DL_UL; 40-6-1b; Association between CSI-RS and SRS for noncodebook single-DCI based STx2P SDM scheme for PUSCH; 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook based STxMP SDM scheme for PUSCH 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously; [2-15a, 40-6-1a; Yes; N/A; Associated CSI-RS resources for non-codebook single-DCI based STxMP SDM scheme for PUSCH is not supported; [Per Band]; n/a; FR2 only; n/a; Optional with capability signalling. |

| 40. NR_MIMO_evo_DL_UL | 40-6-1-2 | New DMRS port entry for single-DCI based SDM scheme | Support of new DMRS port entry {0, 2, 3} | 40-6-1,40-6-1a | Yes | N/A | New DMRS port entry for single-DCI based SDM scheme is not supported | [Per Band] | n/a | FR2 only | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] | - We propose to clarify that this is for STx2P in the FG name. - Regarding the type, we propose to follow RAN2 guidance and define FG 40-6-1-2 per band. 40-6-1-2; New DMRS port entry for single-DCI based STx2P SDM scheme; Support of new DMRS port entry {0, 2, 3}; 40-6-1,40-6-1a; New DMRS port entry for single-DCI based SDM scheme is not supported; Per Band. |
| ZTE [4] |  |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-6-1-2; New DMRS port entry for single-DCI based SDM scheme; Support of new DMRS port entry {0, 2, 3}; 40-6-1,40-6-1a; Yes; N/A; New DMRS port entry for single-DCI based SDM scheme is not supported; Per Band; No; FR2 only; Optional with capability signalling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-6-1-2; New DMRS port entry for single-DCI based SDM scheme; Support of new DMRS port entry {0, 2, 3}; 40-6-1,40-6-1a; Yes; N/A; New DMRS port entry for single-DCI based SDM scheme is not supported; Per band; n/a; FR2 only; n/a; Optional with capability signalling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-6-1-2; New DMRS port entry for single-DCI based SDM scheme; Support of new DMRS port entry {0, 2, 3}; 40-6-1,40-6-1a; Yes; N/A; New DMRS port entry for single-DCI based SDM scheme is not supported; Per Band; n/a; FR2 only; n/a; Optional with capability signalling. |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-6-1-2; New DMRS port entry for single-DCI based SDM scheme; Support of new DMRS port entry {0, 2, 3}; 40-6-1,40-6-1a; Yes; N/A; New DMRS port entry for single-DCI based SDM scheme is not supported; Per Band; n/a; FR2 only; n/a; Optional with capability signalling. |

| 40. NR_MIMO_evo_DL_UL | 40-6-2 | Single-DCI based STx2P SFN scheme for PUSCH—codebook | 1. Support of single-DCI based STx2P SFN scheme for codebook based PUSCH 2.Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP 3. 1 PTRS port for single-DCI based STx2P SFN scheme for PUSCH—codebook 4. Support of two SRS resource sets with usage set to 'codebook' 5. Supported number of SRS resources in one SRS resource set 6. Supported maximal number of MIMO layers of each SRS resource set for CB PUSCH with SFN scheme [7. Maximum number of SRS antenna ports for each SRS resource in each SRS resource set] [8. Maximum total number of layers across both panels for Single-DCI STx2P with SFN] [9. Max number of PUSCH ports associated with SRS resource set] | 2-14 | Yes | n/a | Single-DCI based STx2P SFN scheme for PUSCH—codebook is not supported | [Per FS] | n/a | FR2 only | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-6-2; Single-DCI based STx2P SFN scheme for PUSCH—codebook; 1. Support of single-DCI based STx2P SFN scheme for codebook based PUSCH 2.Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP 3. 1 PTRS port for single-DCI based STx2P SFN scheme for PUSCH—codebook 4. Support of two SRS resource sets with usage set to ‘codebook’ 5. Supported number of SRS resources in one SRS resource set 6. Supported maximal number of MIMO layers of each SRS resource set for CB PUSCH with SFN scheme [7. Maximum number of SRS antenna ports for each SRS resource in each SRS resource set] [8. Maximum total number of layers across both panels for Single-DCI STx2P with SFN] [9. Max number of PUSCH ports associated with SRS resource set]; 2-14; Yes; n/a; Single-DCI based STx2P SFN scheme for PUSCH—codebook is not supported; [Per FS]; n/a; FR2 only; n/a; Candidate value for component 5:{1,2,4} Candidate value for component 6{1,2} Candidate value for component 7:{1,2,4} Candidate value for component 8:{2,3,4} Note: 1+3 and 3+1 are not supported Candidate value for component 9:(1,2,4}; Optional with capability signaling. |
| Ericsson [3] | This is the FG for supporting codebook-based STxMP SFN scheme. There are three highlighted components in FG 40-6-2: Maximum number of SRS antenna ports for each SRS resource in each SRS resource set. We do not see that this is needed. Maximum total number of layers across both panels for Single-DCI STx2P with SFN. We do not see that the number of layers across both panels need to be reported separately: the number of layers per SRS resource set in component 6 is sufficient. In fact, with SFN, the number of layers per panel is the same as the number of layers over panels. Max number of PUSCH ports associated with SRS resource set. There are no RAN1 agreements on supporting a max number of PUSCH ports per SRS resource set. Hence, since this is a new feature, which should not be introduced in UE-capability discussions, this component is not needed. Regarding the type, we propose to follow RAN2 guidance and define FG 40-6-2 per band. 40-6-2; Single-DCI based STx2P SFN scheme for PUSCH—codebook; 1. Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP 2. 1 PTRS port for single-DCI based STx2P SFN scheme for PUSCH—codebook 3. Support of two SRS resource sets with usage set to 'codebook' 4. Supported number of SRS resources in one SRS resource set 5. Supported maximal number of MIMO layers of each SRS resource set for CB PUSCH with SFN scheme; 2-14; Single-DCI based STx2P SFN scheme for PUSCH—codebook is not supported; Per Band. |
| ZTE [4] | - For FG 40-6-2, - Component-8 is not needed and should be removed, because it can be derived from component-6 that same maximum layers per panel as agreed in RAN1. - Component-9 is not needed and should be removed, because it can be derived from component-7 that same maximum ports per panel as agreed in RAN1. 40. NR_MIMO_evo_DL_UL; 40-6-3a; codebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG; 1. Support of multi-DCI based STx2P PUSCH+PUSCH for codebook-based PUSCH with fully[/partial] overlapping PUSCHs in time and [non-overlapping in frequency] 2. Support of two SRS resource sets with usage set to 'codebook' associated with two coresetPoolIndex values 3. Maximum number of SRS resources in one SRS resource set 4. Maximum number of layers of each PUSCH of PUSCH+PUSCH overlapping in time domain [5. Supported maximum number of NZP PUSCH ports for each PUSCH of PUSCH+PUSCH overlapping in time domain] 6. Maximum number of PUSCHs per CORESETPoolIndex per slot 7. Maximum total number of layers across two overlapping PUSCH 8. Maximum number of SRS antenna ports for each SRS resource in each SRS resource set; 2-14; Yes; N/A; Codebook multi-DCI based STx2P PUSCH+PUSCH is not supported; [Per Band]; n/a; FR2 only; n/a; Note: Processing capability 2 is not supported in any CC if at least one CC is configured with two values of CORESETPoolIndex. Component 3 candidate values: {1,2,4} Note: If value 4 is reported for component 3, UE also reports value 4 in FG 16-5c. Component 4 candidate values: {1,2} [Component 5 candidate values: {1,2,4} Note: If a row of the TPMI consists of all 0’s, the corresponding PUSCH port is not counted.] Component 6 candidate values: {1,2,3,4,7} Note: per SCS, similar with Rel-15 Component 7 candidate values: {2 ,3, 4} Component 8 candidate values: {1, 2 ,4}; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-6-2; Single-DCI based STx2P SFN scheme for PUSCH—codebook; 1. Support of single-DCI based STx2P SFN scheme for codebook based PUSCH 2.Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP 3. 1 PTRS port for single-DCI based STx2P SFN scheme for PUSCH—codebook 4. Support of two SRS resource sets with usage set to 'codebook' 5. Supported number of SRS resources in one SRS resource set 6. Supported maximal number of MIMO layers of each SRS resource set for CB PUSCH with SFN scheme 7. Maximum number of SRS antenna ports for each SRS resource in each SRS resource set 8. Maximum total number of layers across both panels for Single-DCI STx2P with SFN 9. Max number of PUSCH ports associated with SRS resource set; 2-14, one of {40-1-1/1a, 40-1-2/2a}; Yes; n/a; Single-DCI based STx2P SFN scheme for PUSCH—codebook is not supported; [Per FS]; n/a; FR2 only; n/a; Optional with capability signaling. |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-6-2: - As maximum number of SRS antenna ports for each SRS resource in each SRS resource set and the maximum total number of layers across both panels for SFN are the same, it is our view to keep component 7 while remove component 8.. Besides, number of PUSCH ports can be indirectly reflected by maximum number of SRS antenna ports for each SRS resource in each SRS resource set, therefore, it is then not necessary to introduce component 9. Proposal 14: Keep component 7 while remove component 8 and 9 in FG 40-6-2. |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-6-2; Single-DCI based STx2P SFN scheme for PUSCH—codebook; 1. Support of single-DCI based STx2P SFN scheme for codebook based PUSCH 2.Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP 3. 1 PTRS port for single-DCI based STx2P SFN scheme for PUSCH—codebook 4. Support of two SRS resource sets with usage set to 'codebook' 5. Supported number of SRS resources in one SRS resource set 6. Supported maximal number of MIMO layers of each SRS resource set for CB PUSCH with SFN scheme; 2-14; Yes; n/a; Single-DCI based STx2P SFN scheme for PUSCH—codebook is not supported; Per FS; No; FR2 only; Candidate value for component 5: {1,2,4} Candidate value for component 6: {1,2}; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-6-2 - Component: - Candidate value of component 5: {1,2,4} - Candidate value of component 6: {1,2} - Support component 7 with candidate value as {1,2,4} - Component 8 seems duplicate with component 6 and should be removed. - Support component 9 with candidate value as {1,2,4} |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-6-2; Single-DCI based STx2P SFN scheme for PUSCH—codebook; 1. Support of single-DCI based STx2P SFN scheme for codebook based PUSCH 2.Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP 3. 1 PTRS port for single-DCI based STx2P SFN scheme for PUSCH—codebook 4. Support of two SRS resource sets with usage set to 'codebook' 5. Supported number of SRS resources in one SRS resource set 6. Supported maximal number of MIMO layers of each SRS resource set for CB PUSCH with SFN scheme 7. Maximum number of SRS antenna ports for each SRS resource in each SRS resource set [8. Maximum total number of layers across both panels for Single-DCI STx2P with SFN] [9. Max number of PUSCH ports associated with SRS resource set]; 2-14; Yes; n/a; Single-DCI based STx2P SFN scheme for PUSCH—codebook is not supported; Per FS; n/a; FR2 only; n/a; Component 5 candidate values: {1, 2 ,4} Component 6 candidate values: {1, 2} Component 7 candidate values: {1, 2 [,4]} Note: If value 4 is reported for component 5, UE also reports value 4 in FG 16-5c.; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | - For 40-6-2 / 40-6-2a (SFN): Some of the points discussed above for SDM are also applicable to SFN scheme as shown in the proposal/table below. 40. NR_MIMO_evo_DL_UL; 40-6-2; Single-DCI based STx2P SFN scheme for PUSCH—codebook; 1. Support of single-DCI based STx2P SFN scheme for codebook based PUSCH 2.Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP 3. 1 PTRS port for single-DCI based STx2P SFN scheme for PUSCH—codebook 4. Support of two SRS resource sets with usage set to 'codebook' 5. Supported number of SRS resources in one SRS resource set 6. Supported maximal number of MIMO layers of each SRS resource set for CB PUSCH with SFN scheme 7. Max number of PUSCH ports associated with one SRS resource set; 2-14; Yes; n/a; Single-DCI based STx2P SFN scheme for PUSCH—codebook is not supported; Per FSPC; n/a; FR2 only; n/a; Component 5 candidate values: {1,2,4} Note: If value 4 is reported for component 5, UE also reports value 4 in FG 16-5c. Component 6 candidate values: {1,2} Component 7 candidate values: {1,2,4} Note: If a row of the TPMI consists of all 0’s, the corresponding PUSCH port is not counted.; Optional with capability signaling. |
| Huawei/HiSilicon [19] | For FG 40-6-2 and 40-6-2-1, we have the following proposal: Proposal 5-4: - Regarding FG 40-6-2, support component 7, 8 and 9. - Regarding FG 40-6-2 and FG 40-6-2-1, support Per FS reporting granularity. 40. NR_MIMO_evo_DL_UL; 40-6-2; Single-DCI based STx2P SFN scheme for PUSCH—codebook; 1. Support of single-DCI based STx2P SFN scheme for codebook based PUSCH 2.Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP 3. 1 PTRS port for single-DCI based STx2P SFN scheme for PUSCH—codebook 4. Support of two SRS resource sets with usage set to 'codebook' 5. Supported number of SRS resources in one SRS resource set 6. Supported maximal number of MIMO layers of each SRS resource set for CB PUSCH with SFN scheme [7. Maximum number of SRS antenna ports for each SRS resource in each SRS resource set] [8. Maximum total number of layers across both panels for Single-DCI STx2P with SFN] [9. Max number of PUSCH ports associated with SRS resource set]; 2-14; Yes; n/a; Single-DCI based STx2P SFN scheme for PUSCH—codebook is not supported; [Per FS]; n/a; FR2 only; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-6-2-1 | 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—codebook | Support of 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—codebook | 40-6-2 | Yes | n/a | 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—codebook is not supported | [Per FS] | n/a | FR2 only | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] | - Regarding the type, we propose to follow RAN2 guidance and define FG 40-6-2-1 per band. 40-6-2-1; 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—codebook; Support of 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—codebook; 40-6-2; 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—codebook is not supported; Per Band. |
| ZTE [4] |  |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-6-2-1; 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—codebook; Support of 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—codebook; 40-6-2; Yes; n/a; 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—codebook is not supported; Per FS; No; FR2 only; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-6-2-1; 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—codebook; Support of 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—codebook; 40-6-2; Yes; n/a; 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—codebook is not supported; Per FS; n/a; FR2 only; n/a; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-6-2-1; 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—codebook; Support of 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—codebook; 40-6-2; Yes; n/a; 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—codebook is not supported; Per band; n/a; FR2 only; n/a; Optional with capability signaling. |
| Huawei/HiSilicon [19] | For FG 40-6-2 and 40-6-2-1, we have the following proposal: Proposal 5-4: - Regarding FG 40-6-2, support component 7, 8 and 9. - Regarding FG 40-6-2 and FG 40-6-2-1, support Per FS reporting granularity. 40. NR_MIMO_evo_DL_UL; 40-6-2-1; 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—codebook; Support of 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—codebook; 40-6-2; Yes; n/a; 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—codebook is not supported; [Per FS]; n/a; FR2 only; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-6-2a | Single-DCI based STx2P SFN scheme for PUSCH—noncodebook | 1. Support of single-DCI based STx2P SFN scheme for noncodebook based PUSCH 2.Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP 3. 1 PTRS port for single-DCI based STx2P SFN scheme for PUSCH—noncodebook 4. Support of two SRS resource sets with usage set to 'noncodebook' 5. Supported number of SRS resources in one SRS resource set 6. Supported maximal number of MIMO layers of each SRS resource set for NCB PUSCH with SFN scheme [7. Maximum number of simultaneous transmitted SRS resources at one symbol] [8. Maximum number of simultaneous transmitted SRS resources from one SRS resource set at one symbol] | FFS | Yes | n/a | Single-DCI based STx2P SFN scheme for PUSCH—noncodebook is not supported | [Per FS] | n/a | FR2 only | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-6-1a and 40-6-2a - There are no agreements regarding “Maximum number of simultaneous transmitted SRS resources at one symbol” and “Maximum number of simultaneous transmitted SRS resources from one SRS resource set at one symbol] in rel. 18. Thus, they should be deleted. 40. NR_MIMO_evo_DL_UL; 40-6-2a; Single-DCI based STx2P SFN scheme for PUSCH—noncodebook; 1. Support of single-DCI based STx2P SFN scheme for noncodebook based PUSCH 2.Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP 3. 1 PTRS port for single-DCI based STx2P SFN scheme for PUSCH—noncodebook 4. Support of two SRS resource sets with usage set to ‘noncodebook’ 5. Supported number of SRS resources in one SRS resource set 6. Supported maximal number of MIMO layers of each SRS resource set for NCB PUSCH with SFN scheme [7. Maximum number of simultaneous transmitted SRS resources at one symbol] [8. Maximum number of simultaneous transmitted SRS resources from one SRS resource set at one symbol]; FFS; Yes; n/a; Single-DCI based STx2P SFN scheme for PUSCH—noncodebook is not supported; [Per FS]; n/a; FR2 only; n/a; Candidate value for component 5:{1,2,4} Candidate value for component 6:{1,2}; Optional with capability signaling. |
| Ericsson [3] | This is the FG for supporting non-codebook-based STxMP SFN scheme. There are two highlighted components in FG 40-6-2a: - Maximum number of simultaneous transmitted SRS resources at one symbol. One of component 7 and 8 is needed, which one may depend on whether simultaneous transmission of two SRS resource sets is supported or not, which has not yet been decided in RAN1. Since component 8 works irrespectively of whether simultaneous transmission of two SRS resource sets is supported it can be kept. - Maximum number of simultaneous transmitted SRS resources from one SRS resource set at one symbol. One of component 7 and 8 is needed, which one may depend on whether simultaneous transmission of two SRS resource sets is supported or not, which has not yet been decided in RAN1. Since component 8 works irrespectively of whether simultaneous transmission of two SRS resource sets is supported it can be kept. The pre-requisite FG should be FG 2-15, and regarding the type, we propose to follow RAN2 guidance and define FG 40-6-2-1 per band. 40-6-2a; Single-DCI based STx2P SFN scheme for PUSCH—noncodebook; 1.Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP 2. 1 PTRS port for single-DCI based STx2P SFN scheme for PUSCH—noncodebook 3. Support of two SRS resource sets with usage set to 'noncodebook' 4. Supported number of SRS resources in one SRS resource set 5. Supported maximal number of MIMO layers of each SRS resource set for NCB PUSCH with SFN scheme 6. Maximum number of simultaneous transmitted SRS resources from one SRS resource set at one symbol; 2-15; Single-DCI based STx2P SFN scheme for PUSCH—noncodebook is not supported; Per Band. |
| ZTE [4] | - For FG 40-6-2a, - Component-7 is needed according to our elaboration in R1-2309018, where the candidate values can include {1, 2, 3, 4}. - Component-8 is needed according to our elaboration in R1-2309018, where the candidate values can include {1, 2}. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-6-2a; Single-DCI based STx2P SFN scheme for PUSCH—noncodebook; 1. Support of single-DCI based STx2P SFN scheme for noncodebook based PUSCH 2.Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP 3. 1 PTRS port for single-DCI based STx2P SFN scheme for PUSCH—noncodebook 4. Support of two SRS resource sets with usage set to 'noncodebook' 5. Supported number of SRS resources in one SRS resource set 6. Supported maximal number of MIMO layers of each SRS resource set for NCB PUSCH with SFN scheme [7. Maximum number of simultaneous transmitted SRS resources at one symbol] 8. Maximum number of simultaneous transmitted SRS resources from one SRS resource set at one symbol; 2-15, one of {40-1-1/1a, 40-1-2/2a}; Yes; n/a; Single-DCI based STx2P SFN scheme for PUSCH—noncodebook is not supported; [Per FS]; n/a; FR2 only; n/a; Optional with capability signaling. |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-6-2a: - As it is agreed to implement single-DCI based STx2P SFN scheme for non-codebook based PUSCH transmission, similar to FG 40-6-1a, FG 2-15 should be applied as the prerequisite FG. - It is not agreed whether different SRS resource set can be transmitted in one symbol. Therefore, we propose to remove component 7. As the prerequisite FG is FG 2-15, the component regarding the maximum number of simultaneous transmitted SRS resources at one symbol is included. Also, it is not necessary to change the values of this component. Therefore, we propose to remove component 8 in FG 40-6-2a. Proposal 15: Adopt the following changes for FG 40-6-2a: - The prerequisite of FG 40-6-2a is FG 2-15; - Remove component 7 and component 8 for FG 40-6-2a. |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-6-2a; Single-DCI based STx2P SFN scheme for PUSCH—noncodebook; 1. Support of single-DCI based STx2P SFN scheme for noncodebook based PUSCH 2.Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP 3. 1 PTRS port for single-DCI based STx2P SFN scheme for PUSCH—noncodebook 4. Support of two SRS resource sets with usage set to 'noncodebook' 5. Supported number of SRS resources in one SRS resource set 6. Supported maximal number of MIMO layers of each SRS resource set for NCB PUSCH with SFN scheme; 2-15; Yes; n/a; Single-DCI based STx2P SFN scheme for PUSCH—noncodebook is not supported; Per FS; No; FR2 only; Candidate value for component 5: {1,2,4} Candidate value for component 6: {1,2}; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-6-2a - Component: - Candidate value of component 5: {1,2,3,4} - Candidate value of component 6: {1,2} - Support component 8 with candidate value as {1,2,3,4} - For component 7, we have not agreed on simultaneous transmission of two SRS resource sets. It can be discussed later. - Prerequisite FG: 2-15. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-6-2a; Single-DCI based STx2P SFN scheme for PUSCH—noncodebook; 1. Support of single-DCI based STx2P SFN scheme for noncodebook based PUSCH 2.Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP 3. 1 PTRS port for single-DCI based STx2P SFN scheme for PUSCH—noncodebook 4. Support of two SRS resource sets with usage set to 'noncodebook' 5. Supported number of SRS resources in one SRS resource set 6. Supported maximal number of MIMO layers of each SRS resource set for NCB PUSCH with SFN scheme [7. Maximum number of simultaneous transmitted SRS resources at one symbol] 8. Maximum number of simultaneous transmitted SRS resources from one SRS resource set at one symbol; FFS; Yes; n/a; Single-DCI based STx2P SFN scheme for PUSCH—noncodebook is not supported; Per FS; n/a; FR2 only; n/a; Component 5 candidate values: {1, 2 ,3, 4} Component 6 candidate values: {1, 2} Component 8 candidate values: {1, 2, 3, 4}; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | - For 40-6-2 / 40-6-2a (SFN): Some of the points discussed above for SDM are also applicable to SFN scheme as shown in the proposal/table below. 40. NR_MIMO_evo_DL_UL; 40-6-2a; Single-DCI based STx2P SFN scheme for PUSCH—noncodebook; 1. Support of single-DCI based STx2P SFN scheme for noncodebook based PUSCH 2.Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP 3. 1 PTRS port for single-DCI based STx2P SFN scheme for PUSCH—noncodebook 4. Support of two SRS resource sets with usage set to 'noncodebook' 5. Supported number of SRS resources in one SRS resource set 6. Supported maximal number of MIMO layers of each SRS resource set for NCB PUSCH with SFN scheme 7. Maximum number of simultaneous transmitted SRS resources from one SRS resource set at one symbol; 2-15; Yes; n/a; Single-DCI based STx2P SFN scheme for PUSCH—noncodebook is not supported; Per FSPC; n/a; FR2 only; n/a; Component 5 candidate values: {1,2,3,4} Component 6 candidate values: {1,2} Component 7 candidate values: {1,2,3,4}; Optional with capability signaling. |
| Huawei/HiSilicon [19] | For FG 40-6-2a and 40-6-2a-1, we have the following proposal: Proposal 5-5: - Regarding FG 40-6-2a, support component 7 and 8, prerequisite FG 2-15. - Regarding FG 40-6-2a, and FG 40-6-2a-1, support Per FS reporting granularity. 40. NR_MIMO_evo_DL_UL; 40-6-2a; Single-DCI based STx2P SFN scheme for PUSCH—noncodebook; 1. Support of single-DCI based STx2P SFN scheme for noncodebook based PUSCH 2.Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP 3. 1 PTRS port for single-DCI based STx2P SFN scheme for PUSCH—noncodebook 4. Support of two SRS resource sets with usage set to 'noncodebook' 5. Supported number of SRS resources in one SRS resource set 6. Supported maximal number of MIMO layers of each SRS resource set for NCB PUSCH with SFN scheme [7. Maximum number of simultaneous transmitted SRS resources at one symbol] [8. Maximum number of simultaneous transmitted SRS resources from one SRS resource set at one symbol]; FFS 2-15; Yes; n/a; Single-DCI based STx2P SFN scheme for PUSCH—noncodebook is not supported; [Per FS]; n/a; FR2 only; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-6-2a-1 | 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—noncodebook | Support of 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—noncodebook | 40-6-2a | Yes | n/a | 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—noncodebook is not supported | [Per FS] | n/a | FR2 only | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Ericsson [3] | - Regarding the type, we propose to follow RAN2 guidance and define FG 40-6-2a-1 per band. 40-6-2a-1; 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—noncodebook; Support of 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—noncodebook; 40-6-2a; 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—noncodebook is not supported; Per Band. |
| ZTE [4] |  |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-6-2a-1; 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—noncodebook; Support of 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—noncodebook; 40-6-2a; Yes; n/a; 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—noncodebook is not supported; Per FS; No; FR2 only; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-6-2a-1; 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—noncodebook; Support of 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—noncodebook; 40-6-2a; Yes; n/a; 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—noncodebook is not supported; Per FS; n/a; FR2 only; n/a; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-6-2a-1; 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—noncodebook; Support of 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—noncodebook; 40-6-2a; Yes; n/a; 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—noncodebook is not supported; Per band; n/a; FR2 only; n/a; Optional with capability signaling. |
| Huawei/HiSilicon [19] | For FG 40-6-2a and 40-6-2a-1, we have the following proposal: Proposal 5-5: - Regarding FG 40-6-2a, support component 7 and 8, prerequisite FG 2-15. - Regarding FG 40-6-2a, and FG 40-6-2a-1, support Per FS reporting granularity. 40. NR_MIMO_evo_DL_UL; 40-6-2a-1; 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—noncodebook; Support of 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—noncodebook; 40-6-2a; Yes; n/a; 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—noncodebook is not supported; [Per FS]; n/a; FR2 only; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-6-2b | Association between CSI-RS and SRS for noncodebook single-DCI based STx2P SFN scheme for PUSCH | 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook based STxMP SFN scheme for PUSCH 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously | 2-15a | Yes | n/a | Associated CSI-RS resources for non-codebook single-DCI based STxMP SFN scheme for PUSCH is not supported | [Per Band] | n/a | FR2 only | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-6-2b; Association between CSI-RS and SRS for noncodebook single-DCI based STx2P SFN scheme for PUSCH; 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook based STxMP SFN scheme for PUSCH 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously; 2-15a; Yes; n/a; Associated CSI-RS resources for non-codebook single-DCI based STxMP SFN scheme for PUSCH is not supported; [Per Band]; n/a; FR2 only; n/a; Candidate value for component 2:{1 to 8} Candidate value for component 3:{1 to 8} Candidate value for component 4:{0 to 8} Candidate value for component 5:{ 1 to 16} Candidate value for component 6: {1,2}; Optional with capability signalling. |
| Ericsson [3] | The pre-requisite for this FG should be FG 2-15a. The type should be per band. 40-6-2b; Association between CSI-RS and SRS for noncodebook single-DCI based STx2P SFN scheme for PUSCH; 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook based STxMP SFN scheme for PUSCH 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously; 2-15a; Associated CSI-RS resources for non-codebook single-DCI based STxMP SFN scheme for PUSCH is not supported; Per Band. |
| ZTE [4] |  |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-6-2b; Association between CSI-RS and SRS for noncodebook single-DCI based STx2P SFN scheme for PUSCH; 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook based STxMP SFN scheme for PUSCH 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously; 2-15a; Yes; n/a; Associated CSI-RS resources for non-codebook single-DCI based STxMP SFN scheme for PUSCH is not supported; Per Band; No; FR2 only; n/a; Optional with capability signalling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-6-2b - Component: - Candidate value of component 2 can be {1 to 8}. - Candidate value of component 3 can be {1 to 8}. - Candidate value of component 4 can be {0 to 8}. - Candidate value of component 5 can be {1 to 16}. - Candidate value of component 6 can be {1, 2}. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-6-2b; Association between CSI-RS and SRS for noncodebook single-DCI based STx2P SFN scheme for PUSCH; 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook based STxMP SFN scheme for PUSCH 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously; 2-15a; Yes; n/a; Associated CSI-RS resources for non-codebook single-DCI based STxMP SFN scheme for PUSCH is not supported; Per FS; n/a; FR2 only; n/a; Component 2 candidate values: {1 to 8} Component 3 candidate values: {1 to 8} Component 4 candidate values: {0 to 8} Component 5 candidate values: {1 to 16} Component 6 candidate values: {1 to 2}; Optional with capability signalling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-6-2b; Association between CSI-RS and SRS for noncodebook single-DCI based STx2P SFN scheme for PUSCH; 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook based STxMP SFN scheme for PUSCH 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously; 2-15a; Yes; n/a; Associated CSI-RS resources for non-codebook single-DCI based STxMP SFN scheme for PUSCH is not supported; Per Band; n/a; FR2 only; n/a; Component 2: {1 to 8} Component 3: {1 to 8} Component 4: {0 to 8} Component 5: {1 to 16} Component 6: {1,2}; Optional with capability signalling. |
| Huawei/HiSilicon [19] | For FG 40-6-2b, similar to the CSI-RS and SRS association for sTRP-based NCB transmission in 2-15a, the reporting granularity should be Per band. Proposal 5-6: For FG 40-6-2b, support Per band reporting granularity. 40. NR_MIMO_evo_DL_UL; 40-6-2b; Association between CSI-RS and SRS for noncodebook single-DCI based STx2P SFN scheme for PUSCH; 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook based STxMP SFN scheme for PUSCH 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously; 2-15a; Yes; n/a; Associated CSI-RS resources for non-codebook single-DCI based STxMP SFN scheme for PUSCH is not supported; [Per Band]; n/a; FR2 only; n/a; Optional with capability signalling. |

| 40. NR_MIMO_evo_DL_UL | 40-6-3a | codebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG | 1. Support of multi-DCI based STx2P PUSCH+PUSCH for codebook-based PUSCH with fully[/partial] overlapping PUSCHs in time and [non-overlapping in frequency] 2. Support of two SRS resource sets with usage set to 'codebook' associated with two coresetPoolIndex values 3. Maximum number of SRS resources in one SRS resource set 4. Maximum number of layers of each PUSCH of PUSCH+PUSCH overlapping in time domain [5. Supported number of NZP PUSCH ports for each PUSCH of PUSCH+PUSCH overlapping in time domain] 6. Maximum number of PUSCHs per CORESETPoolIndex per slot 7. Maximum total number of layers across two overlapping PUSCH 8. Maximum number of SRS antenna ports for each SRS resource in each SRS resource set | 2-14 | Yes | N/A | Codebook multi-DCI based STx2P PUSCH+PUSCH is not supported | [Per Band] | n/a | FR2 only | n/a | Note: Processing capability 2 is not supported in any CC if at least one CC is configured with two values of CORESETPoolIndex. Component 3 candidate values: {1,2,4} Note: If value 4 is reported for component 3, UE also reports value 4 in FG 16-5c. Component 4 candidate values: {1,2} [Component 5 candidate values: {1,2,4} Note: If a row of the TPMI consists of all 0’s, the corresponding PUSCH port is not counted.] Component 6 candidate values: {1,2,3,4,7} Note: per SCS, similar with Rel-15 Component 7 candidate values: {2 ,3, 4} Component 8 candidate values: {1, 2 ,4} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-6-3a and FG 40-6-3b - Remove the square brackets to confirm the ”partial” and ”non overlapping in frequency” based on the following agreement in RAN1#100bis-e Agreement (RAN1#110bis-e) The multi-DCI based STxMP PUSCH+PUSCH transmission supports fully/partially/non-overlapping in frequency domain and fully/partially overlapping in time domain. - FFS whether/how to handle the PUSCH power adjustment when two PUSCHs are fully/partially overlapped in time domain (Depending on RAN4’s input on Pcmax requirements). - Note: No symbol-level power adjustment within a PUSCH transmission occassion in the case of fully/partially overlapping in time domain. - Confirm the component 5 by removing squre brackets 40. NR_MIMO_evo_DL_UL; 40-6-3a; codebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG; 1. Support of multi-DCI based STx2P PUSCH+PUSCH for codebook-based PUSCH with fully[/partial] overlapping PUSCHs in time and [non-overlapping in frequency] 2. Support of two SRS resource sets with usage set to 'codebook' associated with two coresetPoolIndex values 3. Maximum number of SRS resources in one SRS resource set 4. Maximum number of layers of each PUSCH of PUSCH+PUSCH overlapping in time domain [5. Supported number of NZP PUSCH ports for each PUSCH of PUSCH+PUSCH overlapping in time domain] 6. Maximum number of PUSCHs per CORESETPoolIndex per slot 7. Maximum total number of layers across two overlapping PUSCH 8. Maximum number of SRS antenna ports for each SRS resource in each SRS resource set; 2-14; Yes; N/A; Codebook multi-DCI based STx2P PUSCH+PUSCH is not supported; [Per Band]; n/a; FR2 only; n/a; Note: Processing capability 2 is not supported in any CC if at least one CC is configured with two values of CORESETPoolIndex. Component 3 candidate values: {1,2,4} Note: If value 4 is reported for component 3, UE also reports value 4 in FG 16-5c. Component 4 candidate values: {1,2} [Component 5 candidate values: {1,2,4} Note: If a row of the TPMI consists of all 0’s, the corresponding PUSCH port is not counted.] Component 6 candidate values: {1,2,3,4,7} Note: per SCS, similar with Rel-15 Component 7 candidate values: {2 ,3, 4} Component 8 candidate values: {1, 2 ,4}; Optional with capability signaling. |
| Ericsson [3] | This is the FG for supporting codebook based multi-DCI STxMP: For component 1, we propose to follow the structure of FG 16-2a. For component 5, since there is no RAN1 agreements on this, it should be removed. We propose to define this per band, according to the RAN2 recommendation [1] 40-6-3a; codebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG; 1. Support of multi-DCI based STx2P PUSCH+PUSCH for codebook-based PUSCH with fullypartial overlapping PUSCHs in time and non-overlapping in frequency 2. Support of two SRS resource sets with usage set to 'codebook' associated with two coresetPoolIndex values 3. Maximum number of SRS resources in one SRS resource set 4. Maximum number of layers of each PUSCH of PUSCH+PUSCH overlapping in time domain 5. Maximum number of PUSCHs per CORESETPoolIndex per slot 6. Maximum total number of layers across two overlapping PUSCH; 2-14; Codebook multi-DCI based STx2P PUSCH+PUSCH is not supported; Per Band. |
| ZTE [4] | - For FG 40-6-3a, - The brackets in component-1 can be removed to keep alignment with the principle of MDCI based STxMP PUSCH+PUSCH transmission. - Component-5 can be kept due to we can understand the motivation is to limit the used number of PUSCH ports among two panels for this transmission scheme as per the explanation from some UE vendors in the last meeting. Consequently, its candidate values can also be kept. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-6-3a; codebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG; 1. Support of multi-DCI based STx2P PUSCH+PUSCH for codebook-based PUSCH with fully[/partial] overlapping PUSCHs in time and [non-overlapping in frequency] 2. Support of two SRS resource sets with usage set to 'codebook' associated with two coresetPoolIndex values 3. Maximum number of SRS resources in one SRS resource set 4. Maximum number of layers of each PUSCH of PUSCH+PUSCH overlapping in time domain [5. Supported number of NZP PUSCH ports for each PUSCH of PUSCH+PUSCH overlapping in time domain] 6. Maximum number of PUSCHs per CORESETPoolIndex per slot 7. Maximum total number of layers across two overlapping PUSCH 8. Maximum number of SRS antenna ports for each SRS resource in each SRS resource set; 2-14, 16-2a, one of {40-1-7/7a, 40-1-9/9a}; Yes; N/A; Codebook multi-DCI based STx2P PUSCH+PUSCH is not supported; [Per Band]; n/a; FR2 only; n/a; Note: Processing capability 2 is not supported in any CC if at least one CC is configured with two values of CORESETPoolIndex. Component 3 candidate values: {1,2,4} Note: If value 4 is reported for component 3, UE also reports value 4 in FG 16-5c. Component 4 candidate values: {1,2} [Component 5 candidate values: {1,2,4} Note: If a row of the TPMI consists of all 0’s, the corresponding PUSCH port is not counted.] Component 6 candidate values: {1,2,3,4,7} Note: per SCS, similar with Rel-15 Component 7 candidate values: {2 ,3, 4} Component 8 candidate values: {1, 2 ,4}; Optional with capability signaling. |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-6-3a and 40-6-3b: - In Rel-16, support of fully/partially overlapping PDSCHs in time and non-overlapping in frequency is basic UE feature of multi-DCI based multi-TRP feature, and support of fully overlapping and partially overlapping in frequency and time are optional features of multi-DCI based multi-TRP feature. Different from PDSCH reception, transmitting PUSCHs in an overlapping manner or in a non-overlapping require similar UE capabilities. Therefore, we propose that different overlapping cases are basic features of 40-6-3a or 40-6-3b, and no other separate features are needed. - For 40-6-3a, supported number of NZP PUSCH ports can be indirectly reflected by maximum number of SRS antenna ports for each SRS resource in each SRS resource set. Therefore we propose to remove it. - The prerequisite FG of 40-6-3b is not 2-14 (which is corresponding to codebook based PUSCH transmission). The correct prerequisite FG should be 2-15 (corresponding to non-codebook based PUSCH transmission). - Similar to S-DCI based STx2P transmission, component 5 of FG 40-6-3b can be removed. - Similar to TDM based M-TRP PUSCH repetition (FG 23-3-1and FG 23-3-1-2), UE feature granularities of FG 40-6-3a and FG 40-6-3b can be per FS. |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-6-3a; codebook multi-DCI based STx2P PUSCH+PUSCH; FFS: separate rows for - DG-PUSCH+CG-PUSCH - CG-PUSCH+CG-PUSCH - DG-PUSCH+DG-PUSCH FFS: how to signal - Partial-overlapping PUSCHs in time and non-overlapping in frequency - Overlapping PUSCHs in time and partially overlapping in frequency - fully overlapping in both frequency and time - fully overlapping in frequency, partially overlapping in time - partially or non-overlapping in frequency, partially or fully overlapping in time - support for DG-PUSCH+DG-PUSCH configured as codebook based； - suppot the PUSCHs of fully-overlapping in time and non-overlapping in frequency; - 1 PTRS port for single-DCI based STx2P SFN scheme for PUSCH—codebook - Support of two SRS resource sets with usage set to 'codebook' related to different CoreserPoolIndex; - Supported number of SRS resources in one SRS resource set related to different CoreserPoolIndex; - Supported number of layers for each PUSCH related to different CoresetPoolIndex;; 2-15; Yes; n/a; codebook multi-DCI based STx2P PUSCH+PUSCH is not supported; Per band; No; FR2 only; Candidate value for component 5: {1,2,4} Candidate value for component 6: {1,2}; Optional with capability signalling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-6-3a - Component: - Support full/partial overlapping in time and non-overlapping in frequency in 40-6-3a. - Support component 5. 40-6-3c/3d/3e/3f/3g. - Support one separate row for full/partial overlapping in time and partial overlapping in frequency, and one separate row for full/partial overlapping in time and full overlapping in frequency, i.e., keep 40-6-3d, merge 40-6-3e and 40-6-3f, remove 40-6-3c and 40-6-3g. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-6-3a; codebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG; 1. Support of multi-DCI based STx2P PUSCH+PUSCH for codebook-based PUSCH with fully overlapping PUSCHs in time and frequency 2. Support of two SRS resource sets with usage set to 'codebook' associated with two coresetPoolIndex values 3. Maximum number of SRS resources in one SRS resource set 4. Maximum number of layers of each PUSCH of PUSCH+PUSCH overlapping in time domain [5. Supported number of NZP PUSCH ports for each PUSCH of PUSCH+PUSCH overlapping in time domain] 6. Maximum number of PUSCHs per CORESETPoolIndex per slot 7. Maximum total number of layers across two overlapping PUSCH 8. Maximum number of SRS antenna ports for each SRS resource in each SRS resource set; 2-14; Yes; N/A; Codebook multi-DCI based STx2P PUSCH+PUSCH is not supported; per FS; n/a; FR2 only; n/a; Note: Processing capability 2 is not supported in any CC if at least one CC is configured with two values of CORESETPoolIndex. Component 3 candidate values: {1,2,4} Note: If value 4 is reported for component 3, UE also reports value 4 in FG 16-5c. Component 4 candidate values: {1,2} [Component 5 candidate values: {1,2,4} Note: If a row of the TPMI consists of all 0’s, the corresponding PUSCH port is not counted.] Component 6 candidate values: {1,2,3,4,7} Note: per SCS, similar with Rel-15 Component 7 candidate values: {2 ,3, 4} Component 8 candidate values: {1, 2 ,4}; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-6-3c; Multi-DCI STx2P scheme – fully overlapping in frequency, partially overlapping in time for codebook based PUSCH; - Support of Multi-DCI STx2P schedule with two PUSCHs fully overlapping in frequency and partially overlapping in time for codebook based PUSCH.; Per band; FR2 only; Optional with capability signaling. |
| Samsung [16] | If the UE can support multi-DCI based STxMP PUSCH scheme, two PUSCHs which are overlapped in time domain can be scheduled to the UE. RAN1 discussed possible overlapping cases and the following agreement were endorsed: Agreement The multi-DCI based STxMP PUSCH+PUSCH transmission supports fully/partially/non-overlapping in frequency domain and fully/partially overlapping in time domain. - FFS whether/how to handle the PUSCH power adjustment when two PUSCHs are fully/partially overlapped in time domain (Depending on RAN4’s input on Pcmax requirements). - Note: No symbol-level power adjustment within a PUSCH transmission occassion in the case of fully/partially overlapping in time domain. In our view, some of overlapping case can require further enhanced UE capability, e.g. the case with partially overlapping in time domain. If two PUSCHs are partially overlapped in time domain, it can effect that the UE determines uplink power control (total transmit power or each transmit power per panel etc.) and prepares two overlapped PUSCHs. On the other hand, we think the fully overlapping case in time domain can be kept in main FG 40-6-3a and 40-6-4b because less capability can be required than partially overlapping cases. Therefore, we suggest making separate a FG to support two partially overlapping PUSCHs in time domain. Proposal 18: Separate a FG to support two partially overlapping PUSCHs in time domain. When multi-DCI based STxMP PUSCH scheme is supported, the UE should have the capability to transmit two different PUSCHs simultaneously. In contrast with single-DCI based STxMP PUSCH that one TB is divided into two parts for each panel, two independent TBs are transmitted via two overlapped PUSCHs which are associated with each CORESETPoolIndex. If the gNB schedules two different PUSCHs which are associated with two different CORESETPoolIndex and are overlapped in time domain, the UE will prepare and process two independent TBs to transmit PUSCH+PUSCH STxMP. To prepare two TBs simultaneously, the UE should have the capability on parallel processing. Therefore, we suggest adding the support of parallel processing as basic component of FG 40-6-31 and 40-6-3b. Proposal 19: Add the support of parallel processing as basic component of FG 40-6-3a and 40-6-3b. |
| MediaTek Inc. [17] | FG 40-6-3a/ FG 40-6-3b We don’t see the need to signal the UE can support what types of overlapping in time/frequency domain, i.e., any type of overlapping/non-overlapping should be supported as baseline. There is no such capability for Rel-16 M-DCI based PDSCH+PDSCH. Proposal 25: No need to introduce separate rows for the following, i.e., any type of overlapping/non-overlapping should be supported as baseline - Partial-overlapping PUSCHs in time and non-overlapping in frequency - Overlapping PUSCHs in time and partially overlapping in frequency - Fully overlapping in both frequency and time - Fully overlapping in frequency, partially overlapping in time - Partially or non-overlapping in frequency, partially or fully overlapping in time |
| Qualcomm Incorporated [18] | - The reporting type for FGs 40-6-3a / 40-6-3b should be “per FSPC” consistent with the corresponding Rel-16 FG 16-2a for DL multi-DCI as well as Rel-15 FGs 2-14 / 2-15. - For 40-6-3a: Component 5 should be kept as UE needs to indicate its true capability with respect to max number of simultaneously transmitted PUSCH ports (Tx chains or digital ports). - Regarding the overlapping types, given the additional complexity / RF requirements in case of UL transmissions of partially-overlapping PUSCHs in time domain, we propose the following: - The basic FGs 40-6-3a / 40-6-3b include fully-overlapping PUSCHs in time and non-overlapping in frequency. - When transmissions are from different panels / RF chains, inter-modulation for “non-overlapping in frequency” may not be an issue. However, if there is a concern, we can also discuss “fully overlapping in frequency” for the baseline, but fully overlapping in time should be ensured anyway. 40. NR_MIMO_evo_DL_UL; 40-6-3a; codebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG; 1. Support of multi-DCI based STx2P PUSCH+PUSCH for codebook-based PUSCH with fully overlapping PUSCHs in time and non-overlapping in frequency 2. Support of two SRS resource sets with usage set to 'codebook' associated with two coresetPoolIndex values 3. Maximum number of SRS resources in one SRS resource set 4. Maximum number of layers of each PUSCH of PUSCH+PUSCH overlapping in time domain 5. Supported number of NZP PUSCH ports for each PUSCH of PUSCH+PUSCH overlapping in time domain 6. Maximum number of PUSCHs per CORESETPoolIndex per slot 7. Maximum total number of layers across two overlapping PUSCH 8. Maximum number of SRS antenna ports for each SRS resource in each SRS resource set; 2-14; Yes; N/A; Codebook multi-DCI based STx2P PUSCH+PUSCH is not supported; Per FSPC; n/a; FR2 only; n/a; Note: Processing capability 2 is not supported in any CC if at least one CC is configured with two values of CORESETPoolIndex. Component 3 candidate values: {1,2,4} Note: If value 4 is reported for component 3, UE also reports value 4 in FG 16-5c. Component 4 candidate values: {1,2} Component 5 candidate values: {1,2,4} Note: If a row of the TPMI consists of all 0’s, the corresponding PUSCH port is not counted. Component 6 candidate values: {1,2,3,4,7} Note: per SCS, similar with Rel-15 Component 7 candidate values: {2 ,3, 4} Component 8 candidate values: {1, 2 ,4}; Optional with capability signaling. |
| Huawei/HiSilicon [19] | Regarding FG 40-6-3a, similar to FG 16-2a, support fully/partial overlapping PUSCHs in time and non-overlapping in frequency for multi-DCI based STx2P PUSCH+PUSCH for codebook-based PUSCH and FG 40-6-3c should be removed. Proposal 5-7: - For FG 40-6-3a component 1, support fully/partial overlapping PUSCHs in time and non-overlapping in frequency for multi-DCI based STx2P PUSCH+PUSCH for codebook-based PUSCH, and remove FG 40-6-3c. - For FG 40-6-3a component 5, support to report number of NZP PUSCH ports for each PUSCH. - support Per band reporting granularity. 40. NR_MIMO_evo_DL_UL; 40-6-3a; codebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG; 1. Support of multi-DCI based STx2P PUSCH+PUSCH for codebook-based PUSCH with fully[/partial] overlapping PUSCHs in time and [non-overlapping in frequency] 2. Support of two SRS resource sets with usage set to 'codebook' associated with two coresetPoolIndex values 3. Maximum number of SRS resources in one SRS resource set 4. Maximum number of layers of each PUSCH of PUSCH+PUSCH overlapping in time domain [5. Supported number of NZP PUSCH ports for each PUSCH of PUSCH+PUSCH overlapping in time domain] 6. Maximum number of PUSCHs per CORESETPoolIndex per slot 7. Maximum total number of layers across two overlapping PUSCH - 8. Maximum number of SRS antenna ports for each SRS resource in each SRS resource set; 2-14; Yes; N/A; Codebook multi-DCI based STx2P PUSCH+PUSCH is not supported; [Per Band]; n/a; FR2 only; n/a; Note: Processing capability 2 is not supported in any CC if at least one CC is configured with two values of CORESETPoolIndex. Component 3 candidate values: {1,2,4} Note: If value 4 is reported for component 3, UE also reports value 4 in FG 16-5c. Component 4 candidate values: {1,2} [Component 5 candidate values: {1,2,4} Note: If a row of the TPMI consists of all 0’s, the corresponding PUSCH port is not counted.] Component 6 candidate values: {1,2,3,4,7} Note: per SCS, similar with Rel-15 Component 7 candidate values: {2 ,3, 4} Component 8 candidate values: {1, 2 ,4}; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-6-3c | [Codebook multi-DCI based STx2P PUSCH+PUSCH – Partial-overlapping PUSCHs in time and non-overlapping in frequency] | [Support of partial-overlapping PUSCHs in time and non-overlapping in frequency] | [40-6-3a] | [Yes] | [N/A] | [Partial-overlapping PUSCHs in time and non-overlapping in frequency for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported] | [Per Band] | [n/a] | [FR2 only] | [n/a] |  | [Optional with capability signaling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-6-3c to FG40-6-3g and FG40-6-3j to FG 40-6-3n - Remove the square brackets and confirm the FGs based on the agreement above 40. NR_MIMO_evo_DL_UL; 40-6-3c; [Codebook multi-DCI based STx2P PUSCH+PUSCH – Partial-overlapping PUSCHs in time and non-overlapping in frequency]; [Support of partial-overlapping PUSCHs in time and non-overlapping in frequency]; [40-6-3a]; [Yes]; [N/A]; [Partial-overlapping PUSCHs in time and non-overlapping in frequency for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |
| Ericsson [3] | We propose to remove this, as it is part of FG 40-6-3a |
| ZTE [4] | - For FG 40-6-3c, FG 40-6-3f and FG 40-6-3g, these should be removed due to that is duplicated with component-1 in FG40-6-3a. 40. NR_MIMO_evo_DL_UL; 40-6-3c; [Codebook multi-DCI based STx2P PUSCH+PUSCH – Partial-overlapping PUSCHs in time and non-overlapping in frequency]; [Support of partial-overlapping PUSCHs in time and non-overlapping in frequency]; [40-6-3a]; [Yes]; [N/A]; [Partial-overlapping PUSCHs in time and non-overlapping in frequency for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-6-3c; [Codebook multi-DCI based STx2P PUSCH+PUSCH – Partial-overlapping PUSCHs in time and non-overlapping in frequency]; [Support of partial-overlapping PUSCHs in time and non-overlapping in frequency]; [40-6-3a]; [Yes]; [N/A]; [Partial-overlapping PUSCHs in time and non-overlapping in frequency for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-6-3c; Codebook multi-DCI based STx2P PUSCH+PUSCH – Partial/fully-overlapping PUSCHs in time and non-overlapping in frequency; [Support of partial-overlapping PUSCHs in time and non-overlapping in frequency]; [40-6-3a]; [Yes]; [N/A]; [Partial-overlapping PUSCHs in time and non-overlapping in frequency for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |
| OPPO [8] | - For FG 40-6-3c, 40-6-3d, 40-6-3e, 40-6-3f, 40-6-3g, support separate rows of fully/partial overlapping PUSCH in time and fully/partial overlapping in frequency. |
| CATT [9] | 40-6-3c/3d/3e/3f/3g and 40-6-3j/3k/3l/3m/3n: - According to the discussion in FG 40-6-3a and FG 40-6-3b, support of fully/partially overlapping PDSCHs in time and fully/partially/non-overlapping in frequency is basic feature. Correspondingly, FG 40-6-3c/3d/3e/3f/3g and 40-6-3j/3k/3l/3m/3n can be removed. |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-6-3c; M-DCI based PUSCHs of Fully overlapping both in time and frequency; Support the M-DCI based PUSCHs of Fully overlapping both in time and partially overlapping in frequency; Yes; n/a; M-DCI based PUSCHs of Fully overlapping both in time and frequency is not supported; Per band; No; FR2 only. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-6-3d; Multi-DCI STx2P scheme – partially or non-overlapping in frequency, fully overlapping in time for codebook based PUSCH; Support of Multi-DCI STx2P schedule with two PUSCHs partially or non-overlapping in frequency and fully overlapping in time for codebook based PUSCH; Per band; FR2 only; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | - FG 40-6-3c for partial-overlapping PUSCHs in time and non-overlapping in frequency should be kept 40. NR_MIMO_evo_DL_UL; 40-6-3c; Multi-DCI based STx2P PUSCH+PUSCH – Partial-overlapping PUSCHs in time and non-overlapping in frequency; Support of partial-overlapping PUSCHs in time and non-overlapping in frequency; Yes; N/A; Partial-overlapping PUSCHs in time and non-overlapping in frequency for multi-DCI based STx2P PUSCH+PUSCH is not supported; Per Band; n/a; FR2 only; n/a; Optional with capability signaling. |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-6-3c; [Codebook multi-DCI based STx2P PUSCH+PUSCH – Partial-overlapping PUSCHs in time and non-overlapping in frequency]; [Support of partial-overlapping PUSCHs in time and non-overlapping in frequency]; [40-6-3a]; [Yes]; [N/A]; [Partial-overlapping PUSCHs in time and non-overlapping in frequency for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |

| 40. NR_MIMO_evo_DL_UL | 40-6-3d | [Codebook multi-DCI based STx2P PUSCH+PUSCH – Overlapping PUSCHs in time and partially overlapping in frequency] | [Support of overlapping PUSCHs in time and partially overlapping in frequency] | [40-6-3a] | [Yes] | [N/A] | [Overlapping PUSCHs in time and partially overlapping in frequency for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported] | [Per Band] | [n/a] | [FR2 only] | [n/a] |  | [Optional with capability signaling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-6-3c to FG40-6-3g and FG40-6-3j to FG 40-6-3n - Remove the square brackets and confirm the FGs based on the agreement above 40. NR_MIMO_evo_DL_UL; 40-6-3d; [Codebook multi-DCI based STx2P PUSCH+PUSCH – Overlapping PUSCHs in time and partially overlapping in frequency]; [Support of overlapping PUSCHs in time and partially overlapping in frequency]; [40-6-3a]; [Yes]; [N/A]; [Overlapping PUSCHs in time and partially overlapping in frequency for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |
| Ericsson [3] | - These features groups provide a higher granularity than for DL. Preferably, these feature groups should be combined into one. 40-6-3d; [Codebook multi-DCI based STx2P PUSCH+PUSCH – Overlapping PUSCHs in time and partially overlapping in frequency]; [Support of overlapping PUSCHs in time and partially overlapping in frequency]; [40-6-3a]; [Per Band]. |
| ZTE [4] | - For FG 40-6-3d, FG 40-6-3e, these can be kept to capture the cases other than component-1 in FG40-6-3a. 40. NR_MIMO_evo_DL_UL; 40-6-3d; [Codebook multi-DCI based STx2P PUSCH+PUSCH – Overlapping PUSCHs in time and partially overlapping in frequency]; [Support of overlapping PUSCHs in time and partially overlapping in frequency]; [40-6-3a]; [Yes]; [N/A]; [Overlapping PUSCHs in time and partially overlapping in frequency for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-6-3d; [Codebook multi-DCI based STx2P PUSCH+PUSCH – full/partial Overlapping PUSCHs in time and partially overlapping in frequency]; [Support of overlapping PUSCHs in time and partially overlapping in frequency]; [40-6-3a]; [Yes]; [N/A]; [Overlapping PUSCHs in time and partially overlapping in frequency for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-6-3d; [Codebook multi-DCI based STx2P PUSCH+PUSCH – Overlapping PUSCHs in time and partially overlapping in frequency]; [Support of overlapping PUSCHs in time and partially overlapping in frequency]; [40-6-3a]; [Yes]; [N/A]; [Overlapping PUSCHs in time and partially overlapping in frequency for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |
| OPPO [8] | - For FG 40-6-3c, 40-6-3d, 40-6-3e, 40-6-3f, 40-6-3g, support separate rows of fully/partial overlapping PUSCH in time and fully/partial overlapping in frequency. |
| CATT [9] | 40-6-3c/3d/3e/3f/3g and 40-6-3j/3k/3l/3m/3n: - According to the discussion in FG 40-6-3a and FG 40-6-3b, support of fully/partially overlapping PDSCHs in time and fully/partially/non-overlapping in frequency is basic feature. Correspondingly, FG 40-6-3c/3d/3e/3f/3g and 40-6-3j/3k/3l/3m/3n can be removed. |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-6-3d; M-DCI based PUSCHs of Fully overlapping in time and partially overlapping in frequency; Support the M-DCI based PUSCHs of Fully overlapping in time and partially overlapping in frequency; Yes; n/a; M-DCI based PUSCHs of Fully overlapping in time and partially overlapping in frequency is not supported; Per band; No; FR2 only. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-6-3e; Multi-DCI STx2P scheme – partially or non-overlapping in frequency, partially overlapping in time for codebook based PUSCH; - Support of Multi-DCI STx2P schedule with two PUSCHs partially or non-overlapping in frequency and partially overlapping in time for codebook based PUSCH.; Per band; FR2 only; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | - FG 40-6-3d for overlapping PUSCHs in time and partially overlapping in frequency (similar to Rel-16 FG 16-2a-1) should be kept. 40. NR_MIMO_evo_DL_UL; 40-6-3d; Multi-DCI based STx2P PUSCH+PUSCH – Overlapping PUSCHs in time and partially overlapping in frequency; Support of overlapping PUSCHs in time and partially overlapping in frequency; Yes; N/A; Overlapping PUSCHs in time and partially overlapping in frequency for multi-DCI based STx2P PUSCH+PUSCH is not supported; Per Band; n/a; FR2 only; n/a; Optional with capability signaling. |
| Huawei/HiSilicon [19] | For FG 40-6-3d, similar to FG 16-2a-1, support overlapping PUSCHs in time and partially overlapping in frequency. Similar to FG 16-2a-0 that is the pre-requisite of FG 16-2a-1, we think the fully overlapping feature 40-6-3e should be the pre-requisite for this partial overlapping feature FG 40-6-3d. 40. NR_MIMO_evo_DL_UL; 40-6-3d; [Codebook multi-DCI based STx2P PUSCH+PUSCH – Overlapping PUSCHs in time and partially overlapping in frequency]; [Support of overlapping PUSCHs in time and partially overlapping in frequency]; [40-6-3a] 40-6-3e; [Yes]; [N/A]; [Overlapping PUSCHs in time and partially overlapping in frequency for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |

| 40. NR_MIMO_evo_DL_UL | 40-6-3e | [Codebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping in both frequency and time] | [Support of fully overlapping in both frequency and time] | [40-6-3a] | [Yes] | [N/A] | [Fully overlapping in both frequency and time for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported] | [Per Band] | [n/a] | [FR2 only] | [n/a] |  | [Optional with capability signaling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-6-3c to FG40-6-3g and FG40-6-3j to FG 40-6-3n - Remove the square brackets and confirm the FGs based on the agreement above 40. NR_MIMO_evo_DL_UL; 40-6-3e; [Codebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping in both frequency and time]; [Support of fully overlapping in both frequency and time]; [40-6-3a]; [Yes]; [N/A]; [Fully overlapping in both frequency and time for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |
| Ericsson [3] | - We propose to keep this FG, it corresponds to FG 16-2a-0. We propose to define this per band, according to the RAN2 recommendation [1]. 40-6-3e; Codebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping in both frequency and time; Support of fully overlapping in both frequency and time; 40-6-3a; Per Band. |
| ZTE [4] | - For FG 40-6-3d, FG 40-6-3e, these can be kept to capture the cases other than component-1 in FG40-6-3a. 40. NR_MIMO_evo_DL_UL; 40-6-3e; [Codebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping in both frequency and time]; [Support of fully overlapping in both frequency and time]; [40-6-3a]; [Yes]; [N/A]; [Fully overlapping in both frequency and time for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-6-3e; [Codebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping in both frequency and full/partial overlapping in time]; [Support of fully overlapping in both frequency and time]; [40-6-3a]; [Yes]; [N/A]; [Fully overlapping in both frequency and time for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |
| Intel Corporation [7] |  |
| OPPO [8] | - For FG 40-6-3c, 40-6-3d, 40-6-3e, 40-6-3f, 40-6-3g, support separate rows of fully/partial overlapping PUSCH in time and fully/partial overlapping in frequency |
| CATT [9] | 40-6-3c/3d/3e/3f/3g and 40-6-3j/3k/3l/3m/3n: - According to the discussion in FG 40-6-3a and FG 40-6-3b, support of fully/partially overlapping PDSCHs in time and fully/partially/non-overlapping in frequency is basic feature. Correspondingly, FG 40-6-3c/3d/3e/3f/3g and 40-6-3j/3k/3l/3m/3n can be removed. |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-6-3e; M-DCI based PUSCHs of Partially overlapping in time and non-overlapping in frequency; Support the M-DCI based PUSCHs of Partially overlapping in time and non-overlapping in frequency; Yes; n/a; M-DCI based PUSCHs of Partially overlapping in time and non-overlapping in frequency is not supported; Per band; No; FR2 only. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] |  |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | - FG 40-6-3e for fully-overlapping PUSCHs in frequency and time (similar to Rel-16 FG 16-2a-0) should be kept. - This assumes baseline is with “fully-overlapping PUSCHs in time and non-overlapping in frequency”. 40. NR_MIMO_evo_DL_UL; 40-6-3e; Multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping in both frequency and time; Support of fully overlapping in both frequency and time; Yes; N/A; Fully overlapping in both frequency and time for multi-DCI based STx2P PUSCH+PUSCH is not supported; Per Band; n/a; FR2 only; n/a; Optional with capability signaling. |
| Huawei/HiSilicon [19] | For FG 40-6-3e, similar to FG 16-2a-0, support fully overlapping in both frequency and time. Proposal 5-8: - Support FG 40-6-3e and FG 40-6-3f with pre-requisite of FG 40-6-3a and per band reporting granularity; 40. NR_MIMO_evo_DL_UL; 40-6-3e; [Codebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping in both frequency and time]; [Support of fully overlapping in both frequency and time]; [40-6-3a]; [Yes]; [N/A]; [Fully overlapping in both frequency and time for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |

| 40. NR_MIMO_evo_DL_UL | 40-6-3f | [Codebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping in frequency, partially overlapping in time] | [Support of fully overlapping in frequency, partially overlapping in time] | [40-6-3a] | [Yes] | [N/A] | [Fully overlapping in frequency, partially overlapping in time for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported] | [Per Band] | [n/a] | [FR2 only] | [n/a] |  | [Optional with capability signaling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-6-3c to FG40-6-3g and FG40-6-3j to FG 40-6-3n - Remove the square brackets and confirm the FGs based on the agreement above 40. NR_MIMO_evo_DL_UL; 40-6-3f; [Codebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping in frequency, partially overlapping in time]; [Support of fully overlapping in frequency, partially overlapping in time]; [40-6-3a]; [Yes]; [N/A]; [Fully overlapping in frequency, partially overlapping in time for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |
| Ericsson [3] | - These features groups provide a higher granularity than for DL. Preferably, these feature groups should be combined into one. 40-6-3f; [Codebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping in frequency, partially overlapping in time]; [Support of fully overlapping in frequency, partially overlapping in time]; [40-6-3a]; Per Band. |
| ZTE [4] | - For FG 40-6-3c, FG 40-6-3f and FG 40-6-3g, these should be removed due to that is duplicated with component-1 in FG40-6-3a. 40. NR_MIMO_evo_DL_UL; 40-6-3f; [Codebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping in frequency, partially overlapping in time]; [Support of fully overlapping in frequency, partially overlapping in time]; [40-6-3a]; [Yes]; [N/A]; [Fully overlapping in frequency, partially overlapping in time for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-6-3f; [Codebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping in frequency, partially overlapping in time]; [Support of fully overlapping in frequency, partially overlapping in time]; [40-6-3a]; [Yes]; [N/A]; [Fully overlapping in frequency, partially overlapping in time for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-6-3f; [Codebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping in frequency, partially overlapping in time]; [Support of fully overlapping in frequency, partially overlapping in time]; [40-6-3a]; [Yes]; [N/A]; [Fully overlapping in frequency, partially overlapping in time for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |
| OPPO [8] | - For FG 40-6-3c, 40-6-3d, 40-6-3e, 40-6-3f, 40-6-3g, support separate rows of fully/partial overlapping PUSCH in time and fully/partial overlapping in frequency. |
| CATT [9] | 40-6-3c/3d/3e/3f/3g and 40-6-3j/3k/3l/3m/3n: - According to the discussion in FG 40-6-3a and FG 40-6-3b, support of fully/partially overlapping PDSCHs in time and fully/partially/non-overlapping in frequency is basic feature. Correspondingly, FG 40-6-3c/3d/3e/3f/3g and 40-6-3j/3k/3l/3m/3n can be removed. |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-6-3f; M-DCI based PUSCHs of Partially overlapping in time and fully overlapping in frequency; Support the M-DCI based PUSCHs of Partially overlapping in time and fully overlapping in frequency; Yes; n/a; M-DCI based PUSCHs of Partially overlapping in time and fully overlapping in frequency is not supported; Per band; No; FR2 only. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] |  |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | - FG 40-6-3f and 40-6-3g seem unnecessary and can be removed. |
| Huawei/HiSilicon [19] | For FG 40-6-3f, generally we are ok to support fully overlapping in frequency and partially overlapping in time Proposal 5-8: - Support FG 40-6-3d with pre-requisite of FG 40-6-3e and per band reporting granularity; 40. NR_MIMO_evo_DL_UL; 40-6-3f; [Codebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping in frequency, partially overlapping in time]; [Support of fully overlapping in frequency, partially overlapping in time]; [40-6-3a]; [Yes]; [N/A]; [Fully overlapping in frequency, partially overlapping in time for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |

| 40. NR_MIMO_evo_DL_UL | 40-6-3g | [Codebook multi-DCI based STx2P PUSCH+PUSCH – Partially or non-overlapping in frequency, partially or fully overlapping in time] | [Support of partially or non-overlapping in frequency, partially or fully overlapping in time] | [40-6-3a] | [Yes] | [N/A] | [Partially or non-overlapping in frequency, partially or fully overlapping in time for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported] | [Per Band] | [n/a] | [FR2 only] | [n/a[ |  | [Optional with capability signaling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-6-3c to FG40-6-3g and FG40-6-3j to FG 40-6-3n - Remove the square brackets and confirm the FGs based on the agreement above 40. NR_MIMO_evo_DL_UL; 40-6-3g; [Codebook multi-DCI based STx2P PUSCH+PUSCH – Partially or non-overlapping in frequency, partially or fully overlapping in time]; [Support of partially or non-overlapping in frequency, partially or fully overlapping in time]; [40-6-3a]; [Yes]; [N/A]; [Partially or non-overlapping in frequency, partially or fully overlapping in time for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |
| Ericsson [3] | - These features groups provide a higher granularity than for DL. Preferably, these feature groups should be combined into one. |
| ZTE [4] | - For FG 40-6-3c, FG 40-6-3f and FG 40-6-3g, these should be removed due to that is duplicated with component-1 in FG40-6-3a. 40. NR_MIMO_evo_DL_UL; 40-6-3g; [Codebook multi-DCI based STx2P PUSCH+PUSCH – Partially or non-overlapping in frequency, partially or fully overlapping in time]; [Support of partially or non-overlapping in frequency, partially or fully overlapping in time]; [40-6-3a]; [Yes]; [N/A]; [Partially or non-overlapping in frequency, partially or fully overlapping in time for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a[; [Optional with capability signaling]. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-6-3g; [Codebook multi-DCI based STx2P PUSCH+PUSCH – Partially or non-overlapping in frequency, partially or fully overlapping in time]; [Support of partially or non-overlapping in frequency, partially or fully overlapping in time]; [40-6-3a]; [Yes]; [N/A]; [Partially or non-overlapping in frequency, partially or fully overlapping in time for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a[; [Optional with capability signaling]. |
| Intel Corporation [7] |  |
| OPPO [8] | - For FG 40-6-3c, 40-6-3d, 40-6-3e, 40-6-3f, 40-6-3g, support separate rows of fully/partial overlapping PUSCH in time and fully/partial overlapping in frequency. |
| CATT [9] | 40-6-3c/3d/3e/3f/3g and 40-6-3j/3k/3l/3m/3n: - According to the discussion in FG 40-6-3a and FG 40-6-3b, support of fully/partially overlapping PDSCHs in time and fully/partially/non-overlapping in frequency is basic feature. Correspondingly, FG 40-6-3c/3d/3e/3f/3g and 40-6-3j/3k/3l/3m/3n can be removed. |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-6-4g; M-DCI based CG PUSCH+CG PUSCH STxMP; Support of the M-DCI based CG PUSCH+CG PUSCH; Yes; n/a; M-DCI based CG PUSCH+CG PUSCH STxMP is not supported; Per band; No; FR2 only. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] |  |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] |  |
| Huawei/HiSilicon [19] | For FG 40-6-3g, it is not required as non-overlapping in frequency and partially or fully overlapping in time is included in FG 40-6-3a, and partially overlapping in frequency and fully overlapping in time is included in FG 40-6-3d. Proposal 5-8: - Do not support FG 40-6-3g since this FG has already been covered in FG 40-6-3a and FG 40-6-3d; 40. NR_MIMO_evo_DL_UL; 40-6-3g; [Codebook multi-DCI based STx2P PUSCH+PUSCH – Partially or non-overlapping in frequency, partially or fully overlapping in time]; [Support of partially or non-overlapping in frequency, partially or fully overlapping in time]; [40-6-3a]; [Yes]; [N/A]; [Partially or non-overlapping in frequency, partially or fully overlapping in time for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a[; [Optional with capability signaling]. |

| 40. NR_MIMO_evo_DL_UL | 40-6-3h | Codebook multi-DCI based STx2P PUSCH+PUSCH for CG+CG | Support of multi-DCI based STxMP CG-PUSCH+CG-PUSCH [FFS: other components] | [40-6-3a] | Yes | N/A | Multi-DCI based STx2P for CG+CG is not supported for codebook | [Per Band] | n/a | FR2 only | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-6-3h; Codebook multi-DCI based STx2P PUSCH+PUSCH for CG+CG; Support of multi-DCI based STxMP CG-PUSCH+CG-PUSCH [FFS: other components]; [40-6-3a]; Yes; N/A; Multi-DCI based STx2P for CG+CG is not supported for codebook; [Per Band]; n/a; FR2 only; n/a; Optional with capability signaling. |
| Ericsson [3] | - We propose to keep these. We propose to define this per band, according to the RAN2 recommendation [1]. 40-6-3h; Codebook multi-DCI based STx2P PUSCH+PUSCH for CG+CG; Support of multi-DCI based STxMP CG-PUSCH+CG-PUSCH; 40-6-3a; Per Band. |
| ZTE [4] |  |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-6-3h/3i and 40-6-3o/3p: - The prerequisite of 40-6-3h/3i is 40-3-3a. - The prerequisite of 40-6-3o/3p is 40-3-3b. - Similar to CG PUSCH transmission towards M-TRPs (FG 23-3-1g), UE feature granularities of 40-6-3h/3i and 40-6-3o/3p can be per band. Proposal 18: Adopt the following for FG 40-6-3h/3i and 40-6-3o/3p: - The prerequisite of 40-6-3h/3i is 40-3-3a. - The prerequisite of 40-6-3o/3p is 40-3-3b. - UE feature granularities of FG 40-6-3h/3i and 40-6-3o/3p are per band. |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-6-3h; M-DCI based DG PUSCH+CG PUSCH STxMP; Support of the M-DCI based DG PUSCH+CG PUSCH; Yes; n/a; M-DCI based DG PUSCH+CG PUSCH STxMP is not supported; Per band; No; FR2 only. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-6-3j/3k/3l/3h/3n - Support one separate row for full/partial overlapping in time and partial overlapping in frequency, and one separate row for full/partial overlapping in time and full overlapping in frequency, i.e., keep 40-6-3k, merge 40-6-l and 40-6-3m, remove 40-6-3j and 40-6-3n. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-6-3h; Codebook multi-DCI based STx2P PUSCH+PUSCH for CG+CG; Support of multi-DCI based STxMP CG-PUSCH+CG-PUSCH [FFS: other components]; [40-6-3a]; Yes; N/A; Multi-DCI based STx2P for CG+CG is not supported for codebook; Per band; n/a; FR2 only; n/a; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-6-3h; Multi-DCI based STx2P PUSCH+PUSCH for CG+CG; Support of multi-DCI based STxMP CG-PUSCH+CG-PUSCH; Yes; N/A; Multi-DCI based STx2P for CG+CG is not supported; Per Band; n/a; FR2 only; n/a; Optional with capability signaling. |
| Huawei/HiSilicon [19] | For FG 40-6-3h and FG 40-6-3i, no other components seem to be necessary as FG 40-6-3a (and all its components) should be supported as a pre-requisite of FG 40-6-3h and FG 40-6-3i. Proposal 5-9: - For FG 40-6-3h and FG 40-6-3i, no other component is necessary. Prerequisite is FG 40-6-3a. 40. NR_MIMO_evo_DL_UL; 40-6-3h; Codebook multi-DCI based STx2P PUSCH+PUSCH for CG+CG; Support of multi-DCI based STxMP CG-PUSCH+CG-PUSCH [FFS: other components]; [40-6-3a]; Yes; N/A; Multi-DCI based STx2P for CG+CG is not supported for codebook; [Per Band]; n/a; FR2 only; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-6-3i | Codebook multi-DCI based STx2P PUSCH+PUSCH for DG+CG | Support of multi-DCI based STxMP DG-PUSCH+CG-PUSCH [FFS: other components] | [40-6-3a] | Yes | N/A | Multi-DCI based STx2P for DG+CG is not supported for codebook | [Per Band] | n/a | FR2 only | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-6-3i; Codebook multi-DCI based STx2P PUSCH+PUSCH for DG+CG; Support of multi-DCI based STxMP DG-PUSCH+CG-PUSCH [FFS: other components]; [40-6-3a]; Yes; N/A; Multi-DCI based STx2P for DG+CG is not supported for codebook; [Per Band]; n/a; FR2 only; n/a; Optional with capability signaling. |
| Ericsson [3] | - We propose to keep these. We propose to define this per band, according to the RAN2 recommendation [1]. 40-6-3i; Codebook multi-DCI based STx2P PUSCH+PUSCH for DG+CG; Support of multi-DCI based STxMP DG-PUSCH+CG-PUSCH; 40-6-3a; Per Band. |
| ZTE [4] |  |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-6-3h/3i and 40-6-3o/3p: - The prerequisite of 40-6-3h/3i is 40-3-3a. - The prerequisite of 40-6-3o/3p is 40-3-3b. - Similar to CG PUSCH transmission towards M-TRPs (FG 23-3-1g), UE feature granularities of 40-6-3h/3i and 40-6-3o/3p can be per band. Proposal 18: Adopt the following for FG 40-6-3h/3i and 40-6-3o/3p: - The prerequisite of 40-6-3h/3i is 40-3-3a. - The prerequisite of 40-6-3o/3p is 40-3-3b. - UE feature granularities of FG 40-6-3h/3i and 40-6-3o/3p are per band. |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-6-3i; Codebook multi-DCI based STx2P PUSCH+PUSCH for DG+CG; Support of multi-DCI based STxMP DG-PUSCH+CG-PUSCH [FFS: other components]; [40-6-3a]; Yes; N/A; Multi-DCI based STx2P for DG+CG is not supported for codebook; Per band; n/a; FR2 only; n/a; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-6-3i; Multi-DCI based STx2P PUSCH+PUSCH for DG+CG; Support of multi-DCI based STxMP DG-PUSCH+CG-PUSCH; Yes; N/A; Multi-DCI based STx2P for DG+CG is not supported; Per Band; n/a; FR2 only; n/a; Optional with capability signaling. |
| Huawei/HiSilicon [19] | For FG 40-6-3h and FG 40-6-3i, no other components seem to be necessary as FG 40-6-3a (and all its components) should be supported as a pre-requisite of FG 40-6-3h and FG 40-6-3i. Proposal 5-9: - For FG 40-6-3h and FG 40-6-3i, no other component is necessary. Prerequisite is FG 40-6-3a. 40. NR_MIMO_evo_DL_UL; 40-6-3i; Codebook multi-DCI based STx2P PUSCH+PUSCH for DG+CG; Support of multi-DCI based STxMP DG-PUSCH+CG-PUSCH [FFS: other components]; [40-6-3a]; Yes; N/A; Multi-DCI based STx2P for DG+CG is not supported for codebook; [Per Band]; n/a; FR2 only; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-6-3b | Noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG | 1. Support of multi-DCI based STxMP PUSCH+PUSCH for noncodebook-based PUSCH with fully[/partial] overlapping PUSCHs in time and [non-overlapping in frequency] 2. Support of two SRS resource sets with usage set to 'noncodebook' associated with two coresetPoolInde values 3. Maximum number of SRS resources in one SRS resource set 4. Maximum number of layers of each PUSCH of PUSCH+PUSCH overlapping in time domain [5. Maximum number of simultaneously transmitted SRS resources in one symbol per SRS resource set] 6. Maximum number of PUSCHs per CORESETPoolIndex per slot 7. Maximum total number of layers across two overlapping PUSCH | 2-14 | Yes | N/A | Noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported | [Per Band] | n/a | FR2 only | n/a | Note: Processing capability 2 is not supported in any CC if at least one CC is configured with two values of CORESETPoolIndex. Component 3 candidate values: {1,2,[3,]4} Component 4 candidate values: {1,2} [Component 5 candidate values: {1,2,[3,4]}] Component 6 candidate values: {1,2,3,4,7} Note: per SCS, similar with Rel-15 Component 7 candidate values: {2 ,3, 4} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-6-3a and FG 40-6-3b - Remove the square brackets to confirm the ”partial” and ”non overlapping in frequency” based on the following agreement in RAN1#100bis-e Agreement (RAN1#110bis-e) The multi-DCI based STxMP PUSCH+PUSCH transmission supports fully/partially/non-overlapping in frequency domain and fully/partially overlapping in time domain. - FFS whether/how to handle the PUSCH power adjustment when two PUSCHs are fully/partially overlapped in time domain (Depending on RAN4’s input on Pcmax requirements). - Note: No symbol-level power adjustment within a PUSCH transmission occassion in the case of fully/partially overlapping in time domain. - Confirm the component 5 by removing squre brackets 40. NR_MIMO_evo_DL_UL; 40-6-3b; Noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG; 1. Support of multi-DCI based STxMP PUSCH+PUSCH for noncodebook-based PUSCH with fully[/partial] overlapping PUSCHs in time and [non-overlapping in frequency] 2. Support of two SRS resource sets with usage set to 'noncodebook' associated with two coresetPoolInde values 3. Maximum number of SRS resources in one SRS resource set 4. Maximum number of layers of each PUSCH of PUSCH+PUSCH overlapping in time domain [5. Maximum number of simultaneously transmitted SRS resources in one symbol per SRS resource set] 6. Maximum number of PUSCHs per CORESETPoolIndex per slot 7. Maximum total number of layers across two overlapping PUSCH; [40-6-3a]; Yes; N/A; Noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported; [Per Band]; n/a; FR2 only; n/a; Note: Processing capability 2 is not supported in any CC if at least one CC is configured with two values of CORESETPoolIndex. Component 3 candidate values: {1,2,[3,]4} Component 4 candidate values: {1,2} [Component 5 candidate values: {1,2,[3,4]}] Component 6 candidate values: {1,2,3,4,7} Note: per SCS, similar with Rel-15 Component 7 candidate values: {2 ,3, 4}; Optional with capability signaling. |
| Ericsson [3] | This is the main FG for supporting non-codebook-based multi-DCI STxMP: For component 1, we propose to follow the structure of FG 16-2a. For component 5, since there is no RAN1 agreements on this, it should be removed. The pre-requisite FG should be 2-15 – non-codebook-based transmission We propose to define this per band, according to the RAN2 recommendation [1] 40-6-3b; Noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG; 1. Support of multi-DCI based STxMP PUSCH+PUSCH for noncodebook-based PUSCH with fully/partial overlapping PUSCHs in time and non-overlapping in frequency 2. Support of two SRS resource sets with usage set to 'noncodebook' associated with two coresetPoolInde values 3. Maximum number of SRS resources in one SRS resource set 4. Maximum number of layers of each PUSCH of PUSCH+PUSCH overlapping in time domain 5. Maximum number of PUSCHs per CORESETPoolIndex per slot 6. Maximum total number of layers across two overlapping PUSCH; 2-15; Noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported; Per Band. |
| ZTE [4] | - For FG 40-6-3b, - Its prerequisite FG should be FG 2-15 instead of FG 2-14. - The brackets in component-1 can be removed to keep alignment with the principle of MDCI based STxMP PUSCH+PUSCH transmission. - Candidate value 3 of component-3 should be kept due to it was supported from Rel-15 (cf. FG 2-15). - Candidate values 3 and 4 of component-5 should be kept due to it is feasible to transmit up to 4 SRS resources from each SRS set simultaneous for sounding the UL channel condition. 40. NR_MIMO_evo_DL_UL; 40-6-3b; Noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG; 1. Support of multi-DCI based STxMP PUSCH+PUSCH for noncodebook-based PUSCH with fully[/partial] overlapping PUSCHs in time and [non-overlapping in frequency] 2. Support of two SRS resource sets with usage set to 'noncodebook' associated with two coresetPoolInde values 3. Maximum number of SRS resources in one SRS resource set 4. Maximum number of layers of each PUSCH of PUSCH+PUSCH overlapping in time domain [5. Maximum number of simultaneously transmitted SRS resources in one symbol per SRS resource set] 6. Maximum number of PUSCHs per CORESETPoolIndex per slot 7. Maximum total number of layers across two overlapping PUSCH; 2-14 2-15; Yes; N/A; Noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported; [Per Band]; n/a; FR2 only; n/a; Note: Processing capability 2 is not supported in any CC if at least one CC is configured with two values of CORESETPoolIndex. Component 3 candidate values: {1,2,[3,]4} Component 4 candidate values: {1,2} [Component 5 candidate values: {1,2,[3,4]}] Component 6 candidate values: {1,2,3,4,7} Note: per SCS, similar with Rel-15 Component 7 candidate values: {2 ,3, 4}; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-6-3b; Noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG; 1. Support of multi-DCI based STxMP PUSCH+PUSCH for noncodebook-based PUSCH with fully[/partial] overlapping PUSCHs in time and [non-overlapping in frequency] 2. Support of two SRS resource sets with usage set to 'noncodebook' associated with two coresetPoolInde values 3. Maximum number of SRS resources in one SRS resource set 4. Maximum number of layers of each PUSCH of PUSCH+PUSCH overlapping in time domain [5. Maximum number of simultaneously transmitted SRS resources in one symbol per SRS resource set] 6. Maximum number of PUSCHs per CORESETPoolIndex per slot 7. Maximum total number of layers across two overlapping PUSCH; 2-154, 16-2a, one of {40-1-7/7a, 40-1-9/9a}; Yes; N/A; Noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported; [Per Band]; n/a; FR2 only; n/a; Note: Processing capability 2 is not supported in any CC if at least one CC is configured with two values of CORESETPoolIndex. Component 3 candidate values: {1,2,[3,]4} Component 4 candidate values: {1,2} [Component 5 candidate values: {1,2,[3,4]}] Component 6 candidate values: {1,2,3,4,7} Note: per SCS, similar with Rel-15 Component 7 candidate values: {2 ,3, 4}; Optional with capability signaling. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-6-3b; Noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG; 1. Support of multi-DCI based STxMP PUSCH+PUSCH for noncodebook-based PUSCH with fully/partial overlapping PUSCHs in time and non-overlapping in frequency 2. Support of two SRS resource sets with usage set to 'noncodebook' associated with two coresetPoolInde values 3. Maximum number of SRS resources in one SRS resource set 4. Maximum number of layers of each PUSCH of PUSCH+PUSCH overlapping in time domain [5. Maximum number of simultaneously transmitted SRS resources in one symbol per SRS resource set] 6. Maximum number of PUSCHs per CORESETPoolIndex per slot 7. Maximum total number of layers across two overlapping PUSCH; 2-14; Yes; N/A; Noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported; [Per Band]; n/a; FR2 only; n/a; Note: Processing capability 2 is not supported in any CC if at least one CC is configured with two values of CORESETPoolIndex. Component 3 candidate values: {1,2,[3,]4} Component 4 candidate values: {1,2} [Component 5 candidate values: {1,2,[3,4]}] Component 6 candidate values: {1,2,3,4,7} Note: per SCS, similar with Rel-15 Component 7 candidate values: {2 ,3, 4}; Optional with capability signaling. |
| OPPO [8] |  |
| CATT [9] | 40-6-3a and 40-6-3b: - In Rel-16, support of fully/partially overlapping PDSCHs in time and non-overlapping in frequency is basic UE feature of multi-DCI based multi-TRP feature, and support of fully overlapping and partially overlapping in frequency and time are optional features of multi-DCI based multi-TRP feature. Different from PDSCH reception, transmitting PUSCHs in an overlapping manner or in a non-overlapping require similar UE capabilities. Therefore, we propose that different overlapping cases are basic features of 40-6-3a or 40-6-3b, and no other separate features are needed. - For 40-6-3a, supported number of NZP PUSCH ports can be indirectly reflected by maximum number of SRS antenna ports for each SRS resource in each SRS resource set. Therefore we propose to remove it. - The prerequisite FG of 40-6-3b is not 2-14 (which is corresponding to codebook based PUSCH transmission). The correct prerequisite FG should be 2-15 (corresponding to non-codebook based PUSCH transmission). - Similar to S-DCI based STx2P transmission, component 5 of FG 40-6-3b can be removed. - Similar to TDM based M-TRP PUSCH repetition (FG 23-3-1and FG 23-3-1-2), UE feature granularities of FG 40-6-3a and FG 40-6-3b can be per FS. |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-6-3b; noncodebook multi-DCI based STx2P PUSCH+PUSCH; FFS: separate rows for - DG-PUSCH+CG-PUSCH - CG-PUSCH+CG-PUSCH - DG-PUSCH+DG-PUSCH FFS: how to signal - Partial-overlapping PUSCHs in time and non-overlapping in frequency - Overlapping PUSCHs in time and partially overlapping in frequency - fully overlapping in both frequency and time - fully overlapping in frequency, partially overlapping in time - partially or non-overlapping in frequency, partially or fully overlapping in time - support for DG-PUSCH+DG-PUSCH configured as non-codebook based； - suppot the PUSCHs of fully-overlapping in time and non-overlapping in frequency; - 1 PTRS port for single-DCI based STx2P SFN scheme for PUSCH—codebook - Support of two SRS resource sets with usage set to 'codebook' related to different CoreserPoolIndex; - Supported number of SRS resources in one SRS resource set related to different CoreserPoolIndex; - Supported number of layers for each PUSCH related to different CoresetPoolIndex;; 2-15a; Yes; n/a; noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported.; Per band; No; FR2 only; Candidate value for component 5: {1,2,4} Candidate value for component 6: {1,2}; Optional with capability signalling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-6-3b - Component: - Support full/partial overlapping in time and non-overlapping in frequency in 40-6-3b. - Support component 5. - Prerequisite FG: 2-14 should be replaced by 2-15. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-6-3b; Noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG; 1. Support of multi-DCI based STxMP PUSCH+PUSCH for noncodebook-based PUSCH with fully overlapping PUSCHs in time and frequency 2. Support of two SRS resource sets with usage set to 'noncodebook' associated with two coresetPoolIndex values 3. Maximum number of SRS resources in one SRS resource set 4. Maximum number of layers of each PUSCH of PUSCH+PUSCH overlapping in time domain 5. Maximum number of simultaneously transmitted SRS resources in one symbol per SRS resource set 6. Maximum number of PUSCHs per CORESETPoolIndex per slot 7. Maximum total number of layers across two overlapping PUSCH; 2-14; Yes; N/A; Noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported; Per FS; n/a; FR2 only; n/a; Note: Processing capability 2 is not supported in any CC if at least one CC is configured with two values of CORESETPoolIndex. Component 3 candidate values: {1,2,3,4} Component 4 candidate values: {1,2} Component 5 candidate values: {1,2,3,4} Component 6 candidate values: {1,2,3,4,7} Note: per SCS, similar with Rel-15 Component 7 candidate values: {2 ,3, 4}; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-6-3j; Multi-DCI STx2P scheme – fully overlapping in frequency, partially overlapping in time for noncodebook based PUSCH; - Support of Multi-DCI STx2P schedule with two PUSCHs fully overlapping in frequency and partially overlapping in time for noncodebook based PUSCH.; Per band; FR2 only; Optional with capability signaling. |
| Samsung [16] | If the UE can support multi-DCI based STxMP PUSCH scheme, two PUSCHs which are overlapped in time domain can be scheduled to the UE. RAN1 discussed possible overlapping cases and the following agreement were endorsed: Agreement The multi-DCI based STxMP PUSCH+PUSCH transmission supports fully/partially/non-overlapping in frequency domain and fully/partially overlapping in time domain. - FFS whether/how to handle the PUSCH power adjustment when two PUSCHs are fully/partially overlapped in time domain (Depending on RAN4’s input on Pcmax requirements). - Note: No symbol-level power adjustment within a PUSCH transmission occassion in the case of fully/partially overlapping in time domain. In our view, some of overlapping case can require further enhanced UE capability, e.g. the case with partially overlapping in time domain. If two PUSCHs are partially overlapped in time domain, it can effect that the UE determines uplink power control (total transmit power or each transmit power per panel etc.) and prepares two overlapped PUSCHs. On the other hand, we think the fully overlapping case in time domain can be kept in main FG 40-6-3a and 40-6-4b because less capability can be required than partially overlapping cases. Therefore, we suggest making separate a FG to support two partially overlapping PUSCHs in time domain. Proposal 18: Separate a FG to support two partially overlapping PUSCHs in time domain. When multi-DCI based STxMP PUSCH scheme is supported, the UE should have the capability to transmit two different PUSCHs simultaneously. In contrast with single-DCI based STxMP PUSCH that one TB is divided into two parts for each panel, two independent TBs are transmitted via two overlapped PUSCHs which are associated with each CORESETPoolIndex. If the gNB schedules two different PUSCHs which are associated with two different CORESETPoolIndex and are overlapped in time domain, the UE will prepare and process two independent TBs to transmit PUSCH+PUSCH STxMP. To prepare two TBs simultaneously, the UE should have the capability on parallel processing. Therefore, we suggest adding the support of parallel processing as basic component of FG 40-6-31 and 40-6-3b. Proposal 19: Add the support of parallel processing as basic component of FG 40-6-3a and 40-6-3b. |
| MediaTek Inc. [17] | FG 40-6-3a/ FG 40-6-3b We don’t see the need to signal the UE can support what types of overlapping in time/frequency domain, i.e., any type of overlapping/non-overlapping should be supported as baseline. There is no such capability for Rel-16 M-DCI based PDSCH+PDSCH. Proposal 25: No need to introduce separate rows for the following, i.e., any type of overlapping/non-overlapping should be supported as baseline - Partial-overlapping PUSCHs in time and non-overlapping in frequency - Overlapping PUSCHs in time and partially overlapping in frequency - Fully overlapping in both frequency and time - Fully overlapping in frequency, partially overlapping in time - Partially or non-overlapping in frequency, partially or fully overlapping in time |
| Qualcomm Incorporated [18] | - The reporting type for FGs 40-6-3a / 40-6-3b should be “per FSPC” consistent with the corresponding Rel-16 FG 16-2a for DL multi-DCI as well as Rel-15 FGs 2-14 / 2-15. - For 40-6-3b: Component 5 should be kept as this is similar to the legacy (FG 2-15, component 3). Furthermore, same as legacy, the candidate values for both components 3 and 5 should be {1,2,3,4}. - Regarding the overlapping types, given the additional complexity / RF requirements in case of UL transmissions of partially-overlapping PUSCHs in time domain, we propose the following: - The basic FGs 40-6-3a / 40-6-3b include fully-overlapping PUSCHs in time and non-overlapping in frequency. - When transmissions are from different panels / RF chains, inter-modulation for “non-overlapping in frequency” may not be an issue. However, if there is a concern, we can also discuss “fully overlapping in frequency” for the baseline, but fully overlapping in time should be ensured anyway. 40. NR_MIMO_evo_DL_UL; 40-6-3b; Noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG; 1. Support of multi-DCI based STxMP PUSCH+PUSCH for noncodebook-based PUSCH with fully overlapping PUSCHs in time and non-overlapping in frequency 2. Support of two SRS resource sets with usage set to 'noncodebook' associated with two coresetPoolInde values 3. Maximum number of SRS resources in one SRS resource set 4. Maximum number of layers of each PUSCH of PUSCH+PUSCH overlapping in time domain 5. Maximum number of simultaneously transmitted SRS resources in one symbol per SRS resource set 6. Maximum number of PUSCHs per CORESETPoolIndex per slot 7. Maximum total number of layers across two overlapping PUSCH; 2-14; Yes; N/A; Noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported; Per FSPC; n/a; FR2 only; n/a; Note: Processing capability 2 is not supported in any CC if at least one CC is configured with two values of CORESETPoolIndex. Component 3 candidate values: {1,2,3,4} Component 4 candidate values: {1,2} Component 5 candidate values: {1,2, 3,4} Component 6 candidate values: {1,2,3,4,7} Note: per SCS, similar with Rel-15 Component 7 candidate values: {2 ,3, 4}; Optional with capability signaling. |
| Huawei/HiSilicon [19] | For Noncodebook multi-DCI based STx2P PUSCH+PUSCH, similar to the FGs for codebook multi-DCI based STx2P PUSCH+PUSCH, we have the following proposal Proposal 5-10: - For FG 40-6-3b - For component 1, support fully/partial overlapping PUSCHs in time and non-overlapping in frequency for multi-DCI based STx2P PUSCH+PUSCH for noncodebook-based PUSCH, and remove FG 40-6-3j. - support Per band reporting granularity - Support FG 40-6-3l with pre-requisite of FG 40-6-3b. - Support FG 40-6-3k with pre-requisite of FG 40-6-3l; - Do not support FG 40-6-3n since the FG is already included in FG 40-6-3b and FG 40-6-3k; - For FG 40-6-3k, FG 40-6-3l, and FG 40-6-3m, support Per band reporting granularity. For FG 40-6-3o and FG 40-6-3p, no other component is necessary. Prerequisite is FG 40-6-3b. 40. NR_MIMO_evo_DL_UL; 40-6-3b; Noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG; 1. Support of multi-DCI based STxMP PUSCH+PUSCH for noncodebook-based PUSCH with fully[/partial] overlapping PUSCHs in time and [non-overlapping in frequency] 2. Support of two SRS resource sets with usage set to 'noncodebook' associated with two coresetPoolInde values 3. Maximum number of SRS resources in one SRS resource set 4. Maximum number of layers of each PUSCH of PUSCH+PUSCH overlapping in time domain [5. Maximum number of simultaneously transmitted SRS resources in one symbol per SRS resource set] 6. Maximum number of PUSCHs per CORESETPoolIndex per slot 7. Maximum total number of layers across two overlapping PUSCH; 2-14; Yes; N/A; Noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported; [Per Band]; n/a; FR2 only; n/a; Note: Processing capability 2 is not supported in any CC if at least one CC is configured with two values of CORESETPoolIndex. Component 3 candidate values: {1,2,[3,]4} Component 4 candidate values: {1,2} [Component 5 candidate values: {1,2,[3,4]}] Component 6 candidate values: {1,2,3,4,7} Note: per SCS, similar with Rel-15 Component 7 candidate values: {2 ,3, 4}; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-6-3j | [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Partial-overlapping PUSCHs in time and non-overlapping in frequency] | [Support of partial-overlapping PUSCHs in time and non-overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH] | [40-6-3a] | [Yes] | [N/A] | [Partial-overlapping PUSCHs in time and non-overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported] | [Per Band] | [n/a] | [FR2 only] | [n/a] |  | [Optional with capability signaling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-6-3c to FG40-6-3g and FG40-6-3j to FG 40-6-3n - Remove the square brackets and confirm the FGs based on the agreement above 40. NR_MIMO_evo_DL_UL; 40-6-3j; [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Partial-overlapping PUSCHs in time and non-overlapping in frequency]; [Support of partial-overlapping PUSCHs in time and non-overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH]; [40-6-3b]; [Yes]; [N/A]; [Partial-overlapping PUSCHs in time and non-overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |
| Ericsson [3] | We propose to remove this, as it is part of FG 40-6-3a |
| ZTE [4] | - For FG 40-6-3j, FG 40-6-3m and FG 40-6-3n, these should be removed due to that is duplicated with component-1 in FG40-6-3b. 40. NR_MIMO_evo_DL_UL; 40-6-3j; [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Partial-overlapping PUSCHs in time and non-overlapping in frequency]; [Support of partial-overlapping PUSCHs in time and non-overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH]; [40-6-3a]; [Yes]; [N/A]; [Partial-overlapping PUSCHs in time and non-overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-6-3j; [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Partial-overlapping PUSCHs in time and non-overlapping in frequency]; [Support of partial-overlapping PUSCHs in time and non-overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH]; [40-6-3a]; [Yes]; [N/A]; [Partial-overlapping PUSCHs in time and non-overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-6-3j; [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Partial-overlapping PUSCHs in time and non-overlapping in frequency]; [Support of partial-overlapping PUSCHs in time and non-overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH]; [40-6-3a]; [Yes]; [N/A]; [Partial-overlapping PUSCHs in time and non-overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |
| OPPO [8] | - For FG 40-6-3j, 40-6-3k, 40-6-3l, 40-6-3m, 40-6-3n, support separate rows of fully/partial overlapping PUSCH in time and fully/partial overlapping in frequency. |
| CATT [9] | 40-6-3c/3d/3e/3f/3g and 40-6-3j/3k/3l/3m/3n: - According to the discussion in FG 40-6-3a and FG 40-6-3b, support of fully/partially overlapping PDSCHs in time and fully/partially/non-overlapping in frequency is basic feature. Correspondingly, FG 40-6-3c/3d/3e/3f/3g and 40-6-3j/3k/3l/3m/3n can be removed. |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-6-3j/3k/3l/3h/3n - Support one separate row for full/partial overlapping in time and partial overlapping in frequency, and one separate row for full/partial overlapping in time and full overlapping in frequency, i.e., keep 40-6-3k, merge 40-6-l and 40-6-3m, remove 40-6-3j and 40-6-3n. |
| Apple [15] |  |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | - For the overlapping type as well as CG+DG/CG+CG, we do not see a strong reason to differentiate codebook versus non-codebook. Even though more FGs is generally good for UE implementation, but in this case, duplicating so many FGs may not be justified. Hence, we suggest to remove FGs 40-6-3j to 40-6-3p. |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-6-3j; [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Partial-overlapping PUSCHs in time and non-overlapping in frequency]; [Support of partial-overlapping PUSCHs in time and non-overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH]; [40-6-3a]; [Yes]; [N/A]; [Partial-overlapping PUSCHs in time and non-overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |

| 40. NR_MIMO_evo_DL_UL | 40-6-3k | [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Overlapping PUSCHs in time and partially overlapping in frequency] | [Support of overlapping PUSCHs in time and partially overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH] | [40-6-3a] | [Yes] | [N/A] | [Overlapping PUSCHs in time and partially overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported] | [Per Band] | [n/a] | [FR2 only] | [n/a] |  | [Optional with capability signaling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-6-3c to FG40-6-3g and FG40-6-3j to FG 40-6-3n - Remove the square brackets and confirm the FGs based on the agreement above 40. NR_MIMO_evo_DL_UL; 40-6-3k; [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Overlapping PUSCHs in time and partially overlapping in frequency]; [Support of overlapping PUSCHs in time and partially overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH]; [40-6-3b]; [Yes]; [N/A]; [Overlapping PUSCHs in time and partially overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |
| Ericsson [3] | - These features groups provide a higher granularity than for DL. Preferably, these feature groups should be combined into one. |
| ZTE [4] | - For FG 40-6-3k, FG 40-6-3l, these can be kept to capture the cases other than component-1 in FG40-6-3b. 40. NR_MIMO_evo_DL_UL; 40-6-3k; [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Overlapping PUSCHs in time and partially overlapping in frequency]; [Support of overlapping PUSCHs in time and partially overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH]; [40-6-3a]; [Yes]; [N/A]; [Overlapping PUSCHs in time and partially overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-6-3k; [Noncodebook multi-DCI based STx2P PUSCH+PUSCH –full/partial Overlapping PUSCHs in time and partially overlapping in frequency]; [Support of overlapping PUSCHs in time and partially overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH]; [40-6-3ba]; [Yes]; [N/A]; [Overlapping PUSCHs in time and partially overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-6-3k; [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Overlapping PUSCHs in time and partially overlapping in frequency]; [Support of overlapping PUSCHs in time and partially overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH]; [40-6-3a]; [Yes]; [N/A]; [Overlapping PUSCHs in time and partially overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |
| OPPO [8] | For FG 40-6-3j, 40-6-3k, 40-6-3l, 40-6-3m, 40-6-3n, support separate rows of fully/partial overlapping PUSCH in time and fully/partial overlapping in frequency. |
| CATT [9] | 40-6-3c/3d/3e/3f/3g and 40-6-3j/3k/3l/3m/3n: - According to the discussion in FG 40-6-3a and FG 40-6-3b, support of fully/partially overlapping PDSCHs in time and fully/partially/non-overlapping in frequency is basic feature. Correspondingly, FG 40-6-3c/3d/3e/3f/3g and 40-6-3j/3k/3l/3m/3n can be removed. |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-6-3j/3k/3l/3h/3n - Support one separate row for full/partial overlapping in time and partial overlapping in frequency, and one separate row for full/partial overlapping in time and full overlapping in frequency, i.e., keep 40-6-3k, merge 40-6-l and 40-6-3m, remove 40-6-3j and 40-6-3n. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-6-3k; Multi-DCI STx2P scheme – partially or non-overlapping in frequency, fully overlapping in time for noncodebook based PUSCH; - Support of Multi-DCI STx2P schedule with two PUSCHs partially or non-overlapping in frequency and fully overlapping in time for noncodebook based PUSCH.; Per band; FR2 only; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | - For the overlapping type as well as CG+DG/CG+CG, we do not see a strong reason to differentiate codebook versus non-codebook. Even though more FGs is generally good for UE implementation, but in this case, duplicating so many FGs may not be justified. Hence, we suggest to remove FGs 40-6-3j to 40-6-3p. |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-6-3k; [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Overlapping PUSCHs in time and partially overlapping in frequency]; [Support of overlapping PUSCHs in time and partially overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH]; [40-6-3a] 40-6-3l; [Yes]; [N/A]; [Overlapping PUSCHs in time and partially overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |

| 40. NR_MIMO_evo_DL_UL | 40-6-3l | [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping in both frequency and time] | [Support of fully overlapping in both frequency and time for noncodebook multi-DCI based STx2P PUSCH+PUSCH] | [40-6-3a] | [Yes] | [N/A] | [Fully overlapping in both frequency and time for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported] | [Per Band] | [n/a] | [FR2 only] | [n/a] |  | [Optional with capability signaling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-6-3c to FG40-6-3g and FG40-6-3j to FG 40-6-3n - Remove the square brackets and confirm the FGs based on the agreement above 40. NR_MIMO_evo_DL_UL; 40-6-3l; [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping in both frequency and time]; [Support of fully overlapping in both frequency and time for noncodebook multi-DCI based STx2P PUSCH+PUSCH]; [40-6-3b]; [Yes]; [N/A]; [Fully overlapping in both frequency and time for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |
| Ericsson [3] | - We propose to keep this FG, it corresponds to FG 16-2a-0. We propose to define this per band, according to the RAN2 recommendation [1]. 40-6-3l; Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping in both frequency and time; Support of fully overlapping in both frequency and time for noncodebook multi-DCI based STx2P PUSCH+PUSCH; 40-6-3a; Per Band. |
| ZTE [4] | - For FG 40-6-3k, FG 40-6-3l, these can be kept to capture the cases other than component-1 in FG40-6-3b. 40. NR_MIMO_evo_DL_UL; 40-6-3l; [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping in both frequency and time]; [Support of fully overlapping in both frequency and time for noncodebook multi-DCI based STx2P PUSCH+PUSCH]; [40-6-3a]; [Yes]; [N/A]; [Fully overlapping in both frequency and time for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-6-3l; [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping in both frequency and full/partial overlapping in time]; [Support of fully overlapping in both frequency and time for noncodebook multi-DCI based STx2P PUSCH+PUSCH]; [40-6-3ba]; [Yes]; [N/A]; [Fully overlapping in both frequency and time for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |
| Intel Corporation [7] |  |
| OPPO [8] | For FG 40-6-3j, 40-6-3k, 40-6-3l, 40-6-3m, 40-6-3n, support separate rows of fully/partial overlapping PUSCH in time and fully/partial overlapping in frequency. |
| CATT [9] | 40-6-3c/3d/3e/3f/3g and 40-6-3j/3k/3l/3m/3n: - According to the discussion in FG 40-6-3a and FG 40-6-3b, support of fully/partially overlapping PDSCHs in time and fully/partially/non-overlapping in frequency is basic feature. Correspondingly, FG 40-6-3c/3d/3e/3f/3g and 40-6-3j/3k/3l/3m/3n can be removed. |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-6-3j/3k/3l/3h/3n - Support one separate row for full/partial overlapping in time and partial overlapping in frequency, and one separate row for full/partial overlapping in time and full overlapping in frequency, i.e., keep 40-6-3k, merge 40-6-l and 40-6-3m, remove 40-6-3j and 40-6-3n. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-6-3l; Multi-DCI STx2P scheme – partially or non-overlapping in frequency, partially overlapping in time for noncodebook based PUSCH; - Support of Multi-DCI STx2P schedule with two PUSCHs partially or non-overlapping in frequency and partially overlapping in time for noncodebook based PUSCH.; Per band; FR2 only; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | - For the overlapping type as well as CG+DG/CG+CG, we do not see a strong reason to differentiate codebook versus non-codebook. Even though more FGs is generally good for UE implementation, but in this case, duplicating so many FGs may not be justified. Hence, we suggest to remove FGs 40-6-3j to 40-6-3p. |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-6-3l; [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping in both frequency and time]; [Support of fully overlapping in both frequency and time for noncodebook multi-DCI based STx2P PUSCH+PUSCH]; [40-6-3ba]; [Yes]; [N/A]; [Fully overlapping in both frequency and time for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |

| 40. NR_MIMO_evo_DL_UL | 40-6-3m | [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping in frequency, partially overlapping in time] | [Support of fully overlapping in frequency, partially overlapping in time for noncodebook multi-DCI based STx2P PUSCH+PUSCH] | [40-6-3a] | [Yes] | [N/A] | [Fully overlapping in frequency, partially overlapping in time for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported] | [Per Band] | [n/a] | [FR2 only] | [n/a] |  | [Optional with capability signaling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-6-3c to FG40-6-3g and FG40-6-3j to FG 40-6-3n - Remove the square brackets and confirm the FGs based on the agreement above 40. NR_MIMO_evo_DL_UL; 40-6-3m; [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping in frequency, partially overlapping in time]; [Support of fully overlapping in frequency, partially overlapping in time for noncodebook multi-DCI based STx2P PUSCH+PUSCH]; [40-6-3b]; [Yes]; [N/A]; [Fully overlapping in frequency, partially overlapping in time for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |
| Ericsson [3] | - These features groups provide a higher granularity than for DL. Preferably, these feature groups should be combined into one. |
| ZTE [4] | - For FG 40-6-3j, FG 40-6-3m and FG 40-6-3n, these should be removed due to that is duplicated with component-1 in FG40-6-3b. 40. NR_MIMO_evo_DL_UL; 40-6-3m; [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping in frequency, partially overlapping in time]; [Support of fully overlapping in frequency, partially overlapping in time for noncodebook multi-DCI based STx2P PUSCH+PUSCH]; [40-6-3a]; [Yes]; [N/A]; [Fully overlapping in frequency, partially overlapping in time for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-6-3m; [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping in frequency, partially overlapping in time]; [Support of fully overlapping in frequency, partially overlapping in time for noncodebook multi-DCI based STx2P PUSCH+PUSCH]; [40-6-3a]; [Yes]; [N/A]; [Fully overlapping in frequency, partially overlapping in time for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-6-3m; [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping in frequency, partially overlapping in time]; [Support of fully overlapping in frequency, partially overlapping in time for noncodebook multi-DCI based STx2P PUSCH+PUSCH]; [40-6-3a]; [Yes]; [N/A]; [Fully overlapping in frequency, partially overlapping in time for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |
| OPPO [8] | For FG 40-6-3j, 40-6-3k, 40-6-3l, 40-6-3m, 40-6-3n, support separate rows of fully/partial overlapping PUSCH in time and fully/partial overlapping in frequency. |
| CATT [9] | 40-6-3c/3d/3e/3f/3g and 40-6-3j/3k/3l/3m/3n: - According to the discussion in FG 40-6-3a and FG 40-6-3b, support of fully/partially overlapping PDSCHs in time and fully/partially/non-overlapping in frequency is basic feature. Correspondingly, FG 40-6-3c/3d/3e/3f/3g and 40-6-3j/3k/3l/3m/3n can be removed. |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] |  |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | - For the overlapping type as well as CG+DG/CG+CG, we do not see a strong reason to differentiate codebook versus non-codebook. Even though more FGs is generally good for UE implementation, but in this case, duplicating so many FGs may not be justified. Hence, we suggest to remove FGs 40-6-3j to 40-6-3p. |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-6-3m; [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping in frequency, partially overlapping in time]; [Support of fully overlapping in frequency, partially overlapping in time for noncodebook multi-DCI based STx2P PUSCH+PUSCH]; [40-6-3ba]; [Yes]; [N/A]; [Fully overlapping in frequency, partially overlapping in time for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |

| 40. NR_MIMO_evo_DL_UL | 40-6-3n | [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Partially or non-overlapping in frequency, partially or fully overlapping in time] | [Support of partially or non-overlapping in frequency, partially or fully overlapping in time for noncodebook multi-DCI based STx2P PUSCH+PUSCH] | [40-6-3a] | [Yes] | [N/A] | [Partially or non-overlapping in frequency, partially or fully overlapping in time for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported] | [Per Band] | [n/a] | [FR2 only] | [n/a] |  | [Optional with capability signaling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-6-3c to FG40-6-3g and FG40-6-3j to FG 40-6-3n - Remove the square brackets and confirm the FGs based on the agreement above 40. NR_MIMO_evo_DL_UL; 40-6-3n; [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Partially or non-overlapping in frequency, partially or fully overlapping in time]; [Support of partially or non-overlapping in frequency, partially or fully overlapping in time for noncodebook multi-DCI based STx2P PUSCH+PUSCH]; [40-6-3b]; [Yes]; [N/A]; [Partially or non-overlapping in frequency, partially or fully overlapping in time for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |
| Ericsson [3] | - These features groups provide a higher granularity than for DL. Preferably, these feature groups should be combined into one. |
| ZTE [4] | - For FG 40-6-3j, FG 40-6-3m and FG 40-6-3n, these should be removed due to that is duplicated with component-1 in FG40-6-3b. 40. NR_MIMO_evo_DL_UL; 40-6-3n; [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Partially or non-overlapping in frequency, partially or fully overlapping in time]; [Support of partially or non-overlapping in frequency, partially or fully overlapping in time for noncodebook multi-DCI based STx2P PUSCH+PUSCH]; [40-6-3a]; [Yes]; [N/A]; [Partially or non-overlapping in frequency, partially or fully overlapping in time for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-6-3n; [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Partially or non-overlapping in frequency, partially or fully overlapping in time]; [Support of partially or non-overlapping in frequency, partially or fully overlapping in time for noncodebook multi-DCI based STx2P PUSCH+PUSCH]; [40-6-3a]; [Yes]; [N/A]; [Partially or non-overlapping in frequency, partially or fully overlapping in time for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |
| Intel Corporation [7] |  |
| OPPO [8] | For FG 40-6-3j, 40-6-3k, 40-6-3l, 40-6-3m, 40-6-3n, support separate rows of fully/partial overlapping PUSCH in time and fully/partial overlapping in frequency. |
| CATT [9] | 40-6-3c/3d/3e/3f/3g and 40-6-3j/3k/3l/3m/3n: - According to the discussion in FG 40-6-3a and FG 40-6-3b, support of fully/partially overlapping PDSCHs in time and fully/partially/non-overlapping in frequency is basic feature. Correspondingly, FG 40-6-3c/3d/3e/3f/3g and 40-6-3j/3k/3l/3m/3n can be removed. |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-6-3j/3k/3l/3h/3n - Support one separate row for full/partial overlapping in time and partial overlapping in frequency, and one separate row for full/partial overlapping in time and full overlapping in frequency, i.e., keep 40-6-3k, merge 40-6-l and 40-6-3m, remove 40-6-3j and 40-6-3n. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-6-3o; Noncodebook multi-DCI based STx2P PUSCH+PUSCH for CG+CG; Support of multi-DCI based STxMP CG-PUSCH+CG-PUSCH for noncodebook [FFS: other components]; [40-6-3a]; Yes; N/A; Multi-DCI based STx2P for CG+CG is not supported for noncodebook is not supported; Per band; n/a; FR2 only; n/a; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | - For the overlapping type as well as CG+DG/CG+CG, we do not see a strong reason to differentiate codebook versus non-codebook. Even though more FGs is generally good for UE implementation, but in this case, duplicating so many FGs may not be justified. Hence, we suggest to remove FGs 40-6-3j to 40-6-3p. |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-6-3n; [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Partially or non-overlapping in frequency, partially or fully overlapping in time]; [Support of partially or non-overlapping in frequency, partially or fully overlapping in time for noncodebook multi-DCI based STx2P PUSCH+PUSCH]; [40-6-3a]; [Yes]; [N/A]; [Partially or non-overlapping in frequency, partially or fully overlapping in time for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported]; [Per Band]; [n/a]; [FR2 only]; [n/a]; [Optional with capability signaling]. |

| 40. NR_MIMO_evo_DL_UL | 40-6-3o | Noncodebook multi-DCI based STx2P PUSCH+PUSCH for CG+CG | Support of multi-DCI based STxMP CG-PUSCH+CG-PUSCH for noncodebook [FFS: other components] | [40-6-3a] | Yes | N/A | Multi-DCI based STx2P for CG+CG is not supported for noncodebook is not supported | [Per Band] | n/a | FR2 only | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-6-3o; Noncodebook multi-DCI based STx2P PUSCH+PUSCH for CG+CG; Support of multi-DCI based STxMP CG-PUSCH+CG-PUSCH for noncodebook [FFS: other components]; [40-6-3b]; Yes; N/A; Multi-DCI based STx2P for CG+CG is not supported for noncodebook is not supported; [Per Band]; n/a; FR2 only; n/a; Optional with capability signaling. |
| Ericsson [3] | - We propose to keep these. We propose to define this per band, according to the RAN2 recommendation [1]. 40-6-3o; Noncodebook multi-DCI based STx2P PUSCH+PUSCH for CG+CG; Support of multi-DCI based STxMP CG-PUSCH+CG-PUSCH for noncodebook; 40-6-3a; Per Band. |
| ZTE [4] |  |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-6-3h/3i and 40-6-3o/3p: - The prerequisite of 40-6-3h/3i is 40-3-3a. - The prerequisite of 40-6-3o/3p is 40-3-3b. - Similar to CG PUSCH transmission towards M-TRPs (FG 23-3-1g), UE feature granularities of 40-6-3h/3i and 40-6-3o/3p can be per band. Proposal 18: Adopt the following for FG 40-6-3h/3i and 40-6-3o/3p: - The prerequisite of 40-6-3h/3i is 40-3-3a. - The prerequisite of 40-6-3o/3p is 40-3-3b. - UE feature granularities of FG 40-6-3h/3i and 40-6-3o/3p are per band. |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] |  |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | - For the overlapping type as well as CG+DG/CG+CG, we do not see a strong reason to differentiate codebook versus non-codebook. Even though more FGs is generally good for UE implementation, but in this case, duplicating so many FGs may not be justified. Hence, we suggest to remove FGs 40-6-3j to 40-6-3p. |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-6-3o; Noncodebook multi-DCI based STx2P PUSCH+PUSCH for CG+CG; Support of multi-DCI based STxMP CG-PUSCH+CG-PUSCH for noncodebook [FFS: other components]; [40-6-3ba]; Yes; N/A; Multi-DCI based STx2P for CG+CG is not supported for noncodebook is not supported; [Per Band]; n/a; FR2 only; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-6-3p | Noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+CG | Support of multi-DCI based STxMP DG-PUSCH+CG-PUSCH for noncodebook [FFS: other components] | [40-6-3a] | Yes | N/A | Multi-DCI based STx2P for DG+CG is not supported for noncodebook is not supported | [Per Band] | n/a | FR2 only | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-6-3p; Noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+CG; Support of multi-DCI based STxMP DG-PUSCH+CG-PUSCH for noncodebook [FFS: other components]; [40-6-3b]; Yes; N/A; Multi-DCI based STx2P for DG+CG is not supported for noncodebook is not supported; [Per Band]; n/a; FR2 only; n/a; Optional with capability signaling. |
| Ericsson [3] | - We propose to keep these. We propose to define this per band, according to the RAN2 recommendation [1]. 40-6-3p; Noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+CG; Support of multi-DCI based STxMP DG-PUSCH+CG-PUSCH for noncodebook; 40-6-3a; Per Band. |
| ZTE [4] |  |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-6-3h/3i and 40-6-3o/3p: - The prerequisite of 40-6-3h/3i is 40-3-3a. - The prerequisite of 40-6-3o/3p is 40-3-3b. - Similar to CG PUSCH transmission towards M-TRPs (FG 23-3-1g), UE feature granularities of 40-6-3h/3i and 40-6-3o/3p can be per band. Proposal 18: Adopt the following for FG 40-6-3h/3i and 40-6-3o/3p: - The prerequisite of 40-6-3h/3i is 40-3-3a. - The prerequisite of 40-6-3o/3p is 40-3-3b. - UE feature granularities of FG 40-6-3h/3i and 40-6-3o/3p are per band. |
| Xiaomi [10] |  |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-6-3p; Noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+CG; Support of multi-DCI based STxMP DG-PUSCH+CG-PUSCH for noncodebook [FFS: other components]; [40-6-3a]; Yes; N/A; Multi-DCI based STx2P for DG+CG is not supported for noncodebook is not supported; Per band; n/a; FR2 only; n/a; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | - For the overlapping type as well as CG+DG/CG+CG, we do not see a strong reason to differentiate codebook versus non-codebook. Even though more FGs is generally good for UE implementation, but in this case, duplicating so many FGs may not be justified. Hence, we suggest to remove FGs 40-6-3j to 40-6-3p. |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-6-3p; Noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+CG; Support of multi-DCI based STxMP DG-PUSCH+CG-PUSCH for noncodebook [FFS: other components]; [40-6-3ba]; Yes; N/A; Multi-DCI based STx2P for DG+CG is not supported for noncodebook is not supported; [Per Band]; n/a; FR2 only; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-6-3b-1 | Associated CSI-RS resources for noncodebook multi-DCI based STx2P PUSCH+PUSCH |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-6-3b-1; Associated CSI-RS resources for noncodebook multi-DCI based STx2P PUSCH+PUSCH; Yes; n/a; [Per Band]; n/a; FR2 only; n/a; Optional with capability signaling. |
| Ericsson [3] | 40-6-3b-1; Associated CSI-RS resources for noncodebook multi-DCI based STx2P PUSCH+PUSCH; 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for multi-DCI non-codebook based STxMP scheme for PUSCH 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously; 40-6-3a; Per Band. |
| ZTE [4] | - For FG 40-6-3b-1, similar to the update of FG 40-6-3a, - FG 2-15a and FG 40-6-3b should be adopted as the prerequisite FGs. 40. NR_MIMO_evo_DL_UL; 40-6-3b-1; Associated CSI-RS resources for noncodebook multi-DCI based STx2P PUSCH+PUSCH; 2-15a, 40-6-3b. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-6-3b-1; Associated CSI-RS resources for noncodebook multi-DCI based STx2P PUSCH+PUSCH; 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook PUSCH of M-DCI based STxMP 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously; 2-15a,40-6-3b; Yes; n/a; Associated CSI-RS resources for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported.; Per band; No; FR2 only; Optional with capability signalling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-6-3b-1 - Component: same components as 40-6-1b and 40-6-2b should be included. - 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook based STxMP SFN scheme for PUSCH - 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP. Candidate value {1 to 8}. - 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP. Candidate value {1 to 8} - 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP. Candidate value {0 to 8} - 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS. Candidate value {1 to 16} - 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously. Candidate value {1,2} - Prerequisite FG: 40-6-3b, 2-15a |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-6-3b-1; Associated CSI-RS resources for noncodebook multi-DCI based STx2P PUSCH+PUSCH; 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook multi-DCI based STx2P PUSCH+PUSCH 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously; Per band; Component 2 candidate values: {1 to 8} Component 3 candidate values: {1 to 8} Component 4 candidate values: {0 to 8} Component 5 candidate values: {1 to 16} Component 6 candidate values: {1 to 2}; Optional with capability signalling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | - For 40-6-3b-1: The proposed changes are similar to 40-6-1b or 40-6-2b. 40. NR_MIMO_evo_DL_UL; 40-6-3b-1; Associated CSI-RS resources for noncodebook multi-DCI based STx2P PUSCH+PUSCH; 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook-based multi-DCI based STxMP 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously; 2-15a; Yes; N/A; Per band; n/a; FR2 only; n/a; Component 2: {1 to 8} Component 3: {1 to 8} Component 4: {0 to 8} Component 5: {1 to 16} Component 6: {1,2}; Optional with capability signaling. |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-6-4 | Single-DCI based STx2P SFN scheme for PUCCH | 1. Support of single-DCI based STx2P SFN scheme for PUCCH 2. Supported PUCCH formats for STxMP SFN scheme | 23-3-2d | Yes | n/a | Single-DCI based STx2P SFN scheme for PUCCH is not supported | [Per FS] | n/a | FR2 only | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-6-4; Single-DCI based STx2P SFN scheme for PUCCH; 1. Support of single-DCI based STx2P SFN scheme for PUCCH 2. Supported PUCCH formats for STxMP SFN scheme; 23-3-2d; Yes; n/a; Single-DCI based STx2P SFN scheme for PUCCH is not supported; [Per FS]; n/a; FR2 only; n/a; Optional with capability signaling. |
| Ericsson [3] | - The type for FG 40-6-4 should be per band, according to the RAN2 recommendation [1]. 40-6-4; Single-DCI based STx2P SFN scheme for PUCCH; 1. Support of single-DCI based STx2P SFN scheme for PUCCH 2. Supported PUCCH formats for STxMP SFN scheme; 23-3-2d; Single-DCI based STx2P SFN scheme for PUCCH is not supported; Per Band. |
| ZTE [4] |  |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-6-4; Single-DCI based STx2P SFN scheme for PUCCH; 1. Support of single-DCI based STx2P SFN scheme for PUCCH 2. Supported PUCCH formats for STxMP SFN scheme; 23-3-2d; Yes; n/a; Single-DCI based STx2P SFN scheme for PUCCH is not supported; Per FS; No; FR2 only; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] |  |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-6-4; Single-DCI based STx2P SFN scheme for PUCCH; 1. Support of single-DCI based STx2P SFN scheme for PUCCH 2. Supported PUCCH formats for STxMP SFN scheme; 23-3-2d; Yes; n/a; Single-DCI based STx2P SFN scheme for PUCCH is not supported; Per FS; n/a; FR2 only; n/a; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-6-4a; Rel-18 STxMP SFN scheme and the Rel-17 repetition number parameter pucch-RepetitionNrofSlots in a same PUCCH resource at the same time; Support of enabling Rel-18 STxMP SFN scheme and the Rel-17 repetition number parameter pucch-RepetitionNrofSlots in a same PUCCH resource at the same time; Per band. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | - For FG 40-6-4 (SFN PUCCH): - The pre-requisite “23-3-2d” should be deleted. Although this seems to be non-highlighted (i.e., agreed), we believe this was not the intention (was a typo or mistake) as Rel-17 FG “23-3-2d” has nothing to do with SFN PUCCH (FG 23-3-2d is related to updating two spatial relation info’s for a group of PUCCH resources, and SFN PUCCH is not even based on spatial relation info framework. - Reporting type should be per FS. - Detail of candidate value for component 2 is also added. - A new FG (40-6-4a) is added given the agreement in RAN1 #114 on “It is subject to UE capability to support enabling Rel-18 STxMP SFN scheme and the Rel-17 repetition number parameter pucch-RepetitionNrofSlots in a same PUCCH resource at the same time.” 40. NR_MIMO_evo_DL_UL; 40-6-4; Single-DCI based STx2P SFN scheme for PUCCH; 1. Support of single-DCI based STx2P SFN scheme for PUCCH 2. Supported PUCCH formats for STxMP SFN scheme; Yes; n/a; Single-DCI based STx2P SFN scheme for PUCCH is not supported; Per FS; n/a; FR2 only; n/a; Component 2 candidate values: {PF0/2, PF1/3/4, PF0-4}; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-6-4a; Dynamic indication of repetition number for SFN scheme for PUCCH; Support STxMP SFN PUCCH scheme together with the Rel-17 dynamic indication of repetition number; 40-6-4; Yes; n/a; Per band; n/a; FR2 only; n/a; Optional with capability signaling. |
| Huawei/HiSilicon [19] | 40. NR_MIMO_evo_DL_UL; 40-6-4; Single-DCI based STx2P SFN scheme for PUCCH; 1. Support of single-DCI based STx2P SFN scheme for PUCCH 2. Supported PUCCH formats for STxMP SFN scheme; 23-3-2d; Yes; n/a; Single-DCI based STx2P SFN scheme for PUCCH is not supported; [Per FS]; n/a; FR2 only; n/a; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-6-5 | Support grouped-based beam reporting for STx2P |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-6-5; Support grouped-based beam reporting for STx2P; Yes; n/a; grouped-based beam reporting for STx2P is not supported; [Per Band]; n/a; FR2 only; n/a; Optional with capability signaling. |
| Ericsson [3] | RAN1 agreed two alternatives for group-based beam reporting for STx2P. For Alt1, simultaneous UL transmission is possible. For Alt 2, simultaneous UL transmission and simultaneous DL reception is possible. There is a need for separate rows: the existing FG 40-6-5 will correspond to simultaneous UL transmission, and the new FG 40-6-5a will correspond to simultaneous UL transmission and simultaneous DL reception. We have added components to these two feature groups, corresponding to FG 23-5-1. 40-6-5; Support UL-only grouped-based beam reporting for STx2P; 1. Max number N of beam groups (M=2 beams per beam group) in a single L1-RSRP reporting instance based on measurement on two CMR resource sets 2. Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs 3. Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets; Per band. 40-6-5a; Support UL and DL group-based beam reporting for STx2P; 1. Max number N of beam groups (M=2 beams per beam group) in a single L1-RSRP reporting instance based on measurement on two CMR resource sets 2. Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs 3. Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets; Per band. |
| ZTE [4] | - For FG 40-6-5, - Note that two Alts were adopted to enhance the Rel-17 group-based beam L1-RSRP reporting to support STxMP-based transmission and each of them is subject to UE capability, hence it is needed to capture this in the part of component. Agreement To enhance the Rel-17 group-based beam L1-RSRP reporting to support STxMP-based transmission, support the system to configure the UE to report one of the followings: - Alt1: For each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously, and the reported pair of CRIs or SSBRIs can be received simultaneously. - Alt2: For each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously. - Supporting Alt1 and/or Alt2 is subject to UE capability. - Its prerequisite should include FG 23-5-1. 40. NR_MIMO_evo_DL_UL; 40-6-5; Support grouped-based beam reporting for STx2P; Support of one of {Mode 1, Mode 2, both Mode 1 and Mode 2} - Note: Mode 1 is that the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously, and the reported pair of CRIs or SSBRIs can be received simultaneously. - Note: Mode 2 is that the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously.; 23-5-1. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] | - For FG 40-6-5, support component 1 and component 2 for group-based beam reporting. 40. NR_MIMO_evo_DL_UL; 40-6-5; Support grouped-based beam reporting for STx2P; 1. For each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously 2. The reported pair of CRIs or SSBRIs can be received simultaneously.; 40-6-1, 40-6-1a, 40-6-2, 40-6-2a; Yes; n/a; grouped-based beam reporting for STx2P is not supported; Per Band; n/a; FR2 only; n/a; Optional with capability signaling. |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-6-5; Support grouped-based beam reporting for STx2P; - Support of the reported mode for group-based beam reporting for STxMP transmission; - Max number N of beam pairs in a single L1-RSRP reporting instance for STxMP transmission;; 23-5-1; Yes; n/a; Group-based beam reporting for STx2P is not supported; Per Band; No; FR2 only; Candidate value for component 1: {Joint ULand DL, UL only, both}; Candidate value for component 2: {1,2,3,4}; Optional with capability signalling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-6-5 - Component: similar components as Rel-17 group based beam reporting can be included. Following components can be included. - Max number of beam groups in a beam reporting. Candidate values: {1,2,3,4}. - Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs. Candidate values: {2,3,4,8,16,32,64} - Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs. Candidate values: {8,16,32,64,128} |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-6-5; Support grouped-based beam reporting for STx2P; - Support grouped-based beam reporting for STx2P.; Per band; FR2 only; Component 1 candidate value: {mode 1, mode 2, both} Note: Mode 1: For each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously, and the reported pair of CRIs or SSBRIs can be received simultaneously. Mode 2: For each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously.; Optional with capability signalling. |
| Samsung [16] | RAN1 agreed enhance group-based beam reporting to support UL STxMP schemes. Agreement To enhance the Rel-17 group-based beam L1-RSRP reporting to support STxMP-based transmission, support the system to configure the UE to report one of the followings: - Alt1: For each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously, and the reported pair of CRIs or SSBRIs can be received simultaneously. - Alt2: For each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously. - Supporting Alt1 and/or Alt2 is subject to UE capability. In the agreement, Alt1 (the reported beam pair(s) can be used for both UL simultaneous transmission and DL simultaneous reception) and/or Alt2 (the reported beam pair(s) can be used for only UL simultaneous transmission) are supported depending on UE capability reporting. Therefore, if the UE supports UL STxMP scheme(s), the UE should report the possible scheme(s) of group based beam reporting. As additional FFS point, we think RAN1 needs to discuss on prerequisite for FG 40-6-5, especially, whether FG 23-5-1 can be prerequisite or not. 40-6-5; Support grouped-based beam reporting for STx2P; 1. Support group based L1-RSRP reporting for STxMP based transmission - Component 1 candidate values: {DL and UL, UL only, both} 2. Max number N of beam groups (M=2 beams per beam group) in a single L1-RSRP reporting instance based on measurement on two CMR resource sets 3. Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs 4. Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs. |
| MediaTek Inc. [17] | FG 40-6-5 In this FG, based on the following agreement, supporting Alt1 and/or Alt2 is subject to UE capability. Thus, a UE can report a value from the candidate {JointULandDL, ULOnly, both JointULandDL and ULOnly}. Agreement To enhance the Rel-17 group-based beam L1-RSRP reporting to support STxMP-based transmission, support the system to configure the UE to report one of the followings: - Alt1: For each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously, and the reported pair of CRIs or SSBRIs can be received simultaneously. - Alt2: For each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously. - Supporting Alt1 and/or Alt2 is subject to UE capability. Proposal 27: Support the following candidate value for the component “Support grouped-based beam reporting for STx2P” in FG 40-6-5 - {JointULandDL, ULOnly, both JointULandDL and ULOnly} |
| Qualcomm Incorporated [18] | - For FG 40-6-5: - For component 1, the proposal captures the following RAN1 agreement. Agreement To enhance the Rel-17 group-based beam L1-RSRP reporting to support STxMP-based transmission, support the system to configure the UE to report one of the followings: - Alt1: For each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously, and the reported pair of CRIs or SSBRIs can be received simultaneously. - Alt2: For each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously. - Supporting Alt1 and/or Alt2 is subject to UE capability Components 2/3/4, their candidate values, and the note are consistent with Rel-17 FG 23-5-1 (Rel-17 group-based L1-RSRP). 40. NR_MIMO_evo_DL_UL; 40-6-5; Support group-based beam reporting for STx2P; 1. Supported mode of group-based beam reporting for STx2P 2. Max number N of beam groups (M=2 beams per beam group) in a single L1-RSRP reporting instance based on measurement on two CMR resource sets 3. Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs 4. Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs; Yes; N/A; Per band; n/a; FR2 only; n/a; Component 1 candidate values: {JointULandDL, ULOnly, both} Component 2 candidate values: {1,2,3,4} Component 3 candidate values: {2,3,4,8,16,32,64} Component 4 candidate values: {8, 16, 32, 64, 128} Note: components 3 and 4 are also counted in FG 16-1g, 16-1g-1, and 23-5-1.; Optional with capability signaling. |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-7-1 | Basic features for Codebook-based 8Tx PUSCH | 1. Supported maximal PUSCH MIMO layers for codebook based PUSCH 2. Supported maximum number of 8 port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 8Tx PUSCH | FFS | Yes | n/a | Codebook-based 8Tx PUSCH is not supported | [Per FSPC] | No | No | No | A UE that supprts FG 40-7-1 must support at least one of FGs 40-7-1a/b/c/d | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-7-1; Basic features for Codebook-based 8Tx PUSCH; 1. Supported maximal PUSCH MIMO layers for codebook based PUSCH 2. Supported maximum number of 8 port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 8Tx PUSCH; FFS; Yes; n/a; Codebook-based 8Tx PUSCH is not supported; [Per FSPC]; No; No; No; A UE that supprts FG 40-7-1 must support at least one of FGs 40-7-1a/b/c/d; Optional with capability signaling. |
| Ericsson [3] |  |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-7-1; Basic features for Codebook-based 8Tx PUSCH; 1. Supported maximal number of PUSCH MIMO layers for codebook based PUSCH 2. Supported maximum number of 8 port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 8Tx PUSCH; FFS; Yes; n/a; Codebook-based 8Tx PUSCH is not supported; Per FSPC; No; No; No; 1. Component candidate values: {1,2,3,4,5,6,7,8} A UE that supprts FG 40-7-1 must support at least one of FGs 40-7-1a/b/c/d; Optional with capability signaling. |
| Fujitsu [5] | 40-7-1; Basic features for Codebook-based 8Tx PUSCH; 1. Supported maximal PUSCH MIMO layers for codebook based PUSCH 2. Supported maximum number of 8 port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 8Tx PUSCH; FFS; A UE that supports FG 40-7-1 must support at least one of FGs 40-7-1a/b/c/d. |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-7-1; Basic features for Codebook-based 8Tx PUSCH; 1. Supported maximal PUSCH MIMO layers for codebook based PUSCH 2. Supported maximum number of 8 port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 8Tx PUSCH; Yes; n/a; Codebook-based 8Tx PUSCH is not supported; Per FSPC; No; No; No; A UE that supprts FG 40-7-1 must support at least one of FGs 40-7-1a/b/c/d; Optional with capability signaling. |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-7-1: - The prerequisite feature group of FG 40-7-1 can be the basic UE features for codebook based PUSCH transmission in Rel-15, i.e., FG 2-14. - Similar as FG 2-14 (basic UE features for codebook based PUSCH transmission in Rel-15), for FG 40-7-1, - It can be a FG per FSPC. - The candidate values of component-1 are { 1, 2, 4, 8}. - The candidate values of component-2 are {1, 2}. Proposal 19: Adopt the following for FG 40-7-1: - The prerequisite of FG 40-7-1 is FG 2-14; - FG 40-7-1 is a FG per FSPC; - The candidate values of component-1 are {1, 2, 4, 8}; - The candidate values of component-2 are {1, 2}. |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-7-1; Basic features for Codebook-based 8Tx PUSCH; 1. Supported maximal PUSCH MIMO layers for codebook based PUSCH 2. Supported maximum number of 8 port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 8Tx PUSCH; FFS 2-14; Yes; n/a; Codebook-based 8Tx PUSCH is not supported; [Per FSPC]; No; No; No; A UE that supprts FG 40-7-1 must support at least one of FGs 40-7-1a/b/c/d Component 1 candidate values: {1,2,3,4,5,6,7,8} Component 2 candidate values: {1,2}; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-7-1 - Prerequisite FG: 2-14 (CB based PUSCH). - Component - For component 1, candidate values are {1,2,3,4,5,6,7,8}. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-7-1; Basic features for Codebook-based 8Tx PUSCH; 1. Supported maximal PUSCH MIMO layers for codebook based PUSCH 2. Supported maximum number of 8 port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 8Tx PUSCH; FFS; Yes; n/a; Codebook-based 8Tx PUSCH is not supported; Per FSPC; No; No; No; Component 1 candidate values: {1, 2, [3,] 4, [5, 6, 7,] 8} Component 2 candidate values: {1, 2} A UE that supports FG 40-7-1 must support at least one of FGs 40-7-1a/b/c/d; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-7-1; Basic features for Codebook-based 8Tx PUSCH; 1. Supported maximal PUSCH MIMO layers for codebook based PUSCH 2. Supported maximum number of 8 port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 8Tx PUSCH; YES; N/A; [Per FSPC]; NO; NO; NO; Component 1 candidate values: {1,2,3,4,5,6,7,8} A UE that supprts FG 40-7-1 must support at least one of FGs 40-7-1a/b/c/d; Optional with capability signaling. |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-7-1a | Codebook-based 8Tx PUSCH—[full-coherent codebook] | Support of codebook-based 8Tx PUSCH—[full-coherent codebook] | 40-7-1 | Yes | n/a | Codebook-based 8Tx PUSCH—[full-coherent] codebook is not supported | [Per FSPC] | No | No | No | FFS: whether to (N1,N2) as component or new row | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-7-1a - FFS: whether to (N1,N2) as component or new row: we recommend to include (N1.N2) as components. We also included the candidate values. 40. NR_MIMO_evo_DL_UL; 40-7-1a; Codebook-based 8Tx PUSCH—[full-coherent codebook]; Support of codebook-based 8Tx PUSCH—[full-coherent codebook] Ng= 1; 40-7-1; Yes; n/a; Codebook-based 8Tx PUSCH—[full-coherent] codebook is not supported; [Per FSPC]; No; No; No; FFS: whether to (N1,N2) as component or new row components (N1, N2) ={ (2,2), (4,1), both (2,2) and (4,1)}; Optional with capability signaling. |
| Ericsson [3] | Regarding the naming of features 40-7-1a/b/c/d, these can be named according to what is used in 38.211 now, as shown below. We note that this is related to the issue of how to define coherence as discussed by the feature lead in the last meeting [6]. However, since the terms full-, partial-, or non-coherent are not used for 8 Tx in the specifications at present, and there are no codebook subsets defined with these terms, they should not be added to UE capabilities unless they are defined. Regarding the FFS for 40-7-1a: “whether to (N1,N2) as component or new row”, the RRC parameter for the fully coherent / Ng=1 codebook is defined as follows in our understanding: {codebook1=ng1n4n1 and/or codebook1=ng1n4n1}, so we can define the components as “Supported codebooks: {codebook1=ng1n4n1 and/or codebook1=ng1n4n1}”. 40-7-1a; Codebook-based 8Tx PUSCH—codebook1 [full-coherent codebook]; Supported codebooks: {codebook1=ng1n4n1 and/or codebook1=ng1n4n1}; 40-7-1. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-7-1a; Codebook-based 8Tx PUSCH—[full-coherent codebook], Ng=1; 1. Support of codebook-based 8Tx PUSCH—[full-coherent codebook] Ng=1 2. Support of (N1, N2) for codebook-based 8Tx PUSCH—full-coherent codebook, Ng=1; 40-7-1; Yes; n/a; Codebook-based 8Tx PUSCH—[full-coherent codebook], Ng=1 codebook is not supported; Per FSPC; No; No; No; FFS: whether to (N1,N2) as component or new row 2. Component candidate values: {(4,1), (2,2), both}; Optional with capability signaling. |
| Fujitsu [5] | 40-7-1a; Codebook-based 8Tx PUSCH—[full-coherent codebook]; Support of codebook-based 8Tx PUSCH—[full-coherent codebook]; 40-7-1; FFS: whether to put (N1,N2) as component or new row The full coherent UE shall support FG 40-7-1a and can also support FG 40-7-1b, 40-7-1c, 40-7-1d.. |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-7-1a; Codebook-based 8Tx PUSCH—full-coherent codebook; Support of codebook-based 8Tx PUSCH—full-coherent codebook - Supported combinations of (N1, N2); 40-7-1; Yes; n/a; Codebook-based 8Tx PUSCH—full-coherent codebook is not supported; Per FSPC; No; No; No; Optional with capability signaling Component 1: {(2,2), (4,1)}. |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-7-1a: - In RAN1 #112 meeting, it has been agreed that the following combinations of (N1, N2) for Ng =1 are supported: (N1, N2) = (4, 1) and (N1, N2) = (2, 2). There is no motivation for an UE to report both of (4, 1) and (2, 2) for (N1, N2). Therefore reporting only one of (4, 1) and (2, 2) by FG 40-7-1a can be considered, i.e., (N1,N2) can be a component of FG 40-7-1a, with 2 candidate values corresponds to (4, 1) and (2, 2) respectively. - Since the the prerequisite FG for 40-7-1a is 40-7-1, and FG 40-7-1 is per FSPC, FG 40-7-1a shall be a FG per FSPC. |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-7-1a; Codebook-based 8Tx PUSCH—[full-coherent codebook]; 1. Support of codebook-based 8Tx PUSCH—[full-coherent codebook] 2. Supported number of antenna ports in the horizontal and vertical dimensions (N1,N2).; 40-7-1; Yes; n/a; Codebook-based 8Tx PUSCH—[full-coherent] codebook is not supported; [Per FSPC]; No; No; No; FFS: whether to (N1,N2) as component or new row Component 1 candidate values: {(4,1),(2,2)}; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-7-1a/1b/1c/1d - Component - Based on current CR, ‘[full-coherent codebook]’ can be replaced by ‘Codebook1’, ‘[partial-coherent codebook], Ng=2’ can be replaced by ‘Codebook2’, ‘[partial-coherent codebook], Ng=4’ can be replaced by ‘Codebook3’, ‘[non-coherent codebook]’ can be replaced by ‘Codebook4’. - For the FFS for 40-7-1a, we prefer to make (N1,N2) as component, instead of new row. For the candidate values for (N1, N2), (4,1) or (2,2) can be reported by UE. There is no need to support ‘both’ in our views. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-7-1a; Codebook-based 8Tx PUSCH—[full-coherent codebook] Higher-layer parameter CodebookType = “codebook1”; Support of codebook-based 8Tx PUSCH— Higher-layer parameter CodebookType = “codebook1”; 40-7-1; Yes; n/a; Codebook-based 8Tx PUSCH— Higher-layer parameter CodebookType = “codebook1” is not supported; Per FS; No; No; No; Candidate values: {N1=4 and N1=1, N1=2 and N2=2}; Optional with capability signaling. |
| Samsung [16] | - FG 40-7-1a: - following running RAN2 CR on Rel.18 RRC parameter, where ng1n4n1 and ng1n2n2 are used for two values Ng,N1,N2= (1,4,1) and (1,2,2), (N1,N2) can be included as a component of this row. In particular, the component can be Ng,N1,N2=1,4,1, (1,2,2). - Re the text on full-coherent, since this has been used in legacy (Rel.15 UL codebooks), for consistency, it should be included in the description of this row. Besides, since Ng=1 is included in the RRC parameter, both Ng=1 and full-coherent can be included. |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-7-1a; Codebook-based 8Tx PUSCH [full-coherent codebook]; Support of codebook-based 8Tx PUSCH [full-coherent codebook]; 40-7-1; Yes; n/a; Codebook-based 8Tx PUSCH [full-coherent] codebook is not supported; [Per FSPC]; No; No; No; FFS: whether to (N1,N2) as component or new row; Optional with capability signaling. |
| Huawei/HiSilicon [19] | For FG 40-7-1a, we have the following proposal: Proposal 5-11: - Regarding FG 40-7-1a, support (N1,N2) as component. 40. NR_MIMO_evo_DL_UL; 40-7-1a; Codebook-based 8Tx PUSCH—[full-coherent codebook]; 1. Support of codebook-based 8Tx PUSCH—[full-coherent codebook] 2. (N1, N2); 40-7-1; Yes; n/a; Codebook-based 8Tx PUSCH—[full-coherent] codebook is not supported; [Per FSPC]; No; No; No; FFS: whether to (N1,N2) as component or new row Candidates of component 2: {(4, 1), (2,2)}; Optional with capability signaling. |

| 40. NR_MIMO_evo_DL_UL | 40-7-1b | Codebook-based 8Tx PUSCH—[partial-coherent codebook], Ng=2 | Support of codebook-based 8Tx PUSCH—[partial-coherent codebook], Ng=2 | 40-7-1 | Yes | n/a | Codebook-based 8Tx PUSCH—[partial-coherent codebook], Ng=2 is not supported | [Per FSPC] | No | No | No |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-7-1b; (N1,N2) for coherent codebook Codebook-based 8Tx PUSCH—[partial-coherent codebook], Ng=2; Support of codebook-based 8Tx PUSCH—[partial-coherent codebook], Ng=2; 40-7-1; Yes; n/a; Codebook-based 8Tx PUSCH—[partial-coherent codebook], Ng=2 is not supported; [Per FSPC]; No; No; No; Optional with capability signaling. |
| Ericsson [3] | Regarding the naming of features 40-7-1a/b/c/d, these can be named according to what is used in 38.211 now, as shown below. We note that this is related to the issue of how to define coherence as discussed by the feature lead in the last meeting [6]. However, since the terms full-, partial-, or non-coherent are not used for 8 Tx in the specifications at present, and there are no codebook subsets defined with these terms, they should not be added to UE capabilities unless they are defined. 40-7-1b; Codebook-based 8Tx PUSCH— codebook2 [partial-coherent codebook], Ng=2; 40-7-1. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-7-1b; Codebook-based 8Tx PUSCH—[partial-coherent codebook], Ng=2; Support of codebook-based 8Tx PUSCH—[partial-coherent codebook], Ng=2; 40-7-1; Yes; n/a; Codebook-based 8Tx PUSCH—[partial-coherent codebook], Ng=2 is not supported; Per FSPC; No; No; No; Optional with capability signaling. |
| Fujitsu [5] | 40-7-1b; Codebook-based 8Tx PUSCH—[partial-coherent codebook], Ng=2; Support of codebook-based 8Tx PUSCH—[partial-coherent codebook], Ng=2; 40-7-1; The partial coherent UE with two antenna groups shall support FG 40-7-1b and can also support FG 40-7-1c, 40-7-1d.. |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-7-1b; Codebook-based 8Tx PUSCH—partial-coherent codebook, Ng=2; Support of codebook-based 8Tx PUSCH—partial-coherent codebook, Ng=2; 40-7-1; Yes; n/a; Codebook-based 8Tx PUSCH—partial-coherent codebook, Ng=2 is not supported; Per FSPC; No; No; No; Optional with capability signaling. |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-7-1b/1c/1d: - Similar as FG 40-7-1a, FG 40-7-1b/1c/1d shall be FGs per FSPC. Proposal 20: Adopt (N1,N2) as a component for 40-7-1a. Proposal 21: The prerequisite FG of FG(s) 40-7-1a, 40-7-1b, 40-7-1c and 40-7-1d is FG 2-12. |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-7-1b; (N1,N2) for coherent codebook Codebook-based 8Tx PUSCH—[partial-coherent codebook], Ng=2; Support of codebook-based 8Tx PUSCH—[partial-coherent codebook], Ng=2; 40-7-1; Yes; n/a; Codebook-based 8Tx PUSCH—[partial-coherent codebook], Ng=2 is not supported; [Per FSPC]; No; No; No; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-7-1a/1b/1c/1d - Component - Based on current CR, ‘[full-coherent codebook]’ can be replaced by ‘Codebook1’, ‘[partial-coherent codebook], Ng=2’ can be replaced by ‘Codebook2’, ‘[partial-coherent codebook], Ng=4’ can be replaced by ‘Codebook3’, ‘[non-coherent codebook]’ can be replaced by ‘Codebook4’. - For the FFS for 40-7-1a, we prefer to make (N1,N2) as component, instead of new row. For the candidate values for (N1, N2), (4,1) or (2,2) can be reported by UE. There is no need to support ‘both’ in our views. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-7-1b; (N1,N2) for coherent codebook Codebook-based 8Tx PUSCH—[partial-coherent codebook], Higher-layer parameter CodebookType = “codebook2”; Support of codebook-based 8Tx PUSCH—[partial-coherent codebook], Higher-layer parameter CodebookType = “codebook2”; 40-7-1; Yes; n/a; Codebook-based 8Tx PUSCH - Higher-layer parameter CodebookType = “codebook2” is not supported; Per FS; No; No; No; Optional with capability signaling. |
| Samsung [16] | - FG 40-7-1b/1c/1d: likewise, since the text (partial-coherent and non-coherent) has been used in legacy (Rel.15 UL codebooks), for consistency, it should be included in the description of respective rows. Besides, since Ng=2,4,8 are included in the RRC parameter, the following can be included in respective rows. - FG 40-7-1b: both Ng=2 and partial-coherent |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-7-1b; Codebook-based 8Tx PUSCH [partial-coherent codebook], Ng=2; Support of codebook-based 8Tx PUSCH [partial-coherent codebook], Ng=2; 40-7-1; Yes; n/a; Codebook-based 8Tx PUSCH [partial-coherent codebook], Ng=2 is not supported; [Per FSPC]; No; No; No; Optional with capability signaling. |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-7-1c | Codebook-based 8Tx PUSCH— [partial-coherent codebook], Ng=4 | Support of codebook-based 8Tx PUSCH— [partial-coherent codebook], Ng=4 | 40-7-1 | Yes | n/a | Codebook-based 8Tx PUSCH— [partial-coherent codebook], Ng=4 is not supported | [Per FSPC] | No | No | No |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-7-1c; Codebook-based 8Tx PUSCH— [partial-coherent codebook], Ng=4; Support of codebook-based 8Tx PUSCH— [partial-coherent codebook], Ng=4; 40-7-1; Yes; n/a; Codebook-based 8Tx PUSCH— [partial-coherent codebook], Ng=4 is not supported; [Per FSPC]; No; No; No; Optional with capability signaling. |
| Ericsson [3] | Regarding the naming of features 40-7-1a/b/c/d, these can be named according to what is used in 38.211 now, as shown below. We note that this is related to the issue of how to define coherence as discussed by the feature lead in the last meeting [6]. However, since the terms full-, partial-, or non-coherent are not used for 8 Tx in the specifications at present, and there are no codebook subsets defined with these terms, they should not be added to UE capabilities unless they are defined. 40-7-1c; Codebook-based 8Tx PUSCH— codebook3 [partial-coherent codebook], Ng=4; 40-7-1. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-7-1c; Codebook-based 8Tx PUSCH—[partial-coherent codebook], Ng=4; Support of codebook-based 8Tx PUSCH—[partial-coherent codebook], Ng=4; 40-7-1; Yes; n/a; Codebook-based 8Tx PUSCH—[partial-coherent codebook], Ng=4 is not supported; Per FSPC; No; No; No; Optional with capability signaling. |
| Fujitsu [5] | 40-7-1c; Codebook-based 8Tx PUSCH— [partial-coherent codebook], Ng=4; Support of codebook-based 8Tx PUSCH— [partial-coherent codebook], Ng=4; 40-7-1; The partial coherent UE with four antenna groups shall support FG 40-7-1c and can also support FG 40-7-1d.. |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-7-1c; Codebook-based 8Tx PUSCH— partial-coherent codebook, Ng=4; Support of codebook-based 8Tx PUSCH— partial-coherent codebook, Ng=4; 40-7-1; Yes; n/a; Codebook-based 8Tx PUSCH— partial-coherent codebook, Ng=4 is not supported; Per FSPC; No; No; No; Optional with capability signaling. |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-7-1b/1c/1d: - Similar as FG 40-7-1a, FG 40-7-1b/1c/1d shall be FGs per FSPC. Proposal 20: Adopt (N1,N2) as a component for 40-7-1a. Proposal 21: The prerequisite FG of FG(s) 40-7-1a, 40-7-1b, 40-7-1c and 40-7-1d is FG 2-12. |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-7-1c; Codebook-based 8Tx PUSCH— [partial-coherent codebook], Ng=4; Support of codebook-based 8Tx PUSCH— [partial-coherent codebook], Ng=4; 40-7-1; Yes; n/a; Codebook-based 8Tx PUSCH— [partial-coherent codebook], Ng=4 is not supported; [Per FSPC]; No; No; No; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-7-1a/1b/1c/1d - Component - Based on current CR, ‘[full-coherent codebook]’ can be replaced by ‘Codebook1’, ‘[partial-coherent codebook], Ng=2’ can be replaced by ‘Codebook2’, ‘[partial-coherent codebook], Ng=4’ can be replaced by ‘Codebook3’, ‘[non-coherent codebook]’ can be replaced by ‘Codebook4’. - For the FFS for 40-7-1a, we prefer to make (N1,N2) as component, instead of new row. For the candidate values for (N1, N2), (4,1) or (2,2) can be reported by UE. There is no need to support ‘both’ in our views. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-7-1c; Codebook-based 8Tx PUSCH— [partial-coherent codebook], Higher-layer parameter CodebookType = “codebook3”; Support of codebook-based 8Tx PUSCH— [partial-coherent codebook], Higher-layer parameter CodebookType = “codebook3”; 40-7-1; Yes; n/a; Codebook-based 8Tx PUSCH— Higher-layer parameter CodebookType = “codebook3” is not supported; Per FS; No; No; No; Optional with capability signaling. |
| Samsung [16] | - FG 40-7-1b/1c/1d: likewise, since the text (partial-coherent and non-coherent) has been used in legacy (Rel.15 UL codebooks), for consistency, it should be included in the description of respective rows. Besides, since Ng=2,4,8 are included in the RRC parameter, the following can be included in respective rows. - FG 40-7-1c: both Ng=4 and partial-coherent |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-7-1c; Codebook-based 8Tx PUSCH [partial-coherent codebook], Ng=4; Support of codebook-based 8Tx PUSCH [partial-coherent codebook], Ng=4; 40-7-1; Yes; n/a; Codebook-based 8Tx PUSCH [partial-coherent codebook], Ng=4 is not supported; [Per FSPC]; No; No; No; Optional with capability signaling. |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-7-1d | Codebook-based 8Tx PUSCH— [non-coherent codebook] | Support of codebook-based 8Tx PUSCH— [non-coherent codebook] | 40-7-1 | Yes | n/a | Codebook-based 8Tx PUSCH— [non-coherent codebook] is not supported | [Per FSPC] | No | No | No |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-7-1d; Codebook-based 8Tx PUSCH— [non-coherent codebook]; Support of codebook-based 8Tx PUSCH— [non-coherent codebook] Ng=8; 40-7-1; Yes; n/a; Codebook-based 8Tx PUSCH— [non-coherent codebook] is not supported; [Per FSPC]; No; No; No; Optional with capability signaling. |
| Ericsson [3] | Regarding the naming of features 40-7-1a/b/c/d, these can be named according to what is used in 38.211 now, as shown below. We note that this is related to the issue of how to define coherence as discussed by the feature lead in the last meeting [6]. However, since the terms full-, partial-, or non-coherent are not used for 8 Tx in the specifications at present, and there are no codebook subsets defined with these terms, they should not be added to UE capabilities unless they are defined. 40-7-1d; Codebook-based 8Tx PUSCH— codebook4 [non-coherent codebook]; 40-7-1. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-7-1d; Codebook-based 8Tx PUSCH— [non-coherent codebook]; Support of codebook-based 8Tx PUSCH— [non-coherent codebook]; 40-7-1; Yes; n/a; Codebook-based 8Tx PUSCH— [non-coherent codebook] is not supported; Per FSPC; No; No; No; Optional with capability signaling. |
| Fujitsu [5] | 40-7-1d; Codebook-based 8Tx PUSCH— [non-coherent codebook]; Support of codebook-based 8Tx PUSCH— [non-coherent codebook]; 40-7-1; The non-coherent UE shall support FG 40-7-1d.. |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-7-1d; Codebook-based 8Tx PUSCH— non-coherent codebook; Support of codebook-based 8Tx PUSCH— non-coherent codebook; 40-7-1; Yes; n/a; Codebook-based 8Tx PUSCH—non-coherent codebook is not supported; Per FSPC; No; No; No; Optional with capability signaling. |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-7-1b/1c/1d: - Similar as FG 40-7-1a, FG 40-7-1b/1c/1d shall be FGs per FSPC. Proposal 20: Adopt (N1,N2) as a component for 40-7-1a. Proposal 21: The prerequisite FG of FG(s) 40-7-1a, 40-7-1b, 40-7-1c and 40-7-1d is FG 2-12. |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-7-1d; Codebook-based 8Tx PUSCH— [non-coherent codebook]; Support of codebook-based 8Tx PUSCH— [non-coherent codebook]; 40-7-1; Yes; n/a; Codebook-based 8Tx PUSCH— [non-coherent codebook] is not supported; [Per FSPC]; No; No; No; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-7-1a/1b/1c/1d - Component - Based on current CR, ‘[full-coherent codebook]’ can be replaced by ‘Codebook1’, ‘[partial-coherent codebook], Ng=2’ can be replaced by ‘Codebook2’, ‘[partial-coherent codebook], Ng=4’ can be replaced by ‘Codebook3’, ‘[non-coherent codebook]’ can be replaced by ‘Codebook4’. - For the FFS for 40-7-1a, we prefer to make (N1,N2) as component, instead of new row. For the candidate values for (N1, N2), (4,1) or (2,2) can be reported by UE. There is no need to support ‘both’ in our views. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-7-1d; Codebook-based 8Tx PUSCH, Higher-layer parameter CodebookType = “codebook4”; Support of codebook-based 8Tx PUSCH— Higher-layer parameter CodebookType = “codebook4”; 40-7-1; Yes; n/a; Codebook-based 8Tx PUSCH— Higher-layer parameter CodebookType = “codebook4” is not supported; Per FS; No; No; No; Optional with capability signaling. |
| Samsung [16] | - FG 40-7-1b/1c/1d: likewise, since the text (partial-coherent and non-coherent) has been used in legacy (Rel.15 UL codebooks), for consistency, it should be included in the description of respective rows. Besides, since Ng=2,4,8 are included in the RRC parameter, the following can be included in respective rows. - FG 40-7-1d: both Ng=8 and non-coherent |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-7-1d; Codebook-based 8Tx PUSCH [non-coherent codebook]; Support of codebook-based 8Tx PUSCH [non-coherent codebook]; 40-7-1; Yes; n/a; Codebook-based 8Tx PUSCH [non-coherent codebook] is not supported; [Per FSPC]; No; No; No; Optional with capability signaling. |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-7-1e | UL full power transmission mode 0 | Support of UL full power transmission mode of fullpower when UE is capable of 8 Tx codebook based PUSCH operation | FFS | yes | n/a | FFS | [Per FSPC] | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-7-1e; UL full power transmission mode 0; Support of UL full power transmission mode of fullpower when UE is capable of 8 Tx codebook based PUSCH operation; FFS; yes; n/a; FFS UL full power transmission mode 0 is not supported; [Per FSPC]; n/a; n/a; n/a; Optional with capability signalling. |
| Ericsson [3] | Regarding the prerequisites for UL FP Tx modes, they all require codebook based operation, and so 40-7-1 should be sufficient as a prerequisite. We note that for Rel-16, the basic codebook based operation 2-14 is a prerequisite as is the PUSCH codebook coherency subset, 2-13. However, there are no coherency subsets for 8 Tx, and so using basic codebook based operation for 8 Tx would be as consistent as possible with the Rel-16 prerequisites. 40-7-1e; UL full power transmission mode 0; Support of UL full power transmission mode of fullpower when UE is capable of 8 Tx codebook based PUSCH operation; FFS 40-7-1. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-7-1e; UL full power transmission mode 0 —partial-coherent codebook, Ng=2; Support of UL full power transmission mode of fullpower when UE is capable of 8 Tx codebook based PUSCH operation with Ng=2; 40-7-1b; yes; n/a; UL full power transmission mode 0 —partial-coherent codebook, Ng=2 is not supported; Per FSPC; n/a; n/a; n/a; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-7-1e-1; UL full power transmission mode 0 —partial-coherent codebook, Ng=4; Support of UL full power transmission mode of fullpower when UE is capable of 8 Tx codebook based PUSCH operation with Ng=4; 40-7-1c; yes; n/a; UL full power transmission mode 0 —partial-coherent codebook, Ng=4 is not supported; Per FSPC; n/a; n/a; n/a; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-7-1e-2; UL full power transmission mode 0 —partial-coherent codebook, Ng=8; Support of UL full power transmission mode of fullpower when UE is capable of 8 Tx codebook based PUSCH operation with Ng=8; 40-7-1d; yes; n/a; UL full power transmission mode 0 —partial-coherent codebook, Ng=8 is not supported; Per FSPC; n/a; n/a; n/a; Optional with capability signalling. |
| Fujitsu [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-7-1e; UL full power transmission mode 0; Support of UL full power transmission mode of fullpower when UE is capable of 8 Tx codebook based PUSCH operation; 40-7-1; yes; n/a; Per FSPC; n/a; n/a; n/a; A UE that supprts FG 40-7-1e must support at least one of FGs 40-7-1b/c/d; Optional with capability signalling. |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-7-1e: - Supporting 8Tx codebook based PUSCH should be the prerequisite of the full power transmission enhancement for Rel-18. Therefore, FG 40-7-1 is applied as the prerequisite FG. - Add the consequence if the feature is not supported by the UE: UL full power transmission mode 0 is not supported. Proposal 22: Adopt the following changes for FG 40-7-1e: - The prerequisite of FG 40-7-1e is FG 40-7-1; - Add the consequence if the feature is not supported by the UE: UL full power transmission mode 0 for 8 Tx codebook based PUSCH operation is not supported. |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-7-1e; UL full power transmission mode 0; Support of UL full power transmission mode of fullpower when UE is capable of 8 Tx codebook based PUSCH operation; FFS 16-5a; yes; n/a; FFS UL full power transmission mode 0 is not supported; [Per FSPC]; n/a; n/a; n/a; Optional with capability signalling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-7-1e - Prerequisite FG: 40-7-1b, 40-7-1c, or 40-7-1d. - Consequence if the feature is not supported: UL full power transmission mode 0 is not supported. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-7-1e; UL full power transmission mode 0; Support of UL full power transmission mode of fullpower when UE is capable of 8 Tx codebook based PUSCH operation; FFS; yes; n/a; FFS; Per FS; n/a; n/a; n/a; Optional with capability signalling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-7-1e; Full power mode 0 for 8 Tx PUSCH; Support of UL full power transmission mode of fullpower when UE is capable of 8 Tx codebook based PUSCH operation; YES; N/A; [Per FSPC]; NO; NO; NO; Optional with capability signaling. |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-7-1f | UL full power transmission mode 1 | Support of UL full power transmission mode of fullpowerMode1 when UE is capable of 8 Tx codebook based PUSCH operation This is a WA | FFS | yes | n/a | FFS | [Per FSPC] | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-7-1f - The working assumption was confirmed The full power mode1 is agreed in RAN1 #113 agreement below. The ”working assmption” is removed Agreement (RAN1#113 Incheon) For full power PUSCH transmission by an 8TX UE, confirm the Working Assumption for Mode1 with updates: - To support full power transmission with Mode1, Rel-16 Mode1 (fullPowerMode1) is re-used. - FFS if more than one of the 8TX full coherent precoders is used. - FFS: identification of precoders per rank / per Ng. 40. NR_MIMO_evo_DL_UL; 40-7-1f; UL full power transmission mode 1; Support of UL full power transmission mode of fullpowerMode1 when UE is capable of 8 Tx codebook based PUSCH operation This is a WA; FFS; yes; n/a; FFS UL full power transmission mode 1 is not supported; [Per FSPC]; n/a; n/a; n/a; Optional with capability signalling. |
| Ericsson [3] | Regarding the prerequisites for UL FP Tx modes, they all require codebook based operation, and so 40-7-1 should be sufficient as a prerequisite. We note that for Rel-16, the basic codebook based operation 2-14 is a prerequisite as is the PUSCH codebook coherency subset, 2-13. However, there are no coherency subsets for 8 Tx, and so using basic codebook based operation for 8 Tx would be as consistent as possible with the Rel-16 prerequisites. 40-7-1f; UL full power transmission mode 1; Support of UL full power transmission mode of fullpowerMode1 when UE is capable of 8 Tx codebook based PUSCH operation This is a WA; FFS 40-7-1. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-7-1f; UL full power transmission mode 1 —partial-coherent codebook, Ng=2; Support of UL full power transmission mode of fullpowerMode1 when UE is capable of 8 Tx codebook based PUSCH operation with Ng=2 This is a WA; 40-7-1b; yes; n/a; UL full power transmission mode 1 —partial-coherent codebook, Ng=2 is not supported; Per FSPC; n/a; n/a; n/a; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-7-1f-1; UL full power transmission mode 1 —partial-coherent codebook, Ng=4; Support of UL full power transmission mode of fullpowerMode1 when UE is capable of 8 Tx codebook based PUSCH operation with Ng=4; 40-7-1c; yes; n/a; UL full power transmission mode 1 —partial-coherent codebook, Ng=4 is not supported; Per FSPC; n/a; n/a; n/a; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-7-1f-2; UL full power transmission mode 1 —partial-coherent codebook, Ng=8; Support of UL full power transmission mode of fullpowerMode1 when UE is capable of 8 Tx codebook based PUSCH operation with Ng=8; 40-7-1d; yes; n/a; UL full power transmission mode 1 —partial-coherent codebook, Ng=8 is not supported; Per FSPC; n/a; n/a; n/a; Optional with capability signalling. |
| Fujitsu [5] | 40-7-1f; UL full power transmission mode 1; Support of UL full power transmission mode of fullpowerMode1 when UE is capable of 8 Tx codebook based PUSCH operation This is a WA; FFS. |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-7-1f; UL full power transmission mode 1; Support of UL full power transmission mode of fullpowerMode1 when UE is capable of 8 Tx codebook based PUSCH operation; 40-7-1; yes; n/a; Per FSPC; n/a; n/a; n/a; A UE that supprts FG 40-7-1f must support at least one of FGs 40-7-1b/c/d; Optional with capability signalling. |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-7-1f: - It is agreed to implement full power transmission mode1 in RAN1 #113 meeting and the corresponding enhancements are adopted in RAN1 #114 meeting. In this case, this UE feature should be supported. Therefore, the wording “This is a WA” should be removed. - Similar to the FG 40-7-1e, the prerequisite feature group should be supporting the 8Tx codebook based PUSCH. - Add the consequence if the feature is not supported by the UE: UL full power transmission mode 1 for 8 Tx codebook based PUSCH operation is not supported. Proposal 23: Adopt the following changes for FG 40-7-1f: - Remove the wording “This is a WA” ; - The prerequisite of FG 40-7-1f is FG 40-7-1; - Add the consequence if the feature is not supported by the UE: UL full power transmission mode 1 for 8 Tx codebook based PUSCH operation is not supported. |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-7-1f; UL full power transmission mode 1; Support of UL full power transmission mode of fullpowerMode1 when UE is capable of 8 Tx codebook based PUSCH operation This is a WA; FFS 16-5b; yes; n/a; FFS UL full power transmission mode 1 is not supported; [Per FSPC]; n/a; n/a; n/a; Optional with capability signalling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-7-1f - Prerequisite FG: 40-7-1b, 40-7-1c, or 40-7-1d. - Component: remove ‘This is a WA’. - Consequence if the feature is not supported: UL full power transmission mode 1 is not supported. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-7-1f; UL full power transmission mode 1; Support of UL full power transmission mode of fullpowerMode1 when UE is capable of 8 Tx codebook based PUSCH operation; FFS; yes; n/a; FFS; Per FS; n/a; n/a; n/a; Optional with capability signalling. |
| Samsung [16] | - FG 40-7-1f/g: the text on WA should be deleted. |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-7-1f; Full power mode 1 for 8 Tx PUSCH; Support of UL full power transmission mode of fullpowerMode1 when UE is capable of 8 Tx codebook based PUSCH operation This is a WA; YES; N/A; [Per FSPC]; NO; NO; NO; Optional with capability signaling. |
| Huawei/HiSilicon [19] | For full power transmission, we have following proposal: Proposal 5-12: - Removing “This is a WA” for FGs 40-7-1f and 40-7-1g. - Support component 2 of 40-7-1g - Add a component for reporting of TPMI groups for full power mode 2. 40. NR_MIMO_evo_DL_UL; 40-7-1f; UL full power transmission mode 1; Support of UL full power transmission mode of fullpowerMode1 when UE is capable of 8 Tx codebook based PUSCH operation This is a WA; FFS; yes; n/a; FFS; [Per FSPC]; n/a; n/a; n/a; Optional with capability signalling. |

| 40. NR_MIMO_evo_DL_UL | 40-7-1g | UL full power transmission mode 2 | 1. Support of UL full power transmission mode of fullpowerMode2 when UE is capable of 8 Tx codebook based PUSCH operation [2. Maximum number of SRS resources in one SRS resource set with usage set to 'codebook' for 8Tx codebook based PUSCH for Mode 2] This is a WA | FFS | yes | n/a | FFS | [Per FSPC] | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - For FG 40-7-1g - The working assumption was confirmed The full power mode1 is agreed in RAN1 #113 agreement below. The ”working assmption” is removed - Maximum number of SRS resources in one SRS resource set is based on the agreement RAN1 #114 below. Agreement (RAN1#113 Incheon) For full power PUSCH transmission by an 8TX UE, confirm the Working Assumption for Mode2 with updates: - To support full power transmission with Mode2, Rel-16 Mode2 (fullPowerMode2) is re-used. - FFS definition of precoder groups (G0, G1, …) - FFS enhancements for SRS configuration. Agreement (RAN1#114 Toulouse) For an 8TX UE, configured for full power transmission with ‘fullpowerMode2’, - Subject to UE capability, a maximum of 2 or 4 SRS resources are supported in an SRS resource set with usage set to 'codebook', - An SRS resource set can be configured with one or more of 1-, 2-, 4-, or 8-port SRS resources.. 40. NR_MIMO_evo_DL_UL; 40-7-1g; UL full power transmission mode 2; 1. Support of UL full power transmission mode of fullpowerMode2 when UE is capable of 8 Tx codebook based PUSCH operation [2. Maximum number of SRS resources in one SRS resource set with usage set to 'codebook' for 8Tx codebook based PUSCH for Mode 2] This is a WA; FFS; yes; n/a; FFS UL full power transmission mode 2 is not supported; [Per FSPC]; n/a; n/a; n/a; Candidate values for component 2: {1, 2, 4, 8}; Optional with capability signalling. |
| Ericsson [3] | Regarding the prerequisites for UL FP Tx modes, they all require codebook based operation, and so 40-7-1 should be sufficient as a prerequisite. We note that for Rel-16, the basic codebook based operation 2-14 is a prerequisite as is the PUSCH codebook coherency subset, 2-13. However, there are no coherency subsets for 8 Tx, and so using basic codebook based operation for 8 Tx would be as consistent as possible with the Rel-16 prerequisites. 40-7-1g; UL full power transmission mode 2; 1. Support of UL full power transmission mode of fullpowerMode2 when UE is capable of 8 Tx codebook based PUSCH operation [2. Maximum number of SRS resources in one SRS resource set with usage set to 'codebook' for 8Tx codebook based PUSCH for Mode 2] = 2 or 4 This is a WA; FFS 40-7-1. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-7-1g; UL full power transmission mode 2 —partial-coherent codebook, Ng=2; 1. Support of UL full power transmission mode of fullpowerMode2 when UE is capable of 8 Tx codebook based PUSCH operation with Ng=2 [2. Maximum number of SRS resources in one SRS resource set with usage set to 'codebook' for 8Tx codebook based PUSCH for Mode 2] This is a WA; 40-7-1b; yes; n/a; UL full power transmission mode 2 —partial-coherent codebook, Ng=2 is not supported; Per FSPC; n/a; n/a; n/a; Optional with capability signalling. |
| Fujitsu [5] | 40-7-1g; UL full power transmission mode 2; 1. Support of UL full power transmission mode of fullpowerMode2 when UE is capable of 8 Tx codebook based PUSCH operation [2. Maximum number of SRS resources in one SRS resource set with usage set to 'codebook' for 8Tx codebook based PUSCH for Mode 2] This is a WA; FFS. |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-7-1g; UL full power transmission mode 2; 1. Support of UL full power transmission mode of fullpowerMode2 when UE is capable of 8 Tx codebook based PUSCH operation 2. Maximum number of SRS resources in one SRS resource set with usage set to 'codebook' for 8Tx codebook based PUSCH for Mode 2: {1,2,4}; 40-7-1; yes; n/a; Per FSPC; n/a; n/a; n/a; A UE that supprts FG 40-7-1g must support at least one of FGs 40-7-1b/c/d; Optional with capability signalling. |
| Intel Corporation [7] |  |
| OPPO [8] | Based on the following agreements and legacy full power mode 2 design in Rel-16, we have some proposals for UE feature for 8Tx codebook-based transmission, e.g. full power mode 2. Agreement For an 8TX UE, configured for full power transmission with ‘fullpowerMode2’ for Ng=2 - UE power capability is indicated per antenna group, where for an indicated group, full power is supported for all ranks - For when Ng=2, a single bit is used to indicate which of the antenna group has full power capability. Agreement For an 8TX UE, configured for full power transmission with ‘fullpowerMode2’, - Subject to UE capability, a maximum of 2 or 4 SRS resources are supported in an SRS resource set with usage set to 'codebook', - An SRS resource set can be configured with one or more of 1-, 2-, 4-, or 8-port SRS resources. Conclusion - Full power transmission with ‘fullpowerMode2’ with TPMI capability reporting when Ng=4 is not supported in Rel-18 - Full power transmission with ‘fullpowerMode2’ with TPMI capability reporting when Ng=8 is not supported in Rel-18 40. NR_MIMO_evo_DL_UL; 40-7-1g; UL full power transmission mode 2; 1. Support of UL full power transmission mode of fullpowerMode2 when UE is capable of 8 Tx codebook based PUSCH operation 2. Maximum number of SRS resources in one SRS resource set with usage set to 'codebook' for 8Tx codebook based PUSCH for Mode 2; 40-7-1; yes; n/a; FFS; [Per FSPC]; n/a; n/a; n/a; Component 2 candidate values: {1,2,4} A UE that supports FG 40-7-1g supports at least full power operation with single port; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-7-1g-1; UL full power transmission mode 2 for 8Tx – SRS resources; The SRS configuration with different number of antenna ports per SRS resource for Mode 2 with 8Tx codebook; 40-7-1g; yes; n/a; FFS; [Per FSPC]; n/a; n/a; n/a; candidate values: {1_8, 1_2_8, 1_4_8,1_2_4_8} 1st state (1_8): each SRS resource can be configured with 1 port or 8 ports 2nd state (1_2_8): each SRS resource can be configured with 1 port or 2 ports or 8 ports 3rd state (1_4_8): each SRS resource can be configured with 1 port or 4 ports or 8 ports 4rd state (1_2_4_8): each SRS resource can be configured with 1 port or 2 ports or 4 ports or 8 ports Note: The four states can be used if 40-7-1g is reported as 2 or 4 for maximum number of SRS resources in one SRS resource set.; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-7-1g-2; UL full power transmission fullpowerMode2 – full power for codebook2; indicate which of the two antenna port groups has full power capability; 40-7-1g; yes; n/a; FFS; [Per FSPC]; n/a; n/a; n/a; candidate values: {1st, 2nd}; Optional with capability signalling. |
| CATT [9] | 40-7-1g: - It is agreed to implement full power transmission mode2 in RAN1 #113 meeting and the corresponding enhancements are adopted in RAN1 #114 meeting. In this case, this UE feature should be supported. Therefore, the wording “This is a WA” should be removed. - It is agreed that a maximum of 2 or 4 SRS resource can be applied for full power mode2 enhancement, therefore, such candidate values should be adopted. Similar to FG 16-5c-2, such feature can be captured as a separate FG. - Similar to the FG 40-7-1e, supporting the 8Tx codebook based PUSCH should be one of the prerequisite feature groups. Besides, maximal number of SRS resources in one SRS in Rel-16 is re-used as the values for that in Rel-18, thus the basic feature of full power mode2 in Rel-16 should also be one prerequisite feature group. - Add the consequence if the feature is not supported by the UE: UL full power transmission mode 2 is not supported. Proposal 24: Adopt the following changes for FG 40-7-1g: - Remove the wording “This is a WA” ; - The prerequisite of FG 40-7-1g are FG 40-7-1 and FG 16-5c; - Add the consequence if the feature is not supported by the UE: UL full power transmission mode 2 is not supported for 8 Tx codebook based PUSCH operation; - Remove component 2 and add a separate FG for SRS configuration with different number of antenna ports per SRS resource for full power mode2. |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-7-1g; UL full power transmission mode 2; 1. Support of UL full power transmission mode of fullpowerMode2 when UE is capable of 8 Tx codebook based PUSCH operation [2. Maximum number of SRS resources in one SRS resource set with usage set to 'codebook' for 8Tx codebook based PUSCH for Mode 2] This is a WA; FFS 16-5c; yes; n/a; FFS UL full power transmission mode 2 is not supported; [Per FSPC]; n/a; n/a; n/a; Component 2 candidate values: {1,2,3,4}; Optional with capability signalling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-7-1g - Prerequisite FG: 40-7-1b. - Component: - Remove ‘This is a WA’. - Support the 2nd component, and its candidate values include {1,2,4}. - Consequence if the feature is not supported: UL full power transmission mode 2 is not supported for Codebook2. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-7-1g; UL full power transmission mode 2; 1. Support of UL full power transmission mode of fullpowerMode2 when UE is capable of 8 Tx codebook based PUSCH operation 2. Maximum number of SRS resources in one SRS resource set with usage set to 'codebook' for 8Tx codebook based PUSCH for Mode 2; FFS; yes; n/a; FFS; Per FS; n/a; n/a; n/a; Component 2 candidate values: {1, 2, 4}; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-7-1h; UL full power transmission mode of fullpowerMode2 for 8 Tx – SRS resource; The supported SRS configurations with different number of antenna ports per SRS resource for full power Mode 2 when UE is capable of and configured with 8 Tx codebook based PUSCH operation; Per FS; Component 1 candidate values: {1_8, 1_2_8, 1_4_8, 1_2_4_8} 1st state (1_8): each SRS resource can be configured with 1 port or 8 ports. 2nd state (1_2_8): each SRS resource can be configured with 1 port or 2 ports or 8 ports. 3rd state (1_4_8): each SRS resource can be configured with 1 port or 4 ports or 8 ports. 4th state (1_2_4_8): each SRS resource can be configured with 1 port or 2 port or 4 ports or 8 ports.; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-7-1i; UL full power transmission mode of fullpowerMode2 for 8 Tx – full power TPMI groups, Higher-layer parameter CodebookType = “codebook2”; - TPMI group(s) which delivers full power when UE is capable of and configured with 8 Tx codebook based PUSCH operation, Higher-layer parameter CodebookType = “codebook2”; Per FS; Component 1 candidate values: {first coherent antenna port group, second coherent antenna port group}; Optional with capability signaling. |
| Samsung [16] | - FG 40-7-1f/g: the text on WA should be deleted. - FG 40-7-1g: re component 2, we can include a value 8 in addition to legacy values {1,2,4}. |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-7-1g; Full power mode 2 for 8 Tx PUSCH; 1. Support of UL full power transmission mode of fullpowerMode2 when UE is capable of 8 Tx codebook based PUSCH operation 2. Maximum number of SRS resources in one SRS resource set with usage set to 'codebook' for 8Tx codebook based PUSCH for Mode 2 This is a WA FFS how to indicate full power precoders under full power mode 2.; YES; N/A; [Per FSPC]; NO; NO; NO; Optional with capability signaling. |
| Huawei/HiSilicon [19] | For full power transmission, we have following proposal: Proposal 5-12: - Removing “This is a WA” for FGs 40-7-1f and 40-7-1g. - Support component 2 of 40-7-1g - Add a component for reporting of TPMI groups for full power mode 2. 40. NR_MIMO_evo_DL_UL; 40-7-1g; UL full power transmission mode 2; 1. Support of UL full power transmission mode of fullpowerMode2 when UE is capable of 8 Tx codebook based PUSCH operation [2. Maximum number of SRS resources in one SRS resource set with usage set to 'codebook' for 8Tx codebook based PUSCH for Mode 2] 3. Support of TPMI reporting for UE power capability for Ng=2 This is a WA; FFS; yes; n/a; FFS; [Per FSPC]; n/a; n/a; n/a; Candidates of component 2: {2, 4} Candidate of component 3: {G1, G2}, where G1 are TPMIs with antenna ports #0,#1,#4,#5 non-zero power, and G2 are TPMIs with antenna ports #2,#3,#6,#7 non-zero power.; Optional with capability signalling. |

| 40. NR_MIMO_evo_DL_UL | 40-7-2 | Basic features for Non-Codebook-based 8Tx PUSCH | 1. Supported maximal PUSCH MIMO layers for non-codebook based PUSCH 2. Supported maximum number of SRS resources per SRS resource set with usage set to 'nonCodebook’ 3. Maximum number of simultaneous transmitted SRS resources at one symbol | FFS | Yes | n/a | Non-codebook based 8Tx PUSCH is not supported | [Per FSPC] | No | No | No |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-7-2; Basic features for Non-Codebook-based 8Tx PUSCH; 1. Supported maximal PUSCH MIMO layers for non-codebook based PUSCH 2. Supported maximum number of SRS resources per SRS resource set with usage set to 'nonCodebook’ 3. Maximum number of simultaneous transmitted SRS resources at one symbol; FFS; Yes; n/a; Non-codebook based 8Tx PUSCH is not supported; [Per FSPC]; No; No; No; Optional with capability signaling. |
| Ericsson [3] | For non-codebook based 8 Tx 40-7-2, similar to Rel-16 non-codebook based feature, we can use basic PUSCH transmission 2-12 as the prerequisite. 40-7-2; Basic features for Non-Codebook-based 8Tx PUSCH; 1. Supported maximal PUSCH MIMO layers for non-codebook based PUSCH 2. Supported maximum number of SRS resources per SRS resource set with usage set to 'nonCodebook’ 3. Maximum number of simultaneous transmitted SRS resources at one symbol; FFS 2-12. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-7-2; Basic features for Non-Codebook-based 8Tx PUSCH; 1. Supported maximal number of PUSCH MIMO layers for non-codebook based PUSCH 2. Supported maximum number of SRS resources per SRS resource set with usage set to 'nonCodebook’ 3. Maximum number of simultaneous transmitted SRS resources at one symbol; Yes; n/a; Non-codebook based 8Tx PUSCH is not supported; Per FSPC; No; No; No; Optional with capability signaling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] | 40-7-2: - The prerequisite feature group of FG 40-7-2 shall be the basic UE features for non-codebook based PUSCH transmission, i.e., FG 2-15. - Similar as that for FG 2-15, for FG 40-7-2, the candidate values of component-1 can be {1, 2, 4, 8}, and the candidate values of component-3 can be {1, 2, 3, 4, 5, 6, 7, 8}. - Similar as that for FG 2-15, FG 40-7-2 can be a FG per FSPC. Proposal 27: Adopt the following for FG 40-7-2: - The prerequisite feature group of FG 40-7-2 is FG 2-15; - The candidate values of component-1are {1, 2, 4, 8}; - The candidate values of component-3 are {1, 2, 3, 4, 5, 6, 7, 8}; - FG 40-7-2 is a FG per FSPC. |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-7-2; Basic features for Non-Codebook-based 8Tx PUSCH; 1. Supported maximal PUSCH MIMO layers for non-codebook based PUSCH 2. Supported maximum number of SRS resources per SRS resource set with usage set to 'nonCodebook’ 3. Maximum number of simultaneous transmitted SRS resources at one symbol; FFS 2-15; Yes; n/a; Non-codebook based 8Tx PUSCH is not supported; [Per FSPC]; No; No; No; Component 1 candidate values: {1,2,3,4,5,6,7,8} Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: {1,2,3,4,5,6,7,8}; Optional with capability signaling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-7-2 - Prerequisite FG: 2-15 (NCB based PUSCH). - Component - For component 1, candidate values are {1,2,3,4,5,6,7,8}. - For component 2, candidate values are {1,2,3,4,5,6,7,8}. - For component 3, candidate values are {1,2,3,4,5,6,7,8}. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-7-2; Basic features for Non-Codebook-based 8Tx PUSCH; 1. Supported maximal PUSCH MIMO layers for non-codebook based PUSCH 2. Supported maximum number of SRS resources per SRS resource set with usage set to 'nonCodebook’ 3. Maximum number of simultaneous transmitted SRS resources at one symbol; FFS; Yes; n/a; Non-codebook based 8Tx PUSCH is not supported; Per FSPC; No; No; No; Component 1 candidate values: {1, 2, [3, ]4, [5, 6, 7, ]8} Component 2 candidate values: {5, 6, 7, 8} Component 3 candidate values: {1, 2, 3, 4, 5, 6, 7, 8}; Optional with capability signaling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-7-2; Basic features for Non-Codebook-based 8Tx PUSCH; 1. Supported maximal PUSCH MIMO layers for non-codebook based PUSCH 2. Supported maximum number of SRS resources per SRS resource set with usage set to 'nonCodebook’ 3.Maximum number of simultaneous transmitted SRS resources at one symbol; YES; N/A; [Per FSPC]; NO; NO; NO; Component 1 candidate values: {1,2,3,4,5,6,7,8}; Optional with capability signaling. |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-7-2a | Association between CSI-RS and SRS for non-codebook case | 1. Support association between NZP-CSI-RS and SRS resource set via RRC parameter "SRS-ResourceSet" for noncodebook 8Tx PUSCH operation [2. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources, and total # of Tx ports} across all CCs simultaneously.] | FFS | yes | n/a | Association between CSI-RS and SRS for non-codebook case is not supported | [Per FSPC] | No | No | No |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-7-2a; Association between CSI-RS and SRS for non-codebook case; 1. Support association between NZP-CSI-RS and SRS resource set via RRC parameter "SRS-ResourceSet" for noncodebook 8Tx PUSCH operation [2. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources, and total # of Tx ports} across all CCs simultaneously.]; FFS 2-15; yes; n/a; Association between CSI-RS and SRS for non-codebook case is not supported; [Per FSPC]; No; No; No; Candidate values for component 2: The candidate values for the max # of Tx port in one resource is {2, 4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports is: {from 2 to 256}; Optional with capability signalling. |
| Ericsson [3] |  |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-7-2a; Association between CSI-RS and SRS for non-codebook case; 1. Support association between NZP-CSI-RS and SRS resource set via RRC parameter "SRS-ResourceSet" for noncodebook 8Tx PUSCH operation [2. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources, and total # of Tx ports} across all CCs simultaneously.]; 40-7-2; yes; n/a; Association between CSI-RS and SRS for non-codebook case is not supported; Per FSPC; No; No; No; Optional with capability signalling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-7-2a; Association between CSI-RS and SRS for non-codebook case; 1. Support association between NZP-CSI-RS and SRS resource set via RRC parameter "SRS-ResourceSet" for noncodebook 8Tx PUSCH operation [2. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources, and total # of Tx ports} across all CCs simultaneously.]; FFS 2-15a; yes; n/a; Association between CSI-RS and SRS for non-codebook case is not supported; [Per FSPC]; No; No; No; Optional with capability signalling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-7-2a - Prerequisite FG: 40-7-2, 2-15a. - Component: - OK to keep component 2. The candidate value of component 2 refers to FG 2-15a: Maximum size of the list is 16. The candidate values for the max # of Tx port in one resource is {2, 4, 8, 12, 16, 24, 32}. The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports is: {from 2 to 256}. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-7-2a; Association between CSI-RS and SRS for non-codebook case; 1. Support association between NZP-CSI-RS and SRS resource set via RRC parameter "SRS-ResourceSet" for noncodebook 8Tx PUSCH operation 2. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources, and total # of Tx ports} across all CCs simultaneously.; FFS; yes; n/a; Association between CSI-RS and SRS for non-codebook case is not supported; Per band; No; No; No; Component 2 candidate value: Maximum size of the list is 16. The candidate values for the max # of Tx port in one resource is {2, 4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {1 to 64} The candidate value set of total # of ports is: {2 to 256}; Optional with capability signalling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-7-2a; Association between CSI-RS and 8Tx SRS; 1. Support association between NZP-CSI-RS and SRS resource set via RRC parameter "SRSresoureset" 2. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously.; YES; N/A; [Per FSPC]; NO; NO; NO; Component-2: Maximum size of the list is 16. the candidate values for the max # of Tx port in one resource is {2, 4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports is: {from 2 to 256}; Optional with capability signaling. |
| Huawei/HiSilicon [19] |  |

| 40. NR_MIMO_evo_DL_UL | 40-7-3 | CBG based 2 CWs PUSCH with rank >4 | Support CBG based transmission for 2 CWs PUSCH | FFS | yes | n/a | CBG based transmission for 2 CWs PUSCH is not supported | [Per FSPC] | No | No | No |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 40. NR_MIMO_evo_DL_UL; 40-7-3; CBG based 2 CWs PUSCH with rank >4; Support CBG based transmission for 2 CWs PUSCH; FFS; yes; n/a; CBG based transmission for 2 CWs PUSCH is not supported; [Per FSPC]; No; No; No; Optional with capability signalling. |
| Ericsson [3] |  |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-7-3; CBG based 2 CWs PUSCH with rank >4; Support CBG based transmission for 2 CWs PUSCH; 40-7-2; yes; n/a; CBG based transmission for 2 CWs PUSCH is not supported; Per FSPC; No; No; No; Optional with capability signalling. |
| Fujitsu [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| OPPO [8] |  |
| CATT [9] |  |
| Xiaomi [10] | 40. NR_MIMO_evo_DL_UL; 40-7-3; CBG based 2 CWs PUSCH with rank >4; Support CBG based transmission for 2 CWs PUSCH; FFS 40-7-1, 40-7-2; yes; n/a; CBG based transmission for 2 CWs PUSCH is not supported; [Per FSPC]; No; No; No; Optional with capability signalling. |
| LG Electronics [11] |  |
| CMCC [12] |  |
| Google [13] |  |
| NTT DOCOMO, INC. [14] | 40-7-3 - Prerequisite FG: 40-7-1 or 40-7-2. In addition, for all the FGs above, the granularity could be per FSPC. We also support to add new FGs for SRS resource for full power mode 2, and full power TPMI group indication for codebook2. 40. NR_MIMO_evo_DL_UL; 40-7-1g-1; UL full power transmission mode of fullpowerMode2 for 8 Tx – SRS resource; 1. The supported SRS configurations with different number of antenna ports per SRS resource for full power Mode 2 for 8 Tx codebook based PUSCH operation; 40-7-1g; yes; n/a; [Per FSPC]; Component 1 candidate values: {(1, 8), (2, 8), (4, 8)}; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-7-1g-2; UL full power transmission mode of fullpowerMode2 – full power TPMI groups; - TPMI group(s) which delivers full power; 40-7-1g; yes; n/a; TPMI group(s) for UL full power transmission mode 2 for 8 Tx codebook based PUSCH operation is not supported; [Per FSPC]; n/a; n/a; n/a; Component 1 candidate values: {first antenna group, second antenna group}; Optional with capability signalling. |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-7-3; CBG based 2 CWs PUSCH with rank >4; Support CBG based transmission for 2 CWs PUSCH; FFS; yes; n/a; CBG based transmission for 2 CWs PUSCH is not supported; Per FS; No; No; No; Optional with capability signalling. |
| Samsung [16] |  |
| MediaTek Inc. [17] |  |
| Qualcomm Incorporated [18] | 40. NR_MIMO_evo_DL_UL; 40-7-3; CBG based 2 CWs PUSCH with rank >4; 1.Support CBG based transmission for 2 CWs PUSCH; YES; N/A; [Per FSPC]; NO; NO; NO; Optional with capability signaling. |
| Huawei/HiSilicon [19] |  |

Other

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | - The following agreement about TCI state(s) selection for PUSCH, including dynamic switching from single-TRP to multi-TRP is not really explicitly covered in any FG. This indication leverage existing field but the operation itself would need to be considered as a UE feature Agreement (RAN1#112 Athens) On unified TCI framework extension for S-DCI based MTRP, when two SRS resource sets for CB/NCB are configured, support the followings for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 (including DG and Type2 CG): - If the DCI format 0_1/0_2 indicates codepoint "00" for the existing SRS resource set indicator, the UE shall apply the first indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If the DCI format 0_1/0_2 indicates codepoint "01" for the existing SRS resource set indicator, the UE shall apply the second indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If the DCI format 0_1/0_2 indicates codepoint "10" or “11” for the existing SRS resource set indicator: - For TDM based PUSCH Tx scheme, the UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set for CB/NCB (note: the association between an SRS resource set for CB/NCB and PUSCH transmission occasions(s) is defined according to TS 38.214) - FFS: SDM and SFN based PUSCH Tx schemes FFS: The case that the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission. Observation: The dynamic switching from single-TRP to multi-TRP is not explicitly covered in any FG for the uTCI framework extension for S-DCI-based MTRP when two SRS resource sets for CB/NCB are configured to support PUSCH transmission scheduled by DCI format 0_1/0_2. - For FG 40-3-1-24 - Minimum CSI calculation delay requirements for AP-CSI reporting for Rel-16/17-based CJT codebooks. Additional delay parameter r applicable to Z2 and Z'2. FFS: candidate values Agreement(RAN1#113) For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding Z/Z’: - For NTRP=1: reuse legacy Z/Z’ values - For NTRP>1, introduce two UE capabilities: - Capability 1: Reuse legacy Z/Z’ values - Capability 2: Legacy Z/Z’ values + r - The value(s) of r>0 can depend on the configured NTRP value - FFS: exact value(s) of r Note: Since this pertains Type-II, the relevant values are Z2/Z2’. 40. NR_MIMO_evo_DL_UL; 40-3-1-24; Additional minimum CSI calculation delay requirements for AP-CSI reporting for Rel-16/17-based CJT codebooks; Minimum delay requirements are: (Z, Z’) = (Z2+Z'2, 2Z'2); Yes; N/A; Additional minimum CSI calculation delay requirements for AP-CSI reporting for Rel-16/17-based CJT codebooks not supported; No; N/A; N/A; If reported, the minimum delay requirements are (Z2+Z'2, 2Z'2), otherwise, they are (Z2, Z'2); Optional with capability signaling. - For FG 40-3-1-25 - Number of CPUs occupied by a CMR for Rel-16/17-based CJT codebooks. It follows from the agreement Agreement(RAN1#113) For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding the CPU occupation: OCPU = X.NTRP where - X≥1 when NTRP>1, is defined based on UE capabilities and determined by the UE - FFS: Whether the supported value(s) of X are common or can depend on the value of NTRP, NL, total sum of {Ln}, and/or other CJT features (e.g. dynamic TRP selection) - The legacy specification on CPU pools is fully reused - Note: When NTRP=1 is configured, legacy OCPU applies, i.e. OCPU =1. 40. NR_MIMO_evo_DL_UL; 40-3-1-25; Number of CPUs occupied by a CMR for Rel-16/17-based CJT codebooks; A CMR occupies X CPUs; Yes; N/A; Number of CPUs occupied by a CMR for Rel-16/17-based CJT codebooks not supported; No; N/A; N/A; Candidate values: {1,1.5,2}; Optional with capability signaling. We proposed the following new FGs: - For FG 40-3-2-8 - Number of CPUs occupied by an AP-CMR, for Rel-16/17-based Doppler codebooks. Each of the K AP-CMRs occupies Y1 CPUs 40. NR_MIMO_evo_DL_UL; 40-3-2-8; Number of CPUs occupied by an AP-CMR, for Rel-16/17-based Doppler codebooks; Each of the K AP-CMRs occupies Y1 CPUs; Yes; N/A; No; N/A; N/A; Candidate values: {2/3, 1, 2, 3}; Optional with capability signaling. - For FG 40-3-2-9 - Number of CPUs occupied by a P- or SP-CMR, with N4>1, for Rel-16/17-based Doppler codebooks. A P- or SP-CMR occupies Y2*N4 CPUs, for N4>1 40-3-2-8/9 follow from the agreement on the CPU occupancy: Agreement(RAN1#113) Previous agreement is revised as follows For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the CPU occupation: OCPU = Y.N4 [+4] when P/SP-CSI-RS is configured for CMR, or OCPU = Y.K when AP-CSI-RS is configured for CMR - Y≥1 is defined based on UE capabilities and determined by the UE, and can be different between P/SP-CSI-RS and AP-CSI-RS. - FFS: Whether the supported value(s) of Y can depend on codebook parameter values - The legacy specification on CPU pools is fully reused - When N4=1, OCPU =4 - OCPU ≥ 4 when P/SP-CSI-RS is configured for CMR. 40. NR_MIMO_evo_DL_UL; 40-3-2-9; Number of CPUs occupied by a P- or SP-CMR, with N4>1, for Rel-16/17-based Doppler codebooks; A P- or SP-CMR occupies Y2*N4 CPUs, for N4>1; Yes; N/A; No; N/A; N/A; Candidate values: {2/3, 1, 2, 3}; Optional with capability signaling. - For FG 40-3-2-10 - Multiplying factor, Kp, for counting active P- or SP-CSI-RS resources/ports, for Rel-16/17-based Doppler codebooks Agreement(RAN1#113) For the Rel-18 Type-II codebook refinement for high/medium velocities, the value of KP for P/SP-CSI-RS active resource counting is determined based on UE capability, where the candidate values are {1, 2, 4}.. 40. NR_MIMO_evo_DL_UL; 40-3-2-10; Multiplying factor, Kp, for counting active P- or SP-CSI-RS resources/ports, for Rel-16/17-based Doppler codebooks; An active P- or SP-CSI-RS resource is counted Kp times; Yes; N/A; No; N/A; N/A; Candidate values: {1,2,4}; Optional with capability signaling. - For FG 40-3-2-11 - Additional Z2 delay for AP-CSI reporting on AP-CSI-RS resources, for Rel-16/17-based Doppler codebooks. Additional delay parameter m applicable to Z2. FFS: candidate values 40. NR_MIMO_evo_DL_UL; 40-3-2-11; Additional Z2 delay for AP-CSI reporting on AP-CSI-RS resources, for Rel-16/17-based Doppler codebooks; Additional delay parameter m applicable to Z2; Yes; N/A; No; N/A; N/A; FFS: candidate values; Optional with capability signaling. - For FG 40-3-2-12 - Additional Z2 delay for AP-CSI reporting on P- or SP-CSI-RS resources, for Rel-16/17-based Doppler codebooks. Additional delay parameter w applicable to Z2. FFS: candidate values 40. NR_MIMO_evo_DL_UL; 40-3-2-12; Additional Z2 delay for AP-CSI reporting on P- or SP-CSI-RS resources, for Rel-16/17-based Doppler codebooks; Additional delay parameter w applicable to Z2; Yes; N/A; No; N/A; N/A; FFS: candidate values; Optional with capability signaling. - For FG 40-3-2-13 - Additional Z’2 delay for AP-CSI reporting with N4>1, for Rel-16/17-based Doppler codebooks. Additional delay parameter r applicable to Z’2. FFS: candidate values 40. NR_MIMO_evo_DL_UL; 40-3-2-13; Additional Z2 and Z’2 delay for AP-CSI reporting with N4>1, for Rel-16/17-based Doppler codebooks; Additional delay parameter r applicable to (Z2, Z’2); Yes; N/A; No; N/A; N/A; If reported r= Z'2, otherwise r=0; Optional with capability signaling. 40-3-2-11/12/13 follow from the agreements: Agreement(RAN1#113) For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z/Z’ - For N4=1: reuse legacy Z’ values - For N4>1, introduce two UE capabilities: - Capability 1: Reuse legacy Z’ values - Capability 2: Legacy Z’ values + r - The value(s) of r>0 can depend on the configured N4 value - FFS: exact value(s) of r Note: Since this pertains Type-II, the relevant values are Z2/Z2’ Agreement(RAN1#113) For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z - Based on the two UE capabilities agreed for Z’: - Capability 1: - For AP CSI-RS: Z=legacy Z+14.(K–1).m - For P/SP CSI-RS: Z= legacy Z+w where w>0 - TBD: Value of w - Capability 2: - For AP CSI-RS: Z= legacy Z+14.(K–1).m + r - For P/SP CSI-RS: Z= legacy Z+w+r - Note: r corresponds to the agreed value for Z’ relaxation in previous agreement. Note: Since this pertains Type-II, the relevant values are Z2/Z2’. - For FG 40-4-4a - Based on the following agreements in RAN1 #114bis and #114, we proposed a new FG for ”Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports for FDM scheme” Agreement(RAN1#114bis Xiamen) Introduce a separate UE capability to report the orphan RE capability (i.e. UE can receive PDSCH without the scheduling restriction for FD-OCC length 4 in Rel.18 eType 1 DMRS) for PDSCH with fdmSchemeA or fdmSchemeB. Agreement (RAN1#114 Toulouse) If UE does not sSupport the orphan RE capability (i.e. UE can receive PDSCH without the scheduling restriction for FD-OCC length 4 in Rel.18 eType 1 DMRS), all the following scheduling restriction is applied for PDSCH transmission with fdmSchemeA or fdmSchemeB: - 1) The number of consecutively scheduled PRBs for PDSCH for each TCI-state is even. - If the precoding granularity is set to ‘wideband’, the total number of PRBs allocated to UE should be multiple of 4 to ensure the number of consecutively scheduled PRBs for PDSCH for each TCI-state is even. - 2) The number of PRBs offset of scheduled PDSCH for each TCI-state from point A (common resource block 0) is even.. 40. NR_MIMO_evo_DL_UL; 40-4-4a; Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports for FDM scheme; Support reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports for fdmSchemeA or fdmSchemeB; 40-4-1, 40-4-4; Yes; n/a; Reception of PDSCH with fdmSchemeA or fdmSchemeB without the scheduling restriction for Rel.18 eType1 DMRS ports is not supported; [Per FS]; No; No; n/a; Note: If this feature is not supported, UE expects that gNB shall apply at least the following scheduling restriction for PDSCH for FD-OCC 4 in Rel.18 eType 1 DMRS - 1) The number of consecutively scheduled PRBs for PDSCH for each TCI-state is even. - If the precoding granularity is set to ‘wideband’, the total number of PRBs allocated to UE should be multiple of 4 to ensure the number of consecutively scheduled PRBs for PDSCH for each TCI-state is even. 2) The number of PRBs offset of scheduled PDSCH for each TCI-state from point A (common resource block 0) is even.; Optional with capability signaling. - For FG 40-4-11: Rel-18 DMRS and PDSCH processing capability 2 simultaneously - Based on the following agreement in RAN1 #114bis we proposed a new FG Agreement(RAN1#114bis Xiamen) Introduce a UE feature group to indicate the whether/how to support Rel-18 DMRS and PDSCH processing capability 2 simultaneously - In this feature group, the UE can additionally report relaxation on processing delay for PDSCH processing capability 2 - FFS: Details of the relaxation on processing delay. 40. NR_MIMO_evo_DL_UL; 40-4-11; Rel-18 DMRS and PDSCH processing capability 2 simultaneously; Support for Rel-18 DMRS and PDSCH processing capability 2 simultaneously Support for relaxation on processing delay for PDSCH processing capability 2; Yes; n/a; Rel-18 DMRS and PDSCH processing capability 2 are not supported simultaneously; No; No; n/a; {d0 (no additional relaxation), d1, d2, d3, d4}. * dn: additional n symbol delay; Optional with capability signaling. |
| Ericsson [3] | The unified TCI framework in Rel-17 supports inter-cell beam management. The corresponding support should be added for the Rel-18 extension, both for single-DCI based multi-TRP and multi-DCI-based multi-TRP. Note that inter-cell operation requires multiple activated TCI states, which is captured in the corresponding pre-requisite FG. 40-1-1d; Unified TCI with joint DL/UL TCI update for single-DCI based inter-cell multi-TRP with single activated TCI codepoint per CC; - Support of unified TCI with joint DL/UL TCI update for single-DCI based inter-cell multi-TRP. - Support K additional MAC-CE activated joint TCI states per CC in a band - Support K additional MAC-CE activated joint TCI states across all CC(s) in a band; 40-1-1; Unified TCI with joint DL/UL TCI update for single-DCI based inter-cell multi-TRP with multiple activated TCI codepoints per CC is not supported; Per band. 40-1-2d; Unified TCI with separate DL/UL TCI update for single-DCI based inter-cell multi-TRP with single activated TCI codepoint per CC; - Support of unified TCI with separate DL/UL TCI update for single-DCI based inter-cell multi-TRP. - Support K additional MAC-CE activated DL TCI states per CC in a band - Support K additional MAC-CE activated DL TCI states across all CC(s) in a band; 40-1-2; Unified TCI with separate DL/UL TCI update for single-DCI based inter-cell multi-TRP is not supported; Per band. 40-1-7d; Unified TCI with joint DL/UL TCI update for multi-DCI based inter-cell multi-TRP with single activated TCI codepoint per CC; - Support of unified TCI with joint DL/UL TCI update for multi-DCI based inter-cell multi-TRP. - Support K additional MAC-CE activated joint TCI states per CC in a band - Support K additional MAC-CE activated joint TCI states across all CC(s) in a band; 40-1-7; Unified TCI with joint DL/UL TCI update for single-DCI based inter-cell multi-TRP with multiple activated TCI codepoints per CC is not supported; Per band. 40-1-9d; Unified TCI with separate DL/UL TCI update for multi-DCI based inter-cell multi-TRP with single activated TCI codepoint per CC; - Support of unified TCI with separate DL/UL TCI update for multi-DCI based inter-cell multi-TRP. - Support K additional MAC-CE activated DL TCI states per CC in a band - Support K additional MAC-CE activated DL TCI states across all CC(s) in a band; 40-1-9; Unified TCI with separate DL/UL TCI update for multi-DCI based inter-cell multi-TRP is not supported; Per band. There is a need to add the counterpart to FG 40-1-3: 40-1-8; Per aperiodic CSI-RS resource/resource set configuration for TCI selection in mDCI based mTRP; Support of per aperiodic CSI-RS resource/resource set configuration for TCI selection in mDCI based mTRP; Per band; Component candidate values: {per resource, per resource set, both}. The extended unified TCI framework supports all schemes specified in Rel-16 and Rel-17. The features introduced so far describe how the TCI states are updated. For example, the FG 40-1-1 describes this for single-DCI based multi-TRP. However, it is not clear which Rel-16/Rel-17 multi-TRP schemes are supported with the unified TCI framework. There are essentially two options: A UE that supports single-DCI based multi-TRP (FG 40-1-1x, FG 40-1-2x) support all the single-DCI schemes from Rel-16/Rel-17, provided that the corresponding Rel-16/Rel-17 feature is supported, i.e., - NC-JT (corresponding Rel-16 feature: FG 16-2b-x) - PDCCH repetition (corresponding Rel-17 feature: FG 23-2-1x ) - PUSCH repetition (corresponding Rel-17 feature: FG 23-3-1x) - PUCCH repetition (corresponding Rel-17 feature: FG 23-3-2x) - HST-SFN (corresponding Rel-17 feature: FG 23-6-1x) - mTRP BFR (corresponding Rel-17 feature: FG 23-5-2x) The UE must report support for each Rel-16/Rel-17 feature in combination with the unified TCI framework - We note that option 2 has been implemented for mTRP BFR (FG 40-1-1b), but not for any of the other schemes. Overall, our preference is option 1. With option 1, we should clarify in the description of FG 40-1-1, or in a note, that the corresponding Rel-16/Rel-17 feature is supported together with the unified TCI framework. As already mentioned, cross-TRP PDCCH ordered PRACH transmission for inter-cell should have its own feature group: 40-2-4a; Support of a PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different inter-cell TRP based on CFRA; Support of cross-TRP PDCCH order based on CFRA for inter-cell multi-DCI based mTRP; Inter-cell cross-TRP PDCCH ordered PRACH transmission is not supported; Per band. In RAN1#113, RAN1 made the following agreement: Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, for the case when the UE does not support UL STxMP transmission, - for the baseline feature, the UE does not expect the two UL transmissions to overlap (i.e., scheduling restriction is applied to avoid overlap between the two UL transmissions) - as an optional feature, the overlapping duration of the later of the two UL transmissions is reduced. - FFS: for the optional feature, whether or not the overlapping duration needs to be specified as 1 (in case 2) or 2 (in case 1) OFDM symbols where - Case 1 applies when UE is capable of supporting MRTD > CP, SCS=60 kHz and frequency range is FR1. Case 2 applies in all other cases We propose to add the new FG 40-2-9 according to this agreement: Allow scheduling overlapping UL transmissions for UEs not supporting STxMP. 40-2-9; Support dropping the overlapping part of the later of two overlapping UL transmissions for UEs not supporting STxMP; Allow two overlapping UL transmissions, in which case the UE drops the overlapping part of the later transmission.; The UE does not expect to be scheduled with overlapping UL transmissions; Per band. In RAN1#113, it was agreed to support hopping over a subset of cyclic shifts and comb offsets, as a UE-optional feature: Agreement (RAN1#113) Support configuring a subset of comb offsets when comb offset hopping is configured, and configuring a subset of cyclic shifts when cyclic shift hopping is configured. - The subset configuration applies to all the port(s) in the SRS resource, and all the port(s) in the SRS resource has (have) the same hopping offset value khopping(pi) on an OFDM symbol. - This is a UE-optional feature. Two new FGs are needed for the above agreement, one for comb offset hopping and one for cyclic shift hopping. In RAN1#113, it was further agreed to support finer-granularity cyclic-shift hopping, as a UE-optional feature: Agreement (RAN1#113) For SRS cyclic shift hopping, support finer time-delay-domain granularity, e.g., αi=2πnSRScs,inSRScs,max+2πnSRScs,offsetK×nSRScs,max, where nSRScs,offset can be randomly chosen from 0,1,…K×nSRScs,max-1 at each SRS transmission. - Note: The finer granularity above only applies to the cyclic shift offsets when cyclic shift hopping is enabled. If a subset for cyclic shifts is configured, this feature cannot be configured. Above is a UE optional feature. A new FG is needed to support finer-granularity cyclic-shift hopping, with FG 40-5-2 as a prerequisite. Below, we list the new FG, including those that were identified in Section 2.3.2.1 and Section 2.3.2.2. 40. NR_MIMO_evo_DL_UL; 40-5-1c; Hopping within a subset for SRS comb offset hopping; Support for comb offset hopping within a configured subset of comb offsets; 40-5-1; Yes; n/a; Comb offset hopping within a configured subset of comb offsets is not supported; Per band; n/a; n/a; n/a; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-5-2c; Hopping within a subset for SRS cyclic shift hopping; Support cyclic shift for hopping within a configured subset of cyclic shifts; 40-5-2; Yes; n/a; Cyclic shift for hopping within a configured subset of cyclic shifts is not supported; Per band; n/a; n/a; n/a; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-5-2d; Finer-granularity SRS cyclic shift hopping; Supported comb offset hopping granularity in time when repetition factor R>1 is configured; 40-5-2; Yes; n/a; Finer-granularity SRS cyclic shift hopping is not supported; Per band; n/a; n/a; n/a; Optional with capability signaling. |
| ZTE [4] | 40. NR_MIMO_evo_DL_UL; 40-3-1-24; Z/Z’ for CJT type-II codebook; For NTRP>1, support one of capabilities for Z/Z’ for CJT type-II codebook.; 40-3-1-1, 40-3-1-5; Yes; N/A; Z/Z’ for CJT type-II codebook for NTRP>1 is NOT reported; Per-band and per BC; N/A; N/A; N/A; Component candidate values: {Capability 1, Capability 2} Capability 1: Reuse legacy Z/Z’ values Capability 2: Legacy Z/Z’ values + legacy Z’ Note: For NTRP=1: reuse legacy Z/Z’ values; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-2-8; Support of the number of AP-CSI-RS resources for the CMR, i.e., K, equal to 12; For the number of AP-CSI-RS resources for the CMR, i.e., K, we have an individual UE feature of supporting K=12.; 40-3-2-1, 40-3-2-4; Yes; N/A; The number of AP-CSI-RS resources for the CMR, i.e., K, equal to 12, is not supported; Per band and per BC; N/A; N/A; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-2-9; Support of L=6 for Rel-16-based doppler codebook; Support of L=6 for Rel-16-based doppler codebook; 40-3-2-1; Yes; N/A; L=6 for Rel-16-based doppler codebook is not supported; Per band and per BC; N/A; N/A; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-2-10; Support of RANK =3 and 4 for Rel-16-based doppler codebook; Support of RANK =3 and 4 for Rel-16-based doppler codebook; 40-3-2-1; Yes; N/A; RANK =3 and 4 for Rel-16-based doppler codebook is not supported; Per band and per BC; N/A; N/A; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-2-11; Support of RANK =3 and 4 for Rel-17-based doppler codebook; Support of RANK =3 and 4 for Rel-17-based doppler codebook; 40-3-2-1; Yes; N/A; RANK =3 and 4 for Rel-17-based doppler codebook is not supported; Per band and per BC; N/A; N/A; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-2-12; Z/Z’ for doppler codebook; For N4>1, support one of capabilities for Z/Z’ for Doppler Type-II codebook:; 40-3-2-1, 40-3-2-4; Component candidate values: {Capability 1, Capability 2} Capability 1: - Regarding Z - For AP CSI-RS: Z=legacy Z+14.(K–1).m - For P/SP CSI-RS: Z= legacy Z+w - Reuse legacy Z’ values Capability 2: - Regarding Z, - For AP CSI-RS: Z= legacy Z+14.(K–1).m + legacy Z’ - For P/SP CSI-RS: Z= legacy Z+w+ legacy Z’ - Z’ = Legacy Z’ values + legacy Z’ values Note: For N4=1: reuse legacy Z’ values. 40. NR_MIMO_evo_DL_UL; 40-3-2-12a; Support of w for Z for doppler codebook in Capability 2; Support of w for Z for doppler codebook in Capability 2; 40-3-1-12; Component candidate values: {4, 8, 14, 28} symbols. - For FG 40-4-5b, it was added according to the following agreement reached in RAN1#114bis meeting. Agreement Introduce a separate UE capability to report the orphan RE capability (i.e. UE can receive PDSCH without the scheduling restriction for FD-OCC length 4 in Rel.18 eType 1 DMRS) for PDSCH with fdmSchemeA or fdmSchemeB.. 40. NR_MIMO_evo_DL_UL; 40-4-5b; Reception of single DCI based MTRP PDSCH in FDMSchemeA or FDMSchemeB without the scheduling restriction for Rel.18 eType1 DMRS ports; Support reception of single DCI based MTRP PDSCH in FDMSchemeA or FDMSchemeB without the scheduling restriction for Rel.18 eType1 DMRS ports; 40-4-1, 16-2b-2, 16-2b-3; Yes; n/a; Reception of single DCI based MTRP PDSCH in FDMSchemeA or FDMSchemeB without the scheduling restriction for Rel.18 eType1 DMRS ports is not supported; [Per FS] Per FS; No; No; n/a; Note: If this feature is not supported, UE shall assume that the number of consecutively scheduled PRBs for PDSCH for each TCI-state is even, and the offset of the scheduled PRB from common resource block 0 for PDSCH for each TCI-state is even number.; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-5-2c; SRS cyclic shift hopping with finer granularity; Support of SRS cyclic shift hopping with finer granularity with K = 2 Note: K is the parameter determining the cyclic shift hopping granularity; 40-5-2; Yes; n/a; Per band; n/a; n/a; n/a; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-5-2d; SRS cyclic shift hopping within a cyclic shift subset; Support of SRS cyclic shift hopping within a cyclic shift subset; 40-5-2; Yes; n/a; Per band; n/a; n/a; n/a; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-5-1c; Comb offset hopping within a comb offset subset; Support of comb offset hopping within a comb offset subset; 40-5-1; Yes; n/a; Per band; n/a; n/a; n/a; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-7-1h; Support the configuration of different number of SRS ports in different SRS resources; 1. Maximum number of SRS resources in one SRS resource set with usage set to 'codebook' for 8Tx codebook based PUSCH for full power transmission mode 2; 40-7-1g; yes; n/a; Per FSPC; n/a; n/a; n/a; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-7-1i; Support the antenna group with full power capability.; 1. Support of first or second antenna group with full power capability; 40-7-1g; yes; n/a; Per FSPC; n/a; n/a; n/a; Component candidate values: {first antenna group, second antenna group}; Optional with capability signalling. |
| Fujitsu [5] | For another issue on CPU occupation, we prefer to define a new FG only for the number of CPUs, which is also similar as Rel-17 NCJT. Regarding the following agreement in the previous meetings, OCPU = X.NTRP and X ={1, 1.5, 2}. Thus, the separate FG 40-3-1-1b for mode 2 and FG 40-3-1-5b for mode 1 can be considered. Agreement @#113 For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding the CPU occupation: OCPU = X.NTRP where - X≥1 when NTRP>1, is defined based on UE capabilities and determined by the UE - FFS: Whether the supported value(s) of X are common or can depend on the value of NTRP, NL, total sum of {Ln}, and/or other CJT features (e.g. dynamic TRP selection) - The legacy specification on CPU pools is fully reused - Note: When NTRP=1 is configured, legacy OCPU applies, i.e. OCPU =1 Agreement@#114 For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding the CPU occupation, - X is a “common value” and not dependent on NTRP or any other parameter value, reported by the UE from a set of candidate values {1, 1.5, 2} according to UE capability. 40-3-1-1b; Support of X for the number of CPUs for mode 2 based CJT type-II codebook; Number of CPUs X* N_TRP occupied by CMRs within one set for CJT CSI hypotheses; 40-3-1-1; Component candidate values for X: {1,1.5,2}. 40-3-1-5b; Support of X for the number of CPUs for mode 1 based CJT type-II codebook; Number of CPUs X* N_TRP occupied by CMRs within one set for CJT CSI hypotheses; 40-3-1-5; Component candidate values for X: {1,1.5,2}. In RAN1 #113 meeting, there was the following agreement. Agreement For an 8TX UE, Option 1 is supported, - Option 1 – Subject to its capability, an 8TX UE may report more than one Ng value, based on which, gNB may RRC configure UE with a codebook corresponding to only one of the supported Ng values.. Based on the agreement, the UE can support more than one Ng value. However, this is not reflected in the latest UE feature list on 8Tx UL. In legacy spec, the full coherent UE can also support the partial coherent and non-coherent precoders. The partial coherent UE can support non-coherent precoders. The same scheme could be applied for UE with 8Tx. For example, for full coherent UE, the UE can support full coherent precoders, partial coherent precoders with two antenna groups and four antenna groups, and non-coherent precoders. For partial coherent UE with two antenna groups, it can support partial coherent precoders with two antenna groups and four antenna groups, and non-coherent precoders. For partial coherent UE with four antenna groups, it can support partial coherent precoders with four antenna groups, and non-coherent precoders. For non-coherent UE, it can only support non-coherent precoders. In addition, in RAN1 #114 meeting, the following agreement and conclusions were reached regarding full power Mode 2 operation. Agreement For an 8TX UE, configured for full power transmission with ‘fullpowerMode2’ for Ng=2 - UE power capability is indicated per antenna group, where for an indicated group, full power is supported for all ranks - For when Ng=2, a single bit is used to indicate which of the antenna group has full power capability. Conclusion - Full power transmission with ‘fullpowerMode2’ with TPMI capability reporting when Ng=4 is not supported in Rel-18 - Full power transmission with ‘fullpowerMode2’ with TPMI capability reporting when Ng=8 is not supported in Rel-18. Therefore, a new row (40-7-1h) should be added for full power Mode 2 operation, which is 1 bit to indicate which antenna group can deliver full power for partial coherent UE with Ng=2. 40-7-1h; UL full power transmission fullpowerMode2 – full power TPMI groups; TPMI group(s) which delivers full power; 40-7-1g; For partial coherent UE with two antenna groups, 1 bit is reported to indicate which antenna group can support full power. For full coherent UE, the reporting is the same as partial coherent UE with two antenna groups.. |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-3-1-24; Applied Z/Z’ values when N4>1 for Rel-16 based Type II refinement for CJT; Support of Z/Z’ capability 1 when N4>1 for Rel-16 based Type II refinement for CJT; 40-3-1-1; Yes; N/A; Only support capability 2; Per-band and per BC; N/A; N/A; N/A; Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding Z/Z’: - For NTRP=1: reuse legacy Z/Z’ values - For NTRP>1, introduce two UE capabilities: - Capability 1: Reuse legacy Z/Z’ values - Capability 2: Legacy Z/Z’ values + r - The value(s) of r>0 can depend on the configured NTRP value - FFS: exact value(s) of r Note: Since this pertains Type-II, the relevant values are Z2/Z2’; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-1-24a; Applied Z/Z’ values when N4>1 for Rel-17 based Type II refinement for CJT; Support of Z/Z’ capability 1 when N4>1 for Rel-17 based Type II refinement for CJT; 40-3-1-1; Yes; N/A; Only support capability 2; Per-band and per BC; N/A; N/A; N/A; Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding Z/Z’: - For NTRP=1: reuse legacy Z/Z’ values - For NTRP>1, introduce two UE capabilities: - Capability 1: Reuse legacy Z/Z’ values - Capability 2: Legacy Z/Z’ values + r - The value(s) of r>0 can depend on the configured NTRP value - FFS: exact value(s) of r Note: Since this pertains Type-II, the relevant values are Z2/Z2’; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-2-8; Applied Z/Z’ values when N4>1 for Rel-16 based Type II refinement for Doppler; Support of Z/Z’ capability 1 when N4>1 for Rel-16 based Type II refinement for Doppler; 40-3-2-1; Yes; N/A; Only support capability 2; Per-band and per BC; N/A; N/A; N/A; Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z/Z’ - For N4=1: reuse legacy Z’ values - For N4>1, introduce two UE capabilities: - Capability 1: Reuse legacy Z’ values - Capability 2: Legacy Z’ values + r - The value(s) of r>0 can depend on the configured N4 value - FFS: exact value(s) of r Note: Since this pertains Type-II, the relevant values are Z2/Z2’; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-1-8a; Applied Z/Z’ values when N4>1 for Rel-17 based Type II refinement for Doppler; Support of Z/Z’ capability 1 when N4>1 for Rel-17 based Type II refinement for Doppler; 40-3-2-1; Yes; N/A; Only support capability 2; Per-band and per BC; N/A; N/A; N/A; Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z/Z’ - For N4=1: reuse legacy Z’ values - For N4>1, introduce two UE capabilities: - Capability 1: Reuse legacy Z’ values - Capability 2: Legacy Z’ values + r - The value(s) of r>0 can depend on the configured N4 value - FFS: exact value(s) of r Note: Since this pertains Type-II, the relevant values are Z2/Z2’; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-4-4a; Reception of PDSCH for fdmSchemeA and fdmSchemeB without the scheduling restriction for Rel.18 eType1 DMRS ports; Support reception of PDSCH for fdmSchemeA and fdmSchemeB without the scheduling restriction for Rel.18 eType1 DMRS ports; 40-4-1; Yes; n/a; Reception of PDSCH for fdmSchemeA and fdmSchemeB without the scheduling restriction for Rel.18 eType1 DMRS ports is not supported; [Per FS]; No; No; n/a; Note: If this feature is not supported, UE expects that gNB shall apply both the following scheduling restriction for PDSCH for fdmSchemeA and fdmSchemeB for FD-OCC 4 in Rel.18 eType 1 DMRS 1) The number of consecutively scheduled PRBs for PDSCH is even for each TCI state 2) The number of PRBs offset of each set of consecutively scheduled PDSCH from point A (common resource block 0) is even for each TCI state; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-5-2c; The subset of cyclic shifts when cyclic shift hopping is configured; Support of the subset of cyclic shifts when cyclic shift hopping is configured; 40-5-2; Yes; n/a; The subset of cyclic shifts when cyclic shift hopping is configured is not supported; Per band; n/a; n/a; n/a; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-5-2d; Finer time-delay-domain granularity for SRS cyclic shift hopping; Support of finer time-delay-domain granularity for SRS cyclic shift hopping; 40-5-2; Yes; n/a; Finer time-delay-domain granularity for SRS cyclic shift hopping is not supported; Per band; n/a; n/a; n/a; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-7-1h; UL full power transmission mode 2 – SRS resource; 1. The SRS configuration with different number of antenna ports per SRS resource for Mode 2; 40-7-1, 40-7-1g; Per FSPC; n/a; n/a; n/a; Note: when 2 or 4 is reported for 40-7-1g Component candidate values: {1_2, 1_4, 1_8, 2_4, 2_8, 1_2_4_8}; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-7-1i; UL full power transmission mode 2 – full power TPMI group; 1. TPMI group(s) which delivers full power; 40-7-1, 40-7-1b,40-7-1g; Per FSPC; n/a; n/a; n/a; Component candidate values: 1 bit {‘0’ indicates TPMIs corresponding to one port group, ‘1’ indicates TPMIs corresponding to another port group }; Optional with capability signaling. |
| Intel Corporation [7] |  |
| OPPO [8] | When RAN1 enters into maintenance phase, there were a few of pending issues on unified TCI framework extension. When enough progress has been made, we could step forward to handle the following potential issues on UE capability. - For S-DCI MTRP, study the UE capability on dynamic switching between Rel-17 TCI framework and Rel-18 TCI framework - For STxMP, study UE capability on panel-specific UE-configured maximum output power - Note per panel or per indicated UL/joint TCI states maximum output power is supported in RAN4 Proposal 1: Considered above-mentioned UE capability change and remaining issues for Rel-18 unified TCI state extension. In previous RAN1 meeting, the following agreement was made with key part highlighted. To support such an optional feature, we should establish a separate row for it. Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, for the case when the UE does not support UL STxMP transmission, - for the baseline feature, the UE does not expect the two UL transmissions to overlap (i.e., scheduling restriction is applied to avoid overlap between the two UL transmissions) - as an optional feature, the overlapping duration of the later of the two UL transmissions is reduced. - FFS: for the optional feature, whether or not the overlapping duration needs to be specified as 1 (in case 2) or 2 (in case 1) OFDM symbols where - Case 1 applies when UE is capable of supporting MRTD > CP, SCS=60 kHz and frequency range is FR1. - Case 2 applies in all other cases 40. NR_MIMO_evo_DL_UL; 40-2-7; Support of the overlapping duration reduction of the later of two UL transmissions; Support of the overlapping duration reduction of the later of two UL transmissions when the UE doesn’t support UL STxMP Tx; yes; N/A; [Per band]; No; No; N/A; Optional with capability signaling. Proposal 2: Considered and reported via UE capability (if supported) for Rel-18 two TA enhancement. 40. NR_MIMO_evo_DL_UL; 40-4-10; DMRS port configuration for PUSCH with rank 5-8 and Rel-15 DMRS; DMRS port configuration for PUSCH with rank 5-8 and 8Tx for Rel 15 DMRS; [2-16]; Yes; n/a; DMRS port configuration for PUSCH with rank 5-8 and Rel-15 DMRS is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-4-11; DMRS port configuration for PUSCH with rank 5-8 and Rel-18 DMRS; DMRS port configuration for PUSCH with rank 5-8 and 8Tx for Rel 18 DMRS; [40-4-6, 40-4-10]; Yes; n/a; DMRS port configuration for PUSCH with rank 5-8 and Rel-18 DMRS is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-4-12; Rel-18 UL DMRS with StxMP; Support of Single DCI based STx2P SDM/SFN and multi-DCI based STx2P PUSCH+PUSCH with Rel-18 DMRS; 40-4-6, FFS more; Yes; n/a; Single DCI based STx2P SDM/SFN and multi-DCI based STx2P PUSCH+PUSCH with Rel-18 DMRS is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. Based on the following agreements, we add three new FG for Rel-18 SRS. Agreement Support configuring a subset of comb offsets when comb offset hopping is configured, and configuring a subset of cyclic shifts when cyclic shift hopping is configured. - The subset configuration applies to all the port(s) in the SRS resource, and all the port(s) in the SRS resource has (have) the same hopping offset value khopping(pi) on an OFDM symbol. - This is a UE-optional feature. Agreement For SRS cyclic shift hopping, support finer time-delay-domain granularity, e.g., , where nSRScs,offset can be randomly chosen from at each SRS transmission. - Note: The finer granularity above only applies to the cyclic shift offsets when cyclic shift hopping is enabled. If a subset for cyclic shifts is configured, this feature cannot be configured. Above is a UE optional feature. Agreement Adopt the text proposal for TS38.214 on allowing 2 SP SRS resource sets for 8T8R per UE capability: 6.2.1.2 UE sounding procedure for DL CSI acquisition <Unchanged text is omitted> - For 1T=1R, or 2T=2R, 4T=4R or 8T=8R, up to two SRS resource sets each with one SRS resource can be configured, where the number of SRS ports for each resource is equal to 1, 2, 4 or 8 if the UE is not indicating srs-AntennaSwitching2SP-1Periodic. Two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' can be configured and the two SRS resource sets configured with 'semi-persistent' are not activated at the same time, or up to two SRS resource sets can be configured, if the UE is indicating srs-AntennaSwitching2SP-1Periodic or [srs-AntennaSwitching2SP-1Periodic8T8R], where each SRS resource set has one SRS resource, the number of SRS ports for each resource is equal to 1, 2, 4[, or 8] or <Unchanged text is omitted>. Proposal 4: The following can be applied to UE feature for SRS enhancement. 40. NR_MIMO_evo_DL_UL; 40-5-1c; SRS comb offset hopping with subset; Support a subset of comb offsets for comb offset hopping; 40-5-1; Yes; n/a; A subset of comb offsets for comb offset hopping is not supported; Per band; n/a; n/a; n/a; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-5-2c; SRS cyclic shift hopping with finer time-delay-domain granularity; Support finer time-delay-domain granularity for cyclic shift hopping; 40-5-2; Yes; n/a; finer time-delay-domain granularity for cyclic shift hopping is not supported; Per band; n/a; n/a; n/a; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-5-8; Maximum 2 SP and 1 periodic SRS sets for 8T8R antenna switching; Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for 8T8R antenna switching; 40-5-4, 40-5-6; Yes; n/a; maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for 8T8R antenna switching is not supported; Per band; n/a; n/a; n/a; Note: - If UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS. - The two SP-SRS resource sets are not activated at the same time; Optional with capability signaling. Based on the agreement in RAN1#114 meeting, a UE capability can be introduced to support enabling single-DCI based STxMP SFN scheme for PUCCH and the Rel-17 repetition number parameter pucch-RepetitionNrofSlots in a same PUCCH resource at the same time. We think this feature can be a separate row from FG 40-6-4. Agreement Introduce one RRC parameter in PUCCH-config to configure STxMP SFN scheme. When this RRC parameter is configured: - When two indicated TCI states are applied to one PUCCH resource, the STxMP SFN scheme is enabled to this PUCCH resource. - When one TCI state is applied to one PUCCH resource, the sTRP transmission is enabled to this PUCCH resource. - It is subject to UE capability to support enabling Rel-18 STxMP SFN scheme and the Rel-17 repetition number parameter pucch-RepetitionNrofSlots in a same PUCCH resource at the same time. When this RRC parameter is not configured: - When two indicated TCI states are applied to one PUCCH resource, the Rel-17 TDM scheme with unified TCI states is enabled to this PUCCH resource. - When one TCI state is applied to one PUCCH resource, the sTRP transmission is enabled to this PUCCH resource.. According to the agreement in RAN1#113 meeting, following capability can be introduced to support group-based beam reporting. For FG 40-6-5, support component 1 and component 2 for group-based beam reporting. Agreement To enhance the Rel-17 group-based beam L1-RSRP reporting to support STxMP-based transmission, support the system to configure the UE to report one of the followings: - Alt1: For each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously, and the reported pair of CRIs or SSBRIs can be received simultaneously. - Alt2: For each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously. - Supporting Alt1 and/or Alt2 is subject to UE capability. Proposal 5: The following can be applied to UE feature for UL precoding indication for multi-panel transmission. - Add a separate row FG 40-6-4a to support enabling Single-DCI based STx2P SFN scheme for PUCCH and Slot based dynamic PUCCH repetition in a same PUCCH resource at the same time. 40. NR_MIMO_evo_DL_UL; 40-6-4a; Single-DCI based STx2P SFN scheme for PUCCH; Support enabling Single-DCI based STx2P SFN scheme for PUCCH and Slot based dynamic PUCCH repetition in a same PUCCH resource at the same time.; 40-6-4; Yes; n/a; Single-DCI based STx2P SFN scheme for PUCCH and slot based dynamic PUCCH repetition in a same PUCCH resource is not supported; Per FS; n/a; FR2 only; n/a; Optional with capability signaling. |
| CATT [9] | 40-7-1g-1: - Similar to FG 16-5c-2, we propose to add a separate feature regarding the SRS configurations for UL full power transmission mode 2 to the UE feature list. - It is agreed that one or more of 1-, 2-, 4-, or 8-port SRS resources in one SRS resource set can be applied for full power mode2 enhancement, therefore, such enhancement should be captured. Proposal 25: Adopt the following UE feature FG 40-7-1g-1: 40. NR_MIMO_evo_DL_UL; 40-7-1g-1; UL full power transmission fullpowerMode2 – SRS resources; The SRS configuration with different number of antenna ports per SRS resource for Mode 2; 40-7-1g; yes; n/a; Configure different number of antenna ports per SRS resource for full power Mode 2 is not supported.; [Per FSPC]; n/a; n/a; n/a; Component candidate value: {at least support one 8-port SRS resource in one SRS resrouce set, other values FFS}; Optional with capability signalling. 40-7-1g-2: - For full power transmission mode2, it was agreed that TPMI capability reporting is supported for Ng = 2 and a single bit is used to indicate which of the antenna group has full power capability. Therefore, similar to FG 16-5c-3, a new feature on TPMI groups which delivers full power shall be specified for full power transmission mode2. The prerequisite feature groups for the new added feature can be FG 40-7-1b and 40-7-1g. Proposal 26: Adopt the following UE feature FG 40-7-1g-2: 40. NR_MIMO_evo_DL_UL; 40-7-1g-2; UL full power transmission fullpowerMode2 – full power TPMI groups; TPMI group(s) which delivers full power for Ng=2; 40-7-1b, 40-7-1g; yes; n/a; TPMI group(s) reporting for UL full power transmission mode 2 for 8 Tx codebook based PUSCH operation is not supported; Per FSPC; n/a; n/a; n/a; Component candidate values: {a single bit is used to indicate which of the antenna group for Ng=2 has full power capability}; Optional with capability signalling. |
| Xiaomi [10] | Refer to TS 38.306[2], the following two UE feature were defined that UE need to indicate whether the UE supports dynamic switching between single-TRP and PDSCH SFN scheme A/B by TCI state field in DCI formats 1_1 and 1_2. But in Rel-18, a new DCI field, i.e., [TCI selection field] will be introduced into DCI formats 1_1 and 1_2 to indicate the TCI state for PDSCH. Thus it is necessary to define a new UE capability to indicate whether the UE supports dynamic switching between single-TRP and PDSCH SFN scheme A/B by [TCI selection field] in DCI formats 1_1 and 1_2. sfn-SchemeA-DynamicSwitching-r17 Indicates whether the UE supports dynamic switching between single-TRP and PDSCH SFN scheme A by TCI state field in DCI formats 1_1 and 1_2. The UE supporting this feature shall indicate sfn-SchemeA-r17 or sfn-SchemeA-PDSCH-only-r17.; FS; No; N/A; N/A. sfn-SchemeB-DynamicSwitching-r17 Indicates whether the UE supports dynamic switching between single-TRP and PDSCH SFN scheme B by TCI state field in DCI formats 1_1 and 1_2. The UE supporting this feature shall indicate sfn-schemeB-r17 or sfn-schemeB-PDSCH-only-r17.; FS; No; N/A; N/A. Proposal 2-3: Define a new UE capability to indicate whether the UE supports dynamic switching between single-TRP and PDSCH SFN scheme A/B by [TCI selection field] in DCI formats 1_1 and 1_2. 40. NR_MIMO_evo_DL_UL; 40-1-14a; Dynamic switching - scheme A; Support of dynamic switching between single-TRP and PDSCH SFN scheme A by [TCI selection field] in DCI formats 1_1, 1_2; 40-1-4, 23-6-1a; Yes; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-1-14b; Dynamic switching - scheme B; Support of dynamic switching between single-TRP and PDSCH SFN scheme B by [TCI selection field] in DCI formats 1_1, 1_2; 40-1-4, 23-6-2a; Yes; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-2-9; Overlap between the two UL transmissions; 1. Support of reducing the overlapping duration of the later of the two UL transmissions when the UE does not support UL STxMP transmission.; 40-2-1, 40-2-2; yes; n/a; Per band; n/a; n/a; n/a; Optional with capability signalling. According to the following agreement, the number of supported SD basis combination can be reported based on UE capability. Agreement (RAN1-112 meeting) On the Type-II codebook refinement for CJT mTRP, for Rel-16-based refinement, support at least the following combinations of {Ln} for the higher-layer-configured value of NTRP (FFS by RAN1#112: whether the bracketed permutations are also supported): - FFS by RAN1#112: whether other combinations can be supported FFS (by RAN1#112bis-e): Whether/how the supported combinations of {n} for Rel-17-based refinement are derived from the supported combinations of {Ln} for Rel-16-based refinement FFS: Whether the total number of Ln is a UE capability NTRP; {Ln} combination. 1; {2}. {4}. {6} (analogous to legacy, only for total # ports =32, rank 1-2, R=1. 2; {2,2}. {2,4}, [{4,2}]. {4,4}. 3; {2,2,2}. {2,2,4} [and its other permutations]. {4,4,4}. 4; {2,2,2,2}. {2,2,2,4} [and its other permutations]. {2,2,4,4} [and its other permutations]. {4,4,4,4}. Proposal 4-1: define a UE capability to indicate the total number of supported SD basis combinations. 40. NR_MIMO_evo_DL_UL; 40-3-1-24; The maxmum total number of supported SD basis for M-TRP CJT; The maxmum total number of supported SD basis for M-TRP CJT. According to the following agreement, the supported subset of linkages can be reported based on UE capability. Agreement (RAN1-112b e-meeting) On the Parameter Combination of Type-II codebook refinement for CJT mTRP, only the following linkages are supported (marked ‘x’), for Rel-16 eType-II based - For NTRP =1, - fully reuse seven out of the eight Parameter Combinations from Rel-16 eType-II as indicated in the table below - FFS (by RAN1#112bis-e): whether to add one more Parameter Combination for L=4 based on the legacy Rel-16 eType-II FD combo {½, ½, ¼, ¼; ½} or the agreed FD combo {½, ½, ½, ½; ½}, or not to add from the indicated seven below - For NTRP >1, only the following linkages are supported (marked ‘x’) - Note: Configured linkage(s) are associated with the configured value of NTRP, regardless whether the dynamic TRP selection (the dynamic change of N given NTRP) is configured. Also, the configured linkage(s) are valid for any dynamically selected SD basis and/or any dynamically selected CSI-RS resource (TRP). - FFS: UE feature/capability to support only a subset of linkages NTRP; SD combo; FD combo {pv},. {1/8, 1/8, 1/16, 1/16}, ¼; {1/8, 1/8, 1/16, 1/16}, ½; {1/4, ¼, 1/8, 1/8}, ¼; {1/4, ¼, 1/8, 1/8}, ½; {1/4, ¼, ¼, ¼}, ¾; {1/2, ½, ½, ½}, ½. 1; 2; x; x. 4; x; x; x. 6 w/ restriction; x; x. 2; {2,2}; x. {2,4} {4,2}; x. {4,4}; x; x; x. 3; {2,2,2}; x; x. {2,2,4} {2,4,2} {4,2,2}; x; x. {4,4,4}; x; x; x; x; x; x. 4; {2,2,2,2}; x; N/A. {2,2,2,4}; x; N/A. {2,2,4,4}; x; x; N/A. {4,4,4,4}; x; x; x; N/A. Proposal 4-2: define a UE capability to indicate the supported subset of linkages. 40. NR_MIMO_evo_DL_UL; 40-3-1-25; The supported subset of linkages; The supported subset of linkages. According to the following agreement, the supported value of Z/Z’ can be reported based on UE capability for NTRP>1. Agreement(RAN1-113 meeting) For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding Z/Z’: - For NTRP=1: reuse legacy Z/Z’ values - For NTRP>1, introduce two UE capabilities: - Capability 1: Reuse legacy Z/Z’ values - Capability 2: Legacy Z/Z’ values + r - The value(s) of r>0 can depend on the configured NTRP value - FFS: exact value(s) of r Note: Since this pertains Type-II, the relevant values are Z2/Z2’ Proposal 4-3: define a UE capability to indicate the supported value of Z/Z’ for NTRP>1. 40. NR_MIMO_evo_DL_UL; 40-3-1-26; The supported value of Z/Z’ for NTRP>1; The supported value of Z/Z’ for NTRP>1. Agreement(RAN1-113 meeting) For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding the CPU occupation: OCPU = X.NTRP where - X≥1 when NTRP>1, is defined based on UE capabilities and determined by the UE - FFS: Whether the supported value(s) of X are common or can depend on the value of NTRP, NL, total sum of {Ln}, and/or other CJT features (e.g. dynamic TRP selection) - The legacy specification on CPU pools is fully reused - Note: When NTRP=1 is configured, legacy OCPU applies, i.e. OCPU =1 Agreement(RAN1-114 meeting) For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding the CPU occupation, - X is a “common value” and not dependent on NTRP or any other parameter value, reported by the UE from a set of candidate values {1, 1.5, 2} according to UE capability According to the following agreement, the supported value of CPU oocupation can be reported based on UE capability for NTRP>1. Proposal 4-4: define a UE capability to indicate the supported value of CPU occupation for NTRP>1. 40. NR_MIMO_evo_DL_UL; 40-3-1-27; The supported value of CPU occupation for NTRP>1; The supported value of CPU occupation for NTRP>1; {1, 1.5, 2}. According to the following agreement, L=6 should be as a additional UE feature/capability, which is similar with UE feature of Rel-16 Type II codebook. Agreement For the Type-II codebook refinement for high/medium velocities based on Rel-16 eType-II regular codebook, at least the following Parameter Combinations are supported L; pυ; β. υ ∈1,2; υ ∈3,4. 4; 1/4; 1/4; 1/4. 4; 1/4; 1/4; 1/2. 4 (*); 1/2; 1/4; 1/2. 4 (*); 1/4; 1/4; 3/4. 6 (*); 1/4; --; 1/2. 6 (*); 1/4; --; 3/4. (*) Note: From legacy. For L=6, the same restriction and UE optionality as legacy apply Proposal 4-5: L=6 should be as an additional UE feature. 40. NR_MIMO_evo_DL_UL; Support parameter combinations with L=6; 40-3-2-1; Yes; N/A; parameter combinations with L=6 is not supported; Per-band and Per-BC; N/A; N/A; N/A; Optional with capability signalling. Acorrding to the following agreement, the supported value of w is a UE feature and needs to be discussed in this part. Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z for Capability 2 associated with P/SP-CSI-RS, decide, in RAN1#114, based on the following alternatives: - Alt1: w=14.(KP–1).d or 14. KP.d, where d denotes the CSI-RS periodicity - Alt2: w=14 or 28 (fixed) Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z associated with P/SP-CSI-RS, - W (unit of symbols) is reported by UE - To be finalized as part of Rel-18 UE feature discussions In our view, the value of w should be assocated with the periodic of CSI-RS, since the CSI processing time needs to increase with the periodic increases. Therefore, Alt1, i.e.,w=14.(KP–1).d or 14. KP.d is preferred. Proposal 4-6: The value of w should be as an additional UE feature and Alt1, i.e.,w=14.(KP–1).d or 14. KP.d is preferred. 40. NR_MIMO_evo_DL_UL; The supported value of w; 40-3-2-1; Yes; N/A; The value of w is not reported.; Per-band and Per-BC; N/A; N/A; N/A; - Alt1: w=14.(KP–1).d or 14. KP.d, where d denotes the CSI-RS periodicity; Optional with capability signalling. According to the following agreement, configuring a subet of comb offset when comb offset hopping is configured and configgureing as subset of cyclic shift when cyclic shift hopping is configured is a UE-optional feature. Agreement Support configuring a subset of comb offsets when comb offset hopping is configured, and configuring a subset of cyclic shifts when cyclic shift hopping is configured. - The subset configuration applies to all the port(s) in the SRS resource, and all the port(s) in the SRS resource has (have) the same hopping offset value khopping(pi) on an OFDM symbol. - This is a UE-optional feature. Proposal 6-2: Configuring a subet of comb offset when comb offset hopping is configured and configuring as subset of cyclic shift when cyclic shift hopping is configured should be a UE-optional feature. 40. NR_MIMO_evo_DL_UL; A subet configuration of comb offset; Support configuring a subset of comb offsets; 40-5-1; Yes; n/a; Per band; n/a; n/a; n/a; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; A subset configuration of cyclic shift; Support configuring a subset of cyclic shift; 40-5-2; Yes; n/a; Per band; n/a; n/a; n/a; Optional with capability signaling. According to the following agreement, finger time-delay-domain granularity is a UE optional feature as well, and the feature cannot be configured when a subset for cyclic shift is configured. Agreement For SRS cyclic shift hopping, support finer time-delay-domain granularity, e.g., αi=2πnSRScs,inSRScs,max+2πnSRScs,offsetK×nSRScs,max, where nSRScs,offset can be randomly chosen from 0,1,…K×nSRScs,max-1 at each SRS transmission. - Note: The finer granularity above only applies to the cyclic shift offsets when cyclic shift hopping is enabled. If a subset for cyclic shifts is configured, this feature cannot be configured. Above is a UE optional feature. 40. NR_MIMO_evo_DL_UL; Finer time-delay-domain granularity confiugration; Support finer time-delay-domain granularity for SRS cyclic shift hopping; 40-5-2; Yes; n/a; Per band; n/a; n/a; n/a; If a subset for cyclic shifts is configured, this feature cannot be configured.; Optional with capability signaling. |
| LG Electronics [11] | 40-5-1c - Based on the agreement below, add an FG for UEs supporting configuring a subset of comb offsets when comb offset hopping is configured. 40-5-2c - Based on the agreement below, add an FG for UEs supporting configuring a subset of cyclic shifts when cyclic shift hopping is configured. Agreement Support configuring a subset of comb offsets when comb offset hopping is configured, and configuring a subset of cyclic shifts when cyclic shift hopping is configured. - The subset configuration applies to all the port(s) in the SRS resource, and all the port(s) in the SRS resource has (have) the same hopping offset value khopping(pi) on an OFDM symbol. - This is a UE-optional feature.. Proposal 7: Add the following two FGs in 40-5-1c and 40-5-2c. 40. NR_MIMO_evo_DL_UL; 40-5-1c; Subset for SRS comb offset hopping; Support of configuring a subset of comb offsets when comb offset hopping is configured; 40-5-1; Yes; n/a; Subset of SRS comb offset hopping is not supported; Per band; n/a; n/a; n/a; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-5-2c; Subset for SRS cyclic shift hopping; Support of configuring a subset of cyclic shifts when cyclic shift hopping is configured; 40-5-2; Yes; n/a; Subset of SRS cyclic shift hopping is not supported; Per band; n/a; n/a; n/a; Optional with capability signaling. |
| CMCC [12] | Agreement@RAN1#114 On unified TCI framework extension for S-DCI based MTPR, a UE capability is used as the threshold for PDSCH reception scheduled/activated by DCI format 1_0/1_1/1_2 if the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2 - Note: Whether to reuse the legacy UE capability (timeDurationForQCL) as the threshold and corresponding candidate values are discussed in Rel-18 UE feature AI. In RAN1#114 meeting [1], it has been agreed that a UE capability is used as the threshold for PDSCH reception scheduled/activated by DCI format 1_0/1_1/1_2 if the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2. If the UE doesn’t support the capability of two default beams, UE must determine a default beam to buffer PDSCH before the threshold, the threshold should include the time for both DCI decoding and beam switching. In Rel-15, the TCI state of PDSCH is indicated in the DCI, before DCI decoding, UE does not know the TCI state of PDSCH, so a larger time is needed for preparing the Rx beam for PDSCH reception. In Rel-18, the TCI state for PDSCH reception is one or both TCI states that UE applied for unified TCI, which are already known for UE. After DCI decoding, UE only needs to determine whether one or both TCI states are used for PDSCH reception, so the time for PDSCH Rx beam preparing time is smaller than Rel-15. It has been agreed that before the threshold, the first indicated joint/DL TCI state is used for PDSCH reception. If some advanced UE could support a threshold smaller than legacy UE capability (timeDurationForQCL), it will be beneficial for gNB to choose a suitable TRP for PDSCH transmission when the scheduling gap is small. So, A new UE capability smaller than legacy UE capability (timeDurationForQCL) is introduced as the threshold for PDSCH reception. Similarly, a new UE capability smaller than the legacy UE capability (beamSwitchTiming/beamSwitchTiming-r16) can be also introduced as the threshold for AP CSI-RS reception. Proposal 1: A new UE capability smaller than legacy UE capability (timeDurationForQCL) is introduced as the threshold for PDSCH reception scheduled/activated by DCI format 1_0/1_1/1_2. Proposal 2: A new UE capability smaller than legacy UE capability (beamSwitchTiming/beamSwitchTiming-r16) is introduced as the threshold for AP CSI-RS reception. |
| Google [13] | For DMRS, one UE FG on supported MU-MIMO operation with different DMRS types is missing, which is a new feature introduced in Rel-18. Therefore, the following UE FG is proposed. Proposal 3: Introduce the following UE FG 40-4-11 based on the new feature introduced in Rel-18 40. NR_MIMO_evo_DL_UL; 40-4-11; Support of MU-MIMO operation from different DMRS types; 1. Support of MU-MIMO operation for current UE and co-scheduling UEs with different DMRS types; 40-4-1; Yes; n/a; MU-MIMO operation for current UE and co-scheduling UEs with different DMRS types is not supported; Per FS; No; No; n/a; Candidate value: {Type1+eType1, eType1+Type1, Type2+eType2, eType2+Type2}; Optional with capability signaling. For SRS, there is a new feature agreed in RAN1 #114, which introduces configuration of subset of comb offsets and cyclic shifts for SRS comb offset hopping and cyclic shift hopping. Therefore, the corresponding UE FG should be introduced. In RAN1 #113, it was agreed that the joint comb offset hopping and cyclic shift hopping can be discussed in UE feature session. Current spec does not prohibit such joint hopping. Therefore, corresponding UE FG is introduced. Proposal 4: Introduce the following UE FGs based on the new feature agreed in RAN1 #114 and agreement in RAN1 #113. 40. NR_MIMO_evo_DL_UL; 40-5-1c; Comb offset hopping based on configured subset of comb offsets; Support of comb offset hopping based on configured subset of comb offsets; 40-5-1; Yes; n/a; Per band; n/a; n/a; n/a; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-5-2a; Cyclic shift hopping based on configured subset of cyclic shifts; Support of cyclic shift hopping based on configured subset of cyclic shifts; 40-5-2; Yes; n/a; Per band; n/a; n/a; n/a; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-5-3; Comb offset hopping and cyclic shift hopping for the same SRS resource; Support of comb offset hopping and cyclic shift hopping for the same SRS resource; 40-5-1, 40-5-2; Yes; n/a; Per band; n/a; n/a; n/a; Optional with capability signaling. For STxMP, there is a hidden feature for mDCI based STxMP PUSCH, where the PUSCH may be based on DFT-s-OFDM waveform. Although not explicitly agreed, current spec does not preclude the network to schedule one of or both PUSCHs from DFT-s-OFDM waveform. Therefore, the following UE FG is proposed with regard to the hidden feature. Proposal 5: Introduce the following UE FGs with regard to DFT-s-OFDM waveform for one of the PUSCHs for mDCI based STxMP. 40. NR_MIMO_evo_DL_UL; 40-6-3-1; Support of mDCI based STxMP PUSCH with DFT-s-OFDM + DFT-s-OFDM waveform; Support of mDCI based STxMP PUSCH with DFT-s-OFDM + DFT-s-OFDM waveform; 40-6-3, 40-6-3a; Yes; n/a; mDCI based STxMP PUSCH with DFT-s-OFDM + DFT-s-OFDM waveform is not supported; Per FS; n/a; FR2 only; n/a; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-6-3-2; Support of mDCI based STxMP PUSCH with DFT-s-OFDM + CP-OFDM waveform; Support of mDCI based STxMP PUSCH with DFT-s-OFDM + CP-OFDM waveform; 40-6-3, 40-6-3a; Yes; n/a; mDCI based STxMP PUSCH with DFT-s-OFDM + CP-OFDM waveform is not supported; Per FS; n/a; FR2 only; n/a; Optional with capability signaling. For 8Tx, Rel-18 introduces one UE feature on UCI multiplexing on a PUSCH with two codewords. Therefore, corresponding UE FG should be introduced. Proposal 6: Introduce the following UE FG on UCI multiplexing on a PUSCH with two codewords, which is based on the agreed Rel-18 feature. 40. NR_MIMO_evo_DL_UL; 40-7-6; Support of UCI multiplexing on PUSCH with 2 codeword; Support of UCI multiplexing on PUSCH with 2 codeword; 40-7-1; Yes; n/a; Support of UCI multiplexing on PUSCH with 2 codeword is not supported; Per FSPC; No; No; No; Optional with capability signaling. |
| NTT DOCOMO, INC. [14] | Others: - Add a new FG for cyclic shift offset hopping with finer time-delay-domain granularity - Prerequisite: FG 40-5-2 - Type: Per band - Add a new FG for hopping subset configuration for each of comb offset hopping and cyclic shift hopping (a FG for each) - Prerequisite: FG 40-5-1 and 40-5-2 - Type: per band |
| Apple [15] | 40. NR_MIMO_evo_DL_UL; 40-1-5; For unified TCI framework for single-DCI multi-TRP, spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission; - For unified TCI framework for single-DCI multi-TRP, support of spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission that is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission. Note: This is still an FFS since RAN1#112. Depends on whether this is supported by Rel-18, needs further discussion; Per band; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-2-9; Overlapping UL transmission reduction; - Support of reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions, when the UE does not support UL STx2P (Simultaneous Transmission cross 2 Panels), for multi-DCI based multi-TRP operation with two TA enhancement; Per band; Note: If UE does not support this feature, UE does not expect the two UL transmissions to overlap (i.e., scheduling restriction is applied to avoid overlap between the two UL transmissions); Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-3-1-5b; Maximum number of TRPs supported for Rel-17-based CJT Type-II codebook; 1, Maximum number of TRPs supported for Rel-17-based CJT Type-II codebook; 40-3-1-1; Yes; N/A; Rel-17-based CJT type-II codebook with FD basis selection integer frequency offset is not supported; Per band and Per BC; N/A; N/A; N/A; Component 1 candidate values: {2,3,4}; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-1-24; CPU counting for multi-TRP CJT; Scaling factor X for CPU occupation of multi-TRP CJT CSI based on Type-II codebook; Per FS; Component 1 candidate values: {1, 1.5, 2} Note: When NTRP=1 TRP is configured, OCPU =1. When NTRP>1 TRPS are configured, OCPU =X * NTRP; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-3-2-1b; Maximum number of aperiodic CSI-RS resources that can be configured in the same CSI report setting for Rel-16-based doppler measurement; 1. Maximum number of aperiodic CSI-RS resources that can be configured in the same CSI report setting for Rel-16-based doppler measurement; 40-3-2-1; Yes; N/A; Per-band and Per-BC; N/A; N/A; N/A; Component 1 candidate values: {4, 8, 12}; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-2-2a; Support Rel-16-based predicted PMI codebook with parameter combination with L=6; Support of Rel-16 eType-II regular codebook refinement for predicted PMI with parameter combination with L=6; Per band and per BC; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-3-2-2b; Support Rel-16-based predicted PMI codebook with rank 3,4; Support of Rel-16 eType-II regular codebook refinement for predicted PMI with rank 3,4; Per band and per BC; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-3-2-4b; Maximum number of aperiodic CSI-RS resources that can be configured in the same CSI report setting for Rel-17-based doppler measurement; 1. Maximum number of aperiodic CSI-RS resources that can be configured in the same CSI report setting for Rel-17-based doppler measurement; 40-3-2-4; Yes; N/A; Per-band and Per-BC; N/A; N/A; N/A; Component 1 candidate values: {4, 8, 12}; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-2-8; Support Rel-17-based predicted PMI codebook with rank 3,4; Support of Rel-17 FeType-II port selection codebook refinement for predicted PMI with rank 3,4; Per band and per BC; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-3-2-9; Aperiodic CSI report timing relaxation for predicted PMI codebook based on Type-II codebook; - Aperiodic CSI report timing relaxation, w, for predicted PMI codebook based on Type-II codebook. - Aperiodic CSI report timing relaxation, Z’, for predicted PMI codebook based on Type-II codebook; Per band and per BC; Component 1 candidate values: UE reports candidate value, w, independently for each SCS in unit of symbols For 15kHz SCS: {0, 4, 8, 16, 32} For 30kHz SCS: {0, 8, 16, 32, 64} For 60kHz SCS: {0, 16, 32, 64, 128} For 120kHz SCS: {0, 16, 32, 64, 128} For 480kHz SCS: {0, 64, 128, 256, 512} For 960kHz SCS: {0, 128, 256, 512, 1024} Component 2 candidate values: {CAP1, CAP2} For CAP1 in component 2 1) For AP CSI-RS: Z=legacy Z+14*(K–1)*m 2) For P/SP CSI-RS: Z= legacy Z+w For CAP2 in component 3 1) For AP CSI-RS: Z= legacy Z+14*(K–1)*m + r 2) For P/SP CSI-RS: Z= legacy Z+w+r where Legacy Z/Z’ values are defined as Z2/Z'2 in Table 5.4-2 in TS38.214 K = {4,8,12}, is the number of AP CSI-RS resources for the CMR in a CSI report setting M = {1,2}, is the offset between two adjacent AP CSI-RS resources for the CMR in slots; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-4-8; Support of Rel-18 DMRS and PDSCH processing capability 2 simultaneously; Support Rel-18 DMRS and PDSCH processing capability 2 simultaneously; 40-4-1/1a, 5-5a/5b; Yes; n/a; Rel-18 DMRS and PDSCH processing capability 2 are not support simultaneously; Per FS; No; No; n/a; Note: PDSCH processing capability #2 related UE capability follows legacy FG5-5a, FG5-5b, FG5-13, FG13-a, FG5-13c; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-4-6b; Support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PUSCH; 1. Support of 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PUSCH; Per FS; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-5-1c; Configuration of subset of comb offsets for comb offset hopping; Support of configuration of a subset of comb offsets when comb offset hopping is configured; Per band; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-5-2c; Configuration of subset of cyclic shifts for cyclic shift hopping; Support of configuration of a subset of cyclic shifts when cyclic shift hopping is configured; Per band; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-5-2d; Finer time-delay-domain granularity for SRS cyclic shift hopping; Support of finer time-delay-domain granularity for SRS cyclic shift hopping; Per band; Note: The finer granularity above only applies to the cyclic shift offsets when cyclic shift hopping is enabled. If a subset for cyclic shifts is configured, this feature cannot be configured; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-5-8; Maximum 2 SP and 1 periodic SRS sets for 8T8R antenna switching; Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for 8T8R antenna switching; 40-5-4; Yes; n/a; Maximum 2 SP and 1 periodic SRS sets for 8T8R antenna switching is not supported; Per FS; n/a; n/a; n/a; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-6-3r-1; Single-DCI based STx2P SDM scheme for PUSCH and repetition in time; 1. single-DCI based STx2P and semi-static indication of PUSCH repetitions over multiple slots 2. single-DCI based STx2P and dynamic indication of repetition Type-A 3. single-DCI based STx2P SDM and dynamic indication of repetition Type-B; 40-6-1, or 40-6-1a; Yes; N/A; UE cannot be indicated to perform STx2P SDM scheme over R15/16 PUSCH repetitions in time; Per FSPC; No; n/a; Notes: For component 1, UE also reports 5-17, and/or 5-16, and/or 5-14. For component 2, UE also reports 11-6. For component 3, UE also reports 11-5.; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-6-3r-2; Single-DCI based STx2P SFN scheme for PUSCH and repetition in time; 1. single-DCI based STx2P and semi-static indication of PUSCH repetitions over multiple slots 2. single-DCI based STx2P and dynamic indication of repetition Type-A 3. single-DCI based STx2P SDM and dynamic indication of repetition Type-B; 40-6-2, or 40-6-2a; Yes; N/A; UE cannot be indicated to perform STx2P SFN scheme over R15/16 PUSCH repetitions in time; Per FSPC; No; n/a; Notes: For component 1, UE also reports 5-17, and/or 5-16, and/or 5-14. For component 2, UE also reports 11-6. For component 3, UE also reports 11-5.; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-6-3r-3; multi-DCI based STx2P for PUSCH+PUSCH and repetition in time for at least one of the PUSCHs; 1. multi-DCI based STx2P for PUSCH+PUSCH and semi-static indication of PUSCH repetitions over multiple slots 2. multi-DCI based STx2P for PUSCH+PUSCH and dynamic indication of repetition Type-A 3. multi-DCI based STx2P for PUSCH+PUSCH and dynamic indication of repetition Type-B; 40-6-3a, or 40-6-3b; Yes; N/A; UE cannot be indicated to perform STx2P SDM scheme over R15/16 PUSCH repetitions in time; Per FSPC; No; n/a; Notes: For component 1, UE also reports 5-17, and/or 5-16, and/or 5-14. For component 2, UE also reports 11-6. For component 3, UE also reports 11-5.; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-6-3q; Multi-DCI STx2P with different PHY priorities; Support of Multi-DCI STx2P with different PHY priorities; FG12-1; Per FS; FR2 only; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-6-4a; Rel-18 STxMP SFN scheme and the Rel-17 repetition number parameter pucch-RepetitionNrofSlots in a same PUCCH resource at the same time; Support of enabling Rel-18 STxMP SFN scheme and the Rel-17 repetition number parameter pucch-RepetitionNrofSlots in a same PUCCH resource at the same time; Per band. 40. NR_MIMO_evo_DL_UL; 40-7-1h; UL full power transmission mode of fullpowerMode2 for 8 Tx – SRS resource; The supported SRS configurations with different number of antenna ports per SRS resource for full power Mode 2 when UE is capable of and configured with 8 Tx codebook based PUSCH operation; Per FS; Component 1 candidate values: {1_8, 1_2_8, 1_4_8, 1_2_4_8} 1st state (1_8): each SRS resource can be configured with 1 port or 8 ports. 2nd state (1_2_8): each SRS resource can be configured with 1 port or 2 ports or 8 ports. 3rd state (1_4_8): each SRS resource can be configured with 1 port or 4 ports or 8 ports. 4th state (1_2_4_8): each SRS resource can be configured with 1 port or 2 port or 4 ports or 8 ports.; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-7-1i; UL full power transmission mode of fullpowerMode2 for 8 Tx – full power TPMI groups, Higher-layer parameter CodebookType = “codebook2”; - TPMI group(s) which delivers full power when UE is capable of and configured with 8 Tx codebook based PUSCH operation, Higher-layer parameter CodebookType = “codebook2”; Per FS; Component 1 candidate values: {first coherent antenna port group, second coherent antenna port group}; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-7-4e; UL full power transmission mode of fullpowerMode2; FFS How to support downgrade configuration, i.e., UE is configured with 2/4 Tx while UE supports 8Tx. Whether and how to use the legacy FG16-5c, FG16-5c-2, FG16-5c-3; Per FS; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-7-1a-1; PUSCH codebook coherency subset when an 8 Tx capable UE is configured with 2 Tx codebook based PUSCH operation; 1.Supported codebook coherency subset type when an 8 Tx capable UE is configured with 2 Tx codebook based PUSCH.; Per band; Component 1 candidate values: {non-coherent, non-coherent and full-coherent}; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-7-1a-2; PUSCH codebook coherency subset when an 8 Tx capable UE is configured with 4 Tx codebook based PUSCH operation; Supported codebook coherency subset type when an 8 Tx capable UE is configured with 4 Tx codebook based PUSCH operation.; Per band; Component 1 candidate values: {non-coherent, non-coherent and partial coherent, non-coherent and partial coherent and full-coherent} FFS: whether to reuse legacy FG2-13 assuming UE reports that UE supports 4Tx in legacy; Optional with capability signaling. |
| Samsung [16] | Proposal 1: support the following FG (separate row(s) from the basic FGs) for Rel-18 eUTCI - support of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies only one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH. 40. NR_MIMO_evo_DL_UL; 40-1-14; Additional support of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies only one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH; n/a; Optional with capability signalling. In RAN1#114, the below agreement was made to address some ambiguity issues from UE’s operation perspective when a UE is configured to follow the indicated unified TCI state(s) with a capability of per resource set TCI selection for a configured aperiodic CSI-RS resource set for NCJT CSI measurement/reporting – two Resource Groups are configured. Agreement (RAN1#114) On unified TCI framework extension, if the UE supports NCJT CSI, the UE should support resource-level RRC configuration for informing that the UE shall apply the first or the second indicated joint/DL TCI state to AP CSI-RS resource. The above agreement, however, implicitly bundles the support of NCJT scheme and per-CSI-RS resource configuration for TCI selection in S-DCI based MTRP as in FG 40-1-3. To illustrate, a UE supporting both FG 40-1-3 and FG 23-7-1 cannot indicate ‘per resource set’ for FG 40-1-3. The only way to avoid that is that UE declines either FG 40-1-3 or FG 23-7-1. Since both FG 40-1-3 and FG 23-7-1 have ‘per band’ granularity, the UE would be forced to drop either of them in a band manner which can be considered as under-reporting where UE could have indicated ‘per resource set’ for FG 40-1-3 supporting eUTCI CSI-RS transmission in any other scheme than NCJT. To address the issue of under-reporting, one possible solution is that a separate capability is introduced for joint operation of NCJT scheme and UTCI CSI-RS for S-DCI based MTRP for a UE that indicates ‘per resource’ for FG 40-1-3. Proposal 3: a new capability is introduced for joint operation of NCJT scheme and eUTCI CSI-RS for SDCI based MTRP operation as below: 40. NR_MIMO_evo_DL_UL; 40-1-15; Joint operation of NCJT CSI and per aperiodic CSI-RS resource configuration for TCI selection in S-DCI based MTRP; Support of joint operation of NCJT CSI and per aperiodic CSI-RS resource configuration for TCI selection in S-DCI based MTRP; 23-7-1, 40-1-3; yes; n/a; Per UE; n/a; n/a; n/a; Optional with capability signalling. Similar to the discussion for NCJT scheme in RAN1#114 to address UE behavior when UE is configured to follow unified TCI state with capability of per resource set, we also need to address UE behavior ambiguity for CJT scheme with capability of more than one unified TCI states. For a configured aperiodic CSI-RS resource set for CJT CSI measurement/reporting with K NZP CSI-RS resources each corresponding to one TRP/TRP-group, there would be ambiguity on UE behavior when UE is configured to follow unified TCI state with capability of per resource set in FG 40-1-3 and two TCI states for CJT Tx scheme for PDSCH in FG 40-1-4. Following the same rule as agreed for NCJT in RAN1#114, we may condition the support of CJT CSI on the support of resource-level RRC configuration for informing that the UE shall apply the first or the second indicated joint/DL TCI state to AP CSI-RS resource. Hence, a separate capability can be introduced for joint operation of CJT scheme and UTCI aperiodic CSI-RS transmission. Proposal 4: a new capability is introduced for joint operation of CJT scheme and UTCI CSI-RS for S-DCI based MTRP as below: 40. NR_MIMO_evo_DL_UL; 40-1-16; Joint operation of CJT with more than one unified TCI states and per aperiodic CSI-RS resource configuration for TCI selection in S-DCI based MTRP; Support of joint operation of CJT with more than one unified TCI states and per aperiodic CSI-RS resource configuration for TCI selection in S-DCI based MTRP; 40-1-3, 40-1-4, 40-3-1-1, 40-3-1-5; yes; n/a; Per UE; n/a; n/a; n/a; Optional with capability signalling. In RAN1#113, the following agreement has been made: Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, for the case when the UE does not support UL STxMP transmission, - for the baseline feature, the UE does not expect the two UL transmissions to overlap (i.e., scheduling restriction is applied to avoid overlap between the two UL transmissions) - as an optional feature, the overlapping duration of the later of the two UL transmissions is reduced. - FFS: for the optional feature, whether or not the overlapping duration needs to be specified as 1 (in case 2) or 2 (in case 1) OFDM symbols where - Case 1 applies when UE is capable of supporting MRTD > CP, SCS=60 kHz and frequency range is FR1. - Case 2 applies in all other cases. Based on this agreement, there can be UEs that can handle overlap of the two uplink transmissions with different TAs and UEs that can’t handle the overlap of the two uplink transmissions with different TAs. Therefore, we can consider the following two cases: - UEs not capable of handling overlap (basic feature) This refers to a UE that does not expect any overlapping of two UL transmissions in multi-DCI based multi-TRP operation with two TA enhancement as network would avoid any overlap between the two UL transmissions through scheduling restriction. If such overlap happens as an error case, UE behaviour can then be dropping the later transmission. - UEs capable of handling overlap This refers to a UE that can handle specific overlap duration for two UL transmissions in multi-DCI based multi-TRP operation with two TA enhancement. That is UE can expect two UL transmissions overlap with some specific duration and if it happens, the overlapping duration of the later of the two UL transmissions would be reduced. The signalling granularity of such UE capabilities can be FS or FSPC, following the DL counterpart design. Proposal 8: For multi-DCI based Multi-TRP operation with two TA enhancement, introduce new UE capability as described below with signalling granularity of ‘per FS’ or ‘per FSPC’. 40. NR_MIMO_evo_DL_UL; 40-2-7; UL overlapping in multi-DCI based Multi-TRP operation with two TA enhancement; Support of UL overlapping by reducing the latter UL transmission in multi-DCI based Multi-TRP operation with two TA enhancement; 40-2-1, 40-2-2; yes; n/a; Per FS; n/a; n/a; n/a; Optional with capability signalling. A new feature group (e.g., FG 40-3-1-A) for OCPU and Z/Z’ is needed based on the agreements below. Based on the agreements, X∈1,1.5,2 is UE capability and determined by the UE, where OCPU = X.NTRP, when NTRP>1. Regarding Z/Z’, based on the agreements below, Capability 1 and Capability 2 need to be supported via another FG. [113] Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding the CPU occupation: OCPU = X.NTRP where - X≥1 when NTRP>1, is defined based on UE capabilities and determined by the UE - FFS: Whether the supported value(s) of X are common or can depend on the value of NTRP, NL, total sum of {Ln}, and/or other CJT features (e.g. dynamic TRP selection) - The legacy specification on CPU pools is fully reused - Note: When NTRP=1 is configured, legacy OCPU applies, i.e. OCPU =1 [114] Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding the CPU occupation, - X is a “common value” and not dependent on NTRP or any other parameter value, reported by the UE from a set of candidate values {1, 1.5, 2} according to UE capability [113] Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding Z/Z’: - For NTRP=1: reuse legacy Z/Z’ values - For NTRP>1, introduce two UE capabilities: - Capability 1: Reuse legacy Z/Z’ values - Capability 2: Legacy Z/Z’ values + r - The value(s) of r>0 can depend on the configured NTRP value - FFS: exact value(s) of r Note: Since this pertains Type-II, the relevant values are Z2/Z2’ [114] Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding Z/Z’ for Capability 2 when NTRP>1, r=legacy Z2’. (New) FG 40-3-2-A: Support of K=12 aperiodic NZP CSI-RS resources (according to the agreement below) [112] Agreement For the Type-II codebook refinement for high/medium velocities, regarding the parameter K (the number of AP-CSI-RS resources for the CMR), optionally support only K=12 as an additional candidate value. (New) FG 40-3-2-B: Support of L=6 (according to the agreement below) [112bis-e] Agreement For the Type-II codebook refinement for high/medium velocities based on Rel-16 eType-II regular codebook, at least the following Parameter Combinations are supported … Note: From legacy. For L=6, the same restriction and UE optionality as legacy apply. Regarding Z/Z’, based on the agreements below, Capability 1 and Capability 2 need to be supported via another FG. On the candidate values of w as a UE capability, we support the possible values of w should be in the set of the values that are generated from alternative 1 or alternative 2 of the following agreement. [114] Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z for Capability 2 associated with P/SP-CSI-RS, decide, in RAN1#114, based on the following alternatives: - Alt1: w=14.(KP–1).d or 14. KP.d, where d denotes the CSI-RS periodicity - Alt2: w=14 or 28 (fixed). The candidate values of w should also be chosen such that Z+w+r≤32×14 (in symbols), where r= legacy Z2', because the max value of reportSlotOffset is 32 slots, as discussed during the previous meeting. In our view, the candidate values of w should be chosen such that - w=14 or 28, - w=14(Kp-1)d, where Kp=1,2, or 4 is UE capability and d=4,5,8,10,16,20,40,80,160,320 is periodicity, and - Z+w+r≤32×14, where r=legacy Z2'. Since Z+r can be given by (up to) 141+140 =281 (when μ=2), w should be less than or equal to 167 to hold Z+w+r≤32×14. Among the possible values that are generated from W=14(Kp-1)d (shown in the table below), 0, 56, 70, 112, and 140 are only the candidate values less than or equal to 167. Values of W=14(Kp-1)d d=4; d=5; d=8; d=10; d=16; d=20; d=40; d=80; d=160; d=320. Kp=1; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0. Kp=2; 56; 70; 112; 140; 224; 280; 560; 1120; 2240; 4480. Kp=4; 168; 210; 336; 420; 672; 840; 1680; 3360; 6720; 13440. Hence we propose w∈{0, 14, 28, 56, 70, 112, 140}. [113] Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the CPU occupation: OCPU = Y.N4 when P/SP-CSI-RS is configured for CMR, or OCPU = Y.K when AP-CSI-RS is configured for CMR - Y≥1 is defined based on UE capabilities and determined by the UE, and can be different between P/SP-CSI-RS and AP-CSI-RS. - FFS: Whether the supported value(s) of Y can depend on codebook parameter values - The legacy specification on CPU pools is fully reused - When N4=1, OCPU =4 - OCPU ≥ 4 when P/SP-CSI-RS is configured for CMR [114] Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the CPU occupation, the candidate values of Y are {2/3, 1, 2, 3} [113] Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z/Z’ - For N4=1: reuse legacy Z’ values - For N4>1, introduce two UE capabilities: - Capability 1: Reuse legacy Z’ values - Capability 2: Legacy Z’ values + r - The value(s) of r>0 can depend on the configured N4 value - FFS: exact value(s) of r Note: Since this pertains Type-II, the relevant values are Z2/Z2’ [113] Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z - Based on the two UE capabilities agreed for Z’: - Capability 1: - For AP CSI-RS: Z=legacy Z+14.(K–1).m - For P/SP CSI-RS: Z= legacy Z+w where w>0 - TBD: Value of w - Capability 2: - For AP CSI-RS: Z= legacy Z+14.(K–1).m + r - For P/SP CSI-RS: Z= legacy Z+w+r - Note: r corresponds to the agreed value for Z’ relaxation in previous agreement. Note: Since this pertains Type-II, the relevant values are Z2/Z2’ [114] Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z/Z’ for Capability 2, r=legacy Z2’ [114] Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z for Capability 2 associated with P/SP-CSI-RS, decide, in RAN1#114, based on the following alternatives: - Alt1: w=14.(KP–1).d or 14. KP.d, where d denotes the CSI-RS periodicity - Alt2: w=14 or 28 (fixed) [114] Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z associated with P/SP-CSI-RS, - W (unit of symbols) is reported by UE - To be finalized as part of Rel-18 UE feature discussions. Proposal 9: - For CJT, - (New) FG 40-3-1-A for Ocpu - OCPU=XNTRP, where X∈1,1.5,2 - (New) FG 40-3-1-B for Z/Z’ - Support Capability 1 and Capability 2 as the relevant agreements stated - For Doppler - (New) FG 40-3-2-A: Support of K=12 aperiodic NZP CSI-RS resources - (New) FG 40-3-2-B: Support of L=6 - (New) FG 40-3-2-D for Z/Z’ - Support Capability 1 and Capability 2 as the relevant agreements stated - Support the candidate of w should be in w∈{0, 14, 28, 56, 70, 112, 140} that satisfies - w=14 or 28, - w=14(Kp-1)d, where Kp=1,2, or 4 is UE capability and d=4,5,8,10,16,20,40,80,160,320 is periodicity, and - Z+w+r≤32×14, where r=legacy Z2'. Since Rel-18 DMRS eType-I/II are introduced, there are 4 different types of DMRSs: Rel-15 DMRS Type-I/II, and Rel-18 DMRS eType-I/II. Given supportable types of DMRS for each different DCI format defined in NR, a UE shall prepare up to 4 different DMRS types whenever the UE receives same or different DCI formats, which could be burden to UE. Hence, our view is that defining the maximum number of different DMRS types considering for all DCI formats as a UE capability could inform to gNB the processing limit of the UE. Proposal 13: Support a UE capability introducing the maximum number of different DMRS types considering for all DCI formats. 40-4-XX; Maximum number of different DMRS types for all DCI formats; Support maximum number of different DMRS types for all DCI formats; 40-4-1g; Per FS; Candidate values: {1, 2, 3, 4}. In RAN1#114 [4], a detailed signaling mechanism for subset configuration has been agreed to limit the hopping region of comb-offsets for comb-offset hopping and cyclic shifts for cyclic shift hopping, as follows: Agreement When a subset of comb offsets for comb offset hopping is configured, and when a subset of cyclic shifts for cyclic shift hopping is configured, support the following option for configuring the subset S={S(0), S(1), …, S(z-1)} with 1<z<Z, where Z=KTC for comb offset hopping and Z=nSRScs,max for cyclic shift hopping, and: - Option 1b: S(0), S(1), …, S(z-1) are configured via a Z-length bitmap with S(i-1) being the i-th bit set as 1.. Since FGs for comb-offset hopping and cyclic shift hopping are defined separately as in Table. 3, our view is that FGs for subset of comb-offsets and cyclic shifts for comb-offset hopping and cyclic shift hopping, respectively, can be defined separately as well. Hence, we support the following table for supporting subset of comb-offsets and cyclic shifts for comb-offset hopping and cyclic shift hopping, respectively. Proposal 15: Define a FG supporting subset of comb-offsets and cyclic shifts for comb-offset hopping and cyclic shift hopping, respectively. 40-5-XX; Subset of comb-offsets for comb-offset hopping; Support subset of comb-offsets for comb-offset hopping; 2-52 or 2-53, 40-5-1; Per band. 40-5-XX; Subset of cyclic shifts for cyclic shift hopping; Support subset of cyclic shifts for cyclic shift hopping; 2-52 or 2-53, 40-5-2; Per band; This feature and cyclic shift hopping with finer granularity are not configured with a UE at the same time.. In RAN1#113, a concept of finer granularity for cyclic shift hopping has been agreed as follows: Agreement in RAN1#113 For SRS cyclic shift hopping, support finer time-delay-domain granularity, e.g., αi=2πnSRScs,inSRScs,max+2πnSRScs,offsetK×nSRScs,max, where nSRScs,offset can be randomly chosen from 0,1,…K×nSRScs,max-1 at each SRS transmission. - Note: The finer granularity above only applies to the cyclic shift offsets when cyclic shift hopping is enabled. If a subset for cyclic shifts is configured, this feature cannot be configured. Above is a UE optional feature.. Based on the above agreement, an optional UE feature is needed to be defined, hence we would like to suggest the following FG. Proposal 16: Define a FG supporting finer granularity of cyclic shift hopping. 40-5-XX; Cyclic shift hopping with finer granularity; Support cyclic shift hopping with finer granularity as K=2; 2-52, 40-5-1; Per band; This feature and a subset of cyclic shifts for cyclic shift hopping are not configured with a UE at the same time.. When the UE can support multi-DCI based STx2P PUSCH+PUSCH, the UE will prepare two TBs in parallel to transmit both PUSCH simultaneously. However, depending on UE’s capability for processing time, additional timeline may be required. Therefore, new processing capability for additional timeline of multi-DCI based STx2P PUSCH+PUSCH can be introduced as follow: 40-6-3a-1; UE STxMP processing capability for codebook; 1. Require additional timeline to process multiple TBs for codebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG Note. This FG can be applied for CG+DG also if UE can support those FG.; 40-6-3a. 40-6-3b-2; UE STxMP processing capability for noncodebook; 1. Require additional timeline to process multiple TBs for noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG Note. This FG can be applied for CG+DG also if UE can support those FG.; 40-6-3b. Proposal 20: Introduce new processing capability for additional timeline of multi-DCI based STx2P PUSCH+PUSCH as follow: 40-6-3a-1; UE STxMP processing capability for codebook; 1. Require additional timeline to process multiple TBs for codebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG Note. This FG can be applied for CG+DG also if UE can support those FG.; 40-6-3a. 40-6-3b-2; UE STxMP processing capability for noncodebook; 1. Require additional timeline to process multiple TBs for noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG Note. This FG can be applied for CG+DG also if UE can support those FG.; 40-6-3b. - (New) FG 40-7-A: support of a first sub-feature of mode2 that is based on SRS virtualization - (New) FG 40-7-B: support of a second sub-feature of mode2 that is based on full power TPMI group(s) (New) FG 40-7-A UL full power transmission 8TxfullpowerMode2 – SRS resources; 1. The SRS configuration with different number of antenna ports per SRS resource for Mode 2; Component 1: TBD. (New) FG 40-7-B UL full power transmission 8TxfullpowerMode2 – full power TPMI groups; 1. TPMI group(s) which delivers full power; Component 1: TBD. |
| MediaTek Inc. [17] | The Number of TDCP reports: In RAN1#113, an FG for the “Number of CSI-RS resources for TDCP” is agreed. This includes both per-CC and across-CC resources, as well as configured and simultaneous resources. The configured resources can be used in different ways to create various TDCP reports. Hence, we propose adding a separate FG to indicate the maximum number of active (simultaneous) and configured TDCP reports that the UE supports, for both per-CC and across-CC. Proposal 10: Add an FG to indicate the “Maximum number of TDCP reports” per-CC and across-CC, with indicators for both the total number of configured reports and for active reports in each case: 40. NR_MIMO_evo_DL_UL; 40-3-3-x; Maximum number of TDCP reports; - Maximum number of configured TDCP reports per-CC. - Maximum number of configured TDCP reports across-CC. - Maximum number of active TDCP reports per-CC. - Maximum number of active TDCP reports across-CC.. A-TRS based TDCP reporting: Since aperiodic TRS (A-TRS) is an optional UE feature, we propose adding a separate FG to indicate whether UE supports TDCP reporting based on A-TRS. Proposal 12: Add an FG for “Aperiodic-TRS-based” to indicate whether the UE supports TDCP reporting based on Aperiodic-TRS: 40. NR_MIMO_evo_DL_UL; 40-3-3-x; Aperiodic-TRS-based; - A Flag to indicate support of aperiodic-TRS-based TDCP reporting.; Optional with UE capability. Number of CPUs: In RAN1-114, we have the following agreement regarding the number of CPUs RAN1 #114 Agreement For the Rel-18 TRS-based TDCP reporting, when Y delay(s) are configured, regarding CPU occupation, the value of X={1,2} is reported and not dependent on the configured value of D or whether phase reporting is ON/OFF Proposal 13: Add an FG for “Number of CPUs” to indicate the value X∈{1,2} 40. NR_MIMO_evo_DL_UL; 40-3-3-x; Number of CPUs; - The value for X∈{1,2}, where the number of CPUs OCPU=Y+1. X; Optional with UE capability. Number of periodic TRS sets: RAN1 #114 Agreement For the Rel-18 TRS-based TDCP reporting, the supported values of KTRS (number of configured TRS resource sets) are {1,2,3} - The candidate values {2,3} are UE optional Proposal 14: Add an FG for the “Number of periodic TRS sets” to indicate the value(s) supported by UE. 40. NR_MIMO_evo_DL_UL; 40-3-3-x; Number of periodic TRS; - Number of supported periodic TRS sets KTRS∈{1,2,3}; Optional with UE capability. RAN1# 113 Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding the CPU occupation: OCPU = X.NTRP where - X≥1 when NTRP>1, is defined based on UE capabilities and determined by the UE - FFS: Whether the supported value(s) of X are common or can depend on the value of NTRP, NL, total sum of {Ln}, and/or other CJT features (e.g. dynamic TRP selection) - The legacy specification on CPU pools is fully reused - Note: When NTRP=1 is configured, legacy OCPU applies, i.e. OCPU =1 RAN1# 114 Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding the CPU occupation, - X is a “common value” and not dependent on NTRP or any other parameter value, reported by the UE from a set of candidate values {1, 1.5, 2} according to UE capability Based on this agreement, we have the following proposal: Proposal: Introduce the capability on number of CPUs occupied per active CSI-RS resource for Rel-18 Type II codebook for CJT. RAN1# 113 Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding Z/Z’: - For NTRP=1: reuse legacy Z/Z’ values - For NTRP>1, introduce two UE capabilities: - Capability 1: Reuse legacy Z/Z’ values - Capability 2: Legacy Z/Z’ values + r - The value(s) of r>0 can depend on the configured NTRP value - FFS: exact value(s) of r Note: Since this pertains Type-II, the relevant values are Z2/Z2’ In RAN1# 111, it was agreed to support the value of DD unit size d=m for AP-CSI-RS, and in RAN1# 112, it was further agreed to support d=1 for AP-CSI-RS and d= CSI-RS periodicity for P/SP-CSI-RS. RAN1# 112 Agreement For the Type-II codebook refinement for high/medium velocities, regarding the parameter d (in slots), - for P/SP-CSI-RS, support d equal to the periodicity of the CSI-RS resource - for AP-CSI-RS, also support d =1 While agreeing to support d=1, it was observed by many companies that this feature requires UE to perform per-slot channel interpolation in addition to channel prediction, which increases UE complexity. Therefore, we propose to include d=m as part of the basic feature group, and an additional UE capability for supporting d =1. Proposal: Include the support of DD unit size d=m for AP-CSI-RS and d= CSI-RS periodicity for P/SP-CSI-RS in the basic feature group of Doppler CSI. Proposal: Include a new feature group for the support of DD unit size d=1 for AP-CSI-RS. 40. NR_MIMO_evo_DL_UL; 40-3-2-x; Support of DD unit size d=1; Support of DD unit size d=1 when A-CSI-RS is configured for CMR; Optional with capability signalling. In RAN# 113, two UE capabilities were agreed for CSI computation timeline: RAN1# 113 Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z - Based on the two UE capabilities agreed for Z’: - Capability 1: - For AP CSI-RS: Z=legacy Z+14.(K–1).m - For P/SP CSI-RS: Z= legacy Z+w where w>0 - TBD: Value of w - Capability 2: - For AP CSI-RS: Z= legacy Z+14.(K–1).m + r - For P/SP CSI-RS: Z= legacy Z+w+r - Note: r corresponds to the agreed value for Z’ relaxation in previous agreement. Note: Since this pertains Type-II, the relevant values are Z2/Z2’ |
| Qualcomm Incorporated [18] | We note the following regarding UE features for two TAs for multi-DCI: Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, for the case when the UE does not support UL STxMP transmission, - for the baseline feature, the UE does not expect the two UL transmissions to overlap (i.e., scheduling restriction is applied to avoid overlap between the two UL transmissions) - as an optional feature, the overlapping duration of the later of the two UL transmissions is reduced. - FFS: for the optional feature, whether or not the overlapping duration needs to be specified as 1 (in case 2) or 2 (in case 1) OFDM symbols where - Case 1 applies when UE is capable of supporting MRTD > CP, SCS=60 kHz and frequency range is FR1. - Case 2 applies in all other cases A new FG 40-2-9 is needed to capture the following agreement in RAN1 #113: For SRS enhancements on TDD CJT, a number of agreements were made in RAN1 #113 that require new FGs as detailed below: - A pre-requisite is not needed for either FG 40-5-1 or 40-5-2. It is obvious that for a UE supporting these Rel-18 SRS hopping schemes, the basic SRS feature should be already supported, and there is no need to spell out / debate the basic SRS features. - The new FGs 40-5-1c (for comb offset hopping) and 40-5-2c (for cyclic shift hopping) below are added as a result of the following agreement: Agreement Support configuring a subset of comb offsets when comb offset hopping is configured, and configuring a subset of cyclic shifts when cyclic shift hopping is configured. - The subset configuration applies to all the port(s) in the SRS resource, and all the port(s) in the SRS resource has (have) the same hopping offset value khopping(pi) on an OFDM symbol. - This is a UE-optional feature. Agreement For SRS cyclic shift hopping, support finer time-delay-domain granularity, e.g., αi=2πnSRScs,inSRScs,max+2πnSRScs,offsetK×nSRScs,max, where nSRScs,offset can be randomly chosen from 0,1,…K×nSRScs,max-1 at each SRS transmission. - Note: The finer granularity above only applies to the cyclic shift offsets when cyclic shift hopping is enabled. If a subset for cyclic shifts is configured, this feature cannot be configured. Above is a UE optional feature. The new FG 40-5-2a (for cyclic shift hopping) is added as a result of the following agreement. 40. NR_MIMO_evo_DL_UL; 40-2-4a; Support of a PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA for intra-cell; Support of cross-TRP PDCCH order based on CFRA for intra-cell multi-DCI based mTRP; yes; N/A; Per band; No; No; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-2-9; Partial dropping in case of overlap; Support of reducing the overlapping duration of the later of the two UL transmissions, when two logically TDMed UL transmissions overlap due to different TAs; Yes; N/A; band; No; No; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-1-24; Support of for Rel-16-based CJT type-II codebook with NTRP={3,4}; Parameter combination {1,2,3,4,5} with NTRP=3, and {1,2,3,4} with NTRP=4; 40-3-1-1 or 40-3-1-a; Yes; N/A; Per band and Per-BC; N/A; N/A; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-1-25; Timeline for regular eType-II-CJT CSI, or for port selection FeType-II-CJT CSI; Timeline relaxation parameter: r=Z2’ (for both Z' and Z);; 40-3-1-1, or 40-3-1-5; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-2-8; Support of rank 3,4 for Rel-16-based doppler CSI; 40-3-2-1; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-2-9; Support of rank 3,4 for Rel-17-based doppler CSI; 40-3-2-4; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-2-10; Timeline parameter “r” for Rel-16/17-based Doppler CSI; Timeline relaxation parameter: r=Z2’ (for both Z' and Z);; 40-3-2-1, or 40-3-2-4; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Optional with capability signaling. 41. NR_MIMO_evo_DL_UL; 40-3-2-11; Support of periodic CSI-RS resources for Rel-16/17-based Doppler CSI; 40-3-2-1, or 40-3-2-4; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-2-12; Active CSI-RS resources/ports counting for P/SP-CSI-RS; Counting number Kp for periodic CSI-RS; 40-3-2-11; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Candidate values: Kp={1,2,4}; Optional with capability signaling. 41. NR_MIMO_evo_DL_UL; 40-3-2-13; Timeline parameter “w” for Rel-16/17-based Doppler CSI; Maximum periodicity supported for P/SP-CSI-RS, which is used to determine measurement window size “w” for AP-report based on P/SP-CSI-RS; 40-3-2-11 and 40-3-2-12; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Candidate values: {4, 5, 8, 12} 2. For each periodicity of P/SP-CSI-RS no larger than reported in component 1 (maximum periodicity), w=Kp*periodicity; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-x; Active CSI-RS resources and ports for mixed Type-II-Doppler codebook types in any slot; 1. List of codebook combinations 2. List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination; 40-3-2-1,40-3-2-4,2-36,2-40; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Codebook 1 = {Type I SP, Type I MP} (Codebook 2, Codebook 3) = {(eType-II-Doppler R=1, NULL), (eType-II-Doppler R=2, NULL), (FeType-II-Doppler PS R=1 M=1, NULL), (FeType-II-Doppler PS R=1 M=2, NULL), (FeType-II-Doppler PS R=2 M=2, NULL), (FFS CB combos…)}; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-3-7; Maximum number of TDCP report settings per-BWP; Maximum number of CSI-ReportConfig with reportQuantity configured as [tdcp], configured with resourcesForChannelMeasurement linked to a same BWP ID; 40-3-3-1; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Candidate value: {1,2,3,4}; Optional with capability signaling. w40. NR_MIMO_evo_DL_UL; 40-4-2; Support Rel-18 DMRS with PDSCH processing capability 2; 1) Support Rel-18 DMRS with PDSCH processing capability 2 2) Additional number of OFDM symbols needed to support Rel-18 DMRS with PDSCH processing capability 2; 40-4-1; Yes; n/a; Per FS; No; No; n/a; Candidate values for component 2 = {0, 1, 2, 3, 4} OFDM symbols.; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-4-6b; Support 1+2 DMRS (uplink); 1. Support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port; YES; N/A; [Per FS]; NO; NO; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-4-11; Additional row(s) for antenna ports (0,2,3) for Rel.18 DMRS ports for single-DCI based M-TRP for PUSCH; Support of new DMRS port entry {0,2,3} for single-DCI based SDM STxMP PUSCH with Rel-18 DMRS; YES; N/A; [Per FS]; NO; NO; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-4-11a; Support Rel-18 UL DMRS with single-DCI based M-TRP; 1. Support Rel-18 UL DMRS with Single-DCI based M-TRP; YES; N/A; [Per FS]; NO; NO; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-4-11b; Support Rel-18 UL DMRS with M-DCI based M-TRP; 1. Support Rel-18 UL DMRS with M-DCI based M-TRP; YES; N/A; [Per FS]; NO; NO; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-4-12; PTRS for 8 Tx PUSCH; 1. Supported PTRS power boost configurations 2. Supported number of PTRS ports; YES; N/A; [Per FS]; NO; NO; N/A; Candidate values for component 1 = {ptrs-Power configures 00, ptrs-Power configures 01, both ptrs-Power configures 00 and 01} Candidate values for component 2 = {1 port, 2 ports}; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-5-2a; Smaller cyclic shift granularity for cyclic shift hopping; Support configuration of cyclic shift hopping with smaller granularity (with factor K=2); 40-5-2; Yes; N/A; Per band; n/a; n/a; n/a; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-5-1c; Comb offset hopping within a subset; Support configuration of subset of comb offsets for comb offset hopping; 40-5-1; Yes; N/A; Per band; n/a; n/a; n/a; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-5-2c; Cyclic shift hopping within a subset; Support configuration of subset of cyclic shifts for cyclic shift hopping; 40-5-2; Yes; N/A; Per band; n/a; n/a; n/a; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-5-8; 8-ports SRS for noncodebook PUSCH; 1. Support 8-ports SRS for noncodebook PUSCH, without SRS repetition and without SRS frequency hopping 2. Support 8-ports SRS for noncodebook PUSCH, with SRS repetition and without SRS frequency hopping 3. Support 8-ports SRS for noncodebook PUSCH, without SRS repetition and with SRS frequency hopping 4. Support 8-ports SRS for noncodebook PUSCH, with SRS repetition and with SRS frequency hopping 5. Supported repetition factor R 6. Supported number of SRS symbols m in one slot 7. Supported max number of SRS resource per set (SRS set use is configured as for non-codebook transmission). 8. Maximum number of simultaneous transmitted SRS resources at one OFDM symbol; Yes; N/A; Per FSPC; No; No; N/A; Candidate value for component 5 = a subset of {1, 2, 4, 5, 6, 7, 8, 10, 12, 14} Candidate value for component 6 = a subset {1, 2, 4, 8, 10, 12, 14} Candidate value for component 7 = {1,2,3,4,5,6,7,8} Candidate value for component 8 = {1,2,4,8}; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-7-4; (N1,N2) for coherent codebook; (N1,N2) for coherent codebook; YES; N/A; Per FSPC; NO; NO; NO; Candidate values = {(4,1) only, (2,2) only, both (4,1) and (2,2)}; Optional with capability signaling. |
| Huawei/HiSilicon [19] | Moreover, it has been agreed that a set of NL combinations of values for {L1, ..., LNTRP} could be gNB-configured and UE should select one and report. The total number of Ln can be up to 16 for the candidate {L1, ..., LNTRP} combinations, which is larger than the value of sTRP codebooks (i.e. 6), and it would increase the UE complexity significantly. It is natural to add a UE capability of the max supported total number of SD basis vectors, so that gNB can configure the {Ln} combinations properly. Hence, we have the following proposal. Proposal 3-2: adopt the adopt the following additional FGs for CJT CSI enhancement. 40. NR_MIMO_evo_DL_UL; 40-3-1-x; The max total number of SD basis vectors (Ltot,max) for Rel-16-based CJT type-II codebook; Candidate values of Ltot,max for one CJT CSI measurement: {6, 8,12,16}. 40. NR_MIMO_evo_DL_UL; 40-3-1-x; The max total number of selected ports (2Ltot,max) for Rel-17-based CJT type-II codebook; Candidate values of alpha_tot,max for one CJT CSI measurement: {0.5,0.75,1} And the max total number of selected ports (2Ltot,max) equals (alpha_tot,max * max # of CSI-RS ports).. Considering the higher channel estimation complexity brought by length-4-FD-OCC-based Rel.18 DMRS, the support of the combination of Rel.18 DMRS and other features with high timing requirement/channel estimation complexity should be enabled by separate UE features if supported. First, for PDSCH processing time, it is agreed in RAN1#114bis to introduce a UE feature group to indicate whether/how to support Rel-18 DMRS and PDSCH processing capability 2 simultaneously Agreement Introduce a UE feature group to indicate the whether/how to support Rel-18 DMRS and PDSCH processing capability 2 simultaneously - In this feature group, the UE can additionally report relaxation on processing delay for PDSCH processing capability 2 - FFS: Details of the relaxation on processing delay Thus we have the following proposal Proposal 4-1: Following Rel.18-DMRS-related UE capabilities should be introduced: - FG 40-4-1x: indicate whether UE can simultaneously support Rel-18 DMRS and PDSCH processing capability 2. - The candidate value can be {support, support with x symbols relaxation on processing delay} where x can be 1 or 2 - Consequence if the feature is not supported by the UE: UE cannot simultaneously support Rel-18 DMRS and PDSCH processing capability 2 Next, due to the high complexity of multi-CDM-group channel estimation, a UE capability indicating the simultaneous support of cross-CDM-group Rel.18 DMRS port combination for one codeword should be introduced. Proposal 4-2: Following Rel.18-DMRS-related UE capabilities should be introduced: - UE capability 2: indicate whether UE can support cross-CDM-group Rel.18 DMRS port combinations for one codeword. In addition, in current spec., there exists a UE capability indicating the supported maximum number of DL MIMO layers with candidate value being chosen from 2, 4, and 8, among which 8 aims at boosting the SU performance. Given that Rel.18 DMRS targets at enabling higher MU-MIMO layers to improve MU performance, the ‘mutually-exclusive’ application scenario makes it unreasonable to simultaneously support Rel.18 DMRS and 8-layer DL MIMO, letting alone the unacceptable multiplicative channel estimation complexity of them. Or at least, a UE capability indicating whether Rel.18 DMRS and >4 layers DL MIMO are simultaneously supported should be introduced. Proposal 4-3: Support to introduce a UE capability indicating whether Rel.18 DMRS and >4 layers DL MIMO are simultaneously supported. 40. NR_MIMO_evo_DL_UL; 40-4-1x; simultaneously support Rel-18 DMRS and PDSCH processing capability 2; simultaneously support Rel-18 DMRS and PDSCH processing capability 2; 40-4-1; Yes; n/a; simultaneously support Rel-18 DMRS and PDSCH processing capability 2 is not supported; [Per FS]; No; No; n/a; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-4-1y; Simultaneously support Rel.18 DMRS and >4 layers DL MIMO; Simultaneously support Rel.18 DMRS and >4 layers DL MIMO; 40-4-1; Yes; n/a; Simultaneously support Rel.18 DMRS and >4 layers DL MIMO is not supported; Per FS; No; No; n/a; Optional with capability signaling. |

# Conclusion

Agreements reached during RAN1 #115 as part of this agenda item are summarized in [20].

# References

- R1-2310635, Updated RAN1 UE features list for Rel-18 NR after RAN1#114bis, Moderators (AT&T, NTT DOCOMO, INC.)

- R1-2310889, On UE features for Rel. 18 MIMO evolution WI, Nokia/Nokia Shanghai Bell

- R1-2310907, Discussion on UE features for Rel18 MIMO, Ericsson

- R1-2310956, Discussion on UE features for NR MIMO evolution, ZTE

- R1-2311049, Discussion on UE features for NR MIMO evolution, Fujitsu

- R1-2311117, Discussion on Rel-18 MIMO UE features, vivo

- R1-2311150, UE features for NR-MIMO evolution, Intel Corporation

- R1-2311220, UE features for Rel-18 MIMO evolution, OPPO

- R1-2311328, Further discussion on UE features for NR MIMO evolution, CATT

- R1-2311392, Further discussion on UE features for NR MIMO evolution, xiaomi

- R1-2311438, Discussion on Rel-18 MIMO UE features, LG Electronics

- R1-2311501, Discussion on UE features for NR MIMO evolution, CMCC

- R1-2311576, UE feature on NR MIMO evolution, Google

- R1-2311642, Discussion on UE features for NR MIMO evolution, NTT DOCOMO, INC.

- R1-2311706, Views on UE features for Rel-18 NR MIMO evolution, Apple

- R1-2311867, UE features for Rel-18 MIMO, Samsung

- R1-2311974, UE Features for NR MIMO Evolution, MediaTek Inc.

- R1-2312061, UE features for NR MIMO evolution, Qualcomm Incorporated

- R1-2312152, UE features for NR MIMO evolution, Huawei/HiSilicon

- R1-2312608, Session Notes of AI 8.16.1, Ad-Hoc Chair (AT&T)