3GPP TSG-RAN WG1 Meeting #115	Tdoc R1-2312097

Chicago, IL, November 13th – 17th, 2023

Agenda Item:	7.2

Source:	Ericsson

Title:	Discussion on CSI related timeline for 480/960 kHz

Document for:	Discussion, Decision

# 1	Introduction

Issue#3: CSI related timeline for 480/960 kHz

R1-2309300	Discussion on CSI related timeline for 480/960 kHz	LG Electronics

Conclusion

This issue can be discussed again at a future meeting.

The Chairman notes for RAN#114bis contains the following guidance for discussion on FR2_2 maintenance at a future meeting:

A summary of the discussion on Issue#3 from RAN#114bis is contained in [1].

# 2	Discussion

According to 38.214, only the CSI computation delay requirement 2 is applicable for 480/960 kHz SCS. The relevant values of Z and Z’ are highlighted in the table below, where Z is the minimum number of symbols between the last symbol of the PDCCH triggering an ap-CSI report and the first symbol of the PUSCH carrying the report, and Z’ is the minimum number of symbols between the last symbol of the ap-CSI-RS and the first symbol of the PUSCH carrying the report.

Table 5.4-2: CSI computation delay requirement 2

|  | Z1 [symbols] |  | Z2 [symbols] |  | Z3 [symbols] |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Z1 | Z'1 | Z2 | Z'2 | Z3 | Z'3 |
| 0 | 22 | 16 | 40 | 37 | 22 | X0 |
| 1 | 33 | 30 | 72 | 69 | 33 | X1 |
| 2 | 44 | 42 | 141 | 140 | min(44, X3X2+ KB1) | X2 |
| 3 | 97 | 85 | 152 | 140 | min(97, X3+ KB2) | X3 |
| 5 | 388 | 340 | 608 | 560 | min(388, X5+ KB3) | X5 |
| 6 | 776 | 680 | 1216 | 1120 | min(776, X6+ KB4) | X6 |

According to 38.213 Section 9.2.5, for the case when there is an aperiodic CSI report multiplexed in a PUSCH in a group of overlapping PUCCHs and PUSCHs in a slot, a small amount of extra CSI processing time is allowed. In this case Z and Z’ are increased by d symbols, where d=2 for μ=0,1, d=3 for μ=2, and d=4 for μ=3.

Currently, there is no definition of d for the case of 480/960 kHz, i.e, μ=5/6. In RAN1#114bis, three alternatives were discussed, but there was no conclusion:

- Alt-1: d=16 for 480 kHz and d=32 for 960 kHz

- Alt-2: d=0 for both 480 kHz and 960 kHz

- Alt-3: d=4 for both 480 kHz and 960 kHz

In our view, the CSI processing time is very long to start with since only processing requirement 2 is supported for 480/960 kHz SCS. For example for 480 kHz SCS, Z2 is 608 symbols (~44 slots) already, hence extending this by 16 symbols (~1 slot), is not well motivated. Hence we prefer Alt-2.

Support Alt-2, i.e., in 38.213 Section 9.2.5, define d in Tproc,CSImux and Z'proc,CSI mux as d=0 for both μ=5 and μ=6.

# Conclusions

Based on the discussion in this paper we propose the following:

Proposal 1	Support Alt-2, i.e., in 38.213 Section 9.2.5, define d in Tproc,CSImux and Z'proc,CSI mux as d=0 for both μ=5 and μ=6.

# References

R1-2310446, “Summary #2 of FR2-2 maintenance,” Moderator (LG Electronics), RAN1#114bis, October 2023.