3GPP TSG-RAN WG1 Meeting #115	R1-2312149

Chicago, USA, November 13 – 17, 2023

Agenda Item:	8.15.1

Source:	Huawei, HiSilicon

Title:	Evaluation methodology of IMT-2020 satellite

Document for:	Discussion and Decision

# Introduction

In RAN1#114bis meeting, the evaluation methodologies and evaluation configurations for the link budget template were discussed, and the following agreements were made [1].

Agreement Proposal 1.1 in section 6 of R1-2310471 is endorsed with TBS value 1736 for NR NTN Rural eMBB-s table. Agreement For the link budget template, the target elevation angle is 30 degrees, and the link budget template includes the link margin (in dB) at that elevation angle. Agreement Include the receiver interference in the link budget template for NTN, and reuse the following average INR results from TR 38.821: - 4.4 dB for FRF3 DL, - 7.6 dB for FRF3 UL

In this contribution, remaining issues for evaluation methodologies for eMBB-s and evaluation results for the link budget template and are discussed.

# Discussion on bandwidth used for spectral efficiency related evaluations

In RAN1#113 meeting, it was agreed that “For FRF3, 30MHz is the total maximum bandwidth (i.e., each beam will have 10MHz maximum bandwidth)”. However, for spectrum efficiency calculation of FRF=3, companies seem have different calculation in last meeting. Some companies use 10MHz channel bandwidth, while other companies use 30MHz channel bandwidth.  According to section 7.2.5 of M.2514 [2], “Average spectral efficiency is the aggregate throughput of all users (the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time) divided by the channel bandwidth of a specific band divided by the number of TRxPs and is measured in bit/s/Hz/TRxP”, “The channel bandwidth for this purpose is defined as the effective bandwidth times the frequency reuse factor.”.  Therefore 30MHz (instead of 10MHz) should be used for calculation of spectrum efficiency for FRF=3.

In addition, for the evaluation related to spectral efficiency and bandwidth, i.e., user experienced data rate and area traffic capacity, 30MHz should be used for calculation for FRF=3 as well.

Proposal 1: 30 MHz should be used for the calculation of average spectral efficiency, 5th percentile user spectral efficiency, user experienced data rate and area traffic capacity for FRF=3. Results from different sources should be checked before being captured in the TR for averaging.

# Discussion on link budget

## Link budget template

According to TR 37.910 [3], the link budget template only shows one required SNR value for one configuration (combination of a scenario of a physical channel), while other templates capture the results from multiple companies and calculate a mean value for one configuration.

As simulation assumptions for link budget evaluation are aligned based on the agreements in last meeting, the required SNR values from different companies should be captured in link budget template and a mean required SNR value can be calculated. Then the coverage margin at minimum elevation angle should be calculated with the mean value of required SNR.

Proposal 2:  Required SNR values from different companies should be captured in link budget template and a mean required SNR value can then be calculated. Coverage margin at minimum elevation angle should be calculated against the mean required SNR value.

## Link budget results

### NR NTN Rural eMBB-s

Based on the agreements in last meeting [4], the simulation assumptions for NR NTN Rural eMBB-s PDSCH and PUSCH are listed in Table 1.

Table 1: Simulation assumptions for NR NTN Rural eMBB-s PDSCH and PUSCH

|  | PDSCH |  |  |  | PUSCH |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | TBS | BW | #reps | Target BLER | TBS | BW | #reps | Target BLER |
| Value | 1736 | 9.36 MHz | 1 | 10% | 184 | 1PRB | 20 slots | 10% |

The excel document Link budget template NTN.xlsx captures these results for FRF=3 and PDSCH/PUSCH of NR NTN Rural eMBB-s. The evaluation results for required SNR and coverage margin at minimum elevation angle are shown in Table 2.

Table 2: Link budget evaluation results for NR NTN Rural eMBB-s PDSCH and PUSCH

|  | PDSCH | PUSCH |
| --- | --- | --- |
| Required SNR (dB) | -4.5 | -4.6 |
| Coverage margin at minimum elevation angle (dB) | 2.7 | 4.7 |

### NR NTN Rural HRC-s

Based on the agreements in last meeting [4], the simulation assumptions for NR NTN Rural HRC-s PDSCH and PUSCH are listed in Table 3.

Table 3: Simulation assumptions for NR NTN Rural HRC-s PDSCH and PUSCH

|  | PDSCH |  |  |  | PUSCH |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | TBS | BW | #reps | Target BLER | TBS | BW | #reps | Target BLER |
| Value | 256 | 1.44MHz | 4 | 0.1% | 256 | 1PRB | 16 slots, HARQ enabled | 0.1% |

The evaluation results for required SNR and coverage margin at minimum elevation angle are shown in Table 4.

Table 4: Link budget evaluation results for NR NTN Rural HRC-s PDSCH and PUSCH

|  | PDSCH | PUSCH |
| --- | --- | --- |
| Required SNR (dB) | -4.6 | -1.0 |
| Coverage margin at minimum elevation angle (dB) | 2.8 | 1.1 |

The excel document Link budget template NTN.xlsx captures these results for PDSCH and PUSCH of NR NTN Rural HRC-s.

### NR NTN Rural mMTC-s

Based on the agreements in last meeting [4], the simulation assumptions for NR NTN Rural mMTC-s PDSCH and PUSCH are listed in Table 5.

Table 5: Simulation assumptions for NR NTN Rural mMTC-s PDSCH and PUSCH

|  | PDSCH |  |  |  | PUSCH |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | TBS | BW | #reps | Target BLER | TBS | BW | #reps | Target BLER |
| Value | 184 | 1.08MHz | 1 | 10% | 256 | 1PRB | 20 | 10% |

The evaluation results for required SNR and coverage margin at minimum elevation angle are shown in Table 6.

Table 6: Link budget evaluation results for NR NTN Rural mMTC-s PDSCH and PUSCH

|  | PDSCH | PUSCH |
| --- | --- | --- |
| Required SNR (dB) | -4.6 | -3.5 |
| Coverage margin at minimum elevation angle (dB) | 2.8 | 3.6 |

The excel document Link budget template NTN.xlsx captures these results for PDSCH and PUSCH of NR NTN Rural mMTC-s.

### NB-IoT NTN Rural mMTC-s

Based on the agreements in last meeting [4], the simulation assumptions for NB-IoT NTN Rural mMTC-s NPDSCH and NPUSCH fmt1 are listed in Table 7.

Table 7: Simulation assumptions for NB-IoT NTN Rural mMTC-s NPDSCH and NPUSCH fmt1

|  | NPDSCH |  |  |  | NPUSCH fmt1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | TBS | BW | #reps | Target BLER | TBS | BW | #reps | Target BLER |
| Value | 256 | 180kHz | 4 reps, 10subframes per rep (total 40ms) | 10% | 256 | 15kHz | 8 reps, 4 RUs (8 subframes per RU) | 10% |

The evaluation results for required SNR and coverage margin at minimum elevation angle are shown in Table 8.

Table 8: Link budget evaluation results for NB-IoT NTN Rural mMTC-s NPDSCH and NPUSCH fmt1

|  | NPDSCH | NPUSCH fmt1 |
| --- | --- | --- |
| Required SNR (dB) | -3.3 | -4.7 |
| Coverage margin at minimum elevation angle (dB) | 1.5 | 15.6 |

The attached excel document Link budget template NTN.xlsx captures these results for PDSCH/PUSCH of NR NTN and NPDSCH/NPUSCH fmt1 of NB-IoT NTN with FRF=3.

Proposal 3: Capture the evaluation results in the attached excel in TR 37.911.

# Conclusions

This contribution provides our considerations on configurations and simulation results for link budget, and our proposals are as follows:

Proposal 1: 30 MHz should be used for the calculation of average spectral efficiency, 5th percentile user spectral efficiency, user experienced data rate and area traffic capacity for FRF=3. Results from different sources should be checked before being captured in the TR for averaging.

Proposal 2:  Required SNR values from different companies should be captured in link budget template and a mean required SNR value can then be calculated. Coverage margin at minimum elevation angle should be calculated against the mean required SNR value.

Proposal 3: Capture the evaluation results in the attached excel in TR 37.911.

# References

- Report of RAN1#114bis, Xiamen, China, 9th – 13th October 2023.

- ITU-R M.2514, "Vision, requirements and evaluation guidelines for satellite radio interface(s) of IMT-2020".

- TR 37.910, “Study on self evaluation towards IMT-2020 submission”.

- R1-2310471, “Feature lead summary on Evaluation Methodology for IMT-2020 satellite”, RAN1#114bis, Xiamen, China, 9th – 13th October 2023.