3GPP TSG-RAN WG1 Meeting #115	R1-2312214

Chicago, USA, 13 – 17 November, 2023

Agenda Item:	7.2

Source:	Huawei, HiSilicon, China Unicom

Title:	Discussion on SRS spatial relation

Document for:	Discussion and decision

# Introduction

In RAN1#114bis, RAN1 discussed the issue of UE SRS spatial relation in R1-2310258 [1] and reached the following conclusion.

Conclusion Current specification does support the scenario that “two (or more) positioning SRS resources are configured/activated with the same spatial relation RS”. The enhancement regarding this aspect is not pursued in RAN1 in Rel-17 maintenance.

In this paper, we discuss the possibility of resolving this in Rel-18 specification.

# Discussion

Based on the discussion in FL summary R1-2310306 [2], it appears that most companies believed that interpretation 1 should be the case, i.e. UE uses the same Tx beam for SRS resources configured with the same spatial relation, while at the same time, the exact Tx beam behavior is still up to UE implementation, since there may be another factors that impact UE beamforming behavior.

Question 3-3: If your answer is “Yes” to Question 3-2, what is your view on the following proposal from Huawei”? Proposal 1: RAN1 to discuss UE behavior when two positioning SRS resources are configured/activated with the same spatial relation RS, especially for the case when UE supports path specific power measurement. Proposal 2: RAN1 should discuss which of the following interpretation should be used when two positioning SRS resources are configured/activated with the same spatial relation RS. - Interpretation 1: UE is required to transmit the positioning SRS with the same Tx beam - Interpretation 2: UE is required to transmit the positioning SRS with different Tx beams Company; comments. Samsung; Discussion is not needed. The UE beahvoir should be interpretation 1 but the exact behaviour of determining the tx beam will be up to UE implementation in the case of Figure 1, because UE can find two tx directions are good for same PRS. The general BM procedure (if ends up using the DL PRS as spatial relation), it’s DL measurement done by UE reported to gNB and UE understandts for which tx beam(s) is good. Although this proposal’s intention is to focusing on the path specific optimization, but the spatial relation indication is based on resource specific, if one resource is capable of two tx beam from UE implementation perspective, then it should leave UE to use which one(s). because at end of the day, it’s about gNB reception of the SRS, if by previous BM procedure, which tx (path beam) does not matter, meaning can qualify the RAN4 requirement, then the gNB reception should be fine as well.. Qualcomm; We don’t believe that such a discussion should be done within Rel-17 maintenance. Spatial relation is resource specific and not path-specific, so generally “interpretation 1” is closer to what we would typically expect, but still, it should be up to UE implementation to use a different Tx beam assuming any legacy RAn4 requirements are met. So, a UE, up to implementation, and assuming all requirements can be met, may use same or different Tx beams for 2 SRS resources that have same spatial relation.. ZTE; The proposal my be beneficial in FR2. However, it is to introduce a totally new feature which needs more study and evaluation. It is more like a Rel-19 enhancement. We don’t think it is appropriate to discuss it here. We agree the current spec is not perfect, but that is the true situation.. CATT; Share the similar view as Qualcomm that such a discussion might be difficult within Rel-17 maintenance.. Huawei, HiSilicon; Our understanding is that the spatial relation is key to FR2 positioning, and if we leave everything up to UE implementation, the performance, under multi-path condition cannot be guaranteed.. FL; All of the feedbacks acknowledge that current specification support “two (or more) positioning SRS resources are configured/activated with the same spatial relation RS”. However, most companies do not think this issue should/can be resolved within Rel-17 maintenance..

Observation 1: There is consensus that no clear behavior was defined for the UE Tx beamforming on multiple positioning SRS resources configured with the same spatial relation.

Afterwards, there was also discussion on whether it can be considered in Rel-18, and due to lack of discussion, there was no conclusion [2], either.

(Round 2) Proposal 3 (as a conclusion) - Current specification does support the scenario that “two (or more) positioning SRS resources are configured/activated with the same spatial relation RS”. However, defining UE behavior for this scenario may need significant effort, and thus, may be considered in a future release but not in Rel-17 maintenance. Company; comments. Samsung; Suggested wording, “However, defining UE behavior for this scenario may need significant effort, and thus, may be considered in a future release but not in Rel-17 maintenance.further enhancement regarding this aspect is not pursued in RAN1 for Rel18.”. FL; For Samsung’s comment, we are discussing the proposals submitted for Rel-17 maintenance. Thus, it is not suitable to say not pursued in Rel-18. We could simply the conclusion as: - Current specification does support the scenario that “two (or more) positioning SRS resources are configured/activated with the same spatial relation RS”. The enhancement regarding this aspect is not pursued in RAN1 in Rel-17 maintenance..

Observation 2: Companies have not expressed their view sufficiently whether this issue can be fixed in Rel-18.

In our view, there could be different network expectations on the UE beamforming schemes:

If network expects to use multiple SRS resources for the Rx beam sweeping for the sake of UL AoA measurement, the UE should use the same Tx beam.

If network expects to explore additional spatial propagation information based on multiple SRS resources, the UE should use different Tx beams.

It is worth noting that it now has become a critical issue for implementing mmWave positioning based on the current specification.

Observation 3: The ambiguity of UE beamforming is a critical issue for implementing mmWave positioning.

In this way, we think it should be settled within Rel-18 timeframe. A simple way of correcting this would be introducing a new signaling from network indicating whether UE is expected to transmit SRS using the same Tx beam or different Tx beams for SRS resources configured with the same spatial relation.

Proposal 1: Introduce a new RRC parameter for a positioning SRS resource set in Rel-18, indicating whether the UE is expected to use the same or different spatial transmission filters for the positioning SRS resources configured with the same spatial relation RS.

No new RAN4 requirement is expected from RAN1 perspective.

Send an LS to RAN2 on the higher layer signaling impact.

Proposal 2: Agree the draft CR in R1-2312215 for Rel-18 TS38.214.

# Conclusion

In this contribution, we discussed the positioning SRS spatial relation associated with the same DL PRS. Based on the discussion, we have the following observations and proposals.

Observation 1: There is consensus that no clear behavior was defined for the UE Tx beamforming on multiple positioning SRS resources configured with the same spatial relation RS.

Observation 2: Companies have not expressed their view sufficiently whether this issue can be fixed in Rel-18.

Observation 3: The ambiguity of UE beamforming is a critical issue for implementing mmWave positioning.

Proposal 1: Introduce a new RRC parameter for a positioning SRS resource set in Rel-18, indicating whether the UE is expected to use the same or different spatial transmission filters for the positioning SRS resources configured with the same spatial relation RS.

No new RAN4 requirement is expected from RAN1 perspective.

Send an LS to RAN2 on the higher layer signaling impact.

Proposal 2: Agree the draft CR in R1-2312215 for Rel-18 TS38.214.

# Reference

- R1-2310258, Discussion on SRS spatial relation, Huawei, HiSilicon, RAN1#114bis, Xiamen, China, 9 – 13 Oct., 2023.

- R1-2310306, FL Summary #1 for Maintenance on Rel-17 NR Positioning, Moderator (CATT), RAN1#114bis, Xiamen, China, 9 – 13 Oct., 2023.