3GPP TSG-RAN WG1 Meeting #115	R1-2312239

Chicago, USA, November 13 – 17, 2023

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	Discussion on required DCI signalling for advanced receiver on MU-MIMO scenario

Document for:	Discussion and Decision

Introduction

Revised WID on NR demodulation performance evaluation [1] was approved at RAN meeting #100, and one RAN1 related objective is to consider how to introduce the DCI based assistant signalling to support advanced receiver to cancel inter-user interference in MU-MIMO scenario:

- Introduce network assistant signaling to support advanced receiver to cancel inter-user interference for MU-MIMO according to R4-2309895 [RAN4, RAN1, RAN2]: - Identify the required signalling for supporting advanced receiver to cancel inter-user interference for MU-MIMO [RAN4] - Introduce DCI based assistant signalling [RAN1] - Introduce RRC based assistant signalling and UE capability [RAN2]

As a response to RAN1 LS (R1-2308598), RAN4 has sent back the reply LS [2], answering the questions from RAN1. This contribution provides our views based on RAN4’s answers.

Discussion

For the introduction of new signalling to DCI format 1_2, the answer from RAN4 is as below

Question 1: Whether this new signaling in DCI is introduced in DCI format 1_2 in addition to format 1_1? Answer: The understanding in RAN4 is that URLLC is not a common scenario for MU-MIMO, but if there are relevant use cases with MU-MIMO scheduling with DCI format 1_2, the signalling in DCI can be introduced in DCI format 1_2, otherwise not.

Although DCI format 1_2 is designed under URLLC WI, it can also be used for eMBB also. DCI format 1_2 provides much flexibility in DCI configuration for network, e.g., to reduce DCI overhead. The benefits from R-ML receiver can be achieved in various scenarios supported by both DCI format 1_1 and format 1_2. Therefore, we propose to introduce the field for both DCI formats.

Proposal 1: The field to indicate information of modulation order is supported in DCI format 1_2.

On the support of the new DCI field for multi-TRP scenarios, the answer from RAN4 is as below

Question 2: Whether this new signaling in DCI is supported for one or more DL multi-TRP schemes? Answer: The understanding in RAN4 is that there are limited scenarios for MU-MIMO with mTRP operation. RAN4 suggests that this new signalling in DCI is not supported for multi-TRP schemes.

To receive multi-TRP PDSCH, UE has to receive two TCI states. As a result, the complexity of detection of co-scheduled UE’s interference is increased using two TCI states. Considering the further UE complexity, whether to support multi-TRP schemes needs further discussion in RAN1.

Proposal 2: whether to support the new DCI field for DL multi-TRP schemes needs further discussion.

For the configuration of maxNrofCodeWordsScheduledByDCI as 2, the answer from RAN4 is as below

Question3: Whether this new signaling in DCI is supported when the RRC parameter maxNrofCodeWordsScheduledByDCI is configured as 2? Answer: This new DCI is supported if RRC parameter maxNrofCodeWordsScheduledByDCI is configured as 2 and target UE is only scheduled 1 codeword.

From RAN4’s answer, there may be MU-MIMO UEs when only one codeword is scheduled for the target UE, and the target UE can interpret the new DCI field to extract the knowledge of co-scheduled UEs. When 2 codewords are scheduled for the target UE, UE should not interprete the field to avoid ambiguity in interpreting the field.

Proposal 3: The new field is reserved if 2 codewords are scheduled.

Based on the above discussion, we have the following text proposal:

Proposal 4: Endorse the following text proposal to 38.212:

#### 7.3.1.2 DCI formats for scheduling of PDSCH ##### 7.3.1.2.2 Format 1_1 DCI format 1_1 is used for the scheduling of one or multiple PDSCH in one cell. The following information is transmitted by means of the DCI format 1_1 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: - Identifier for DCI formats – 1 bits < Unchanged parts are omitted > - PUCCH Cell indicator – 0 or 1 bit. - 1 bit if higher layer parameter pucch-sSCellDyn is configured. - 0 bit otherwise. If the UE is configured with a PUCCH-SCell, pucch-sSCellDyn is replaced by pucch-sSCellDynSecondaryPUCCHgroup for the secondary PUCCH group. - Co-scheduled UE information – 0 or 3 bits. - 3 bits as defined in Table 7.3.1.2.2-12 if higher layer parameter XYZ is configured; - 0 bit otherwise. The field is reserved if two codewords are scheduled. < Unchanged parts are omitted > Table 7.3.1.2.2-12: Co-scheduled UE information Bit field mapped to index; Co-scheduled UE information. 0; In all the PRBs allocated to the UE, there is no co-scheduled UE or there is co-scheduled UE but with a different root DMRS sequence. 1; In all the PRBs allocated to the UE, all the co-scheduled UE(s), which have the same root DMRS sequence as the UE, are scheduled with modulation scheme QPSK. 2; In all the PRBs allocated to the UE, all the co-scheduled UE(s), which have the same root DMRS sequence as the UE, are scheduled with modulation scheme 16QAM. 3; In all the PRBs allocated to the UE, all the co-scheduled UE(s), which have the same root DMRS sequence as the UE, are scheduled with modulation scheme 64QAM. 4; In all the PRBs allocated to the UE, all the co-scheduled UE(s), which have the same root DMRS sequence as the UE, are scheduled with modulation scheme 256QAM. 5; In all the PRBs allocated to the UE, all the co-scheduled UE(s), which have the same root DMRS sequence as the UE, are scheduled with modulation scheme 1024QAM. 6; In each individual PRB allocated to the UE, all the co-scheduled UE(s), which have the same root DMRS sequence as the UE, are scheduled with the same modulation scheme, except the cases corresponding to index 0~5. 7; All cases not covered above. Note: Root DMRS sequence is rn as defined in clause 7.4.1.1.1 of [4, TS 38.211]. < Unchanged parts are omitted > ##### 7.3.1.2.3 Format 1_2 DCI format 1_2 is used for the scheduling of PDSCH in one cell. < Unchanged parts are omitted > - PUCCH Cell indicator - 0 or 1 bit. - 1 bit if higher layer parameter pucch-sSCellDynDCI-1-2 is configured. - 0 bit otherwise. - Co-scheduled UE information – 0 or 3 bits. - 3 bits as defined in Table 7.3.1.2.2-12 if higher layer parameter XYZ is configured; - 0 bit otherwise. The field is reserved if two codewords are scheduled.

Conclusion

In this paper, we provide our views on required DCI signalling for advanced receiver on MU-MIMO scenario, with following proposal.

Proposal 1: The field to indicate information of modulation order is supported in DCI format 1_2.

Proposal 2: whether to support the new DCI field for DL multi-TRP schemes needs further discussion.

Proposal 3: The new field is reserved if 2 codewords are scheduled.

Proposal 4: Endorse the text proposal in section 2 to 38.212.

- References

- RP-231492 Revised WID on NR demodulation performance evolution

- R1-2310794 (R4-2317011) Reply LS on required DCI signalling for advanced receiver on MU-MIMO scenario, From: RAN4, To: RAN1, CC: RAN2.