3GPP TSG RAN WG1 #115			                                                                   R1-2312331

Chicago, USA, November 13rd- 17th, 2023

Agenda item:	8.7.2

Source: 	Moderator (CATT)

Title: 	Moderator summary on timing advance management for LTM: Round 1

Document for:	Discussion and Decision

- Introduction

In RAN #94e, the Rel-18 WID of Further NR mobility enhancements are approved [1]. In the approved WID, Timing Advance management is a part of RAN1 objectives,

To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction: - Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3] - Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1] - L1 enhancements for inter-cell beam management, including L1 measurement and reporting, and beam indication [RAN1, RAN2] - Note 1: Early RAN2 involvement is necessary, including the possibility of further clarifying the interaction between this bullet with the previous bullet - Timing Advance management [RAN1, RAN2] - CU-DU interface signaling to support L1/L2 mobility, if needed [RAN3] Note 2: FR2 specific enhancements are not precluded, if any. Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios: - Standalone, CA and NR-DC case with serving cell change within one CG - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected) - Both intra-frequency and inter-frequency - Both FR1 and FR2 - Source and target cells may be synchronized or non-synchronized

This summary includes the following:

- Summary of companies’ views on each of open issues raised by interested companies

- Observations and recommended proposals based on the summary of companies’ views

- Issue 1: Enhancement for PDCCH-ordered RACH

Open issues on PDCCH-ordered RACH for TA acquisition of candidate target cell(s) and company views are summarized below.

Table 1. Summary of views on Issue 1

| # | Issue | Companies’ views |
| --- | --- | --- |
| 1-1 | According to RAN2 agreement in RAN2 #123 meeting, when PRACH transmission for candidate cell(s) and Msg1/MsgA or Msg3 serving cell overlaps, it is up to UE implementation to handle collisions regardless simultaneous transmission is supported by UE. This is not aligned with RAN1’s agreement. RAN2#123 agreement It is up to UE implementation to handle the RACH initiation collisions where the early RACH is getting involved. No specification change can be foreseen.. | - UE behavior needs further clarification when PRACH transmission for candidate cell(s) and Msg 1, or Msg A, or Msg3 for the serving cell overlaps in the time domain - vivo |

Companies are encouraged to show views/comments/suggestions in the following tables.

P1-1: Clarification of UE behaviour when PRACH transmission for candidate cell(s) and Msg 1, or Msg A, or Msg3 for the serving cell overlaps in the time domain

Proposal 1-1: TBD

.

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |
| Ericsson | The intent of the RAN1 agreement was not MsgA, Msg3: it was other types of UL transmissions. We could be OK to clarify that. |
| QC | This is up to UE implementation based on RAN2 agreement. RAN1 agreement is for other cases |
| Samsung | We don’t think RANs’s that agreement requesting any change in RAN1’s agreement. LS for sharing RAN1’s decision will be beneficial |
| NOKIA | As per the following RAN1 agreement, Msg1/MsgA/Msg3 to serving cell will be dropped, which may create some issues as its more important to keep the connection with the serving cell instead of performing a PRACH for a candidate cell. We prefer to keep the decision for this scenario on the UE implementation (assuming a good implementation will prioritize PRACH to the serving cell) instead of following the below agreement - the RAN1 agreement should only apply to other cases. Agreement When the UE does not support simultaneous/parallel transmissions of PRACH in candidate cell and UL channels and signals in serving cell, support - serving cell UL TX is dropped. |
| Lenovo | Same view with Ericsson and we are OK to clarify it. |
| ZTE | UE implementation is preferred. |
| NTT DOCOMO | According to RAN2 colleague, this agreement does not mean the collision of RACH. It means that the collision of RACH does not happen as in legacy. That’s why if there are any problems, it should be discussed in RAN2. |
| Futurewei | RAN2 decision should be fine. It is for the collision of the serving cell RACH and PDCCH ordered RACH. Since in different scenarios, when concurrent serving cell and candidate/target cell random access occurs, the priority of RA to the serving cell vs RA to the candidate cell can be different and only UE knows all the causes of the RA. For example, if the serving cell RA is triggered by serving cell link failure while candidate cell RA is only for PDCCH ordered early PRACH, then the serving cell RA should take high priority, but if the candidate cell RA is triggered for target cell HO, then the target cell RA should have high priority. Therefore, it is the best to let UE make the decision. |
| Mod | Based on discussions above, clarification on this issue would be beneficial. Proposal 1-1 (proposed conclusion): It is up to UE implementation to handle the RACH initiation collisions where the early RACH is getting involved. The following RAN1 agreement applies to other types of UL transmissions. Agreement When the UE does not support simultaneous/parallel transmissions of PRACH in candidate cell and UL channels and signals in serving cell, support - serving cell UL TX is dropped. |
| Huawei, HiSilicon | RAN1 agreed the PRACH transmission for candidate cell has higher priority than other UL transmission in serving cell. To our understanding, the RAN2 agreement is general for the whole RACH procedure not specific for a channel. So, we think no need to clarify. |
| Xiaomi | Agree with Nokia. |

- Issue 2: UE-based measurement for TA acquisition

Open issues on TA acquisition of the candidate target cell(s) for UE-based measurement for TA acquisition and company views are summarized below.

Table 2. Summary of views on Issue 2

| # | Issue | Companies’ views |
| --- | --- | --- |
| 2.1 | UE-based TA acquisition for candidate cell(s) | - Spreadtrum - Only support synchronized case for UL-based TA measurement in Rel-18 LTM, not support asynchronous case. - Huawei - UE based TA acquisition is only performed in the scenario that TAE between the target cell and source cell is smaller than 260ns in FR1. The TAE is negligible in such scenario and no need to indicate to UE. Adopt TP#2 in clause 21 of TS38.213. - ZTE: Support UE-based TA acquisition in FR1 only. - UE determined TA value and the corresponding candidate cell identification should be reported to network prior to the reception of the cell switch command. - Ericsson - It is up to UE implementation to determine for which candidate cells the TA is estimated. - Samsung: - UE is configured by RRC for which cell UE-based TA acquisition should be performed. - It is up to UE when to start UE-based candidate cell TA calculation, but the calculation should be finished before UE’s UL transmission for LTM process. - OPPO: To complete UE-based TA acquisition - The system provides the UE with the Tx time difference between the serving cell and one candidate cell - The UE reports whether it has a valid TA value of the candidate cell to the system through a MAC CE. - CMCC: - With the configuration of UE based TA measurement, the MAC CE for TCI state activation for the candidate cell and/or cell switching command MAC CE can be used for the activation of TA measurement. Two options can be considered for the UE based TA measurement. - Option 1: UE can derive the TA of the candidate cells without any reporting and confirmation from the serving cell. - Option 2: UE should report the timing gap or estimated TA to the serving cell. And the serving cell can indicate the actual TA values within the cell switching command. - Nokia: - Candidate cell TCI activation/deactivation MAC CE is used to initiate UE-based TA measurements on the LTM candidate cells/RS for which TCI state(s) are activated and stop the UE-based TA measurements on other configured LTM candidate cells/RSs. - The UE is provided with the estimation error for a candidate cell which the UE uses with all future UE-based TA estimation for the candidate cell. - NTTDoCoMo: - UE-based TA measurement for indicated candidate cells is triggered by MAC CE for TCI state activation for candidate cells, or by cell switch command MAC CE if not triggered by MAC CE for TCI state activation for candidate cells. - CAICT: - DL Tx timing difference between the serving and candidate cells need to be provided to UE - gNB can dynamically (de)activate the UE based TA update for a candidate cell via MAC-CE - Interdigital - Triggering TA measurement is left to UE implementation for UE-based TA measurement. - For asynchronous scenarios, gNB should at least provide DL Tx timing difference between the serving cell and candidate cells. |
| 2.2 | Coexistence of TA acquisition mechanisms | - Introduce the priority rule for UE to determine TA in the target cell for first UL transmission when UE is configured multiple TA acquisition mechanisms, i.e. TA indicated in Cell Switch Command > TA acquired by UE itself - ZTE, Ericsson, Samsung, Huawei(TA indicated in CSC > TA acquired by UE itself > TA acquired by RACH after CSC on the pre-configured resource) - OPPO, LGE: Do not support configuring both UE-based TA measurement and PDCCH order RACH-based TA acquisition simultaneously. - Nokia: When TAs available from both RACH-based TA acquisition and UE-based TA Estimation, the TA acquired most recently should be used. |

P2-1: Use case for UE based TA acquisition

Proposal 2-1: TBD

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |
| Ericsson | Not sure if anything is needed. It is up to the NW to configure which cell pairs use UE-based TA estimation: there is no need to introduce restrictions in the specification (e.g., TAE, or frequency range). We will not support configuring the Tx timing difference, since that is unknown in asynchronous cases. If it would be known, the NW would adjust the Tx timing. |
| QC | For 2.1, no any restriction is needed For 2.2, fine for TA in CSC > UE based TA |
| Futurewei | UE based TA determination is the only scheme in Rel-18 supporting the true RACH-less and fast cell switch. It is desirable to employ this solution to common mobility scenarios where source and target cells are not collocated and therefore not precisely synchronized. On the other hand, RAN4 indicates a very strict TAE requirement (less than 260 ns) for UE based TA determination to meet the TA accuracy requirement. As noted in the LS, only collocated source/target CCs could meet such a precise synchronization requirement, which represents only corner scenarios. To allow the UE based TA measurement is usable in normal mobility scenarios, the network should provide DL Tx timing difference between the serving and candidate cells to the UE for the UE to compensate the TAE. After the compensation, the residual TAE can be less than 260ns and the UE determined TA can meet the accuracy requirement in common mobility scenarios. @Ericsson, the NW can easily measure the timing difference between two nodes. But the cost to precisely synchronize all the nodes in the network can be increased significantly and not affordable. So practically we don’t do it and it is not necessary to perform high precise network synchronization. |
| NOKIA | We can first focus on the synchronous case as mentioned by RAN4 with TAE requirement (less than 260 ns). |
| Lenovo | Similar view with Ericsson that it is up to the NW to configure which cell use UE-based TA estimation. |
| NTT DOCOMO | We support synchronized case according to RAN4 LS reply because UE-based TA measurement should cover the any frequency cases (i.e., FR1 and FR2). |
| Huawei, HiSilicon | According to Ran4 feedback, only case with TAE smaller than 260ns in FR1 is feasible. |
| Xiaomi | In the LS from RAN4, there are some specific scenarios that UE may be able to derive the TA based on UE based TA measurement from RAN4’s perspective. Then, we do not need to further discuss the use case in RAN1, right? |

P2-2: Network assistance for UE based TA acquisition

Proposal 2-2: For UE based TA acquisition, gNB should at least provide the following info for UE to derive the TA for candidate cell

- For each of measured serving cell and candidate cell

  - Cell ID, measured SSB ID [and corresponding measured occasions]

  - DL Tx timing difference between the serving and candidate cells (for asynchronous scenarios)

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |
| Google | Support |
| Ericsson | Cell Id is already there. If the Tx timing difference is known, the NW would compensate for it. Do not support additional NW assistance – this can be left to UE implementation. |
| QC | Support. It does not work if not knowing above info, since it is impossible for UE to measure all possibilities |
| Futurewei | Support Proposal 2-2. Under such a strict synchronization requirement of TAE less than 260ns, most common mobility scenarios are source/target non-collocated and cannot meet such a precise synchronization requirement. They are considered as asynchronous scenarios. Therefore, DL TX timing difference have to be provided to the UE so that the UE based TA measurement solution can be practically useful in normal mobility scenarios. |
| Samsung | O.K. in principle with FL proposal. |
| NOKIA | Measured occasions may not be needed. Also, Cell ID and SSB ID could be provided implicitly or explicitly depending on the decision in P2-4. Could we add that in the main bullet? |
| Lenovo | We are fine with it in principle. |
| NTT DOCOMO | We think Cell ID is not needed because whether UE-based TA measurement is enabled or not is configured by RRC parameter per cell in current spec. Also, DL Tx timing difference is not needed because UE-based TA measurement should be used for synchronized case. |
| LGE | Similar view as NOKIA. If we go with Alt 1 in P2-4, then Cell ID and SSB ID could be provided implicitly or explicitly. Possible one way for P2-4 can be that SSB ID is provided implicitly when RRC signaling configure specific candidate cell(s) for UE-based TA measurement, as suggested by QC for compromise. |
| Mod | Update of P2-2 based on discussions above. Proposal 2-2: For UE based TA acquisition, gNB should at least provide the following info for UE to derive the TA for candidate cell - For each of measured serving cell and candidate cell - Cell ID and measured SSB ID which could be provided implicitly or explicitly - DL Tx timing difference between the serving and candidate cells (for asynchronous scenarios) |
| Huawei, HiSilicon | RAN2 already designed the signaling for UE based TA acquisition and captured in their running CR on how to determine the source cell and target cell for UE based time acquisition. As for the DL Tx timing difference, RAN4 reply it may be feasible if TAE is smaller than 260ns and in FR1. As the TAE is smaller than CP, the indication of DL Tx timing difference is not necessary. |
| Xiaomi | Fine |
| IDCC | Same view as NTT Docomo. |
| CMCC | General fine with the proposal. And we also think the measured occasion is not needed. The cell id should be indicated to UE. And the SSB id can be indicated or identified by the UE for the UE based measurement of TA. |

P2-3: priority of TA acquisition method

Proposal 2-3: Regarding whether two different TA acquisition methods can be supported or not,

- Alt 1: Priority rule is not needed.

  - UE is not expected to be configured with both PDCCH order RACH and UE based TA measurement for a candidate cell

- Alt 2: Priority rule is needed. Details to be discussed.

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |
| Google | Alt2. TA in CSC, if present, should be prioritized over UE measured TA |
| Ericsson | Agreed with Google |
| QC | Fine for Alt2 if simultaneous enablement of the 2 schemes is further up to UE capability for UE supporting both PDCCH order based and UE based TA measurement. Otherwise, we prefer Alt1, i.e. always no simultaneous enablement |
| Futurewei | We would prefer Alt2. It depends on whether the two schemes are configured to the same candidate cell at the same time. If both schemes are configured to the same cell at the same time, we think priority rule is needed but it can be determined at the source serving cell and indicated to the UE. We think it is not needed to specify priority rule to the UE. |
| Samsung | Alt 2. We prefer higher priority on RACH based TA than UE-based TA. But the discussion is for the TA acquisition before the CSC. We don’t think RACH after CSC is under the consideration. |
| NOKIA | We support the configuration of both UE-based and early UL synch parameters for a candidate cell. In case TAs are available from both methods, priority should be used in such a way that the UE uses the latest TA. |
| Lenovo | We prefer Alt1. |
| ZTE | Support Alt2. RACH based TA is prioritized. |
| NTT DOCOMO | We prefer Alt2 and have same view as Google. |
| LGE | Support Alt 1. |
| Mod | Based on inputs shown above, the following update is made. Proposal 2-3: If both RACH based and UE based TA acquisition methods are configured to the same cell at the same time. - TA in CSC, if present, should be prioritized over UE measured TA |
| Huawei, HiSilicon | We think it is possible for gNB to configure both UE based TA and early RACH for a UE for the flexibility, considering the limited accuracy of UE based scheme, and larger overhead from early RACH. Priority rule should be defined (Alt 2), as gNB may indicate TA in CSC as gNB may have no idea when/whether UE based TA acquisition is performed. Support the latest proposal 2-3. |
| Xiaomi | Support Alt.1. If the PDCCH order RACH will be by triggered by serving cell to measure TA, why bother to enable UE based TA measurement? Therefore, from our understanding, if UE based TA measurement is enabled by serving cell, then it is UE itself to perform measurement to get the TA of candidate cell. UE is not expected to receive PDCCH order to trigger RACH to candidate cell. |
| IDCC | Alt 2. |
| NOKIA | With the above proposal, it seems the UE is not allowed to perform UE-Based TA acquisition for a candidate cell after a PDCCH ordered PRACH transmission for that candidate cell? If that is not the case, then the UE should be allowed use the latest TA, e.g., TA acquired more recently. |

P2-4: triggering of UE-based TA acquisition

- Alt 1: UE-based TA measurement is configured by RRC

- Alt 2: UE-based TA measurement is configured by RRC and activated/triggered by MAC CE for TCI state activation for candidate cells or cell switch command MAC CE

  - UE-based TA measurement for indicated candidate is performed if MAC CE for TCI state activation for candidate cells is provided.

  - If MAC CE for TCI state activation for candidate cells is not provided, UE-based TA measurement for indicated target cell is performed when cell switch command MAC CE is received.

  - [Upon reception of cell switch command MAC CE, the UE calculates the final target cell TA and applies it for UL transmission.]

- Alt 3: UE-based TA measurement is activated/triggered by MAC CE for TCI state activation for candidate cells or cell switch command MAC CE

  - UE-based TA measurement for indicated candidate is performed if MAC CE for TCI state activation for candidate cells is provided.

  - If MAC CE for TCI state activation for candidate cells is not provided, UE-based TA measurement for indicated target cell is performed when cell switch command MAC CE is received.

- Alt4: Left to UE implementation

- Alt 5: UE-based TA measurement is configured by RRC. Starting of RSTD measurement may be at DL synchronization phase and left to UE implementation. The final target TA calculation is performed by the UE upon the reception of CSC.

Proposal 2-4: TBD

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above alternatives. |
| Google | Alt 1 |
| Ericsson | Alt1 is already supported. Maybe formulate only alternatives in addition to RRC? |
| QC | If Alt1 intention is enabled UE based TA measurement per individual candidate cell based on RRC, we can compromise with it. Not support one RRC flag asking UE to measure all candidate cells. Both Alt2 and Alt3 tie the UE based TA with either TCI pre-activation or post CSC, which are not necessary. Fine if it is based on new MAC-CE dedicated to UE based TA |
| Futurewei | We prefer Alt 5. In our view, the UE based TA measurement includes two major activities: 1. RSTD measurement, 2. Target TA calculation. To minimize the latency, RSTD measurement should be started with DL synchronization and as early as possible. The activation of RSTD measurement can left to UE implementation. To ensure target TA accuracy, the calculation of target TA should be triggered by CSC, and using the latest serving cell TA and RSTD. Enabling of the UE based TA measurement feature at a candidate cell should be based on the RRC configuration at a per candidate cell basis. We think Alt 5 is essential. Any MAC CEs used before CSC maybe useful for UE to focus the UE based activities on network selected candidate SSBs for operation and power saving. |
| Samsung | Alt 1. We think required UE operation for UE-based TA is already clear. UE acquire candidate cell DL timing for L1 measurement, then UE extract candidate cell TA value at certain moment not too late to support LTM. RRC configuration is the only required spec change to inform which cell UE-based TA is required. |
| NOKIA | We support the additional triggering via TCI state activation as that can help the UE to minimize the number of cells for which the TA needs to be measured and only make or continue measurements for the cell which have more potential to become the target cell. |
| Lenovo | We support Alt1. |
| ZTE | Prefer Alt4. When the UE starts to measure RSTD and calculate the final TA value can be totally up to UE implementation. |
| NTT DOCOMO | First, we agree with E/// the formulation could be further refined because Alt1 itself has been supported and we think the intention is to discuss the starting timing to perform UE-based TA measurement. Second, we're not sure whether Alt1 and Alt4 have the same intention, i.e., it is up to UE implementation when to start performing UE-based TA measurement after UE-based TA measurement is configured by RRC . Third, to reduce UE complexity on UE-based TA measurement, Alt2 is preferred. |
| LGE | Alt 1. Compromise point suggested by QC can be the middle ground. |
| Mod | Update of P2-4. P2-4: in addition to RRC configuration of UE-based TA acquisition, triggering of UE-based TA acquisition - Alt 1: UE-based TA measurement is configured by RRC only: Google, Ericsson, QC(UE based TA measurement per individual candidate cell based on RRC), Samsung, Lenovo, LGE - Alt 2: UE-based TA measurement is configured by RRC and activated/triggered by MAC CE for TCI state activation for candidate cells or cell switch command MAC CE: Nokia(MAC CE for TCI state activation), DOCOMO - UE-based TA measurement for indicated candidate is performed if MAC CE for TCI state activation for candidate cells is provided. - If MAC CE for TCI state activation for candidate cells is not provided, UE-based TA measurement for indicated target cell is performed when cell switch command MAC CE is received. - [Upon reception of cell switch command MAC CE, the UE calculates the final target cell TA and applies it for UL transmission.] - Alt 3: UE-based TA measurement is activated/triggered by MAC CE for TCI state activation for candidate cells or cell switch command MAC CE - UE-based TA measurement for indicated candidate is performed if MAC CE for TCI state activation for candidate cells is provided. - If MAC CE for TCI state activation for candidate cells is not provided, UE-based TA measurement for indicated target cell is performed when cell switch command MAC CE is received. - Alt4: Left to UE implementation: ZTE - Alt 5: UE-based TA measurement is configured by RRC. Starting of RSTD measurement may be at DL synchronization phase and left to UE implementation. The final target TA calculation is performed by the UE upon the reception of CSC: Futurewei |
| Huawei, HiSilicon | According to RAN2#123bis, RAN2 adopted 2nd part of Alt 2 that UE based TA acquisition is configured by RRC and triggered by CSC. If UE is configured by RRC to perform UE based TA measurement, UE applies the measured TA value and performs RACH-less LTM, upon LTM cell switch. (assume similar config as for L2 reset) |
| Xiaomi | Support alt.2. If UE based TA measurement is enabled by serving cell, UE is not expected to receive PDCCH order to trigger RACH to candidate cell. Then, it is better to introduce a signaling to trigger UE based TA measurement to ensure that the TA is already calculated by UE before cell switch command. |
| IDCC | Support Alt1. |
| CMCC | Alt 1 is already supported. And we are fine with QC’s comments to introduce new MAC CE. We also have no problem for the TCI state activation MAC CE to trigger or activate the UE based measurements. The only concern is that if the activation or the triggering of the UE based TA measurement is based on CSC, whether it is a little bit late for the TA acquisition and if the delay of TA measurement of UE can fulfil the delay requirement of LTM. |

- TPs

TP1: To specify that UE does not detect PDCCH RAR for a PRACH transmission triggered for a candidate cell. (ZTE)

Reason for change:

RACH based TA acquisition mechanism for candidate cells does not include UE receiving RAR corresponding to the PRACH transmission, however, such characteristic has not been captured in clause 8.2 TS 38.213. In other words, from perspective of RAN1 specification, UE still needs to detect PDCCH for RAR for a PRACH transmission towards a candidate cell. Hence, we suggest to capture the following text proposal for clarification.

Summary of change:

To specify that UE does not detect PDCCH RAR for a PRACH transmission triggered for a candidate cell.

Consequence if not approved:

RACH procedure without RAR for candidate cells is not supported from perspective of RAN1 specification.

Text Proposal 1: To adopt the following changes in section 8.2, TS 38.213-i00. 8.2 Random access response - Type-1 random access procedure In response to a PRACH transmission, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a window controlled by higher layers [11, TS 38.321]. If a PRACH transmission is triggered by PDCCH order for a candidate cell, UE does not detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in clause 10.1, that is at least one symbol, after the last symbol of the last PRACH occasion corresponding to the PRACH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set as defined in clause 10.1. If NTA,adjUE or NTA,adjcommon, as defined in [4, TS 38.211], is not zero, the window starts after an additional TTA+kmac msec where TTA is defined in [4, TS 38.211] and kmac is provided by kmac or kmac=0 if kmac is not provided. The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by ra-ResponseWindow. <Unchanged parts are omitted>.

- ==============================TP2 for TS38.213============================== - L1/L2-triggered mobility procedures - -------------------------------------------Unchanged parts are omitted------------------------------------------- If a UE is provided ueMeasuredTA, the UE estimates based on the UE implementation a timing advance to apply from a first transmission on a candidate cell that is after the reception of a cell switch command for the candidate cell [11, TS 38.321].A UE can be provided configurations, by EarlyUlSyncConfig, for PRACH transmission parameters for each of the candidate cells. The UE can be triggered a contention free PRACH transmission on a candidate cell by a PDCCH order that the UE receives on a serving cell and includes an indication of the candidate cell for the PRACH transmission [4, TS 38.212], where there is no random access response for the PRACH transmission. If the serving cell and the candidate cell operate in a same frequency range and the UE would have transmissions that overlap in time, or when a gap between a first or last symbol of a PRACH transmission to the candidate cell is less than 𝑁 symbols from a last or first symbol, respectively, of an UL transmission to the serving cell, where N is defined in Clause TBD, the UE - drops the transmissions on the serving cell when the UE does not support transmissions that overlap in time or are separated by less than the gap on the serving cell and the candidate cell - prioritizes power allocation to the PRACH transmission on the candidate cell in clause 7.5 when the UE supports transmissions that overlap in time or are separated by less than the gap, and a total UE transmit power in the frequency range would exceed PCMAX -------------------------------------------Unchanged parts are omitted--------------------------

Moved the above TP to TP6

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above TP. |
| Google | We don’t think this UP needed. We neither configure RAR window for TA acquisition for LTM, nor specify any behavior corresponding to receiving RAR after transmitting a RA for LTM. |
| Ericsson | We think this is needed. There is a RAR window configured as part of the RACHConfigGeneric |
| QC | Fine |
| Futurewei | Fine |
| NOKIA | OK. It is good to clarify this. |
| Lenovo | Support to clarify it. |
| ZTE | We think that this TP is needed to clarify RAR not to be needed for Rel-18 LTM |
| NTT DOCOMO | We are OK. |
| LGE | OK. |
| Mod | Companies please also check the information needed for the cover page, i.e., reason for change, summary for change as well as consequences if not approved. |
| Huawei, HiSilicon | Agree to add the description of not receiving RAR for early RACH in TP1. Where to capture in the spec can be left to editor. No need to have TP2 |
| CMCC | Fine. |

TP2: Capture the agreement about the interpretation of Cell indicator

Reason for change: It is agreed that the bit field codepoint ‘0’ of cell indicator field in PDCCH order indicates PRACH for current serving cell, the rest bit field codepoints are mapped to candidate cells configured with EarlyUlSyncConfig-r18 where there is one-to-one mapping between bit field codepoint from 1 to C to candidate cell IDs in an ascending order. While the agreement is not captured in current specification yet.

Summary for change: Capture the agreement about the interpretation of Cell indicator.

Consequences if not approved: The interpretation of Cell indicator is lacked.

### TP from Ericsson:

A.2 Text proposal to 38.212 ##### 7.3.1.2.1 Format 1_0 DCI format 1_0 is used for the scheduling of PDSCH in one DL cell. The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: - Identifier for DCI formats - 1 bits - The value of this bit field is always set to 1, indicating a DL DCI format - Frequency domain resource assignment - bits where is given by Clause 7.3.1.0 If the CRC of the DCI format 1_0 is scrambled by C-RNTI and the "Frequency domain resource assignment" field are of all ones, the DCI format 1_0 is for random access procedure initiated by a PDCCH order, with all remaining fields set as follows: - Random Access Preamble index - 6 bits according to ra-PreambleIndex in Clause 5.1.2 of [8, TS38.321] - UL/SUL indicator - 1 bit. If the value of the "Random Access Preamble index" is not all zeros and if the UE is configured with supplementaryUplink in ServingCellConfig in the cell, this field indicates which UL carrier in the cell to transmit the PRACH according to Table 7.3.1.1.1-1; otherwise, this field is reserved - SS/PBCH index - 6 bits. If the value of the "Random Access Preamble index" is not all zeros, this field indicates the SS/PBCH that shall be used to determine the RACH occasion for the PRACH transmission; otherwise, this field is reserved. - PRACH Mask index - 4 bits. If the value of the "Random Access Preamble index" is not all zeros, this field indicates the RACH occasion associated with the SS/PBCH indicated by "SS/PBCH index" for the PRACH transmission, according to Clause 5.1.1 of [8, TS38.321]; otherwise, this field is reserved - Cell indicator - log2C+1 bits indicating the cell for the corresponding PRACH transmission if the UE is configured with higher layer parameter EarlyUlSyncConfig, where C is the number of candidate cells configured with higher layer parameter EarlyUlSyncConfig; 0 bit otherwise. The bit field index 0 of the cell indicator field initiates a random access procedure on a serving cell, , and other bit field indexes initiates a random access procedure to an LTM candidate cell configured with higher layer parameter EarlyUlSyncConfig according to an ascending order of a candidate identity configured by ltm-CandidateId, with the bit field index 1 mapped to the candidate cell with the smallest candidate identity.

### TP from Lenovo:

- ==============================TP1 for TS38.212============================== - 7.3.1.2.1 Format 1_0 - -------------------------------------------Unchanged parts are omitted------------------------------------------- If the CRC of the DCI format 1_0 is scrambled by C-RNTI and the "Frequency domain resource assignment" field are of all ones, the DCI format 1_0 is for random access procedure initiated by a PDCCH order, with all remaining fields set as follows: - Random Access Preamble index - 6 bits according to ra-PreambleIndex in Clause 5.1.2 of [8, TS38.321] - UL/SUL indicator - 1 bit. If the value of the "Random Access Preamble index" is not all zeros and if the UE is configured with supplementaryUplink in ServingCellConfig in the cell, this field indicates which UL carrier in the cell to transmit the PRACH according to Table 7.3.1.1.1-1; otherwise, this field is reserved - SS/PBCH index - 6 bits. If the value of the "Random Access Preamble index" is not all zeros, this field indicates the SS/PBCH that shall be used to determine the RACH occasion for the PRACH transmission; otherwise, this field is reserved. - PRACH Mask index - 4 bits. If the value of the "Random Access Preamble index" is not all zeros, this field indicates the RACH occasion associated with the SS/PBCH indicated by "SS/PBCH index" for the PRACH transmission, according to Clause 5.1.1 of [8, TS38.321]; otherwise, this field is reserved - Cell indicator - log2C+1 bits indicating the cell for the corresponding PRACH transmission if the UE is configured with higher layer parameter EarlyUlSyncConfig, where C is the number of candidate cells configured with higher layer parameter EarlyUlSyncConfig, and codepoint ‘0’ indicates the serving cell and codepoints from 1 to C are one-to-one mapped to candidate cell IDs configured with EarlyUlSyncConfig in an ascending order; 0 bit otherwise.

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above TP. |
| Google | Both TPs are fine to us |
| Ericsson | Suppoort |
| QC | Both are fine |
| Futurewei | Fine with both |
| NOKIA | Fine with the TP from Ericsson. TP from Lenovo is not needed as this has already been reflected in the latest CR 38.312 [R1-2310747]. The proposed TP is based on an older CR. |
| Lenovo | Both are fine. |
| ZTE | Both are OK. |
| NTT DOCOMO | Both are fine. |
| LGE | Either way is fine. |
| Mod | Let’s focus on Ericsson’s version. Companies please also check the information needed for the cover page, i.e., reason for change, summary for change as well as consequences if not approved. |
| Huawei, HiSilicon | As for TP from Ericsson, we think the initiation of RACH procedure should be captured in RAN2 spec. as for the TP from Lenovo, editor already implement the content in the endorsed editor CR. |
| CMCC | Both are fine. |

TP3: Capture the values for TSSB and ∆RF/BB preparation in 38.213 (vivo)

- Reason for change:

According to RAN4 agreement about the preparation time of PDCCH ordered RACH [3], when the PDCCH order triggers PRACH transmission toward the candidate cell, regardless of BWP switch, required delay will be considered and reflected in other new components. Thus, ∆BWPSwitching is not needed.

< Agreement> - RAN4 shall introduce a new term for additional time for RF and/or BB preparation and retuning in PDCCH ordered PRACH toward target cell before cell switch. - ∆BWPSwitching is not needed because all required components will be addressed separately.

However, in the current CR, only the PDCCH ordered RACH toward a candidate cell which is a non-serving cell follows the new delay requirement designed by RAN4.  The PDCCH ordered RACH toward a candidate cell which is a current serving cell follows the legacy delay requirement. It is not aligned with the RAN4 agreement.

Summary of change:

In TS 38.213, revise some current wordings to align with the corresponding RAN4 agreement.

Consequences if not approved:

If the TP is not approved, the current wording is not aligned with RAN4 agreement.

38.213 21 L1/L2-triggered Mobility Procedure <*unchanged is omitted*> A UE can be provided configurations, by EarlyUlSyncConfig, for PRACH transmission parameters for each of the candidate cells. The UE can be triggered a PRACH transmission on a candidate cell by a PDCCH order that the UE receives on a serving cell and includes an indication of the candidate cell for the PRACH transmission [4, TS 38.212]. The time between the last symbol of the PDCCH order reception and the first symbol of the corresponding PRACH transmission on a candidate cell is larger than or equal to msec, where - is a time duration of symbols corresponding to a PUSCH preparation time for UE processing capability 1 [6, TS 38.214] assuming corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH order and the SCS configuration of the corresponding PRACH transmission - msec for FR1 and msec for FR2 - is a switching gap duration as defined in [6, TS 38.214] - is SSB based T/F tracking duration as defined in TS 38.133 - is RF and/or BB preparation and retuning duration as defined in FFS If the serving cell and the candidate cell operate in a same frequency range and the UE would have transmissions that overlap in time, or when a gap between a first or last symbol of a PRACH transmission to the candidate cell is less than 𝑁 symbols from a last or first symbol, respectively, of an UL transmission to the serving cell, where N is defined in Clause TBD, the UE - drops the transmissions on the serving cell when the UE does not support transmissions that overlap in time or are separated by less than the gap on the serving cell and the candidate cell - prioritizes power allocation to the PRACH transmission on the candidate cell in clause 7.5 when the UE supports transmissions that overlap in time or are separated by less than the gap, and a total UE transmit power in the frequency range would exceed PCMAX <*unchanged is omitted*>

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above TP. |
| NOKIA | Not clear. The time gap is currently captured in section 8.1. |
| Mod | Companies please also check the information needed for the cover page, i.e., reason for change, summary for change as well as consequences if not approved. |
| Huawei, HiSilicon | The “non-serving cell” is trying to exclude candidate cell configured with early RACH but is still the serving cell. Referring to 212 is sufficient in current endorsed CR. |

TP4:  Capture the following agreements of power ramping counter in TS38.213 section 7.4 (CATT)

Reason of Change: In RAN1#114 meeting, following agreements were achieved on the condition to reset the power ramping counter for the power control of PDCCH-ordered CFRA in LTM. These agreements were not captured in current spec.

Agreement For the power control of PDCCH-ordered CFRA in LTM, power-ramping counter is reset at least when UE receives a PDCCH order indicating the initial transmission of PRACH Agreement - In addition to case 1, power-ramping counter is reset in the following cases: - Case 2: The candidate cell indicated in the PDCCH order, indicating retransmission, is different from that indicated in the last PDCCH order.

Summary of change: Capture the agreement on the condition to reset the power-ramping counter for the power control of PDCCH-ordered CFRA in LTM.

Consequences if not approved:  UE behavior to reset the power-ramping counter is ambiguous for PDCCH-ordered CFRA in LTM.

## *** Unchanged parts are omitted *** ## Section 7.4 of TS38.213 If prior to a PRACH retransmission, a UE changes the spatial domain transmission filter, Layer 1 notifies higher layers to suspend the power ramping counter as described in [11, TS 38.321]. If the UE receives a PDCCH order indicating the initial transmission of PRACH, or the candidate cell indicated in the PDCCH order indicating retransmission is different from that indicated in the last PDCCH order, Layer 1 may notify higher layers to suspend the corresponding power ramping counter. If due to power allocation to PUSCH/PUCCH/PRACH/SRS transmissions as described in clause 7.5, or due to power allocation in EN-DC or NE-DC or NR-DC operation, or due to slot format determination as described in clause 11.1, or due to the PUSCH/PUCCH/PRACH/SRS transmission occasions are in the same slot or the gap between a PRACH transmission and PUSCH/PUCCH/SRS transmission is small as described in clause 8.1, or due to DAPS operation as described in clause 15, or due to HD-UE operation in paired spectrum as described in clause 17.2, the UE does not transmit a PRACH in a transmission occasion, Layer 1 notifies higher layers to suspend the corresponding power ramping counter. If due to power allocation to PUSCH/PUCCH/PRACH/SRS transmissions as described in clause 7.5, or due to power allocation in EN-DC or NE-DC or NR-DC operation, the UE transmits a PRACH with reduced power in a transmission occasion, Layer 1 may notify higher layers to suspend the corresponding power ramping counter. ## *** Unchanged parts are omitted ***

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above TP. |
| Google | Support |
| Ericsson | Why “may notify”? Should be “notifies”, right? |
| QC | Fine |
| Futurewei | Fine |
| NOKIA | Do not support. As per understanding, suspend notification does not mean to “reset” the counter. The proposed TP is describing the scenarios when the counter is reset which is already captured in running CR 38.321 [R2-2311595]. Suspend notification is only needed when only SSB is changed which is already captured in section 7.4: If prior to a PRACH retransmission, a UE changes the spatial domain transmission filter, Layer 1 notifies higher layers to suspend the power ramping counter as described in [11, TS 38.321]. |
| Lenovo | Fine |
| ZTE | Agree with NOKIA. We think reset is to set the counter to be 0, but suspend is to not increase the counter by 1. |
| Mod | Revision based on Ericsson’s suggestion. Further discussion is needed. ## *** Unchanged parts are omitted *** ## Section 7.4 of TS38.213 If prior to a PRACH retransmission, a UE changes the spatial domain transmission filter, Layer 1 notifies higher layers to suspend the power ramping counter as described in [11, TS 38.321]. If the UE receives a PDCCH order indicating the initial transmission of PRACH, or the candidate cell indicated in the PDCCH order indicating retransmission is different from that indicated in the last PDCCH order, Layer 1 notifies higher layers to suspend the corresponding power ramping counter. If due to power allocation to PUSCH/PUCCH/PRACH/SRS transmissions as described in clause 7.5, or due to power allocation in EN-DC or NE-DC or NR-DC operation, or due to slot format determination as described in clause 11.1, or due to the PUSCH/PUCCH/PRACH/SRS transmission occasions are in the same slot or the gap between a PRACH transmission and PUSCH/PUCCH/SRS transmission is small as described in clause 8.1, or due to DAPS operation as described in clause 15, or due to HD-UE operation in paired spectrum as described in clause 17.2, the UE does not transmit a PRACH in a transmission occasion, Layer 1 notifies higher layers to suspend the corresponding power ramping counter. If due to power allocation to PUSCH/PUCCH/PRACH/SRS transmissions as described in clause 7.5, or due to power allocation in EN-DC or NE-DC or NR-DC operation, the UE transmits a PRACH with reduced power in a transmission occasion, Layer 1 may notify higher layers to suspend the corresponding power ramping counter. *** Unchanged parts are omitted *** Companies please also check the information needed for the cover page, i.e., reason for change, summary for change as well as consequences if not approved. |
| Huawei, HiSilicon | It should be reflected in RAN2 spec. no need in ran1 spec |
| NOKIA | Not needed. Reset conditions are captured in 38.321. Suspend notification is NOT same as the reset. |

TP5:  Capture the configuration mechanism of UE based TA estimation in 38.213 (Ericsson)

Reason for change: specify the condition of performing UE-based TA estimation

Summary for change: the configuration of each candidate cell includes a field ltm-UE-MeasuredTA-ID. If the value of ltm-UE-MeasuredTA-ID for the serving cell is the same as the value of ltm-UE-MeasuredTA-ID for the candidate, the UE would perform UE-based TA estimation. If the value of ltm-UE-MeasuredTA-ID for the serving cell is different from the value of ltm-UE-MeasuredTA-ID for the candidate, the UE would not perform UE-based TA estimation.

Consequences if not approved: condition of performing UE-based TA estimation is not clear

38.213 21 L1/L2-triggered Mobility Procedure A UE can be indicated, by LTM-Config, candidate cells and SS/PBCH blocks per candidate cell for the UE to obtain synchronization and measure corresponding L1-RSRPs [10, TS 38.133]. A MAC CE command can activate TCI states, provided by LTM-Candidate-TCI-State-r18 or/and LTM-Candidate-TCI-UL-State-r18, associated with SS/PBCH blocks or TRS of corresponding candidate cells. The UE is provided configurations by LTM-CSI-ReportConfigToAddModList for reporting L1-RSRP measurements [6, TS 38.214] that include a number of candidate cells and a number of SS/PBCH blocks per candidate cell from the number of candidate cells. If the ltm-UE-MeasuredTA-ID of a candidate cell is equal to the ltm-UE-MeasuredTA-ID of the serving cell, a, the UE estimates a timing advance to apply from a first transmission on a candidate cell that is after the reception of a cell switch command for the candidate cell [11, TS 38.321]. A UE can be provided configurations, by EarlyUlSyncConfig, for PRACH transmission parameters for each of the candidate cells. The UE can be triggered a PRACH transmission on a candidate cell by a PDCCH order that the UE receives on a serving cell and includes an indication of the candidate cell for the PRACH transmission [4, TS 38.212]. If the serving cell and the candidate cell operate in a same frequency range and the UE would have transmissions that overlap in time, or when a gap between a first or last symbol of a PRACH transmission to the candidate cell is less than 𝑁 symbols from a last or first symbol, respectively, of an UL transmission to the serving cell, where N is defined in Clause TBD, the UE - drops the transmissions on the serving cell when the UE does not support transmissions that overlap in time or are separated by less than the gap on the serving cell and the candidate cell - prioritizes power allocation to the PRACH transmission on the candidate cell in clause 7.5 when the UE supports transmissions that overlap in time or are separated by less than the gap, and a total UE transmit power in the frequency range would exceed PCMAX The UE transmits the PRACH on the candidate cell as described in Clause 8.1 with a power determined as described in Clause 7.4. A UE can be provided by a MAC CE in a PDSCH reception on the serving cell [11, TS 38.321] a TCI-State and/or TCI-UL-State in LTM-dl-OrJointTCI-StateToAddModList and/or LTM-ul-TCI-ToAddModList indicating a unified TCI state [6, TS 38.214] for applicable receptions or transmissions on a candidate cell from the number of candidate cells. The UE applies the TCI-State and/or TCI-UL-State, if indicated by the MAC CE, from a first slot that is TBD after the last symbol of a PUCCH or PUSCH with HARQ-ACK information for the PDSCH providing the MAC CE, and μ is the SCS configuration for the TBD.

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above TP. |
| ZTE | It is basically up to RRC signalling. If ltm-UE-MeasuredTA-ID rather than ueMeasuredTA (an enable indication) is finally captured by RRC signalling, the TP is fine to us. |
| NTT DOCOMO | We are OK. |
| Mod | Companies please also check the information needed for the cover page, i.e., reason for change, summary for change as well as consequences if not approved. |
| Huawei, HiSilicon | Support. We had similar TP in R1-2310849. When the value of ltm-UE-MeasuredTA-ID of a candidate cell, if configured, indicated in cell switch command is equal to the value of ltm-ServingCellUE-MeasuredTA-ID of current serving cell, if configured, , the UE estimates based on the UE implementation a timing advance to apply from a first transmission on a candidate cell that is after the reception of a cell switch command for the candidate cell [11, TS 38.321]. |

TP6:  Capture of agreement related to RACH-based TA acquisition for candidate cell (Lenovo)

Reason for change: It is agreed that RACH-based TA acquisition for candidate cell supported CFRA other than CBRA triggered by a PDCCH order and there is no RAR corresponding to the PRACH towards a candidate cell. While these agreements are not captured in current specification yet.

Summary for change: For RACH-based TA acquisition for candidate cell only support CFRA triggered by a PDCCH order and no RAR corresponding to the PRACH towards a candidate cell.

Consequences if not approved: Incorrect capture of agreement related to RACH-based TA acquisition for candidate cell.

- ==============================TP2 for TS38.213============================== - 21 L1/L2-triggered mobility procedures - -------------------------------------------Unchanged parts are omitted------------------------------------------- If a UE is provided ueMeasuredTA, the UE estimates based on the UE implementation a timing advance to apply from a first transmission on a candidate cell that is after the reception of a cell switch command for the candidate cell [11, TS 38.321].A UE can be provided configurations, by EarlyUlSyncConfig, for PRACH transmission parameters for each of the candidate cells. The UE can be triggered a contention free PRACH transmission on a candidate cell by a PDCCH order that the UE receives on a serving cell and includes an indication of the candidate cell for the PRACH transmission [4, TS 38.212], where there is no random access response for the PRACH transmission. If the serving cell and the candidate cell operate in a same frequency range and the UE would have transmissions that overlap in time, or when a gap between a first or last symbol of a PRACH transmission to the candidate cell is less than 𝑁 symbols from a last or first symbol, respectively, of an UL transmission to the serving cell, where N is defined in Clause TBD, the UE - drops the transmissions on the serving cell when the UE does not support transmissions that overlap in time or are separated by less than the gap on the serving cell and the candidate cell - prioritizes power allocation to the PRACH transmission on the candidate cell in clause 7.5 when the UE supports transmissions that overlap in time or are separated by less than the gap, and a total UE transmit power in the frequency range would exceed PCMAX -------------------------------------------Unchanged parts are omitted------------------------------------------

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above TP. Companies please also check the information needed for the cover page, i.e., reason for change, summary for change as well as consequences if not approved. |
| Huawei, HiSilicon | No need to have this TP. It is already clarified in TP1. |

- Other issues

Open issues on the coexistence of TA acquisition mechanisms are summarized below.

Table 3. Summary for other issues

| # | Issue | Companies’ views |
| --- | --- | --- |
| 4.1 | TAG related issues | - Associate TA/TAG and candidate cell explicitly - Proponents: QC, CMCC - Opponents: Google, Samsung - QC: For candidate cell with 2 TAGs (if supported), reuse the association decided for Rel-18 multi-DCI based multi-TRP with 2 TAGs |
| 4.2 | n-TimingAdvanceOffset configuration | - Vivo: Whether to provide the n-TimingAdvanceOffset for each candidate cell before cell switch needs discussion, if not provided, UE always assumes the serving cell n-TimingAdvanceOffset also applies for the cell where PRACH transmitted - CMCC: The values of n-TimingAdvanceOffset for each candidate cell should be provided in the LTM configurations. If not provided, either a default value of TS38.133 or the same value of current serving cell can be considered. - Apple: NTA_offset is pre-configured to UE for each candidate cell on FR1. |
| 4.3 | PRACH repetition number | Qualcomm - For PDCCH-order based PRACH, the PDCCH order DCI can dynamically indicate the PRACH repetition number |
| 4.4 | whether SUL is supported for early TA acquisition in LTM | - Google - RAN1 confirms SUL is supported for early TA acquisition in LTM. |
| 4.5 | Combination of RACH-based and UE-based TA estimation | - Nokia - Combination of RACH-based and UE-based TA estimation can be used to minimize the inaccurate TA estimation from UE based TA measurement. - The UE is provided with the estimation error for a candidate cell which the UE uses with all future UE-based TA estimation updates for the candidate cell. |
| 4.6 | Handling the overlap between UL transmission on serving cell and PRACH on a candidate cell | - Nokia - Transmission overlapping scenario in Clause 8.1(of 213) between a PRACH transmission and a PUSCH/PUCCH/SRS transmission on the serving cell is relevant to the LTM scenario more compared to the transmission overlapping scenario in clause 15 between PUSCH/PUCCH/SRS transmissions to the source and target MCGs. - Use clause 8.1 as the reference clause for the value of N used to determine the overlap scenario between the PRACH transmission to a candidate cell and an UL transmission to the serving cell. |

P4-1: n-TimingAdvanceOffset configuration

Proposal 4-1: TBD

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above issues. |
| Google | Seems needed to us. Open to discuss. |
| Ericsson | Support |
| Futurewei | Fine with the legacy approach if applicable. |
| Lenovo | We are fine to discuss it. |
| NTT DOCOMO | Support. |
| Mod | The following proposal is drafted for discussion. Proposal 4-1: n-TimingAdvanceOffset is pre-configured to UE for each candidate cell. |
| CMCC | As we proposed in the our contribution, the n-TimingAdvanceOffset should be provided for the candidate cells. |

P4-4: whether SUL is supported for early TA acquisition in LTM

Proposal 4-1: TBD

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above issues. |
| Google | To clarify our intention more. In previous meetings, most companies think SUL is needed or useful for TA acquisition for LTM. However, current SPEC does not support it with two reasons. First, we don’t have RA configuration corresponding to SUL in EarlyUlSyncConfig. Second, in current SPEC, the presence of UL/SUL field in PDCCH order still depends on serving cell configuration even the cell indicator indicates a LTM candidate cell, which make no sense. Hence, we suggest that RAN1 first confirm SUL is supported, and proceed related SPEC changes mentioned above. |
| Ericsson | In our view, SUL is supported with the current specification: the NW provides the UE with the correct RACH configuration, and if a candidate cell is SUL, the RACH configuration will be mapped to that. We should clarify that the UE ignores the SUL bit in the PDCCH order, to avoid error cases. |
| Huawei, HiSilicon | In legacy release, SUL can be configured to a cell to improve the UL coverage, similar benefit can be obtained in LTM if a candidate cell can be configured as SUL. We do not see any additional effort to support PRACH transmission on SUL. We support to have SUL for LTM. As for the comment of Ericsson, it depends on how SUL is configured, whether it is under a candidate cell include both UL and SUL or separate candidate cells. If follow legacy, it is not a separate candidate. |
| CMCC | Our thinking is that the SUL is supported for LTM. If the SUL is supported by the candidate cells, then the SUL/UL indication should be included in the PDCCH order. |

P4-6: Handling the overlap between UL transmission on serving Cell and PRACH on a candidate Cell

Proposal 4-6: TBD

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above issues. |
| Ericsson | Support |
| Mod | The following proposal is listed for discussion. Proposal 4-6: Use clause 8.1 of 213 as the reference clause for the value of N used to determine the overlap scenario between the PRACH transmission to a candidate cell and an UL transmission to the serving cell. |
| NOKIA | Support. This is to resolve one “TBD” in the current CR. |

- Issues to be discussed in online/offline sessions

5.1 Offline discussion round 1

P2-3

Proposal 2-3: If both RACH based and UE based TA acquisition methods are configured to the same cell at the same time.

- TA in CSC, if present, should be prioritized over UE measured TA

TP 1 (ZTE)

Reason for change:

RACH based TA acquisition mechanism for candidate cells does not include UE receiving RAR corresponding to the PRACH transmission, however, such characteristic has not been captured in clause 8.2 TS 38.213. In other words, from perspective of RAN1 specification, UE still needs to detect PDCCH for RAR for a PRACH transmission towards a candidate cell. Hence, we suggest to capture the following text proposal for clarification.

Summary of change:

To specify that UE does not detect PDCCH RAR for a PRACH transmission triggered for a candidate cell.

Consequence if not approved:

RACH procedure without RAR for candidate cells is not supported from perspective of RAN1 specification.

Text Proposal 1: To adopt the following changes in section 8.2, TS 38.213-i00. 8.2 Random access response - Type-1 random access procedure In response to a PRACH transmission, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a window controlled by higher layers [11, TS 38.321]. If a PRACH transmission is triggered by PDCCH order for a candidate cell, UE does not detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in clause 10.1, that is at least one symbol, after the last symbol of the last PRACH occasion corresponding to the PRACH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set as defined in clause 10.1. If NTA,adjUE or NTA,adjcommon, as defined in [4, TS 38.211], is not zero, the window starts after an additional TTA+kmac msec where TTA is defined in [4, TS 38.211] and kmac is provided by kmac or kmac=0 if kmac is not provided. The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by ra-ResponseWindow. <Unchanged parts are omitted>.

TP 2 (Ericsson)

Reason for change: It is agreed that the bit field codepoint ‘0’ of cell indicator field in PDCCH order indicates PRACH for current serving cell, the rest bit field codepoints are mapped to candidate cells configured with EarlyUlSyncConfig-r18 where there is one-to-one mapping between bit field codepoint from 1 to C to candidate cell IDs in an ascending order. While the agreement is not captured in current specification yet.

Summary for change: Capture the agreement about the interpretation of Cell indicator.

Consequences if not approved: The interpretation of Cell indicator is lacked.

A.2 Text proposal to 38.212 ##### 7.3.1.2.1 Format 1_0 DCI format 1_0 is used for the scheduling of PDSCH in one DL cell. The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: - Identifier for DCI formats - 1 bits - The value of this bit field is always set to 1, indicating a DL DCI format - Frequency domain resource assignment - bits where is given by Clause 7.3.1.0 If the CRC of the DCI format 1_0 is scrambled by C-RNTI and the "Frequency domain resource assignment" field are of all ones, the DCI format 1_0 is for random access procedure initiated by a PDCCH order, with all remaining fields set as follows: - Random Access Preamble index - 6 bits according to ra-PreambleIndex in Clause 5.1.2 of [8, TS38.321] - UL/SUL indicator - 1 bit. If the value of the "Random Access Preamble index" is not all zeros and if the UE is configured with supplementaryUplink in ServingCellConfig in the cell, this field indicates which UL carrier in the cell to transmit the PRACH according to Table 7.3.1.1.1-1; otherwise, this field is reserved - SS/PBCH index - 6 bits. If the value of the "Random Access Preamble index" is not all zeros, this field indicates the SS/PBCH that shall be used to determine the RACH occasion for the PRACH transmission; otherwise, this field is reserved. - PRACH Mask index - 4 bits. If the value of the "Random Access Preamble index" is not all zeros, this field indicates the RACH occasion associated with the SS/PBCH indicated by "SS/PBCH index" for the PRACH transmission, according to Clause 5.1.1 of [8, TS38.321]; otherwise, this field is reserved - Cell indicator - log2C+1 bits indicating the cell for the corresponding PRACH transmission if the UE is configured with higher layer parameter EarlyUlSyncConfig, where C is the number of candidate cells configured with higher layer parameter EarlyUlSyncConfig; 0 bit otherwise. The bit field index 0 of the cell indicator field initiates a random access procedure on a serving cell, , and other bit field indexes initiates a random access procedure to an LTM candidate cell configured with higher layer parameter EarlyUlSyncConfig according to an ascending order of a candidate identity configured by ltm-CandidateId, with the bit field index 1 mapped to the candidate cell with the smallest candidate identity.

TP 5 (Ericsson)

Reason for change: specify the condition of performing UE-based TA estimation

Summary for change: the configuration of each candidate cell includes a field ltm-UE-MeasuredTA-ID. If the value of ltm-UE-MeasuredTA-ID for the serving cell is the same as the value of ltm-UE-MeasuredTA-ID for the candidate, the UE would perform UE-based TA estimation. If the value of ltm-UE-MeasuredTA-ID for the serving cell is different from the value of ltm-UE-MeasuredTA-ID for the candidate, the UE would not perform UE-based TA estimation.

Consequences if not approved: condition of performing UE-based TA estimation is not clear

38.213 21 L1/L2-triggered Mobility Procedure A UE can be indicated, by LTM-Config, candidate cells and SS/PBCH blocks per candidate cell for the UE to obtain synchronization and measure corresponding L1-RSRPs [10, TS 38.133]. A MAC CE command can activate TCI states, provided by LTM-Candidate-TCI-State-r18 or/and LTM-Candidate-TCI-UL-State-r18, associated with SS/PBCH blocks or TRS of corresponding candidate cells. The UE is provided configurations by LTM-CSI-ReportConfigToAddModList for reporting L1-RSRP measurements [6, TS 38.214] that include a number of candidate cells and a number of SS/PBCH blocks per candidate cell from the number of candidate cells. If the ltm-UE-MeasuredTA-ID of a candidate cell is equal to the ltm-UE-MeasuredTA-ID of the serving cell, a, the UE estimates a timing advance to apply from a first transmission on a candidate cell that is after the reception of a cell switch command for the candidate cell [11, TS 38.321]. A UE can be provided configurations, by EarlyUlSyncConfig, for PRACH transmission parameters for each of the candidate cells. The UE can be triggered a PRACH transmission on a candidate cell by a PDCCH order that the UE receives on a serving cell and includes an indication of the candidate cell for the PRACH transmission [4, TS 38.212]. If the serving cell and the candidate cell operate in a same frequency range and the UE would have transmissions that overlap in time, or when a gap between a first or last symbol of a PRACH transmission to the candidate cell is less than 𝑁 symbols from a last or first symbol, respectively, of an UL transmission to the serving cell, where N is defined in Clause TBD, the UE - drops the transmissions on the serving cell when the UE does not support transmissions that overlap in time or are separated by less than the gap on the serving cell and the candidate cell - prioritizes power allocation to the PRACH transmission on the candidate cell in clause 7.5 when the UE supports transmissions that overlap in time or are separated by less than the gap, and a total UE transmit power in the frequency range would exceed PCMAX The UE transmits the PRACH on the candidate cell as described in Clause 8.1 with a power determined as described in Clause 7.4. A UE can be provided by a MAC CE in a PDSCH reception on the serving cell [11, TS 38.321] a TCI-State and/or TCI-UL-State in LTM-dl-OrJointTCI-StateToAddModList and/or LTM-ul-TCI-ToAddModList indicating a unified TCI state [6, TS 38.214] for applicable receptions or transmissions on a candidate cell from the number of candidate cells. The UE applies the TCI-State and/or TCI-UL-State, if indicated by the MAC CE, from a first slot that is TBD after the last symbol of a PUCCH or PUSCH with HARQ-ACK information for the PDSCH providing the MAC CE, and μ is the SCS configuration for the TBD.

P4-1

Proposal 4-1: n-TimingAdvanceOffset is pre-configured to UE for each candidate cell.

P4-6

Proposal 4-6: Use clause 8.1 of 213 as the reference clause for the value of N used to determine the overlap scenario between the PRACH transmission to a candidate cell and an UL transmission to the serving cell.

5.2 Online discussion round 1

5.3 Offline discussion round 2

5.4 Online discussion round 2

- References

- RP-222332	Revised WID on Further NR mobility enhancements		 	MediaTek (Moderator)

- R1-2310831	Remaining issues with UE based TA determination			FUTUREWEI

- R1-2310849	Maintenance of timing advance management to reduce latency		Huawei, HiSilicon

- R1-2310955	Maintenance on TA management to reduce latency			ZTE

- R1-2310973	Maintenance of TA management for LTM				Ericsson

- R1-2311106	Discussion on TA management for L1/L2-triggered Mobility			vivo

- R1-2311173	Remaining issues on timing advance management to reduce latency		Spreadtrum Communications

- R1-2311231	Remaining Issues of Timing Advance Management			OPPO

- R1-2311351	Remaining issues on TA management to reduce latency			CATT

- R1-2311369	Remaining issues of timing advance management for L1/L2 mobility		Lenovo

- R1-2311439	Remaining issues on TA management for mobility				LG Electronics

- R1-2311443	Maintenance on timing advance management to reduce latency		Google

- R1-2311491	Remaining issues on timing advance management to reduce latency		CMCC

- R1-2311592	Remaining issues on TA management for LTM				FGI

- R1-2311630	Remaining issues on TA enhancements for inter-cell mobility			NTT DOCOMO, INC.

- R1-2311693	Timing advance management for L1/L2 Mobility				Apple

- R1-2311853	Remaining issues on candidate cell TA management for NR L1/L2 mobility enhancement	Samsung

- R1-2311890	Remaining Issues on Timing Advance Management for L1/L2-triggered Mobility	Nokia, Nokia Shanghai Bell

- R1-2311952	Remaining details on timing advance management				InterDigital, Inc.

- R1-2312045	TA management to reduce latency for L1/L2 based mobility			Qualcomm Incorporated

- R1-2312134	Discussion on TA management to reduce latency				CAICT

- Previous agreements

RAN1 #110bis-e

Agreement

Support TA acquisition of candidate cell(s) before cell switch command is received in L1/L2 based mobility.

- FFS: whether this can be applied to candidate cell when it is deactivated SCell (if defined in RAN2)

Agreement

On mechanism to acquire TA of the candidate cells, the following solutions can be further studied:

•         RACH-based solutions

e.g., PDCCH ordered RACH, UE-triggered RACH, higher layer triggered RACH from NW other than L3 HO cmd

•         RACH-less solutions

e.g., SRS based TA acquisition, Rx timing difference based, RACH-less mechanism as in LTE, UE based TA measurement (including UE based TA measurement with one TAC from serving cell)

Agreement

For TA acquisition of a candidate cell before cell switch command is received, study at least the following alternatives of associating TA/TAG to candidate cell:

- Alt1: Associate TA/TAG and candidate cell implicitly, e.g.,

- the association between TA/TAG and TCI states can be configured

- Alt2: Associate TA/TAG and candidate cell explicitly, e.g.,

- the association is provided as a part of candidate cell(s) configuration

- the association between TA/TAG and SSB(s)/TRS(s) is provided as a part of candidate cell(s) configuration

RAN1 #111

Agreement

On mechanism to acquire TA of the candidate cell(s) in Rel-18 LTM, at least support PDCCH ordered RACH.

- The PDCCH order is only triggered by source cell

- FFS: the details including content of DCI, RACH resource configuration, RAR transmission mechanism, etc.

- Note: any other RACH-based solutions are for discussion separately

Agreement (Made in RAN1#110b-e)

Support TA acquisition of candidate cell(s) before cell switch command is received in L1/L2 based mobility.

- FFS: whether this can be applied to candidate cell when it is deactivated SCell (if defined in RAN2)

Agreement

For PDCCH ordered RACH in LTM, at least the following enhancements are supported

- Introduce indication of candidate cell and/or RO of candidate cell in DCI

- configuration of RACH resource for candidate cell(s) is provided prior to the PDCCH order

- FFS: whether/how to transmit RAR

Agreement

On whether RAR is needed for PDCCH ordered RACH for a candidate cell in LTM, the following alternatives are considered for further study

- Alt 1: RAR is needed

- Alt 2: RAR is not needed

    - Note: If Alt 2 is supported, TA value of candidate cell is indicated in cell switch command

- Alt 3: whether RAR is needed can be configured

Agreement

- TA updating (i.e. re-acquisition of TA) for candidate cell can be triggered by NW.

same triggering mechanism reuse the initial TA acquisition, i.e., PDCCH order triggered RACH in a candidate cell

RAN1 #112

Agreement

For Rel-18 LTM, Random Access Preamble indices and indication of RACH occasions with the associated SSB indices are configured for each candidate cell.

Note: the detailed signalling is left to RAN2

Agreement

The PDCCH order from the source cell contains the indication of candidate cell.

- The reserved bit(s) in DCI format 1_0 for PDCCH order can be used for indication of cell identity

Agreement

For PDCCH ordered-RACH for candidate cell(s), RAR reception can be configured/indicated

- If reception of RAR is not configured/indicated (without RAR)

  - TA value of candidate cell is indicated in cell switch command

  - FFS: whether UE should re-transmit PRACH when reception of RAR is not configured/indicated

  - FFS: how UE determine the transmit power of subsequent PRACH triggered by PDCCH order

- If reception of RAR is configured/indicated (with RAR), FFS

  - whether RAR is received from serving cell or candidate cell

    - if RAR is received from candidate cell, whether Type1-PDCCH CSS of the candidate cell is configured to the UE

  - content of RAR

- FFS: signaling for configuration/indication of whether RAR needs to be received

- UE can report the support combination of with RAR only and without RAR only, where support of one default scheme is the baseline UE approach for LTM

- Send LS to RAN2 and RAN3 to check the feasibility about this agreement

- Note: Definition of candidate cells is up to RAN2

Agreement

- For PDCCH-order based RACH for TA measurement for candidate cells, legacy CBRA is not supported

Agreement

on whether UE should initiate re-transmit PRACH when reception of RAR is not configured/indicated, down select one from the following alternatives.

- Alt 1: UE autonomous re-transmission of PRACH is not allowed (e.g., by setting the number of allowed PRACH transmission to the minimum value of PreambleTransMax=1)

  - Alt 2: UE autonomous Re-transmission of PRACH is allowed,

    - The number of PRACH transmission will be defined e.g. set the times of RACH transmission to the minimum value of PreambleTransMax

Agreement

If reception of RAR is configured/indicated, RAR contains at least TA of candidate cell.

- The maximum number of TA values memorized by UE is a UE capability

- FFS: whether other parameters such as UE ID, candidate cell ID etc. is contained in RAR

Agreement

Whether RAR needs to be received is configured by RRC.

Agreement

study at least the following issues on PDCCH-order based PRACH for candidate cell that is not UL serving cell, i.e. without PUCCH/PUSCH configured

- Whether gap between the DCI and PRACH longer than timeline defined in spec is needed

- Any impact/interruption on UL Tx of serving CCs due to the PRACH Tx

Working Assumption

UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported.

- Corresponding UE capability is to be introduced to support UE-based TA measurement

- For a UE reports support of this capability, configuration of UE-based TA measurement is supported

- FFS: other impacts on RAN1 spec

RAN1 #112bis

Agreement

For PDCCH ordered-RACH, if reception of RAR is not configured, UE autonomous re-transmission of PRACH is not allowed, regardless of the configuration of PreambleTransMax.

Agreement

When reception of RAR is configured, support RAR is received from serving cell at least in intra-DU case.

Agreement

When reception of RAR is configured, support RAR is received from serving cell in inter-DU case.

- FFS: RA response window related issues

Agreement

For PDCCH ordered RACH mechanism in R18 LTM, when reception of RAR is configured,

- the UE stores(remembers/maintains/handles) a TA for at least one candidate cell

- storing(remembering/maintaining/handling) corresponding TAs for more than one candidate cell is up to UE capability

  - detailed number of candidate cell is up to UE capability

Agreement

For PDCCH-order based PRACH for candidate cell study the following issues:

  - whether/how prioritizations for transmission power reduction for a PRACH transmission to a LTM candidate cell is performed

  - whether/how prioritizations for prioritization of a PARCH transmission to a LTM candidate cell compared to an overlapped (in time and frequency) serving cell UL transmission

Agreement

For PDCCH ordered-RACH, if reception of RAR is not configured

- Whether power ramping is performed or not is determined from PDCCH order

    - If power ramping is performed,

      - whether PRACH is an initial transmission or retransmission is explicitly indicated in PDCCH order (FFS exact indication mechanism)

      - power ramping-up value is configured

    - else, the power should be determined by open-loop power control

Agreement

Send LS to RAN4 with the following info

- RAN1 discussed the time gap between a PDCCH order and the corresponding PRACH transmission for LTM.

- RAN1 believes that this will require that the time gap is increased at least for the following scenario

    - For PDCCH-order based PRACH on a candidate cell that is not a current serving cell with PUCCH/PUSCH or inter-frequency with the current serving cell

- RAN1 relies on RAN4 to verify the need for the above additional latency and, if so, the corresponding value is needed

- RAN1 relies on RAN4 to investigate any impact/interruption on UL Tx of serving cell due to the PRACH Tx on a candidate cell that is not a current serving cell with PUCCH/PUSCH

- RAN1 relies on RAN4 to verify the need for any update is required to ΔBWPSwitching, ΔDelay if so, the corresponding values and whether UE capability is needed

Potential RAN1 spec update will be based on RAN4’s feedback.

RAN1 #113

Agreement

Confirm the following Working Assumption, and sent LS to RAN4 to clarify the feasibility of supporting this mechanism

Working Assumption

From RAN 1 perspective, UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported.

- Corresponding UE capability is to be introduced to support UE-based TA measurement

- For a UE reports support of this capability, configuration of UE-based TA measurement is supported

- FFS: other impacts on RAN1 spec

Agreement

From RAN 1 perspective, without performing PDCCH-ordered RACH for candidate cell(s), RACH-less mechanism can be supported by indicating TA value of target cell as TA=0 or keeping the same value as source cell in cell switch command.

- Note 1: this doesn’t mean to preclude TA values other than 0 and the same value as source cell in cell switch command for PDCCH-ordered RACH when RAR is not configured for the PDCCH order.

- Note 2: The feasibility and signalling can be further concluded by RAN2

Agreement

For PDCCH order based PRACH to candidate cell, the candidate cell SSB indicated in the PDCCH order serves as the path loss RS for PRACH Tx power determination.

Agreement

On the determination of the PRACH transmission power when reception of RAR is not configured, a [1-bit] field in PDCCH order explicitly indicating initial transmission or retransmission of PRACH is supported.

Note:

From RAN1 perspective, when reception of RAR is configured, there may have 4 alternatives to determine the randomaccess response window

- Alt1: Postpone the starting point of the random access response window

- Alt2: Extend the length of the random access response window

- Alt3: Length and offset of the starting point of RAR window can be configured by RRC

- Alt4: if MAC CE is used to carry TA and PDCCH is scrambled by C-RNTI in USS, RAR window is not needed

[Note: the random access response window for candidate cell(s) is separately configured from the normal RAR window.]

Note:

In addition to TA, when reception of RAR is configured, RAN1 discussed the following alternatives:

- Alt 1: when there is only one ongoing RACH procedure at each time, the identification of candidate cell is not needed

- Alt 2: when more than one RACH procedures are allowed at each time, the identification of candidate cell is contained in RAR

Agreement

On the determination of the PRACH transmission power when reception of RAR is not configured, a 1-bit field in PDCCH order explicitly indicating initial transmission or retransmission of PRACH, FFS

- UE will increase the power with the value of power ramping configuration if it is indicated as re-transmission, unless the max allowed power is achieved

- whether/how to reset the counter

Agreement

- For PDCCH-order based PRACH for candidate cell, If UE capability does not support simultaneous/parallel transmissions, when the PRACH transmission to a candidate cell other than current serving cell(including any interruption due to processing time to build the PRACH transmission, carrier or/and BWP switching time if any, UL or DL RF retuning time if any, additional preparation time if any) happen to overlap over one or more symbols or have a time gap below a certain threshold (e.g., N symbols, FFS: the value of N) with following UL transmission to one of the serving cells

- PRACH transmission

- PUCCH/PUSCH transmission carrying HARQ-ACK, SR, P/SP CSI, aperiodic CSI

- SRS transmission

- Any other PUCCH/PUSCH transmission

- Down-select the UE behavior in this case

- Alt 1: Dropping rule is needed

- Alt 2: up to UE implementation

RAN1 #114

Agreement

For the power control of PDCCH-ordered CFRA in LTM, the UE can maintain only one power ramping counter

Agreement

For the power control of PDCCH-ordered CFRA in LTM, power-ramping counter is reset at least when UE receives a PDCCH order indicating the initial transmission of PRACH

Agreement

When a PDCCH order is sent for a candidate cell,

- The bit size of N in DCI format 1_0 for cell indicator is determined by the number (e.g., C) of configured candidate cells with RACH configuration provided for early TA acquisition, downselect one from the following alternatives.

    - Alt 1: N=log2C

    - Alt 2: N=log2C+1 (update the equation with (C+1))

        - The number of cells used to calculate the bit width is the number of candidate cells with RACH configuration provided for early TA acquisition + 1 (serving cell)

Agreement

For the power control of PDCCH-ordered CFRA in LTM,

- When a UE receives a PDCCH order indicating a re-transmission of PRACH with the same associated SSB and same candidate cell as the previous PRACH, the counter is increased by 1.

- In addition to case 1, power-ramping counter is reset in the following cases:

  - Case 2: The candidate cell indicated in the PDCCH order, indicating retransmission, is different from that indicated in the last PDCCH order.

Note: the initial counter is 0 before receiving any PDCCH order.

Agreement

when a PDCCH order is sent for a candidate cell,

- The bit size of N in DCI format 1_0 for cell indicator is determined by the number (e.g., C) of configured candidate cells with RACH configuration provided for early TA acquisition, the following alternative is supported:

    - Alt 2: N=log2(C+1)

- The number of cells used to calculate the bit width is the number of candidate cells with RACH configuration provided for early TA acquisition + 1 (serving cell)

Agreement

When the UE does not support simultaneous/parallel transmissions of PRACH in candidate cell and UL channels and signals in serving cell, support

  - serving cell UL TX is dropped.

Agreement

If the UE supports simultaneous/parallel transmissions of PRACH in candidate cell and UL channels and signals in serving cell in the same frequency range, support:

- A PRACH transmission to a LTM candidate cell has the highest priority for power allocation

Note: up to UE whether performs power scale-down or drop of UL transmission with lower priority when UL transmission power is insufficient.

RAN1 #114bis

Agreement

Remove the bracket [] and add the following two child IEs into RRC parameters list for LTM to determine the frequency location of PRACH resource for early UL sync procedure:

| # | RAN2 parent IE | Parameter name in the spec | New or existing? | Description | Value Range |
| --- | --- | --- | --- | --- | --- |
| 1 | ltm-EarlyUlSyncConfig-r18 | EarlyUlSyncConfig-r18 | new | Configuration used by the UE to perform the early UL synchronization procedure. | SEQUENCE { FrequencyInfoUL BWP-genericParameters rach-ConfigGeneric, ssb-perRACH-Occasion, prach-RootSequenceIndex } |
| 2 | EarlyUlSyncConfig-r18 | BWP-genericParameters | new | configure the common parameters of an uplink BWP of candidate cell | BWP |
| 3 | BWP-genericParameters | BWP | Existing | Configure the common parameters of an uplink BWP of candidate cell | SEQUENCE { locationAndBandwidth, subcarrierSpacing, cyclicPrefix } |

Agreement

Introduce new components of TSSB and ∆RF/BB_preparation in the time gap between the last symbol of the PDCCH order reception and the first symbol of the PRACH transmission in RAN1 formula according to the RAN4 reply LS [R4-2314454].

- BWP switching related delay (∆BWPSwitching) is not needed or equals to zero for LTM early TA acquisition.

- The corresponding TP is to be discussed in RAN1 #114-bis.

Agreement

The bit field codepoint ‘0’ of cell indicator field in PDCCH order indicates PRACH for current serving cell, the rest bit field codepoints are mapped to candidate cells configured with EarlyUlSyncConfig-r18.

- One-to-one mapping between bit field codepoint from 1 to C to candidate cell IDs in an ascending order

Conclusion

TA acquisition of candidate cell(s) before cell switch command is received in L1/L2 based mobility is supported when the candidate cell(s) is deactivated SCell(s).