# 3GPP TSG RAN WG1 #115                                                                          R1-2312482

# Chicago, USA, November 13th – November 17th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0461 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on resource allocation in time domain for multi-PXSCH scheduling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (LG Electronics), vivo, Ericsson, NTT DOCOMO, INC., Samsung, ZTE, Sanechips, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Core |  |  |  |  |  | Date: |  |  | 2023-11-14 |
| Category: | A |  |  |  |  |  | Release: |  |  | Release 18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following agreement was achieved in RAN1#105-e meeting. Agreement: - If a PDSCH among multiple PDSCHs that are scheduled by a single DCI is collided with uplink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, the UE does not receive the PDSCH. - FFS on how to handle HARQ-related issue for the PDSCH (e.g., HARQ process numbering) - The UE does not expect to be scheduled with multiple PDSCHs by a single DCI, where every PDSCH is collided with uplink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated. - If a PUSCH among multiple PUSCHs that are scheduled by a single DCI is collided with downlink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, the UE does not transmit the PUSCH. - FFS on how to handle HARQ-related issue for the PUSCH (e.g., HARQ process numbering) - The UE does not expect to be scheduled with multiple PUSCHs by a single DCI, where every PUSCH is collided with downlink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated. The first bullet in the above agreement is captured by the following text in clause 5.1 of TS38.214. HARQ process ID is not incremented for PDSCH(s) not received if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided. Besides, the third bullet in the above agreement is captured by the following text in clause 6.1 of TS38.214. HARQ process ID is not incremented for PUSCH(s) not transmitted if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. However, the second and the fourth bullets in the above agreement are not captured in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture the second and the fourth bullets in the above agreement in TS38.214. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is incomplete. In addition, the UE or the network may regard the case where each of multiple PDSCHs or PUSCHs scheduled by a single DCI format is invalid, as allowed. However, it is inconsistent with RAN1’s common understanding. Besides, this case may result in unspecified UE behaviour, in terms of HARQ process operation, HARQ-ACK reporting, etc. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.2.1, 6.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Unchanged text omitted ***

#### 5.1.2.1	Resource allocation in time domain

*** Unchanged text omitted ***

For pdsch-TimeDomainAllocationListForMultiPDSCH in pdsch-Config each PDSCH has a separate SLIV, mapping type and K0. The number of scheduled PDSCHs is signalled by the number of indicated SLIVs in the row of the pdsch-TimeDomainAllocationListForMultiPDSCH signalled in DCI format 1_1.

If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH in which one or more rows contain multiple SLIVs for PDSCH on a DL BWP of a serving cell, and the UE is indicated re-transmission of PDSCH corresponding to a DL SPS by DCI format 1_1, the UE does not expect that the number of indicated SLIVs in the row of the pdsch-TimeDomainAllocationListForMultiPDSCH by the DCI is more than one.

If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH in which one or more rows contain multiple SLIVs for PDSCH on a DL BWP of a serving cell, the UE does not expect to be scheduled with one or multiple PDSCH receptions by a single DCI format 1_1, where each PDSCH reception overlaps with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided.

*** Unchanged text omitted ***

#### 6.1.2.1	Resource allocation in time domain

*** Unchanged text omitted ***

For pusch-TimeDomainAllocationListForMultiPUSCH in pusch-Config, if a row indicates resource allocation for two to eight contiguous PUSCHs and extendedK2 is not configured, K2 given by k2-r16 indicates the slot where UE shall transmit the first PUSCH of the multiple PUSCHs. Each PUSCH has a separate SLIV and mapping type. The number of scheduled PUSCHs is signalled by the number of indicated valid SLIVs in the row of the pusch-TimeDomainAllocationListForMultiPUSCH signalled in DCI format 0_1.

For pusch-TimeDomainAllocationListForMultiPUSCH in pusch-Config, if a row indicates resource allocation of more than one PUSCH and extendedK2 is configured, each PUSCH has a separate SLIV, mapping type and K2 given by extendedK2. If a row indicates resource allocation of a single PUSCH, the PUSCH has a single SLIV, mapping type, and K2, where K2 is given by extendedK2, if configured, otherwise K2 is given by k2-r16. The number of scheduled PUSCHs is signalled by the number of indicated SLIVs in the row of the pusch-TimeDomainAllocationListForMultiPUSCH signalled in DCI format 0_1.

If a UE is configured with extendedK2 in pusch-TimeDomainAllocationListForMultiPUSCH in which one or more rows contain multiple SLIVs for PUSCH on a UL BWP of a serving cell, and the UE is indicated re-transmission of PUSCH by DCI format 0_1, where the PUSCH is correspond to a configured grant Type 1 or Type 2, the UE does not expect that the number of indicated SLIVs in the row of the pusch-TimeDomainAllocationListForMultiPUSCH by the DCI is more than one.

If a UE is configured with pusch-TimeDomainAllocationListForMultiPUSCH in which one or more rows contain multiple SLIVs for PUSCH on a UL BWP of a serving cell, the UE does not expect to be scheduled with one or multiple PUSCH transmissions by a single DCI format 0_1, where each PUSCH transmission overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

*** Unchanged text omitted ***