3GPP TSG RAN WG1 #116									 R1-2400518

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	8.12.1

Source:	Ericsson

Title:	Discussion on UE features for Rel-18 MIMO

Document for:	Discussion, Decision

# 1	Introduction

In this contribution, we discuss the UE features for Rel18 MIMO.

We follow the recommendations in the RAN2 LSs [1] and [2].

# 2	Discussion

In [5], RAN1 provided a first list UE features for Rel-18 MIMO. In this document, we will suggest updates and amendments to the list. The focus will be on defining the required rows, the description, and the type.

## 2.1	Multi-TRP enhancement

## 2.1.1 Unified TCI framework extension for multi-TRP

When designing the features for unified TCI framework extension for multi-TRP, it is useful to revisit how the UE features for the unified TCI framework in Rel-17 was designed [3]. Many of the choices made and principles used in Rel-17 are relevant also when designing the UE features for the Rel-18 framework.

## 2.1.1.1	FGs included in [5]

For unified TCI, [5] contains a number of feature groups, where the prerequisite FGs are not settled. We discuss them one by one.

FG 40-1-1: This functionality relies on the unified TCI framework defined in Rel-17, i.e., FG 23-1-1. It also relies on single-DCI mTRP, i.e., FG 16-2b-1.

FG 40-1-1a: There was a question from RAN2 if component 1 should have candidate values, i.e., UE to indicate the support with or without DL assignment, or both. In Rel-17, this was not case: a UE that supports DCI-based indication supports both with and without DL assignment. However, from a testability point of view, it would make sense that the UE can report these capabilities separately. Hence, we propose to introduce candidate values for component 1: {with DL assignment, without DL assignment, both}.

FG 40-1-1b: This feature relies on FG 40-1-1 and also mTRP BFR, i.e., FG 23-5-2.

FG 40-1-1c: Here the prerequisites are listed as “At least….” Here it is enough to include FG 40-1-1 as a pre-requisite: this is the only FG that is strictly required to support FG 40-1-1c. The other listed FGs are needed in case the NW deploys the corresponding, more advanced, functionality, and hence all the other FGs require that 40-1-1 is supported.

FG 40-1-2a: This feature relies on FG 40-1-2.

FG 40-1-3: This feature relies on FG 40-1-1.

FG 40-1-4: This feature relies on FG 40-1-1, and also on CJT: FG 40-3-1-1.

FG 40-1-7: There is a long list of pre-requisites in yellow highlighting. We note that this relies on the unified TCI state framework, i.e., FG 23-1-1. It also relies on basic multi-DCI mTRP, i.e., FG 16-2a. However, it does not rely on inter-cell mTRP – the feature can be implemented without it. It does not rely on any of the STxMP features (40-6-x). Of course, if the NW wants to deploy inter-cell mTRP, the UE must support FG 24-4, and if the NW wants to deploy STxMP, the UE must support some of the 40-6-x feature, but multi-DCI multi-TRP can be implemented without them, so they should not be listed as pre-requisites.

FG 40-1-7a: In our view, only FG 40-1-7 is needed.

FG 40-1-9:

  - Here the following pre-requisites are needed: FG 23-10-1 (support for separate DL/UL TCI states from Rel-17), FG 40-1-7 (support for joint is a pre-requisite, cf. FG 40-1-2).

  - There should be candidate values for component 1, similar to component 1 in FG 40-1-7: Component 1 candidate values {intra-cell, intra-cell and inter-cell}

FG 40-1-9a: Only FG 40-1-9 is a prerequisite, following the principle from FG 40-1-1a (only FG 40-1-1 is a prerequisite).

FG 40-1-12: Only FG 40-1-1 is a pre-requisite, following the principle from Rel-17. (only FG 23-1-1 is a prerequisite for FG 23-1-1f)

FG 40-1-13: Only FG 40-1-7 is a pre-requisite, following the principle from Rel-17.

## 2.1.1.2	New FG

There is a need to add the counterpart to FG 40-1-3 for multi-DCI based mTRP:

| 40-1-14 | Per aperiodic CSI-RS resource/resource set configuration for TCI selection in mDCI based mTRP | Support of per aperiodic CSI-RS resource/resource set configuration for TCI selection in mDCI based mTRP |  |  | Per band | Component candidate values: {per resource, per resource set, both} |
| --- | --- | --- | --- | --- | --- | --- |

## 2.1.1.3	Support for Rel-16 and Rel-17 multi-TRP schemes

The extended unified TCI framework supports all schemes specified in Rel-16 and Rel-17. The features introduced so far describe how the TCI states are updated. For example, the FG 40-1-1 describes this for single-DCI based multi-TRP. However, it is not clear which Rel-16/Rel-17 multi-TRP schemes are supported with the unified TCI framework. There are essentially two options:

A UE that supports the multi-TRP extension of the unified TCI framework (FG 40-1-1, FG 40-1-7) supports any mTRP scheme from Rel-16/Rel-17, provided that the corresponding Rel-16/Rel-17 feature is supported, i.e.,

    - NC-JT (corresponding Rel-16 feature: FG 16-2b-x/16-2a). This is actually explicitly stated in FG 40-1-1 and 40-1-7.

    - PDCCH repetition (corresponding Rel-17 feature: FG 23-2-1x )

    - PUSCH repetition (corresponding Rel-17 feature: FG 23-3-1x)

    - PUCCH repetition (corresponding Rel-17 feature: FG 23-3-2x)

    - HST-SFN (corresponding Rel-17 feature: FG 23-6-1x)

    - mTRP BFR (corresponding Rel-17 feature: FG 23-5-2x)

The UE must report support for each Rel-16/Rel-17 feature in combination with the unified TCI framework

- We note that option 2 has been implemented for mTRP BFR (FG 40-1-1b), but not for any of the other schemes.

Overall, our preference is option 1, and we propose to clarify that with notes in FG 40-1-1 and FG 40-1-7.

## 2.1.1.4	Updated list of features for FG 40-1-x

| Index | Feature group | Components | Prerequisite FGs | Consequence if the feature is not supported by the UE | Type | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 40-1-1 | Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC | 1. Maximum number of configured joint TCI states per CC per BWP 2. Maximum number of activated joint TCI states across all CCs | FFS 23-1-1, 16-2b-1 | Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC is not supported | Per band | Component 1 candidate values: {8, 12, 16, 24, 32, 48, 64, 128} Component 2 candidate values: {2, 4, 6, 8, 16, 32} Note: FG 16-2b-0 can be used to indicate support of two default beams Note: A UE that supports FG 40-1-1 supports any of the single-DCI schemes from Rel-16/Rel-17 with the mTRP extension of the unified TCI framework, provided that the corresponding Rel-16/Rel-17 feature is supported. |
| 40-1-1a | Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC | 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2) with DL assignment b) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2) without DL assignment 2. M Maximum number of activated joint TCI states per CC | 40-1-1 | Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC is not supported | Per band | Component 1 candidate values: {with DL assignment, without DL assignment, both} Component 2 candidate values: {2,3, 4, 5, 6, 7, 8} Note: FG 16-2b-0 can be used to indicate support of two default beams |
| 40-1-1b | TRP-specific BFR with unified TCI framework with Unified TCI | Support for TRP-specific BFR with unified TCI framework with Unified TCI | FFS 40-1-1, 23-5-2 | TRP-specific BFR with unified TCI framework with Unified TCI is not supported | Per band |  |
| 40-1-1c | DCI format 1_1 and if supported 1_2 configured with TCI selection field | Support of DCI format 1_1 and if supported 1_2 configured with TCI selection field | At least one of {40-1-1/1a/2/2a} | DCI format 1_1 and if supported 1_2 configured with TCI selection field are not supported | Per band |  |
| 40-1-2 | Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC | Maximum number of configured DL TCI states per CC per BWP Maximum number of configured UL TCI states per CC per BWP Maximum number of activated DL TCI states across all CCs Maximum number of activated UL TCI states across all CCs | 40-1-1, 23-10-1 | Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC is not supported | Per band | Component 1 candidate values: {4,8,12,16,24,32,48,64,128} Component 2 candidate values: {4,8,12,16,24,32,48,64} Component 3 candidate values: {2,4,8,16} Component 4 candidate values: {2,4,8,16} Note: FG 16-2b-0 can be used to indicate support of two default beams |
| 40-1-2a | Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC | 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2) with DL assignment b) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2) without DL assignment 2. M aximum number of activated DL TCI states across all CCs 3. Maximum number of activated UL TCI states across all CCs | FFS 40-1-2 | Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC is not supported | Per band | Component 2 candidate values: {2,4,8,16} Component 3 candidate values: {2,4,8,16} Note: FG 16-2b-0 can be used to indicate support of two default beams |
| 40-1-3 | Per aperiodic CSI-RS resource/resource set configuration for TCI selection in S-DCI based MTRP | Support of per aperiodic CSI-RS resource/resource set configuration for TCI selection in S-DCI based MTRP | 40-1-1 | Per aperiodic CSI-RS resource/resource set configuration for TCI selection in S-DCI based MTRP is not supported | Per band | Component candidate values: {per resource, per resource set, both} |
| 40-1-4 | Two TCI states for CJT Tx scheme for PDSCH | Support of two TCI states for CJT Tx scheme for PDSCH | 40-1-1, 40-3-1 | Two TCI states for CJT Tx scheme for PDSCH are not supported | Per band | Component candidate values: {CJT Scheme-A, CJT scheme-B, both} CJT Scheme-A: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint/DL TCI states with respect to QCL-TypeA CJT Scheme-B: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint/DL TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint/DL TCI state |
| 40-1-7 | Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC | 1. Support of mTRP operation for M-DCI with joint TCI state 3. Maximum number of configured joint TCI states per BWP per CC 4. Maximum number of activated joint TCI states across all CCs per ‘coresetPoolIndex’ value 5. One MAC-CE activates one joint TCI-states per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value | [16-2a, 23-4, 40-6-1, 40-6-2, 40-6-3a, 40-6-3b, 40-6-4] 23-1-1 | Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC is not supported | Per band | Component 1 candidate values {intra-cell, intra-cell and inter-cell} Component 3 candidate values: {8, 12, 16, 24, 32, 48, 64, 128} Component 4 candidate values: {1, 2, 4, 8, 16} Note: activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH transmissions Note: FG 16-2a-6 can be used to indicate support of two default beams Note: A UE that supports FG 40-1-7 supports any of the multi-DCI schemes from Rel-16/Rel-17 with the mTRP extension of the unified TCI framework, provided that the corresponding Rel-16/Rel-17 feature is supported. |
| 40-1-7a | Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC | 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2) with DL assignment b) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2) without DL assignment 2. Maximum number of MAC-CE activated joint TCI states per CC per coresetpoolindex | 40-1-7 [, FFS] | Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC is not supported | Per band | Component 2 candidate values: {2,3,4,5,6,7,8) |
| 40-1-9 | Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC | 0. Support of mTRP operation for M-DCI with separate DL/UL TCI state 1. Maximum number of configured DL TCI states per BWP per CC 2. Maximum number of configured UL TCI states per BWP per CC 3. Maximum number of activated DL TCI states across all CC 4. Maximum number of activated UL TCI states across all CC 5. One MAC-CE activated DL TCI-state per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value. 6. One MAC-CE activated UL TCI-state per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value. | [23-10-1], 40-1-7 | Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC is not supported | Per band | Component 0 candidate values {intra-cell, intra-cell and inter-cell} Component 1 candidate value {8, 12, 16, 24, 32, 48, 64, 128} Component 2 candidate value {8, 12, 16, 24, 32, 48, 64} Component 3 candidate values: {1, 2, 4, 8, 16} Component 4 candidate values: {1, 2, 4, 8, 16} |
| 40-1-9a | Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC | 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1 and if supported 1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1 and if supported 1_2 without DL assignment) 2. maximum number of activated DL TCI states per CORESETPoolIndex per BWP per CC 3. maximum number of activated UL TCI states per CORESETPoolIndex per BWP per CC | [23-10-1b, 40-1-9] | Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC is not supported | Per band | Component 2 candidate values: {1, 2, 3, 4, 5, 6, 7, 8} Component 3 candidate values: {1, 2, 3, 4, 5, 6, 7, 8,} |
| 40-1-12 | Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP | 1. Support of common multi-CC TCI state ID update and activation for single-DCI based multi-TRP 2. Maximum number of CC list(s) | [40-1-1, 40-1-1, 40-1-2] | Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP is not supported | Per band | Component 2 candidate values: {1,2,3,4} |
| 40-1-13 | Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP | 1. Support of common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP 2. Maximum number of CC list(s) | [40-1-7] | Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP is not supported | Per band | Component 2 candidate values: {1,2,3,4} |
| 40-1-14 | Per aperiodic CSI-RS resource/resource set configuration for TCI selection in M-DCI based MTRP | Support of per aperiodic CSI-RS resource/resource set configuration for TCI selection in M-DCI based MTRP | 40-1-7 | Per aperiodic CSI-RS resource/resource set configuration for TCI selection in S-DCI based MTRP is not supported | Per band | Component candidate values: {per resource, per resource set, both} |

## 2.1.2	Two TAs for multi-DCI

## 2.1.2.1	FGs included in [5]

For two TA, [5] contains a few feature groups with some yellow highlighting or missing content. We address them one by one.

FG 40-2-2: Two TA enhancement for multi-DCI based intra-cell Multi-TRP operation should be a prerequisite. FG 16-2a is then superfluous.

FG 40-2-3: This only works if there are two TAG Id configured, which requires the UE to support 40-2-1.

FG 40-2-4: This requires that the UE supports the indication of a second TAG Id in RAR, which requires the UE to support 40-2-2.

FG 40-2-4a: This requires that the UE supports the indication of a second TAG Id in RAR, which requires the UE to support 40-2-1.

FG 40-2-6: There is no need for a prerequisite here.

FG 40-2-7: This requires that UE supports 40-2-1. It also requires support of STxMP, either codebook-based (40-6-3a) or non-codebook based (40-6-3b).

FG 40-2-8: This requires that UE supports 40-2-1.

FG 40-2-9: This requires that UE supports 40-2-1.

The changes are summarized below.

| Index | Feature group | Components | Prerequisite feature groups | Consequence if the feature is not supported by the UE | Type | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 40-2-1 | Basic feature for multi-DCI based intra-cell Multi-TRP operation with two TA enhancement | Support of two TA enhancement for multi-DCI based intra-cell Multi-TRP operation | 16-2a | Two TA enhancement for multi-DCI based intra-cell Multi-TRP operation is not supported | Per FSPC |  |
| 40-2-2 | Basic feature for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement | 1. Support of two TA enhancement for multi-DCI based inter-cell Multi-TRP operation 2. Maximum number of n-TimingAdvanceOffset value per serving cell | 16-2a 40-2-1 | Two TA enhancement for multi-DCI based inter-cell Multi-TRP operation is not supported | Per FSPC | Component 2 candidate values: {1,2} |
| 40-2-3 | TAG ID indication via absolute TA command MAC CE | Support indicating one of two TAG IDs configured in the SpCell via absolute TA command MAC CE. | 40-2-1 | TAG ID indication via absolute TA command MAC CE is not supported | Per band |  |
| 40-2-4 | PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA for inter-cell | Support of cross-TRP PDCCH order based on CFRA for inter-cell multi-DCI based mTRP | 40-2-2 | Inter-cell cross-TRP PDCCH ordered PRACH transmission is not supported | Per band |  |
| 40-2-4a | PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA for intra-cell | Support of cross-TRP PDCCH order based on CFRA for intra-cell multi-DCI based mTRP | 40-2-1 | Intra-cell cross-TRP PDCCH ordered PRACH transmission is not supported | Per band |  |
| 40-2-6 | Rx timing difference larger than CP length | 1. Support of the Rx timing difference between the two DL reference timings is larger than CP length |  | Rx timing difference larger than CP is not supported | Per FSPC |  |
| 40-2-7 | Two TAs for multi-DCI STxMP PUSCH+PUSCH | Support of two TAs for multi-DCI STxMP PUSCH+PUSCH | [40-2-1 or 40-2-2, FFS more], 40-6-3a or 40-6-3b | Two TAs for multi-DCI STxMP PUSCH+PUSCH is not supported | Per FSPC |  |
| 40-2-8 | Maximum number of TAGs across all CCs | Maximum number of TAGs across all CCs | FFS 40-2-1 | Maximum number of TAGs across all CCs is unknown when UE supports two TAGs per CC | Per BC | Component candidate values: {2,3,4} Note: UE only supports the configuration where all UL CCs of the same frequency band are configured with up to 2 Timing Advance Group ID Note: The same description of “supportedNumberTAG” in 38.306 applies to this FG as well |
| 40-2-9 | Overlapping UL transmission reduction | Support of reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions when the UE is not configured with UL STx2P for multi-DCI based multi-TRP operation with two TA enhancement | 40-2-1 | Reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions is not supported | Per band | Note: If UE does not support this feature, UE does not expect the two UL transmissions to overlap (i.e., scheduling restriction is applied to avoid overlap between the two UL transmissions) |

## 2.2	CSI enhancement

## 2.2.1 	FGs included in [5]

For CSI, [5] contains a few feature groups with some yellow highlighting or missing content. We address them one by one.

FG 40-3-1-1: for this feature the basic CSI reporting framework feature 2-35 should be the pre-requisite.

FG 40-3-1-3: Component 2 currently has a yellow part on ‘[across all CCs]’.  In our view, the description for component 2 of this FG should be aligned with the description of component 5 of FG 40-3-1-1.  Our proposed change is given in the table below.

FG 40-3-1-5: for this feature the basic CSI reporting framework feature 2-35 should be the pre-requisite.

FG 40-3-3-1: for this feature the basic CSI reporting framework feature 2-35 should be the pre-requisite.  Regarding candidate values for item 4, they are agreed in RAN1 already to be {1, 2}.  The candidate values for item 6 can be {4, 6, 8, 10, 12, ..., 64} where the minimum is chosen to be 4 since the basic feature for TDCP reporting needs TRSs in two slots with four NZP CSI-RSs.

FG 40-3-3-1a: for this feature, the brackets around the ‘Note’ in 2nd last column can be removed.

FG 40-3-3-5: for this feature, since each CC can have 1, 2, or 3 TRS resource sets, the number of configured CSI-RS resources for TDCP per CC can be {4, 6, 8, 10, 12} for component 1.  For component 2, the candidate values can be {4, 6, 8, 10, 12, ..., 64}.  For component 3, the candidate values are {4, 6, 8, 10, 12}.  Furthermore, the components in this feature also need to be defined for the basic feature.  Hence, we propose to that feature 40-3-3-5 be reported together with feature 40-3-1-1.

The changes are summarized below:

| Index | Feature group | Components | Prerequisite feature groups | Consequence if the feature is not supported by the UE | Type | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 40-3-1-1 | Basic feature for Rel-16-based CJT type-II codebook | Support of N=N_TRP only Support of N_L=1 only 1. Support of mode 2 for Rel-16 eType-II codebook refinement for multi-TRP CJT 2. Support for PMI subband R=1. 3. Support of parameter combinations with L=2,4 4. Support of rank 1,2 5. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT] 6. Supported frequency basis selection mode 2, i.e., common frequency basis selection among different TRPs 7. Scaling factor X for CPU occupation counting for Rel-16-based CJT type-II codebook 8. Maximum number of NZP CSI-RS resources in one NZP CSI-RS resource set associated with multi-TRP CJT | [2-35] 2-35 | Mode 2 for Rel-16-based CJT type-II codebook is not supported | Per Band and Per BC | Component 5 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} Component 7 candidate values: {1, 1.5, 2} Component 8 candidate values: {2,3,4} Note: When NTRP=1 TRP is configured, OCPU =1. When NTRP>1 TRPS are configured, OCPU = ceil(X * NTRP) Note: A-CSI is supported, and whether UE supports SP-CSI on PUSCH is dependent on FG2-32b Note: A UE that supports CSI enhancement for Rel. 16 based type-II CJT must support this FG |
| 40-3-1-3 | Support R=2 for Rel-16-based CJT codebook | 1. Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with PMI subbands R=2 2. {Max # of Tx ports in one resource set, Max # of resource sets, total # of Tx ports}, [across all CCs] simultaneously, with R=2 2. A list of supported combinations, up to 16, across all CCs simultaneously with R=2, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT | 40-3-1-1 | R=2 for Rel-16-based CJT codebook is not supported | Per band Per BC | Component 2 candidate values: a) {4,8,12,16,24,32} b) {2 to 64} c) {4 to 256} |
| 40-3-1-5 | Basic feature for Rel-17-based CJT type-II codebook | Support of N=N_TRP only Support of N_L=1 only 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT 2. Support of PMI subband R=1. 3. Support of parameter combinations with M=1 4. Support of rank 1,2 5. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT 6. Supported frequency basis selection mode 2, i.e., common frequency basis selection among different TRPs 7. Scaling factor X for CPU occupation counting for Rel-17-based CJT type-II codebook 8. Maximum number of NZP CSI-RS resources in one NZP CSI-RS resource set associated with multi-TRP CJT | [2-35] 2-35 | Mode 2 for Rel-17-based CJT type-II codebook is not supported | Per band and Per BC | Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} Component 7 candidate values: {1, 1.5, 2} Component 8 candidate values: {2,3,4} Note: When NTRP=1 TRP is configured, OCPU =1. When NTRP>1 TRPS are configured, OCPU = ceil(X * NTRP) Note: A-CSI is supported, and whether UE supports SP-CSI on PUSCH is dependent on FG2-32b Note: A UE that supports CSI enhancement for Rel 17 based type-II CJT must support this FG |
| 40-3-3-1 | TDCP (Time Domain Channel Properties) report | 1. Support of Y=1 delay value for TDCP report 2. Basic delay value, component candidate value <= D_basic = 1 slot 3. Support of amplitude report 4. Value of X for CPU occupation (OCPU=(Y+1).X) 5. Support to configure KTRS = 1 TRS resource set 6. Maximum number of simultaneously active CSI-RS resources for TDCP across all CCs | [2-35] 2-35 | Time Domain Channel Properties report is not supported | Per band and Per BC | Note: counting of simultaneously active CSI-RS resources follows existing specification TS38.214 Candidate values for Component 4: {1, 2} Candidate values for Component 6: {4, 6, 8, 10, 12, ...., 64} |
| 40-3-3-1a | Supported maximum delay value larger than D_basic | Support of maximum delay value larger than D_basic =1 slot | 40-3-3-1 | delay value(s) larger than D_basic are not supported | Per FS | Candidate values: {2 slots, 3 slots, 4 slots, 5 slots, 6 slots, 10 slots} [Note: 10 slots is only applicable for SCS >= 30 kHz, and 6 slots is maximum for SCS = 15 kHz] Note: 10 slots is only applicable for SCS >= 30 kHz, and 6 slots is maximum for SCS = 15 kHz |
| 40-3-3-5 | Number of CSI-RS resources for TDCP | 1. Maximum number of configured CSI-RS resources for TDCP per CC 2. Maximum number of configured CSI-RS resources for TDCP across all CCs 3. Maximum number of simultaneously active CSI-RS resources for TDCP per CC Note: counting of simultaneously active CSI-RS resources follows existing specification TS38.214 | 40-3-3-1 | Number of CSI-RS resources for TDCP is not reported | Per band and Per BC | Candidate values for Component 1: {4, 6, 8, 10, 12} Candidate values for Component 2: {4, 6, 8, 10, 12, ..., 64} Candidate values for Component 1: {4, 6, 8, 10, 12} Note: this featue is reported together with 40-3-3-1 |

## 2.3		Reference signal enhancement

## 2.3.1	Increased number of orthogonal DMRS ports

We continue to address the remaining capabilities for DMRS enhancement. From 38.822 UE feature list and 38.306 UE radio access capabilities one can find the legacy mirror features [3] [4]. Following these two specs as reference, we propose to make updates marked with red text in the list of FGs below. The red text mainly addresses the prerequisite each feature. In addition, a clarification in 40-4-1j is suggested.

| Index | Feature group | Components/comment | Prerequisite feature groups | Type |
| --- | --- | --- | --- | --- |
| 40-4-1 | Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type A | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol | [2-5] | Per FS |
| 40-4-1a | Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type B | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol | [2-5] | Per FS |
| 40-4-1i | 2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8 | Support of 2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8 | 40-4-1, FFS more40-4-1h | Per FS |
| 40-4-1j | Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port for mapping type A (downlink) | Support of Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port for mapping type A | 40-4-1 | Per FS |
| 40-4-7 | Rel-18 DL DMRS with M-DCI based M-TRP | Support of Rel-18 DL DMRS with multi- DCI based M-TRP PDSCH operation | FFS40-4-1 | Per FS |
| 40-4-6 | Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type A for Rel.18 enhanced DMRS ports | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbols 3) Support 1 symbol FL DMRS and 2 additional DMRS symbols | [2-16] | Per FS |
| 40-4-6a | Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type B for Rel.18 enhanced DMRS ports | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol | [2-16a] | Per FS |
| 40-4-6g | 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 | Support of 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 | 40-4-6, FFS more | Per FS |
| 40-4-6h | 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 | Support of 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 | 40-4-6, FFS more, 40-4-6g | Per FS |
| 40-4-6i | 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 | Support of 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 | 40-4-6, FFS more40-4-6g | Per FS |
| 40-4-6j | 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 | Support of 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 | 40-4-6, FFS more40-4-6h, 40-4-6g | Per FS |
| 40-4-10 | DMRS port configuration for PUSCH [with rank 5-8/ with 8Tx] | DMRS port configuration for PUSCH [with rank 5-8 / with 8Tx] for Rel 15 and Rel. 18 | [40-4-6] | Per FS |

## 2.3.2	SRS enhancements targeting TDD CJT and 8 TX operation

A set of FGs for SRS are provided in [5], some of which contain FFSs and/or yellow highlighting. In the below table, suggest amendments to the set of FGs to address said FFS and/or yellow highlighting . In our view, FG 2-52 (Basic SRS) should be a prerequisite for FG 40-5-1 (SRS comb offset hopping) and 40-5-2 (SRS cyclic shift hopping). Furthermore, FG 2-55 (SRS Tx switch) should be a prerequisite for FG 40-5-4 (SRS 8 Tx ports—antenna switching).

| 40. NR_MIMO_evo_DL_UL | 40-5-1 | SRS comb offset hopping | Support of SRS comb offset hopping | FFS 2-52 | Yes | n/a | SRS comb offset hopping is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-5-2 | SRS cyclic shift hopping | Support of SRS cyclic shift hopping | [2-53] 2-52 | Yes | n/a | SRS cyclic shift hopping is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-2a | Smaller cyclic shift granularity for cyclic shift hopping | Support configuration of cyclic shift hopping with smaller granularity (with factor K=2) | 40-5-2 | Yes | N/A |  | Per band | n/a | n/a | n/a | Configuration of cyclic shift hopping with smaller granularity (with factor K=2) is not supported | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-1c | Comb offset hopping within a subset | Support configuration of subset of comb offsets for comb offset hopping | 40-5-1 | Yes | N/A |  | Per band | n/a | n/a | n/a | Configuration of subset of comb offsets for comb offset hopping is not supported | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-2c | Cyclic shift hopping within a subset | Support configuration of subset of cyclic shifts for cyclic shift hopping | 40-5-2 | Yes | N/A |  | Per band | n/a | n/a | n/a | Configuration of subset of cyclic shifts for cyclic shift hopping is not supported | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-1b | SRS comb offset hopping combined with legacy group/sequence hopping | Support of SRS comb offset hopping combined with legacy group/sequence hopping | 40-5-1 | Yes | n/a | SRS comb offset hopping combined with legacy group/sequence hopping is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-2b | SRS cyclic shift hopping combined with legacy group/sequence hopping | Support of SRS cyclic shift hopping combined with legacy group/sequence hopping | 40-5-2 | Yes | n/a | SRS cyclic shift hopping combined with legacy group/sequence hopping is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-3 | SRS cyclic shift hopping combined with SRS comb offset hopping | Support of SRS cyclic shift hopping combined SRS comb offset hopping | 40-5-1, 40-5-2 | Yes | n/a | SRS cyclic shift hopping combined with SRS comb offset hopping is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-1a | Comb offset hopping time-domain behavior when repetition factor R>1 | Supported comb offset hopping granularity in time when repetition factor R>1 is configured | 40-5-1 | Yes | n/a | Comb offset hopping time-domain behavior when repetition factor R>1 is not supported | Per band | n/a | n/a | n/a | Component candidate values: {‘per SRS symbol’, ’per R SRS symbols’, ‘both’} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-4 | SRS 8 Tx ports—antenna switching | 1. Support of 8T8R for antenna switching 2. Downgrade antenna switching configurations 3. Report the entry number of the first-listed band with UL in the band combination that affects this DL 4. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL | FFS 2-55 | Yes | n/a | SRS with 8 Tx ports—antenna switching is not supported | Per FS | n/a | n/a | n/a | Component 1 candidate values: {noTDM, TDM and noTDM} Component 2 candidate value: combination (including empty) of {1T1R, 1T2R, 1T4R, 1T6R, 1T8R, 2T2R, 2T4R, 2T6R, 2T8R, 4T4R, 4T8R} Component 3 candidate value: {1,2,…,32} Component 4 candidate value: {1,2,…,32} Note: UE reports support of SRS with 8 Tx ports and Comb8 mapping —antenna switching via FG 23-8-8 | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-5 | Maximum 2 SP and 1 periodic SRS sets for 8T8R antenna switching | Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for 8T8R antenna switching | 40-5-4 | Yes |  | Maximum 2 SP and 1 periodic SRS sets for 8T8R antenna switching is not supported | Per FS | n/a | n/a | n/a | Note: If UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS Note: The two SP-SRS resource sets are not activated at the same time | Optional with capability signaling |

## 2.4 	Enhanced uplink transmission

## 2.4.1	UL precoding indication for multi-panel transmission

A set of FGs for STxMP are provided in [5], some of which contain FFSs and/or yellow highlighting. In the below table, suggest amendments to the set of FGs to address said FFS and/or yellow highlighting. In our view, FG 40-6-3a should be the prerequisite for FGs 40-6-3h and 40-6-4i, i.e., remove the brackets. In addition, clarify in fourth column (components) that these FGs are for codebook (for consistency with corresponding FGs 40-6-3o and 40-6-3p for noncodebook). For an issue raised on the RAN1 reflector, the brackets around FGs 40-6-3j, 40-6-3k, 40-6-3l, 40-6-3m, and 40-6-3n should be removed. FG 40-6-3b should be the prerequisite for FGs 40-6-3o and 40-6-4p. For FG 40-6-3b, the prerequisite should be 2-15a and 40-6-3b. We think that FG 40-6-3b-1 doesn’t need any prerequisites. Furthermore, correcting a minor typo: a left bracket in the fifth column (prerequisites) of FG 40-6-1b should be removed.

| 40. NR_MIMO_evo_DL_UL | 40-6-1 | Single-DCI based STx2P SDM scheme for PUSCH—codebook | 1. Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP for PUSCH—codebook 2. 1 PTRS port for single-DCI based STx2P SDM scheme for PUSCH—codebook 3. Support of two SRS resource sets with usage set to 'codebook' 4. Maximum number of SRS resources in one SRS resource set 5. Maximum number of layers of each panel for Single-DCI STx2P with SDM 7. Max number of NZP PUSCH ports associated with one SRS resource set 8. Maximum number of SRS antenna ports for each SRS resource in each SRS resource set | 2-14 | Yes | N/A | Single-DCI based STx2P SDM scheme for PUSCH—codebook is not supported | Per FSPC | No | FR2 only | n/a | Component 4 candidate values: {1, 2 ,4} Component 5 candidate values: {1, 2} Component 7 candidate values: {1, 2 ,4} Component 8 candidate values: {1, 2 ,4} Note: For component 7, if a row of the TPMI consists of all 0’s, the corresponding PUSCH port is not counted Note: If value 4 is reported for component 4, UE also reports value 4 in FG 16-5c | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-6-1-1 | 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—codebook | Support of 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—codebook | 40-6-1 | Yes | N/A | 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—codebook is not supported | Per Band | No | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-1a | Single-DCI based STx2P SDM scheme for PUSCH—noncodebook | 1. Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP for PUSCH—noncodebook 2. 1 PTRS port for single-DCI based STx2P SDM scheme for PUSCH—noncodebook 3. Support of two SRS resource sets with usage set to 'noncodebook' 4. Maximum number of SRS resources in one SRS resource set 5. Maximum number of layers of each panel for Single-DCI STx2P with SDM 8. Maximum number of simultaneous transmitted SRS resources from one SRS resource set at one symbol | 2-15 | Yes | N/A | Single-DCI based STx2P SDM scheme for PUSCH—noncodebook is not supported | Per FSPC | No | FR2 only | n/a | Component 4 candidate values: {1, 2 ,3, 4} Component 5 candidate values: {1, 2} Component 8 candidate values: {1, 2, 3, 4} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-1a-1 | 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—noncodebook | Support of 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—noncodebook | 40-6-1a | Yes | N/A | 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—noncodebook is not supported | Per Band | No | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-1b | Association between CSI-RS and SRS for noncodebook single-DCI based STx2P SDM scheme for PUSCH | 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook based STxMP SDM scheme for PUSCH 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously | [2-15a, 40-6-1a | Yes | N/A | Associated CSI-RS resources for non-codebook single-DCI based STxMP SDM scheme for PUSCH is not supported | Per Band | n/a | FR2 only | n/a | Component 2 candidate values: {1, 2,…,8} Component 3 candidate values: {1, 2,…,8} Component 4 candidate values: {0,1, 2,…,8} Component 5 candidate values: {1, 2,…,16} Component 6 candidate values: {1,2} | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-6-1-2 | New DMRS port entry for single-DCI based SDM scheme | Support of new DMRS port entry {0, 2, 3} | 40-6-1,40-6-1a | Yes | N/A | New DMRS port entry for single-DCI based SDM scheme is not supported | Per Band | n/a | FR2 only | n/a |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-6-2 | Single-DCI based STx2P SFN scheme for PUSCH—codebook | 2.Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP 3. 1 PTRS port for single-DCI based STx2P SFN scheme for PUSCH—codebook 4. Support of two SRS resource sets with usage set to 'codebook' 5. Maximum number of SRS resources in one SRS resource set 6. Maximum number of MIMO layers of each SRS resource set for CB PUSCH with SFN scheme 7. Maximum number of SRS antenna ports for each SRS resource in each SRS resource set 9. Max number of NZP PUSCH ports associated with one SRS resource set | 2-14 | Yes | n/a | Single-DCI based STx2P SFN scheme for PUSCH—codebook is not supported | Per FSPC | n/a | FR2 only | n/a | Component 5 candidate values: {1, 2 ,4} Component 6 candidate values: {1, 2} Component 7 candidate values: {1, 2, 4} Component 9 candidate values: {1, 2, 4} Note: For component 9, if a row of the TPMI consists of all 0’s, the corresponding PUSCH port is not counted Note: If value 4 is reported for component 5, UE also reports value 4 in FG 16-5c | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-2-1 | 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—codebook | Support of 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—codebook | 40-6-2 | Yes | n/a | 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—codebook is not supported | Per Band | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-2a | Single-DCI based STx2P SFN scheme for PUSCH—noncodebook | 2.Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP 3. 1 PTRS port for single-DCI based STx2P SFN scheme for PUSCH—noncodebook 4. Support of two SRS resource sets with usage set to 'noncodebook' 5. Maximum number of SRS resources in one SRS resource set 6. Maximum number of MIMO layers of each SRS resource set for NCB PUSCH with SFN scheme 8. Maximum number of simultaneous transmitted SRS resources from one SRS resource set at one symbol | 2-15 | Yes | n/a | Single-DCI based STx2P SFN scheme for PUSCH—noncodebook is not supported | Per FSPC | n/a | FR2 only | n/a | Component 5 candidate values: {1, 2 ,3, 4} Component 6 candidate values: {1, 2} Component 8 candidate values: {1, 2, 3, 4} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-2a-1 | 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—noncodebook | Support of 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—noncodebook | 40-6-2a | Yes | n/a | 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—noncodebook is not supported | Per Band | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-2b | Association between CSI-RS and SRS for noncodebook single-DCI based STx2P SFN scheme for PUSCH | 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook based STxMP SFN scheme for PUSCH 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously | 2-15a, 40-6-2a | Yes | n/a | Associated CSI-RS resources for non-codebook single-DCI based STxMP SFN scheme for PUSCH is not supported | Per Band | n/a | FR2 only | n/a | Component 2 candidate values: {1, 2,…,8} Component 3 candidate values: {1, 2,…,8} Component 4 candidate values: {0,1, 2,…,8} Component 5 candidate values: {1, 2,…,16} Component 6 candidate values: {1,2} | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3a | codebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG | 1. Support of multi-DCI based STx2P PUSCH+PUSCH for codebook-based PUSCH with fully overlapping PUSCHs in time and overlapping in frequency 2. Support of two SRS resource sets with usage set to 'codebook' associated with two coresetPoolIndex values 3. Maximum number of SRS resources in one SRS resource set 4. Maximum number of layers of each PUSCH of PUSCH+PUSCH overlapping in time domain 5. Maximum number of NZP PUSCH ports for each PUSCH of PUSCH+PUSCH overlapping in time domain 6. Maximum number of PUSCHs per CORESETPoolIndex per slot 7. Maximum total number of layers across two overlapping PUSCH 8. Maximum number of SRS antenna ports for each SRS resource in each SRS resource set | 2-14 | Yes | N/A | Codebook multi-DCI based STx2P PUSCH+PUSCH is not supported | Per FSPC | n/a | FR2 only | n/a | Note: Processing capability 2 is not supported in any CC if at least one CC is configured with two values of CORESETPoolIndex. Component 3 candidate values: {1,2,4} Note: If value 4 is reported for component 3, UE also reports value 4 in FG 16-5c. Component 4 candidate values: {1,2} Component 5 candidate values: {1,2,4} Note: If a row of the TPMI consists of all 0’s, the corresponding PUSCH port is not counted Component 6 candidate values: {1,2,3,4,7} Note: per SCS, similar with Rel-15 Component 7 candidate values: {2 ,3, 4} Component 8 candidate values: {1, 2 ,4} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3c | Codebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping PUSCHs in time and fully overlapping in frequency | Support of fully overlapping PUSCHs in time and fully overlapping in frequency | 40-6-3a | Yes | N/A | Fuly overlapping PUSCHs in time and fully overlapping in frequency for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported | Per Band | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3d | Codebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping PUSCHs in time and partially overlapping in frequency | Support of fully overlapping PUSCHs in time and partially overlapping in frequency | 40-6-3a | Yes | N/A | Fully overlapping PUSCHs in time and partially overlapping in frequency for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported | Per Band | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3e | Codebook multi-DCI based STx2P PUSCH+PUSCH – Partially overlapping PUSCHs in time and fully overlapping in frequency | Support of partially overlapping PUSCHs in time and fully overlapping in frequency | 40-6-3a | Yes | N/A | Partially overlapping PUSCHs in time and and fully overlapping in frequency for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported | Per Band | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3f | Codebook multi-DCI based STx2P PUSCH+PUSCH – Partially overlapping PUSCHs in time, partially overlapping in frequency | Support of partially overlapping PUSCHs in time, partially overlapping in frequency | 40-6-3a | Yes | N/A | Partially overlapping PUSCHs in time, partially overlapping in frequency for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported | Per Band | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3g | Codebook multi-DCI based STx2P PUSCH+PUSCH – Partially =overlapping PUSCHs in time, non- overlapping in frequency | Support of partially overlapping PUSCHs in time, non-overlapping in frequency | 40-6-3a | Yes | N/A | Partially overlapping PUSCHs in time, non-overlapping in frequencyfor codebook multi-DCI based STx2P PUSCH+PUSCH is not supported | Per Band | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3h | Codebook multi-DCI based STx2P PUSCH+PUSCH for CG+CG | Support of multi-DCI based STxMP CG-PUSCH+CG-PUSCH for codebook | [40-6-3a] 40-6-3a | Yes | N/A | Multi-DCI based STx2P for CG+CG is not supported for codebook | Per Band | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3i | Codebook multi-DCI based STx2P PUSCH+PUSCH for DG+CG | Support of multi-DCI based STxMP DG-PUSCH+CG-PUSCH for codebook | [40-6-3a] 40-6-3a | Yes | N/A | Multi-DCI based STx2P for DG+CG is not supported for codebook | Per Band | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3b | Noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG | 1. Support of multi-DCI based STxMP PUSCH+PUSCH for noncodebook-based PUSCH with fully overlapping PUSCHs in time and non-overlapping in frequency 2. Support of two SRS resource sets with usage set to 'noncodebook' associated with two coresetPoolInde values 3. Maximum number of SRS resources in one SRS resource set 4. Maximum number of layers of each PUSCH of PUSCH+PUSCH overlapping in time domain 5. Maximum number of simultaneously transmitted SRS resources in one symbol per SRS resource set 6. Maximum number of PUSCHs per coresetPoolIndex per slot 7. Maximum total number of layers across two overlapping PUSCH | 2-15 | Yes | N/A | Noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported | Per FSPC | n/a | FR2 only | n/a | Note: Processing capability 2 is not supported in any CC if at least one CC is configured with two values of CORESETPoolIndex. Component 3 candidate values: {1,2,3,4} Component 4 candidate values: {1,2} Component 5 candidate values: {1,2,3,4} Component 6 candidate values: {1,2,3,4,7} Note: per SCS, similar with Rel-15 Component 7 candidate values: {2 ,3, 4} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3j | [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping PUSCHs in time and fully overlapping in frequency] | [Support of fully overlapping PUSCHs in time and fully overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH] | 40-6-3b | Yes | N/A | Fully overlapping PUSCHs in time and fully overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported | Per Band | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3k | [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping PUSCHs in time and partially overlapping in frequency] | [Support of fully overlapping PUSCHs in time and partially overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH] | 40-6-3b | Yes | N/A | Fully overlapping PUSCHs in time and partially overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported | Per Band | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3l | [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Partially overlapping PUSCHs in time and fully overlapping in frequency] | [Support of partially overlapping PUSCHs in time and fully overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH] | 40-6-3b | Yes | N/A | Partially overlapping PUSCHs in time and fully overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported | Per Band | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3m | [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Partially overlapping PUSCHs in time, partially overlapping in frequency] | [Support of partially overlapping PUSCHs in time, partially overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH] | 40-6-3b | Yes | N/A | Partially overlapping PUSCHs in time, partially overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported | Per Band | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3n | [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Partially overlapping PUSCHs in time, non-overlapping in frequency] | [Support of partially overlapping PUSCHs in time, non-overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH] | 40-6-3b | Yes | N/A | Partially overlapping PUSCHs in time, non-overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported | Per Band | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3o | Noncodebook multi-DCI based STx2P PUSCH+PUSCH for CG+CG | Support of multi-DCI based STxMP CG-PUSCH+CG-PUSCH for noncodebook | [40-6-3a] 40-6-3b | Yes | N/A | Multi-DCI based STx2P for CG+CG is not supported for noncodebook | Per Band | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3p | Noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+CG | Support of multi-DCI based STxMP DG-PUSCH+CG-PUSCH for noncodebook | [40-6-3a] 40-6-3b | Yes | N/A | Multi-DCI based STx2P for DG+CG is not supported for noncodebook | Per Band | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3b-1 | Associated CSI-RS resources for noncodebook multi-DCI based STx2P PUSCH+PUSCH | 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for multi-DCI non-codebook based STxMP scheme for PUSCH 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously | FFS 2-15a, 40-6-3b | Yes | n/a | Associated CSI-RS resources for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported | Per Band | n/a | FR2 only | n/a | Component 2 candidate values: {1 to 8} Component 3 candidate values: {1 to 8} Component 4 candidate values: {0 to 8} Component 5 candidate values: {1 to 16} Component 6 candidate values: {1 to 2} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-4 | Single-DCI based STx2P SFN scheme for PUCCH | 1. Support of single-DCI based STx2P SFN scheme for PUCCH 2. Supported PUCCH formats for STxMP SFN scheme |  | Yes | n/a | Single-DCI based STx2P SFN scheme for PUCCH is not supported | Per FS | n/a | FR2 only | n/a | Component 2 candidate values: {PF0/2, PF1/3/4, PF0-4) | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-4a | Dynamic indication of repetition number for SFN scheme for PUCCH | Support STxMP SFN PUCCH scheme together with the Rel-17 dynamic indication of repetition number | 40-6-4, 30-5 | Yes | n/a | Dynamic indication of repetition number for SFN scheme for PUCCH is not supported | Per band | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-5 | Support grouped-based beam reporting for STx2P | 1. Support group based L1-RSRP reporting for STxMP based transmission 2. Max number N of beam groups (M=2 beams per beam group) in a single L1-RSRP reporting instance based on measurement on two CMR resource sets 3. Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs 4. Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs | FFS | Yes | n/a | Grouped-based beam reporting for STx2P is not supported | Per Band | n/a | FR2 only | n/a | Component 1 candidate values: {JointULandDL, ULOnly, both} Component 2 candidate values: {1,2,3,4} Component 3 candidate values: {2,3,4,8,16,32,64} Component 4 candidate values: {8, 16, 32, 64, 128} Note: components 3 and 4 are also counted in FG 16-1g, 16-1g-1, and 23-5-1 | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-6 | Out-of-order operation for multi-DCI based STx2P PUSCH+PUSCH | Support of out-of-order operation for multi-DCI based STx2P PUSCH+PUSCH | 40-6-3a or 40-6-3b | Yes | N/A | Out-of-order operation for multi-DCI based STx2P PUSCH+PUSCH is not supported | Per FSPC | N/A | N/A | N/A |  | Optional with capability signaling |

## 2.4.2	SRI/TPMI enhancement for enabling 8 TX UL transmission

In this section, we discuss 8 Tx UL MIMO features as currently reflected in [5], and all changes are with respect to this version of the features.

For 40-7-1:

- Regarding the FFS prerequisites there are no codebook coherency subsets for 8 Tx, and so they can’t be prerequisites like they are in Rel-15 for 2-14, ‘Codebook based PUSCH MIMO transmission’.  This leaves 2-12, ‘basic PUSCH transmission’ as the remaining dependency.

- On whether the maximum number of supported PUSCH MIMO layers can be one or more of 3, 5, 6, and 7 layers (i.e. if the brackets can be removed from any or all of them): we do not see a need for high granularity in the number of MIMO layers, since the WID targeted relatively complex CPE/FWA/vehicle/Industrial devices.  To simplify network operation and avoid excessive market fragmentation, we think {1, 2, 4, 8} layers is enough.  We also note that 3 layer capability can be discussed in the context of the Rel-19 3 Tx UL MIMO work, in case of high interest in 3 layer maximum UE capability.

Regarding the components for 40-7-1a, the IE defining the codebook in the current version of the RRC spec (CodebookTypeUL, copied below) can be codebook1=ng1n4n1 and/or codebook1=ng1n4n1.  In our understanding both can be independently supported, which means they are different components.  Furthermore, there is no independent RRC IE for (N1,N2); the 8 Tx codebook is configured by CodebookTypeUL.  Therefore, in order to reflect RAN2 decisions, we suggest to update component 1 to identify support for codebook1=ng1n4n1 and replace component 2 with support for codebook1=ng1n2n2, indicating that either can be supported with ‘and/or’.

CodebookTypeUL-r18 ::= CHOICE { codebook1-r18 ENUMERATED {ng1n4n1, ng1n2n2}, codebook2-r18 ENUMERATED {ng2}, codebook3-r18 ENUMERATED {ng4}, codebook4-r18 ENUMERATED {ng8} }

For non-codebook based 8 Tx 40-7-2 prerequisites, similar to Rel-16 non-codebook based feature, we can use basic PUSCH transmission 2-12 as the prerequisite.

Our proposals for features 40-7-1, 40-7-1a, and 40-7-2 are then:

| Index | Feature group | Components | Prerequisite feature groups | Note |
| --- | --- | --- | --- | --- |
| 40-7-1 | Basic features for Codebook-based 8Tx PUSCH | 1. Maximum number of PUSCH MIMO layers for codebook based PUSCH 2. Maximum number of 8 port SRS resources per SRS resource set with usage set to ‘codebook’ for codebook-based 8Tx PUSCH 3. SRS 8 Tx ports—codebook | FFS 2-12 | Component 1 candidate values: {1,2 [,3],4 [,5,6,7],8} Component 2 candidate values: {1,2} Component 3 candidate values: {noTDM, TDM and noTDM} A UE that supprts FG 40-7-1 must support at least one of FGs 40-7-1a/b/c/d |
| 40-7-1a | Codebook-based 8Tx PUSCH—codebook1 | 1. Support of codebook-based 8Tx PUSCH— codebook1=ng1n4n1, and/or 2. Support of codebook-based 8Tx PUSCH— codebook1=ng1n2n2 2. Support of (N1, N2) for codebook-based 8Tx PUSCH—codebook1 | 40-7-1 |  |
| 40-7-2 | Basic features for Non-Codebook-based 8Tx PUSCH | 1. Supported maximal PUSCH MIMO layers for non-codebook based PUSCH 2. Supported maximum number of SRS resources per SRS resource set with usage set to ‘nonCodebook’ 3. Maximum number of simultaneous transmitted SRS resources at one symbol | FFS 2-12 |  |

Regarding the prerequisites for UL FP Tx modes 40-7-1e/f/g, they all require codebook based operation, and so 40-7-1 should be sufficient as a prerequisite.  We note that for Rel-16, the prerequisite for the uplink full power modes includes basic codebook based operation (FG 2-14), so using basic codebook based operation for 8 Tx is consistent with the Rel-16 prerequisites. The prerequisite FGs for 40-7-1e/f/g can therefore be set as follows:

| Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- |
| 40-7-1e | UL full power transmission mode 0 | Support of UL full power transmission mode of fullpower when UE is capable of 8 Tx codebook based PUSCH operation | FFS 40-7-1 |
| 40-7-1f | UL full power transmission mode 1 | Support of UL full power transmission mode of fullpowerMode1 when UE is capable of 8 Tx codebook based PUSCH operation | FFS 40-7-1 |
| 40-7-1g | UL full power transmission mode 2 with 1/2/4 resources | 1. Support of UL full power transmission mode of fullpowerMode2 when UE is capable of 8 Tx codebook based PUSCH operation 2. Maximum number of SRS resources in one SRS resource set with usage set to 'codebook' for 8Tx codebook based PUSCH for Mode 2 | FFS 40-7-1 |

For the candidate values of 40-7-1g-1: The SRS resource size combinations in Rel-16 UL FP Tx mode 2 used a simple approach where all combinations of Rel-16 SRS resource sizes were specified, with the constraint that all UEs must support single SRS port transmission with full power. Therefore, combinations 1_2, 1_4, and 1_2_4 were specified. If we extend the Rel-16 approach to the 8 port case, we get the following: {1_2, 1_4, 1_8, 1_2_4, 1_2_8, 1_4_8, 1_2_4_8}.  The Rel-16 feature also allows any of the SRS resource size combinations to be used regardless of if the capability for maximum number of SRS resources is 2 or 4.  Since the capability for the maximum number of SRS resources for UL FP Tx Mode 2 is also 2 or 4 for 8 Tx, we extend the Rel-16 note to be ‘Note: Any of the above states can be used if 40-7-1g is reported as 2 or 4.’  Therefore, we suggest that FG 40-7-1g-1 be updated as follows, where we also make a typo correction to the agreed table: ‘respource’.

| Index | Feature group | Components | Prerequisite feature groups | Note |
| --- | --- | --- | --- | --- |
| 40-7-1g-1 | SRS resources for UL full power transmission mode 2 | 1. SRS configurations with different number of antenna ports per SRS respource for mode 2 | 40-7-1g | Component (1) candidate values: FFS {1_2, 1_4, 1_8, 1_2_4, 1_2_8, 1_4_8, 1_2_4_8} 1st state (1_2): each SRS resource can be configured with 1 port or 2 ports 2nd state (1_4): each SRS resource can be configured with 1 port or 4 ports 3rd state (1_8): each SRS resource can be configured with 1 port or 8 ports 4th state (1_2_4): each SRS resource can be configured with 1 port or 2 ports or 4 ports 5th state (1_2_8): each SRS resource can be configured with 1 port or 2 ports or 8 ports 6th state (1_4_8): each SRS resource can be configured with 1 port or 4 ports or 4 ports 7th state (1_2_4_8): each SRS resource can be configured with 1 port or 2 ports or 4 ports or 8 ports Note: Any of the above states can be used if 40-7-1g is reported as 2 or 4. |

UE features for 8 Tx codebook and non-codebook-based operation are defined according to the recommendations in section 2.4.2.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	UE features for 8 Tx codebook and non-codebook-based operation are defined according to the recommendations in section 2.4.2.

# References

R1-2001513, Guidelines for UE capability definitions, RAN2, RAN1#100bis, April 2020.

R1-2306381, Further Guidelines on UE capability definitions, RAN2, RAN2#122, Incheon, South Korea, May 2023.

3GPP TR 38.822, v17.1.0, NR; User Equipment (UE) feature list, RAN#100, June 2023.

3GPP TR 38.306, v17.6.0, NR; User Equipment (UE) radio access capabilities, RAN#101, September 2023.

R1-2312705, Corrected RAN1 UE features list for Rel-18 NR after RAN1#115, Moderators (AT&T, NTT DOCOMO, INC.), RAN1#116, Chicago, USA, November 2023.