3GPP TSG RAN WG1 #116                                                     R1-2400536

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	8.12.3

Source:	Xiaomi

Title:	Discussion on UE features for expanded and improved NR positioning

Document for:	Discussion

- Introduction

In RAN1#115 meeting, the UE features for expanded and improved NR positioning were discussed and summarized in R1- 2312572 [1]. In this contribution, we provide our views on UE features based on the current related RAN1 agreements.

- Sidelink positioning

| 41. NR_pos_enh2 | 41-1-2 | Receiving SL-PRS in a shared resource pool | 1. Support SL-PRS in shared resource pool 2. Support receiving SCI format 2D | [15-1, 15-4, 41-1-1] | Yes | No | Receiving SL-PRS in a shared resource pool is not supported | Per band | n/a | n/a | n/a | Need for location server/ UE to know if the feature is supported [UE indicating support of FG 41-1-1 must indicate either this feature group or feature group 41-1-3 is supported or both are supported] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41. NR_pos_enh2 | 41-1-3 | Receiving SL-PRS in a dedicated resource pool | 1. Support SL-PRS in dedicated resource pool 2. Support receiving SCI format 1B | [15-1, 15-4 41-1-1] | Yes | No | Receiving SL-PRS in a dedicated resource pool is not supported | Per band | n/a | n/a | n/a | Need for location server/ UE to know if the feature is supported [UE support of FG 41-1-1 must indicate either this feature group or feature group 41-1-2 is supported or both are supported] | Optional with capability signaling |

FG 15-4 is the UE capability of receiving Sidelink synchronization signal, which shall not be included in the prerequisite FGs of FG 41-1-2 and 41-2-3. In addition, as in RAN1#114bis meeting the following agreement is made, an additional component can be added for FG 41-1-3 on the number of PSCCH a UE can receive in a dedicated resource pool. Moreover, we agree that UE support of FG 41-1-1 must indicate either feature group 41-1-2 or feature group 41-1-3 or or both are supported. Therefore, we think the corresponding brackets can be removed.

Agreement

The maximum number of SL PRS resources that can be (pre)configured in a slot of a dedicated resource pool is 12.

The maximum number of SL PRS resources that can be (pre)configured in a slot of a shared resource pool is 17.

Proposal 2-1: For FG 41-1-2 and 41-2-3, adopt the following revisions

| 41. NR_pos_enh2 | 41-1-2 | Receiving SL-PRS in a shared resource pool | 1. Support SL-PRS in shared resource pool 2. Support receiving SCI format 2D | 15-1, 15-4, 41-1-1 | Yes | No | Receiving SL-PRS in a shared resource pool is not supported | Per band | n/a | n/a | n/a | Need for location server/ UE to know if the feature is supported [UE indicating support of FG 41-1-1 must indicate either this feature group or feature group 41-1-3 is supported or both are supported] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41. NR_pos_enh2 | 41-1-3 | Receiving SL-PRS in a dedicated resource pool | 1. Support SL-PRS in dedicated resource pool 2. Support receiving SCI format 1B 3. UE can receive X PSCCH in a dedicated resource pool in a slot; | 15-1, 15-4 ,41-1-1 | Yes | No | Receiving SL-PRS in a dedicated resource pool is not supported | Per band | n/a | n/a | n/a | Need for location server/ UE to know if the feature is supported [UE support of FG 41-1-1 must indicate either this feature group or feature group 41-1-2 is supported or both are supported] | Optional with capability signaling |

For FG 41-1-4c, we think the “dedicated resource pool” should also be changed to “dedicated SL-PRS resource pool” to align with the description in FG41-1-4b.

Proposal 2-2: For FG 41-1-4c, adopt the following revisions

| 41. NR_pos_enh2 | 41-1-4c | Transmitting SL-PRS mode 2 in a dedicated SL PRS resource pool | 1. UE can transmit SL-PRS and PSCCH within a slot without PSSCH in dedicated resource pool 2. UE can transmit SL-PRS according to the mapping rule between PSCCH and SL-PRS 3. Support transmitting SCI format 1B | [15-[x], 41-1-3], at least one of {41-1-8, 41-1-10} | Yes | No | Transmitting SL-PRS mode 2 in a dedicated resource pool is not supported | Per band | n/a | n/a | n/a | Need for location server/ UE to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- NR DL and UL carrier phase positioning

According to the agreement in R1- 2312572 [1], the following FGs/rows are agreed as baseline for further discussions on carrier phase positioning.

| 41. NR_pos_enh2 | 41-2-4 | UE-based Carrier Phase Positioning | 1. Support of carrier phase measurement for UE-based positioning 2. Support of Assistance data for UE-based Carrier Phase Positioning | FFS | No | N.A. | UE-based Carrier Phase Positioning is not supported | Per band | N.A. | N.A. | N.A. | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

It seems FG 41-2-11 is the reduplicated one of FG 41-2-4. We suggest to delete FG 41-2-11.

Proposal 3-1: delete FG 41-2-11 since it is same as FG 41-2-4.

- Bandwidth aggregation for positioning measurements

According to the agreement in R1- 2312572 [1], the following FGs/rows are agreed as baseline for further discussions on bandwidth aggregation for positioning measurement.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41. NR_pos_enh2 | 41-4-6 | Positioning SRS bandwidth aggregation in RRC_CONNECTED | 1. The number of supported aggregated carriers in intra band contiguous carriers 2. Maximum aggregated UL SRS bandwidth in MHz, which is supported and reported by UE 5. Max number of aggregated SRS resource sets for positioning supported by UE for SRS bandwidth aggregation 6. Maximum number of aggregated SRS resources for bandwidth aggregation 7. Maximum number of aggregated SRS resources for bandwidth aggregation per slot 8. Support the same SRS power reduction across aggregated carriers | 13-8 [, 6-2] | Yes | n/a | Positioning SRS bandwidth aggregation in RRC_CONNECTED is not supported | Per FS | n/a | n/a | n/a | Component 1 candidate values: {2,3,2and3} Component 2 candidate values: For 2 in Component 1: FR1 bands: {80, 100, 160, 200M} FR2 bands: {50, 100, 200, 400, 600, 800} For 3 in Component 1: FR1 bands: {80, 100, 160, 200, 300} FR2 bands: {50, 100, 200, 400, 600, 800, 1000, 1200} Component 5 candidate values: {1, 2, 4, 8, 12, 16} Component 6 candidate values: Periodic: {1,2,4,8,16,32,64} Aperiodic: {0,1,2,4,8,16,32,64} Semi-persistent: {0,1,2,4,8,16,32,64} Component 7 candidate values: Periodic: {1,2,3,4,5,6,8,10,12,14} Aperiodic: {0,1,2,3,4,5,6,8,10,12,14} Semi-persistent: {0,1,2,3,4,5,6,8,10,12,14} Note: The UE supports the simultaneous transmission in a coherent manner of 2 or 3 SRS resources in 2 or 3 intra-band contiguous CCs. Note: each two or three linked SRS resources are counted as 1 resource Note: A UE that support FG [13-8a] must signal a non-zero value for components 6 and 7 for aperiodic Need for location server to know if the feature is supported. UE only reports the number on bands for the current configured CA band combination. | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-4-7 | Positioning SRS bandwidth aggregation independent from UL communication CA in RRC_CONNECTED | 1. The number of supported aggregated carriers in intra band contiguous carriers 2. Maximum aggregated UL SRS bandwidth in MHz, which is supported and reported by UE 5. Max number of aggregated SRS resource sets for positioning supported by UE for SRS bandwidth aggregation 6. Maximum number of aggregated SRS resources for bandwidth aggregation 7. Maximum number of aggregated SRS resources for bandwidth aggregation per slot 8. Support the same SRS power reduction across aggregated carriers 9. Guard period | 6-6, 41-4-6 | Yes | n/a | Positioning SRS bandwidth aggregation independent from UL communication CA in RRC_CONNECTED is not supported | Per FS | n/a | n/a | n/a | Component 1 candidate values: {2,3,2and3} Component 2 candidate values: For 2 in Component 1: FR1 bands: {80, 100, 160, 200M} FR2 bands: {50, 100, 200, 400, 600, 800} For 3 in Component 1: FR1 bands: {80, 100, 160, 200, 300} FR2 bands: {50, 100, 200, 400, 600, 800, 1000, 1200} Component 5 candidate values: {1, 2, 4, 8, 12, 16} Component 6 candidate values: Periodic: {1,2,4,8,16,32,64} Aperiodic: {0,1,2,4,8,16,32,64} Semi-persistent: {0,1,2,4,8,16,32,64} Component 7 candidate values: Periodic: {1,2,3,4,5,6,8,10,12,14} Aperiodic: {0,1,2,3,4,5,6,8,10,12,14} Semi-persistent: {0,1,2,3,4,5,6,8,10,12,14} Note: The UE supports the simultaneous transmission in a coherent manner of 2 or 3 SRS resources in 2 or 3 intra-band contiguous CCs. Note: each two or three linked SRS resources are counted as 1 resource Need for location server to know if the feature is supported. UE only reports the number on bands for the current configured CA band combination. Note: Guard period is needed before and after the aggregated SRS transmissions when SRS resource is configured within a CC without PUSCH/PUCCH is linked for aggregation with an SRS resource configured within an UL active BWP of a UL communication CC | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-4-8 | Positioning SRS bandwidth aggregation in RRC_INACTIVE | 1. The number of supported aggregated carriers in intra band contiguous carriers 2. Maximum aggregated UL SRS bandwidth in MHz, which is supported and reported by UE 5. Max number of aggregated SRS resource sets for positioning supported by UE for SRS bandwidth aggregation 6. Maximum number of aggregated SRS resources for bandwidth aggregation 7. Maximum number of aggregated SRS resources for bandwidth aggregation per slot 8. Support the same SRS power reduction across aggregated carriers | 27-15b | Yes | n/a | Positioning SRS bandwidth aggregation in RRC_INACTIVE is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: {2,3,2and3} Component 2 candidate values: For 2 in Component 1: FR1 bands: {80, 100, 160, 200M} FR2 bands: {50, 100, 200, 400, 600, 800} For 3 in Component 1: FR1 bands: {80, 100, 160, 200, 300} FR2 bands: {50, 100, 200, 400, 600, 800, 1000, 1200} Component 5 candidate values: {1, 2, 4, 8, 12, 16} Component 6 candidate values: Periodic: {1,2,4,8,16,32,64} Semi-persistent: {0,1,2,4,8,16,32,64} Component 7 candidate values: Periodic: {1,2,3,4,5,6,8,10,12,14} Semi-persistent: {0,1,2,3,4,5,6,8,10,12,14} Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-4-9 | Support of Single MAC CE activating SP SRS resource sets for positioning across aggregated carriers. |  | 41-4-6 |  |  |  |  |  |  |  |  |  |

According to the following agreement and WA, single MAC CE activating positioning SRS resource sets across the linked carriers is supported. It is necessary to define a corresponding UE capability.

Agreement(RAN1-112 meeting)

At least support periodic positioning SRS and semi-persistent positioning SRS for bandwidth aggregation

- Support single MAC CE activating positioning SRS resource sets across the linked carriers

- FFS whether support aperiodic positioning SRS for bandwidth aggregation for UEs in RRC_CONNECTED state. Study a single DCI scheduling positioning SRS across the linked carriers, and check whether the conclusion/agreements in agenda of multi-cell PUSCH/PDSCH scheduling with a single DCI can be reused

- FFS MIMO SRS can be supported for bandwidth aggregation, e.g. with UE transparent way

Working assumption(RAN1-113 meeting)

For semi-persistent positioning SRS for bandwidth aggregation, a single MAC CE can activate or deactivate:

- SRS resource set(s) in one or two or three of three aggregated carriers

- SRS resource set(s) in one or two of two aggregated carriers.

Note: the single spatial relation is indicated by the MAC CE for each of two or three aggregated SRS resources.

Send an LS to RAN2 to confirm the feasibility.

Proposal 4-1: define a new UE capability to indicate support single MAC CE activating positioning SRS resource sets across aggregated carrier.

Conclusion

Based on the discussion in the previous sections we propose the following:

Sidelink positioning

Proposal 2-1: For FG 41-1-2 and 41-2-3, adopt the following revisions

| 41. NR_pos_enh2 | 41-1-2 | Receiving SL-PRS in a shared resource pool | 1. Support SL-PRS in shared resource pool 2. Support receiving SCI format 2D | 15-1, 15-4, 41-1-1 | Yes | No | Receiving SL-PRS in a shared resource pool is not supported | Per band | n/a | n/a | n/a | Need for location server/ UE to know if the feature is supported [UE indicating support of FG 41-1-1 must indicate either this feature group or feature group 41-1-3 is supported or both are supported] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41. NR_pos_enh2 | 41-1-3 | Receiving SL-PRS in a dedicated resource pool | 1. Support SL-PRS in dedicated resource pool 2. Support receiving SCI format 1B 3. UE can receive X PSCCH in a dedicated resource pool in a slot; | 15-1, 15-4 ,41-1-1 | Yes | No | Receiving SL-PRS in a dedicated resource pool is not supported | Per band | n/a | n/a | n/a | Need for location server/ UE to know if the feature is supported [UE support of FG 41-1-1 must indicate either this feature group or feature group 41-1-2 is supported or both are supported] | Optional with capability signaling |

Proposal 2-2: For FG 41-1-4c, adopt the following revisions

| 41. NR_pos_enh2 | 41-1-4c | Transmitting SL-PRS mode 2 in a dedicated SL PRS resource pool | 1. UE can transmit SL-PRS and PSCCH within a slot without PSSCH in dedicated resource pool 2. UE can transmit SL-PRS according to the mapping rule between PSCCH and SL-PRS 3. Support transmitting SCI format 1B | [15-[x], 41-1-3], at least one of {41-1-8, 41-1-10} | Yes | No | Transmitting SL-PRS mode 2 in a dedicated resource pool is not supported | Per band | n/a | n/a | n/a | Need for location server/ UE to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Carrier phase positioning

Proposal 3-1: delete FG 41-2-11 since it is same as FG 41-2-4.

| 41. NR_pos_enh2 | 41-2-4 | UE-based Carrier Phase Positioning | 1. Support of carrier phase measurement for UE-based positioning 2. Support of Assistance data for UE-based Carrier Phase Positioning | FFS | No | N.A. | UE-based Carrier Phase Positioning is not supported | Per band | N.A. | N.A. | N.A. | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Bandwidth aggregation for positioning measurements

Proposal 4-1: define a new UE capability to indicate support single MAC CE activating positioning SRS resource sets across aggregated carrier.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41. NR_pos_enh2 | 41-4-6 | Positioning SRS bandwidth aggregation in RRC_CONNECTED | 1. The number of supported aggregated carriers in intra band contiguous carriers 2. Maximum aggregated UL SRS bandwidth in MHz, which is supported and reported by UE 5. Max number of aggregated SRS resource sets for positioning supported by UE for SRS bandwidth aggregation 6. Maximum number of aggregated SRS resources for bandwidth aggregation 7. Maximum number of aggregated SRS resources for bandwidth aggregation per slot 8. Support the same SRS power reduction across aggregated carriers | 13-8 [, 6-2] | Yes | n/a | Positioning SRS bandwidth aggregation in RRC_CONNECTED is not supported | Per FS | n/a | n/a | n/a | Component 1 candidate values: {2,3,2and3} Component 2 candidate values: For 2 in Component 1: FR1 bands: {80, 100, 160, 200M} FR2 bands: {50, 100, 200, 400, 600, 800} For 3 in Component 1: FR1 bands: {80, 100, 160, 200, 300} FR2 bands: {50, 100, 200, 400, 600, 800, 1000, 1200} Component 5 candidate values: {1, 2, 4, 8, 12, 16} Component 6 candidate values: Periodic: {1,2,4,8,16,32,64} Aperiodic: {0,1,2,4,8,16,32,64} Semi-persistent: {0,1,2,4,8,16,32,64} Component 7 candidate values: Periodic: {1,2,3,4,5,6,8,10,12,14} Aperiodic: {0,1,2,3,4,5,6,8,10,12,14} Semi-persistent: {0,1,2,3,4,5,6,8,10,12,14} Note: The UE supports the simultaneous transmission in a coherent manner of 2 or 3 SRS resources in 2 or 3 intra-band contiguous CCs. Note: each two or three linked SRS resources are counted as 1 resource Note: A UE that support FG [13-8a] must signal a non-zero value for components 6 and 7 for aperiodic Need for location server to know if the feature is supported. UE only reports the number on bands for the current configured CA band combination. | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-4-7 | Positioning SRS bandwidth aggregation independent from UL communication CA in RRC_CONNECTED | 1. The number of supported aggregated carriers in intra band contiguous carriers 2. Maximum aggregated UL SRS bandwidth in MHz, which is supported and reported by UE 5. Max number of aggregated SRS resource sets for positioning supported by UE for SRS bandwidth aggregation 6. Maximum number of aggregated SRS resources for bandwidth aggregation 7. Maximum number of aggregated SRS resources for bandwidth aggregation per slot 8. Support the same SRS power reduction across aggregated carriers 9. Guard period | 6-6, 41-4-6 | Yes | n/a | Positioning SRS bandwidth aggregation independent from UL communication CA in RRC_CONNECTED is not supported | Per FS | n/a | n/a | n/a | Component 1 candidate values: {2,3,2and3} Component 2 candidate values: For 2 in Component 1: FR1 bands: {80, 100, 160, 200M} FR2 bands: {50, 100, 200, 400, 600, 800} For 3 in Component 1: FR1 bands: {80, 100, 160, 200, 300} FR2 bands: {50, 100, 200, 400, 600, 800, 1000, 1200} Component 5 candidate values: {1, 2, 4, 8, 12, 16} Component 6 candidate values: Periodic: {1,2,4,8,16,32,64} Aperiodic: {0,1,2,4,8,16,32,64} Semi-persistent: {0,1,2,4,8,16,32,64} Component 7 candidate values: Periodic: {1,2,3,4,5,6,8,10,12,14} Aperiodic: {0,1,2,3,4,5,6,8,10,12,14} Semi-persistent: {0,1,2,3,4,5,6,8,10,12,14} Note: The UE supports the simultaneous transmission in a coherent manner of 2 or 3 SRS resources in 2 or 3 intra-band contiguous CCs. Note: each two or three linked SRS resources are counted as 1 resource Need for location server to know if the feature is supported. UE only reports the number on bands for the current configured CA band combination. Note: Guard period is needed before and after the aggregated SRS transmissions when SRS resource is configured within a CC without PUSCH/PUCCH is linked for aggregation with an SRS resource configured within an UL active BWP of a UL communication CC | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-4-8 | Positioning SRS bandwidth aggregation in RRC_INACTIVE | 1. The number of supported aggregated carriers in intra band contiguous carriers 2. Maximum aggregated UL SRS bandwidth in MHz, which is supported and reported by UE 5. Max number of aggregated SRS resource sets for positioning supported by UE for SRS bandwidth aggregation 6. Maximum number of aggregated SRS resources for bandwidth aggregation 7. Maximum number of aggregated SRS resources for bandwidth aggregation per slot 8. Support the same SRS power reduction across aggregated carriers | 27-15b | Yes | n/a | Positioning SRS bandwidth aggregation in RRC_INACTIVE is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: {2,3,2and3} Component 2 candidate values: For 2 in Component 1: FR1 bands: {80, 100, 160, 200M} FR2 bands: {50, 100, 200, 400, 600, 800} For 3 in Component 1: FR1 bands: {80, 100, 160, 200, 300} FR2 bands: {50, 100, 200, 400, 600, 800, 1000, 1200} Component 5 candidate values: {1, 2, 4, 8, 12, 16} Component 6 candidate values: Periodic: {1,2,4,8,16,32,64} Semi-persistent: {0,1,2,4,8,16,32,64} Component 7 candidate values: Periodic: {1,2,3,4,5,6,8,10,12,14} Semi-persistent: {0,1,2,3,4,5,6,8,10,12,14} Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-4-9 | Support of Single MAC CE activating SP SRS resource sets for positioning across aggregated carriers. |  | 41-4-6 |  |  |  |  |  |  |  |  |  |

References

- R1-2312572 Updated RAN1 UE features list for Rel-18 NR after RAN1 #115, RAN1#115, Nov. 2023