3GPP TSG RAN WG1 #116							    R1-2401663

Athens, Greece, February 26th – March 1st, 2024

Source:	Moderator (MediaTek)

Title:	FL summary 3 for on-demand SIB1 in idle/inactive mode

Agenda item:	9.5.2

Document for:	Discussion and Decision

Introduction

This document is used to assist the discussions for “9.5.2 On-demand SIB1 for idle/inactive mode UEs” study item of the Rel-19 working item on “Enhancements of network energy savings for NR” based on proposals from companies.

This contribution provides discussion points in Section 2, resulted RAN1 conclusion/agreement in Section 3 (TBD), and reference (companies tdoc list) in Section 4.

Discussion points

This section is used to discuss the critical factors that are brought up by multiple companies’ contributions. For each issue, a brief background is provided with reference, and then proposals are bought up to collect company views.

- Issue 1: Cell Scenarios to be discussed for on-demand SIB1

Background

- Most of the companies discussed the cell scenarios to apply on-demand SIB1.

  - Exemplary figure form Nokia and DOCOMO are shown below (while many companies also draw gorgeous figures)

[4, Nokia]:

| Scenario-2-1 | Scenario-2-2 |
| --- | --- |
| Scenario-2-3 | Scenario-2-4 |
| Figure-2 |  |

[24, DOCOMO]:

We first try to summarize companies’ stand in RAN1 #116 contributions.

Assume Cell A to be a cell which normally transmits SIB1, and Cell B to be a cell which requires a UL WUS to trigger on-demand SIB1.

- Alt 1: To trigger the on-demand SIB1 on cell B, UE transmits WUS to Cell B (18 companies)

  - Spreadtrum, Futurewei, Huawei, Nokia, vivo, CMCC, Intel, Google, CATT, ZTE, China Telecom, Samsung, Apple, Ericsson, Sharp, Fujitsu (no inter-cell coordination required), Qualcomm, MediaTek

- Alt 2: To trigger the on-demand SIB1 on cell B, UE transmits WUS to Cell A (7 companies)

  - Futurewei, Nokia, Intel, Google, CATT, Samsung, Apple

Assume Cell A to be a cell which normally transmits SIB1, and Cell B to be a cell which requires a UL WUS to trigger on-demand SIB1.

- Alt 1: For the WUS transmitted by UE to trigger on-demand SIB1 on cell B, UE obtains the WUS configuration from Cell B

  - Futurewei, Nokia, Intel, Google, Samsung, NEC, Sony, DCM, Ericsson, Sharp, Vodafone (11 companies)

- Alt 2: For the WUS transmitted by UE to trigger on-demand SIB1 on cell B, UE obtains the WUS configuration from Cell A (27 companies)

  - Spreadtrum, Futurewei, Huawei, Nokia, vivo, CMCC, Intel, Google, CATT, ZTE, Xiaomi, OPPO, China Telecom, Samsung, NEC, Sony, Panasonic, Apple, FGI, DCM, Ericsson, Sharp, Fujitsu, Vodafone, LG, Qualcomm, MediaTek

[24, DOCOMO]

- FL Proposal 1-1

For the further study of on-demand SIB1 in idle/inactive mode about target cell of UL WUS transmission, RAN1 to prioritize one of the following two alternatives in RAN1 #116b. Assume Cell A to be a cell which normally transmits SIB1, and Cell B to be a cell which requires a UL WUS to trigger on-demand SIB1:

- Alt 1: To trigger the on-demand SIB1 on cell B, UE transmits WUS to Cell B

- Alt 2: To trigger the on-demand SIB1 on cell B, UE transmits WUS to Cell A

| Company | Support or not | Comment |
| --- | --- | --- |
| Moderator |  | To pursue an efficient study, we tend to think we can try to narrow down the supported scenarios in R19 by next RAN1 meeting to facilitate the progress, if possible, and then focus on the remaining design details. Companies are welcomed to elaborate on the pros and cons of Alt1/Alt2 in the comment. |
| Google |  | By prioritizing one of the alternatives, does it mean to support only one of them or both of them but start the work from the high priority one. In our view, both have pros and cons. Alt1 is with lower latency, but Alt2 is good for NES for cell B. |
| Panasonic | Support | When cell A and cell B are same coverage of co-location, alt 2 can maximize NES gain as WUS receiver in cell B can be off. When there are multiple small cell Bs in cell A coverage, alt 1 allows which cell corresponding cell B to be wake-up can be understood by gNB. The similar can be realized if small cell B also receive WUS in cell A frequency. It is assumed Cell B transmits SSB. On the other hand, if cell B does not transmit SSB, to transmit WUS to cell B means no timing synchronization but just to transmit WUS to cell B is uncertain operation. |
| Intel | Comment | The sub bullets should also refer to “UL WUS” and not “WUS” to not be confused with the WUS in the downlink. In our understanding when Cell A is used besides physical layer signalling it is also possible to use higher layer signalling, thus we would like to change Alt 2 to: - Alt 2: To trigger the on-demand SIB1 on cell B, UE transmits UL WUS or higher layer signaling (e.g. MAC CE or RRC) to Cell A |
| CEWiT | Support with comment | Alt-1 is more suitable for UEs in idle mode to acheive low latency. It is also suitable for UEs not in coverage of anchor cell. Thus in a multi cell case with an NES cell (i.e. cell B ) or a single cell scenario where the cell is the NES cell with on demand SIB-1, transmitting WUS to the NES cell (or cell B) should be considered. |
| Spreadtrum | Y | For necessity of on-demand SIB1, we want to emphasize that on-demand SIB1 is similar to on-demand SSB for SCell in CA, but in non-CA, SIB1 transmission should be addressed together with SSB for non-anchor cell. |
| Qualcomm |  | We suggest the following update: For the further study of on-demand SIB1 in for idle/inactive mode UE about target cell of UL WUS transmission, RAN1 to prioritize down-selects one of the following two alternatives in RAN1 #116b. Assume Cell A to be a cell which normally periodically transmits SIB1, and Cell B to be a cell which requires a UL WUS to trigger on-demand SIB1: |
| vivo | Support | Our preference is Alt 1. For Alt. 2, the cons are the following: - Need of coordination between cells, i.e. when UE WUS is detected on Cell A, Cell A needs to tell Cell B that SIB1 should be transmitted - Large latency to acquire SIB1, due to coordination between cells When UE moves to Cell B’s coverage that is not within Cell A’s coverage, there is no possibility to obtain cell B’s SIB1 anymore, which means it can only be applied to a scenario where the whole Cell B is within the whole coverage of Cell A. |
| NEC | Support | Our preference is Alt 1. |
| CATT |  | Of course alt1 is one options, however, for alt2 to be considered, need to define what is cell A, cannot say ‘Assume Cell A to be a cell which normally transmits SIB1’ |
| Fujitsu | Support | Alt 1can achieve lower latency for on-demand SSB triggering while Alt 2 may provide slightly higher NES gain as the RF receiver for UL-WUS monitoring can be turned off. However, for Alt 2, inter-cell coordination is needed which also results in larger latency to obtain SIB1. |
| China Telecom | Support | We prefer Alt1. We agree with vivo and Panasonic that Alt1 can be applied to all the scenarios, while Alt2 can only applied when Cell A and B are co-located. Thus Alt1 should be support at first, and we are also open to discuss Alt2. |
| Apple |  | We are open for both alternatives. For Alt 2, we think the spec impact regarding the WUS configuration and procedure could be small compared to Alt 1, basically, it could largely follow the legacy PRACH procedure for On-demand SIBx on cell A. For Alt 1, if the WUS configuration is also provided by cell B, we think the configuration flexibility would be highly restricted. Regarding the latency to acquire SIB1, we think the coordination between cells could be performed before WUS is sent so that cell A has a copy of SIB1 of cell B in advance, so the latency could be controlled. The pros of Alt 1 would be that for the scenario where UE is moving towards cell B, finds that cell B satisfies cell-reselection criterion, the reliability on requesting SIB1 on cell B would be higher. |
| Futurewei | Support with clarifications | The two alternatives can have different trade-offs and may be used in different use cases. Alt 2 can save more energy for Cell B but may have more spec impact. More clarifications are also helpful here. For example, it should be important to note that for Alt 1, Cell B SSB has to be transmitted periodically in order for the UE to transmit WUS to Cell B, whereas for Alt 2, Cell B SSB can be turned off to save energy. We suggest making this clear. - Alt 1: To trigger the on-demand SIB1 on cell B, UE transmits WUS to Cell B - Cell B SSB is periodically transmitted. - Alt 2: To trigger the on-demand SIB1 on cell B, UE transmits WUS to Cell A - Cell B SSB is not transmitted. |
| Xiaomi | Support | Alt 1 is our preference. Alt 2 increases the implementation complexity and the delay in obtaining SIB1 due to coordination between gNBs as commented by other companies. Besides, if UE has been camped on cell B and it requires SIB1, does the UE has to reselect cell A to transmit WUS for Alt 2? If that is the case, the procedure for WUS triggering is too complicated. |
| Huawei, HiSilicon | Y | Although it is for next meeting, our preference is Alt 1. |
| Nokia/NSB | Support both Alt 1 and Alt 2 | For Alt1, the cell B could manage the on-demand SIB1 operation independently from cell A, where the specification impact on backhaul signalling can be avoided. For Alt2, there can be better energy saving for cell B, i.e. with NW cell reception off, where the NW monitoring of WUS is handled by cell A. On the other hand, some backhaul signalling need to be specified for cell A informing of cell B about WUS reception for on-demanding SIB1. |
| Samsung |  | Alt.1: PRACH can be considered as a design of WUS for UEs in idle/inactive mode, which may not be established with uplink synchronization. Alt.2: Depending on whether having a connection to Cell A, other UL channels, e.g. PUCCH and PUSCH, in addition to PRACH, may be taken into account for WUS. |
| Fraunhofer | Support | We prefer Alt1 or having both possibilities. We agree with vivo on the cons of Alt2. Also noting that Alt2 is applicable only when cells A and B are co-located or the coverage regions of the cells overlap sufficiently, the opportunities for applying this feature would be limited. If Alt2 is considered, a fair performance comparison with Atl1 should be done by properly modelling the potential coverage issues in evaluation. |
| InterDigital | Support | Alt1 can apply for single cell and multi-cell/carrier scenarios and provides good balance in terms of deployment flexibility and access. Alt2 can provide certain NES gains depending on whether Cell B does not transmit only SIB1 or both SSB and SIB1. |
| ZTE, Sanechips | Support | Alt 1 is preferred. For Alt 2, following points need to be clarified: - Coordination between gNBs, - Spec impact of framework of multi-cell for RRC idle/inactive mode. RACH access |
| ETRI | Support | We prefer Alt. 1 because it is a universal solution that can be applicable for all the scenarios discussed on Monday. |
| ASUSTeK | Support | We prefer Alt.1. Alt.1 functions well irrespective of whether there is overlaying cell providing SIB1. Alt.2 only works when there is overlaying cell providing SIB1 and therefore Alt.2 may not be future-proof. |
| Sharp | Support | At least Alt. 1 should be considered since it can be operated with less impact on back-haul signalling with low latency. |
| Vodafone |  | We prefer Alt.1 over Alt.2 |
| Sony | Support | We prefer Alt 1. Alt 2 has concern for necessary of cell coordination and latency to obtain SIB1. |
| DCM | Support | We prefer Alt.1 as it has no/less restriction on NES cell deployment and small latency of on-demand SIB1 transmission. Alt.2 may require that cell A and B within similar coverage or co-located. |

- FL Proposal 1-1-v2

For the further study of on-demand SIB1 in for idle/inactive mode UE about target cell of UL WUS transmission, RAN1 to prioritize study the following two alternatives in RAN1 #116b. Assume Cell A to be a cell which normally periodically transmits SIB1 and provides UL-WUS configuration, and Cell B to be a cell which requires a UL WUS to trigger on-demand SIB1:

- Alt 1: To trigger the on-demand SIB1 on cell B, UE transmits WUS to Cell B

- Alt 2: To trigger the on-demand SIB1 on cell B, UE transmits WUS to Cell A

Companies view during Wed. (2/28) morning offline session:

- Futurewei: No down selection for now, Alt 2 can save more power

- Ericsson: Coverage discussion of cell A and cell B? (Corresponding spec impact?)

- NEC: Isolated cell or other cases?

- LG: Do not down-select for now. Revise the wording

- OPPO: No need for isolated cell case.

- CEWIT: Isolate cell case is needed for discussion. Configuration can be more flexible. Single cell should not be excluded.

- Nokia: Single and multi-cell case should both be considered.

- CMCC: Isolated cell does not work. Configuration is provided from Cell A or B.

- Spreadtrum: Support Alt 1 which can have earlier sync.

- Ericsson: Prefer Alt 1. Information change between gNBs would be more complicated otherwise.

- Xiaomi: Support Alt 1, otherwise large delay for SIB1 reception.  Should work for both isolated and non-isolated cell.

- Vivo: Should consider single cell case. Prefer Alt 1 which can work for single/multi cell.

- FL Proposal 1-1-v3

For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 studies the following options. Assume Cell A to be a cell which periodically transmits SIB1, and Cell B to be a cell which requires a UL WUS to trigger on-demand SIB1.

On target cell of UL WUS transmission:

- Option 1: To trigger the on-demand SIB1 on cell B, UE transmits WUS to Cell B

- Option 2: To trigger the on-demand SIB1 on cell B, UE transmits WUS to Cell A

On configuration provision for UL WUS transmission

- Option 1: UE obtains the WUS configuration from Cell B

- Option 2: UE obtains the WUS configuration from Cell A

- FL Proposal 1-2

For the further study of on-demand SIB1 in idle/inactive mode about configuration provision for UL WUS transmission, RAN1 to prioritize one of the following two alternatives in RAN1 #116b. Assume Cell A to be a cell which normally transmits SIB1, and Cell B to be a cell which requires a UL WUS to trigger on-demand SIB1:

- Alt 1: For the WUS transmitted by UE to trigger on-demand SIB1 on cell B, UE obtains the WUS configuration from Cell B

- Alt 2: For the WUS transmitted by UE to trigger on-demand SIB1 on cell B, UE obtains the WUS configuration from Cell A

| Company | Support or not | Comment |
| --- | --- | --- |
| Moderator |  | To pursue an efficient study, we tend to think we can try to narrow down the supported scenarios in R19 by next RAN1 meeting to facilitate the progress, if possible, and then focus on the remaining design details. Companies are welcomed to elaborate on the pros and cons of Alt1/Alt2 in the comment. |
| Google |  | Similar question as above. Besides, we think the two alternatives are connected to the alternatives in proposal 1-1. |
| Panasonic | Not support | We do not think down selection is needed. Both options can be supported. When cell B is not transmitting SIB1, UE obtains the WUS configuration from cell A. When cell B is already transmitting SIB1 requested by other UEs, UE can obtain SIB1 from cell B. |
| Intel | Support with comment | The sub bullets should also refer to “UL WUS” and not “WUS” to not be confused with the WUS in the downlink.’ We think there is an overlap with proposals connected to the UL WUS configuration. Thus, down selection might not be possible in RAN1#116b. |
| CEWiT | Support with comment | As mention in our previous comment, Alt-1 is more suitable for UEs in idle mode to acheive low latency to access the cell as mentioned by Docomo. It is also suitable for UEs not in coverage of anchor cell to directly interact with the NES cell (cell B). Thus in a multi cell case with an NES cell (i.e. cell B ) or a single cell scenario where the cell is the NES cell with on demand SIB-1, UE obtains the WUS configuration from Cell B or the NES cell should be considered. |
| Spreadtrum | Y |  |
| Qualcomm |  | We suggest the following update. Furthermore, we’ve added Alt 3, which was discussed in our Proposal 7 and illustrated below: For the further study of on-demand SIB1 in for idle/inactive mode UE about configuration provision for UL WUS transmission, RAN1 to study prioritize one of the following two alternatives in RAN1 #116b. Assume Cell A to be a cell which normally periodically transmits SIB1, and Cell B to be a cell which requires a UL WUS to trigger on-demand SIB1: - Alt 1: For the WUS transmitted by UE to trigger on-demand SIB1 on cell B, UE obtains the WUS configuration from Cell B - Alt 2: For the WUS transmitted by UE to trigger on-demand SIB1 on cell B, UE obtains the WUS configuration from Cell A - Alt 3: For the WUS transmitted by UE to trigger on-demand SIB1 on cell B, UE obtains the WUS configuration from another UE. |
| vivo |  | We think both alternatives can exist for on-demand SIB1 and there is no need for down-selection. First, Alt 2 should be the baseline to support on demand SIB1, i.e. when UE camps on Cell A, it can obtain WUS configuration from Cell A when camping in Cell A. Second, Alt 1 could be additionally supported. For example, when UE is already camping on Cell B, WUS configuration from Cell A may become invalid or need to be updated, so the SIB1 triggered WUS can include WUS configuration for itself, i.e. Cell B. |
| NEC |  | We think for multi-cell with an anchor cell case, UE obtains the WUS configuration from Cell A whenever possible; for single-cell case or UE not in coverage of anchor cell, UE can only obtain the WUS configuration from Cell B. |
| CATT |  | Same comment as for the previous one. Definition for cell A need to be updated. |
| Fujitsu | Support | Alt 2 gives a clearer configuration framework which leads to less spec impact. Study under Alt 2 should be prioritized. |
| China Telecom | Support | We prefer Alt 2. If Alt 1 is adopted, the configuration for SIB1 should be required in MIB or existing SIB1. The former will bring large spec impact while the latter is not practical for multi-cell scenario. |
| Apple |  | We think Alt 2 should be prioritized at least. For Alt 1, if the WUS configuration is also from NES cell, only the MIB can be used to convey the WUS configuration which highly limits the configuration flexibility of WUS, especially considering the power control related parameters of the WUS signal. |
| Futurewei | Support with comment | We also think both alternatives can be studied at least for now. Both can work as described in our tdoc: Scenario 1: The cell can be standalone, by transmitting always-on SSB and long-periodicity SIB1/WUS configuration and monitoring the configured WUS transmission opportunities. WUS transmission is based on this cell’s SSB. This cell transmits on-demand SIB1 if it detects WUS. Scenario 2: The cell transmits always-on SSB and transmits on-demand SIB1 if it detects WUS. WUS transmission is based on this cell’s SSB. WUS configuration is transmitted by an assisting cell in the assisting cell’s SIBs. Scenario 3: The cell transmits on-demand SSB / SIB1 if a UE transmits WUS for this cell. WUS configuration is transmitted by an assisting cell in the assisting cell’s SIBs, and WUS transmission is based on the assisting cell’s SSB. WUS is monitored by the assisting cell only or by both cells if both cells are synchronized. If a down-selection is needed to limit the scope of the study, we prefer Alt 2. |
| Xiaomi | Support | We are open to discuss both options. From our understanding, the applicable scenarios for Alt 1and Alt 2 should be discussed first. It seems that Alt 1 works for standalone cell, while Alt 2 works where a neighbouring cell, i.e., cell A can be searched by the UE. |
| Huawei, HiSilicon | Y | Although it is for next meeting, our preference is Alt 2. |
| Nokia/NSB | Support both Alts | Alt1 can be the case for the UEs that originally camp and serve in cell B. Alt2 can be the case for the UEs that is moving from cell A to/towards cell B. |
| Samsung |  | Alt.1: WUS configuration including time/frequency information for UL WUS transmission needs to be pre-configured or fixed. Then, there would be so much spec. impact to set WUS configuration on Cell B. Alt.2: WUS configuration can be delivered by some SIB from Cell A which may be simple compared to Alt.1. But, how for UE to identify Cell A and Cell B can be one of issues. Then, if Rel-19 NES UEs continue to receive SSB and detect SIB1 on each cell, it would cause so much UE burden. |
| Fraunhofer |  | We think both options should be supported. Again, as noted in previous comment, Alt2 would limit the opportunities for enabling the feature due to scenario and coverage issues. We recognize the limitation of Alt1 noted by Apple above and think RAN1 should focus on solving it via specification. Since the WID objective explicitly states, “No modification of SSB will be discussed under this objective”, extensions to SSB/MIB to provision UL cell wake-up signal configuration is ruled out. Other ways, as we noted in our contribution [R1-2400928], can be: - UL cell wake-up signal configurations are predefined, in a way that no provisioning is needed. - When SIB-1 is omitted (for energy saving) the gNB transmits another signal (e.g. a DCI on CORESET#0) which carries the configuration of UL cell wake-up signal. |
| InterDigital |  | We tend to agree with companies that both Alt1 and Alt2 are feasible and there no need to prioritize between them. Nevertheless, Alt2 can be the baseline. |
| ZTE, Sanechips |  | Alt 2 is preferred. SSB has fixed resources and the PBCH has only 1 reserved bit, which makes it difficult to use SSB to carry additional information. Besides, the modification of SSBs has large impact on the initial access, especially for legacy UEs. |
| ETRI | Support | Alt. 2 works when there is an overlaid anchor cell, but cannot cover the single cell scenarios. Therefore Alt. 1 can be considered as baseline. Depending on the estimated specification workload, both Alt. 1 and Alt. 2 may be supported targeting different scenarios. |
| ASUSTeK | Support | Alt.1 is preferred with similar reason we provide for proposal 1-1, i.e. Alt.1 functions well irrespective of whether there is overlaying cell providing SIB1. |
| Sharp |  | Both alternatives can be considered. Alt. 1 works with any cell scenarios though Alt. 2 works well with multiple cell scenario. |
| Vodafone |  | We prefer both options to capture both single and multi-cell scenarios |
| Sony | Support | We think both alternatives could be supported. There are pros and cons for alternatives. Alt 1 could be considered for single cell scenario while it would be concerned for large specification impact. Alt 2 would have less specification impact while single cell scenario would be ruled out. |
| DCM | Not support | It is too early to take a down-selection between two alternatives. More investigation is needed on both Alt.1 and Alt.2. |

- FL Proposal 1-2-v2

For the further study of on-demand SIB1 in for idle/inactive mode UE about configuration provision for UL WUS transmission, RAN1 to study prioritize one of the following two alternatives in RAN1 #116b. Assume Cell A to be a cell which normally periodically transmits SIB1, and Cell B to be a cell which requires a UL WUS to trigger on-demand SIB1:

- Alt 1: For the WUS transmitted by UE to trigger on-demand SIB1 on cell B, UE obtains the WUS configuration from Cell B

- Alt 2: For the WUS transmitted by UE to trigger on-demand SIB1 on cell B, UE obtains the WUS configuration from Cell A

- Alt 3: For the WUS transmitted by UE to trigger on-demand SIB1 on cell B, UE obtains the WUS configuration from another UE.

Companies view during Wed. (2/28) morning offline session:

- Issue 2: UL WUS signal structure

Background

- For selecting a starting point to design the UL WUS, companies have the following proposed candidates and design considerations:

  - PRACH (Msg 1/preamble): Spreadtrum, Huawei, Nokia, Transsion, vivo, CMCC, Google, CATT, Xiaomi, China Telecom, InterDigital, Sony, Fraunhofer, Lenovo, DOCOMO, Ericsson, Sharp, ASUSTeK, Fujitsu, ETRI, CEWiT, LG, Qualcomm, MediaTek (24 companies)

  - Msg 3: Lenovo, Qualcomm (2 companies)

  - PUSCH: Xiaomi (1 company)

[1, Spreadtrum]

- Proposal 5: A segment for all preambles can be used for UL WUS for on-demand SIB1.

[7, CMCC]:

- Observation 1: If PRACH or preambles are determined as the wake-up signal, the UE has to send PRACH twice for the uplink transmission.

[9, Google]

- Proposal 2: Support the UL-WUS based on a PRACH and the NW to configure the followings for the UL-WUS

  - Time-domain resources for each RO

  - Frequency-domain resource for each RO based on the frequency domain resource for the SSB

  - Power control parameters, i.e., P0, SSB transmission power, power ramping step size

  - Each RO is one-to-one associated with each SSB in the target cell

[24, DCM]:

- Proposal 6: Study whether the wake-up procedure can be part of random-access procedure to reduce the initial access delay.

  - Study in which condition that wake-up procedure can be part of random-access procedure.

[26, Sharp]:

- Proposal 5: RAN 1 discuss to select whether Alt. 1 or Alt. 2 for on-demand SIB1 procedure:

  - Alt. 1: While SIB1 is being transmitted within on-demand SIB1 procedure, UE and gNB perform specific initial access procedure for Rel-19 NES UE.

  - Alt. 2:  While SIB1 is being transmitted within on-demand SIB1 procedure, UE and gNB perform

  - Rel-18 initial access procedure.

[34, Qualcomm]:

- Proposal 4: As a starting point, RAN1 follows the on-demand OSI procedure principles for on-demand SIB1 procedure (as shown in Figure 3). RAN1 to further discuss the following aspects:

  - Whether to follow MSG1-based and/or MSG3-based SI request principle

  - Whether or not to support an explicit response to the SIB1 request, prior to sending on-demand SIB1

  - Whether  the  resources  and  configurations  of  the  DL/UL  signals  associated  with  on-demand  SIB1 procedure can be at least partially shared with the common RACH procedure.

- FL Proposal 2-1

For the further study of on-demand SIB1 in idle/inactive mode, RAN1 considered PRACH as a starting point for the design of the UL WUS.

- FFS: To serve as UL WUS, which PRACH parameters should be indicated to UE

- FFS: RACH occasion applicable to UL WUS

- FFS: A subset of legacy RACH configurations is applicable to UL WUS

- FFS: Power control of UL WUS

- FFS: Whether the initial access procedure can be simplified with the PRACH-based UL WUS

| Company | Support or not | Comment |
| --- | --- | --- |
| Moderator |  | There is unanimous support to at least include PRACH as the UL WUS, so I think we can try this proposal. Some companies already mentioned some design considerations based on PRACH, where I put them in the FFS part. Companies are welcomed to add more FFS points in the comment for further design considerations. |
| Google | Support |  |
| Panasonic | Support |  |
| Intel | Support |  |
| CEWiT | Support | Need clarification on “FFS: A subset of legacy RACH configurations is applicable to UL WUS” whether the legacy RACH configuration means the PRACHconfigurationIndex to indicate the RACH preamble and RACH occasions? |
| Spreadtrum | Y | PRACH can be also used for initial UL sync |
| Qualcomm |  | We don’t think down-selection should be done in this meeting. Before going into this detailed discussion, we should discuss under which scenarios should UE request on-demand SIB1 and make related agreements. - Scenario 1: UE requests SIB1 to camp and perform idle mode operation (e.g., cell resection, paging) on the cell with on-demand SIB1 (Cell B in the earlier proposals). - Scenario 2: UE requests SIB1 to perform RRC connection setup (for transition from idle/inactive state to connected state) and to perform RRC connected mode operation in Cell B. If Scenario 1 is supported, using PRACH as UL WUS looks fine. However, if scenario 2 is supported, using PRACH as UL WUS shall require duplication of PRACH transmission (one for SIB1 request and the other for RRC connection setup), which is not efficient for both UE and NW operation as well as access latency. Hence, we suggest the following proposals: Revised proposal For the further study of on-demand SIB1 for idle/inactive UE, - RAN1 to further study the following scenarios: - Scenario 1: UE requests SIB1 to camp and perform idle mode operation (e.g., cell resection, paging) on the cell with on-demand SIB1 (Cell B in the earlier proposals). - Scenario 2: UE requests SIB1 to perform RRC connection setup (for transition from idle/inactive state to connected state) and to perform RRC connected mode operation in Cell B. - RAN1 to further study the following signals for UL WUS - Msg1 - Msg3 - MsgA |
| vivo | Support in general | For the 3rd sub-bullet “FFS: A subset of legacy RACH configurations is applicable to UL WUS”, we can’t understand the intention here since it is already covered by the second sub-bullet. We suggest to remove it. |
| NEC | Support | For the 3rd sub-bullet “FFS: A subset of legacy RACH configurations is applicable to UL WUS”, we prefer to specify new RACH configurations for UL WUS. We suggest to remove it. |
| CATT | OK but remove all the FFS | Otherwise we need to discuss the wording for these FFS, which is meaning less and ineffective use of time |
| Fujitsu | Support |  |
| China Telecom | Support |  |
| Apple | Support in general | We think only the first sub-bullet and the last bullet need to be kept, the other bullets are all details of the first bullet. |
| Futurewei | Support the main bullet | We support the main bullet; but the FFS points are too general and not mature enough to discuss at this stage. They also heavily depend on the outcomes of Issue 1 discussions. For example, which cell’s SSB, Cell A’s or Cell B’s, is to be used for RACH’s PL RS and timing reference? We could replace all the FFS with a single FFS Details |
| Xiaomi | Support | Fine to follow the guideline by the Proposal. And if preamble is adopted for WUS design, existing on-demand SI scheme can be a baseline. For the 3rd sub-bullet, we share the similar view with vivo and NEC to remove it. For the Last sub-bullet, a clarification would be appreciated. Does it mean to introduce a specific initial access procedure for Rel-19 NES UE? Finally, we suggest a minor modification for Proposal 2-1: - FL Proposal 2-1 For the further study of on-demand SIB1 in idle/inactive mode, RAN1 considered PRACH as a starting point for the design of the UL WUS. - FFS: To serve as UL WUS, which PRACH parameters should be indicated to UE - FFS: RACH occasion applicable to UL WUS - FFS: A subset of legacy RACH configurations is applicable to UL WUS - FFS: Power control of UL WUS - FFS: Whether the initial access procedure can be simplified with the PRACH-based UL WUS - Note: Existing on-demand SI scheme can be a baseline for the design of UL WUS. |
| Huawei, HiSilicon | Y | And we think RACH is the only possible option given WID requirement of “using an existing signal/channel” for idle mode UE. |
| Nokia/NSB | OK | Generally, we are OK, and the details can be FFS. |
| Samsung |  | Fine to consider PRACH as a starting point. |
| Fraunhofer | Support | Agree with Apple to limit the FFS bullets. |
| InterDigital | Support |  |
| ETRI | Support |  |
| ASUSTeK | Support |  |
| Sharp | Support |  |
| Vodafone | Support |  |
| Sony | Support |  |
| DCM | Support |  |

- FL Proposal 2-1-v2

For the further study of on-demand SIB1 in idle/inactive mode, RAN1 considered at least PRACH (Msg 1) as a starting point for the design of the UL WUS.

- FFS: Design details of using PRACH as UL WUS

- FFS: Msg 3 or Msg A as UL WUS

For the further study of on-demand SIB1 for idle/inactive UE,

- RAN1 to further study the following scenarios:

  - Scenario 1: UE requests SIB1 to camp and perform idle mode operation (e.g., cell resection, paging) on the cell with on-demand SIB1 (Cell B in the earlier proposals).

  - Scenario 2: UE requests SIB1 to perform RRC connection setup (for transition from idle/inactive state to connected state) and to perform RRC connected mode operation in Cell B.

- RAN1 to further study the following signals for UL WUS

  - Msg1

  - Msg3

  - MsgA

Companies view during Wed. (2/28) morning offline session:

- Huawei: Reuse legacy on-demand SIBx procedure as more as possible.

- QC: Scenarios that Msg 1 or Msg 3 is needed should be considered first.

- Samsung: Care about the whole picture of PRACH configuration and transmission, including procedure.

- NEC: Motivation of Msg 3 or Msg A?

- Lenovo: Msg 1 is more useful than Msg 3.

- QC: Not agree with Lenovo, UE can choose the desired cell to camp (so Msg 3 is useful).

- CEWIT: Msg 3 can not be helpful.

- Ericsson: What waveform would UE transmit (PRACH most likely)?. Procedure can be discussed in another topic.

- Apple: Msg 1 or Msg 3 would be depending on the cell UE is camping on. Msg 1 and Msg 3 should both be considered.

- LG: We can put Msg 1 as the baseline and further discuss about Msg 3 later. We may need help from RAN2 on the Msg 3 part.

- Xiaomi: There should be no change on the RACH procedure.

- FL Proposal 2-1-v3

For the further study of on-demand SIB1 in idle/inactive mode, RAN1 consider Msg 1 or Msg 3 or Msg A as a starting point for the design of the UL WUS.

- FFS: Design details of using Msg1/Msg3/MsgA as UL WUS

- FL Proposal 2-1-v3-1

For the further study of on-demand SIB1 for idle/inactive UE,

- RAN1 to further study the following scenarios:

  - Scenario 1: UE requests SIB1 to camp and perform idle mode operation (e.g., cell resection, paging) on the cell with on-demand SIB1 (Cell B in the earlier proposals).

  - Scenario 2: UE requests SIB1 to perform RRC connection setup (for transition from idle/inactive state to connected state) and to perform RRC connected mode operation in Cell B.

- RAN1 to further study the following signals for UL WUS

  - Msg1

  - Msg3

  - MsgA

- FL Proposal 2-1-v4

For the further study of on-demand SIB1 for idle/inactive UE,

- RAN1 to consider waveform of PRACH as a starting point for the design of the UL WUS.

- RAN1 to further study the following scenarios for the design of the UL WUS

  - Scenario 1: UE requests SIB1 to camp and perform idle mode operation (e.g., cell resection, paging) on the cell with on-demand SIB1 (Cell B in the earlier proposals).

  - Scenario 2: UE requests SIB1 to perform RRC connection setup (for transition from idle/inactive state to connected state) and to perform RRC connected mode operation in Cell B.

| Company | Support or not | Comment |
| --- | --- | --- |
| Moderator |  | There is based on latest off-line/on-line discussion to consider PRACH as a starting point (mentioned by Nokia) and list the 2 scenarios that UE would request SIB1 (mentioned by Qualcomm/Samsung), when designing the waveform of UL WUS. |
| LG Electronics | Support |  |
| Futurewei | Support |  |
| Qualcomm |  | We believe it is too early to make decision on waveform for UL WUS while the procedure is ambiguous/unclear. We’d like to understand OD-SIB1 procedure first before discussing how to reuse the existing signals/channels for UL WUS. On-demand OSI procedure was specified in R15. In addition, in R17, on-demand PRS was introduced by following design of on-demand OSI procedure. Hence, to facilitate discussion, we suggest considering on-demand OSI procedure as a starting point for discussing design of on-demand SIB1 procedure. The scenarios under discussion are not only for UL WUS design consideration but also for on-demand SIB1 procedure design. We suggest the following revision: - Revised FL Proposal 2-1-v4 For the further study of on-demand SIB1 for idle/inactive UE, - RAN1 to consider on-demand OSI procedure waveform of PRACH as a starting point for the design of the UL WUS on-demand SIB1 procedure. - Whether to support MSG1-based and/or MSG3-based SIB1 request - RAN1 to further study the following scenarios for the design of the UL WUS on-demand SIB1 procedure - Scenario 1: UE requests SIB1 to camp and perform idle mode operation (e.g., cell resection, paging) on NES cell the cell with on-demand SIB1 (Cell B in the earlier proposals). Scenario 2: UE requests SIB1 to perform RRC connection setup (for transition from idle/inactive state to connected state) and to perform RRC connected mode operation in NES cell Cell B. |
| Xiaomi |  | We share similar view with QC that on-demand SI should be a starting point for WUS design. Besides, Msg1 based is our preference. If adopting Msg3, the overhead for WUS configuration and WUS transmission is an issue. Besides, the delay for WUS requesting also increases. |
| CEWiT | Support | The proposal seems fine to us. Any change regarding waveform will restrict the technique to one solution which is not feasible at this point. The details of procedure can be discussed later. |
| Vodafone | Support | Since we’re discussing the waveform and not the overall procedure, in our view it is OK to use PRACH as a starting point with Msg1 being our preference. |
| ETRI | Support | We agree that both the two scenarios need to be further studied. For multi-cell scenarios where there is an anchor cell coverage, scenario 1 may be less motivated than scenario 2 because idle UEs can just camp on the anchor cell. For single cell scenarios (if considered to be feasible), both scenarios would be valid cases. |
| DCM2 | Support |  |
| LG Electronics2 |  | After reviewing the proposal again, we are not sure that scenarios 1 and 2 in the second bullet are in scope for further study in RAN1. We are fine with the first bullet. |
| Sharp | Support | Before getting SIB1, a UE have to send a UL signal firstly and PRACH should be the baseline for the first signal. The detailed procedure can be further discussed. |
| Ericsson | Support | First bullet – Support |
| Lenovo |  | We share similar view with QC and others to use on-demand OSI as a starting point for on-demand SIB1. Msg1 based solution can be baseline and Msg3 based solution to be FFS. |
| Fraunhofer | Support |  |
| vivo | Support | We support the first bullet. |

- FL Proposal 2-1-v5

For the further study of on-demand SIB1 for idle/inactive UE,

- RAN1 to consider on-demand OSI procedure waveform of PRACH as a starting point for the design of the UL WUS on-demand SIB1 procedure.

  - Whether to support MSG1-based and/or MSG3-based SIB1 request

Preferred by QC, Apple, and Xiaomi

RAN1 to consider waveform of PRACH as a starting point for the design of the UL WUS.

Preferred by 12 companies

RAN1 to consider the existing RACH procedure as a starting point for the design of the on-demand SIB1 procedure

Preferred by QC, Xiaomi, Panasonic

- FL Proposal 2-1-v6

- For study of UL WUS design, consider at least waveform of PRACH as a starting point

- For study of UL WUS design on-demand SIB1 procedure, consider RACH procedure as a starting point

- Other option(s) not precluded

- FL Proposal 2-2-v1

- RAN1 to further study the following scenarios for the design of the UL WUS on-demand SIB1 procedure

  - Scenario 1: UE requests SIB1 to camp and perform idle mode operation (e.g., cell resection, paging) on NES cell.

  - Scenario 2: UE requests SIB1 to perform RRC connection setup (for transition from idle/inactive state to connected state) and to perform RRC connected mode operation in NES cell.

- Issue 3: Achievable NES gain with on-demand SIB1 and baseline system settings

Background

Multiple companies discussed the achievable gain of on-demand SIB1 although this was studied during R18 SI phase:

- [3, Huawei]: 5.9%/11.2%/20.2%/33.6% NES gain with 160/80/40/20 ms SIB1 periodicity

- [6, vivo]: 12.1%~45.2% NES gain

- [10, CATT]: The NES gain heavily depends on the ratio of zero system load cell (6.9%~18.5% NES gain)

- [11, ZTE]: With 160ms SIB1 periodicity, little NES gain is achieved with on-demand SIB1 (0.39%~5.63%)

- [12, Xiaomi]: The overhead of SIB1 is 2.85%~34.28%

- [15, Samsung]: NES gain of on-demand SIB1 may be limited because of periodic SSB transmission and UL WUS monitoring

- [20, Fraunhofer]: 13.4%~42.3% NES gain

- [21, Apple]: 5.9% to  53.8% NES gain  with  20ms  SIB1 periodicity, <4% for 160ms SIB1 periodicity

- [25, Ericsson]: With 160ms SIB1 periodicity, little NES gain (<2%)

- [32, MediaTek]:

  - For FR1, the NES gain is evident (>10%) for empty/light load, 4/8 beams, and 20/40ms SIB1 periodicity.

  - For FR2, the NES gain is small (<3%) (SSB/SIB1 multiplexing pattern 3) (quoting results from RP-233379)

[32, MediaTek]

| Low load (RU=9.83%) Video traffic with DRX (160, 8, 100) | SIB1 period = 20ms Cat 1 BS | SIB1 period = 40ms Cat 1 BS | SIB1 period = 80ms Cat 1 BS | SIB1 period = 160ms Cat 1 BS | SIB1 on-demand (no SIB1) Cat 1 BS |
| --- | --- | --- | --- | --- | --- |
| 8 Tx beams |  |  |  |  |  |
| BS Power (/ms) | 81.49 | 69.00 | 62.49 | 59.14 | 55.91 |
| BS NES gain | 31.39% | 18.97% | 10.53% | 5.46% | N/A |
| 4 Tx beams |  |  |  |  |  |
| BS Power (/ms) | 68.03 | 61.25 | 57.74 | 55.91 | 54.19 |
| BS NES gain | 20.34% | 11.53% | 6.15% | 3.08% | N/A |
| 1 Tx beam |  |  |  |  |  |
| BS Power (/ms) | 58.74 | 56.09 | 54.72 | 54.00 | 53.34 |
| BS NES gain | 9.19% | 4.90% | 2.52% | 1.22% | N/A |

Observing from the values reported from companies, let’s try the following conclusion. Or we may need to discuss a baseline system setting for further NES gain evaluation.

- Conslusion 3-1

Regarding achievable NES gain of on-demand SIB1 in idle/inactive mode,

- For FR1, the NES gain is evident (>10%) for empty/light load, 4/8 beams, and 20/40ms SIB1 periodicity.

- For FR1, the NES gain is small (<6%) for 160ms SIB1 periodicity.

- For FR2, the NES gain is small (<3%).

| Company | Support or not | Comment |
| --- | --- | --- |
| Moderator |  | It seems the NES gain is larger under some system settings and smaller under some other settings. Hence, I try to capture some critical settings in this conclusion and record some gain values. Companies are welcomed to suggest better wording or formulation to capture the current NES gain results reported by companies. |
| Intel | Comment | There is at the moment no consistent evaluation assumptions across the simulations of different companies. Thus, before we make such a conclusion, we would prefer to define a limited set of evaluation scenarios and afterwards decide a conclusion like this based on the aggregated results from multiple companies. |
| CEWiT | Not support | The conclusion is not clear about the significant gains provided by companies such as 45.2% by Vivo or 53.8 % by Apple. Hence the proposal should indicate the maximum gain possible. Indicating the maximum gain provided will help to identify the scenarios to consider for the study for e.g. on demand SSB in FR1. Following update can be considered. - Conslusion 3-1 Regarding achievable NES gain of on-demand SIB1 in idle/inactive mode, - For FR1, a maximum NES gain of 53.8% is acheived. - For FR2, the NES gain is small (<3%). |
| Spreadtrum | N | It can be postponed. In this meeting, we can set evaluation assumptions. In our view, SSB adaption should be jointly considered for on-demand SIB1, which is more realistic. For a Cell without SIB1, initial access may not be enabled. In other words, the cell is barred. Thus, before on-demand SIB1 is effective, SSB may be only used for neighbouring cell measurement. |
| Qualcomm |  | We should capture assumption on PRACH configuration for WUS monitoring which leads to the NES. In particular, 10ms PRACH configuration periodicity shall have very different NES than 40ms PRACH configuration periodicity. We should leave this discussion to the next meeting. In this meeting, if needed, we can discuss which scenarios we should further evaluate e.g., - SSB periodicity - # of SSBs - SIB1 transmission periodicity - PRACH configuration periodicity for WUS monitoring |
| vivo |  | We support the first sub-bullet. For the second sub-bullet, we don’t agree with the observation. First, in our updated Tdoc R1-2401565, we also provided evaluation results for 160ms SIB1 period, which provide >10% NES gain. Second, 160ms SIB1 period is not a typical case in real deployment. Third, for on-demand SIB1 case, 160ms SSB period should be assumed. If on demand SIB1 is used by a cell, the cell can’t accept initial access UEs anymore so that there is no need to maintain 20ms SSB period. So if evaluating on-demand SIB1 enhance case, 160ms SSB and no SIB1 should be used. For the third sub-bullet for FR2, it is not reasonable to draw a conclusion from a single result. If discussing system setting for further evaluation, we think both baseline case based on legacy and enhanced case with on demand SIB1 should be defined. For baseline cases, two cases can be considered: Case 1 in a typical deployment: 20ms SSB+20ms SIB1; Case 2 that can achieve the most NES gain based on legacy: 160ms SSB+160ms SIB1 For enhanced case, as discussed above, it should be 160ms+no SIB1 since there is no need to serve initial access UEs so that SSB doesn’t need to be 20ms anymore. Finally, NES gain should be evaluated between baseline case and enhanced case. |
| CATT |  | Not sure we can draw conclusion at this time. All of the simulation are per companies and have not been agreed by RAN1. |
| Fujitsu |  | We share the same view as Intel that there should be a certain level of alignment in the evaluation assumptions before we draw a conclusion. |
| China Telecom |  | Agree with Intel/CATT/Fujitsu. |
| Apple |  | Agree with the other proponents that discussion on evaluation assumption on baseline and enhanced schemes needed first. |
| Futurewei |  | Agree with other companies that the assumptions should be first clarified and aligned. |
| Xiaomi |  | We share similar view with vivo that the baseline should be determined first. For the NES gain for FR2, we observe that only 1 company provides the evaluation result. It is too early to draw the conclusion. |
| Huawei, HiSilicon |  | Typo in “Conslusion”. We understand the intention while consider the conclusion/TR from R18 SI is already clear. We are also ok to confirm the observation if needed and add new observations for more information. The point here perhaps is whether new simulations are needed in order to provide additional insights on the potential of on -demand SIB1. - In our view, new simulation is not needed. And we echo OPPO comments that the main focus is to provide study on “procedures and signalling”, as the main reason that this objective is “study” rather than “specify” is due to the potential spec impact, as captured in RAN plenary note. If new simulations are to be provided, then need to discuss what new assumptions are needed in addition to R18 SI. |
| Samsung |  | In general, we think it is premature to make a conclusion about the NES gain before discussing potential scenarios and assumptions for on-demand SIB1 operation. For the detail, our understanding is that 160ms SIB1 periodicity supported in current spec. can provide the best NES gain without any enhancement for on-demand SIB1. So, it should be clear 160ms SIB1 periodicity should be baseline in order to check how much NES gain can be achieved with on-demand SIB1. |
| Fraunhofer |  | We share the above view that a unified evaluation assumption is needed for fair comparison and for making such conclusions. |
| ZTE, Sanechips |  | As many companies comment, scenarios should be discussed firstly. |
| ETRI |  | The formulation with the three cases looks good but it seems premature to conclude on the numbers about the gains. Maybe no need to make such a conclusion since it was already done in the Rel-18 SI. |
| DCM |  | The assumption of SSB periodicity and SSB number have large impact on the gain of on-demand SIB1. Before draw the conclusion, these assumptions needs to be discussed and aligned. |

- Define a limited set of targeted evaluation scenarios for companies to report: Intel, Spreadtrum (joint adaptation with SSB), QC, vivo, CATT, Fujitsu, China Telecom, Apple, Futurewei, xiaomi, Samsung (160ms SIB1 periodicity should be baseline), Fraunhofer, ZTE, DCM

- Capture the largest gain: CEWiT,

- No need for new gain evaluation: Huawei, ETRI

System settings to be applied for NES gain evaluation:

- SSB adaptation: Spreadtrum

- SSB periodicity: QC, DCM

- # of SSBs in a SSB burst: QC, MTK, DCM

- SIB1 transmission periodicity: QC, MTK, vivo (same SSB and SIB1 periodicity as baseline, no SIB1 as enhanced), Samsung (160ms SIB1 periodicity as baseline)

- PRACH configuration periodicity for UL WUS monitoring: QC

- FL Proposal 3-1

For further study of achievable NES gain of on-demand SIB1 for idle/inactive mode UE, adopt the following assumptions in FR1 for baseline network energy calculation:

- 30kHz SCS, DDDSU TDD pattern

- Case A1: 20ms SSB period with 20ms SIB1 period; Case B1: 160ms SSB period with 160ms SIB1 period

- 4 or 8 SSBs in a SSB burst

- [20ms] PRACH monitoring period

For enhanced network energy calculation with on-demand SIB1, adopt the following assumptions in FR1:

- 30kHz SCS, DDDSU TDD pattern

- Case A2: 20ms SSB period with no SIB1 transmitted; Case B2: 160ms SSB period with no SIB1 transmitted

  - Note: Case A2 is compared with Case A1; Case B2 is compared with Case B1

- 4 or 8 SSBs in a SSB burst

- [20ms] UL WUS monitoring period

| Company | Support or not | Comment |
| --- | --- | --- |
| Moderator |  | It seems majority of companies want to define unified evaluation assumption for NES gain calculation. So this proposal is brought up combining the inputs from companies. To Spreadtrum: Joint SSB/SIB1 adaptation was discussed in RAN #102 but not included in the scope, so I did not include it in the list. |
| Futurewei |  | The proposal seems to be limited to cases where SSB is always transmitted periodically by the NES cell. However, as in the figure provided for Issue 1 (by Nokia), in Scenario-2-1 and Scenario-2-2, SSB seems not needed. So whether the periodic SSB transmission is needed should be discussed. Therefore, we suggest to add: - Case C: no periodic SSB or SIB1 transmitted - Note: Case C is compared with both Case A1 and Case B1 If companies think periodic SSB should always be transmitted instead, the group should confirm this. |
| Qualcomm |  | We suggest also adding analysis assumption for FR2: For further study of achievable NES gain of on-demand SIB1 for idle/inactive mode UE, adopt the following assumptions in FR2 for baseline network energy calculation: - 120kHz SCS, DDDSU TDD pattern - 20ms SSB period with 20ms SIB1 period - 32 or 64 SSBs in a SSB burst - [20ms] PRACH monitoring period For enhanced network energy calculation with on-demand SIB1, adopt the following assumptions in FR2: - 120kHz SCS, DDDSU TDD pattern - 20ms SSB period with no SIB1 transmitted; - 32 or 64 SSBs in a SSB burst - [20ms] UL WUS monitoring period Note: SSB/RMSI multiplexing pattern 1 is used |
| Vodafone |  | Suggest to include assumptions on the load for the further study. |
| Ericsson |  | There should be a case that reflects what can be achieved already with Release 18 functionality. Therefore we think that case B1 should be modified: Case B1: 160ms 20ms SSB period with 160ms SIB1 period, we want to have The reason is that 160ms SSB period cannot be used for idle/inactive users and the intended scenarios (cell reselection, RRC connection setup) with Release 18. |
| Fraunhofer | Support |  |
| vivo |  | We generally support the direction. Regarding enhanced case, we prefer to prioritize Case B2. Since if on demand SIB1 is enabled, the cell can’t be used for initial access UEs. In this case, there is no need to transmit 20ms SSB for NES purpose and 160ms SSB is sufficient. So, case A2 is not a reasonable case if on demand SIB1 is enabled. For PRACH and WUS monitoring period, we suggest to add 160ms as well to align with SSB and SIB1 period. |

- FL Proposal 3-1-v2

For further study of achievable NES gain of on-demand SIB1 for idle/inactive mode UE, adopt the following assumptions in FR1 for network energy calculation:

- Empty/Low Cell load as defined in 38.864

- 30kHz SCS, DDDSU TDD pattern

- Possible configuration for non-NES cell:

  - Case A1: 20ms SSB period with 20ms SIB1 period;

  - Case B1: 160ms SSB period with 160ms SIB1 period;

  - Case C1: 20ms SSB period with 160ms SIB1

Note: Other period assumptions are not precluded (up to companies to report)

- 4 or 8 SSBs in a SSB burst with SSB pattern case C

- [20ms/160] PRACH monitoring period

For enhanced network energy calculation with on-demand SIB1, adopt the following assumptions in FR1:

- Empty/Low Cell load as defined in 38.864

- 30kHz SCS, DDDSU TDD pattern

  - Case A2: 20ms SSB period with no SIB1 transmitted;

  - Case B2: 160ms SSB period with no SIB1

  - Case C2: 20ms SSB period with no SIB1 transmitted

  - [Case D: no periodic SSB or SIB1 transmitted]

  - Note: Case A2 is compared with Case A1; Case B2 is compared with Case B1; Case C2 is compared with Case C1

  - Note: Case D is compared with both Case A1 and Case B1

Note: Other period assumptions are not precluded  (up to companies to report)

- 4 or 8 SSBs in a SSB burst with SSB pattern case C

- [20/160] UL WUS monitoring period

- Note: SSB/CORESET0 multiplexing pattern 1 is used

For further study of achievable NES gain of on-demand SIB1 for idle/inactive mode UE, adopt the following assumptions in FR2 for baseline network energy calculation:

- 120kHz SCS, DDDSU TDD pattern

- Case A1: 20ms SSB period with 20ms SIB1 period; Case B1: 160ms/20ms SSB period with 160ms SIB1 period

- 32 or 64 SSBs in a SSB burst

- [20ms] PRACH monitoring period

For enhanced network energy calculation with on-demand SIB1, adopt the following assumptions in FR2:

- 120kHz SCS, DDDSU TDD pattern

- Case A2: 20ms SSB period with no SIB1 transmitted; Case B2: 160ms SSB period with no SIB1 transmitted; Case C: no periodic SSB or SIB1 transmitted

  - Note: Case A2 is compared with Case A1; Case B2 is compared with Case B1

  - Note: Case C is compared with both Case A1 and Case B1

- 32 or 64 SSBs in a SSB burst

- [20ms] UL WUS monitoring period

- Note: SSB/CORESET0 multiplexing pattern 1/2/3 is used

- FL Proposal 3-1-v3

For further study of achievable NES gain with on-demand SIB1 for idle/inactive mode UE,

- Assume the following for network energy calculation of non-NES cell in FR1:

- Empty/low/medium cell load as defined in 38.864

- Cat 1/Cat 2 BS as defined in 38.864

- 30kHz SCS, DDDSU TDD pattern

  - Case A: 20ms SSB period with 20ms SIB1 period;

  - Case B: 160ms SSB period with 160ms SIB1 period;

  - Case C: 20ms SSB period with 160ms SIB1 period;

Note: Other SSB/SIB1 periodicity assumptions are not precluded (up to companies to report)

- 4 or 8 SSBs in a SSB burst with SSB pattern case C

- 20ms/160ms PRACH monitoring period

- Assume the following for network energy calculation of NES cell in FR1:

- Empty/low/medium cell load as defined in 38.864

- Cat 1/Cat 2 BS as defined in 38.864

- 30kHz SCS, DDDSU TDD pattern

  - Case 1: 20ms SSB period with no SIB1 transmitted;

  - Case 2: 160ms SSB period with no SIB1 transmitted;

  - [Case 3: no periodic SSB or SIB1 transmitted;]

Note: Other SSB/SIB1 assumptions are not precluded (up to companies to report)

- 4 or 8 SSBs in a SSB burst with SSB pattern case C

- 20ms/160ms UL WUS monitoring period

- Note: SSB/CORESET0 multiplexing pattern 1 is used

- FL Proposal 3-2

For further study of achievable NES gain with on-demand SIB1 for idle/inactive mode UE,

- Assume the following for network energy evaluation of non-NES cell in FR2:

- Empty/low/medium cell load as defined in 38.864

- Cat 1/Cat 2 BS as defined in 38.864

- 120kHz SCS, DDDSU TDD pattern

  - Case A: 20ms SSB period with 20ms SIB1 period;

  - Case C: 20ms SSB period with 160ms SIB1 period;

  - Case D: 20ms SSB period with 40ms SIB1 period;

Note: Other SSB/SIB1 periodicity assumptions are not precluded (up to companies to report)

- 32/64 SSBs in a SSB burst with SSB pattern case D

- 20ms or 160ms PRACH monitoring period

- Assume the following for network energy evaluation of NES cell in FR2:

- Empty/low/medium cell load as defined in 38.864

- Cat 1/Cat 2 BS as defined in 38.864

- 120kHz SCS, DDDSU TDD pattern

  - Case 1: 20ms SSB period with no SIB1 transmitted;

Note: Other SSB/SIB1 assumptions are not precluded (up to companies to report)

- 32/64 SSBs in a SSB burst with SSB pattern case D

- 20ms/160ms UL WUS monitoring period

- Note: SSB/CORESET0 multiplexing pattern 1 or 3 is used

- Issue 4: Supported operation for the NES cell with on-demand SIB1

Background

Which operations (Ex. SSB, paging, RACH receiving, …) should be supported for the on-demand SIB1 NES cell is widely discussed as copied below.

[2, Futurewei]

Proposal 1: For potential enhancements of on-demand SIB1 of a cell for UEs in idle/inactive mode,

consider at least the following two scenarios for the cell’s SSB:

- The cell’s SSB is always on (transmitted periodically regardless of its SIB1 on/off status).

  - This cell’s MIB may be configured to indicate the cell as barred and/or SIB1 absent to avoid backward compatibility issue.

- The cell transmits only on-demand SSB and SIB1, and one of SSB/SIB1 is transmitted only when the other is transmitted.

[6, vivo]

Proposal 2: Clarify the following assumptions in WID for specifying on demand SIB1 by UE WUS

•  WUS configuration is provided to UEs via the SIB1 of its camping cell;

•  WUS signal is transmitted to a corresponding NES cell for triggering its SIB1 transmission;

•  UE can camp on a NES cell to monitor paging and perform RACH.

[12, xiaomi]

Proposal 3:  UE can camp on the on-demand SIB1 cell following current cell selection/reselection procedure.

[25, Ericsson]

Proposal 6:  For  the  study  of  procedures  and  signaling  for  operation  of  a  cell  with  on-demand SIB1 (or NES cell), the assumption on the transmission/reception of the following channels/signals on the NES cell should be clarified.

a.  SSB transmission

b.  WUS configuration transmission (for requesting SIB1)

c.  WUS reception

d.  SIB1 transmission

e.  Initial access/RACH procedure (including RACH periodicity)

f.  Paging transmission

g.  OSI transmission

[33, LG]

The following issues can be discussed to support on-demand SIB1 operation.

- Whether SSB needs to be transmitted on non-anchor cell or not

- Failure handling when SIB1 is not received after UE’s PRACH transmission

- Whether SIB1 for non-anchor cell is permanently transmitted

- How UE performs random access procedure for a non-anchor cell or paging after receiving SIB1 for the non-anchor cell

[34, Qualcomm]

Proposal 7: RAN1 to discuss other methods for providing a UE with the on-demand SIB1 procedure configuration – for the following cases and examples:

•  Idle/inactive UE camping (if allowed) on the NES cell with on-demand SIB1.

•  Idle/inactive UE was connected to the NES cell, whose connection was released/suspended.

•  UE may acquire the information through other means, such as other UEs.

- FL Proposal 4-1

For the study of procedures and signaling for operation of a NES cell with on-demand SIB1, RAN1 to decide the assumption on the transmission/reception of the following channels/signals on the NES cell in RAN1 #116b:

- SSB transmission

- WUS configuration transmission (for requesting SIB1)

- WUS reception

- SIB1 transmission

- Initial access/RACH procedure (including RACH periodicity)

- Paging transmission

- OSI transmission

| Company | Support or not | Comment |
| --- | --- | --- |
| Moderator |  | For the supported operation of the NES cell, companies are welcomed to provide comments and list other aspects which have not been captured above, so RAN1 can make decision in RAN1 #116b. |
| Panasonic | Support |  |
| Intel | Comments | The sub bullets should also refer to “UL WUS” and not “WUS” to not be confused with the WUS in the downlink. In our understanding It is not necessary to consider anything after SIB1 is transmitted in the NES cell as this case is already covered by the already standardized system operation. |
| CEWiT | Support |  |
| Spreadtrum |  | Paging may not be included for discussion, since paging is not tightly coupled with SIB1 compared with SSB. OSI may not be included for discussion, since on-demand OSI is specified already. |
| Qualcomm |  | We don’t support this proposal which is too detailed at this stage. It is important to get high level framework first before going into detailed signaling/design. |
| vivo | Support |  |
| NEC | Support | Paging may be used for the transmission of WUS configuration. |
| CATT |  | Why do we need to decide the assumption for all of the subbullet ? We may not be able to make all the decision by RAN1#116b. |
| Fujitsu | Support |  |
| China Telecom | Support. | We think the motivation of this proposal is just to provide a framework for further discussion. In case we missed something (though we don’t find it currently), we suggest to add a note: ‘if necessary, other assumptions are not precluded’ |
| Apple |  | We think WUS confirmation also needs to be added to the list to make it complete. However, we wonder what is the intention of this proposal and what “assumption” means. Besides SSB transmission, we think the others are procedures that need to be considered for On-demand SIB1. Suggest keep only the first bullet, to keep the discussion focused. The other WUS configuration/transmission related parts can be discussed under Proposal 1. |
| Futurewei |  | This is a long list and potentially significant spec impact. We suggest reducing the list to keep only the most essential items, which will also ensure that the scope is more realistic or postpone the discussion at a later stage. |
| Xiaomi | Support with modification | For the first sub-bullet, ‘No modification of SSB will be discussed under this objective’ has been specified by the WID. There is no need to discuss the SSB transmission on NES cell as the transmission is same with legacy cell. For the second and third sub-bullets, they have been discussed under Proposal 1-1 and Proposal 1-2. Repeated discussion should be avoided. For the last four sub-bullets, we are fine to discuss them. |
| Huawei, HiSilicon | Simplified | If the above bullets are for simulation purpose, we can discuss although we do not prefer. If the above bullets are for discussion of procedures&signalling (as in WID) purpose, we think there could be some focus without performing comprehensive study on all aspects – some of them could be left to WI, if proceeded. Therefore, the main points in our view is, describing from UE perspective: - SSB transmission - WUS configuration reception transmission (for requesting SIB1) - WUS reception transmission - SIB1 transmission - Initial access/RACH procedure (including RACH periodicity) - Paging transmission - OSI transmission |
| Samsung |  | - SSB transmission: Given WID saying “No modification of SSB will be discussed under this objective”, it should be clear that there would be SSB transmission on NES cell. WUS configuration transmission (for requesting SIB1): Different depending on scenario WUS reception: Different depending on scenario SIB1 transmission: Different depending on scenario Initial access/RACH procedure (including RACH periodicity): FFS Paging transmission: FFS OSI transmission: FFS |
| Fraunhofer | Support |  |
| InterDigital | Comments | Similar with Intel, it is not clear to us the need to consider the legacy procedures and transmission of signals/channels (e.g. RACH, OSI) after SIB1 transmission |
| ETRI | Support in general | We are okay with the direction, but not sure about some of the bullets (e.g., paging and OSI) for now. Also, it may not be realistic to make all the decisions until the next meeting. “Further study ~” seems sufficient at this moment. |
| ASUSTeK | Comments | Probably better to start with a shorted list. From our perspective, the first three bullets seem sufficient for now, - SSB transmission - WUS configuration transmission (for requesting SIB1) - WUS reception The other could be added later when proper. And we share similar view as Xiaomi that it’s more natural to assume SSB transmission is available as legacy as per WID (i.e. no need to support both on-demand SSB and on-demand SIB1 on a same cell). |
| Vodafone | Support |  |
| Sony | Support |  |
| DCM | Support |  |

- FL Proposal 4-1-v2

For the study of procedures and signaling for operation of a NES cell with on-demand SIB1, RAN1 to discuss the assumption on the transmission/reception of the following channels/signals on the NES cell:

- SSB transmission

- WUS configuration transmission (for requesting SIB1)

- WUS reception

- SIB1 transmission

- Initial access/RACH procedure (including RACH periodicity)

- Paging transmission

- OSI transmission

Note: If necessary, other assumptions are not precluded

| Company | Support or not | Comment |
| --- | --- | --- |
| Moderator |  | Combining companies input for FL Proposal 4-1-v1 and try to reformulate. |
| LG Electronics |  | We are generally fine with the proposal. In addition, the response of WUS reception as a confirmation and CORESET/SS configuration for SIB1 scheduling are also considered. |
| Futurewei |  | We suggest changing the first bullet to: - Whether SSB is transmitted (No modification of SSB will be discussed) |
| Qualcomm |  | We should discuss to have common understanding on overall procedure first as we suggest in Proposal 2-1-v4 before going into this discussion. |
| Xiaomi | Preclude SIB1 transmission | Based on the agreement newly achieved: a NES cell is a cell that may transmit SIB1 transmission in response to UL WUS from a UE. That has been specified the SIB1 transmission on the NES cell. The motivation for further discussion of SIB1 transmission is not clear to us. |
| CEWiT |  | CORESET/SS configuration for SIB1 scheduling can also be considered. Also we agree with Xiomi that the clarification for including discussion of SIB1 transmission is needed. |
| ETRI | OK |  |
| DCM2 |  | As Proposal 2-1-v4 discussed, PRACH can be a start point for the WUS. As PRACH is highly related to the initial access procedure, we of course will touch it in future discussion. We suggest keeping the “initial access” in the discussion list. For the study of procedures and signaling for operation of a NES cell with on-demand SIB1, RAN1 to discuss the assumption on the transmission/reception of the following channels/signals on the NES cell: - SSB transmission - WUS configuration transmission (for requesting SIB1) - WUS reception - SIB1 transmission - Initial access/RACH procedure (including RACH periodicity) - Paging transmission - OSI transmission Note: If necessary, other assumptions are not precluded |
| Ericsson |  | Agree with Qualcomm, We don’t see that we need this proposal. |
| Fraunhofer |  | Agree with DCM2 and want a further edit as follows, because we think at this point, we should study different possibilities before narrowing to specific assumptions later: For the study of procedures and signaling for operation of a NES cell with on-demand SIB1, RAN1 to discuss the different possibilities and related assumptions on the transmission/reception of the following channels/signals on the NES cell: - SSB transmission - WUS configuration transmission (for requesting SIB1) - WUS reception - SIB1 transmission - Initial access/RACH procedure (including RACH periodicity) - Paging transmission - OSI transmission Note: If necessary, other assumptions are not precluded |

- FL Proposal 4-1-v3

For the study of procedures and signaling for operation of a NES cell with on-demand SIB1, RAN1 to discuss the different possibilities and related assumptions on the transmission/reception of the following channels/signals on the NES cell:

- SSB transmission

- UL WUS configuration and UP WUS transmission from UE (for requesting SIB1)

- UL WUS monitoring at the gNB side

- SIB1 transmission

- Initial access/RACH procedure (including RACH periodicity)

- Paging transmission

- OSI transmission

Note: If necessary, other assumptions are not precluded

- FL Proposal 4-1-v4

For the study of procedures and signaling for UE operation in idle/inactive mode on a NES cell with on-demand SIB1, RAN1 to discuss at least the following aspects:

- Details of the UL WUS configuration

- Details of UL WUS transmission from UE (for requesting SIB1)

- SIB1 reception details and SIB1 PDCCH monitoring from UE side for receiving the requested SIB1

- Details of initial access/RACH procedure

- [DCI 1_0 reception with CRC scrambled by P-RNTI]

- [OSI reception]

- Issue 5: Triggering condition for UE to transmit uplink WUS

Background

The triggering condition for UE to transmit uplink WUS is also discussed among multiple contributions.

[6, vivo]

Proposal 5: Design the on-demand SIB1 by UL WUS based on the following:

•  RAN1 designs WUS signalling and related UE behaviours after WUS transmission.

•  RAN2 designs WUS configuration and WUS triggering conditions for on-demand SIB1.

[10, CATT]

Proposal 4: If on-demand SIB1 is supported, the triggering condition for UL WUS transmission should be

further studied.

[34, Qualcomm]

Proposal 5: RAN1 to discuss conditions/restrictions for sending UL-WUS to the NES cell.

- FL Proposal 5-1

For the further study of on-demand SIB1 in idle/inactive mode, RAN1 to discuss triggering conditions and restrictions for sending UL-WUS to the NES cell, and inform RAN2 of RAN1’s preference in RAN1 #116b.

| Company | Support or not | Comment |
| --- | --- | --- |
| Moderator |  | Looking at companies’ proposals, I tend to think RAN1 can first discuss this topic and then communicate with RAN2 later. |
| Google | Support |  |
| Panasonic | Support |  |
| Intel | comment | We agree to study triggering conditions and restrictions for UL WUS transmissions. It would be nice if we could agree on a terminology as in this document the following three are used: UL WUS, WUS, and UL-WUS. We do not see a motivation to already agree to interact with RAN2 in RAN1#116b, considering that the discussion on this topic has not yet started in RAN2 and RAN2 impact is unclear. |
| Spreadtrum | Y |  |
| Qualcomm |  | We suggest the following update since (1) it was not agreed yet UL WUS is sent to NES cell (although it is our preference); (2) when to inform RAN2 could be decided/discussed later. For the further study of on-demand SIB1 in for idle/inactive mode UE, RAN1 to discuss triggering conditions and restrictions for sending UL-WUS to the NES cell, and inform RAN2 of RAN1’s preference in RAN1 #116b. |
| vivo | Support |  |
| CATT |  | No need for such agreement. We are not sure we can decide ran1’s preference by ran1#116b, and inform ran2. |
| Fujitsu | Support |  |
| China Telecom | Support |  |
| Apple |  | Open to discuss but not sure whether RAN1 could decide on the triggering conditions which is normally RAN2 work. |
| Futurewei |  | We are not sure if this proposal is useful. We agree with the moderator’s input. |
| Xiaomi |  | We share the similar view with QC. |
| Huawei, HiSilicon | Maybe (but positive) | In addition to the above, other aspects with ran1 progress can also be informed to RAN2 whenever needed, e.g. WUS configuration details, impact on RACH procedures if any. Therefore, a general conclusion could be considered in order to facilitate the discussion/progress, e.g. For the further study of on-demand SIB1 in idle/inactive mode, RAN1 to identify the aspects that have RAN2 impact and inform RAN2 in RAN1 #116b. |
| Samsung |  | OK with discuss. Whether the outcome should be sent to RAN2 can be discussed later. |
| InterDigital | Comment | In our view, the question on triggering conditions for UL-WUS can be revisited after progress is made on other earlier proposals |
| ZTE, Sanechips |  | This issue seems belong to RAN2. |
| ETRI | Support | Suggest to remove the last part “and inform RAN2 of RAN1’s preference in RAN1 #116b” which is not clear for now. This proposal seems also related to Issue 7. |
| ASUSTeK | Support |  |
| Sony | Support |  |

- FL Proposal 5-1-v2

For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 to discuss triggering conditions and restrictions for sending UL-WUS.

| Company | Support or not | Comment |
| --- | --- | --- |
| Moderator |  | Combining companies input for FL Proposal 5-1-v1 and try to reformulate. |
| LG Electronics |  | We are open to discuss. Based on the discussion, we can provide input if there is something that needs to be decided by RAN2. |
| Futurewei |  | Open to discuss. |
| Xiaomi |  | Open to discuss it. |
| Vodafone | Support | And report any conclusions/suggestions to RAN2 |
| ETRI | Support |  |
| DCM2 | Support |  |
| Sharp | Support |  |
| Ericsson | Support | Support with a small modification: to discuss triggering conditions and restrictions for sending UL-WUS. |
| Lenovo | Support | This is also related with the scenarios discussed in proposal 2-1-v4. |
| Fraunhofer | Support | And agree with LGE and Vodafone on reporting to RAN2. |

- FL Proposal 5-1-v3

For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 to discuss triggering conditions and restrictions for sending UL-WUS.

- Issue 6: Using which signal/channel to transmit the UL WUS configuration to the UE

Background

About using which signal/channel to transmit the UL WUS configuration to the UE the following company views are collected:

Assume Cell A to be a cell which normally transmits SIB1, and Cell B to be a cell which requires a UL WUS to trigger on-demand SIB1. For the WUS transmitted by UE to trigger on-demand SIB1 on cell B, UE obtains the WUS configuration from:

- Alt 1: SIBx of Cell A

  - Nokia, Xiaomi, MediaTek, Ericsson, DOCOMO

- Alt 2: DCI on Cell A or Cell B

  - NEC (paging/DCI1_0), Fraunhofer (DCI on CORESET#0), ETRI (new DCI on CORESET#0 based on DCI 1_0 and new RNTI)

- Alt 3: Msg 4 or Msg 2

  - NEC, Qualcomm

- Alt 4: PBCH on Cell B

  - DOCOMO, NEC

- Alt 5: Predefined configuration

  - Fraunhofer

- FL Proposal 6-1

For the further study of using which signal/channel to transmit the UL WUS configuration to the UE, RAN1 to down-select from the following alternatives in RAN1 #116b. Assume Cell A to be a cell which normally transmits SIB1, and Cell B to be a cell which requires a UL WUS to trigger on-demand SIB1. For the WUS transmitted by UE to trigger on-demand SIB1 on cell B, UE obtains the WUS configuration from:

- Alt 1: SIBx of Cell A

- Alt 2: DCI on Cell A or Cell B

- Alt 3: Msg 4 or Msg 2

- Alt 4: PBCH on Cell B

- Alt 5: Predefined configuration

| Company | Support or not | Comment |
| --- | --- | --- |
| Moderator |  | This may be related to FL proposal 1-2, but I tend to think they can be discussed in parallel. Companies are welcomed to revise the wording and add more alternatives if not being captured in the list yet. |
| Google | Support |  |
| Panasonic | Support with suggested revision | When think combinations of the alternatives are needed. So suggested to change the main bullet to “…, UE obtains the WUS configuration from one or more from:” |
| Intel | comment | The main text should also refer to “UL WUS” and not “WUS” to not be confused with the WUS in the downlink. As this is a critical part of the UL WUS transmission, we would like to avoid restricting the decision or RAN1#116b. In our understand this topic is connected to the way that a UE detects that the cell is operating with on-demand SIB1 (see proposal 7-1). We would like to first discuss this before discussing where to get the UL WUS transmission configuration from. We think there are other options for the related signalling, thus other options should not be precluded at this stage. |
| CEWiT | Support with comment | we prefer Alt-4 and Alt-5 which are more suitable for idle UEs to access the cell with low latency. |
| Spreadtrum | Y |  |
| Qualcomm |  | This proposal is too detail - We can discuss it after we make further progress related to Proposal 1-2. Furthermore, our position was not captured properly. We don’t propose using Msg2/4 to carry WUS config. Since the legacy specs define on-demand OSI procedure (and on-demand PRS reusing on-demand OSI procedure), we suggest following the on-demand OSI procedure principles. In particular, Msg2/Msg4 is the gNB confirmation on reception of UE request in Msg1/Msg3 respectively. Hence, instead of having this proposal, we think the following proposal should be studied and we provided it to Issue 9. Suggested proposal: For the further study of on-demand SIB1 in for idle/inactive mode UE, RAN1 to further study whether feedback from gNB in response to the SIB1 request is supported before the UE starts monitoring SIB1 PDCCH. |
| Vivo |  | As indicated in comment of Issue 1, WUS configuration of Cell B can also be contained in SIBx of Cell B. So, for Alt 1, we suggest to add Cell B to Alt 1, i.e. Alt 1: SIBx of Cell A and/or Cell B |
| NEC |  | We think all options as proposed in the background section should be included at this stage. |
| CATT |  | Need to clearly define cell A; for these alternatives, some are not even possible. For example DCI of cell B. |
| Fujitsu | Support the proposal and vivo’s update |  |
| China Telecom | Support | We are fine with the proposal and prefer Alt1. However, we think this can be discussed after the discussion of proposal 1-2, so that some alts can be reduced and the description of remain alts can be more accurate. |
| Apple |  | We propose to add another alternative: Alt 6: RRC release message of cell A |
| Futurewei |  | Agree with vivo’s suggestion. We also echo Qualcomm’s view that this proposal is too detailed. In addition, many of the idle/inactive procedures and signalling are handled by RAN2, so we think RAN1 should discuss with RAN2 to reach some general guideline on which WG can make a decision on issues like this. |
| Xiaomi |  | For the proposal suggested by QC, we think it is a different topic and should be discussed separately. For Alt 1, we support vivo’s version. Alt 2、Alt 4 and Alt 5 can be discussed if UE obtains WUS configuration on Cell B is conformed. |
| Huawei, HiSilicon | Y |  |
| Nokia/NSB | Support and select from Alt1, Alt3, Alt4 |  |
| Samsung |  | It can be discussed after a discussion on a feasibility of each scenario |
| Fraunhofer | Support |  |
| InterDigital | Support |  |
| ETRI | Support | The due date RAN1#116b may be deleted. Alt. 1 may be related to Alt. 2 depending on details of SIBx. |
| ASUSTeK | Support | Ok to study further these alternatives, while maybe not all alternative is reasonable. For example, if UL WUS configuration is provided by DCI, the energy saving gain could be reduced since the UL WUS may needs to be provided periodically (which replaces SIB1). Since we also prefer to provide UL WUS configuration by Cell B (as our answers to proposal 1-1/1-2), we are open to Alt. 3, Alt.4, Alt.5. |
| Sharp | Support |  |
| Vodafone | Support |  |
| Sony | Support | We are fine with the proposal then we could discuss next meeting. We prefer alt 1 for alternative 1 in proposal 1-2 and alt 4, alt 5 for alternative 2 in proposal 1-2. |
| DCM | Support |  |

- FL Proposal 6-1-v2

For the further study of UL WUS transmitted by UE to trigger on-demand SIB1 on NES cell, UE obtains the WUS configuration from one or more of:

- Alt 1: SIBx of Cell A and/or NES cell

- Alt 2: DCI on Cell A or NES cell

- Alt 3: Msg 4 or Msg 2

- Alt 4: PBCH on NES Cell

- Alt 5: Predefined configuration

- Alt 6: RRC release message of cell A

| Company | Support or not | Comment |
| --- | --- | --- |
| Moderator |  | Combining companies input for FL Proposal 6-1-v1 and try to reformulate. |
| Futurewei | Support |  |
| Xiaomi |  | For Alt 6, the RRC release message of cell B is also a candidate for WUS configuration. Hence we propose: - Alt 6: RRC release message of cell A and/or NES cell |
| CEWiT | Support |  |
| Vodafone | Support |  |
| ETRI | Support |  |
| DCM2 | Support |  |
| LG Electronics2 |  | According to the Note in WID description, no modification of SSB will be discussed under 9.5.3. Therefore, Alt 4 should be removed until we clarify whether the modification of PBCH is allowed or not |
| Sharp | Support |  |
| Fraunhofer |  | Agree with Xiaomi. |

- FL Proposal 6-1-v3

For the further study of UL WUS transmitted by UE to trigger on-demand SIB1 on NES cell, UE obtains the WUS configuration from one or more of:

- Alt 1: SIBx of Cell A and/or NES cell

- Alt 2: DCI on Cell A or NES cell

- Alt 3: Msg 4 or Msg 2

- Alt 4: [PBCH on NES Cell]

- Alt 5: Predefined configuration

- Alt 6: RRC release message of cell A

- Issue 7: How UE identifies a NES cell is with on-demand SIB1

Background

About how UE identifies a NES cell is with on-demand SIB1, the following company views are collected:

- Alt 1: By WUS configuration: Xiaomi, MTK

- Alt 2: By MIB of NES cell: Futurewei, Qualcomm, NEC, ETRI, Samsung

- Alt 3: By DCI 1_0 (SI_RNTI) from NES cell: NEC

- Alt 4: By UE blind detection: ETRI

- RAN1 should discuss this issue: Ericsson, Sharp, Vodafone (NW should inform this to UE)

- FL Proposal 7-1

For the further study of how UE identifies a NES cell is with on-demand SIB1, RAN1 to down-select from the following alternatives in RAN1 #116b.

- Alt 1: By WUS configuration

- Alt 2: By MIB of NES cell

- Alt 3: By DCI 1_0 from NES cell

- Alt 4: By UE blind detection

| Company | Support or not | Comment |
| --- | --- | --- |
| Moderator |  | Companies are welcomed to add more alternatives if not being captured in the list yet. |
| Google | Support |  |
| Panasonic | Support |  |
| Intel | Comment | The first bullet should also refer to “UL WUS” and not “WUS” to not be confused with the WUS in the downlink. We would like to avoid deciding this critical part in RAN1#116. In addition, we should not preclude other options at this stage. |
| CEWiT | Support with Comment | we prefer Alt-2 and Alt-3. We are also ok for Alt-1 with lower priority. Alt-4 may cause negative impact in UE power consumption due to blind detection. |
| Spreadtrum | Y |  |
| Qualcomm |  | Alt 1, Alt 3 and Alt 4 do not work since UE performing cell search shall need to search every sync raster until it finds CD-SSB. However, CD-SSB may have always-on SIB1 transmission (legacy) or may have on-demand SIB1 (if being specified in R19). In this case, without indication, the UE shall not know whether it should receive SIB1 or need to request SIB1. Hence, blind detection (Alt 4) means that UE always has to assume there is SIB1 transmission and after a long time it can’t find SIB1 if SIB1 is on-demand, which significantly impacts UE power and latency. In addition, indication from WUS config (Alt 1) or DCI 1-0 from NES (Alt 3) requests UE to find assisting cell first. |
| vivo | Support |  |
| CATT |  | What is the definition of the procedure ‘UE identifies a NES cell’ ? what does this mean in 3GPP term? We cannot agree anything without clear definition. |
| Fujitsu | Support |  |
| China Telecom | Support. |  |
| Apple |  | We think this is related to the decision of Proposal 1-2, and also on whether the NES cell transmits CD-SSB. For CD-SSB, ALt 2 is more straightforward, for NCD-SSB, other options could be considered, e.g. from the configuration of cell A, we proposa to add another Alternative: Alt 5: By SIB or RRC messages of cell A |
| Futurewei |  | We agree with other companies that on Alt 4 which significantly impacts UE and Alt 4 should be removed. We suggest further discussions of the proposal. |
| Xiaomi |  | Just to clarify our position. We support Alt 1 is there is a ‘cell A’ to transmit the WUS configuration of cell B. In that case, anyway UE needs to obtain WUS configuration before triggering SIB1. It is natural that the NES ID is carried by WUS configuration. Otherwise, if UE has to obtain WUS configuration on NES cell, we are open to discuss Alt 2 and Alt 3. For Alt 4, if UE cannot find SIB1 after a long time, the possible situation may be： - The cell periodically transmits SIB1 while UE fails to receive SIB1 due to poor DL performance; - The cell is an on-demand SIB1 cell and the SIB1 is not transmitted without the WUS triggering. To be concluded, Alt 4 is not workable for identifying the NES cell. Besides, we share different view with QC. For Alt.1, if WUS configuration has been obtained by the UE, the UE will know the target cell for requesting SIB1. If WUS configuration has not been obtained, the UE cannot request SIB1. If UE searches for an on-demand SIB1 cell, it can not obtain SIB1 without SIB1 requesting. The cell will be treated as barred due to current mechanism. The UE will continue to search for other cells. For Alt.3, the UE can receive DCI 1_0 (SI-RNTI) before SIB1. Hence, UE can obtain it on NES cell and assisting cell is not mandatory. |
| Huawei, HiSilicon | Y | We would guess there may be other alternatives not precluded, or combination of the above. But Ok with the current one. |
| Nokia/NSB | Support Alt1 and Alt2 |  |
| Samsung |  | It can be discussed after a discussion on a feasibility of each scenario |
| InterDigital |  | We tend to share same view with Intel to not preclude other options at this state. As such, we suggest adding “Alt 5: Others” to proposal 7-1. |
| ZTE, Sanechips |  | We should not exclude other possibilities at this state, thus we suggest to add following note. Note: other options are not precluded |
| ETRI | Support in general | The proposal seems a good starting point for discussion. Some alternatives may need a clarification. |
| ASUSTek | Support | Prefer Alt 1 and Alt 2. |
| Vodafone | Support |  |
| Sony | Support | We are fine with the proposal. |
| DCM | Support with comment | - For Alt.2, we suggest to update as “Alt 2: By MIB/PBCH of NES cell” as PBCH carries more information bits in addition to MIB, then it has the potential for the indication. For the further study of how UE identifies a NES cell is with on-demand SIB1, RAN1 to down-select from the following alternatives in RAN1 #116b. - Alt 1: By WUS configuration - Alt 2: By MIB/PBCH of NES cell - Alt 3: By DCI 1_0 from NES cell - Alt 4: By UE blind detection |

- FL Proposal 7-1-v2

For the further study of how UE identifies a NES cell is with on-demand SIB1, RAN1 to discuss the following alternatives.

- Alt 1: By WUS configuration

- Alt 2: By MIB/PBCH of NES cell

- Alt 3: By DCI 1_0 from NES cell

- Alt 4: By UE blind detection

- Alt 5: By SIB or RRC messages of cell A

Note: other alternatives are not precluded

| Company | Support or not | Comment |
| --- | --- | --- |
| Moderator |  | Combining companies input for FL Proposal 7-1-v1 and try to reformulate. |
| LG Electronics2 | Support but comment | As per the Note in the WID description, modification of SSB is not discussed in 9.5.3, so if Alt 2 requires modification of the legacy MIB/PBCH, Alt 2 should be removed. |
| Futurewei | Support |  |
| Xiaomi |  | We fail to find the feasibility of Alt 4 and prefer to remove it. |
| CEWiT |  | Motivation of Alt-4 is not clear to us. |
| Vodafone | Support | OK to preclude some options such as Alt 4 as it is not very clear what it means in this context |
| ETRI | Support | For Alt 4, although we suggested to study Alt. 4 as another possible approach, we also share the view that there would be UE complexity and reliability issues. So if there are concerns, we are okay to remove it. |
| DCM2 | Support |  |
| Sharp | Support | We are fine to remove Alt 4 for the discussion. |
| Lenovo | Support | Share similar views with others to remove Alt.4. |
| Fraunhofer |  | As suggested by others above we prefer to remove Alt 4. Additionally, we would to make Alt3 more generic and add another alternative as follows: - FL Proposal 7-1-v2 For the further study of how UE identifies a NES cell is with on-demand SIB1, RAN1 to discuss the following alternatives. - Alt 1: By WUS configuration - Alt 2: By MIB/PBCH of NES cell - Alt 3: By DCI 1_0 from NES cell (e.g., DCI 1_0) - Alt 4: By UE blind detection - Alt 5: By SIB or RRC messages of cell A - Alt 6: By monitoring new RNTI on Coreset#0 from NES cell Note: other alternatives are not precluded |

- FL Proposal 7-1-v3

For the further study of how UE identifies a NES cell is with on-demand SIB1, RAN1 to discuss the following alternatives.

- Alt 1: By WUS configuration

- Alt 2: By MIB[/PBCH] of NES cell

- Alt 3: By DCI 1_0 from NES cell (e.g., DCI 1_0)

- Alt 4: By UE blind detection

- Alt 5: By SIB or RRC messages of cell A

- Alt 6: By monitoring new RNTI on Coreset#0 from NES cell

Note: other alternatives are not precluded

- Issue 8: UL WUS is cell-specific or shared among multiple cells

Background

About UL WUS is cell-specific or shared among multiple cells, the following company views are collected:

[7, CMCC]

- Proposal 3: It should be further discussed whether the wake-up signal is a cell-specific one or can be shared among multiple cells.

[12, Xiaomi]

- Proposal 4:  The following mechanisms can be considered for how to carry WUS configuration:

  - One cell carries WUS configuration of multiple cells.

  - Identical WUS configuration is broadcasted by multiple cells.

[24, DOCOMO]

Proposal 4:

- Study the validity range of WUS configuration with following alternatives,

  - Alt.1: An area specific WUS configuration, where the area compromising multiple cells.

  - Alt.2: A cell specific WUS configuration.

[31, CEWiT]

Proposal 1: Following alternatives can be considered for the design of WUS

- Alt.1. Common WUS for the network.

- Alt.2. Cell specific WUS.

- Alt.3. SSB(s) specific WUS.

- FL Proposal 8-1

For the further study on whether UL WUS configuration is cell-specific or shared among multiple cells, RAN1 aims to down-select from the following alternatives in RAN1 #116b.

- Alt 1: Common WUS for the network

- Alt 2: Common WUS for multiples cells

  - FFS: how to define the multiple cells

- Alt 3: Cell specific WUS

- Alt 4: SSB(s) specific WUS

| Company | Support or not | Comment |
| --- | --- | --- |
| Moderator |  | Companies are welcomed to add more alternatives if not being captured in the list yet. |
| Google | Support |  |
| Panasonic | Support |  |
| Intel | Comment | The sub bullets should also refer to “UL WUS” and not “WUS” to not be confused with the WUS in the downlink. This proposal does overlap with proposal 6-1. In our understanding it is better to combine this discussion as any, single options in on of the proposals restricts the options on the other. |
| CEWiT | Support |  |
| Spreadtrum | Y | But in our view Alt-1 is not feasible, which gives zero flexibility for network |
| vivo | Support |  |
| CATT |  | Need to remove the restriction of ‘in RAN1 #116b’ |
| Fujitsu | Support |  |
| China Telecom | Support. |  |
| Apple | Support | Need to remove the restriction of ‘in RAN1 #116b’ |
| Futurewei | Support with comment | We are not sure Alt 1 can work. Suggest removing. |
| Xiaomi |  | The issue is high related with Proposal 1-1 and Proposal 1-2. Hence the discussion can be postponed after Proposal 1-1 and Proposal 1-2 have been determined. |
| Huawei, HiSilicon | Y |  |
| Nokia/NSB |  | It can be discussed later when the whole procedures are clarified. |
| Samsung |  | It can be discussed after a discussion on a feasibility of each scenario |
| InterDigital | Support |  |
| ETRI | Support |  |
| Sharp | Support |  |
| Vodafone | Support |  |
| Sony | Support |  |
| DCM | Support |  |

- FL Proposal 8-1-v2

For the further study on whether UL WUS configuration is cell-specific or shared among multiple cells, RAN1 study the following options:

- Option 1: Common UL WUS for the network

- Option 2: Common UL WUS for multiples cells

  - FFS: how to define the multiple cells

- Option 3: Cell specific UL WUS

- Option 4: SSB(s) specific UL WUS

| Company | Support or not | Comment |
| --- | --- | --- |
| Moderator |  | Combining companies input for FL Proposal 8-1-v1 and try to reformulate. |
| LG Electronics |  | We think there needs to be more discussion about the need and feasibility of each option. |
| Futurewei | Support | Open to discuss. |
| Xiaomi |  | We think the issue is high related with the target cell and target signal/channel for WUS configuration. We prefer to postpone it until the way for WUS configuration is determined. |
| Vodafone | Support |  |
| ETRI | Support |  |
| DCM2 | Support |  |
| Sharp | Support |  |
| Lenovo |  | Suggest to add “configuration” after WUS in each option to make the proposal clear. |
| Fraunhofer | Support with additional note | Note: supporting multiple options above is not precluded. |

- FL Proposal 8-1-v3

For the further study on whether UL WUS configuration is cell-specific or shared among multiple cells, RAN1 study the following options:

- Option 1: Common UL WUS configuration for the network

- Option 2: Common UL WUS configuration for multiples cells

  - FFS: how to define the multiple cells

- Option 3: Cell specific UL WUS configuration

- Option 4: SSB(s) specific UL WUS configuration

- Issue 9: Confirmation of reception of UL WUS transmission

Background

About confirmation of reception of UL WUS transmission, the following company views are collected:

[10, CATT]

Proposal 6:  If on-demand SIB1 is supported, the confirmation of reception of UL WUS transmission should be supported to guarantee the understanding of SIB1 transmission decision between NES mode cell and UE is

[34, Qualcomm]

Proposal 4: As a starting point, RAN1 follows the on-demand OSI procedure principles for on-demand SIB1 procedure (as shown in Figure 3). RAN1 to further discuss the following aspects:

- Whether to follow MSG1-based and/or MSG3-based SI request principle

- Whether or not to support an explicit response to the SIB1 request, prior to sending on-demand SIB1

- Whether the resources and configurations of the DL/UL signals associated with on-demand SIB1 procedure can be at least partially shared with the common RACH procedure.

- FL Proposal 9-1

For the further study of on-demand SIB1 in idle/inactive mode, RAN1 to study whether to support an explicit response to the UL WUS requesting SIB1, prior to sending on-demand SIB1, and aims to make a decision in RAN1 #116b.

| Company | Support or not | Comment |
| --- | --- | --- |
| Moderator |  | Companies are welcomed to revise the wording or make suggestions. |
| Google | Support |  |
| Panasonic | Support |  |
| Intel | Support | In our understanding any necessary information is already contained in SIB1 thus no further information is necessary. It is also unclear to use what are the benefits of transmitting this additional information in other signalling than SIB1. |
| CEWiT | Comment | transmission of SIB1 itself is an implicit indication for the response to the UL WUS hence no further information is necessary. |
| Spreadtrum | Y |  |
| Qualcomm |  | Since the legacy specs define on-demand OSI procedure (and on-demand PRS reusing on-demand OSI procedure), we suggest following the on-demand OSI procedure principles. In particular, Msg2/Msg4 is the gNB confirmation on reception of UE request in Msg1/Msg3 respectively. Hence, we propose the following revision. Suggested revision: For the further study of on-demand SIB1 in for idle/inactive mode UE, RAN1 to further study whether feedback from gNB in response to the SIB1 request is supported before the UE starts monitoring SIB1 PDCCH. |
| vivo | Support |  |
| NEC | Support |  |
| CATT |  | Ok. Fine to say ‘ aim to make decision …’ . |
| Fujitsu | Support |  |
| China Telecom | Support. |  |
| Apple |  | Support QC’s suggested version. |
| Futurewei | Support |  |
| Xiaomi | Support |  |
| Huawei, HiSilicon | Y | Although we think it is part of the previous question for potential impact on RACH procedure. Ok to be more specific. |
| Samsung |  | It can be discussed after a discussion on a feasibility of each scenario |
| Fraunhofer | Support |  |
| InterDigital | Support |  |
| ETRI | Support |  |
| ASUSTeK | Support | As Qualcomm mentioned, RAR could serve as the explicit response when Msg1 is used as UL WUS (similarly as legacy on-demand OSI design). |
| Sharp | Support |  |
| Vodafone |  | We don’t see the need of such indication, as the transmission of SIB1 would serve as an indicator that the UL WUS was successfully received. |
| Sony | Support |  |
| DCM | Support | Also fine with QC’s version. |

- FL Proposal 9-1-v2

For the further study of on-demand SIB1 in for idle/inactive mode UE, RAN1 to further study whether feedback from gNB in response to the SIB1 request is supported before the UE starts monitoring SIB1 PDCCH.

| Company | Support or not | Comment |
| --- | --- | --- |
| Moderator |  | Combining companies input for FL Proposal 9-1-v1 and try to reformulate. |
| LG Electronics | Support | The CORESET/search space set configuration for reception to SIB1 could also be included in the gNB’s response. |
| Futurewei | Support |  |
| Xiaomi |  | The response is for the request for SIB1, not for SIB1 PDCCH. Hence we prefer the way of previous version: FL Proposal 9-1-v2 For the further study of on-demand SIB1 in for idle/inactive mode UE, RAN1 to further study whether feedback from gNB in response to the SIB1 request is supported before the UE starts receiving SIB1starts monitoring SIB1 PDCCH.. |
| Vodafone |  | OK to consider for the study but not clear why it is needed. |
| ETRI | Support |  |
| Sharp | Support | We OK to discuss it. |
| Lenovo |  | Share same view with Xiaomi. |

- FL Proposal 9-1-v3

For the further study of on-demand SIB1 in for idle/inactive mode UE, RAN1 to further study whether feedback from gNB in response to the SIB1 request is supported before the UE starts receiving SIB1starts monitoring SIB1 PDCCH.

- Issue 10: How long on-demand SIB1 is transmitted after BS receives a UL WUS

Background

About how long on-demand SIB1 is transmitted after BS receives a UL WUS, the following company views are collected:

[16, NEC]

Proposal 10: The UE should monitor the on-demand SIB1 transmissions within a specified time window.

[24, DCM]

Proposal 5: Consider the following time domain behaviours of on-demand SIB1 TX, potential indication of the TX, and the way to configure these necessary parameters.

- Alt.1 Aperiodic SIB1 transmission

- Alt.2 Semi-persistence SIB1 transmission

- Alt.3 Periodic SIB1 transmission

[28, Fujitsu]

Proposal 7: Study further about detail of the one-shot SIB1 transmission scheme.

- FL Proposal 10-1

For the further study of time domain behaviours of on-demand SIB1 transmission, RAN1 to study the way to configure corresponding necessary parameters of the following alternatives, and aims to down-select from them in RAN1 #116b.

- Alt.1 Aperiodic SIB1 transmission

- Alt.2 Semi-persistence SIB1 transmission

- Alt.3 Periodic SIB1 transmission

| Company | Support or not | Comment |
| --- | --- | --- |
| Moderator |  | Companies are welcomed to revise the wording or make suggestions on the alternatives. |
| Google |  | We think this proposal can be reformulated based on when the UE can start to monitor SIB1 after transmitting the UL-WUS. |
| Panasonic | Support |  |
| Intel | Comment | In our understanding as the SIB1 is only transmitted based on a user request it is not necessary to perform periodic or semi-persistent SIB1 transmissions as long as SIB1 remains the same information. |
| CEWiT | Support with comment | we prefer Alt-1, it will help gNB to avoid additional SIB1 transmissions thereby helping in acheiving more energy savings. |
| Spreadtrum | Y | Alt.3 is enough |
| Vivo |  | We are fine to discuss time domain issues of on demand SIB1 transmission. In our understanding, SIB1 can be transmitted for a time window. So we suggest to add the following Alt: Alt. 4: SIB1 transmission within a time window |
| NEC |  | Agree with Vivo’s comment that SIB1 can be transmitted and monitored for a time window from UE’s perspective. So we support to add the following Alt: Alt. 4: SIB1 transmission within a time window Periodic or semi-persistent SIB1 transmissions does not provide gain for NES. |
| CATT |  | For alt2/alt3, both should be ‘within a time window’ |
| Fujitsu | Support | From the perspective of maximizing NES gain, Alt 1 is preferred. |
| China Telecom | Support | We prefer Alt1 or Alt2/Alt3 ‘within a time window’. |
| Apple |  | Support adding Alt 4. SIB1 transmission within a time window |
| Futurewei | Support | The Alt 4 suggested by vivo may be incorporated into Alt 1? |
| Xiaomi |  | We think it is just a gNB implementation issue. However, we are open to discuss the Proposal if it is the majority view. |
| Huawei, HiSilicon |  | The proposal is a bit unclear. Are we to study the way to configure parameters for all the alternatives, or study and determine which alternative is to be adopted. Additionally, it is not clear what does semi persistence mean ? or how it will work for on-demand SIB1? Perhaps we can simplify the proposal: For the time domain behaviours of on-demand SIB1 transmission, RAN1 to study and aims to down-select from the following alternatives in RAN1 #116b. - Alt.1 Aperiodic SIB1 transmission - Alt.2 Semi-persistence SIB1 transmission - Alt.3 Periodic SIB1 transmission |
| Nokia/NSB |  | After the NW reception of WUS from one or more UEs, the NW switch to legacy SIB1 mode with periodic SIB1 transmission, and continue with the periodic SIB1 transmission until the NW decide to go to the on-demand SIB1 mode again. |
| Fraunhofer |  | Agree with Apple. |
| InterDigital | Support |  |
| ETRI | Support |  |
| ASUSTeK | Support |  |
| Sharp | Support |  |
| Vodafone |  | In our understanding, upon reception of UL WUS it is up to gNB to transmit SIB1 as it was doing before disabling it. |
| Sony | Support |  |
| DCM | Support |  |

- FL Proposal 10-1-v2

For the time domain behaviours of on-demand SIB1 transmission, RAN1 to study the following options.

- Option 1: Aperiodic SIB1 transmission

- Option 2: Semi-persistence SIB1 transmission

- Option 3: Periodic SIB1 transmission

- Option 4: SIB1 transmission within a time window

| Company | Support or not | Comment |
| --- | --- | --- |
| Moderator |  | Combining companies input for FL Proposal 10-1-v1 and try to reformulate. |
| LG Electronics | Support | We prefer Option 4. |
| Futurewei | Support |  |
| Xiaomi |  | We think it can be realized by gNB implementation. |
| CEWiT |  | We prefer option 1 for more oppurtunity at gNB for NES. |
| Vodafone |  | We think it should up to gNB implementation, but it should be fine to compare the different options in the study. |
| ETRI | Support |  |
| DCM2 | Support |  |
| LG Electronics2 |  | There seems to be an overlap between the options, what is the difference between Option 2 and Option 4? |
| Sharp | Support |  |
| Lenovo | Support |  |

- FL Proposal 10-1-v3

For the time domain behaviours of on-demand SIB1 reception for UE, RAN1 to study the following options.

- Option 1: Aperiodic SIB1 reception

- Option 2: Semi-persistent SIB1 reception

- Option 3: Periodic SIB1 reception after UE transmits the UL WUS

- Option 4: SIB1 reception within a time window

Note: SIB1 reception is scheduled by the DCI as in legacy procedure

Resulted RAN1 conclusion/agreement

Agreement

For discussion purpose, the following assumption will be used in RAN1

- Cell A: A cell that is periodically transmitting at least its own SIB1

- NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE

For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 studies the following options.

On target cell of UL WUS transmission:

- Option 1: UE transmits UL WUS to NES Cell

- Option 2: UE transmits UL WUS to Cell A

On configuration provision for UL WUS transmission

- Option A: UE obtains the UL WUS configuration from NES Cell

- Option B: UE obtains the UL WUS configuration from Cell A

Other options are not precluded

Agreement

For further study of achievable NES gain with on-demand SIB1 for idle/inactive mode UE,

- Assume the following for network energy evaluation of non-NES cell in FR1:

- Empty/low/medium cell load as defined in 38.864

- Cat 1/Cat 2 BS as defined in 38.864

- 30kHz SCS, DDDSU TDD pattern

  - Case A: 20ms SSB period with 20ms SIB1 period;

  - Case C: 20ms SSB period with 160ms SIB1 period;

  - Case D: 20ms SSB period with 40ms SIB1 period;

Note: Other SSB/SIB1 periodicity assumptions are not precluded (up to companies to report)

- 4 or 8 SSBs in a SSB burst with SSB pattern case C

- 20ms or 160ms PRACH monitoring period

- Assume the following for network energy evaluation of NES cell in FR1:

- Empty/low/medium cell load as defined in 38.864

- Cat 1/Cat 2 BS as defined in 38.864

- 30kHz SCS, DDDSU TDD pattern

  - Case 1: 20ms SSB period with no SIB1 transmitted;

Note: Other SSB/SIB1 assumptions are not precluded (up to companies to report)

- 4 or 8 SSBs in a SSB burst with SSB pattern case C

- 20ms/160ms UL WUS monitoring period

- Note: SSB/CORESET0 multiplexing pattern 1 is used

Agreement

- For study of UL WUS design, consider at least PRACH as a starting point

  - FFS: Whether there is dedicated PRACH resource for SIB1 request

- Other option(s) not precluded

Agreement

For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 to discuss triggering conditions for sending UL-WUS.

Agreement

For the study of on-demand SIB1 for idle/inactive mode UE, RAN1 to further study whether feedback from gNB in response to the SIB1 request is supported including associated details.

Possible Agreement

For the time domain behaviours of on-demand SIB1 transmission, RAN1 to study the following options.

- Option 1: Aperiodic SIB1 transmission

- Option 2: Semi-persistent SIB1 transmission

- Option 3: Periodic SIB1 transmission

- Option 4: SIB1 transmission within a time window

Note: SIB1 transmission is scheduled by DCI

- References (all from RAN1 #116)

[1]	R1-2400063, Discussion on on-demand SIB1 for idle/inactive mode UEs, Spreadtrum Communications

[2]	R1-2400098, Discussion of on-demand SIB1 for idle/inactive mode UEs, FUTUREWEI

[3]	R1-2400120, Discussion on on-demand SIB1 for NES, Huawei, HiSilicon

[4]	R1-2400181, On-demand SIB1 for Idle/Inactive mode UEs, Nokia, Nokia Shanghai Bell

[5]	R1-2400209, Discussion on on-demand SIB1 transmission, Transsion Holdings

[6]	R1-2400251, Discussions on on-demand SIB1 for idle/inactive mode UEs, vivo

[7]	R1-2400336, Disucssion on-demand SIB1 for UEs in idle/inactive mode, CMCC

[8]	R1-2400370, Study of on-demand SIB1 for idle/inactive mode UEs, Intel Corporation

[9]	R1-2400400, On-demand SIB1 for Idle/Inactive Mode UE, Google

[10]	R1-2400442, Discussion on on-demand SIB1, CATT

[11]	R1-2400493, Discussion on on-demand SIB1 for UEs in idle or inactive mode, ZTE, Sanechips

[12]	R1-2400567, Discussion on on-demand SIB1 for idle/inactive mode UEs, xiaomi

[13]	R1-2400600, Discussion on the enhancement to support on demand SIB1 for idle/inactive mode UE, OPPO

[14]	R1-2400668, Discussion on on-demand SIB1 for idle/inactive mode UEs, China Telecom

[15]	R1-2400741, On-demand SIB1 for idle/inactive mode UEs, Samsung

[16]	R1-2400807, Discussion on on-demand SIB1 for UEs in idle/inactive mode, NEC

[17]	R1-2400822, Discussion on On-demand SIB1 for idle/inactive mode UEs, InterDigital, Inc.

[18]	R1-2400861, Considerations on on-demand SIB1 for idle/inactive mode UEs, Sony

[19]	R1-2400902, Discussion on on-demand SIB1 for idle/inactive mode UEs, Panasonic

[20]	R1-2400928, On-demand SIB1 for NES, Fraunhofer IIS, Fraunhofer HHI

[21]	R1-2401021, On On-demand SIB1 for IDLE/INACTIVE mode UEs, Apple

[22]	R1-2401054, Scope of on-demand SIB study, NTPU

[23]	R1-2401086, On-demand SIB1 for idle/inactive mode UEs, Lenovo

[24]	R1-2401124, Discussion on on-demand SIB1 for idle/inactive mode UEs, NTT DOCOMO, INC.

[25]	R1-2401144, Study of on-demand SIB1 for UEs in idle/inactive mode for NES, Ericsson

[26]	R1-2401176, Discussion on on-demand SIB1 transmission for idle UEs, Sharp

[27]	R1-2401197, Triggering of on-demand SIB1, ASUSTeK

[28]	R1-2401204, Discussion on on-demand SIB1 transmission for network energy savings, Fujitsu

[29]	R1-2401233, Discussion on on-demand SIB1 for idle/inactive mode UEs, ETRI

[30]	R1-2401253, Views on On-demand SIB1 operation for idle/inactive UEs, Vodafone

[31]	R1-2401280, Discussion on    on-demand SIB1, CEWiT

[32]	R1-2401292, On-demand SIB1 for idle or inactive mode UEs, MediaTek Inc.

[33]	R1-2401333, On-demand SIB1 for idle/inactive mode UEs, LG Electronics

[34]	R1-2401450, On-demand SIB1 procedure, Qualcomm Incorporated