3GPP TSG RAN WG1 #116-bis	R1-2402039

Changsha, China, Apr. 15th – 19th, 2024

Agenda Item:	9.1.2

Source:		Fraunhofer IIS, Fraunhofer HHI

Title:		AI/ML positioning accuracy enhancement

Document for:	Discussion & Decision

# Introduction

The positioning related objectives of the WID in RP-234039 [1] with regards to potential specification impact consists of:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

In this contribution, we delve into the aspects of Model monitoring, functionality/model management, and model Inference measurement enhancements that are essential for the operation of the identified positioning cases.

# Model Input

## DL PRS and UL SRS

In RAN1#116, the following agreement was concluded

Agreement For Rel-19 AI/ML based positioning, the measurements for determining model input are based on the DL PRS and UL SRS defined in TS38.211. - Note: The use of SRS for MIMO resource is transparent to UE.

Based on the findings from evaluations conducted during the study phase, it has been shown that the utilization of multiple transmit/receive antenna port pairs (Nport) has a significant positive impact on performance. Multi-port operation additionally reduces the number of OFDM symbols needed to support positioning. However, in Rel. 16, SRS transmission was limited to a single-port for positioning.

Proposal 1: 	Consider multi-port UL SRS support for AI/ML positioning.

# Discussion on potential specification impact for the identified cases

## Direct AI/ML positioning

### Case 1: UE-based positioning with UE-side model

In Case 1, the UE performs measurements on the DL-PRS, utilizing these measurements as input for model inference, which in the context of direct positioning it is the position. To ensure reliable operation, signaling should encompass validity conditions, such as applicable areas and UE capabilities. Additionally, the requirement for assistance signaling depends on various factors, including the synchronization of DL-PRS resources from multiple NG-RAN nodes, which may be necessary based on the model and network configuration.

Figure 1: UE-based positioning with UE-sided model (direct positioning).

The UE is responsible for deriving monitoring metrics in Case 1. It can monitor error sources, faults, or performance-related conditions and relay this information to the network. The network utilizes this data to verify the performance of the AI/ML model on the UE side and gain a general overview of each (logical) model’s performance, as well as the performance of the activated functionality in each case, by gathering monitoring data from multiple UEs.

Observation 1: 	UE monitoring reports in Case 1 assist the network in verifying AI/ML model/functionality performance and gathering multi-UE monitoring data for performance management.

In scenarios where the UE supports multiple models or functionalities, network management may facilitate the selection, activation, deactivation, or switching of AI/ML (logical) models or functionalities, as well as fallback to non-AI/ML operation. Although the AI/ML model is monitored by the UE, the network can request crucial monitoring information from the UE for functionality monitoring, without compromising the proprietary nature of the UE-side model. Specifically, if the UE updates its supported functionalities or requests a change in the functionality (i.e., (de)activation/switching or fallback), it should provide information to the network on whether this request was due to i) update in UE capability; ii) the current AI/ML (logical) model underperforming; or iii) an inactive functionality/model is predicted to perform better.

Observation 2: 	In Case 1, if the UE requests a change in the functionality (i.e., (de)activation/switching or fallback) or updates its supported functionalities, without communicating the UE-side monitoring event that triggered this request or change, the network may lack crucial information that could lead to inefficiencies in network management.

Proposal 2: 	Support UE-sided models to inform the network on the monitoring event that triggered a request to switch functionality or an update of the supported functionalities.

### LMF-side model (Case 2b: UE-assisted & Case 3b: NG-RAN node assisted)

The following agreements were reached in the previous meeting with regards to cases 2b and 3b:

Agreement For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting: - timing information; - paired timing information and power information. Agreement For AI/ML based positioning case 2b, at least the following types of time domain channel measurements are supported for UE reporting to LMF: - timing information; - paired timing information and power information. Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to a reference time. - FFS: Whether any specification impact of the reference time used to represent the timing information. Details of the reference time

In Case 2b the UE provides measurements on DL-PRS to the network (LMF). Similarly, in Case 3b the NG-RAN nodes assist in the positioning process by providing measurements to the network. The network utilizes these measurements as input to the LMF-side model, which then outputs the UE position. In this case, the assistance information typically includes information on reference signal configuration and the measurements needed to be generated to support the LMF-model’s positioning estimation. Depending on the LMF model functionality, different RS configurations (beam configuration, periodicity, etc.) and measurements may be required.

Figure 2: LMF-based positioning with LMF-side model: Case 2b: UE-assisted and Case 3b: NG-RAN-assisted (1st priority).

Figure 3 shows possible building blocks of the measurement unit (UE in case 2b, TRP in case 3b). The UE may receive (and process) RS from several TRP. For Case 3b one or several NG_RAN nodes will receive a RS from the UE. Accordingly, several NG-RAN nodes may report to the LMF. For OFDM based systems, the processing of the reference signal is performed in the frequency domain using the FFT output. Multiplying the FFT output with the conjugate reference signal (RS) sequence implements the cyclic correlation. The intermediate output is the CIR in the frequency domain (“CIR_FD”).

Figure 3: Overview of measurement unit building blocks.

Channel estimation algorithms may use this data directly as input for further processing. The number of relevant samples in the frequency domain depends on the number of sub-carriers assigned to the RS. The frequency domain signal can be transformed into the time domain by the inverse FFT (iFFT) resulting in the CIR_TD. The CIR_TD may be the input for building blocks estimating the paths in the time domain. For AI/ML based algorithms it may be sufficient to use the CIR_TD samples as input to the model. To reduce the reporting complexity the relevant parts of the CIR_TD may be extracted. The LMF can request for performance or monitoring operation, reports including relevant samples of the CIR_TD together with measurements already defined in earlier positioning releases. Depending on the measurement report configuration, the LMF can evaluate the report and further process the measurements, as shown in the example of Figure 4.

Figure 4: Building blocks of LMF with AI/ML support.

Observation 3: 	LMF can utilize reports that include AI/ML measurements, along with measurements from earlier releases, based on the same resources.

Proposal 3: 	Define new measurement, including sample-based time domain CIR to provide model input for cases 2b and 3b.

## AI/ML assisted positioning (UE side or gNB side model)

Case 2a involves UE performing an intermediate step by providing a measurement or classification as output of an AI/ML model, assisting in positioning within the network. Similarly, Case 3a entails the gNB conducting an intermediate step by providing a measurement or classification as output of an AI/ML model. Both cases are illustrated in Figure 5.

AI/ML functionality/model specific information might be provided. The assistance information typically includes measurements or reference signal configuration that need to be generated to support the LMF-model’s positioning estimation. Additionally, further information on the receive scenario (e.g., TRPs or beams for a given area) may be provided to the measurement unit.

Figure 5: AI/ML assisted positioning – Case 2a: UE-assisted with UE-side model. Case 3a: NG-RAN node assisted positioning with gNB-side model (1st priority)..

Figure 6 shows that the parts of the building blocks already shown in Figure 3 now incorporate an AI/ML model. In cases 2a and 3a, [2] concluded that the model can generate new measurements, retain measurements from earlier releases, or enhance existing measurements. While introducing a new measurement could potentially address the need for higher resolution measurements, refining existing measurements through AI/ML maintains consistency with current protocols, avoiding the need for extensive changes.

Figure 6: Overview to measurement unit building blocks with UE or NG-RAN node sided model.

In [2], the LMF is identified as the entity to derive the monitoring metric at least when monitoring is based on provided ground-truth label for case 2a and case 3a. The LMF can collect monitoring metrics from the UE or gNB (e.g., predicted vs true ToA, AoA, LOS/NLOS, statistics of input data/measurements compared to the statistics of training data, etc.) and associate them with the estimated position. Additional channel information (the input data of the model, for example) can be reported to monitor at the LMF the model’s performance under given conditions.

Proposal 4: 	Consider refining existing measurements over introducing entirely new measurements to support AI/ML assisted positioning.

# LCM operations: necessary measurements, signaling/mechanism(s)

## Functionality/Model monitoring

In the previous meeting, the following agreement was made with regards to model monitoring:

Agreement For LMF-side model, RAN1 studies whether/what assistance information and/or measurement report may be sent from UE/PRU, and/or gNB to LMF to assist at least for the performance monitoring. - RAN1 understands that it is out of RAN1 scope to define monitoring metric calculation and related model management decisions for LMF-side model.

One of the core components of the LCM that enables functionality/model management decisions is the outcome of functionality/model monitoring. As identified in [2], different monitoring metrics can be applied, focusing on input or output data statistics, system performance metric(s) that can be a proxy for the AI/ML model performance or comparison with the true label (e.g., 3D position, ToA/AoA estimation, LOS/NLOS classification, etc.).

The fact is that different monitoring metrics (or analytics) come with different benefits and requirements. Some are easier to apply (e.g., monitor input data) but might not necessarily reflect a problem with the model (for example, even if input data statistics are “far” from the training data statistics, the model could still reach sufficient accuracy). Other monitoring metrics, like utilizing a ground truth label for monitoring, are far more accurate when it comes to determining an underperforming model, might however necessitate more resources (e.g., the availability of a PRU).

Even when acquiring ground truth labels for monitoring, the label validity, quality, and confidence also depends on the method used for generating the label. For example, a label reliability might differ depending on the source such as a PRU or a landmark (when available) compared to labels generated by other RAT or non-RAT methods.

A reasonable strategy here is to monitor metrics that are easy to infer locally (e.g., input data statistics) and if these indicate a potential problem with the functionality/model, a plurality of monitoring metrics can be further triggered, for determining if a problem with the model is indeed present. Depending on the monitoring quality requirements, more frequent monitoring, with more accurate monitoring metrics (e.g., ground truth labels of the highest quality/confidence) and with more detailed reporting (e.g., F1-score vs accuracy, confidence of monitoring result vs simple problem confirmation, etc.) could be configured.

Proposal 5: 	Consider a sequential monitoring process: upon identifying of a potential issue through early metrics, activate a broader set of monitoring metrics to confirm the presence of the issue.

Proposal 6: 	For functionality/model monitoring, support diverse monitoring configurations encompassing:

      - Offering options for event-based or scheduled monitoring in different time intervals, with the flexibility to adjust the monitoring frequency.

      - Providing customization for reporting detail, notifying monitoring entity on the level of detail in monitoring reports.

      - Supporting different methods of generating ground truth labels for monitoring purposes, with varying quality of labels.

## Functionality/Model management

In functionality-based LCM, after the initial functionality identification and determination of applicable functionalities, functionality management (i.e., decision on functionality activation/deactivation/switching/fallback) should be handled by the NW (LMF), due to the extensive information it has on the (radio) conditions in the area. This can be initiated by the NW-side (e.g., due to monitoring of the active and inactive functionalities) or requested by the UE (e.g., due to capability update).

Proposal 7: 	For all positioning use cases, functionality management is provided by the NW (LMF).

Since the LMF has limited or no information on the models available at the UE and the NG-RAN node in functionality-based LCM, the decision on which functionality would perform best in each case is not trivial. And for LMF-side positioning with LMF-side model, the LMF is required to manage both functionality-based LCM for aligning with the UE/NG-RAN node and model-based LCM for managing its own AI/ML models.

To achieve this, the LMF needs to have information on specific properties of the (logical or physical) models in the UE/NG-RAN node side. First and foremost, for cases that inference time / latency is an issue, the LMF should be informed on this property per model or per functionality from the UE/NG-RAN node side. Considering the dynamics within a positioning area, the effective management of AI/ML functionality shall ensure seamless operation and meet latency consideration. By considering inference time, the LMF can adaptively choose the most suitable functionality for each scenario, ensuring that latency constraints are addressed.

Proposal 8: 	Provide inference time information associated with the UE-/NG-RAN node-side AI/ML models for adaptive functionality management by the LMF.

Going one step further, the LMF needs to balance the expected performance of applying a specific functionality/model against the potential cost/overhead, especially for the UE-/gNB-side. For example, a UE can support several functionalities in Case 1 (or Case 2a), but for a given functionality it has a plurality of physical models that have a trade-off between prediction accuracy, (computational and storage) complexity and power consumption. Or it might need to download some models (or parts of models) from the NW or from the User Plane.

Proposal 9: 	In functionality-based LCM, UE/gNB provides an expected performance vs cost/overhead information to the LMF that is associated with a functionality or model. The cost/overhead information includes:

      - information on (logical or physical) model properties, such as size, computational complexity or power consumption

      - availability of the physical model on the device

      - latency for activating functionality or model.

Having all the pieces of information available, can enable the LMF to reduce the delay between functionality switching. The network could configure several functionalities at the UE/gNB (e.g., for switching to different functionality/model in different areas or radio conditions) and provide a configuration determining when to automatically switch between functionalities, based on some criteria/threshold values. As the conditions or UE capability change dynamically, the NW/LMF can update the configuration on demand.

Proposal 10: 	The NW configures several functionalities at the UE/gNB and provides a configuration for functionality switching, depending on specific parameters or thresholds. The configuration is updated on demand, based on dynamic radio conditions and UE capability.

Determining when to switch to a functionality (or model) is only part of the story. There is always the case that this switch was not the best option, due to several factors that can only become known if monitoring continues for some time interval after the switch. For example:

- The newly-activated functionality/model underperforms directly or after few inference steps and switching to a different functionality/model or fallback is required;

- There is a constant switching of functionalities/models (ping-pong effect).

- The newly-activated functionality/model has higher complexity/overhead (e.g., model size/complexity, more – and more frequent – measurements required, etc.), but the performance gain is insignificant.

Availability of such “follow-up” performance-related information is critical for assessing the quality of the functionality/model management entity and must be logged and/or reported.

Proposal 11: 	Support monitoring the events resulting from functionality/model management decisions (fallback or (de)activation/switching) for a pre-defined time window.

Proposal 12: 	Support a monitoring report including the outcome (performance, overhead/cost, robustness, etc.) of functionality or model management decisions when required.

## Model update/re-training

When a functionality or a model are indeed detected to under-perform, the central question is whether this is due to a temporal (e.g., a temporary blockage that does not require any model update), a periodic (e.g., an area changes at known or easy-to-determine time periods, which can imply that a dedicated model needs to be trained for these periods) or permanent effect (so the model needs to be updated/re-trained).

As these variations are area-specific, it makes sense that the NW (LMF) monitors for such effects and informs the incoming UEs. So, when several functionalities or models (in any positioning case) appear to have performance issues, based on data collected from several UEs, the NW (LMF) can configure a wide-range monitoring campaign, to determine if these problems have temporal, periodic or permanent causes. In any case, the NW can mark areas (and/or time-periods) where a functionality/model is expected to perform well. This information can be made available to UEs entering these areas, while data collection for model update/re-train can focus on areas that are marked as problematic for specific functionalities/models.

Proposal 13: 	Support validity indication for the AI/ML models. The indication shall include at least information about the existence of ML assisted areas.

# Necessary signaling of necessary measurement enhancements

The following agreements were reached regarding sample-based and path-based measurements and the support of phase information:

Agreement In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, RAN1 investigate the following alternatives: - Alternative (a). Sample-based measurements, where the timing information is an integer multiple of sampling periods. - Alternative (b). Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods. The issues to be studied include, but not limited to, the following: - Tradeoff of positioning accuracy and signaling overhead - Impact and necessary details of gNB/UE implementation to obtain the channel measurement values. - Whether the same Alternative(s) applies to all cases or not - Applicability and necessity of specifying the Alternative(s) to different cases - Note: different sub-cases may have different issues. Note: In addition to timing information, the components for the channel measurement for model input may also include power and potentially phase. To provide the type of the channel measurement in their investigation.

Agreement For AI/ML based positioning for all use cases, RAN1 investigate the necessity and feasibility of using phase information (in addition to timing information and power information) for determining model input. The issues to study include: - Tradeoff of positioning accuracy and signaling overhead - The impact of transmitter and receiver implementation - Specification impact - Other aspects are not precluded Note: the phase information may be used in different ways, e.g., one phase value for the first path or first sample only; triplet of {timing information, power information, phase information} for CIR, etc.

Both aspects of channel measurements are related and discussed in this section. For the trade-off analysis positioning accuracy versus signaling overhead it should be noted that the evaluations during the study phase focused on positioning of UEs assuming “random dropping”. The evaluation scenarios did not cover applications like tracking of moving UEs. The evaluation of a sequence of measurements provides a lot of additional information on the channel characteristics. Accordingly, the trade-off analysis should not be restricted to results of the simulation performed within the study phase. Maintaining the information captured by the measurements may enable advanced AI/ML models.

## Measurements of paths vs samples

The issue regarding the criteria for reporting additional paths to resolve ambiguity has been discussed in Release 17. However, there was of consensus on how the additional paths should be determined, leaving reporting of additional paths ambiguous. For example, in the FL summary in R1-2112494 identified various criteria for the selection for additional paths.

Proposal 3.1-A during Release 17 in [3]: For additional path reporting criteria support one of the following options: - Option 1: UE/TRP are configured with a power threshold for additional paths - Option 2: UE/TRP have a power threshold which is fixed in specification for additional paths - Option 3: UE/TRP reports at least the strongest path in addition to first path - Option 4: UE/TRP report additional paths when uncertain that the first path is correct - Option 5: UE/TRP reports the strongest path and the N-2 paths between first and strongest paths, if first and strongest paths are same then first N paths - Option 6: UE/TRP reports additional paths within a certain time span of the first path - Option 7: Left to UE/TRP implementation.

The drawback of the ambiguity surrounding the criteria for reporting additional paths is that it can lead to inconsistencies in the implementation of this feature across different devices and network elements. Different implementations may interpret and handle the reporting of additional paths differently. This can result in interoperability issues, degraded performance, and difficulties in comparing and analyzing positioning data.

Observation 4: 	Path-based measurements especially for cases 3b and 2b results in ambiguity. Resolving it requires defining additional criteria which introduces more standardization overhead compared to sample-based.

Figure 7 gives an example of (the magnitude of) a CIR measurement for InF_LOS:

- The red lines represent the (magnitude of) taps of the true CIR. For this scenario the CIR includes taps arriving with a small delay difference (<< TS, TS = 1/fS, fS is the sampling frequency of the signal selected according to the bandwidth)

- Blue “diamonds” () represent the (magnitude of the) correlator output with a sampling frequency of 61.44MHz

- The blue line shows the underlying (bandwidth limited) CIR represented with a sampling frequency >> fS

Figure 7: Example CIR with (true) paths (in red) and “samples” (blue diamonds)

In our contributions we use the following terminology

- A sequence of samples (vector) represents a signal, e.g., channel impulse response (CIR), with a sampling frequency fS according to the bandwidth of the signal. Hence, the distance between samples and the related time resolution depends on the bandwidth of the signal. The minimum sampling frequency is defined by the Nyquist theorem. Beside the bandwidth of the signal, the 5G numerology is taken into account for the selection of the sampling frequency.

- The time of arrival (ToA) of a path may be detected with a higher resolution than the TS with TS = 1/fS. For high accuracy positioning the ToA must be estimated with an accuracy better than TS.  Example: For a bandwidth of 50MHz the typical sampling frequency is 61.44MHz (or higher) resulting in TS=16.276ns (equivalent to 4.88m).

Figure 8: Zoom of the correlation “lobe” related to first arriving path (FAP)

Figure 8 shows details of the first “lobe” of the correlation function. The width of the lobe depends on the selected bandwidth. The measured CIR can be considered as a superposition of lobes (SINC functions) related to each tap of the true CIR. The lobes related to each tap may overlap constructive (in-phase) or destructive (out-of-phase). The shape of the resulting superposition of the lobes may differ from a lobe related to a discrete path. In the example a second path arrives with a small delay relative to the first arriving path. AI/ML based processing may derive from the shape of the lobe further information. Accordingly, AI/ML based ToA estimation (in case of AI/ML assisted positioning, for example) may perform better especially for receive scenario where several paths overlap in the bandwidth limited correlation function. This is the cases if several paths arrive with a delay difference in the range of or less than TS. Especially for the first lobe a superposition of several paths may degrade the performance of the ToA estimation. Examples for such reception scenarios are ground reflections or objects close to the UE or TRP or embedded UEs (e.g., UE mounted on or inside vehicles), where several “near reflection” can be expected. The peak of the first lobe may be no longer at the position of the first arriving path. This can be observed in Figure 8.

- The blue line depicts the output of an interpolator (re-sampling or up-sampling) used to generate a higher time resolution. This curve (“lobe shape”) can be reconstructed from the samples @ 61.44MHz using a “re-sampler”.

- The peak value (of the lobe) is app. 2.5ns after the true ToA of the first path. This would result in a distance error of 75cm.

- For the given sampling frequency fS = 61.44MHz the samples of the correlator output may not include the peak value. For the example the peak value for the samples at 61.44MHz sampling frequency has an offset of app. 5ns compared to the true time-of-arrival of the first path. The resulting distance error would be app. 1.5m.

Observation 5: 	The correlation function resulting for a bandwidth limited reference signal can be captured by (a vector of) samples spaced according to the sampling frequency selected in-line with the Nyquist theorem. From this vector of samples, the correlation function with higher time resolution can be derived.

The interpolation (resampling) represents the CIR only, if the complex valued samples are used as input. Otherwise, if the magnitude or power (“PDP”) is reported only, significant information may be lost.

Observation 6: 	Complex valued CIR reporting maintains the phase information of the CIR.

Proposal 14: 	Consider a sample based approach for the new measurement report:

- Vector(s) of samples including the CIR or PDP

- Time stamp of the first sample of the vector (or offset of the first sample of the relative to the reported ToA).

- Sampling frequency used for the vector

  - Opt 1: Sampling frequencies inline with the numerology are supported only.

  - Opt 2: The measurement unit may select the sampling frequency and reports the selected sampling frequency

## Information included in CIR (complex valued correlation function)

We identified the following AI/ML scenarios that utilize phase information as input:

- Scenario 1: Improved CIR Reconstruction for AI/ML assisted Channel Estimation

As described above a better reconstruction of the underlying true CIR becomes feasible. This may enable alternative channel estimation algorithms such as MUSIC, for example.

- Scenario 2: AI/ML Multipath Assisted Tracking

AI/ML  models can make use of phase measurements to detect changes in distance, which enables the estimation of velocity.  AI/ML can make use of the phase difference information of the LOS as well as multipath information.

- Scenario 3: Path Characterization and Phase Relationship Analysis

Characterization of path characteristics and the phase relationship between different paths.  The complex-valued CIR includes a more comprehensive understanding of multipath effects. AI/ML techniques can make use of this additional information to better estimate the LOS/NLOS state or identify channel effects like near reflections.

- Scenario 4: Phase-Based High-Resolution Positioning Algorithm

AI/ML-enhanced algorithm utilizes phase information from multipath signals to achieve high-resolution positioning and overcome limitations of conventional method.

When considering for example scenario 3, the detection of the change of the phase relationship between two paths allows, for example, the estimation of movements. Figure 9 and Figure 10 depicts the correlation function of 3 closely spaced positions (distance between two positions is just 1 millimeter). Additional details on this evaluation scenario are provided in the annex.

- As expected the magnitude is nearly identical. Fast fading effects cause a minor variation of the path power only.

- The I/Q diagram demonstrates the change of the phase of a second path is different to the phase change of the first arriving path (FAP). To highlight the different behavior the correlations are normalized to the phase of the first position.

Figure 9: Correlation functions for 3 closely spaced positions: Magnitude

Phase change of this part is different from phase change of the FAP

Figure 10: Correlation functions for 3 closely spaced positions: I/Q diagram, with phase alignment to FAP.

Based on the above example, it becomes clear that utilizing phase information by the AI/ML model in identifying path characteristics can be straightforward.

Observation 7:	Especially for moving devices the phase provides valuable additional  information to the AI/ML model. Examples are: 
- Identifying path characteristics 
- Estimation of the velocity 
- Phase assisted UE tracking

## Phase Information: Impact of frequency offsets and phase noise

The impact of frequency offset and phase noise is most noticeable in scenarios 2 and 4, whereas it is minimal in scenarios 1 and 3. In scenarios where phase-based (absolute) positioning algorithms are utilized, challenges such as phase ambiguity and frequency offset arise. However, the effects of frequency offset can be mitigated by incorporating either the phase difference itself or the change in phase difference as additional information. The difference can be calculated by the measurement unit or at the input of the AI/ML model or by the AI/ML model itself.

The first aspect of our evaluation focuses on the impact of frequency offset between UE and the TRPs. Using the same scenario and method as depicted in Figure 10, we compare the CIR applied to a signal without or with compensated frequency offset in Figure 11. Detailed analysis of the impact of frequency offset and compensation its impact is provided in the annex for further reference.

| No frequency offset | Very high frequency offset |
| --- | --- |

Figure 11: Impact of frequency offset

Moving on to another important consideration, the phase difference of arrival is equivalent to a ToA difference with an ambiguity according to the wavelength of the signal. Evaluating the phase difference CHANGE between two consecutive measurements allows the detection of small (< wavelength/2) position changes. The related phase difference change for successive measurements can be used to detect this position change with high accuracy and low latency. The underlying phase information may be also useful for AI/ML based positioning methods.

Observation 8	The phase relationship between the paths and the CHANGE of the phase relationship caused by movements is maintained in case of frequency offsets. This phase information may be used for the characterization of the path (e.g. specular or diffuse reflection) or for the detection of movements.

Observation 9:	At least for selected application examples the effects resulting from frequency offset are not relevant or can be cancelled out enabling the use of phase (or phase difference) information as additional input to the AI/ML model.

In line with the potential enhancements and applicability scenarios, and to prevent the loss of valuable information that could benefit future AI/ML models, it's crucial not to omit phase information in the reports. Therefore, we propose:

Proposal 15:	Enable the phase information as input to the AI/ML model.

## Reduction of the reporting complexity

In [2], it was observed that reporting of the CIR (complex valued reporting) may generate a higher traffic compared to PDP or DP reporting. The reporting complexity can be significantly reduced by reporting the relevant part of the vector only. In the simplest case the first arrival path is detected and only samples around this path are reported (“truncation”).

As an example (Figure 12), we pick 13 samples out of the CIR and report these values only. Taking into account the signal characteristics of the reference signal (bandwidth, used SC, etc.) the relevant parts of the CIR may be reconstructed just from this subset. Figure 12 depicts the example.

Figure 12: Truncated CIR reporting and reconstructed CIR

In Figure 13 the (magnitude of the) CIR reconstructed from the 13 samples is compared to the full (re-sampled) CIR and the reconstructed (magnitude of the) CIR in case of “magnitude only” (without truncation) reporting. It is observed that for the part covered by the “truncated CIR” can be reconstructed with minor degradation compared to the full reporting, whereas for the reconstruction from the “magnitude only” reporting a significant difference to the real lope shape is observed. Further studies (not subject of this contribution) showed that different ToA estimation algorithms, including algorithms using the frequency domain signal as input such as MUSIC, for example, still provide good accuracy with this subset.

Figure 13: Comparison of full reporting and truncated CIR reporting

It may be worthwhile to make the reporting length configurable. Alternatively, the measurement unit can select the reporting length (and/or the relevant parts of the CIR) according to the estimated CIR.

We performed a statistical analysis of the reconstruction error versus the reporting length. A reconstruction of the complex valued CIR is not possible for magnitude reporting. Accordingly, we calculated the reconstruction error for the magnitude of the CIR only. The reconstruction error is the difference between interpolated CIR using a high vector length as input and the CIR reconstructed from the truncated CIR. The error is normalized to the power. The reconstruction error was calculated in a window length according to the reporting length. The minimum window size was set to 6 samples around the estimated first arriving path (FAP).

Figure 14 compares the reconstruction error for I/Q reporting and magnitude only reporting versus reporting window length. The figure is based on the following assumption:

- AI/ML models may estimate the ToA with high accuracy (resolution << 1/fS). In this case the lope around the estimated first arriving path (FAP) should be reported with sufficient accuracy. Accordingly, a reconstruction of the CIR around the FAP should be feasible. In the figure the reconstruction error of the CIR is calculated for a length equivalent to 6 samples (window length 97.66ns for fS = 61.44MHz as used for the example, equivalent to app. 30m) even if the reporting length is shorter. The window length also defines the required accuracy of the initial estimation of the FAP.

- The reporting of other path may depend on the application or the detected PDP (power delay profile). This may be configurable or derived from the detected PDP characteristics.

- The reconstruction error was measured in the CIR part of interest, wherein the part of interest is the window length minus 2 samples or at least 6 samples.

Assumed minimum reconstruction length: 6 samples

Reporting length not sufficient for reconstruction of the first lobe

Figure 14: Reconstruction error for different reporting length

Observation 10: 	For CIR reporting the part of interest can be reconstructed with high accuracy if the reporting window length covers the part of interest plus 2 samples.

Observation 11: 	For PDP reporting the reconstruction error is much higher than for CIR reporting. This may limit the performance of algorithms/models targeting high accuracy.

In the evaluations performed within the SI a vector length of 128 or 256 was used, for example. Compared to this vector length the “truncated CIR reporting” may significantly reduce the reporting complexity (in the example by the factor 10 or more).

Observation 12: 	Using truncated CIR reporting significantly reduces the reporting complexity. This may have no or a minor impact to the model design. The model inference input can be reconstructed from the truncated CIR.

Proposal 16: 	Support truncated CIR reporting, wherein the report may include

- (Short) vector of complex samples

- Time stamp of the first sample of the vector

- Applied sampling frequency

- Estimated delay of the first path or N-paths relative to the time stamp of the first sample (this may allow to convert the report directly into a RSTD report).

## Segmentation of CIR

The truncated CIR reporting applies mainly to scenarios with low and medium delay spread or positioning methods focusing on the first arriving paths. The length of the reporting window may be selected according to the delay spread. To avoid high window length in case of late arriving paths additional segments of the CIR may be reported by “segmentation”. Instead of estimating the delay of the path a few samples around the estimated path delay can be reported. Reporting a few samples around the detected path includes information on the shape of the lobe and may allow the characterization of the path (specular reflection, diffuse reflection, distributed cluster (large objects), etc.). The model may benefit from this additional input especially for applications tracking the positions of UEs.

Figure 15 : InF-LOS CIR generated with ray-tracing

The principle of the segmentation is depicted in Figure 12. In this example we define Segment 1 for the early clusters and Segment 2 for the late clusters. Depending on the channel characteristics, later reflections with varying power levels and K-factors might be evident in other CIR examples. For Segment 1, data surrounding the earliest arrivals is of significance. These early cluster may result from single bounce reflections at near objects and may have different characteristics (e.g., specular reflection) than late arriving clusters which may undergo several reflections and may typically include several sub-paths, which can’t be distinguished. With accurate information on selected segments, the LMF can precisely reconstruct this portion of the CIR. By reporting complex-valued samples (or magnitude and phase), a full CIR reconstruction becomes feasible, ensuring acceptable accuracy including interpolating. This enables different pre-processing or post-processing methods inside the LMF. An example is the analysis of the phase relationship of paths, which may allow to detect the AoA, moving direction or type of cluster (specular reflection or diffuse scatterer). Also, LOS/NLOS detection methods may perform better with complex valued data. As for Segment 2, Power Delay Profile (PDP) details on distinct paths— as identified by the receiver— might be sufficient for certain applications.

Proposal 17: 	Support segmented CIR reporting, wherein for each segment the following data are reported

- Offset of the segment relative to the first segment

- Number of samples used for the segment

- Vector including I/Q values (or magnitude and phase) of the CIR within the segment

Proposal 18: 	Consider multiple reporting configurations that emphasize CIR/PDP information depending on the varied applications, channel conditions or requirements.

# Conclusion

Based on the discussion in the document, regarding UL SRS enhancements, the following proposal is presented:

Proposal 1: 	Consider multi-port UL SRS support for AI/ML positioning.

Regarding model monitoring, the subsequent proposals are suggested:

Proposal 2: 	Support UE-sided models to inform the network on the monitoring event that triggered a request to switch functionality or an update of the supported functionalities.

Proposal 5: 	Consider a sequential monitoring process: upon identifying of a potential issue through early metrics, activate a broader set of monitoring metrics to confirm the presence of the issue.

Proposal 6: 	For functionality/model monitoring, support diverse monitoring configurations encompassing:

      - Offering options for event-based or scheduled monitoring in different time intervals, with the flexibility to adjust the monitoring frequency.

      - Providing customization for reporting detail, notifying monitoring entity on the level of detail in monitoring reports.

      - Supporting different methods of generating ground truth labels for monitoring purposes, with varying quality of labels.

Additionally, we have proposed solutions for Functionality/model management:

Proposal 7: 	For all positioning use cases, functionality management is provided by the NW (LMF).

Proposal 8: 	Provide inference time information associated with the UE-/NG-RAN node-side AI/ML models for adaptive functionality management by the LMF.

Proposal 9: 	In functionality-based LCM, UE/gNB provides an expected performance vs cost/overhead information to the LMF that is associated with a functionality or model. The cost/overhead information includes:

      - information on (logical or physical) model properties, such as size, computational complexity or power consumption

      - availability of the physical model on the device

      - latency for activating functionality or model.

Proposal 10: 	The NW configures several functionalities at the UE/gNB and provides a configuration for functionality switching, depending on specific parameters or thresholds. The configuration is updated on demand, based on dynamic radio conditions and UE capability.

Proposal 11: 	Support monitoring the events resulting from functionality/model management decisions (fallback or (de)activation/switching) for a pre-defined time window.

Proposal 12: 	Support a monitoring report including the outcome (performance, overhead/cost, robustness, etc.) of functionality or model management decisions when required.

Proposal 13: 	Support validity indication for the AI/ML models. The indication shall include at least information about the existence of ML assisted areas.

Furthermore, we have following proposals for Model Inference related measurements:

Proposal 3: 	Define new measurement, including sample-based time domain CIR to provide model input for cases 2b and 3b.

Proposal 4: 	Consider refining existing measurements over introducing entirely new measurements to support AI/ML assisted positioning.

Proposal 14: 	Consider a sample based approach for the new measurement report:

- Vector(s) of samples including the CIR or PDP

- Time stamp of the first sample of the vector (or offset of the first sample of the relative to the reported ToA).

- Sampling frequency used for the vector

  - Opt 1: Sampling frequencies inline with the numerology are supported only.

  - Opt 2: The measurement unit may select the sampling frequency and reports the selected sampling frequency

Proposal 15:	Enable the phase information as input to the AI/ML model.

Proposal 16: 	Support truncated CIR reporting, wherein the report may include

- (Short) vector of complex samples

- Time stamp of the first sample of the vector

- Applied sampling frequency

- Estimated delay of the first path or N-paths relative to the time stamp of the first sample (this may allow to convert the report directly into a RSTD report).

Proposal 17: 	Support segmented CIR reporting, wherein for each segment the following data are reported

- Offset of the segment relative to the first segment

- Number of samples used for the segment

- Vector including I/Q values (or magnitude and phase) of the CIR within the segment

Proposal 18: 	Consider multiple reporting configurations that emphasize CIR/PDP information depending on the varied applications, channel conditions or requirements.

# References

- 3GPP RP-234039, “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, 3GPP TSG-RAN #102, Edinburg, Scotland, Dec 11th – Dec 15th, 2023.

- 3GPP TR38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”, V2.0.0 (2023-12).

- 3GPP R1-2112494, “Feature Lead Summary #3 for Potential multipath/NLOS mitigation”. e-Meeting, November 11th – 19th, 2021.

# Annex

## Detection of movements using the phase

The use of the phase of the LOS path is mainly considered in the context of CPP (Carrier phase based positioning). AI/ML models may also incorporate the phase of multipath components. Real-world models may consider moving devices and are able to derive further information from a sequence of measurements. A simple example to demonstrate the information included in the phase is given in Figure 16.

Figure 16: Evaluation scenario for phase information in multipath components (R1-2305214)

The evaluation scenario includes:

- The UE is moving in a multipath environment. In the simulation we used a short track to demonstrate the high sensitivity of the phase-based methods. Position changes in the range of millimeter can be detected.

- For the multipath environment we assume

  - Reflections on walls (modelled by a deterministic placement of the scattering clusters)

  - Ground reflections as described on TR38.901

  - Other multipath components (random positions of the related scattering clusters using the methods described in TR38.901).

The characteristics of the expected CIR are depicted in Figure 17 and Figure 18. Depending on the AoA relative to the movement direction the phase may change clockwise (distance is reduced) or counter-clockwise (distance is increased).

Figure 17: CIR example for scenario given in Figure 16

Figure 18: I/Q diagram of complex valued CIR (3 taps only)

Taking into account the limited bandwidth it may be difficult (or not possible) to distinguish between path arriving with a low delay difference. An example of the bandwidth limited CIR is depicted in the following figures. Instead of the (discrete) paths of the CIR the cyclic correlation function defined by

ci(τ)=ifft( RXω*conj( RS(ω) )

With

ci(τ)	correlation function in the time (delay) domain

RXω	received signal in the frequency domain (OFDM demodulator output)

RS(ω)	transmitted reference signal (SRS or DL-PRS) in the frequency domain

Figure 19: Correlation function for 3 positions – Magnitude vs delay

Counter clockwise change of the phase

Clockwise change of the phase

Figure 20: Correlation function for 3 positions - I/Q diagram

In the example we assume a movement by 1 millimeter per position. The distance of 1 millimeter is equivalent to a velocity of 0.1m/s and a measurement every 10ms. According to the small distance between positions a minor change in the magnitude of the correlation function is expected. In the I/Q diagram the different behavior of the different paths can be observed. If the correlation function is aligned to the phase of the first detected path (FAP) the change of the correlation function can be better highlighted. For the example this is depicted in Figure 21

Phase change of this part is different from phase change oft he FAP

Figure 21: Correlation function for 3 positions - I/Q diagram with phase alignment to FAP

## Impact of frequency offsets

If the UE and the network are not synchronized an additional phase rotation between subsequent measurements will result. The resulting effects can be canceled out. In chapter 6.2.1 we give an example for the application “Detection of the path characteristics”. The preferred method may depend on the application and may be not in the scope of the standard.

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Figure 22: Common phase rotation caused by frequency offsets

Figure 22 shows the impact of the frequency offset.

- No frequency offset. The rotation of the correlation function depends on the movement only

- A very small (-2.6Hz, equivalent to -0.00065ppm @ 4GHz) frequency offset is assumed. The additional phase rotation is in the same order of magnitude as the phase rotation resulting from the movement.

- A moderate frequency offset (-94Hz, -0.024ppm @ 4GHz) is assumed. For the example a measurement every 10ms is assumed. -94Hz results in a phase rotation of 338 degree between two measurements. An ambiguity for the detection of the phase rotation may result.

- Very high frequency offset (-5438Hz, -1.36ppm @ 4GHz, 18% of the subcarrier spacing of 30kHz). The phase rotation between two measurements covers several 360degree periods.

For the separation of the phase rotation caused by frequency offsets and movement the same method as in Figure 21 can be applied. The correlation functions of subsequent measurements are aligned to the phase of the FAP of the first measurement. It can be observed that the phase rotation resulting from the frequency offset can be canceled out and the (relative) path characteristics can be still detected.

| - No frequency offset (same as Figure 21) | - Very low frequency offset |
| --- | --- |
| - Moderate frequency offset | - High frequency offset |

Figure 23: Compensation of frequency offset effects