3GPP TSG RAN WG1 #116bis		R1-2402300

Changsha, China, April 15th – 19th, 2024

Source:	OPPO

Title:	Text Proposals on Expanded and Improved NR Positioning

Agenda Item:	8.2

Document for:	Discussion and Decision

- Introduction

In this contribution, we will provide text proposal for the Rel-18 expanded and improved NR positioning

- Text Proposal

Proposal 1: Adopt the following TP for TS38.214:

- Reason for change: (1) A few high layer parameter names are in [] now and are not aligned with TS 37.355. (2) The description of “DL carrier phase measurement” is not clear. - Summary of change: (1) Align the high layer parameter names in 214 with that in 37.355. (2) Clarify that the UE can measure DL RSCP or DL RSCPD in RRC_INACTIVE or RRC_IDLE state. - Consequences if not approved: The description of carrier phase positioning measurement is ambiguous.

TS 38.214 ##### 5.1.6.5.2 PRS for carrier phase positioning *** Unchanged parts are omitted *** If the UE reports LoS/NLoS indicator(s) via higher layer parameter nr-los-nlos-Indicator along with a measurement report containing DL RSCP or DL RSCPD, the LoS/NLoS indicator(s) are assumed to also apply to the DL RSCP or DL RSCPD measurements. The UE may be provided with nr-PRU-RSCP-MeasInfo or nr-PRU-DL-TDOA-MeasInfo which contains DL RSCP/RSCPD measurements together with DL RSTD, DL PRS-RSRP, and/or DL PRS-RSRPP measurement(s) associated with the RSCP/RSCPD measurements performed by a positioning reference unit (PRU) [20, TS 38.305] the timestamps associated with the measurements, and the location information of the PRU. The UE may be configured to report quality metrics NR-PhaseQuality corresponding to the DL RSCP and RSCPD measurements which include the following fields [17, TS 37.355]: - phaseQualityValue which provides the uncertainty of the measurement - phaseQualityResolution which specifies the resolution levels used in the [phase quality index] field. The UE in RRC_INACTIVE or RRC_IDLE mode is expected to perform the DL RSCP or DL RSCPD measurement from the bandwidth of a DL PRS resource including outside of the initial downlink bandwidth part.

- Conclusion

In this contribution, we presented text proposal for Rel-18 expanded and improved NR positioning:

Proposal 1: Adopt the above TP for TS38.214

- Reference

TS 38.214 V18.2.0