3GPP TSG-RAN WG1 Meeting #116bis                   	        R1-2402400

Changsha, Hunan Province, China, April 15th – 19th, 2024

Title:	Draft reply LS on UE Capability of Multicast Reception in RRC_INACTIVE

Response to:	R1-2401941(R2-2401661)

Release:	Rel-18

Work Item:	NR_MBS_enh-Core

Source:	RAN1

To:	RAN2

Contact Person:

Name:	Na Li

E-mail Address:	lina5g@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	No

1. Overall Description:

RAN1 would like to thank RAN2 for their LS in R1-2401941(R2-2401661) on Reply LS on UE Capability of Multicast Reception in RRC_INACTIVE.

RAN1 discussed the detailed FG for the support of intra-slot TDMed unicast/broadcast/multicast PDSCHs in RRC_INACTIVE state and defined the following components for the new FG for support of intra-slot TDMed unicast/broadcast/multicast PDSCHs in RRC_INACTIVE state.

- Support TDM between one unicast PDSCH and one group-common PDSCH for multicast in a slot.

- For any two consecutive slots n and n+1, if there are more than 1 multicast/unicast PDSCH in either slot, whether to require the minimum time separation between starting time of any two multicast/unicast PDSCHs within the duration of these slots is 4 OFDM symbol for 30kHz and 7 OFDM symbol for 60kHz

Specifically, RAN1 defined the following FGs for the support of intra-slot TDMed unicast/multicast PDSCHs in RRC_INACTIVE state.

| Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| xx-y | Dynamic scheduling for multicast in RRC_INACTIVE state | 1. Support of group-common PDCCH/PDSCH for multicast with CRC scrambled by Multicast MCCH-RNTI. 2. Support of group-common PDCCH/PDSCH for multicast with CRC scrambled by G-RNTI. 2. Support of CFR configuration for multicast. 3. Support of CORESET and common search space configuration for multicast. 4. Support of DCI format 4_0 with CRC scrambled with Multicast MCCH-RNT for multicast MCCH. 5. Support of DCI format 4_1 with CRC scrambled with G-RNTI for multicast MTCH. 6. Support one G-RNTI for multicast reception. 5. Support of inter-slot TDM between group-common PDSCH for multicast and other PDSCHs in different slots. |  |  |  | N/A | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH for multicast in RRC_INACTIVE state | 1. Support TDM between one unicast PDSCH and one group-common PDSCH for multicast in a slot. 2. For any two consecutive slots n and n+1, if there are more than 1 multicast/unicast PDSCH in either slot, whether to require the minimum time separation between starting time of any two multicast/unicast PDSCHs within the duration of these slots is 4 OFDM symbol for 30kHz and 7 OFDM symbol for 60kHz | xx-y, 5-11 and/or 5-11a and/or 5-11b |  |  | N/A | N/A | Candidate value for component 2: require the minimum time separation time {yes, no} | Optional with capability signalling |

2. Actions:

To RAN2

ACTION: 	RAN1 respectfully ask RAN2 to take above information into account.

3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #117		May 20th – 24th, 2024	      	Fukuoka, JP

TSG RAN WG1 Meeting #118	    Oct.19th – 23th, 2024	      	Maastricht, NL