3GPP TSG RAN WG1 Meeting #116bis                                                               R1-2402538

Changsha, China, April 15th - April 19th, 2024

Source:	New H3C

Title:	Discussion on Rel-19 CSI enhancements

Agenda Item:	9.2.2

Document for:	Discussion and Decision

- Introduction

In the RAN1 #116 meeting, Rel-19 CSI enhancements was discussed. Some agreements were made as below [1].

Agreement

For the Rel-19 NR Type-I/II codebook refinement and CRI-based CSI for up to 128 CSI-RS ports, reuse the EVM agreed for Rel-18 NR Type-II Doppler codebooks (cf. R1-2205289) for SLS with the following refinement:

- Center frequencies can be replaced by the following options (companies should state their assumption): 3.5GHz, 4.9GHz, 6GHz, 7GHz.

- Companies should state their assumptions on TXRU structure and the associated TXRU-to-port virtualization for >32 ports

- The UEs can be placed both indoor (with 80% probability, 3kmph) and outdoor (with 20% probability, companies should state the assumed UE speed)

- Lower RU can also be assumed (companies should state the assumed RU)

- Companies should state the assumed traffic models and ISD

Agreement

For the Rel-19 NR CJT calibration reporting, reuse the EVM agreed for Rel-18 NR Type-II CJT codebooks (cf. R1-2205289) for SLS with the following refinement:

- Adding ISD=500m as an option for inter-site CJT

- Delay offsets caused by propagation delays from the UE to the TRPs and, whenever applicable, the ISI incurred by the composite (summed across selected TRP(s)) channel impulse response should be modelled/accounted

  - In addition, initial timing offsets among TRPs can be included, e.g. from TR 38.855 for positioning or uniformly distributed [-0.5CP:0.5CP]

- Per-TRP frequency offset (relative to the UE) is modelled as a uniform random variable with [-x:x]

  - Companies should state the assumed value of x, e.g. 0.05ppm, 0.1ppm (per Table 6.5.1.2-1 of TS.38.104)

- For TDD, per-TRP DL/UL phase misalignment is modelled as a uniform random variable with [-y:y] where y is {}

- For TDD, per-TRP DL/UL timing misalignment is modelled as a uniform random variable with [-z:z]

  - Companies should state the assumed value of z, e.g. 65ns(from RAN4)

- For FDD scenarios, Rel-18 Type-II CJT codebook is assumed

- For TDD scenario, SRS-based (e.g. SVD) precoding is assumed

- Companies should state the assumed sub-carrier spacing, e.g. 15kHz, 30kHz

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, support the following:

- The UE is configured with NTRP NZP CSI-RS resources/resource sets via higher-layer (RRC) signalling where NTRP∈{1, 2, 3, 4}

  - FFS (by RAN1#116bis): Whether further restriction(s) on applicable NZP CSI-RS resources/resource sets need to be introduced (e.g. number of ports, only TRS with multiple resource sets, TD/FD locations, QCL assumptions)

- For the purpose of CJT calibration reporting, decide, by RAN1#116bis, from the following

  - Opt1:  The UE reports for all the configured NTRP NZP CSI-RS resources/resource sets

  - Opt2: The UE reports for N out of NTRP NZP CSI-RS resources/resource sets where the selection of N resources/resource sets is dynamically signalled by the NW to the UE

  - Opt3: The UE reports for N out of NTRP NZP CSI-RS resources/resource sets where the selection of N resources/resource sets is performed by the UE and included in the CSI report

- Interference measurement is not supported, hence neither CSI-IM nor NZP CSI-RS resource for interference measurement can be configured (analogous to Rel-18 TDCP)

- FFS: One-part or two-part UCI on PUSCH (analogous to Rel-18 TDCP)

- The priority of the CSI report(s) is the same as CSI report(s) not carrying L1-RSRP or L1-SINR (analogous to Rel-18 TDCP)

Agreement

For the Rel-19 Type-I codebook refinement for up to 128 CSI-RS ports, at least for RI=1-4, study and decide, by RAN1#116bis, from the following:

- Scheme1 (baseline): Adding new (N1, N2) values for the Rel-15 Type-I single-panel codebook where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources

  - FFS: Whether to further down-select between mode-1 (L=1) and mode-2 (L=4)

  - FFS: For rank-3/4, follow legacy mechanisms for <16 ports, or for >=16 ports

- Scheme2: Adding new (N1, N2) values where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources, and

  - W1 structure:

    - For each layer, reuse legacy Rel-16 eType-II SD basis with L=1 to determine the DFT-based SD basis candidates

      - FFS: Whether the indication of selected SD basis indices follows Rel-16 eType II or Rel-15 Type I

    - For 4≥RI>1, L=1 SD basis vector is independently selected for different layers

      - FFS: SD basis selection restriction to reduce SD overhead for RI>4

  - W2 structure: Layer-specific inter-polarization M-PSK co-phasing where M is further down-selected from {2, 4, 8, 16}

  - FFS: Common SD vector selection for a pair of layers (reduced total number of bits for SD basis vector selection); layer multiplexing via orthogonal polarization co-phasing for the layer pairs with common SD vector (reduced number of bits for co-phasing indication for the layer pairs with common SD vector).

  - FFS: Additional support for L>1

- Scheme2B: Adding new (N1, N2) values where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources, and

  - W1 structure:

    - For each layer, determine L=1 DFT-based SD basis candidate

      - FFS: Whether the indication of selected SD basis indices follows Rel-16 eType-II or Rel-15 Type-I

    - For 4≥RI>1, L=1 SD basis vector is independently selected for different layers

    - FFS: Common SD vector selection for a pair of layers (reduced total number of bits for SD basis vector selection), SD basis selection restriction to reduce SD overhead for RI>4

  - W2 structure:

    - Option 1: Layer-specific inter-polarization amplitude and phase scaling (single scaling coefficient per polarization)

      - FFS: WB/SB amplitude and phase reporting.

    - Option 2: Layer-specific intra-polarization (two scaling coefficients per polarization) amplitude and phase scaling.

      - FFS: WB/SB amplitude and phase reporting.

    - FFS: Rel-15 3-bit WB amplitude and M-PSK co-phasing and M is further down-selected from {2, 4, 8, 16}.

- Scheme3: Adding new (N1, N2) values where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources, and

  - W1 structure:

    - Reuse legacy Rel-16 eType-II SD basis with L>1 to determine the DFT-based SD basis candidates, and indication of SD basis indices follows Rel-16 eType-II

    - For 4≥RI>1, L>1 SD basis vectors are commonly selected across layers

      - FFS: SD basis selection restriction to reduce SD overhead for RI>4

  - W2 structure:

    - Option 1: Layer-specific sub-band SD basis selection (1 out of L) and inter-polarization M-PSK co-phasing where M is further down-selected from {2, 4, 8, 16}

    - Option 2: Layer-specific wideband SD basis linear combination and inter-polarization scaling coefficient (e.g., amplitude scaling + M-PSK co-phasing) where M is further down-selected from {2, 4, 8, 16}

- Scheme4: Using legacy Rel-15 Type-I codebook including legacy (N1, N2) values per NZP CSI-RS resource (or port group) where the PMI (associated with W1 and W2) is calculated according to

  - W1 structure: Reuse legacy Rel-15 Type-I SD basis with L=1 or L=4 for either each or some of the NZP CSI-RS resources (or port groups)

  - W2 structure: inter-NZP CSI-RS resource (or port group) co-phasing along with reusing legacy Rel-15 Type-I inter-polarization co-phasing per NZP CSI-RS resource (or port group)

    - inter-CSI-RS resource (or port group) co-phasing is used to combine the different PMIs to come up with a single precoder with >32 ports

- Scheme5: Adding new (N1, N2) values where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources, and extending the set of orthogonal beams for the selection of the second beam based on the Rel-15 Type-I single-panel codebook

  - (i1,1, i1,2) is used to refer to the 1st beam as in legacy Rel-15 Type-I

  - The 2nd beam is selected from the extended set of orthogonal beams of size: N1-1N2O2+N2-1N1O1-N1-1N2-1

  - FFS: whether to apply any restrictions to the extended orthogonal set of beams

- Scheme6: Adding new (N1, N2) values where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources, and

  - Beam(s) is(are) selected for each antenna group or NZP CSI-RS resource.

  - Inter-group (or CSI-RS resource) co-phasing along with inter-polarization co-phasing per group (or CSI-RS resource) are used to combine different beam(s), FFS using scalar quantization or vector quantization for the co-phasings

FFS (by RAN1#116bis): Down-select (O1, O2) value between (2,2) and (4,4), whether (O1, O2) and/or (q1, q2) is layer-common or layer-specific

FFS (by RAN1#116bis): Whether extension of Rel-15 Type-I MP codebook for Rel-19 Type-I is also supported

FFS (by RAN1#116bis): Whether to introduce larger L values (e.g. 6, 8, 10)

FFS: Whether to refine CBSR design to reduce RRC overhead

This contribution continue to discuss about Rel-19 CSI enhancements.

- Discussion

2.1 Type I/II

In terms of discussion in RAN#116 meeting[1], there are 7 options on Rel-19 Type-I codebook as follows:

Agreement For the Rel-19 Type-I codebook refinement for up to 128 CSI-RS ports, at least for RI=1-4, study and decide, by RAN1#116bis, from the following: - Scheme1 (baseline): Adding new (N1, N2) values for the Rel-15 Type-I single-panel codebook where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources - FFS: Whether to further down-select between mode-1 (L=1) and mode-2 (L=4) - FFS: For rank-3/4, follow legacy mechanisms for <16 ports, or for >=16 ports - Scheme2: Adding new (N1, N2) values where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources, and - W1 structure: - For each layer, reuse legacy Rel-16 eType-II SD basis with L=1 to determine the DFT-based SD basis candidates - FFS: Whether the indication of selected SD basis indices follows Rel-16 eType II or Rel-15 Type I - For 4≥RI>1, L=1 SD basis vector is independently selected for different layers - FFS: SD basis selection restriction to reduce SD overhead for RI>4 - W2 structure: Layer-specific inter-polarization M-PSK co-phasing where M is further down-selected from {2, 4, 8, 16} - FFS: Common SD vector selection for a pair of layers (reduced total number of bits for SD basis vector selection); layer multiplexing via orthogonal polarization co-phasing for the layer pairs with common SD vector (reduced number of bits for co-phasing indication for the layer pairs with common SD vector). - FFS: Additional support for L>1 - Scheme2B: Adding new (N1, N2) values where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources, and - W1 structure: - For each layer, determine L=1 DFT-based SD basis candidate - FFS: Whether the indication of selected SD basis indices follows Rel-16 eType-II or Rel-15 Type-I - For 4≥RI>1, L=1 SD basis vector is independently selected for different layers - FFS: Common SD vector selection for a pair of layers (reduced total number of bits for SD basis vector selection), SD basis selection restriction to reduce SD overhead for RI>4 - W2 structure: - Option 1: Layer-specific inter-polarization amplitude and phase scaling (single scaling coefficient per polarization) - FFS: WB/SB amplitude and phase reporting. - Option 2: Layer-specific intra-polarization (two scaling coefficients per polarization) amplitude and phase scaling. - FFS: WB/SB amplitude and phase reporting. - FFS: Rel-15 3-bit WB amplitude and M-PSK co-phasing and M is further down-selected from {2, 4, 8, 16}. - Scheme3: Adding new (N1, N2) values where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources, and - W1 structure: - Reuse legacy Rel-16 eType-II SD basis with L>1 to determine the DFT-based SD basis candidates, and indication of SD basis indices follows Rel-16 eType-II - For 4≥RI>1, L>1 SD basis vectors are commonly selected across layers - FFS: SD basis selection restriction to reduce SD overhead for RI>4 - W2 structure: - Option 1: Layer-specific sub-band SD basis selection (1 out of L) and inter-polarization M-PSK co-phasing where M is further down-selected from {2, 4, 8, 16} - Option 2: Layer-specific wideband SD basis linear combination and inter-polarization scaling coefficient (e.g., amplitude scaling + M-PSK co-phasing) where M is further down-selected from {2, 4, 8, 16} - Scheme4: Using legacy Rel-15 Type-I codebook including legacy (N1, N2) values per NZP CSI-RS resource (or port group) where the PMI (associated with W1 and W2) is calculated according to - W1 structure: Reuse legacy Rel-15 Type-I SD basis with L=1 or L=4 for either each or some of the NZP CSI-RS resources (or port groups) - W2 structure: inter-NZP CSI-RS resource (or port group) co-phasing along with reusing legacy Rel-15 Type-I inter-polarization co-phasing per NZP CSI-RS resource (or port group) - inter-CSI-RS resource (or port group) co-phasing is used to combine the different PMIs to come up with a single precoder with >32 ports - Scheme5: Adding new (N1, N2) values where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources, and extending the set of orthogonal beams for the selection of the second beam based on the Rel-15 Type-I single-panel codebook - (i1,1, i1,2) is used to refer to the 1st beam as in legacy Rel-15 Type-I - The 2nd beam is selected from the extended set of orthogonal beams of size: N1-1N2O2+N2-1N1O1-N1-1N2-1 - FFS: whether to apply any restrictions to the extended orthogonal set of beams - Scheme6: Adding new (N1, N2) values where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources, and - Beam(s) is(are) selected for each antenna group or NZP CSI-RS resource. - Inter-group (or CSI-RS resource) co-phasing along with inter-polarization co-phasing per group (or CSI-RS resource) are used to combine different beam(s), FFS using scalar quantization or vector quantization for the co-phasings FFS (by RAN1#116bis): Down-select (O1, O2) value between (2,2) and (4,4), whether (O1, O2) and/or (q1, q2) is layer-common or layer-specific FFS (by RAN1#116bis): Whether extension of Rel-15 Type-I MP codebook for Rel-19 Type-I is also supported FFS (by RAN1#116bis): Whether to introduce larger L values (e.g. 6, 8, 10) FFS: Whether to refine CBSR design to reduce RRC overhead

Scheme 1 can be considered as a basic mode because it provides considerable gain with low feedback overhead.   Evaluation results indicate that Scheme 2 can provide additional gains and hence Scheme 2 can be considered an advanced mode in Rel-19.

Proposal 1: For the Rel-19 Type-I single-panel (SP) codebook refinement for up to 128 CSI-RS ports, for RI=1-4, support the following:

- Scheme1: Adding new (N1, N2) values for the Rel-15 Type-I single-panel codebook mode-1 (L=1) where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources, and for rank-3/4, follow legacy mechanisms for <16 ports

- Scheme2: Adding new (N1, N2) values where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources, and

  - W1 structure:

    - For each layer, reuse legacy Rel-16 eType-II SD basis with L=1 to determine the DFT-based SD basis candidates

    - For 1<RI≤4, L=1 SD basis vector is independently selected for different layers

      - The SD basis selection indication includes layer-common (q1,q2) and log2N1N2 bits for each layer

      - Note: This implies that each of the SD basis vectors is selected from a group of N1N2 orthogonal basis vectors

  - W2 structure: Layer-specific inter-polarization co-phasing with the alphabet {+1, +j, -1, -j}

For the Rel-19 Type-I single-panel (SP) codebook refinement for up to 128 CSI-RS ports，we fail to see any benefit of O1=O2=2 value.

Proposal 2: For the Rel-19 Type-I single-panel (SP) codebook refinement for up to 128 CSI-RS ports，(O1, O2) value = (4,4) should be supported.

Proposal 3: For the Rel-19 Type-I single-panel (SP) codebook refinement for up to 128 CSI-RS ports，if (O1, O2) value = (4,4)  can’t be agreed, (O1, O2) value  can be configured between (2,2) and (4,4).

2.2 CRI-based CSI refinement

In terms of discussion in RAN#116 meeting[1], there are Rel-19 CRI-based CSI refinement  agreement as follows:

Agreement For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, in accordance to the WID, extend the Rel-15 CRI-based CSI reporting as follows: - A UE is configured to measure KS>1 NZP CSI-RS resources with equal number of ports, with up to 32 ports per NZP CSI-RS resource - Note: The maximum number of ports per NZP CSI-RS resource for a given value of KS will be discussed separately - Containing the information of M “quadruplets” {(CRIn, RIn, PMIn, CQIn), n=0, …, M–1} in one CSI reporting instance where the value range of M (≤KS) is {1, …, min(X, KS)} - FFS (by RAN1# 116bis): The supported value(s) of X (candidates are 2, 4, 6, KS) - FFS (by RAN1# 116bis): Whether the value of M is NW-configured via higher-layer (RRC) signalling, or UE-selected (as a part of CSI report), or a combination of the two - A same legacy codebook (with up to 32 ports) is configured for (associated with) all M “quadruplets” FFS: detailed UCI design/optimization (e.g. overhead reduction) FFS: Whether solution to allow CSI reporting for larger number of CSI-RS resources across multiple CSI reports is supported FFS: whether further restriction(s) on CMR configuration is needed, including relation with IMR FFS: the packing order of the information of M “quadruplets”, CSI omission rule FFS: Whether all the K CSI-RS resources are associated with a same CSI-RS resource set or not FFS: Whether KS, maximum # ports per resource, and X depend on codebook type

We support RRC-configured M.

For Rel-15 SP Type I, we support X=4.

For Rel-16 eType II, X=1 can at least be supported.

Proposal 4: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, the value of M is NW-configured.

Proposal 5: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, X=4 is supported for Rel-15 SP Type I.

Proposal 6: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, X=1 can at least be supported for Rel-16 eType II.

2.3 CJT Calibration

In terms of discussion in RAN#116 meeting[1], there are Rel-19 CJT calibration reporting agreement as follows:

Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, support the following: - The UE is configured with NTRP NZP CSI-RS resources/resource sets via higher-layer (RRC) signalling where NTRP∈{1, 2, 3, 4} - FFS (by RAN1#116bis): Whether further restriction(s) on applicable NZP CSI-RS resources/resource sets need to be introduced (e.g. number of ports, only TRS with multiple resource sets, TD/FD locations, QCL assumptions) - For the purpose of CJT calibration reporting, decide, by RAN1#116bis, from the following - Opt1: The UE reports for all the configured NTRP NZP CSI-RS resources/resource sets - Opt2: The UE reports for N out of NTRP NZP CSI-RS resources/resource sets where the selection of N resources/resource sets is dynamically signalled by the NW to the UE - Opt3: The UE reports for N out of NTRP NZP CSI-RS resources/resource sets where the selection of N resources/resource sets is performed by the UE and included in the CSI report - Interference measurement is not supported, hence neither CSI-IM nor NZP CSI-RS resource for interference measurement can be configured (analogous to Rel-18 TDCP) - FFS: One-part or two-part UCI on PUSCH (analogous to Rel-18 TDCP) - The priority of the CSI report(s) is the same as CSI report(s) not carrying L1-RSRP or L1-SINR (analogous to Rel-18 TDCP) Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, given the NTRP configured NZP CSI-RS resources/resource sets and the selected N resources/resource sets, study and decide, by RAN1#116bis, whether to support reporting, in one CSI reporting instance, {n,m n=0, 1, …, N – 1, n≠nref, m=0,1,…,M-1}, where n,m denotes the measured phase offset between the n-th CSI-RS resource/resource set and the reference CSI-RS resource/resource set/ nref for the m-th frequency unit - FFS: whether M>1 (sub-band reporting) is needed or not (M=1, i.e. wideband reporting) - For the reference CSI-RS resource/resource set nref, the value of nref is assumed 0 and not reported - FFS (by RAN1#116bis): Whether nref is fixed, NW-configured, or is included in the report (selected by the UE) - The value n,m indicates a uniformly quantized phase between –A and A, or 0 and A - FFS (by RAN1#116bis): supported quantization alphabet(s) (including A and resolution) for n,m - FFS: Detailed UCI design

For the purpose of CJT calibration reporting， we prefer Opt.1 because N = NTRP would not increase much reporting overhead.

Proposal7: For the Rel-19 aperiodic standalone CJT calibration reporting, the UE reports for all the configured NTRP NZP CSI-RS resources/resource sets

For the Rel-19 aperiodic standalone CJT calibration reporting, nref, can be is selected by the UE and reported as a part of the CJT calibration report.

Proposal 8: For the Rel-19 aperiodic standalone CJT calibration reporting, nref is selected by the UE and reported as a part of the CJT calibration report

For the Rel-19 aperiodic standalone CJT calibration reporting, M=1 should be at least supported.

Proposal 9: For the Rel-19 aperiodic standalone CJT calibration reporting, given the NTRP configured NZP CSI-RS resources/resource sets and the selected N resources/resource sets, support reporting, in one CSI reporting instance, {n,m n=0, 1, …, N – 1, n≠nref, m=0,1,…,M-1}, where n,m denotes the measured phase offset between the n-th CSI-RS resource/resource set and the reference CSI-RS resource/resource set/ nref for the m-th frequency unit

- M=1 (i.e. wideband reporting) is at least supported

- Conclusion

In this contribution, we discussed about Rel-19 CSI enhancements. The proposals are as follows:

Proposal 1: For the Rel-19 Type-I single-panel (SP) codebook refinement for up to 128 CSI-RS ports, for RI=1-4, support the following:

- Scheme1: Adding new (N1, N2) values for the Rel-15 Type-I single-panel codebook mode-1 (L=1) where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources, and for rank-3/4, follow legacy mechanisms for <16 ports

- Scheme2: Adding new (N1, N2) values where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources, and

  - W1 structure:

    - For each layer, reuse legacy Rel-16 eType-II SD basis with L=1 to determine the DFT-based SD basis candidates

    - For 1<RI≤4, L=1 SD basis vector is independently selected for different layers

      - The SD basis selection indication includes layer-common (q1,q2) and log2N1N2 bits for each layer

      - Note: This implies that each of the SD basis vectors is selected from a group of N1N2 orthogonal basis vectors

  - W2 structure: Layer-specific inter-polarization co-phasing with the alphabet {+1, +j, -1, -j}

Proposal 2: For the Rel-19 Type-I single-panel (SP) codebook refinement for up to 128 CSI-RS ports，(O1, O2) value = (4,4) should be supported.

Proposal 3: For the Rel-19 Type-I single-panel (SP) codebook refinement for up to 128 CSI-RS ports，if (O1, O2) value = (4,4)  can’t be agreed, (O1, O2) value  can be configured between (2,2) and (4,4).

Proposal 4: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, the value of M is NW-configured.

Proposal 5: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, X=4 is supported for Rel-15 SP Type I.

Proposal 6: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, X=1 can at least be supported for Rel-16 eType II.

Proposal7: For the Rel-19 aperiodic standalone CJT calibration reporting, the UE reports for all the configured NTRP NZP CSI-RS resources/resource sets

Proposal 8: For the Rel-19 aperiodic standalone CJT calibration reporting, nref is selected by the UE and reported as a part of the CJT calibration report

Proposal 9: For the Rel-19 aperiodic standalone CJT calibration reporting, given the NTRP configured NZP CSI-RS resources/resource sets and the selected N resources/resource sets, support reporting, in one CSI reporting instance, {n,m n=0, 1, …, N – 1, n≠nref, m=0,1,…,M-1}, where n,m denotes the measured phase offset between the n-th CSI-RS resource/resource set and the reference CSI-RS resource/resource set/ nref for the m-th frequency unit

- M=1 (i.e. wideband reporting) is at least supported

- Reference

- Chairman’s Notes, RAN1#116, final, February 26th – March 1st, 2024