3GPP TSG RAN WG1 #116-bis	R1-2403030

Changsha, Hunan Province, China, April 15th – 19th, 2024

Agenda item:	9.11.2

Source: 	ETRI

Title:	Discussion on HD UEs with NR NTN

Document for:	Discussion/Decision

Introduction

This contribution presents ETRI’s views on RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands based on the agreements in Table 1.

Table 1. Agreements on support of NTN HD UEs [1].

Agreement Study at least the following scenarios for (e)RedCap HD-FDD UEs for NTN: - Whether existing handling rules for the following cases should be reused or updated when taking into account TA mismatch between actual TA used by UE and assumed TA at the gNB based on available TA report: - Case 1: Dynamically scheduled DL reception collides with semi-statically configured UL transmission - Case 2: Semi-statically configured DL reception collides with dynamically scheduled UL transmission - Case 3: Semi-statically configured DL reception collides with semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Case 5: Configured SSB collides with dynamically scheduled or configured UL transmission - Case 6: Dynamic or semi-static DL collides with valid RO - Case 7: Collision due to direction switching - At least the following potential issues can be further considered for (e)RedCap HD-FDD UEs - Error cases in case 3 and case 4 - SIB19 reception collides with UL transmission - Slot counting for UL repetition transmission colliding with SSB reception - Invalid symbol determination for PUSCH repetition type B - Actual TDW determination due to the collision between DL reception and UL transmission with DMRS bundling - CPU occupation due to omitted DL reception or UL transmission Note: Both GSO and Non-GSO should be considered.

Discussion

UE-specific TA report

As in Table 2, a UE can report the least integer number of slots for the timing advance using subcarrier spacing of 15 kHz (i.e., with 1 msec reporting granularity). Therefore, when the actual length of uplink slot or downlink slot is less than 1 msec, then the network may have a certain level of ambiguity on the time reference for the half-duplex (HD) prioritization rule given that the reported TA value may represent multiple uplink slots rather than a single fixed slot.

Observation 1. Due to the 1 msec granularity of TA report MAC CE for NTN, the network may have a certain level of ambiguity on the time reference for the half-duplex (HD) prioritization rule given that the reported TA value may represent multiple uplink slots rather than a single fixed slot.

Table 2. Timing advance report MAC CE in TS 38.321 [2]

#### 6.1.3.56 Timing Advance Report MAC CE The Timing Advance Report MAC CE is identified by MAC subheader with LCID as specified in Table 6.2.1-2. It has a fixed size and consists of two octets defined as follows (Figure 6.1.3.56-1): - R: Reserved bit, set to 0; - Timing Advance: In FR1 except for ATG, the Timing Advance field indicates the least integer number of slots for NTN or symbols for ATG, using subcarrier spacing of 15 kHz for NTN and either 15 kHz or 30 kHz for ATG, greater than or equal to the Timing Advance value (see TS 38.211 [8], clause 4.3.1). For ATG, the Timing Advance field indicates the least integer number of symbols greater than or equal to the Timing Advance value (see TS 38.211 [8], clause 4.3.1). The symbol duration is based on the subcarriers spacing the UE is currently configured with. In this release of the specification, only 15 kHz and 30 kHz SCS are applicable and only values 1 … 56 are used. The length of the field is 14 bits. Figure 6.1.3.56-1: Timing Advance Report MAC CE

Collision between higher-layer configured DL reception and higher-layer configured UL transmission

As clarified by the yellow highlighted part in Table 3, the network should avoid collision between higher-layer configured DL reception and higher-layer configured UL transmission by its implementation. Compared with TN where this network implementation-based solution works well, it seems that some of the NTN environments enforces extremely high time/frequency resource overhead, if we only rely on the implementation-based solution to avoid the collision. For instance, given that the distance between satellite and UE may vary from 500km to 1500km according to the elevation angle to the LEO satellite in 500km altitude, the network implementation-based solution may require (implicit) guard period between the configured DL reception and the configured UL transmission, which is longer than several msec. In this regard, we think a new HD prioritization rule is needed for collision between higher-layer configured DL reception and higher-layer configured UL transmission in case of NTN.

Table 3. Specification support for HD UE [3]

A HD-UE does not expect to receive both dedicated higher layer parameters configuring transmission in a set of symbols and dedicated higher layer parameters configuring reception in the set of symbols. A HD-UE does not expect to receive both a Type-0/0A/0B/1/2-PDCCH CSS set configuration for PDCCH reception in a set of symbols and dedicated higher layer parameters configuring transmission in the set of symbols, except Type-2-PDCCH CSS set configuration for PDCCH reception in a set of symbols and configured-grant based PUSCH transmission as described in clause 19.1 in the set of symbols for which case the UE follows the procedure as in clause 5.1B.2.6 in [10, TS 38.133]. The UE expects to be configured with a Type-2-PDCCH CSS set configuration for PDCCH reception such that there is at least one paging occasion that does not overlap with configured-grant based PUSCH transmission as described in clause 19.1 per SI modification period.

Observation 2. A new HD prioritization rule is needed for collision between higher-layer configured DL reception and higher-layer configured UL transmission in case of NTN.

- Otherwise, GP over multiple slots is required for every interval between the configured DL and configured UL.

Collision between PDCCH CSS and UL transmission configured by dedicated higher layer signaling

As clarified by the cyan highlighted part in Table 3, the network should avoid collision between PDCCH CSS monitoring and higher-layer configured UL transmission by its implementation. Since the same analysis from section 2.2 can be applied in this section, some of the NTN environments enforces extremely high time/frequency resource overhead, if the existing implementation-based solution is available.

Observation 3. A new HD prioritization rule is needed for collision between PDCCH CSS monitoring and higher-layer configured UL transmission in case of NTN.

- Otherwise, GP over multiple slots is required for every interval between the PDCCH CSS and configured UL.

CPU occupation with omitted CMR/IMR due to the HD prioritization rules

A UE reports its capability on the number of CSI processing units (CPUs) to the network according to the green highlighted parts in Table 4 and Table 5. It is noted that the reported number of CPUs in a CC can be “one” per the higher layer parameter simultaneousCSI-ReportsPerCC as captured in Table 5. Therefore, any impact on CPU occupation could be critical for such UEs with a single CPU, which is a practical assumption for RedCap or eRedCap UEs.

Table 4 shows definitions of start time of CPU occupation (yellow highlighted part in Table 4) and end time of CPU occupation (cyan highlighted part in Table 4). One or more receptions, measurements, or monitoring on the CSI-RS/CSI-IM/PDCCH can be omitted by HD prioritization rule and they cause the following issues to NTN HD UE:

- Ambiguous start time of CPU occupation.

- Large margin (longer than CSI-RS/CSI-IM period or PDCCH monitoring period, at least) on CPU occupation from network side.

- Misunderstanding on the omitted CSI between network and UE.

Above issues are critical to NTN HD UE given that the CSI-RS/CSI-IM period or PDCCH monitoring period in NTN can be larger than that for TN scenarios, in general.

Table 4. Specification support on CPU occupation [4].

The UE indicates the number of supported simultaneous CSI calculations NCPU with parameter simultaneousCSI-ReportsPerCC in a component carrier, and simultaneousCSI-ReportsAllCC across all component carriers. If a UE supports NCPU simultaneous CSI calculations it is said to have NCPU CSI processing units for processing CSI reports. … For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity not set to 'none', the CPU(s) are occupied for a number of OFDM symbols as follows: - A periodic or semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report and a semi-persistent CSI report on PUSCH configured with the higher layer parameter codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18') occupies CPU(s) from the first symbol of the earliest one of each CSI-RS/CSI-IM/SSB resource, or each CSI-RS/CSI-IM resource associated with all configured sub-configurations for periodic CSI report corresponding to a CSI-ReportConfig that contains a list of sub-configurations provided by csi-ReportSubConfigList, or each CSI-RS/CSI-IM resource associated with all triggered sub-configurations for semi-persistent CSI report corresponding to a CSI-ReportConfig that contains a list of sub-configurations provided by csi-ReportSubConfigList, for channel or interference measurement, respective latest CSI-RS/CSI-IM/SSB occasion no later than the corresponding CSI reference resource, until the last symbol of the configured PUSCH/PUCCH carrying the report. - An aperiodic CSI report occupies CPU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining the CPU occupation duration, the PDCCH candidate that ends later in time is used. - An initial semi-persistent CSI report on PUSCH after the PDCCH trigger occupies CPU(s) from the first symbol after the PDCCH until the last symbol of the scheduled PUSCH carrying the report. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining the CPU occupation duration, the PDCCH candidate that ends later in time is used. - A semi-persistent CSI report on PUSCH configured with the higher layer parameter codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18' occupies CPU(s) from the first symbol of KP-th latest consecutive periodic/semi-persistent CSI-RS occasions no later than CSI reference resource, until the last symbol of the PUSCH carrying the report, where the value of KP∈{1,2,4} is indicated by UE capability.

Table 5. Specification support on CPU occupation [5].

CSI-ReportFramework ::= SEQUENCE { maxNumberPeriodicCSI-PerBWP-ForCSI-Report INTEGER (1..4), maxNumberAperiodicCSI-PerBWP-ForCSI-Report INTEGER (1..4), maxNumberSemiPersistentCSI-PerBWP-ForCSI-Report INTEGER (0..4), maxNumberPeriodicCSI-PerBWP-ForBeamReport INTEGER (1..4), maxNumberAperiodicCSI-PerBWP-ForBeamReport INTEGER (1..4), maxNumberAperiodicCSI-triggeringStatePerCC ENUMERATED {n3, n7, n15, n31, n63, n128}, maxNumberSemiPersistentCSI-PerBWP-ForBeamReport INTEGER (0..4), simultaneousCSI-ReportsPerCC INTEGER (1..8) } CA-ParametersNR-v1540 ::= SEQUENCE { simultaneousSRS-AssocCSI-RS-AllCC INTEGER (5..32) OPTIONAL, csi-RS-IM-ReceptionForFeedbackPerBandComb SEQUENCE { maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC INTEGER (1..64) OPTIONAL, totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC INTEGER (2..256) OPTIONAL } OPTIONAL, simultaneousCSI-ReportsAllCC INTEGER (5..32) OPTIONAL, dualPA-Architecture ENUMERATED {supported} OPTIONAL }

It is also noted that the available/desirable periodicity of CMR and IMR can be quite large value (e.g., 360 or 640 msec by [5]) and the effective/actual periodicity of valid CMR/IMR could be multiple of those (i.e., several seconds or tens of second).

Observation 4. One or more receptions, measurements, or monitoring on the CSI-RS/CSI-IM/PDCCH can be omitted by HD prioritization rule and they cause the following issues to NTN HD UE:

- Ambiguous start time of CPU occupation.

- Large margin (longer than CSI-RS/CSI-IM/PDCCH monitoring period, i.e., multiples of 360 msec or 640 msec) on CPU occupation from network side.

- Misunderstanding on the omitted CSI between network and UE.

CPU occupation with omitted PUCCH/PUSCH due to the HD prioritization rules

Table 4 shows definitions of start time of CPU occupation (yellow highlighted part in Table 4) and end time of CPU occupation (cyan highlighted part in Table 4). One or more PUCCH/PUSCH transmissions can be omitted by HD prioritization rule and they cause the following issues to NTN HD UE:

- Ambiguous end time of CPU occupation.

- Large margin (longer than PUCCH period, at least) on CPU occupation from network side or requires an additional CSI report triggering in case of PUSCH-based CSI report.

- Misunderstanding on the omitted CSI between network and UE.

Above issues are critical to NTN HD UE given that the CSI report periodicity in NTN can be larger than that for TN scenarios, in general.

Observation 5. One or more PUCCH/PUSCH transmissions can be omitted by HD prioritization rule and they cause the following issues to NTN HD UE:

- Ambiguous start time of CPU occupation.

- Large margin (longer than PUCCH period, i.e., multiples of 360 msec or 640 msec) on CPU occupation from network side or requires an additional CSI report triggering in case of PUSCH-based CSI report.

- Misunderstanding on the omitted CSI between network and UE.

Figure 1. Examples of missing CSI report due to HD prioritization rules.

Figure 1 shows examples of missing CSI report due to HD prioritization rules. In Figure 1, the orange line represents slant range (between LEO 600km and a UE with 1.5m height) and the corresponding propagation delay while the initial elevation angle is 30o at 0 second. Similarly, the blue line depicts slant range (between LEO 1200km and a UE with 1.5m height) and the corresponding propagation delay while the initial elevation angle is 30o at 0 second. In Figure 1, the black horizontal lines stand for potential boundaries of slots, i.e., the gap between two adjacent black lines is 1 msec. For instance, we can assume that the network allocates proper DL receptions and UL transmissions for LEO 1200 with HD UEs, which secure ZERO omission on CMR/IMR measurement or CSI report (marked as HD collision pattern #1 in Figure 1). However, this does NOT mean that such stainless collision handling will be automatically achieved, since there can be different HD collision cases (HD collision pattern #2 in Figure 1) due to the propagation delay variation. Given that such duration (HD collision pattern #2 in Figure 1) can be a quite long time period (e.g., tens of second), it is desirable to resolve the issue from missing CMR/IMR measurement due to HD prioritization rules. For example, the following could be clarified for CSI measurement and report of NTN HD UE:

- If one or more of the CSI-RS/CSI-IM/SSB occasions are omitted for NTN HD UE, the corresponding CSI report shall be omitted and the corresponding CPU occupation shall be released.

Proposal 1. RAN1 to clarify the following for CSI measurement and report of NTN HD UE:

- If one or more of the CSI-RS/CSI-IM/SSB occasions are omitted for NTN HD UE, the corresponding CSI report shall be omitted and the corresponding CPU occupation shall be released.

It is noted that the mean and standard deviation values of basic pathloss for NTN UE can vary significantly within a dwell time, as captured by Table 6 (larger than 20dB gap between elevation angles of 30o and 90o, which can be converted into multiple steps of CQI/MCS indexes).

Table 6. Basic pathloss (Eq. 6.6-4 in [6]) for LEO 600km

|  | Elevation angle | Slant range | ΕPLb | σPLb |
| --- | --- | --- | --- | --- |
| LEO 600 | 30o | 1075 km | 176.56 dB | 17.25 |
|  | 90o | 600 km | 154.52 dB | 3.90 |

HARQ-disabled PDSCH for HD UEs

Table 7 shows the existing specification support on HARQ-disabled PDSCH for NTN in TS 38.214 [4]. As in Table 7, when HARQ feedback for the HARQ process ID is disabled, the network shall guarantee a gap of larger than Tproc,1 between one PDSCH and the other PDSCH for that HARQ process. In case of HD UE in NTN, however, there could be an exception that reception of the former PDSCH is omitted due to the HD priority rule. Since Tproc,1 can span multiple slots, we think it should be clarified that the current scheduling gap of Tproc,1 between one PDSCH and the other PDSCH for HARQ-disabled HARQ process is not applied when the reception of the former PDSCH is omitted due to the HD priority rule.

Table 7. Specification support on HARQ-disabled PDSCH for NTN [4].

When HARQ feedback for the HARQ process ID is disabled, the UE is not expected to receive another PDCCH carrying a DCI scheduling a PDSCH or set of slot-aggregated PDSCH scheduled for the given HARQ process or to receive another PDSCH without corresponding PDCCH for the given HARQ process that starts until Tproc,1 after the end of the reception of the last PDSCH or slot-aggregated PDSCH for that HARQ process.

Observation 6. The current scheduling gap of Tproc,1 between one PDSCH and the other PDSCH for HARQ-disabled HARQ process is not needed, when the reception of the former PDSCH is omitted due to the HD priority rule.

Based on the observations above, we draw the following proposal.

Proposal 2. Send an LS to RAN to inform that specification supports are required for NTN HD UEs at least in the following scenarios:

- Ambiguous location of DL-UL colliding slot(s) due to the 1 ms granularity of UE-specific TA reporting value.

- High time/frequency resource overhead to avoid collisions between DL reception and UL transmission, which are configured by dedicated higher layer signaling, respectively

- High time/frequency resource overhead to avoid collisions between PDCCH CSS and UL transmission configured by dedicated higher layer signaling

- Clarification on the CPU occupation for a CSI report, while the reception of corresponding CMR and/or IMR is dropped due to the HD prioritization rule

- Clarification on the CPU occupation for a CSI report, while the transmission of corresponding PUCCH or PUSCH is dropped due to the HD prioritization rule

- Unnecessary scheduling restriction for HARQ-disabled PDSCH, while reception of the PDSCH is dropped due to the HD prioritization rule

Conclusion

In this contribution, ETRI’s views on NTN HD UEs were shown and the following observations and proposals were made:

Observation 1. Due to the 1 msec granularity of TA report MAC CE for NTN, the network may have a certain level of ambiguity on the time reference for the half-duplex (HD) prioritization rule given that the reported TA value may represent multiple uplink slots rather than a single fixed slot.

Observation 2. A new HD prioritization rule is needed for collision between higher-layer configured DL reception and higher-layer configured UL transmission in case of NTN.

- Otherwise, GP over multiple slots is required for every interval between the configured DL and configured UL.

Observation 3. A new HD prioritization rule is needed for collision between PDCCH CSS monitoring and higher-layer configured UL transmission in case of NTN.

- Otherwise, GP over multiple slots is required for every interval between the PDCCH CSS and configured UL.

Observation 4. One or more receptions, measurements, or monitoring on the CSI-RS/CSI-IM/PDCCH can be omitted by HD prioritization rule and they cause the following issues to NTN HD UE:

- Ambiguous start time of CPU occupation.

- Large margin (longer than CSI-RS/CSI-IM/PDCCH monitoring period, i.e., multiples of 360 msec or 640 msec) on CPU occupation from network side.

- Misunderstanding on the omitted CSI between network and UE.

Observation 5. One or more PUCCH/PUSCH transmissions can be omitted by HD prioritization rule and they cause the following issues to NTN HD UE:

- Ambiguous start time of CPU occupation.

- Large margin (longer than PUCCH period, i.e., multiples of 360 msec or 640 msec) on CPU occupation from network side or requires an additional CSI report triggering in case of PUSCH-based CSI report.

- Misunderstanding on the omitted CSI between network and UE.

Observation 6. The current scheduling gap of Tproc,1 between one PDSCH and the other PDSCH for HARQ-disabled HARQ process is not needed, when the reception of the former PDSCH is omitted due to the HD priority rule.

Proposal 1. RAN1 to clarify the following for CSI measurement and report of NTN HD UE:

- If one or more of the CSI-RS/CSI-IM/SSB occasions are omitted for NTN HD UE, the corresponding CSI report shall be omitted and the corresponding CPU occupation shall be released.

Proposal 2. Send an LS to RAN to inform that specification supports are required for NTN HD UEs at least in the following scenarios:

- Ambiguous location of DL-UL colliding slot(s) due to the 1 ms granularity of UE-specific TA reporting value.

- High time/frequency resource overhead to avoid collisions between DL reception and UL transmission, which are configured by dedicated higher layer signaling, respectively

- High time/frequency resource overhead to avoid collisions between PDCCH CSS and UL transmission configured by dedicated higher layer signaling

- Clarification on the CPU occupation for a CSI report, while the reception of corresponding CMR and/or IMR is dropped due to the HD prioritization rule

- Clarification on the CPU occupation for a CSI report, while the transmission of corresponding PUCCH or PUSCH is dropped due to the HD prioritization rule

- Unnecessary scheduling restriction for HARQ-disabled PDSCH, while reception of the PDSCH is dropped due to the HD prioritization rule

References

- 3GPP RAN1#116 Chair notes

- 3GPP TS 38.321 V18.0.0

- 3GPP TS 38.213 V18.1.0

- 3GPP TS 38.214 V18.1.0

- 3GPP TS 38.331 V18.0.0

- 3GPP TR 38.811