3GPP TSG-RAN WG1 Meeting #117 	R1-2404149

Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.211 | CR |  | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on DMRS symbol in 38.211 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-05-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When a DMRS symbol conflicts with the configured second starting symbol in the resource pool, the Rx UE may need to avoid using that DMRS symbol for channel estimation, e.g., by puncturing the DMRS symbol at the second starting symbol. Essentially it means that the UE is required to support new DMRS pattern (i.e., puncturing one DMRS symbol of the existing pattern) for decoding, which increases the UE implementation complexity. Moreover, the punctured DMRS pattern will lead to a non-uniform distribution of the DMRS symbols within the slot, which further increases the implementation complexity and degrades the decoding performance. It is worth noting that, from the implementation point of view, the complexity would highly depend on the worst case. Whenever such kind of collision may happen in some cases, e.g., due to the hidden node issue, the implementation complexity is inevitable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the UE does not expect the DMRS symbol to be overlapped with the startingSymbolSecond. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The conflict between the DMRS symbol and the second starting symbol may increase the UE implementation complexity. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

8.4.1.1.2	Mapping to physical resources

< Unchanged parts are omitted >

The position(s) of the DM-RS symbols is given by l according to Table 8.4.1.1.2-1 where the number of PSSCH DM-RS is indicated in the SCI, and ld is the duration of the scheduled resources for transmission of PSSCH according to clause 8.1.2.1 of [6, TS 38.214] and the associated PSCCH, including the OFDM symbol duplicated as described in clauses 8.3.1.5 and 8.3.2.3. The UE does not expect the DMRS symbol to be overlapped with the startingSymbolSecond if configured within a slot.

< Unchanged parts are omitted >