3GPP TSG RAN WG1 #117                                                                          R1-2404232

Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | [DRAFT] | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on search space of DCI format 0_3 and DCI format 1_3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MC_enh-Core |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The DCI size and BD/CCE of DCI format 0_3/1_3 is counted on the reference cell and there are two cases to determine the reference cell according to the agreement in RAN1#111. For a set of cells which is configured for multi-cell scheduling, - Existing DCI size budget is maintained on each cell of the set of cells. - DCI size of DCI format 0_X/1_X is counted on one cell among the set of cells. - DCI size of the DCI format 0_X/1_X is counted on the reference cell. - BD/CCE of DCI format 0_X/1_X is counted on one cell among the set of cells. - BD/CCE of the DCI format 0_X/1_X is counted on the reference cell. - Same reference cell is used for both DCI format 0_X and DCI format 1_X. - The reference cell is - [Case 1]the scheduling cell if the scheduling cell is included in the set of cells and search space of the DCI format 0_X/1_X is configured only on the scheduling cell; - [Case2 ]one cell of the set of cells which search space of DCI format 0_X/1_X is configured on and associated with the search space of the scheduling cell with the same search space ID if search space of the DCI format 0_X/1_X is configured on the cell in addition to the scheduling cell. - It is up to gNB on which cell the SS of the DCI format 0_X/1_X is configured on.. For Case 2, the reference cell is one cell of the set of cells. The search space of DCI format 0_3/1_3 is configured on the reference cell. It is associated with the search space of the scheduling cell with the same search space ID. While, how to derive the Ms,nCIL of DCI format 0_3/1_3 based on the nCI-Value for the set of cells should be clarified, because both the USS with same search space ID on the scheduling cell and scheduled cell are configured. Note, in this case, the BD/CCE of the candidates in the USS with DCI format 0_3/1_3 configured on scheduling cell should not be counted on the scheduling cell, because the scheduling cell is out of the set of cells. |
| Summary of change: |  | In case the reference cell is one cell of the set of cells which search space of DCI format 0_3/1_3 is configured on both the reference cell and the scheduling cell with the same search space ID, use the nCI-Value for the set of cells to derive the CCE resources of the candidates of the same aggregation level in the USS with same ID configured in the reference cell, and the UE is not required to monitor the candidates of the USS with same ID configured in the scheduling cell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ms,nCIL is not clear if the number of PDCCH candidates in both the USS with same ID of DCI format 0_3/1_3 are configured on the scheduling cell which is out of the set of cells, and the reference cell in the set of cells. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version for this CR. |  |  |  |  |  |  |  |  |

10.1	UE procedure for determining physical downlink control channel assignment

<Unchanged parts are omitted>

For a search space set s associated with CORESET p, the CCE indexes for aggregation level L corresponding to PDCCH candidate ms,nCI(L)  of the search space set in slot ns,fμ for an active DL BWP of a serving cell corresponding to carrier indicator field value nCI, or corresponding to value nCI of nCI-Value associated with a set of serving cells MC-DCI-SetofCells, are given by

L⋅Yp,ns,fμ+ms,nCI(L)⋅NCCE,pL⋅Ms,maxL+nCImodNCCE,pL+i

where

for any CSS, Yp,ns,fμ=0;

for a USS, Yp,ns,fμ=Ap⋅Yp,ns,fμ−1modD, Yp,−1=nRNTI≠0, Ap=39827 for pmod3=0, Ap=39829 for pmod3=1, Ap=39839 for pmod3=2, and D=65537;

i=0,⋯,L−1;

NCCE,p is the number of CCEs, numbered from 0 to NCCE,p−1, in CORESET p and, if any, per RB set;

nCI is

-	the carrier indicator field value, if provided by cif-InSchedulingCell in CrossCarrierSchedulingConfig for the serving cell on which PDCCH is monitored, except for scheduling of the serving cell from the same serving cell in which case nCI=0;

-	the nCI-Value, if provided, for the set of serving cells MC-DCI-SetofCells;

-	otherwise, including for any CSS, nCI=0

ms,nCI(L)=0,⋯,Ms,nCIL−1, where Ms,nCIL is

-	the number of PDCCH candidates the UE is configured to monitor for aggregation level L of a search space set s for a serving cell corresponding to nCI;

-	the number of PDCCH candidates the UE is configured in the scheduling cell to monitor for aggregation level L of a search space set s for a set of serving cells corresponding to nCI if the scheduling cell is included in the set of serving cells and the UE is provided search space sets for the PDCCH candidates only on the scheduling cell;

-	the number of PDCCH candidates the UE is configured in the scheduled cell to monitor for aggregation level L of a search space set s for a set of serving cells corresponding to nCI  if search space sets s for one or both of DCI format 0_3 and DCI format 1_3, respectively, are provided on the scheduled cell and on the scheduling cell. The UE is not required to monitor the number of PDCCH candidates configured in the scheduling cell for aggregation level L of the search space set s.

for any CSS, Ms,maxL=Ms,0L;

for a USS, Ms,maxL is the maximum of Ms,nCIL over all configured nCI values for a CCE aggregation level L of search space set s ;

the RNTI value used for nRNTI is the C-RNTI.

<Unchanged parts are omitted>