3GPP TSG-RAN WG1 Meeting #117	R1-2404378

Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT]CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | xxx | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on HARQ-ACK information skipping due to BWP change for second Type-2 HARQ-ACK codebook |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MC_Enh-Core |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In current TS 38.213, the UE behaviour of second type-2 HARQ-ACK sub-codebook generation is not specified when the following conditions are satisfied. - PDCCH MO of DCI format 1_3 is before an active DL BWP change on a cell of co-scheduled cells by the DCI format 1_3, and - The DCI format 1_3 does not trigger an active DL BWP change for the cell. Considering the efficiency of type-2 HARQ-ACK codebook for generating the second sub-codebook, the following scheme should be considered: - If at least one cell of co-scheduled cells has not an active DL BWP change ,the corresponding HARQ-ACK information for that cell with BWP switching is generated with NACK bit, - Otherwise the corresponding HARQ-ACK information for the cell with BWP switching should be skipped. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the ‘while c<NcellsDL’ struction of second type-2 HARQ-ACK sub-codebook, add the condition of ‘active DL BWP(s) change on all serving cells that scheduled by DCI format 1_3 and active DL BWP(s) change on all serving cells are not triggered in PDCCH monitoring occasion m’. In the ‘while mc<Ncells,setDL,max’ struction of second type-2 HARQ-ACK sub-codebook, add the condition of ‘if PDCCH monitoring occasion m is not before an active DL BWP change’. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure of second type-2 HARQ-ACK sub-codebook generating is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

9.1.3.1	Type-2 HARQ-ACK codebook in physical uplink control channel

< Unchanged parts are omitted >

while m<M

c=0

if harq-ACK-SpatialBundlingPUCCH is not provided,

while c<NcellsDL

if PDCCH monitoring occasion m is before active DL BWP change(s) on all serving cells that scheduled by DCI format 1_3 or an active UL BWP change on the serving cell of PUCCH transmission if the UE is provided pucch-sSCellDyn, or an active UL BWP change on the PCell if the UE is not provided pucch-sSCellDyn, and active DL BWP change(s) on all serving cells are not triggered in PDCCH monitoring occasion m

c=c+1;

else

if there is a PDSCH reception on serving cell c that is scheduled by a DCI format scheduling more than one PDSCHs that provide respective more than one transport blocks with enabled HARQ-ACK information on respective more than one serving cells, where the DCI format is associated with a PDCCH reception in PDCCH monitoring occasion m and c is the smallest serving cell index among the more than one serving cells

if VC-DAI,c,mDL≤Vtemp

j=j+1;

end if

Vtemp=VC-DAI,c,mDL;

if VT-DAI,mDL=∅

Vtemp,2=VC-DAI,c,mDL;

else

Vtemp,2=VT-DAI,mDL;

end if

cnt=0;

mc=0;

while mc<Ncells,setDL,max

if the UE is scheduled PDSCH reception on serving cell mc, if any, from the more than one serving cells, and if PDCCH monitoring occasion m is not before an active DL BWP change

if maxNrofCodeWordsScheduledByDCI is 2 for serving cell mc, if any, from the more than one serving cells

oNsetsTB,max⋅TD⋅j+NsetsTB,max⋅VC-DAI,c,mDL-1+cntACK = HARQ-ACK information bit corresponding to the first transport block of this cell

oNsetsTB,max⋅TD⋅j+NsetsTB,max⋅VC-DAI,c,mDL-1+1+cntACK = HARQ-ACK information bit corresponding to the second transport block of this cell

cnt=cnt+2;

else

oNsetsTB,max⋅TD⋅j+NsetsTB,max⋅VC-DAI,c,mDL-1+cntACK = HARQ-ACK information bit corresponding to the transport block of this cell

cnt=cnt+1;

end if

end if

mc=mc+1;

end while

while cnt< NsetsTB,max

oNsetsTB,max⋅TD⋅j+NsetsTB,max⋅VC-DAI,c,mDL-1+cntACK= NACK;

cnt=cnt+1;

end while

Vs=Vs∪NsetsTB,max⋅TD⋅j+NsetsTB,max⋅VC-DAI,c,mDL-1, …, NsetsTB,max⋅TD⋅j+NsetsTB,max⋅VC-DAI,c,mDL-1+NsetsTB,max-1;

end if

c=c+1;

end if

end while

else

while c<NcellsDL

if PDCCH monitoring occasion m is before active DL BWP change(s) on all serving cells that scheduled by DCI format 1_3 or an active UL BWP change on the serving cell of PUCCH transmission if the UE is provided pucch-sSCellDyn, or an active UL BWP change on the PCell if the UE is not provided pucch-sSCellDyn, and active DL BWP change(s) on all serving cells are not triggered in PDCCH monitoring occasion m

c=c+1;

else

if there is a PDSCH reception on serving cell c that is scheduled by a DCI format scheduling more than one PDSCHs that provide respective more than one transport blocks with enabled HARQ-ACK information on respective more than one serving cells, where the DCI format is associated with a PDCCH reception in PDCCH monitoring occasion m and c is the smallest serving cell index among the more than one serving cells

if VC-DAI,c,mDL≤Vtemp

j=j+1;

end if

Vtemp=VC-DAI,c,mDL;

if VT-DAI,mDL=∅

Vtemp,2=VC-DAI,c,mDL;

else

Vtemp,2=VT-DAI,mDL;

end if

cnt=0;

mc=0;

while mc<Ncells,setDL,max

if the UE is scheduled PDSCH reception for transport blocks with enabled HARQ-ACK information on serving cell mc, if any, from the more than one serving cells, and if PDCCH monitoring occasion m is not before an active DL BWP change

if maxNrofCodeWordsScheduledByDCI is 2 for serving cell mc

if the PDSCH reception provides two transport blocks

oNcells,setDL,max⋅TD⋅j+Ncells,setDL,max⋅VC-DAI,c,mDL-1+cntACK = binary AND operation of the HARQ-ACK information bits corresponding to the first and second transport blocks of this cell

else

oNcells,setDL,max⋅TD⋅j+Ncells,setDL,max⋅VC-DAI,c,mDL-1+cntACK = HARQ-ACK information bit corresponding to the first transport block of this cell

end if

else

oNcells,setDL,max⋅TD⋅j+Ncells,setDL,max⋅VC-DAI,c,mDL-1+cntACK= HARQ-ACK information bit of this cell

end if

cnt=cnt+1;

end if

mc=mc+1;

end while

< Unchanged parts are omitted >