3GPP TSG RAN WG1 #117		 							R1-2404824

Fukuoka City, Fukuoka, Japan, May 20th– 24th, 2024

Source:	OPPO

Title:	UE features for other Rel-18 work items (Topics B)

Agenda Item:	8.5.3

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss the UE feature for Rel-18 MIMO evolution.

UE feature for Rel-18 MIMO

  - UE feature for SRS enhancements

- In RAN1#113 meeting, the following was agreed for SRS power scaling of 8 ports TDMed SRS.

Agreement For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, the UE splits a linear value PSRS of SRS transmission power equally across the SRS ports configured on each OFDM symbol, if the UE is capable of transmitting at PCMAX per OFDM symbol with 8/s ports, where PCMAX is specified in the current specifications. - Note: This may be captured in the specification in a few different but equivalent ways, and it is up to the editor to decide.

In current 38.213, the UE splits a linear value of SRS transmission power equally across the SRS ports configured on each OFDM symbol. However, this can only be applied to a UE capable of transmitting at PCMAX per OFDM symbol. For a UE without this capability, the UE behaviour is not defined, and the transmit power in one symbol may exceed the maximal power supported by UE. Considering the limited time for maintenance, a simple solution with minimized standardization effort is preferred. For example, TDM-based 8Tx SRS (s=2) is not supported by UE not capable of transmitting at PCMAX per OFDM symbol with 8/s ports.

Proposal 1: A UE supporting TDMed 8 ports SRS should be able to transmit at PCMAX per OFDM symbol with 4 ports and applied the following note:

| 40. NR_MIMO_evo_DL_UL | 40-5-4 | SRS 8 Tx ports—antenna switching | 1. Support of 8T8R for antenna switching 2. Downgrade antenna switching configurations 3. Report the entry number of the first-listed band with UL in the band combination that affects this DL 4. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL | 2-53 | Yes | n/a | SRS with 8 Tx ports—antenna switching is not supported | Per FS | n/a | n/a | n/a | Component 1 candidate values: {noTDM, TDM and noTDM} Component 2 candidate value: combination (including empty) of {1T1R, 1T2R, 1T4R, 1T6R, 1T8R, 2T2R, 2T4R, 2T6R, 2T8R, 4T4R, 4T8R} Component 3 candidate value: {1,2,…,32} Component 4 candidate value: {1,2,…,32} Note: UE reports support of SRS with 8 Tx ports and Comb8 mapping —antenna switching via FG 23-8-8 Note: UE reporting support of TDM SRS should be able to transmit at P_CMAX per OFDM symbol with 4 ports | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-7-1 | Basic features for Codebook-based 8Tx PUSCH | 1. Maximum number of PUSCH MIMO layers for codebook based PUSCH 2. Maximum number of 8 port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 8Tx PUSCH 3. SRS 8 Tx ports—codebook |  | Yes | n/a | Codebook-based 8Tx PUSCH is not supported | Per FSPC | No | No | No | Component 1 candidate values: {1,2 ,3,4 ,5,6,7,8} Component 2 candidate values: {1,2} Component 3 candidate values: {noTDM, TDM and noTDM} A UE that supports FG 40-7-1 must support at least one of FGs 40-7-1a/b/c/d Note: UE reporting support of TDM SRS should be able to transmit at P_CMAX per OFDM symbol with 4 ports | Optional with capability signaling |

  - UE feature for SRI/TPMI enhancement for enabling 8 TX UL transmission

In Rel-16, full power mode 2 is supported for 2/4Tx with the following FGs:

| 16-5c | UL full power transmission fullpowerMode2 | The maximum number of SRS resources in one SRS resource set with usage set to ‘codebook’ for Mode 2: {1, 2, 4} | 2-13, 2-14 | Yes | N/A |  | Per FS | No | No |  | A UE that supports FG 16-5c supports at least full power operation with single port | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16-5c-2 | UL full power transmission fullpowerMode2 – SRS resources | The SRS configuration with different number of antenna ports per SRS resource for Mode 2 | 16-5c | Yes | N/A |  | Per FS | No | No |  | Component (1) candidate values: {1_2, 1_4, 1_2_4} 1st state (1_2): each SRS resource can be configured with 1 port or 2 ports 2nd state (1_4): each SRS resource can be configured with 1 port or 4 ports 3rd state (1_2_4): each SRS resource can be configured with 1 port or 2 ports or 4 ports Note: The first, second, or third state can be used if 16-5c is reported as 2 or 4.t | Optional with capability signaling |

For 8Tx, similar two FGs (40-7-1g/40-7-1g-1) were introduced as below with the same functionality. In RAN1#117 meeting, a note was added similar to 2/4Tx  to clarify that full power operation with single port is mandatory for UE supporting full power mode 2 with 8Tx. However, in FG 40-7-1g-1, there is still one bit (b0) to indicate whether SRS resource can be configured with one port for full power mode 2. It is redundant considering the newly added note and can be deleted.

Proposal 2: The following can be applied to UE feature for 8Tx PUSCH transmission:

| 40. NR_MIMO_evo_DL_UL | 40-7-1g | UL full power transmission mode 2 with 1/2/4 resources | 1. Support of UL full power transmission mode of fullpowerMode2 when UE is capable of 8 Tx codebook based PUSCH operation 2. Maximum number of SRS resources in one SRS resource set with usage set to 'codebook' for 8Tx codebook based PUSCH for Mode 2 | 40-7-1 | yes | n/a | UL full power transmission mode 2 is not supported | Per FSPC | n/a | n/a | n/a | Component 2 candidate values: {1, 2, 4} Note: A UE that supports FG 40-7-1g supports at least full power operation with single port | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-7-1g-1 | SRS resources for UL full power transmission mode 2 | 1. SRS configurations with different number of antenna ports per SRS resource for mode 2 | 40-7-1g | yes | n/a | SRS resources for UL full power transmission mode 2 cannot be signaled | Per FSPC | n/a | n/a | n/a | Component 1 candidate values: 32 bit bitmap {b0, b1, b2} b0 indicates whether SRS resource can be configured with 12 port b1 indicates whether SRS resource can be configured with 24 port b2 indicates whether SRS resource can be configured with 4 port | Optional with capability signalling |

Based on current UE feature, capability for different codebook types are independently reported. If UE reports capability for a codebook type, UE should support it regardless of non-TDMed and TDMed SRS.  However, according to RAN4’s LS[1], a UE supporting codebook1 for non-TDMed SRS may not be able to achieve coherency across TDMed SRS. For a UE supporting codeobok1 for non-TDMed SRS, additional UE feature is needed to indicate whether codebook1 is also supported for TDMed SRS. Furthermore, considering coherent SRS antenna ports are transmitted in the same symbol for partial coherent UE even with TDMed SRS, it is not need to differentiate TDMed SRS and non-TDMed SRS for codebook2/3/4. A UE cannot report capability of TDMed SRS if it doesn’t support any codebook type for TDMed SRS.

Proposal 3: Modify current FG 40-7-1a to support UE to report one of the following UE features:

- UE supports codebook1 for both non-TDMed SRS and TDMed SRS

- UE supports codebook1 for non-TDMed SRS and not for TDMed SRS

Note: Whether UE supports codebook 2/3/4 or not for TDMed and non-TDMed SRS is up to FG 40-7-1.

| 40. NR_MIMO_evo_DL_UL | 40-7-1 | Basic features for Codebook-based 8Tx PUSCH | 1. Maximum number of PUSCH MIMO layers for codebook based PUSCH 2. Maximum number of 8 port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 8Tx PUSCH 3. SRS 8 Tx ports for codebook2/ codebook 3/ codebook 4 |  | Yes | n/a | Codebook-based 8Tx PUSCH is not supported | Per FSPC | No | No | No | Component 1 candidate values: {1,2 ,3,4 ,5,6,7,8} Component 2 candidate values: {1,2} Component 3 candidate values: {noTDM, TDM and noTDM} A UE that supports FG 40-7-1 must support at least one of FGs 40-7-1a/b/c/d | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-7-1a | Codebook-based 8Tx PUSCH—codebook1 | 1. Support of codebook-based 8Tx PUSCH—codebook1 2. Support of (N1, N2) for codebook-based 8Tx PUSCH—codebook1 3. SRS 8 Tx ports for codebook1 | 40-7-1 | Yes | n/a | Codebook-based 8Tx PUSCH—codebook1 is not supported | Per FSPC | No | No | No | Component 2 candidate values: {(4,1), (2,2), both} Component 3 candidate values: {noTDM, TDM and noTDM} | Optional with capability signaling |

Conclusion

In this contribution, we discuss the remaining issues on UE features for Rel-18 MIMO with the following proposals:

Proposal 1: A UE supporting TDMed 8 ports SRS should be able to transmit at PCMAX per OFDM symbol with 4 ports and applied the following note:

| 40. NR_MIMO_evo_DL_UL | 40-5-4 | SRS 8 Tx ports—antenna switching | 1. Support of 8T8R for antenna switching 2. Downgrade antenna switching configurations 3. Report the entry number of the first-listed band with UL in the band combination that affects this DL 4. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL | 2-53 | Yes | n/a | SRS with 8 Tx ports—antenna switching is not supported | Per FS | n/a | n/a | n/a | Component 1 candidate values: {noTDM, TDM and noTDM} Component 2 candidate value: combination (including empty) of {1T1R, 1T2R, 1T4R, 1T6R, 1T8R, 2T2R, 2T4R, 2T6R, 2T8R, 4T4R, 4T8R} Component 3 candidate value: {1,2,…,32} Component 4 candidate value: {1,2,…,32} Note: UE reports support of SRS with 8 Tx ports and Comb8 mapping —antenna switching via FG 23-8-8 Note: UE reporting support of TDM SRS should be able to transmit at P_CMAX per OFDM symbol with 4 ports | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-7-1 | Basic features for Codebook-based 8Tx PUSCH | 1. Maximum number of PUSCH MIMO layers for codebook based PUSCH 2. Maximum number of 8 port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 8Tx PUSCH 3. SRS 8 Tx ports—codebook |  | Yes | n/a | Codebook-based 8Tx PUSCH is not supported | Per FSPC | No | No | No | Component 1 candidate values: {1,2 ,3,4 ,5,6,7,8} Component 2 candidate values: {1,2} Component 3 candidate values: {noTDM, TDM and noTDM} A UE that supports FG 40-7-1 must support at least one of FGs 40-7-1a/b/c/d Note: UE reporting support of TDM SRS should be able to transmit at P_CMAX per OFDM symbol with 4 ports | Optional with capability signaling |

Proposal 2: The following can be applied to UE feature for 8Tx PUSCH transmission:

| 40. NR_MIMO_evo_DL_UL | 40-7-1g | UL full power transmission mode 2 with 1/2/4 resources | 1. Support of UL full power transmission mode of fullpowerMode2 when UE is capable of 8 Tx codebook based PUSCH operation 2. Maximum number of SRS resources in one SRS resource set with usage set to 'codebook' for 8Tx codebook based PUSCH for Mode 2 | 40-7-1 | yes | n/a | UL full power transmission mode 2 is not supported | Per FSPC | n/a | n/a | n/a | Component 2 candidate values: {1, 2, 4} Note: A UE that supports FG 40-7-1g supports at least full power operation with single port | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-7-1g-1 | SRS resources for UL full power transmission mode 2 | 1. SRS configurations with different number of antenna ports per SRS resource for mode 2 | 40-7-1g | yes | n/a | SRS resources for UL full power transmission mode 2 cannot be signaled | Per FSPC | n/a | n/a | n/a | Component 1 candidate values: 32 bit bitmap {b0, b1, b2} b0 indicates whether SRS resource can be configured with 12 port b1 indicates whether SRS resource can be configured with 24 port b2 indicates whether SRS resource can be configured with 4 port | Optional with capability signalling |

Proposal 3 Modify current FG 40-7-1a to support UE to report one of the following UE features:

- UE supports codebook1 for both non-TDMed SRS and TDMed SRS

- UE supports codebook1 for non-TDMed SRS and not for TDMed SRS

Note: Whether UE supports codebook 2/3/4 or not for TDMed and non-TDMed SRS is up to FG 40-7-1.

| 40. NR_MIMO_evo_DL_UL | 40-7-1 | Basic features for Codebook-based 8Tx PUSCH | 1. Maximum number of PUSCH MIMO layers for codebook based PUSCH 2. Maximum number of 8 port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 8Tx PUSCH 3. SRS 8 Tx ports for codebook2/ codebook 3/ codebook 4 |  | Yes | n/a | Codebook-based 8Tx PUSCH is not supported | Per FSPC | No | No | No | Component 1 candidate values: {1,2 ,3,4 ,5,6,7,8} Component 2 candidate values: {1,2} Component 3 candidate values: {noTDM, TDM and noTDM} A UE that supports FG 40-7-1 must support at least one of FGs 40-7-1a/b/c/d | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-7-1a | Codebook-based 8Tx PUSCH—codebook1 | 1. Support of codebook-based 8Tx PUSCH—codebook1 2. Support of (N1, N2) for codebook-based 8Tx PUSCH—codebook1 3. SRS 8 Tx ports for codebook1 | 40-7-1 | Yes | n/a | Codebook-based 8Tx PUSCH—codebook1 is not supported | Per FSPC | No | No | No | Component 2 candidate values: {(4,1), (2,2), both} Component 3 candidate values: {noTDM, TDM and noTDM} | Optional with capability signaling |

References

- R4-2403632, LS reply on Relative Phase/Power Error Requirements within Port Groups for 8TX UE, RAN4 (Nokia), RAN4#110 meeting