3GPP TSG RAN WG1 Meeting #117			 		 	R1-2405401

Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024

Source:	Moderator (Intel Corporation)

Title:	FL summary #1 on SL positioning reference signal

Agenda item:	8.1

Document for:	Discussion and Decision

- Introduction

This document presents the Feature Lead (FL) summary of submitted contributions to AI 8.1 on maintenance of SL PRS design as part of maintenance of Rel-18 WI on expanded and improved NR positioning.

Based on the submitted contributions to RAN1 #117, the only issue to be discussed is listed in the table below.

| Issue # | Tdoc reference | Issue/Proposal |
| --- | --- | --- |
| 1 | [4] R1-2404052, Nokia | Specify that a Tx UE may report whether “different SL PRS resources can be used to perform sidelink positioning measurements with multiple measurement samples”. |

Please follow the naming convention in this example:

- SLPRS_FLS -v000.docx

- SLPRS_FLS -v001-CompanyA.docx

- SLPRS_FLS -v002-CompanyA-CompanyB.docx

- SLPRS_FLS -v003-CompanyB-CompanyC.docx

If needed, you may “lock” a spreadsheet file for 30 minutes by creating a checkout file, as in this example:

- Assume CompanyC wants to update SLPRS_FLS-v002-CompanyA-CompanyB.docx.

- CompanyC uploads an empty file named SLPRS_FLS-v003-CompanyB-CompanyC.checkout

- CompanyC checks that no one else has created a checkout file simultaneously, and if there is a collision, CompanyC tries to coordinate with the company who made the other checkout (see, e.g., contact list below).

- CompanyC then has 30 minutes to upload SLPRS_FLS-v003-CompanyB-CompanyC.docx

- If no update is uploaded in 30 minutes, other companies can ignore the checkout file.

- Note that the file timestamps on the server are in UTC time.

To avoid excessive email load on the RAN1 email reflector, please note that there is NO need to send an info email to the reflector just to inform that you have uploaded a new version of this document. Companies are invited to enter the contact info in the table below.

## FL1 Question 1-1

- Please consider entering contact info below for the points of contact for this agenda item:

| Company | Point of contact | Email address |
| --- | --- | --- |

- Discussion on Identified Issues

  - Issue #1

Reference [4] (Nokia) proposes the following:

Proposal 3: Support the following text proposal of Clause 8.4.4 of TS 38.214 8.2.4 SL PRS transmission procedure <omitted text> The UE may report the association information between the already transmitted SL PRSs of SL PRS resources and UE Tx ARP ID. The association information includes ARP ID(s) indicated by sl-POS-ARP-ID-Tx, SL PRS transmission timestamp(s) indicated by sl-TimeStamp, and optional SL PRS resource ID(s) indicated by sl-PRS-ResourceID. The UE may report whether transmitted SL PRSs of SL PRS resources can be used to perform sidelink positioning measurements with multiple measurement samples. <omitted text>. Reason for change:; RAN4 is now considering both single measurement sample and multiple measurement samples of SL PRS measurements. In RAN1, a specific SL PRS resource is uniquely defined within a slot, so a receiving UE can perform a positioning measurement based on a single measurement sample only. There is no way for the Rx UE to assume the same Tx antenna across different SL PRS resources. In order for Rx UE to perform SL PRS measurement with multiple measurement samples, RAN1 should support additional feature. At least, the Rx UE needs information which SL PRS resources have been transmitted with the same Tx antenna port. It should be noted that antenna port number of DL PRS resources is the same, but it does not mean that the UE can use measurements from different DL PRS resources as multiple measurement samples to derive a single measurement. The UE reports a specific positioning measurement based on multiple measurement samples from a single DL PRS resource which is transmitted periodically. The positioning measurement based on multiple measurement samples cannot be done by randomly selecting different SL PRS resources by the Rx UE.. Summary of change:; Add the proposed text in Clause 8.2.4 of TS 38.214.. Consequences if not approved:; The Rx UE cannot obtatin positioning measurements with multiple measurement samples based on SL PRS, which is not aligned with the current RAN4 discussion. If this is not approved, RAN1 needs discussion with RAN4 to address the issue..

Moderator comments:

- As discussed during RAN1 #116bis, for SL PRS transmission, use of multiple samples at the receiver should assume non-coherent combining. Thus, for RAN4 to define measurements on SL PRS using multiple samples, it is not necessary that the samples can be combined coherently.

- As long as the Tx ARPs for different SL PRS transmissions from a Tx UE (if different Tx ARPs are used for SL PRS transmissions from a Tx UE) are not considerably far apart relative to each other (i.e., not the case as Tx antenna(s) at front and rear bumpers of a vehicle), any changes to precoding/antenna selection should still be able to be combined.

  - Further, the signalling of assistance data in the form of association information of Tx ARP ID to already transmitted SL PRS, when provided, can be utilized by LMF/server UE/Rx UE to decide on potential combining of multiple samples.

- Moreover, the TP proposed in [4] may not help much in addressing the issue raised in [4] unless further identification of the specific SL PRS resources is provided.

### FL1 Proposal 2.1-1

- Agree on TP#1 for TS 38.214, Clause 8.2.4 to support indication of use of same antenna port for transmission of different SL PRS resources.

------------------------------ TP#1: TS 38.214 ----------------------------------- 8.2.4 SL PRS transmission procedure <omitted text> The UE may report the association information between the already transmitted SL PRSs of SL PRS resources and UE Tx ARP ID. The association information includes ARP ID(s) indicated by sl-POS-ARP-ID-Tx, SL PRS transmission timestamp(s) indicated by sl-TimeStamp, and optional SL PRS resource ID(s) indicated by sl-PRS-ResourceID. The UE may report whether transmitted SL PRSs of SL PRS resources can be used to perform sidelink positioning measurements with multiple measurement samples. <omitted text>

| Reason for change: | RAN4 is not considering multiple measurement samples of SL PRS measurements. In RAN1, a specific SL PRS resource is uniquely defined within a slot, so a receiving UE can perform a positioning measurement based on a single measurement sample only. There is no way for the Rx UE to assume the same antenna port across different SL PRS resources. In order for Rx UE to perform SL PRS measurement with multiple measurement samples, RAN1 should support additional feature. At least, the Rx UE needs information which SL PRS resources have been transmitted with the same Tx antenna port. |
| --- | --- |
| Summary of change: | Add the proposed text in Clause 8.2.4 of TS 38.214. |
| Consequences if not approved: | The Rx UE cannot obtatin positioning measurements with multiple measurement samples based on SL PRS, which is not aligned with the current RAN4 discussion. If this is not approved, RAN1 needs discussion with RAN4 to address the issue. |

| Company | Comments |
| --- | --- |
| Qualcomm | Not needed. We discussed it also last meeting. |
| vivo | Not needed. |
| CATT | Not needed. |
| ZTE | Not needed |
| Sharp | Not needed. |
| Moderator | Based on received feedback so far and the discussions during RAN1 #116bis, the following is proposed as a conclusion: Proposed Conclusion: - Reporting of information from Tx UE on whether SL PRS resources can be used for sidelink positioning measurements with multiple measurement samples is not pursued in Rel-18. |

- Proposals for Monday GTW

Proposed Conclusion:

- Reporting of information from Tx UE on whether SL PRS resources can be used for sidelink positioning measurements with multiple measurement samples is not pursued in Rel-18.

- List of Text Proposals for Monday GTW

…

- Outcome from RAN1 #117

…

# References

- R1-2403959	Remaining issues of Rel-18 positioning	Huawei, HiSilicon

- R1-2403970	Correction on collision handling of positioning SRS with frequency hopping in TDD systems	Intel Corporation

- R1-2403971	Corrections on positioning SRS with frequency hopping	Intel Corporation

- R1-2404052	Maintenance on Expanded and Improved NR Positioning	Nokia

- R1-2404153	Draft CR on PRS for carrier phase positioning	vivo

- R1-2404154	Draft CR on SRS bandwidth aggregation for positioning measurements	vivo

- R1-2404155	Draft CR on SRS frequency hopping for positioning	vivo

- R1-2404156	Draft CR on Sidelink congestion control in dedicated SL PRS resource pool	vivo

- R1-2404366	Correction on the definition of sidelink PRS channel busy ratio	CATT, CICTCI

- R1-2404372	Correction on the UE procedure for transmitting PSCCH in dedicated SL PRS resource pool	CATT, CICTCI

- R1-2404373	Discussion on the higher layer parameters in a dedicated SL PRS resource pool	CATT, CICTCI

- R1-2404986	Correction on PRS bandwidth aggregation for 38.214	ZTE

- R1-2404987	Correction on SRS bandwidth aggregation in RRC_INACTIVE for 38.214	ZTE

- R1-2404988	Draft CR for dropping rule on SRS bandwidth aggregation	ZTE

- R1-2404989	Draft CR on PRS processing for bandwidth aggregation	ZTE

- R1-2404990	Correction on SRS configuration and UE behavior in validity area for LPHAP for 38.214	ZTE

- R1-2404991	Correction on UE behavior in validity area for LPHAP for 38.213	ZTE

- R1-2404992	Draft CR for carrier phase positioning	ZTE

- R1-2404993	Draft CR for collision handling of positioning SRS with Tx hopping in TDD system	ZTE

- R1-2404994	Discussion on collision handling of positioning SRS with Tx hopping in TDD system	ZTE

- R1-2404995	Draft CR for RedCap UE frequency hopping	ZTE

- R1-2404996	Draft CR for UE behavior on monitoring DCI format 3_2	ZTE

- R1-2404997	Discussion on UE behavior on monitoring DCI format 3_2	ZTE

- R1-2404998	Correction on SL positioning for 38.214	ZTE

- R1-2404999	Correction on SL positioning for 38.212	ZTE

- R1-2405000	Correction on SL positioning for 38.211	ZTE

- R1-2405001	Missed RRC parameters for sidelink positioning	ZTE

- R1-2405288	Draft CR for correction to SRS for positioning with tx hopping in 38.211	Ericsson

- R1-2405289	Draft CR for correction to SRS for positioning with tx hopping in 38.214	Ericsson

- R1-2405313	Correction to 38.202 on support of combination of SL CA and SL positioning	Huawei, HiSilicon

- R1-2405314	Correction to 38.213 on support of DCI formats for SL positioning	Huawei, HiSilicon

- R1-2405315	Correction to 38.211 on the transmission counter	Huawei, HiSilicon

- R1-2405320	Correction to the impact from DL reception on the affected bands	Huawei, HiSilicon

- R1-2405321	Correction to the provision of RTD in SL positioning	Huawei, HiSilicon

- 3GPP TR 38.859,  Study on expanded and improved NR positioning (Release 18)”, December 2022.

- R1-2401828, RAN1 agreements for Rel-18 WI on Expanded and Improved NR Positioning	Rapporteur (Intel Corporation), RAN1 #116bis, April 2024.