3GPP TSG RAN WG1 #117			R1-2405489

Fukuoka City, Fukuoka, Japan, May 20th - 24th, 2024

Agenda Item:	9.1.3.1

Source: 	Moderator (LG Electronics)

Title: 	Summary #1 of CSI prediction

Document for:	Discussion and decision

Introduction

This is the summary document for agenda item 9.1.3.1 CSI prediction.

Contact information

Please provide your contact information or any updates.

| Company | Name | Email |
| --- | --- | --- |
| Moderator (LG Electronics) | Haewook Park | haewook.park@lge.com |
| New H3C | Lei Zhou | Zhou.leih@h3c.com |
| Samsung | Ameha Tsegaye Abebe | amehat.abebe@samsung.com |
| Indian Institute of Technology Madras (IIT Madras) | Sai Prasad Pirati | venkatasiva@5gtbiitm.in |
| SK telecom | Yunesung Kim | yunesung.kim@sktelecom.com |
| MediaTek | Pedram Kheirkhah Sangdeh Yuching Huang | Pedram.kheirkhah@mediatek.com yuching.huang@mediatek.com |
| Panasonic | Tetsuya Yamamoto | yamamoto.tetsuya001@jp.panasonic.com |
| vivo | Hao Wu | hao.wu@vivo.com |
| Huawei, HiSilicon | Keyvan Zarifi, Yuan Li | Keyvan.Zarifi@huawei.com liyuan3@huawei.com |
| NEC | Zhen He, Zhaobang Miao, Caroline Liang | he_zhen@nec.cn miao_zhaobang@nec.cn caroline.liang@emea.nec.com |
| Fujitsu | WANG Guotong (David) | wangguotong@fujitsu.com |
| Qualcomm | Jay Kumar Sundararajan | jsundara@qti.qualcomm.com |
| OPPO | Wendong Liu Wenqiang Tian | liuwendong1@oppo.com tianwenqiang@oppo.com |
| NTT DOCOMO | Xin Wang Haruhi Echigo | wangx@DOCOMOlabs-beijing.com.cn haruhi.echigo.fw@nttDOCOMO.com |
| Spreadtrum | Shijia Shao Hualei Wang | Shijia.shao@unisoc.com Hualei.wang@unisoc.com |
| Ericsson | Jingya Li Siva Muruganathan Jianwei Zhang | Jingya.li@ericsson.com Siva.muruganathan@ericsson.com Jianwei.zhang@ericsson.com |
| ZTE | Xingguang WEI Lun LI | wei.xingguang@zte.com.cn li.lun1@zte.com.cn |
| IIT Kanpur | Shyam Vijay Gadhai Abhishek Kumar Singh | svgadhai@iitk.ac.in abhishekks@iitk.ac.in |
| AT&T | Isfar Tariq | Isfar.tariq@att.com |
| Xiaomi | Liuzhengxuan Liumin | liuzhengxuan@xiaomi.com liumin10@xiaomi.com |
| Intel | Victor Sergeev | Victor.sergeev@intel.com |
| CMCC | Yuhua Cao Yi Zheng Dan Song | caoyuhua@chinamobile.com zhengyi@chinamobile.com songdan@chinamobile.com |
| Lenovo, MotM | Ahmed Hindy Vahid Pourahmadi | ahmedhindy@motorola.com vpourahmadi@lenovo.com |
| CEWiT | Shiv Shankar | shivshankar@cewit.org.in |
| NVIDIA | Xingqin Lin | xingqinl@nvidia.com |
| CATT | Qianrui Li | liqianrui@catt.cn |
| TCL | Yunsheng Kuang | yunsheng.kuang@tcl.com |
| LGE | Junhyuk Yoo | Junhyuk.yoo@lge.com |
| Sony | Sam Atungsiri | Sam.atungsiri@sony.com |
| InterDigital | Moon-il Lee | moonil.lee@interdigital.com |
| ETRI | Anseok Lee | alee@etri.re.kr |
| Mavenir | Fan Yang Yuanlong Yang | fan.yang@mavenir.com yuanlong.yang@mavenir.com |

Evaluation on AI/ML based CSI prediction

In RAN#103, modification for study objectives were agreed [1] as captured below:

Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - CSI feedback enhancement [RAN1]: - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843. - For CSI prediction (UE-sided model), further study performance gain over Rel-18 non-AI/ML based approach and associated complexity, while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843 (e.g., cell/site specific model could be considered to improve performance gain).

Initial evaluation results

- Performance comparison compared to Benchmark 1 & 2

- Summary

From the contribution submitted to RAN1#117, several companies provide their evaluation results and assumptions. The table below captures observations in their contribution.

| Ericsson | From the perspective of basic performance gain over non-AI/ML benchmark, under the assumption of the same UE speed of 30km/h for training and inference, observation window of 5/5ms, and prediction window of 1/5ms/5ms, and non-ideal CSI channel estimation: - AI/ML based CSI prediction outperforms the benchmark of the Rel-16 Type II with nearest historical CSI (baseline #1), where - 9% and 37% gains in terms of mean-UPT are observed at RU of 20% and 50%, respectively. - 27% and 100% gains in terms of 5%-UPT are observed at RU of 20% and 50%, respectively. - AI/ML based CSI prediction outperforms the benchmark of Rel-18 Type II non-AI/ML based CSI prediction (baseline #2), where - 9% and 24% gains in terms of mean-UPT are observed at RU of 20% and 50%, respectively. - 18% and 46% gains in terms of 5%-UPT are observed at RU of 20% and 50%, respectively. - Rel-18 Type II non-AI/ML based CSI prediction (baseline #2) outperforms Rel-16 Type II with nearest historical CSI (baseline #1) From the perspective of basic performance gain over non-AI/ML benchmark, under the assumption of the same UE speed of 30km/h for training and inference, observation window of 5/5ms, and prediction window of 4/5ms/5ms, and non-ideal CSI channel estimation: - AI/ML based CSI prediction outperforms the benchmark of the Rel-16 Type II with nearest historical CSI (baseline #1), where - 7% and 29% gains in terms of mean-UPT are observed at RU of 20% and 50%, respectively. - 14% and 77% gains in terms of 5%-UPT are observed at RU of 20% and 50%, respectively. - AI/ML based CSI prediction outperforms the benchmark of Rel-18 Type II non-AI/ML based CSI prediction (baseline #2), where - 13% and 35% gains in terms of mean-UPT are observed at RU of 20% and 50%, respectively. - 23% and 73% gains in terms of 5%-UPT are observed at RU of 20% and 50%, respectively. - Rel-16 Type II with nearest historical CSI (baseline #1) outperforms Rel-18 Type II non-AI/ML based CSI prediction (baseline #2) From the perspective of basic performance gain over non-AI/ML benchmark, under the assumption of the same UE speed of 60km/h for training and inference, observation window of 5/5ms, and prediction window of 1/5ms/5ms, and non-ideal CSI channel estimation: - AI/ML based CSI prediction outperforms the benchmark of the Rel-16 Type II with nearest historical CSI (baseline #1), where - 5% and 21% gains in terms of mean-UPT are observed at RU of 20% and 50%, respectively. - 9% and 45% gains in terms of 5%-UPT are observed at RU of 20% and 50%, respectively. - AI/ML based CSI prediction outperforms the benchmark of Rel-18 Type II non-AI/ML based CSI prediction (baseline #2), where - 11% and 31% gains in terms of mean-UPT are observed at RU of 20% and 50%, respectively. - 17% and 66% gains in terms of 5%-UPT are observed at RU of 20% and 50%, respectively. - Rel-16 Type II with nearest historical CSI (baseline #1) outperforms Rel-18 Type II non-AI/ML based CSI prediction (baseline #2) From the perspective of basic performance gain over non-AI/ML benchmark, under the assumption of the same UE speed of 60km/h for training and inference, observation window of 5/5ms, and prediction window of 4/5ms/5ms, and non-ideal CSI channel estimation: - AI/ML based CSI prediction outperforms the benchmark of the Rel-16 Type II with nearest historical CSI (baseline #1), where - 5% and 21%gains in terms of mean-UPT are observed at RU of 20% and 50%, respectively. - 4% and 26% gains in terms of 5%-UPT are observed at RU of 20% and 50%, respectively. - AI/ML based CSI prediction outperforms the benchmark of Rel-18 Type II non-AI/ML based CSI prediction (baseline #2), where - 13% and 32% gains in terms of mean-UPT are observed at RU of 20% and 50%, respectively. - 19% and 56% gains in terms of 5%-UPT are observed at RU of 20% and 50%, respectively. - Rel-16 Type II with nearest historical CSI (baseline #1) outperforms Rel-18 Type II non-AI/ML based CSI prediction (baseline #2) |
| --- | --- |
| Spreadtrum | Observation 1: For CSI prediction, with observation window of 5/5ms and prediction window of 1/5ms/5ms, - Compared to AR based CSI prediction (non-AI based CSI prediction), about 4% performance gain is observed with AI/ML based CSI prediction. |
| InterDigital | N4=1 At speed 10 km/h without spatial consistency, for N4=1, AIML-based CSI prediction achieves roughly the same performance as the non-AIML based CSI prediction (KF) in terms of mean UPT and exhibits less than 1% loss in terms of 5% UPT. At speed 30 km/h without spatial consistency, for N4=1, AIML-based CSI prediction achieves roughly the same performance (in the range of -0.4% to 0.4%) as the non-AIML based CSI prediction (KF) in terms of mean UPT. In terms of 5% UPT, the AI/ML model exhibits a 7.4% gain over KF for low RU, and 2% loss for high RU. At speed 60 km/h without spatial consistency, for N4=1, the non-AIML based CSI prediction (KF) outperforms the AIML-based CSI prediction by up to 6% for mean UPT and up to 10% for 5% UPT. The performance of the AI/ML model degrades relative to the non-AI/ML CSI prediction (KF) as the resource utilization increases N4=4 At speed 10 km/h without spatial consistency, for N4=4, AIML-based CSI prediction outperforms the non-AIML based CSI prediction (KF) in terms of mean UPT, with a relative gain of 1%-5%. For cell edge users and high RU, the AI/ML exhibits about 10% degradation compared to KF. At speed 30 km/h without spatial consistency, for N4=4, AIML-based CSI prediction outperforms the non-AIML based CSI prediction (KF) for both mean and 5th percentile UPT. The relative throughput gain is in the range of 1%-5% for mean UPT, while the largest improvement of about 28% is for cell edge users and large RU. At speed 60 km/h without spatial consistency, for N4=4, AI/ML shows a relative mean UPT loss of 13% and 20% compared to KF, for medium RU and high RU, respectively. For the 5%th percentile UPT, the AI/ML shows a relative gain of 10% and 27% over the KF, for medium RU and high RU, respectively. |
| Vivo | w/ Spatial consistency, N4=1 Compared to benchmark 1, the AI-based CSI prediction method can achieve SGCS gain of 113.9% and SE gain of 75.5% in the case of spatial consistency. Compared to benchmark 2, the AI-based CSI prediction method can achieve SGCS gain of 48.8% and SE gain of 14.8% in the case of spatial consistency. N4=3 Compared to benchmark 2, the AI-based CSI prediction method can achieve SGCS gain of 63.6% in the case of N4=3. The SGCS gain becomes higher when predicted CSIs are compressed by Rel-18 DD codebook. w/o spatial consistency Compared to benchmark 1, the AI-based CSI prediction method can achieve SGCS gain of 86.5% and SE gain of 86.4% in the case of non-spatial consistency. Compared to benchmark 2, the AI-based CSI prediction method can achieve SGCS gain of 26.6% and SE gain of 20.0% in the case of non-spatial consistency. |
| Apple | Table II: SGCS comparison of layer 1 eigen-vector with different baseline method 1/5ms/5ms. Sample and hold; 0.81. Wiener filter (ap-CSI-RS, offline calculated filter coefficients); 0.83. Wiener filter (filter update every 5ms with p-CSI-RS); 0.99. AI (LSTM) based approach; 0.97. |
| CATT | Observation 1: For CSI prediction with observation window of 5/5ms and prediction window of 1/5ms/5ms, compared with AR based CSI prediction, - For FTP traffic with UE speed at 30km/h, AI/ML based CSI prediction achieves the SGCS gain of 8.5%, the mean UPT loss of 2.1%, and the 5% UPT gain of 50.1%; - For FTP traffic with UE speed at 60km/h, AI/ML based CSI prediction achieves the SGCS gain of 7.83%, the mean UPT loss of 6.1%, and the 5% UPT gain of 47.1%； - For full buffer traffic with UE speed at 30km/h, AI/ML based CSI prediction achieves the SGCS gain of 17.8%, the mean UPT gain of 1.2% , and the 5% UPT gain of 6%； - For full buffer traffic with UE speed at 60km/h, AI/ML based CSI prediction achieves the SGCS gain of 12.5%, the mean UPT gain of 0.2%, and the 5% UPT gain of 0.4%. |
| China Mobile | Observation 1: AI/ML based CSI prediction can achieve very high prediction accuracy compared with baseline non-prediction in terms of SGCS. |
| CMCC | Observation 1: AI/ML based CSI prediction can achieve very high prediction accuracy compared with baseline non-prediction in terms of SGCS. Observation 2: The performance of both baseline (without CSI prediction) and AI based CSI prediction will decrease when UE moves faster. |
| Lenovo | Intermediate KPI Compared with sample-and hold CSI prediction scheme, NMSE for AI/ML-based CSI prediction scheme achieves a 13.11 dB improvement in NMSE and 16.25% improvement in MIMO Layer-1 SGCS gain for a 5ms CSI prediction window AI/ML-based CSI prediction achieves marginal gains over auto-regression based CSI prediction scheme with respect to NMSE and MIMO Layer-1 SGCS for a 5ms CSI prediction window The performance of the AI/ML-based CSI prediction scheme for a 10ms prediction window, with respect to NMSE and SGCS KPIs, degrades significantly compared with the same scheme for a 5ms prediction window UPT System performance evaluations show that with respect to mean UPT, the AI/ML-based CSI prediction achieves 24% and 51% gains over AR-based and sample-and-hold CSI prediction schemes, respectively, over a 5ms CSI prediction window System performance evaluations show that with respect to 5%-ile UPT over a 5ms CSI prediction window, the AI/ML based CSI prediction achieves: - Similar performance to AR-based CSI prediction scheme - 10.5% gain over sample-and-hold CSI prediction scheme |
| Fujitsu | Observation 1: - AI/ML based CSI prediction can provide significant performance gain over sampling and hold. For full buffer traffic, the mean UPT gain of 27% and the 5%-tile UPT gain of 7% can be observed; For FTP traffic, the mean UPT gain of 31.7% and the 5%-tile UPT gain of 51.7% can be observed. Observation 2: - AI/ML based CSI prediction can also provide obvious gain over AR based prediction. For full buffer traffic, the mean UPT gain of 10.8% and the 5%-tile UPT gain of 4.4% can be observed; For FTP traffic, the mean UPT gain of 9.2% and the 5%-tile UPT gain of 20.7% can be observed. |
| Oppo | Observation 2: Both non-AI and AI CSI prediction outperform sample-and-hold from the perspective of NMSE on raw channel and SGCS on CSI eigenvector. Observation 3: The SGCS performance increases with larger observation window for both non-AI and AI CSI prediction. Observation 4: AI-based CSI prediction outperforms auto-regression CSI prediction on all predicted slots. Proposal 1: For the intermediate KPI comparison on CSI prediction, focus on SGCS performance on CSI eigenvector, instead of NMSE on raw channel. |
| Nokia | Observation 2: With ideal CSI-RS, the MMSE predictor outperforms the AI/ML CSI predictor in relative throughput when operating in frequency-port-time domain. |
| MediaTek | Based on the intermediate KPI and eventual KPI results, it is observed that non-AI based CSI prediction can achieve similar performance as AI/ML-based CSI prediction. |
| IIT | Observation 1: Based on Intermediated KPI as NMSE it’s been observed that AI/ML based CSI Prediction performs better than Benchmark of Nearest Historical CSI |

Based on submitted result in excel sheet (i.e. Table 6,7, and X), proposed observations up to this meeting are as follows:

Table 2-1. For SGCS gain over Benchmark 1

|  | 10km/h | 30km/h | 60km/h | 120km/h | Remarks |
| --- | --- | --- | --- | --- | --- |
| Lenovo |  | 16.25% |  |  |  |
| Apple |  | 20% |  |  |  |
| Samsung | [17%/62.94%/23.12%/-1.61%] | 9.55% |  |  |  |
| Vivo |  | 62.8%, 65.5%, Depending on channel estimation error modelling (std: 0.3, 0.1) | 90.6%, (OW: 10/4ms) | 68.2% (OW:10/2ms) | Spatial consistency on, additive Gaussian error modelling |
| China Telecom |  | 13.48% | 10.52% |  |  |
| Spreadtrum |  | 9.99% |  |  |  |
| Fujitsu |  | 31.65% 29.89% (w/ channel estimation error+phase discontinuity w/o phase correction) 52.87% (w/phase correction) |  |  |  |
| OPPO |  | [20.6%, 25.7%, 23.8%, 19.7%] OW:5/5ms [21%, 26.5%, 27.5%, 24.4%] OW:10/5ms |  |  |  |
| CMCC |  | 10.81% |  |  |  |
| ZTE |  | 17.83%, 25.5% 34.66% Depending on traffic load (20%,50%,70%) |  |  |  |
| Intel |  | 31.1%, 30.8%, 23.2%, Depending on channel estimation error (none/6dB/9dB) |  |  | additive Gaussian error modelling |
| Qualcomm |  | 8.5% | 2.8% |  | Eigenvector |
| MediaTek |  | [76.4%, 59.4%, 30.5%, 20.9%] | [16.2%, 5.96%, 10.8%, 9.54%] |  | Spatial consistency on |
| Huawei |  |  | 18% |  |  |
| Ericsson |  | [23%, 27%, 34%, 29%] | [22%, 14%, 7%, 7%] |  | Spatial consistency on, channel estimation error is modelled |

Table 2-2. For NMSE gain over Benchmark 1

|  | 10km/h | 30km/h | 60km/h | 120km/h | Remarks |
| --- | --- | --- | --- | --- | --- |
| Lenovo |  | 13.11dB |  |  |  |
| ZTE |  | 7.694dB,7.055dB,7.354dB Depending on traffic load (20%,50%,70%) |  |  |  |
| Intel |  | 10.4 dB 8.27 dB 7.48 dB Depending on channel estimation error (none/6dB/9dB) |  |  |  |
| Ericsson |  | [6 9 8 5] | [8 5 7 6] |  | Mod: is it gain in a unit of dB?? |

Observation #2.1-A (SGCS performance over Benchmark 1)

For the CSI prediction using UE-sided model, till the RAN1#117 meeting, compared to the Benchmark#1 of the nearest historical CSI, in terms of SGCS, from UE speed perspective, in general the gain of AI/ML based solution is related with the UE speed:

- If spatial consistency is not adopted,

  - For 10km/h UE speed, 1 source [Samsung] observes 17% gain

  - For 30km/h UE speed, 11 sources [Lenovo, Apple, Samsung, China Telecom, Spreadtrum, OPPO, CMCC, ZTE, Intel, Fujitsu] observe 8.5%~31.65% gain

  - For 60km/h UE speed, 3 sources [Qualcomm, China Telecom, Huawei] observe 2.8%~18% gain

- If spatial consistency is adopted,

  - For 30km/h UE speed, 1 source [Ericsson] observe 23% gain, 2 sources [vivo, MediaTek] observe 62.8%~76.4% gain

  - For 60km/h UE speed, 2 sources [Ericsson, MediaTek] observe 16.2~-22% gain, 1 sources [vivo] observe 90.6% gain

  - For 120km/h UE speed, 1 source [vivo] observes 68.2% gain which are in general smaller than 30km/h UE speed.

- Note: the above results are based on the following assumptions

- The observation window considers to start as early as 15ms~50ms.

- A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

- 1 source [Qualcomm] considers eigenvector as model input, and other sources considers Raw channel matrix as model input.

- 1 source [Ericsson] considers beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, and other sources do not consider pre/post processing.

- The performance metric is SGCS in linear value for layer 1.

- Note: Results refer to Table 2-1 of [TBD: R1-240xxxx]

Observation #2.1-B (NMSE performance over Benchmark 1)

For the CSI prediction using UE-sided model, till the RAN1#117 meeting, compared to the Benchmark#1 of the nearest historical CSI, in terms of NMSE, from UE speed perspective, in general the gain of AI/ML based solution is related with the UE speed:

- For 30km/h UE speed, 3 sources [Lenovo, ZTE, Intel] observe 7.694 dB~13.11dB gain

- Note: the above results are based on the following assumptions

- The observation window considers to start as early as 15ms~50ms.

- A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

- The performance metric is NMSE in linear value for layer 1.

- No post processing is considered.

- No spatial consistency is considered.

- Note: Results refer to Table 2-2 of [TBD: R1-240xxxx]

Please provide your view on Observation #2.1-A/B.

| Company | Views |
| --- | --- |
| Mod | @ Ericsson: please check your unit for NMSE results. |
| Huawei, HiSilicon | questions/comments to our FL that apply to both Observation #2.1A and #2.1-B: - It is not clear for us the following two bullets are based on which rows of Table. 6. We appreciate our FL clarification about these. - The observation window considers to start as early as 15ms~50ms. - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric. Mod: You can refer to observation window and prediction window. Actually, this comes from Rel-18 observation. Based on submitted results so far, I updated accordingly. Comment regarding Observation #2.1-A: Suggest not to draw any conclusion regarding simulation results with spatial consistency due to limited companies that simulated this case with very diverse results. Mod: In Rel-18, the # of results w/ spatial consistency is 4 and it was captured in the TR. But, I understand your concern, so I made bracket and discuss it further whether we need to capture it or not. Comment regarding Observation #2.1-B: Since only 30 km/h is simulated by three companies, we think this should be removed from the observation: “from UE speed perspective, in general the gain of AI/ML based solution is related with the UE speed” Mod: As Ericsson provide 60km/h result, let’s keep the wording as it is. |
| Xiaomi | For observation #2.1-B, it seems that the pre-processing is not considered as well. Hence, “No post processing is considered” should be reworded as “No pre/post processing is considered”. Mod: Thanks for pointing this out. |
| Qualcomm | For 30km/h, we also see 8.5% gain, but it’s not captured in the observation #2.1-A. Besides, - In observation A/B, the range of gain is a bit large, especially for 30km/h, no spatial consistency modelling 8.5%~31.5%, would be better narrow down to make the observation more specific and concrete. - Codebook type need to be captured in the observation (the codebook type for benchmark 1 is ideal SVD or Rel-16 eTypeII or Rel-18) Mod: Sorry for missing your results. I added Qualcomm in updated version. Thanks for pointing this out. For your 1st comment, I updated, please check below. For your 2nd comment, since this is intermediate KPI, we don’t need to mention codebook type here. |
| Ericsson | Our NMSE results are provided in the unit of dB. Please capture our results in Observation #2.1-B. Thanks. |
| Intel | As we also use the beam-delay domain we need to be added to the related bullet. This would transform the bullet into (changes in red): - 12 sources [Ericsson, Intel] considers beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, and other sources do not consider pre/post processing. |
| Samsung | Thank you FL. Sorry for not making our input clear. We have also considered per, processing. We considered Rank-1 full channel matrix H_1=uvH, where u and v are the dominant left and right eigenvectors. - Note: the above results are based on the following assumptions - The observation window considers to start as early as 15ms~50ms. - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric. - 1 source [Qualcomm] considers eigenvector as model input, and other sources considers Raw channel matrix as model input. - 1 source [Ericsson] considers beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, and other sources do not consider pre/post processing. - 1 source [Samsung] considers Rank-1 full channel matrix H1=uvH, where u and v are the dominant left and right eigenvectors. - The performance metric is SGCS in linear value for layer 1. - Note: Results refer to Table 2-1 of [TBD: R1-240xxxx] Mod: Then, your assumption may be belonging to raw channel matrix, right? - 1 source [Qualcomm] considers eigenvector as model input, and other sources considers Raw channel matrix as model input. |
| ZTE | We are generally fine with the proposal structure. However, maybe one more important question is, if only few companies (e.g., 1 or 2) provided simulation results for one particular case, do we include them in the observation and do we use them to draw conclusion? Mod: I’m ok to remove the bullet with few companies results. Let’s hear other views from companies. Anyway, this observation will be updated based on more results (hopefully) in the next meeting. |

- Observation #2.1-A (SGCS performance over Benchmark 1) (updated)

For the CSI prediction using UE-sided model, till the RAN1#117 meeting, compared to the Benchmark#1 of the nearest historical CSI, in terms of SGCS, from UE speed perspective, in general the gain of AI/ML based solution is related with the UE speed:

- If spatial consistency is not adopted,

  - For 10km/h UE speed, 1 source [Samsung] observes 17% gain

  - For 30km/h UE speed, 11 sources [Lenovo, Apple, Samsung, China Telecom, Spreadtrum, OPPO, CMCC, ZTE, Intel, Fujitsu, Qualcomm] observe 8.5%~31.65% gain

  - For 30km/h UE speed, 9 sources [Lenovo, Apple, Samsung, China Telecom, Spreadtrum, OPPO, CMCC, ZTE, Qualcomm] observe 8.5%~20.6% gain and 2 sources [Intel, Fujitsu] observe 31.1%~31.65% gain.

  - For 60km/h UE speed, 3 sources [Qualcomm, China Telecom, Huawei] observe 2.8%~18% gain

- [If spatial consistency is adopted,

  - For 30km/h UE speed, 1 source [Ericsson] observe 23% gain, 2 sources [vivo, MediaTek] observe 62.8%~76.4% gain

  - For 60km/h UE speed, 2 sources [Ericsson, MediaTek] observe 16.2~-22% gain, 1 sources [vivo] observe 90.6% gain

  - For 120km/h UE speed, 1 source [vivo] observes 68.2% gain which are in general smaller than 30km/h UE speed.]

- Note: the above results are based on the following assumptions

- The observation window considers to start as early as 2015ms~50ms.

- A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

- 1 source [Qualcomm] considers eigenvector as model input, and other sources considers Raw channel matrix as model input.

- 21 source [Ericsson, Intel] considers beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, and other sources do not consider pre/post processing.

- The performance metric is SGCS in linear value for layer 1.

- Note: Results refer to Table 2-1 of R1-2405489 [TBD: R1-240xxxx]

- Observation #2.1-B (NMSE performance over Benchmark 1) (updated)

For the CSI prediction using UE-sided model, till the RAN1#117 meeting, compared to the Benchmark#1 of the nearest historical CSI, in terms of NMSE, from UE speed perspective, in general the gain of AI/ML based solution is related with the UE speed:

- For 30km/h UE speed, 4 sources [Lenovo, ZTE, Intel, Ericsson] observe 6 dB~13.11dB gain

- For 60km/h UE speed, 1 source [Ericsson] observe 8 dB gain

- Note: the above results are based on the following assumptions

- The observation window considers to start as early as 15ms~50ms. 20ms~25ms.

- A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

- The performance metric is NMSE in linear value for layer 1.

- No pre/post processing is considered.

- 1 source [Ericsson] considers spatial consistency, and other sources do not consider spatial consistency.

- Note: Results refer to Table 2-2 of R1-2405489 [TBD: R1-240xxxx]

Please provide your view on updated version Observation #2.1-A/B. And provide your view whether or not to capture results from one or two companies.

| Company | Views |
| --- | --- |

Table 2-3. For SGCS gain over Benchmark 2

|  | 10km/h | 30km/h | 60km/h | 120km/h | Remarks |
| --- | --- | --- | --- | --- | --- |
| Lenovo |  | 1% |  |  |  |
| Apple |  | 17%, -2% Depending on whether filter update is done for Benchmark 2 |  |  |  |
| Vivo |  | 25.7%,10.4%, Depending on channel estimation error modelling (std: 0.3, 0.1) | 48.8%, (OW: 10/4ms) | 20.6% (OW:10/2ms) | Spatial consistency on, Gaussian channel estimation nmodeling |
| Spreadtrum |  | 4.88% |  |  |  |
| Fujitsu |  | 26.29% 7.26% (w/ channel estimation error+phase discontinuity w/o phase correction) 25.6% (w/phase correction) |  |  |  |
| OPPO |  | 1.8~5.6% |  |  |  |
| ZTE |  | 2.54%, 14.55% 26.45% Depending on traffic load (20%,50%,70%) |  |  |  |
| Intel |  | 3.08%, 10.7%, 3.67%, Depending on channel estimation error (none/6dB/9dB) |  |  | additive Gaussian error modelling |
| Qualcomm |  | 0% | -20.6%, -21.7% For OW: 5/5ms, 10/5ms |  | Eigenvector |
| MediaTek |  | [0.43% 0.45% 0.97% 0.45%] | [-1.01% 0.51% 1.16% 1.52%] |  | Spatial consistency on |
| Huawei |  |  | 3.6% |  |  |
| Ericsson |  | [5%, 19%, 28%, 29%] | [3%, 11%, 11% 16%] |  | Spatial consistency on, channel estimation error is modelled |
| CATT |  | 8.5% | 7.8% |  | additive Gaussian error modelling |
| InterDigital | [3% 5% 3% 1%] | [48% 21% -1% -14%] | [42% 9% -5% -11%] |  |  |

Table 2-4. For NMSE gain over Benchmark 2

|  | 10km/h | 30km/h | 60km/h | 120km/h | Remarks |
| --- | --- | --- | --- | --- | --- |
| Lenovo |  | 1.29dB |  |  |  |
| ZTE |  | 1.424dB, 2.531dB, 3.372dB Depending on traffic load (20%,50%,70%) |  |  |  |
| Intel |  | 1.22 dB 0.13 dB -0.45 dB Depending on channel estimation error (none/6dB/9dB) |  |  |  |
| Ericsson |  | [1 2 2 2] | [1 2 2 2] |  |  |

Observation #2.1-C (SGCS performance over Benchmark 2)

For the CSI prediction using UE-sided model, till the RAN1#117 meeting, compared to the Benchmark#2 of non-AI based CSI prediction, in terms of SGCS, from UE speed perspective, in general the gain of AI/ML based solution is related with the UE speed:

- If spatial consistency is not adopted,

  - For 10km/h UE speed, 1 source [InterDigital] observes 3% gain

  - For 30km/h UE speed, 6 sources [Qualcomm, Lenovo, Spreadtrum, Apple, Intel, CATT] observe -2 %~8.5% gain, and 2 sources [Fujitsu, InterDigital] observe 26.3%~48% gain, 1 source [Apple] observe

  - For 60km/h UE speed, 1 source [Qualcomm] observes -20.6% gain, 2 sources [Huawei, CATT] observe 3.6%~7.8% gain, 1 source [InterDigital] observes 42% gain

- If spatial consistency is adopted,

  - For 30km/h UE speed, 2 sources [Ericsson, MediaTek] observe 0.43%~5% gain, 1 source [vivo] observes 25.7% gain

  - For 60km/h UE speed, 2 sources [Ericsson, MediaTek] observe -1%~3% gain, 1 source [vivo] observes 48.8% gain

  - For 120km/h UE speed, 1 source [vivo] observes 20.6% gain

- Note: the above results are based on the following assumptions

- The observation window considers to start as early as 15ms~50ms.

- A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

- 1 source [Qualcomm] considers eigenvector as model input, and other sources considers Raw channel matrix as model input.

- 1 source [Ericsson] considers beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, and other sources do not consider pre/post processing.

- The performance metric is SGCS in linear value for layer 1.

- Note: Results refer to Table 2-3 of [TBD: R1-240xxxx]

Observation #2.1-D (NMSE performance over Benchmark 2)

For the CSI prediction using UE-sided model, till the RAN1#117 meeting, compared to the Benchmark#1 of non-AI based CSI prediction, in terms of NMSE, from UE speed perspective, in general the gain of AI/ML based solution is related with the UE speed:

- For 30km/h UE speed, 3 sources [Lenovo, ZTE, Intel] observe 1.22 dB~1.42dB gain

- Note: the above results are based on the following assumptions

- The observation window considers to start as early as 15ms~50ms.

- A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

- The performance metric is NMSE in linear value for layer 1.

- No post processing is considered.

- No spatial consistency is considered.

- Note: Results refer to Table 2-4 of [TBD: R1-240xxxx]

Please provide your view on Observation #2.1-C/D.

| Company | Views |
| --- | --- |
| Mod | @ Ericsson: please check your unit for NMSE results. @ OPPO: please provide gain in a unit of % |
| Huawei, HiSilicon | questions/comments to our FL that apply to both Observation #2.1C and #2.1-D: - It is not clear for us the following two bullets are based on which rows of Table. 6. We appreciate our FL clarification about these. - The observation window considers to start as early as 15ms~50ms. - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric. Mod: please refer to previous comment Comment regarding Observation #2.1-C: The results for different speeds are very diverse. Only one company simulated 10 km/h. The gain for 30 km/h ranges from -2% to 48% (based on 8 companies results) and the gain for 60 km/h ranges from -20% to 42% (based on 4 companies results). Based on these results, we don’t think it is warranted to mention that “in general the gain of AI/ML based solution is related with the UE speed” since such conclusion implies that there is an obvious trend in the AI/ML based prediction gain when the speed changes (i.e., gain clearly decreases or increases). We don’t see such trend in the simulation results. It might be more accurate to mention that “several companies observed considerable gain for 30 km/h speed. The gain/loss for other speeds is less conclusive.” Comment regarding Observation #2.1-D: Since only 30 km/h is simulated by three companies, we think this should be removed from the observation: “from UE speed perspective, in general the gain of AI/ML based solution is related with the UE speed”. There is no speed other than 30 km/h simulated by any company to draw such conclusion. Mod: please refer to previous comment |
| Xiaomi | For observation #2.1-D, it seems that the pre-processing is not considered as well. Hence, “No post processing is considered” should be reworded as “No pre/post processing is considered”. For Observation #2.1-C, larger performance gain divergence is observed. |
| Qualcomm | Same comments as for observation A/B for range of the gain and codebook type. |
| Ericsson | Our results for NMSE gain over benchmark #2 are not correct in the table, the NMSE gains are [1 2 2 2] and [1 2 2 2] for 30km/h and 60km/h, respectively. The results are provided in the unit of dB. |
| Intel | As we also use the beam-delay domain we need to be added to the related bullet. This would transform the bullet into (changes in red): - 12 sources [Ericsson, Intel] considers beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, and other sources do not consider pre/post processing. |
| Apple | For Apple’s results, depending on whether filter is updated, the gain is different. To perform filter update, we are assuming the filter update window is not the same as observation window, to get accurate covariance update. Therefore, we separate ap-CSI-RS and p-CSI-RS. The updated version only applies to p-CSI-RS. The summary missed many detailed assumption. If we do not want to capture the details, please update Apple’s result on benchmark 2 to -2% - 18% depending on filter update. We also suggest adding a note these results assume no channel estimation error and no phase continuity is modelled in filter update. |
| OPPO | Thanks FL. Regarding the SGCS performance gain over benchmark 2, our results are 1.8%~5.6%. And please add our results in the observation. |
| Mod | Based on the comments, I made some modification as below |

- Observation #2.1-C (SGCS performance over Benchmark 2) (updated)

For the CSI prediction using UE-sided model, till the RAN1#117 meeting, compared to the Benchmark#2 of non-AI based CSI prediction, in terms of SGCS, from UE speed perspective, in general the gain of AI/ML based solution is related with the UE speed:

- If spatial consistency is not adopted,

  - For 10km/h UE speed, 1 source [InterDigital] observes 3% gain

  - For 30km/h UE speed,

    - 6 sources [Qualcomm, Lenovo, Spreadtrum, Apple, Intel, CATT, OPPO] observe -2 0%~8.5% gain,

    - 2 sources [Fujitsu, InterDigital] observe 26.3%~48% gain,

    - 1 source [Apple] observe -2% - 18% depending on filter update

  - For 60km/h UE speed, 1 source [Qualcomm] observes -20.6% gain, 2 sources [Huawei, CATT] observe 3.6%~7.8% gain, 1 source [InterDigital] observes 42% gain

- If spatial consistency is adopted,

  - For 30km/h UE speed, 2 sources [Ericsson, MediaTek] observe 0.43%~5% gain, 1 source [vivo] observes 25.7% gain

  - For 60km/h UE speed, 2 sources [Ericsson, MediaTek] observe -1%~3% gain, 1 source [vivo] observes 48.8% gain

  - For 120km/h UE speed, 1 source [vivo] observes 20.6% gain

- Note: the above results are based on the following assumptions

- The observation window considers to start as early as 15ms~50ms.

- A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

- 1 source [Qualcomm] considers eigenvector as model input, and other sources considers Raw channel matrix as model input.

- 21 source [Ericsson, Intel] considers beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, and other sources do not consider pre/post processing.

- [No channel estimation error and no phase continuity is modelled in filter update for Benchmark 2]

- The performance metric is SGCS in linear value for layer 1.

- Note: Results refer to Table 2-3 of R1-2405489 [TBD: R1-240xxxx]

- Observation #2.1-D (NMSE performance over Benchmark 2)(updated)

For the CSI prediction using UE-sided model, till the RAN1#117 meeting, compared to the Benchmark#1 of non-AI based CSI prediction, in terms of NMSE, from UE speed perspective, in general the gain of AI/ML based solution is related with the UE speed:

- For 30km/h UE speed, 4 3sources [Ericsson, Lenovo, ZTE, Intel] observe 1 dB~1.42dB gain

- For 60km/h UE speed, 1 source [Ericsson] observe 1 dB gain

- Note: the above results are based on the following assumptions

- The observation window considers to start as early as 15ms~50ms. 20ms~25ms.

- A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

- The performance metric is NMSE in linear value for layer 1.

- [No channel estimation error and no phase continuity are modelled in filter update for Benchmark 2]

- No pre/post processing is considered.

- No spatial consistency is considered.

- Note: Results refer to Table 2-4 of R1-2405489 [TBD: R1-240xxxx]

Please provide your view on updated Observation #2.1-C/D.

| Company | Views |
| --- | --- |
| Mod | @ZTE, with add bullet, “No channel estimation error and no phase continuity is modelled in filter update for Benchmark 2” I cannot able to capture your results. Please provide results w/o channel estimation error modelling. |

Table 2-5. UPT gain over Benchmark 1 for traffic model

|  | 30km/h |  |  |  |  |  | 60km/h |  |  |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mean UPT |  |  | 5% UE UPT |  |  | Mean UPT |  |  | 5% UE UPT |  |  |  |
|  | RU: low | RU: mid | RU: high | RU: low | RU: mid | RU: high | RU: low | RU: mid | RU: high | RU: low | RU: mid | RU: high |  |
| Huawei |  |  |  |  |  |  | 1.2%/1.7%/2.1% | 1.8%/3.9%/3.5% | 2.5%/3.1%/4.2% | 4.5%/18.3%/8% | 11.3%/9.3%/8.6% | 17.8%/13.4%/16.4% | PW: 5/5ms |
| Ericsson | 9% | 37% |  | 27% | 100% |  | 5% | 21% |  | 9% | 45% |  | PW: 1/5/5ms w/ spatial consistency |
|  | 7% | 29% |  | 14% | 77% |  | 5% | 21% |  | 4% | 26% |  | PW: 4/5/5ms w/ spatial consistency |

Table 2-6. UPT gain over Benchmark 2 for traffic model

|  | 30km/h |  |  |  |  |  | 60km/h |  |  |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mean UPT |  |  | 5% UE UPT |  |  | Mean UPT |  |  | 5% UE UPT |  |  |  |
|  | RU: low | RU: mid | RU: high | RU: low | RU: mid | RU: high | RU: low | RU: mid | RU: high | RU: low | RU: mid | RU: high |  |
| Huawei |  |  |  |  |  |  | 0.7%/0.9%/0.8% | 2.3%/3.1%/2.5% | 2.4%/2%/2.5% | 2.5%/5.2%/4.3% | 7.7%/7%/8.6% | 14.8%/6.7%/10.2% |  |
| CATT | -2.1% |  |  | 50.1% |  |  | -6.1% |  |  | 47.1% |  |  |  |
| Ericsson | 9% | 24% |  | 18% | 46% |  | 11% | 31% |  | 17% | 66% |  | PW: 1/5/5ms w/ spatial consistency |
|  | 13% | 35% |  | 23% | 73% |  | 13% | 32% |  | 19% | 56% |  | PW: 4/5/5ms w/ spatial consistency |
| InterDigital | 2.1% | 2.5% | 5.7% | 5.7% | 6.8% | 28.6% | -13.1% | -19.6% | -17.2% | 9.5% | 27.2% | 27.2% | PW: 4/5/5ms |
| MediaTek | 0.02% | -0.25% | 0.11% |  |  |  | 0.14% | 0.25% | 0.92% |  |  |  | PW: 4/5/5ms w/ spatial consistency |

Table 2-7. UPT gain over Benchmark 1 for full buffer model

|  | 30km/h |  | 60km/h |  | Remarks |
| --- | --- | --- | --- | --- | --- |
|  | Mean UPT | 5% UE UPT | Mean UPT | 5% UE UPT |  |
| Lenovo | 51% | 10.5% |  |  | PW: 1/5/5ms |
| vivo |  |  | 28.9% |  | PW: 1/4/4ms w/ spatial consistency |
| Fujitsu | 27% | 9.9% |  |  | PW: 1/5/10ms |
|  | 11.6% | 2.1% |  |  | PW: 4/5/10ms |
| MediaTek | 8.7% | 7.7% |  |  | PW: 1/5/5ms w/ spatial consistency |

Table 2-8. UPT gain over Benchmark 2 for full buffer model

|  | 30km/h |  | 60km/h |  | Remarks |
| --- | --- | --- | --- | --- | --- |
|  | Mean UPT | 5% UE UPT | Mean UPT | 5% UE UPT |  |
| CATT | 1,2% | 6.0% | 0.2% | 0.4% | PW: 1/5/5ms |
| Lenovo | 24% | 0.2% |  |  | PW: 1/5/5ms |
| vivo | 24.7% | 81.9% | 41.3% | 48.9% | PW: 3/5/5ms w/ spatial consistency w/ phase discontinuity modelled |
|  |  |  | 6.5% |  | PW: 1/4/4ms w/ spatial consistency w/o phase discontinuity modelled |
| Fujitsu | 9.7% | 2.6% |  |  | PW: 1/5/10ms |
|  | 7% | 6.3% |  |  | PW: 4/5/10ms |
| MediaTek | 0.6% | -2% |  |  | PW: 1/5/5ms w/ spatial consistency |

- Observation #2.1-E (Mean UE UPT performance) (updated)

For the CSI prediction using UE-sided model, till the RAN1#117 meeting, in terms of mean UPT, gains are observed compared to both Benchmark#1 of the nearest historical CSI and Benchmark#2 of a non-AI/ML based CSI prediction approach:

- Compared to the benchmark#1 of the nearest historical CSI:

  - For FTP traffic:

    - 1 source [Huawei] observes 4.5%~18.3% gain;

    - 1 source [Ericsson] observes 9%~ 100% gain.

  - For full buffer traffic:

    - 1 source [Lenovo] observes 10.5% gain.

    - 1 source [Fujitsu] observes 2.1%~9.9%11.6%~27% gain.

    - 1 source [MediaTek] observes 7.7% gain.

- Compared to the benchmark#2 of non-AI/ML based CSI prediction:

  - For FTP traffic:

    - 1 source [Huawei] observes 0.2%~14.8% gain;

    - 1 source [CATT] observes 47.1%~50.1%  -6.1%~-2.1% gain.

    - 1 source [Ericsson] observes 17%~73% gain.

    - 1 source [InterDigital] observes 5.7%~28.6% gain.

  - For full buffer traffic:

    - 1 source [CATT] observes 0.4%~6%  0.2%~1.2% gain.

    - 1 source [Lenovo] observes 0.2% gain.

    - 1 source [vivo] observes 6.5% gain.

    - 1 source [Fujitsu] observes 2.6%~6.3%7%~9.7% gain.

    - 1 source [MediaTek] observes -2% gain.

- Note: the above results are based on the following assumptions

  - The UE speed is 30km/h or 60km/h.

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms ~ 20ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - The performance metric is mean UPT for Max rank 1.

  - 1 source [Ericsson] considers beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, and other sources considers no pre/post processing.

  - 3 sources [vivo, Ericsson, MediaTek] consider spatial consistency. Other sources do not consider spatial consistency.

  - No channel estimation error and no phase continuity are modelled

  - Note: Results refer to Table 2-5/2-4/2-4/2-8 of [TBD: R1-240xxxx]

- Observation #2.1-F (5% UE UPT performance)(updated)

For the CSI prediction using UE-sided model, till the RAN1#117 meeting, in terms of 5% UE UPT, gains are observed compared to both Benchmark#1 of the nearest historical CSI and Benchmark#2 of a non-AI/ML based CSI prediction approach:

- Compared to the benchmark#1 of the nearest historical CSI:

  - For FTP traffic:

    - 1 source [Huawei] observes 1.2%~4.2% gain;

    - 1 source [Ericsson] observes 9%~ 35% gain.

  - For full buffer traffic:

    - 1 source [vivo] observes 86.5% gain.

    - 1 source [Lenovo] observes 51% gain.

    - 1 source [Fujitsu] observes 11.6%~27%2.1%~9.9% gain.

    - 1 source [MediaTek] observes 8.7% gain.

- Compared to the benchmark#2 of non-AI/ML based CSI prediction:

  - For FTP traffic:

    - 1 source [Huawei] observes 0.7%~3.1% gain;

    - 1 source [CATT] observes -6.1%~-2.1% 47.1%~50.1% gain.

    - 1 source [Ericsson] observes 9%~35% gain.

    - 1 source [InterDigital] observes 2.1%~19.6% gain.

    - 1 source [MediaTek] observes 0.02%~0.92% gain.

  - For full buffer traffic:

    - 1 source [CATT] observes 0.2%~1.2%  0.4%~6% gain.

    - 1 source [Lenovo] observes 24% gain.

    - 1 source [vivo] observes 24.7%~41.3% gain.

    - 1 source [Fujitsu] observes 7%~9.7%2.6%~6.3% gain.

    - 1 source [MediaTek] observes 0.6% gain.

- Note: the above results are based on the following assumptions

  - The UE speed is 30km/h or 60km/h.

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms ~ 20ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - The performance metric is mean UPT for Max rank 1.

  - 1 source [Ericsson] considers beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, and other sources considers no pre/post processing.

  - 3 sources [vivo, Ericsson, MediaTek] consider spatial consistency. Other sources do not consider spatial consistency.

  - No channel estimation error and no phase continuity are modelled

  - Note: Results refer to Table 2-5/2-4/2-4/2-8 of [TBD: R1-240xxxx]

Please provide your view on Observation #2.1-E/F.

| Company | Views |
| --- | --- |
| Huawei/HiSilicon | For a Table 2.5 and 2.6, please add PW: 1/5/5ms in the Remark section for Huawei/HiSilicon results. |
| Fujitsu | Thanks FL for the great efforts. It looks our results were not correctly captured in Observation #2.1-E/F. Please update as below, thanks. - Observation #2.1-E (Mean UE UPT performance) - Compared to the benchmark#1 of the nearest historical CSI: - For full buffer traffic: - 1 source [Fujitsu] observes 2.1%~9.9%11.6%~27% gain. - Compared to the benchmark#2 of non-AI/ML based CSI prediction: - For full buffer traffic: - 1 source [Fujitsu] observes 2.6%~6.3%7%~9.7% gain. - Observation #2.1-F (5% UE UPT performance) - Compared to the benchmark#1 of the nearest historical CSI: - For full buffer traffic: - 1 source [Fujitsu] observes 11.6%~27%2.1%~9.9% gain. - Compared to the benchmark#2 of non-AI/ML based CSI prediction: - For full buffer traffic: - 1 source [Fujitsu] observes 7%~9.7%2.6%~6.3% gain. |
| CATT | - It seems that the mean UPT and 5%-ile UPT performance are not correctly captured. Regarding CATT’s results, please use the following: - Observation #2.1-E (Mean UE UPT performance) - Compared to the benchmark#2 of non-AI/ML based CSI prediction: - For FTP traffic: - 1 source [CATT] observes 47.1%~50.1% -6.1%~-2.1% gain. - For full buffer traffic: - 1 source [CATT] observes 0.4%~6% 0.2%~1.2% gain. - Observation #2.1-F (5% UE UPT performance) - Compared to the benchmark#2 of non-AI/ML based CSI prediction: - For FTP traffic: - 1 source [CATT] observes -6.1%~-2.1% 47.1%~50.1% gain. - For full buffer traffic: - 1 source [CATT] observes 0.2%~1.2% 0.4%~6% gain. In addition, we suggest the FL to further summarize in the note of observations whether channel estimation error is modelled or not |
| Mod | Thanks for the comment, I revised observation as above. Please keep comment, if any. |
| vivo | vivo result in Observation #2.1-F consider phase discontinuity modelling and we update some incorrectly captured |
| Mod | Thanks for your input. |

- Complexity comparison compared to Non-AI based CSI prediction (Benchmark 2)

- Summary

For RAN1#117, several companies provide complexity comparison in terms of FLOPs compared to Non-AI based CSI prediction. The table below captures observations in their contribution

| Ericsson | For generating a CSI report, the computational complexity of the considered non-AI AR-based CSI prediction scheme consist of two parts, the complexity for calculating the AR model coefficients (AR-model parameter derivation) and the complexity of applying the AR model with the calculated coefficients to generate predicted CSIs (AR-model inference). Our AI model has a similar level of computational complexity in units of FLOPs per CSI report generation as compared to the considered non-AI AR-based CSI prediction benchmark. |
| --- | --- |
| Intel | Observation 1: - Complexity of non-AI/ML CSI prediction based on autoregressive statistical model varies from 1.5 MFLOPs to 0.05 MFLOPs depending on the applied pre-processing algorithm (with or without dimensionality reduction). - Dimensionality reduction for CSI prediction has minor impact on the CSI feedback performance since CSI available at the gNB side has reduced dimensionality due to CSI compression. Proposal 1: - RAN1 to consider CSI prediction based on autoregressive model as basic assumption for benchmark 2. |
| InterDigital | For 2 Rx antennas, 16 Tx antennas, 52 RBs, with K=5, N4=1, the overall number of FLOPs required for CSI prediction with the two models is approximately: - 3.5 million FLOPs for non AIML Kalman filter - 79 million FLOPs for the AI/ML CNN_Xformer model. Observation 7: The transformer-based AI/ML CSI prediction model has significantly higher complexity compared to the non-AI/ML CSI prediction using Kalman filters. |
| Apple | Observation 1: For CSI prediction using a non-AI based approach, complexity and performance varies with different assumption such as whether the filter is designed per UE with frequent filter updates, the granularity of filter etc. Observation 2: For non-AI based solution, main computation complexity is dominated by filter update. Observation 3: For CSI prediction using non-AI based approach, when ap-CSI-RS is used, filter update is difficult. |
| Fujitsu | Observation 3: - The complexity of AI/ML based CSI prediction is much larger than AR based CSI prediction in terms of FLOPS. |
| NEC | Proposal 1: Study to use model compression to reduce the complexity of the AI/ML model for CSI prediction. |
| OPPO | Observation 1: FLOPs of AI-based CSI prediction is about 13 times of non-AI benchmark when 4 predicted slots with 52RBs per channel sample. |
| Nokia | Observation 4: An initial complexity analysis suggests that Rel-18 non-AI/ML prediction is less computationally complex than AI/ML-based CSI prediction. However, additional issues must be considered including complexity reduction of prediction models. Proposal 2: Continue to study the relative complexity of AI/ML and non-AI/ML-based CSI prediction techniques, considering the possibility of AI/ML model complexity reduction/optimization. |
| MediaTek | Further study complexity reduction techniques to evaluate their potential in reducing the storage and computational complexities of AI/ML models for CSI prediction. |
| CEWiT | Proposal-2: Study the effect of pre-processing techniques to exploit sparsity better in the evaluation of CSI prediction. |

From the complexity assessments from almost all companies as in above, the complexity of non-AI CSI prediction is normally lower than that of AI/ML based CSI prediction although the complexity of non-AI CSI prediction varies according to assumptions such as frequency of filter updates, pre-processing algorithm. Meanwhile, Ericsson, MediaTek, NEC, Nokia and CEWiT propose to study on complexity reduction. As we discussed in RAN1#116bis, our first priority is performance comparison between AI/ML based CSI prediction and non-AI based CSI prediction. Hence, Moderator suggests that performance and complexity trade-off can be studied by each companies’ willingness.

Please provide your view on complexity issue, if any.

| Company | Views |
| --- | --- |
| Huawei/HiSilicon | Agree with the moderator to leave complexity reduction schemes to individual companies. No need to formally discuss such schemes in this or next meeting. |
| Fujitsu | Agree with FL. |
| Xiaomi | Agree with FL’s assessment. |
| Sony | Agree with FL |
| ETRI | We agree with FL’s view. |
| LGE | Agree with FL |

Additional evaluation assumptions.

- Channel estimation error and phase discontinuity

- Summary

In the contribution submitted to RAN1#117, several companies provide their evaluation results regarding chennal estimation error and phase discontinuity. The table below captures observations/proposals in their contribution.

Channel estimation error

| Ericsson | Proposal 1. To study impact of channel estimation errors on AI and non-AI CSI prediction, simulations should either include non-ideal/practical CSI-RS channel estimation, or use a model of channel estimation error with the error modeling assumptions presented together with the SLS results. |
| --- | --- |
| Intel | - Table 1. Preliminary evaluation of intermediate metric SGCS for cases with and without channel estimation error. Prediction Method; SGCS for Layer 1 (ideal CE); SGCS for Layer 1 (channel estimation error delta = 9 dB); SGCS for Layer 1 (channel estimation error delta = 6 dB). benchmark #1; 0.689; 0.665; 0.608. benchmark #2; 0.876 (+27.1 %); 0.790 (+18.8 %); 0.718 (+18.1 %). ML prediction; 0.903 (+31.1 %); 0.819 (+23.2 %); 0.795 (+30.8 %). - Table 2: Preliminary evaluation of intermediate metric NMSE of the channel matrix in dB for cases with and without channel estimation error. Prediction Method; NMSE in dB (ideal CE); NMSE in dB (channel estimation error delta = 9 dB); NMSE in dB (channel estimation error delta = 6 dB). Benchmark #1; 0.425; 0.560; 0.795. Benchmark #2; -8.75; -8.16; -6.55. ML prediction; -9.97; -7.71; -6.68. Observation 2: - AI/ML-based CSI prediction is more robust to channel estimation errors compared to non-AI/ML based CSI prediction (benchmark #2). |
| vivo | Compared to benchmark 2, the AI-based CSI prediction method can achieve SGCS gain of 25.7% when channel estimation error gets larger. The gain of AI-based scheme over benchmark 2 increases with the standard deviation of the channel estimation error. |
| LG Electronics | Proposal #1: For performance comparison with respect to channel estimation error, compare performance of 5% UE UPT, median UE UPT and 95% UE UPT. |
| ZTE | Observation 1: Regarding CSI prediction considering the channel estimation error - As the traffic load increases (as the interference increases), the SGCS of CSI prediction decreases. Under full buffer traffic (high interference case), the SGCS of AI/ML based CSI prediction is still as high as 0.7, while the SGCS of non-AI/ML based CSI prediction goes down to 0.51. - As the traffic load increases (as the interference increases), the SGCS gain of AI vs Wiener CSI prediction increases, and up to 37.67% gain can be observed under full buffer traffic. - As the traffic load increases (as the interference increases), overall, the SGCS gain of AI vs Sample&hold CSI prediction increases, and up to 43.02% gain can be observed under full buffer traffic. |
| Nokia | Observation 3: With non-ideal CSI-RS, the AI/ML CSI predictor operating in frequency-port-time domain yields about 10% performance gain over ZoH (measured in GCS). Proposal 1: Continue to study the performance of CSI predictors for non-ideal CSI-RS conditions, comparing SLS throughput results. |
| Mavenir | Proposal 1: Channel measurement error needs to be considered when evaluating and improving CSI prediction models. |

Tx/Rx Phase discontinuity

| vivo | The AI-based CSI prediction achieves significantly higher prediction accuracy and UPT over both benchmark 1 and benchmark 2 under impairments like channel estimation error and phase discontinuity. With the presence of phase discontinuity, AI achieves about 11%~54% SGCS gain over benchmark1 and 19%~30% SGCS gain over benchmark 2 With phase discontinuity modelled, AI based prediction shows much larger gain over non-AI based methods, e.g., up to 48.6% over benchmark 2 when Twindow=20ms. When phase discontinuity is high, e.g., ϕmax≥30o, AI based prediction can still provide satisfactory SGCS, e.g., around 0.8 - 0.9. With phase discontinuity modelling as ϕmax=40 degrees and Twindow=20ms, the AI-based prediction improves the gain on the 5% SE and average SE by 81.9% and 24.7% respectively compared to the non-AI prediction method at speed of 30km/h. With phase discontinuity modelling as ϕmax=40 degrees and Twindow=20ms, the AI-based prediction improves the gain on the 5% SE and average SE by 48.9% and 41.3% respectively compared to the non-AI prediction method at speed of 60km/h. |
| --- | --- |
| Qualcomm | For UE speed of 30km/h, AI/ML prediction yields 12.3% gain over historical CSI before CSI compression and reporting. After compression using R18 eT2 codebook, the gain decreases to 8.5%. For UE speed of 60km/h, the gain is 8% and 3.8% for predicted CSI and compressed CSI, respectively. Comparing with non-AI based prediction with h-in-h-out, before CSI compression, AI/ML with v-in-v-out shows 13.9% gain under the modelled phase discontinuity for UE speed of 30km/h and observation window of 10/5ms (SGCS 0.82 vs. 0.72). For UE speed of 60km/h, the gain is 13.3% (0.68 vs. 0.6). For CSI after compression using R18 eT2 codebook, AI/ML with v-in-v-out shows 11.7% gain under the modelled phase discontinuity for UE speed of 30km/h and observation window of 10/5ms (SGCS 0.76 vs. 0.68) For UE-side CSI prediction use case, further study the performance gain before discussion of specification impact. |

As shown above, it is observed that in the environment where channel estimation error or phase discontinuity is modelled, AI/ML based CSI prediction outperforms non-AI CSI prediction. [TBU: Based on the submitted results in excel sheet, observation for channel estimation error and phase discontinuity will be provided during the meeting period.]

Please provide your input, if any.

| Company | Views |
| --- | --- |
| Huawei/HiSilicon | There are limited simulation results for phase discontinuity (only two companies). So, we suggest to draw any conclusions regarding phase discontinuity if/when more companies submit their corresponding results. For channel estimation errors, multiple companies provided their UPT/intermediate KPI results but based on quite different modelling approaches (Variance of estimation error inversely proportional to SINR/geometry, fixed variance of estimation error, and “realistic channel modelling”). We think this should be captured in any possible observation/conclusion. |
| Fujitsu | Fine with FL’s suggestion. |
| Xiaomi | More simulation results are needed before giving any conclusions. |

- 20ms CSI-RS periodicity

- Summary

In the contribution submitted to RAN1#117, several companies provide their evaluation results under the assumption of 20ms CSI-RS periodicity. The table below captures observations/proposals in their contribution.

| Ericsson | Having CSI-RS periodicity of 20 ms makes CSI prediction hard, with both AI and non-AI models underperforming compared to Rel-16 baseline in terms of system KPI for the considered configuration: observation window of 5/20ms, prediction window of 1/20ms/20ms, UE speed of 3km/h. When the number of prediction time instances increases (prediction window increases), the AI and Rel-16 baseline achieve the similar level of system performance. For the CSI prediction use case with practical CSI-RS periodicities (e.g., 20 ms), study channel measurements on combined periodic and aperiodic CSI-RS resources to improve CSI prediction performance. |
| --- | --- |
| InterDigital | For 20ms CSI-RS periodicity, the non-AI/ML KF based approach consistently outperforms the AI/ML based prediction in terms of mean throughput, for the UE speeds considered (3, 10 and 30 km/h, and all considered resource utilizations). The AI/ML model outperforms the non-AI/ML KF approach only for the 5th percentile throughput and UE speed of 30 km/h. |
| Apple | Observation 4: When p-CSI-RS with 20ms periodicity is assumed, prediction fails for 30km/h and 60km/h speed. |
| SK Telecom | Periodic CSI-RS and CSI reporting with 5ms periodicity are not practical setting in commercial 5G network. Periodic CSI-RS and CSI reporting with 20ms periodicity should be considered in Release 19 AI/ML-based CSI prediction study. There are still many potential implementation options to achieve meaningful positive gain under practical periodic CSI-RS and CSI report with 20ms periodicity (e.g., prediction window of 4/5ms/5ms with 20ms p-CSI-RS). If is concluded that there is no meaningful gain of AI/ML based CSI prediction with 20ms p-CSI-RS only despite of additional investigation and evaluation, we should consider to transmit addition aperiodic CSI-RS for high-speed UE for prediction accuracy enhancement in practical scenarios - Consider to study combined P-CSI-RS + AP CSI-RS for AI/ML-based CSI prediction, if it is concluded that there is no meaningful gain of AI/ML based CSI prediction with 20ms p-CSI-RS only. |

Similar to RAN1#116bis, according to Ericsson and Apple’s observation, 20ms CSI-RS periodicity makes CSI prediction hard. For solving this issue, Ericsson and SK Telecom propose to consider combining p-CSI-RS and a-CSI-RS. Meanwhile, InterDigital provides the results with 20ms CSI-RS periodicity that AI/ML performance is lower than that of non-AI CSI prediction in terms of mean UPT. Moderator thinks that more evaluations are needed, but situation will be the same since the channel coherence time may be smaller than 20ms CSI-RS periodicity. Hence, we can discuss based on following proposal.

Proposal  #2.2-A:

Study whether and how to combined periodic and aperiodic CSI-RS resources for AI/ML based CSI prediction at least for 20ms CSI-RS periodicity.

Please provide your view on this issue if any.

| Company | Views |
| --- | --- |
| Huawei/HiSilicon | We don’t see the value in such study at this time since only one meeting is left from the study item. If necessary, such schemes may be discussed in the work item. We think the main focus during this and next meeting should be observations/conclusions based on the evaluation results. For evaluations, companies may use 5 ms CSIRS periodicity as agreed. |
| Fujitsu | We would like to hear more details on how the combination of periodic and aperiodic CSI-RS could provide more benefit. But we could be open for discussion. |
| Xiaomi | We prefer to study 20 ms CSI-RS periodicity with low priority. |
| Sony | Support the proposal |
| SK telecom | Yes, the first priority of next meeting is to draw conclusions based on the evaluation results with 5ms periodic CSI-RS. However, at the same time, it is clear to us that AI/ML based CSI prediction with 5ms periodic CSI-RS cannot be used in commercial network due to its large overhead. So, we think that it is worthwhile to conduct study to determine whether combination of P-CSI-RS and AP-CSI-RS can solve this problem or not. Detail specification work for combination of P-CSI-RS and AP-CSI-RS can be left to the normative phase. However, we need initial evaluation result to include combination of P-CSI-RS and AP-CSI-RS in the scope of normative work. |
| Intel | We think this proposal is too narrow. If companies agree we could study solution to improve performance for CSI reporting with 20 ms in general. Thus, we would like to change to the following proposal: Study how to improve AI/ML based CSI prediction at least for 20ms CSI-RS reporting periodicity. |
| ETRI | We suggest to collect more result on this. |
| SPRD | We prefer to study 20 ms CSI-RS periodicity with low priority. |
| Apple | Low priority |
| ZTE | From our perspective, we can first wait and see if we can make some observations or conclusions for the concerned case. If it is identified an issue, we can then discuss how to address the potential issue. |
| Mod | Thanks for the comments. I think Intel’s wording is good since it is more generic. So, I will try again with proposal below. Please provide your view on this proposal. |

- Proposal  #2.2-A (updated):

Study how to improve AI/ML based CSI prediction at least for 20ms CSI-RS reporting periodicity.

| Company | Views |
| --- | --- |

- Case of 80% indoor UE + 20% outdoor UE

- Summary

In the contribution submitted to RAN1#117, one company provides evaluation results under the assumption of 80% indoor UE and 20% outdoor UE as below. From this observation, compared to 100% outdoor UE, the gain over benchmark 1 is reduced. Moderator thinks that this is because indoor UEs move slowly so that channel statistic is relatively stable over outdoor UE. This means there is not much performance difference between benchmark 1 and AI/ML based CSI prediction. In order to make observation, it seems more results are needed.

| NTT DOCOMO | Observation 2 - The more significant gains on SGCS can be expected to predict distant future CSI. - With 80% indoor-20% outdoor UE distribution, minor performance gain (<10% at 20ms in the future) is observed compared with the sample-and-holding scheme, even considering the raw output of the CSI prediction model without the Rel. 18 codebook quantization. - With 100% outdoor UE distribution, the performance gain is more significant (7% at 5ms and 23% at 20ms in the future). - Compared to the results of the two cases, the outdoor UEs in cars contribute almost all performance gains. Compared with sample-and-hold, indoor UEs do not benefit from the AI/ML-based CSI prediction. |
| --- | --- |

Please provide your view on this issue if any.

| Company | Views |
| --- | --- |
| Huawei, HiSilicon | Agree with moderator assessment that more results are needed to make quantitative observations. |
| Fujitsu | Agree with FL. |
| Xiaoxi | Agree with FL’s assessment. |
| Sony | Agree with FL |

Generalization/Scalability evaluation

- Summary

In the contribution submitted to RAN1#117, several companies provide their evaluation results on generalization/scalability. The table below captures observations/proposals in their contribution.

Generalization performance over UE speed

| Ericsson | Comparing with non-AI based prediction, AI-based prediction can improve the CSI prediction performance when the AI model is trained with matched data statistics for inference scenario, or when the AI model is trained with a mixed dataset that contains the inference data statistics. It is more robust to train an AI model using a dataset for higher UE speed with inference at a scenario with lower UE speed, than to train an AI model using a dataset for lower UE speed and inference at a scenario with higher UE speed. |
| --- | --- |
| vivo | The generalization of AI-based CSI prediction over speed is not good if the training set contains only one speed. When the testing speed is higher than the training speed, the degradation of prediction accuracy is more significant than the other way around. The generalization of AI-based CSI prediction over speed can be improved using training set with mixed speed, whose prediction accuracy is still worse than that of speed-specific models. |
| CATT | Observation 2: For CSI prediction of generalization over UE speeds, compared to generalization Case 1 where the AI/ML model is trained with dataset with UE speed 30km/h and applied for inference with the same UE speed, - For generalization Case 2, where the AI/ML model is trained with dataset with UE speed 60km/h, evaluation shows about 6% degradation in terms of SGCS ； - For generalization Case 3, where the where the AI/ML model is trained with mixed dataset with UE speed 30 km/h and 60km/h, evaluation results show that it can achieve nearly the same SGCS performance. Observation 3: For CSI prediction of generalization over UE speeds, compared to generalization Case 1 where the AI/ML model is trained with dataset with UE speed 60km/h and applied for inference with the same UE speed, - For generalization Case 2, where the AI/ML model is trained with dataset with UE speed 30km/h, evaluation shows about 1.9% degradation in terms of SGCS ； - For generalization Case 3, where the where the AI/ML model is trained with mixed dataset with UE speed 30 km/h and 60km/h, evaluation results show that it can achieve slightly better (+2.4%) SGCS performance. |
| CMCC | Observation 4: The unified AI model trained with mixed dataset achieve good generalization performance over different UE speeds for CSI prediction. |
| Fujitsu | - For AI/ML based CSI prediction, the significant performance degradation can be observed when the AI/ML model trained by the dataset with UE speed X is tested on the dataset with UE speed Y (≠X). - For AI/ML based CSI prediction, the AI/ML model trained by the mixed dataset has good generalization for various UE speeds. |
| MediaTek | Training at a mixed speed can improve the performance, but it still experiences a loss of up to 10dB in NMSE (and 44% in SGCS at high speed) when compared to optimal results. AR method do not suffer from generalization issue. |

Generalization performance over deployment scenarios including LOS/NLOS cases

| Ericsson | An AI-based CSI prediction model trained using dataset for Dense Urban scenario generalize well when used for inference in UMi or UMa scenarios. |
| --- | --- |
| vivo | The generalization over the deployment scenarios, e.g., LOS/NLOS, Uma/Umi, is not good if the training set contains only one scenario. Training with mixed scenarios can improve the generalization performance while its data collection needs to be carefully designed |
| MediaTek | For AI/ML-based CSI prediction, the AI/ML model trained on a certain deployment (e.g., UMa/UMi) can be generalized and performed inference on other deployment (e.g., UMi/UMa). - Use mixed datasets over deployment for AI/ML-based CSI prediction model to improve the generalization. For AI/ML-based CSI prediction, the AI/ML model trained on a LOS or NLOS scenario can be generalized and performed inference on the other scenario. - Use mixed datasets over LOS/NLOS for AI/ML-based CSI prediction model to improve the generalization. |

Generalization performance over carrier frequency

| vivo | When the testing carrier frequency is higher than the training carrier frequency, the degradation of prediction accuracy is more significant than the other way around. The model trained by the mixed scenarios can improve the generalization performance while there still exist performance gap with generalization Case 1. |
| --- | --- |
| MediaTek | For AI/ML-based CSI prediction, the AI/ML model trained on a certain carrier frequency may not be generalized on other carrier frequencies. Training at a mixed carrier frequencies can improve the performance, but it still experiences a loss of up to 5dB in NMSE (and 5% in SGCS) when compared to optimal results. |

Generalization performance over mixed aspects

| AT&T | Proposal 8: For generalization performance of multiple aspects, the following combinations can be considered. - Option 1: Deployment scenarios + carrier frequency. - Option 2: Deployment scenarios + frequency granularity. - Option 3: Deployment scenarios + antenna port number. - Option 4: Deployment scenarios + carrier frequency + frequency granularity. - Option 5: Deployment scenarios + carrier frequency + antenna port number. - Option 6. UE speed + deployment scenarios + carrier frequency - Option 7. UE speed + deployment scenarios + frequency granularity |
| --- | --- |
| NTT DOCOMO | Observation 3 - For generalization case 2 over multiple aspects, significant performance loss is observed compared to generalization case 1. The loss increases with the prediction period, and the generalization performance is worse than the sample-and-hold scheme for some cases. - For generalization case 3 over multiple aspects, the generalization performance depends on the detailed case. For some cases, the performance loss is observable (>5%) compared to generalization case 1 and increases with the prediction period. |

Generalization performance over different observation/prediction windows

| Samsung | Condition 1: 5 measurements of observation window and 2 prediction instances, and a CSI-RS periodicity and the gap between prediction instances (‘m’ and ‘d’ in FIGURE 3, respectively) are set to 1ms. Condition 2: The observation and prediction window parameters are the same for condition 1, but the CSI-RS periodicity is m=5ms. - Large: 5-layer 2D CNN with 100 output channels and kernel size =5 - Medium: 3-layer 2D CNN with 50 output channels and kernel size =5 - Small: 2-layer 2D CNN with 10 output channels and kernel size =5 Table 1 Generalization performance for different observation and prediction window configurations (NTx,NRx)=(16,4), Test NMSE (dB) Model tested on Model trained on; Condition 1; Condition 2. Model size; Model size. Large; Medium; Small; Large; Medium; Small. Condition 1; -34.0; -25.3; -20.3; 13.4; 9.2; 4.8. Condition 2; -3.1; -3.37; -3.36; -7.07; -7.06; -7.03. Mixed condition (generic) 1 &2; -29.0; -22.3; -10.09; -7.04; -6.95; -5.54. Performance degradation (dB) from condition specific model to the generic one; Δ=5.0; Δ=3.0; Δ=10.2; Δ=.03; Δ=.11; Δ=1.49. |
| --- | --- |

Scalability over frequency granularity assumption

| Apple | Observation 8: Both filter-based CSI predictor and AI (LSTM) based predictor generalize/scalable to different frequency granularity and antenna ports. |
| --- | --- |
| CMCC | Observation 3: The AI model trained under one subband could perform well when testing under a different subband. |
| Nokia | Observation 5: Scalability parameters are generally known at the UE and the gNB and, typically, do not change during the active time of a UE in a certain cell. Therefore, one straight forward solution is to provide optimally trained ML models per scalability value. Observation 6: There exist different options to deal with various scalability parameters such as - Switching to ML models trained for specific scalability parameters; - Reconfigurations of ML models depending on known scalability parameters such as the number of antenna ports, bandwidth, or carrier frequency; - Specific type of data preprocessing converting various input parameters to the same type of input signal for the ML model; - Using one larger ML model inherently adapting to various scalability parameter values. Proposal 3: Consider the following alternative solutions to cope with varying scalability and generalization parameters: - Scenario specific ML model selection, switching and (de)activation, where each model is trained for specific UE speeds, SINRs, etc. - One single ML model, or very few ML models, with high generalization and high scalability capabilities. - UE sided finetuning of generalized ML models based on most recent channel observations over one to few hundreds of ms such that a single ML model can be used in many scenarios. - Cell and/or location specific retraining of ML models based on training data sets provided by the gNB. Proposal 4: Consider in a first step fine tuning performance as an upper bound of what can be achieved by localized models. |
| MediaTek | Compared with training at single RB, more complex models need to be considered when training at multiple RBs, otherwise the performance cannot be improved. The AI/ML model trained on single/joint RB(s) can be generalized and inferenced on other single/joint RB(s). |

Based on the submitted results, following observations are proposed. Note that cases other than UE speeds and deployment scenarios needs more evaluation samples. Therefore, in this meeting, we will discuss on generalization for UE speed and deployment scenarios.

Table 3-1. Generalization performance across different deployment scenarios

|  |  |  | Case 2 | Case 3 | Remarks |
| --- | --- | --- | --- | --- | --- |
| vivo | UMa>UMi | SGCS | -4.0% | -0.6% | PW: 1/3ms/3ms w/ Spatial consistency |
|  |  | NMSE | -0.80dB | -0.38dB |  |
|  | UMi>UMa | SGCS | -6.8% | -5.2% |  |
|  |  | NMSE | -2.18dB | -1.72dB |  |
| Ericsson | UMa>UMi | SGCS | -3.03%, -4.01%, -3.19%, -0.70% |  | PW: 4/5ms/5ms w/ Spatial consistency |
|  |  | NMSE | Please provide gain in dB -0.10 dB -0.03 dB -0.04 dB -0.07 dB |  |  |
|  | UMi>UMa | SGCS | -1.88%, -0.99%, -1.88%, -3.02% |  |  |
|  |  | NMSE | Please provide gain in dB -0.24 dB -0.12 dB -0.18 dB -0.18 dB |  |  |
| MediaTek | UMa>UMi | SGCS | 0% | 0% | w/ Spatial consistency |
|  |  | NMSE | -0.45dB | -0.27dB |  |
|  | UMi>UMa | SGCS | 0% | 0% |  |
|  |  | NMSE | -1.69dB | -0.27dB |  |

Table 3-2. Generalization performance across UE speeds

|  |  |  | Case 2 | Case 3 | Remarks |
| --- | --- | --- | --- | --- | --- |
| Huawei | 60km/h>30km/h | SGCS | -6% | -0.9% |  |
|  | 30km/h>60km/h | SGCS | -31% | -1.4% |  |
| CATT | 60km/h>30km/h | SGCS | -6.05% | 0.35% |  |
|  | 30km/h>60km/h | SGCS | -1.9% | 2.4% |  |
| Apple | 60km/h>30km/h | SGCS |  | 2% |  |
|  | 30km/h>60km/h | SGCS |  | 0% |  |
| vivo | 60km/h>30km/h | SGCS | -2.7% | -1.8% | w/ Spatial consistency Mod: Please check the gain |
|  | 30km/h>60km/h | SGCS | -28.6% | -2.5% |  |
| Fujitsu | 30km/h>60km/h | SGCS | -29.2% | -1.8% | 10+30+60km/h |
|  | 30km/h>10km/h | SGCS | 0% | 0% |  |
|  | 10km/h>30km/h | SGCS | -51% | -0.5% |  |
| CMCC | 20km/h>10km/h | SGCS | -1.95% | -16.87% | 10km/h + 20km/h + 30km/h + 60km/h |
|  | 60km/h>10km/h | SGCS | -26.79% |  |  |
|  | 10km/h>20km/h | SGCS | -57.83% | -16.53% |  |
|  | 60km/h>20km/h | SGCS | -28.21% |  |  |
|  | 10km/h>30km/h | SGCS | -72.37% | -15.44% |  |
|  | 60km/h>30km/h | SGCS | -25.58% |  |  |
|  | 10km/h>60km/h | SGCS | -76.85% | -6.77% |  |
|  | 20km/h>60km/h | SGCS | -33.14% |  |  |
| Ericsson | 60km/h>30km/h | SGCS | -11.39%, -13.32%, -17.07%, -11.06% | -2.77%, -1.24%, -3.01%, -2.53% | w/ Spatial consistency PW:4/5ms/5ms |
|  | 30km/h>60km/h | SGCS | -31.36%, -38.88%, -24.23%, -30.81% | -3.63%, 0%, 3.34%, 4.94% |  |
| MediaTek | 30km/h>10km/h | SGCS | 0% | -0.2% | w/ Spatial consistency 10km/h + 30km/h + 60km/h + 120km/h |
|  | 120km/h>10km/h | SGCS | -5.5% |  |  |
|  | 120km/h>30km/h | SGCS | -51.5% | -0.7% |  |
|  | 30km/h>60km/h | SGCS | -56.3% | -13.5% |  |
|  | 120km/h>60km/h | SGCS | -32.6% |  |  |
|  | 30km/h>120km/h | SGCS | -56.3% | -43.6% |  |

- Observation #2.3-A:

For the generalization verification of CSI prediction using UE sided model over various UE speeds, till the RAN1#117 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain UE speed#B and applied for inference with a same UE speed#B,

- For generalization Case 2, generalized performance may be achieved for some certain combinations of UE speed#A and UE speed#B but not for others:

  - If UE speed#B is 10 km/h and

    - UE speed#A is 30 km/h or 120km/h, 2 sources [Fujitsu, MediaTek] observe a generalized performance of less than -5.5% degradation.

    - UE speed#A is 60 km/h, 1 source [CMCC] observes -26.79% degradation.

  - If UE speed#B is 30 km/h and

    - UE speed#A is 60km/h, 2 sources [Huawei, CATT, Ericsson] observe a generalized performance of  -11.4%~-6.05% degradation and 2 sources [vivo, CMCC] observe  a generalized performance of -25.6%~-32% degradation.

    - UE speed#A is 10km/h or 120km/h, 2 sources [CMCC, MediaTek] observe a generalized performance of  -51.5%~-72.37% degradation.

  - If UE speed#B is 60 km/h and

    - UE speed#A is 30km/h, 1 source [CATT] observes a generalized performance of -1.9% degradation and 4 sources [Huawei, Fujitsu, MediaTek, Ericsson] observe a generalized performance of  -29.2%~-56.3% degradation

  - If UE speed#B is 120 km/h and

    - UE speed#A is 30km/h, 1 source [MediaTek] observes a generalized performance of -56.3% degradation

- For generalization Case 3, generalized performance of the AI/ML model can be achieved in general (0%~-4.45% loss) for UE speed#B subject to any of 10 km/h, 30 km/h, 60 km/h and 120 km/h, if the training dataset is constructed with data samples subject to multiple UE speeds including UE speed#B, as observed by 9 sources [Xiaomi, Interdigital, Apple, Huawei, ZTE, Samsung, ETRI, vivo, Spreadtrum].

  - If UE speed#B is 10 km/h

    - 2 sources [Fujitsu, MediaTek] observe a generalized performance of less than -0.2% degradation, and 1 source [CMCC] observes -16.87% degradation.

  - If UE speed#B is 30 km/h

    - 7 sources [Huawei, CATT, vivo, Apple, Fujitsu, Ericsson, MediaTek] observe a generalized performance of less than -2.77% degradation, 1 source [CMCC] observes a generalized performance of -15.44% degradation.

  - If UE speed#B is 60 km/h and

    - 6 sources [Huawei, CATT, vivo, Apple, Fujitsu, Ericsson] observe a generalized performance of less than -3.63% degradation, 2 sources [CMCC, MediaTek] observe a generalized performance of -6.77%~-13.5% degradation.

  - If UE speed#B is 120 km/h and

    - 1 source [MediaTek] observes a generalized performance of -43.6% degradation

  - Note: For generalization Case 3, 2 sources [CMCC, MediaTek] observe performance degradations (-13.5%~-43.6% loss) for UE speed#B subject to 10 km/h, 30 km/h, 60 km/h, 120 km/h, but compared with generalization Case 2, in general the performance are still improved.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Raw channel matrix is used as the model input.

  - The performance metric is SGCS in linear value for layer 1/2/3/4.

  - 3 sources [vivo, Ericsson, MediaTek] consider spatial consistency. Other sources do not consider spatial consistency.

- Note: Results refer to Table 3-2 of [R1-240xxxx]

Please provide your view on Observation #2.3-A.

| Company | Views |
| --- | --- |
| Mod | @vivo: please check the gain in Table 3-2. |
| Qualcomm | The degradation is relative to case 1, maybe also capture how it is compared to benchmark schemes, otherwise we don’t understand the gain / loss relative the non-AI schemes. Besides, for benchmark scheme 2, companies may need to report how the algorithm is adapted to variable scenarios. |
| Ericsson | We have added the values for NMSE gain in dB for table 3-1. |
| vivo | We add the related gain in the table 3-2 |

Cell/site specific model evaluation

- Summary

From the contribution submitted to RAN1#117, several companies provide their view and/or evaluation results on cell/site specific model evaluation. The table below captures observations in their contribution.

| Ericsson | Localized model 1 A localized model gives in most cases only minor gain compared to a model trained on general data. |
| --- | --- |
| Samsung | Localized model: Option 1. Proposal#1: For the evaluation of AI/ML-based CSI prediction using localized models in Release 19, among the options for modeling the spatial correlation in the dataset for a local region consider - A single drop on a single sector with spatial consistency turned on and a large number of UE per drop. Observation#1: Site-specific AI/ML prediction model trained based on dataset collected from a single drop on a single sector with spatial consistency turned on and a large number of UE per drop shows better performance (SGCS) as compared to generic model. |
| NVIDIA | Proposal 2: Site-specific AI/ML models for CSI prediction should be considered to improve performance gain. Proposal 3: Define a common reference scenario with site specificity as a basis for further study of AI/ML based CSI prediction. Proposal 4: Select one the following options to define a common reference scenario with site specificity as a basis for further study of AI/ML based CSI prediction: - Option 1: Real-scenario map that is a virtual representation of a real area on earth. - Option 2: Synthetic-scenario map that is artificially constructed to mimic a certain environment such as urban macro, rural macro, indoor office, or indoor factory. Proposal 5: Consider the Madrid grid developed by the METIS project for urban scenarios for further study of site-specific AI/ML based CSI prediction. Proposal 6: With a common reference scenario with site specificity, ray tracing is used to generate channel data for the development and evaluation of site-specific AI/ML models for CSI prediction. |
| LG Electronics | Proposal #2: Prefer option 1 (spatial consistency based) for cell/site specific model evaluation which is optional evaluation. |
| Oppo | Localized model: Option 1. Observation 5: With raw channel as the input, localized model outperforms both generalized model and non-AI benchmark. Observation 6: With the input of CSI eigenvector, in comparison to the application of a generalized model on diverse cells, localized models can achieve more extra benefits, e.g. SGCS gain for CSI prediction (5ms) has been improved from ~6% to ~10%. Observation 7: Within a cell, the introduction of AI/ML-based CSI prediction may effectively improve the CSI prediction performance of some users in the cell, e.g., - [17.79%~81.45%] for [50%, 20%, 5%] users with highest localized performance gain within 5ms/10ms prediction window, - [16.85%~65.81%] for [50%, 20%, 5%] users with worst performance of Rel-16 eTypeII w/o prediction within 5ms/10ms prediction window. |
| Ericsson | Localized model 1 A localized model gives in most cases only minor gain compared to a model trained on general data. |
| Samsung | Localized model: Option 1. Proposal#1: For the evaluation of AI/ML-based CSI prediction using localized models in Release 19, among the options for modeling the spatial correlation in the dataset for a local region consider - A single drop on a single sector with spatial consistency turned on and a large number of UE per drop. Observation#1: Site-specific AI/ML prediction model trained based on dataset collected from a single drop on a single sector with spatial consistency turned on and a large number of UE per drop shows better performance (SGCS) as compared to generic model. |
| NVIDIA | Proposal 2: Site-specific AI/ML models for CSI prediction should be considered to improve performance gain. Proposal 3: Define a common reference scenario with site specificity as a basis for further study of AI/ML based CSI prediction. Proposal 4: Select one the following options to define a common reference scenario with site specificity as a basis for further study of AI/ML based CSI prediction: - Option 1: Real-scenario map that is a virtual representation of a real area on earth. - Option 2: Synthetic-scenario map that is artificially constructed to mimic a certain environment such as urban macro, rural macro, indoor office, or indoor factory. Proposal 5: Consider the Madrid grid developed by the METIS project for urban scenarios for further study of site-specific AI/ML based CSI prediction. Proposal 6: With a common reference scenario with site specificity, ray tracing is used to generate channel data for the development and evaluation of site-specific AI/ML models for CSI prediction. |
| LG Electronics | Proposal #2: Prefer option 1 (spatial consistency based) for cell/site specific model evaluation which is optional evaluation. |
| Oppo | Localized model: Option 1. Observation 5: With raw channel as the input, localized model outperforms both generalized model and non-AI benchmark. Observation 6: With the input of CSI eigenvector, in comparison to the application of a generalized model on diverse cells, localized models can achieve more extra benefits, e.g. SGCS gain for CSI prediction (5ms) has been improved from ~6% to ~10%. Observation 7: Within a cell, the introduction of AI/ML-based CSI prediction may effectively improve the CSI prediction performance of some users in the cell, e.g., - [17.79%~81.45%] for [50%, 20%, 5%] users with highest localized performance gain within 5ms/10ms prediction window, - [16.85%~65.81%] for [50%, 20%, 5%] users with worst performance of Rel-16 eTypeII w/o prediction within 5ms/10ms prediction window. |

In the RAN1#116, following was agreed for evaluation of AI/ML based CSI prediction using cell/site specific models.

Agreement For the evaluation of AI/ML-based CSI prediction using localized models in Release 19, consider the following options as a starting point to model the spatial correlation in the dataset for a local region: - Option 1: The dataset is derived from UEs dropped within the local region, with spatial consistency modelling as per TR 38.901. - E.g., Dropped in a specific cell or within a specific boundary. - Option 2: By using a scenario/configuration specific to the local region. - E.g., Indoor-outdoor ratio, LOS-NLOS ratio, TXRU mapping, etc. Note: While modelling the spatial correlation, strive to ensure that the dataset distribution also correctly captures the decorrelation due to temporal variations in the channel. To report methods to generate training and testing dataset.

For the observation for localized model, only Ericsson have been provided evaluation results in Excel sheet during the result collection period. Moderator thinks more results are needed, so it will be discussed in the next meeting it there are enough samples. Also, as shown above summary, no one considers option 2 for localized model simulation. Therefore, it would be helpful to down-select between two options in ordered to further align the results. Therefore, following is proposed.

- Proposal #2.4-A:

For the evaluation of AI/ML-based CSI prediction using localized models in Release 19, consider option 1 as a baseline to model the spatial correlation in the dataset for a local region.

- Option 1: The dataset is derived from UEs dropped within the local region, with spatial consistency mod-elling as per TR 38.901.

  - E.g., Dropped in a specific cell or within a specific boundary.

Please provide your view on Proposal #2.4-A

| Company | Views |
| --- | --- |
| Huawei, HiSilicon | OK |
| Xiaomi | We prefer to Option 2 with specific indoor-outdoor ratio configuration as a baseline due to its simplicity. |
| Ericsson | Support |
| ETRI | Support |
| ZTE | OK |

Finetuning

- Summary

From the contribution submitted to RAN1#117, one company propose to evaluate finetuning approach. Moderator thinks that it can be simulated and submitted to the excel sheet for generalization case 2a.

| Nokia | Proposal 7: Use CSI-RS burst with K resources and time interval m slots (based on R18 MIMO eType-II) as a starting point for fine tuning methods. Furthermore, consider more efficient triggering of CSI RS configurations for fine tuning. Proposal 8: In case for fine tuning some more efficient configurations of AP-CSI RSs compared to legacy methods can be identified then a common approach with the configuration of Nsamps,2 /2ms, Nsamps,2 =2, 4, 8, 12 AP-CSI RSs should be defined. Proposal 9: We propose to evaluate the benefits of fine tuning as it might be able to adapt to any relevant channel conditions for a single or few generalized ML models. Furthermore, it might provide an upper bound for what can be achieved with cell/cell area specific model retraining without the need to define new localized channel conditions. |
| --- | --- |

Please provide your view on fine-tuning case if any.

| Company | Views |
| --- | --- |
| Huawei, HiSilicon | We don’t think there is a need for adding another generalization case 2a at this time as the next meeting is the last meeting in SI phase. |
| Xiaomi | Considering limited time for oncoming check point, suggest to study it with low priority. |

Others assumptions

- Summary

In the following, other evaluation assumptions are proposed by some companies.

| Smasung | Proposal#3: For the evaluation of AI/ML-based CSI prediction • CSI reporting periodicity: {5, 10, 20} ms Proposal#4: For the evaluation of AI/ML-based CSI prediction • adopt the CSI feedback overhead rate as reference, where the CSI feedback overhead rate is the average bit-rate of CSI feedback overhead across time. |
| --- | --- |
| Nokia | Proposal 5: Evaluate further details of observation windows including Nsamps,2/2ms AP-CSI RS in addition to Nsamps/5ms P-CSI RS allowing the inference of the time domain fine structure of high speed UEs. Proposal 6: Consider UE specific configuration methods of Nsamps,2/2ms AP-CSI RS for the observation window. Specific configurations will depend on UE speed, time variance, regularity of channel evolution, etc. Observation 11: Some predefined CSI-RS configurations might save some DCI or UCI traffic and configuration latency. In case Nsamps,2 /2ms AP-CSI RS become an integral part of channel prediction and more efficient triggering of AP-CSI RS can be verified then one might consider a related specification enhancement. |
| CEWiT | Proposal-1: In our view, eigen vector to be considered as the input of the AI/ML model. This is in alignment with CSI compression, which has a use-case for CSI prediction followed by compression at UE side. Proposal-1: Study semi-persistent or aperiodic based report for AI/ML based CSI prediction. |

Please provide your input on above issue, if any. And, please provide essential issues related to EVM not handled above, if any.

| Company | Views |
| --- | --- |

Template updates

- Summary

During the results collection for RAN1#117, some companies raised issue that there is duplicated excel sheet for “intermediate KPI”.  In my understanding, it is for some companies who do not submit results for eventual KPI or consider other configuration only for intermediate KPI. Therefore, please just copy and paste your results of intermediate KPI in both sheets for the next meeting. For this meeting, I will take care of this and please let me know if I missed your results.

Please provide your view on template updates, if any.

| Company | Views |
| --- | --- |

Other aspects including potential specification impact

Data collection

- Summary

In this meeting, data collection related proposals/observations are listed below:

| Ericsson | For the CSI prediction use case, at least for training data collection, study potential specification impacts on CSI-RS configuration, including at least following aspects: - Indication of the association between CSI-RS resources used for measurements in an observation window and CSI-RS resource(s) used for ground-truth labels in a prediction window. For the CSI prediction use case, at least for monitoring data collection, study potential specification impacts on CSI-RS configuration, including at least following aspects: - Indication of the association between CSI-RS resources used for measurements in an observation window and CSI-RS resource(s) used for ground-truth labels in a prediction window At least for inference, for UE-sided model based CSI prediction, legacy CSI-RS configuration is a starting point of discussion. Study the necessity and potential specification impacts including at least following aspects: - CSI-RS configuration to improve prediction performance with long CSI-RS periodicity (e.g., 20 ms) |
| --- | --- |
| Huawei | Proposal 2: For the configuration of CSI measurement and report for AI/ML based CSI prediction, the mechanism of Rel-18 MIMO may be reused. - As minor difference between training and inference, the UE may or may not transmit the predicted CSI to gNB for training data collection. Proposal 3: For the functionality based LCM (activation/fallback) for AI/ML based CSI prediction, the functionality based LCM of UE side model of BM can be reused. |
| Intel | Proposal 5: - RAN1 to consider at least the following changes for observation window and prediction window configurations for AI/ML-based CSI prediction: - Support of observation window configuration for CSI with periodic and semi-persistent CSI-RS. - Changes to CSI-RS burst configuration for CSI with aperiodic CSI-RS. - Changes to configuration for prediction window (e.g., value range for slot offset, time interval duration and number of time intervals). |
| Spreadtrum | Proposal 1: The discussion in Rel-19 focuses on the UE-side data collection. Proposal 2 : Regarding the data collection at UE side for CSI prediction with UE-side AI/ML model, study the potential specification impact (if any) to initiate/trigger data collection from RAN1 point of view by considering the following options as a starting point - Option 1: data collection initiated/triggered by configuration from NW - Option 2: request from UE for data collection - FFS: details Proposal 3: CSI-RS configuration for non-AI-based CSI prediction in Rel-18 MIMO can be reused for model inference of AI-based CSI prediction. Proposal 4: Configuration of aperiodic CSI-RS for AI-based prediction should be enhanced. |
| InterDigital | Proposal 1: Study further the following aspects for CSI prediction performance improvement: - benefits of reporting the prediction accuracy - CSI-RS configuration enhancements based on variable observation and prediction window sizes - CSI reporting enhancements based on variable observation and prediction window sizes. |
| Samsung | Proposal#5: For the AI/ML based CSI prediction sub-use case, consider the following aspects for data collection - CSI measurement and reporting framework. - Data collection procedure and priority. |
| CATT | Proposal 3: In UE side CSI prediction use case, for data collection for model training and performance monitoring, study the signalling and procedures for the following schemes in Rel-19: - Data collection triggered by NW; - Requested from UE for data collection. |
| China Mobile | Proposal 1: Support the UE to report the preferred CSI-RS configuration for CSI prediction including at least the preferred intervals between every two consecutive CSI-RS instances and minimum number of CSI-RS instances for CSI prediction. Proposal 2: Support the report of the CSIs with a timestamp indicator as assistance information so as to guarantee the continuity and sequential order for data collection of historical CSIs or future CSIs. |
| CMCC | Proposal 3: For data collection of AI/ML based CSI prediction, data collection mechanism of AI/ML based beam management can be reused as much as possible |
| Lenovo | Study potential configurations of the observation window and the prediction window for UE-based CSI prediction |
| Fujitsu | - For AI/ML based CSI prediction, RAN1 to further study the signaling and procedure for the UE to send the request on the preferred configuration for CSI prediction operation according to the UE’s situation/condition. |
| Xiaomi | Proposal 1: At least requested from UE for data collection of model training should be supported. Proposal 2: CSI-RS configuration for Rel-18 Type II Doppler codebook could be considered as a starting point for data collection. |
| NEC | Proposal 12: For data collection for CSI prediction using UE-side model, at least the CSI measurement period in the observation window and the CSI prediction period in the prediction window need to be provided from UE to NW. |
| Google | Proposal 1: Support the UE reports the preferred CSI-RS configuration for CSI prediction including at least the preferred intervals between every two consecutive CSI-RS instances and minimum number of CSI-RS instances for CSI prediction. Proposal 5: Support to maintain the same understanding between the NW and UE on when to perform the measurement for UE side data collection for CSI prediction based on the following options: - Option 1: The measurement for UE side data collection for CSI prediction is configured by the NW - Option 2: UE request CSI-RS for data collection for CSI prediction Proposal 6: Corresponding CPU(s) are occupied when UE performs CSI measurement for data collection |
| ZTE | Proposal 4: Further study the data collection for model inference and performance monitoring for AI/ML CSI prediction with UE-side model. |
| Panasonic | Observation 2: Data collection for model training and non-real time (slow) monitoring is not required to be real-time and then latency requirement can be relaxed. Observation 3: Ground-truth CSI reporting could be realized through U-plane at least for data collection for model training and non-real time (slow) monitoring. Observation 4: Assuming fast monitoring is 100s of ms order, U-plane, RRC or MAC-CE can be sufficient. Observation 5: For NW-side data collection, at least time stamps / situation of measurement, cell ID and UE location should be considered as the UE-side additional condition. Observation 6: For NW-side data collection, the necessity and feasibility of UE reporting Rx filter assumption to network should be studied. Instead of informing actual configuration, UE-side associated ID is necessary |
| Nokia | Observation 19: There might be different sets of training data, i.e., i) for the offline training of the generalized ML models, ii) for the fine tuning of ML models and, iii) for the cell/cell area specific retraining of ML models. Observation 20: Training of the baseline generalized ML models is expected to be UE vendor specific which allows for UE vendor specific data formats. Cell/cell area specific retraining data sets are provided by gNBs to UEs which may require some data format specification. Similarly, ML model fine tuning requires configurations of CSI-RSs to fit to current UE radio channel conditions, which might therefore benefit from additional CSI-RS configuration options. |
| Mavenir | Proposal 2: Enhanced CSI-RS configurations are critical for AI/ML model training because models need accurate historical CSI data to learn the changing patterns of the channel. |
| CEWiT | Proposal-3: Study signalling for data collection for CSI prediction in case of model finetuning. Proposal-4: Consider Dataset-ID based categorization for additional data collected for finetuning process. |

Regarding data collection for CSI prediction using UE sided model, following was agreed in RAN1#114. In order to avoid duplicated discussion, similar proposals as in above will not be treated

Observation In CSI prediction using UE sided model use case, at least the following aspects have been proposed by companies on data collection, including: - Signaling and procedures for the data collection - data collection indicated by NW - Requested from UE for data collection - CSI-RS configuration - Assistance information for categorizing the data, if needed The provision of assistance information needs to consider feasibility of disclosing proprietary information to the other side

Meanwhile, there are several proposals to study on association between measurements in an observation window and ground-truth CSI in a prediction window. Also, some companies proposed to consider UE capability reporting of preferred CSI-RS configurations. Therefore, following is proposed.

- Proposal #3.1-A:

For the CSI prediction using UE-sided model, study necessity and potential specification impacts on CSI-RS configuration, including at least following aspects:

- Association between measurements in an observation window and ground-truth CSI in a prediction window

- UE to report preferred configurations for observation/[prediction] window

Please provide your view on proposal #3.1-A.

| Company | Views |
| --- | --- |
| Huawei, HiSilicon | We are not sure we understand the exact intention of the first bullet. However, considering that the CSI-RS resource for measurement of ground-truth CSI (located in the prediction window) may not be in the slot of the predicted CSI but, for instance, in a neighboring slot, UE may need to be indicated with the association between the predicted CSI and the measurement of ground-truth CSI. Therefore, we suggest the following modification: - Proposal #3.1-A: (modified) For the CSI prediction using UE-sided model, study necessity and potential specification impacts on CSI-RS configuration, including at least following aspects: - Association between measurements in an observation window predicted CSI and ground-truth CSI in a prediction window - UE to report preferred configurations for observation/[prediction] window Mod: I think that is another issue, so it can be discussed whether we included it or not. |
| Fujitsu | We think it should be clarified whether the proposal is for training data collection, or performance monitoring, or inference, or all the three purposes. For the first sub-bullet, looks it is applied to training data collection and performance monitoring since it requires the ground truth CSI. For the second sub-bullet, looks it could be applied to all the three purposes. |
| Xiaomi | In our view, for model inference by using UE-sided model, such association is not needed. Only model training or model monitoring needs such association. This is should be clarified in the main bullet. UE can also report such addition information, such as time domain channel propriety (TDCP) to indicated its preferred configuration for observation/[prediction] window. gNB can configure preferred the observation/prediction window of UE according to the received addition information. Hence, we suggest to study how to indicate the UE’s preferred configurations for observation/[prediction] window to gNB for the second bullet. |
| Panasonic | We are OK with the proposal. |
| Sony | We agree with Huawei’s modification |
| Ericsson | We do not think we need UE reporting of preferred configurations for observation/[prediction] windows. In Rel-18, TDCP reporting was introduced that gives the network information regarding how fast the channel varies. Using the TDCP measurement reported by the UE, the network can decide the configurations of observation and prediction window for a UE. Note that during the Rel-18 MIMO work item, UE reporting its preferred configurations was discussed as an alternative to autocorrelation based TDCP reporting. However, the UE reporting its preferred configuration was not agreed in Rel-18 and autocorrelation based TDCP was agreed in the end. Hence, the second sub bullet should be removed. |
| ETRI | Support |
| NTT DOCOMO | Support |
| SPRD | Agree with Fujitsu. It should be clarified in the main bullet which phase is included. |
| NEC | OK. |
| Apple | Support |
| CATT | We are fine with the 1st bullet. For the 2nd bullet, ok for UE to report configurations for observation/[prediction] window for model training. For model inference, NW can obtain such information via functionality identification. For performance monitoring, we prefer to wait and adopt the discussions and agreements in AI BM. |
| ZTE | Similar other companies, we need to first identify the phase of this proposal, e.g., training, inference or monitoring. Regarding the 1st bullet, if it is for the training phase, we see some value of this. |

- Proposal #3.1-A:

For the CSI prediction using UE-sided model, at least for training and performance monitoring, study necessity and potential specification impacts on CSI-RS configuration, including at least following aspects:

- Association between measurements in an observation window and ground-truth CSI in a prediction window

- [Association between predicted CSI and ground-truth CSI in a prediction window]

- [UE to report preferred configurations for observation/[prediction] window]

Please provide your view on updated proposal #3.1-A.

| Company | Views |
| --- | --- |

Inference and related CSI reporting

- Summary

In the RAN1#116bis, following regarding inference aspects were agreed.

Agreement For AI/ML based CSI prediction, at least for inference, legacy CSI-RS configuration can be a starting point. Further study on whether there is a need for specification enhancement. Agreement - At least for inference, for UE-sided model based CSI prediction, legacy feedback mechanism using codebook type set to “typeII-Doppler-r18” is a starting point of discussion. Study the necessity and potential specification impacts including at least following aspects: - CSI processing criteria and timeline

Also, proposals/observations related to inference and corresponding CSI reporting are copied below:

| Vivo | Study the following two options to quantize the processing criteria of AI based CSI - Option 1: Joint processing unit pool for AI based CSI and legacy CSI - Option 2: Separate processing unit pools for AI based CSI and legacy CSI |
| --- | --- |
| CMCC | Proposal 4: For CSI prediction, some CSI related parameters in Rel-18 MIMO might need revision to adapt AI/ML-enabled CSI prediction. |
| Sony | Proposal 3: RAN1 should study configuration options that allow the UE side CSI prediction model to carry out and report CSI predictions at [N] possible TDRA candidates to maintain allowing scheduling flexibility. |
| Lenovo | The legacy Type-II CSI feedback format is used as the baseline for CSI feedback for AI-based CSI prediction, at least with respect to the configured CSI fields, the spatial domain and frequency domain representation within the PMI For UE-based CSI prediction, study potential enhancements of the CSI feedback format for predicted CSI, at least with respect to time-domain representation Study the CPU calculation for AI/ML-based CSI report(s) for UE-based CSI prediction |
| LG Electronics | Proposal #4: Study whether legacy CSI processing criteria and timeline (e.g., the required number and/or occupation time of CPUs, the values of Z/Z’, and total number active/simultaneous CSI-RS resource/ports) is sufficient. Proposal #5: Further study whether there is a need for improvement/refinement of Rel-18 CSI. |
| Fujitsu | Proposal 2: - For UE-sided AI/ML-based CSI prediction, for inference operation, the CSI processing criteria and timeline in Rel-18 MIMO could be a starting point. - RAN1 to further discuss the CSI processing criteria and timeline for performance monitoring and training data collection. |
| Xiaomi | Proposal 5: Rel-18 Type II Doppler codebook could be considered as a reference for performance monitoring of the UE side AI/ML model based CSI prediction. Proposal 6: For AI/ML based CSI prediction, legacy CSI processing criteria for Rel-18 Type II Doppler codebook could be reused, and CSI computation time can be determined by UE capability indication. |
| NEC | Proposal 13: For CSI prediction, study the mechanism of discontinuous periodic CSI measurement and reporting. Proposal 14: For CSI prediction, the CSI reporting periodicity may be updated autonomously upon reaching a significant point of variation (determined by time, location or distance). |
| Google. | Proposal 2: Support that one CSI report for model inference occupies N1 eCPU(s) and N2 CPU(s) - Note: eCPU is assumed for model inference, and CPU is used for model input processing and uplink signal generation for model output - The value of N1 and N2 can be decided after the supported configuration for AI/ML based CSI prediction is finalized Proposal 3: Support the minimum processing delay (Z, Z’) for AI/ML based CSI prediction as (Z0, Z0’)+x, where (Z0, Z0’) indicates the minimum processing delay for Rel-18 eType2 codebook enhancement for PMI prediction and x indicates additional processing delay reported by UE capability. Proposal 4: Support to reuse the active resource counting rule for Rel-18 eType2 codebook enhancement for PMI prediction for AI/ML based CSI prediction as the starting point. |
| NTT DOCOMO | Proposal 1 - Conclude in RAN1 that there is no need to enhance the CSI-RS configurations and codebooks at least for the inference of CSI prediction. |

For CSI prediction using UE sided model, regarding CSI-RS configuration and CSI feedback, NTT DOCOMO thinks there is no need for enhancement/refinement. Meanwhile, Lenovo, CMCC and LG Electronics think some enhancement may be needed. This issue was already discussed in the last meeting, but it was not agreed. Therefore, it would be good to further study on this issue before making conclusion.

Also, regarding CSI processing criteria and timeline, detailed discussion may belong to working phase discussion. Therefore, we need to first study on whether legacy (Rel-18) CSI processing criteria and timeline is sufficient or not. Please keep studying on these issues.

Please provide your view on inference aspects, and suggest essential issue to be handled if any.

| Company | Views |
| --- | --- |
| Fujitsu | For inference, we think the legacy CSI processing criteria and timeline could be applied. For training data collection and performance monitoring, we think the CSI processing criteria and timeline should be further studied since these two operations are new procedures compared with Rel-18 MIMO. |
| Sony | Agree with FL |
| ETRI | Agree with FL |

LCM aspects

- Summary

In the last meeting, following was agreed.

Agreement For performance monitoring for functionality-based LCM, further study on details of type 1,2 and 3, e.g., potential specification impact, pros/cons aspects. - To clarify the boundary between type 1 and type 3 - To clarify definition of monitoring output and performance metric

Regarding above agreements, many proposals are provided by companies in this meeting as captured below:

| Ericsson | The difference between type 1 and type 3 is that whether UE reports performance metric (e.g., SGCS related statistics) or performance monitoring output (e.g., a flag indicating whether a model is functioning) to the NW. For performance monitoring for CSI prediction use case with UE side model, deprioritize Type 2. |
| --- | --- |
| Huawei | Proposal 4: For the monitoring of CSI prediction, further study and elaborate the details of each type before making down-selection. As examples: - For Type 1, potential spec impacts include: - The metric calculation approach for comparison with the threshold criterion, e.g., correspond to a per sample monitoring result, or the statistical value (e.g., mean or x% of CDF). - The type of the threshold criterion, e.g., SGCS could be prioritized. - Type of monitoring output to be reported to NW, and the NW indication of the action as the monitoring decision if needed. - For Type 2, potential spec impacts include: - The type of ground-truth CSI, e.g., channel matrix/raw eigenvectors or legacy codebook/PMI converted from channel matrix. - NW indication of the action as monitoring decision; e.g., the actions may include activation and fallback. - For Type 3, potential spec impacts include: - The metric calculation approach, e.g., correspond to a per sample monitoring result, or the statistical value (e.g., mean or x% of CDF). - The type of the metric, e.g., SGCS could be prioritized. - NW indication of the action as monitoring decision; e.g., the actions may include activation and fallback. Proposal 5: For the monitoring of CSI prediction, the boundary between Type 1 and Type 3 lies in the type of report: - For Type 1, the report is the monitoring output, e.g., candidate actions of functionality control, or the event of monitoring. - For Type 3, the report is the calculated metric, e.g., SGCS. Proposal 6: For the monitoring Type 1 and Type 3 of CSI prediction, study the type of predicted CSI and ground-truth CSI for calculating the metric of intermediate KPI, including two candidates: - Candidate 1: Channel matrix/raw eigenvectors. - Candidate 2: Legacy codebook/PMI converted from channel matrix Proposal 7: For the monitoring of CSI prediction, study how to indicate the association between the CSI report of predicted CSI and the CSI report of ground-truth CSI. |
| Intel | Proposal 2 - For CSI prediction using UE-sided AI/ML model, model performance monitoring with performance metrics calculated at the UE side (type 1 and type 3) is prioritized for discussion. Proposal 3: - Consider the following options for performance metrics F calculation for performance monitoring: - F = F(W(n), Wp(n)), where W(n) – precoding matrix for channel measured in slot n, Wp(n) – precoding matrix for channel predicted for slot n based on previous CSI measurements. - Benchmark performance metrics can be additionally used for model performance monitoring. - Benchmark performance metrics may correspond to performance of sample-and-hold operation calculated as FB = F(W(n), W(n-P)), where P is CSI-RS periodicity in slots. - Performance metrics F(V1, V2) may correspond to intermediate metrics (GCS, SGCS or NMSE) or difference in SINR (or, CQI difference) for hypothetical PDSCH transmission for precoding with matrix V1 and matrix V2. Proposal 4: - Consider the following options for calculation of performance monitoring output at the UE for Type 1 performance monitoring: - Option 1: UE compares performance metrics F to threshold T, where threshold T can be configured by the gNB. - Option 2: UE compares performance metrics F to benchmark FB. |
| Spreadtrum | Proposal 5: For the boundary between type 1 and type 3, the difference is content of UE report. - For Type 3, UE only need to calculate and directly report performance metric to gNB regardless of prediction performance.. - For Type 1, UE will first calculate the performance metric. Based on some information provided by the gNB (such as the threshold), UE will make some judgments, such as whether the current report needs to be reported, or only report some relevant information to indicate the current performance. Proposal 6: Regarding CSI prediction with UE-sided model, support performance monitoring type 1 and type 3 with intermediate KPI, e.g., SGCS, as performance metric. Proposal 7: Regarding CSI prediction with UE-sided model, deprioritize performance monitoring type 2. Proposal 8: For performance monitoring, both periodic trigger and event trigger can be considered. |
| InterDigital | Proposal 2: Study further the following aspects for UE-side model monitoring: - metrics for monitoring the UE-side CSI prediction model more indicative of the end-to-end performance - benefits of using out-of-distribution metrics for UE-side model monitoring Proposal 3: Clarify Type-1 and Type-3 performance monitoring in functionality-based LCM for CSI prediction as follows. - Type 1: - UE performs performance monitoring based on the thresholds configured by gNB - UE reports comparison of prediction accuracy against configured thresholds - Type 3: - UE provides assistance information for gNB to perform the performance monitoring of the UE-side CSI prediction model +UE reports intermediate KPIs (e.g., SGCS, NMSE) between predicted CSI and measured CSI |
| Samsung | Proposal#6: For the AI/ML based CSI prediction sub-use case, for Type 1 monitoring, consider - Configuration of CSI-RS resources for performance monitoring - Configuration for baseline CSI and threshold for UE’s calculation of performance metric - Configuration and time-domain properties for monitoring outcome reporting. Proposal#7: For the AI/ML based CSI prediction sub-use case, for Type 2 monitoring, consider - Configuration of CSI-RS resources for performance monitoring - Configuration and potential enhancement on Type II CSI for ground truth CSI reporting corresponding to multiple time instances. - Priority and CSI processing timeline |
| Vivo | At least prioritize type 3 for CSI prediction performance monitoring. Further study the specified reporting metric. |
| Apple | Proposal 1: Deprioritize type 2 performance monitoring and corresponding accuracy evaluation. Proposal 2: For potential specification impact, for type 1 performance monitoring, consider RLM/BFD like mechanism to define performance monitoring output. |
| CATT | Observation 4: On Type 2 performance monitoring of UE side CSI prediction use case, the reporting overhead is large and the codebook based reporting may impact the accuracy of performance metric. . Proposal 1: On Type 1 performance monitoring of UE side CSI prediction use case, UE can report the performance metric related event to the network. Proposal 2: On performance monitoring of UE side CSI prediction use case, Type 1 and Type 3 performance monitoring is prioritized for discussion. |
| China Mobile | Proposal 3: For type1/2/3, potential specification impact on reporting contents of performance monitoring should be studied for AI/ML-based CSI prediction, at least including: - Performance metrics of all monitoring occasions - An statistical performance metric over monitoring occasions Proposal 4: For Type 1 performance monitoring, the details of performance monitoring output should be further studied, at least including: - An indicator of monitoring performance An indicator of enabling decision recommendation |
| CMCC | Proposal 1: For the definition of performance metric in Type 3 performance monitoring, intermediate KPI can be considered as starting point. Proposal 2: For the definition of performance metric in Type 1 performance monitoring, there might be two alternatives: - Alt1: the calculation or comparison outcome based on the performance metric and the threshold criterion configured by NW. - Alt2: the recommended LCM decision reported to NW. |
| Lenovo | Study the specification impact corresponding to AI/ML model monitoring, considering the following monitoring decisions: (i) No model change, (ii) CSI parameters update, (iii) Model parameter update, (iv) Model switching, and (v) Fallback to non-AI/ML scheme For performance monitoring under UE-based CSI prediction, three reference time instants are considered: (i) at the first slot of the prediction window, (ii) at the median slot of the prediction window, and (iii) at the last slot of the prediction window For performance monitoring under UE-based CSI prediction, study and evaluate the pros and cons of the following alternatives: - Implicit performance monitoring: the UE feeds back CSI measurements based on CSI report quantities or intermediate KPIs that enable the network to derive performance monitoring decisions - Explicit performance monitoring: the monitoring feedback includes performance monitoring recommendation based on a set of network-configured performance monitoring metrics For CSI prediction using a UE side model, adopt the following definition for ‘performance metric’: a parameter computed at the UE based on channel measurements at the UE side, where the performance metric is implicit, i.e., not reported, for Type 1 monitoring, and is explicit, i.e., reported by the UE, for Type 3 monitoring For CSI prediction using a UE side model, adopt the following definition for ‘monitoring output’: a parameter comprising a decision (generated at the NW side) or a feedback of a recommended decision (generated at the UE side), based on performance metric(s) computed at the UE side |
| NVIDIA | Proposal 7: For CSI prediction using UE side model use case, study the potential specification impact of different types of performance monitoring for functionality-based LCM. |
| LG Electronics | Proposal #3: Discuss the definition of monitoring output. For example, monitoring output can include - accuracy/confidence level of model - recommendation for functionality fallback - performance monitoring. |
| Fujitsu | Proposal 3: - For AI/ML based CSI prediction with UE-side model, Type 1 and Type 3 performance monitoring should be prioritized. Proposal 4: - For AI/ML based CSI prediction with UE-side model, the intermediate KPI-based model performance monitoring should be prioritized. Observation 6: - For AI/ML based CSI prediction with UE-side model, the boundary between type 1 and type 3 performance monitoring is whether the performance metric is compared to some threshold(s) or not. Proposal 5: - For AI/ML based CSI prediction with UE-side model, regarding Type-1 monitoring, the following definitions of the monitoring output could be discussed. - Alt 1: the recommendation of the follow-up functionality LCM operations provided by UE. - Alt 2: the level that the performance metric is in. The criterion of setting the performance levels can be further studied. Proposal 6: - For the intermediate KPI based performance monitoring in AI/ML based CSI prediction with UE-side model, RAN1 to further discuss the performance metric taking the following factors into consideration: - The information of the prediction time instance(s), e.g., a particular prediction time instance, average over all prediction time instances (if multiple future CSIs are predicted). - The information of the spatial layer(s), e.g., a particular layer, average over all layers. - The frequency-domain information, e.g., a particular subband, average over all subbands. |
| Xiaomi | Proposal 3: Type 1 (UE calculates the performance metrics and reports performance monitoring output for NW making decision) or Type 3 (UE calculates the performance metrics and reports the metrics for NW making decision) could be supported for performance monitoring. Proposal 4: For Type 1, the performance monitoring output could be one bit indication information which is used to help NW to make decision of functionality fallback operation. For Type 3, the reported performance metric could be average of SGCS or NMSE of predicted N slots, where the value N could be FFS Proposal 7: In addition to performance monitoring output or performance metric reporting, the assistance information, e.g., TDCP, could be reported to NW as well by UE for NW making decision of UE sided model/functionality selection. |
| NEC | Observation 1: For performance monitoring, the primary distinction between Type 1 performance monitoring and Type 3 performance monitoring lies in the divergent content reported by UE. Specifically, - Under Type 1, the report content mainly comprises performance monitoring output which is determined based on the calculated performance metric(s). - Under Type 3, the report content mainly comprises the calculated performance metric(s). Proposal 2: The definition of performance metric should focus on Type 1 and 3 performance monitoring (i.e., calculation of performance metric is performed at UE side). Proposal 3: For Type 1 and 3 performance monitoring (i.e., calculation of performance metric is performed at UE side), the performance metric should be intermediate KPI (e.g., SGCS, NMSE), or input/output data distribution related metric (e.g., similarity, divergence, distance). Proposal 4: For Type 1 performance monitoring, the performance monitoring output that facilitates functionality fallback decision at NW should comprise one or more states indicating the following: - For the current AI/ML model, the calculated performance metric(s) does not meet a threshold (provided by NW). - For each of the other candidate AI/ML models, the calculated performance metric(s) does not meet a threshold (provided by NW). Proposal 5: For Type 1 and 3 performance monitoring, the performance metric(s) may derive from the results of a single calculation, or the results of multiple calculations (i.e., statistical performance metric). Proposal 6: Potential spec impact of performance monitoring may be as follows: - For Type 1 performance monitoring: - Determination of the performance monitoring output, e.g., definition of criterion/threshold related to performance metric(s) - Reporting of the performance monitoring output - For Type 2 performance monitoring: - Quantization of the performance metric(s) or statistical performance metric(s) - Reporting of the performance metric(s) or statistical performance metric(s) - For Type 3 performance monitoring: Association between reporting the predicted CSI and reporting the ground-truth CSI Proposal 7: Pros/Cons of Type 1, Type 2 and Type 3 performance monitoring are captured in the following table: Type of performance monitoring; Pros; Cons. Type 1; Minimal reporting overhead; Definition of the criterion/threshold related to performance metric may be challengeable. Type 2; More quantifiable evaluation the performance of AI/ML at NW side; Moderate reporting overhead; Quantization of the performance metric may be challengeable. Type 3; More comprehensive assessment of the performance of AI/ML Circumventing any standardization efforts related to performance metric; Huge reporting overhead. Proposal 8: Study how to refine the performance monitoring procedure when the target timing of predicted CSI is not aligned with the timing of available ground-CSI truth. Proposal 9: Study simultaneous performance monitoring for multiple AI/ML models. Proposal 10: For model switching, model selection and model update, if the input or/and output (e.g., observation window length, the number of measurement time instance, prediction window length, the number of future time instances) of the new AI/ML model change, UE is necessary to report the change(s) to NW. Proposal 11: AI/ML-based CSI prediction and reporting should be performed under NW configurations. |
| Google | Proposal 7: Consider the following potential spec impact for the 3 types of performance monitoring - Type1: - NW configuration for the downlink signal for UE to calculate the performance metric - NW configuration for UE to determine whether model performance failure happens - Mechanism for UE to report the detected event that model performance failure happens - Type2: - CSI framework for ground-truth CSI report - Type3: - NW configuration for the downlink signal for UE to calculate the performance metric - Mechanism for UE to report the calculated performance metric Proposal 8: With regard to the report overhead, for performance monitoring Type1 should be prioritized and Type2 should be deprioritized |
| ZTE | Proposal 1: Study CSI-RS configuration/triggering enhancement for performance monitoring for AI/ML CSI prediction with UE-side model Proposal 3: Regarding performance monitoring for AI/ML CSI prediction with UE-side model, Type 1 and Type 3 are grouped together as following. Type 1&3: - UE calculates the performance metric(s) - UE reports performance monitoring output to the network - Performance monitoring output details can be further defined, e.g., recommendation on whether to deactivate the current Functionality, prediction accuracy, SGCS, etc. - NW may configure threshold criterion to facilitate UE side performance monitoring (if needed). NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). |
| Panasonic | Proposal 1: For discussion purpose of types, boundary between Type 1 and Type 3 is clarified as follows. - Type 1: UE report monitoring output - In addition to performance metric calculation, UE performs additional calculation(s) and report the result of additional calculation(s). - Example of additional calculation - Detection of performance metric degradation based on the comparison between intermediate KPI and the configured threshold. - Statistic information calculation based on intermediate KPIs over multiple monitoring data samples. - Type 3: UE report performance metric - UE only report performance metric (i.e., an intermediate KPI such as SGCS) of each monitoring data Observation 7: There are at least two cases for performance monitoring. One is the network to judge whether the CSI prediction model / function at UE is reliable or not. The other is to judge enable / disable the AI/ML functionality in certain environment. These needs separate discussion although the signaling framework can be aligned. Observation 8: For performance monitoring of CSI prediction, performance monitoring output (Type 1), corresponding ground-truth CSI (Type 2) or performance monitoring metric (Type 3) for the previous predicted CSI can be piggybacked in the predicted CSI report. |
| Oppo | Proposal 5: For the CSI prediction performance monitoring, prioritize type 1 and type 3 CSI prediction performance monitoring. Proposal 6: Stability of the performance evaluation and decision-making mechanism should be considered to mitigate the impact of random effects on evaluation outcomes, includes: - Obtaining a consistent evaluation result by considering multiple evaluation samples within an evaluation window. - Assessing whether model monitoring should be handled at the UE level or the cell level. |
| Nokia | Proposal 10: For performance monitoring Type 1, the NW predefines the conditions for a fallback operation like defining a UE individual threshold value (to determine the monitoring outcome/event) and fallback configuration. Proposal 11: Further investigate the details of performance monitoring Type 3 including reference signal timing and overhead, feedback metrics, and supported actions by the gNB. |
| AT&T | Proposal 4: The following are the pro/con for performance monitoring type 1/2/3 Type 1; Type 2; Type 3. UE signaling overhead; Low; High; Medium. Additional NW signaling; Configuring threshold; None; None. NW side fault detection and recovery; Very limited (only fallback); Full detection and recovery; Limited. Proposal 5: For type 1 the performance monitoring output is a single bit to indicate if the UE side performance metrics are within the thresholds configured by the NW. Proposal 6: Evaluate the performance monitoring accuracy for CSI prediction for the reported performance monitoring metrics (e.g. intermediate KPI) and the eventual KPI (e.g. throughput). |
| NTT DOCOMO | Proposal 2 - Deprioritize the study on performance monitoring Type 2. - For the proper NW final decision on LCM, UE should report some performance-related information in Type 1 and Type 3. - The boundary between Type 1 and Type 3 is whether a UE calculates binary metrics from the other metrics. - Note: The NW configures the thresholds and makes the final decisions about LCM (fallbacks, model switches) if UE calculates binary metrics. |
| Mavenir | Proposal 4: Regarding CSI prediction with UE-sided model, for NW side performance monitoring, using an existing CSI feedback scheme as the reference can be considered Proposal 5: The fallback to legacy CSI prediction mechanism should be set to achieve balance of performance and complexity. Proposal 6: The correlation metrics(SGCS) between the legacy CSI and the predicted CSI can be used to monitor the performance of AI based CSI prediction |
| Qualcomm | For performance monitoring, study following aspects - CSI-RS and/or CSI report configuration for ground-truth measurement - UE-initiated and NW configured monitoring report - Report quantity includes intermediate results over a monitoring window (e.g., SGCS before compression or after compression, report SGCS value or number of instances that the SGCS not satisfying a threshold) |
| CEWiT | Proposal-7: In our view, the performance monitoring metric and performance monitoring output needs to be clarified. Is the latter the output/decision made from the former needs to be clarified. Proposal-8: In terms of specification impact with regards to various monitoring types, we propose the following: - Type-I based monitoring : - Signalling to report the monitoring output to the NW side. - NW based LCM configuration for further action - Type-II based monitoring : - Format for ground truth and predicted CSI. - Reporting instance of ground truth and predicted CSI - Type-III based monitoring: - Signalling to report the monitoring metric to the NW side. - NW based LCM configuration for further action |
| IIT | Proposal 1: Use the following relation between model and model monitoring and it’s Performance measurement. Where Monitoring, Performance metric and actual model can reside at any of N/W, UE or any other node. Proposal 2: To use the following definitions of Monitoring Output and Performance Metric for clarification. Monitoring Output: “Monitoring output for an AI/ML model involves examining the actual predictions or decisions made by the model when presented with new data. This involves comparing the model's outputs to the ground truth or expected outcomes to assess the accuracy and reliability of the model's predictions. “ Performance Metric: “A performance metric for an AI/ML model is a quantitative measure used to evaluate the effectiveness and quality of the model's predictions or decisions.” Proposal 3: For the AI/ML based CSI prediction sub-use case, for Type 1 monitoring, consider at least - Configuration of CSI-RS resources for performance monitoring - Configuration for Reference Model Output CSI and threshold for UE’s calculation of performance metric - Configuration of Performance Metrix exchange |

As shown above, regarding the boundary between type 1 and type 3 performance monitoring and/or model output and performance metric, most of companies are in the same page. Therefore, following conclusion is proposed.

Proposed conclusion #3.4-A:

For the boundary between Type 1 and Type 3 performance monitoring, the difference is whether UE reports performance metric or performance monitoring output to UE.

- For Type 1 performance monitoring, monitoring output can be information about whether a model is functioning or not.

- For Type 3 performance monitoring, performance metric can include at least intermediate KPI (e.g., SGCS, NMSE).

- FFS on details of monitoring output and performance metric

Please provide your view on proposed conclusion #3.4-A.

| Company | Views |
| --- | --- |
| Huawei, HiSilicon | We think the following modification makes the proposal clearer. - Proposed conclusion #3.4-A: (modified) For the boundary between Type 1 and Type 3 performance monitoring, the difference is whether UE reports performance metric or performance monitoring output to UE. - For Type 1 performance monitoring, UE reports monitoring output. mMonitoring output can be information about whether a model is functioning or not, candidate actions of functionality control, or the event of monitoring. - For Type 3 performance monitoring, UE reports performance metric. pPerformance metric can include at least intermediate KPI (e.g., SGCS, NMSE). - FFS on details of monitoring output and performance metric |
| Fujitsu | For Type-1 monitoring, we think the UE could report whether model is functioning or not, or the UE could report some quality level of the model performance to facilitate NW’s decision. We suggest the following update (including typo in the main bullet). For the boundary between Type 1 and Type 3 performance monitoring, the difference is whether UE reports performance metric or performance monitoring output to UEnetwork. - For Type 1 performance monitoring, monitoring output can be information about whether a model is functioning or not, or the quality level of the model performance. - For Type 3 performance monitoring, performance metric can include at least intermediate KPI (e.g., SGCS, NMSE). - FFS on details of monitoring output and performance metric Mod: information can be hard indicator or soft indicator. In case of soft indicator, it can be quality level of model performance. Is it address your comment? |
| Xiaomi | We are fine with proposal. |
| Panasonic | We are OK with the proposed conclusion. |
| Qualcomm | Suggest to combine type 1 and 3, the boundary is unclear. What is the information about whether a model is functioning or not. In our understanding, they can be harmonize into one option, and we can list the possible options of the reporting content, e.g., monitoring metrics, etc. From spec impact perspective, they also share lots of commonality, e.g., reporting mechanism (UE-initiated, NW configured (time-based)), reporting content, threshold configuration, etc. |
| Ericsson | For the boundary between Type 1 and Type 3 performance monitoring, the difference is whether UE reports performance metric or performance monitoring output to NW. - For Type 1 performance monitoring, monitoring output can be information about whether a model is functioning or not, based on conditions configured by the NW. |
| ETRI | Ok |
| LGE | We are OK with the proposed conclusion. In our understanding, quality level of the model performance can be included in information about whether a model is functioning or not as e.g. |
| NTT DOCOMO | We would like to suggest a note under Type 1. Note: The criteria or threshold on whether a model is functioning or not is configured or indicated by the NW. |
| SPRD | We are fine with proposal. |
| NEC | Agree with NTT DOCOMO. |
| Apple | Agree with Huawei’s edit. |
| CATT | Regarding Type1, further clarification is needed on “how to define whether a model is functioning or not”, an event like performance metric below a threshold can be defined. Mod: Thanks for the comment. Maybe your comment can be addressed by FFS bullet. |
| ZTE | Even for Type1 performance monitoring, the intermediate KPI should also be discussed since UE needs to know which KPI is used to determine the monitoring output. If it is difficult to clarify the difference between Type 1 and type 3, we propose to combine them together. Proposal : Regarding performance monitoring for AI/ML CSI prediction with UE-side model, Type 1 and Type 3 are grouped together as following. Type 1&3: - UE calculates the performance metric(s) - UE reports performance monitoring output to the network - Performance monitoring output details can be further defined, e.g., recommendation on whether to deactivate the current Functionality, prediction accuracy, SGCS, etc. - NW may configure threshold criterion to facilitate UE side performance monitoring (if needed). - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). Mod: Thanks for the comments. I will try to merge if current version is not agreeable. |
| Mod | Thanks for the valuable comment. Based on your comments, I revised as below. |

- Proposed conclusion #3.4-A(updated):

For the boundary between Type 1 and Type 3 performance monitoring, the difference is whether UE reports performance metric or performance monitoring output to NW.

- For Type 1 performance monitoring, UE reports monitoring output. Monitoring output can be information about whether a model is functioning or not, candidate actions of functionality control, or the event of monitoring.

  - Note: The criteria or threshold may be configured or indicated by the NW.

- For Type 3 performance monitoring, UE reports performance metric. Performance metric can include at least intermediate KPI (e.g., SGCS, NMSE).

- FFS on details of monitoring output and performance metric

Please provide your input on proposed conclusion #3.4-A, if any. And provide your view on whether Type 1 and 3 can be merged.

| Company | Views |
| --- | --- |

Proposed observation #3.4-B:

For CSI prediction using UE-sided model, for performance monitoring, following specification impacts are identified.

- Type 1

  - Type of threshold criterion, if configured

  - Type of monitoring output, and reporting mechanism of monitoring output

- Type 2

  - Type of ground truth CSI, and reporting mechanism of monitoring output

- Type 3

  - Type of performance metric, and reporting mechanism of performance metric

Please provide your view on Proposal #3.4-B.

| Company | Views |
| --- | --- |
| Huawei, HiSilicon | We think for all three types, the final decision regarding the monitoring action is made by NW and may need to be indicated to the UE. Such an indication also has specification impact. For Type 1, although monitoring output is reported, UE first need to calculate the performance metric. So, the type of performance metric (e.g., per sample monitoring result or the statistical value (e.g., mean or x% of CDF) should also be considered in Type 1. Therefore, we suggest the following modification to the proposal. - Proposed observation #3.4-B: (modified) For CSI prediction using UE-sided model, for performance monitoring, following specification impacts are identified. - Type 1 - Type of threshold criterion, if configured - Type of performance metric - Type of monitoring output, and reporting mechanism of monitoring output - NW indication to the UE of the decision regarding the monitoring action - Type 2 - Type of ground truth CSI, and reporting mechanism of monitoring output - NW indication to the UE of the decision regarding the monitoring action - Type 3 - Type of performance metric, and reporting mechanism of performance metric - NW indication to the UE of the decision regarding the monitoring action |
| Fujitsu | Generally fine with the update version from Huawei. |
| Xiaomi | We are fine with proposal. But we prefer to first discuss Proposal #3.4-C before discussing the proposal. We think one monitoring type is sufficient. At this stage, it is not necessary to discuss the specification impact for each type. |
| Panasonic | We are fine with the proposed observation. |
| Qualcomm | Maybe merge Type1 and 3 with same comment as above. Study further if threshold is needed per option of report content. |
| Intel | We think that type here is not clear. In our view it is better to change word ‘Type’ to ‘Definition’ considering that detailed definition in spec is required. We prefer to add the following for Type 1: - Definition of performance metric, if monitoring output is based on specified performance metrics |
| SPRD | Agree with xiaomi. We can discussed this proposal when the proposal 3.4-C has conclssion. |
| NEC | Ok in general. For Type 1, we suggest the following updates: - Type 1 - Type of performance metric - Type of threshold criterion, if configured - Type of monitoring output, and reporting mechanism of monitoring output |
| Apple | For type 1, add sub-bullet of event of reporting. We think RLM/BFD type of mechanism can be used for UE to generate the monitoring output. |
| ZTE | Maybe we need to clarify whether the “monitoring output” in type 2 is the same as that in type 1. Alternatively, we can use another terminology for type 2 performance monitoring. Mod: Sorry, it was a typo, it can be ground truth CSI. |
| Mod | Thanks for the valuable comments. I modified the proposal based on your comments. |

- Proposed observation #3.4-B:

For CSI prediction using UE-sided model, for performance monitoring, following specification impacts are identified.

- Type 1

  - Definition of performance metric

  - Definition of threshold criterion, if configured

  - Definition of monitoring output, and reporting mechanism of monitoring output

- Type 2

  - Definition of ground truth CSI, and reporting mechanism of monitoring output ground truth CSI.

- Type 3

  - Definition of performance metric, and reporting mechanism of performance metric

- For all types of performance monitoring, NW indication to the UE of the decision regarding the monitoring action

Please provide your input on proposed conclusion #3.4-B, if any

| Company | Views |
| --- | --- |

Also, there are several proposals on de-prioritization of Type 2 performance monitoring. Based on Tdoc review, the companies’ positions are as follows:

|  | Prioritize | De-prioritize |
| --- | --- | --- |
| Type 1 | Intel, OPPO, Google, Fujitsu, CATT |  |
| Type 2 |  | Ericsson, Spreadtrum, Apple, Google, NTT DOCOMO |
| Type 3 | Intel, vivo, OPPO, Fujitsu, CATT |  |

Therefore, following is proposed.

- Proposal #3.4-C:

For CSI prediction using UE-sided model, de-prioritize Type 2 performance monitoring.

Please provide your view on Proposal #3.4-C.

| Company | Views |
| --- | --- |
| Huawei, HiSilicon | Not support. We think that at least some companies that propose to deprioritize Type2 may assume that the reported ground-truth CSI in type 2 needs to be the channel matrix or raw eigenvectors and, therefore, the report entails a large overhead. We think that the reported ground-truth can actually be legacy codebook/PMI converted from channel matrix. Note that, during inference, it is very likely that UE will convert the predicted CSI of channel matrix to the legacy codebook and feed it back to gNB in forms of legacy PMI, e.g., Type I/II CB, eType II CB, Doppler CB, etc. Therefore, it actually makes more sense that UE reports ground-truth CSI in a similar form (legacy PMI) so the gNB can simply compare the two reported PMIs to derive the monitoring performance metric. The bias between the two PMIs (subject to predicted CSI and ground-truth CSI, respectively) may better reflect the impact of the AI/ML model accuracy to the eventual precoder and UPT. Also, since the overhead of reporting PMI of the ground-truth CSI is much smaller than reporting the channel matrix, we don’t see why Type 2 should be deprioritized. |
| Fujitsu | Support FL proposal. |
| Xiaomi | We are fine with the proposal. |
| Panasonic | We are OK with the proposal. |
| Sony | Support FL proposal |
| Intel | Support |
| ETRI | We tend to agree with Huawei. Moreover, in our view, reporting the GT CSI can also be useful to NW to obtain the CSI at the current time, especially when the prediction performance is low. |
| NTT DOCOMO | Support |
| SPRD | Support FL proposal |
| NEC | Open to this. To our understanding, however, the frequency of performance degradation in a well-trained model is actually quite low. Consequently, the frequency of performance monitoring can be correspondingly reduced. From this perspective, the overhead caused Type 2 appears to be acceptable as well. |
| Apple | Support |
| CATT | OK |

Also, in this meeting, followings regarding on general framework/LCM aspects for AI/ML based CSI prediction were proposed by companies as captured below:

| Huawei | - Proposal 1: The continued study of AI/ML based CSI prediction in Rel-19 focuses on functionality based LCM. |
| --- | --- |
| InterDigital | Proposal 4: Study the following mechanisms for model switching, activation/deactivation overhead reduction: - Improved model generalization techniques - fallback mechanism which can be applicable for both AI/ML based CSI prediction and non-AI/ML based CSI prediction |
| vivo | At least include the following information in functionality identification procedure for AI based prediction - Maximum observation window for AI based prediction - Maximum prediction window for AI based prediction - Processing unit related to AI based prediction |
| Sony | Observation 1: For site specific AI/ML CSI prediction models, the model is trained at the network side after gNB installation at the site/cell concerned. Observation 2: For site specific AI/ML CSI prediction models, the model is stored at the network side and delivered to the UE when it registers or is handed over to the gNB concerned. Proposal 1: To alleviate / resolve some issues related to inter-vendor training, storage and transfer of cell/site-specific AI/ML-based UE-side CSI prediction models, RAN1 will study the following options: - Option 1: Standardized reference model structure + Parameter delivery from NW-side to UE-side - Option 2: Standardized dataset format - Option 3: Standardized model structure + Standardized dataset format Proposal 2: RAN1 study pre-training using standardised format data of the CSI prediction model during gNB manufacturing. |
| ZTE | Proposal 2: The LCM for AI/ML CSI prediction reuses the outcomes defined for AI/ML temporal beam prediction with UE-side model. |
| AT&T | Proposal 1: In CSI prediction using UE sided model use case, study the necessity, feasibility, potential specification impact for at least the following aspects. - Data collection - Performance monitoring for AI/ML model activation/deactivation/switching/fallback - CSI configuration and report - LCM - Aspects specified as condition for functionality based LCM - Aspects that are considered as assistance information. Proposal 3: Study the model ID based LCM for the cell/site/scenario specific models or other model ID based operation like model transfer/delivery. Proposal 7: For CSI prediction using UE sided model consider the following set configurations and their granularity that will be signaled and the corresponding specification impact for the AI/ML LCM. - UE speed - Frequency PRBs - Prediction window - Observation window - Scenario (UMa etc.) - Performance requirement/monitoring - Other additional configurations |
| CEWiT | Proposal-5: In functionality based LCM for UE-sided - model based prediction, consider the existing UE capability report for send AI/ML functionalities. Proposal-6: Study the various AI/ML functionalities which is applicable for AI/ML based CSI prediction. |

Please provide your view if there are essential issues to be handled, if any.

| Company | Views |
| --- | --- |

Conditions and additional conditions

- Summary

Proposals/observations related to conditions and additional conditions are copied below:

| Samsung | Proposal#2: In CSI prediction use case using UE-sided model, consider TRP related aspects for network-side additional condition indication. |
| --- | --- |
| vivo | For training-inference consistency of CSI prediction use case, study to identify whether there are NW additional conditions which impacts the performance of AI based CSI prediction, e.g., CSI-RS virtualization, gNB transmitter’s phase coherent time and phase jumping level. |
| Fujitsu | Proposal 7: - For CSI prediction with UE side model, to ensure the consistency between training and inference, the option of consistency assisted by monitoring should be prioritized. - NW should provide configuration for the monitoring. - RAN1 to further discuss the related signaling and procedures to facilitate the model/functionality selection/activation/deactivation operation. |
| Panasonic | Observation 1: For UE-side data collection for UE-side training, in order to identify the scenario / configuration, how to share the NW-side additional condition should be studied. Instead of informing actual configuration, some kind of configuration ID and /or change timing of NW-side additional condition is necessary. |
| Oppo | Proposal 2: Regarding the data collection for CSI prediction, cell/site/scenario related “condition information” and “additional condition information” should be considered during the data collection stage. Proposal 3: For the “condition” part, following information should be considered: - CSI type to be predicted, e.g. raw channel H or eigenvector W, - CSI-RS configurations, e.g. pattern, time/frequency domain configuration, - transmission related configuration, e.g. bandwidth and sub-band info, antenna ports, rank, SCS, frequency band, - cell/site/scenario related information, e.g. Cell ID. Proposal 4: For the “additional condition” part, following information should be considered: - Cell/site/scenario related information, e.g. region/scenario indication, indoor/outdoor info, UE speed, UE ID, timestamp of data samples, observed SNR - CSI prediction related information, e.g. observation window, prediction window, sample number/interval. |
| Qualcomm | Study data collection and model identification procedure considering NW side additional information |

Moderator thinks that this issue can be firstly handled in 9.1.3.3, and if there are any CSI prediction specific issues, it can be discussed here.

If you have further comment, please comment below.

| Company | Views |
| --- | --- |
| Huawei, HiSilicon | Agree with the moderator’s assessment. |
| Ericsson | Support |
| ETRI | Agree |
| LGE | Agree with FL. |
| Apple | Agree to identify potential CSI prediction specific NW side additional condition. |

Miscellaneous issues

- Summary

Proposals/observations not handled above are copied below:

| NVIDIA | Proposal 1: The inference of one-sided AI/ML model for CSI prediction can be performed at either gNB or UE. Besides studying CSI prediction at UE side, companies are encouraged to study CSI prediction at gNB side to understand the potential gains of performing CSI prediction at gNB side vs. UE side. |
| --- | --- |
| Rakuten | Proposal1 : RAN1 discusses and agrees to dynamically modify PRB group size for preparing CSI feedback information. |
| Mavenir | Proposal 3: Further discuss in TDD mode, how the uplink measurement reference signal can be utilized to reduce the overhead of the CSI-RS, while improving the downlink CSI prediction accuracy. |

Please provide your views on above issues, and please provide essential issues not handled above, if any.

| Company | Views |
| --- | --- |

Summary and proposals for offline session

Discussions for Tuesday.

Observation #2.1-A (SGCS performance over Benchmark 1) (updated)

For the CSI prediction using UE-sided model, till the RAN1#117 meeting, compared to the Benchmark#1 of the nearest historical CSI, in terms of SGCS, from UE speed perspective

- If spatial consistency is not adopted,

  - For 10km/h UE speed, 1 source [Samsung] observes 17% gain

  - For 30km/h UE speed, 9 sources [Lenovo, Apple, Samsung, China Telecom, Spreadtrum, OPPO, CMCC, ZTE, Qualcomm] observe 8.5%~20.6% gain and 2 sources [Intel, Fujitsu] observe 31.1%~31.65% gain.

  - For 60km/h UE speed, 3 sources [Qualcomm, China Telecom, Huawei] observe 2.8%~18% gain

- If spatial consistency is adopted,

  - For 30km/h UE speed, 1 source [Ericsson] observe 23% gain, 2 sources [vivo, MediaTek] observe 62.8%~76.4% gain

  - For 60km/h UE speed, 2 sources [Ericsson, MediaTek] observe 16.2~-22% gain, 1 source [vivo] observes 90.6% gain

  - For 120km/h UE speed, 1 source [vivo] observes 68.2% gain

- Note: the above results are based on the following assumptions

- The observation window considers to start as early as 20ms~50ms.

- A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

- 1 source [Qualcomm] considers eigenvector as model input, and other sources considers Raw channel matrix as model input.

- 2 source [Ericsson, Intel] considers beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, and other sources do not consider pre/post processing.

- The performance metric is SGCS in linear value for layer 1.

- Note: Results refer to Table 2-1 of R1-2405489

Observation #2.1-B (NMSE performance over Benchmark 1) (updated)

For the CSI prediction using UE-sided model, till the RAN1#117 meeting, compared to the Benchmark#1 of the nearest historical CSI, in terms of NMSE, from UE speed perspective:

- For 30km/h UE speed, 4 sources [Lenovo, ZTE, Intel, Ericsson] observe 6 dB~13.11dB gain

- For 60km/h UE speed, 1 source [Ericsson] observe 8 dB gain

- Note: the above results are based on the following assumptions

- The observation window considers to start as early as 20ms~25ms.

- A future 5ms instance from the prediction output is considered for calculating the metric.

- The performance metric is NMSE in linear value for layer 1.

- No pre/post processing is considered.

- 1 source [Ericsson] considers spatial consistency, and other sources do not consider spatial consistency.

- Note: Results refer to Table 2-2 of R1-2405489

Observation #2.1-C (SGCS performance over Benchmark 2) (updated)

For the CSI prediction using UE-sided model, till the RAN1#117 meeting, compared to the Benchmark#2 of non-AI based CSI prediction, in terms of SGCS, from UE speed perspective:

- If spatial consistency is not adopted,

  - For 10km/h UE speed, 1 source [InterDigital] observes 3% gain

  - For 30km/h UE speed,

    - 6 sources [Qualcomm, Lenovo, Spreadtrum, Intel, CATT, OPPO] observe 0%~8.5% gain,

    - 2 sources [Fujitsu, InterDigital] observe 26.3%~48% gain,

    - 1 source [Apple] observe -2% - 18% depending on filter update

  - For 60km/h UE speed, 1 source [Qualcomm] observes -20.6% gain, 2 sources [Huawei, CATT] observe 3.6%~7.8% gain, 1 source [InterDigital] observes 42% gain

- If spatial consistency is adopted,

  - For 30km/h UE speed, 2 sources [Ericsson, MediaTek] observe 0.43%~5% gain, 1 source [vivo] observes 25.7% gain

  - For 60km/h UE speed, 2 sources [Ericsson, MediaTek] observe -1%~3% gain, 1 source [vivo] observes 48.8% gain

  - For 120km/h UE speed, 1 source [vivo] observes 20.6% gain

- Note: the above results are based on the following assumptions

- The observation window considers to start as early as 15ms~50ms.

- A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

- 1 source [Qualcomm] considers eigenvector as model input, and other sources considers Raw channel matrix as model input.

- 21 source [Ericsson, Intel] considers beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, and other sources do not consider pre/post processing.

- [No channel estimation error and no phase continuity is modelled in filter update for Benchmark 2]

- The performance metric is SGCS in linear value for layer 1.

- Note: Results refer to Table 2-3 of R1-2405489 [TBD: R1-240xxxx]

Observation #2.1-D (NMSE performance over Benchmark 2)(updated)

For the CSI prediction using UE-sided model, till the RAN1#117 meeting, compared to the Benchmark#1 of non-AI based CSI prediction, in terms of NMSE, from UE speed perspective, in general the gain of AI/ML based solution is related with the UE speed:

- For 30km/h UE speed, 4 3sources [Ericsson, Lenovo, ZTE, Intel] observe 1 dB~1.42dB gain

- For 60km/h UE speed, 1 source [Ericsson] observe 1 dB gain

- Note: the above results are based on the following assumptions

- The observation window considers to start as early as 15ms~50ms. 20ms~25ms.

- A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

- The performance metric is NMSE in linear value for layer 1.

- [No channel estimation error and no phase continuity are modelled in filter update for Benchmark 2]

- No pre/post processing is considered.

- No spatial consistency is considered.

- Note: Results refer to Table 2-4 of R1-2405489 [TBD: R1-240xxxx]

Observation #2.1-E (Mean UE UPT performance) (updated)

For the CSI prediction using UE-sided model, till the RAN1#117 meeting, in terms of mean UPT, gains are observed compared to both Benchmark#1 of the nearest historical CSI and Benchmark#2 of a non-AI/ML based CSI prediction approach:

- Compared to the benchmark#1 of the nearest historical CSI:

  - For FTP traffic:

    - 1 source [Huawei] observes 4.5%~18.3% gain;

    - 1 source [Ericsson] observes 9%~ 100% gain.

  - For full buffer traffic:

    - 1 source [Lenovo] observes 10.5% gain.

    - 1 source [Fujitsu] observes 2.1%~9.9%11.6%~27% gain.

    - 1 source [MediaTek] observes 7.7% gain.

- Compared to the benchmark#2 of non-AI/ML based CSI prediction:

  - For FTP traffic:

    - 1 source [Huawei] observes 0.2%~14.8% gain;

    - 1 source [CATT] observes 47.1%~50.1%  -6.1%~-2.1% gain.

    - 1 source [Ericsson] observes 17%~73% gain.

    - 1 source [InterDigital] observes 5.7%~28.6% gain.

  - For full buffer traffic:

    - 1 source [CATT] observes 0.4%~6%  0.2%~1.2% gain.

    - 1 source [Lenovo] observes 0.2% gain.

    - 1 source [vivo] observes 6.5% gain.

    - 1 source [Fujitsu] observes 2.6%~6.3%7%~9.7% gain.

    - 1 source [MediaTek] observes -2% gain.

- Note: the above results are based on the following assumptions

  - The UE speed is 30km/h or 60km/h.

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms ~ 20ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - The performance metric is mean UPT for Max rank 1.

  - 1 source [Ericsson] considers beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, and other sources considers no pre/post processing.

  - 3 sources [vivo, Ericsson, MediaTek] consider spatial consistency. Other sources do not consider spatial consistency.

  - No channel estimation error and no phase continuity are modelled

  - Note: Results refer to Table 2-5/2-4/2-4/2-8 of R1-2405489

Observation #2.1-F (5% UE UPT performance)(updated)

For the CSI prediction using UE-sided model, till the RAN1#117 meeting, in terms of 5% UE UPT, gains are observed compared to both Benchmark#1 of the nearest historical CSI and Benchmark#2 of a non-AI/ML based CSI prediction approach:

- Compared to the benchmark#1 of the nearest historical CSI:

  - For FTP traffic:

    - 1 source [Huawei] observes 1.2%~4.2% gain;

    - 1 source [Ericsson] observes 9%~ 35% gain.

  - For full buffer traffic:

    - 1 source [vivo] observes 86.5% gain.

    - 1 source [Lenovo] observes 51% gain.

    - 1 source [Fujitsu] observes 11.6%~27%2.1%~9.9% gain.

    - 1 source [MediaTek] observes 8.7% gain.

- Compared to the benchmark#2 of non-AI/ML based CSI prediction:

  - For FTP traffic:

    - 1 source [Huawei] observes 0.7%~3.1% gain;

    - 1 source [CATT] observes -6.1%~-2.1% 47.1%~50.1% gain.

    - 1 source [Ericsson] observes 9%~35% gain.

    - 1 source [InterDigital] observes 2.1%~19.6% gain.

    - 1 source [MediaTek] observes 0.02%~0.92% gain.

  - For full buffer traffic:

    - 3 sources [CATT, Fujitsu, MediaTek] observe 0.4%~6.3% gain.

    - 1 source [Lenovo] observes 24% gain.

    - 1 source [vivo] observes 24.7%~41.3% gain.

- Note: the above results are based on the following assumptions

  - The UE speed is 30km/h or 60km/h.

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms ~ 20ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - The performance metric is mean UPT for Max rank 1.

  - 1 source [Ericsson] considers beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, and other sources considers no pre/post processing.

  - 3 sources [vivo, Ericsson, MediaTek] consider spatial consistency. Other sources do not consider spatial consistency.

  - No channel estimation error and no phase continuity are modelled

  - Note: Results refer to Table 2-5/2-4/2-4/2-8 of R1-2405489

Proposed agreement #3.4-A(updated):

For the boundary between Type 1 and Type 3 performance monitoring, the difference is whether UE reports performance metric or performance monitoring output to NW.

- The monitoring output is calculated based on performance metric and/or threshold criterion if configured.

- The monitoring output can include indication/event of monitoring.

  - FFS: definition of an event

Proposed observation #3.4-B:

For CSI prediction using UE-sided model, for performance monitoring, following specification impacts are identified.

- Type 1

  - Definition of performance metric

  - Definition of threshold criterion, if configured

  - Definition of monitoring output, and reporting mechanism of monitoring output

- Type 2

  - Definition of ground truth CSI, and reporting mechanism of monitoring output ground truth CSI.

- Type 3

  - Definition of performance metric, and reporting mechanism of performance metric

- For all types of performance monitoring, NW indication to the UE of the decision regarding the monitoring action

Discussions for Wednesday.

Summary and proposals for online session

Proposals for Tuesday.

Proposal #3.4-A:

For the boundary between Type 1 and Type 3 performance monitoring, the difference is whether UE reports performance metric or performance monitoring output to NW.

- The monitoring output is calculated based on performance metric and/or threshold criterion if configured.

- Monitoring output can be information about whether a model is functioning or not, or the event of monitoring.

  - FFS: definition of an event

Proposed observation #3.4-B:

For CSI prediction using UE-sided model, for performance monitoring, following specification impacts are identified.

- Type 1

  - Definition of performance metric

  - Definition of threshold criterion, if configured

  - Definition of monitoring output, and reporting mechanism of monitoring output

- Type 2

  - Definition of ground truth CSI, and reporting mechanism of ground truth CSI.

- Type 3

  - Definition of performance metric, and reporting mechanism of performance metric

- For all types of performance monitoring, NW indication to the UE of the decision regarding the monitoring action

Proposal #3.4-C:

For CSI prediction using UE-sided model, de-prioritize Type 2 performance monitoring.

Proposals for Thursday.

FL note for RAN1#118

Agreements

Agreements in RAN1#116bis

Agreement

- For the AI/ML based CSI prediction, adopt following assumptions as a baseline for evaluation purpose

  - UE speed: 30km/h, 60km/h

    - Others can be additionally submitted, e.g., 10km/h, 120km/h

  - Observation window (number/distance): 5/5ms,10/5ms

    - Others can be additionally submitted, e.g., 4/5ms, 15/5ms

  - Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance):  1/5ms/5ms, 4/5ms/5ms

    - Others can be additionally submitted, e.g., 2/5ms/5ms, 3/5ms/5ms, 1/5ms/10ms

  - For other assumptions, reuse Rel-18 baseline

Agreement

- For the AI/ML based CSI prediction, for CSI report, adopt following as a baseline for evaluation purpose

  - N4 value: 1, 4e

    - Others can be additionally submitted, e.g., 2, 8

  - paramCombination-Doppler-r18: 6,7 or paramCombination -r16 = 5,6 (for Benchmark 1)

    - Others can be additionally submitted.

    - Note: The same selected parameter combination shall be applied for benchmarks.

  - CSI report periodicity: 5ms, 20ms (encouraged)

    - Others can be additionally submitted, e.g., 10ms

Conclusion

Consider error modelling in TR36.897 Table A.1-2 as a baseline if channel estimation error is modeled.

  - Other modelling is not precluded, and companies should report how to model channel estimation error if other modelling is considered.

Conclusion

If phase discontinuity is modeled, it is modelled as a uniform distribution between -ϕmax,ϕmax within a time window ofTwindow, where ϕmax=40 degrees and Twindow=20ms can be a baseline.

- Other modelling is not precluded, and companies should report how to model phase discontinuity if other modelling is considered, and additional Twindow.if adopted

Conclusion

For the phase discontinuity modelling, it is clarified that

- A fixed phase for all CSI-RS observations within the time window, and another fixed phase for the next time window. The phases are according to uniform distribution.

Conclusion

- For evaluation of the UE-sided model based CSI prediction, UE distribution of (80% indoor, 20% outdoor) can be optionally simulated.

Note: Indoor speed is 3 km/h, outdoor speed is chosen from the following options: 30 km/h, 60 km/h. Assumption on O2I car penetration loss and spatial consistency follow the Rel-18 AI/ML based CSI prediction

Agreement

For the results template used to collect evaluation results for UE -sided model based CSI prediction, adopt Table 6 used in Rel-18 as starting point with the following addition:

- Assumption

  - UE distribution (Baseline: 100% outdoor, Optional: 80% indoor, 20% outdoor)

  - Whether/how channel estimation error is modelled

  - Whether/how phase discontinuity is modelled

  - Methods used to handle the phase discontinuity (if applied)

- Benchmark 2

  - FLOPs/M

  - Details of complexity calculation, e.g., complexity of prediction and complexity of filter update

Agreement

- For the results template used to collect evaluation results for UE-sided model based CSI prediction using localized models, adopt Table 6 used in Rel-18 as starting point, capturing the generalized model result and the localized model result as separate columns, with the following additions for the localized model:

- Dataset description

  - Local region modelling: e.g., Option 1 or Option 2, and further details

  - Temporal modelling: e.g., how temporal variation is modelled in train and test sets

  - Dataset description for generalized model

Agreement

For the UE-sided model based CSI prediction, for optional evaluation using AP CSI-RS, consider following assumption on observation window (number/distance)

- Observation window: 12/2ms, 8/2ms, 4/2ms

- Others can be additionally submitted

Agreement

For AI/ML based CSI prediction, at least for inference, legacy CSI-RS configuration can be a starting point. Further study on whether there is a need for specification enhancement.

Agreement

- At least for inference, for UE-sided model based CSI prediction, legacy feedback mechanism using codebook type set to “typeII-Doppler-r18” is a starting point of discussion. Study the necessity and potential specification impacts including at least following aspects:

- CSI processing criteria and timeline

Agreement

For performance monitoring for functionality-based LCM, further study on details of type 1,2 and 3, e.g., potential specification impact, pros/cons aspects.

- To clarify the boundary between type 1 and type 3

- To clarify definition of monitoring output and performance metric

Agreements in RAN1#116

Agreement

For Rel-19 study on CSI prediction, consider EVM agreed in Rel-18 CSI prediction based on UE-sided model as a starting point.

- FFS on additional assumptions, e.g., channel estimation error, phase discontinuity, CSI-RS periodicity.

- Note: Rel-18 CSI-RS configuration/reporting can be reused.

- Note: additional EVM and corresponding template to collect the results can be updated.

Agreement

For Rel-19 study on CSI prediction, companies are encouraged to evaluate throughput performance by comparing performance with non-AI/ML based CSI prediction.

- R18 eType II doppler codebook is assumed for CSI report for both AI/ML and Non AI/ML prediction.

- Companies to report the assumption for N4, which could be 1, 2, 4, 8.

Note: Non-AI/ML based CSI prediction (Benchmark 2) can include statistical model based CSI prediction (e.g., based on Kalman filter, Wiener filter, Auto-regression).

Agreement

For evaluation, to report computational complexity in unit of FLOPs including additional complexity if applicable, e.g., update of filter, and their assumption on non-AI based CSI prediction when performance results are provided.

Conclusion

For the evaluation of the AI/ML based CSI prediction, it is up to companies to choose the modelling method and companies should report if ‘Channel estimation’ and/or ‘phase discontinuity’ is/are considered by companies.

Agreement

For the evaluation of the AI/ML based CSI prediction, consider following CSI-RS configuration

- Periodic: 5 ms periodicity (baseline), 20 ms periodicity (encouraged)

- Aperiodic: Optional, CSI-RS burst with K resources and time interval m slots (based on R18 MIMO eType-II)

Note: Companies to report observation window (number/distance) and prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance) on their evaluation.

Conclusion

For Rel-19 study on CSI prediction only, consider UE-sided model only.

Agreement

- For CSI prediction evaluations, to verify the generalization/scalability performance of an AI/ML model over various configurations, to evaluate one or more of the following aspects:

- Various UE speeds (e.g., 10km/h, 30km/h, 60km/h, 120km/h)

- Various deployment scenarios

- Various carrier frequencies (e.g., 2GHz, 3.5GHz)

- Various frequency granularity assumptions

- Various antenna port numbers (e.g., 32 ports, 16 ports)

- To report the selected configurations for generalization verification

- To report the method to achieve generalization over various configurations and/or to achieve scalability of the AI/ML input/output, including pre-processing, post-processing, etc.

- To report generalization cases where multiple aspects (e.g., combination of above) are involved in one dataset, if adopted.

- To report the performance and requirement (e.g., updating filter parameters, convergence of filter) for non-AI/ML-based CSI prediction to handle the various scenarios/configurations.

Agreement

For the evaluation of AI/ML-based CSI prediction using localized models in Release 19, consider the following options as a starting point to model the spatial correlation in the dataset for a local region:

- Option 1: The dataset is derived from UEs dropped within the local region, with spatial consistency modelling as per TR 38.901.

    - E.g., Dropped in a specific cell or within a specific boundary.

- Option 2: By using a scenario/configuration specific to the local region.

    - E.g., Indoor-outdoor ratio, LOS-NLOS ratio, TXRU mapping, etc.

Note: While modelling the spatial correlation, strive to ensure that the dataset distribution also correctly captures the decorrelation due to temporal variations in the channel. To report methods to generate training and testing dataset.

Reference

- RP-240774	Revised WID: AI/ML for NR air interface	Qualcomm

- R1-2403909	AI/ML for CSI prediction	Ericsson

- R1-2403931	Discussion on AI/ML for CSI prediction 	Huawei, HiSilicon

- R1-2403961	Discussion on study for AI/ML CSI prediction	Tejas Networks Limited

- R1-2403975	AI/ML for CSI prediction	Intel Corporation

- R1-2404016	Discussion on AIML for CSI prediction	Spreadtrum Communications, BUPT

- R1-2404053	Discussion on AI/ML-based CSI prediction	InterDigital, Inc.

- R1-2404103	Discussion for further study on AI/ML-based CSI prediction	Samsung

- R1-2404167	Discussion on CSI prediction	vivo

- R1-2404274	Discussion on AI based CSI prediction	Apple

- R1-2404386	Study on AI/ML for CSI prediction	CATT

- R1-2404422	Discussion on AI/ML-based CSI prediction	China Telecom

- R1-2404446	Discussion on AI/ML for CSI prediction	CMCC

- R1-2404492	Discussion on CSI prediction inference in AI/ML	Sony

- R1-2404527	On AI/ML for CSI prediction	Lenovo

- R1-2404538	Additional study on AI-enabled CSI prediction	NVIDIA

- R1-2404547	Study on CSI prediction	LG Electronics

- R1-2404569	Discussion on AI/ML for CSI prediction	SK Telecom

- R1-2404584	Discussion on CSI prediction with AI/ML	Fujitsu

- R1-2404603	Discussion on AI/ML model based CSI prediction	Xiaomi

- R1-2404653	Discussion on CSI prediction	NEC

- R1-2404684	AI/ML based CSI Prediction	Google

- R1-2404702	Discussion on study for AI/ML CSI prediction	ZTE

- R1-2404744	Discussion on AI/ML for CSI prediction	Panasonic

- R1-2404810	Varying CSI feedback granularity based on channel conditions	Rakuten Mobile, Inc

- R1-2404879	Additional study on AI/ML-based CSI prediction	OPPO

- R1-2404906	AI/ML for CSI Prediction	Nokia

- R1-2405015	Discussion on AI/ML for CSI prediction	AT&T

- R1-2405032	Discussion on AI/ML for CSI prediction	NTT DOCOMO, INC.

- R1-2405088	Additional Study on AI/ML for CSI Prediction	MediaTek Inc.

- R1-2405125	AI/ML for CSI prediction	Mavenir

- R1-2405145	Additional study on CSI prediction	Qualcomm Incorporated

- R1-2405236	Discussion on  AI/ML for CSI Prediction	CEWiT

- R1-2405290	Discussion on study of AIML for CSI prediction	IIT Kanpur, Indian Institute of Tech (M)