3GPP TSG RAN WG1 #118	R1-2406194

Maastricht, NL, August 19th – 23rd, 2024

Source:	vivo

Title:	Discussion on LP-WUS operation in IDLE/INACTIVE modes

Agenda Item:	9.6.2

Document for:	Discussion and Decision

- Introduction

In RAN#117 meeting, the following agreements and conclusion on LP-WUS operation in RRC idle/inactive modes are provided in [1].

Conclusion Regarding the “FFS: How to determine the received power of LP-SS in OOK ON symbols” for LP-RSRP, no additional work in RAN1. Agreement It is supported that UEs monitoring the same PO are divided into multiple subgroups, where LP-WUS can provide wake-up indication for each subgroup. Consider the following options: - Option 1: UEs monitoring the same PO monitor the same LO - Option 2: UEs corresponding to different POs monitor the same LO - Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO. - Combinations of the above options can be considered. R1-2404301 Summary #3 on LP-WUS operation in IDLE/INACTIVE mode Moderator (Apple) Agreement For LP-SS based LP-RSRQ, LP-RSRP and LP-RSSI are measured within the same bandwidth. Agreement For LP-RSSI definition for LP-RSRQ, the following is agreed - Option 1: LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols. Note: Above does not constrain LP-SS sequence design for OOK Working Assumption From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively. - Above is applicable for both time-domain processing or frequency-domain processing - Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications Agreement For idle/inactive mode, the maximum number of information bits (excluding CRC) in a LP-WUS is Z, where Z <= [8 or 16]. - FFS the exact value of Z Agreement For idle/inactive mode, the maximum number of subgroups per PO is X, where 8 <= X <= 256. - FFS the exact value of X.

Based on the discussion in RAN1#117, we provide our views on the procedure and configuration of LP-WUS monitoring in RRC idle/inactive mode.

- Procedure of LP-WUS indicating paging monitoring

  - UE LP-WUS monitoring behavior

In R17 PEI discussion, the specified UE behaviour for PEI monitoring is: PEI indicates UE should monitor a PO if UE’s group/subgroup is paged. And UE is not required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO.

For the UE behaviour of LP-WUS monitoring, we can comply with the same UE behaviour of R17 PEI for the sake of saving LP-WUS resource overhead. But having to say, since LP-WUS may not achieve the same detection performance as that of PDCCH based PEI, the case that UE fails to detect LP-WUS due to out of LP-WUS coverage should be separately considered as provided in section 2.3 of the entry/exit condition for LP-WUS monitoring.

Proposal 1: UE is not required to monitor a PO, if UE does not detect LP-WUS at all LP-WUS monitoring occasion(s) for the PO.

  - Subgrouping indication of LP-WUS

Different from that of R17 PEI, the transition energy for MR wake up from ultra-deep sleep state is much larger e.g., 15000 or 40000. The more frequent waking up, the less power saving gain is obtained. Given that, the effective paging rate of UE, which is determined by: 1) per UE paging rate, 2) number of UEs in the same subgroup and 3) FAR, should be kept as lower as possible. To achieve similar or finer waking-up granularity as PEI, at least 8 subgroups are required for LP-WUS design. Besides, when the number of UEs associated with a PO increases, the number of subgroups shall be increased accordingly to keep the effective paging rate, otherwise, the power saving gain is not anticipated. It is observed when the target UE effective paging rate of LP-WUS is no larger than 1%, significant power saving gain e.g., 85% can be guaranteed by applying LP-WUS [2]. Targeting an effective paging rate to keep such power saving gain, i.e., 1%, the number of required subgroups, the size of LP-WUS information bits, and the corresponding resource overhead can be analyzed, where the assumptions of connection density and packets arrival rate provided by ITU for IoT case (the detail is given in the last row of table 1) can be referred.

However, for the connection density of 106 UEs per km2, not all users desire both low latency and low power consumption, which mainly benefits from LP-WUR/WUS, and thus, it is more practical to consider a reduced connection density for LP-WUS. Given the connection density of AIoT devices is in the order of 105 per km2, a lower density of LP-WUS device i.e., <=105 per km2 will be more realistic, considering a higher WUR cost than AIoT device. Besides, considering the network paging strategy that an IDLE/INACTIVE mode UE will be firstly paged in its last serving cell. And the network can find it at most cases for a LP-WUS UE with low mobility. In case this UE cannot be reached, the paging range is gradually extended until the entire tracking area, and thus, the actual paging range will be probably quite smaller than the whole tracking area as provided by 500 sites, especially for the IoT UEs with low mobility.

Based the analysis above, targeting 1% UE effective paging rate, the corresponding required number of subgroups per PO and paging area size can be derived and the overhead of LP-WUS for different cases are also listed in Table 1. The calculation process of resource overhead of LP-WUS in the system can be found in Appendix A.

Table 1 The required number of subgroups, the size of LP-WUS information bits with the corresponding resource overhead of LP-WUS to achieve the 1% target UE effective paging rate and up to 85% power saving gain.

| Paging area size | #UEs with LP-WUS /km2 | The number of UE per paging range | #PO per cell | #Required subgroups per PO | Subgroup indication method | #info bits per LP-WUS | #Total bits per LP-WUS | #Resource per LP-WUS (5MHz) | Network resource overhead for LP-WUS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 127 | 105 | 917504 | 64 | 256 | Codepoint with encoded bits | 8 | 10 | 20 OS | 18.29% |
| 32 | 105 | 229376 | 64 | 64 | Codepoint with encoded bits | 6 | 10 | 20 OS | 4.57% |
| 16 | 105 | 114688 | 64 | 32 | Codepoint with encoded bits | 5 | 8 | 16 OS | 1.83% |
| 8 | 105 | 57344 | 64 | 16 | Codepoint with encoded bits | 4 | 8 | 16 OS | 0.91% |
| 4 | 105 | 28672 | 64 | 8 | Codepoint with encoded bits | 3 | 8 | 16 OS | 0.46% |
| 127 | 105 | 917504 | 64 | 256 | Codepoint with 1-stage sequence | 8 | 256 | 256 | 234.06% |
| 32 | 105 | 229376 | 64 | 64 | Codepoint with 1-stage sequence | 6 | 64 | 64 | 14.63% |
| 16 | 105 | 114688 | 64 | 32 | Codepoint with 1-stage sequence | 5 | 32 | 32 | 3.66% |
| 8 | 105 | 57344 | 64 | 16 | Codepoint with 1-stage sequence | 4 | 16 | 16 | 0.91% |
| 4 | 105 | 28672 | 64 | 8 | Codepoint with 1-stage sequence | 3 | 8 | 8 | 0.23% |
| 127 | 105 | 917504 | 64 | 256 | Codepoint with 2-stage sequence | 8 | 32 | 32 | 29.26% |
| 32 | 105 | 229376 | 64 | 64 | Codepoint with2-stage sequence | 6 | 16 | 16 | 3.66% |
| 4 | 105 | 28672 | 64 | 8 | Bitmap | 8 | 16 | 32 OS | 0.88% |
| 8 | 105 | 57344 | 64 | 16 | Bitmap | 16 | 24 | 48 OS | 2.55% |
| Notes: - 1,000,000 user/km2 connection density, with packet arrival every 1 message/2 hours/device i.e., 0.018% per 1.28s. [ITU M.2412] - TA (tracking area) include 500 sites and each site with 3 sectors/cells, ISD=500m. - To ensure sufficient UE power saving benefit by LP-WUS, per UE effective wake-up rate is set as up to 1% - For 256, 64, 32, 16, 8 subgroups, by codepoint method, 8, 6, 5, 4, 3 bits are accordingly required. But, to guarantee the FAR requirement by detection i.e., false alarm from noise targeting to be 1%, 10, 10, 8, 8, 8 are assumed as total number of information bits per LP-WUS respectively. |  |  |  |  |  |  |  |  |  |

From Table 1, in case the required number of subgroups per PO is >=32, the derived total number of LP-WUS UE in paging area will be more than 110,000 which is too large to be realistic. Moreover, the larger number of LP-WUS UE, the bigger system resource overhead will be caused by LP-WUS. for example, 256 subgroups per PO targeting 1% per UE effective paging rate will result in 19% or 234% system overhead for LP-WUS, which is totally unacceptable to the NR system. However, if we consider more realistic cases e.g., around 20000 ~50000 UEs using LP-WUS per paging area as shown in the highlighted rows, the required number of subgroups per PO decrease to 8~16 and the resource overhead of LP-WUS will be controlled to less than 3%. Hence, targeting to a significant power saving gain e.g., 85% and under a reasonable number of LP-WUS UE assumption, 8~16 subgroups per PO is sufficient.

Observation 1: With more realistic assumptions e.g., around 20000 ~50000 UEs using LP-WUS per paging area, 8~16 subgroups per PO is sufficient to guarantee significant UE power saving gain e.g., 85% with the system overhead for LP-WUS transmissions to be less than 3%.

Proposal 2: Support the maximum number of subgroups to be 8~16.

  - Entry/exit condition for LP-WUS monitoring

In the previous meeting, a working assumption for the entry and exit condition of LP-WUS monitoring is endorsed which should be further confirmed. Since before UE enters LP-WUS monitoring, RRM measurement by UE MR is performed, it is proper to use MR measurement for the entry condition. Besides, RRM measurement via LR could be further considered into the determination of the entry condition if UE enables LR in advance before entering LP-WUS monitoring. And for the exit condition, in case the serving cell quality based on LR RRM measurement is lower than a threshold during LP-WUS monitoring, UE can decide to exit the LP-WUS monitoring. Additionally, if the RRM measurement is not fully offloaded to LR i.e., UE still does relaxed MR RRM measurement, the exit criteria can also consider MR RRM measurement.

Proposal 3:  Confirm the working assumption for the entry/exit conditions for LP-WUS monitoring agreed in RAN1 #116bis meeting.

Working Assumption From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode, - The UE may start LP-WUS monitoring if - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB, and/or - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up - The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB, and/or - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - FFS the serving cell measurement metrics - The entry/exit thresholds can be configured separately for different types of LR - It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB. - Note: This may be revisited based on the RAN2/RAN4 discussion.

Besides, for the UE behavior of RRM measurement when entering or exiting LP-WUS monitoring has also been discussed in the previous meeting, but no agreement has been achieved due to lack of time. According to TR 38.869 conclusion [2], significant power saving gain by LP-WUS monitoring can be only achieved when MR based RRM measurement is relaxed e.g., more than 8 times or fully offloaded to LR. And the network can have the flexibility to configure either relaxed MR based RRM measurement with suitable relaxation factor or fully offload MR measurement to LR i.e., MR does not perform measurement during LP-WUS monitoring. Given that, for the UE behavior on RRM measurement when UE starts LP-WUS monitoring, both serving cell and neighbor cell RRM measurement relaxation or offloading should be considered. And different conditions for different relaxation levels i.e., RRM measurement relaxation or offloading can be further studied in RAN2.

Proposal 4: Support the following UE behavior on RRM measurement when UE starts LP-WUS monitoring,

- UE performs LR based serving cell measurement, and performs relaxed or does not perform MR based serving cell measurement;

- UE performs MR based relaxed neighbour cell measurement or UE stops neighbour cell measurement.

- FFS the conditions for different RRM measurement cases.

Besides, regarding the specific threshold of the RRM relaxation/offloading criteria, it could be same as or different from the threshold of entry condition for LP-WUS monitoring. In order to guarantee serving cell measurement is relaxed/offloading and neighbor cell measurement is relaxed when UE monitors LP-WUS, thresholds of the criteria for the serving/neighbor cell measurement relaxation should be lower than or equal to the threshold of the entry condition for LP-WUS monitoring. Based on the analysis above, we provide two examples to illustrate the thresholds and UE behaviors for the serving cell measurement relaxation/offloading and neighbor cell measurement relaxation comparing to the LP-WUS monitoring in the following figures.

Figure 1: Thresholds and UE behaviors for serving cell offloading/neighbor cell measurement relaxation and LP-WUS monitoring

Figure 2: Thresholds and UE behaviors for serving/neighbor cell measurement relaxation and LP-WUS monitoring

For simplicity, the serving cell measurement offloading and serving cell measurement relaxation are provided in Figure 1 and Figure 2, respectively. All the criteria for serving cell measurement relaxation/offloading and neighbor cell measurement relaxation and entry condition for LP-WUS monitoring are based on the serving cell quality via MR.

For neighbor cell measurement relaxation, it refers to the neighbor cell measurement for intra-frequency/inter-frequency with equal or lower priority frequency. As for the inter-frequency with higher priority, it will be more complicated and can be further discussed later. When the serving cell quality based on the measurement via MR exceeds the legacy S-measure threshold, there is no neighbor cell measurement to be performed. Besides, we think the reasonable configuration is that the threshold of the legacy S-measure threshold is higher than or equal to the threshold for the neighbor cell measurement relaxation (i.e. further relaxation is used to differentiate the RRM relaxation in Rel-16/17), because the neighbor cell measurement can be further relaxed only if there is neighbor cell measurement.

In Figure 1, there are two alternatives for the threshold of entry condition for LP-WUS monitoring and threshold for serving cell offloading criteria, which could either higher or lower than legacy S-measure threshold. Besides, the threshold of entry condition for LP-WUS monitoring is the same as the threshold of serving cell offloading criteria. The reason is the entry condition for LP-WUS monitoring is based on the service cell measurement via MR, otherwise, there is no serving cell measurement via MR when serving cell measurement is offloaded. For the green arrow, it represents that UE performs serving cell offloading if the serving cell offloading criteria are fulfilled.

For Case 1, i.e. the threshold of serving cell offloading criteria and the threshold of criteria for LP-WUS monitoring is higher than legacy S-measure threshold, when the entry condition for LP-WUS monitoring is fulfilled and the serving cell offloading criteria is fulfilled, UE will monitor LP-WUS and perform serving cell measurement offloading from MR to LR and no neighbor cell measurement. For Case 2, i.e. the threshold of serving cell offloading criteria and the threshold of criteria for LP-WUS monitoring is lower than legacy S-measure threshold, UE will monitor LP-WUS and perform serving cell measurement offloading from MR to LR, and MR neighbor cell measurement relaxation.

In Figure 2, similar to Figure 1, there are two alternatives for the threshold of entry condition for LP-WUS monitoring and serving cell measurement relaxation criteria. Besides, the threshold of entry condition for LP-WUS monitoring is higher than or equal to the threshold of the serving cell relaxation criteria. For Case 1, i.e. the threshold of serving cell relaxation criteria and the threshold of criteria for LP-WUS monitoring is higher than legacy S-measure threshold, when the entry condition for LP-WUS monitoring is fulfilled and the serving cell measurement relaxation is fulfilled, UE will monitor LP-WUS and perform serving cell measurement relaxation and no neighbor cell measurement. For Case 2, i.e. the threshold of serving cell relaxation criteria and the threshold of criteria for LP-WUS monitoring is lower than legacy S-measure threshold, UE will monitor LP-WUS and perform serving cell measurement relaxation, and MR neighbor cell measurement relaxation.

Proposal 5: Thresholds of for serving/neighbor cell measurement relaxation should be lower than or equal to the threshold of entry condition for LP-WUS monitoring.

  - RRM measurement metrics by OFDM-based LR

In the last meeting, for LP-RSSI definition, we have agreed on option 1 that LP-RSSI is linear average of total received power in ON and OFF OOK symbols. It is still unclear which ON/OFF OOK symbols of the LP-SS can be measured. If it can be up to UE implementation to choose some of OOK ON/OFF symbols to be measured, LP-RSSI would vary with different ratio of measured OOK ON/OFF symbols. To keep the consistent understanding on the definition of LP-RSSI, we propose to support that LP-RSSI is the liner average of the same number of ON and OFF OOK symbols.

Agreement For LP-RSSI definition for LP-RSRQ, the following is agreed - Option 1: LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols. Note: Above does not constrain LP-SS sequence design for OOK Agreement From RAN1 perspective, at least the following metrics can be supported for RRM serving cell measurement performed by OOK-based receiver based on LP-SS: - LP-RSRP LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols. - FFS: How to determine the received power of LP-SS in OOK ON symbols Agreement For LP-SS based LP-RSRQ, LP-RSRP and LP-RSSI are measured within the same bandwidth.

Proposal 6: For LP-RSSI definition for LP-RSRQ, LP-RSSI is the liner average of total received power in the same number of ON and OFF OOK symbols.

In the current TS 38.215, SS-RSRP is defined as the linear average over the power contributions (in [W]) of the resource elements that carry secondary synchronization signals. And SS-RSRQ is defined as the ratio of N×SS-RSRP/NR carrier RSSI, where N is the number of resource blocks in the NR carrier RSSI measurement bandwidth. So, the definition and derivation of NR RSRP and NR RSRQ is clear. But for LP-RSRP and LP-RSRQ definitions, the current progress as above is not enough to let people on the same page. In our view, the frequency domain measurement granularity of LP-RSRP and LP-RSSI for LP-RSRQ e.g., a RE, an RB or the bandwidth of LP-SS etc. need to be further defined. Besides, for a simple derivation of LP-RSRQ, the same frequency granularity can be used for both LP-RSRP and LP-RSSI.

Proposal 7: Define the frequency domain granularity of LP-RSRP and LP-RSSI for LP-RSRQ.

In the last meeting, the following working assumption on RRM measurement metrics based on SSS for OFDM-based LP-WUR has been agreed as follow.

Working Assumption From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively. - Above is applicable for both time-domain processing or frequency-domain processing - Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications

In some extent, we support to use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ. But, for the time resource used to measure LP-SSS-RSRP and LP-SSS-RSSI, we still think it needs to be further considered. In NR, for the definition of SS-RSRP and SS-RSSI, the measurement time resource(s) usually used for serving cell and neighbour cell measurements are confined within SMTC window duration. While SSS for OFDM based receiver is used only for serving cell measurement according to WID. There seems no need to restrict the time measurement resource for LP-SSS-RSRP and LP-SSS-RSSI to be within the SMTC. Hence, for flexibility consideration, the network can configure the measurements time resource for LP-SSS-RSRP and LP-SSS-RSSI different from the current NR SMTC window.

Proposal 8: The network configures the measurement time resource for LP-SSS-RSSI, which can be different from NR SMTC.

- LP-WUS monitoring configuration

  - The relationship between LO and PO monitoring

In the previous meetings, companies may have different understanding on the term LO. Some companies view that different MO of a LO can carry different LP-WUS content, but the others may not. To avoid misunderstanding on this, we want to clarify our understanding on LO as multiple MOs of a LO needs to contain the same subgrouping indication information. And different LOs can carry different subgrouping information.

Proposal 9: Clarify that the same subgrouping information will be carried in the MOs of a LO.

Besides, according to the progress on the LO and PO relationship achieved in the last meeting, we further support option 1 i.e., LO and PO keeps one to one mapping relationship which is the baseline option in fact. And for option 2, one LO indicates multiple POs will no doubt increase the carrying bits of LP-WUS and give the challenge to LP-WUS coverage with the assumption that one LO indicates multiple POs will directly double the number of required bits. For option 3, there will be multiple LOs associated with one PO, so the system overhead of LP-WUS will accordingly increase.

Agreement It is supported that UEs monitoring the same PO are divided into multiple subgroups, where LP-WUS can provide wake-up indication for each subgroup. Consider the following options: - Option 1: UEs monitoring the same PO monitor the same LO - Option 2: UEs corresponding to different POs monitor the same LO - Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO. - Combinations of the above options can be considered.

Proposal 10: For the relationship between LO and PO, support Option 1 i.e., UEs monitoring the same PO monitor the same LO.

  - Determination of LP-WUS offset

According to the discussion in R18 SI on the required wake-up delay including both ramp up time and sync time from detecting LP-WUS to be ready for PO monitoring [2], it is explicit that the wake-up delay is vary from different UE capabilities. For example, for UE1, due to its high capability, the wake-up delay can be shortened to 400ms, while for UE2 with low capability, 1 second wake-up delay may be required. Hence, UE should report its required wake-up delay from woken up by LP-WUS to be ready for PO reception.

Proposal 11: Support to report UE capability on wake-up delay which is the time offset between receiving LP-WUS targeted to itself and being ready for PO reception.

Regarding the number of wake-up delay which can be reported by a UE, we think it should be at the UE's discretion to select and report only the highest priority wake-up delay based on the timing and power consumption requirements, even if multiple wake-up delays can be supported by UE. That is to say, there seems no certain needs for UE to report multiple wake-up delays since the network will basically send LP-WUS based on one of UE’s wake-up capabilities. Additionally, the network can configure one or multiple offsets according to the reported wake-up delays from different UEs. For example, if the range of the reported wake-up delays from different UEs is wide, then the network can effectively configure multiple offsets to address different wake up requirements.

Proposal 12: Support the network to configure one or multiple offsets between LO and PO.

  - Multiple beam operation for LP-SS and LP-WUS

For LP-SS design, multi-beam operation can be the same as that for SSB. For example, as shown in Figure 3, there is 4 Tx beams. And the network is required to transmit multiple times of LP-SS by using 4 beams in turn. And one LP-SS occasion is associated with one Tx beam. The LP-SS monitoring occasions associated with beams respectively is fixed. Furthermore, LP-SS can indicate its beam index to avoid the LP-SS timing error due to the sync offset of LR.

Figure 3  One example of LP-SS with multiple MOs per duty cycle

Proposal 13: For multi-beam operation of LP-SS, each LP-SS MO is associated with each transmitted beam in turn.

Multi-beam operation for LP-WUS is agreed in the last meeting, while there still needs further discussion on the functionality of the number of LP-WUS MOs for each beam i.e., K. Considering the flexibility of the network, there can be multiple MOs associated with one beam i.e., K can be larger than 1, so that the network can have multiple opportunity to transmit LP-WUS with a certain beam. Figure 4 provides one example for 12 MOs for LP-WUS monitoring within a period, wherein N=4 beams with K=3 candidate MOs per beam ordered in similar way as legacy multiple PEI/PO MOs, i.e., the [x*N+M]th LP-WUS MO corresponds to the xth candidate MO for Mth beam of LP-WUS, where x=0,1,…,K-1 and M=1, 2,…N. In this example, Tx beam is transmitted in the order of 2-4-3-1. Additionally, companies also consider that multiple MOs for one beam is used to do LP-WUS repetition in the last meeting’s discussion. From our view, LP-WUS repetition within one MO is sufficient and no specific need to define repetition among multiple MOs. Besides, similar as the UE monitoring behavior for PO/PEI-O, when the UE detects a LP-WUS within its LO, the UE is not required to monitor the subsequent MO(s) associated with the same LO. And one more thing needs to be pointed out is that since there are two types of LR i.e., OOK based LR and OFDM based LR, different types of LR needs to receive its corresponding sync reference signal i.e., LP-SS and SSB. And whether there is any mapping relation between LP-SS beams and SSB beams can be further studied, for example, it is up to the network implementation.

Figure 4  One example of LP-WUS with multiple MOs per duty cycle

Observation 2: LP-WUS repetition can be achieved within a LP-WUS MO not among multiple MOs.

Proposal 14: Support K i.e., the number of LP-WUS MOs for each beam can be larger than or equal to 1 to provide the flexibility for the network.

- when the UE detects a LP-WUS within its LO, the UE is not required to monitor the subsequent MO(s) associated with the same LO.

  - Duty-cycled or continuous LP-WUS monitoring

As mentioned before, LP-WUS is assumed to be configured relative to PO/PF. Hence, if the time offset between LP-WUS monitoring and PO is equal to the required wake up delay of UE, the smallest latency can be achieved already. In this sense, it seems no need to configure continuous LP-WUS monitoring for RRC idle/inactive mode. Besides, considering continuous LP-WUS monitoring will potentially result in the risk of high false alarm, duty-cycled LP-WUS monitoring will be more effective for RRC idle/inactive mode.

Proposal 15: Support duty-cycled LP-WUS monitoring for RRC idle/inactive mode.

Proposal 16: The periodicity of LP-WUS occasion (LO) for a UE is the same as its IDRX cycle.

  - MR and LR operation in same or different frequency carriers/bands

In the last meeting, there are some discussions on the carrier or band deployment of LP-WUS functionality. If LR and MR are always confined in the same carrier/band, it will limit the opportunity for LP-WUS operation, considering LR may only support a limited number of candidate carriers/bands to reduce complexity and power consumption.  In this case, for the carriers/bands supported by MR but not supported by LR, LP-WUS cannot be used. An alternative way is to ask UEs to camp on only the carriers/bands that supported by LR, however, this forces UEs which support LP-WUS operation to prioritize the carriers/bands supported by LR, which changes current camping priority rules and limits the camping priority from operators. Further, such camping priority will cause camping congestion on LR operated carriers/bands.

Therefore, LR and MR operating in different carriers/bands shall be supported. To support this, minor spec effort is observed, e.g., the existing NR mechanism has already supported different timing, SCS and duplex modes across different carriers/bands, which can be reused here.

Observation 3: For operating MR and LR in the same carrier/band by forcing UEs camped on only the carriers/bands supported by LR, it will limit the camping priority from operators and causes UE camping congestion.

Observation 4: Supporting LR and MR in different carriers/bands requires minor spec effort.

Proposal 17: Support LR and MR to work in the same or different Carriers/bands.

- Conclusion

In this contribution, the procedure of LP-WUS monitoring in RRC idle/inactive mode is discussion on the aspect of LP-WUS subgrouping, entry/exit condition, RRM measurement behavior during LP-WUS monitoring and LP-WUS monitoring configuration etc., with the following observations and proposals:

Observation 1: With more realistic assumptions e.g., around 20000 ~50000 UEs using LP-WUS per paging area, 8~16 subgroups per PO is sufficient to guarantee significant UE power saving gain e.g., 85% with the system overhead for LP-WUS transmissions to be less than 3%.

Observation 2: LP-WUS repetition can be achieved within a LP-WUS MO not among multiple MOs.

Observation 3: For operating MR and LR in the same carrier/band by forcing UEs camped on only the carriers/bands supported by LR, it will limit the camping priority from operators and causes UE camping congestion.

Observation 4: Supporting LR and MR in different carriers/bands requires minor spec effort.

Proposal 1: UE is not required to monitor a PO, if UE does not detect LP-WUS at all LP-WUS monitoring occasion(s) for the PO.

Proposal 2: Support the maximum number of subgroups to be 8~16.

Proposal 3:  Confirm the working assumption for the entry/exit conditions for LP-WUS monitoring agreed in RAN1 #116bis meeting.

Proposal 4: Support the following UE behavior on RRM measurement when UE starts LP-WUS monitoring,

- UE performs LR based serving cell measurement, and performs relaxed or does not perform MR based serving cell measurement;

- UE performs MR based relaxed neighbour cell measurement or UE stops neighbour cell measurement.

- FFS the conditions for different RRM measurement cases.

Proposal 5: Thresholds of for serving/neighbor cell measurement relaxation should be lower than or equal to the threshold of entry condition for LP-WUS monitoring.

Proposal 6: For LP-RSSI definition for LP-RSRQ, LP-RSSI is the liner average of total received power in the same number of ON and OFF OOK symbols.

Proposal 7: Define the frequency domain granularity of LP-RSRP and LP-RSSI for LP-RSRQ.

Proposal 8: The network configures the measurement time resource for LP-SSS-RSSI, which can be different from NR SMTC.

Proposal 9: Clarify that the same subgrouping information will be carried in the MOs of a LO.

Proposal 10: For the relationship between LO and PO, support Option 1 i.e., UEs monitoring the same PO monitor the same LO.

Proposal 11: Support to report UE capability on wake-up delay which is the time offset between receiving LP-WUS targeted to itself and being ready for PO reception.

Proposal 12: Support the network to configure one or multiple offsets between LO and PO.

Proposal 13: For multi-beam operation of LP-SS, each LP-SS MO is associated with each transmitted beam in turn.

Proposal 14: Support K i.e., the number of LP-WUS MOs for each beam can be larger than or equal to 1 to provide the flexibility for the network.

- when the UE detects a LP-WUS within its LO, the UE is not required to monitor the subsequent MO(s) associated with the same LO.

Proposal 15: Support duty-cycled LP-WUS monitoring for RRC idle/inactive mode.

Proposal 16: The periodicity of LP-WUS occasion (LO) for a UE is the same as its IDRX cycle.

Proposal 17: Support LR and MR to work in the same or different Carriers/bands.

# References

- 3GPP RAN1#117 meeting, Chairman notes.

- 3GPP TR 38.869 V2.0.0, Study on low-power wake up signal and receiver for NR, Release 18.

# Appendix A

The system resource overhead of LP-WUS is calculated with the following assumptions:

- The bandwidth of LP-WUS denoted as BWLP-WUS is 4.32MHz +2RB (30kHz) = 5.04MHz.

- The bandwidth of a serving cell denoted as BWCell is 20MHz.

- The number of PO in the system is NPO.

- The duration of LP-WUS represented by DLP-WUS:

  - Case 1: codepoint with encoded bits, using Manchester coding rate= 1/2 with 2 transmissions and 0 bits CRC, M=2.

    - Note: Though only 3 or 4 bits is required for 8 or 16 subgroups, minimum number of bits is 8 (additional 5 or 4 bits is reserved) to guarantee the total FAR from noise is no larger than 1%. Assuming no threshold is set for FAR, considering it would be difficult to adjust threshold according to SNR in practical deployment, per bit FAR from noise is 50%. Then, FAR is derived by 1-(1-50%)L, where L is the minimum number of bits.

|  | L=3 | L=4 | L=6 | L=8 | L=10 | L=12 |
| --- | --- | --- | --- | --- | --- | --- |
| FARO | 0.125 | 0.063 | 0.016 | 0.004 | 9.8e-04 | 2e-04 |

    - Note: the duration of LP-WUS is derived under the assumption that at a lower SNR, i.e., SNR=0 dB

  - Case 2: codepoint with OOK sequences: using Walsh sequence with sequence length = 8 or 16 for 8 or 16 subgroups, Manchester coding rate =1/2 and single transmission, M=2. So, the time resource of a LP-WUS is 8 symbols or 16 symbols.

  - Case 3: bitmap, using Manchester coding rate= 1/2 with 2 transmissions, 8 bits CRC, M=2. So, the time resource of a LP-WUS with 8-bit bitmap is 32 symbols, 16-bit bitmap is 48 symbols.

- For RRC idle/inactive mode, the number of beams for LP-WUS transmission i.e., Nbeam is assumed to be 8, which means the LP-WUS is transmitted repeatedly 8 times.

- The number of transmitted LP-WUS per PO is calculated as:

  - 1) Nsubgroup*Psubgroup for codepoint subgrouping method or

  - 2) P(subgroup>=1) for bitmap subgrouping method.

- Note1: Psubgroup is the probability of per subgroup paging rate e.g., 1%. P (subgroup>=1) is the probability of >=1 subgroup is paged, it can be derived by 1-(1-Psubgroup )^Nsubgroup.

- Note2: Nsubgroup is the number of subgroups per PO.

- Note 3: One more thing needs to emphasize is that a LP-WUS will be immediately transmitted once there is the need. That is to say, for codepoint subgrouping method, for a PO, the number of transmitted LP-WUS is equal to the number of paged subgroups which can be reflected by the formula 1).

The resource overhead of LP-WUS to the overall system resource can be calculated as formula below

RLP_WUS=BWLPWUS*DLPWUS*NPO* NLPWUS*Nbeam BWCell*TIDRXperiodicity

The target effective paging rate is 3%. And L represents the ISD which is the distance in red in the following figure. Assuming a hexagon shape of the coverage of a site, the area of a cell is 36L2.

M represents the number of sites per tracking area, and ρ represents the density of UEs, then the number of UEs in a tracking area is 336L2Mρ. And then the paging range is X cells, the number of UEs in X cells is 36L2*X*ρ.

According to the TR [2], when we assume FAR to be 0, the UE effective paging rate is equal to per group paging rate i.e., RG=1-(1-RE)Nand a per UE paging rate RE, where N is the number of UEs in a subgroup. Thus, to achieve a target effective paging rate (i.e. the paging rate for a group/subgroup), N=log1-RE(1-RG).

Then, the required number of subgroups to page UE in different paging range is 3L2Mρ2log1-RE(1-RG) or 3L2*X*ρ6log1-RE(1-RG).