3GPP TSG RAN WG1 Meeting #118	R1-2406312

Maastricht, NL, August 19th – 23rd, 2024

Source:	Fujitsu

Title:	Discussion on uplink enhancement for UE with 3Tx

Agenda item:	9.2.3

Document for:	Discussion and Decision

- Introduction

In RAN1 #117 meeting, the following agreements were achieved on 3Tx uplink transmission [1].

Agreement Update the agreement made in RAN1 #116bis as the following. Agreement For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, support the following for muting one of the ports of the configured 4-port SRS resource, Option 3: Always a same port is muted, e.g., i.e., the 4th port. Agreement For codebook-based M-TRP PUSCH repetition by a 3TX UE, scheduled by DCI format 0_1/0_2, - Reuse Rel-17 M-TRP PUSCH repetition design, where the second precoding information field only indicates TPMI index, and applies same rank as indicated by the first precoding information field. Agreement For codebook-based M-TRP PUSCH repetition by a 3TX UE, scheduled by DCI format 0_1/0_2, - Introduce new tables as Table I, II, III for the second precoding information field, for maxRank=1 or 2 or 3, respectively. - Table I: Second precoding information for 3 antenna ports if maxRank=1 Bit field; codebookSubset=NonCoherent. 0; 1 layer: TPMI=0. 1; 1 layer: TPMI=1. 2; 1 layer: TPMI=2. 3; Reserved. - Table II: Second precoding information for 3 antenna ports if maxRank=2 Bit field; codebookSubset=NonCoherent. 0; 1 layer: TPMI=0. 1; 1 layer: TPMI=1. 2; 1 layer: TPMI=2. 3; 1 layer: Reserved. 0; 2 layer: TPMI=0. 1; 2 layer: TPMI=1. 2; 2 layer: TPMI=2. 3; 2 layer: Reserved. - Table III: Second precoding information for 3 antenna ports if maxRank=3 Bit field; codebookSubset=NonCoherent. 0; 1 layer: TPMI=0. 1; 1 layer: TPMI=1. 2; 1 layer: TPMI=2. 3; 1 layer: Reserved. 0; 2 layer: TPMI=0. 1; 2 layer: TPMI=1. 2; 2 layer: TPMI=2. 3; 2 layer: Reserved. 0; 3 layer: TPMI=0. 1~3; 3 layer: reserved. Agreement For codebook-based M-TRP PUSCH repetition by a 3TX UE, for indication of PTRS-DMRS association, - When 1 PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig, reuse Rel-17 multi-TRP TDM repetition, - When 2 PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, and maxRank = 2 or 3, - A second PTRS-DMRS association field (1 bit) is used to indicate the association between PTRS ports and DMRS ports for the 2nd SRS resource set (i.e.,2nd TRP). Agreement For codebook-based UL transmission by a 3TX UE, subject to its capability, - A 3TX UE may report a maximum number of 3 layers - A 3TX UE may report a maximum number of SRS ports of up to 3 Note: SRS resource definition is not changed nor the number of SRS ports in the SRS resource.

The main issues to support 3Tx UE have been resolved. In this contribution, we provide discussions on some remaining details of the uplink enhancement for 3Tx UE in Rel-19.

- Discussion on uplink enhancement for UE with 3Tx

2.1 Downgrade operation from 4Tx UE to 3Tx UE

In RAN1 #116bis meeting, it was agreed that for 3Tx UE, the power scaling factor is the ratio of the number of antenna ports with non-zero PUSCH power to 3, as shown below.

Agreement For 3-port codebook-based PUSCH transmission for a 3TX UE, scale factor s should be the ratio of the number of antenna ports with a non-zero PUSCH transmission power to 3 (except for full-power Mode 0). - FFS: Whether specification needs to be updated to reflect the above

The UE downgrade operation is always supported in NR, for example, the UE with 4Tx could be configured with uplink operation of 3 ports.

In this case, how to determine the power scaling factor should be clarified, since the UE with 4Tx will report capability on maximum number of SRS ports of 4. There could be two options:

- Option 1: The power scaling factor is the ratio of the number of antenna ports with non-zero PUSCH power to 3, i.e., same as 3Tx UE.

- Option 2: The power scaling factor is the ratio of the number of antenna ports with non-zero PUSCH power to the maximum number of SRS ports supported by the UE, i.e., 4.

Whether Option 1 or Option 2 should be applied when the 4Tx UE is configured with 3-port transmission should be clarified to avoid confusion.

Proposal 1:

- RAN1 to discuss and clarify the power scaling factor setting when 4Tx UE is downgraded to 3-port transmission.

- Conclusion

In conclusion, we have the following proposals on supporting PUSCH transmission with 3Tx in Rel-19.

Proposal 1:

- RAN1 to discuss and clarify the power scaling factor setting when 4Tx UE is downgraded to 3-port transmission.

References

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #117, May 20th – 24th, 2024.