3GPP TSG RAN WG1 #118                           	                                                      R1-2406350

Maastricht, NL, August 19th – 23rd, 2024

Source:	CATT

Title:	Discussion on channel measurements derivation for computing CSI

Agenda Item:	8.1

Document for:	Discussion and Decision

- Introduction

In this contribution, we provide our views on channel measurement and interference measurement derivation to compute CSI for type II Doppler codebook.

- Discussion on channel measurements derivation for CSI computation

The RRC parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig can be configured to enable time domain restriction for channel measurements. UE behaviors on channel measurement derivation to compute CSI value reported in the UL slot n for CSI report under different timeRestrictionForChannelMeasurements configurations are described in TS 38.214 [1]  5.2.2.1.

The derivation of CSI value for all CSI reporting except Type II Doppler is crystal clear. If timeRestrictionForChannelMeasurements is set to "Configured ", channel measurement on the most recent  occasion of NZP CSI-RS which is no later than the CSI reference resource is used for CSI computing.

However, when Type II Doppler based CSI prediction is introduced, UE behavior on channel measurement derivation to compute CSI for CSI reporting may be different. For example, multiple NZP CSI-RS occasions in the measurement window can be used to derive N4 predicted CSI, as is illustrated in Figure 1 below.

Figure 1 The measurement, CSI derivation and reporting for Type II Doppler

In order to make the specification clear, the following options can be considered. One option is clarifying multiple occasions of CSI-RS are used by Type II Doppler based CSI prediction for time restricted channel measurement. Another option is providing the detailed number of occasions. The number of occasions can be defined as Kp, which refers to the definition of CPU occupation and active resource counting for Type II Doppler based CSI prediction. If the number of occasions depends on UE implementation and it cannot be restricted by the specification, a clarification on the applicability of the time restricted channel measurement for Type II Doppler based CSI prediction is also needed. For example, timeRestrictionForChannelMeasurements in CSI-ReportConfig cannot be set to "Configured" for Type II Doppler based CSI prediction. No matter which modification method is adopted, separate description for UE behavior on channel measurement derivation should be adopted for Type II Doppler based CSI reporting and all other CSI reportings.

Observation 1: UE behaviors on channel measurement derivation to compute CSI for CSI reporting have been specified. However, Type II Doppler based CSI reporting has distinct behavior compared to other CSI reporting and cannot follow current description in the specification.

Proposal 1:  Support to specify separate UE channel measurement derivation behavior for CSI computation of Type II Doppler based CSI reporting and all other CSI reportings.

Therefore, we propose the following modification in TS 38.214 [1] 5.2.2.1.

5.2.2.1 Channel quality indicator (CQI) <Unrelated part are omitted> If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "Configured", except for codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18', the UE shall derive the channel measurements for computing CSI reported in uplink slot n based on only the most recent, no later than the CSI reference resource, in cell DTX active period of a serving cell if cell DTX is activated, occasion of NZP CSI-RS (defined in [4, TS 38.211]) associated with the CSI resource setting on the serving cell. If the codebookType is set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18', the UE shall derive the channel measurements for computing CSI reported in uplink slot n based on KP most recent, no later than the CSI reference resource, in cell DTX active period of a serving cell if cell DTX is activated, occasions of NZP CSI-RS associated with the CSI resource setting on the serving cell. <Unrelated part are omitted> If the higher layer parameter timeRestrictionForInterferenceMeasurements in CSI-ReportConfig is set to "Configured", except for codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18', the UE shall derive the interference measurements for computing the CSI value reported in uplink slot n based on the most recent, no later than the CSI reference resource, in cell DTX active period of a serving cell if cell DTX is activated, occasion of CSI-IM and/or NZP CSI-RS for interference measurement (defined in [4, TS 38.211]) associated with the CSI resource setting on the serving cell. If the codebookType is set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18', the UE shall derive the interference measurements for computing the CSI value reported in uplink slot n based on KP most recent, no later than the CSI reference resource, in cell DTX active period of a serving cell if cell DTX is activated, occasions of CSI-IM and/or NZP CSI-RS for interference measurement associated with the CSI resource setting on the serving cell. <Unrelated part are omitted>

We provide a draft CR in our company contribution [2] to capture the Proposal 1.

- Conclusions

In this contribution, we provide our views on channel measurement derivation to compute CSI for type II Doppler codebook. The following observation and proposal are made:

Observation 1: UE behaviors on channel measurement derivation to compute CSI for CSI reporting have been specified. However, Type II Doppler based CSI reporting has distinct behavior compared to other CSI reporting and cannot follow current description in the specification.

Proposal 1:  Support to specify separate UE channel measurement derivation behavior for CSI computation of Type II Doppler based CSI reporting and all other CSI reportings.

- References

- TS 38.214, Physical layer procedures for data (Release 18), v18.3.0 (2024-06).

- R1-2406347, Correction on channel measurements derivation for computing CSI, CATT, 3GPP TSG-RAN WG1 Meeting #116, Maastricht, Netherlands, August 19th – 23th, 2024.