3GPP TSG RAN WG1 #118	        	R1-2406486

Maastricht, NL, August 19th – 23rd, 2024

Agenda Item:					    9.9.1

Source:	Sony

Title:	Measurements related enhancements for LTM

Document for:	Discussion

# Introduction

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1]

  - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM

  - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1]

    - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work

  - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

According to the WID for Rel-19 mobility enhancement [1], RAN1 will work measurement related enhancements for LTM.

In this contribution, Sony’s views of enhancement to support event triggered L1 measurement reporting are provided.

# Discussion

In Release 19 MIMO, UE initiated/event-driven beam management (UEIBM) has been discussed since RAN1#116 meeting, which is similar mechanism to event triggered L1 measurement reporting for LTM in Rel-19 mobility. On the other hand, as the time units for Rel-19 mobility enhancement is quite limited and Rel-19 mobility WID [1] implies that duplicated works between MIMO and Mobility enhancement should be avoided, specification works for Rel-19 mobility should be minimized. Therefore, in our view, if agreements/conclusions made in UEIBM of MIMO would also be applicable to discussions in event-triggered L1 measurement reporting for LTM, they should be reused as much as possible and RAN1 in mobility enhancement should focus on discussion about difference from UE-initiated/event-driven beam management.

Observation: Agreements/conclusions made in UE-initiated/event-driven beam management of MIMO could be reused to discussion in event-triggered L1 measurement reporting for LTM.

Proposal 1: To minimize specification works in Rel-19, RAN1 in mobility enhancement should focus on discussion for difference between event-triggered L1 measurement reporting for LTM and UE-initiated/event-driven beam management.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, following modes are supported:

- Mode A (dynamically scheduling UCI by gNB):

  - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report

    - FFS: Request format, e.g., SR or a new UCI type.

  - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report.

  - Step 3: Beam report is transmitted in second UL channel.

    - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both

  - This mode is basic UE capability (i.e. all UE supporting UE-initiated/event-driven beam reporting should support this feature).

  - No new DCI format is introduced.

- Mode B (UCI in pre-configured resource(s) for second UL channel):

  - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report

    - FFS: Notification format, e.g., SR or a new UCI type.

  - Step 2: UE transmits the beam report in the second UL channel.

    - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both

  - The notification in Step1 is in a separate reporting instance from the beam report in Step 2.

FFS: Whether UE receives acknowledge information with response to each step for all modes

For above procedures, cross-CC beam reporting is supported for both modes.

- FFS: Details.

In RAN1#116bis [2], the following beam reporting transmission procedure for UE-initiated beam management has been agreed.

In reporting procedure for UEIBM, although two modes (mode A and mode B) have been defined, both modes contain Step 1 that UE transmits a first PUCCH. Without step 1, a lot of reserved UL resources for second UL channel containing beam report would be necessary, which leads to waste of radio resources. In our view, similar reporting procedure could be applied to event-triggered L1 measurement reporting for LTM since a lot of information bits would also be required to transmit L1 measurement results (e.g. L1-RSRP) for LTM.

On the other hands, details of the procedure could be optimized, taking into account the purpose of LTM. For example, although both mode A and mode B have been supported for UEIBM, it would be further studied whether either mode A or mode B, or both should be supported for event triggered L1 measurement report for LTM.

Proposal 2: Procedures for event triggered L1 measurement reporting for LTM should be studied. Mode A and Mode B defined in UE initiated beam management should be a baseline for discussion.

# Conclusions

The following observations and proposals were made in this contribution.

Observation: Agreements/conclusions made in UE-initiated/event-driven beam management of MIMO could be reused to discussion in event-triggered L1 measurement reporting for LTM.

Proposal 1: To minimize specification works in Rel-19, RAN1 in mobility enhancement should focus on discussion for difference between event-triggered L1 measurement reporting for LTM and UE-initiated/event-driven beam management.

Proposal 2: Procedures for event triggered L1 measurement reporting for LTM should be studied. Mode A and Mode B defined in UE initiated beam management should be a baseline for discussion.

# References

RP-241515	, “Revised Work Item: NR mobility enhancements Phase 4,” 3GPP TSG RAN Meeting #104, June 2024

RAN1#116bis chairman’s note