3GPP TSG RAN WG1 #118		                                       R1-2406507 
Maastricht, NL, Aug 19th – 23rd, 2024

Agenda Item:	9.5.1

Source:	InterDigital Inc.

Title:	Discussion on on-demand SSB SCell operation

Document for:	Discussion

# Introduction

The Rel-19 New WID on Network Energy Savings for NR has the following objective on on-demand SSB SCell [1]:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

The agreements made in the RAN1#117 [2] meeting are provided in the appendix. In this contribution, we discuss the enhancements for enabling on-demand SSB SCell under the scope of RAN1.

# Discussion

For CA deployments, intra-band CA with SSB-less SCell has been supported since Rel-15 for both FR1 and FR2. In Rel-18 the support for SSB-less SCell is extended for collocated cells in FR1 that are in inter-band CA (e.g. within max receive time, frequency and power difference). In the inter-band CA case, the NW signals to the UE which cell is the reference cell (e.g. a PCell or another SCell), where the reference cell acts as the timing reference and AGC source of the SSB-less Scell.

Supporting inter-band CA for FR2 comes with challenges, e.g. achieving synchronization, L1/L3 measurements and beam management, especially when the cells are non-collocated and only the SCells operate in FR2 while other cells in FR1. For energy saving purposes in both intra- and inter-band CA, it is important for the SCells to operate in NES mode as long as possible (e.g. before and after SCell activation) with limited SSB transmission.

## Scenarios where OD-SSB is applicable

In the RAN1#116bis meeting [3], the scenarios where OD-SSB can be transmitted was discussed, considering Case #1 (no always-on SSB on the cell) and Case #2 (always-on SSB is periodically transmitted on the cell). The scenarios/cases were discussed during RAN1#117 but no related agreements were made. The following agreement was made in RAN1#116bis:

Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.

When SCell is configured to operate as an NES cell, both Case #1 and Case #2 are possible in Scenario #3A, i.e. no SSBs may be transmitted or SSBs may be transmitted with long periodicity. In both cases, OD-SSB transmission is beneficial to expedite the SCell activation procedure. For example, any issues related to T/F sync and beam selection can be avoided by transmitting OD-SSB after SCell activation command is sent so that sufficient measurements can be made by the UE before SCell activation is completed.

Observation 1: Any issues (e.g. T/F sync) during the SCell activation procedure can be avoided by transmitting OD-SSB after sending SCell activation command to ensure sufficient measurements are made by UE before SCell activation is completed

Proposal 1: Support on-demand SSB transmission in Scenario #3A for both Case #1 and Case #2

In Scenraio #3B after SCell activation is completed, it is possible that the SCell can be transitioned to an NES mode and operate completely SSB-less (Case #1) or SSBs may be transmitted sparsely or with longer periodicity (Case #2). This scenario is reasonable as it provides more opportunities for the SCell for energy savings, especially during load conditions at the SCell. In Scenario #3B, triggering OD-SSB trasmisison after SCell activation is completed can be beneficial to improve sycnchronization, timing reference and AGC at the UE, especially when the last transmission of SSBs may be outdated.

Observation 2: Since the SCell can be transitioned to NES mode (e.g. SSB-less or SSBs are transmitted with long periodicity) after SCell activation is completed, triggering OD-SSB transmission can be beneficial to improve synchronization, timing reference and AGC at the UE

In addition, such benefits for using OD-SSBs can also apply in the cases outside of the SCell activation procedure where the UE can be triggered to perform RRM measurements, CSI measurements/reporting and PDCCH decoding. The spec impacts for supporting OD-SSB (e.g. in terms of signaling for indicating OD-SSB) are similar regardless of its applicability for Scenarios 2/2A and Scenarios 3A/3B as well as for Case#1 and Case#2. As such, we see no reason to restrict OD-SSB transmission to only certain scenarios and cases. Given the overall benefits, we propose the following:

Proposal 2: Support on-demand SSB transmission in Scenario #3B for both Case #1 and Case #2

## Assumption on cell supporting OD-SSB transmission

Another issue that was discussed during RAN1#117 without reaching an agreement/conclusion was on whether OD-SSB is a cell-defining SSB or not. The agreement made in RAN1#116bis to downselect between different alternatives is:

Agreement - For a cell supporting on-demand SSB SCell operation, - Note: It is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not. - For on-demand SSB on the cell, downselect between the following alternatives - Alt-1: It is up to gNB implementation whether on-demand SSB is cell-defining SSB or not. - Alt-2: On-demand SSB is limited to non-cell-defining SSB. FFS: Further limitations to on-demand SSB

Non-cell defining SSB was introduced in Rel-17 for supporting RedCap UEs, where the SSBs can be transmitted without RMSI (e.g. without associated SIB1). During RAN#116bis, there was a discussion on whether OD-SSBs can be cell-defining or non-cell defining SSBs, based on which two alternatives were proposed.

In Alt-1, it is left to gNB implementation to decide on the SSB type applied for OD-SSB. Since this approach is similar to legacy for always-on SSBs, the spec impact in terms of UE behavior is expected to be similar when receiving OD-SSBs. In addition, it provides flexibility to NW to decide on what/when such OD-SSBs are transmitted. Also, gNB can decide to transmit NCD-SSBs as OD-SSBs if it decides to prevent any idle mode UEs from triggering initial access to the SCell.

In Alt-2, OD-SSBs are limited to only non-cell defining SSBs. In our view, having such limitation for the OD-SSB is not necessary since any NES gains by not transmitting RMSI are marginal anyways, and this also introduces unnecessary impacts in UE behavior with no clear benefits. As such, our preference is Alt-1 for OD-SSB.

Proposal 3: For OD-SSB on the cell, it is up to gNB implementation whether on-demand SSB is cell-defining SSB or not

## Options for OD-SSB transmission

In RAN1#116bis, the following updated agreement was made to study the options for OD-SSB:

Agreement The following agreement from RAN1#116 is modified (in red) - For SSB burst(s) triggeredindicated by on-demand SSB SCell operation, study at least the following options. - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A. - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B. - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted. - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B. - FFS: The combination of above options - FFS: How to define time instance A/B and the value of N per option - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement)

The different options above can be broadly categorized as follows:

- Category 1: Periodic OD-SSB transmission with single periodicity

- Category 2: Periodic OD-SSB transmission with periodicity switching

- Category 3: Aperiodic/semi-persistent OD-SSB transmission

In Category 1 (corresponding to Option 1 in the agreement), the SCell is assumed to transmit OD-SSBs regularly with certain periodicity. This option makes sense, as once the SCell is activated, there will be a need for recurring SSBs, as the UE is in connected mode and will periodically measure and report the channel measurements. The applicability of this category is thus suitable at least for both Scenario #3A and Scenario #3B.

Category 2 (correspoding to Option 4) requires configuring parameters for time instances (point A and B) and periodicity values where the SSB transmission assumptions will change. However, to make the best use of the OD-SSBs, different parameter values should be configured for different scenarios, hence incurring high spec effort.

Category 3 (corresponding to Options 1A, 2 and 3) is suitable when the SSB periodicity is already too long, where transmission of at least one occasion of OD-SSB is useful. Also, the behavior associated with Option 2 can be realized by configuring proper value for N in Option 3, while avoiding any need for keeping track of time instance B. In Category 3, the network has more flexibility to transmit OD-SSB in a limited time period, only when there is a need (e.g. for measurements at UE), hence allowing higher NES gains. The aperiodic/semi-persistent OD-SSB transmisison can be useful for not only in all scenarios (e.g. #2, #2A, #3A and #3B), but also in the case outside of the SCell activation procedure such as in SSB-less SCell operation or prolonged SSB periods.

Proposal 4: Support one of the following options:

- Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A.

- Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB.

- Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted. FFS: A, N values.

## Timeline for OD-SSB transmission

In RAN1#117, the following agreement was made on the timing for when UE can expect the OD-SSB transmisison:

Agreement For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - FFS: Whether the value of T is predefined or indicated/configured by gNB - FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB” Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

The description on existing timing for SCell activation/deactivation in 38.213 [5] is as follows:

4.3 Timing for secondary cell activation / deactivation With reference to slots for PUCCH transmissions, when a UE receives in a PDSCH an activation command [11, TS 38.321] for a secondary cell ending in slot n, the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133] and no earlier than slot n+k, except for the following: - the actions related to CSI reporting on a serving cell that is active in slot n +k - the actions related to the sCellDeactivationTimer associated with the secondary cell [11, TS 38.321] that the UE applies in slot n +k - the actions related to CSI reporting on a serving cell which is not active in slot n +k that the UE applies in the earliest slot after n +k in which the serving cell is active. The value of k is m+3 Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-ACK information for the PDSCH reception as described in clause 9.2.3 and Nslotsubframe,μ is a number of slots per subframe for the SCS configuration μ of the PUCCH transmission as defined in [4, TS 38.211].

As per the legacy spec, upon receiving the SCell activation indication in slot n there can be up to k slots (i.e. n+k’th slot) before the SCell activation procedure is complete (i.e. UE transmits CSI report). For OD-SSB transmission, the corresponding indication can be received by UE in a slot either prior to receiving the SCell activation indication (for Scenario #2) or after receiving the activation indication (for Scenario #3A). That is, for Scenario #2 when receiving the indication on OD-SSB in slot m, the UE can expect the first SSB in slot m+T (where m+T is before slot n). For Scenario 3#A, when receiving the indication on OD-SSB in slot n+q (where q < k), the UE can expect the first SSB in slot n+q+T.

Since the indication for OD-SSB is received via a MAC CE, and considering the processing time at the UE for the MAC CE (as per the Note in the agreement above), it is reasonable to assume slot-level timeline for defining the time instance A. Upon receiving the MAC CE in PDSCH, the UE can transmit HARQ-ACK per legacy timeline. The UE need not wait for the OD-SSB until after HARQ-ACK transmission and can expect the first SSB of OD-SSB in T slots upon receiving the indication on OD-SSB. As such, we propose to confirm the following from the previous agreement for time instance A:

Proposal 5: Confirm the following: For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows.

- Time instance A is [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission

One of the FFS from the previous agreement is on whether the value of T is predefined or indicated/configured by gNB. Since OD-SSB can be triggered dynamically by gNB (e.g. before/during SCell activation), it is not necessary to predefine the value T. On the other hand, if the value T is configured (in RRC signaling), the UE can expect reception of the first SSB within T slots after receiving the indication on OD-SSB. However, if the gNB has triggered OD-SSB transmission earlier (e.g. for another UE), it is possible that by the slot when UE expects SSB according to the configured T value, the OD-SSB is no longer transmitted on the SCell. To address this issue and to provide more flexibility to the gNB to decide on whether/when to transmit OD-SSB, it is is beneficial to signal the value T in the indication on OD-SSB. For example, providing the value T (e.g. in MAC CE) allows the gNB to use the same OD-SSB transmission to multiple UEs by indicating different T values to different UEs. The details on how the value T can be indicated (e.g. in MAC CE) with minimal overhead can be left to RAN2 discussion.

Observation 3: Signaling the timing for the reception of the first SSB of OD-SSB in the OD-SSB indication (e.g. MAC CE) provides more flexibility to gNB to decide on OD-SSB transmission and reuse the same OD-SSB for multiple UEs

Proposal 6: The timing for the reception of the first SSB of OD-SSB burst (i.e. value of T) is signalled in the indication on OD-SSB

## Configuration for OD-SSB

In RAN1#117, the following agreement was made regarding the configuration parameters on OD-SSB that are provided via RRC signaling:

Agreement - For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported. - Frequency of the on-demand SSB - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst - Periodicity of the on-demand SSB - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE - FFS: Whether the RRC is newly introduced or existing RRC is reused

Regarding the FFS, supporting multiple OD-SSB configurations where each configuration may correspond to different transmission pattern, periodicity, and start timing increases the spec impact unnecessarily. Given that OD-SSB applies to only SCell, it does not appear there is much benefit to support multiple types of OD-SSB configurations. In the interest of keeping the spec effort manageable, the propose the following:

Proposal 7: Configuration of more than one OD-SSB configurations is not supported

## Signaling for indicating OD-SSB

In RAN1#117, the following agreement was made on the signaling to indicate OD-SSB transmission:

Agreement - For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable

On the FFS in the agreement, supporting DCI provides another means to indicate the OD-SSB transmission with relatively low latency. On the other hand, in view of the agreement to support both RRC and MAC CE based signaling, there is no further motivation to introduce DCI based signaling to indicate the same info on OD-SSB. The RRC and MAC CE based signaling also provides higher reliability and are similar to the legacy signalling used for SCell activation/deactivation, hence the corresponding UE behavior when receiving the indication on OD-SSB in such signaling is reuseable. Given the reasoning, we propose the following:

Proposal 8: DCI based signaling to indicate OD-SSB transmission is not introduced in Rel-19

Another issue that was discussed during the last RAN1#117 was related to the MAC CE indicating OD-SSB transmission. The options considered for the MAC CE was whether separate signaling is used for providing OD-SSB indication and SCell (de)activation or a single signaling is used for both. In Scenario #2 using separate signaling is the only feasible way for the gNB to indicate the start of OD-SSB transmission before sending the SCell activation command. Separate signaling is also preferable as it is useful in Scenarios #3A, #3B and any scenarios outside of the SCell activation procedure as it provides full flexibility to the NW to trigger OD-SSB whenever deemed necessary. Further discussions on whether a single or separate signaling is used for the MAC CE can be left to RAN2.

Observation 4: Discussions on whether a single or separate signaling is used for the MAC CE for indicating OD-SSB transmission and SCell (de)activation is left to RAN2

## UL WUS for requesting on-demand SSB

In addition to the ongoing discussions on gNB initiated OD-SSB, it also important and practical to discuss transmitting OD-SSB only when there is a need from the UE (e.g. based on measured channel conditions when the UE moves in or out of coverage, when cells are not collocated). Also in SCell activation procedure, in contrast to the gNB-initiated case, transmitting OD-SSB in the time occasions expected by the UE (e.g. for making timely measurements and reporting) can reduce the SCell activation delay.

Observation 5: Transmitting OD-SSB in the time occasions expected by the UE (e.g. for making timely measurements and reporting) can reduce the SCell activation delay

The need or demand for OD-SSB can thus be effectively conveyed by the UE, as likely the conditions for requesting such demand is known at the UE. Such conditions can include, for example, lack of sufficient SSB beams for sync and L1/L3 measurements. In such conditions, the UE can send the UL WUS indication to the gNB to request for OD-SSB tranmisison.

Using UL WUS for requesting for OD-SSB transmission also results in considerable NES gains. The Rel-18 TR [3] mentions that when only sparse SSBs with periodicity of 160ms is transmitted before the reception of UL WUS at gNB, 5 sources show UE WUS triggering for SSB/SIB/RACH could achieve BS energy savings by 6.2% – 80.7% in range with UPT loss by 0%-24.2%, where the gain increases as a function of the WUS period. Given the benefits in terms of low delay, energy savings and low UPT losses, UL WUS can be supported for requesting OD-SSB transmission.

Proposal 9: Support UL WUS for requesting on-demand SSB transmission at SCell

## Signal/channel to use for UL WUS

Regarding the type of channel/signal to use for UL WUS, the following criteria can be considered:

- Allows flexible provisioning of resource configuration,

- Provides timely indication to the gNB on the need for OD-SSB,

- Provides reasonable coverage and reliability for detection at NW,

- Reuses existing synchronization and power control procedure at UE

During Rel-18 SI, the options considered by companies for UL WUS include using PRACH resource and PUCCH resource (e.g. SR). Using a PUCCH resource for triggering on demand SSBs can be useful for UEs in connected mode, but this may require introducing a new procedure for triggering the transmission and ensuring suffient reliability/coverage when receiving at SCell and hence, less preferable for UL WUS. Using PRACH is thus simpler, given the gNB monitors PRACH on the PCell regardless of NES mode.

Reusing PRACH resources for UL WUS allows meeting the above criteria and provides the benefit of simplyfing any PHY layer design effort while reusing many of legacy principles/procedures. For example, to allow more flexibility when accessing the resources, the contention-based PRACH resources for UL WUS can be configured to consist of a set of PRACH preambles and RACH occasions. Such resources for UL WUS can be configured in NES-capable UEs as an additional set which may not overlap with the PRACH resources provided to the legacy UEs. For ensuring proper detection of the UL WUS, the NW can configure the RACH occasions for UL WUS such that they are aligned with the monitoring/reception occasions at NW. Given there is no CBRA on the SCell, the UL WUS would have to be transmitted by the UE on the PCell.

The resources for UL WUS can be provided by the serving/reference cell (e.g. PCell) to the NES-capable UEs as a separate configuration or a sub-configuration of the RACH config. Such resource configuration for UL WUS can be provided in the same way as legacy via higher layer signaling (e.g. with RRCReconfiguration for UE in connected mode or via SIB1).

Proposal 10: Support using PRACH preamble on the PCell as UL WUS for requesting OD-SSB on the SCell

# Conclusion

In this contribution, the following observations are made:

Observation 1: Any issues (e.g. T/F sync) during the SCell activation procedure can be avoided by transmitting OD-SSB after sending SCell activation command to ensure sufficient measurements are made by UE before SCell activation is completed

Observation 2: Since the SCell can be transitioned to NES mode (e.g. SSB-less or SSBs are transmitted with long periodicity) after SCell activation is completed, triggering OD-SSB transmission can be beneficial to improve synchronization, timing reference and AGC at the UE

Observation 3: Signaling the timing for the reception of the first SSB of OD-SSB in the OD-SSB indication (e.g. MAC CE) provides more flexibility to gNB to decide on OD-SSB transmission and reuse the same OD-SSB for multiple UEs

Observation 4: Discussions on whether a single or separate signalling is used for the MAC CE for indicating OD-SSB transmission and SCell (de)activation is left to RAN2

Observation 5: Transmitting OD-SSB in the time occasions expected by the UE (e.g. for making timely measurements and reporting) can reduce the SCell activation delay

The following proposals are made:

Proposal 1: Support on-demand SSB transmission in Scenario #3A for both Case #1 and Case #2

Proposal 2: Support on-demand SSB transmission in Scenario #3B for both Case #1 and Case #2

Proposal 3: For OD-SSB on the cell, it is up to gNB implementation whether on-demand SSB is cell-defining SSB or not

Proposal 4: Support one of the following options:

- Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A.

- Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB.

- Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted. FFS: A, N values.

Proposal 5: Confirm the following: For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows.

- Time instance A is [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission

Proposal 6: The timing for the reception of the first SSB of OD-SSB burst (i.e. value of T) is signalled in the indication on OD-SSB

Proposal 7: Configuration of more than one OD-SSB configurations is not supported

Proposal 8: DCI based signaling to indicate OD-SSB transmission is not introduced in Rel-19

Proposal 9: Support UL WUS for requesting on-demand SSB transmission at SCell

Proposal 10: Support using PRACH preamble on the PCell as UL WUS for requesting OD-SSB on the SCell

# Appendix: Agreements on OD-SSB SCell operation

RAN1#117 Agreements

Agreement - For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable Agreement - For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported. - Frequency of the on-demand SSB - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst - Periodicity of the on-demand SSB - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE - FFS: Whether the RRC is newly introduced or existing RRC is reused Agreement - At least support L1 measurement based on on-demand SSB - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported. - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB Agreement For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - FFS: Whether the value of T is predefined or indicated/configured by gNB - FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB” Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology. Agreement - For a cell supporting on-demand SSB SCell operation, at least the followings for on-demand SSB are known to UE. - Sub-carrier spacing of the on-demand SSB - Physical Cell ID of the on-demand SSB - Location of on-demand SSB burst - Downlink transmit power of on-demand SSB - FFS: Other parameters - FFS: Whether each of above parameters is configured/indicated explicitly or not

RAN1#116bis Agreements

Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed. Agreement - For a cell supporting on-demand SSB SCell operation, - Note: It is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not. - For on-demand SSB on the cell, downselect between the following alternatives - Alt-1: It is up to gNB implementation whether on-demand SSB is cell-defining SSB or not. - Alt-2: On-demand SSB is limited to non-cell-defining SSB. - FFS: Further limitations to on-demand SSB Agreement - For a cell supporting on-demand SSB SCell operation, - L1 and/or L3 measurement based on on-demand SSB is supported for the cell. - FFS further details on L1 and/or L3 measurement Agreement The following agreement from RAN1#116 is modified (in red) - For SSB burst(s) triggeredindicated by on-demand SSB SCell operation, study at least the following options. - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A. - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B. - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted. - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B. - FFS: The combination of above options - FFS: How to define time instance A/B and the value of N per option - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement) Agreement For a cell supporting on-demand SSB SCell operation, further study the following options. - Option 1: Separate signaling between legacy/existing signaling (e.g., RRC, MAC CE) providing SCell activation/deactivation and signaling providing On-demand SSB transmission indication. - Option 2: A single signaling in which both SCell activation/deactivation and On-demand SSB transmission indication are provided. - FFS: Details of the signaling - Other options are not precluded. - FFS: Details on On-demand SSB transmission indication

RAN1#116 Agreements

Agreement Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study: - Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell - FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted. FFS: Which scenario the above applies for Agreement RAN1 to strive for a common design for on-demand SSB operation considering all applicable CA configurations. Agreement For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios. - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321) - Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321) - This does not preclude SCell for which activation is completed - FFS: The case where SCell activation is completed FFS: Application timing between NW triggering message and on demand SSB transmission Agreement Support on-demand SSB SCell operation triggered by gNB. - FFS Details of associated signaling/indication/configuration provided to UE Agreement - For SSB burst(s) triggered by on-demand SSB SCell operation, study at least the following options. - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A. - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B. - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted. - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B. - FFS: The combination of above options - FFS: How to define time instance A/B and the value of N per option - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement)

# References

RP-240170, Revised WID: Enhancements of network energy savings for NR, Mar 2023

Chair notes, RAN1#117, May 2024

Chair notes, RAN1#116bis, April 2024

3GPP TR 38.864, Study on network energy savings for NR (Release 18), V18.1.0, Mar 2023

3GPP TS 38.213, Physical layer procedures for control (Release 18), V18.1.0, Dec 2023