3GPP TSG RAN WG1 #118		                                                    R1-2406554

Maastricht, The Netherlands, 19th – 23rd August, 2024

Agenda Item:	9.11.1

Source:	NEC

Title:	NR-NTN downlink coverage enhancement

Document for:	Discussion

Introduction

In RAN#104, the WID: Non-Terrestrial Networks (NTN) for NR Phase 3 is updated [1]:

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

In RAN1#117, additional LLS and SLS results are observed [2]. The corresponding agreements are summarised in Appendix B.

In this contribution, we further discussed the downlink coverage enhancement for NR-NTN.

Discussion

For a phased array antenna, the beam size (beam width) is a function of the active transmit antenna element number. Fig. 1 illustrates the different beam widths under 16, 32, and 64 active transmit antenna elements.

Fig. 1 Phased array beam pattern with various active antenna elements

Observation 1: Adjusting the number of active transmit antennae could provide flexible physical beam patterns/size (i.e. wide or narrow) across the satellite footprint.

EIRP determines the link budget (CNR) and can be calculated by

EIRP dBW=PT dBW-LC dB+GT [dBi]

where PT is antenna transmit power, LC is cable loss, and GT is transmit antenna gain and can be derived by

GT dBi=GT,e dBi+10log10NT,a,  &array antenna10log10η∙π2∙D[m]2λ[m]2,  &parabolic antenna

where GT,e is transmit antenna element gain and NT,a is the number of transmit antenna elements [4].

For a satellite with a fixed payload total power level (e.g. LEO600 km Set 1-1, 1-2, or 1-3 FR1[2]), the satellite could reduce the simultaneously active beam numbers by enlarging the beam size (reducing the antenna gain) and resharing the DL power with a smaller number of active DL beams.

Proposal 1: Support using different beam sizes to provide DL service of different PHY channels to meet the link-level DL coverage requirements, and to improve the rate of (N2+N3)/(N1+N2+N3) by flexible power-sharing at the system level.

Fig. 2 illustrates the key points to determine the maximum serving time of one beam or one satellite under earth-fixed cell (from t0 to t1) and earth-moving cell (from t2 to t3), respectively. The determinations of t1-t0 and t3-t2 are based on the beam diameter and elevation angle assumptions in [3].

Fig. 2 Beam/cell serving time under earth-fixed cell and earth-moving cell circumstances

Under the earth-fixed cell circumstance, the E0 and E1 positions are constrained by both the elevation angle and the beam diameter to calculate the one-beam serving time; and the E0 and E1 positions are constrained by the far edge beam elevation angle to calculate the one satellite serving time.

Under the earth-moving cell circumstance, the E2 and E4 positions are constrained by both the elevation angle and the beam diameter to calculate the one-beam serving time; and the E2 and E4 positions are constrained by the edge beam elevation angle to calculate the one-satellite serving time.

Table. 1 Maximal serving time

| Satellite Type |  | Elevation Angle | Beam Diameter (km) | Earth Fixed |  | Earth-Moving |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Beam Serving Time (s) | Satellite Serving Time (s) | Beam Serving Time (s) | Satellite Serving Time (s) |
| Set-1 | LEO600 | 30 | 50 | 239.6 | 123.4 | 7.2 | 246.8 |
|  | LEO1200 | 30 | 90 | 466.4 | 240.6 | 14.7 | 481.1 |
| Set-2 | LEO600 | 30 | 90 | 233.8 | 123.4 | 13.0 | 246.8 |
|  | LEO1200 | 30 | 190 | 450.0 | 240.6 | 31.1 | 481.1 |

The calculation results are summarized in Table. 1. The minimum serving time for one beam is 7.2s, and for one satellite is 123.4s. One of the Rel-18 network energy-saving techniques is specified as cell DTX. The serving time of a satellite/beam is long enough to apply the cell DTX–like configurations.

Proposal 2: Support beam-level time domain configurations to flexible sharing of the DL transmission power.

Conclusion

From the discussion, we have the following observations and proposals.

Observation 1: Adjusting the number of active transmit antennae could provide flexible physical beam patterns/size (i.e. wide or narrow) across the satellite footprint.

Proposal 1: Support using different beam sizes to provide DL service of different PHY channels to meet the link-level DL coverage requirements, and to improve the rate of (N2+N3)/(N1+N2+N3) by flexible power-sharing at the system level.

Proposal 2: Support beam-level time domain configurations to flexible sharing of the DL transmission power.

- References

- RP-241667, “New WID: Non-Terrestrial Networks (NTN) for NR Phase 3”, Moderator, RAN1 Vice-Chair (Huawei)

- Chair’s Notes RAN1 #117

- TR 38.821, Solutions for NR to support non-terrestrial networks (NTN)

- Appendix A

In RAN1#116, the following agreements have been made [2].

Agreement

For DL coverage study, consider the following additional reference satellite parameters scenarios for LEO600km Set1 in FR1 (i.e., S-band), referred to as Set1-1 FR1, Set1-2 FR1 and Set1-3 FR1:

| LEO600km Set1-1 FR1 (i.e., S-band) |
| --- |
| Maximum Bandwidth per beam | 5 MHz |
| SCS | 15 kHz |
| Beam size(Note 1) | 50km |
| Satellite EIRP density /beam (dBW/MHz) | 34 |
| Payload Total DL power level (dBW) | 31.24 |
| Aggregated EIRP (Total) (dBW) | 61.24* |
| Satellite Tx max Gain | 30 dBi |
| Maximum EIRP per Satellite beam (dBW) | 41 |
| Total number of beam footprints*** | 1058 |
| Total number of simultaneously active beams ** | 106 |
| % simultaneously active beams** | 10.02 % |
| *Note: EIRP limit is 61.24 dBm for the reference configuration. **Assuming 100 % Resource Block utilization within the same beam at max power. Absolute number of simultaneously active beams is up to 212 (due to limitation of RF) *** For a constellation design at 600km with low elevation angle with 30° and selected (i.e Set 1 parameters) beam size Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies |  |

| LEO600km Set1-2 FR1 (i.e., S-band) |
| --- |
| Maximum Bandwidth per beam | 5 MHz |
| SCS | 15 kHz |
| Beam size (note 1) | 50km |
| Satellite EIRP density /beam (dBW/MHz) | 34 |
| Payload Total DL power level (dBW) | 23 |
| Aggregated EIRP (Total) (dBW) | 53* |
| Satellite Tx max Gain | 30 dBi |
| Maximum EIRP per Satellite beam (dBW) | 41 |
| Total number of beam footprints | 1058 |
| Total number of simultaneously active beams** | 16 |
| % simultaneously active beams** | 1.5 % |
| *Note: EIRP limit is 53 dBm for the reference configuration. **Absolute number of simultaneously active beams is up to 16 (due to limitation of RF) Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies |  |

| LEO600km Set 1-3 FR1 (i.e., S-band) |
| --- |
| Maximum Bandwidth per beam | 5 MHz |
| SCS | 15 kHz |
| Beam size (note 1) | 50km |
| Satellite EIRP density /beam (dBW/MHz) | 26 |
| Payload Total DL power level (dBW) | 23.24 |
| Aggregated EIRP (Total) (dBW) | 53.24* |
| Satellite Tx max Gain | 30 dBi |
| Maximum EIRP per Satellite beam (dBW) | 33 |
| Total number of beam footprints | 1058 |
| Total number of simultaneously active beams** | 106 |
| % simultaneously active beams** | 10.02 % |
| *Note: EIRP limit is 53.24 dBm for the reference configuration. **Absolute number of simultaneously active beams is up to 212 (due to limitation of RF) Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies |  |

Note: RAN1 will aim to identify necessary enhancements for these scenarios in the study phase. At the end of the study phase, RAN1 will further discuss whether the potential enhancements will be specified within Rel-19 framework.

Agreement

For DL coverage study at system level, consider the following additional reference satellite payload parameters for LEO600km in FR2 (i.e., Ka-band):

| LEO600km Set1-1 FR2 (i.e., Ka-band) |
| --- |
| Maximum Bandwidth per beam | 400 MHz |
| SCS | 120 kHz |
| Beam size | TBD in next meeting |
| Satellite EIRP density /beam (dBW/MHz) |  |
| Payload Total DL power level (dBW) |  |
| Aggregated EIRP (Total) (dBW) |  |
| Satellite Tx max Gain |  |
| EIRP per Satellite beam (dBW) |  |
| Total number of beam footprints | 800 (note 1) |
| Total number of simultaneously active beams | 12 |
| % simultaneously active beams | 1.5 % |
| Note 1: A typical deployment scenario in FR2 should consider 800 satellites beams per a single satellite coverage area with an absolute number of simultaneously active beams equal to 16 (due to limitation of RF) |  |

Agreement

Adopt the following phased array antenna parameters for LEO 600km in FR1:

| Satellite phased array antenna Characteristics | LEO-600 |
| --- | --- |
| Orbit | LEO-600km |
| Frequency range/band | FR1/S-Band |
| Antenna element pattern | Table7.3-1 in TR 38.901 |
| Horizontal/vertical 3 dB beam width of single element (degree) | [65] for H [65] for V |
| Antenna polarization | Circular (RHCP or LHCP) |
| Number of antenna elements | [400 elements (20 x 20)] |
| Equivalent satellite antenna aperture | 2m |
| Element maximum gain | 4 dBi |
| Antenna maximum gain | 30 dBi |
| Steering loss at 30° elevation angle | [4dB] |

Agreement

RAN1 to consider the following performance metrics for DL Coverage enhancement evaluation at system level:

At least:

- CDF of the received SINR

- The dwell time and revisit time interval for each beam illumination across the coverage

- Periodicity of common control channels (e.g. SSB, CORESET0/SIB1, SIB19) and corresponding coverage ratio

Other metrics may be reported such as

- CDF of the cell throughput

- CDF of user perceived throughput (UPT)

- CDF of Latency

- Ratio of mean served cell throughput and offered cell throughput, denoted by 𝜌 (refer to TR36.889)

For system level study based on analytical evaluation:

- N1 beam footprints are in state “off”

  - These beam footprints are not served by any signal (no satellite service in this area)

- N2 beam footprints are in state “common messages only”

  - These beam footprints do not have any active user traffic, and are served the necessary information for cell discovery and initial access.

  - Optionally, companies may consider user arrival (e.g. RACH access) in this type of cell, and should describe how this is taken into account in the analytical evaluation

- N3 beam footprints are in state “active traffic”

  - These beam footprints have X active (e.g. VoNR) users each.

  - These beam footprints are also served the necessary information for cell discovery and initial access

- N1 + N2 + N3 = “Total number of beam footprints “

- N1, N2, N3, X are to be reported by companies.

- Resource utilization obtained under the assumptions above is to be reported by companies.

- Other assumptions made in the evaluation are to be reported by companies, e.g. power sharing scheme, beam hopping scheme, etc.

Agreement

For NR NTN Rel-19 DL coverage evaluation, UE characteristics for handheld terminals in Table 6.1.1.1-3 in TR 38.821 can be reused, with the following:

- -5.5 dBi antenna gain is assumed

- at least 2Rx are considered at the UE

  - 4Rx can be optionally considered and reported

Note: Redcap device is not considered in the scope of DL coverage study

Agreement

The following traffic models are considered for system level evaluation of DL coverage:

- FTP3: as in Table 6.1.1.1-7 of TR 38.821: 0.5MB as packet size, 200ms as mean inter-arrival time

- FTP3 IM: 0.1MB as packet size, 2s as mean inter-arrival time

- VoIP can be considered in the evaluation.

It is up to company report which traffic model is used among the discussed traffic models in their evaluations.

- Other models may be used as well, and parameter (e.g. packet size and arrival rate) adjustment can be optionally considered and reported.

| Traffic type | FTP | IM | VoIP |
| --- | --- | --- | --- |
| Model | FTP model 3 | FTP model 3 | As defined in Rel-18 NTN CE. |
| Packet size | 0.5 Mbytes | 0.1 Mbytes |  |
| Mean inter-arrival time | 200 ms | 2 sec |  |

Agreement

For NR NTN Rel-19 DL coverage evaluation, Beam layout defined in Table 6.1.1.1-4 in TR 38.821 can be reused.

- Using other beam layouts is not precluded, and should be reported by companies

Agreement

For NR NTN Rel-19 DL coverage evaluation, a value of beam steering latency equal to 0 at least if phase array antenna is assumed.

Values different from 0 can be optionally reported

Agreement

DL coverage is evaluated at link level with the following considerations:

- NGSO at LEO-600 operating in FR1 is considered in priority

- Additional satellite payload parameters defined for system level evaluation are used

- FFS: Antenna gain reduction due to steering loss can be considered

Agreement

For the evaluation of NTN downlink coverage at link level, reuse the target data rate from Rel-18 NTN Coverage enhancements:

- For VoIP: AMR 4.75 kbps (TBS of 184 bits without CRC in physical layer) with 20 ms data arriving interval

- For data rate service: both 3 kbps and 1Mbps can be considered

  - Companies can also use the data rates corresponding to the traffic types used for system level evaluations

Agreement

For link-level study, downlink coverage performance in NR NTN is evaluated according to the following steps.

Step 1: CNR is calculated as defined in 6.1.3.1 of TR 38.821

Step 2: Required SNR of target service is evaluated by LLS

Step 3: The CNR and the required SNR are compared

Agreement

For link-level study, for NR NTN DL coverage enhancement, the following channels/signals can be considered for evaluations:

- PDSCH for VoIP

- PDSCH for low data rate service

- PDSCH Msg.2

- PDSCH Msg.4

- PDSCH carry SIB, e.g., SIB1, SIB 19

- PDSCH for paging

- PDCCH

- Broadcast PDCCH (e.g. PDCCH of Msg.2, paging)

- SSB

Note: RAN1 will aim to identify necessary link-level enhancements for these channels in the study phase. At the end of the study phase, RAN1 will further discuss whether the potential link-level enhancements will be specified within Rel-19 framework.

Agreement

For DL coverage performance evaluation, the following are assumed for all channels/signals

- Channel model/Delay spread:

  - Channel model as in Table 6.1.2-4 of TR38.821, NTN-TDL-C (LOS)

- Evaluation scenario:

  - Rural (LOS)

- Channel estimation: Realistic estimation:

  - Companies are encouraged to report channel estimation method.

- SCS:

  - 15 kHz only

- UE speed: 3 km/h

- Frequency drift: TBD

- Frequency offset: 0.1 ppm

Agreement

For link budget calculation, parameters in the following table are assumed:

| Parameters |  |
| --- | --- |
| Carrier frequency | 2 GHz for DL (S-band) |
| Satellite altitude | 600 km |
| Target elevation angle | 30° (LEO) |
| Atmospheric loss | Equation (6.6-8) in [38.811] |
| Shadowing margin | 3 dB |
| Scintillation loss | Section 6.6.6 in [38.811] Ionospheric loss: = 2.2 dB Tropospheric loss: Table 6.6.6.2.1-1 of [38.811] |
| Additional loss | 0 dB |
| Clear sky conditions | Yes |
| Satellite antenna polarization | Circular polarization |
| Terminal type | [S band: (M, N, P) = (1,1,2)] |
| UE antenna gain | -5.5dBi |
| Free space path loss | Equation (6.6-2) in [38.811] |
| Polarization loss | 3dB |
| Outcome | CNR |

- Appendix B

In RAN1#117, the following agreements have been made [2].

Observation

Based on LLS results on PDCCH coverage evaluation collected from different sources:

- It is observed that the required SNR for PDCCH is in average equal to -6dB (17 sources)

  - With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

    - 17 sources observed that there is no coverage gap with Set1-1/1-2 FR1.

      - The coverage margin is around 4 dB compared to CNR of -1.9 dB.

  - With parameter LEO600km Set1-3 FR1:

    - 15 sources observed that there is a PDCCH coverage gap of 3.9dB in average compared to CNR of -9.9 dB.

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDCCH.

Observation

Based on LLS results on PDSCH Msg2 coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH carrying Msg2 is in average equal to – 10.9 dB (14 sources)

  - With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

    - 14 sources observed that there is no coverage gap for PDSCH with Msg2:

      - The coverage margin is around 9 dB compared to CNR of -1.9 dB

  - With parameter LEO600km Set1-3 FR1:

    - 12 sources observed that there is no coverage gap for PDSCH with Msg2:

      - The coverage margin is around 1 dB on average compared to CNR of -9.9 dB

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH Msg2.

Observation

Based on LLS results on PDSCH Msg4 coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH carrying Msg4 is in average equal to – 5.2 dB (14 sources)

  - With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

      - 14 sources observed that there is no coverage gap for PDSCH with Msg4:

        - The coverage margin is around 3.3 dB on average compared to CNR of -1.9 dB

  - With parameter LEO600km Set1-3 FR1:

    - 11 sources observed that there is a coverage gap for PDSCH with Msg4:

      - The coverage gap is around 4.7 dB on average compared to CNR of -9.9 dB

    - 1 source observed that there is no coverage gap for PDSCH with Msg4 with a coverage margin of 0.3 dB compared to CNR of -9.9 dB

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH Msg4.

Observation

Based on LLS results on PDSCH SIB1 coverage evaluation collected from different sources:

- For PDSCH carrying SIB1 option 1 (with a payload size of 800bits) it is observed that the required SNR is in average equal to – 5.8 dB (14 sources)

- For PDSCH carrying SIB1 option 2 (with a payload size of 1280bits) it is observed that the required SNR is in average equal to – 3.4 dB (12 sources)

  - With parameter LEO600km Set1-1 FR1 and 1-2FR1:

    - 14 sources observed that there is no coverage gap for PDSCH with SIB1 option 1:

        - The coverage margin is around 3.9 dB on average compared to CNR of -1.9 dB

    - 12 sources observed that there is no coverage gap for PDSCH with SIB1 option 2:

        - The coverage margin is around 1.5 dB on average compared to CNR of -1.9 dB

  - With parameter LEO600km Set1-3 FR1:

    - 11 sources observed that there is a coverage gap for PDSCH with SIB1 option 1:

        - The coverage gap is around 4.1 dB on average compared to CNR of -9.9 dB

    - 1 source observed that there is no coverage gap for PDSCH with SIB1 option 1:

        - The coverage margin is 3.4 dB compared to CNR of -9.9 dB

    - 10 sources observed that there is a coverage gap for PDSCH with SIB1 option 2:

        - The coverage gap is around 6.5 dB on average compared to CNR of -9.9 dB

- Note: some results assumed SIB1 combination (where SIB1 is repeated within 160 ms) and some results assumed no SIB1 combination

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH carrying SIB1.

Observation

Based on LLS results on PDSCH SIB19 coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH carrying SIB19 is in average equal to – 6.9 dB (14 sources)

  - With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

    - 12 sources observed that there is no coverage gap for PDSCH with SIB19:

      - The coverage margin is around 4.2 dB on average compared to CNR of -1.9 dB

  - With parameter LEO600km Set1-3 FR1:

    - 10 sources observed that there is a coverage gap for PDSCH with SIB19:

      - The coverage gap is around 3.5 dB on average compared to CNR of -9.9 dB

- Note: all the results above assumed no SIB19 combination

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH carrying SIB19.

Observation

Based on the results of DL coverage ratio evaluation at system level collected from 7 sources for all the three LEO600km satellite parameter sets where the beam footprint diameter is 50 km:

- For Set 1-1/1-3, the coverage ratio can be improved from 10% to 100% if the SSB periodicity is increased from 20ms to 80ms and beam hopping is applied

- For Set 1-2, the coverage ratio can be improved from 1.5% to 96.8% if the SSB periodicity is increased from 20ms to 320ms and beam hopping is applied.

- Note: coverage ratio is N2+N3/ total beam footprints

- Note: the baseline assumes no beam hopping. TDM between SIB1 and SIB19 is assumed in those results, following current specs.

Based on the results of DL coverage ratio evaluation at system level collected from 3 sources for a deployment scenario implementing wide beam footprint:

- 1 source reports that with a deployment of wide beam covering 4 narrow (of 50km size) beams, which means Set 1-2 FR1 with additional EIRP reduction of 6dB, using SSB periodicity of 80 ms can provide coverage ratio of 96.8%, and Set 1-1/1-3 FR1 with additional EIRP reduction of 6dB, SSB periodicity of 80 ms can provide coverage of 100%.

- 1 source observed that for Set 1-1, 1-2 and 1-3, the coverage ratio can be improved from 1.5% to 100% using the legacy default SSB periodicity of 20ms during initial access, by choosing a wide beam footprint with beam footprint sizes of 84 km and 56 km respectively.

  - Note: the PDCCH and the PDSCH for SIB19 is assumed to be transmitted within 2 OFDM symbols and 5 MHz bandwidth. the PDSCH for SIB1 is assumed to be transmitted within 3 OFDM symbols and 5 MHz bandwidth. This assumes no SIB1 and SIB19 transmission in N2 beam footprints. This assumes non-aligned SFN timing across different beams.

- 1 source observed, for Set 1-1 with increased beam size, that the legacy SSB periodicity of 20ms during initial access is usable with NTN beam hopping, by choosing a deployment scenario implementing wide beam footprint with beam footprint sizes of 70.7 km and 86.6 km, leading to a total of 529 and 353 beam footprints within the satellite coverage area, respectively, and the coverage ratio is 80% and 90%, respectively, and a ratio of simultaneously active beam footprints to the total number of beam foot prints equal to 20% and 30%.

  - Note: Beam footprint size is increased by increasing only the adjacent beam spacing without increasing the 3dB beamwidth.

Note: RAN1 will further investigate the impact of SSB periodicity extension

Note: Any needed clarification “SSB channel enhancement is not considered” in the WID is up to RAN plenary

Note: RAN1 will further investigate the impact of wider beam of SSB and/or other channels on performance (e.g. link budget, capacity...)

Observation

Based on LLS results on PDSCH for VoIP coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH for VoIP is in average equal to – 11 dB (11 sources)

- With parameter LEO600km Set1-1 and 1-2 FR1:

  - When PDSCH repetition is enabled, 11 sources observed that there is no coverage gap for VoIP:

    - The coverage margin is around 9.1 dB on average, compared to CNR of -1.9 dB

- With parameter LEO600km Set1-3 FR1:

  - When PDSCH repetition is enabled, 9 sources observed that there is no coverage gap for VoIP:

    - The coverage margin is around 2.3 dB on average, compared to CNR of -9.9 dB

  - 1 source observed that even with 8 PDSCH repetitions there is a coverage gap of 1.5 dB compared to CNR of -9.9 dB

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH for VoIP.

Observation

Based on LLS results on PDSCH 3kbps coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH for low data rate is in average equal to – 11 dB (8 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

  - When PDSCH repetition is enabled, 8 sources observed that there is no coverage gap for PDSCH with 3kbp:

  - The coverage margin is around 9.1 dB on average, compared to CNR of -1.9 dB

- With parameter LEO600km Set1-3 FR1:

  - When PDSCH repetition is enabled, 6 sources observed that there is no coverage gap for PDSCH with 3kbp:

  - The coverage margin is around 1.6 dB on average, compared to CNR of -9.9 dB

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH 3kbps.

Observation

Based on LLS results on PDSCH 1Mbps coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH with 1Mbps data rate is in average equal to – 4.1 dB (7 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

  - 7 sources observed that there is no coverage gap for PDSCH with 1Mbps:

  - The coverage margin is around 2.2 dB on average, compared to CNR of -1.9 dB

- With parameter LEO600km Set1-3 FR1:

  - 5 sources observed that, there is a coverage gap for PDSCH with 1Mbps:

  - The coverage gap is around 5.5 dB on average, compared to CNR of -9.9 dB

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH 1Mbps.