3GPP TSG RAN WG1 #118	R1-2406748

Maastricht, Netherland. 19th – 23rd Aug 2024

Agenda item:		9.2.4

Source:	Nokia

Title:	Enhancement for asymmetric DL sTRP/UL mTRP scenarios

Document for:		Discussion and Decision

# Introduction

A New Work Item for Rel19 MIMO evolution was approved in RAN#102 [1].

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

Figure 1: Asymmetric DL sTRP/UL mTRP scenarios (UL-only TRP) deployment

Figure 1 depicts the scenario defined in the WID [1]. The DL transmission is from only a single TRP. This TRP is also capable of UL reception. Several UL-only TRPs are deployed in the area closer to the cell edge. The DL and UL-capable TRP functions as an anchor point and has the backhaul connection with all UL-only TRPs. The UL-only TRPs won’t be able to transmit reference signals in the DL. The UEs may listen to the SSB and DL control information from the single serving TRP and transmit data and control information to serving sTRP or UL-only TRP or both, depending on the deployed MIMO transmission reception technique. Both the FR1 and FR2 bands can be used in the deployment.

# Discussion

Figure 2: Uplink transmission in the presence of UL-only TRP

The WID states that two closed-loop PC adjustment states need to be specified for SRS, which are both separate from PUSCH adjustment states. In RAN1 #116bis, the following agreement was made:

Agreement(RAN1 #116bis Changsha) For indicating TPC command for those two SRS CLPC adjustment states through DCI when the UE is configured with two SRS CLPC adjustment states, support Option3: - Option 3: enhance the legacy DCI format 2_3 of higher layer parameter srs-TPC-PDCCH-Group = typeA and typeB;

As the group common control signalling may have some coverage and reliability limitations, additional techniques on using UE-specific control signalling (specifically, DCI format 1_1 and 0_1, using which SRS can be triggered) has been considered. The following agreement was made in this regard at RAN1 #116bis:

Agreement(RAN1 #116bis Changsha) For the asymmetric DL sTRP/UL mTRP deployment scenarios, study whether and how to indicate TPC command for SRS CLPC adjustment states through DCI format 1_1 and/or 0_1 when the UE is configured two SRS CLPC adjustment states.

It should be noted that, DCI format 0_1 supports up to two PUSCH CLPC command fields, and DCI format 1_1 supports up to two PUCCH CLPC command fields, where the cases with two TPC command fields were introduced in Rel-17 for multi-TRP UL scenarios.

Indicating TPC command for SRS would require adding one or two TPC command fields (e.g., 2 or 4 bits) in each of DCI formats 1_1 and 0_1, in addition to existing up to TPC command fields. One issue with such a proposal is that it increases the DCI size/overhead.

Observation 1: As group common control signalling may have some coverage and reliability limitations, UE-specific DCI format 1_1 and 0_1 could be further considered in the discussions regarding indicating TPC command for SRS. However, that would require adding one or two TPC command fields (for SRS) in each of DCI formats 1_1 and 0_1, in addition to existing up to TPC command fields (for PUCCH/PUSCH).

Proposal 1: Consider indicating TPC command(s) for SRS CLPC through DCI format 1_1 and/or 0_1 when the UE is configured two SRS CLPC adjustment states.

When the PL offset value is updated via MAC CE, it needs to be quantized. There is a trade-off between quantization error and overhead when selecting the number of bits. The number of bits used to represent the PL offset should not change with different deployment scenarios. Our evaluation results in the Appendix show that the range of the PL offsets is quite similar for different deployment scenarios.

Observation 2: Our evaluations show the pathloss offset range can be around 50 to 60 dB, and thus, quantization level of (at least) 4 bits for pathloss offset indication with 3 dB steps or 5 bits for pathloss offset indication with 2dB steps would be required.

Proposal 2: For pathloss offset indication, considering evaluation results, consider a quantization level of at least 4 bits for pathloss offset indication with 3 dB steps or 5 bits for pathloss offset indication with 2dB steps.

According to section 7 of 3GPP TS 38.213, if a UE transmits PUSCH on active UL BWP b of carrier f of serving cell c using parameter set configuration with index 𝑗 and PUSCH power control adjustment state with index 𝑙, the UE determines the PUSCH transmission PPUSCH,b,f,c(i,j,qs,l) in transmission occasion i as

PPUSCH,b,f,ci,j,qd,l=minPCMAX,f,ci,POPUSCH,b,f,cqs+10log102μ∙MRB, b,f,cPUSCHi+αb,f,cqs∙PLb,f,cqd-αb,f,cqs∙GPL,b,f,ci,l+∆TF,b,f,ci+fb,f,ci,l

(1)

A new term, the pathloss offset GPL,b,f,ci,l is introduced in the expression (1). It may be defined as follows,

GPL,b,f,ci,l=|Pathloss of UE to serving gNB -Pathloss of UE to ULonlyTRP |. In situations where Pathloss of UE to serving gNB< Pathloss of UE to UL_only TRP, ‘‘ +αb,f,cqs∙GPL,b,f,ci,l” will be used in equation (1).

The UE calculates SRS transmission power in dBm for the UE to UL-only TRP link

PSRS,b,f,ci,q,l=minPCMAX,f,ci,POSRS,b,f,cqs+10log102μ∙MSRS,b,f,ci+αSRS,b,f,cqs∙PLb,f,cqd-αSRS,b,f,cqs∙GPL,b,f,ci,l+hb,f,ci,l

(2)

Where, active UL BWP b of carrier f of serving cell c using SRS power control adjustment state with index l, during transmission occasion 'i'. The UE to serving gNB link pathloss is applied to PLb,f,c(qd).

The UE determines its Tx power of PUCCH on UL BW part b of carrier f of serving cell c using PUCCH power control adjustment state with index l, using parameter set qu, path loss reference qd  in PUCCH transmission period i as shown in the equation below.

PPUCCHb,f,ci,qu,qd,l=minPCMAXf,ci,                                        P0PUCCHb,f,cqu+10log102μ*MRBb,f,cPUCCHi+PLb,f,cqd-GPL,b,f,ci,l+∆FPUCCHF+∆TF,b,f,ci+gb,f,c(i,l)

(3)

Proposal 3. For the transmit power calculation of a UL channel/signal towards a UL-only TRP, account for the pathloss offset in the corresponding transmission power formula by considering the pathloss offset parameter subtracted from or added to the existing pathloss component.

To calculate a Type 1 PHR based on an actual PUSCH transmission, if a joint/UL TCI state associated with a PL offset with value Gb,f,c is applied on this PUSCH transmission, the UE determines the Type 1 PHR as:

PHtype1,b,f,ci,j,qd,l= PCMAX,f,ci-PO_PUSCH,b,f,cj+10log102μ⋅MRB,b,f,cPUSCH(i)+αb,f,cj⋅(PLb,f,cqd-Gb,f,c)+∆TF,b,f,ci+fb,f,ci,l

To calculate a Type 3 PHR based on an actual SRS transmission, if a joint/UL TCI state associated with a PL offset with value Gb,f,c is applied on this SRS transmission, the UE determines the Type 3 PHR as:

PHtype3,b,f,ci,qs= PCMAX,f,ci-PO_SRS,b,f,cj+10log102μ⋅MSRS,b,f,c(i)+αSRS, b,f,cj⋅(PLb,f,cqs-Gb,f,c)+fb,f,ci

Proposal 4: For the PHR type 1 and 3 calculation, account for the pathloss offset in the corresponding PHR formulations by adding or subtracting from the pathloss component.

- Study the PHR triggering conditions to account for PL offset and send an LS to RAN2 regarding the update of the triggering conditions in TS 38.321.

In the presence of UL-Only TRPs, the RACH process (both the CFRA and CBRA) may use a lower transmission power level to reach one of the closest UL-Only TRPs and improve the RACH performance in the cell. By the PDCCH order or other means, gNB may indicate the suitable UL-Only TRP for the RACH and the required pathloss gap values. In the RAN1#117, the following alternatives were down selected.

Agreement(RAN1 #117 Fukuoka) For indicating a PL offset for PDCCH-order PRACH transmission at least for FR1, further study and down-select one from the Alt1 and Alt3 by RAN1#118 meeting: - Alt1: RRC configures multiple PL offset values [in PRACH-Config] and PDCCH-order DCI indicates one of them through one DCI field - Alt3: The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission - FFS: the details of DCI field design.

Indicating the PL offset using PDCCH-Order DCI is a straightforward solution. We believe that applying the PL offset value associated with the joint or UL TCI state to the PDCCH order is also a feasible solution and doesn’t incur additional overhead. Moreover, the alternative where PDCCH order DCI indicates one TCI state associated with the PL offset is also a possibility.

For indicating a PL offset for PDCCH-order PRACH transmission, our preference is Alt3.

Proposal 5: For indicating a PL offset for PDCCH-order PRACH transmission, adopt Alt3:

- Alt3: The PL offset associated with one of the indicated joint/UL TCI states for UL TRP in the unified TCI framework is applied on the PDCCH-order PRACH transmission.

In addition, in the presence of asymmetric sTRP/mTRP deployment with pathloss offsets, the enhancements to the power control of the PUSCH and PUCCH transmissions after the successful transmission of the RACH preamble can be studied. Actually, it should be clarified whether such transmissions are in the scope of the working item.

Proposal 6: Study the impact of pathloss offsets on UL transmissions of RACH procedures, such as PRACH Msg3 (for the 4-step RACH procedure) and MsgA (for the 2-step RACH procedure).

There is no DL transmission from UL-only TRPs towards UEs in the UL-only TRP deployments. The UE doesn’t have the pathloss between UL-Only TRP to UE or the pathloss offset value indicated by the gNB to make the initial SRS power level calculation. The accurate initial SRS power is necessary for accurate channel estimation at the UL-Only TRP. It is beneficial to indicate the initial SRS power level to the UE from gNB via MAC CE or RRC signaling. The MAC CE used for pathloss offset could also be leveraged for SRS initial power level indication.

Proposal 7: Discuss SRS initial transmit power level. Consider the SRS initial power level indicated to UE via MAC CE or RRC signaling.

The Rel. 18 MIMO WI included the two TA enhancements for UL multi-DCI for multi-TRP operation. UL-Only TRPs don’t have the capability to transmit DL control and data signals. Thus, it requires 2 TA enhancements for single DCI for multi TRP operation. It was discussed in the item 5 of the WID for the Rel. 19, asymmetric sTRP/mTRP (UL-Only TRP) but was not included. We propose to explore the possibility of including the limited scope 2TA enhancements for single DCI mode to enable UL-Only TRPs.

Proposal 8: Consider including the 2TA enhancements for single-DCI mode to enable transmissions towards UL-only nodes/TRPs.

# Conclusion

In this contribution, we make the following observations and proposals. Please consider adopting them.

Observation 1: As group common control signalling may have some coverage and reliability limitations, UE-specific DCI format 1_1 and 0_1 could be further considered in the discussions regarding indicating TPC command for SRS. However, that would require adding one or two TPC command fields (for SRS) in each of DCI formats 1_1 and 0_1, in addition to existing up to TPC command fields (for PUCCH/PUSCH).

Observation 2: Our evaluations show the pathloss offset range can be around 50 to 60 dB, and thus, quantization level of (at least) 4 bits for pathloss offset indication with 3 dB steps or 5 bits for pathloss offset indication with 2dB steps would be required.

Proposal 1: Consider indicating TPC command(s) for SRS CLPC through DCI format 1_1 and/or 0_1 when the UE is configured two SRS CLPC adjustment states.

Proposal 2: For pathloss offset indication, considering evaluation results, consider a quantization level of at least 4 bits for pathloss offset indication with 3 dB steps or 5 bits for pathloss offset indication with 2dB steps.

Proposal 3. For the transmit power calculation of a UL channel/signal towards a UL-only TRP, account for the pathloss offset in the corresponding transmission power formula by considering the pathloss offset parameter subtracted from or added to the existing pathloss component.

Proposal 4: For the PHR type 1 and 3 calculation, account for the pathloss offset in the corresponding PHR formulations by adding or subtracting from the pathloss component.

- Study the PHR triggering conditions to account for PL offset and send an LS to RAN2 regarding the update of the triggering conditions in TS 38.321.

Proposal 5: For indicating a PL offset for PDCCH-order PRACH transmission, adopt Alt3:

- Alt3: The PL offset associated with one of the indicated joint/UL TCI states for UL TRP in the unified TCI framework is applied on the PDCCH-order PRACH transmission.

Proposal 6: Study the impact of pathloss offsets on UL transmissions of RACH procedures, such as PRACH Msg3 (for the 4-step RACH procedure) and MsgA (for the 2-step RACH procedure).

Proposal 7: Discuss SRS initial transmit power level. Consider the SRS initial power level indicated to UE via MAC CE or RRC signaling

Proposal 8: Consider including the 2TA enhancements for single-DCI mode to enable transmissions towards UL-only nodes/TRPs.

# References

- RP-234007	New WID: NR MIMO Phase 5

# Appendix: Performance evaluation

As depicted in Figures 1 and 2 and in the introduction, when the UL-only TRPs are deployed, it may improve the average and cell-edge spectral efficiencies significantly. It may be helpful to establish the performance gains using some performance evaluations. On the infrastructure side, the serving gNB is expected to have backhaul connections with the UL-only TRPs. Thus, the serving gNB can have timely channel state information and pathloss values of the UE to UL-only TRPs and UE to serving gNB links. The pathloss difference or the offset needs to be sent to UE for accurate TPC towards the UL-only TRPs. Obtaining the typical range of the pathloss gap value in different deployment scenarios would help to decide on the range of the pathloss gap indication by the gNB. Moreover, the pathloss gap indication may need to carry the quantized values of the pathloss gap to reduce the control overhead. Thus, evaluating the performance with different pathloss gap quantization levels would be helpful to decide on the performance versus quantization level trade-off. The UL system simulations assumptions are listed in Table 1. The node attachment to the UL-only TRP is based on RSRP_UL-only TRP + 3 dB > RSRP_gNB.

Figure 3 Fixed UL-only TRP deployment scenario

The fixed UL-only TRP deployment is shown in Figure 3. The corresponding pathloss offset range is depicted in Figure 4 for UMi, UMa and RMa channel models.

Figure 4. Pathloss difference offset (gap) for Umi, Uma and RMa scenarios for fixed deployments

The random UL-only TRP deployment is shown in Figure 6. The corresponding pathloss offset ranges are depicted in Figures 7, 8, and 9 for UMi, UMa and RMa channel models, respectively.

Figure 5 Fixed UL-only TRP deployment scenario

Figure 6. Pathloss difference offset for Umi scenario for random UL-Only TRP deployments

Figure 7. Pathloss difference offset for Uma scenario for random UL-Only TRP deployments

Figure 8. Pathloss difference offset for RMa scenario for random UL-Only TRP deployments

For studying the benefits of introducing the UL-Only TRPs in a cell coverage area and deciding on the operational parameters, UL system simulations may be useful. In Table 2, our simulation assumptions are listed.

| Parameter | gNB | UL-only TRP |
| --- | --- | --- |
| Carrier frequency | 4 GHz |  |
| Bandwidth | 100 MHz |  |
| SCS | 30 kHz |  |
| Network layout | 3 cell | 3 UL-only TRPs placed in a sector |
| Channel model | Umi, Uma, RMa 3GPP TR 38.901 |  |
| BS antenna structure and TXRU | 128Rx = (M,N,P,Mg,Ng) = (8,8,2,1,1), (dH,dV) = (0.5, 0.5)λ. TXRU: 16TXRU=(Mp,Np,P,Mg,Ng) =(1,8,2,1,1) | 64Rx = (M,N,P,Mg,Ng) = (8,4,2,1,1), (dH,dV) = (0.5, 0.5)λ. TXRU: 8TXRU=(Mp,Np,P,Mg,Ng) =(1,4,2,1,1) |
| UE antenna structure and TXRU | 4Tx = (M,N,P,Mg,Ng) = (1,2,2,1,1), (dH,dV) = (0.5, 0.5)λ TXRU: 4TXRU=(Mp,Np,P,Mg,Ng)=(1,2,2,1,1) |  |
| CSI-T | CB-based UL TX |  |
| Channel estimation | Real |  |
| Scheduling | PF |  |
| MIMO receiver (CSI/data) | MMSE |  |
| Traffic model | Full buffer |  |
| ISD | 200m, 500m, 1.732km |  |
| Number of average UEs per macro sector | 30 |  |
| UE power | 23dBm |  |
| UE mobility | Fixed 30 km/h, [mix:80% indoor (3 km/h), 20% outdoor (30 km/h)] |  |
| UL power control | Open Loop TPC | Closed loop TPC |
| Modulation | Upto 64QAM |  |
| UL RX node decision | Legacy | RSRP or Distance based with 3 dB offset |
| CSI feedback delay | 5 ms |  |
| Antenna height | 10m | 10m |
| receiver noise figure | 5 dB | 5dB |
| UE receiver noise figure | 9 dB |  |

TABLE 1. UL System Simulations parameter assumptions