3GPP TSG RAN WG1 #118			R1-2406970

Maastricht, NL, Aug 19th – Aug 23rd, 2024

Source:	Sharp

Title:	Discussion on specification support for AI/ML based positioning accuracy enhancements

Agenda Item:	9.1.2

Document for:	Discussion and Decision

Introduction

In the last RAN1 meeting, the following agreements and working assumption are made [1].

Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved. Note: Previous related working assumption made in RAN1#116bis for training data generation of AI/ML based positioning Case 3b will not need to be confirmed. Agreement Sample-based measurement is defined as: - The measurement is composed of Nt' samples of the estimated channel response in time domain. The timing information for the Nt' samples are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The corresponding measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' samples. - Nt' and k can be signalled - FFS: the value range of Nt'; the value range of integer k for the timing granularity T. - The timing information is defined relative to a reference time Further discussion is expected on the determination of Nt' and k (including signaling) , and a rule to be introduced for selecting Nt' samples. Note: It doesn’t imply the definition of Sample-based measurement will be captured into the spec. Agreement Path-based measurement refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed). Agreement For training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are: - for a same UE (PRU or Non-PRU UE), and - for a same location associated with Part B. Note: the association can be discussed Agreement Draft LS R1-2405577 is endorsed in principle by adding the latest agreements made in this meeting and adding “agreements” to “Note: the working assumptions above are based on RAN1 understanding for RAN work item (NR_AIML_air).” Agreement - Final LS R1-2405578 is endorsed.

In this contribution, we discuss specification support for positioning accuracy enhancements.

Discussions

Model input

Sample-based and path-based measurement for model input

Sample-based vs path-based measurement

In RAN1#116 and RAN1#117 meeting, it was agreed that RAN1 investigate sample-based and path-based measurement [1][3].

Agreement on RAN1#116 In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, RAN1 investigate the following alternatives: - Alternative (a). Sample-based measurements, where the timing information is an integer multiple of sampling periods. - Alternative (b). Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods. The issues to be studied include, but not limited to, the following: - Tradeoff of positioning accuracy and signaling overhead - Impact and necessary details of gNB/UE implementation to obtain the channel measurement values. - Whether the same Alternative(s) applies to all cases or not - Applicability and necessity of specifying the Alternative(s) to different cases - Note: different sub-cases may have different issues. Note: In addition to timing information, the components for the channel measurement for model input may also include power and potentially phase. To provide the type of the channel measurement in their investigation. Agreement on RAN1#117 Sample-based measurement is defined as: - The measurement is composed of Nt' samples of the estimated channel response in time domain. The timing information for the Nt' samples are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The corresponding measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' samples. - Nt' and k can be signalled - FFS: the value range of Nt'; the value range of integer k for the timing granularity T. - The timing information is defined relative to a reference time Further discussion is expected on the determination of Nt' and k (including signaling) , and a rule to be introduced for selecting Nt' samples. Note: It doesn’t imply the definition of Sample-based measurement will be captured into the spec. Agreement on RAN1#117 Path-based measurement refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed).

In this contribution, we analyse the advantages and disadvantages of sample-based measurement and path-based measurement for model input.

In our understanding, both sample-based measurement and path-based measurement have timing granularity T=2kxTc to represent samples/paths. The difference between sample-based measurement and path-based measurement is whether the method of extracting the sample/path is defined in the spec. For the sample-based measurement, firstly an entity measures a channel with specified number of samples (Nt) and period, then extracts N’t samples within Nt samples by a specified rule (e.g. the strongest sample selection). Thus, the ambiguity due to the difference of implementation of each receiver can be minimized. On the other hand, path-based measurement refers to the measurement in the existing specifications. Thus, how to select the paths is up to implementation manner and that is why path-based measurement has ambiguity issue causing inconsistency between training and inference. The inconsistency may require much more training data for generalization and degrade positioning accuracy. Therefore, sample-based measurement should be supported for AI/ML based positioning.

Observation 1: Sample-based measurement can minimize the ambiguity due to variations in the implementations of each receiver.

Observation 2: Path-based measurement has ambiguity issue and requires more training data for generalization, otherwise the positioning accuracy is degraded due to the ambiguity.

Proposal 1: For model input of the AI/ML based positioning, sample-based measurement should be supported.

Sample selecting rule of sample-based measurement

In the RAN1#117 meeting, there was a discussion about selecting rule of the N’t samples for the sub-sampling in the sample-based measurement. One of the sampling rules is to select samples with strongest power and it is straightforward method for selecting the sub-samples. In TR38.843 [5], the selecting rule of N’t samples also adopts the strongest power-based sample selection, and the TR shows that there is little impact on performance even when signalling size shrinks to 1/4 (256 to 64). Therefore, for AI/ML based positioning, sample-based measurement with selecting samples with strongest power should be supported.

For the evaluation of direct AI/ML positioning, when N't time domain samples with the strongest power are selected as model input, evaluation results show that: - For model input of CIR or PDP and Nt=256, using different N't while holding other parameters constant, - Reducing N't from 256 to 64 does not appreciably degrade the positioning accuracy, while the measurement size and signalling overhead shrink to (approximately) 1/4 that of Nt=N't=256. - Positioning error of N't=128 is 1.02 ~ 1.07 times the positioning error of Nt=N't=256; - Positioning error of N't=64 is 1.02 ~ 1.21 times the positioning error of Nt=N't=256; - Reducing N't from 256 to 32~16 degrade the positioning accuracy, while the measurement size and signalling overhead shrink to (approximately) 1/8 ~ 1/16 that of Nt=N't=256. - Positioning error of N't=32 is 1.14 ~ 2.03 times the positioning error of Nt=N't=256; - Positioning error of N't=16 is 1.12 ~ 2.54 times the positioning error of Nt=N't=256; - Reducing N't from 256 to 9~8 degrade the positioning accuracy, while the measurement size and signalling overhead shrink to (approximately) 1/32 that of Nt=N't=256. - Positioning error of N't=9~8 is 1.42 ~ 3.29 times the positioning error of Nt=N't=256; - For model input of DP and Nt=256, using different N't while holding other parameters constant, - One source (R1-2304339) showed that reducing N't from 64 to 32 does not degrade the positioning accuracy while the measurement size and signalling overhead shrink by (approximately) 1/2. - Positioning error of N't=32 is 1.03 times the positioning error of N't=64. - Note: the evaluation results based on the other model input (e.g., multiple path) can be added in next meeting For AI/ML assisted positioning, when N't time domain samples with the strongest power are selected as model input, evaluation results show that for model input of CIR or PDP and Nt=256, using different N't while holding other parameters the same, - Reducing N't from 256 to 64 does not appreciably degrade the positioning accuracy, while the measurement size and signalling overhead shrink to (approximately) 1/4 that of Nt=N't=256. - Positioning error of N't=128 is 1.00 ~ 1.33 times the positioning error of Nt=N't=256; - Positioning error of N't=64 is 0.98 ~ 1.23 times the positioning error of Nt=N't=256; - Reducing N't from 256 to 32~16 may degrade the positioning accuracy, while the measurement size and signalling overhead shrink to (approximately) 1/8 ~ 1/16 that of Nt=N't=256. - Positioning error of N't=32 is 1.15 ~ 1.69 times the positioning error of Nt=N't=256; - Positioning error of N't=16 is 1.04 ~ 2.67 times the positioning error of Nt=N't=256; - Reducing N't from 256 to 9 degrade the positioning accuracy, while the measurement size and signalling overhead shrink to (approximately) 1/32 that of Nt=N't=256. - Positioning error of N't=9 is 1.66 ~ 4.40 times the positioning error of Nt=N't=256;

Observation 3: TR38.843 shows that the strongest power-based sample selection has little impact on the positioning accuracy performance even when the signalling size of channel measurement shrinks to 1/4 (256 to 64) for direct AI/ML positioning and AI/ML assisted positioning.

Proposal 2: For AI/ML based positioning, sample-based measurement with selecting samples with strongest power should be supported.

We assume two ways to represent the timing information of sub-sampled measurement for sample-based DP, PDP and CIR. The one is to use legacy-like format that the timing information of each N’t sub-samples are represented by integer number in log2Nt bits (i.e. the total bit length will be Nt'∙log2Nt). The other one is to use bitmap format that the timing information is represented by a bitmap where the length of the bitmap is Nt. The legacy-like format offers benefits when the Nt is large value but N’t is not. The bitmap format, on the other hand, offers benefit when the Nt is not too large. The examples of these two formats are shown in bellow:

Figure 1: Legacy-like format where the timing information of each N’t sub-samples are represented with integer number in log2Nt bits

Figure 2: Bitmap format that the timing information is represented with bitmap where the length is Nt bits

For these two formats, the following table shows the comparison of the required bit length to represent the timing information:

Table 1. Required bit lengths for legacy-like format and bitmap format

| Nt | N't | Legacy-like format [bits] | Bitmap format [bits] |
| --- | --- | --- | --- |
| 256 | 128 | 1024 | 256 |
| 256 | 64 | 512 | 256 |
| 256 | 32 | 256 | 256 |
| 256 | 16 | 128 | 256 |
| 128 | 64 | 448 | 128 |
| 128 | 32 | 224 | 128 |
| 128 | 16 | 112 | 128 |
| 64 | 32 | 192 | 64 |
| 64 | 16 | 96 | 64 |
| 32 | 16 | 80 | 32 |

In the above cases, legacy-like format requires more bits than bitmap format except for the cases where (Nt, N’t) = (256, 16) and (128, 16). In other words, the bitmap format can reduce the signalling overhead to signal the channel measurement for AI/ML based positioning. Therefore, bitmap format should be introduced for sub-sampled channel measurement, if the sample-based measurement is supported. How to use each format properly for each case can be discussed further.

Observation 4: The legacy-like format requires more bits than bitmap format in the most cases.

Proposal 3: The bitmap format should be introduced for channel measurement for sample-based measurement of AI/ML based positioning.

Proposal 4: How to properly use legacy-like format and bitmap format for each AI/ML positioning case can be further discussed.

Phase information for model input

In RAN1#116 meeting, phase information for model input was discussed and following agreement was made [3].

Agreement For AI/ML based positioning for all use cases, RAN1 investigate the necessity and feasibility of using phase information (in addition to timing information and power information) for determining model input. The issues to study include: - Tradeoff of positioning accuracy and signaling overhead - The impact of transmitter and receiver implementation - Specification impact - Other aspects are not precluded Note: the phase information may be used in different ways, e.g., one phase value for the first path or first sample only; triplet of {timing information, power information, phase information} for CIR, etc.

In our understanding, there are mainly two ways to report phase information of the channel: One is (a) continuous phase information for each sample/path (e.g. including CIR), and the other one is (b) phase information for single sample/path (e.g. the first sample/path phase). The phase reporting is already supported in the Rel-18 spec, so additional specification effort may be not so large. And we should not concern about the signalling overhead for phase information report because whether to report the phase information can be up to LMF decision. For (a), the positioning accuracy enhancement is observed in the TR 38.843, so the CIR report should be supported. However, in our understanding, (b) may not contribute to the positioning accuracy enhancement because the single-phase information may be randomized. Therefore, we make a following proposal:

Proposal 5: For AI/ML based positioning, continuous phase information is supported for model input.

Timing reference of downlink measurement

In the RAN1#117 meeting, there are three options for timing reference of downlink measurement and following proposal was made but not agreed:

Proposal 2.2.2-1 For AI/ML based positioning Case 2b, for UE channel measurements reported to LMF, the timing information is represented relative to a reference time, where the reference time is selected from the following. - Option 1: The reference time is TSubframeRxi, as defined in TS 38.215, clause 5.1.29. - Option 2: The reference time is TUE-TX, as defined in TS 38.215, clause 5.1.30. - Option 3: Introduce a DL RTOA reference time T0+tPRS

The following descriptions are definitions of DL-RSTD, UE Rx-Tx time difference and UL-RTOA, respectively and descriptions of TSubframeRxi, TUE-TX and UL RTOA reference time are highlighted. The UL-RTOA reference time can reuse for DL-RTOA with few modifications.

DL reference signal time difference (DL RSTD) is the DL relative timing difference between the Transmission Point (TP) [18] j and the reference TP i, defined as TSubframeRxj – TSubframeRxi, Where: TSubframeRxj is the time when the UE receives the start of one subframe from TP j. TSubframeRxi is the time when the UE receives the corresponding start of one subframe from TP i that is closest in time to the subframe received from TP j. Multiple DL PRS resources can be used to determine the start of one subframe from a TP. For frequency range 1, the reference point for the DL RSTD shall be the antenna connector of the UE. For frequency range 2, the reference point for the DL RSTD shall be the antenna of the UE.

The UE Rx – Tx time difference is defined as TUE-RX – TUE-TX Where: TUE-RX is the UE received timing of downlink subframe #i from a Transmission Point (TP) [18], defined by the first detected path in time. TUE-TX is the UE transmit timing of uplink subframe #j that is closest in time to the subframe #i received from the TP. Multiple DL PRS or CSI-RS for tracking resources, as instructed by higher layers, can be used to determine the start of one subframe of the first arrival path of the TP. The time of the beginning of a subframe is determined by assuming the time durations of the OFDM symbols at the receiver are the same as defined in TS 38.211 [3]. For frequency range 1, the reference point for TUE-RX measurement shall be the Rx antenna connector of the UE and the reference point for TUE-TX measurement shall be the Tx antenna connector of the UE. For frequency range 2, the reference point for TUE-RX measurement shall be the Rx antenna of the UE and the reference point for TUE-TX measurement shall be the Tx antenna of the UE.

The UL Relative Time of Arrival (TUL-RTOA) is the beginning of subframe i containing SRS received in Reception Point (RP) [18] j, relative to the RTOA Reference Time [16]. The UL RTOA reference time is defined as T0+tSRS, where - T0 is the nominal beginning time of SFN 0 provided by SFN Initialization Time [15, TS 38.455] - tSRS=10nf+nsf×10-3, where nf and nsf are the system frame number and the subframe number of the SRS, respectively. Multiple SRS resources can be used to determine the beginning of one subframe containing SRS received at a RP. The reference point for TUL-RTOA shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Rx antenna (i.e. the centre location of the radiating region of the Rx antenna), - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector.

For Option 3, we have concerns about synchronization issues. The RTOA reference time is signaled from LMF but the actual reference time is based on the UE clock. Therefore, the UE must have accurate timing clock for synchronization with other measurement. Otherwise, inconsistency issue between training and inference may appear. Moreover, DL-RTOA reference time is not supported in the current specification. Therefore, the Option 3 should not be supported.

Figure 3: Example of timing reference - DL-RTOA reference time

Observation 5: The DL RTOA reference time imposes the UE to implement accurate clock.

Proposal 6: DL RTOA reference time is not supported for AI/ML based positioning Case 2b.

For Option 1, TSubframeRxi is measured with certain reference TRP as shown in below. In other words, TSubframeRxi depends on not only the TRP which is used to measure CIR, PDP or DP but also the reference TRP. Therefore, measurement data associated with one AI/ML model shall be generated with same reference TRP to keep consistency between training and inference. On the other hand, TSubframeRxi has the least ambiguity due to UE implementation among the three options.

Figure 4: Example of timing reference - TSubframeRxi

For Option 2, TUE-TX can be generated without reference TRP. Thus, the inconsistency issue due to the reference TRP may not be appeared. Although the ambiguity due to UE implementation may be little larger than Option 1.

Figure 5: Example of timing reference - TUE-TX

Furthermore, Option 1 and Option 2 can reuse current specifications. Therefore, we make a following proposal:

Proposal 7: For AI/ML based positioning Case 2b, for UE channel measurements reported to LMF, the timing information is represented relative to a reference time, where the reference time is selected from the following.

•	Option 1: The reference time is TSubframeRxi, as defined in TS 38.215, clause 5.1.29.

•	Option 2: The reference time is TUE-TX, as defined in TS 38.215, clause 5.1.30.

Additional information for data collection

For AI/ML based positioning, two agreements were made for training data collection in RAN1#116bis and RAN1#117 meeting[2][1]:

Agreement on RAN1#116bis For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Agreement on RAN1#117 For training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are: - for a same UE (PRU or Non-PRU UE), and - for a same location associated with Part B. Note: the association can be discussed

It was agreed that the Part A and Part B does not only include channel measurement and ground truth label, respectively, but also include quality indicator and time stamp. The quality indicator and time stamp for Part A and Part B can reuse the legacy LPP parameters without any issues (i.e. NR-TimingQuality and NR-TimeStamp, respectively) for data collection.

Proposal 8: Legacy quality indicator and time stamp are reused for data collection of AI/ML based positioning.

Moreover, the quality indicator can be used by the LMF to request Part A and/or Part B from UE/PRU for training data collection, or the quality indicator can be used by the UE to request dataset of Part A and/or Part B from LMF for its training. By using the quality indicator for data requests, the timing quality of the collected data can be ensured, and signalling overhead can be reduced. For example, LMF may request the UE to provide the Part A data with the quality indicator, then the UE generates channel measurement for training data collection. The UE report the Part A data for the LMF which the channel measurement fulfils the quality indicator.

Observation 6: By using the quality indicator for data requests, the timing quality of the collected data can be ensured, and signalling overhead can be reduced.

Proposal 9: For training data collection of AI/ML based positioning, quality indicator can be used to request Part A and Part B.

In addition to the agreed components, Area ID and/or identifier for reference signal (e.g. DL-PRS-ID) can help to ensure the consistency of model training and inference. Therefore, these components should be provided along with training data.

Observation 7: Area ID and/or identifier for reference signal (e.g. DL-PRS-ID) can help to ensure the consistency of model training and inference.

Proposal 10: For training data collection of AI/ML based positioning, Area ID and/or identifier for reference signal (e.g. DL-PRS-ID) are provided together with training data.

Model output for AI/ML assisted positioning

In RAN1#116 meeting, for AI/ML positioning case 2a and case 3a, at least LOS/NLOS indicator and/or timing information (UL-RTOA and gNB Rx-Tx Time Difference for uplink, and DL-RSTD and UE Rx-Tx Time Difference for downlink) are supported for reporting [3].

Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion. Agreement For AI/ML assisted positioning Case 2a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via DL RSTD or UE Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion.

In the legacy positioning, the LOS/NLOS indicator is not reported alone; it is always associated with the timing information as specified in TS 38.214.

TS 38.214 5.1.6.5 PRS reception procedure … The UE may be requested, subject to UE capability, to report LoS/NLoS indicator(s) via higher layer parameter nr-los-nlos-IndicatorRequest. The UE can report LoS/NLoS indicator(s) via higher layer parameter nr-los-nlos-Indicator associated with each DL RSTD, DL PRS-RSRP, DL PRS-RSRPP, and UE Rx-Tx time difference measurements. The UE can report LoS/NLoS indicator(s) via higher layer parameter nr-los-nlos-Indicator associated with each dl-PRS-ID in a measurement report. For the LoS/NLoS indicator(s) associated with DL RSTD, the UE may report one indicator associated with the dl-PRS-ID indicated by higher layer parameter dl-PRS-ReferenceInfo and one indicator associated with the dl-PRS-ID of the DL RSTD measurement. A UE may be provided with LoS/NLoS indicator(s) via higher layer parameter nr-los-nlos-Indicator, and it may be associated with each DL PRS resource of each configured dl-PRS-ID or may be associated with each configured dl-PRS-ID. The values of the higher layer parameter LOS-NLOS-Indicator may be soft values (0, 0.1, …, 0.9, 1) or hard values (0, 1) with the values corresponding to the likelihood of LoS, with a value of 1 corresponding to LoS and a value of 0 corresponding to NLoS.

Both LOS/NLOS indicator and timing information derived by AI/ML should not be included in one measurement report because these two measurements must be associated with each other. For example, in case that LOS/NLOS indicator is derived by AI/ML, the model outputs LOS/NLOS indicator which indicates more precise LOS or NLOS state of the path associated with non-AI/ML based timing information. Further, since the AI/ML model should learn the labels of timing information generated in only LOS environment, and AI/ML model strives to predict timing information in LOS environment. Therefore, since the LOS/NLOS indicator shall indicate LOS, the LOS/NLOS indicator may be a wasted signalling in this case. Therefore, RAN1 should consider following cases:

Case 1: LOS/NLOS indicator derived by AI/ML and timing information derived by legacy positioning method are reported
Case 2: Only timing information derived by AI/ML is reported

Observation 8: It is not advisable to report both of timing information and LOS/NLOS indicator derived by AI/ML in a same measurement report for AI/ML assisted positioning.

Proposal 11: For Rel-19 AI/ML assisted positioning, RAN1 supports one or more of following cases regarding LOS/NLOS indicator and timing information to be reported:
Case 1: LOS/NLOS indicator derived by AI/ML and timing information derived by legacy measurement are reported
Case 2: Only timing information derived by AI/ML is reported

Performance monitoring

Performance monitoring for Case 1

In RAN1#116 bis meeting, the following agreement was made for performance monitoring for AI/ML positioning case-1[2]:

Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side. - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded.

Based on our understanding, example of sequence diagrams for each option are shown in Fig.1 (Option A) and Fig.2 (Option B):

Figure 6: Option A. The target UE side performs monitoring metric calculation

Figure 7: Option B. The LMF performs monitoring metric calculation.

In our understanding, the calculation node (target UE or LMF) should have at least inference result and corresponding ground truth label in order to calculate label-based performance monitoring metric in case 1. And the accuracy of performance monitoring depends on the ground truth label.

Figure 6 and Figure 7 shows our understanding of performance metric procedure of Option A and Option B in case 1. In perspective of ground truth label generation, for Option A-1, A-2 and B-1, the ground truth label for performance monitoring is generated based on legacy positioning method performed using measurement made by target UE since the PRU is not involving for the performance monitoring. Thus, these three options may not be useful in NLOS environment. Therefore, we should discuss Option A-3, A-4 and B-2, and Option A-1, A-2 and Option B-1 separately, and down-select two options where PRU is involving and not involving, respectively.

Among Option A-3, A-4 and B-2, Option A-3 has less specification impact and the quality of measurement and position can be guaranteed by the specification thus we slightly prefer Option A-3.

Among Option A-1, A-2 and Option B-1, Option A-2 may have less signalling overhead. Therefore, we prefer the Option A-2 in case that PRU is not involving in the performance monitoring.

Observation 9: For label-based performance monitoring for AI/ML positioning case-1, Option A-1, A-2 and B-1 may not be useful in NLOS environment that AI/ML based positioning focuses on.

Proposal 12: For performance monitoring in AI/ML based positioning Case 1, support Option A-3 in case that PRU is involving in the performance monitoring.

Proposal 13: For performance monitoring in AI/ML based positioning Case 1, support Option A-2 in case that PRU is not involving in the performance monitoring.

Conclusion

In this contribution, we have discussed our views on specification support for positioning accuracy enhancement and have the following observations and proposals.

Observation 1: Sample-based measurement can minimize the ambiguity due to difference of implementation of each receiver.

Observation 2: Path-based measurement has ambiguity issue and requires more training data for generalization, otherwise the positioning accuracy is degraded due to the ambiguity.

Proposal 1: For model input of the AI/ML based positioning, sample-based measurement should be supported.

Observation 3: TR38.843 shows that the strongest sample selection has little impact on the positioning accuracy performance even when the signalling size of channel measurement shrink to 1/4 (256 to 64) for direct AI/ML positioning and AI/ML assisted positioning.

Proposal 2: For AI/ML based positioning, sample-based measurement with selecting samples of strongest power should be supported.

Observation 4: The legacy-like format requires more bit length than bitmap format in the most cases.

Proposal 3: The bitmap format should be introduced for channel measurement for sample-based measurement of AI/ML based positioning.

Proposal 4: How to properly use legacy-like format and bitmap format for each AI/ML positioning case can be further discussed.

Proposal 5: For AI/ML based positioning, continuous phase information is supported for model input.

Observation 5: The DL RTOA reference time imposes the UE to implement accurate clock.

Proposal 6: DL RTOA reference time is not supported for AI/ML based positioning Case 2b.

Proposal 7: For AI/ML based positioning Case 2b, for UE channel measurements reported to LMF, the timing information is represented relative to a reference time, where the reference time is selected from the following.

•	Option 1: The reference time is TSubframeRxi, as defined in TS 38.215, clause 5.1.29.

•	Option 2: The reference time is TUE-TX, as defined in TS 38.215, clause 5.1.30.

Proposal 8: Legacy quality indicator and time stamp are reused for data collection of AI/ML based positioning.

Observation 6: By using the quality indicator for data requests, the timing quality of the collected data can be ensured, and signalling overhead can be reduced.

Proposal 9: For training data collection of AI/ML based positioning, quality indicator can be used to request Part A and Part B.

Observation 7: Area ID and/or identifier for reference signal (e.g. DL-PRS-ID) can help to ensure the consistency of model training and inference.

Proposal 10: For training data collection of AI/ML based positioning, Area ID and/or identifier for reference signal (e.g. DL-PRS-ID) are provided together with training data.

Observation 8: It should not be considered the case that both of timing information and LOS/NLOS indicator are derived by AI/ML in a same measurement report for AI/ML assisted positioning.

Proposal 11: For Rel-19 AI/ML assisted positioning, RAN1 supports one or more of following cases regarding LOS/NLOS indicator and timing information to be reported:
Case 1: LOS/NLOS indicator derived by AI/ML and timing information derived by legacy measurement are reported
Case 2: Only timing information derived by AI/ML is reported

Observation 9: For label-based performance monitoring for AI/ML positioning case-1, Option A-1, A-2 and B-1 may not be useful in NLOS environment that AI/ML based positioning focuses on.

Proposal 12: RAN1 to further primally discuss benefits and potential specification impact for Option A-3, A-4 and B-2.

References

“RAN1 Chairman’s Notes,” RAN1 #117, May. 2024.

“RAN1 Chairman’s Notes,” RAN1 #116 bis, Apr. 2024.

“RAN1 Chairman’s Notes,” RAN1 #116, Mar. 2024.

RP-234039 “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN#102, Qualcomm.

3GPP TR 38.843 V18.0.0 (2023-12); TSG RAN; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)