3GPP TSG RAN WG1 Meeting #120	R1-2500056

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.2.1

Source:	Futurewei

Title:	Enhancements for UE-initiated/Event-driven Beam Management

Document for:	Discussion and decision

# Introduction

During the RAN1#119 meeting, many agreements were reached [1], including measures to reduce beam application latency following a UE-initiated/Event-driven (UEI/ED) beam report, support for multiple events in UEI/ED beam report, evaluation periodicity for determining event instances, the first PUCCH channel configuration for UEI/ED beam report, handling the case that pre-configured Type-1 CG PUSCH does NOT carry the beam report, and procedures for cross-CC beam report transmission. However, some important issues and details of UEI/ED beam management remain unclear or are still under discussion (FFS/TBD). In this paper, we will discuss the unsolved issues related to UEI/ED beam management, focusing on reducing signaling overhead and beam application latency, as well as other leftover topics. We also provide potential solutions and outline our preferences, particularly for procedure(s) required to facilitate fast beam switching and activation. Other specific considerations include whether RS resource set for new beam in Event-1 need configuration, how to introduce additional indications to specify whether the CRI/SSBRI meets the conditions of Event-2, whether a dedicated CSI trigger state is needed for UEI/ED beam report, and how NW confirms beam(s) reported by the UEI/ED beam report.

# Discussion

## UEI/ED beam management for reducing overhead and latency

### Overhead and beam application latency reduction after UEI/ED beam report

In terms of beam application latency reduction after UEI/ED beam report, the following agreement was reached during RAN1#119 meeting [1]:

Agreement Study the following to reduce beam application latency after a UEI/ED beam report is sent - Alt-1: After confirmation/acknowledgement from NW, apply new beam without RRC configuration signaling or MAC-CE signaling - after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report - update TCI state(s) with the reported new beam(s) - activate new beam(s) without additional SSB reception - Alt-2: After receiving a TCI state activation command to activate a TCI state(s), if the new beam(s) associated with the TCI state(s) is reported as synchronized in the UEI/ED beam report, the TCI state(s) becomes applicable for DL reception without additional SSB reception. - Note: A reported new beam is determined as synchronized by UE, if the UE stores the QCL properties associated with the reported new beam(s) after the UEI/ED beam report is sent - FFS: How to inform a reported new beam in a UEI/ED beam report (i.e., introducing one-bit indicator for each reported new beam or all the reported new beam are assumed to be synchronized) - Alt-3: After sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report. - In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception. - Other alternatives are not precluded.

To reduce beam application latency after UEI/ED beam report, three alternatives (i.e., Alt-1, Alt-2, and Alt-3) were proposed and discussed in last RAN1 meeting.

In Alt-1, after receiving confirmation/acknowledgement from NW, UE can directly apply the reported new beam without requiring additional RRC reconfiguration message or MAC-CE activation signaling, as illustrated in Figure 1.

For instance, if Event-2 or Event-7 is fulfilled, but the reported new beam(s) are not included in the preconfigured TCI state pool, UE can update the preconfigured TCI state pool and the activated TCI state list based on the reported new beam(s) after confirmation/acknowledgement from NW. This eliminates the need for the RRC reconfiguration message and MAC CE activation command typically required in legacy beam management procedure, replacing them with just a confirmation or acknowledgment from the NW. Some may argue that the reported new beam(s) can be always preconfigured in the original TCI state pool, therefore the RRC reconfiguration of TCI state pool is not needed.  However, in high-frequency scenarios, it is common for UE to experience challenges, especially when moving at high speeds. Considering the potentially long interval between the configuration of the original TCI state pool and the occurrence of the triggering event, it becomes difficult for the NW to predict all the potential new beams the UE may experience and preconfigure them in the original TCI state pool. As a result, it is highly likely that the new beam(s) will not be included in the original TCI state pool. With the legacy approach, updating the original TCI state pool would require an additional RRC reconfiguration message. Furthermore, if Event-2 or Event-7 is fulfilled and the reported new beam(s) are included in the preconfigured TCI state pool, UE can update the activated TCI state list based on the reported new beam(s) after confirmation/acknowledgement from the NW. In this case, the MAC CE activation/deactivation command of legacy beam management procedure is also not needed. This approach (i.e., Alt-1) reduces both signaling overhead and beam application latency in UEI/ED beam management, as we discussed in previous meetings.

Since UE determines the reported new beam(s)/reference signal(s) (RS(s)) through its own beam measurements, it inherently obtains the QCL properties of these reported beam(s)/RS(s) before sending the UE-initiated beam report, particularly for beam(s)/RS(s) satisfying event conditions. After sending the beam report, UE can store the QCL properties (e.g., those of the SSB) associated with the RS(s) reported in the beam report, without the need of additional SSB reception for activating the corresponding beam(s).

From our perspective, the reported new beam(s)/RS(s) in the UEI/ED beam report that satisfy event conditions should be used as soon as possible to update the preconfigured TCI state pool/activated TCI state list and/or switch the serving beam (depending on the event type). Delays in utilization may render the QCL properties stored by the UE obsolete, as channel conditions can change rapidly over time, particularly in FR2 and for high-velocity UEs. This could render the information in the UEI/ED beam report ineffective and should be avoided.

To address this, Alt-1 replaces the time consuming signaling in conventional beam management procedures (such as RRC reconfiguration messages and/or MAC CE activation commands) with just a confirmation indication (e.g., L1 signaling or MAC CE) from the NW. This significantly reduces signaling overhead and beam application latency for updating the preconfigured TCI state pool/activated TCI state list and/or for switching the serving beam. In our view, this method is preferred.

Figure 1: Beam application latency reduction based on confirmation from NW after UEI/ED beam report

Both Alt-2 and Alt-3 aim to reduce beam application latency specifically related to activating new beams based on a legacy TCI state activation command (such as the TCI state activation/deactivation MAC CE). In our understanding, the key difference between Alt-2 and Alt-3 lies in whether/how to convey synchronization information for a new beam reported in the UEI/ED beam report.

In Alt-2, after receiving a TCI state activation command to activate a TCI state(s), if the new beam(s) associated with the TCI state(s) is reported as synchronized in the UEI/ED beam report, the TCI state(s) becomes applicable for DL reception without additional SSB reception. A reported new beam is considered synchronized by the UE if the UE stores the QCL properties associated with the reported new beam(s) after sending the UEI/ED beam report. However, an FFS point still exists in Alt-2: how to inform a reported new beam in a UEI/ED beam report—whether by introducing a one-bit indicator for each reported new beam or by assuming that all reported new beams are synchronized. In our view, introducing a one-bit indicator in the UEI/ED beam report (i.e., for each reported new beam to indicate whether it is synchronized or not) is unnecessary for two reasons. First, it would lead to high signaling overhead if a one-bit indicator is introduced for each reported new beam. Second, all reported new beams should be assumed to be synchronized, as the UE determines the reported new beam(s)/RS(s) based on its beam measurement and can also obtain the QCL properties of the reported new beam(s)/RS(s) before sending the UE-initiated beam report, especially for those reported new beam(s)/RS(s) that meet event criteria.

In Alt-3, after sending a UE-initiated beam report, the UE may store the QCL properties of the SSB associated with the reference signal reported in the beam report. In such a case, upon receiving a subsequent Unified TCI States Activation/Deactivation MAC CE, the UE can activate new beam(s) without additional SSB reception.

If all reported new beams are assumed to be synchronized (i.e., the UE can obtain the QCL properties of the reported new beam(s)/RS(s) before sending the UE-initiated beam report), Alt-2 is equivalent to Alt-3. In essence, both Alt-2 and Alt-3 aim to reduce beam application latency related to activating new beams based on the legacy TCI state activation command (i.e., the TCI state activation/deactivation MAC CE) without additional SSB reception. However, the TCI state activation/deactivation MAC CE is encapsulated within a transport block and transmitted over the PDSCH channel. While the associated HARQ process ensures reliability for the transport block, it may also contribute to increased latency in activating the new beam. Moreover, if the new beam reported in the UEI/ED beam report falls outside the preconfigured TCI state pool for Alt-2 and Alt-3, an additional RRC reconfiguration message will be required, potentially further increasing the latency of new beam application.

To provide a clearer and more accurate comparison, the three alternatives are summarized based on two cases (i.e., new beam(s) being within or outside the preconfigured TCI state pool) in Table 1 below:

Table 1: Comparisons of beam application latency and signaling overhead for the three alternatives

| case {1}/ case {2} | Latency of case {1} that new beam is outside the preconfigured TCI state pool | Signaling overhead of case {1} that new beam is outside preconfigured TCI state pool | Latency of case {2} that new beam is within preconfigured TCI state pool | Signaling overhead of case {2} that new beam is within preconfigured TCI state pool |
| --- | --- | --- | --- | --- |
| Alt-1 | Low | Low | Low [L1 signaling] | Low |
| Alt-2 | High [Legacy signaling] | High [Legacy signaling] | Medium [Legacy signaling] | Low [Legacy signaling] |
| Alt-3 | High [Legacy signaling] | High [Legacy signaling] | Medium [Legacy signaling] | Low [Legacy signaling] |

Based on the above discussions and analysis, we have the following observations and proposals:

Observation 1: The reported new beam(s) in the UEI/ED beam report that satisfy event conditions should be used as soon as possible to update the activated TCI state list/preconfigured TCI state pool as delays in utilization may render the QCL properties stored by the UE obsolete.

Observation 2: Alt-1 can significantly reduce signaling overhead and beam application latency for both cases of reported new beam(s) being within and outside preconfigured TCI state pool.

Observation 3: For Alt-2 and Alt-3, the latency and overhead are higher than those of Alt-1, especially for the case of reported new beam(s) being outside preconfigured TCI state pool.

Proposal 1: All reported new beams should be assumed to be synchronized, as UE determines the reported new beams/RS(s) based on its beam measurements.

Proposal 2: To reduce beam application latency after sending UE-initiated beam report, a unified solution, for both cases of reported new beam(s) being outside and within the preconfigured TCI state pool, is preferred.

Proposal 3: To reduce beam application latency after sending UE-initiated beam report, support Alt-1.

- Alt-1: After confirmation/acknowledgement from NW, apply new beam without RRC configuration signaling or MAC-CE signaling

  - after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report

  - update TCI state(s) with the reported new beam(s)

  - activate new beam(s) without additional SSB reception

### Confirmation procedures for UE-initiated/event-driven beam report

As mentioned earlier, the UEI/ED beam report including the reported new beam(s), together with the direct confirmation from NW (i.e., Alt-1 in Section 2.1.1), can significantly reduce signaling overhead and latency for fast beam switching or update at the UE. For instance, as discussed in Section 2.1.1, if the UEI/ED beam report is used to update new TCI state(s)/beam(s) in the preconfigured TCI state pool (a scenario where TCI state configuration with RRC reconfiguration message is required in legacy beam management), direct confirmation from the NW can reduce beam application latency by about 30 ms due to the removal of RRC reconfiguration message and additional SSB reception [2]. Even if the UEI/ED beam report is only utilized to update the active TCI state list (where no RRC reconfiguration is necessary), direct confirmation from the NW can still reduce latency by over 20 ms due to the removal of additional SSB reception [2].

Regarding the confirmation of the UEI/ED beam report, two options can be considered.

In the first option, the confirmation procedure for the beam failure recovery of the SpCell in the current standard can serve as a reference or baseline. For instance, in current standard, if the contention-free Random-Access Preamble for beam failure recovery request was transmitted by UE for beam failure recovery procedure of SpCell, UE may monitor for a PDCCH transmission, on a dedicated search space indicated by “recoverySearchSpaceId” of the SpCell, identified by the C-RNTI. If notification of a reception of a PDCCH transmission on the dedicated search space indicated by “recoverySearchSpaceId” is received from lower layers on the Serving Cell where the preamble was transmitted and PDCCH transmission is addressed to the C-RNTI, UE may consider the Random-Access procedure/beam failure recovery procedure is successfully completed for the SpCell.

Therefore, the confirmation procedure for UEI/ED beam report can be leveraged based on the confirmation procedure for beam failure recovery procedure for SpCell, in particular for aspects such as, a dedicated search space (i.e., L1 signalling) can be defined and used for the confirmation and indicating which reported new beam(s) can be used for beam switching/update by the UE, as illustrated in Figure 2 below.

Figure 2: Confirmation procedure for the UEI/ED beam report indicating reported beam(s) by UE for beam switching/update

In the second option, a new MAC CE or an enhancement of the legacy TCI state activation/deactivation command MAC CE may be used for confirming the UEI/ED beam report. The MAC CE may indicate one or multiple beams of the reported new beams as a confirmation from the NW. The UE can then use the confirmed reported beam(s) to update the preconfigured TCI state pool, if needed, without requiring an additional RRC reconfiguration message, and also update the activated TCI state list.

Therefore, based on the above discussions and analysis, we have the following proposal:

Proposal 4: The confirmation for the UEI/ED beam report can either leverage the confirmation procedure of the beam failure recovery procedure of the SpCell or utilize a new MAC CE (or an enhancement of the legacy TCI state activation/deactivation command MAC CE).

### Update preconfigured TCI state pool and activated TCI state list

To update the preconfigured TCI state pool and the activated TCI state list, both two scenarios (i.e., Case 1 and Case 2 as aforementioned) need to be considered. A potential solution is that each of the new beam(s) can be preconfigured with an associated TCI state, which may be outside the preconfigured TCI state pool for Case 1 or within the preconfigured TCI state pool for Case 2. In addition, the association between a new beam and a preconfigured TCI state can be updated, such as by a MAC CE.

Case 1: The reported new beam is outside the preconfigured TCI state pool

In this case, both the preconfigured TCI state pool and the activated TCI state list need to be updated by the UE based on the reported new beam(s) after receiving the confirmation from NW, as illustrated in Figure 3 below. The confirmation from NW may explicitly indicate one or multiple beams of the reported new beam(s). The confirmation may also not explicitly indicate any of the reported new beam(s), which implies that all the reported new beam(s) are confirmed. The UE can then update both the preconfigured TCI state pool, and the activated TCI state list based on TCI state(s) associated with the confirmed reported new beam(s).

Figure 3: The reported new beam(s) being outside the preconfigured TCI state pool

Case 2: The reported new beam is within the preconfigured TCI state pool

In this case, only the activated TCI state list needs to be updated by the UE based on the reported new beam(s) after receiving the confirmation from NW, as illustrated in Figure 4 below. As in Case 1, the confirmation from NW may explicitly indicate one or multiple beams of the reported new beam(s). The confirmation may also not explicitly indicate any of the reported new beam(s), which implies that all the reported new beam(s) are confirmed. The UE can then update the activated TCI state list based on TCI state(s) associated with the confirmed reported new beam(s).

Figure 4: The reported new beam(s) being within the preconfigured TCI state pool

Therefore, based on the above discussions and analysis, we have the following proposal:

Proposal 5: Support that each of the new beam(s) can be preconfigured with an associated TCI state and the confirmation from NW can indicate one or multiple beams of the reported new beam(s). The TCI state(s) associated with the confirmed reported new beam(s) are used to update the activated TCI state list and the TCI state pool, if applicable.

## UL signaling content(s) for UEI/ED beam report

Regarding UL signaling content(s) for UEI/ED beam report, the below agreement was reached during RAN1#119 meeting [1]:

Agreement On UE-initiated/event-driven beam reporting, at least one of the following is supported as an extension on L1-RSRP report format depending on Event-2. - Option-1: For each of reported CRI/SSBRI, to introduce additional indication of whether the CRI/SSBRI satisfies the condition of Event-2. - The presence of this field is enabled by RRC with subjective to UE capability. - The presence of this field is only for the case that N > 1 and the time window and M are configured - Option-2: For each of reported CRI/SSBRI, to introduce additional indication of the number of Event-2 instances for the CRI/SSBRI(s) within a time window. - The presence of this field is enabled by RRC with subjective to UE capability. - The presence of this field is only for the case that N > 1 and the time window and M are configured. - Option-3: No further enhancement. Note: As agreed in RAN1#117, at least one of the reported CRI/SSBRI(s) should satisfy the condition of Event-2 FFS: Whether/how to handle the case if UE has not sent any first PUCCH associated for UE-initiated/event-driven beam reporting, for Mode-A.

In last RAN1 meeting, three options (Option-1, Option-2, and Option-3) were proposed and discussed to extend the L1-RSRP report format for UEI/ED beam report, specifically addressing Event-2. From our perspective, enabling the NW to identify which CRI/SSBRI(s) meet the Event-2 criteria would potentially benefit subsequent beam management processes. Under Event-2, UE determines if a CRI/SSBRI(s) meets the event criteria based on a configured number of Event-2 instances within a defined time window, and subsequently reports the corresponding L1-RSRP value(s) associated with the CRI/SSBRI(s). However, based solely on the reported L1-RSRP value(s) (such as whether averaged or one-shot measurement value) for each of the reported CRI/SSBRI, the NW may not reliably determine if the reported CRI/SSBRI meets the Event-2 criteria. Therefore, there is a clear need for further enhancements to accurately indicate if the Event-2 criteria is satisfied by the CRI/SSBRI(s). In that regard, Option-3, which does not address this need, is not preferred. To indicate whether the CRI/SSBRI meets the Event-2 criteria, both Option-1 and Option-2 are feasible alternatives, but they are different in signaling overhead. More specifically, Option-1 proposes adding an indication for each reported CRI/SSBRI to specify whether it meets the Event-2 criteria. For example, a single bit per reported CRI/SSBRI would suffice for this purpose. In contrast, Option-2 introduces an indication of the number of Event-2 instances for each reported CRI/SSBRI within a configured time window. While Option-2 provides significantly more information to the network, it also results in much higher signaling overhead compared to Option-1. Moreover, from a beam management perspective, the additional information provided by Option-2 may not offer significant benefits or be necessary.

Therefore, based on the above discussions and analysis, we have the following proposal:

Proposal 6: For an extension on L1-RSRP report format depending on Event-2, support Option-1.

  - Option-1: For each of reported CRI/SSBRI, to introduce additional indication of whether the CRI/SSBRI satisfies the condition of Event-2.

    - The presence of this field is enabled by RRC with subjective to UE capability.

    - The presence of this field is only for the case that N > 1 and the time window and M are configured

## UL signaling medium/container considering the UE-initiated/event-driven UL transmission for beam report

During RAN1#119 meeting, the below agreements were reached [1], and FL proposal was also discussed [3]:

Agreement Confirm the following working assumption in RAN1#117: On beam report transmission procedure for UE-initiated/event-driven beam reporting - For mode-A, at least support one-bit indication in the first PUCCH channel to request a resource for a second UL channel to carry beam report. - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling. - For mode-B, at least support one-bit indication in the first PUCCH channel to notify a second UL channel to carry beam report. - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling. - FFS: Whether/how to support multi-bit indication in the first PUCCH for mode-A and mode-B, e.g., when multi-event(s) are approved. - FFS: details on the dedicated RRC signaling - Above applies at least for the single CC case. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A, down-select one of the following options in RAN1#120 - Option-1: A CSI trigger state is dedicated to UE-initiated/event-driven beam reporting, i.e., not associated with legacy AP-CSI report configuration. - Option-2: A CSI trigger state can be associated with - only UE-initiated/event-driven beam reporting - or only legacy AP-CSI configuration - or UE-initiated/event-driven beam reporting and legacy AP-CSI configuration FL Proposal 3.1.3: On beam report transmission procedure for UE-initiated/event-driven beam reporting for a CSI report configuration, regarding a prohibit timer for Mode-A and/or Mode-B, further study the following: - Option-1: For both mode-A and mode-B: In the case that triggering-event associated with the CSI report configuration is determined, if the prohibit timer is NOT running, the UE can transmit first PUCCH; - At the first symbol after the end of the second PUSCH transmission, the UE starts the prohibit timer - Option-2: For both mode-A and mode-B: In the case that triggering-event associated with the CSI report configuration is determined, if the prohibit timer is NOT running, the UE can transmit first PUCCH; - At the first symbol after the end of the first PUCCH transmission, the UE starts the prohibit timer - Note: The above does not imply that the prohibit timer is introduced. - If the prohibit timer is running, the first PUCCH is not allowed to be transmitted.

Regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-A and Mode-B, an FFS point remains in the agreement above. That is, whether/how to support multi-bit indication in the first PUCCH for mode-A and mode-B (e.g., when multi-event(s) are supported).

For Mode-A, UE needs to transmit a first PUCCH to request resource(s) for a second UL channel to carry beam report. In addition, the second channel in Step-3 of Mode-A is PUSCH as per agreements in previous RAN1 meetings. In legacy UL transmission for transport block(s), UE needs to send an SR (i.e., via PUCCH) to request UL resource(s) for PUSCH channel to carry the transport block(s). Upon receiving an uplink grant, UE needs to transmit a buffer status report (BSR) MAC CE to inform NW about the status of data buffers (e.g., buffer size) in UE side. After that, UE may detect another uplink grant to schedule resource(s) for PUSCH channel to carry the transport block(s) and then transmits the transport block(s) via the scheduled resource(s) for PUSCH channel.

If the beam report has variable size as discussed in previous RAN1 meetings, the size of the beam report needs to be informed to NW. In that sense, the transmission of BSR MAC CE may still be needed if the first PUCCH is SR like signal as agreed in last RAN1 meeting, which may introduce more latency for beam report and is not preferred. Instead, if the first PUCCH has multiple bits in new UCI type, the first PUCCH can explicitly indicate the exact size of uplink resources required for the beam report. Thus, the transmission of BSR MAC CE may not be needed, which can reduce the latency for beam report.

However, the beam report with a fixed size was already agreed in previous RAN1 meetings. Since the size of beam report can be preconfigured to UE, the size of the beam report doesn’t need to be informed to NW. In that regard, the first PUCCH is just used to specify whether to request resource(s) for the second UL channel to carry beam report. Thus, one bit for the first PUCCH is sufficient to address that need. In addition, multiple events can also be supported in this case, since the second UL channel (to carry beam report) can provide additional information (such as Event ID, etc.) to NW for the multiple events if required.

For Mode-B, UE needs to transmit a first PUCCH notifying whether a second UL channel to carry beam report. The resources (i.e., CG-PUSCH) of the second UL channel are preconfigured to UE. If the first PUCCH is only used to indicate whether the preconfigured resources of the second channel are used or not for beam report, it is also sufficient to use one bit for the first PUCCH. In addition, as aforementioned, the beam report with a fixed size was already agreed in previous RAN1 meetings. In other words, the size of beam report can be preconfigured to UE and thereby the size of the beam report doesn’t need to be informed to NW. Similarly, multiple events can also be supported in this case, since the second UL channel (to carry beam report) can provide additional information (such as Event ID, etc.) to NW for the multiple events if required.

Based on the above discussions and analysis, we have the following obeservation and proposal:

Observation 4: One bit for the first PUCCH is sufficient for both Mode-A and Mode-B, even with multiple events.

Proposal 7: Not support multi-bit indication in the first PUCCH for both mode-A and mode-B.

Regarding triggering procedure in Step-2 of Mode-A, two options (i.e. Option-1 and Option-2) were proposed and discussed in last RAN1 meeting. Further down-select one of the two options is still needed. In Option-1, a CSI trigger state is dedicated to UEI/ED beam report (i.e., not associated with legacy AP-CSI report configuration). In Option-2, on the other hand, a CSI trigger state can be associated with either only UEI/ED beam reporting, or only legacy AP-CSI configuration, or both. For simplification and low signaling overhead in physical layer, we prefer that the CSI trigger state is dedicated to UE-initiated/event-driven beam reporting (i.e., Option-1). Otherwise, as in Option-2, additional signaling is needed to help UE differentiate the transmission requests for UEI/ED beam report and legacy AP-CSI report, such as one additional bit in uplink grant, which could introduce more complexity and signaling overhead in physical layer and is not preferred. Therefore, we have the following proposal:

Proposal 8: It is preferred that a CSI trigger state is dedicated to UE-initiated/event-driven beam reporting (i.e., Option-1).

Regarding a prohibit timer for Mode-A and/or Mode-B, two options (i.e., Option-1 and Option-2) were proposed and discussed in last RAN1 meeting. In Option-1, UE starts the prohibit timer at the first symbol after the end of the second PUSCH transmission for both Mode-A and Mode-B. In Option-2, UE starts the prohibit timer at the first symbol after the end of the first PUCCH transmission for both Mode-A and Mode-B. However, in our understanding, whether the prohibit timer is applied or not is different for Mode-A and Mode-B.

For Mode-A, UE transmits a first PUCCH to request resources for a second UL channel to carry beam report and then dynamically scheduling UL grant is used by NW to schedule uplink resources for the second UL channel. As agreed in last RAN1 meeting, the same concept of Scheduling Request (SR) like signal can be reused for the first PUCCH in Mode-A. Since prohibit timer (e.g., used to reduce unnecessary SR requests [4]) has been defined for SR in legacy system, similar concept can also be applied to the first PUCCH in Mode-A to avoid unnecessary resource requests. Furthermore, UE could stop the prohibit timer if it received a UL grant for the second UL channel to carry beam report.

For Mode-B, the second UL channel carrying the beam report is based on pre-configured resource(s). Even though the concept of legacy SR like signal can be reused for the first PUCCH in Mode-B, the prohibit timer for the first PUCCH may not be needed. Note that it has been agreed to have a one-to-one mapping between the first PUCCH and the second UL channel. Since the second UL channel is pre-configured, its transmission timing is known at UE. Therefore, once UE transmits the first PUCCH, it knows when to send the second UL channel. Before the time for transmitting the second UL channel, there is no need for UE to retransmit the first PUCCH. This rule can be applied without introducing a prohibit timer. After sending the second UL channel, if UE receives a negative acknowledgement, UE should be allowed to retransmit the first PUCCH without further delay. In this case, a prohibit timer is also not needed. Therefore, based on the above discussions and analysis, we have the following proposal:

Proposal 9: Support Option-2, but only for mode-A.

  - Option-2: For both mode-A and mode-B: In the case that triggering-event associated with the CSI report configuration is determined, if the prohibit timer is NOT running, the UE can transmit first PUCCH;

    - At the first symbol after the end of the first PUCCH transmission, the UE starts the prohibit timer

## Trigger-event detection for UEI/ED beam report

On the definition of event to drive UEI/ED beam report, the below agreement was reached during RAN1 #119 meeting:

Agreement On UE-initiated/event-driven beam reporting, for Event 1, down-select at least one among the following options for RS measurement - Option-1: RS resource set for new beam is NOT configured in the CSI reporting configuration, and an explicit RRC selection for scheme-1 and scheme-2 is introduced. - Option-2: RS resource set for new beam is configured in the CSI reporting configuration, and the following implicit manner for enabling one of either scheme-1 or scheme-2 is used: - If the RS(s) for new beam are CSI-RS configured in a CSI-RS resource set configured with repetition, Scheme-1 is enabled; otherwise, Scheme-2 is enabled. Note: For Event-2, we have the following definition for scheme-1 and scheme-2 - Scheme-1: RS for current beam is the QCL RS in the indicated TCI state - Scheme-2: RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state.

For Event-1, two options (Option-1 and Option-2) were proposed and discussed for RS measurement of the current beam. Further selection between these options is needed. In Option-1, the RS resource set for the new beam is not configured in the CSI reporting configuration. In addition, explicit RRC selection for schemes 1 and 2 is introduced. In Option-2, the RS resource set for the new beam is configured in the CSI reporting configuration. It utilizes an implicit method to enable either scheme 1 or scheme 2: Scheme-1 is enabled if the RS(s) for the new beam are CSI-RS configured in a CSI-RS resource set configured with repetition; otherwise, Scheme-2 is enabled. In our view, the purpose of Event-1 for UEI/ED beam report needs reconsideration before proceeding with the selection between these options. Specifically, Event-1 is being redefined based on legacy Event A2 (where the serving cell degrades beyond a threshold) for mobility handover. Typically, Event A2 triggers mobility procedures as a UE moves towards the edge of the serving cell. For instance, the network may configure measurement gaps and inter-frequency or inter-system measurements once Event A2 triggers. Moreover, Event A2 may prompt the UE to perform neighbor cell measurements, which can facilitate a measurement-based mobility procedure.

If Event-1's purpose is solely to pre-alarm the serving beam with poor quality and subsequently configure beam measurement parameters for other events (e.g., Event-2 or Event-7), then the RS resource set for the new beam may not need to be configured in the CSI reporting configuration. Conversely, if Event-1 is intended not only for pre-alarming the serving beam with poor quality but also for monitoring the beam quality of new beams, then the RS resource set for the new beam should be configured in the CSI reporting configuration. Based on our understanding and previous discussions, the primary purpose of Event-1 for UEI/ED beam report is to pre-alarm the serving beam with poor quality and optimize the signaling overhead of measurement RS(s) for other events (such as Event-2 or Event-7). To that end, the RS resource set for the new beam may not need to be configured in the CSI reporting configuration.

Therefore, based on the above discussions and analysis, we have the following observation and proposal:

Observation 5: The primary purpose of Event-1 for UEI/ED beam report is to pre-alarm the serving beam with poor quality and optimize the signaling overhead of measurement RS(s) for other events (such as Event-2 or Event-7).

Proposal 10: Option-1 is preferred.

  - Option-1: RS resource set for new beam is NOT configured in the CSI reporting configuration, and an explicit RRC selection for scheme-1 and scheme-2 is introduced.

# Conclusions

Based on above discussions, we have the following observations and proposals:

Observation 1: The reported new beam(s) in the UEI/ED beam report that satisfy event conditions should be used as soon as possible to update the activated TCI state list/preconfigured TCI state pool as delays in utilization may render the QCL properties stored by the UE obsolete.

Observation 2: Alt-1 can significantly reduce signaling overhead and beam application latency for both cases of reported new beam(s) being within and outside preconfigured TCI state pool.

Observation 3: For Alt-2 and Alt-3, the latency and overhead are higher than those of Alt-1, especially for the case of reported new beam(s) being outside preconfigured TCI state pool.

Proposal 1: All reported new beams should be assumed to be synchronized, as UE determines the reported new beams/RS(s) based on its beam measurements.

Proposal 2: To reduce beam application latency after sending UE-initiated beam report, a unified solution, for both cases of reported new beam(s) being outside and within the preconfigured TCI state pool, is preferred.

Proposal 3: To reduce beam application latency after sending UE-initiated beam report, support Alt-1.

- Alt-1: After confirmation/acknowledgement from NW, apply new beam without RRC configuration signaling or MAC-CE signaling

  - after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report

  - update TCI state(s) with the reported new beam(s)

  - activate new beam(s) without additional SSB reception

Proposal 4: The confirmation for the UEI/ED beam report can either leverage the confirmation procedure of the beam failure recovery procedure of the SpCell or utilize a new MAC CE (or an enhancement of the legacy TCI state activation/deactivation command MAC CE).

Proposal 5: Support that each of the new beam(s) can be preconfigured with an associated TCI state and the confirmation from NW can indicate one or multiple beams of the reported new beam(s). The TCI state(s) associated with the confirmed reported new beam(s) are used to update the activated TCI state list and the TCI state pool, if applicable.

Proposal 6: For an extension on L1-RSRP report format depending on Event-2, support Option-1.

  - Option-1: For each of reported CRI/SSBRI, to introduce additional indication of whether the CRI/SSBRI satisfies the condition of Event-2.

    - The presence of this field is enabled by RRC with subjective to UE capability.

    - The presence of this field is only for the case that N > 1 and the time window and M are configured

Observation 4: One bit for the first PUCCH is sufficient for both Mode-A and Mode-B, even with multiple events.

Proposal 7: Not support multi-bit indication in the first PUCCH for both mode-A and mode-B.

Proposal 8: It is preferred that a CSI trigger state is dedicated to UE-initiated/event-driven beam reporting (i.e., Option-1).

Proposal 9: Support Option-2, but only for mode-A.

  - Option-2: For both mode-A and mode-B: In the case that triggering-event associated with the CSI report configuration is determined, if the prohibit timer is NOT running, the UE can transmit first PUCCH;

    - At the first symbol after the end of the first PUCCH transmission, the UE starts the prohibit timer

Observation 5: The primary purpose of Event-1 for UEI/ED beam report is to pre-alarm the serving beam with poor quality and optimize the signaling overhead of measurement RS(s) for other events (such as Event-2 or Event-7).

Proposal 10: Option-1 is preferred.

  - Option-1: RS resource set for new beam is NOT configured in the CSI reporting configuration, and an explicit RRC selection for scheme-1 and scheme-2 is introduced.

# References

Draft_Minutes_report_RAN1#119_v030.

TS 38.133, NR: Requirements for support of radio resource management (Release 16).

R1-2410872-Moderator Summary #4 on UE-initiated/event-driven beam management- ZTE, RAN1#119.

TS 38.321, NR: Medium Access Control (MAC) protocol specification (Release 18).