3GPP TSG-RAN WG1 Meeting #120	R1-2500081

# Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.9.1

Source:	Huawei, HiSilicon

Title:	Measurements related enhancements for LTM

Document for:	Discussion and Decision

Introduction

In this paper, we continue discussing the CSI-RS based measurement/report for LTM beam management, event triggered L1 measurement reporting, CSI acquisition before and during LTM cell switch and RAN1 related issues in conditional LTM based on previous agreements.

CSI-RS based measurements for LTM beam management

Time domain property of CSI RS transmission

Agreement For gNB scheduled reporting and event triggered reporting - At least periodic CSI-RS is supported for L1-RSRP measurement for candidate cell - FFS: aperiodic and semi-persistent CSI-RS - At least CSI-RS for beam management is supported for L1-RSRP measurement for candidate cell - FFS: CSI-RS for mobility Working Assumption In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective - Send an LS to RAN3 (CC RAN2) to ask for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload). Support of semi-persistent CSI-RS is subject to UE capability.

For candidate cell L1-RSRP measurement, whether to support semi-persistent CSI-RS was discussed in RAN#118 [3]. The main concern from opponents were whether it is feasible to specify signaling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s), and an LS was sent to RAN3 to inquire about this. According to the reply from RAN3 [4], the feasibility of specifying the signaling was confirmed. Hence, semi-persistent CSI-RS can be supported for candidate cell measurement. So, the above working assumption should be confirmed.

Proposal 1: Support to confirm the working assumption.

- In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting.

- Support of semi-persistent CSI-RS is subject to UE capability.

To allow UE to acquire more prompt measurement result with less overhead of CSI-RS transmission, we think the measurement of aperiodic CSI-RSs should be supported from RAN1 perspective. Similar as that for semi-persistent CSI-RS, an LS can be sent to RAN3 asking for the feasibility of the coordination between serving cell and candidate cells in inter-DU and inter-CU scenarios.

Proposal 2: Aperiodic CSI-RS can be supported for L1-RSRP measurement for candidate cell(s) from RAN1 perspective. Send LS to RAN3 asking for feasibility of coordination between serving cell and candidate cells in inter-DU and inter-CU scenarios.

CSI report configuration framework

In Rel-18 LTM, to support SSB-based L1 measurement on candidate cells, SSB resources are configured per candidate cell. In particular, ltm-SSB-Config-r18 is configured in LTM-Candidate-r18 that includes RRC configuration for one candidate cell. Although CSI-RS based measurement for candidate cell is not supported, a set of CSI-RS Resource (i.e. ltm-NZP-CSI-RS-ResourceToAddModList-r18) and a set of CSI-RS Resource Set (i.e. ltm-NZP-CSI-RS-ResourceSetToAddModList-r18) are configured in the LTM-TCI-Info-r18 for a candidate cell which are only used as QCL RS of an LTM TCI state.

In Rel-19 LTM, to support CSI-RS based L1 measurement for a candidate cell, we think CSI-RS resources can be defined per candidate cell similar as SSB configuration. For example, the configuration of CSI-RS resource and CSI-RS resource set can be under LTM-Candidate-r18. To avoid unnecessary efforts, the similar IE structure as ltm-NZP-CSI-RS-ResourceToAddModList-r18 and ltm-NZP-CSI-RS-ResourceSetToAddModList-r18 can be reused to the configuration of CSI-RS resource and CSI-RS resource set. The field repetition and trs-Info can be added in the configuration of CSI-RS Resource Set to differentiate CSI-RS for BM and TRS for LTM candidate cell(s).

Proposal 3: For CSI-RS based L1 measurement for Rel-19 LTM, the configuration of CSI-RS Resource and CSI-RS Resource Set can be configured under candidate cell’s RRC configuration.

In Rel-18 LTM, it was supported to include measurement results derived from SSB of both serving cell and candidate cell(s) in the same reporting. This design can help the NW to make a quick and fair comparison on beam quality across multiple cells. In order to support the reporting scheme, an LTM-CSI-ReportConfig-r18 is associated with an LTM-CSI-SSB-ResourceSet-r18, which consists of an ltm-CSI-SSB-ResourceList-r18 including a sequence of SSB index and a sequence of LTM-CandidateId-r18 correspondingly.

Similarly, it’s better that CSI-RS measurement results from multiple cells can be included in the same UCI. However, in current CSI-RS configuration for serving cell, the function of CSI-RS (e.g. “repetition” configured for BM, “trs-Info” configured for TRS) is configured per CSI-RS resource set rather than per CSI-RS resource. And the offset between DCI triggering aperiodic CSI-RS resources and CSI-RS resources transmission is also configured for CSI-RS resource set. If an LTM CSI report configuration is directly associated with CSI-RS resource(s), these parameters should be additionally introduced in the IE of NZP-CSI-RS-Resource. In addition, the CSI-RS resources in a CSI-RS resource set should have the same periodicity, the same starting RB and number of RBs, the same density, nrofPorts, and cdm-type. It is quite restrictive to configure common values for these parameters of CSI-RS resources in different candidate cells. Therefore, we prefer an LTM CSI report configuration is associated with multiple NZP-CSI-RS resource sets from multiple candidate cells, where each NZP-CSI-RS resource set corresponds to one candidate cell. In particular, an LTM CSI report configuration can be associated with an ltm-NZP-CSI-RS-ResourceSetList, which consists of a sequence of NZP-CSI-RS-ResourceSet and a sequence of LTM-CandidateId correspondingly.

Proposal 4: An LTM CSI report configuration can be associated with multiple NZP-CSI-RS resource sets from multiple candidate cells.

According to above CSI report configuration framework, CRI reported in CSI report should be redefined. UE can derive CRI of individual NZP-CSI-RS-Resource in the ltm-NZP-CSI-RS-ResourceSetList from its entry index in the NZP-CSI-RS-ResourceSet and entry index in the ltm-NZP-CSI-RS-ResourceSetList. For example, the CRI for the kth NZP-CSI-RS-Resource in the mth NZP-CSI-RS-ResourceSet in an ltm-NZP-CSI-RS-ResourceSetList is CRI=s=0m-1Ks+k-1,where Ks is the number of NZP-CSI-RS-Resource in the sth NZP-CSI-RS-ResourceSet.

Proposal 5: CRI of individual NZP-CSI-RS-Resource reported in CSI report can be derived from the following formula.

CRI=s=0m-1Ks+k-1

where

- m is the entry index of NZP-CSI-RS-ResourceSet in the ltm-NZP-CSI-RS-ResourceSetList,

- k is the entry index of NZP-CSI-RS-Resource in the mth NZP-CSI-RS-ResourceSet,

- Ks is the number of NZP-CSI-RS-Resource in the sth NZP-CSI-RS-ResourceSet.

In legacy CSI framework, a CSI-RS resource set configured with “repetition” is used for beam management. When “repetition” is set to “on”, UE can assume that CSI-RSs in this set are transmitted with the same downlink spatial domain transmission filter. It is usually used for quickly Rx beam refinement. In LTM, we think Rx beam refinement for multiple candidate cells may cause large measurement overhead and a waste of RS resource. Hence, we support to always configure the field of “repetition” as “off”, if applicable, for NZP-CSI-RS resource set of candidate cell(s).

Proposal 6: Support to always configure the field of “repetition” as “off” for NZP-CSI-RS resource set of candidate cell(s).

CSI acquisition on candidate cell(s)

CSI acquisition framework

Working Assumption As baseline, CSI-RS measurement and CSI reporting operations are performed after reception of LTM CSC MAC CE. - The report is sent directly to target cell - Introduce UE capability for CSI-RS measurement can start before reception of LTM CSC MAC CE - Other than UE capability, strive for no additional spec impact compared to the baseline (only one triggering mechanism will be specified)

In RAN1#119 [1], the timeline of CSI-RS measurement and report for candidate cell(s) has been discussed. A working assumption was achieved that CSI measurement before LTM CSC MAC CE reception and CSI measurement after LTM CSC MAC CE reception are both supported, with CSI measurement before LTM CSC MAC CE subject to additional UE capability. To our understanding, the working assumption provides a good tradeoff between transmission performance and measurement complexity for UE, and a good flexibility for gNB configuration. For advanced UE, it can conduct CSI measurement before LTM CSC MAC CE reception to avoid latency of CSI acquisition and thus ensure the transmission performance after cell switch. For UE with low capability, it can conduct CSI measurement immediately after LTM CSC MAC CE reception to avoid additional processing capability. In addition to CSI measurement, CSI report sent directly to target cell can avoid unnecessary high layer exchange between source cell and target cell. Therefore, we support to confirm the working assumption.

Proposal 7: Support to confirm the following working assumption:

As baseline, CSI-RS measurement and CSI reporting operations are performed after reception of LTM CSC MAC CE.

- The report is sent directly to target cell

- Introduce UE capability for CSI-RS measurement can start before reception of LTM CSC MAC CE

- Other than UE capability, strive for no additional spec impact compared to the baseline (only one triggering mechanism will be specified)

CSI-RS resource transmission for CSI acquisition of candidate cell

- CSI-RS resource configuration for CSI acquisition of candidate cell

Considering CSI-RS measurement for BM for candidate cell is supported, RAN1 should strive to design a unified CSI-RS resource configuration for BM and CSI acquisition. Similar to CSI-RS resource configuration as discussed in section 2, the configuration of CSI-RS resource and CSI-RS resource set for CSI acquisition can be under candidate cell’s RRC configuration (i.e. LTM-Candidate-r18). Furthermore, the CSI-RS resource set for CSI acquisition should not be configured with “repetition” or “trs-info” which are used to BM and TRS respectively. Different from BM, CSI comparison among candidate cells seems unnecessary, hence, an LTM CSI report associate with one CSI-RS resource set from one candidate cell for CSI acquisition would be more flexible.

Proposal 8: Support unified CSI-RS resource configuration design for BM and CSI acquisition for LTM candidate cells with the restriction that an LTM CSI report can be associate with only one CSI-RS resource set from one candidate cell for CSI acquisition.

- Time domain property of CSI RS transmission

As discussed in section 2.1, periodic CSI-RS and semi-persistent CSI-RS are supported for L1 measurement for BM. Aperiodic CSI RS for BM is still under discussion. The concern on supporting semi-persistent and aperiodic CSI-RS for BM is the feasibility of coordination between serving cell and candidate cell. Similar situation happens to CSI-RS for CSI acquisition. Thus, we propose to support periodic and semi-persistent CSI-RS for CSI acquisition. Whether to support aperiodic CSI-RS for CSI acquisition can be discussed together with CSI-RS for BM.

Proposal 9: At least periodic and semi-persistent CSI-RS transmission for CSI acquisition should be supported from RAN1 perspective. Whether to support Aperiodic CSI-RS for CSI acquisition could be discussed together with CSI-RS for BM.

CSI reporting for CSI acquisition of candidate cell

- Time domain property of CSI reporting

To support CSI reporting on target cell, aperiodic CSI reporting is adequate for CSI acquisition during cell switch procedure. UE doesn’t need to transmit CSI reporting periodically since CSI acquisition for candidate cell is to improve the performance of data transmission in the initial stage after cell switch. After UE sends the RRC reconfiguration complete message, the legacy serving cell CSI report can be used for CSI acquisition.

Proposal 10: Only support aperiodic CSI reporting for CSI acquisition to target cell after reception of LTM CSC MAC CE .

- UL channel of CSI reporting on candidate cell

In legacy CSI reporting framework, the CSI information is carried in the UCI multiplexed the on PUCCH or PUSCH. The UCI format is also defined in TS38.212. To minimize the standard effort, we support UCI as the container of CSI report to target cell.

Proposal 11: Support UCI-based CSI reporting to target cell after reception of LTM CSC MAC CE.

Furthermore, we think the UL channel carrying CSI report in target cell can be discussed based on the following options.

- Alt-1: UCI-based CSI reporting is carried in PUSCH scheduled by RAR UL grant if 4-step CFRA based LTM cell switch is performed.

- Alt-2: UCI-based CSI reporting is carried in PUSCH of MsgA if 2-step CFRA based LTM cell switch is performed.

- Alt-3: UCI-based CSI reporting is carried in the first UL channel after RA procedure is successfully completed if CBRA-based LTM cell switch is performed.

- Alt-4: UCI-based CSI reporting is carried in the first UL channel (either CG-based or DG-based PUSCH) if RACH-less LTM cell switch is performed.

- Alt-5: UCI-based CSI reporting is carried in CG-PUSCH dedicated for the CSI reporting after reception of LTM CSC MAC CE.

- Alt-6: UCI-based CSI reporting is carried in UL channel scheduled by LTM CSC MAC CE.

For Alt-1 and Alt-2, CSI report can be transmitted in RACH procedure, which can help gNB acquire CSI information immediately. In addition, it does not introduce any extra UL resource. Hence, we propose that CSI report can be reported on PUSCH during CFRA procedure.

Proposal 12: For CFRA based LTM cell switch, CSI report can be reported on PUSCH scheduled by RAR UL grant (Alt 1) or PUSCH of MsgA to target cell if configured by NW(Alt 2).

For CBRA-based LTM cell switch, PUSCH scheduled by RAR UL grant or PUSCH of MsgA may collide with other UEs. Hence, PUSCH during CBRA procedure is not reliable to carry CSI report of target cell. In this case, Alt 3 that UCI-based CSI reporting can be carried in the first UL channel (msg 5) after RA procedure is successfully completed, which has higher reliability. Meanwhile, such UL channel carry the RRC reconfiguration complete message, which means no extra UL channel is introduced.

Proposal 13: For CBRA based LTM cell switch, CSI reporting is carried in the first UL channel after RA procedure is successfully completed to target cell (Alt 3).

For Alt-4, it is for RACH-less cell switch. In this case, the first UL channel (either CG-based or DG-based PUSCH) should carry the RRC reconfiguration complete message. Hence, we think it can be used to carry the CSI report for reducing UL overhead.

Proposal 14: For RACH-less LTM cell switch, CSI reporting is carried in the first UL channel (either CG-based or DG-based PUSCH) (Alt 4).

For both Alt-5 and Alt-6, the UL channel are dedicated for CSI reporting, which may introduce additional UL overhead for reporting and have a longer latency than Alt-4. It also results more standard effort to specify timeline and configuration of the dedicated resource. Hence, we think Alt-5 and Alt-6 can be precluded.

Considering the CSI report is reported together with PUSCH for msg 3/msg A/msg 5, the existing UCI multiplexing procedure defined in TS38.212/213 can be reused, which can minimize the standard effort. In addition, the reporting mechanism can also be easily extended to other existing mobility schemes other than LTM (e.g. L3 mobility), because no LTM specific configuration of UL resource for CSI report is required.

Proposal 15: The existing UCI multiplexing procedure can be reused for CSI reporting during the LTM cell switch, which can be further extended to other existing mobility schemes in the future.

- Reporting content of CSI acquisition of candidate cell

Regarding the CSI reporting quantities, there were a lot of discussion during past releases in MIMO topic. At least cri-RI-PMI-CQI could be supported, which can be used in most scenarios. As for the PMI and CQI, two reporting granularities can be configured through reportFreqConfiguration in frequency domain, i.e. subband and wideband. The subband measurement and reporting provides more precise channel state information in frequency selective channel, but it requires a larger reporting overhead. Considering the tradeoff between overhead and complexity, the wideband measurement and report are sufficient for mobility purpose.

According to TS 38.214 [5], the CSI-ReportConfig can contain CodebookConfig when PMI is reported, which contains configuration parameters for Type-I, Type II, Enhanced Type II CSI, Further Enhanced Type II Port Selection, Enhanced Type II for coherent joint transmission (CJT), Further Enhanced Type II Port Selection for CJT, Enhanced Type II for predicted PMI, or Further Enhanced Type II Port Selection for predicted PMI including codebook subset restriction when applicable, and configurations of group-based reporting. Many types of CSI reporting are designed for different MIMO transmission scenarios. In general, Type-I and Type II codebook are basic choices for most scenarios. Compare to the Type-I, Type II codebook provides more accurate CSI reporting with the larger overhead and higher processing complexity in UE. For LTM, UE needs to measure the CSI in addition to its serving cell, it may be challenge considering the complexity. Meanwhile, if the CSI reporting is sent after cell switch on reserved UL resources, Type II codebook reporting requires candidate cell to reserve more UL resources. Thus, Type-I codebook based measurement and reporting is sufficient.

Proposal 16: For the CSI acquisition before or during the LTM cell switch, at least cri-RI-PMI-CQI with wideband CQI/PMI and Type-I codebook should be supported.

Event triggered L1 measurement reporting

Measured RS for event detection

RAN2#126 Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. RAN2#128 Either CSI-RS or SSB could be configured as candidate beam and the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type, i.e. the RS for current beam of serving cell is same as or QCLed with the QCL RS of the indicated TCI state, according to what is agreed in RAN1.

In RAN2#126 [6], Event LTM2~5 are agreed as L1 LTM measurement events. For Event LTM3 and event LTM5, the measurement RS of the serving cell beam is determined based on the candidate beam explicitly configured by gNB to ensure same RS type [7]. While there is no comparison between serving cell and candidate cell for Event LTM2 so that the RS type of serving cell cannot be determined by candidate cells. Therefore, we propose that the RS type of serving cell for Event LTM2 is explicitly configured by gNB.

Proposal 17: For Event LTM2, the RS type of serving cell should be explicitly configured by gNB.

Filtering of L1 measurement results

L3 filtering has been specified in legacy L3 measurement and allows network to configure some coefficient for UE to perform weighted averaging across multiple measurement samples in time/spatial domain. It helps alleviate the ping-pong effect in handover, which cause large signaling overhead. For the L1 measurement, the most important advantage is to acquire measurement report timely. If the L1 filtering is specified and configured, UE may need more time, e.g. averaging across multiple transmission of periodic reference signal, to derive L1 measurement result. For the ping-pong effect, it may be an issue for reporting to triggering the mobility, however it will not have impact on just triggering a report. Thus, the L1 filtering should be left to UE implementation and not to be specified.

Proposal 18: The L1 filtering should be left to UE implementation and not to be specified.

Summary and conclusion

In this contribution, we have following proposals:

Proposal 1: Support to confirm the working assumption.

- In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting.

- Support of semi-persistent CSI-RS is subject to UE capability.

Proposal 2: Aperiodic CSI-RS can be supported for L1-RSRP measurement for candidate cell(s) from RAN1 perspective. Send LS to RAN3 asking for feasibility of coordination between serving cell and candidate cells in inter-DU and inter-CU scenarios.

Proposal 3: For CSI-RS based L1 measurement for Rel-19 LTM, the configuration of CSI-RS Resource and CSI-RS Resource Set can be configured under candidate cell’s RRC configuration.

Proposal 4: An LTM CSI report configuration can be associated with multiple NZP-CSI-RS resource sets from multiple candidate cells.

Proposal 5: CRI of individual NZP-CSI-RS-Resource reported in CSI report can be derived from the following formula.

CRI=s=0m-1Ks+k-1

where

- m is the entry index of NZP-CSI-RS-ResourceSet in the ltm-NZP-CSI-RS-ResourceSetList,

- k is the entry index of NZP-CSI-RS-Resource in the mth NZP-CSI-RS-ResourceSet,

- Ks is the number of NZP-CSI-RS-Resource in the sth NZP-CSI-RS-ResourceSet.

Proposal 6: Support to always configure the field of “repetition” as “off” for NZP-CSI-RS resource set of candidate cell(s).

Proposal 7: Support to confirm the following working assumption:

As baseline, CSI-RS measurement and CSI reporting operations are performed after reception of LTM CSC MAC CE.

- The report is sent directly to target cell

- Introduce UE capability for CSI-RS measurement can start before reception of LTM CSC MAC CE

- Other than UE capability, strive for no additional spec impact compared to the baseline (only one triggering mechanism will be specified)

Proposal 8: Support unified CSI-RS resource configuration design for BM and CSI acquisition for LTM candidate cells with the restriction that an LTM CSI report can be associate with only one CSI-RS resource set from one candidate cell for CSI acquisition.

Proposal 9: At least periodic and semi-persistent CSI-RS transmission for CSI acquisition should be supported from RAN1 perspective. Whether to support Aperiodic CSI-RS for CSI acquisition could be discussed together with CSI-RS for BM.

Proposal 10: Only support aperiodic CSI reporting for CSI acquisition to target cell after reception of LTM CSC MAC CE .

Proposal 11: Support UCI-based CSI reporting to target cell after reception of LTM CSC MAC CE.

Proposal 12: For CFRA based LTM cell switch, CSI report can be reported on PUSCH scheduled by RAR UL grant (Alt 1) or PUSCH of MsgA to target cell if configured by NW(Alt 2).

Proposal 13: For CBRA based LTM cell switch, CSI reporting is carried in the first UL channel after RA procedure is successfully completed to target cell (Alt 3).

Proposal 14: For RACH-less LTM cell switch, CSI reporting is carried in the first UL channel (either CG-based or DG-based PUSCH) (Alt 4).

Proposal 15: The existing UCI multiplexing procedure can be reused for CSI reporting during the LTM cell switch, which can be further extended to other existing mobility schemes in the future.

Proposal 16: For the CSI acquisition before or during the LTM cell switch, at least cri-RI-PMI-CQI with wideband CQI/PMI and Type-I codebook should be supported.

Proposal 17: For Event LTM2, the RS type of serving cell should be explicitly configured by gNB.

Proposal 18: The L1 filtering should be left to UE implementation and not to be specified.

References

[1] RAN1#119, Chairman, November 18-22, 2024.

[2] RAN1#118bis, Chairman, October 14-18, 2024.

[3] RAN1#118, Chairman, August 19-23, 2024.

[4] R3-247822 Reply LS for LS on the support of semi-persistent CSI-RS resource for LTM candidate cells

[5] 3GPP TS 38.214 version 18.3.0 Release 18

[6] RAN2#126

[7] RAN2#128