3GPP TSG RAN WG1 #120  	          	              R1-2500131

Athens, Greece, February 17th–21st, 2025

Source: 	ZTE Corporation, Sanechips

Title:	Discussion on LP-WUS design

Agenda item: 	9.6.1

Document for:	Discussion and decision

Introduction

In RAN #105 meeting, a revised WID on low-power Wake-up Signal and Receiver for NR was approved in [1]. In RAN1 #119 meeting, the following agreements were made in [2].

Agreement Update the existing agreement as below In case of overlaid OFDM sequence carrying information, support option 2: - Option 2: One sequence is selected from multiple candidates overlaid OFDM sequences on each OOK ‘ON’ symbol, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). Consider the following two sub-options for potential down-selection. - Option 2-1: The overlaid OFDM sequence(s) carry part of information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by OFDM sequence(s) and location of the OFDM sequence(s)/OOK ‘ON’ symbols. - Option 2-2: The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the overlaid OFDM sequence(s) - FFS the how the information bits are carried by the overlaid OFDM sequence(s) Note: the overlaid OFDM sequence in each OOK ‘ON’ symbol can be different according to information bits to be carried by the overlaid OFDM sequence within the LP-WUS. Agreement For the overlaid OFDM sequence in time domain, the sequence is generated based on , - M=1, is given by the largest prime number such that,is the overlaid OFDM sequence length. - The base overlaid sequence is generated by extension of , - With CS(s) applied to the base overlaid OFDM sequence if any:denotes the potential cyclic shift (s) ,, - M=2, 4, is down-selected from the following: - Alt1: is given by the largest prime number such that,is the overlaid OFDM sequence length. - The base overlaid sequenceis generated by extension of , - With CS(s) applied to the base overlaid OFDM sequence if any:denotes the potential cyclic shift (s) , - Note it doesn’t preclude any pulse shaping scheme if any. - Alt2:is given by the largest prime number such that, is the overlaid OFDM sequence length. - The base overlaid sequenceis generated by inserting zeros before and/or after . The total number of zeros is. For example,, - With CS(s) applied to the base overlaid OFDM sequence if any: denotes the potential cyclic shift (s) For example,, where, - FFS the value of G, with G>0. - FFS value(s) of root q and/or CS for generating the candidate sequences if applied - FFS on whether changes are needed to handle intra-/inter-cell interference - Note: the overlaid OFDM sequence in time domain is based on potential modification to ZC sequence as listed above. - Above overrides any previous RAN1 agreement / working assumption Agreement At least for M>1, for the overlaid OFDM sequence in time domain, down-select the sequence length from the following: - Alt1:, with - FFS how to map the generated overlaid OFDM sequence to the frequency domain - Alt2: - Note: X is the number of RBs of LP-WUS/LP-SS bandwidth (blanked guard RBs are not included) Agreement CS(s) and/or root(s) used for overlaid OFDM sequence in the time domain are derived from RRC signalling: - FFS: the set of values of CS and root for configuration - FFS: details of the RRC signaling Agreement For the M value for LP-WUS and LP-SS, down-select one alternative from the following in RAN1#120: - Alt1: The M values for LP-WUS and LP-SS are always same. - Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS. - Alt3: The M values for LP-WUS and LP-SS can be configured to be same or different. Agreement Regarding the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell: - support maximum 4 candidates overlaid sequences for M=4 Agreement Regarding whether to support additional sync signal to LP-SS, down-selection from the following: - Option 1A: No additional sync signal, LP-SS periodicity =320ms - Additionally support at least one of [80ms,160ms] - FFS: whether different values can be applicable for different M values - Option 1B: No additional sync signal, LP-SS periodicity =320ms - No additional support of other periodicities - Option 2: With additional sync signal, LP-SS periodicity =320ms - FFS additionally support other LP-SS periodicities - FFS details on additional sync signal - FFS additional sync signal is configurable and/or conditionally present Agreement For RRC idle/inactive state, for the case where associated CD-SSB and initial DL BWP have the same SCS at least when the associated CD-SSB and LP-WUS are on the same carrier - The single SCS for LP-WUS/LP-SS is same as the associated CD-SSB - FFS: Case where associated CD-SSB and initial DL BWP have different SCSs - FFS: Which initial BWP for RedCap R1-2410925 Template for collecting LP-SS binary sequence results Moderator (vivo) Agreement For collecting LP-SS binary sequence results - Endorse the template for collecting LP-SS binary sequence results in R1-2410925 - Email discussion for collecting LP-SS binary sequences by 10th Jan. - Please include your cross-checking results (e.g., a separate excel sheet) in the t-doc for #120.

In this contribution, issues on waveform design and signal structure for LP-WUS/LP-SS are discussed.

Waveform generation

M values for OOK-4

In RAN1#118 meeting, RAN1 made following agreement.

Agreement For OOK-4 with M >1, support M=2 & M=4 (working assumption) for LP-WUS. - M=4 for 15KHz SCS - M=4 for 30KHz SCS (working assumption) - FFS M=1 for OOK-4

It is recommended that the same solution is supported for both 15kHz SCS and 30kHz SCS. With the same allocated resources and smaller residual time error, such as time error <=1us, LP-WUS with OOK-4 with M=4 can achieve the same or better detection performance than OOK-4 with M=1,2.

For the case of larger residual time error, LP-SS or additional sync signal can be used to ensure the residual time error before LP-WUS detection smaller than 1us. Moreover, as description in section 5.2, inserting Gaps at the beginning and the ending of each OOK symbol of OOK-4, can also be used to solve the issue of LP-WUS detection performance degradation caused by larger residual time error.

Therefore, multiple solutions can be used to solve the issue of time error for OOK-4. It is recommended to support M =4 for both 15kHz SCS and 30kHz SCS.

Proposal 1: Confirm the working assumption in below agreement:

Agreement For OOK-4 with M >1, support M=2 & M=4 (working assumption) for LP-WUS. - M=4 for 15KHz SCS - M=4 for 30KHz SCS (working assumption) FFS M=1 for OOK-4

In RAN1 #119 meeting, RAN1 made following agreement on the M value for LP-WUS and LP-SS.

Agreement For the M value for LP-WUS and LP-SS, down-select one alternative from the following in RAN1#120: - Alt1: The M values for LP-WUS and LP-SS are always same. - Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS. - Alt3: The M values for LP-WUS and LP-SS can be configured to be same or different.

For Alt1, it restricts the gNB configuration which may cause resource wasting. For example, for LP-SS and M=1, using LP-WUS and M=1 would occupy more resources than M=2.

For Alt2, M value for LP-WUS is not larger than that of LP-SS which may benefit to the LP-WUS reception due to the timing drifting. However, when the M value is the same, it is still possible that the LP-WUS is out of sync after LP-SS for a time duration. Moreover, LP-WUS using smaller M values bring more resources occupation.

For Alt3, WUR has the stronger capability than Ambient-IoT UE, and there is no issue to support multiple M values. Moreover, different M value does not impact implementation much. The WUR side still can use similar or same decoding algorithm. And Alt3 is more beneficial for the NW to configure the M value and allocate the resources.

Therefore, it is not necessary to make a relationship between the M values of LP-WUS and LP-SS. Thus, Alt3 is supported.

Proposal 2: For the M value for LP-WUS and LP-SS, it should be configured by gNB respectively.

OOK1 and OOK4 with M=1 waveform generation

In RAN1#118b meeting, the following working assumption on overlaid OFDM sequence(s) of LP-WUS was made.

Working Assumption For overlaid OFDM sequences for LP-WUS in time or frequency domain, for OOK-1 and OOK-4 M=1 - Option 1-1(compromised): overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing for OOK-4 with M=1. - ZC sequence in time domainDFT/LSIFFT to result in a frequency domain sequence that is ZC sequence or a sequence that is close to ZC sequence, no additional operation - It doesn’t preclude additional operation for OOK-4 with M>1. - DFT size is 2^n Note1: OOK-1 is considered as specific case of OOK-4 with M=1.

Considering that ZC sequence after DFT operation is still a ZC sequence, in order to generate sequence in frequency domain for OOK-1 and OOK-4 with M=1, ZC sequence with DFT operation can be considered.

Additionally, using 2^n DFT size is beneficial for the NW since it is easiser to be implemented based on 2^n DFT size. Moverover, if the DFT size is not 2^n, e.g., 132 or 132, the simples received in time domain is not matched with the local oscillator, 7.68MHz, which may bring additional processing complexity due to the clock counting. Therefore, the working assumption could be confirmed.

Proposal 3: Confirm the working assumption on overlaid OFDM sequence(s) of LP-WUS.

Working Assumption For overlaid OFDM sequences for LP-WUS in time or frequency domain, for OOK-1 and OOK-4 M=1 - Option 1-1(compromised): overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing for OOK-4 with M=1. - ZC sequence in time domainDFT/LSIFFT to result in a frequency domain sequence that is ZC sequence or a sequence that is close to ZC sequence, no additional operation - It doesn’t preclude additional operation for OOK-4 with M>1. - DFT size is 2^n Note1: OOK-1 is considered as specific case of OOK-4 with M=1.

SCS and CP

In RAN1#119 meeting, the following agreement on SCS of LP-WUS and LP-SS was made.

Agreement For RRC idle/inactive state, for the case where associated CD-SSB and initial DL BWP have the same SCS at least when the associated CD-SSB and LP-WUS are on the same carrier - The single SCS for LP-WUS/LP-SS is same as the associated CD-SSB - FFS: Case where associated CD-SSB and initial DL BWP have different SCSs - - FFS: Which initial BWP for RedCap

For OOK based LP-WUR, the SCS is better to be the same as initial DL BWP, since the LP-SS/LP-WUS could be used for channel estimation for the MR, and there is no need for the OOK based WUR to keep the same SCS with CD-SSB.

For OFDM sequence based LP-WUR, it depends on the LP-WUR whether it can support both SCS like NR UE. If so, to keep align with OOK based WUR, the SCS could be the same as initial DL BWP. If the OFDM sequence based LP-WUR can not support different SCS, then the OFDM sequence based LP-WUR may support LP-WUS/LP-SS with the same SCS with CD-SSB, since the OFDM sequence based LP-WUR needs to monitor SSS for measurement.

Observation 1: For OOK based LP-WUR, the SCS of LP-WUS/LP-SS could be the same as initial DL BWP, since OOK based WUR is not needed to monitor SSS and it may be beneficial for the paging reception.

Observation 2: For OFDM sequence based LP-WUR, if it can support different SCS like NR UE, the SCS of LP-WUS/LP-SS could be the same as initial DL BWP; otherwise, the SCS of LP-WUS/LP-SS is better to be the same as CD-SSB.

Additionally, regarding the CP length, the definition for CP is preferred not to be changed. If the SCS of LP-WUS/LP-SS is the same as CD-SSB, then normal CP is used. If the SCS of LP-WUS/LP-SS is the same as initial DL BWP, we may need to discuss whether the extended CP could be supported.

LP-SS design

Principle for binary sequence selection

- In order to maintain same powering boosting level of multiple OFDM symbols allocated for LP-SS, same number of OOK-ON symbols within one OFDM symbol should be ensured. Otherwise, gNB is required to do separate power control implementation for each OFDM symbol and it is too complicated for gNB.

For OOK based LR, LP-RSRP/LP-RSRQ should be calculated based on LP-SS sequence. In the corresponding calculation process, OOK-OFF symbols should be used to evaluate the received power of Interference+Noise, OOK-ON symbols should be used to evaluate the received power of the received signal including LP-SS and Interference + Noise. In this calculation process, it is preferred to configure equal number of OOK-ON symbols and OOK-OFF symbols to ensure the balanced power and obtain better LP-RSRP/LP-RSRQ performance. Moreover, to configure equal number of OOK-ON symbols and OOK-OFF symbols also has benefits of achieving ideal AGC gain.

- Observation 3: For LP-SS, to configure equal number of OOK-ON symbols (carrying bit “1”) and OOK-OFF symbols (carrying bit “0”) within M OOK symbols for one OFDM symbol has benefits of

- Ensuring the power control implementation at gNB side unchanged;

- Improving LP-RSRP/LP-RSRQ calculation accuracy;

- Ensuring the AGC performance;

- Proposal 4:Candidate LP-SS binary sequences should satisfy that

- For OOK-4 with M>1, equal number of OOK-ON symbols (carrying bit “1”) and OOK-OFF symbols (carrying bit “0”) within M OOK symbols of one OFDM symbol for OOK-4 with M>1.

- For OOK-4 with M=1, equal number of OOK-ON symbols (carrying bit “1”) and OOK-OFF symbols (carrying bit “0”) within the binary sequence.

- In [3], all candidate binary sequence for LP-SS are selected by companies. In order to facilitate cross-check of the binary sequences by each company,some principles for cross-check should be decided. For example,

- For OOK-4 with a M value, under the premise of meeting the time offset requirement, the shortest binary sequence should be selected as LP-SS. It has benefits of resource overhead reduction for network.

For OOK-4 with a M value and a given length of binary sequence, there may have multiple LP-SS detection methods. For a given LP-SS detection method, the sequences with best detection performance are preferred. If there are some binary sequences that have best detection performance under different LP-SS detection method, these binary sequences are preferred as LP-SS. If the number of the preferred binary sequences are less than 4, a tradeoff between binary sequences with different detection methods should be considered for sequence selection.

Proposal 5: For candidate binary sequences cross-check,

- For OOK-4 with a M value, under the premise of meeting the time offset requirement, the shortest binary sequence should be selected as LP-SS.

- For OOK-4 with a M value and a given length of binary sequence,

  - Common candidate binary sequences(if any) that have best detection performance under different LP-SS detection method are preferred as LP-SS.

    - If the number of the common candidate binary sequences are less than 4, a tradeoff between binary sequences with different detection methods should be considered for sequence selection.

Evaluation results for TO correction

For OOK-4 with M=1

In [3], all candidate binary sequence for LP-SS are included. The following simulation results are based on the candidate binary sequences under the principle of balanced OOK-ON and OOK-OFF symbols. Moreover, both  mismatch/bipolar detection and unipolar detection are evaluated.

- 4-length binary sequence

For OOK-4 with M=1, all candidate 4-length binary sequence under the principle of balanced OOK-ON and OOK-OFF symbols and the corresponding TO correction results are listed in Table 1.

Table 1. Summary of TO correction results for OOK-4 with M=1 and 4-length binary sequences

| Sequence index | Candidate sequences | TO correction results by mismatch/bipolar detection (us) | TO correction results by unipolar detection(us) |
| --- | --- | --- | --- |
| 0 | 0 1 0 1 | 4.84 | 4.01 |
| 1 | 1 0 0 1 | 4.10 | 4.41 |
| 2 | 1 0 1 0 | 4.92 | 4.92 |
| 3 | 0 1 1 0 | 9.03 | 8.32 |
| 4 | 1 1 0 0 | 9.10 | 6.97 |
| 5 | 0 0 1 1 | 10.65 | 8.90 |
| Note1: Based on our simulation results, same TO correction performance can be achieved by mismatch detection and bipolar detection. |  |  |  |

From the simulation results, it can be seen that only three binary sequences can meet the requirement of TO correction accuracy <5us.

Observation 4: For OOK-4 with M=1 and 4-length binary sequences, only three 4-length binary sequences can meet the requirement of TO correction accuracy <=5us.

- 6-length binary sequence

For OOK-4 with M=1, all candidate 6-length binary sequence under the principle of balanced OOK-ON and OOK-OFF symbols are listed in Table 2.

Table 2. Summary of TO correction results for OOK-4 with M=1 and 6-length binary sequences

| Sequence index | Candidate sequences | TO correction results by mismatch/bipolar detection (us) | TO correction results by unipolar detection(us) |
| --- | --- | --- | --- |
| 0 | 1 0 1 0 0 1 | 2.58 | 2.71 |
| 1 | 1 0 0 1 0 1 | 2.84 | 2.84 |
| 2 | 0 1 0 1 0 1 | 2.94 | 2.69 |
| 3 | 1 0 1 0 1 0 | 3.03 | 2.60 |
| 4 | 1 0 0 1 1 0 | 5.06 | 4.32 |
| 5 | 0 1 1 0 0 1 | 3.97 | 4.35 |
| 6 | 1 1 0 0 0 1 | 4.29 | 4.04 |
| 7 | 1 0 0 0 1 1 | 4.76 | 4.31 |
| 8 | 1 0 1 1 0 0 | 4.06 | 3.84 |
| 9 | 1 1 1 0 0 0 | 9.41 | 7.10 |
| 10 | 1 1 0 1 0 0 | 3.99 | 4.18 |

From the simulation results, it can be seen

- For mismatch/bipolar detection, there are 9 binary sequences with index 0,1,2,3,5,6,7,8,10 can meet the requirement of TO correction accuracy <5us.

- For unipolar detection, there are 10 binary sequences with index 0,1,2,3,4,5,6,7,8,10 can meet the requirement of TO correction accuracy <5us.

Considering both mismatch/bipolar detection performance and unipolar detection performance, 4 binary sequences with index 0,1,2,3 are selected and can be used as LP-SS binary sequence for OOK-1 and OOK-4 with M=1.

- Observation 5: For OOK-4 with M=1 and 6-length binary sequences

- For mismatch/bipolar detection, there are 9 binary sequences with index 0,1,2,3,5,6,7,8,10 can meet the requirement of TO correction accuracy <5us.

- For unipolar detection, there are 10 binary sequences with index 0,1,2,3,4,5,6,7,8,10 can meet the requirement of TO correction accuracy <5us.

Proposal 6:For OOK-4 with M=1, the following four 6-length binary sequences can be used as LP-SS.

| Sequence index | Candidate sequences |
| --- | --- |
| 0 | 1 0 1 0 0 1 |
| 1 | 1 0 0 1 0 1 |
| 2 | 0 1 0 1 0 1 |
| 3 | 1 0 1 0 1 0 |

- 8-length binary sequence

For OOK-4 with M=1, all candidate 8-length binary sequence under the principle of balanced OOK-ON and OOK-OFF symbols are listed in Table 3.

Table 3. Summary of TO correction results for OOK-4 with M=1 and 8-length binary sequences

| Sequence index | Candidate sequences | TO correction results by mismatch/bipolar detection (us) | TO correction results by unipolar detection(us) |
| --- | --- | --- | --- |
| 0 | 1 0 1 0 1 0 0 1 | 2.02 | 1.90 |
| 1 | 1 0 1 0 0 1 0 1 | 2.16 | 1.88 |
| 2 | 1 0 0 1 0 1 0 1 | 2.16 | 1.92 |
| 3 | 1 0 1 0 0 1 1 0 | 3.25 | 3.1 |
| 4 | 0 1 0 1 0 1 0 1 | 1.97 | 1.83 |
| 5 | 1 0 0 0 1 1 0 1 | 2.66 | 2.65 |
| 6 | 0 1 1 0 0 1 0 1 | 2.6 | 2.32 |
| 7 | 0 1 0 1 1 0 0 1 | 2.55 | 2.32 |
| 8 | 0 1 0 1 0 1 1 0 | 2.95 | 2.90 |
| 9 | 1 0 1 1 0 0 0 1 | 2.88 | 2.85 |
| 10 | 1 1 0 0 1 0 1 0 | 3.12 | 3.01 |
| 11 | 1 1 1 0 1 0 0 0 | 4.79 | 3.68 |
| 12 | 1 0 0 1 0 1 1 0 | 2.88 | 2.79 |
| 13 | 0 1 0 1 1 0 1 0 | 2.86 | 2.93 |
| 14 | 0 1 1 0 1 0 0 1 | 2.45 | 2.45 |
| 15 | 1 0 1 0 1 0 1 0 | 2.23 | 2.07 |

From the simulation results, it can be seen

- For mismatch/bipolar detection, all the 16 binary sequences can meet the requirement of TO correction accuracy <5us.

- For unipolar detection, all the 16 binary sequences can meet the requirement of TO correction accuracy <5us.

Considering both mismatch/bipolar detection performance and unipolar detection performance, 4 binary sequences with index 0,1,2,4 are selected and can be used as LP-SS binary sequence for OOK-1 and OOK-4 with M=1.

- Observation 6: For OOK-4 with M=1 and 8-length binary sequences

- For mismatch/bipolar detection, all the 16 binary sequences can meet the requirement of TO correction accuracy <5us.

- For unipolar detection, all the 16 binary sequences can meet the requirement of TO correction accuracy <5us.

Proposal 7:For OOK-4 with M=1, if 8-length binary sequences is considered, the following 4 sequences be used.

| Sequence index | Candidate sequences |
| --- | --- |
| 0 | 1 0 1 0 1 0 0 1 |
| 1 | 1 0 1 0 0 1 0 1 |
| 2 | 1 0 0 1 0 1 0 1 |
| 4 | 0 1 0 1 0 1 0 1 |

For OOK-4 with M=2

- 8-length binary sequence

For OOK-4 with M=2, all candidate 8-length binary sequence under the principle of balanced OOK-ON and OOK-OFF symbols are listed in Table 4.

Table 4. Summary of TO correction results for OOK-4 with M=2 and 8-length binary sequences

| Sequence index | Candidate sequences | TO correction results by mismatch/bipolar detection (us) | TO correction results by unipolar detection(us) |
| --- | --- | --- | --- |
| 0 | 1 0 1 0 0 1 0 1 | 2.1 | 2.09 |
| 1 | 0 1 1 0 1 0 0 1 | 1.9 | 1.97 |
| 2 | 1 0 0 1 0 1 1 0 | 2.4 | 2.37 |
| 3 | 0 1 1 0 0 1 0 1 | 1.96 | 2.52 |
| 4 | 1 0 0 1 1 0 0 1 | 1.95 | 2.06 |
| 5 | 1 0 1 0 1 0 0 1 | 2.11 | 1.80 |
| 6 | 0 1 0 1 0 1 0 1 | 2.47 | 2.88 |
| 7 | 0 1 0 1 1 0 1 0 | 2.73 | 2.58 |
| 8 | 1 0 1 0 0 1 1 0 | 2.80 | 2.02 |
| 9 | 1 0 1 0 1 0 1 0 | 2.67 | 1.80 |
| 10 | 0 1 0 1 1 0 0 1 | 2.25 | 2.61 |
| 11 | 0 1 1 0 0 1 1 0 | 2.43 | 2.59 |
| 12 | 1 0 0 1 0 1 0 1 | 2.13 | 2.55 |
| 13 | 0 1 0 1 0 1 1 0 | 2.62 | 2.88 |

From the simulation results, it can be seen

- For mismatch/bipolar detection, there are 3 binary sequences with index 1,3,4 can meet the requirement of TO correction accuracy <2us.

- For unipolar detection, there are 2 binary sequences with index 1,5 can meet the requirement of TO correction accuracy <2us.

- Observation 7: For OOK-4 with M=2 and 8-length binary sequences

- For mismatch/bipolar detection, there are 3 binary sequences with index 1,3,4 can meet the requirement of TO correction accuracy <2us.

- For unipolar detection, there are 2 binary sequences with index 1,5 can meet the requirement of TO correction accuracy <2us.

Observation 8: For OOK-4 with M=2 and 8-length binary sequences, only two or three 8-length binary sequences can meet the requirement of TO correction accuracy <=2us.

- 12-length binary sequence

For OOK-4 with M=2, all candidate 12-length binary sequence under the principle of balanced OOK-ON and OOK-OFF symbols are listed in Table 5.

Table 5. Summary of TO correction results for OOK-4 with M=2 and 12-length binary sequences

| Sequence index | Candidate sequences | TO correction results by mismatch/bipolar detection (us) | TO correction results by unipolar detection(us) |
| --- | --- | --- | --- |
| 0 | 0 1 1 0 0 1 0 1 1 0 0 1 | 1.19 | 1.88 |
| 1 | 0 1 1 0 0 1 1 0 1 0 0 1 | 1.18 | 1.30 |
| 2 | 0 1 1 0 1 0 0 1 1 0 0 1 | 1.16 | 1.30 |
| 3 | 1 0 0 1 1 0 0 1 0 1 0 1 | 1.39 | 1.98 |
| 4 | 0 1 1 0 1 0 1 0 1 0 0 1 | 1.39 | 1.34 |
| 5 | 0 1 1 0 1 0 0 1 0 1 0 1 | 1.50 | 1.94 |
| 6 | 0 1 0 1 0 1 1 0 0 1 0 1 | 1.64 | 2.22 |
| 7 | 0 1 0 1 0 1 0 1 0 1 0 1 | 1.91 | 2.58 |
| 8 | 1 0 0 1 1 0 1 0 0 1 1 0 | 1.65 | 1.69 |
| 9 | 1 0 1 0 1 0 1 0 1 0 1 0 | 1.88 | 1.34 |
| 10 | 1 0 1 0 1 0 1 0 1 0 0 1 | 1.43 | 1.30 |
| 11 | 1 0 0 1 1 0 1 0 1 0 0 1 | 1.37 | 1.28 |
| 12 | 1 0 1 0 1 0 0 1 1 0 0 1 | 1.33 | 1.27 |
| 13 | 1 0 0 1 1 0 0 1 1 0 0 1 | 1.03 | 1.35 |
| 14 | 1 0 1 0 1 0 1 0 0 1 1 0 | 1.86 | 1.49 |
| 15 | 1 0 1 0 0 1 1 0 1 0 0 1 | 1.30 | 1.31 |
| 16 | 1 0 0 1 0 1 1 0 0 1 0 1 | 1.57 | 2.11 |
| 17 | 0 1 0 1 1 0 1 0 1 0 0 1 | 1.57 | 1.63 |
| 18 | 1 0 0 1 0 1 1 0 0 1 1 0 | 1.58 | 1.86 |
| 19 | 1 0 0 1 1 0 0 1 0 1 1 0 | 1.39 | 1.65 |

From the simulation results, it can be seen

- For mismatch/bipolar detection, all the 20 binary sequences can meet the requirement of TO correction accuracy <2us.

- For mismatch/bipolar detection, 4 binary sequences with index 0,1,2,13 have the best TO correction accuracy.

- For unipolar detection, there are 18 binary sequences with index 0,1,2,3,4,5,8,9,10,11,12,13,14,15,17,18,19 can meet the requirement of TO correction accuracy <2us.

- For unipolar detection, 4 binary sequences with index 1,2,11,12 have the best TO correction accuracy.

- Observation 9: For OOK-4 with M=1 and 8-length binary sequences

- For mismatch/bipolar detection, all the 20 binary sequences can meet the requirement of TO correction accuracy <2us.

- For mismatch/bipolar detection, 4 binary sequences with index 0,1,2,13 have the best TO correction accuracy.

- For unipolar detection, there are 18 binary sequences with index 0,1,2,3,4,5,8,9,10,11,12,13,14,15,17,18,19 can meet the requirement of TO correction accuracy <2us.

- For unipolar detection, 4 binary sequences with index 1,2,11,12 have the best TO correction accuracy.

Proposal 8:For OOK-4 with M=2 and mismatch/bipolar detection, the following four 12-length binary sequences can be used as LP-SS.

| Sequence index | Candidate sequences |
| --- | --- |
| 0 | 0 1 1 0 0 1 0 1 1 0 0 1 |
| 1 | 0 1 1 0 0 1 1 0 1 0 0 1 |
| 2 | 0 1 1 0 1 0 0 1 1 0 0 1 |
| 13 | 1 0 0 1 1 0 0 1 1 0 0 1 |

Proposal 9:For OOK-4 with M=2 and unipolar detection, the following four 12-length binary sequences can be used as LP-SS.

| Sequence index | Candidate sequences |
| --- | --- |
| 1 | 0 1 1 0 0 1 1 0 1 0 0 1 |
| 2 | 0 1 1 0 1 0 0 1 1 0 0 1 |
| 11 | 1 0 0 1 1 0 1 0 1 0 0 1 |
| 12 | 1 0 1 0 1 0 0 1 1 0 0 1 |

- 16-length binary sequence

For OOK-4 with M=2, all candidate 16-length binary sequence under the principle of balanced OOK-ON and OOK-OFF symbols are listed in Table 6.

Table 6. Summary of TO correction results for OOK-4 with M=2 and 16-length binary sequences

| Sequence index | Candidate sequences | TO correction results by mismatch/bipolar detection (us) | TO correction results by unipolar detection(us) |
| --- | --- | --- | --- |
| 0 | 1 0 0 1 1 0 1 0 0 1 0 1 1 0 0 1 | 1.12 | 1.42 |
| 1 | 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 | 1.67 | 1.04 |
| 2 | 1 0 0 1 1 0 0 1 1 0 0 1 1 0 0 1 | 1.05 | 1.14 |
| 3 | 0 1 1 0 0 1 1 0 0 1 1 0 0 1 1 0 | 1.02 | 1.16 |
| 4 | 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0 | 1.16 | 1.14 |
| 5 | 1 0 1 0 0 1 1 0 1 0 1 0 1 0 0 1 | 1.18 | 1.09 |
| 6 | 1 0 0 1 1 0 0 1 0 1 1 0 0 1 0 1 | 1.23 | 1.65 |
| 7 | 1 0 1 0 1 0 0 1 1 0 1 0 1 0 0 1 | 1.38 | 1.17 |
| 8 | 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1 | 1.31 | 1.39 |
| 9 | 1 0 0 1 0 1 0 1 0 1 0 1 1 0 0 1 | 1.32 | 2.16 |
| 10 | 1 0 1 0 0 1 0 1 1 0 0 1 0 1 0 1 | 1.30 | 1.92 |
| 11 | 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 | 1.91 | 2.55 |
| 12 | 0 1 1 0 1 0 1 0 0 1 1 0 0 1 1 0 | 1.17 | 1.16 |
| 13 | 0 1 1 0 0 1 1 0 1 0 0 1 0 1 0 1 | 1.25 | 1.58 |
| 14 | 1 0 1 0 0 1 1 0 0 1 0 1 0 1 1 0 | 1.30 | 1.44 |
| 15 | 1 0 1 0 0 1 1 0 0 1 0 1 0 1 1 0 | 1.30 | 1.44 |
| 16 | 1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1 | 1.16 | 1.37 |
| 17 | 1 0 0 1 0 1 0 1 1 0 0 1 1 0 0 1 | 1.13 | 1.67 |
| 18 | 0 1 0 1 1 0 1 0 0 1 1 0 0 1 0 1 | 1.28 | 1.61 |
| 19 | 0 1 1 0 1 0 0 1 0 1 0 1 1 0 0 1 | 1.26 | 1.60 |
| 20 | 0 1 1 0 0 1 1 0 1 0 1 0 1 0 0 1 | 1.16 | 1.17 |
| 21 | 0 1 0 1 1 0 1 0 1 0 0 1 1 0 0 1 | 1.25 | 1.28 |
| 22 | 1 0 0 1 0 1 0 1 1 0 1 0 1 0 0 1 | 1.29 | 1.68 |
| 23 | 1 0 0 1 1 0 0 1 0 1 0 1 1 0 1 0 | 1.28 | 1.60 |
| 24 | 1 0 1 0 1 0 0 1 1 0 0 1 0 1 0 1 | 1.28 | 1.52 |
| 25 | 1 0 0 1 1 0 1 0 1 0 0 1 1 0 0 1 | 1.11 | 1.13 |
| 26 | 1 0 1 0 0 1 1 0 1 0 0 1 1 0 0 1 | 1.06 | 1.13 |
| 27 | 1 0 0 1 1 0 0 1 1 0 1 0 1 0 0 1 | 1.09 | 1.13 |
| 28 | 1 0 1 0 1 0 1 0 0 1 1 0 1 0 1 0 | 1.35 | 1.10 |
| 29 | 0 1 1 0 1 0 0 1 1 0 1 0 1 0 0 1 | 1.21 | 1.10 |
| 30 | 0 1 1 0 1 0 0 1 1 0 0 1 1 0 0 1 | 1.04 | 1.17 |
| 31 | 0 1 1 0 0 1 1 0 0 1 1 0 1 0 0 1 | 1.04 | 1.19 |
| 32 | 0 1 1 0 1 0 1 0 1 0 0 1 1 0 1 0 | 1.25 | 1.22 |
| 33 | 0 1 1 0 1 0 0 1 1 0 1 0 1 0 1 0 | 1.31 | 1.11 |
| 34 | 1 0 1 0 1 0 0 1 1 0 0 1 1 0 0 1 | 1.11 | 1.14 |
| 35 | 1 0 0 1 1 0 0 1 1 0 0 1 0 1 0 1 | 1.13 | 1.53 |
| 36 | 1 0 0 1 0 1 1 0 0 1 1 0 1 0 0 1 | 1.11 | 1.33 |

From the simulation results, it can be seen

- For mismatch/bipolar detection, all the 36 binary sequences can meet the requirement of TO correction accuracy <2us.

- For mismatch/bipolar detection, 4 binary sequences with index 2,3,30,31 have the best TO correction accuracy.

- For unipolar detection, except sequences with index 9,11, there are 34 binary sequences can meet the requirement of TO correction accuracy <2us.

- For unipolar detection, 4 binary sequences with index 1,5,28,29 have the best TO correction accuracy.

- Observation 10: For OOK-4 with M=2 and 16-length binary sequences

- For mismatch/bipolar detection, all the 36 binary sequences can meet the requirement of TO correction accuracy <2us.

- For mismatch/bipolar detection, 4 binary sequences with index 2,3,30,31 have the best TO correction accuracy.

- For unipolar detection, except sequences with index 9,11, there are 34 binary sequences can meet the requirement of TO correction accuracy <2us.

- For unipolar detection, 4 binary sequences with index 1,5,28,29 have the best TO correction accuracy.

Proposal 10:For OOK-4 with M=2 and mismatch/bipolar detection, the following four 16-length binary sequences can be used as LP-SS.

| Sequence index | Candidate sequences |
| --- | --- |
| 2 | 1 0 0 1 1 0 0 1 1 0 0 1 1 0 0 1 |
| 3 | 0 1 1 0 0 1 1 0 0 1 1 0 0 1 1 0 |
| 30 | 0 1 1 0 1 0 0 1 1 0 0 1 1 0 0 1 |
| 31 | 0 1 1 0 0 1 1 0 0 1 1 0 1 0 0 1 |

Proposal 11:For OOK-4 with M=2 and unipolar detection, the following four 16-length binary sequences can be used as LP-SS.

| Sequence index | Candidate sequences |
| --- | --- |
| 1 | 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 |
| 5 | 1 0 1 0 0 1 1 0 1 0 1 0 1 0 0 1 |
| 28 | 1 0 1 0 1 0 1 0 0 1 1 0 1 0 1 0 |
| 29 | 0 1 1 0 1 0 0 1 1 0 1 0 1 0 0 1 |

For OOK-4 with M=4

- 16-length binary sequence

For OOK-4 with M=4, all candidate 16-length binary sequence under the principle of balanced OOK-ON and OOK-OFF symbols are listed in Table 7.

Table 7. Summary of TO correction results for OOK-4 with M=4 and 16-length binary sequences

| Sequence index | Candidate sequences | TO correction results by mismatch/bipolar detection (us) | TO correction results by unipolar detection(us) |
| --- | --- | --- | --- |
| 0 | 1 0 1 0 1 0 0 1 1 0 1 0 0 1 0 1 | 0.77 | 0.92 |
| 1 | 0 1 0 1 0 1 0 1 1 0 1 0 1 0 0 1 | 1.16 | 1.21 |
| 2 | 1 0 1 0 1 0 1 0 0 1 1 0 0 1 0 1 | 0.92 | 0.92 |
| 3 | 1 0 0 1 1 0 1 0 1 0 0 1 0 1 0 1 | 0.97 | 1.08 |
| 4 | 0 0 1 1 0 1 1 0 0 0 1 1 1 0 0 1 | 1.35 | 1.52 |
| 5 | 0 0 1 1 1 0 0 1 1 0 1 0 0 0 1 1 | 1.16 | 1.34 |
| 6 | 0 1 0 1 0 1 0 1 1 1 0 0 1 0 0 1 | 1.26 | 1.60 |
| 7 | 0 1 1 0 0 0 1 1 1 0 1 0 1 0 1 0 | 1.26 | 1.12 |
| 8 | 0 0 1 1 0 0 1 1 0 0 1 1 0 0 1 1 | 2.02 | 2.59 |
| 9 | 1 0 1 0 0 1 0 1 1 0 1 0 0 1 0 1 | 0.85 | 0.97 |
| 10 | 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 | 16.60 | 1.10 |
| 11 | 1 1 0 0 0 0 1 1 0 0 1 1 1 1 0 0 | 1.70 | 2.39 |
| 12 | 0 1 1 0 1 0 0 1 1 0 1 0 1 0 1 0 | 1.14 | 1.02 |
| 13 | 0 1 1 0 1 0 1 0 1 0 0 1 1 0 1 0 | 1.22 | 1.10 |
| 14 | 1 0 1 0 0 1 1 0 1 0 1 0 1 0 0 1 | 1.03 | 1.05 |
| 15 | 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0 | 1.06 | 0.91 |
| 16 | 0 1 1 0 0 1 0 1 1 0 1 0 0 1 0 1 | 0.75 | 0.90 |
| 17 | 1 1 0 0 0 1 0 1 1 0 1 0 0 0 1 1 | 0.75 | 1.01 |
| 18 | 1 0 1 0 0 1 0 1 1 0 1 0 0 0 1 1 | 0.75 | 0.86 |
| 19 | 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1 | 0.76 | 0.87 |
| 20 | 1 0 1 0 1 0 0 1 1 0 1 0 1 0 1 0 | 0.85 | 0.79 |
| 21 | 1 0 1 0 1 1 0 0 1 0 1 0 1 0 1 0 | 1.15 | 0.81 |
| 22 | 1 0 1 0 0 1 0 1 1 0 1 0 0 1 1 0 | 0.88 | 0.84 |
| 23 | 1 0 0 1 1 0 1 0 1 0 1 0 1 0 0 1 | 0.94 | 0.84 |
| 24 | 1 1 0 0 0 1 0 1 1 0 1 0 0 1 0 1 | 0.77 | 0.87 |
| 25 | 0 1 0 1 0 0 1 1 0 0 1 1 0 1 0 1 | 1.43 | 2.20 |
| 26 | 0 1 0 1 0 1 0 1 1 0 0 1 1 0 0 1 | 1.71 | 1.97 |
| 27 | 0 1 1 0 0 1 1 0 0 1 0 1 0 1 0 1 | 1.27 | 1.44 |
| 28 | 1 0 0 1 1 0 1 0 1 0 1 0 0 1 1 0 | 1.37 | 0.97 |
| 29 | 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 | 16.32 | 16.32 |
| 30 | 1 0 0 1 1 0 0 1 1 0 0 1 1 0 0 1 | 1.44 | 1.77 |
| 31 | 0 1 1 0 0 1 1 0 0 1 1 0 0 1 1 0 | 1.85 | 1.60 |
| 32 | 1 0 1 0 0 1 0 1 1 0 0 1 0 1 0 1 | 0.97 | 1.26 |
| 33 | 1 0 0 1 0 1 0 1 0 1 0 1 1 0 0 1 | 1.27 | 1.73 |
| 34 | 1 0 1 0 1 0 0 1 1 0 1 0 1 0 0 1 | 0.88 | 1.01 |

From the simulation results, it can be seen

- For mismatch/bipolar detection, binary sequences with index 0,2,3,9,16,17,18,19,20,22,23,24,32,34can meet the requirement of TO correction accuracy <1us.

- For mismatch/bipolar detection, 4 binary sequences with index 16,17,18,19 have the best TO correction accuracy.

- For unipolar detection, binary sequences with index 0,2,9,15,16,18,19,20,21,22,23,24,28 can meet the requirement of TO correction accuracy <1us.

- For unipolar detection, 4 binary sequences with index 20,21,22,23 have the best TO correction accuracy.

- Observation 11: For OOK-4 with M=4 and 16-length binary sequences

- For mismatch/bipolar detection, binary sequences with index 0,2,3,9,16,17,18,19,20,22,23,24,32,34can meet the requirement of TO correction accuracy <1us.

- For mismatch/bipolar detection, 4 binary sequences with index 16,17,18,19 have the best TO correction accuracy.

- For unipolar detection, binary sequences with index 0,2,9,15,16,18,19,20,21,22,23,24,28 can meet the requirement of TO correction accuracy <1us.

- For unipolar detection, 4 binary sequences with index 20,21,22,23 have the best TO correction accuracy.

Proposal 12:For OOK-4 with M=4 and mismatch/bipolar detection, the following four 16-length binary sequences can be used as LP-SS.

| Sequence index | Candidate sequences |
| --- | --- |
| 16 | 0 1 1 0 0 1 0 1 1 0 1 0 0 1 0 1 |
| 17 | 1 1 0 0 0 1 0 1 1 0 1 0 0 0 1 1 |
| 18 | 1 0 1 0 0 1 0 1 1 0 1 0 0 0 1 1 |
| 19 | 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1 |

Proposal 13:For OOK-4 with M=4 and unipolar detection, the following four 16-length binary sequences can be used as LP-SS.

| Sequence index | Candidate sequences |
| --- | --- |
| 20 | 1 0 1 0 1 0 0 1 1 0 1 0 1 0 1 0 |
| 21 | 1 0 1 0 1 1 0 0 1 0 1 0 1 0 1 0 |
| 22 | 1 0 1 0 0 1 0 1 1 0 1 0 0 1 1 0 |
| 23 | 1 0 0 1 1 0 1 0 1 0 1 0 1 0 0 1 |

- 32-length binary sequence

For OOK-4 with M=4, all candidate 32-length binary sequence under the principle of balanced OOK-ON and OOK-OFF symbols are listed in Table 8.

Table 8. Summary of TO correction results for OOK-4 with M=4 and 32-length binary sequences

| Sequence index | Candidate sequences | TO correction results by mismatch/bipolar detection (us) | TO correction results by unipolar detection(us) |
| --- | --- | --- | --- |
| 0 | 1 0 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 1 0 0 1 0 1 0 1 0 1 0 1 0 1 | 0.57 | 0.56 |
| 1 | 1 0 1 0 1 0 1 0 1 0 1 0 0 1 0 1 0 1 0 1 0 1 1 0 1 0 1 0 1 0 0 1 | 0.65 | 0.56 |
| 2 | 1 0 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 1 | 0.65 | 1.29 |
| 3 | 1 0 1 0 0 1 0 1 0 1 1 0 1 0 1 0 1 0 1 0 1 0 1 0 0 1 0 1 0 1 0 1 | 0.51 | 0.55 |
| 4 | 0 1 1 0 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1 1 0 1 0 1 0 0 1 0 1 1 0 | 0.52 | 0.55 |
| 5 | 1 0 1 0 0 1 0 1 1 0 0 1 1 0 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1 1 0 | 0.63 | 0.64 |
| 6 | 1 0 1 0 0 1 0 1 1 0 1 0 1 0 0 1 0 1 1 0 0 1 1 0 1 0 0 1 1 0 1 0 | 0.57 | 0.71 |
| 7 | 1 0 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1 1 0 1 0 1 0 0 1 0 1 1 0 0 1 | 0.56 | 0.70 |
| 8 | 1 0 1 0 0 1 0 1 1 0 1 0 0 1 0 1 1 0 1 0 0 0 1 1 1 1 0 0 0 0 1 1 | 0.47 | 0.54 |
| 9 | 0 0 1 1 1 1 0 0 0 0 1 1 1 1 0 0 1 0 1 0 0 1 0 1 1 0 1 0 0 1 0 1 | 0.49 | 0.58 |
| 10 | 1 0 1 0 0 1 0 1 1 0 1 0 0 1 0 1 0 1 1 0 1 0 0 1 1 1 0 0 0 0 1 1 | 0.51 | 0.58 |
| 11 | 1 0 1 0 0 1 0 1 1 0 1 0 0 1 0 1 1 1 0 0 0 0 1 1 1 1 0 0 0 0 1 1 | 0.51 | 0.56 |
| 12 | 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 0 1 0 1 | 0.64 | 0.51 |
| 13 | 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 0 1 0 1 0 1 | 0.65 | 0.55 |
| 14 | 1 1 0 0 0 1 1 0 1 0 1 0 0 1 0 1 1 0 1 0 0 1 0 1 0 1 1 0 0 0 1 1 | 0.56 | 0.55 |
| 15 | 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 0 1 0 1 0 1 0 1 | 0.58 | 0.55 |
| 16 | 1 0 1 0 1 0 1 0 0 1 1 0 0 1 1 0 1 0 0 1 0 1 1 0 0 1 0 1 0 1 0 1 | 0.56 | 0.63 |
| 17 | 0 1 1 0 0 1 1 0 1 0 0 1 0 1 1 0 0 1 0 1 0 1 0 1 1 0 1 0 1 0 1 0 | 0.62 | 0.68 |
| 18 | 1 0 0 1 0 1 1 0 0 1 0 1 0 1 0 1 1 0 1 0 1 0 1 0 0 1 1 0 0 1 1 0 | 0.57 | 0.69 |
| 19 | 0 1 0 1 0 1 0 1 1 0 1 0 1 0 1 0 0 1 1 0 0 1 1 0 1 0 0 1 0 1 1 0 | 0.75 | 0.83 |
| 20 | 0 1 0 1 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1 | 0.57 | 0.70 |
| 21 | 0 1 1 0 0 1 0 1 0 1 1 0 0 1 0 1 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1 | 0.51 | 0.58 |
| 22 | 0 1 0 1 0 1 0 1 1 0 1 0 1 0 0 1 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0 | 0.58 | 0.69 |
| 23 | 0 1 0 1 0 1 1 0 0 1 0 1 1 0 1 0 0 1 1 0 0 1 1 0 1 0 1 0 0 1 0 1 | 0.55 | 0.57 |
| 24 | 0 1 0 1 0 1 0 1 0 1 0 1 1 0 0 1 1 0 0 1 1 0 0 1 1 0 0 1 1 0 0 1 | 0.75 | 1.18 |
| 25 | 0 1 1 0 0 1 0 1 0 1 0 1 0 1 0 1 0 0 1 1 0 0 1 1 0 0 1 1 0 0 1 1 | 0.80 | 1.74 |
| 26 | 0 1 1 0 0 1 1 0 0 1 1 0 0 1 1 0 0 1 0 1 0 1 0 1 0 1 0 1 0 0 1 1 | 0.62 | 0.96 |
| 27 | 1 0 0 1 1 0 0 1 1 0 0 1 1 0 0 1 1 0 0 1 1 0 1 0 1 0 1 0 1 0 1 0 | 0.73 | 0.90 |
| 28 | 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 | 2.13 | 16.44 |
| 29 | 0 1 1 0 0 1 1 0 0 1 1 0 0 1 1 0 0 1 1 0 0 1 1 0 0 1 1 0 0 1 1 0 | 0.89 | 0.83 |
| 30 | 1 0 0 1 1 0 0 1 1 0 0 1 1 0 0 1 1 0 0 1 1 0 0 1 1 0 0 1 1 0 0 1 | 0.94 | 1.37 |
| 31 | 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 | 0.74 | 0.60 |

From the simulation results, it can be seen

- For mismatch/bipolar detection, except binary sequences with index 28, other 31 binary sequences can meet the requirement of TO correction accuracy <1us.

- For mismatch/bipolar detection, 4 binary sequences with index 8,9,10,11 have the best TO correction accuracy.

- For unipolar detection, except binary sequence with index 2,24,25,28,30, other binary sequences can meet the requirement of TO correction accuracy <1us.

- For unipolar detection, 4 binary sequences with index 4,8,12,14 have the best TO correction accuracy.

- Observation 12: For OOK-4 with M=4 and 32-length binary sequences

- For mismatch/bipolar detection, except binary sequences with index 28, other 31 binary sequences can meet the requirement of TO correction accuracy <1us.

- For mismatch/bipolar detection, 4 binary sequences with index 8,9,10,11 have the best TO correction accuracy.

- For unipolar detection, except binary sequence with index 2,24,25,28,30, other binary sequences can meet the requirement of TO correction accuracy <1us.

- For unipolar detection, 4 binary sequences with index 4,8,12,14 have the best TO correction accuracy.

Proposal 14:For OOK-4 with M=4 and mismatch/bipolar detection, the following four 32-length binary sequences can be used as LP-SS.

| Sequence index | Candidate sequences |
| --- | --- |
| 8 | 1 0 1 0 0 1 0 1 1 0 1 0 0 1 0 1 1 0 1 0 0 0 1 1 1 1 0 0 0 0 1 1 |
| 9 | 0 0 1 1 1 1 0 0 0 0 1 1 1 1 0 0 1 0 1 0 0 1 0 1 1 0 1 0 0 1 0 1 |
| 10 | 1 0 1 0 0 1 0 1 1 0 1 0 0 1 0 1 0 1 1 0 1 0 0 1 1 1 0 0 0 0 1 1 |
| 11 | 1 0 1 0 0 1 0 1 1 0 1 0 0 1 0 1 1 1 0 0 0 0 1 1 1 1 0 0 0 0 1 1 |

Proposal 15:For OOK-4 with M=4 and unipolar detection, the following four 32-length binary sequences can be used as LP-SS.

| Sequence index | Candidate sequences |
| --- | --- |
| 4 | 0 1 1 0 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1 1 0 1 0 1 0 0 1 0 1 1 0 |
| 8 | 1 0 1 0 0 1 0 1 1 0 1 0 0 1 0 1 1 0 1 0 0 0 1 1 1 1 0 0 0 0 1 1 |
| 12 | 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 0 1 0 1 |
| 14 | 1 1 0 0 0 1 1 0 1 0 1 0 0 1 0 1 1 0 1 0 0 1 0 1 0 1 1 0 0 0 1 1 |

Additional sync signal

Time error

Time error for OOK based LR

In RAN1 #119 meeting, RAN1 made following agreement.

Agreement Regarding whether to support additional sync signal to LP-SS, down-selection from the following: - Option 1A: No additional sync signal, LP-SS periodicity =320ms - Additionally support at least one of [80ms,160ms] - FFS: whether different values can be applicable for different M values - Option 1B: No additional sync signal, LP-SS periodicity =320ms - No additional support of other periodicities - Option 2: With additional sync signal, LP-SS periodicity =320ms - FFS additionally support other LP-SS periodicities - FFS details on additional sync signal - FFS additional sync signal is configurable and/or conditionally present

In this section, a rough calculation of TO value before LP-WUS detection is made in Table 9.

Table 9. A rough calculation of TO value before LP-WUS detection based on various configuration

|  | Residual frequency error=20ppm |  |  | Residual frequency error=10ppm |  |  | Residual frequency error=5ppm |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LP-SS periodicity(ms) | Worst time drift before LP-WUS detection, T,(us) | LP-SS TO estimation accuracy, ∆T,(us) | Total TO before LP-WUS detection,(us) | Worst time drift before LP-WUS detection, T,(us) | LP-SS TO estimation accuracy, ∆T,(us) | Total TO before LP-WUS detection,(us) | Worst time drift before LP-WUS detection, T,(us) | LP-SS TO estimation accuracy, ∆T,(us) | Total TO before LP-WUS detection,(us) |
| 80 | 1.6 | 5 | 6.6 | 0.8 | 5 | 5.8 | 0.4 | 5 | 5.4 |
|  |  | 2 | 3.6 |  | 2 | 2.8 |  | 2 | 2.4 |
|  |  | 1 | 2.6 |  | 1 | 1.8 |  | 1 | 1.4 |
| 160 | 3.2 | 5 | 8.2 | 1.6 | 5 | 6.6 | 0.8 | 5 | 5.8 |
|  |  | 2 | 5.2 |  | 2 | 3.6 |  | 2 | 2.8 |
|  |  | 1 | 4.2 |  | 1 | 2.6 |  | 1 | 1.8 |
| 320 | 6.4 | 5 | 11.4 | 3.2 | 5 | 8.2 | 1.6 | 5 | 6.6 |
|  |  | 2 | 8.4 |  | 2 | 5.2 |  | 2 | 3.6 |
|  |  | 1 | 7.4 |  | 1 | 4.2 |  | 1 | 2.6 |
| Note1: The residual frequency error is assumed after frequency error correction by RTC calibration. |  |  |  |  |  |  |  |  |  |

From Table 9, it can be seen that

- For the case of LP-SS periodicity=320ms and assuming time interval between target LP-WUS and LP-SS used for TO estimation equals to the LP-SS periodicity

- When residual frequency error = 20 ppm, even though the TO estimation error is smaller than 1us, additional sync signal is necessary.

- When residual frequency error = 10 ppm, when the TO estimation error is larger than 1us, additional sync signal is necessary.

- When residual frequency error = 5 ppm, when the TO estimation error is larger than 2us, additional sync signal is necessary.

- For the case of LP-SS periodicity=160ms and assuming time interval between target LP-WUS and LP-SS used for TO estimation equals to the LP-SS periodicity

- When residual frequency error = 20 ppm, when the TO estimation error is larger than 1us, additional sync signal is necessary.

- When residual frequency error = 10 ppm, when the TO estimation error is larger than 2us, additional sync signal is necessary.

- When residual frequency error = 5 ppm, when the TO estimation error is larger than 2us, additional sync signal is necessary.

- For the case of LP-SS periodicity=80ms and assuming time interval between target LP-WUS and LP-SS used for TO estimation equals to the LP-SS periodicity

- When residual frequency error = 20 ppm, when the TO estimation error is larger than 2us, additional sync signal is necessary.

- When residual frequency error = 10 ppm, when the TO estimation error is larger than 2us, additional sync signal is necessary.

- When residual frequency error = 5 ppm, when the TO estimation error is larger than 2us, additional sync signal is necessary.

Observation 13: For LP-SS periodicity with 80ms,160ms and 320ms, there are many cases that may cause residual time error before LP-WUS detection larger than the tolerance of the corresponding LP-WUS structure.

Proposal 16: Support additional sync signal to LP-SS at least for LP-SS periodicity =320ms.

Time error for OFDM sequence based LR

- For OFDM sequence based LR, since the clock precision is better and the power consumption is higher, the frequency drift is slower (0.05 ppm/s). The initial frequency offset has different precision for different OFDM sequence based LRs. For example, if OFDM sequence based LR has FFT block, it can do frequency and phase tracking by PLL and the corresponding power consumption is about 20~30, and the initial frequency error after frequency correction is about 2 ppm. If OFDM sequence based LR has a FLL but may not have a FFT block, it can also do frequency tracking but the accuracy is not as good as OFDM sequence based LR with FFT block, thus the initial frequency error after frequency correction is about 5 ppm.

- Time error calculation assumptions

- oscillator frequency drift [ppm/s] = 0.05ppm/s, residual frequency error[ppm] =5ppm for OFDM based LR without FFT block

- Time duration is [80, 160, 320]ms

- Time error is calculated based on the formula defined in TR38.869 which is listed as bellow:

The relationship between a frequency drift ( F’), and corresponding timing drift(ΔT) over a time(T) is ΔT = Fr*T ±0.5 * F’ *T2 (transient region)

- Time error calculation results

Table 10. Time error results when residual frequency error=5ppm, frequency drift=0.05ppm/s, without considering the residual timing error

| Periodicity(ms) | 80 | 160 | 320 |
| --- | --- | --- | --- |
| Timing drift(us) | 0.4 | 0.8 | 1.6 |
| Margin value of time error based on maximum tolerance of 4us | 3.6 | 3.2 | 2.4 |
| Note: the LP-WUS timing error tolerance 4us cited from TR 38.869 - For OFDMA, tolerance to timing error varies with sliding window size assumed by a receiver and was shown to tolerate timing error up to 4us if proper sliding window size is assumed by a receiver. |  |  |  |

From the calculation results, it can be seen that for OFDM sequence based LR additional sync signal is not necessary when LP-SS periodicity is not larger than 320ms.

- Proposal 17: For OFDM sequence based LR, additional sync signal is not necessary when LP-SS periodicity is not larger than 320ms.

Additional sync signal overhead

Case : target 5000 UEs in a cell, number of subgroups is 32, per UE paging rate 0.001

Assuming M value for LP-SS and LP-WUS is 1

The LP-WUS overhead is 0.77% assuming per UE paging rate 0.1% and each LP-WUS occupies 2 slots with 4 beams.

The LP-SS overhead is 0.18% with 320ms periodicity assuming LP-SS occupies 8 symbols with 4 beams.

The LP-SS overhead is 0.36% with 160ms periodicity assuming LP-SS occupies 8 symbols with 4 beams.

The LP-SS overhead is 0.72% with 80ms periodicity assuming LP-SS occupies 8 symbols with 4 beams.

Assuming M=2 L=8 for preamble and occupies 4symbols with 4 beams and the overhead is 0.11%

Obviously, reducing the LP-SS periodicity to achieve the sync performance would cause more system overhead.

Observation 14: Compared with reducing LP-SS periodicity to satisfy sync performance, additional sync signal has less system overhead in some scenarios.

Since additional sync signal is also used for TO correction, it is a straightforward way to reuse the structure of LP-SS and the binary sequences can be selected from candidate binary sequences for LP-SS in [3].

Proposal 18: Support additional sync signal design, and select binary sequence from candidate binary sequences for LP-SS.

LP-WUS

Maximum number of overlaid ZC sequences per OOK symbols for M=1,2

A link level simulation is made to decide the maximum number of overlaid ZC sequences for OOK-4 with M=1 and M=2.

- Simulation assumptions

- SCS=30KHz

- Number of SCs=128

- No time error

- No frequency error

- OOK-1 or OOK-4 with M=1

- Support detection with sliding window for OFDM sequence receiver without FFT

  - Sliding window = [-24:24] sampling points

- Other simulation assumptions refer to Appendix A

- Simulation results

2^N ZC sequences are configured to indicate N bit information carried by one OOK-ON symbol. For a target UE, since it knows the expression of the information carried by ZC sequence per OOK-ON symbol, it only detect the target ZC sequence per OOK-On symbol.

Table 11. Detection performance for 4 ZC sequences,OOK-4 with M=1

| SNR(dB) | 4 ZC sequences with Ncs=16, Root =1 |  | 4 ZC root sequences, Root=1,2,3,4 |  |
| --- | --- | --- | --- | --- |
|  | Miss detection rate | Non-target UE false wake up rate | Miss detection rate | Non-target UE false wake up rate |
| -2 | 0.0285 | 0.0109 | 0.0302 | 0.0923 |
| -1 | 0.0170 | 0.0113 | 0.0195 | 0.1262 |
| 0 | 0.0092 | 0.0120 | 0.0108 | 0.1732 |
| 1 | 0.0060 | 0.0126 | 0.0055 | 0.2272 |
| Note1: In simulation, only overlaid ZC sequence for Target UE is transmitted, but in receiver side all potential UEs(including Target UE and Non-target UEs) try to detect the transmitted overlaid ZC sequence. Therefore, Non-target UE false wake up rate = Successful(by false) detection times of the Non--target UEs / Total detection times of the Non--target UEs |  |  |  |  |

Table 12. Detection performance for 8 ZC sequences,OOK-4 with M=1

| SNR(dB) | 8 ZC sequences with Ncs=16, Root =1 |  | 8 ZC root sequences, Root=1,2,3,4,5,6,7,8 |  |
| --- | --- | --- | --- | --- |
|  | Miss detection rate | Non-target UE false wake up rate | Miss detection rate | Non-target UE false wake up rate |
| -2 | 0.0268 | 0.0314 | 0.0275 | 0.1043 |
| -1 | 0.0174 | 0.0409 | 0.0176 | 0.1440 |
| 0 | 0.0102 | 0.0523 | 0.0108 | 0.1959 |
| 1 | 0.0070 | 0.0664 | 0.0062 | 0.2612 |

Table 13. Detection performance for 16 ZC sequences,OOK-4 with M=1,FAR=0.1%

| SNR(dB) | 16 ZC sequences with Ncs=16, Root =1,2 |  | 16 ZC root sequences, Root=1~16 |  |
| --- | --- | --- | --- | --- |
|  | Miss detection rate | Non-target UE false wake up rate | Miss detection rate | Non-target UE false wake up rate |
| -2 | 0.0221 | 0.0737 | 0.0229 | 0.1134 |
| -1 | 0.0136 | 0.0988 | 0.0134 | 0.1551 |
| 0 | 0.0083 | 0.1313 | 0.0079 | 0.2106 |
| 1 | 0.0039 | 0.1713 | 0.0043 | 0.2797 |

From Table 11-13, it can be seen that

- MDR is kept to the same level when the number of overlaid ZC sequences is increased from 4 to 16;

- Non-target UE false wake up rate is increased seriously when the number of overlaid ZC sequences is increased from 4 to 16, especially when the overlaid ZC sequences are selected from multiple ZC root sequences.

Therefore, when configuring larger number of overlaid ZC sequences, a large number of non-target UEs may be woken up and will cause a serious waste of UE power consumption.

In order to decrease “Non-target UE false wake up rate”, FAR can be restricted to FAR =0.01%, and the detection performance for 16 ZC sequences is shown in Table 14.

Table 14: Detection performance for 16 ZC sequences,OOK-4 with M=1,FAR=0.01%

| SNR(dB) | 16 ZC sequences with Ncs=16, Root =1,2 |  | 16 ZC root sequences, Root=1~16 |  |
| --- | --- | --- | --- | --- |
|  | Miss detection rate | Non-target UE false wake up rate | Miss detection rate | Non-target UE false wake up rate |
| -2 | 0.0417 | 0.0217 | 0.0419 | 0.0361 |
| -1 | 0.0262 | 0.0338 | 0.0265 | 0.0565 |
| 0 | 0.0160 | 0.0524 | 0.0162 | 0.0879 |
| 1 | 0.0097 | 0.0780 | 0.0088 | 0.1335 |

From Table 14, it can be seen that

- Though the “Non-target UE false wake up rate” is decreased, the performance is still unacceptable;

- It causes the increase of MDR for target ZC sequence detection.

From the above simulation results, it is recommended that maximum number of candidate overlaid ZC sequences to carry LP-WUS information per OOK ON chip for one cell is 4. The 4 candidate overlaid ZC sequences is enumerated in increasing order of first increasing cyclic shift of a root ZC sequence and then in increasing order of the root index.

Observation 15: With the increase of number of ZC sequences overlaid on OOK-ON symbol, Non-target UE false wake up rate is increased seriously, especially when the overlaid ZC sequences are selected from multiple ZC root sequences. It will cause a large number of non-target UEs may be woken up and lead to a serious waste of UE power consumption.

Observation 16: A stricter restriction on FAR can be used to reduce the “Non-target UE false wake up rate”, but the penalty is that the target ZC sequence detection performance is also reduced.

Proposal 19: For OOK-4 with M=1 and M=2, maximum number of candidate overlaid sequences to carry LP-WUS information per OOK ON chip for one cell is 4.

Proposal 20: For OFDM sequence based LP-WUS , the overlaid ZC sequences is enumerated in increasing order of first increasing cyclic shift of a root ZC sequence and then in increasing order of the root index.

Overlaid ZC sequences design for M=2,4

Sequence length LZC

In RAN1 #119 meeting, RAN1 made following agreement.

Agreement At least for M>1, for the overlaid OFDM sequence in time domain, down-select the sequence length LZC from the following: - Alt1:, with - FFS how to map the generated overlaid OFDM sequence to the frequency domain - Alt2: - Note: X is the number of RBs of LP-WUS/LP-SS bandwidth (blanked guard RBs are not included)

For both Alt1 and Alt2, BZC is given by the largest prime number such that. The value of BZC based on different LZC is shown in Table 15.

Table 15: BZC Value for OOK-4 with M=2,4

| Solutions | M=2 |  | M=4 |  |
| --- | --- | --- | --- | --- |
|  | LZC | BZC | LZC | BZC |
| Alt 1 | 64 | 61 | 32 | 31 |
| Alt 2 | 66 | 61 | 33 | 31 |

It can be seen that no matter whether Alt1 or Alt2 is used, the value of BZC is the same. Moreover, cyclic shift elements of ZC sequence are used to supplement the sequence to the length of LZC. For Alt1, 3 or 1 additional sequence element are needed for M=2 or M=4. For Alt2, 5 or 2 additional sequence element are needed for M=2 or M=4. Obviously, for Alt2, more sequence elements need to be added, then the feature of orthogonality will be broken more seriously.

Observation 17: If cyclic shift elements of ZC sequence are used to supplement the sequence to the length of LZC,  cross-correlation feature will be broken since it it not a whole ZC sequence.

Proposal 21:For OOK-4 with M>1, Alt 1 is supported to decide the sequence length LZC for the overlaid OFDM sequence in time domain.

ZC Sequence length BZC

In RAN1#119 meeting, the following agreement is made

- M=2, 4, is down-selected from the following: - Alt1: is given by the largest prime number such that,is the overlaid OFDM sequence length. - The base overlaid sequenceis generated by extension of , - With CS(s) applied to the base overlaid OFDM sequence if any:denotes the potential cyclic shift (s) , - Note it doesn’t preclude any pulse shaping scheme if any. - Alt2:is given by the largest prime number such that, is the overlaid OFDM sequence length. - The base overlaid sequenceis generated by inserting zeros before and/or after . The total number of zeros is. For example,, - With CS(s) applied to the base overlaid OFDM sequence if any: denotes the potential cyclic shift (s) For example,, where, - FFS the value of G, with G>0. - FFS value(s) of root q and/or CS for generating the candidate sequences if applied - FFS on whether changes are needed to handle intra-/inter-cell interference - Note: the overlaid OFDM sequence in time domain is based on potential modification to ZC sequence as listed above. - Above overrides any previous RAN1 agreement / working assumption

In this section, a link level simulation is made to select the alternative of length of ZC sequences for OOK-4 with M=2,4

- Simulation assumptions

- SCS=30KHz

- Number of SCs=128

- No time error

- No frequency error

- OOK-4 with M=2

- 4 overlaid ZC sequences

- FAR =0.1%

- Support detection with sliding window for OFDM sequence receiver without FFT

  - Sliding window = [-24:24] sampling points

- Other simulation assumptions refer to Appendix A

- Simulation results

Figure 1: MDR performance for Alt1 and Alt2, OOK-4 with M=2

Based on the above simulation results, it can be seen that for OOK-4 with M=2,4, Alt2(inserting Gap) have the same ZC sequence detection performance with Alt1(No Gap).

Moreover, as analysis in section 5.3, it can be seen that inserting Gaps for each OOK symbols can significant improve OOK based LP-WUS detection performance with large UE time error.

Therefore, for OOK-4 with M=2,4, the overlaid OFDM sequence in time domain the sequence is generated based on Alt2.

Proposal 22:For OOK-4 with M=2,4, the overlaid OFDM sequence in time domain, the sequence is generated based on Alt2.

TO tolerance for OOK based LP-WUS

During the study of Rel-18 LP-WUS SID, it is a common understanding by most companies that OOK-1 or OOK-4 with M=1 has better tolerant to time error than OOK-4 with M>1 since each OOK symbol has CP to suppress the time error caused by the frequency drift of oscillator of LP-WUR. But from our perspective of view, frequency drift of oscillator has two directions, it can cause the corresponding timing drift (ΔT) over a time(T) also having two directions, as in TR 38.869 the formula of ΔT is defined as ΔT = Fr*T ±0.5 * F’ *T2 (transient region).

- Figure 3: A diagram of OOK-1 based LP-WUS detection with time error

As shown in the above figure, CP of OOK-1 can solve the time error with maximum negative value smaller than the length of CP, and thus the detection performance will not be decreased. But for the case of time error with positive value, the CP can not solve it and the detection performance will be decreased.

Moreover, a simulation is made to evaluate OOK-1 performance with time error caused by the frequency drift of oscillator of LP-WUR.

- Simulation assumptions

- 8 bits Payload size+8bit CRC

- LP-WUS BW=12RB (not including guardband)

- Guardband=1PRB

- SCS=30KHz

- OOK=1 or OOK-4 with M=1

- Manchester 1/2

- Not support envelope detection with sliding window in LP-WUS receiver

- Other simulation assumptions refer to Appendix A

- Simulation results

Figure 4: OOK-1 detection performance with time error

The simulation results are shown in Figure 4, it can be seen that time error with positive value has significant impact on OOK-1 detection performance

Observation 18: Time error with positive value has significant impact on OOK-1 detection performance

Moreover, for OOK-4 with M>1, since only the first OOK has a CP, the time error with both positive and negative has significant impact on OOK-4 detection performance.

- Preferred solution

To solve the problem of time error, it is proposed to insert Gaps at the beginning and the end of each OOK symbol, as illustrated in Figure 5, taking OOK-4 with M=2 for example, a Gap with length of G0 is added at the beginning and the end of each OOK symbol. The value of G0 is determined by the maximum value of the UE time error.

Figure 5: OOK-4 with Gap inserted within each OOK symbol

- TX:

Based on the time domain based OOK waveform generation described in section 2, a gap with length of G1 is inserted into Esi before DFT and then Esi is changed to

Esi =0,0, ⋯,0G1 , wi0∙si,wi1∙si,⋯,wiA−1∙siA,0,0, ⋯,0G1

Wherein, the length of G1 is depended on the length of G0, and a simple conversion formula can be

G1=G0/(NFFT/NSC,LP-WUS)

- RX:

- No additional operation is introduced for LP-WUS detection, normal detection procedure including step1 and step2 is used and is listed bellow.

- Step1: CP is dropped when OFDM symbol is received.

Step 2: OOK envelope detection is made for each OOK symbol.

Figure 6: LP-WUS envelope detection with UE time error

- Simulation assumptions

- 8 bits Payload size+8bit CRC

- LP-WUS BW=128 SCs

- Guardband=1PRB

- SCS=30KHz

- OOK-4 with M=2

- Manchester 1/2

- Not support envelope detection with sliding window in LP-WUS receiver

- Inserted gap length of G0 is 72Ts.

- Other simulation assumptions refer to Appendix A

- Simulation results

Figure 7: OOK-4 with M=2, Manchester 1/2, LP-WUS envelope detection results with/without UE time error

From the above simulation results, it can be seen that inserting Gaps for each OOK symbols can significant improve OOK envelope detection performance with large UE time error.

Proposal 23: For OOK-1/OOK-4, Gap can be inserted in the beginning and ending of each OOK symbol to solve the problem of LP-WUS detection performance loss caused by larger UE time error.

How to carry LP-WUS information for OOK

In RAN1#118b meeting, the following agreement was made for idle mode

Agreement Down-select among the following alternatives for LP-WUS information for OOK to meet target requirements: - adding CRC - channel coding other than Manchester coding, including rate matching if any - mapped to a binary sequence/codeword, include truncation if any - repetition, including rate matching if any - Note: combination of options above is not precluded - Note: combination of Manchester coding with above is assumed FFS how LP-WUS is transmitted within a MO/MOs

- Adding CRC

CRC is necessary for OOK based LP-WUS information transmission. The CRC can be used to guarantee FAR depending on the length of CRC bits. In Table 16, based on different set of {Payload size, CRC length}, corresponding FAR results is evaluated. In the evaluation, OOK-4 with M=2 and Manchester code rate =0.5 is configured.

Table 16: FAR results based on different set of {Payload size, CRC length}

| Index | Encoded bits | CRC length | FAR |
| --- | --- | --- | --- |
| 0 | 5 | 6 | 0.0150 |
| 1 | 6 | 6 | 0.0156 |
| 2 | 8 | 6 | 0.0158 |
| 3 | 5 | 8 | 0.0037 |
| 4 | 6 | 8 | 0.0038 |
| 5 | 8 | 8 | 0.0040 |

From the evaluation results, it can be seen that when encoded bits is smaller than 8 bits, 8-length CRC can ensure FAR<1%.

Observation 19: For LP-WUS consist of encoded bits and CRC, when encoded bits is smaller than 8 bits, 8-length CRC can ensure FAR<1%.

- Repetition

Additionally, as analysis in section 6.1.2, for OOK based LP-WUS, repetition can provide significant coverage gain. If LP-WUS coverage can not meet the requirement based on existing technologies, repetition can be used to enhance the coverage level. However, assuming the same LP-WUS bits, if repetition is used for LP-WUS, we do not think the hamming distance can be guaranteed for the FAR target.

Observation 20: For OOK based LP-WUS, repetition can provide significant coverage gain but hamming distance can not be guaranteed.

- Mapped to a binary sequence/codeword

Take LP-WUS information bits size equals to 5bits for example, 2^5=32 binary sequences are necessary to be configured. In order to ensure good binary sequence detection performance, it is recommended to configure 32 orthogonal binary sequences, thus the length of the binary sequence should be equal or larger than 32.

For example, the 32 binary sequences are selected from a 32x32 Hadamard Matrix. In order to maintain same powering boosting level of multiple OFDM symbols allocated for LP-WUS, same number of OOK-ON symbols within one OFDM symbol should be ensured. Thus, there are only 24 binary sequences can be used as binary sequence from the 32x32 Hadamard Matrix. To make up 32 binary sequences, 8 non-orthogonal 32-length binary sequences can be included or extend the size of Hadamard Matrix larger than 32.

If non-orthogonal binary sequences are configured, the binary sequence detection performance will be reduced since the cross-correlation results become larger than orthogonal binary sequences. Moreover, if the 32 binary sequences are all non-orthogonal, the length of the binary sequence may smaller than 32, but the binary sequence detection performance will be reduced seriously.In order to ensure good detection performance, much higher Tx power or larger LP-WUS resources may be necessary.

When LP-WUS information bits size is larger than 5bits, such as 6 or 8bits, the required binary sequence resource(including sequence length ) may become larger.

Careful evaluation should be taken for “LP-WUS consist of encoded bits and CRC” and “LP-WUS is mapped to binary sequence/codeword” in terms of resource overhead and detection performance(at least including MDR and FAR)

Proposal 24: Evaluation should be taken for “LP-WUS consist of encoded bits and CRC” and “LP-WUS is mapped to binary sequence/codeword” in terms of resource overhead and detection performance(at least including MDR and FAR)

The following agreement was made for connected mode.

Agreement Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs (for non-CA case), select at least one from the following - Option 1: A bitmap with each bit corresponding to [one or more] UEs - Option 3: A codepoint value corresponding to [one or more] UEs - FFS details for extension of option 1 and/or 3 when UE is configured with CA

Bitmap method is simple and can help save more system overhead, whcih should be prioritized. For the codepoint method, if binary sequence or other detailed schemes should be studied further, we feel it would be quite complicated especially when we consider this together with CA.

Proposal 25: In connected mode, bitmap method for LP-WUS to wake up UE is preferred.

Coverage improvement to achieve coverage of PUSCH for Msg3

In RAN1#117 meeting, the following agreement on coverage performance was made

Agreement For evaluation of LP-WUS and LP-SS design to achieve coverage of PUSCH for message3 from RAN1 perspective, at least the following SNR values should be considered: - SNR=-3dB for NF of LR = NF of MR+ 8dB - SNR= -0.5dB for NF of LR = NF of MR+ 5dB - SNR=2dB for NF of LR = NF of MR+ 2dB - Note 1: The NF of MR is assumed as 7dB

As our OOK based LP-WUR simulation results shown in section 6.3.1, SNR at about 0.8dB can achieve BLER=1% for LP-WUS carrying 8bits information and 8 bits CRC. In order to achieve the coverage of PUSCH Msg3, at least [1.2dB, 3.8dB] coverage improvement should be supported according to different NF value of LR.

In this section, some coverage improvement schemes are discussed and preliminary simulation results are also provided to show the detection performance gain.

Detection with sliding window

- Simulation assumptions

- 8-bit payload size+8bit CRC

- LP-WUS BW=12RB (not including guardband)

- Guardband=1PRB

- SCS=30KHz

- M=2

- Manchester 1/2

- Support envelope detection with sliding window in LP-WUS receiver

  - Sliding window = [-72:12:72] sampling points

- Other simulation assumptions refer to Appendix A

- Simulation results

Figure 8: OOK based LP-WUS detection performance with/without sliding window

Table 17: Summary of OOK based LP-WUS detection performance with/without sliding window

| SNR@BLER=1% (dB) | OOK-4, M=2, No sliding window | OOK-4, M=2, with sliding window |
| --- | --- | --- |
| BW = 12RBs | 1.89 | 0.61 |

Observation 21: For OOK based LP-WUS, LP-WUR with sliding window can provide about 1.28dB coverage gain.

Repetition

- Simulation assumptions

- 8-bit payload size+8bit CRC

- LP-WUS BW=12RB (not including guardband)

- Guardband=1PRB

- SCS=30KHz

- M=2

- Manchester 1/2

- Not support envelope detection with sliding window in LP-WUS receiver

- LP-WUS transmission with repetitions

- Other simulation assumptions refer to Appendix A

- Simulation results

Case 1: Repetition is performed for the whole payload information

Figure 9: OOK based LP-WUS detection performance with whole payload information repetition

Table 18: Summary of OOK based LP-WUS detection performance with whole payload information repetition

| SNR@BLER=1% (dB) | OOK-4, M=2, | OOK-4, M=2, Repetition=2 | OOK-4, M=2, Repetition=4 | OOK-4, M=2, Repetition=8 |
| --- | --- | --- | --- | --- |
| BW = 12RBs | 1.89 | 0.72 | -0.82 | -2.26 |

Case 2: Repetition is performed per bit information

Figure 10: OOK based LP-WUS detection performance with whole bit information repetition

Table 19: Summary of OOK based LP-WUS detection performance with whole payload information repetition

| SNR@BLER=1% (dB) | OOK-4, M=2, | OOK-4, M=2, BitRepetition=2 | OOK-4, M=2, BitRepetition=4 | OOK-4, M=2, BitRepetition=8 |
| --- | --- | --- | --- | --- |
| BW = 12RBs | 1.89 | 0.21 | -1.82 | -3.75 |

Observation 22: For OOK based LP-WUS, repetition can provide significant coverage gain.

Observation 23: For OOK based LP-WUS, bit repetition has better detection performance than whole payload repetition.

Frequency hopping

- Simulation assumptions

- 8 bits payload size+8bit CRC

- LP-WUS BW=12RB (not including guardband)

- Guardband=1PRB

- SCS=30KHz

- M=2

- Manchester 1/2

- Not support envelope detection with sliding window in LP-WUS receiver

- LP-WUS transmission with repetitions

- Frequency hopping for the repetitions of whole payload information

- Other simulation assumptions refer to Appendix A

- Simulation results

Figure 11: OOK based LP-WUS detection performance with frequency hopping

Table 20: Summary of OOK based LP-WUS detection performance with frequency hopping

| SNR@BLER=1% (dB) | OOK-4, M=2, | OOK-4, M=2, Repetition=2，FH | OOK-4, M=2, Repetition=4，FH | OOK-4, M=2, Repetition=8，FH |
| --- | --- | --- | --- | --- |
| BW = 12RBs | 1.89 | -1.09 | -2.6 | -3.55 |

Observation 24: For OOK based LP-WUS, frequency hopping with repetition can provide significant coverage improvement gain.

Proposal 26: For OOK based LP-WUS, at least the following coverage improvement schemes should be further studied to achieve the coverage of PUSCH Msg3:

- Detection with sliding window

- Bit level repetition is prioritized

- Frequency hopping

Conclusions

In this contribution, we have discussed issues on waveform design and signal structure for LP-WUS. We make the following observations and proposals:

Observation 1: For OOK based LP-WUR, the SCS of LP-WUS/LP-SS could be the same as initial DL BWP, since OOK based WUR is not needed to monitor SSS and it may be beneficial for the paging reception.

Observation 2: For OFDM sequence based LP-WUR, if it can support different SCS like NR UE, the SCS of LP-WUS/LP-SS could be the same as initial DL BWP; otherwise, the SCS of LP-WUS/LP-SS is better to be the same as CD-SSB.

- Observation 3: For LP-SS, to configure equal number of OOK-ON symbols (carrying bit “1”) and OOK-OFF symbols (carrying bit “0”) within M OOK symbols for one OFDM symbol has benefits of

- Ensuring the power control implementation at gNB side unchanged;

- Improving LP-RSRP/LP-RSRQ calculation accuracy;

- Ensuring the AGC performance;

Observation 4: For OOK-4 with M=1 and 4-length binary sequences, only three 4-length binary sequences can meet the requirement of TO correction accuracy <=5us.

- Observation 5: For OOK-4 with M=1 and 6-length binary sequences

- For mismatch/bipolar detection, there are 9 binary sequences with index 0,1,2,3,5,6,7,8,10 can meet the requirement of TO correction accuracy <5us.

- For unipolar detection, there are 10 binary sequences with index 0,1,2,3,4,5,6,7,8,10 can meet the requirement of TO correction accuracy <5us.

- Observation 6: For OOK-4 with M=1 and 8-length binary sequences

- For mismatch/bipolar detection, all the 16 binary sequences can meet the requirement of TO correction accuracy <5us.

- For unipolar detection, all the 16 binary sequences can meet the requirement of TO correction accuracy <5us.

- Observation 7: For OOK-4 with M=2 and 8-length binary sequences

- For mismatch/bipolar detection, there are 3 binary sequences with index 1,3,4 can meet the requirement of TO correction accuracy <2us.

- For unipolar detection, there are 2 binary sequences with index 1,5 can meet the requirement of TO correction accuracy <2us.

Observation 8: For OOK-4 with M=2 and 8-length binary sequences, only two or three 8-length binary sequences can meet the requirement of TO correction accuracy <=2us.

- Observation 9: For OOK-4 with M=1 and 8-length binary sequences

- For mismatch/bipolar detection, all the 20 binary sequences can meet the requirement of TO correction accuracy <2us.

- For mismatch/bipolar detection, 4 binary sequences with index 0,1,2,13 have the best TO correction accuracy.

- For unipolar detection, there are 18 binary sequences with index 0,1,2,3,4,5,8,9,10,11,12,13,14,15,17,18,19 can meet the requirement of TO correction accuracy <2us.

- For unipolar detection, 4 binary sequences with index 1,2,11,12 have the best TO correction accuracy.

- Observation 10: For OOK-4 with M=2 and 16-length binary sequences

- For mismatch/bipolar detection, all the 36 binary sequences can meet the requirement of TO correction accuracy <2us.

- For mismatch/bipolar detection, 4 binary sequences with index 2,3,30,31 have the best TO correction accuracy.

- For unipolar detection, except sequences with index 9,11, there are 34 binary sequences can meet the requirement of TO correction accuracy <2us.

- For unipolar detection, 4 binary sequences with index 1,5,28,29 have the best TO correction accuracy.

- Observation 11: For OOK-4 with M=4 and 16-length binary sequences

- For mismatch/bipolar detection, binary sequences with index 0,2,3,9,16,17,18,19,20,22,23,24,32,34can meet the requirement of TO correction accuracy <1us.

- For mismatch/bipolar detection, 4 binary sequences with index 16,17,18,19 have the best TO correction accuracy.

- For unipolar detection, binary sequences with index 0,2,9,15,16,18,19,20,21,22,23,24,28 can meet the requirement of TO correction accuracy <1us.

- For unipolar detection, 4 binary sequences with index 20,21,22,23 have the best TO correction accuracy.

- Observation 12: For OOK-4 with M=4 and 32-length binary sequences

- For mismatch/bipolar detection, except binary sequences with index 28, other 31 binary sequences can meet the requirement of TO correction accuracy <1us.

- For mismatch/bipolar detection, 4 binary sequences with index 8,9,10,11 have the best TO correction accuracy.

- For unipolar detection, except binary sequence with index 2,24,25,28,30, other binary sequences can meet the requirement of TO correction accuracy <1us.

- For unipolar detection, 4 binary sequences with index 4,8,12,14 have the best TO correction accuracy.

Observation 13: For LP-SS periodicity with 80ms,160ms and 320ms, there are many cases that may cause residual time error before LP-WUS detection larger than the tolerance of the corresponding LP-WUS structure.

Observation 14: Compared with reducing LP-SS periodicity to satisfy sync performance, additional sync signal has less system overhead in some scenarios.

Observation 15: With the increase of number of ZC sequences overlaid on OOK-ON symbol, Non-target UE false wake up rate is increased seriously, especially when the overlaid ZC sequences are selected from multiple ZC root sequences. It will cause a large number of non-target UEs may be woken up and lead to a serious waste of UE power consumption.

Observation 16: A stricter restriction on FAR can be used to reduce the “Non-target UE false wake up rate”, but the penalty is that the target ZC sequence detection performance is also reduced.

Observation 17: If cyclic shift elements of ZC sequence are used to supplement the sequence to the length of LZC,  cross-correlation feature will be broken since it it not a whole ZC sequence.

Observation 18: Time error with positive value has significant impact on OOK-1 detection performance

Observation 19: For LP-WUS consist of encoded bits and CRC, when encoded bits is smaller than 8 bits, 8-length CRC can ensure FAR<1%.

Observation 20: For OOK based LP-WUS, repetition can provide significant coverage gain but hamming distance can not be guaranteed.

Observation 21: For OOK based LP-WUS, LP-WUR with sliding window can provide about 1.28dB coverage gain.

Observation 22: For OOK based LP-WUS, repetition can provide significant coverage gain.

Observation 23: For OOK based LP-WUS, bit repetition has better detection performance than whole payload repetition.

Observation 24: For OOK based LP-WUS, frequency hopping with repetition can provide significant coverage improvement gain.

Proposal 1: Confirm the working assumption in below agreement:

Agreement For OOK-4 with M >1, support M=2 & M=4 (working assumption) for LP-WUS. - M=4 for 15KHz SCS - M=4 for 30KHz SCS (working assumption) FFS M=1 for OOK-4

Proposal 2: For the M value for LP-WUS and LP-SS, it should be configured by gNB respectively.

Proposal 3: Confirm the working assumption on overlaid OFDM sequence(s) of LP-WUS.

Working Assumption For overlaid OFDM sequences for LP-WUS in time or frequency domain, for OOK-1 and OOK-4 M=1 - Option 1-1(compromised): overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing for OOK-4 with M=1. - ZC sequence in time domainDFT/LSIFFT to result in a frequency domain sequence that is ZC sequence or a sequence that is close to ZC sequence, no additional operation - It doesn’t preclude additional operation for OOK-4 with M>1. - DFT size is 2^n Note1: OOK-1 is considered as specific case of OOK-4 with M=1.

- Proposal 4:Candidate LP-SS binary sequences should satisfy that

- For OOK-4 with M>1, equal number of OOK-ON symbols (carrying bit “1”) and OOK-OFF symbols (carrying bit “0”) within M OOK symbols of one OFDM symbol for OOK-4 with M>1.

- For OOK-4 with M=1, equal number of OOK-ON symbols (carrying bit “1”) and OOK-OFF symbols (carrying bit “0”) within the binary sequence.

Proposal 5: For candidate binary sequences cross-check,

- For OOK-4 with a M value, under the premise of meeting the time offset requirement, the shortest binary sequence should be selected as LP-SS.

- For OOK-4 with a M value and a given length of binary sequence,

  - Common candidate binary sequences(if any) that have best detection performance under different LP-SS detection method are preferred as LP-SS.

    - If the number of the common candidate binary sequences are less than 4, a tradeoff between binary sequences with different detection methods should be considered for sequence selection.

Proposal 6:For OOK-4 with M=1, the following four 6-length binary sequences can be used as LP-SS.

| Sequence index | Candidate sequences |
| --- | --- |
| 0 | 1 0 1 0 0 1 |
| 1 | 1 0 0 1 0 1 |
| 2 | 0 1 0 1 0 1 |
| 3 | 1 0 1 0 1 0 |

Proposal 7:For OOK-4 with M=1, if 8-length binary sequences is considered, the following 4 sequences be used.

| Sequence index | Candidate sequences |
| --- | --- |
| 0 | 1 0 1 0 1 0 0 1 |
| 1 | 1 0 1 0 0 1 0 1 |
| 2 | 1 0 0 1 0 1 0 1 |
| 4 | 0 1 0 1 0 1 0 1 |

Proposal 8:For OOK-4 with M=2 and mismatch/bipolar detection, the following four 12-length binary sequences can be used as LP-SS.

| Sequence index | Candidate sequences |
| --- | --- |
| 0 | 0 1 1 0 0 1 0 1 1 0 0 1 |
| 1 | 0 1 1 0 0 1 1 0 1 0 0 1 |
| 2 | 0 1 1 0 1 0 0 1 1 0 0 1 |
| 13 | 1 0 0 1 1 0 0 1 1 0 0 1 |

Proposal 9:For OOK-4 with M=2 and unipolar detection, the following four 12-length binary sequences can be used as LP-SS.

| Sequence index | Candidate sequences |
| --- | --- |
| 1 | 0 1 1 0 0 1 1 0 1 0 0 1 |
| 2 | 0 1 1 0 1 0 0 1 1 0 0 1 |
| 11 | 1 0 0 1 1 0 1 0 1 0 0 1 |
| 12 | 1 0 1 0 1 0 0 1 1 0 0 1 |

Proposal 10:For OOK-4 with M=2 and mismatch/bipolar detection, the following four 16-length binary sequences can be used as LP-SS.

| Sequence index | Candidate sequences |
| --- | --- |
| 2 | 1 0 0 1 1 0 0 1 1 0 0 1 1 0 0 1 |
| 3 | 0 1 1 0 0 1 1 0 0 1 1 0 0 1 1 0 |
| 30 | 0 1 1 0 1 0 0 1 1 0 0 1 1 0 0 1 |
| 31 | 0 1 1 0 0 1 1 0 0 1 1 0 1 0 0 1 |

Proposal 11:For OOK-4 with M=2 and unipolar detection, the following four 16-length binary sequences can be used as LP-SS.

| Sequence index | Candidate sequences |
| --- | --- |
| 1 | 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 |
| 5 | 1 0 1 0 0 1 1 0 1 0 1 0 1 0 0 1 |
| 28 | 1 0 1 0 1 0 1 0 0 1 1 0 1 0 1 0 |
| 29 | 0 1 1 0 1 0 0 1 1 0 1 0 1 0 0 1 |

Proposal 12:For OOK-4 with M=4 and mismatch/bipolar detection, the following four 16-length binary sequences can be used as LP-SS.

| Sequence index | Candidate sequences |
| --- | --- |
| 16 | 0 1 1 0 0 1 0 1 1 0 1 0 0 1 0 1 |
| 17 | 1 1 0 0 0 1 0 1 1 0 1 0 0 0 1 1 |
| 18 | 1 0 1 0 0 1 0 1 1 0 1 0 0 0 1 1 |
| 19 | 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1 |

Proposal 13:For OOK-4 with M=4 and unipolar detection, the following four 16-length binary sequences can be used as LP-SS.

| Sequence index | Candidate sequences |
| --- | --- |
| 20 | 1 0 1 0 1 0 0 1 1 0 1 0 1 0 1 0 |
| 21 | 1 0 1 0 1 1 0 0 1 0 1 0 1 0 1 0 |
| 22 | 1 0 1 0 0 1 0 1 1 0 1 0 0 1 1 0 |
| 23 | 1 0 0 1 1 0 1 0 1 0 1 0 1 0 0 1 |

Proposal 14:For OOK-4 with M=4 and mismatch/bipolar detection, the following four 32-length binary sequences can be used as LP-SS.

| Sequence index | Candidate sequences |
| --- | --- |
| 8 | 1 0 1 0 0 1 0 1 1 0 1 0 0 1 0 1 1 0 1 0 0 0 1 1 1 1 0 0 0 0 1 1 |
| 9 | 0 0 1 1 1 1 0 0 0 0 1 1 1 1 0 0 1 0 1 0 0 1 0 1 1 0 1 0 0 1 0 1 |
| 10 | 1 0 1 0 0 1 0 1 1 0 1 0 0 1 0 1 0 1 1 0 1 0 0 1 1 1 0 0 0 0 1 1 |
| 11 | 1 0 1 0 0 1 0 1 1 0 1 0 0 1 0 1 1 1 0 0 0 0 1 1 1 1 0 0 0 0 1 1 |

Proposal 15:For OOK-4 with M=4 and unipolar detection, the following four 32-length binary sequences can be used as LP-SS.

| Sequence index | Candidate sequences |
| --- | --- |
| 4 | 0 1 1 0 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1 1 0 1 0 1 0 0 1 0 1 1 0 |
| 8 | 1 0 1 0 0 1 0 1 1 0 1 0 0 1 0 1 1 0 1 0 0 0 1 1 1 1 0 0 0 0 1 1 |
| 12 | 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 0 1 0 1 |
| 14 | 1 1 0 0 0 1 1 0 1 0 1 0 0 1 0 1 1 0 1 0 0 1 0 1 0 1 1 0 0 0 1 1 |

Proposal 16: Support additional sync signal to LP-SS at least for LP-SS periodicity =320ms.

- Proposal 17: For OFDM sequence based LR, additional sync signal is not necessary when LP-SS periodicity is not larger than 320ms.

Proposal 18: Support additional sync signal design, and select binary sequence from candidate binary sequences for LP-SS.

Proposal 19: For OOK-4 with M=1 and M=2, maximum number of candidate overlaid sequences to carry LP-WUS information per OOK ON chip for one cell is 4.

Proposal 20: For OFDM sequence based LP-WUS , the overlaid ZC sequences is enumerated in increasing order of first increasing cyclic shift of a root ZC sequence and then in increasing order of the root index.

Proposal 21:For OOK-4 with M>1, Alt 1 is supported to decide the sequence length LZC for the overlaid OFDM sequence in time domain.

Proposal 22:For OOK-4 with M=2,4, the overlaid OFDM sequence in time domain, the sequence is generated based on Alt2.

Proposal 23: For OOK-1/OOK-4, Gap can be inserted in the beginning and ending of each OOK symbol to solve the problem of LP-WUS detection performance loss caused by larger UE time error.

Proposal 24: Evaluation should be taken for “LP-WUS consist of encoded bits and CRC” and “LP-WUS is mapped to binary sequence/codeword” in terms of resource overhead and detection performance(at least including MDR and FAR)

Proposal 25: In connected mode, bitmap method for LP-WUS to wake up UE is preferred.

Proposal 26: For OOK based LP-WUS, at least the following coverage improvement schemes should be further studied to achieve the coverage of PUSCH Msg3:

- Detection with sliding window

- Bit level repetition is prioritized

- Frequency hopping

References

RP-241824, “Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, RAN#105.

Draft_Minutes_report_RAN1#119_v030

R1-2410939, “Collection of LP-SS binary sequences”, vivo, RAN1#120

- Appendix A:Simulation assumptions for LP-WUS

Table A-1. Simulation assumptions for LP-WUS

| Attributes | Assumptions |
| --- | --- |
| Carrier Frequency | 2.6GHz |
| SCS of OFDM generator for NR signal | 30kHz |
| gNB Channel BW | 20MHz and with Normal CP=2.344us。 |
| Filter | 5rd order Butterworth LPF |
| Sampling Rate | 7.68 MHz |
| ADC bit width | 4bits ADC |
| Channel Model | TDL-C 300ns |
| Number of Rx for LP-WUS | 1 Rx |
| UE speed | 3 km/h |
| Inter-cell interference | No |
| Phase noise modeling | No |