3GPP TSG RAN WG1 #120			R1-2500149

Athens, Greece, February 17th – 21st, 2025

Source:	ZTE

Title:	Discussion on UE features for Rel-19 SBFD

Agenda item:	9.15.3

Document for:	Discussion and Decision

Introduction

In this contribution, we provide our views on UE features for Rel-19 SBFD including basic transmission/reception behaviour procedures, RACH in SBFD symbols and L1 UE-UE CLI handling.

UE features for transmission/reception behaviour and procedures

Basic UE feature for SBFD operation

Based on the discussion, cell-specific configuration of time and frequency domain location for SBFD subbands is supported. The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one. Furthermore, only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s). In addition, a UE should support resource allocation in frequency domain for PDSCH within one or two DL suband in SBFD symbols. For CSI-RS, a UE should at least support a contiguous CSI-RS resource without overlaps with SBFD subband boundaries or across two DL subbands. The subband boundary may not align with the boundary of RBG, CSI reporting subband and PRG etc. The last but not the least, it was agreed that, for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots, Configuration 1 is a default UE capability.

In our views, at least above behaviour should be the basic or prerequisite feature for all SBFD operations. As a result, we have the following proposal.

Proposal 1: Support the following basic FG for SBFD operation in RRC_CONNECTED mode as a baseline.

| Index | Feature group | Components | Prerequisite feature groups | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| x-1 | Basic SBFD operation in RRC_CONNECTED mode | 1) Semi-static indication of time and frequency location of SBFD subbands 2) Resource allocation in frequency domain for PDSCH within one or two DL subands and for PUSCH within one UL subband. 3) A contiguous CSI-RS resource without overlapping with SBFD subband boundaries or across two DL subbands 4) Unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG 5) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols), support Configuration 1: the transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only |  | [Per UE] | TDD only | n/a | 1) The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one. 2) Only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s) |

Resource allocation in frequency domain for CSI-RS

In AI 9.3.1, the following agreement was reached for CSI-RS resource overlaps with SBFD subband boundaries or across two DL subbands.

Working Assumption For frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, support one contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL usable PRBs. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured) RAN1 to further study the impact on CSI processing timeline in SBFD symbols to process the CSI-RS across the two DL subbands Agreement For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, - Option 3: It is up to UE capability to indicate one of the following: - CSI processing timeline and CPU counting are the same as legacy. - Scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy. Agreement For a contiguous CSI-RS resource which overlaps with SBFD subband boundaries, only CSI-RS frequency resources within DL usable PRBs are valid for SBFD-aware. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured)

With above WA and agreements, we propose the following FG.

Proposal 2: Support the following FG for resource allocation in frequency domain for CSI-RS.

| Index | Feature group | Components | Prerequisite feature groups | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| x-2 | Resource allocation in frequency domain for CSI-RS | 1) A contiguous CSI-RS resource overlaps with SBFD subband boundaries 2) A contiguous CSI-RS resource across two DL subbands, where the CSI processing timeline and CPU counting are reported from {legacy, scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy}. | x-1 | [Per UE] | TDD only | n/a |  |

Partial PRG

For the maximum number of supported partial PRGs in SBFD symbols, the following was agreed.

Agreement For a PRG that overlaps the subband boundary, the part of the DL PRG inside the DL subband can be used subject to UE capability - UE capability reports the maximum number of supported partial PRGs in SBFD symbols.

Based on our contribution submitted under AI 9.3.1, we propose that the maximum number of partial PRGs reported by a UE can be 2 or 4. If two DL subbands are configured and value 2 is reported by the UE, a PDSCH transmission is valid as long as it only contains up to 2 partial PRGs, no matter the partial PRG are caused by the boundary of BWP as legacy or it is caused by the boundary of DL subband. Otherwise, a PDSCH transmission is valid no matter which and how many partial PRGs the PDSCH contains. With above, we have the following proposal.

Proposal 3: Support the following FG for partial PRG.

| Index | Feature group | Components | Prerequisite feature groups | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| x-3 | Support of partial PRG | Support of partial PRG in SBFD symbols. The maximum number of supported partial PRGs in SBFD symbols is {2, 4} | x-1 | [Per UE] | TDD only | n/a |  |

Configuration 2 for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots.

For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots, it was agreed that Configuration 1 is the default capability and support of Configuration 2 is subject to UE capability. So, support of Configuration 1 is included in the basic FG x-1 as shown above. A separate UE capability is needed for Configuration 2.

Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, - Support of configuration 2 is subject to UE capability. Configuration 1 is the default capability. - SBFD-aware UE can be configured with Configuration 2 on a per UL/DL BWP basis. - The configuration for DL BWP at least applies to PDSCH receptions within the DL BWP. - The configuration for UL BWP applies to PUCCH and PUSCH transmissions within the UL BWP. - For SRS, only Configuration 1 is applicable.

Proposal 4: For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots, support the following FG for Configuration 2.

| Index | Feature group | Components | Prerequisite feature groups | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| x-4 | UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots | For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols), support Configuration 2: the transmissions/receptions can be in SBFD symbols and non-SBFD symbols [FFS other components pending on further progress in AI 9.3.1] | x-1 | [Per UE] | TDD only | n/a |  |

Others

It expects separate UE feature groups for other functionalities under AI 9.3.1 are needed, such as CSI reporting and separate configurations etc. At this stage, we prefer to wait for more progress on these aspects in AI 9.3.1 and further discuss the corresponding UE feature groups in subsequent meetings.

UE features for RACH procedure

Similar as enhancements to RACH procedure in previous releases such as Msg3 reptition and PRACH repetition, it is beneficial to let UEs to report their corresponding UE capabilities on the RACH enhancements once entering RRC connected mode. This allows NW to be aware of the number of UEs supporting the enhanced RACH operations so that NW can conduct better RACH configurations.

In AI 9.3.2, the following agreement was reached. Basically, two options are supported for RACH operation in SBFD symbols, while a UE is not required to support both options. With this context, we have the following proposal.

Agreement Confirm the following working assumption. Working Assumption For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported. - For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc. - For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc. UE is not required to support both options.

Proposal 5: For random access in SBFD symbols in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode, support the following two FGs.

| Index | Feature group | Components | Prerequisite feature groups | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| x-5 | Support random access in SBFD symbols via one single RACH configuration | Support of random access in SBFD symbols in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode via Option 1: use one single RACH configuration | x-1 | [Per UE] | TDD only | n/a |  |
| x-6 | Random access in SBFD symbols via two separate RACH configurations | Support of random access in SBFD symbols in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode via Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration | x-1 | [Per UE] | TDD only | n/a |  |

UE features for L1 UE-UE CLI handling

UL resource muting

UL resource muting is introduced under AI 9.3.3. A UE capability is needed to indicate whether the UE supports UL resource muting. It was agreed that the UE capability is separate for CP-OFDM and DFT-S-OFDM. Therefore, two FGs are needed.

Agreement If non-transparent UL resource muting is supported for gNB-to-gNB CLI handling, Option 1 is recommended to be specified - Option 1: Comb-2 for both DFT-S-OFDM and CP-OFDM - Note: Additional details can be discussed within RAN1#117. - Note: Power boosting is assumed for REs in the symbol with UL resource muting. FFS: Details. - Above is subject to separate UE capability (including separate capabilities for DFT-S-OFDM and CP-OFDM).

For UL resource muting, the time location and the frequency location are configured by the RRC signalling. For DG PUSCH and Type-2 CG PUSCH, the DCI can dynamically enable the UL resource muting. In addition, UL resource muting is applied to each PUSCH repetition.  Since UL resource muting is not related to the frequency band, the UE capability should be reported per UE. Considering that UL resource muting is introduced for SBFD and SBFD is deployed in TDD cell, the UE capability is for TDD only.

Proposal 6: Support the following two FGs for UL resource muting.

| Index | Feature group | Components | Prerequisite feature groups | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| x-7 | UL resource muting for CP-OFDM | 1) The UE supports UL resource muting for PUSCH with CP-OFDM waveform. 2) The UE supports UL resource muting for PUSCH repetitions with CP-OFDM. 3) The UL resource muting is dynamically enabled by DCI for DG PUSCH and Type-2 CG PUSCH. 4) At most two OFDM symbols are configured for UL resource muting. 5) Comb-2 offset is configured for UL resource muting. | x-1 | Per UE | TDD only | n/a |  |
| x-8 | UL resource muting for DFT-S-OFDM | 1) The UE supports UL resource muting for PUSCH with DFT-S-OFDM waveform. 2) The UE supports UL resource muting for PUSCH repetitions with DFT-S-OFDM waveform. 3) The UL resource muting is dynamically enabled by DCI for DG PUSCH and Type-2 CG PUSCH. 4) At most two OFDM symbols are configured for UL resource muting. 5) Comb-2 offset is configured for UL resource muting. | x-1 | Per UE | TDD only | n/a |  |

L1-based CLI measurement and reporting

For L1-based CLI measurement and reporting, two metrics are supported, i.e., SRS-RSRP and CLI-RSSI. For SRS-RSRP measurement, the victim UE measures the SRS transmitted from the aggressor UE to obtain the RSRP. For CLI-RSSI measurement, the victim UE measures the RSSI resources configured by the gNB to obtain the RSSI. Therefore, two FGs can be introduced. Note this is in line with the UE features for CLI in Rel-16 as shown in the appendix.

For CLI-RSSI measurement, two measurement methods are supported. For each CLI report, all the resource types should be supported. In addition, similar as UL resource muting, L1-based CLI measurement and reporting is only applicable to TDD.

Proposal 7: Support the following two FGs for L1-based CLI measurement and reporting.

| Index | Feature group | Components | Prerequisite feature groups | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| x-9 | L1-based SRS-RSRP measurement and reporting | 1) Support L1 SRS-RSRP measurement within UL subband. 2) Support periodic, semi-persistent, and aperiodic SRS measurement resource. 2) Support periodic L1 SRS-RSRP reporting in PUCCH. 3) Support semi-persistent L1 SRS-RSRP reporting in PUCCH and PUSCH. 3) Support aperiodic L1 SRS-RSRP reporting in PUSCH. | x-1 | Per UE | TDD only | n/a |  |
| x-10 | L1-based CLI-RSSI measurement and reporting | 1) Support L1 CLI-RSSI measurement within DL subband and UL subband. 2) Support periodic, semi-persistent, and aperiodic CLI-RSSI measurement resource. 2) Support periodic L1 CLI-RSSI reporting in PUCCH. 3) Support semi-persistent L1 CLI-RSSI reporting in PUCCH and PUSCH. 3) Support aperiodic L1 CLI-RSSI reporting in PUSCH. | x-1 | Per UE | TDD only | n/a |  |

Conclusion

In this section, our proposals on UE features for SBFD operation in Rel-19 are summarized.

Proposal 1: Support the following basic FG for SBFD operation in RRC_CONNECTED mode as a baseline.

| Index | Feature group | Components | Prerequisite feature groups | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| x-1 | Basic SBFD operation in RRC_CONNECTED mode | 1) Semi-static indication of time and frequency location of SBFD subbands 2) Resource allocation in frequency domain for PDSCH within one or two DL subands and for PUSCH within one UL subband. 3) A contiguous CSI-RS resource without overlapping with SBFD subband boundaries or across two DL subbands 4) Unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG 5) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols), support Configuration 1: the transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only |  | [Per UE] | TDD only | n/a | 1) The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one. 2) Only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s) |

Proposal 2: Support the following FG for resource allocation in frequency domain for CSI-RS.

| Index | Feature group | Components | Prerequisite feature groups | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| x-2 | Resource allocation in frequency domain for CSI-RS | 1) A contiguous CSI-RS resource overlaps with SBFD subband boundaries 2) A contiguous CSI-RS resource across two DL subbands, where the CSI processing timeline and CPU counting are reported from {legacy, scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy}. | x-1 | [Per UE] | TDD only | n/a |  |

Proposal 3: Support the following FG for partial PRG.

| Index | Feature group | Components | Prerequisite feature groups | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| x-3 | Support of partial PRG | Support of partial PRG in SBFD symbols. The maximum number of supported partial PRGs in SBFD symbols is {2, 4} | x-1 | [Per UE] | TDD only | n/a |  |

Proposal 4: For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots, support the following FG for Configuration 2.

| Index | Feature group | Components | Prerequisite feature groups | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| x-4 | UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots | For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols), support Configuration 2: the transmissions/receptions can be in SBFD symbols and non-SBFD symbols [FFS other components pending on further progress in AI 9.3.1] | x-1 | [Per UE] | TDD only | n/a |  |

Proposal 5: For random access in SBFD symbols in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode, support the following two FGs.

| Index | Feature group | Components | Prerequisite feature groups | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| x-5 | Support random access in SBFD symbols via one single RACH configuration | Support of random access in SBFD symbols in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode via Option 1: use one single RACH configuration | x-1 | [Per UE] | TDD only | n/a |  |
| x-6 | Random access in SBFD symbols via two separate RACH configurations | Support of random access in SBFD symbols in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode via Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration | x-1 | [Per UE] | TDD only | n/a |  |

Proposal 6: Support the following two FGs for UL resource muting.

| Index | Feature group | Components | Prerequisite feature groups | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| x-7 | UL resource muting for CP-OFDM | 1) The UE supports UL resource muting for PUSCH with CP-OFDM waveform. 2) The UE supports UL resource muting for PUSCH repetitions with CP-OFDM. 3) The UL resource muting is dynamically enabled by DCI for DG PUSCH and Type-2 CG PUSCH. 4) At most two OFDM symbols are configured for UL resource muting. 5) Comb-2 offset is configured for UL resource muting. | x-1 | Per UE | TDD only | n/a |  |
| x-8 | UL resource muting for DFT-S-OFDM | 1) The UE supports UL resource muting for PUSCH with DFT-S-OFDM waveform. 2) The UE supports UL resource muting for PUSCH repetitions with DFT-S-OFDM waveform. 3) The UL resource muting is dynamically enabled by DCI for DG PUSCH and Type-2 CG PUSCH. 4) At most two OFDM symbols are configured for UL resource muting. 5) Comb-2 offset is configured for UL resource muting. | x-1 | Per UE | TDD only | n/a |  |

Proposal 7: Support the following two FGs for L1-based CLI measurement and reporting.

| Index | Feature group | Components | Prerequisite feature groups | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| x-9 | L1-based SRS-RSRP measurement and reporting | 1) Support L1 SRS-RSRP measurement within UL subband. 2) Support periodic, semi-persistent, and aperiodic SRS measurement resource. 2) Support periodic L1 SRS-RSRP reporting in PUCCH. 3) Support semi-persistent L1 SRS-RSRP reporting in PUCCH and PUSCH. 3) Support aperiodic L1 SRS-RSRP reporting in PUSCH. | x-1 | Per UE | TDD only | n/a |  |
| x-10 | L1-based CLI-RSSI measurement and reporting | 1) Support L1 CLI-RSSI measurement within DL subband and UL subband. 2) Support periodic, semi-persistent, and aperiodic CLI-RSSI measurement resource. 2) Support periodic L1 CLI-RSSI reporting in PUCCH. 3) Support semi-persistent L1 CLI-RSSI reporting in PUCCH and PUSCH. 3) Support aperiodic L1 CLI-RSSI reporting in PUSCH. | x-1 | Per UE | TDD only | n/a |  |

Appendix

| Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 | Parent IE in TS 38.331 | Need of FDD/TDD differentiation |
| --- | --- | --- | --- | --- | --- |
| CLI-RSSI measurement | 1. Support CLI-RSSI measurement. The max number of resources across all CCs configured to measure RSSI shall not exceed 64. 2. Maximum number of measurement resources configured for CLI-RSSI measurement |  | cli-RSSI-Meas-r16 maxNumberCLI-RSSI-r16 | MeasAndMobParametersFRX-Diff | No (TDD only) |
| SRS-RSRP measurement | 1. Support SRS-RSRP measurement. The max number of SRS resources across all CCs configured to measure SRS-RSRP shall not exceed 32. 2. Maximum number of measurement resources across all CCs configured for SRS-RSRP measurement 3. Maximum number of measurement resources across all CCs configured for SRS-RSRP measurement within a slot - A slot is based on minimum SCS among active BWPs across all CCs configured for SRS-RSRP measurement - A SRS resource occasion that overlaps with the slot is counted as one measurement resource in the slot |  | cli-SRS-RSRP-Meas-r16 maxNumberCLI-SRS-RSRP-r16 maxNumberPerSlotCLI-SRS-RSRP-r16 | MeasAndMobParametersFRX-Diff | No (TDD only) |