3GPP TSG RAN WG1 Meeting RAN1#120	                                                       R1-2500158

Athens, Greece, February 17th – 21st, 2025

Title:	Draft reply LS on CSI-RS measurement with SBFD operation

Response to:	LS on CSI-RS measurement with SBFD operation  (R4-2420165)

Release:	Rel-19

Work Item:	NR_duplex_evo-Core

Source:	[RAN1]

To:	RAN4

Cc:	RAN2

Contact person:

#### Name:	Zhongdan Zhang, Huan Zhou

E-mail Address:	Zhongdan.Zhang@unisoc.com,  Huan.Zhou @unisoc.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: None

- Overall discussion

Issue #1: RAN4 discussed the RRM requirements impact based on the following RAN1 agreement:

RAN1#116bis agreements: If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission) in the SBFD symbol for SBFD-aware UEs, reuse the existing collision handling principles in NR for operation on flexible symbols on a single carrier in unpaired spectrum, i.e. UE does not receive DL channel/signal. - The above does not imply link direction indication is supported - FFS on dynamically scheduled UL transmission with repetition

RAN4 seeks clarification from RAN1 perspective regarding whether the collision scenario between configured DL reception and dynamic UL transmission includes CSI-RS resources used for L1-RSRP/SINR measurement and RLM/BFD/CBD measurement. Specifically, if semi-statically configured CSI-RS resources for above mentioned measurements collide with dynamically scheduled UL transmission in the SBFD slots or symbols, which action should the UE prioritize: performing measurements on the semi-statically configured CSI-RS or executing the dynamically scheduled UL transmission?

Regarding whether the collision scenario between configured DL reception and dynamic UL transmission includes CSI-RS resources used for L1-RSRP/SINR measurement and RLM/BFD/CBD measurement, RAN1 would like to clarify the collision scenario between configured DL reception and dynamic UL transmission includes CSI-RS resources used for L1-RSRP/SINR measurement and RLM/BFD/CBD measurement.

If semi-statically configured CSI-RS resources for above mentioned measurements collide with dynamically scheduled UL transmission in the SBFD slots or symbols, UE performs dynamical UL transmission and does not receive CSI-RS.

Issue #2: RAN4 discussed the RRM requirements impact based on the following RAN1 agreement:

Agreement (RAN1#117) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with one of the configurations. · Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only · Configuration 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols · FFS granularity of the configuration, e.g. per UE, per channel/signal etc. FFS whether support of configuration 2 is subject to UE capability Agreement (RAN1#118bis) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with Configuration 1 or Configurat 2 via RRC configuration. Select from the following options - Option 1: SBFD-aware UE is configured with Configuration 1 or Configuration 2 on a per UE per serving cell basis - Option 2: SBFD-aware UE is configured with Configuration 1 or Configuration 2 on a per UE per BWP per channel/signal basis - Separate configurations for configured and dynamic transmission/receptions for a given channel/signal are not precluded. - Option 3: Configuration 1 or Configuration 2 is on a per cell basis. - No separate UE capabilities for Configuration 1 and 2. Agreement (RAN1#118bis) For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation.

To define the appropriate requirements, RAN4 needs to understand from RAN1 perspective whether the above agreement on resource configuration and/or reporting configuration apply to CSI-RS based L1-RSRP/SINR measurement and RLM/BFD/CBD measurement.

From RAN1 perspective, whether the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured in CSI-ReportConfig has no conclusion. If RAN1 agrees that valid symbol type for CSI derivation is explicitly configured in CSI-ReportConfig, the CSI-RS resources associated with the CSI report includes the CSI-RS resources used for L1-RSRP/SINR measurement and RLM/BFD/CBD measurement. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation.

Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, - Support of configuration 2 is subject to UE capability. Configuration 1 is the default capability. - SBFD-aware UE can be configured with Configuration 2 on a per UL/DL BWP basis. - The configuration for DL BWP at least applies to PDSCH receptions within the DL BWP. - The configuration for UL BWP applies to PUCCH and PUSCH transmissions within the UL BWP. - For SRS, only Configuration 1 is applicable.

In RAN1#119 meeting, the above agreement was reached. Whether the agreement on resource configuration for DL BWP apply to CSI-RS is still under discussion. No conclusion has been reached yet. From our perspective, we already have the valid symbol type for CSI derivation, so there is no need to configure reception behaviour of CSI-RS. UE perform measurements on the CSI-RS of valid symbol type.

- 2	Actions

To RAN4

ACTION: 	RAN1respectfully asks RAN4 to consider the above clarification.

- Dates of next TSG RAN WG1 meetings

TSG-RAN WG1 Meeting #120bis	      7th – 11th April, 2025                  Wuhan, China

TSG-RAN WG1 Meeting #121             19th – 23rd May, 2025                 Malta