3GPP TSG RAN WG1 #120			R1-2500166

Athens, Greece, February 17th – 21st, 2025

Agenda item:	9.3.1

Source: 	Spreadtrum, UNISOC

Title: 		Discussion on SBFD TX/RX/measurement procedures

Document for:	Discussion and decision

Introduction

In RAN#104, the revised WI on evolution of NR duplex operation was approved [1]. As described in the WID, subband non-overlapping full duplex (SBFD) will be specified in Rel-19, which is a key scheme of duplex enhancement in Rel-19 to provide uplink latency reduction, coverage enhancement and spectrum efficiency improvement.

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of time location of SBFD subbands in SIB is not precluded - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of frequency domain location of SBFD subbands in SIB is not precluded - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2] - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon - UL transmissions within UL subband only - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols - Enhancement on resource allocation in frequency domain in SBFD symbols, including - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol - Followings are assumed based on TR 38.858 - SBFD at the gNB side - Half duplex operation at the UE side - FR1 and FR2-1 - SBFD operation Option 4, i.e., both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs - Coexistence between non-SBFD aware UEs (including legacy UEs) and SBFD aware UEs in the cell operating SBFD at gNB side - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies - One UL subband for SBFD operation in an SBFD symbol (excluding legacy UL symbol/slot) within a TDD carrier - Mechanisms for SBFD operation shall also consider the adjacent channel coexistence between two operators

In this contribution, we discuss some technical issues related to SBFD.

Discussion

Semi-static SBFD subband indication

Cell-specific or UE-specific

In RAN1#116 meeting, the following agreement was achieved. Cell-specific configuration on time location of SBFD subbands is supported. Subband time domain locations can be same for all the SBFD-aware UEs in a same serving cell.

Agreement For RRC connected mode UEs, at least cell-specific configuration on time and frequency(working assumption) location of SBFD subbands is supported within a TDD carrier. - FFS: Additional support of UE-specific configuration on time and/or frequency locations of SBFD subbands

In RAN1#116bis meeting, UE-specific configuration on time location of SBFD subbands is excluded. There is no conversion between SBFD symbols and non-SBFD symbols by UE-specific configuration or group-common signaling.

Agreement A symbol configured as SBFD symbol via cell-specific configuration cannot be reverted to a non-SBFD symbol via any UE-specific configuration or group-common signaling. A symbol not configured as SBFD symbol via cell-specific configuration cannot be reverted to an SBFD symbol via any UE-specific configuration or group-common signaling.

The other open issue includes the working assumption to support cell-specific configuration on frequency location of SBFD subbands, and FFS on additional support of UE-specific configuration on frequency domain of SBFD subbands.

From our perspective, we do not support  UE-specific configuration on frequency domain of SBFD subbands.

- It should be single one UL/DL subband locations within a TDD carrier from gNB perstpective to optimize the self-interference. Also RAN4 decide to specify DL/UL subband configuration in BS RF requirements. No matter which active BWP is applied by a UE in the cell, there should be one common resource allocated, in the TDD carrier. There is no need to configure UE dedicated  SBFD subband. The available resources for DL and UL are based on the overlapped frequency resources between UL/DL subbands and the UE active UL/DL BWP.

- The motivation of guard band is for gNB self-interference suppression, not for UE-UE CLI cancellation.  And the size is based on guardbands requirements. So no need to have UE specific subbands due to UL Rx filtering.  One reason to support UE-secific frequency location is to have per-BWP UE-dedicated UL SBFD frequency configurations since different guardband sizes for different SBFD aware UEs due to Rx filter at UE side and/or to mitigate impact on DL reception due to UE-UE CLI. gNB can configure a large guard band in a conservative way and consider the worst interference case.

- The specific guard band size for different Rx filtering capability SBFD aware UEs may improve resource utilization. However, different usable resources for different UEs can be achieved by not scheduling some RBs by gNB in a transparent manner, that is by gNB implementation.

Thus, we prefer to have cell-specific SBFD subband frequency location indication only.

- Only cell-specific configuration on frequency location of SBFD subbands is supported within a TDD carrier.

The signaling design of time and frequency locations was agreed in RAN2#127 meeting.

Cell-specific SBFD time/frequency configuration is provided by SIB1 (or via dedicated signalling to covey cell specific configuration). FFS on UE specific dedicated RRC configuration if needed, pending on RAN1 progress.

- The time domain can be configured within/together with tdd-UL-DL-ConfigurationCommon.

- The frequency location of SBFD subband can use similar structure of FrequencyInfoDL-SIB and FrequencyInfoUL-SIB.

- For Cell-specific SBFD time/frequency configuration is provided by SIB1,

- Time domain can be configured within/together with tdd-UL-DL-ConfigurationCommon

- Frequency location of SBFD subband can use similar structure of FrequencyInfoDL-SIB and FrequencyInfoUL-SIB

Subband time location indication

Subband time location configuration

In RAN1#118 meeting, the following agreement was achieved. Based on the agreement, the starting slot index, starting symbol index within the starting slot, an ending slot index and an ending symbol index within the ending slot can be configured.

Agreement For configuration of SBFD symbols within a TDD-UL-DL pattern period, the following parameters are supported - A starting slot index - A starting symbol index within the starting slot - An ending slot index - An ending symbol index within the ending slot

The remaing open issue is whether or not to explicitly configure guard periods between SBFD and non-SBFD symbols. According to the following agreements in RAN4, it is fixed length and fixed location of transient period. Thus, it does not need to explicitly configure transient period. But it may need to captured in RAN1 to clarify the impact to UE’s behaviour. It can be done after more progress in RAN4.

Agreement: - Transient period length between SBFD and non-SBFD: - The existing TDD BS transmitter transient period reused, i.e., 10us for FR1 and 3us for FR2-1. Agreement: - Location of transient period between SBFD and non-SBFD in RAN4 RF requirement: - FFS the location of transient period between SBFD and non-SBFD shall be located within the SBFD slot/symbol if it needs to be decided.

- For transient period in SBFD operation, it does not need to explicitly configure transient period.

Interaction with TDD-UL-DL-ConfigDedicated

In RAN1#119 meeting, SBFD aware UE behaviors in SBFD symbols with interaction with SFI was discussed. It wa agreed that SFI in DCI format 2_0 is only applicable to non-SBFD symbols but not applicable to SBFD symbols.

Agreement For a serving cell configured with SBFD subband time and frequency location, for an SBFD aware UE, SFI in DCI format 2_0 is only applicable to non-SBFD symbols but not applicable to SBFD symbols - Note: no additional specification efforts are expected to support joint operation of SBFD and dynamic SFI.

For SBFD aware UE behaviors in SBFD symbols with interaction with TDD-UL-DL-ConfigDedicated,  it can use same method as SFI, i.e., TDD-UL-DL-ConfigDedicated is only applicable to non-SBFD symbols but not applicable to SBFD symbols.

There is no consensus on whether link direction indication is supported. If link direction indication is supported, SFI in DCI format 2_0 or TDD-UL-DL-ConfigDedicated may be used to indicate the link direction indication. It needs further discussion.

- For a serving cell configured with SBFD subband time and frequency location, for an SBFD aware UE, TDD-UL-DL-ConfigDedicated is only applicable to non-SBFD symbols but not applicable to SBFD symbols.

Subband frequency location indication

For explicit configuration of frequency locations of UL subband and DL subband, explicit configuration of starting RB and bandwidth of SBFD UL/DL subband was agreed in RAN1#119 mmmeting. It is future-compatible for partial/fully overlapping SBFD operation.

Agreement Confirm the following working assumption made in RAN1#118. Working Assumption For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275. For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList for DL, starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275. - One or two SBFD DL subbands can be configured

Based on WID, subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier is supported. UL subband configured for any SCS would not overlap with DL subband configured for any SCS. It can be guareeted by gNB configuration.

According to WID, only one UL subband for SBFD operation in an SBFD symbol within a TDD carrier is supported. It is defined from the gNB perspective, not from the SCS perspective. Thus, multiple UL subbands in one carrier would not happen even if in different SCSs case.

Transmission/Reception/measurement procedures

TX/RX/measurement behaviors

Link direction from UE perspective

The allowed transmission direction depends on the SBFD time/frequency location indication discussion, which also relative to UE assumption of link direction before collision handling. There are some alternatives was discussed in RAN1#116 meeting.

Agreement For SBFD-aware UE transmission and reception in an SBFD symbol, consider the following options to determine link direction, i.e. whether to transmit or to receive in the SBFD symbol. - Option 1: UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any). - Option 2: link direction is indicated by gNB explicitly. Other options are not precluded.

Whether link direction can be indicated by gNB explicitly was discussed in RAN1#118bis meeting.

#### Proposal 3-1 Proposed Agreement: For SBFD-aware UE transmission and reception in an SBFD symbol, support Option 2 to determine whether to transmit or to receive in the SBFD symbol via dedicated RRC signalling. - Option 2: link direction is indicated by gNB explicitly.

For Option 1, it is similar as link direction in flexible symbol, both of DL and UL assignment can be possible. The difference is legacy flexible symbol has entire same direction for a whole symbol. For SBFD, UE can use either DL or UL subband in frequency domain, depends on the DL reception or UL transmission as shown in the following figure A. This alternative is more complex when handling the collision, but with flexibilities comparing with Option 2.

For Option 2, it is similar as slot format configured with tdd-UL-DL-ConfigurationDedicated in addition to tdd-UL-DL-ConfigurationCommon, explicit singnalling provides further UE’s link direction. The difference is it selects DL subband or UL subband of SBFD symbols. As shown in the following figure B, for UE 1, UE dedicated sigalling indicates the slot 3 to be UL, so only UL subband can be used, while slot 1&2 are DL, only DL subbands can applied. For UE2, slot 1 and 2 are UL, UL transmission in UL subband is valid, and DL in DL subbands are not allowed. In this alternative, semi-statically UL/DL is cancelled if it is against the transmission direction of subband, according to the current collision handling. And dynamic DL/UL should be aligned with transmission direction of subband. Obviously, this alternative can reuse the current collision handling as much as possible, it is easier for all the collision decision, but with less flexibility due to only one of DL or UL subband has been predefined, the opposite direction cannot be used.

Based on the above analysis, Option 1 can provide more scheduling flexibility but Option 2 can reduce some collision handling complexity. Thus, we support Option 1 and Option 2. If dedicated RRC signalling for Opption 2 is optionally configured, UE determines whether to transmit or to receive in the SBFD symbol via dedicated RRC signalling. Otherwise, UE determines whether to transmit or to receive in the SBFD symbol based on agreed collision rules 2 for Option 1.

| A | B |
| --- | --- |

Figure 1: UE assumption of link direction before collision handling

- For SBFD-aware UE transmission and reception in an SBFD symbol, Option 1 and Option 2 can be supported to determine link direction, i.e. whether to transmit or to receive in the SBFD symbol.

- Option 1: UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any).

- Option 2: link direction is indicated by gNB explicitly.

Determination of UL/DL usable PRBs

In RAN1#118b meeting, Option 1 is supported to determine UL/DL usable PRBs if UE-specific configuration on frequency locations of SBFD subbands is not supported nor provided. UL/DL usable PRBs are determined as intersection between cell-specific UL/DL subband and active UL/DL BWP in SBFD symbols.

Agreement If UE-specific configuration on frequency locations of SBFD subbands is not supported nor provided, support Option 1 for determining UL/DL usable PRBs. - Option 1: For a UL BWP, UL usable PRBs are determined as intersection between cell-specific UL subband and the UL BWP in SBFD symbols. For a DL BWP, DL usable PRBs are determined as intersection between cell-specific DL subband(s) and the DL BWP in SBFD symbols.

Based on discussion in section 2.1.3, UE-specific configuration on frequency locations of SBFD subbands is not supported. Thus, Option 1 is supported in any cases.

Tx/Rx occasion mapped to SBFD and non-SBFD symbols within a slot

For the point of “physical channel/signal occasion mapped to SBFD and non-SBFD symbols within a slot”, it was agreed that does not transmit or receive the physical channel/signal within the slot except for PRACH transmission and PUSCH repetition type B.

Agreement For a physical channel/signal occasion mapped to SBFD and non-SBFD symbols within a slot, an SBFD aware UE does not transmit or receive the physical channel/signal within the slot, except - PRACH transmission in a valid RO across SBFD symbols and non-SBFD symbols is subject to the agreements in AI 9.3.2. - For PUSCH repetition type B, down-select from the following options. - Option 1: A nominal repetition is segmented into actual repetitions around boundary of SBFD symbols and non-SBFD symbols. - Option 2: A nominal repetition is segmented into actual repetitions as in legacy operation. An actual repetition mapped to both SBFD and non-SBFD symbols is dropped.

In 9.3.2, it was agreed that configured RO starting from SBFD symbol and ending in non-SBFD symbol either in the same slot or across different slots can be valid based on network configuration and a lot of restrictions.

For PUSCH repetition type B in case a nominal repetition is mapped to SBFD and non-SBFD symbols within a slot, it can be discussed separately.  For Option 1, a nominal repetition can be segmented into multiple actual repetitions around boundary of SBFD symbols and non-SBFD symbols, as illustrated in figure 2. A nominal repetition#3 is segmented into actual repetition#5, actual repetition#6 and actual repetition#7. Each actual repetition includes only SBFD symbols or non-SBFD symbols. So it may no need to satisfy the restrictions, including phase coherency, interruption due to transition time, and same transmission parameter e.g. power control, spatial relation, and frequency domain resource. If nominal repetition is segmented into actual repetitions as in legacy operation and an actual repetition mapped to both SBFD and non-SBFD symbols is dropped, it would cause resource waste and increase lantency. Thus, we support Option 1 to increase resource utilization and reduce latency.

In RAN1#117 meeting, it was agreed that the SBFD-aware UE can be provided with one of the configurations for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots. It can be also applied to PUSCH repetition type B in case a nominal repetition is mapped to SBFD and non-SBFD symbols within a slot. If the SBFD-aware UE is provided with Configuration 2, actual repetition with 2 or more SBFD symbol and actual repetition with 2 or more non-SBFD symbol can be transmited, Otherwise, the actual repetition with invalid symbol type for configuration 1 should be dropped.

Figure 2 : Configuration 2: transmissions in SBFD symbol and non-SBFD symbol

- For PUSCH repetition type B, segmentation around boundary of SBFD symbols and non-SBFD symbols is supported with either Configuration 1/2.

- For Configuration 1, actual repetitions in the invalid symbol type are dropped.

Resource allocation in frequency domain in SBFD symbols

RBG in Resource allocation Type 1

For frequency resource allocation Type 1 for PDSCH or PUSCH in a single slot by DCI format 0_2/0_3/1_2/1_3, PDSCH/PUSCH can be scheduled by starting RBG and length of RBG. It is suffer form the non-contiguous DL subbands and disaligned granularity. It may be allocated with a RBG for PDSCH which is fully/partially outside DL usable PRBs due to RIV mechanism. And PUSCH may be disaligned granularity and allocated RBGs are partially inside UL usable PRBs.

Thus, we have the following proposal:

- For frequency resource allocation Type 1 with RBG for PDSCH or PUSCH in a single slot by DCI format 0_2/0_3/1_2/1_3 based scheduling (without repetition or TBoMS),

- When an allocated RBG overlaps with the subband boundary, only the PRBs within DL usable PRBs are considered to be assigned for PDSCH reception and only the PRBs within UL usable PRBs are considered to be assigned for PUSCH transmission.

- Allocated RBGs that fall outside DL usable PRBs are considered to be not assigned and should not be used for PDSCH resource mapping.

Other issues on non-contiguous DL subbands

CSI-RS

For impacts of non-contiguous DL subbands for CSI-RS and PDSCH, it would be good to have unified solution to resolve this issue. For frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, the following working assumption was made in RAN#118 meeting.

Working Assumption For frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, support one contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL usable PRBs. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured) - RAN1 to further study the impact on CSI processing timeline in SBFD symbols to process the CSI-RS across the two DL subbands

We can confirm the working assumption. Because the same sentence was also used in another agreement.

For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, folloing agreement was made in RAN1#118b meeting. It is up to UE capability to indicate CSI processing timeline are the same as legacy or scaled by factor of 2.

Agreement For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, - Option 3: It is up to UE capability to indicate one of the following: - CSI processing timeline and CPU counting are the same as legacy. - Scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy.

- Confirm the following WA without the last sub-bullet which has been solved:

For frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, support one contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL usable PRBs.

  - No impact on CSI-RS sequence generation

  - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured)

Wideband PRG

Wideband PRG is good for better channel estimation performance. The main concern is whether wideband PRG can be non-contiguous due to discontinuous DL resources. In RAN 1#118 meeting, the folloing conclusion was made. If PRG is determined as wideband, UE does not expect to be scheduled with non-contiguous PRBs in SBFD symbols.

Conclusion If PRG is determined as wideband, UE does not expect to be scheduled with non-contiguous PRBs in SBFD symbols.

The open issue is whether the exsting condition for determination of wideband PRG can be reused. The rule is size of scheduled PRBs is more than half of the size of active DL BWP.  It needs further discussion. Due to the existence of UL subband, the size of usable DL PRBs may be less than half of the size of active DL BWP, or the contiguous PRBs in SBFD symbols are less than half of the size of active DL BWP. So, the condition of wideband PRG is not true.

If the scheduled PRBs are contiguous and the size of the scheduled PRBs is larger than , is the same as the scheduled bandwidth, otherwise is set to the remaining configured value of 2 or 4, respectively.

In RAN1#118b meeting, the following proposal was discussed.

#### Proposal 2-2 Proposed Agreement: For dynamic determination of the precoding bundling for a PDSCH, when DCI bitfield (bundling size indicator) indicates ‘1’ and first set configured with two values as ‘n4-wideband’ or ‘n2-wideband’, the condition for determining wideband or narrowband depends on whether the number of scheduled PRBs is larger than half of the size of intersection between active DL BWP and DL subband where the PDSCH is located.

For SBFD Subband configuration with {DUD} pattern,  DL usable PRBs are two discontigious ones, the condition in the proposal may never be satisfied. According to above agreement, scheduled PRBs is limited to one DL subband if PRG is determined as wideband, so wideband PRG should be determined per DL subband. The condition for determining wideband or narrowband depends on whether the number of scheduled PRBs in a DL subband is larger than half of the number of PRBs in the DL subband.

- For dynamic determination of the precoding bundling for a PDSCH, the condition for determining wideband or narrowband depends on whether the number of scheduled PRBs is larger than half of the size of intersection between active DL BWP and DL subband where the PDSCH is located.

Other issues on resource allocation granularity

PRG

RBG is always larger than or equal size to PRG size applied for UE if both are configured. Considering the case that PRG has smaller or same size as RBG, when part of the DL RBG inside the DL subband is used for PDSCH, it also needs to define UE behaviours for those part of DL PRG inside the DL subband. It is not complete if only partial RBG is allowed but partial PRG is not allowed.

In last meeting, the following agreement was made. It was agreed the part of the DL PRG inside the DL subband can be used subject to UE capability. UE reports the maximum number of supported partial PRGs in SBFD symbols. The candidate value can be discussed in UE features session.

Agreement For a PRG that overlaps the subband boundary, the part of the DL PRG inside the DL subband can be used subject to UE capability UE capability reports the maximum number of supported partial PRGs in SBFD symbols.

Fallback DCI

PDSCH scheduled by DCI format 1_0 in CSS

In current spec, if a PDSCH scheduled with a DCI format 1_0 in a CSS, RB numbering starts from the lowest RB of the CORESET in which the DCI was received. Only resource allocation of type 1 can be used. RBs can be allocated is within the size of CORESET 0 if it is configured or the size of initial DL BWP if CORESET 0 is not configured.

For a CORESET except CORESET0, its frequency resource is easy to be only within DL usable PRBs. Then the PDSCH scheduling is as same as non-SBFD symbols, e.g. no assigned PRBs outside DL usable PRBs.

For CORESET0 configured by MIB, due to it is configured in a continuous resources, it might be overlap with DL subband boundary. Generally, the PDSCH resource allocation by DCI format 1_0 in CSS should resue the legacy method as much as possible, including from the lowest PRB index of CORESET and maximum number of PRBs. Considering there might be SBFD non-aware UE share a same PDCCH and PDSCH, the exclusive of DL non-usable PRBs are not workable, and TBS re-calculation also cannot work. So we suggest the assigned PRBs in PDSCH scheduled by DCI format 1_0 in CSS are all inside DL usable PRBs, without extra handling for PRBs outside of DL usable PRBs.

- Assigned PRBs in PDSCH scheduled by DCI format 1_0 in CSS are all inside DL usable PRBs by scheduling.

Physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots

Resource allocation in frequency domain

For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, two configurations were discussed in RAN1#117 meeting.

Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with one of the configurations. - Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only - Configuration 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols - FFS granularity of the configuration, e.g. per UE, per channel/signal etc. - FFS whether support of configuration 2 is subject to UE capability

In RAN1#119 meeting, it was agreed that upport of configuration 2 is subject to UE capability. Configuration 1 is the default capability.

Granularity of configuration

The granularity of the configuration 2 was discussed. If the granularity of the configuration is per channel/signal, it can provide better flexibility but need more signalling overhead. Thus, it was agreed to support of configuration 2 is subject to UE capability and separate configurations is appplied for UL and DL. It means CG PUSCH and DG PUSCH can share same configuration. For example, it can be configured in BWP-UplinkDedicated .

In last meeting, whether the configuration for DL BWP is applicable to CSI-RS is discussed. From our perspective, we already have the valid symbol type for CSI derivation, so there is no need to configure reception behaviour of CSI-RS. UE receives CSI-RS in valid symbol type for CSI derivation.

Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, - Support of configuration 2 is subject to UE capability. Configuration 1 is the default capability. - SBFD-aware UE can be configured with Configuration 2 on a per UL/DL BWP basis. - The configuration for DL BWP at least applies to PDSCH receptions within the DL BWP. - The configuration for UL BWP applies to PUCCH and PUSCH transmissions within the UL BWP. - For SRS, only Configuration 1 is applicable.

- The configuration for DL BWP is not applicable to CSI-RS.

Tx/Rx for Configuration 1

For configurations 1, the configuration/indication of valid symbol type was discussed in RAN1# 118 meeting and the following agreement was achieved. Based on the agreement, configuration/indication of valid symbol type in Configuration 1 is per signal/channel.

Agreement For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, the valid symbol type is determined as following. - For semi-statically configured transmissions/receptions without activation DCI, the valid symbol type is explicitly configured by RRC. - It is not applicable to PDCCH - For dynamically scheduled transmissions/receptions, the valid symbol type is determined based on the symbol type of the first transmission/reception. - For SP-CSI on PUCCH or PUSCH, type 2 CG PUSCH, SPS PDSCH and semi-persistent SRS, down-select from the following options: - Option 1: - The valid symbol type for SP-CSI on PUCCH or PUSCH is explicitly configured in CSI-ReportConfig. - The valid symbol type for type 2 CG PUSCH is explicitly configured in ConfiguredGrantConfig. - The valid symbol type for SPS PDSCH is explicitly configured in SPS-Config. - The valid symbol type for semi-persistent SRS is explicitly configured in [SRS-Config /SRS-ResourceSet/SRS-Resource]. - Option 2: - The valid symbol type for SP-CSI on PUCCH or PUSCH is determined based on the symbol type of the first PUSCH/ PUCCH after activation. - The valid symbol type for type 2 CG PUSCH is determined based on the symbol type of the first CG PUSCH associated with activation DCI. - The valid symbol type for SPS PDSCH is determined based on the symbol type of the first SPS PDSCH associated with activation DCI. - The valid symbol type for semi-persistent SRS is determined based on the symbol type of the first SRS after activation.

In RAN1#119, the following agreement was achieved.

Agreement For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, - For type 2 CG PUSCH, SPS PDSCH, down-select from the following options: - The valid symbol type for type 2 CG PUSCH is determined based on the symbol type of the first CG PUSCH associated with activation DCI. - The valid symbol type for SPS PDSCH is determined based on the symbol type of the first SPS PDSCH associated with activation DCI.

For SP-CSI on PUCCH or PUSCH, it is activated by DCI or MAC CE and should use similar rule as type 2 CG PUSCH and SPS PDSCH. Option 1 is simpler but has some disadvantage. Option 2 can provide more flexibility and no need explicit indication. Thus, SP-CSI on PUCCH or PUSCH, option 2 is preferred.

For aperiodic SRS, although is dynamically scheduled transmissions, the valid symbol type is determined based on the symbol type of the first transmission/reception. However, we also fine to make an exceptional for aperiodic SRS resource set, which can be configured by RRC.

For semi-persistent SRS, Option 2 may suffer more issues for more than one SRS in a SRS resource set, since all SRS resources in a SRS resource set should with same symbol type. However, if Option 2 applies, semi-persisitent SRS resource may be activated to have different symbol types, due to different slot offset for those SRS resources. So Option 1 is better for semi-persistent SRS. For periodic SRS, it is semi-statically configured transmissions without activation DCI, so Option 1 is applied. We support separate SRS resource sets configurations for SBFD and non-SBFD symbols. So the valid symbol type for semi-persistent SRS and periodic SRS is explicitly configured in SRS-ResourceSet.

- For SP-CSI on PUCCH or PUSCH, the valid symbol type is determined based on the symbol type of the first PUCCH/PUSCH after activation.

- For persistent SRS, periodic SRS and aperiodic SRS, the valid symbol type is explicitly configured.

Tx/Rx for Configuration 2

For configutation 2, there are multiple solutions proposed in RAN1#116b meeting. In RAN1#118 meeting, a unified solution for PUSCH transmission was made. The working assumption can be confirmed.

Working Assumption For an SPS PDSCH configuration without repetitions, if the reception occasions are across SBFD symbols and non-SBFD symbols where each reception occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), PDSCH repetitions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PDSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), - Option 5: Only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid. - For SPS PDSCH, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only - For PDSCH repetitions, FFS whether the number of PRBs for TBS determination is based on assigned PRBs, or based on assigned PRBs within DL usable PRBs only, or based on assigned PRBs within DL usable PRBs of the first repetition. - For multi-PDSCH scheduled by a single DCI, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only.

The remaining open issue is the number of PRBs for TBS determination in SBFD symbols. For multi-PDSCH scheduled by a single DCI with Configuration 2, it was agreed to determine the TBS separately for each PDSCH in different symbol types.

Agreement - For multi-PDSCH scheduled by a single DCI with Configuration 2, the number of PRBs for TBS determination for a PDSCH in SBFD symbols is based on assigned PRBs within DL usable PRBs only. The number of PRBs for TBS determination for a PDSCH in non-SBFD symbols is based on assigned PRBs.

For dynamically scheduled PDSCH repetition, it was agreed that TBS determination is based on the symbol type of first repetition occasion indicated by scheduling DCI.

Agreement For dynamically scheduled PDSCH repetitions with Configuration 2, for TBS determination. - Option 1: - If the first repetition occasion indicated by scheduling DCI is in SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs within DL usable PRBs. - If the first repetition occasion indicated by scheduling DCI is in non-SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs.

For SPS PDSCH, same TBS is applied for all transmission occasions. We prefer to have a unified solution for PDSCH repetition and SPS PDSCH. So the TBS determination for SPS PDSCH is based on the symbol type of the first SPS PDSCH occasion after activation.

#### Proposal 2-9a Proposed Agreement: For SPS PDSCH (with or without repetitions) with Configuration 2, - If the first SPS PDSCH occasion after activation is in SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs within DL usable PRBs. - If the first SPS PDSCH occasion after activation is in non-SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs.

- Confirm the following working assumption:

- For an SPS PDSCH configuration without repetitions, if the reception occasions are across SBFD symbols and non-SBFD symbols where each reception occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), PDSCH repetitions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PDSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2),

- Option 5: Only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid.

- For SPS PDSCH  (with or without repetitions)  with Configuration 2,

- If the first SPS PDSCH occasion after activation is in SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs within DL usable PRBs.

- If the first SPS PDSCH occasion after activation is in non-SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs.

For PUSCH transmissions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols, the frequency location in SBFD symbols is derived from single resource configuration/indication for non-SBFD symbols and RB offset(s), as shown in Figure 3. RB offset represents the frequency domain offset between the starting RB in SBFD symbols and the starting RB in non-SBFD symbols.

Agreement For a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for TBoMS across SBFD symbols and non-SBFD symbols in different slots, where each transmission within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), - Option 2: Single resource configuration/indication for non-SBFD symbols and RB offset(s) configuration/indication/determination to determine frequency resource for SBFD symbols - The numbers of PRBs are the same for PUSCH transmissions in SBFD symbols and PUSCH transmissions in non-SBFD symbols

In RAN1#118bis meeting, it was agreed to consider the following options for determining starting PRB for PUSCH transmissions in SBFD symbols.

Agreement At least in case when PUSCH frequency hopping is not enabled, for a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for TBoMS across SBFD symbols and non-SBFD symbols in different slots, - The number of PRBs for PUSCH transmissions in SBFD and non-SBFD symbols is determined as legacy. - The PRBs for PUSCH transmissions in non-SBFD symbols are determined as legacy. - Consider the following options for determining starting PRB for PUSCH transmissions in SBFD symbols: - Option 1: One or multiple RBoffsetSBFD are configured and the starting PRB for PUSCH in SBFD symbol is determined according to one of the following equation: - Equation 1-A: RBstartSBFD=RBstartnon-SBFD+RBoffsetSBFD - Negative values of RBoffsetSBFD are not precluded. - Equation 1-B: RBstartSBFD=RBstartnon-SBFD+RBoffsetSBFDmodNBWPsize - Equation 1-C: RBstartSBFD=RBstartUL SB+RBstartnon-SBFD+RBoffsetSBFDmodNUL SBsize - Equation 1-D: RBstartSBFD=RBstartUL SB+RBstartnon-SBFD-RBstartUL SB+RBoffsetSBFDmodNUL SBsize - Option 2: The starting PRB for PUSCH in SBFD symbol is determined according to one of the following equation: - Equation 2-A: RBstartSBFD = RBstartUL SB+mod(RBstartnon-SBFD,NUL SBsize -NPUSCHsize) - Equation 2-B: RBstartSBFD = RBstartUL SB+mod(RBstartnon-SBFD,NUL SBsize ): - Equation 2-C: RBstartSBFD = round(RBstartnon-SBFD* NUL SBsizeNBWPsize) + RBstartUL SB - Equation 2-D: RBstartSBFD = RBstartUL SB+RBstartnon-SBFD - The variables are defined as follows: - RBstartSBFD is the starting PRB index of PUSCH in SBFD symbol with reference to the start of UL active BWP - RBstartnon-SBFD is the starting PRB index of PUSCH in non-SBFD symbol with reference to the start of UL active BWP - RBstartUL SB is the starting PRB index of UL usable PRBs with reference to the start of UL active BWP - NBWPsize is the number of PRBs of UL BWP - NUL SBsize is the number of PRBs of UL usable PRBs - NPUSCHsize is the number of PRBs for PUSCH transmissions - No additional configuration/indication to enable/disable whether to apply RBSBFDoffset in SBFD symbols. - FFS: Condition to enable/disable - UE does not expect that the PRBs for PUSCH transmissions in SBFD symbols after applying RBSBFDoffset to be overlapped with PRBs outside UL usable PRBs. - It applies at least to RA type 1. FFS for RA type 0. - Note: Other equations are not precluded.

In RAN1#119 meeting, it was agreed to support the following Equation 1-C2 for determining starting PRB for PUSCH transmissions in SBFD symbols.

Agreement At least in case when PUSCH frequency hopping is not enabled, for a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for TBoMS across SBFD symbols and non-SBFD symbols in different slots, where each transmission within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for determining starting PRB for PUSCH transmissions in SBFD symbols, - Equation 1-C2: RBstartSBFD=RBstartUL SB+RBstartnon-SBFD+RBoffsetSBFDmodNUL SBsize - If RBoffsetSBFD is not configured, it is zero

If the PRBs for PUSCH transmissions in non-SBFD symbols is within the UL usable PRBs, it is not need to apply RBoffsetSBFD. So the condition to disable whether to apply RBSBFDoffset in SBFD symbols is PUSCH frequency resources in non-SBFD symbols is within the UL usable PRBs. In this case, RB_offset is considered as 0.

In case when PUSCH frequency hopping is enabled, the method for determining starting PRB for PUSCH transmissions in SBFD symbols is still applied. Before applying frequency hopping, it should be ensured that the PUSCH of the SBFD symbol is within the UL usable PRBs.

Equation 1-C2 can be supported for RA type 0.

Figure 3: PUSCH resource determination for SBFD symbol

- The condition to disable whether to apply RBSBFDoffset in SBFD symbols is PUSCH frequency resources in non-SBFD symbols is within the UL usable PRBs.

- The Equation 1-C2 can be applied to RA type 1 and RA type 0.

CSI reports across SBFD and non-SBFD symbols

For CSI reports, the following agreements were achieved in RAN1#117 meeting.

Agreement For CSI report associated with periodic/semi-persistent CSI-RS, discuss and decide whether to support the following options. - Option A: For separate CSI reports on SBFD and non-SBFD, one CSI-ReportConfig is associated with CSI-RS(s) restricted to SBFD symbols only and the second CSI-ReportConfig is associated with CSI-RS(s) restricted to non-SBFD symbols only. - gNB configuration may not ensure that the CSI-RS associated with each CSI-ReportConfig is confined to either SBFD symbols or non-SBFD symbols only. - For the CSI-ReportConfig associated with CSI-RS(s) restricted to SBFD symbols only, only CSI-RS transmission occasions within SBFD symbols are used for CSI derivation. For the CSI-ReportConfig associated with CSI-RS(s) restricted to non-SBFD symbols only, only CSI-RS transmission occasions within non-SBFD symbols are used for CSI derivation. - Option B: Enhance Rel-18 NES CSI reporting framework to support one CSI-ReportConfig with one sub-configuration associated with SBFD symbols and the other sub-configuration associated with non-SBFD symbols

For Option A and Option B, they are basically same, except separate CSI reports in Option A, but Option B using separate CSI sub-reports in one CSI report. From function supporting point of view, they have a same ability for CSI reporting. It is really not necessary to suppot both of them. However, from UE capability point of view, Option B require to use Rel-18 NES CSI reporting as prerequisisite. It may have some limits. And the specification impacts of Option A and B have a lot overlapping parts, such as to define CSI-RS transmission occasions within SBFD symbols or non-SBFD symbols. So our suggestion is to support both Options in specification, and Option A/B can be UE capability.

For Option A, the only enhancement is to define occasions used for a CSI report, i.e. CSI-RS transmission occasions within SBFD symbols or non-SBFD symbols.

In Rel 18 NES spatial domain enhancement, single CSI-ReportConfig with multiple sub-configurations is introduced. Each sub-configuration corresponds to a list of one or more CSI-RS resources or a CSI-RS antenna port subset, and/or a power offset for PDSCH relative to CSI-RS. For Option B, besides current two types of sub-configuration (CSI-RS resources and a CSI-RS antenna port subset), a new type of sub-configuration nees to be defined,  which corresponds to CSI-RS transmission occasions subset within SBFD symbols or non-SBFD symbols. Each sub-configuration corresponds to SBFD symbols or non-SBFD symbols in different time instance for one CSI-RS resource. Alternatively, each sub-configuration corresponds to two CSI-RSs which are restricted to SBFD symbols and non-SBFD symbols respectively.

- For SBFD-aware UEs, for CSI report associated with periodic/semi-persistent CSI-RS, at least, across SBFD symbols and non-SBFD symbols in different slots (each CSI-RS resource within a slot has either all SBFD or all non-SBFD symbols), support Option A and B.

- For Option A, it introduces occasions used for a CSI report, CSI-RS transmission occasions within SBFD symbols or non-SBFD symbols

- For Option B, it defines a new type of sub-configuration, corresponding to CSI-RS transmission occasions subset within SBFD symbols or non-SBFD symbols

For Option A, the following agreement was  further achieved in RAN1#118bis meeting.

Agreement For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation.

Accoding to above agreement, whether the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured in CSI-ReportConfig or not is not clear. It should be clarified. Since the validity is related to CSI report, the valid symbol type should be configured in CSI-ReportConfig and applicable to associated CSI-RS resources for the corresponding report. For configuration 2, CSI RS transmissions is in SBFD symbols and non-SBFD symbols and UE uses CSI-RS transmission occasions within the valid symbol types for CSI derivation. For configuration 1, the valid symbol type for CSI-RS transmission should be aligned with the configured valid type for CSI report. UE receives CSI-RS in valid symbol type for CSI derivation.

- For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation is configured in CSI-ReportConfig.

PDCCH

For a CORESET and search space configured that the monitoring occasions of the search space would occur in both SBFD and non-SBFD symbols. Whether the associated CORESET overlaps the boundary of DL usable PRBs in SBFD symbols was discussed and following options was proposed.

#### Proposal 2-18 Proposed Agreement: For a CORESET and a search space configured that the MOs of the search space occur in both SBFD and non-SBFD symbols and the associated CORESET overlaps the boundary of DL usable PRBs in SBFD symbols, down-select from the following options. - Option 1: No enhancements for SBFD-aware UEs in Rel-19. - It is up to gNB to avoid the configuration - Option 2: UE does not monitor a PDCCH candidate if it is mapped to one or more REs that overlap with REs outside DL usable PRBs in SBFD symbols. - Applicable to at least USS

When PDCCH monitoring occasions crossing SBFD and non-SBFD symbols in different slots, it can be guaranteed by PDCCH monitoring occasion and CORESET configuration to never overlap with UL subband in SBFD symbols. Since if the frequency domain location of a CORESET in a SBFD symbol can also work for a non-SBFD symbol, CORESET location does not need to consider how to take full advantage of DL frequency resources, since PDSCH can use the other leftover parts. The location of CORESET only has to ensure the PDCCH capacity, and this part can be satisfied by enough RB number in CORESET.  SBFD is to improve UL delay and coverage, DL resource for is not bottleneck, so a clean CORESET and search space configuration has no problem. In addition, CORESET bitmap configuration has already sufficient flexibility to handle UL subband in frequency domain. Thus, we support Option 1.

In current specification, the frequency resource of CORESRT 0 consists of consecutive resource blocks, as specified in Tables 13-0 through 13-10 of TS38.213. The size of initial DL BWP size is equal to the size of CORESET 0 in RRC_IDLE/INACTIVE mode. Since the centre frequency between the initial UL BWP and initial DL BWP is aligned and  the size of initial UL BWP is no larger than the size of initial DL BWP. If CORESET 0 is confined within one DL subband, there is no usble UL PRBs and random access operation in SBFD symbols can not be achieved.  So the CORESET 0 would overlap the boundary of DL usable PRBs. However, even CORESET 0 overlaps with the boundary of DL usable PRBs in SBFD symbols, the CCE of PDCCH candicate can only be within DL usable PRBs. It can be ensured by the configuration of search space associated with CORESET 0.

In summary, PDCCH monitoring and CCE-REG mapping is the most complex part for UE implementation, existing tools in specifications on CORESET and search space configuration already have sufficient flexibility for PDCCH, no enhancement is required.

- For a CORESET and a search space configured that the MOs of the search space occur in both SBFD and non-SBFD symbols and the associated CORESET overlaps the boundary of DL usable PRBs in SBFD symbols

- No enhancements for SBFD-aware UEs in Rel-19.

  - It is up to gNB to avoid the PDCCH candidate overlap with the boundary of DL usable PRBs in SBFD symbols

Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols

PUCCH

Separate configurations of startingPRB and secondHopPRB for SBFD and non-SBFD symbols was agreed in RAN1#118 meeting and details as below.

Agreement Support separate frequency configurations for SBFD symbols and non-SBFD symbols in the same PUCCH-Resource. - pucch-ResourceId is not separately configured for SBFD and non-SBFD symbols - Support separate configurations of startingPRB and secondHopPRB for SBFD symbols and non-SBFD symbols - Introduce new RRC parameters in PUCCH-Resource to configure starting PRB and second hop PRB for SBFD symbols - FFS whether to support separate configurations of intraSlotFrequencyHopping for Configuration 1 or for both Configuration 1 and 2 - No change on the maximum number of PUCCH resources supported by a UE - Above PUCCH resources with the same pucch-ResourceId is counted as 1 resource FFS: UE behaviour when no separate configuration is provided for SBFD symbols, e.g. PUCCH transmissions in SBFD symbols for this pucch-ResourceId is not expected, or configurations for non-SBFD symbols are applied for SBFD symbols (in which case it is not expected that the configurations would lead to unexpected transmissions) etc.

In RAN1#118bis meeting, the Proposal 2-14 was discussed but no consensus reached.

#### Proposal 2-14 Proposed Conclusion: Separate configurations of intraSlotFrequencyHopping for SBFD and non-SBFD symbols are not supported for both Configuration 1 and 2. When no separate configurations of startingPRB and secondHopPRB are provided for SBFD symbols for a PUCCH resource associated with a pucch-ResourceId, PUCCH transmissions associated with the pucch-ResourceId is not expected in SBFD symbols.

For configurations of intraSlotFrequencyHopping, it is not necessary to have separate configurations of intraSlotFrequencyHopping for SBFD symbol and non-SBFD symbol for configuration 1 and configuration 2.   In addition, separate PUCCH intra-slot frequency hopping for SBFD symbols or non-SBFD symbols can be achieved by configuring different PRBs or same PRB for startingPRB and secondHopPRB as pointed out by other companies. Thus, intra-slot frequency hopping can be enabled or disenabled simultaneously for PUSCH transmission in SBFD symbol and non-SBFD symbols.

When no separate configuration is provided for SBFD symbols, UE behaviour should be clarified. One alternative is PUCCH transmissions in SBFD symbols for this pucch-ResourceId is not expected. It is too restrictive and lead to resources waste especially in configuration 2. It would degrade the performance and impact the latency. Another alternative is configurations for non-SBFD symbols are applied for SBFD symbols when no separate configuration is provided for SBFD symbols. If PUCCH overlaps with DL subband(s) or guardband(s), the PUCCH transmissions associated with the pucch-ResourceId is not expected in SBFD symbols The later alternative is more flexible and improve resource utilization.

- Separate configurations of intraSlotFrequencyHopping for SBFD and non-SBFD symbols are not supported for both Configuration 1 and 2.

- When no separate configuration of startingPRB and secondHopPRB are provided for SBFD symbols, configurations for non-SBFD symbols are applied for SBFD symbols if PUCCH transmissions entirely fall within UL usable RBs in SBFD symbols. PUCCH transmissions in SBFD symbols for this pucch-ResourceId is not expected if PUCCH transmissions outside UL usable RBs in SBFD symbols.

SRS

For periodic/semi-persistent SRS, the frequency domain starting position of SRS is calculated by fomula defined in TS 38.211. It is difficult to modify the formula to ensure that the SRS frequency resource do not overlap with the DL subband. Separate configurations of SRS for SBFD and non-SBFD symbols was agreed in RAN1#118 meeting.

Agreement Support separate configurations for SRS for SBFD symbols and non-SBFD symbols. - FFS whether to have separate configurations of SRS resource sets for SBFD symbols and non-SBFD symbols, or to have separate configurations of SRS resources for SBFD symbols and non-SBFD symbols - FFS applicable SRS usages

In RAN1#118b meeting, it was discussed to support separate SRS-ResourceSets configurations or to support same SRS resource with separate sets of frequency domain parameters for SBFD and non-SBFD symbols.

In RAN1#119, it was agreed to support separate SRS-ResourceSets configurations for SBFD and non-SBFD symbols for a given usage. It is clarified how to associate PUSCH transmission with the separate SRS resource sets for SBFD and non-SBFD. However, there are some remaning issues need to to be clarified.

Agreement Red text is working assumption. For separate SRS configurations for SBFD and non-SBFD symbols, support Option 1. Option 1: Support separate SRS-ResourceSets configurations for SBFD and non-SBFD symbols for a given usage. - SRS-ResourceSet configured for SBFD symbol is only applicable to SRS transmission occasions in SBFD symbols. - For periodic and semi-persistent SRS, the SRS transmissions in non-SBFD symbols are dropped. - For aperiodic SRS with available slot counting, only SBFD symbols are available for SRS transmission. - FFS impact on availableSlotOffsetList - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the SBFD symbols. - FFS: whether there is impact on frequency hopping counter - SRS-ResourceSet configured for non-SBFD symbol is only applicable to SRS transmission occasions in non-SBFD symbols. - For periodic and semi-persistent SRS, the SRS transmissions in SBFD symbols are dropped. - For aperiodic SRS with available slot counting, only non-SBFD symbols are available for SRS transmission. - FFS impact on availableSlotOffsetList - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the non-SBFD symbols. - FFS: whether there is impact on frequency hopping counter - The number of SRS resources in a SRS-ResourceSet for SBFD symbols is the same as the number of SRS resources in a SRS-ResourceSet for non-SBFD symbols for the same usage. - For SRS-ResourceSets with usage set to 'nonCodebook', only one SRS port for each SRS resource is configured as legacy. - For SRS-ResourceSets with usage set to 'Codebook', - Except when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2', the numbers of SRS ports are the same for all the SRS resources in the two SRS-ResourceSets - FFS when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2' - Maintain single TRP behaviour with respect to DCI fields and RRC parameters in rrc-ConfiguredUplinkGrant - SRS resource set indicator (SRSI) field is absent from DCI. srs-ResourceSetId is absent in rrc-ConfiguredUplinkGrant. - If txConfig is set to 'codebook', second precoding information (TPMI) field is absent from DCI and precodingAndNumberOfLayers2 is absent from rrc-ConfiguredUplinkGrant. - Second SRS resource indicator (SRI) field is absent from DCI - For DG PUSCH without repetition within a slot, the indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI in the same symbol type as that of PUSCH transmission, where the SRS resource prior to the PDCCH carrying the SRI. - If txConfig is set to 'codebook', the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to the same symbol type as that of PUSCH transmission - For DG PUSCH with repetition/multi-PUSCH scheduled by a single DCI/TBoMS/Type 2 CG PUSCH with Configuration 1, the indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI in the same symbol type as the valid symbol type of PUSCH transmission, where the SRS resource prior to the PDCCH carrying the SRI. - If txConfig is set to 'codebook', the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to the same symbol type as the valid symbol type of PUSCH transmission - For DG PUSCH with repetition/multi-PUSCH scheduled by a single DCI/TBoMS/Type 2 CG PUSCH with Configuration 2, the SRS resource indicator is applicable to both SBFD and non-SBFD SRS-ResourceSets. For PUSCH transmissions in SBFD symbols, the indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI in SBFD symbols, where the SRS resource is prior to the PDCCH carrying the SRI. For PUSCH transmissions in non-SBFD symbols, the indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI in non-SBFD symbols, where the SRS resource is prior to the PDCCH carrying the SRI. - If txConfig is set to 'codebook', for PUSCH transmission in SBFD symbols, the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to SBFD symbols. For PUSCH transmission in non-SBFD symbols, the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to non-SBFD symbols. - If only a single SRS resource is configured in both SRS resource sets, the SRI field is absent from DCI. In this case “SRS resource identified by the SRI” in the above bullets corresponds to the single SRS resource in each SRS resource set. - srs-ResourceIndicator2 is absent from rrc-ConfiguredUplinkGrant. - For Type 1 CG PUSCH with Configuration 1, srs-ResourceIndicator is associated with the most recent transmission of SRS resource identified by the SRI in the same symbol type as the valid symbol type of PUSCH transmission. - If txConfig is set to 'codebook', precodingAndNumberOfLayers corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to the same symbol type as the valid symbol type of PUSCH transmission. - For Type 1 CG PUSCH with Configuration 2, srs-ResourceIndicator is applicable to both SBFD and non-SBFD SRS-ResourceSets. For PUSCH transmissions in SBFD symbols, the srs-ResourceIndicator is associated with the most recent transmission of SRS resource identified by the SRI in SBFD symbols. For PUSCH transmissions in non-SBFD symbols, the srs-ResourceIndicator is associated with the most recent transmission of SRS resource identified by the SRI in non-SBFD symbols. - If txConfig is set to 'codebook', for PUSCH transmission in SBFD symbols, precodingAndNumberOfLayers corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to SBFD symbols. For PUSCH transmission in non-SBFD symbols, precodingAndNumberOfLayers corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to non-SBFD symbols. - If txConfig is set to 'nonCodebook', each SRS resource set is associated with an associated CSI-RS. - Applicable to usage set to 'codebook' or 'nonCodebook' - FFS usage set to 'beamManagement' - Not applicable to usage set to 'antennaSwitching’

For applicable SRS usage in SBFD senario, it was agreed that the separate SRS resource set can be applied to codebook based PUSCH transmission and non-codebook based PUSCH transmission. Whether it is applied to usage set to  'beamManagement' has no consensus. In R17 MTRP PUSCH repetition, TCI state is configured per SRS resource. One SRS resource in a SRS resource set can be used to beam training for one TRP. The other SRS resource can be used to beam training for another TRP. Same mechanism can be used for SRS transmission in SBFD symbol and non-SBFD symbol. Thus, it is not necessary to configure separate SRS resource set for SBFD symbols and SBFD symbols for usage  'antennaSwitching’.

For aperiodic SRS，the parameter availableSlotOffsetList can be configured up to 4 available slot offset values from the reference slot n + k to the slot where the aperiodic SRS resource set is transmitted, where n is the slot with triggering DCI and k is slotOffset.  In R19 SBFD operation, the SBFD symbol configured in DL symbol can be considered as available slot. Thus, for SRS-ResourceSet configured for SBFD symbol, the UE counts the available slots with the SBFD symbol. For SRS-ResourceSet configured for non-SBFD symbol, the UE counts the available slots with the UL symbols and flexible symbols.

For the number of SRS port,  it was discussed whether to have same number of ports of SRS resource in each SRS-ResourcSet at least for ‘codeoobk’ and ‘non-codebook’ usages. In legacy, only one SRS port for each SRS resource is configured for SRS-ResourceSets with usage set to 'nonCodebook'. It should be kept as legacy. For SRS-ResourceSets with usage set to 'Codebook', according to TS38.214 Clause 6.1.1.1 as follows, except when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2', the numbers of SRS ports are the same for all the SRS resources in the two SRS-ResourceSets. When higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2', UE can be configured with one SRS resource or multiple SRS resources with same or different number of SRS ports. However, we not prefer to combine the SBFD operation and ul-FullPwrMode2 feature. Otherwise, it would increase the UE implementation complexity and specification impact since it needs multiple UE capability reporting including ul-FullPwrMode2-MaxSRS-ResInSet-r16, ul-FullPwrMode2-SRSConfig-diffNumSRSPorts-r16 and ul-FullPwrMode2-TPMIGroup-r16.

Except when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2', when multiple SRS resources are configured by SRS-ResourceSet with usage set to 'codebook', the UE shall expect that a same higher layer parameter nrofSRS-Ports or nrofSRS-Ports-n8 is configured in SRS-Resource in SRS-ResourceSet, and the higher layer parameter nrofSRS-Ports or nrofSRS-Ports-n8 shall be configured with the same value for all these SRS resources. When higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2', - the UE can be configured with one SRS resource or multiple SRS resources with same or different number of SRS ports within an SRS resource set with usage set to 'codebook'. - up to 2 different spatial relations can be configured for all SRS resources in the SRS resource set with usage set to 'codebook' when multiple SRS resources are configured in the SRS resource set. - subject to UE capability, a maximum of 2 or 4 SRS resources are supported in an SRS resource set with usage set to 'codebook'.

In the legacy, TPMI is associated with respect to SRI indicated in DCI. In configuration 2, SRI indicated in DCI corresponds to SRS resource in each SRS-ResourceSet and the corresponding TPMIs associated with each SRS Resource are also applied appropriately based on the symbol type. So the newly added red part related TPMI can be confirmed.

In legacy, if only a single SRS resource is configured in SRS resource set, the SRI field is absent from DCI. It can be keeped as legacy.

- The red text can be confirmed and the following part is clarified.

- Separate SRS-ResourceSets configurations for SBFD and non-SBFD symbols not applicable to usage set to ' beamManagement’.

- For aperiodic SRS with available slot counting,

  - for SRS-ResourceSet configured for SBFD symbol, the UE counts the available slots with the SBFD symbol. For SRS-ResourceSet configured for non-SBFD symbol, the UE counts the available slots with the UL symbols and flexible symbols.

- Same SRS ports is configured for the SRS resources in the two SRS-ResourceSets

- Not support joint operation of SBFD and uplink full power Mode 2

PUSCH Frequency hopping

In RAN1#118 meeting, separate configurations of FH offsets for PUSCH transmissions in SBFD symbols and non-SBFD symbols were supported and agreement as follows.  In legacy, the value range of frequency offsets between two frequency hops is [1, NBWPsize-1]. For PUSCH transmission in SBFD symbols, the value range of frequency offsets between two frequency hops should be updated as [1, NUL SBsize-1].

Agreement Support separate configurations of FH offsets for SBFD symbols and non-SBFD symbols, for a type 1 CG PUSCH with Configuration 2. - Introduce a new RRC parameter in rrc-ConfiguredUplinkGrant in ConfiguredGrantConfig to configure FH offset for SBFD symbols. Support separate configurations of FH offset lists for SBFD symbols and non-SBFD symbols, for PUSCH scheduled by DCI and type 2 CG PUSCH. - Introduce new RRC parameters in PUSCH-Config to configure a set of FH offsets for SBFD symbols. - Support separate configurations for DCI format 0_2 and other DCI formats as legacy - FFS: The number of bits used to indicate FH offset in DCI is determined as legacy UE applies the FH offset/FH offset list according to the symbol type of the PUSCH transmissions. FFS UE behaviour when FH offset/FH offset list is not provided for SBFD symbols, e.g. FH is disabled for PUSCH transmissions in SBFD symbols, or FH offset/FH offset list for non-SBFD symbols are applied for SBFD symbols etc.

In RAN1#118bis meeting, it was agreed that the number of configured FH offsets for SBFD symbols is the same as the number of FH offsets for non-SBFD symbols.

Agreement The number of configured FH offsets of FH offset list for SBFD symbols is the same as the number of FH offsets of FH offset list for non-SBFD symbols. - The number of configured FH offsets of FH offset list is determined based on the UL BWP size as legacy. The number of bits used to indicate FH offset in DCI is determined as legacy.

The remaining open issue is UE behaviour when FH offset/FH offset list is not provided for SBFD symbols. In current specification, PUSCH frequency hopping is performed if the frequency hopping field in a corresponding detected DCI is set to 1, or if higher layer parameter frequencyHoppingOffset is provided for CG type 1 PUSCH transmission. Whether/how to enable FH in non-SBFD symbols or SBFD symbols should follow current specification. When FH offset/FH offset list is not provided for SBFD symbols, UE behaviour should be clarified.  One alternative is to apply the FH list for non-SBFD symbols to SBFD symbols, but PUSCH resource of second hop would overlap with DL subband or guard band. UE may perform drop the transmission of second hop. It would increase UE implementation complexity and not a good option. Another alternative is to disable frequency hopping in SBFD symbols when FH offset/FH offset list is not provided for SBFD symbols. It is simple and  straightforward and we prefer it.

In addition, some companies discussed the case that UE is configured with FH offset list for PUSCH for SBFD symbols but not configured with FH offset list for PUSCH for non-SBFD symbols. If PUSCH frequency hopping is enabled for dynamic PUSCH or type 2 CG, frequencyHoppingOffsetLists should be configured and  MSB bits in FDRA field are used to indicate the frequency offset indicates the applicable FH offset. If frequencyHoppingOffset in rrc-ConfiguredUplinkGrant is not configured, PUSCH frequency hopping is disabled for type 1 CG PUSCH. So frequencyHoppingOffsetLists/frequencyHoppingOffset should be provided for non-SBFD symbols if PUSCH frequency hopping is enabled. Thus, SBFD aware UE does not expected to be configured with FH offset list for PUSCH for SBFD symbols but not configured with FH offset list for PUSCH for non-SBFD symbols.

- For PUSCH transmission in SBFD symbols, the value range of frequency offsets between two frequency hops is [1, NUL SBsize-1].

- For the PUSCH frequency hopping in SBFD symbols, FH is disabled for PUSCH transmissions in SBFD symbols when FH offset/FH offset list is not provided for SBFD symbols. An SBFD aware UE does not expected to be configured with FH offset list for PUSCH for SBFD symbols but not configured with FH offset list for PUSCH for non-SBFD symbols.

Figure 4: Frequency hopping offsets in SBFD symbols

UL power control parameters and spatial relation

In RAN1#117 meeting, separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols was supported in single TRP scenario.

Agreement For a single TRP scenario, support separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols - Decide in RAN1#118 whether separate UL power control for FR2 is supported based on UL spatial relation Info framework and/or unified TCI state framework.

In RAN1#118 meeting, it was agreed that separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols is based on unified TCI state framework and following options can be considered.

Agreement For a single TRP scenario, for separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework, down-select from the following options to support separate configurations of UL power control parameters. - Option 1: One TCI codepoint is mapped to separate TCI states for SBFD symbols and non-SBFD symbols respectively - FFS: No RRC impact - FFS: New MAC CE or reuse existing MAC CE - Option 2: Same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols - FFS whether a new Uplink-powerControlId is introduced for SBFD symbols, or new P0AlphaSets are introduced in Uplink-powerControl for SBFD symbols for PUSCH, PUCCH and SRS respectively - FFS: Impact on MAC CE - Option 3: Same unified TCI state is associated with same UL power control parameters for SBFD symbols and non-SBFD symbols - A power offset is introduced for transmission in SBFD symbols for PUSCH, PUCCH, and SRS - FFS: Same offset or different offsets for all channels - FFS: RRC or MAC CE introduced to convey power offset - FFS whether the maximum number of closed power control loops is increased to be more than 2. - FFS: Whether/How to handle the case where UL power control is not associated within TCI state

In RAN1#119 meeting, it was agreed that same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols. From our perspective, it can be applied to the scenario where SBFD and non-SBFD share a same spatial relation, as shown in Figure 5. There is no  impact on MAC CE. Existing R17 unified TCI MAC CE can be resured. New P0AlphaSets are introduced in Uplink-powerControl for SBFD symbols for PUSCH, PUCCH and SRS respectively.

Figure 5: Single TCI state and separate power control for different symbol type

Agreement For a single TRP scenario, for separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework, - Option 2: Same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols - New P0AlphaSets are introduced in Uplink-powerControl for SBFD symbols for PUSCH, PUCCH and SRS respectively

In RAN1#119 meeting, whether to support separate beam/spatial relations for SBFD and non-SBFD symbols for single-TRP scenario is discussed but no consensus is reached.

#### Proposal 2-10 [HP] Proposed Agreement: Support separate beam/spatial relations for SBFD and non-SBFD symbols for single-TRP scenario for both UL and DL. - It does not preclude using same beam/spatial relation for SBFD and non-SBFD symbols.

One reason to support separate spatial domain comes from the CLI handling, such as the preferred or non-preferred beam is different across SBFD slot and non-SBFD slots. Second reason is different antenna configurations are used by gNB,  so different beam may be selected. So in addition to separate power control, we are also supportive of separate beam/spatial relations for SBFD and non-SBFD symbols for single-TRP scenario for both UL and DL. If separate beam/spatial relations for SBFD and non-SBFD symbols for single-TRP scenario is supported, separate power control parameters for SBFD symbol and non-SBFD symbol can be achieved by associated separate indicated TCI state.

In exsting R18 unified TCI MAC CE for M-TRP, one TCI codepoint can map to separate TCI states for two TRPs. Some companies support to reuse similar scheme for different symbol type, e.g, re-interpret TRP domain into time domain. However, SBFD and mTRP are different feature. There is no need to associate different TRP with different symbol type. New MAC CE can be introduced to indicate separate TCI state for SBFD symbol and non-SBFD symbol. It can use similar structure as R18 unified TCI MAC CE for M-TRP.

- Support separate beam/spatial relations for SBFD and non-SBFD symbols for single-TRP scenario for both UL and DL.

- Introduce a new MAC CE to indicate separate TCI state for SBFD symbol and non-SBFD symbol

- No RRC impact

Figure 6: Separate TCI state and power control for different symbol type

In R17, one default power control parameters set for PUCCH, PUSCH and SRS is configured in BWP-UplinkDedicated when UL power control is not associated within TCI state. One set of default power control parameters would not applied to the case of different symbol type. So in the case of  UL power control is not associated within TCI state, two UL PC parameter settings for PUSCH/PUCCH/SRS can be configured in BWP-UplinkDedicated. Each set of default power control parameters is applied to SBFD symbol and non-SBFD symbol, respectively.

- For the case of UL power control is not associated within TCI state, support a first and a second UL PC parameter settings for PUSCH/PUCCH/SRS configured in BWP-UplinkDedicated.

UE collision handling

Link direction indication is not supported or provided

There was a agreement in RAN1#116 meeting about UE collison handling. According to collision between UL transmissions and DL receptions in the same SBFD symbol, if link direction indication is not supported or provided, the following cases can be discussed. If link direction indication is supported or provided, collision can be separately discussed.

Agreement: For SBFD-aware UEs, collisions between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol may be addressed or alleviated with proper scheduling. The following cases of potential collisions, [if link direction indication is not supported or provided], can be further studied to see if any change to the current specs is necessary: - Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH - Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH - Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission - Case 5: SSB vs. dynamically scheduled or configured UL transmission - e.g., PUSCH, PUCCH, PRACH, SRS - Case 6: Dynamic or semi-static DL vs. valid RO Note: In addition to collision between UL transmission and DL reception in the same SBFD symbol(s), collision between UL transmission and DL reception in different symbol(s) due to lack of sufficient transition time between Tx/Rx at UE side is also included.

In RAN1#116 meeting, there are two agreements about Case 1 and Case 2.

Agreement If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission) in the SBFD symbol for SBFD-aware UEs, reuse the existing collision handling principles in NR for operation on flexible symbols on a single carrier in unpaired spectrum, i.e. UE does not receive DL channel/signal. - The above does not imply link direction indication is supported - FFS on dynamically scheduled UL transmission with repetition Agreement If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 1 (dynamically scheduled DL reception vs. semi-statically configured UL transmission) in the SBFD symbol for SBFD-aware UEs, reuse the existing collision handling principles and timeline in NR for operation on flexible symbols on a single carrier in unpaired spectrum, i.e. UL transmission is cancelled if cancellation timeline is met. - The above does not imply link direction indication is supported - FFS on dynamically scheduled DL reception with repetition

In RAN1#117 meeting, there are two agreements about Case 3 and Case 4.

Agreement If link direction indication is not supported nor provided for a SBFD symbol, collision Case 3 (semi-statically configured DL reception vs. semi-statically configured UL transmission) in the SBFD symbol for SBFD-aware UEs is an error case. - An SBFD-aware UE does not expect to receive both dedicated higher layer parameters configuring transmission in UL usable PRBs from the UE and dedicated higher layer parameters configuring reception in DL usable PRBs in the SBFD symbol - An SBFD-aware UE does not expect to receive both dedicated higher layer parameters configuring transmission in UL usable PRBs from the UE and cell specific higher layer parameters configuring reception in DL usable PRBs in the SBFD symbol - Cell-specifically configured DL reception refers to PDCCH in Type-0/0A/1/2 CSS set - FFS: Additional cases Agreement If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 4 (dynamically scheduled DL reception vs. dynamically scheduled UL transmission) in a SBFD symbol for SBFD-aware UEs, it is considered as an error case if a dynamically scheduled DL reception in DL subband(s) without repetitions overlaps with a dynamically scheduled UL transmission in UL subband without repetitions. - FFS DL reception with repetitions and/or UL transmission with repetitions

For collision Case 1/2/4 with DL receptions and/or UL transmissions with repetitions, the following agreements was made in RAN1#118 meeting.

Agreement For collision Case 1/2/4 with DL receptions and/or UL transmissions with repetitions, the same collision handling rules and timeline for DL receptions and UL transmissions without repetitions are applied for each repetition.

For case 6, the following agreements was made in RAN1#118b meeting.

Agreement If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 6 (Dynamic or semi-static DL vs. valid RO), reuse the existing collision handling rules for HD-FDD RedCap UEs. - UE does not expect collision between PRACH triggered by PDCCH order and dynamically scheduled DL reception. - For collision between PRACH triggered by PDCCH order and semi-statically configured DL reception, UE does not receive DL channel/signal. - For collision between PRACH triggered by higher layer and DL channel/signal, leave it to UE implementation whether to receive the DL or transmit PRACH.

For collision Case 5, it was agreed that re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission.

Agreement Re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission - FFS whether a slot consisting of SSB symbols is considered as a full DL slot or SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs.

The remaining open issue is whether SBFD-aware UEs are allowed to receive within the entire DL BWP in the SSB symbol, or still within the DL usable PRBs. In RAN1#118bis meeting, following three options were discussed.

- Proposal 3-3b Proposed Agreement: For SSB symbols configured with SBFD subbands, down select from the following options. - Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. - Option 2: The SBFD configuration is not applicable to a slot consisting of SSB symbols. - Option 3: The SBFD configuration is not applicable to a TDD pattern period with slot(s) consisting of SSB symbols.

For Option 2, if SBFD configuration is not applicable to a slot consisting of SSB symbols, it means SBFD operation in SSB symbol is disabled and fallback to legacy DL symbol, which can make more transition points in a TDD pattern period. Such as there are two SSB in one slot in Figure 7 (b), there is additonal transition points caused by this fallback. It will increase the implementation complexity in gNB side. Thus, SSB reception within UL usable PRBs in Figure 7 (b) are not allowed. If DL reception only within the DL usable PRBs and left UL subband empty, it would waste some resources but not increase UE implementation complexity. From our perspective, we think SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. The following working assumption should be confirmed.

Working Assumption For SSB symbols configured with SBFD subbands, - Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. - Note: The SSB block is assumed to be within DL subband

| (a) | (b) |
| --- | --- |

Figure 7: SBFD operation in SSB symbols

- Confirm the following working assumption:

- For SSB symbols configured with SBFD subbands,

-	Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs.

-	Note: The SSB block is assumed to be within DL subband

If link direction indication is supported or provided

If link direction indication is supported or provided, the UE can only be allowed to perform UL transmit in UL subband or DL reception in DL subband. In this senario, semi-statically UL/DL is cancelled if it is against the link direction indication, according to the current collision handling.  For example, if link direction indicate by gNB is DL, CG PUSCH transmission in UL subband is invalid and should be cancelled. For  dynamic DL/UL scheduling, it should be aligned with link direction indication. It is reasonable that gNB would indicate a link direction for DL and schedule transmission on UL resource and vice versa. Obviously, if link direction indication is supported or provided, the legacy relus to handle reception/ transmission and symbol direction collison can be reused.

- If link direction indication is supported or provided, semi-statically configured UL/DL is cancelled if it is against the link direction indication.

- If link direction indication is supported or provided, UE does not expect dynamic DL/UL scheduling to contradict link direction indication.

Conclusion

In this contribution, we made the following observations and proposal.

- Only cell-specific configuration on frequency location of SBFD subbands is supported within a TDD carrier.

- For Cell-specific SBFD time/frequency configuration is provided by SIB1,

- Time domain can be configured within/together with tdd-UL-DL-ConfigurationCommon

- Frequency location of SBFD subband can use similar structure of FrequencyInfoDL-SIB and FrequencyInfoUL-SIB

- For transient period in SBFD operation, it does not need to explicitly configure transient period.

- For a serving cell configured with SBFD subband time and frequency location, for an SBFD aware UE, TDD-UL-DL-ConfigDedicated is only applicable to non-SBFD symbols but not applicable to SBFD symbols.

- For SBFD-aware UE transmission and reception in an SBFD symbol, Option 1 and Option 2 can be supported to determine link direction, i.e. whether to transmit or to receive in the SBFD symbol.

- Option 1: UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any).

- Option 2: link direction is indicated by gNB explicitly.

- For PUSCH repetition type B, segmentation around boundary of SBFD symbols and non-SBFD symbols is supported with either Configuration 1/2.

- For Configuration 1, actual repetitions in the invalid symbol type are dropped.

- For frequency resource allocation Type 1 with RBG for PDSCH or PUSCH in a single slot by DCI format 0_2/0_3/1_2/1_3 based scheduling (without repetition or TBoMS),

- When an allocated RBG overlaps with the subband boundary, only the PRBs within DL usable PRBs are considered to be assigned for PDSCH reception and only the PRBs within UL usable PRBs are considered to be assigned for PUSCH transmission.

- Allocated RBGs that fall outside DL usable PRBs are considered to be not assigned and should not be used for PDSCH resource mapping.

- Confirm the following WA without the last sub-bullet which has been solved:

For frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, support one contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL usable PRBs.

  - No impact on CSI-RS sequence generation

  - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured)

- For dynamic determination of the precoding bundling for a PDSCH, the condition for determining wideband or narrowband depends on whether the number of scheduled PRBs is larger than half of the size of intersection between active DL BWP and DL subband where the PDSCH is located.

- Assigned PRBs in PDSCH scheduled by DCI format 1_0 in CSS are all inside DL usable PRBs by scheduling.

- The configuration for DL BWP is not applicable to CSI-RS.

- For SP-CSI on PUCCH or PUSCH, the valid symbol type is determined based on the symbol type of the first PUCCH/PUSCH after activation.

- For persistent SRS, periodic SRS and aperiodic SRS, the valid symbol type is explicitly configured.

- Confirm the following working assumption:

- For an SPS PDSCH configuration without repetitions, if the reception occasions are across SBFD symbols and non-SBFD symbols where each reception occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), PDSCH repetitions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PDSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2),

- Option 5: Only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid.

- For SPS PDSCH  (with or without repetitions)  with Configuration 2,

- If the first SPS PDSCH occasion after activation is in SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs within DL usable PRBs.

- If the first SPS PDSCH occasion after activation is in non-SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs.

- The condition to disable whether to apply RBSBFDoffset in SBFD symbols is PUSCH frequency resources in non-SBFD symbols is within the UL usable PRBs.

- The Equation 1-C2 can be applied to RA type 1 and RA type 0.

- For SBFD-aware UEs, for CSI report associated with periodic/semi-persistent CSI-RS, at least, across SBFD symbols and non-SBFD symbols in different slots (each CSI-RS resource within a slot has either all SBFD or all non-SBFD symbols), support Option A and B.

- For Option A, it introduces occasions used for a CSI report, CSI-RS transmission occasions within SBFD symbols or non-SBFD symbols

- For Option B, it defines a new type of sub-configuration, corresponding to CSI-RS transmission occasions subset within SBFD symbols or non-SBFD symbols

- For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation is configured in CSI-ReportConfig.

- For a CORESET and a search space configured that the MOs of the search space occur in both SBFD and non-SBFD symbols and the associated CORESET overlaps the boundary of DL usable PRBs in SBFD symbols

- No enhancements for SBFD-aware UEs in Rel-19.

  - It is up to gNB to avoid the PDCCH candidate overlap with the boundary of DL usable PRBs in SBFD symbols

- Separate configurations of intraSlotFrequencyHopping for SBFD and non-SBFD symbols are not supported for both Configuration 1 and 2.

- When no separate configuration of startingPRB and secondHopPRB are provided for SBFD symbols, configurations for non-SBFD symbols are applied for SBFD symbols if PUCCH transmissions entirely fall within UL usable RBs in SBFD symbols. PUCCH transmissions in SBFD symbols for this pucch-ResourceId is not expected if PUCCH transmissions outside UL usable RBs in SBFD symbols.

- The red text can be confirmed and the following part is clarified.

- Separate SRS-ResourceSets configurations for SBFD and non-SBFD symbols not applicable to usage set to ' beamManagement’.

- For aperiodic SRS with available slot counting,

  - for SRS-ResourceSet configured for SBFD symbol, the UE counts the available slots with the SBFD symbol. For SRS-ResourceSet configured for non-SBFD symbol, the UE counts the available slots with the UL symbols and flexible symbols.

- Same SRS ports is configured for the SRS resources in the two SRS-ResourceSets

- Not support joint operation of SBFD and uplink full power Mode 2

- For PUSCH transmission in SBFD symbols, the value range of frequency offsets between two frequency hops is [1, NUL SBsize-1].

- For the PUSCH frequency hopping in SBFD symbols, FH is disabled for PUSCH transmissions in SBFD symbols when FH offset/FH offset list is not provided for SBFD symbols. An SBFD aware UE does not expected to be configured with FH offset list for PUSCH for SBFD symbols but not configured with FH offset list for PUSCH for non-SBFD symbols.

- Support separate beam/spatial relations for SBFD and non-SBFD symbols for single-TRP scenario for both UL and DL.

- Introduce a new MAC CE to indicate separate TCI state for SBFD symbol and non-SBFD symbol

- No RRC impact

- For the case of UL power control is not associated within TCI state, support a first and a second UL PC parameter settings for PUSCH/PUCCH/SRS configured in BWP-UplinkDedicated.

- Confirm the following working assumption:

- For SSB symbols configured with SBFD subbands,

-	Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs.

-	Note: The SSB block is assumed to be within DL subband

- If link direction indication is supported or provided, semi-statically configured UL/DL is cancelled if it is against the link direction indication.

- If link direction indication is supported or provided, UE does not expect dynamic DL/UL scheduling to contradict link direction indication.

References

- RP-2401614	Revised WI on evolution of NR duplex operation

- TR38.858, V18.0.0, Study on Evolution of NR Duplex Operation, (Release 18)

- Draft_Minutes_report_RAN1#118bis

- R1-2409088  Summary #3 of 	SBFD TX/RX/measurement procedures, Moderator (CATT)

- R1-2410694, Summary #3 of 	SBFD TX/RX/measurement procedures, Moderator (CATT)