3GPP TSG RAN WG1 Meeting #120	R1-2500242

Athens, Greece, February 17th –21st, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on CSI enhancements

Agenda Item:	9.2.2

Document for:	Discussion and Decision

- Introduction

In RAN#102, a new WID for MIMO evolution for downlink and uplink was approved for Rel-19 [1]. Among items in this WID, two aspects corresponding to CSI enhancement(s) are captured, i.e., Type-I/Type-II codebook refinement supporting up to 128 CSI-RS ports, hybrid CSI enhancement and UE reporting enhancement for CJT calibration in non-ideal synchronization and backhaul scenario. In this contribution, we elaborate our views on above two aspects, respectively.

- CSI enhancement for up to 128 CSI-RS ports on FR1

As the MIMO technology develops from LTE to NR (then to NR-Advanced), the number of antenna ports has been continuously increased to achieve better coverage and higher spectrum efficiency. Among items in the WID for DL/UL MIMO, the aspects for CSI enhancement supporting up to 128 CSI-RS ports, targeting FR1, are listed as below.

- Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design

  - Type-I/Type-II codebook refinement

    - Type-I SP codebook for PCSI-RS≤32

Regarding extension of Rel-19 Type-I single-panel (SP) codebook to ≤32 CSI-RS ports, the following proposal was discussed during RAN1#119 meeting.

Proposal 1.B (RAN1#119) For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, extend the agreed Scheme-A and Scheme-B to 16, 24, and 32 CSI-RS ports, for all applicable RI values with K=1 only, and without any further modification/enhancement of the sub-features pertinent to the Rel-19 Type-I SP design (including, e.g. the Rel-19 Type-I SP CBSR, soft scaling). - For the Rel-19 Type-I SP codebook, the support for 16, 24, and 32ports are 3 separate UE capabilities from the support for the previously agreed number of ports (48, 64, 128 ports) - The Rel-18 SD NES schemes applicable to Rel-15 Type-I SP codebooks are also applicable to the extension of the Rel-19 Type-I SP codebook to 16, 24, and 32 ports - FFS: whether to adopt the extended orthogonal set for the 2nd SD basis for Scheme-A, RI=2-4 and 16, 24, and 32 CSI-RS ports

In current specification, the Rel-15 SP Type-I codebook with rank-3/4 has two respective modes for PCSI-RS<16 and PCSI-RS≥16. In case Rel-19 Type-I SP Type-I codebook is NOT extended to <=32 ports, the codebook structure could be different for PCSI-RS <16, 16<= PCSI-RS <=32, and PCSI-RS >32. To unify the codebook structure for rank-3/4, we propose extending Rel-19 SP Type-I codebook to PCSI-RS≤32. Moreover, extending Rel-19 Type-I SP to PCSI-RS <=32 ports does NOT increase additional UE complexity.

Besides, in Figure 1, the SLS results suggest that Rel-19 Type-I codebook (Scheme-A) for rank-3/4 offers a significant UPT gain (i.e., ~21.2% for cell-edge UE, ~3.8% for near-field UE, ~8.1% in average) over Rel-15 SP Type-I codebook for PCSI-RS = 32, emphasizing the potential benefits of a unified codebook structure. Moreover, it should be noticed that the above proposal does not imply to remove any existing features in Rel-15 Type-I CSI.

Figure 1 Performance comparison between Rel-19 Type-I codebook (Scheme-A) and Rel-15 Type-I codebook for PCSI-RS = 32 and RI=3-4

Observation1: Extending Rel-19 SP Type-I codebook (scheme-A) to ≤32 ports can significantly improve the performance for RI=3-4 (approximately 8.1% gain in average).

Proposal 1: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, at least extend the agreed Scheme-A to 16, 24, and 32 CSI-RS ports.

  - CRI-based CSI reporting for hybrid beamforming for up to 128 CSI-RS ports

Regarding CSI enhancement for up to 128 CSI-RS ports, the following agreement was endorsed in RAN1#119 meeting.

Agreement (RAN1#119) For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding aperiodic CSI-RS resource configuration, an RRC-configured resource-level slot offset (relative to the resource-set-level slot offset, using the same design as Rel-19 Type-I/II codebook refinement for 48, 64, and 128 ports) is supported for aperiodic CSI-RS resource set - FFS: The number of bits for indicating the resource-level slot offset (relative to the resource-set-level slot offset) for KS resources, including the value(s) of the slot offset - FFS: Whether, in addition, configuring an available slot offset for each CSI-RS resource within the aperiodic CSI-RS resource set - Note: “Available slot offset” is analogous to the Rel-17 SRS triggering offset enhancement

As reflected in the agreement, for Rel-19 CRI-based CSI reporting, the number of bits required to indicate the resource-level slot offset for KS CSI-RS resources within an AP resource set remains undecided. In the legacy specification, all CSI-RS resources in an AP resource set are configured within two adjacent slots. However, considering that the number of CSI-RS resources in a resource set may increase for CRI-based CSI reporting and that the network may need to switch beams for different CSI-RS resources, a 1-bit resource-level offset may be insufficient. Therefore, a 2-bit per-resource configuration can be proposed to indicate the resource-level slot offset relative to the resource-set-level slot offset, enabling KS resources to be configured within 4 adjacent slots. The values of the slot offset range from a 0 to 3 slot with a 1-slot step. More specifically, bit values 00, 01, 10, and 11 represent 0-slot, 1-slot, 2-slot, and 3-slot offsets relative to the legacy resource-set-level slot offset, respectively. Regarding the ‘available slot offset’, this issue can be mitigated through proper network implementation and scheduling.

Proposal 2: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, support introducing a 2-bit per-resource configuration to indicate the offset of the KS resources relative to the legacy resource-set-level slot offset.

- The values of the slot offset range from 0 to 3 slot.

- Do NOT support configuring an available slot offset for each CSI-RS resource within an aperiodic CSI-RS resource set.

- CSI enhancement for CJT calibration

Among items in this WID for DL and UL MIMO, the aspects for CSI enhancement for CJT deployments under non-ideal synchronization and backhaul targeting FR1 are listed as below.

- Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP delay misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

  - Joint Dd and PO reporting

Regarding joint Dd and PO reporting, the following proposal was discussed during RAN1#119 meeting.

Proposal 3.G.1 (RAN1#119) For the Rel-19 aperiodic standalone CJT calibration reporting, support joint Dd + phase offset (PO) reporting as follows: - Only wideband (=1) PO is supported - No further optimization of CSI reporting format, e.g. configurability of not reporting {dn} - The UCI parameters are captured in the table below When ReportQuantity is ‘cjtc-Dd-P’ (joint Doffset+d and PO) Parameter; Details/description. nref1; Reference TRS resource set index for Doffset+d, based on the ordering from RRC configuration: logNTRP bits. nref2; Reference TRS resource set index for PO, based on the ordering from RRC configuration: logNTRP bits. {Dn,offset, n=0, 1, …, NTRP – 1 n≠nref1}; Delay offset for CSI-RS resource set n: NTRP-1logMD bits. {dn, n=0, 1, …, NTRP – 1, n≠nref1 }; 1-bit inside/outside indicator for CSI-RS resource set n: NTRP-1 bits. {POn , n=0, 1, …, NTRP –1, n≠nref2}; Wideband phase offset for CSI-RS resource n: NTRP-1logMΦ bits. - The UCI mapping order is as follows: - nref1, - nref2, - {Dn,offset, n=0, 1, …, NTRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource set ID, - {dn, n=0, 1, …, N TRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource set ID - {POn, n=0, 1, …, NTRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource ID

To evaluate the influence of inter-TRP UL/DL timing offset error on reciprocity-based CJT, LLS was performed under both CDL-C and CDL-E channels. Figures 2 and 3 show the CJT spectrum efficiency with different time offset errors for SU-MIMO and MU-MIMO, respectively. Figure 2 suggests that, timing offset error larger than 2ns would cause significant performance loss for SU-MIMO. While Figure 3 suggests that, even 0.5 ns timing offset error would cause very large performance loss for MU-MIMO. Based on these results, the inter-TRP timing quantization granularity should be no larger than 2ns to guarantee the CJT performance.

(a)

(b)

Figure 2 SU-MIMO CJT spectrum efficiency under (a) CDL-C and (b) CDL-E channel

(a)

(b)

Figure 3 MU-MIMO CJT spectrum efficiency under (a) CDL-C and (b) CDL-E channel

Observation 2: To guarantee the performance of reciprocity-based CJT, the inter-TRP UL/DL timing offset quantization granularity should be no larger than 2ns.

For DO quantization, dynamical ranges AD = {1, 0.5} CP and #. of quantization levels MD = {32, 64, 128, 256}. Table 1 shows the inter-TRP timing offset quantization granularity with AD = {1, 0.5, 0.2, 0.1} CP and MD = 256. It suggests that, the agreed configurations cannot provide enough timing offset quantization granularity. To achieve a quantization granularity of 2ns or less, the dynamical range AD should not exceed 0.1CP for 15kHz SCS and 0.2CP for 30kHz SCS.

Table 1 Inter-TRP timing offset quantization granularity for joint Dd and wideband PO reporting with AD = {1, 0.5, 0.2, 0.1} CP and MD = 256

| AD | Timing offset quantization granularity (ns) |  |
| --- | --- | --- |
|  | 15kHz SCS | 30kHz SCS |
| CP | 18.39 | 9.20 |
| 0.5CP | 9.20 | 3.68 |
| 0.2CP | 3.68 | 1.84 |
| 0.1CP | 1.84 | 0.92 |

Proposal 3: For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, support joint Dd + wideband PO reporting.

- For DO quantization, further support AD = {0.1 CP, 0.2 CP}.

  - Timeline relaxion for joint triggering of Dd report and CJT eType-II report

Regarding timeline relaxation for joint triggering, the following agreement was reached during RAN1#119 meeting.

Agreement (RAN1#119) For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with a joint trigger, the timeline (Z/Z’) is determined as Z/Z’ associated with the Rel-18 eType-II CJT, plus Drelax - The value of Drelax is a UE capability, taken from {0, drelax} - FFS: The value of drelax (>0), including whether it depends on SCS - For linking CJTC Dd and Rel-18 eType-II CJT CSI, joint triggering is a separate UE feature group from separate triggering

When the ‘cjt-Dd’ report and CJT eType-II report are jointly triggered, it is natural that UE would firstly process the ‘cjt-Dd’ report and subsequently process the CJT eType-II report based on the ‘cjt-Dd’ report. So, the timeline (Z/Z’) should be extended to adapt the processing procedure, i.e., drelax>0. Regarding the value of drelax, we prefer drelax = Z1’.

Proposal 4: For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, regarding the extended amount of Z/Z’, i.e., Drelax={0, drelax}, for jointly triggered CJTC Dd report and Rel-18 eType-II CJT CSI report, support drelax=Z1’.

- This issue should be discussed in the UE feature session.

  - UE buffering time for separate triggering of Dd report and CJT eType-II report

Regarding UE buffering time for separate triggering of CJTC Dd report and CJT eType-II report, the following proposal was discussed during RAN1#119 meeting.

Proposal 3.C.5 (RAN1#119) For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with two separate triggers, introduce a UE capability for the following: - The UE capability is used to inform the NW on the maximum duration of 2 sec the UE can store the latest CJTC Dd report, measured from the transmission of the linked CJTC Dd report - When the UE does not report this UE capability, it is assumed that the UE can store a CJTC Dd report [indefinitely]

For separate triggering of CJTC Dd report and CJT eType-II report, UE needs to calculate the CJT eType-II report based on a latest CJTC Dd report. Then, after UE reporting a Dd report, this Dd reporte should be buffered at least until NW triggering a linked CJT eType-II report. From NW perspective, the time gap between a reporting instance of a CJTC Dd report and a triggering instance of a linked CJT eType-II report cannot exceed the maximum UE buffering time. Apparently, it cannot be assumed that all UEs can buffer the CJT Dd report infinitely. Therefore, it is necessary to introduce a UE capability of maximum buffering time of the CJT Dd report.

Proposal 5: For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when a linkage between CJTC Dd and Rel-18 eType-II CJT CSI report configurations is configured with two separate triggers, introduce a UE capability of maximum buffering time of the CJTC Dd report.

- This issue should be discussed in the UE feature session.

  - Linkage between FO report and CJT eType-II report

As similar to the linkage between DO report and CJT eType-II report, linking FO report with CJT eType-II report is also necessary. The large FO (up to 0.2 ppm) between TRPs would cause fast phase changing of the channel. Since the CSI-RS resources of CJT eType-II report are allocated on different time locations, the phase changing would reflect in the channels derived from different CSI-RS resources. To accurately calculate the CJT eType-II CSI, especially the CQI, UE needs to perform DO pre-compensation to mitigate the phase changing.

Figure 3 shows the SLS for non-pre-compensated CJT and UE-specific DO/FO pre-compensated CJT. It is observed that, UE-specific FO pre-compensation CJT (130.26% performance for cell-edge UE, 107.77% performance in average) outperforms non-pre-compensated CJT (100% performance for cell-edge UE, 100% performance in average) and DO pre-compensated CJT (127.18% performance for cell-edge UE, 106.61% performance in average), and both DO and FO pre-compensated CJT provides the best performance (156.41% performance for cell-edge UE, 117.81% performance in average). These results clearly suggest the necessity of UE-specific FO pre-compensation.

Figure 4 SLS throughput results for non-compensated CJT and UE-specific DO/FO pre-compensated CJT

Observation 3: UE-specific FO pre-compensation can bring significant UPT gain (approximately 7.77% in average).

Proposal 6: For the Rel-19 aperiodic standalone CJT calibration reporting, support the linkage between CJT calibration FO report and CJT eType-II report.

- Reuse all the mechanisms of linking CJT calibration DO report and CJT eType-II report, including separate triggering and joint triggering.

  - Precoding assumptions for CSI calculation

As discussed in above, UE needs to consider DO/FO pre-compensation in CJT eType-II calculation. In this case, the final precoder is determined by UE-reported DO/FO and CJT PMI. Therefore, new precoding assumptions are needed when DO/FO pre-compensation is considered.

- If the DO is pre-compensated, the real precoder is determined by the CJT PMI and the DO. The precoding assumption can be expressed as

yσ13000(k,l)⋮yσ13000+P-1(k,l)yσ23000(k,l)⋮yσ23000+P-1(k,l)⋮yσN03000(k,l)⋮yσN03000+P-1(k,l)=e-j2π(k-k0)∆f∆τσ1Wσ1e-j2π(k-k0)∆f∆τσ2Wσ2e-j2π(k-k0)∆f∆τσN0⋮WσN0x0k,l⋮xυ-1k,l

- If the FO is pre-compensated, the real precoder is determined by the CJT PMI and the FO. The precoding assumption can be expressed as

yσ13000(k,l)⋮yσ13000+P-1(k,l)yσ23000(k,l)⋮yσ23000+P-1(k,l)⋮yσN03000(k,l)⋮yσN03000+P-1(k,l)=e-j2π(l-l0)∆t∆fσ1Wσ1e-j2π(l-l0)∆t∆fσ2Wσ2e-j2π(l-l0)∆t∆fσN0⋮WσN0x0k,l⋮xυ-1k,l

In the above formulas, yk,l=yσ13000(k,l)⋯yσN03000+P-1(k,l)T is a vector of symbols transmitted on antenna ports [3000,…,3000+P-1] of each of the N0 selected CSI-RS resources corresponding to a frequency point k and a time point l, xk,l=x0k,l⋯xυ-1k,lT is a vector of PDSCH symbols corresponding to a frequency point k and a time point l,  is the number of CSI-RS ports,  {σ1,…,σN0} are the indices of the N0 selected CSI-RS resources in increasing order, such that 1≤σ1<…<σN0≤NTRP, Wσn is the precoding matrix corresponding to the CSI-RS resource with index σn, k0 is a reference frequency point (k0 may equal to 0), (k-k0) denotes the number of frequency units between the time point k and the reference time point k0, ∆f is a frequency unit, ∆τσn is the DO associated with the CSI-RS resource with index σn, l0 is a reference time point (l0 may equal to 0), ∆t is a time unit, (l-l0) denotes the number of time units between the time point l and the reference time point l0, ∆fσn is the FO associated with the CSI-RS resource with index σn.

Proposal 7: For the Rel-19 aperiodic standalone CJT calibration reporting, support introducing new precoding assumption(s) for UE-specific DO/FO pre-compensated CJT.

- If DO is pre-compensated, the precoding assumption can be expressed as

yσ13000(k,l)⋮yσ13000+P-1(k,l)yσ23000(k,l)⋮yσ23000+P-1(k,l)⋮yσN03000(k,l)⋮yσN03000+P-1(k,l)=e-j2π(k-k0)∆f∆τσ1Wσ1e-j2π(k-k0)∆f∆τσ2Wσ2e-j2π(k-k0)∆f∆τσN0⋮WσN0x0k,l⋮xυ-1k,l

- If FO is pre-compensated, the precoding assumption can be expressed as

yσ13000(k,l)⋮yσ13000+P-1(k,l)yσ23000(k,l)⋮yσ23000+P-1(k,l)⋮yσN03000(k,l)⋮yσN03000+P-1(k,l)=e-j2π(l-l0)∆t∆fσ1Wσ1e-j2π(l-l0)∆t∆fσ2Wσ2e-j2π(l-l0)∆t∆fσN0⋮WσN0x0k,l⋮xυ-1k,l

- Conclusion

In summary, we have the following observations and proposals for CSI enhancement:

Observation1: Extending Rel-19 SP Type-I codebook (scheme-A) to ≤32 ports can significantly improve the performance for RI=3-4 (approximately 8.1% gain in average).

Observation 2: To guarantee the performance of reciprocity-based CJT, the inter-TRP UL/DL timing offset quantization granularity should be no larger than 2ns.

Observation 3: UE-specific FO pre-compensation can bring significant UPT gain (approximately 7.77% in average).

Proposal 1: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, at least extend the agreed Scheme-A to 16, 24, and 32 CSI-RS ports.

Proposal 2: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, support introducing a 2-bit per-resource configuration to indicate the offset of the KS resources relative to the legacy resource-set-level slot offset.

- The values of the slot offset range from 0 to 3 slot.

- Do NOT support configuring an available slot offset for each CSI-RS resource within an aperiodic CSI-RS resource set.

Proposal 3: For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, support joint Dd + wideband PO reporting.

- For DO quantization, further support AD = {0.1 CP, 0.2 CP}.

Proposal 4: For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, regarding the extended amount of Z/Z’, i.e., Drelax={0, drelax}, for jointly triggered CJTC Dd report and Rel-18 eType-II CJT CSI report, support drelax=Z1’.

- This issue should be discussed in the UE feature session.

Proposal 5: For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when a linkage between CJTC Dd and Rel-18 eType-II CJT CSI report configurations is configured with two separate triggers, introduce a UE capability of maximum buffering time of the CJTC Dd report.

- This issue should be discussed in the UE feature session.

Proposal 6: For the Rel-19 aperiodic standalone CJT calibration reporting, support the linkage between CJT calibration FO report and CJT eType-II report.

- Reuse all the mechanisms of linking CJT calibration DO report and CJT eType-II report, including separate triggering and joint triggering.

Proposal 7: For the Rel-19 aperiodic standalone CJT calibration reporting, support introducing new precoding assumption(s) for UE-specific DO/FO pre-compensated CJT.

- If DO is pre-compensated, the precoding assumption can be expressed as

yσ13000(k,l)⋮yσ13000+P-1(k,l)yσ23000(k,l)⋮yσ23000+P-1(k,l)⋮yσN03000(k,l)⋮yσN03000+P-1(k,l)=e-j2π(k-k0)∆f∆τσ1Wσ1e-j2π(k-k0)∆f∆τσ2Wσ2e-j2π(k-k0)∆f∆τσN0⋮WσN0x0k,l⋮xυ-1k,l

- If FO is pre-compensated, the precoding assumption can be expressed as

yσ13000(k,l)⋮yσ13000+P-1(k,l)yσ23000(k,l)⋮yσ23000+P-1(k,l)⋮yσN03000(k,l)⋮yσN03000+P-1(k,l)=e-j2π(l-l0)∆t∆fσ1Wσ1e-j2π(l-l0)∆t∆fσ2Wσ2e-j2π(l-l0)∆t∆fσN0⋮WσN0x0k,l⋮xυ-1k,l

- References

- RP-234007, New WID: NR MIMO Evolution Phase 5, Samsung (Moderator)

- Appendix

Table 2 SLS assumption for SP Type-I codebook with PCSI-RS = 32

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 6GHz |
| Channel Model | According to the TR 38.901 3D UMi ISD=200m |
| Antenna setup and port layouts at gNB | - 32 ports: (M, N, P, Mg, Ng, Mp, Np) =(2,8,2,1,1,2,8) (dH, dV) = (0.5, 0.8) |
| Antenna setup and port layouts at UE | (M, N, P, Mg, Ng, Mp, Np) =(1,2,2,1,1,1,2) (dH, dV) = (0.5, 0.5) |
| Modulation | Up to 256QAM |
| gNB Tx power | 46 dBm |
| UE Tx power | 23dBm |
| gNB antenna height | 25 m |
| gNB receiver noise figure | 5dB |
| UE receiver noise figure | 9dB |
| Numerology | 14 OFDM symbol per slot, 15kHz SCS |
| Bandwidth | 10MHz, 52RBs |
| UE receiver | MMSE-IRC |
| Network Layout | 40 UEs per sec (in a total of 21 secs) |
| PMI/CQI feedback | Subband |
| UE distribution | 20% outdoor (30km/h) |
| Traffic model | FTP |
| CSI feedback delay | 5ms |
| MIMO scheme | SU-MIMO with rank adaptation Maximum rank = 4 per UE |
| Performance metrics | 5%-ile and Average UPT |

Table 3 LLS assumptions for the evaluation of the influence of inter-TRP UL/DL timing offset error on reciprocity-based CJT

| Parameter | Value |
| --- | --- |
| Number of antenna ports in single TRP | 64 |
| Number of UE antenna ports | 4 |
| Beamforming Type | SVD |
| SRS periodicity | 20 slots |
| UE speed | 0.3 km/h |
| Number of PRB | 200 |
| PRB bundling size | 2 PRBs |
| Sub-carrier spacing | 30kHz |
| Channel model | CDL-C Delay spread 100 ns or CDL-E Delay spread 30 ns; ASA 0 deg; ZSA 0 deg |
| Rank and MCS | Fixed rank (4); Adaptive MCS |

Table 4 SLS assumptions for evaluation of inter-TRP time/frequency calibration for CJT

| Parameter | Value |
| --- | --- |
| Number of antenna ports | 16T/4R |
| Number of TRPs | Up to 3 |
| Carrier frequency | 2GHz |
| Frequency bandwidth | 52 RBs (10MHz) |
| Frequency synchronization error | 0.01 ppm per TRP |
| Inter-site distance | 500m/800m |
| CSI-RS periodicity | 5 slots |
| Feedback delay | 4 slots |
| CSI subband | 4 RB & R=2 |