3GPP TSG RAN WG1 #120                                                                                           R1-2500366

Athens, Greece, February 17th – 21st, 2025

Source:	vivo

Title:	Discussions on NR-NTN downlink coverage enhancement

Agenda Item:	9.11.1

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss the downlink coverage enhancement for NR NTN, including the discussions on the default SSB periodicity other than 20ms and link level enhancements.

- Default SSB periodicity other than 20ms

The following observations and agreements were captured in RAN1#117 - #119:

Observation Based on the results of DL coverage evaluation at system level collected from different sources, it is observed that extending the default value of SSB periodicity (different from 20ms) in NTN with LEO600km satellite parameter sets where the beam footprint diameter is 50 km, is beneficial in terms of reduction of common control channel overhead, when targeting a full coverage of 1058 beam footprints: - With Set 1-1 FR1 and Set 1-3 FR1, the common messages (SSB, SIB1) overhead is around 40% assuming 5 MHz BW when SSB/SIB1 periodicity of 20ms is in use, this overhead ratio could be reduced to less than 14% when 160ms SSB/SIB1 periodicity is used. - With Set 1-2 FR1, the common message (SSB, SIB1) overhead is greater than 100% assuming 5 MHz BW when SSB/SIB1 periodicity of 20ms is in use, this overhead could be reduced to around 25.8% when 640ms SSB/SIB1 periodicity is used. - Note: the overhead of SIB19 was included in some of the results - Note: an observation when SSB/SIB1 periodicity is 320 ms will be discussed and added to the observation Agreement As part of the NTN DL coverage enhancements at both system level and link level, RAN1 to consider: - Extending the periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - Default value[s] with extended periodicity assumed by NTN UE for initial access can be: - One [or more] values from the list {40ms, 80 ms, 160 ms, 320ms, 640ms} Agreement For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is at least 160 ms. - FFS: whether 320ms can be supported as the maximum of the additional default value instead of or in addition to 160ms

RAN #106 has made the following amendments to the objective of the downlink coverage enhancements in the WID:

Specify solutions, including link level enhancements for FR1-NTN and system level enhancements for FR1-NTN and FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - System-level enhancements are to be specified for the following: - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is 160 ms

In the current spec, for initial cell selection, a UE may assume that half frames with SS/PBCH blocks occur with a periodicity of 2 frames. When the additional default SSB periodicity (i.e., 160ms) is used instead of the 20ms, the UE should be aware of this before or during the initial cell selection. Since the additional default SSB periodicity is only applicable to NTN, there are two options under consideration,

- Option 1: When operating on NTN bands, UEs may assume the SSB periodicity to be 160 ms

- Option 2: For each NTN band, RAN4 to specify the default SSB periodicity (20ms or 160ms)

For option 1, UEs always access the NTN networks assuming the SSB periodicity to be 160ms. However, in some cases, networks may transmit SSB shorter than 160ms, and then UEs have to pay extra unnecessary latency and overheads. This argument particularly holds for the networks capable of larger than 5MHz in FR1 or even larger bandwidth in FR2, where larger SSB periodicity is not needed for ensuring the coverage. Hence, it is preferable to adopt Option 2. Specifically, RAN4 can specify the default SSB periodicity along with the SSB SCS for each NTN band, such that network and UEs can simply align the default SSB periodicity according to the selected bands. Option 2 can also avoid the unnecessary latency and overheads when the UEs access a network with the SSB periodicity shorter than 160ms.

Observation 1: If the SSB periodicity is extended to 160 ms, there may be two options for including the additional default SSB periodicity,

- Option 1: When operating on NTN bands, UEs may assume the SSB periodicity to be 160 ms

- Option 2: For each NTN band, RAN4 to specify the default SSB periodicity (20ms or 160ms)

Compared with Option 1, Option 2 can simplify the implementation and also avoid the unnecessary latency and overheads in the case where the network does not need larger SSB periodicity for ensuring the coverage, e.g., due to sufficient bandwidth.

Proposal 1: For each NTN band, RAN4 to specify the default SSB periodicity (20ms or 160ms).

- Link level enhancements

The following agreement was captured for the link level study in RAN1#118bis and #119:

RAN1#118bis Agreement Support PDCCH CSS Link level enhancement in Rel-19 for all CSS types except type 3. - The following techniques are for further study: - PDCCH repetition, including: - Option 1: Intra-slot PDCCH repetition - Option 2: Inter-slot PDCCH repetition - CORESET length (i.e. number of OFDM symbols) extension - DCI format optimization (e.g. size reduction, etc) - Note: the same technique is intended to apply to all search space types targeted for link level enhancements - For the above techniques, at least the following aspects should be discussed for the relevant candidate techniques: - Configuration - Backward compatibility and UE behaviour of legacy UE - Linked Search Space - Blocking probability - DCI format size budget - Resource overhead - Impact on CORESET0 - Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level. - FFS: whether to apply the selected solution to PDCCH CSS type3 and PDCCH USS RAN1#119 Agreement For PDSCH with Msg4 Link level enhancement: - Support PDSCH repetition - FFS: signalling design including number of repetitions - FFS: impact on UE capability - Note: the target coverage enhancement to bridge the gap with respect to single Msg4 transmission is 2.8 dB - Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level. Agreement For link level enhancement of PDSCH with SIB1: - Support PDSCH repetitions within 20 ms duration - The number of repetitions is fixed to 2 repetitions - Further discuss the specification impact for at least the following: - Procedure and signaling (enabling repetitions, associated time resource determination, etc.) - Note 1: without the above PDSCH repetitions, the coverage gap is 2.2 dB to 4.6 dB depending on SIB1 size. - Note 2: Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level. - Note 3: the above is not related to multiple SIB1 transmissions across 20 ms periodicities of SSB, which may not be available when the SSB periodicity is 160 ms or larger (if supported) depending on the SSB and CORESET multiplexing pattern. Agreement For PDCCH CSS (except Type-3) link level enhancements, support only PDCCH repetition for NTN. - FFS: intra-slot and/or inter-slot

In this section, we discuss the potential enhancements of PDCCH, SIB1 PDSCH and Msg4 PDSCH.

3.1 PDCCH Repetition

For the searchspaces other than searchSpaceZero, its configuration of the PDCCH repetition (e.g., the positions) is flexibly configured by SIB1 or RRC, while the configuration of searchSpaceZero can only be delivered by MIB. Thus, we first analyze the repetition for searchSpaceZero, including the indication issues, and discussions on the intra-/inter-slot schemes, followed by the comparison of several potential schemes. After that, we give some discussions on the PDCCH repetition scheme for the searchspaces other than searchSpaceZero.

Indication of PDCCH repetition for searchSpaceZero

During the acquisition of the PDCCH carried by searchSpaceZero, if the network enables PDCCH repetition, it is questionable of whether the UEs incapable of PDCCH repetition, e.g., pre-R19 UEs or R19 UEs without the corresponding capability (for brevity, only pre-R19 UEs are used in the following), can correctly detect the PDCCH, or find the PDSCH scheduled by the PDCCH. For example, if pre-R19 UEs detect one of the PDCCH candidate but the scheduling timeline is indicated based on the other PDCCH candidate not detected by those UEs, compatibility issue may be raised. Therefore, it should be clarified whether the UEs incapable of PDCCH repetition can access the network enabled the PDCCH repetition of searchSpaceZero, which highly affects the design methodology of the PDCCH repetition for searchSpaceZero.

Proposal 2: RAN1 to clarify whether the UEs incapable of PDCCH repetition can access the network enabling the PDCCH repetition of searchSpaceZero.

For searchSpaceZero, as per agreements, since only two repetitions are considered, there is no need to indicate the repetition number. The other configurations needed to be indicated includes, the enabling/disabling of PDCCH repetition, and the positions for PDCCH repetition. Among them, the positions for PDCCH repetition are optional if the starting symbol is configured by default. There are a few potential ways to delivering the above configurations,

- Option 1: Reserved bit in MIB

- Option 2: Redefine some of the entries in the CORESET#0 table or searchSpaceZero table

  - Option 2-1: Reserved entries

    - E.g., 1 entry in Table 13-1 of TS38.213

  - Option 2-2: Under the assumption of 5MHz max. bandwidth for FR1, the entries with more than 25 PRBs

    - E.g., 9 entries in Table 13-1 of TS38.213

  - Option 2-3: Under the assumption of 5MHz max. bandwidth for FR1, the entries with a non-zero RB offset

    - E.g., 13 entries in Table 13-1 of TS38.213

  - Option 2-4: Under the assumption of more than 1 CORESET symbol, the entries with 1 CORESET symbol

    - E.g., 3 entries in Table 13-1 of TS38.213

For the Option 2-1, the codepoint of the reserved entry is used, which cannot be understood by the UEs, and thus those UEs will not continue to detect the PDCCH. For Option 2-2, 2-3, 2-4, the redefinition of certain entries based on certain assumptions. Based on the study result, the <3dB performance gap is observed for PDCCH with aggregation level 8. For the system with at least 10MHz bandwidth, aggregation level 16 can be used, thus no need to enable PDCCH repetition. Thus, those entries beyond 5MHz bandwidth can be repurposed to enable the PDCCH repetition. However, since pre-R19 UEs cannot correctly understand the entries in the table, they cannot even locate the correct resources for searching the PDCCH. Hence, Option 2-2, 2-3, 2-4 could make pre-R19 UEs incapable to detect the PDCCH at all. Besides, although Option 1 preserves the possibility of the pre-R19 UEs detecting at least one of the PDCCH candidates, pre-R19 UEs may be unable to find the scheduled PDSCH (depending on the design of PDCCH repetition). In this case, certain methods can be adopted for alleviating the compatibility issue, e.g., network transmits PDSCH corresponding to each PDCCH candidates that may be detected by pre-R19 UEs. The details can be found in Section 3.1.3.

Regarding the number of available bits, both Option 1 and Option 2 can only deliver 1 bit, which can only be used to indicate the enabling or disabling of PDCCH repetition. On the other hand, Option 2-2, 2-3 and 2-4 can deliver more information, such as the positions of the PDCCH repetition. Thus, we have the following observation and proposal.

Observation 2: There are some ways to indicate the configuration of PDCCH repetition for searchSpaceZero:

- Option 1: Reserved bit in MIB

- Option 2: Redefine some of the entries in the CORESET#0 table or searchSpaceZero table, e.g.,

  - Option 2-1: Reserved entries

  - Option 2-2: Under the assumption of 5MHz max. bandwidth for FR1, the entries with more than 25 PRBs

  - Option 2-3: Under the assumption of 5MHz max. bandwidth for FR1, the entries with a non-zero RB offset

  - Option 2-4: Under the assumption of more than 1 CORESET symbol, the entries with 1 CORESET symbol

For the above options,

- Both option 1 and option 2-1 can only deliver 1 bit, which can only indicate the enabling or disabling of PDCCH repetition.

- Option 2-2, 2-3 and 2-4 can deliver more information, such as the positions of the PDCCH repetition.

- Option 2-1 bars all pre-R19 UEs if PDCCH repetition is enabled, while the other options may allow the pre-R19 UEs to detect the PDCCH or receive PDSCH at the cost of potential compatibility issues.

Proposal 3: Depending on requirements for the number of bits available for indication and compatibility issues, one of the following methods can be used to indicate the configuration of PDCCH repetition for searchSpaceZero,

- 1 bit with potential compatibility issues: Reserved bit in MIB

- 1 bit with barring all pre-R19 UEs: Redefine reserved entries in the CORESET#0 table or searchSpaceZero table

- More than 1 bits with potential compatibility issues:

  - Under the assumption of 5MHz max. bandwidth for FR1, redefine the entries with more than 25 PRBs in the CORESET#0 table or searchSpaceZero table

  - Under the assumption of 5MHz max. bandwidth for FR1, redefine the entries with a non-zero RB offset in the CORESET#0 table or searchSpaceZero table

  - Under the assumption of more than 1 CORESET symbol, redefine the entries with 1 CORESET symbol in the CORESET#0 table or searchSpaceZero table

Discussions on intra-slot repetition for searchSpaceZero

Recall that NR already supports PDCCH repetitions for connected mode UE. The candidate PDCCH monitoring occasions are determined by the configuration of each search space, including the periodicity and offset. According to the current PDCCH repetition scheme introduced by mTRP, up to two individual search spaces with different id can be linked together, but they are required to have the same periodicities and to have their respective PDCCH monitoring occasions located within a slot. In other words, for each period, two intra-slot monitoring occasions respectively determined by the two linked search spaces carry two copies of a same PDCCH.

Observation 3: The PDCCH repetition scheme introduced by mTRP is to link up to two individual search spaces with different id for connected mode UE. The two search spaces have the same periodicities and have their respective PDCCH monitoring occasions located within a same slot.

For inter-slot repetition, all the repeated PDCCH candidates are contained within a same slot. Therefore, the scheduling timeline, e.g., k0/k1, is clear for determining the PDSCH or the HARQ-ACK information bits.

However, during the cell search, UE needs to detect not only the SSB but also its associated Type-0 CSS PDCCH. For multiplexing pattern 1, if the positions for PDCCH repetition are overlapped with the candidate SSB positions, UE needs to blindly search the PDCCH under two different hypotheses, i.e., whether an SSB exists on the same positions. To avoid this issue, the first design principle is to put PDCCH repetition on the positions without any candidate SSB. An exemplary illustration is shown in Figure x. Typically, for the 2-symbol CORESET, symbols 0 and 1 are occupied by the legacy MOs of searchSpaceZero. Then, symbol 6 and symbol 12 may be chosen to be the starting symbol of the repeated PDCCH candidate.

Figure 1. Illustration of available positions for PDCCH repetition, which are denoted by gray blocks

Another principle should be considered is the scheduling timeline between PDCCH and PDSCH. From current specs, the starting symbol of the PDSCH should not be earlier than the first symbol of the later PDCCH candidate scheduling the PDSCH if the scheduling PDCCH contains more than one PDCCH candidate. Meanwhile, for Multiplexing pattern 1, only same slot scheduling is available. In other words, there should be a few available symbols for SIB1 PDSCH after the repeated PDCCH candidate within the same slot.

From the above, for intra-slot PDCCH repetition of searchSpaceZero, symbol 6 is an appropriate choice for the starting symbol of the repeated PDCCH candidate. Therefore, we have the following observations and proposals.

Observation 4: The following issues should be addressed for intra-slot PDCCH repetition,

- Overlapping of PDCCH repetitions and candidate SSB induces higher complexity for UEs to search for the PDCCH.

- Putting PDCCH repetitions earlier than the scheduled PDSCH causes the out-of-order scheduling issue.

- For multiplexing pattern 1, only same slot scheduling is available.

Proposal 4: The positions for PDCCH repetition of searchSpaceZero should be designed following the two principles,

- The positions for PDCCH repetition should not be overlapped with any candidate SSB, at least for two-symbol CORESET.

- The starting symbol of the PDSCH should not be earlier than the first symbol of the later PDCCH candidate scheduling the PDSCH.

Proposal 5: For intra-slot PDCCH repetition of searchSpaceZero, symbol 6 should be considered as the starting symbol of the repeated PDCCH candidate.

Discussions on inter-slot repetition for searchSpaceZero

For inter-slot repetition, there are more available resources for accommodating the PDCCH repetitions and the PDSCH than intra-slot repetition. Thus, it is easier to find an appropriate position for the PDCCH repetitions satisfying the two principles mentioned in Section 3.1.2.

However, for inter-slot repetition, it introduces an issue of which PDCCH candidate is the reference point for indicating and determining the scheduling timeline, e.g., k0/k1, carried in the PDCCH. In current mTRP PDCCH repetition scheme, if a PDCCH contains two PDCCH candidates, the later candidate serves as the reference point. Hence, it is possible to reuse this rule. Again, if the network bars all pre-R19 UEs, all UEs accessing the network can clearly know where the last PDCCH candidate is and therefore the timeline, no matter whether the last PDCCH candidate is decoded successfully.

Alternatively, if both pre-R19 UEs and R19 UEs can access the network, pre-R19 UEs may have different interpretations of the timeline from R19 UEs, because they may only receive the PDCCH candidates in (one of) the legacy monitoring occasions. We analyze a few potential cases:

- Case 1: for Type0-PDCCH CSS, if the network uses the two candidate monitoring occasions for repetition, pre-R19 UE may receive a PDCCH in either of the monitoring occasions. In this case, no matter how to indicate the scheduling timeline, there is a chance for a pre-R19 to obtain an incorrect scheduling timeline. Alternatively, the network can mute either monitoring occasion and choose other additional monitoring occasions (where pre-R19 UEs do not monitor) for repetition. Meanwhile, the network indicates the scheduling timeline based on the first monitoring occasion, and transmits a corresponding PDSCH according to the first monitoring occasion. An exemplary illustration is shown in Figure 2.

  - Pros: PDSCH repetition and PDCCH repetition can be decoupled

  - Cons:

    - For each SSB index, pre-R19 UEs can only have one MO for PDCCH reception

    - It violates the out-of-order (OOO) scheduling rule, i.e., PDSCH may start earlier than the scheduling PDCCH

Figure 2. Illustration of case 1, where one of the monitoring occasions is muted and the scheduling timeline is based on the unmuted monitoring occasion

- Case 2: the network shifts one of the monitoring occasions, for example, shifting the CORESET#0 by 1 RB for the first monitoring occasion, and meanwhile indicates the scheduling timeline according to the other monitoring occasion, i.e., the second monitoring occasion in our example. Since legacy UEs can only decode the PDCCH carried by the monitoring occasion without shifts, they can find the PDSCH accordingly. Instead, R19 UEs are aware of the shifted monitoring occasion, and thus they can jointly receive the PDCCH using both monitoring occasions. An exemplary illustration is shown in Figure 3.

  - Pros: PDSCH repetition and PDCCH repetition can be decoupled

  - Cons:

    - For each SSB index, pre-R19 UEs can only have one MO for PDCCH reception

    - It violates the OOO scheduling rule

Figure 3. Illustration of case 2, where one of the monitoring occasions is shifted and the scheduling timeline is based on the unshifted monitoring occasion

- Case 3: In this case, to ensure pre-R19 UEs correctly decoding a PDSCH regardless of the scheduling PDCCH received in either monitoring occasion, the network can transmit PDSCHs corresponding to each PDCCH monitoring occasion.  Note that, due to the same DCI carried by the two monitoring occasions, the two PDSCHs should have the same FDRA and TDRA. An exemplary illustration is shown in Figure 4.

  - Pros: For each SSB index, pre-R19 UEs can have two MOs for PDCCH reception

  - Cons:

    - PDSCH repetition and PDCCH repetition are coupled

    - It violates the OOO scheduling rule

Figure 4. Illustration of case 3, where the PDSCHs with the same FDRA and TDRA are transmitted for every monitoring occasion potentially received by the legacy UEs

From the aforementioned analysis, if the network admits pre-R19 UEs, the resulted compatibility issue can be addressed with some restrictions:

- Case 1/2: Network mutes/shifts a monitoring occasion (defined by pre-R19 rules) associated with the SSB index, and indicates the scheduling timeline based on the unmuted/unshifted monitoring occasion.

- Case 3: Network transmits PDSCHs with the same TDRA and FDRA corresponding to each PDCCH monitoring occasion that is possibly monitored by legacy UEs

Unfortunately, although the compatibility issue can be relieved for pre-R19 UEs through above restrictions, it is possible that the scheduled PDSCH is located between the repeated PDCCH candidates, e.g., the exemplary illustrations of Case1/3. In other words, it violates the NR OOO scheduling rule.

Observation 5: For inter-slot PDCCH repetition, if the network admits pre-R19 UEs, the compatibility issue can be achieved with following methods:

  - Network mutes/shifts a monitoring occasion (defined by pre-R19 rules) associated with the SSB index, and indicates the scheduling timeline based on the unmuted/unshifted monitoring occasion.

    - Pros: PDSCH repetition and PDCCH repetition can be decoupled

    - Cons:

      - For each SSB index, pre-R19 UEs can only have one MO for PDCCH reception

      - It violates the OOO scheduling rule

  - Or, network transmits PDSCHs with the same TDRA and FDRA corresponding to each PDCCH monitoring occasion that is possibly monitored by legacy UEs.

    - Pros: For each SSB index, pre-R19 UEs can have two MOs for PDCCH reception

    - Cons:

      - PDSCH repetition and PDCCH repetition are coupled

      - It violates the OOO scheduling rule

Proposal 6: If the network bars all pre-R19 UEs, for inter-slot PDCCH repetition, if a PDCCH contains multiple PDCCH candidates, the later candidate can be used to determine the scheduling timeline.

Proposal 7: If the network allows pre-R19 UEs to access, for inter-slot PDCCH repetition, at least the following methods can be considered for alleviating the compatibility issue:

  - Network mutes/shifts a monitoring occasion (defined by pre-R19 rules) associated with the SSB index, and indicates the scheduling timeline based on the unmuted/unshifted monitoring occasion.

  - Or, network transmits PDSCHs with the same TDRA and FDRA corresponding to each PDCCH monitoring occasion that is possibly monitored by legacy UEs.

  - For above cases, out-of-order scheduling should be allowed for R19 UEs.

Potential Schemes for PDCCH repetition for searchSpaceZero

Although the mTRP PDCCH repetition has been specified, it is only for connected mode UE and except for searchSpaceZero. Since only PDCCH CSS (except Type3) is of interest here, the mTRP PDCCH repetition cannot be applied directly. Next, we list several potential PDCCH repetition schemes for searchSpaceZero.

- Reusing mTRP scheme: A straightforward idea is to reuse the mTRP PDCCH repetition scheme as much as possible, by introducing an additional searchspace associated with a searchspace to be repeated. For searchSpaceZero, an additional searchspace should be introduced, e.g., called searchSpaceZeroRep.  The searchSpaceZeroRep and its linking to searchSpaceZero can be defined by such as table-based indication by MIB. Moreover, the searchSpaceZeroRep and the linking id can be further reconfigured by SIB1 or RRC. Similar to searchSpaceZero, a mapping rule from the monitoring occasions (MOs) of the searchSpaceZeroRep to the associated SSB indexes should be defined, such that the mapping rule between the MOs of the searchSpaceZero and those of the searchSpaceZeroRep can be determined accordingly.

Since Scheme 1 is extended from the mTRP scheme, the UE behavior can maximally reuse the existing specifications. However, it would be difficult for searchSpaceZero. Moreover, even if searchSpaceZeroRep is defined, an additional entry index (which takes 4bits for serachSpaceZero) seems difficult to be introduced in MIB.

Similar to the existing mTRP scheme, Scheme 1 is limited to intra-slot PDCCH repetition, which does not have the scheduling timeline issues as mentioned before. However, it requires additional signals due to the additional searchspace separately defined with its own searchspace id.

To sum up, we have the following observation for Scheme 1,

Observation 6: For the PDCCH repetition Scheme 1, i.e., reusing mTRP scheme,

- Limited to intra-slot PDCCH repetition

- Potentially large spec impacts

- Additional searchspace for repetition are defined separately with its own searchspace id, which generally requires additional signalings. Particularly, the signaling overheads seems impossible for searchSpaceZero, since no more bits than 1 reserved bit can be delivered by MIB.

- Defining additional MOs: This scheme is to define additional MOs on top of the legacy MOs of the searchspace. For example, for an entry defined by searchSpaceZero table, an additional first symbol index is defined to create one more additional MO. Depending on how to define the mapping rule of the MOs to the SSB index or the legacy MOs, the PDCCH repetitions can be flexibly defined to be inter-slot or intra-slot.

- Redefining the association between the MOs and the SSB index: This scheme is to redefine the association between the MOs and the SSB index. In current spec (e.g., Table 13-13 in TS38.213 for Multiplexing pattern 1), when number of MOs per slot is 2, and M=1/2, there are two MOs within a slot and respectively associated with two consecutive SSB indexes. Therefore, it is possible to redefine the mapping between MOs and SSB indexes, such that the two MOs within the same slot can be associated with the same SSB index for PDCCH repetition.

- Linking the two candidate monitoring occasions associated with each SSB index in two consecutive slots: According to the current spec, neither gNB nor UE assumes that the two candidate monitoring occasions are both available and carry the same PDCCH. It is straightforward to specify the two monitoring occasions to be bound for repeating a same PDCCH.

Compared with Scheme 1, all of the schemes do not need to additionally define another searchspace, and thus do not require extra signaling overheads for the additional searchspace id and linking id.

For Scheme 2, both intra-slot and inter-slot PDCCH repetition can be realized by designing the mapping rule of the additional MOs. In the intra-slot case, the scheduling timeline issues can be avoided. The spec impacts include an additional start symbol index in the searchSpaceZero table, and the mapping rule of the additional MOs.

For Scheme 3, it is limited to intra-slot PDCCH repetition, without inducing scheduling timeline issues. However, only selected entries of the searchSpaceZero can be used by the network, imposing the undesirable scheduling restriction. The spec impacts include a redefinition of the mapping rule of SSB indexes to MOs.

For Scheme 4, it is limited to inter-slot PDCCH repetition, which could induce the scheduling timeline issues as mentioned in Section 3.1.3. Several solutions can be used to alleviate the scheduling timeline issues, but the timeline restriction between PDCCH and PDSCH may not be guaranteed, which needs to define new UE behavior. Moreover, among them, if both PDCCH repetition and PDSCH repetition are enabled, network can choose the Case 3 mentioned in Section 3.1.3, that is, to transmit PDSCHs with the same TDRA and FDRA corresponding to each PDCCH monitoring occasion that is possibly monitored by legacy UEs. However, the duplicated PDSCHs with the same TDRA and FDRA may impose additional scheduling restrictions, as well as resulting in many fragmented resources.

To sum up, we have the following observation for Scheme 2, Scheme 3, Scheme 4,

Observation 7: For the PDCCH repetition Scheme 2 (Defining additional MOs), Scheme 3 (Redefining the association between the MOs and the SSB index), Scheme 4 (Linking the two candidate monitoring occasions associated with each SSB index in two consecutive slots),

- All the schemes do not need an additional searchspace, nor induce extra signaling overheads for the additional searchspace id and linking id.

- Scheme 2 can achieve a unified design of intra-slot and inter-slot PDCCH repetition.

- Scheme 2 and Scheme 3 can be applicable to intra-slot PDCCH repetition, without inducing scheduling timeline issues.

- Scheme 3 imposes undesirable scheduling restriction because only selected entries of the searchSpaceZero can be used by the network.

- Scheme 4 is limited to inter-slot repetition, which may induce scheduling timeline issues.

Potential Schemes for PDCCH repetition for searchspaces other than searchSpaceZero

The searchspaces other than searchSpaceZero can be configured flexibly by SIB1 or RRC. Therefore, it is preferable to reuse as much as possible the mTRP PDCCH repetition scheme due to small spec efforts. Specifically, a linking id can be defined for the two searchspaces bound for repetitions, which is typically how the mTRP PDCCH repetition scheme works, except that the linking id needs to be introduced in the common searchspace defined by SIB1 or by RRC. All CSS that are referred to a searchspace with linking id can be automatically enabled with repetition, e.g., CSS Type 0A/1/2, as well as the CSS Type0 defined by searchSpaceSIB1 on non-initial BWP.

Observation 8: The mTRP PDCCH repetition scheme can be extended to all CSS other than searchSpaceZero, by indicating the linked searchspaces through SIB/RRC. Small spec efforts are needed and high flexibility can be achieved.

Proposal 8: It is preferable to reuse the mTRP scheme for the CSS other than searchSpaceZero, if PDCCH repetition is supported for common search spaces beyond searchSpaceZero.

3.2 SIB1 PDSCH repetition

As per agreements, only two repetitions are considered. The SIB1 PDSCH repetition can be inter-slot or intra-slot. The offset between the first repetition and the second repetition can be X symbols for the intra-slot case or X slots for the inter-slot case. To reduce signaling overheads, the TDRA/FDRA should be maximally reused if possible. Particularly, for the intra-slot case, the FDRA should be the same, while for the inter-slot case, both TDRA and FDRA should be the same. For the same reason, X should be predefined.

Regarding the indication of SIB1 PDSCH repetition, there are possible two options,

- Option 1: Follow the enabling/disabling repetition of the CSS associated with the scheduling DCI

- Option 2: 1 bit for enabling/disabling SIB1 PDSCH repetition delivered by DCI scrambled by SI-RNTI

Due to different link budgets of PDCCH and SIB1 PDSCH, the repetition of the scheduling PDCCH is not necessarily applicable to PDSCH. The flexibility of dynamically enabling/disabling SIB1 PDSCH repetition should be maintained. Considering the available reserved bits in the DCI scrambled by SI-RNTI, it is possible to deliver the enabling/disabling indication by the DCI. Therefore, we have the following proposals.

Proposal 9: To reduce signaling overheads, the TDRA/FDRA of SIB1 PDSCH should be maximally reused if possible. Particularly, for the intra-slot case, the FDRA should be the same, while for the inter-slot case, both TDRA and FDRA should be the same.

Proposal 10: The offset (X) between two consecutive SIB1 PDSCH repetitions should be predefined.

Proposal 11: At least the following method can be considered to indicate the configuration of SIB1 PDSCH repetition,

- 1 bit for enabling/disabling SIB1 PDSCH repetition delivered by DCI scrambled by SI-RNTI

3.2 Msg4 PDSCH repetition

As per agreements, two or more repetitions can be supported for Msg4 PDSCH. Similar to SIB1 PDSCH, Msg4 PDSCH repetition can also be inter-slot or intra-slot. Again, the offset between the first repetition and the second repetition can be X symbols or slots, which can be indicated for flexibility. To preserve the scheduling flexibility, the repetition number larger than two is desired, and the repetition number should be indicated rather than predefined. Hence, we have the following proposals.

Proposal 12: More than two repetitions can be considered for Msg4 PDSCH repetition. The repetition number should be indicated.

Proposal 13: The offset (X) between two consecutive Msg4 PDSCH repetitions can be indicated for better flexibility.

Regarding the indication of Msg4 PDSCH repetition, there are possibly four options,

- Option 1: Follow the enabling/disabling repetition of the CSS associated with the scheduling DCI, or the that of SIB1 PDSCH

  - No signaling overheads

  - Only enabling/disabling can be indicated, while repetition number and X can only be indicated via other option

- Option 2: Associated with TBS scaling in the DCI scheduling RAR

  - No signaling overheads

  - A mapping between TBS scaling between enabling/disabling, repetition number, and X should be defined. Since TBS scaling field only has 2 bits, at least some of the configurations needs to be indicated via other options.

- Option 3: 1 or more bits in the DCI scheduling Msg4 PDSCH

  - 1 bit for enabling/disabling Msg4 PDSCH repetition

  - 1 or more bits respectively for repetition number and X

  - However, there are currently no reserved bits available in the DCI scrambled by TC-RNTI, while extending the DCI payload size induces compatibility issue and may affect link budget.

- Option 4: Configured by SIB1

  - Including enabling/disabling Msg4 PDSCH repetition, repetition number, X

  - May slightly increase the SIB1 payload

Although Option 1 and 2 have no extra signaling overheads, they heavily rely on the consistency of the channel quality between the Msg4 PDSCH and the other channels. Moreover, Option 1 and 2 impose undesirable restrictions on the scheduling flexibility. Therefore, Option 1 and 2 are undesirable. Option 3 should also be deprioritized because of the compatibility issue. Instead, Option 4 can provide sufficient scheduling flexibility.

Observation 9: There are several methods to indicate the configuration of Msg4 PDSCH repetition, including

- Option 1: Follow the enabling/disabling repetition of the CSS associated with the scheduling DCI, or the that of SIB1 PDSCH

- Option 2: Associated with TBS scaling in the DCI scheduling RAR

- Option 3: 1 or more bits in the DCI scheduling Msg4 PDSCH

- Option 4: Configured by SIB1

Although Option 1 and 2 have no extra signaling overheads, but they heavily rely on the consistency of the channel quality between the Msg4 PDSCH and the channels being followed, and meanwhile imposing undesirable restrictions on the scheduling flexibility. Option 3 raises compatibility issue.

Proposal 14: It is desirable to indicate the configuration of Msg4 PDSCH repetition by SIB1, including enabling/disabling Msg4 PDSCH repetition, the offset between two consecutive Msg4 PDSCH repetitions, and repetition number.

Another issue related to Msg4 PDSCH repetition is the UE capability indication. There are possibly two options,

- Option 1: Msg1 or MsgA Preamble

  - Dedicated preambles for two different capabilities

- Option 2: Higher layer signaling in MsgA PUSCH or Msg3 PUSCH, e.g., LCID

Since Msg4 PDSCH is behind MsgA/Msg3 PUSCH, it is not necessary to report the UE capability through Msg1/MsgA. Moreover, the dedicated grouping of preambles may significantly reduce the RACH capacity. Therefore, it is more preferable to adopt Option 2.

Proposal 15: It is desirable to indicate the UE capability of Msg4 PDSCH repetition by higher layer signalings in MsgA PUSCH or Msg3 PUSCH, e.g., LCID.

- Conclusion

In this contribution, we provide our views on downlink coverage enhancement for NR-NTN. According to the discussions, we have following observations and proposals:

Observation 1: If the SSB periodicity is extended to 160 ms, there may be two options for including the additional default SSB periodicity,

- Option 1: When operating on NTN bands, UEs may assume the SSB periodicity to be 160 ms

- Option 2: For each NTN band, RAN4 to specify the default SSB periodicity (20ms or 160ms)

Compared with Option 1, Option 2 can simplify the implementation and also avoid the unnecessary latency and overheads in the case where the network does not need larger SSB periodicity for ensuring the coverage, e.g., due to sufficient bandwidth.

Observation 2: There are some ways to indicate the configuration of PDCCH repetition for searchSpaceZero:

- Option 1: Reserved bit in MIB

- Option 2: Redefine some of the entries in the CORESET#0 table or searchSpaceZero table, e.g.,

  - Option 2-1: Reserved entries

  - Option 2-2: Under the assumption of 5MHz max. bandwidth for FR1, the entries with more than 25 PRBs

  - Option 2-3: Under the assumption of 5MHz max. bandwidth for FR1, the entries with a non-zero RB offset

  - Option 2-4: Under the assumption of more than 1 CORESET symbol, the entries with 1 CORESET symbol

For the above options,

- Both option 1 and option 2-1 can only deliver 1 bit, which can only indicate the enabling or disabling of PDCCH repetition.

- Option 2-2, 2-3 and 2-4 can deliver more information, such as the positions of the PDCCH repetition.

- Option 2-1 bars all pre-R19 UEs if PDCCH repetition is enabled, while the other options may allow the pre-R19 UEs to detect the PDCCH or receive PDSCH at the cost of potential compatibility issues.

Observation 3: The PDCCH repetition scheme introduced by mTRP is to link up to two individual search spaces with different id for connected mode UE. The two search spaces have the same periodicities and have their respective PDCCH monitoring occasions located within a same slot.

Observation 4: The following issues should be addressed for intra-slot PDCCH repetition,

- Overlapping of PDCCH repetitions and candidate SSB induces higher complexity for UEs to search for the PDCCH.

- Putting PDCCH repetitions earlier than the scheduled PDSCH causes the out-of-order scheduling issue.

- For multiplexing pattern 1, only same slot scheduling is available.

Observation 5: For inter-slot PDCCH repetition, if the network admits pre-R19 UEs, the compatibility issue can be achieved with following methods:

  - Network mutes/shifts a monitoring occasion (defined by pre-R19 rules) associated with the SSB index, and indicates the scheduling timeline based on the unmuted/unshifted monitoring occasion.

    - Pros: PDSCH repetition and PDCCH repetition can be decoupled

    - Cons:

      - For each SSB index, pre-R19 UEs can only have one MO for PDCCH reception

      - It violates the OOO scheduling rule

  - Or, network transmits PDSCHs with the same TDRA and FDRA corresponding to each PDCCH monitoring occasion that is possibly monitored by legacy UEs.

    - Pros: For each SSB index, pre-R19 UEs can have two MOs for PDCCH reception

    - Cons:

      - PDSCH repetition and PDCCH repetition are coupled

      - It violates the OOO scheduling rule

Observation 6: For the PDCCH repetition Scheme 1, i.e., reusing mTRP scheme,

- Limited to intra-slot PDCCH repetition

- Potentially large spec impacts

- Additional searchspace for repetition are defined separately with its own searchspace id, which generally requires additional signalings. Particularly, the signaling overheads seems impossible for searchSpaceZero, since no more bits than 1 reserved bit can be delivered by MIB.

Observation 7: For the PDCCH repetition Scheme 2 (Defining additional MOs), Scheme 3 (Redefining the association between the MOs and the SSB index), Scheme 4 (Linking the two candidate monitoring occasions associated with each SSB index in two consecutive slots),

- All the schemes do not need an additional searchspace, nor induce extra signaling overheads for the additional searchspace id and linking id.

- Scheme 2 can achieve a unified design of intra-slot and inter-slot PDCCH repetition.

- Scheme 2 and Scheme 3 can be applicable to intra-slot PDCCH repetition, without inducing scheduling timeline issues.

- Scheme 3 imposes undesirable scheduling restriction because only selected entries of the searchSpaceZero can be used by the network.

- Scheme 4 is limited to inter-slot repetition, which may induce scheduling timeline issues.

Observation 8: The mTRP PDCCH repetition scheme can be extended to all CSS other than searchSpaceZero, by indicating the linked searchspaces through SIB/RRC. Small spec efforts are needed and high flexibility can be achieved.

Observation 9: There are several methods to indicate the configuration of Msg4 PDSCH repetition, including

- Option 1: Follow the enabling/disabling repetition of the CSS associated with the scheduling DCI, or the that of SIB1 PDSCH

- Option 2: Associated with TBS scaling in the DCI scheduling RAR

- Option 3: 1 or more bits in the DCI scheduling Msg4 PDSCH

- Option 4: Configured by SIB1

Although Option 1 and 2 have no extra signaling overheads, but they heavily rely on the consistency of the channel quality between the Msg4 PDSCH and the channels being followed, and meanwhile imposing undesirable restrictions on the scheduling flexibility. Option 3 raises compatibility issue.

Proposal 1: For each NTN band, RAN4 to specify the default SSB periodicity (20ms or 160ms).

Proposal 2: RAN1 to clarify whether the UEs incapable of PDCCH repetition can access the network enabling the PDCCH repetition of searchSpaceZero.

Proposal 3: Depending on requirements for the number of bits available for indication and compatibility issues, one of the following methods can be used to indicate the configuration of PDCCH repetition for searchSpaceZero,

- 1 bit with potential compatibility issues: Reserved bit in MIB

- 1 bit with barring all pre-R19 UEs: Redefine reserved entries in the CORESET#0 table or searchSpaceZero table

- More than 1 bits with potential compatibility issues:

  - Under the assumption of 5MHz max. bandwidth for FR1, redefine the entries with more than 25 PRBs in the CORESET#0 table or searchSpaceZero table

  - Under the assumption of 5MHz max. bandwidth for FR1, redefine the entries with a non-zero RB offset in the CORESET#0 table or searchSpaceZero table

  - Under the assumption of more than 1 CORESET symbol, redefine the entries with 1 CORESET symbol in the CORESET#0 table or searchSpaceZero table

Proposal 4: The positions for PDCCH repetition of searchSpaceZero should be designed following the two principles,

- The positions for PDCCH repetition should not be overlapped with any candidate SSB, at least for two-symbol CORESET.

- The starting symbol of the PDSCH should not be earlier than the first symbol of the later PDCCH candidate scheduling the PDSCH.

Proposal 5: For intra-slot PDCCH repetition of searchSpaceZero, symbol 6 should be considered as the starting symbol of the repeated PDCCH candidate.

Proposal 6: If the network bars all pre-R19 UEs, for inter-slot PDCCH repetition, if a PDCCH contains multiple PDCCH candidates, the later candidate can be used to determine the scheduling timeline.

Proposal 7: If the network allows pre-R19 UEs to access, for inter-slot PDCCH repetition, at least the following methods can be considered for alleviating the compatibility issue:

  - Network mutes/shifts a monitoring occasion (defined by pre-R19 rules) associated with the SSB index, and indicates the scheduling timeline based on the unmuted/unshifted monitoring occasion.

  - Or, network transmits PDSCHs with the same TDRA and FDRA corresponding to each PDCCH monitoring occasion that is possibly monitored by legacy UEs.

  - For above cases, out-of-order scheduling should be allowed for R19 UEs.

Proposal 8: It is preferable to reuse the mTRP scheme for the CSS other than searchSpaceZero, if PDCCH repetition is supported for common search spaces beyond searchSpaceZero.

Proposal 9: To reduce signaling overheads, the TDRA/FDRA of SIB1 PDSCH should be maximally reused if possible. Particularly, for the intra-slot case, the FDRA should be the same, while for the inter-slot case, both TDRA and FDRA should be the same.

Proposal 10: The offset (X) between two consecutive SIB1 PDSCH repetitions should be predefined.

Proposal 11: At least the following method can be considered to indicate the configuration of SIB1 PDSCH repetition,

- 1 bit for enabling/disabling SIB1 PDSCH repetition delivered by DCI scrambled by SI-RNTI

Proposal 12: More than two repetitions can be considered for Msg4 PDSCH repetition. The repetition number should be indicated.

Proposal 13: The offset (X) between two consecutive Msg4 PDSCH repetitions can be indicated for better flexibility.

Proposal 14: It is desirable to indicate the configuration of Msg4 PDSCH repetition by SIB1, including enabling/disabling Msg4 PDSCH repetition, the offset between two consecutive Msg4 PDSCH repetitions, and repetition number.

Proposal 15: It is desirable to indicate the UE capability of Msg4 PDSCH repetition by higher layer signalings in MsgA PUSCH or Msg3 PUSCH, e.g., LCID.

References

- RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Huawei, 3GPP TSG RAN Meeting #102, December 11-15, 2023.

- R1-2206063, Discussions on coverage enhancement for NR NTN, vivo, 3GPP TSG RAN WG1 #110, Toulouse, France, August 22nd – 26th, 2022.

- TR 38.811, Study on New Radio (NR) to support non-terrestrial networks.

- R1-2402259, Discussions on NR-NTN downlink coverage enhancement, vivo.

- ITU-T G.711 PULSE CODE MODULATION (PCM) OF VOICE FREQUENCIES.

- TR 37.911, Study on self-evaluation towards the IMT-2020 submission of the 3GPP Satellite Radio Interface Technology (Release 18).