3GPP TSG RAN WG1 #120      					                     R1-2500403

Athens, Greece, Feb 17th – 21st, 2024

Source:                Tejas Networks Pvt. Ltd.

Title:                     Discussion on gNB-gNB CLI handling

Agenda Item:       9.3.3

Document for:     Discussion and decision

# Introduction

The RAN#104 meeting discussed and revised the Work Item Description on the Evolution of NR Duplex Operation in RP-241614[1].

Agreement

Followings are assumed based on TR 38.858

  - SBFD at the gNB side

  - Half duplex operation at the UE side

  - FR1 and FR2-1

  - SBFD operation Option 4, i.e., both time and frequency locations of sub-bands for SBFD operation are known to SBFD aware UEs

  - Coexistence between non-SBFD aware UEs (including legacy UEs) and SBFD aware UEs in the cell operating SBFD at gNB side

  - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies

  - One UL sub-band for SBFD operation in an SBFD symbol (excluding legacy UL symbol/slot) within a TDD carrier

- Mechanisms for SBFD operation shall also consider the adjacent channel coexistence between two operators

For enhancements for CLI handling, the following are recommended from RAN1's perspective:

- Information exchange of semi-static cell-specific SBFD time and frequency location configuration [RAN3]

- Information exchange of measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS [RAN3]

- Information exchange of strongest DL beam information [RAN3]

  - Information exchange of CLI-mitigation request [RAN3]

  - Note: No normative behaviour is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signalling can be discussed in RAN3.

- UL resource muting for PUSCH, including [RAN1, RAN2, RAN4]

  - Indication/Determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain

    - Note: No new DCI field / MAC CE is introduced

  - PUSCH resource mapping, i.e., rate-matching around the muted REs

  - UCI resource determination in symbols with muted REs

    - Note: No impact on data and control multiplexing as defined in section 6.2.7, TS 38.212.

  - Note: UL resource muting does not apply for Msg A PUSCH and Msg 3 PUSCH.

Agreement

Followings are assumed based on TR 38.858

  - SBFD at the gNB side

  - Half duplex operation at the UE side

  - FR1 and FR2-1

  - SBFD operation Option 4, i.e., both time and frequency locations of sub-bands for SBFD operation are known to SBFD aware UEs

  - Coexistence between non-SBFD aware UEs (including legacy UEs) and SBFD aware UEs in the cell operating SBFD at gNB side

  - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies

  - One UL sub-band for SBFD operation in an SBFD symbol (excluding legacy UL symbol/slot) within a TDD carrier

- Mechanisms for SBFD operation shall also consider the adjacent channel coexistence between two operators

For enhancements for CLI handling, the following are recommended from RAN1's perspective:

- Information exchange of semi-static cell-specific SBFD time and frequency location configuration [RAN3]

- Information exchange of measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS [RAN3]

- Information exchange of strongest DL beam information [RAN3]

  - Information exchange of CLI-mitigation request [RAN3]

  - Note: No normative behaviour is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signalling can be discussed in RAN3.

- UL resource muting for PUSCH, including [RAN1, RAN2, RAN4]

  - Indication/Determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain

    - Note: No new DCI field / MAC CE is introduced

  - PUSCH resource mapping, i.e., rate-matching around the muted REs

  - UCI resource determination in symbols with muted REs

    - Note: No impact on data and control multiplexing as defined in section 6.2.7, TS 38.212.

  - Note: UL resource muting does not apply for Msg A PUSCH and Msg 3 PUSCH.

  - Measurement reporting

    - At least aperiodic reporting

      - Note: Periodic and semi-persistent reporting can be considered

    - New report quantities: e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices

      - Note: At least wideband reporting is supported

    - UCI bits generation

    - Priority rules for multiple CSI reporting

    - Note: The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point.

  - CLI measurement accuracy requirement [RAN4]

- Note: The above information exchanges are subject to RAN3’s assessment of feasibility and specification impact.

  - Measurement reporting

    - At least aperiodic reporting

      - Note: Periodic and semi-persistent reporting can be considered

    - New report quantities: e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices

      - Note: At least wideband reporting is supported

    - UCI bits generation

    - Priority rules for multiple CSI reporting

    - Note: The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point.

  - CLI measurement accuracy requirement [RAN4]

- Note: The above information exchanges are subject to RAN3’s assessment of feasibility and specification impact.

In this contribution, we provide our views on the indication/determination of UL resource muting for PUSCH, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in the time domain and UCI resource determination, and power boosting of the UL muting symbol.

# Introduction to SBFD CLI handling

In the case of SBFD UL reception, the gNB receives the signal from its serving UEs (desired signal), and it also receives CLI interference from the neighbouring gNBs of the same network and/or different network gNBs, due to which there is a significant drop in UL throughput performance. To measure the interference for its mitigation, UE needs to mute some of the symbols such that the UL receiver at gNB can measure the interference due to neighbouring gNBs accurately and use it for equalizer weight calculation to suppress interference. The UL Resource muting for PUSCH is done assuming comb-2 for DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in the time domain per slot. As precoding/beamforming applied for PDCCH symbols and PDSCH symbols can be different, this would lead to uplink symbols in a slot experiencing different gNB-gNB CLI. To measure the gNB-gNB CLI, in uplink, we can mute one symbol out of the first three symbols of the slot to capture the interference due to PDCCH channel leakage of the interfering gNB and one OFDM symbol out of the remaining symbols of the slot to measure the gNB-gNB interference due to PDSCH channel leakage. The information pertaining to the symbol(s) to be muted needs to be present at both UE and gNB.

Agreement

If transform precoding is enabled for a PUSCH, study further the following two options:

- Option 1: The DFT size is modified as MscPUSCH2 for symbol(s) with UL resource muting

- Option 2: The DFT size is MscPUSCH for symbol(s) with UL resource muting

Note: MscPUSCH is the total number of subcarriers of the PUSCH.

FFS: Whether any of option 1, option 2 needs to be reflect into RAN1 specifications.

Agreement

If transform precoding is enabled for a PUSCH, study further the following two options:

- Option 1: The DFT size is modified as MscPUSCH2 for symbol(s) with UL resource muting

- Option 2: The DFT size is MscPUSCH for symbol(s) with UL resource muting

Note: MscPUSCH is the total number of subcarriers of the PUSCH.

FFS: Whether any of option 1, option 2 needs to be reflect into RAN1 specifications.

UL resource muting for transform precoding enabled PUSCH transmission

As in case of option 1, the UE need to switch the size of DFT from MscPUSCH point DFT to MscPUSCH/2 point DFT for the UL muting symbol and then switch back to MscPUSCH point DFT for the non-UL muting symbol. But in case of option 2, we need to repeat the input data symbols to DFT twice to get UL muting pattern for the UL muting symbol and the need to adjust the power of signal which will add additional complexity to UE transmitter, and to get the desired comb offset value of muting pattern, we need to multiply the input symbol of the DFT with incremental phase value. Hence, the UL resource muting can be achieved by both the methods, but the complexity of the option 1 is less.

Observation 1: The complexity of the Option 1 is less than the option-2.

Proposal 1: As the implementation complexity of the Option 1 is less than Option 2. Hence, we support Option 1 to reflect in the RAN1 specification.

If transform precoding is enabled for a PUSCH,

- Option 1: The DFT size is modified as MscPUSCH2 for symbol(s) with UL resource muting

Note: MscPUSCH is the total number of subcarriers of the PUSCH.

Agreement

3dB power boosting is assumed for REs in the PUSCH symbol not containing PT-RS with UL resource muting.

- FFS: PUSCH and PT-RS EPRE determination for the PUSCH symbol containing PT-RS with UL resource muting when transform precoding is not enabled.

Agreement

3dB power boosting is assumed for REs in the PUSCH symbol not containing PT-RS with UL resource muting.

- FFS: PUSCH and PT-RS EPRE determination for the PUSCH symbol containing PT-RS with UL resource muting when transform precoding is not enabled.

3dB power bosting for PUSCH symbols containing PT-RS with UL resource muting

When PT-RS is transmitted with PUSCH for more than 1-layer PUSCH transmission, EPRE of the PUSCH and PT-RS is different for per layer per RE as shown in the Table 1. If the precoding matrix of the PUSCH and PT-RS signal is identity matrix for codebook-based transmission or non-code book-based transmission, then the layers are directly mapped to the ports. When PUSCH and PT-RS data is mapped to the subframe symbols, the power of the PUSCH symbol containing PT-RS is more than the symbol containing only PUSCH for full coherent scenarios. In the case of UL muting symbol containing PUSCH and PTRS symbols then 3dB power boosting further increase the muting symbol power as compared to the symbol containing only PUSCH symbols.

Table 1: Factor related to PUSCH to PT-RS power ratio per layer per RE  other than 8TX PUSCH transmission and other than SDM PUSCH

| UL-PTRS-power / |  | The number of PUSCH layers ( ) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 |  | 3 |  | 4 |  |  |
|  | All cases | Full coherent | Partial and non- coherent and non-codebook based | Full coherent | Partial and non- coherent and non-codebook based | Full coherent | Partial coherent | Non-coherent and non-codebook based |
| 00 | 0 | 3 | 3Qp-3 | 4.77 | 3Qp-3 | 6 | 3Qp | 3Qp-3 |
| 01 | 0 | 3 | 3 | 4.77 | 4.77 | 6 | 6 | 6 |
| 10 | Reserved |  |  |  |  |  |  |  |
| 11 | Reserved |  |  |  |  |  |  |  |

Observation 2: The symbol with 3dB power boosting for the REs in the PUSCH symbol containing PT-RS with UL resource muting, have higher power as compared to the symbol with PUSCH symbol containing PT-RS with no UL resource muting.

Proposal 2: We support the 3dB power boosting for the REs in the PUSCH symbol containing PT-RS with UL muting when transform precoding is not enabled if there is not impact on signal quality or MPR.

### UL resource muting for PUSCH that is a retransmission

As we know, PUSCH support up to 3 retransmissions for a HARQ process, it may happen that 2nd and 3rd retransmission happen after large delay with respect to PUSCH new data transmission of a HARQ process. This large delay in 2nd and 3rd retransmission can cause change in the CLI from neighbouring gNBs with respect to PUSCH new data transmission of that HARQ process, which require the UL resource muting for CLI measurement.

Proposal 3: Support UL resource muting for the retransmission if UL resource muting is done for new data transmission for that PUSCH.

### UL resource muting for multi-PUSCH scheduled by a single DCI

The RS, HARQ, RRC signalling and CSI are the high priority UCIs. The UCI type HARQ and RS have very low payload size. We think, the performance of these UCI types does not degrade significantly due to UL resource muting. For UCI type CSI and RRC signalling, we can consider UL resource muting to be not applied.

Proposal 4: Support PUSCH carrying certain UCI type with high priority, UL resource muting is not applied.

### UL resource muting for multi-PUSCH scheduled by a single DCI

As, the time domain resources allocation for the each PUSCH transmission are configured separately for all the PUSCH scheduled by a single DCI. The time location of UL resource muting can be turned ON/OFF dynamically for each PUSCH using TDRA field in DCI.

Proposal 5: Support the UL resource muting for each of the PUSCH transmission of multi-PUSCH scheduled by a single DCI can be turned ON/OFF using their TDRA field in DCI.

### UL resource muting for a PUSCH with the following conditions

- Condition 1: When the number of PUSCH symbols is less than threshold, UL resource muting is not applied

- Condition 2: If the scheduled PUSCH uses a MCS lower than certain reference MCS index, UL resource muting is not applied

- Condition 3: When the number of scheduled RBs is less than threshold, UL resource muting is not applied

- Condition 4: When the number of DMRS symbols is larger than threshold, UL resource muting is not applied

The condition 1, 2, 3 and 4, can be handled using dynamic turn ON/OFF UL resource muting using TDRA field in DCI. Hence, no discussion required on the condition 1,2 3 and 4.

Proposal 6: The UL resource muting can be turned ON/OFF dynamically using TDRA field in DCI. Hence no conditional check required for following conditions,

- Condition 1: When the number of PUSCH symbols is less than threshold, UL resource muting is not applied

- Condition 2: If the scheduled PUSCH uses a MCS lower than certain reference MCS index, UL resource muting is not applied

- Condition 3: When the number of scheduled RBs is less than threshold, UL resource muting is not applied

- Condition 4: When the number of DMRS symbols is larger than threshold, UL resource muting is not applied

# Conclusion

In this contribution, we discuss about CLI handling with the following proposals and observations.

Observation 1: The complexity of the Option 1 is less than the option-2.

Observation 2: The symbol with 3dB power boosting for the REs in the PUSCH symbol containing PT-RS with UL resource muting, have higher power as compared to the symbol with PUSCH symbol containing PT-RS with no UL resource muting.

Proposal 1: As the implementation complexity of the Option 1 is less than Option 2. Hence, we support Option 1 to reflect in the RAN1 specification.

If transform precoding is enabled for a PUSCH,

- Option 1: The DFT size is modified as MscPUSCH2 for symbol(s) with UL resource muting

Note: MscPUSCH is the total number of subcarriers of the PUSCH.

Proposal 2: We support the 3dB power boosting for the REs in the PUSCH symbol containing PT-RS with UL muting when transform precoding is not enabled if there is not impact on signal quality or MPR.

Proposal 3: Support UL resource muting for the retransmission if UL resource muting is done for new data transmission for that PUSCH.

Proposal 4: Support PUSCH carrying certain UCI type with high priority, UL resource muting is not applied.

Proposal 5: Support the UL resource muting for each of the PUSCH transmission of multi-PUSCH scheduled by a single DCI can be turned ON/OFF using their TDRA field in DCI

Proposal 6: The UL resource muting can be turned ON/OFF dynamically using TDRA field in DCI. Hence no conditional check required for following conditions,

- Condition 1: When the number of PUSCH symbols is less than threshold, UL resource muting is not applied

- Condition 2: If the scheduled PUSCH uses a MCS lower than certain reference MCS index, UL resource muting is not applied

- Condition 3: When the number of scheduled RBs is less than threshold, UL resource muting is not applied

- Condition 4: When the number of DMRS symbols is larger than threshold, UL resource muting is not applied

# Reference

- RP-241614 Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD).

- RP-240172, Evolution of NR duplex operation: Sub-band full duplex (SBFD).

- 3GPP TR 38.858: “Study on Evolution of NR Duplex Evolution”.

- R1-2403894, Discussion on gNB-gNB CLI handling, Tejas Networks Ltd.

- RP-240789, Evolution of NR duplex operation: Sub-band full duplex (SBFD).

- Chair notes RAN1 119 eom0.