3GPP TSG RAN WG1 #120			R1-2500406

Athens, Greece, February 17th – 21st, 2025

Agenda item  :	     9.1.3.1

Source           :	Tejas Networks

Title                :	Discussion on AI/ML for CSI prediction

Document for: Discussion

# Introduction

In the RAN#102 meeting, a new work item on air-interface AI/ML was approved for Rel-19 [1]. CSI feedback enhancement is a study objective of the new work item, and CSI prediction is part of the continued study, including the performance gain over non-AI/ML based approach, and other remaining issues that have not been concluded in Rel-18.

Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - CSI feedback enhancement [RAN1]: - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843. - For CSI prediction (one-sided model), further study performance gain over Rel-18 non-AI/ML based approach and associated complexity, while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843 (e.g., cell/site specific model could be considered to improve performance gain). …

# Discussion

The following agreements are agreed in RAN#117 for CSI prediction.

Agreement For the boundary between Type 3 and Type 1 performance monitoring, the difference is whether UE reports performance metric or performance monitoring output to NW, respectively. - The monitoring output is determined based on performance metric, and additionally, baseline and/or threshold criterion if configured. Observation For CSI prediction using UE-sided model, for performance monitoring, at least following specification impacts are additionally identified compared to that has been captured in TR38.843, - Type 1 - Definition/configuration of performance metric - Definition of threshold criterion, if configured - Definition/configuration and report of monitoring output, and corresponding report mechanism - Type 2 - Definition/configuration and report of ground truth CSI, and corresponding report mechanism. - Type 3 - Definition/configuration and report of performance metric, and corresponding report mechanism. - For all types of performance monitoring, NW indication to the UE of the decision regarding the monitoring action Agreement For the evaluation of AI/ML-based CSI prediction using localized models in Release 19, regarding training, - The k-th local model is trained on region #B_k (the k-th local region), 1<=k<=N. - The generalized model is trained on Region #A that may be constructed via any of the following methods that is appropriate for the given generalized/local region modeling approach. - Region #A is the same as the union of regions #B_1, …, #B_N. - Region #A is a proper superset of the union of regions #B_1, …, #B_N. - Region #A is generated separately from regions #B_1, …, #B_N. - Note: companies to report which method was used. For the evaluation of AI/ML-based CSI prediction using localized models in Release 19, regarding testing, - The trained generalized model, local model, and the non-AI/ML benchmark are tested on the regions #B_1, …, #B_N. - In case N>1, when reporting the results, companies may report the performance of the generalized model, the local models, and the non-AI/ML benchmark, by averaging the performance over the regions #B_1,…,B_N. Companies to report the value of N.

- This contribution provides discussion on LCM mode, Data collection and Performance monitoring for CSI prediction.

## Consistency Between Training and Inference

The consistency of network-side additional conditions, such as antenna port configurations and beam characteristics, plays a key role in ensuring that AI/ML models perform reliably during inference. Without proper alignment between training and inference conditions, significant performance degradation can occur.

Observation 1: Variations in interference distributions during training and inference can lead to significant reductions in CSI prediction accuracy.

Observation 2: Differences in network-side additional conditions, such as TxRU mappings, may degrade AI model performance, even if antenna configurations are consistent.

Proposal 1: Use of Associated ID for Consistency

Consider an associated ID-based solution to ensure consistency between training and inference. This method is already explored in AI-based beam management scenarios and could be extended to CSI prediction.

- Step 1: Network signals associated ID during training data collection.

- Step 2: AI/ML models are trained using data corresponding to the associated ID.

- Step 3: The associated ID is used during inference to align the conditions between training and inference phases.

In addition to the associated ID solution, we recommend incorporating performance monitoring to validate consistency. Network and user equipment (UE) can jointly monitor performance metrics and make adjustments based on observed discrepancies. Consider performance monitoring through network configuration, UE-based evaluation, or a joint approach combining both network and UE-based monitoring for real-time feedback.

The agreements mention two aspects that potentially impact the consistency of the training and inference: variations in tilt angle and TXRU mapping. Both variations may happen in real-life deployments. For example, the electronic tilt angle of the gNB can be adjusted semi-statically to adapt the spatial-temporal distributions of the cell traffic.

Diverse simulation observations were presented at RAN1 #119 meetings for these two aspects. However, even for the optimistic results, the performance loss caused by the tilt angle may be up to 3%, which is still comparable to the performance gain of AI/ML-based CSI predictions. Therefore, we have the following proposals.

Proposal 2: For CSI Prediction, Tilt angle and/or TXRU mapping will cause the performance degradation.

## Model Inference

Due to the strong non-linear processing capability of AI/ML models, AI/ML-based CSI prediction is more flexible regarding the prediction distance and intervals of the time instances. For example, the super-resolution capability of AI/ML models can predict the PMI with shorter intervals when the periodicity of the measurements is much longer. AI/ML can also make more predictions far into the future than legacy algorithms, which can reduce partial P/SP CSI-RS transmissions to reduce the overhead. Corresponding enhancements can be considered for the normative work to fully explore the potential capability of AI/ML-based CSI prediction.

Proposal 3: Consider CSI frame work for Resources and reports of AI/ML-based CSI prediction

## LCM mode

For UE side models and UE part of two-sided models we have two flavours for LCM mode, one is based on AI/ML functionality identification and another one is based on AI/ML model identification.

For AI/ML functionality identification:

-	Legacy 3GPP framework of feature is taken as a starting point.

-	UE indicates supported functionalities/functionality for a given sub-use-case.

-	UE capability reporting is taken as starting point.

For AI/ML model identification:

-	Models are identified by model ID at the Network. UE indicates supported AI/ML models.

In functionality-based LCM, network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signalling (e.g., RRC, MAC-CE, DCI). Models 	may not be identified by the Network, and UE may perform model-level LCM. In our view, we consider NW to transfer Model ID to the UE. In model-ID-based LCM, models are identified by the Network, Network will indicate the model ID through signalling (for e.g., RRC, MAC-CE, DCI) provided UE supports the Model.

Proposal 4: For LCM we support AI/ML model identification

- Models are identified by model ID by the Network.

- UE indicates supported AI/ML models.

## Data collection

Data collection may be performed for different purposes in LCM, e.g., model training, model inference, model monitoring, model selection, model update, etc. Each may be done with different requirements and potential specification impact [1]. For CSI prediction sub use case, the model input is channel information measured by UE side. Channel information can be channel matrix or precoding matrix. CSI RS transmission can be periodic or aperiodic within the observation window. To initiate the CSI prediction either NW can trigger the UE for measurements or UE itself can request NW to initiate the CSI RS transmission. CSI prediction accuracy depends on number of CSI RS measurements within the observation window. UE decides the minimum number of CSIRS instances required for accurate CSI prediction and report it to the NW. At least for model inference we can reuse the CSI-RS configuration from Rel-18 Type II Doppler codebook.

Proposal 5: Consider the following options to initiate CSIRS transmission for prediction

- NW trigger CSI RS transmission for prediction

- UE initiate the CSI RS transmission for prediction

Proposal 6: Consider CSIRS transmission can be periodic or aperiodic for CSI prediction

Observation 3: Prediction accuracy depends on number of instances of CSIRS within the observation window.

Proposal 7: Consider that UE should report minimum number of CSIRS instances required for CSI prediction to the NW.

Proposal 8: Reuse the CSI-RS configuration from Rel-18 Type II Doppler codebook for model inference.

Proposal 9: Consider the maximum time gap that allowed between prediction and observation window for better prediction accuracy.

UE must obtain the actual measured CSI at or around the predicted instances for UE-side model training or performance monitoring. Without these data collection requirements, the CSI-RS transmissions may be skipped in these instances since the predicted CSI can be referred. However, additional CSI-RS transmissions are needed to let UE measure the actual CSI for the data collection. For that purpose, transmitting the CSI-RS with the same temporal domain density or periodicity as the inference is unnecessary. A different CSI resource, usually with a different periodicity or separately triggered on demand, can be used for the data collection. The straightforward way, without much specification impact, is to configure CSI resources for model training or monitoring separately. It can be P/SP CSI resources with longer periodicity or AP CSI resources, which can be triggered on demand.

Proposal 10: For data collection of CSI prediction, Consider existing CSI framework.

- CSI resources for inference and training/monitoring.

## Performance monitoring

Regarding the types of monitoring procedures, the main differences are in the role of UE and NW taken to do performance metric calculation and make decisions. We may first elaborate on the details of each type before making down-selection.

Type 1:

For Type 1, UE calculates the performance metric(s). After calculating performance metric(s) UE will not report the performance metric to the NW instead UE reports performance monitoring output to the NW. Upon receiving monitoring

output, the NW facilitates the functionality fallback decision or model switching or model parameter update to the UE. For performance monitoring output NW may define threshold criterion to the UE. The threshold criterion could be statistical value (e.g., mean or x% of CDF) within the observation window or per sample based SCGS value. It may be better to consider that UE reports its monitoring output to the NW only if it falls below the threshold to avoid extra signalling.

Proposal 11: Consider the following as a potential specification impact for performance monitoring:

- Configuration of CSI-RS for performance monitoring

- Configuring threshold criterion

- Measurement of monitoring output

- Reporting of monitoring output (if monitoring output is below the threshold)

- Model switching or Model parameter update

- Functionality fallback operation

Proposal 12: Consider the NW to assign the threshold criterion for UE.

Proposal 13: Consider that UE only reports performance monitoring output only when monitoring output is below the threshold.

Type 3:

For Type 3, UE calculates the performance metric(s). UE reports the performance metric to the NW. NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). For performance matric UE could consider KPIs like SGCS or NMMSE per sample or it can be a statistical value within the monitoring window.

Proposal 14: Consider the following as a potential specification impact for Type 3:

  - Configuration of CSI-RS for performance monitoring

  - Measurement of metric and reporting it to UE

  - Model switching or Model parameter update

  - Functionality fallback operation

Proposal 15: For Type 1 and 3 performance monitoring the performance metric per sample should be intermediate KPI (e.g., SGCS, NMSE), or statistical value within the within the window (e.g., mean value SGCS, NMMSE).

Proposal 16: Type 1 performance should be prioritized.

# Conclusion

According to the discussion, following proposals are provided:

Observation 1: Variations in interference distributions during training and inference can lead to significant reductions in CSI prediction accuracy.

Observation 2: Differences in network-side additional conditions, such as TxRU mappings, may degrade AI model performance, even if antenna configurations are consistent.

Proposal 1: Use of Associated ID for Consistency

Consider an associated ID-based solution to ensure consistency between training and inference. This method is already explored in AI-based beam management scenarios and could be extended to CSI prediction.

- Step 1: Network signals associated ID during training data collection.

- Step 2: AI/ML models are trained using data corresponding to the associated ID.

- Step 3: The associated ID is used during inference to align the conditions between training and inference phases.

Proposal 2: For CSI Prediction, Tilt angle and/or TXRU mapping will cause the performance degradation.

Proposal 3: Consider CSI frame work for Resources and reports of AI/ML-based CSI prediction.

Proposal 4: For LCM we support AI/ML model identification

- Models are identified by model ID by the Network.

- UE indicates supported AI/ML models.

Proposal 5: Consider the following options to initiate CSIRS transmission for prediction

- NW trigger CSI RS transmission for prediction

- UE initiate the CSI RS transmission for prediction

Proposal 6: Consider CSIRS transmission can be periodic or aperiodic for CSI prediction

Observation 3: Prediction accuracy depends on number of instances of CSIRS within the observation window.

Proposal 7: Consider that UE should report minimum number of CSIRS instances required for CSI prediction to the NW.

Proposal 8: Reuse the CSI-RS configuration from Rel-18 Type II Doppler codebook for model inference.

Proposal 9: Consider the maximum time gap that allowed between prediction and observation window for better prediction accuracy.

Proposal 10: For data collection of CSI prediction, Consider existing CSI framework.

- CSI resources for inference and training/monitoring.

Proposal 11: Consider the following as a potential specification impact for performance monitoring:

- Configuration of CSI-RS for performance monitoring

- Configuring threshold criterion

- Measurement of monitoring output

- Reporting of monitoring output (if monitoring output is below the threshold)

- Model switching or Model parameter update

- Functionality fallback operation

Proposal 12: Consider the NW to assign the threshold criterion for UE.

Proposal 13: Consider that UE only reports performance monitoring output only when monitoring output is below the threshold.

Proposal 14: Consider the following as a potential specification impact for Type 3:

  - Configuration of CSI-RS for performance monitoring

  - Measurement of metric and reporting it to UE

  - Model switching or Model parameter update

  - Functionality fallback operation

Proposal 15: For Type 1 and 3 performance monitoring the performance metric per sample should be intermediate KPI (e.g., SGCS, NMSE), or statistical value within the within the window (e.g., mean value SGCS, NMMSE).

Proposal 16: Type 1 performance should be prioritized.

# References

- 3GPP TR 38.843: V18.0.0

- R1-2405492	Summary #4 of CSI prediction	Moderator (LG Electronics)