3GPP TSG RAN WG1 #120		R1-2500451

Athens, Greece, February 17th – 21st, 2025

Source:	OPPO

Title:                     Discussion on physical channels design for A-IoT

Agenda Item:       9.4.2

Document for:	Discussion and Decision

- Introduction

In RAN#106 meeting, the WID on solutions for Ambient IoT was endorsed including following general scope and RAN1-led objectives [1]:

General Scope The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope: - The overall objective shall be to standardize the following Ambient IoT device: Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Deployment scenario 1 with Topology 1, according to D1T1-B. - FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum. - Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. - Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command). - Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B - Proximity determination via Solution 1 in TR 38.769 only. - Device (un)availability via Direction 1 in TR 38.769 only. WGs begin their discussions from the decisions already made in TR 38.769, with the following refinements for the scope: The following objectives are set, within the General Scope: - RAN1 scope: - PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively. - R2D and D2R signal(s) - Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA. - R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations. - R2D transmission supports only the Manchester line code in TR 38.769 - D2R transmission supports: - Either the Manchester line code in TR 38.769 or no line code (one to be down-selected); and - A corresponding small frequency shift method according to the options in TR 38.769. - R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis). - PRDCH and PDRCH both support transmission without CRC, and with CRC as per the generator polynomials in TS 38.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis) for 6-bit CRC and 16-bit CRC. Cases to use which length of CRC, or no CRC, to be decided in RAN1. - D2R supports physical layer repetition transmission. R2D does not support physical-layer repetition transmission.

In this paper, we discuss the physical channel design for R19 A-IoT system.

- Discussion

2.1 CRC for R2D and D2R

The following agreements regarding CRC were reached in RAN1#116-bis.

Agreement Study - baseline: using 6 bits and 16 bits CRC with polynomials from TS 38.212, or no CRC, for PRDCH - baseline: using 6 bits and 16 bits CRC with polynomials from TS 38.212, or no CRC, for PDRCH - FFS: details when different CRC lengths or no CRC may be used - FFS: other 6 bits and 16 bits CRC with different polynomials than from TS 38.212

The agreement shows that both no CRC and with CRC (CRC-6 or CRC-16) can be considered for PRDCH and PDRCH. Regarding in which case no CRC can be used, the mechanism in RF ID can be taken as baseline. In RF ID, the ACK and NAK command without CRC which corresponds to RN-16 reporting from one Tag is supported. Similarly, in A-IoT, if TDMA is used for Msg-1 transmission, a Msg-2 (including ACK command and detected ID) which corresponds to the Msg-1 transmission from device can be transmitted without CRC. While if FDMA is applied for Msg-1 transmission from multiple devices, a Msg-2 transmission can include the response corresponds to multiple Msg-1 transmissions. In that case, CRC can be attached to the Msg-2 to guarantee the detection performance.

Proposal 1: Regarding PRDCH with Msg-2 (including ACK command and detected ID):

- If the Msg-2 corresponds to one device, PRDCH without CRC is supported.

- If the Msg-2 corresponds to more than one device, PRDCH with CRC is supported.

Regarding whether there is some case that PDRCH can be transmitted without CRC, we will consider different transmission for inventory and command use cases respectively. For inventory use case, Msg-1 and Msg-3 (if supported) which corresponds to RN-16 or EPC reporting will be transmitted using PDRCH, CRC should be applied for verification. For command use case, if ACK corresponding to R2D command is supported, whose transmission without CRC can be considered.

Proposal 2: Regarding CRC for PDRCH:

- For inventory use case, CRC should be applied to Msg-1 and Msg-3 (if supported) transmission

- For command use case, if ACK corresponding to R2D command is supported, without CRC can be considered

Regarding whether CRC-6 or CRC-16 can be applied to PRDCH or PDRCH, the following factors can be considered: TBS, false alarm rate or error detection performance, overhead, and message type. Considering the trade-off between overhead and error detection performance, a threshold of TBS can be configured to determine whether CRC-6 or CRC-16 can be used.

Proposal 3: A threshold of TBS can be pre-defined to determine whether CRC-6 and CRC-16 needs to be used.

The following agreements of CRC generator polynomials were reached in RAN1#118bis:

Agreement For the CRC generator polynomials, capture the following observations in the TR: - Sources China Telecom, OPPO, Xiaomi, CATT, Samsung recommend that the same polynomials from TS 38.212 are reused, giving justifications: - Sources CATT, OPPO state that the polynomials from TS 38.212 were already carefully and thoroughly evaluated and ensured in the NR channel coding design, so there is no need of considering other polynomials. - Source Xiaomi states that the device complexity is increased when different polynomials are introduced for the D2R transmission. - Source China Telecom states that link performance is significantly impacted by the CRC lengths, and not the CRC polynomials, hence the polynomials from TS 38.212 should be used. - On the other hand, sources [ZTE], [Qualcomm] recommend that the same polynomial for CRC-6 gCRC6D=D6+D5+1 from TS 38.212 is reused, but a new polynomial for CRC-16 is introduced, gCRC16D=D16+D11+D6+D5+1, incorporating the CRC-6 polynomial, giving justifications: - The proposed new CRC-16 polynomial nests the existing CRC-6 within its structure. - Using separate sets of circuits to implement the polynomials for CRC-6 and CRC-16 as per TS 38.212 requires a higher number of registers and XOR gates when compared to using the new CRC-16 polynomial. - FAR performance is the same for the legacy and proposed new polynomial for CRC-16, but the new polynomial reduces the complexity and power consumption of the device.

The CRC generator polynomials of CRC-6 or CRC-16 were thoroughly studied and specified in LTE and NR system. The same mechanism can be applied to A-IoT system unless serious issue can be addressed.

Proposal 4: The CRC generator polynomials of LTE and NR for CRC-6 or CRC-16 can be reused for A-IoT.

Regarding CRC for R2D data and control, the following options were discussed in RAN1#118bis:

Agreement For the PRDCH design details, following is captured in the TR 38.769: Following design options have been studied for PRDCH: - Option 1: If there is no L1 R2D control information carried by PRDCH, then PRDCH transmission with only R2D data is considered - Note: In this option, R2D control information (if any) maybe carried by higher-layer signaling - Option 2: If any L1 R2D control information is supported & carried by PRDCH, then for the CRC attachment (appended if there is non-zero length CRC), following two options have been studied - Option 2-1: Joint CRC is attached to L1 R2D control information and R2D data - Option 2-2: Separate CRC is attached to L1 R2D control information and R2D data, respectively

Firstly, regarding how to multiplex R2D control information and R2D data packet, we prefer to transmit them separately using same PRDCH, since the control information should have higher reliability requirement than data packet. In that case, it is preferred to apply different CRC for R2D control information and data packet. Whether same or different CRC length for control information and data packet can be FFS.

Proposal 5: If L1 R2D control information is supported & carried by PRDCH, separate CRC is attached to L1 R2D control information and R2D data respectively is preferred.

- FFS whether same or different CRC length for R2D control information and data packet

2.2 Line code and small frequency shift for D2R

According to the WID, for D2R transmission, either Manchester or no line code (to be down-selected) can be considered. The small frequency shift operations for Manchester line code and no line code were discussed in RAN1#118 and RAN1#118bis, the following agreements were reached.

[RAN1#118] Agreement Small frequency shifts for D2R are studied for OOK and BPSK: - For applying with Manchester line codes - Option 1: By repetition of the codewords within the same time duration corresponding to an information bit. FFS how to define this repetition. - Option 2: By multiplying the Manchester codeword with a square wave corresponding to the small frequency-shift. - Companies to report how they perform multiplying for option 2 - For applying with Miller line codes, according to Figure 6-13 of UHF RFID standard. - For FM0, small frequency shift is not defined - If no D2R line code is used, by using a square-wave corresponding to the small frequency-shift. - Potential purposes include: - FDMA of D2R, if supported - CW interference avoidance, if supported - Note: small frequency shifts for D2R are studied for the same potential purposes for relevant identified BFSK variant(s) [RAN1#118bis] Agreement For small frequency shifts in D2R, the following observations are captured in TR 38.769: - Sources [Futurewei], [Vivo] and [Qualcomm] state that Manchester codeword repetitions within the same time duration corresponding to an information bit is equivalent to bit-level repetitions within the same duration prior to Manchester encoding. - Sources [Vivo], [ZTE] and [NEC] state that the output waveform for Manchester line codes by multiplying the codeword with a square wave corresponding to the small frequency shift (Option 2) introduces a phase reversal of the output waveform in the middle of the time duration corresponding to an information bit as compared to Manchester line codes by repetition of the codewords within the same time duration. - Sources [Futurewei], [Ericsson] [CMCC] [QC] [IDC] [LG] state that the output waveform using a square-wave corresponding to the small frequency-shift and no line codes is equivalent to using Manchester line codes by repetition of the codewords within the same time duration corresponding to an information bit. - Source [ZTE] shows that small frequency shift (Option 1) is more sensitive to SFO and the time difference between devices than small frequency shift (Option 2), and small frequency shift via Miller code has the worst performance compared with small frequency shift (Option 1) and small frequency shift (Option 2).

Regarding whether line codes is applied to D2R, we prefer to use Manchester line code over no line code.

Proposal 6: Manchester line code is supported to D2R transmission.

For small frequency shift of Manchester line code, Option 1 has a more concentrated spectrum, and requires lesser bandwidth as compared to Option 2, and Option 1 an achieve additional gain for coverage evaluation due to lower effective noise power. Therefore, we Option 1 is preferred.

How to perform repetition for Option 1 was discussed and agreed as follows in RAN1#118bis:

Agreement For small frequency shifts in D2R using Manchester line codes by repetition of the codewords within the same time duration Tb corresponding to an information bit (Option 1), each Manchester codeword is repeated by a codeword repetition number R, where R = Tb/(2 * chip length), such that the amount of small frequency shift in Hz is R/Tb = 1/(2 * chip length).

Proposal 7: Regarding small frequency shift operation for Manchester line codes, the following option (corresponding to option 1) is supported

- For repetition of the codewords within the same time duration Tb corresponding to an information bit (Option 1), each Manchester codeword is repeated by a codeword repetition number R, where R = Tb/(2 * chip length), such that the amount of small frequency shift in Hz is R/Tb = 1/(2 * chip length).

2.3 Convolutional coding for D2R

According to the WID, only convolutional codes as FEC mechanism for D2R transmission is supported. One open issue is whether to use the same generator polynomials as LTE. In our view, the same mechanism as LTE can be used to A-IoT. The performance of convolutional code based on the generator polynomials of LTE was thoroughly studied in LTE. It can also be supported by Device 1 considering complexity and power consumption.

Proposal 8: The generator polynomials of LTE convolutional code are used for D2R

2.4 Repetition for D2R

Block-level and bit-level repetition type 1 and type 2 are studied for D2R repetition. Block level repetition has following benefits compared to bit level repetition: more diversity gain in time domain considering large time gap of coded bits for a same information bit, lower decoding latency if 1st block can be decoded correctly, better decoding performance. The disadvantage of block level repetition is it requires larger memory size. While considering only inventory and command use cases considered in R19 A-IoT, the TBS of D2R is very small. The memory size is not critical for Device 1. Therefore, we prefer to support block level repetition for D2R transmission.

Proposal 9: Block level repetition is supported for D2R transmission

- Conclusion

In this paper, we discussed physical channel design for A-IoT, the following proposals are given.

Proposal 1: Regarding PRDCH with Msg-2 (including ACK command and detected ID):

- If the Msg-2 corresponds to one device, PRDCH without CRC is supported.

- If the Msg-2 corresponds to more than one device, PRDCH with CRC is supported.

Proposal 2: Regarding CRC for PDRCH:

- For inventory use case, CRC should be applied to Msg-1 and Msg-3 (if supported) transmission

- For command use case, if ACK corresponding to R2D command is supported, without CRC can be considered

Proposal 3: A threshold of TBS can be pre-defined to determine whether CRC-6 and CRC-16 needs to be used.

Proposal 4: The CRC generator polynomials of LTE and NR for CRC-6 or CRC-16 can be reused for A-IoT.

Proposal 5: If L1 R2D control information is supported & carried by PRDCH, separate CRC is attached to L1 R2D control information and R2D data respectively is preferred.

- FFS whether same or different CRC length for R2D control information and data packet

Proposal 6: Manchester line code is supported to D2R transmission.

Proposal 7: Regarding small frequency shift operation for Manchester line codes, the following option (corresponding to option 1) is supported

- For repetition of the codewords within the same time duration Tb corresponding to an information bit (Option 1), each Manchester codeword is repeated by a codeword repetition number R, where R = Tb/(2 * chip length), such that the amount of small frequency shift in Hz is R/Tb = 1/(2 * chip length).

Proposal 8: The generator polynomials of LTE convolutional code are used for D2R

Proposal 9: Block level repetition is supported for D2R transmission

- References

[1]   RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR