3GPP TSG RAN WG1#120		R1-2500488

Athens, Greece, February 18th – 22nd, 2024

Source:	Panasonic

Title: 	Discussion on other aspects of A-IoT

Agenda Item:		9.4.4

Document for:	Discussion

# Introduction

Ambient IoT (A-IoT) is a new type of device, with low complexity and low-power consumption, defined to be integrated in 5G NR. The A-IoT study began in RAN during Rel. 18 and continued in RAN-WGs during Rel. 19 [1, 2]. 3GPP recently defined a new WI for A-IoT in Rel. 19 [3]. The WID specifies the support of A-IoT Device 1 in a Deployment scenario 1 with Topology 1. The spectrum deployment is in-band and standalone FDD in FR1. Accordingly, RAN1 needs to consider the control signals to support the mentioned features. In this contribution, we present our view regarding the L1 control signals.

# Discussions

## R2D control information

A single PRDCH channel is defined to perform the R2D transmissions. Two different approaches were considered for delivering the R2D control information, 1) carried by a higher layer signaling along the data information or 2) carried by L1 control signaling. Regardless of which approach is taken, the reader can include the target device ID(s) and each device only needs to monitor and decode the control information content containing target device ID(s) to determine whether it is targeted. The targeted device(s) further monitors and decodes the rest of the message. This approach would decrease the power consumption of A-IoT devices, which have very limited energy capacity.

Proposal 1: The control information containing target device ID(s) should be detectable by the device without receiving the rest of the message in PRDCH.

The reader may also need to deliver other control information to the A-IoT devices. The possible control information includes system information, configuration of contention-based channel access, the indication of R2D data transmissions, triggering signal for D2R transmissions, and the D2R resource allocations. These can be a higher layer signaling or L1 control signaling. Our view is carrying some control by a higher layer and some other by L1 control signaling prevent simplification and optimization of the overall control design. In addition, the specification maintenance in the feature would be complicated. Hence, the same layer should handle the control information with size variation. In addition, the size of control information can be different case by case.  This requires having the possibility of sending control information with variable sizes. In addition, the reader needs to inform the device regarding the control information contents for each message. These could be achieved by using an indication field in the control field, indicating the present control contents in the message.

Proposal 2: The control information for R2D should be carried alone or along with the data information. The control information would include system information, synchronization, configuration of contention-based channel access, the indication of R2D data transmissions, and D2R resource allocations.

Proposal 3: The variable R2D control information size should be supported.

Proposal 4: Control information should carry an indication field indicating the present control contents and size.

# Conclusions

In this contribution, we provided our views on physical channels and signaling for A-IoT. We made following proposals:

Proposal 1: The control information containing target device ID(s) should be detectable by the device without receiving the rest of the message in PRDCH.

Proposal 2: The control information for R2D should be carried alone or along with the data information. The control information would include system information, synchronization, configuration of contention-based channel access, the indication of R2D data transmissions, and D2R resource allocations.

Proposal 3: The variable R2D control information size should be supported.

Proposal 4: Control information should carry an indication field indicating the present control contents and size.

# Reference

[1] 	TR 38.848, “Study on Ambient IoT (Internet of Things) in RAN,” V18.0.0.

[2]	TR 38.769, “Study on solutions for ambient IoT (Internet of Things) (Release 19),” V2.0.1.

[3]	RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR.”