3GPP TSG RAN WG1 #120		R1-2500547

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.1.3.2

Source:	Google

Title:	ML based CSI prediction

Document for:	Discussion/Decision

# Introduction

In RP-242399, the following objective has been defined for AI/ML based CSI prediction.

- CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025

In RAN1 #118b, the following on consistency of training/inference is agreed.

Agreement For consistency between training and inference, study to identify which potential NW-side additional conditions, if any, may impact on UE assumption for CSI prediction using UE-sided model, resulting non-negligible degradation on model generalization performance. Note: Companies are encouraged to provide generalization performance evaluation on the impact of NW-side additional conditions, if considered. Agreement For generalization evaluation to identify potential NW-side additional conditions to ensure consistency between training and inference, consider one or more of the following aspects, if evaluated: - [Optional evaluation] Various tilt angle (e.g., [102] degrees, [110] degrees) - [Optional evaluation] Various TXRU mapping - With TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (8x8x2) and (12 x 8 x 2) antenna elements) - Without TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (2x8x2) and (4 x 4 x 2) antenna elements, respectively) - Other configurations are not precluded Note: for other evaluation assumptions, reuse baseline of TR 38.843 Note: report how to map antenna port to antenna elements Note: report the backbone/structure of AI/ML model used Agreement The TP1, TP2 and TP3 to TR38.843 in Section 3 of R1-2409236 are endorsed in principle.

In this contribution, we provide some discussion on AI/ML based CSI prediction.

# Consistency of training and inference

The inference configuration procedure for AI/ML based BM has been agreed in RAN1 #119, where two options (Option A and Option B) as in Figure 1 are supported. Similar to AI/ML based BM, for AI/ML based CSI prediction, the alignment between the NW and UE on the supported functionality and applicable functionality for AI/ML based CSI prediction should be necessary. Then one possible way is to reuse the inference configuration procedure for AI/ML based BM for AI/ML based CSI prediction.

Figure 1: Procedure for the inference configuration procedure for AI/ML based BM

Proposal 1: Support to reuse the inference configuration procedure for AI/ML based BM for AI/ML based CSI prediction to maintain the consistency of training and inference with regard to the supported/applicable functionalities for AI/ML based CSI prediction.

# Performance monitoring

## Framework for performance monitoring

The performance monitoring for AI/ML based BM has been agreed to be based on linked CSI report configurations, where the NW can configure one CSI report configuration for inference and the other CSI report configuration for monitoring, which is linked with the CSI report configuration for inference. Then the UE can measure the ground truth results based on the CSI report configuration for monitoring and perform the inference based on the CSI report configuration for inference. It can compare the ground truth results and inference results to check whether the AI/ML based CSI prediction failure happens or not. The AI/ML based CSI prediction is similar to AI/ML based BM case 2 (temporal beam prediction), therefore a unified solution could help to simplify the UE implementations. Thus, the framework for the performance monitoring for AI/ML based CSI prediction can be the same as that for AI/ML based BM.

Proposal 2: Support to reuse the framework for performance monitoring configuration for AI/ML based BM for AI/ML based CSI prediction, i.e.,

- Support the NW configures one CSI report configuration for the inference of the AI/ML based CSI prediction, and configure another CSI report configuration for the monitoring of the AI/ML based CSI prediction which is linked to the CSI report configuration for the inference

- The UE calculates the performance monitoring metric based on the inference results from the CSI report configuration for the inference and the ground-truth results from the CSI report configuration for the monitoring

## Performance metric measurement

For AI/ML based CSI prediction, one possible way is to compare the CQI based on the predicted CSI and the CQI based on ground-truth CSI. If the offset between the CQIs is small, the predicted CSI can be assumed to be accurate enough. The UE can report the CQI offset. Then the NW can decide whether to activate or deactivate the CSI report configuration for AI/ML based CSI prediction.

For the inference of CSI prediction, the NW can configure all types of CSI-RS, e.g., periodic/semi-persistent/aperiodic CSI-RS. For the monitoring of the CSI prediction, the supported type of CSI-RS may depend on the report mechanism, e.g., NW triggered or event triggered. For NW triggered report, all types of CSI-RS can be supported. For event triggered, since UE usually needs to detect the event periodically, the CSI-RS for both inference and monitoring should be periodic CSI-RS. Further, since the measurement window and prediction window could be different. Separate CSI-RS for inference and monitoring should be considered.

Proposal 3: Support the CQI offset between the CQI for the predicted CSI and the CQI for the ground-truth CSI as the metric for performance monitoring.

Proposal 4: Support the NW configures separate CSI-RS resource sets for inference and monitoring

- The CSI-RS can be periodic, semi-persistent or aperiodic

## Report mechanism

The monitoring results can be reported in either NW triggered manner or event triggered manner. Compared to the event triggered mechanism, the NW triggered could be simpler. As both can achieve the same functionality, the NW triggered report should be prioritized.

Further, the latency for the performance monitoring results report is as important as the CSI report, since large latency of the report could result in incorrect activation/deactivation of CSI prediction so as to reduce the performance. Since the CSI report is usually carried by the UCI, the performance monitoring results can also be carried by UCI.

Proposal 5: Support the NW-triggered performance monitoring results report

- Event-triggered performance results report should be deprioritized

Proposal 6: Support to report the performance monitoring results by UCI

# Conclusion

In this contribution, we provided discussion on AI/ML based CSI prediction. Based on the discussion, the following proposals are provided.

Proposal 1: Support to reuse the inference configuration procedure for AI/ML based BM for AI/ML based CSI prediction to maintain the consistency of training and inference with regard to the supported/applicable functionalities for AI/ML based CSI prediction.

Proposal 2: Support to reuse the framework for performance monitoring configuration for AI/ML based BM for AI/ML based CSI prediction, i.e.,

- Support the NW configures one CSI report configuration for the inference of the AI/ML based CSI prediction, and configure another CSI report configuration for the monitoring of the AI/ML based CSI prediction which is linked to the CSI report configuration for the inference

- The UE calculates the performance monitoring metric based on the inference results from the CSI report configuration for the inference and the ground-truth results from the CSI report configuration for the monitoring

Proposal 3: Support the CQI offset between the CQI for the predicted CSI and the CQI for the ground-truth CSI as the metric for performance monitoring.

Proposal 4: Support the NW configures separate CSI-RS resource sets for inference and monitoring

- The CSI-RS can be periodic, semi-persistent or aperiodic

Proposal 5: Support the NW-triggered performance monitoring results report

- Event-triggered performance results report should be deprioritized

Proposal 6: Support to report the performance monitoring results by UCI