3GPP TSG RAN WG1 Meeting #120                                                                                 R1 -2500558

Athens, Greece, February 17th – 21st, 2025

Source:                 TCL

Title:                     Discussion on AI/ML CSI compression

Agenda Item:       9.1.4.1

Document For:    Discussion

- Introduction

CSI compression is a sub-use case of AI/ML for CSI feedback enhancement. Its architecture comprises a two-sided AI/ML model, featuring an encoder implemented on the UE side and a decoder on the NW side. In Release 19, CSI compression will persist as a study item, aiming to optimize the balance between performance and complexity/overhead. Additionally, it seeks to address and resolve challenges associated with inter-vendor training collaboration, in line with the objectives of the new WID on AI/ML for the NR air interface proposed in RAN#106 [1].

The following areas will be further studied as part of this project: - CSI feedback enhancement [RAN1]: - For CSI temporal prediction: further update TR 38.843 with additional evaluations. - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing necessary specification impact analysis, as well as, other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843. - Necessity and details of model Identification concept and procedure in the context of LCM for two-sided models [RAN2/RAN1] - CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]: - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods - Model transfer/delivery [RAN2/RAN1]: - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study

Although the AI/ML CSI compression exhibits significant improvement on overhead and throughput, there are several remaining issues that includes training collaboration types, two-sided model pairing mechanisms, performance monitoring procedures, intermediate KPIs calculation, CSI quantization, and resource/report configuration for model inference, etc. This contribution aims to addresses some of the open issues. Additionally, we provide our perspectives on the conclusions of the study item concerning the AI/ML-based CSI compression sub-use case from the RAN1 viewpoint.

- Study on the Specification Impact on CSI Compression Model Inference

The execution flow for the CSI compression based on two-sided model is shown in Figure 1.

Figure 1 CSI compression procedures

In the figure, w represents the measured CSI information; w represents the recovered CSI information, which is the output of the decoderdecoder. Therefore, the degree of similarity between w and w determines the system performance of AI/ML-based CSI compression. In conventional CSI compression, w is obtained from direct CSI measurement. However, for overhead reduction purpose we may further utilize the time domain correlation of CSI to improve the CSI compression efficiency.

2.1 Temporal domain aspects of CSI compression

The Rel-19 focuses on the temporal domain aspects of AI/ML-based CSI compression, which is captured as follows. Among Cases 1 to 5, Case 2 and Case 3 are prioritized for further discussion.

Agreement For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following categorization for study: Case; Target CSI slot(s); Whether the UE uses past CSI information; Whether the network uses past CSI information. 0; Present slot; No; No. 1; Present slot; Yes; No. 2; Present slot; Yes; Yes. 3; Future slot(s); Yes; No. 4; Future slot(s); Yes; Yes. 5; Present slot; No; Yes. Note 1: For the UE, the past CSI information may include past model inputs and/or any information derived from them. For the network, the past CSI information may include past CSI feedback instances and/or any information derived from them. Note 2: For case 3 and case 4, the UE may perform prediction as a separate step or jointly with compression. Similarly, the network may perform prediction as a separate step or jointly with reconstruction. Companies to report which option is selected, the number of future slots, and whether the prediction is AI/ML-based or not. Note 3: “Target CSI slot(s)” refers to the slot(s) to which the CSI feedback in the report corresponds. “Present slot” refers to the slot of the most recent CSI-RS measurement used to generate the CSI report. “Future slot(s)” includes at least one slot after the present slot and may include the present slot as well. Note 4: Down-selection is not precluded.

2.1.1 Specification Impact on Case 2

Temporal domain compression Case 2 is a CSI compression scheme based on the time domain correlation of CSI. Assuming that the current moment is noted as t-1, the historical measurement CSI information at the moment t-1 is cached at the UE side, and then the measured measurement CSI information measured at the moment t is differentiated from the above cached historical measurement CSI information, and the CSI difference information is obtained to be input into the AI/ML model of the CSI generation part at the UE side for compression, and the resulting data of the compressed CSI difference information for CSI UL uploading.

On the gNB side, the cached historical CSI information is obtained by adding the historical restored CSI information at the moment of t-2 with the restored CSI difference information on the gNB side at the moment of t-1, and the said restored CSI difference information is obtained by adding the compressed CSI difference information received by the gNB by the CSI reconstruction part of the AI/ML model recovery is obtained. The resulting recovery CSI differential information at moment t-1 is added with the historical recovery CSI information to obtain the recovery CSI information at moment t.

The aforementioned operation could be performed slot by slot and it will significantly lower the signaling overhead in the long run. On the other hand, the quantization and compression error may accumulate over time, therefore the prediction accuracy is degraded.

Observation 1: The temporal domain compression Case 2 will improve the CSI compression efficiency.

Proposal 1: RAN1 should study the benefits and potential issues if temporal domain compression Case 2 is adopted, especially in the following aspects:

- The overall performance gain considering both the accuracy loss and the overhead reduction of the temporal domain compression Case 2.

- The performance monitoring metrics of the temporal domain compression Case 2.

- The potential problem of error accumulation and its solutions.

2.1.2 Specification Impact on Case 3

Temporal domain compression Case 3 is CSI compression plus prediction which can be categorized as separate prediction and compression (SPC) and joint prediction and compression (JPC). SPC includes CSI prediction module and CSI compression module, where the CSI prediction module generates the predicted CSIs using the historical CSI measurements and sends the predicted CSIs to the CSI compression module for CSI compression. JPC integrates the CSI prediction module and CSI compression module so that the compressed predicted CSI can be directly generated by inputting the historical CSI measurements.

The following consensus was reached regarding the training data collection and monitoring for temporal domain compression Case 3 [2]:

Agreement For temporal domain aspects Case 3, study LCM aspects and specification impacts, consider the following options for training data collection - Option 1: The target CSI for training is derived based on the predicted CSI of the future slot(s). - Option 2: The target CSI for training is derived based on the measured CSI of the future slot(s). - Note: During inference, the input to the CSI generation part is derived based on the predicted CSI. consider following options for the monitoring labels - Option 1: The monitoring label is derived based on the predicted CSI of the future slot(s). - CSI prediction output is used as input to CSI generation part. - Note: This corresponds to monitoring of CSI compression only. CSI prediction may be monitored separately. - Option 2: The monitoring label is derived based on the measured CSI of the future slot(s) - Option 2a: CSI prediction output is used as input to CSI generation part. - Note: This corresponds to end-to-end monitoring of CSI prediction and compression. - Option 2b: Measured CSI of the future slot(s) is used as input to CSI generation part for monitoring purpose. - Note: This corresponds to monitoring of CSI compression only. CSI prediction may be monitored separately. Study how the functionality/model control (activation, deactivation, switching, and fallback) for CSI prediction and CSI compression interacts.

The monitoring options for CSI compression is discussed. It is generally accepted to further study the necessity and feasibility of NW-side monitoring based on the target CSI reported by UE and UE-side monitoring based on precoded RS (e.g., CSI-RS, DMRS). As for the monitoring of temporal domain compression Case 3, the performance of CSI prediction module also needs to be considered when SPC is adopted, which introduces monitoring data related to CSI prediction. Hence, the specification impact on the monitoring data report configuration needs to be further discussed. One issue is that whether NW or UE monitors the performance of CSI prediction. The report quantity is contingent on the monitoring entity. For example, the predicted CSI should be reported to NW if NW is the CSI prediction monitoring entity. If UE is the CSI prediction monitoring entity, the monitoring metrics or outcome should be reported to NW for functionality/model management. The other issue is whether the monitoring data, i.e., compressed CSI, measured CSI of the future slot(s), predicted CSI of the future slot(s), CSI prediction monitoring metrics/outcome, is reported together or separately. If the monitoring data is reported separately, the indication of data type for scheduled physical resources needs to be considered. In addition, the measured CSI and predicted CSI are reported using the PMI format, which influences the monitoring decision performance. The CSI quality should be introduced to improve the monitoring decision making.

Proposal 2: RAN1 should further study the following specification impact on the monitoring of temporal domain compression Case 3:

- the monitoring data report configuration, e.g., new report quantity, indication of data type.

- the format of CSI quality

2.1 Resource Set (RS) and CSI Report Configuration

The following consensus was reached regarding the CSI-RS resource and CSI report configuration for the CSI compression sub-use case [3][4]:

Agreement in RAN1#112bis-e The study of AI/ML based CSI compression should be based on the legacy CSI feedback signaling framework. Further study potential specification enhancement on - CSI-RS configurations (No discussion on CSI-RS pattern design enhancements) - CSI reporting configurations - CSI report UCI mapping/priority/omission - CSI processing procedures. - Other aspects are not precluded. Agreement in RAN1#113 In CSI compression using two-sided model use case, further study the applicability and potential specification impact for CSI configuration and report: - For network to indicate CSI reporting related information, gNB can indicate the UE with the one or more of following information: - Information indicating CSI payload size - Information indicating quantization method/granularity. - Rank restriction - Other payload related aspects - For UE determination/reporting of the actual CSI payload size, UE reports related information as configured by the NW

The CSI framework, as established in NR, encompasses the setting of CSI-RS resources and report settings, as well as the configuration of the UE with CSI-ReportConfig via RRC signaling. In the use case of CSI compression using a two-sided model, three types of CSI-RS resources associated with the reportQuantity are defined for CSI measurement. These include channel measurement with the first CSI-RS resource configuration, interference measurement with the second CSI-IM resource configuration, and another form of interference executed by the NZP-CSI-RS resource configuration. The information required by the AI/ML model would significantly differ from the traditional CSI framework. Moreover, the conventional CSI-RS resource configuration methodology has not defined the AI or other usage scenarios. Therefore, the method of pairing the AI/ML model with the reference signal configuration needs to be elucidated.

To cater to the specific feedback requirements of the AI/ML model, several methods can be employed to differentiate the AI/ML CSI compression requirement:

- A dedicated reporting quantity has been defined, which is utilized for the operation of the AI/ML-specific model.

- A dedicated reporting configuration is in place, which incorporates information specific to the AI/ML model.

- A dedicated measurement configuration is established, which includes information specific to the AI/ML model.

In prior meetings, several proposals were put forth suggesting that the UE employs multiple AI/ML models to compress the CSI information. For each AI/ML model used for CSI compression, the corresponding CSI-RS resource configuration and CSI reporting configuration would be clarified to align the UE with LCM operations.

Proposal 3: In the case of CSI compression using a two-sided model, the design of an AI/ML-specific CSI-RS resource and CSI reporting configuration that may be compatible with the traditional CSI reporting scenario should be considered in the following aspects:

- AI/ML-specific CSI-RS resource configuration for CSI compression

- AI/ML-specific fields in CSI-ReportConfig IE

- Dedicated report quantities and report configurations for AI

2.2 Priority rules concerning CSI report collision

The following agreements were reached on the priority rules pertaining to CSI report collision for the CSI compression sub-use case:

Agreement In CSI compression using two-sided model use case, further study the feasibility and methods to support the legacy CSI reporting principles including at least: - The priority rule regarding CSI collision handling and CSI omission - Codebook subset restriction - CSI processing Unit Agreement The study of AI/ML based CSI compression should be based on the legacy CSI feedback signaling framework. Further study potential specification enhancement on - CSI-RS configurations (No discussion on CSI-RS pattern design enhancements) - CSI reporting configurations - CSI report UCI mapping/priority/omission - CSI processing procedures. - Other aspects are not precluded.

At times, a UE device may be triggered with multiple CSI reports based on the CSI report configurations. In such instances, it becomes necessary for the UE device to ascertain the priority of these multiple CSI reports. This is to determine whether some CSI reports or portions of the CSI report part 2 should be disregarded when the uplink symbols/slots for multiple CSI reports collide in the temporal domain. In other words, when the UE device is triggered with multiple AI/ML-based CSI reports and non-AI/ML CSI reports, the priority for such AI/ML-based and non-AI/ML-based CSI reports also needs to be established.

In the NR system, a priority rule for CSI reporting has been established, which is associated with multiple factors such as temporal domain behavior (periodic/semi-persistent/aperiodic), CSI report content (whether or not carrying L1-RSRP or L1-SINR), serving cell index, reporting config ID, and so on. Two CSI reports are considered to be in collision if the time occupancy of the physical channels scheduled to carry CSI reports overlaps in at least one OFDM symbol and is transmitted on the same carrier. When a UE is configured to transmit two colliding CSI reports to the gNB, the priority rule will be applied as follows:

- If the y values differ between the two colliding CSI reports, the following rules apply, except in the case where one of the y values is 2 and the other is 3. Here, the parameter y denotes different types of CSI reports, such as y=0 indicating aperiodic CSI reports to be carried on PUSCH, y=1 indicating semi-persistent CSI reports to be carried on PUSCH, y=2 indicating semi-persistent CSI reports to be carried on PUCCH, and y=3 indicating periodic CSI reports to be carried on PUCCH.

- The CSI report with the higher priority value should not be transmitted by the UE.

- Alternatively, the two CSI reports are either multiplexed or one is dropped, depending on the priority values, as outlined in [TS38.213].

At present, AI/ML-based CSI compression has been deliberated multiple times in RAN1/RAN2. For AI/ML models to function properly, dedicated CSI reporting, including AI/ML model life cycle management (LCM), is required. Both non-AI/ML based CSI compression and AI/ML-based CSI compression or related data collection would coexist for a specific radio link. Given the constraints on UCI payload, a new priority rule might be necessary to manage AI/ML and/or non-AI/ML based CSI compression. Consequently, an amendment to the existing CSI reporting priority rule is anticipated.

The establishment of an AI/ML-specific priority rule is essential within the system, distinct from the non-AI/ML specific CSI reporting priority rule currently employed in the NR system. In addition to the factors influencing the value of legacy CSI reporting priority, certain supplementary information may be taken into account as determinants for the value of AI/ML-specific CSI reporting priority. These factors include:

- AI/ML LCM procedures, signifying that CSI reporting associated with different procedures possess varying priority values, such as model monitoring, inference, and training.

- AI/ML characteristics for CSI compression, indicating that CSI reporting associated with different features carries different priority values, such as the model structure for CSI compression and quantization.

- AI/ML CSI reporting contents, suggesting that different CSI reporting information holds varying priority values, such as Rank and CQI, and the codebook subset.

Observation 2: Certain additional information may impact the priority of AI/ML CSI reporting for CSI compression:

- AI/ML LCM procedure

- AI/ML characteristics for CSI compression

- AI/ML reporting contents

- Other factors that may be pertinent to the value of AI/ML-specific priority should not be precluded.

Proposal 4: The definition of AI/ML-specific priority for CSI reporting in relation to CSI compression should be considered in comparison with the traditional CSI priority rules.

Given that both AI/ML CSI reporting and non-AI/ML CSI reporting for CSI compression are likely to coexist in the NR system, when an uplink resource collision occurs, the final priority between AI/ML-based and non-AI/ML-based should be determined. The approach to this determination is as follows:

- Option 1: The priority of AI/ML CSI is always higher than that of non-AI/ML CSI reporting for CSI compression. This implies that non-AI/ML can only be transmitted after the AI/ML CSI, and the CSI priority is determined by the specific priority value within either AI/ML CSI reporting or non-AI/ML CSI reporting.

- Option 2: The priority of AI/ML CSI is always lower than that of non-AI/ML CSI reporting for CSI compression. This implies that AI/ML can only be transmitted after the non-AI/ML CSI, and the CSI priority is determined by the specific priority value within either AI/ML CSI reporting or non-AI/ML CSI reporting.

- Option 3: If both AI/ML CSI and non-AI/ML CSI share the same priority value, the AI/ML CSI will be transmitted first. However, if the priority values differ, the transmission priority is determined by the respective priority value.

- Option 4: If both AI/ML CSI and non-AI/ML CSI share the same priority value, the non-AI/ML CSI will be transmitted first. However, if the priority values differ, the transmission priority is determined by the respective priority value.

Proposal 5: When the UE supports both AI/ML and non-AI/ML CSI reporting, it is necessary to redefine the priority rule considering different types of CSI reporting.

2.3 Mechanism for CSI Compression Processing with Limited UE Capability and Dedicated Timing

For traditional CSI measurement processing criteria, the concept of a CSI Processing Unit (CPU) is introduced to denote the capability of supported simultaneous CSI calculations implemented in a UE. This means that if a UE supports simultaneous CSI calculations, it is said to possess CSI processing units for processing CSI reports. Similarly, for AI/ML models implementing CSI measurement operations, the concept of an AI/ML-specific CPU can also be introduced. However, the requirements for an AI/ML-specific CPU differ significantly from those of a legacy CPU due to the distinct requirements of UE capabilities to implement AI/ML-specific and traditional CSI measurement operations. When AI/ML models are applied to the CSI measurement process for CSI prediction, channel information feedback, beam management, and positioning, the resources (including computing units, memory, storage, and other hardware resources) consumed by AI/ML models differ from those required by traditional CSI computation methods. Consequently, the current CPU and Occupied CPU quantified based on non-AI/ML models are unsuitable for AI/ML models. It is essential to redefine the relevant parameters specifically for AI/ML models.

The utilization of AI/ML models for CSI measurements and calculations may pose challenges to the capabilities of a UE to implement AI/ML models for inference. Firstly, the size of AI/ML models may be substantial due to the complex AI/ML model structures and a large number of AI/ML proprietary parameters, including hyperparameters, learnable parameters, and other types of parameters specific to an AI/ML model. Consequently, to facilitate the running of AI/ML models for inference in a UE, a significant amount of the UE's memory resources must be occupied, which may surpass the available memory resources of the UE specific to AI/ML model operation. Secondly, the computation requirements (e.g., FLOPs) of AI/ML models may be too high for a UE to support. This implies that running AI/ML models for inference in a UE may consume computational resources to a large extent. Lastly, running AI/ML models for inference in a UE may lead to substantial power consumption, which may exceed the tolerable limit of the UE's power-saving requirements.

Observation 3: The memory and computational resources as well as the power consumption may become bottlenecks for UE when implementing AI/ML models for CSI compression and restoration.

Proposal 6: The study of how to describe the capabilities of a UE to implement AI/ML models for inference on CSI compression and calculations should be undertaken.

Upon receipt of the CSI request, which instigates CSI measurements and calculations in the UE via DCI transmitted through PDCCH, the UE commences these measurements and calculations. It then conveys the CSI measurement results to the gNB via the PUSCH/PUCCH, the time slot for which has been predetermined. To ensure the most recent CSI measurement data is reported to the gNB, the CSI measurements and calculations must be completed prior to the first uplink symbol that carries the corresponding CSI report(s), taking into account the CSI computation time.

In the case of CSI measurements and calculations that employ AI/ML models for inference, the estimation of CSI computation time necessitates a reconfiguration for AI/ML. This differs from traditional CSI measurement and calculation operations, which can be executed immediately without any preliminary steps. AI/ML models, on the other hand, require activation within the UE before the implementation of inference, a process that may consume a certain amount of time. Moreover, the size of the AI/ML models may affect the model inference time.

The total operation time for the AI/ML model in CSI computation comprises two durations: the time taken for model activation (if applicable) and the time taken for model inference. This could result in different outcomes for the handling of CSI measurements and calculations (either providing a valid CSI report or disregarding the scheduling request) compared to the time duration from the last symbol of the DCI carrying the CSI request to the first uplink symbol carrying the corresponding CSI report(s).

Observation 4: The computational time for CSI restoration at NW side should be estimated and reconfigured to the UE. This may include the time duration for both model activation and model inference.

2.4 Considerations on inter-vendor training collaboration issues

The following two agreements were reached during RAN1 #116 and #116bis meetings respectively:

Agreement To alleviate / resolve the issues related to inter-vendor training collaboration of AI/ML-based CSI compression using two-sided model, study the following options: - Option 1: Fully standardized reference model (structure + parameters) - Option 2: Standardized dataset - Option 3: Standardized reference model structure + Parameter exchange between NW-side and UE-side - Option 4: Standardized data / dataset format + Dataset exchange between NW-side and UE-side - Option 5: Standardized model format + Reference model exchange between NW-side and UE-side Agreement - For Option 3, further define the two sub-options: - 3a: Parameters received at the UE or UE-side goes through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., potential re-training, re-development of a different model, and/or offline testing. - 3b: Parameters received at the UE are directly used for inference at the UE without offline engineering, potentially with on-device operations. - For Option 5, further define the two sub-options: - 5a: Model received at the UE or UE-side goes through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., potential re-training, re-development of a different model, and/or offline testing. - 5b: Model received at the UE are directly used for inference at the UE without offline engineering, potentially with on-device operations. - For Option 4, it is clarified that: - Dataset received at the UE or UE-side goes through offline engineering at the UE- side (e.g., UE-side OTT server), e.g., model training or offline testing.

Considering the information exchange between the NW and the UE, it should be coordinated by the NW while the UE provides complementary information under the guidance of the NW. The detailed information exchange procedure depends on the options and model monitoring method.

In general, we do not support option 1 and option 2 and think option 3, 4 and 5 are more practical choices for standardization for the following reasons.

Option 1 requires the exchange of fully characterized model, i.e., model structure and parameters, this requires unacceptable overhead for the model exchange. Since the model parameters scenario/condition dependent, even a small change in the scenario/condition can bring about a complete change in the model parameters, which results in significant information exchange overhead in a time varying channel.

Option 2 follow the usual practice in the field of image recognition, where a series of datasets are clearly named and stored in a database, and the AI/ML models for different use cases are trained by choosing the suitable datasets from the database. This approach is good for research purpose, as the model training is done with recognized datasets, then the results is reliable and ease for comparison. However, as the variation features of the channel CSI is more complex than the image does, it is hard to generalize a common database covers all scenarios in the wireless communications, consequently it is difficult to reach consensus in a short period of time.

Proposal 7: Option 1 and 2 should not be standardized, at least they are out of the scope of R19.

Option 3 and option 5 is similar, the model format or structure specifies the skeleton of the AI/ML model, e.g., number of layers, number of nodes, etc., which does not vary across time. And the scenario/condition varying part, namely the parameters or the reference model will be generated in real time and to be exchanges between the NW and UE.

For option 4, the data format refers to the formant of the dataset, that does not involve the content of the dataset. For example, it many just specify which features or parameters are contained within the dataset, and consequently the data type and structure of the features or parameters. This will not bring too much burden for the standardization work.

Considering the two-sided model, if the model format or structure is similar between NW and UE, then there is merely no difference between the options 3, 4 and 5, except that for parameter or reference model exchange the training burden lies on the NW side while for dataset exchange the training burden lies on the UE side.

However, considering the gap of hardware capability between the NW and UE, if no UE-side offline engineering, the UE may suitable for much simpler model format or structure due to the hardware limitation, therefore directly use the same model structure or formant as the NW side may result in problems. In this case the exchange of dataset, i.e., option 4, seems more applicable as the UE is free to choose the suitable model. In contrast, if the hardware capability at the UE side could be kept at similar level as the NW side aided by the OTT server, then the option 3a or option 5a are also applicable.

Proposal 8: RAN 1 should down select among option 3, 4 and 5 considering if unified model format or structure is shared between the NW and UE side model respectively.

- For option 4, there may be no need for offline-engineering.

- Option 3 and 5 are preferred if model structure or format is exchanged from NW to UE.

2.5 Overhead Reduction Issues in CSI Compression

The following two agreements were reached during RAN1 #116 and #116bis meetings respectively:

Agreement Further study following monitoring options in Rel-19, including the necessity and feasibility, - NW-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the target CSI reported by the UE via legacy eT2 codebook or eT2-like high-resolution codebook (Case 1) - SRS-based monitoring - UE-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the output of the CSI reconstruction model at the UE (Case 2-1) - Note: CSI reconstruction model at the UE-side can be the same as the actual CSI reconstruction model used at the NW-side, a reference model provided by NW, or a proxy model developed by the UE side. - Via direct estimation of intermediate KPI (e.g., SGCS) without reconstructing a target CSI (Case 2-2) - Via estimation of monitoring output other than intermediate KPI without reconstructing a target CSI - Based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model - Based on the output of the CSI reconstruction model indicated by the NW via legacy eT2 codebook or eT2-like high-resolution codebook Regarding monitoring metrics: - Monitoring accuracy also includes generalization considerations, if applicable. - Complexity also includes LCM complexity, if applicable. - Monitoring overhead, latency, complexity, and accuracy analysis may have to consider using at N>1 CSI feedback occasions. - Testability of UE reported metrics Discussion may include the following aspects: - Consideration of Options 1-5 and their sub-options for alleviating / resolving the issues related to inter-vendor training collaboration - Temporal domain aspects of CSI compression - How the above monitoring approaches or combination of them may help identifying the cause (e.g., NW side, UE side, data drift) of the performance degradation Note: for UE-side monitoring, the final reported monitoring output, if specified, may be different, e.g., be further derived based on the output of the above approaches. Note: implementation-based monitoring solutions can be considered in assessing the necessity of the above monitoring approaches.

The model monitoring in AI/ML-based CSI compression relies on the computation of intermediate KPIs, which relies on obtaining the original CSI w and the recovered CSI w. However, since w is obtained at the UE side while w is obtained at the NW side, it is necessary to design the corresponding signaling and data transmission to enable the network element who performs the model monitoring obtains both w and w at the same time.

In order to enable the model monitoring, w or w needs to be transferred to the NW or UE for the NW-sided or UE-sided model monitoring, respectively. Both w and w may be high dimensional matrix with very huge data volume due to multiple antenna ports, polarized antennas and large bandwidth in a massive MIMO system. Therefore, transferring the raw CSI, i.e., uncompressed CSI w or w, for the model monitoring purpose may exceed the capacity of the traditional L1 or L3 layer signalling. Therefore, the protocol needs to build a common understanding on the quantization and compression of raw CSI which enables the exchange of the CSI for monitoring in a reduced overhead.

Even if the raw CSI is post processed to a smaller volume, its data size is still relatively large, usually reaching several hundred bits or thousands of bits. An alternative way instead of directly transmit the CSI is to transfer other low-dimensional parameters as a precoding matrix indicator to compute the intermediate KPIs, which may significantly reduce the overhead while guaranteeing the reliability of the model monitoring.

Observation 5: The CSI report or indication for model monitoring in AI/ML based CSI compression introduces considerable overhead, RAN 1 should strive for an efficient signaling and overhead reduction mechanism for the CSI transmission.

Proposal 9: Ran1 should consider whether/how to transmit the raw CSI for monitoring purpose.

- FFS: The overhead reduction scheme based on spectral or temporal processing.

Furthermore, transmitting the quantized and compressed CSI introduces quantization error, which results in the deviation of the calculated intermediate KPIs from the actual values, and makes the model monitoring results less reliable. Therefore, the deviation of the calculated model monitoring metrics caused by quantification errors needs to be considered to determine whether the results of model monitoring are reliable.

Observation 6: The reliability of the overhead reduction scheme for CSI transferring for model monitoring in AI/ML based CSI compression should be guaranteed.

- Conclusions

This contribution has led to the following observations:

Observation 1: The temporal domain compression Case 2 will improve the CSI compression efficiency.

Observation 2: Certain additional information may impact the priority of AI/ML CSI reporting for CSI compression:

- AI/ML LCM procedure

- AI/ML characteristics for CSI compression

- AI/ML reporting contents

- Other factors that may be pertinent to the value of AI/ML-specific priority should not be precluded.

Observation 3: The memory and computational resources as well as the power consumption may become bottlenecks for UE when implementing AI/ML models for CSI compression and restoration.

Observation 4: The computational time for CSI restoration at NW side should be estimated and reconfigured to the UE. This may include the time duration for both model activation and model inference.

Observation 5: The CSI transferring for model monitoring in AI/ML based CSI compression introduces considerable overhead, RAN 1 should strive for an efficient signaling and overhead reduction mechanism for the CSI transferring for model monitoring purpose.

Observation 6: The reliability of the overhead reduction scheme for CSI transferring for model monitoring in AI/ML based CSI compression should be guaranteed.

Furthermore, the following proposals have been put forward:

Proposal 1: RAN1 should study the benefits and potential issues if temporal domain compression Case 2 is adopted, especially in the following aspects:

- The overall performance gain considering both the accuracy loss and the overhead reduction of the temporal domain compression Case 2.

- The performance monitoring metrics of the temporal domain compression Case 2.

- The potential problem of error accumulation and its solutions.

Proposal 2: RAN1 should further study the following specification impact on the monitoring of temporal domain compression Case 3:

- the monitoring data report configuration, e.g., new report quantity, indication of data type.

- the format of CSI quality

Proposal 3: In the case of CSI compression using a two-sided model, the design of an AI/ML-specific CSI-RS resource and CSI reporting configuration that may be compatible with the traditional CSI reporting scenario should be considered in the following aspects:

- AI/ML-specific CSI-RS resource configuration for CSI compression

- AI/ML-specific fields in CSI-ReportConfig IE

- Dedicated report quantities and report configurations for AI

Proposal 4: The definition of AI/ML-specific priority for CSI reporting in relation to CSI compression should be considered in comparison with the traditional CSI priority rules

Proposal 5: When the UE supports both AI/ML and non-AI/ML CSI reporting, it is necessary to redefine the priority rule considering different types of CSI reporting.

Proposal 6: The study of how to describe the capabilities of a UE to implement AI/ML models for inference on CSI compression and calculations should be undertaken.

Proposal 7: Option 1 and 2 should not be standardized, at least they are out of the scope of R19.

Proposal 8: RAN 1 should down select among option 3, 4 and 5 considering if unified model format or structure is shared between the NW and UE side model respectively.

- For option 4, there may be no need for offline-engineering.

- Option 3 and 5 are preferred if model structure or format is exchanged from NW to UE.

Proposal 9: Ran1 should consider whether/how to transmit the raw CSI for monitoring purpose.

- FFS: The overhead reduction scheme based on spectral or temporal processing.

References

- RP-243245 New SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface Phase 2

- Chairman notes of 3GPP TSG RAN WG1 #119 meeting.

- Chairman notes of 3GPP TSG RAN WG1 #112b-e meeting.

- Chairman notes of 3GPP TSG RAN WG1 #112 meeting.