3GPP TSG RAN WG1 #120   	                              R1-2500567

Athens, Greece, February 17th – 21st, 2025

Agenda item:		9.1.4.1

Source:		LG Electronics

Title:			Study on CSI compression

Document for:	Discussion and Decision

Introduction

In RAN#105, the following was made to extend study from RAN1 perspective on AI/ML CSI feedback enhancement for NR air-interface in 2024 [1]:

Study objectives: - CSI feedback enhancement [RAN1]: - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing necessary specification impact analysis, as well as, other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843.

This contribution discusses on CSI compression especially for further improving trade-off between performance and complexity/overhead based on Rel-18 study on AI/ML CSI compression.

Discussion

Aspects on improving the performance for AI/ML CSI compression

In RAN1#118 meeting, the following agreements were made on temporal/spatial/frequency (TSF)-domain CSI compression to evaluate the performance and study the potential specification impacts.

Agreement For temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, among Cases 1, 2, 3, 4, and 5, prioritize further discussion on Case 2 and Case 3. Agreement For studying the standardization of model structure RAN1, RAN1 assumes at least the following: - Precoding matrix as an input (as opposed to raw channel matrix) - Per-layer processing, with common structure across ranks and layers (corresponding to Option 2-1, 2-2, 3-1, or 3-2 for handling rank ≥ 1) - For temporal domain aspects Case 2 and Case 3, strive to reuse the model structure of Case 0 where appropriate, with additional layers or operations either at the input/output domain or at the latent domain. - For Case 0, use precoding matrix, e.g., eigen vector in spatial-frequency domain, and/or angular, delay domain representation such as eType-II W2. - For Case 2 and Case 3, use precoding matrix, e.g., spatial-frequency domain for each CSI observation instance, and/or angular, delay, and Doppler domain representation such as eType-II W2. - Consider scalability approaches over numbers of Tx ports, CSI feedback payload sizes, and bandwidths. Agreement For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, for Case 2, study the performance impact resulting from non-ideal UCI feedback. - Scenario A: no UCI loss - Note: Corresponds to an upper bound or re-aligning missing historical CSI information - Scenario B: UCI loss, known at NW and unknown at UE, with mitigation at NW - Note: Corresponds to implementation-based mitigation at NW but no signaling to UE. - Scenario C: UCI loss, known at NW and UE, with mitigation at NW and UE - Note: Corresponds to reset of historical CSI information at both UE and NW or any other mitigation approach enabled by signaling. - UCI loss modeling - 10% UCI loss probability on all UCI reports - Other options are not precluded, e.g., No UCI loss for the first UCI report of each observation window, and 10% UCI loss probability for the subsequent reports of each observation window, as a shortcut to simplify the evaluation work. - Other values for UCI loss probability are not precluded. - Note: The same UCI loss modeling shall be applied to the benchmark for fair comparison. - FFS: partial PMI-related UCI loss Agreement Adopt all the observations in Section 2 of R1-2407499. Agreement According to the Rel-19 study objective, - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843.. RAN1 is recommending extending the study of the AI/ML CSI compression based on RAN1 understanding the study on CSI compression is not completed.

Regarding TSF-domain CSI compression, AI/ML CSI model considered the design on CSI generation/reconstruction part by taking into account channel information in spatial/frequency domain which is similar to eType-II CSI codebook.

Figure 1. An example of AI/ML based TSF-domain CSI compression procedure

To improve the CSI compression efficiency further, as depicted in the above Figure 1, including CSI in temporal domain, e.g., accumulated past CSI, in addition to spatial/frequency domain can be considered in Rel-19 AI/ML CSI compression since this kind of past channel information is also correlated with current spatial/frequency domain channel. It means that the channel characteristics can be better reflected by utilizing the past channel information as an input to the AI/ML encoder/decoder. Also, this approach can reduce the feedback payload compared to spatial/frequency domain CSI compression when the target channel information is expressed by the differential of accumulated past channel information.

[Methods to manage the similarity/synchronization on past CSI at UE-side and/or NW-side]

Based on the above, it is noted that the performance of TSF-domain CSI compression is improved when the amount of similarity/synchronization of accumulated past CSI at UE-side and/or NW-side becomes high, especially for Case 2 and Case 4 in the above agreement. Therefore, it is necessary to manage the accumulated CSI at UE-side and NW-side with the high similarity/synchronization from a perspective of past channel information itself and/or past channel characteristics derived from the past CSIs. As a potential approach, it is possible to compose CSI feedback not only for the CSI of present slot but also for the CSI(s) on past instance(s) to check the similarity between NW-side decoded past CSI(s) and UE-side reported past CSI(s) in this method. In this case, to reduce the signaling overhead with including past CSI(s), the reporting granularity of past CSI(s) is relatively small compared to that of CSI for present slot and/or this kind of reporting can be operated with relatively larger periodicity. Alternatively, it is also possible to deliver the derived past channel characteristics from UE-side to NW-side, or vice-versa to compare the past channel characteristics for each side. By considering the difference, UE-side and/or NW-side can compensate the past channel characteristics and utilize it for CSI encoding/decoding in an efficient manner.

Proposal #1: Regarding temporal/spatial/frequency (TSF)-domain CSI compression, study methods/mechanisms to manage the similarity/synchronization of accumulated past CSI at UE-side and/or NW-side.

Also, it needs to be discussed what kind of information to represent past channel information. For example, accumulated past channel information and/or multiple past CSI and/or channel coherence time, etc., can be used to represent past channel information. For the purpose of performance monitoring, what kind of type of past CSI including CSI granularity (e.g., WB CSI or SB CSI) and CSI type (e.g., raw channel or precoder type) can be used also can be discussed.

Proposal #2: Regarding TSF-domain CSI compression, discuss the format of past CSI information and how to report it at least for performance monitoring perspective.

[CSI compression and prediction]

Regarding temporal domain aspects of AI/ML-based CSI compression, Case 3 and 4 consider target CSI slot(s) for both present slot and future slot(s) altogether as agreed in RAN1#116. To this end, UE-/NW-side may perform prediction as a separate step or jointly with compression. In RAN1#117, the following agreement was captured for studying LCM aspects on Case 3 and 4. Additionally, in RAN1#119, the detailed aspects of LCM and data collection for training were captured as follows.

@ RAN1#117 Agreement For temporal domain aspects Case 3 and 4, study the impact on LCM aspects of separate prediction and compression, and joint prediction and compression. @ RAN1#119 Agreement For temporal domain aspects Case 3, study LCM aspects and specification impacts, consider the following options for training data collection - Option 1: The target CSI for training is derived based on the predicted CSI of the future slot(s). - Option 2: The target CSI for training is derived based on the measured CSI of the future slot(s). - Note: During inference, the input to the CSI generation part is derived based on the predicted CSI. consider following options for the monitoring labels - Option 1: The monitoring label is derived based on the predicted CSI of the future slot(s). - CSI prediction output is used as input to CSI generation part. - Note: This corresponds to monitoring of CSI compression only. CSI prediction may be monitored separately. - Option 2: The monitoring label is derived based on the measured CSI of the future slot(s) - Option 2a: CSI prediction output is used as input to CSI generation part. - Note: This corresponds to end-to-end monitoring of CSI prediction and compression. - Option 2b: Measured CSI of the future slot(s) is used as input to CSI generation part for monitoring purpose. - Note: This corresponds to monitoring of CSI compression only. CSI prediction may be monitored separately. Study how the functionality/model control (activation, deactivation, switching, and fallback) for CSI prediction and CSI compression interacts. Agreement For NW to collect data for training, study following spec impacts - Data format: codebook-based Rel-16 eType2 or Rel-18 eType2 for PMI prediction. - FFS if enhancement is needed - FFS number of samples in the report. - FFS whether channel or precoder is needed for temporal Cases 3 - Configuration of rank/layer, number of subbands - Mechanism for ground-truth reporting - FFS: Report additional information regarding the samples, e.g., data quality, FFS the definition of data quality and corresponding parameters. - FFS if enhancements in CSI-RS and SRS configuration is needed. - FFS: Report associated information that captures UE side additional condition - FFS: Configuration / reporting of temporal aspects for temporal Case 2 and Case 3, e.g., association between input and output CSI - FFS: details of CSI measurement For UE to collect data for training, study following spec impacts - NW configuration or UE request, e.g., RS configuration/transmission for data collection - Whether enhancements in CSI-RS configuration is needed. - Configuration of temporal aspects for temporal case 2/3, e.g., association between input and output CSI - FFS: Need of configuration of ID, and configuration of ID

To be specific, whether the CSI compression and prediction is performed jointly or separately can affect the impact on LCM such as data collection, training, monitoring, and model control and so on. To our understanding, if two separate AI/ML models for CSI compression and prediction, where each model is identified separately, are comprised to operate Case 3 and 4, it can be considered as the separate CSI compression and prediction. Meanwhile, if single AI/ML model is used for both CSI compression and prediction, it is referred to as the joint CSI compression and prediction.

Based on the above, there may be potential impact on LCM aspects especially for performance monitoring. In case of the separate CSI compression and prediction, it is possible to check whether the performance degradation is originated by whether the AI/ML model of CSI compression or that of CSI prediction by monitoring the performance of each AI/ML model. However, for AI/ML based joint CSI compression and prediction, there is an ambiguity to verify the main reason for performance degradation by monitoring the output related to the corresponding AI/ML model, since the single AI/ML model is operated with CSI compression and prediction at the same time. Hence, when the performance (e.g., intermediate performance and/or eventual performance) is less than a certain threshold, switching, fine-tuning or update of the AI/ML model is needed which may be accompanied with a large signaling/latency overhead to maintain the output performance on CSI compression and prediction even if when the CSI compression works well but only the CSI prediction is poor or vice versa. To handle this issue, it would be beneficial to operate the AI/ML model for compression/prediction only or jointly by adapting different CSI compression ratio and different size of prediction window with respect to the condition of past CSI and/or performance monitoring metric. Then, the adaptive method of CSI compression and prediction can be efficiently performed for various characteristics of the channel and the corresponding past CSI.

Proposal #3: Regarding TSF-domain CSI compression Case 3 and 4, consider performance monitoring method on joint CSI compression and prediction by adapting the operation on the AI/ML model between CSI compression and prediction.

In addition to the above by considering the root cause between CSI compression and CSI prediction, the model update can be performed through the adjustment in training/sample data quality. The quality of sample data plays a critical role to ensure model reliability and performance. Without an effective method to assess the data quality, the model may be trained with inaccurate or suboptimal samples, which leads to performance degradation. To this end, various quality metrics can be considered. For example, prediction confidence quantifies the deviation between predicted CSI and input CSI, providing how reliable the prediction model provides. Similarly, temporal consistency shows whether past and current CSI are in continuous pattern, which is crucial for maintaining stability in time-varying channel. Compression reconstruction error is the amount of discrepancy between reconstructed CSI and input CSI.

Based on these quality assessments, it can be possible to select high-quality samples while dynamically adjusting or filtering out low-quality ones to improve overall robustness and efficiency of model updates. Moreover, the quality assessments may be utilized as a potential approach to help adaptive model (re)training in the performance monitoring. For instance, when the prediction confidence/temporal consistency falls below a certain threshold, the adjustment can be needed on the prediction/measurement window and the corresponding duration to prevent unreliable predictions. Accordingly, the configuration on ground truth CSI needs to be adjusted to reflect model performance and data quality variations. When identifying prediction-related issues, measurement durations or future instance association can be modified based on confidence and consistency level, and the ground truth labels also can be adjusted on modifying measurement duration or redefining the association between current and future instances to improve the prediction accuracy.

Proposal #4: To improve LCM aspects on TSF-domain CSI compression Case 3 and 4, consider the following:

- Data quality-based sample selection for model updates

  - Example of the quality metrics: Prediction confidence, temporal consistency, CSI reconstruction error etc.

Regarding data collection for training, CSI-RS enhancement at least for ground truth CSI is required to enhance the reliability and accuracy on model training/update. As a potential approach, ground truth-associated CMR can be separately configured with higher power level compared to other measurement reference signals, or configured with increased RS density to improve the measurement accuracy. Also, for the purpose of ground truth CSI at least for Case 3, dedicated CSI-RS configuration with specifying usage parameters to distinguish other RSs. For proper performance monitoring, linkage between measurement RS and RS used for ground truth CSI can be defined along with the corresponding transmission timing.

Proposal #5: On data collection for training at least for Case 3, consider CSI-RS enhancement from the following aspects:

- Separate CMR for ground truth CSI: Adjusting power level and RS density

- Temporal alignment: Linkage between inference and ground truth CSI measurements

[Methods to handle non-ideal UCI feedback]

In addition, in RAN1#116, the impact on non-ideal UCI feedback for TSF-domain CSI compression was agreed to study. In case at least of UCI missing and UCI dropping with two-part UCI encoding, utilizing the temporal domain may cause the performance degradation since there can be asymmetry between accumulated past CSI at UE side and NW side. Also, error propagation from the poor past CSI caused by some poor channel environment, e.g. some deep fading or sudden blockage, can consecutively degrade CSI compression performance on the AI/ML model.

Based on the above, to exploit the benefit of the TSF-domain CSI compression, it needs to check that the past CSI and/or its channel property are still useful to compress the target/current CSI. In this sense, the related performance monitoring in order for life-cycle management (LCM) can be necessary to design by considering TSF-domain CSI compression, if specified. For example, when performance monitoring of two-sided model, there can be an ambiguity that the output performance is degraded whether the AI/ML model is not suitable for deployed environment or the quality of accumulated past CSI is bad while the model is suitable for the environment. To handle the issue, one potential solution can be two-step performance monitoring for the TSF-domain CSI compression where it is to firstly check the quality of accumulated past CSI if the monitoring metric value is degraded by NW signaling to UE. Based on the signaling, for example, UE can flush-out some accumulated past channel information (e.g. the oldest CSI, erroneous CSI compared to other accumulated past CSI, etc.) to address the issue on the accumulated past CSI. After that, if the monitoring performance is still in low, the AI/ML model can follow LCM procedure such as model fine-tuning/update/switching. As another approach, it is possible to utilize the management on accumulated past CSI as described above that UE can include not only the present CSI but also a certain past CSI(s) which is based on NW-triggered indication with a certain CSI instances as the CSI feedback where UCI is missed. In case of UCI dropping, it means that some CSI information with low priority is dropped due to the limited size of UCI container while the corresponding UCI is successfully received by NW-side, i.e. partial PMI related CSI dropping when considering two-part UCI encoding method. It also leads the mismatch on past CSI between UE-side and NW-side. For example, when RI=2 and the number of instances is 4, there is a case of dropping CSI on layer 2 for the first and the second instances while CSI on layer 1 is delivered for all the instances. Then, the past CSI on Layer 1 includes the CSI for 4 instances while the past CSI on layer 2 only includes the CSI for 2 instances. Hence, it may be necessary to deliver the dropped CSI information to NW-side, e.g., by defining report mode for dropped CSI(s) at a certain past instance(s) and/or adopting a certain priority to the following CSI report(s) on the dropped CSI information with respect to the number of dropped occasions

Proposal #6: Regarding non-ideal UCI feedback on TSF-domain CSI compression,

- Consider two-step performance monitoring to check that the performance degradation of the AI/ML model is originated from whether the accumulated past CSI has a problem or the AI/ML model is not suitable for the deployed environment

- Consider to report past CSI information via NW-triggered signaling when UCI missing or UCI dropping

[Methods to handle rank adaptation]

As shown in the above agreements, it has been discussed to evaluate the scenario when the rank adaptation is applied to TSF-domain CSI compression (and prediction). In this scenario, the rank depends on the condition of wireless channels and hence it would be varied with respect to the instance of target CSI. Then, the past CSI for different rank/layer(s) may not be available or be outdated. Accordingly, this leads the mismatch of past CSI at UE-/NW-side, which causes the performance degradation for TSF-domain CSI compression as mentioned above. To handle the issue, for a given RI, it can be considered to compose CSI feedback based on the availability of the corresponding layer(s) as a potential approach. For example, when RI is equal to 3, it is assumed that the past CSI on layer 1/2 is available but that of layer 3 is not by evaluating the availability whether the CSI on each layer is reported or not for a certain time-duration. Then, the CSI corresponding to the layer1/2 can be encoded via offset information based on the past CSI and the CSI of layer 3 is encoded in itself. Based on this CSI feedback, NW-side can update/manage the past CSI on layer 3 and correctly decode the CSI feedback from the UE-side.

Proposal #7: Consider the method on the rank adaptation based on the availability check of layer(s) for a given RI.

[CQI determination]

In current non-AI/ML based CSI feedback based on legacy codebook, CQI determination at UE is based on PMI of the codebook, and the gNB can transmit DL data by taking the UE side CSI feedback on CQI, RI, and PMI into account. Meanwhile, there is an issue on the CQI determination for AI/ML two-sided model CSI compression since the CSI reconstruction part is not available at UE-side. Hence, in Rel-18, the following options were discussed to determine the CQI at UE-side (at RAN1#112)

Agreement In CSI compression using two-sided model use case, further study the following options for CQI determination in CSI report, if CQI in CSI report is configured. - Option 1: CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 1a: CQI is calculated based on target CSI with realistic channel measurement - Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment - Option 1c: CQI is calculated based on legacy codebook - Option 2: CQI is calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment - Note: CSI reconstruction part at the UE can be different comparing to the actual CSI reconstruction part used at the NW. - Option 2b: CQI is calculated using two stage approach, UE derive CQI using precoded CSI-RS transmitted with a reconstructed precoder. - Other options are not precluded - Note1: feasibility of different options should be evaluated - Note2: Gap analyses between the UE side CQI calculation results and the NW side results, as well as the impact on the scheduling performance should be evaluated - Note3: Complexity of CQI calculation needs to be evaluated, including the computing complexity and potential RS/signaling overhead

Regarding Option 1, it does not depend on the CSI reconstruction part and based on using realistic channel measurement or legacy codebook but this method can cause a CQI mismatch compared to CQI using AI/ML-based model. As a potential approach to alleviate the issue, Option 1b with some offset signaling from NW-side in order to adjust the CQI gap can be considered. Also, in Option 1c, non-AI/ML based CSI report based on legacy codebook can be triggered in addition to AI/ML based CSI report. Option 2 is based on the CSI reconstruction part but there are several problems on the signaling overhead to deliver the CSI reconstruction part at NW-side and UE computational complexity. Using proxy model(s) to calculate the CQI can alleviate the above issues but still has a problem on reliability compared to the actual CSI reconstruction part. To handle the issue, the model complexity reduction scheme such as knowledge distillation would be adopted for the CSI reconstruction part at NW-side. In case that the knowledge distillation from NW-side CSI reconstruction part is employed, UE-side CSI reconstruction part can be trained based on the knowledge as an assistance information to determine the CQI and the corresponding CQI would be estimated relatively with higher accuracy compared to adopting the proxy model. Option 2b relies on two-step signaling which means more latency is required compared to other options for CQI calculation. So, it could cause CQI mismatch due to channel aging problem which is not preferable.

Proposal #8: For CQI determination in CSI compression using two-sided model, consider to prioritize Option 1. If Option 2 is supported, further consider

- Option 2a: Utilizing AI/ML model complexity reduction method to reduce the signaling overhead to deliver the CSI reconstruction part at NW-side.

Inver-vendor training collaboration

On Rel-19 study for CSI compression, as WID described in section 1, the methods to alleviate/address the training collaboration of two-sided CSI compression model by considering inter-vendor issue have been studied, and corresponding agreements were made in RAN1#119 as follows.

Conclusion For Direction A 4-1 and 3a-1, send LS to RAN2 for the study of feasibility of parameter / dataset exchange Agreement Final LS R1-2410922 is endorsed. Agreement - For Direction A Option 3a-1 and Direction C, study the feasibility of scalable model structure specification over numbers of Tx ports, CSI feedback payload sizes, and bandwidths, number of slots. - Feasibility of Direction A Option 3a-1 is contingent on the feasibility of the scalable model structure specification. - Feasibility of Direction C is contingent on the feasibility of the scalable model structure specification and model parameters specification. - Note: Angular/Delay/Doppler domain conversion and quantization are considered to be part of feasibility study

[Additional aspects on Direction A]

Regarding Direction A 4-1 and 3a-1, it was agreed to send the corresponding LS to RAN2/3 to check the feasibility of parameter/dataset exchange on signaling/payload overhead. Considering the above, as another aspect on model alignment/pairing, Option3a-1 may be needed to provide actual NW-side CSI reconstruction model/parameter to the UE side for exploiting the performance benefit and model alignment unless using some kind of proxy decoder, where there is a proprietary issue to disclose the implementation related information. In addition, while the model structure/format is standardized by considering UE-side CSI generation part only, it still needs to train their model for different UE and NW vendors, which may lead to high computation complexity and signaling overhead especially on training dataset. This problem is also listed as shown in Issue#1 to train UE-side CSI generation part. To alleviate this problem, it can be a potential approach to adopt sub-model/-parameter (i.e. bridge model) which may be concatenated to its own UE-part model. Based on this kind of approach, a certain NW vendor can deliver to UE the bridge model and the small amount of vendor-specific training dataset as an additional information to train/update UE-side CSI generation part in an efficient manner.

Meanwhile, when considering Option 4-1, this kind of separate training by exchanging data/dataset shows some performance loss on CSI accuracy since it is difficult to align the AI/ML model (i.e. CSI generation part and/or CSI reconstruction part) between NW-side and UE-side. In a view of model performance/alignment, training UE-side CSI reconstruction part is more beneficial based on the information {CSI feedback, reconstructed target CSI} not only for training UE-side CSI generation part. Also, it can be utilized as an intermediate KPI estimator at UE side especially on the use case for UE-side performance monitoring. By taking the above into account, Option 4-3 seems to show the best performance by training CSI generation part and CSI reconstruction part altogether at UE-side but the overhead increment on the dataset is inevitable as listed Issue#3. To address the problem, the relationship between target CSI and reconstructed target CSI can be considered. In general, that information is not the same since the reconstructed target CSI experiences some kind of information loss (e.g. compression/decoding via AI/ML model, quantization) and various errors (e.g. distortion, bias). However, from a long-term or averaged perspective, that information can have a correlation as long as the AI/ML model, deployed scenario/area, and channel characteristics are unchanged. Note that the correlation can be expressed by inner product of eigenvector(s) or vector offset between target CSI and reconstructed target CSI. By utilizing this information, UE can generate reconstructed target CSI while the dataset includes {target CSI, CSI feedback} only, or vice versa. Then, it leads to not only the overhead reduction but also the improvement on output performance and model alignment between NW-side and UE-side

Proposal #9: On inter-vendor training collaboration via dataset exchange (i.e., Option 4-1), consider assistant information regarding relationship between target CSI and reconstructed target CSI to reduce/alleviate signaling overhead and model alignment issue.

In addition, the separate training method may lead more performance degradation than joint 1-on-1 training method for a given computational/signaling complexity. To further reduce the complexity and the issue on model alignment, it may be beneficial to consider model complexity reduction method, e.g., knowledge distillation, pruning, etc. In case that the knowledge distillation from NW-side CSI generation part is employed, UE-side CSI generation part can be trained based on the knowledge with/without output dataset. Then, it can also address the overhead concern on dataset exchange related method while maintaining model alignment as much as possible.

Proposal #10: Study on model complexity reduction method, e.g., knowledge distillation, to further reduce the CSI training/signaling complexity and issue on model alignment for Type 3 training collaboration.

[Feasibility of scalable model structure]

As the above, the feasibility study for Direction A and C was agreed on scalable model structure. In Rel-18, for CSI compression based on two-sided model, impact on model generalization and scalability was studied. Regarding the model scalability, one fundamental challenge is input and output dimensions for AI/ML model are typically fixed. Consequently, to support CSI compression across various type of CSI-RS resources, multiple configuration-specific models are required, while it leads to memory/buffer issues especially for a UE due to storing multiple models. The previous studies have explored to mitigating this scalability issue through padding/truncation (on input and output data) and adaptation layers where it utilizes configuration-specific layers with a common encoder/decoder to address the scalability across various configurations.

However, these techniques still have the scalability challenges due to increased model complexity and limited adaptability since the necessity to predefine a maximum input dimension and introduce configuration-specific layers can lead inefficiency, particularly when considering the configuration with higher number of sub-bands and ports. To address the limitation, patch-based CSI encoding/decoding method can be considered as a potential solution. Instead of processing the entire measured CSI at once, this method segments the measured CSI samples into multiple patches for encoding. Then, per-patch decoding can be employed based on the information of patch settings. By adjusting the patch structure (merging or omitting patches with a specific criterion, e.g. using correlation of frequency domain unit (sub-bands)), it enables more efficient CSI feedback with maintaining CSI reconstruction accuracy with scalability across diverse configurations.

Conclusion

In this contribution, we discussed further potential aspects on improving CSI compression/feedback performance and training collaboration with two-sided AI/ML model for CSI compression. The following proposals are provided.

Proposal #1: Regarding temporal/spatial/frequency (TSF)-domain CSI compression, study methods/mechanisms to manage the similarity/synchronization of accumulated past CSI at UE-side and/or NW-side.

Proposal #2: Regarding TSF-domain CSI compression, discuss the format of past CSI information and how to report it at least for performance monitoring perspective.

Proposal #3: Regarding TSF-domain CSI compression Case 3 and 4, consider performance monitoring method on joint CSI compression and prediction by adapting the operation on the AI/ML model between CSI compression and prediction.

Proposal #4: To improve LCM aspects on TSF-domain CSI compression Case 3 and 4, consider the following:

- Data quality-based sample selection for model updates

  - Example of the quality metrics: Prediction confidence, temporal consistency, CSI reconstruction error etc.

Proposal #5: On data collection for training at least for Case 3, consider CSI-RS enhancement from the following aspects:

- Separate CMR for ground truth CSI: Adjusting power level and RS density

- Temporal alignment: Linkage between inference and ground truth CSI measurements

Proposal #6: Regarding non-ideal UCI feedback on TSF-domain CSI compression,

- Consider two-step performance monitoring to check that the performance degradation of the AI/ML model is originated from whether the accumulated past CSI has a problem or the AI/ML model is not suitable for the deployed environment

- Consider to report past CSI information via NW-triggered signaling when UCI missing or UCI dropping

Proposal #7: Consider the method on the rank adaptation based on the availability check of layer(s) for a given RI.

Proposal #8: For CQI determination in CSI compression using two-sided model, consider to prioritize Option 1. If Option 2 is supported, further consider

- Option 2a: Utilizing AI/ML model complexity reduction method to reduce the signaling overhead to deliver the CSI reconstruction part at NW-side.

Proposal #9: On inter-vendor training collaboration via dataset exchange (i.e., Option 4-1), consider assistant information regarding relationship between target CSI and reconstructed target CSI to reduce/alleviate signaling overhead and model alignment issue.

Proposal #10: Study on model complexity reduction method, e.g., knowledge distillation, to further reduce the CSI training/signaling complexity and issue on model alignment for Type 3 training collaboration.

- Reference

[1] RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm