3GPP TSG RAN WG1 #120   	 	  	                 R1-2500568

Athens, Greece, February 17th – 21st, 2025

Agenda item:		9.1.4.2

Source:		LG Electronics

Title:			Discussion on other aspects of AI/ML model and data

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss on the following study objectives described in the revised WID [1].

Study objectives: (omitted) - Necessity and details of model Identification concept and procedure in the context of LCM for two-sided models [RAN2/RAN1] - CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]: - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods - Model transfer/delivery [RAN2/RAN1]: - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study

Discussion

Model/functionality identification and LCM

To determine applicable functionality among the supported functionalities by UE, NW may provide NW-side additional condition to UE. The additional condition may or may not be configured with an ID, i.e. associated ID in previous RAN1 agreements. In other words, the exact form of the additional condition configuration, e.g. whether or not to define an associated ID, is up to sub-use-case signaling design.

Proposal#1. Clarify that NW can configure NW-side additional condition to UE for helping UE to select applicable functionality among supported functionalities, and the exact configuration method including whether or not to use the associated ID is up to sub-use-case signaling design.

The NW-side information in relation to the additional condition is highly dependent on the usage/structure of an AI/ML model, e.g. input/output, sub-use-case, etc. Therefore, details of associated ID, including whether it is needed, the name of the ID, the information inferred from the ID, and how the ID is assigned/used, should be discussed and decided per sub-use-case.

Proposal#2. Details of associated ID, including whether it is needed, the name of the ID, the information inferred from the ID, how the ID is assigned/used, should be discussed and decided per sub-use-case.

For MI-Option 2, ID-X can be used along with dataset transfer. ID-X is for UE to identify the characteristic of the dataset. However, ID-X cannot distinguish UE part model since UE may use same model generalized for multiple ID-Xs or UE may develop multiple models for same ID-X, e.g. depending on UE-oriented factor(s), as clarified in AI-Example2-1 captured below. For model identification purpose, model ID needs to be reported by UE in Step C. For model pairing purpose, ID-X is sufficient since NW does not need to know UE-part model information (e.g. model IDs) if they are trained under a same dataset.

Agreement From RAN1 perspective, for UE part of two-sided model, further study the following example of MI-Option2 (including the feasibility/necessity) - AI-Example2-1 - A: A dataset is transferred from the NW/NW-side to UE/UE-side via standardized signaling. - Note: RAN1 study of Step A only focuses on RAN1 aspect of the dataset transfer from NW to UE. Other solution for dataset exchange is out of RAN1 scope. - B: UE part of two-sided model(s) is(are) developed based on at least the above dataset. - C: UE reports information of its UE part of two-sided model(s) corresponding to the above dataset to the NW. Agreement For study of MI-Option2 (i.e., model identification with dataset transfer) for the two-sided model, - ID-X can be used for pairing the UE-part and the NW-part of a two-sided model - FFS: other information needed for pairing

Proposal#3. For MI-Option 2, clarify that ID-X alone cannot serve model identification purpose, and for the pairing purpose, ID-X is sufficient.

Model delivery/transfer

In previous meetings, it was concluded that the model transfer/delivery Case z2, z3, and z5 are deprioritized for Rel-19 from RAN1 perspective. In our view, given that the analysis was already done in Rel-18 as captured below, it would be more valuable to discuss how to address key challenges of model transfer such as offline cross-vendor collaboration, model storage requirements, and proprietary design disclosure issues by proponent companies.

Agreement For model delivery/transfer to UE (for UE-side models and UE-part of two-sided models): - Model delivery/transfer to UE, if feasible, may be beneficial to handle scenario/configuration specific (including site-specific configuration/channel conditions) models (i.e., when a single model cannot generalize well to multiple scenarios/configurations/sites), to reduce the device storage requirement. - Model delivery/transfer to UE after offline compiling and/or testing may be friendlier from UE’s implementation point of view compared to the case without offline compiling and/or testing. On the other hand, the case without offline compiling and/or testing (that can update parameter with known model structure), may have benefit at least in terms of shorter model parameter update timescale. - For model trained at network side, Case y (w/ network-side training) and Case z2 may incur the burden of offline cross-vendor collaboration such as sending a model to the UE-side and/or compiling a model. - For model trained at UE side/neutral site, Case z1 and Case z3 may incur the burden of offline cross-vendor collaboration to send the trained model from the UE-side to the network, compared to Case y (w/ UE-side training) which does not have such burden. - Model storage at the 3gpp network, compared to storing the model outside the 3gpp network, may come with 3gpp network side burden on model maintenance/storage. - Proprietary design disclosure concern may arise from model training and/or model storage at the network side compared to other cases (such as case y with UE side training) which does not have such issue.

Proposal#4. Focus on discussing the key challenges of model transfer such as offline cross-vendor collaboration, model storage requirements, and proprietary design disclosure issues.

Regarding Case z4, some study points were agreed in RAN1#118bis.

Agreement Regarding model transfer/delivery Case z4 for inference, further study the following aspects: - Whether/when UE needs the transfer of new parameters for a known model structure. - Whether/when the AI model with the transferred parameters is ready for inference. - The transfer of partial of the parameters for a known model structure.

Whether/when UE needs the transfer of new parameters for a known model structure can be known by NW with performance monitoring procedure. It is because NW can determine whether/when model transfer is needed based on the result of performance monitoring, e.g. transferring a new model is needed when the performance of current model is degraded.

Whether/when the AI model with the transferred parameters is ready for inference can be known by NW if model application time (MAT) is specified, which corresponds to the minimum required time for UE to apply the transferred model. Once model is transferred, NW knows from when the transferred model is applicable at UE side from the specified MAT. Regarding this issue, the following two options were identified in RAN1#119. Comparing these two options, we support Option 2 to avoid unnecessary signalling from UE to NW.

Conclusion For the study of model delivery/transfer Case z4, if the model delivery/transfer is directly used for inference, the following options are identified as the candidate solutions to determine the readiness of AI model with the transferred parameters for inference (either or combination of the following options) - Option 1: UE sends signaling to network to notify that the AI model with the transferred parameters is ready for activation - Option 2: The AI model with the transferred parameters can be assumed ready for activation from a minimum applicable time after the completion of model delivery/transfer

Proposal#5. Whether/when UE needs the transfer of new parameters for a known model structure can be known by NW via performance monitoring procedure.

Proposal#6. Whether/when the AI model with the transferred parameters is ready for inference can be known by NW via defining model application time, i.e. the minimum required time for UE to apply the transferred model, i.e. Option 2.

Regarding the model ID options for model transfer Case z4, a model needs to be identified with both structure and corresponding parameter set. Thus, Option 2 is not appropriate option. Both Option 1 and Option 3 are possible but we think that details of such ID should be discussed in WI phase after clarifying signaling procedures related to model ID, the first indication, and the second indication.

Agreement Regarding the study of model transfer/delivery Case z4, for a given known model structure, network can transmit the following information along with the parameters: - a first indication referring to the known model structure at least for the case there are multiple known model structures - a second indication referring to the transmitted parameters - FFS: relationship of model ID, first indication, and second indication - FFS: transfer partial parameters Agreement Regarding the relationship of model ID, first indication, and second indication for model transfer/delivery Case z4, further study the following options: - Opt.1: model ID consists of the information of the first indication and the second indication - E.g., model ID is a combination of the first and second indications - Opt.2: The second indication is assumed as the model ID - Opt.3: Model ID is assigned by network and is separated from the first indication and the second indication

Proposal#7. Model ID should infer both structure information and corresponding parameter set information. Details of model ID, first indication, and second indication should be left to WI phase after clarifying signaling procedures related to such IDs.

Conclusion

In this contribution, the following proposals are provided.

Proposal#1. Clarify that NW can configure NW-side additional condition to UE for helping UE to select applicable functionality among supported functionalities, and the exact configuration method including whether or not to use the associated ID is up to sub-use-case signaling design.

Proposal#2. Details of associated ID, including whether it is needed, the name of the ID, the information inferred from the ID, how the ID is assigned/used, should be discussed and decided per sub-use-case.

Proposal#3. For MI-Option 2, clarify that ID-X alone cannot serve model identification purpose, and for the pairing purpose, ID-X is sufficient.

Proposal#4. Focus on discussing the key challenges of model transfer such as offline cross-vendor collaboration, model storage requirements, and proprietary design disclosure issues.

Proposal#5. Whether/when UE needs the transfer of new parameters for a known model structure can be known by NW via performance monitoring procedure.

Proposal#6. Whether/when the AI model with the transferred parameters is ready for inference can be known by NW via defining model application time, i.e. the minimum required time for UE to apply the transferred model, i.e. Option 2.

Proposal#7. Model ID should infer both structure information and corresponding parameter set information. Details of model ID, first indication, and second indication should be left to WI phase after clarifying signaling procedures related to such IDs.

Reference

[1] RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm