3GPP TSG RAN WG1#120			R1-2500575

Athens, Greece, February 17th-21st, 2025

Agenda item:		9.6.3

Title:			LP-WUS operation in CONNECTED mode

Source:		Nokia

Document for:		Discussion and Decision

# 1. Introduction

In this document, we discuss several different aspects of CONNECTED mode LP-WUS design, including:

- LP-WUS Monitoring Occasions

- Minimum Time Gap

- LP-WUS support of NR-DC and CA

- QCL/TCI-state for LP-WUS

- CSI/L1-RSRP Reporting Specification Impacts

- Coexistence with existing UE power saving features

From those discussions, we provide our observations and proposals.

# 2.   Discussion

## 2.1	LP-WUS Monitoring Occasions

At the last RAN1#119 meeting, the following agreement was made outlining alternative approaches for the configuration of the LP-WUS MOs for Options 1-1 and 1-2.  In this sub-section discuss the approaches outlined in the agreement and provide our proposals.

Agreement - For LP-WUS MOs in connected mode for Option 1-1, following is considered further discussion and possible down-selection - Approach 1: - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s). - UE selectively monitors LP-WUS in the LP-WUS MOs identified by other parameters such as time offset prior to the start of drx-onDurationTimer. - Approach 2: - LP-WUS MOs, including periodicity and offset, are identified based on the slot that drx-onDurationTimer would start and a new RRC parameter indicating the time offset until the slot that drx-onDurationTimer would start. - FFS: An RRC parameter indicates the duration where the UE monitors LP-WUS [consecutively in time] - FFS: the consecutive LP-WUS MOs in time identified by the RRC parameter are associated with a same C-DRX cycle; - FFS: LP-WUS MOs in time identified by the RRC parameter provides consistent indication for the same C-DRX cycle of the UE, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOs associated with the same C-DRX cycle. - For LP-WUS MOs in connected mode for Option 1-2, following is considered further - Approach 1: - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s). - UE monitors LP-WUS in the LP-WUS MOs and starts a new timer for PDCCH monitoring triggered by LP-WUS, after a time offset. - Approach 2: - Periodicity/offset of the slot that the new timer for PDCCH monitoring may be potentially triggered by LP-WUS are configured by RRC parameters. - LP-WUS MOs, including periodicity and offset, are identified based on the slot that new timer would start and a new RRC parameter indicating the time offset until the slot that new timer would start. - FFS: An RRC parameter indicates the duration where the UE monitors LP-WUS [consecutively in time] - FFS: the consecutive LP-WUS MOs in time identified by the RRC parameter are associated with the same slot that the new timer would start; - FFS: LP-WUS MOs in time identified by the RRC parameter provides consistent indication for the same slot that the new timer would start, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOs associated with the same slot that the new timer would start.

For both options 1-1 and 1-2, approach 1 gives the network the most flexibility for scheduling MOs as it is not restricted to the effective start of the drx-ondurationtimer.

Observation 1:		For both options, approach 1 gives the network scheduler the most flexibility.

For both options 1-1 and 1-2, approach 2 can be considered as a special case of approach 1.

Observation 2:		For both options, approach 2 can be considered as a special case of approach 1.

Having a unified approach to defining MOs for both options 1-1 and 1-2 will simplify subsequent specification work.

Observation 3:	A unified approach for specifying MOs for options 1-1 and 1-2, will simplify 3GPP specifications.

Using a unified approach, allows option 1-1 to be defined as a special case of the option 1-2 when the following restriction is applied.

Observation 4:	Option 1-1 is a special case of Option 1-2, where the UE is restricted to monitoring the first set MO(s) that immediately precede the start of drx-onDurationTimer by the smallest period that is no less than the configured minimum time gap.

For the above reasons captured in the above observations, we have the following proposals.

Proposal 1:		For LP-WUS MOs in connected mode for Option 1-1,

      - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

      - UE selectively monitors LP-WUS in the LP-WUS MOs identified by other parameters such as time offset prior to the start of drx-onDurationTimer.

Proposal 2:		For LP-WUS MOs in connected mode for Option 1-2,

      - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

      - UE monitors LP-WUS in the LP-WUS MOs and starts a new timer for PDCCH monitoring triggered by LP-WUS, after a time offset.

For both options 1-1 and 1-2, to increase network MO scheduling opportunities for the same PDCCH monitoring period, we support the use of a RRC parameter to define a time window where the network has multiple opportunities to send the same LP-WUS.

Proposal 3:                    For option 1-1, a RRC parameter is used to define the time window, within which the network has multiple consecutive MOs it can use to trigger PDCCH monitoring in the same C-DRX cycle.

FFS:   If consecutive MOs start in the symbol or slot proceeding the previous MO.

Proposal 4:	For option 1-2, a RRC parameter is used to define the time window, within which the network has multiple consecutive MOs it can use to trigger PDCCH monitoring in the same new PDCCH monitoring period.

FFS:   If consecutive MOs start in the symbol or slot proceeding the previous MO.

The figure below, illustrates how MOs could be configured for both options 1-1 and 1-2, in accordance with the proposals stated above.  Note, no attempt has been made to align the periodicities of the MO configuration to the legacy C-CRX cycle, hence for option 1-1 there is a variable delay between the end of the MO window and the beginning of the corresponding OnDuration.

Lpwus_duration

Lp-wus periodicity

drx-onDurationTimer

Lpwus-drx-onDurationTimer

C-DRX cycle

MinTimeGap

Lp-wus start offset

OPTION 1-2

OPTION 1-1

Figure 1:   Unified approach to defining MOs for Options 1-2 and 1-1

## 2.2	Minimum Time Gap Definition

Below, are the agreements relating the CONNECTED mode minimum time gap from previous meeting along with the draft combined proposal drafted at the end of the last meeting.  In this section we share our thoughts and proposals on the remaining design details of the connected mode Minimum Time Gap.

RAN1#116 Agreement For RRC CONNECTED mode, minimum time gap between LP-WUS reception and MR to start PDCCH monitoring is introduced considering at least following - LP-WUS processing time - MR transition time for ramp up - Time/frequency synchronization of MR - FFS whether UE can report supported minimum time gap from candidate values FFS: Whether the minimum time gap values can be more than one RAN1#117 Agreement For RRC CONNECTED mode, support UE capability report for determination of minimum time gap between LP-WUS reception and MR to start PDCCH monitoring. - FFS: exact value(s) of the minimum time gap - FFS: support of multiple minimum time gaps - FFS whether the reported value includes the duration for time/frequency synchronization of MR RAN1#118 Agreement Select one of the following alternatives in RAN1#118bis Alt 1: For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. - FFS: X - FFS: definition of reported value Alt2: For RRC CONNECTED mode, UE reports multiple values for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. - FFS: X - FFS: definition of reported value - Different minimum time gaps correspond to different sleep states - Companies are encouraged to details on how the reported values are to be used by the network RAN1#118-bis Agreement For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. - FFS: X - FFS: definition of reported candidate value - UE can indicate its preference for configured time offset between LP-WUS and PDCCH reception using UAI mechanism. FFS details. UAI is optional. - FFS: A single time offset is configured for a UE at a given time RAN1#119 #### [Fri]Combined Proposal 3.2-1/3.2-2c: For future meetings: Consider the following alternatives for UE capability report on the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring: - Alt 1: For the X = [2, 3, 4] candidate values for the UE capability report on the minimum time gap, consider {0.25, 0.5, 1, 2, 3, 6, 10, 20}ms as starting point - Note: Other values are not precluded - FFS whether different values are selected for OOK-based WUR and OFDM-based WUR - FFS whether/how to consider the case of CA - Alt 2: For the UE capability report on the minimum time gap, UE reports a pair of values for main radio ramp up and the number of SSBs needed for synchronization (D, NSSB). - Alt 3: UE capability report on the minimum time gap includes the main radio ramp up time. The number of SSBs needed for synchronization is defined in specs, and different number of SSBs can be defined for different UE capability on the main radio ramp up time. - Alt 4: UE capability report on the minimum time gap includes the main radio ramp up time. The time needed for synchronization will be defined by RAN4 requirements later.

During the RAN1#119 meeting, some companies expressed a desire to separately reflect the number of SSBs required by the UE to achieve synchronisation in connected mode, either through additional capability report signalling (Combined proposal Alt 2) or by being explicitly defined within specifications (Combined proposal Alt 3).

In our view, the minimum time gap value should:

- Already include the duration for time/frequency synchronization of the MR

  - Given the device has already woken up from an ultra-deep sleep to be in this CONNECTED mode state and is maintaining some degree of timing/synchronisation from ongoing measurements.

- Not be heavily dependent on the SSB periodicity

  - Same reason as above.

- Be independent of the receiver type (OOK-based WUR and OFDM-based WUR)

  - Unlike in idle mode with its ultra-deep sleep mode and fewer ongoing measurements, we expect a reasonable level of timing/frequency to be maintained in connected mode even for the OOK-based WURs present.

  - If less well synchronized OOK-based WURs are present, the network can always indicate longer minimum time gap values.

- Be selected from as small a set as possible, to simplify network scheduling and allow more opportunities for more efficient sub-group multiplexing.

We therefore support the following proposal:

Proposal 5:                  	For the minimum time gap in a non-CA scenario, the UE reports one value from 2 candidate values.

FFS:   The exact set of candidate values {0.25, 0.5, 1, 2, 3, 6, 10, 20}ms as starting point

FFS:   Whether/how to consider the case of CA

For scenarios where there is CA, we envisage that minimum time gap values will be required to account for additional time to wake up the radio in a controlled fashion (to avoid a sudden power surge) to accommodate other component carriers:

Proposal 6:                      	For the minimum time gap in a CA scenario, RAN1 down-selects from:

Alt 1:     Mandate a value in the candidate set is for CA.

Alt 2:	     Expand the candidate set to include an additional value for CA.

Alt 3:     Providing a separate CA specific set of candidate values.

## 2.3	LP-WUS support of NR-DC and/or CA

In this section we share our thoughts on the design details of the connected mode handling of CA and

4.1.1	Previous agreements and proposals relating to the CA

RAN1#117 Agreement Study whether/how LP-WUS works when UE is configured with CA in RRC CONNECTED mode - FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable - Option 1: one or more serving cells based on gNB indication/configuration - Option 2: all activated serving cells - Note: other options are not precluded RAN1#118 Agreement LP-WUS is at least supported for the case where a UE is configured with CA in RRC CONNECTED mode - FFS: DC RAN1#118-bis Agreement LP-WUS is at least supported for the case where a UE is configured with CA in RRC CONNECTED mode - FFS: DC RAN1#119 [Fri]Proposal 5-2a: For the case when UE is configured with NR-DC and/or CA without dual DRX groups in RRC CONNECTED mode, - LP-WUS is configured on a serving cell per cell-group - LP-WUS indication is applicable to all activated serving cells in the cell-group For the case when UE is configured with NR-DC and/or CA with dual DRX groups in RRC CONNECTED mode, Opt.1 - LP-WUS is configured on a serving cell per cell-group - LP-WUS carries separate indications for each DRX group(s) to trigger PDCCH monitoring of all activated serving cells - Note: This requires RAN2 coordination via LS Opt.2 - LP-WUS is configured on a serving cell per DRX group per cell-group - LP-WUS indication is applicable to all activated serving cells in the DRX group per cell-group - Note: This requires RAN2 coordination via LS Opt.3 - LP-WUS is configured on a serving cell per cell-group - LP-WUS indication is applicable to all activated serving cells of all DRX groups per cell-group. FFS: Whether the serving on which the LP-WUS is configured is only PCell/PSCell or RRC configured cell other than PCell/PSCell

For dual connectivity, a UE is connected to different cells/cell groups that are controlled by different gNBs.  This is unlike CA, where all cells belong to the same gNB.  Because of this looser/non-ideal backhaul connection between cells in DC, we expect the LP-WUS to trigger PDCCH monitoring only in the cell-group controlled by that gNB.

Within a cell-group, cells can either be dormant or non-dormant.  The observation below summarizes the key difference between dormant and non-dormant cells.

Observation 5:    	For RRC CONNECTED mode, the key difference between an ACTIVATED SCell that is in a DORMANT state and one that is a NON-DORMANT state, is that the NON-DORMANT SCell supports both measurements and PDCCH monitoring whereas the DORMANT SCell supports only a minimal set of measurements.

Several options exist for how to handle SCell dormancy after LP-WUS monitoring is begun.  These options include:

Option 1:	Maintain measurements on DORMANT SCells

Option 2:	Deactivate all or a subset DORMANT SCells when LP-WUS monitoring is resumed

This can either be immediately or after some inactivity timer.

Option 3:            Explicitly indicate using the LP-WUS, the preferred DORMANCY state of SCells.

RAN1#119 Agreement LP-WUS in RRC CONNECTED mode does not include Scell dormancy indication. - Note: LP-WUS indication does not impact SCell dormancy behavior

Given the RAN1#119 agreement above we have the following observation.

Observation 6:	Measurements in dormant SCells are unaffected by LP-WUS operation.

Given the discussion above and the original draft proposal from RAN1#119, we have the following proposal.

Proposal 7:	For the case when the UE is configured with NR-DC and/or CA without dual DRX groups in RRC CONNECTED mode,

- LP-WUS is configured on a serving cell per cell-group

- LP-WUS detection triggers PDCCH monitoring on all non-dormant activated cells within the cell-group.

For the case where the UE is configured with NR-DC and/or CA with dual DRX groups in RRC CONNECTED mode, several options (see RAN1#119 draft proposal) for handling the dual DRX groups were presented.

- Option 1 is arguably the most complex but also the most flexible, as it requires the single LP-WUS payload to convey at least 2 bits to indicate which DRX group(s) to trigger PDCCH monitoring in.

- Option 2 does not increase the payload of the individual LP-WUS but does restrict the LP-WUS to indicating the DRX group of the serving cell in which it is applied.  This will mean that to 2 separate LP-WUSs will be required, one in a serving cell for each DRX group.  It also means that the UE is now expected to monitor 2 separate LP-WUSs.

- Option 3 is the simplest approach that requires no LP-WUS payload changes and no requirement on the device to monitor 2 LP-WUSs.  The drawback is that it will activate all serving cells of all DRX groups, even if one DRX group was not immediately required.

Of these 3 options, we believe option 3 has the least impacts on LP-WUS and LR design/deployment and performance.  The only drawback is that all DRX groups are enabled, but once full legacy operation is reestablished, legacy procedures can be applied to disable one of the DRX groups if required.

Proposal 8:	For the case when UE is configured with NR-DC and/or CA with dual DRX groups in RRC CONNECTED mode,

        - LP-WUS is configured on a non-dormant serving cell per cell-group

        - LP-WUS detection triggers PDCCH monitoring on activated non-dormant serving cells of all DRX groups per cell-group.

## 2.4	QCL/TCI-state for LP-WUS

In this section we share our thoughts on the design details of the connected mode TCI/QCL

5.1.1	Previous agreements and proposals relating to the TCI/QCL

RAN1#118-bis IDLE/INACTIVE Agreement Each LP-WUS is QCLed with one SSB. Each LP-SS is QCLed with one SSB. - FFS QCL Type A or Type C and/or Type D FFS implicit QCL determination or some signaling is required RAN1#119 IDLE/INACTIVE Agreement Each LP-WUS or LP-SS is QCLed with an SSB with [select one from ‘typeA’, ‘typeC’], and when applicable, ‘typeD’ with the same SSB. RAN1#117 CONNECTED Agreement For LP-WUS monitoring in RRC CONNECTED mode, a LP-WUS is QCLed with existing NR signal/channel/CORESET for the TCI state - FFS which existing NR signal/channel/CORESET is the QCL source of LP-WUS - FFS exact definition of QCL relationship between LP-WUS and existing NR signal/channel/CORESET RAN1#118 CONNECTED Agreement For LP-WUS monitoring in RRC CONNECTED mode, SSB and/or CSI-RS can be the QCL source of LP-WUS - FFS applicable QCL type(s) RAN1#119 CONNECTED Agreement For LP-WUS monitoring in RRC CONNECTED mode, SSB and/or CSI-RS can be the QCL source of LP-WUS QCL type(s) is {Type A or Type C – for down-selection} and Type D, when applicable #### [Open]Proposal 3.4-2: For LP-WUS monitoring in RRC CONNECTED mode, consider following options on how to define/configure/indicate TCI-state of LP-WUS - Opt.1: Reuse TCI-state framework to indicate TCI-state of LP-WUS - Opt.2: TCI-state association between LP-WUS and CORESET - Opt.3: Default TCI-state - Note: Other options are not precluded

During the RAN1#119 discussions, the following options for how to define/configure/indicate TCI-state of LP-WUS were presented.

- Opt.1: Reuse TCI-state framework to indicate TCI-state of LP-WUS

- Opt.2: TCI-state association between LP-WUS and CORESET

- Opt.3: Default TCI-state

Some companies have discussed if a default TCI state (option 3) should also be defined.   In our view, the only time when a different TCI state is required is when the selected beam can no longer maintain the required quality of service.  However, when this occurs, the existing beam failure recovery procedure (TS38.321 section 5) is expected to determine an alternative beam for the UE to use which will have its own coreset and TCI for the LP-WUS to exploit.

Observation 7:	The existing beam failure recovery procedure means that there is no need to define a default TCI state (option 3) for the LP-WUS.

Since the PDCCH CORESET is already associated with a TCI, the UE can reuse the existing TCI-state framework to determine the QCL relationship for the LP-WUS.  Currently the UE knows

Proposal 9:	The Network re-uses the TCI-state framework for the PDCCH Coreset associated with the LP-WUS (option 1), to determine the QCL relationship with LP-WUS.

The RRC configuration of LP-WUS, includes TCI state information specific to the LP-WUS using the framework/parameters (eg tci-StatesToAddModList) used to define TCI states for the PDCCH Coreset.  These LP-WUS specific tci-states can then be used to define the QCL source (SSB or CSI-RS) and corresponding QCL type to apply to the LP-WUS.

Proposal 10:	Support TCI-state information as part of the RRC configuration for LP-WUS

## 2.5	CSI/L1-RSRP Reporting Specification Impacts

In this section we share our thoughts on the RAN1 specification impacts of the RAN1#119 agreement (see below) regarding CSI/L1-RSRP reporting.

RAN1#119 Agreement For Option 1-2, for LP-WUS CONNECTED mode operation, the periodic CSI/L1-RSRP reporting operation is same as Rel-16 DCP, that is the UE can be configured with a parameter to enable/disable periodic CSI/L1-RSRP reporting, respectively: - If the parameter is NOT configured: - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is not reported. - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time. - If the parameter is configured: - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the time given by the configured drx-onDurationTimer in DRX-Config for the case when UE is outside C-DRX active time. - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time. - FFS: Whether RAN1 spec impact is required.

Before we identify specific RAN1 38.214 specification impacts, it is worth noting that RAN2 has agreed that for option 1-2, a new drx-onDurationTimer parameter is defined:

RAN2#127-bis, Hefei, Agreement For Option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2. In option 1-2, a new timer triggered by LPWUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LPWUS monitoring. When UE is in C-DRX active time, UE PDCCH monitoring behaviors related to other legacy DRX timers (except for drx-onDurationTimer) are not affected..

The key TS38.214 specification extract (subsection 5.2.2.5) where there maybe impacts, is copied below,

When DRX is configured, the UE reports a CSI report with the reportQuantity not set to ‘ssb-Index-RSRP’, ‘ssb-Index-SINR’, ‘ssb-Index-RSRP-Index’ or ‘ssb-Index-SINR-Index’ only if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement in DRX Active Time no later than CSI reference resource and drops the report otherwise. When DRX is configured, for a CSI report configuration containing a list of sub-configurations provided by csi-ReportSubConfigToAddModList, the UE reports a CSI report including one or more sub-reports only if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement in DRX Active Time, per sub-configuration, no later than CSI reference resource and drops the report otherwise, where the sub-configuration is the activated/triggered one for AP/SP-CSI reporting, or the configured one for P-CSI reporting. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitOtherPeriodicCSI to report CSI with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to quantities other than 'cri-RSRP', 'ssb-Index-RSRP', 'cri-RSRP- Index', and 'ssb-Index-RSRP- Index ' when drx-onDurationTimer is not started, the UE shall report CSI with the reportQuantity not set to ‘ssb-Index-SINR’ or ‘ssb-Index-SINR-Index’ during the time duration indicated by drx-onDurationTimer in DRX-Config also outside active time according to the procedure described in Clause 5.2.1.4 if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer in DRX-Config outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise. For a CSI report configuration containing a list of sub-configurations provided by csi-ReportSubConfigToAddModList, when the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitOtherPeriodicCSI to report CSI with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to quantities other than 'cri-RSRP', 'ssb-Index-RSRP', 'cri-RSRP- Index', and 'ssb-Index-RSRP- Index' when drx-onDurationTimer is not started, UE shall report a CSI report including one or more sub-reports only during the time duration indicated by drx-onDurationTimer in DRX-Config also outside active time according to the procedure described in Clause 5.2.1.4 if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer in DRX-Config outside DRX active time or in DRX Active Time, per sub-configuration, no later than CSI reference resource and drops the report otherwise, where the sub-configuration is the configured one for P-CSI reporting. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitPeriodicL1-RSRP to report L1-RSRP with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to 'cri-RSRP', 'ssb-Index-RSRP', 'cri-RSRP- Index', or 'ssb-Index-RSRP- Index' when drx-onDurationTimer is not started, the UE shall report L1-RSRP during the time duration indicated by drx-onDurationTimer in DRX-Config also outside active time according to the procedure described in clause 5.2.1.4 and when reportQuantity set to 'cri-RSRP' or 'cri-RSRP- Index' if receiving at least one CSI-RS transmission occasion for channel measurement during the time duration indicated by drx-onDurationTimer in DRX-Config outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise.

In our view, similar text to that above, should be largely copied for LP-WUS option 1-2 but with specific changes to reflect:

- The UE being configured to monitor LP-WUS rather than DCI format 2_6

- The new option 1-2 specific drx-OnDuration Timer, which according the RAN2 agreements has been agreed.

Observation 8:	TS38.214 specifications require updates to reflect that LP-WUS can affect the periodic CSI/L1-RSRP reporting.

## 2.6	Coexistence with existing UE power saving features

In general, during Active Time for both options 1-1 and 1-2, we do not foresee any issues with the co-existence of LP-WUS and other legacy power saving features, such as PDCCH skipping and SSSG switching.

Proposal 11: 	For RRC CONNECTED mode, LP-WUS can be configured with PDCCH skipping

Proposal 12: 	For RRC CONNECTED mode, LP-WUS can be configured with SSSG switching.

At the RAN1#119 meeting, the question was raised, whether LP-WUS is used outside the Cell DTX active period.

In the box below, are extracts of TS38.321 relating to the operation of Cell DTX when not in the Active Period.

TS38.321 5.34.2 Cell Discontinuous Transmission For each Serving Cell configured with cell DTX, the MAC entity need not: 1> if cell DTX operation is activated and the Serving Cell is not in the cell DTX Active Period: 2> monitor PDCCH for the MAC entity's RNTIs listed in clauses 5.7 and 5.7b, irrespective of the requirements of clauses 5.7 and 5.7b, unless stated otherwise in this clause; 2> instruct the physical layer to receive transport block on the DL-SCH of this Serving Cell according to a configured downlink assignment for SPS; 2> indicate the presence of a configured downlink assignment and deliver the stored HARQ information to the HARQ entity; 2> set the HARQ Process ID to the HARQ Process ID associated with the PDSCH duration of a configured downlink assignment; 2> consider the NDI bit for the HARQ process corresponding to the PDSCH duration of a configured downlink assignment to have been toggled for the configured downlink assignment. 5.7 Discontinuous Reception (DRX) The MAC entity may be configured by RRC with a DRX functionality that controls the UE's PDCCH monitoring activity for the MAC entity's C-RNTI, CI-RNTI, CS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, TPC-SRS-RNTI, AI-RNTI, SL-RNTI, SL-CS-RNTI, SL-PRS-RNTI, SL-PRS-CS-RNTI, SL Semi-Persistent Scheduling V-RNTI and cellDTRX-RNTI. When using DRX operation, the MAC entity shall also monitor PDCCH according to requirements found in other clauses of this specification. When in RRC_CONNECTED, if DRX is configured, for all the activated Serving Cells, the MAC entity may monitor the PDCCH discontinuously using the DRX operation specified in this clause; otherwise the MAC entity shall monitor the PDCCH as specified in TS 38.213 [6].

In our interpretation of the above specification extracts, since the legacy LP-WUS is sent with DCI format 2.6 scrambled by PS-RNTI and PS-RNTI is not listed in clause 5.7, we believe the legacy DCI-based WUS can be monitored outside the Cell-DTX Active Period.

Observation 9:	Legacy DCI format 2_6 based WUS can be monitored outside active time of cell DTX.

Given that LP-WUS is seen as an alternative to DCP WUS, LP-WUS should have at least the same level of flexibility as DCP DCI-based WUS with Cell-DTX, hence the following proposal.

Proposal 13:	LP-WUS can be monitored outside the cell DTX Active time.

## 2.7	Miscellaneous

The following note is included in the WID.

- Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

The note determines that for CONNECTED mode procedures, it is assumed that UE MR can either be in deep, light, or micro sleep. As the MR is assumed not to be in ultra-deep sleep, the MR frequency source (clocks) can be assumed to be operational. Furthermore, as we are considering CONNECTED mode operation, MR needs to be well synchronised so that it can receive PDSCH e.g. at the start of the Active time without the need for any further synchronisation (e.g. via reference signals). Therefore, it can be assumed that MR can provide synchronisation assistance to the LR, and/or LR can benefit from MR time and frequency sources.

Observation 10: 	For CONNECTED mode, LP-WUR time and frequency synchronisation can be provided by the MR.

Given the above observation, we believe that LP-SS is not needed during normal LR CONNECTED mode operation and that the LR time and frequency synchronisation is same as MRs.

Proposal 14: 	For CONNECTED mode, LP-SS is not needed during normal LP-WUR operation with LP-WUR.

It is also noted, that CONNECTED mode measurements (RRM/RLM/BFD/CSI) are performed by MR. As there is no explicit intent nor objective in the work item to change, offload or relax the MR based measurements in CONNECTED mode when LP-WUS is being monitored (contrary to IDLE/Inactive mode) the MR requirements can be kept unchanged.

Proposal 15: 	For CONNECTED mode, there is no change to how existing MR based measurements (RRM/RLM/BFD/CSI) are determined when LP-WUS is enabled.

The continued study of the use of the LP-WUS within the On-Duration itself, Option 1-3, should be deprioritized for the following reasons:

•	Limited TUs for the WI

•	Pre-existing mechanisms such as PDCCH Skipping or Search Space Switching, to reduce PDCCH monitoring in CONNECTED mode.

Proposal 16:     	Option 1-3 is deprioritised.

# 3.   Conclusions

In this document, we have discussed several different aspects of CONNECTED mode LP-WUS design.  From those discussions, we have the following observations and proposals.

OBSERVATIONS

Observation 1:		For both options, approach 1 gives the network scheduler the most flexibility.

Observation 2:		For both options, approach 2 can be considered as a special case of approach 1.

Observation 3:	A unified approach for specifying MOs for options 1-1 and 1-2, will simplify 3GPP specifications.

Observation 4:	Option 1-1 is a special case of Option 1-2, where the UE is restricted to monitoring the first set MO(s) that immediately precede the start of drx-onDurationTimer by the smallest period that is no less than the configured minimum time gap

Observation 5:    	For RRC CONNECTED mode, the key difference between an ACTIVATED SCell that is in a DORMANT state and one that is a NON-DORMANT state, is that the NON-DORMANT SCell supports both measurements and PDCCH monitoring whereas the DORMANT SCell supports only a minimal set of measurements.

Observation 6:	Measurements in dormant SCells are unaffected by LP-WUS operation.

Observation 7:	The existing beam failure recovery procedure means that there is no need to define a default TCI state (option 3) for the LP-WUS.

Observation 8:	TS38.214 specifications require updates to reflect that LP-WUS can affect the periodic CSI/L1-RSRP reporting.

Observation 9:	Legacy DCI format 2_6 based WUS can be monitored outside active time of cell DTX.

Observation 10: 	For CONNECTED mode, LP-WUR time and frequency synchronisation can be provided by the MR.

PROPOSALS

Proposal 1:		For LP-WUS MOs in connected mode for Option 1-1,

      - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

      - UE selectively monitors LP-WUS in the LP-WUS MOs identified by other parameters such as time offset prior to the start of drx-onDurationTimer.

Proposal 2:		For LP-WUS MOs in connected mode for Option 1-2,

      - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

      - UE monitors LP-WUS in the LP-WUS MOs and starts a new timer for PDCCH monitoring triggered by LP-WUS, after a time offset.

Proposal 3:                    For option 1-1, a RRC parameter is used to define the time window, within which the network has multiple consecutive MOs it can use to trigger PDCCH monitoring in the same C-DRX cycle.

FFS:   If consecutive MOs start in the symbol or slot proceeding the previous MO.

Proposal 4:	For option 1-2, a RRC parameter is used to define the time window, within which the network has multiple consecutive MOs it can use to trigger PDCCH monitoring in the same new PDCCH monitoring period.

FFS:   If consecutive MOs start in the symbol or slot proceeding the previous MO.

Proposal 5:                  	For the minimum time gap in a non-CA scenario, the UE reports one value from 2 candidate values.

FFS:   The exact set of candidate values {0.25, 0.5, 1, 2, 3, 6, 10, 20}ms as starting point

FFS:   Whether/how to consider the case of CA

Proposal 6:                      	For the minimum time gap in a CA scenario, RAN1 down-selects from:

Alt 1:     Mandate a value in the candidate set is for CA.

Alt 2:	     Expand the candidate set to include an additional value for CA.

Alt 3:     Providing a separate CA specific set of candidate values.

Proposal 7:	For the case when the UE is configured with NR-DC and/or CA without dual DRX groups in RRC CONNECTED mode,

- LP-WUS is configured on a serving cell per cell-group

- LP-WUS detection triggers PDCCH monitoring on all non-dormant activated cells within the cell-group.

Proposal 8:	For the case when UE is configured with NR-DC and/or CA with dual DRX groups in RRC CONNECTED mode,

        - LP-WUS is configured on a non-dormant serving cell per cell-group

        - LP-WUS detection triggers PDCCH monitoring on activated non-dormant serving cells of all DRX groups per cell-group.

Proposal 9:	The Network re-uses the TCI-state framework for the PDCCH Coreset associated with the LP-WUS (option 1), to determine the QCL relationship with LP-WUS.

Proposal 10:	Support TCI-state information as part of the RRC configuration for LP-WUS

Proposal 11: 	For RRC CONNECTED mode, LP-WUS can be configured with PDCCH skipping

Proposal 12: 	For RRC CONNECTED mode, LP-WUS can be configured with SSSG switching.

Proposal 13:	LP-WUS can be monitored outside the cell DTX Active time.

Proposal 14: 	For CONNECTED mode, LP-SS is not needed during normal LP-WUR operation with LP-WUR.

Proposal 15: 	For CONNECTED mode, there is no change to how existing MR based measurements (RRM/RLM/BFD/CSI) are determined when LP-WUS is enabled.

Proposal 16:     	Option 1-3 is deprioritised.

# 4.   References

[1]     RP-241824	Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

[2]     RAN1 Chair’s Notes            3GPP TSG RAN WG1 #119         Orlando, USA, November 2024

[3]     R1-2410906	FL summary #4 on LP-WUS operation in CONNECTED mode