3GPP TSG RAN WG1 #120		       R1-2500589

Athens, Greece, Feb 17th-21st, 2025

Agenda item:	5

Source: 	NEC

Title: 	Draft reply to LS on CSI-RS based measurements

Document for:	Discussion/Decision

Introduction

This contribution discusses the response to RAN 4 LS (R4-2420165) on CSI-RS-based measurements.

Discussion on LS on CSI-RS based measurements

In R4-2420165, RAN4 raised questions to RAN1 regarding the following CSI-RS based measurement priorities.

RAN4 outlined two questions that require an answer. They are as follows:

Question 1: RAN4 seeks clarification from RAN1 perspective regarding whether the collision scenario between configured DL reception and dynamic UL transmission includes CSI-RS resources used for L1-RSRP/SINR measurement and RLM/BFD/CBD measurement. Specifically, if semi-statically configured CSI-RS resources for above mentioned measurements collide with dynamically scheduled UL transmission in the SBFD slots or symbols, which action should the UE prioritize: performing measurements on the semi-statically configured CSI-RS or executing the dynamically scheduled UL transmission?

It is the understanding of NEC that the configured DL reception certainly includes CSI-RS resources used for L1-RSRP/SINR measurement and RLM/BFD/CBD measurement which are used for beam management. And the dynamically scheduled UL transmission should take priority over L1-RSRP/SINR measurement and RLM/BFD/CBD measurements that may occur simultaneously. Since the gNB is aware of previously scheduled measurements that the UE may be making and is still scheduling the dynamically scheduled UL, it must mean the UL is designed to have a higher priority than the default actions of the UE at the time.

Proposed answer to Q1: RAN1’s answer to Q1 in R2-2404014 is that the dynamically scheduled UL transmissions should take priority over performing measurements on the semi-statically configured CSI-RS including CSI-RS resources used for L1-RSRP/SINR measurement and RLM/BFD/CBD measurement.

In addition, RAN4 also wants clarifications on BC requirements for L1 intra-frequency and intra-frequency LTM measurements.

Question 2: RAN4 needs to understand from RAN1 perspective whether the above agreement on resource configuration and/or reporting configuration apply to CSI-RS based L1-RSRP/SINR measurement and RLM/BFD/CBD measurement.

NEC understands that the agreements outlined in the LS should be applied to CSI-RS-based L1-RSRP/SINR measurement and RLM/BFD/CBD measurement which are used for beam management. As CSI-RS based L1-RSRP/SINR measurement and RLM/BFD/CBD measurement are still required for SBFD aware UE. And the SBFD UE should always use the SBFD behaviors above any other pre-existing non-SBFD behaviour wherever possible.  This will extend onto the scenarios as outlined in the question.

Proposed answer to Q2: RAN1’s answer to Q1 in R2-2404014 is that the resource configuration and/or reporting configuration apply to CSI-RS based L1-RSRP/SINR measurement and RLM/BFD/CBD measurement.

Conclusions

The following are proposed answers to Q1 and Q2 raised in R2-2404014.

Proposed answer to Q1: RAN1’s answer to Q1 in R2-2404014 is that the dynamically scheduled UL transmissions should take priority over performing measurements on the semi-statically configured CSI-RS including CSI-RS resources used for L1-RSRP/SINR measurement and RLM/BFD/CBD measurement.

Proposed answer to Q2: RAN1’s answer to Q1 in R2-2404014 is that the resource configuration and/or reporting configuration apply to CSI-RS based L1-RSRP/SINR measurement and RLM/BFD/CBD measurement.